RTL Design Sherpa

STREAM Characterization Monbus Compression Report

June 2026

Table of Contents

1 Readme	4

2 compression/ — monbus half-beat compression characterization	4

2.1 What “compression” means here	4

2.2 Headline result (real captured traffic)	5

2.3 Live on-board confirmation (all-fixes bitstream)	5

2.4 Half-beat wire format (the 32-bit slot)	6

2.5 Bitstream under test	6

2.6 Reproduce	7

2.7 Files	8

2.8 See also	8

List of Tables

No tables in this document.

# 1	Readme

Generated: 2026-06-15

# 2	compression/ — monbus half-beat compression characterization

Companion to reports/perf/. Where perf measures DMA bus utilization, this measures how well the monbus compressor + half-beat packer shrink the monitor-event stream on real captured traffic.

- Format spec / golden codec: bin/TBClasses/monbus/monbus_compressor.py

- Source capture dataset: reports/compression_dataset/desc_axi_16desc_8ch_1MB.json

- Offline measurement tool: flows-stream-bridge/host/monbus_halfbeat_model.py

- On-board live measurement: flows-stream-bridge/host/hb_measure.py

## 2.1	What “compression” means here

Each monitor record is a 128-bit packet + 64-bit timestamp. The raw (Tier-0) escape costs 3 × 64-bit beats per record. The compressor removes that cost two ways:

- Tier-1 templating (64-bit codec): a CAM-hit record collapses to 1 beat. Structural ceiling = 1 - 1/3 = 66.7%, reached only when every record is a hit.

- Half-beat packing (32-bit codec): two compressible records share one beat (0.5 beat/record), raising the ceiling to 1 - 0.5/3 = 83.3%.

The realized reduction is dominated by the Tier-1 hit rate h, not the slot size:

reduction = h · (1 − b_t / 3)
    current  (b_t = 1.0):  h · 0.667     ceiling 66.7%
    half-beat(b_t = 0.5):  h · 0.833     ceiling 83.3%

So 80% reduction needs h ≈ 0.96. The point of measuring on real traffic is to find the actual h and the fraction of hits that fit a 32-bit half-slot (vs. needing a full 64-bit beat).

## 2.2	Headline result (real captured traffic)

Replaying the committed encoder CAM logic over the 682-record on-board capture (desc_axi_16desc_8ch_1MB.json, the descriptor-fetch read-bus monitor across 8 channels × 16 descriptors × 1 MB):

| Codec | Beats | Reduction | Notes |
| --- | --- | --- | --- |
| raw baseline | 2046 | — | 682 records × 3 beats |
| current 64-bit | 758 | 63.0 % | 1 beat/hit, 3/escape |
| half-beat 32-bit | 498 | 75.7 % | two hits share a beat |

Stream composition: h = 0.944 Tier-1 hit rate — 521 half-fit, 123 full-only, 38 raw escapes. Half-beat packing converts the 63.0 % codec into 75.7 % on this trace: a 12.7-percentage-point gain, or 260 fewer beats drained over UART for the same monitor coverage.

Why 75.7 % and not the 83.3 % ceiling: h = 0.944 caps half-beat at 0.944 × 0.833 = 78.6 %, and of the hits, 123/644 fit only a full 64-bit beat (event_data or delta_ts too wide for a 23-bit half-payload), plus lone-half round-up. The gap to ceiling is real-traffic structure, not codec overhead.

## 2.3	Live on-board confirmation (all-fixes bitstream)

The numbers above are the offline golden codec. The same codec is synthesized into the bitstream (USE_MON_COMPRESSION=USE_MON_HALFBEAT=1). hb_measure.py runs the compressed workload, drains the real on-chip trace SRAM, and decodes the FPGA-emitted 64-bit slots with the golden Decoder — every run below round-trips clean (decode=OK), proving the HW slot stream is bit-exact decodable. Monitor preset debug-compl, reset between runs:

| config | records | half-beat beats | raw-equiv | reduction |
| --- | --- | --- | --- | --- |
| 4desc_1ch_1MB | 16 | 23 | 48 | 52.1 % |
| 8desc_1ch_1MB | 32 | 35 | 96 | 63.5 % |
| 4desc_4ch_1MB | 64 | 94 | 192 | 51.0 % |
| 8desc_4ch_1MB | 125 | 140 | 375 | 62.7 % |
| 8desc_8ch_1MB | 212 | 261 | 636 | 59.0 % |

These live runs are warm-up-dominated and trace-depth-limited: the bulk-trace SRAM holds only 2048 beats, so the largest clean workload is ~200 records — small enough that the one-time CAM-miss escapes (one raw 3-beat escape per first occurrence of each (channel, event) template) are a large fraction of the stream. Adding channels makes this worse, not better: each channel contributes its own cold templates, so 8desc_8ch (8 cold templates × 8 channels) lands at 59.0 % while 8desc_1ch reaches 63.5 %. As the record count grows the escapes amortize and the realized reduction climbs toward the full-capture 75.7 % (682 records). The live data confirms the HW codec is correct and functional; the offline model on the full capture is the representative reduction figure.

## 2.4	Half-beat wire format (the 32-bit slot)

A half-pair beat is {tag[63:60]=0x4, slotA[59:30], slotB[29:0]} — two 30-bit slots. Each slot = sub[1:0] + idx[4:0] + 23-bit payload:

HALF-A (absolute):  sub=1 | idx[4:0] | delta_ts[9:0]      | event_data[12:0]
HALF-C (delta):     sub=2 | idx[4:0] | delta_ts[9:0]      | ed_delta[11:0] signed
NOP (lone-half pad):sub=0

- delta_ts < 1024 cycles (~10.2 µs @ 100 MHz) for either half form.

- HALF-A carries event_data < 2^13; HALF-C carries a signed ed_delta |·| < 2^12 for counters / sequential addresses.

- A record that is Tier-1 but fits neither half form stays a full 64-bit beat. A lone trailing half rounds up to a full beat (flushed before any full/raw beat and at end-of-stream).

These widths are locked in monbus_halfbeat_model.py (lines 50–62) and mirrored bit-exact by rtl/amba/shared/monbus_halfbeat_packer.sv.

## 2.5	Bitstream under test

All numbers are for the all-timing-fixes bitstream (see reports/perf/README.md — WNS +0.048 ns @ 100 MHz, xc7a100t), built with the compressor + packer enabled:

USE_MON_COMPRESSION = 1     # monbus_compressor (Tier-0/1)
USE_MON_HALFBEAT    = 1     # monbus_halfbeat_packer (32-bit slots)
USE_MON_CAM_PIPELINE= 1     # pipelined match CAM (timing)

The on-chip half-beat packer is bit-exact to the Python golden: val/amba/test_monbus_halfbeat.py and test_monbus_compressor.py verify the RTL emits the identical slot stream the model produces for the same record sequence. Therefore the offline reduction above equals what the FPGA emits for that record stream — the model is not an estimate, it is the codec.

## 2.6	Reproduce

source env_python

# 1. Offline, bit-exact, on the locked real capture (no board needed):
cd projects/NexysA7/stream_characterization/flows-stream-bridge
python3 - <<'PY'
import json, sys; sys.path.insert(0, 'host')
from monbus_halfbeat_model import classify, _print
doc = json.load(open('../reports/compression_dataset/desc_axi_16desc_8ch_1MB.json'))
recs = [(int(r['packet'], 16), int(r['timestamp'], 16)) for r in doc['records']]
_print(f'real capture ({len(recs)} records)', classify(recs))
PY

# 2. Synthetic envelope (event_data shape sweep, ceilings 66.7 / 83.3%):
python3 host/monbus_halfbeat_model.py

# 3. Live on-board: run the compressed workload, drain the trace SRAM,
#    decode with the golden, report reduction. Keep the workload small —
#    the bulk-trace SRAM is 2048 beats; a 1 MB debug-all run overflows it.
python3 host/hb_measure.py --port /dev/ttyUSB2 \
    --channels 1 --config 1desc_1ch_1MB --mon-config debug-compl

Trace-SRAM bound. hb_measure.py reads the 2048-beat bulk-trace SRAM and refuses to report on an overflowed trace (the write pointer wraps, so a partial count would be wrong). For a clean live count use a lighter --mon-config (e.g. perf-mon / debug-compl) and/or a small workload so the compressed beat count stays under 2048. For full-workload reduction, use the offline model on a per_source_capture dump (method 1) — it has no depth limit and is bit-exact to the RTL.

## 2.7	Files

| File | Source | Notes |
| --- | --- | --- |
| reports/compression_dataset/desc_axi_16desc_8ch_1MB.json | on-board capture, 682 records | locked dataset; see its README_COMPRESSION_DATASET.md |
| reports/compression_dataset/README_COMPRESSION_DATASET.md | — | full Tier-0/1 format handoff (CAM=32, idx=5b) |

## 2.8	See also

- reports/perf/README.md — DMA bus-utilization characterization.

- bin/TBClasses/monbus/monbus_compressor.py — locked codec (Tier-0/1 + half-beat).

- rtl/amba/shared/monbus_halfbeat_packer.sv — RTL half-beat packer.

- rtl/amba/shared/monbus_compressor.sv — RTL compressor (CAM_PIPELINE, HALF_BEAT_EN).