RTL Design Sherpa

STREAM Characterization DMA Performance Report

June 2026

Table of Contents

1 Readme	4

2 perf/ — STREAM DMA characterization runs	4

2.1 Files	4

2.1.1 matrix_2026-06-15 — all-timing-fixes bitstream	4

2.2 Picking the right efficiency number	5

2.3 Columns (current runner)	6

2.4 Methodology notes	7

2.5 Reproduce	7

List of Tables

No tables in this document.

# 1	Readme

Generated: 2026-06-15

# 2	perf/ — STREAM DMA characterization runs

CSV exports of FPGA characterization measurements. Methodology: see projects/NexysA7/stream_characterization/DMA_UTILIZATION_MEASUREMENT.md.

## 2.1	Files

| File | Source | Rows | Notes |
| --- | --- | --- | --- |
| matrix_2026-06-15.csv / .json | full default matrix | 40 | current — all-timing-fixes bitstream (see below) |
| char_matrix_2026-06-05.csv | full default matrix | 40 | desc x ch sweep at 1MB/desc (pre-cleanup, see note) |
| respdelay_sweep_2026-06-05.csv | resp-delay axis | 32 | 4 configs x 8 delays (0..64 cyc) |

### 2.1.1	matrix_2026-06-15 — all-timing-fixes bitstream

Captured on the bitstream that closes timing with WNS +0.048 ns at 100 MHz (xc7a100t), carrying the full set of timing/feature work: both CAM pipelines (monbus_cam_pipe route-bound match + monitor TRANS_CAM_PIPELINE with gated MAX-3 block margin), half-beat packing, monbus compression, the wrapping trace write pointer, and the pblock_monbus floorplan + timing-directive build.

Result: all 40 configs pass, datapath/bus end-to-end utilization sits in a 94.06–94.12 % band across the entire desc×ch matrix (engine ceiling, flat regardless of channel count or descriptor depth), and net bus throughput is 1435–1436 MB/s against the 1526 MB/s one-direction ceiling. The timing fixes change none of the functional throughput — they make the bitstream valid (positive slack) without regressing the 94.1 % engine efficiency measured on earlier builds.

The trace.overflow flag trips on the eight 16desc_* configs because the on-chip debug trace SRAM is only 2048 beats deep (16 desc × 128 beats > 2048). This is benign: it bounds only the captured waveform trace, not the axi_bus_meter counters or the timer window, so every efficiency number above is unaffected.

Note on the 2026-06-05 CSVs. These pre-date the runner cleanup documented below. Their throughput_MBps, E2E_util_pct, and overhead_delta_pp columns are host-wall-clock measurements that were polluted by the runner’s 50 ms poll-loop sleep and a wrong-by-2x bus-ceiling denominator. Read those three columns as host-observed end-to-end metrics, not as bus efficiency. The bus efficiency numbers in those files are datapath_{R,W,E2E}_pct (steady-state, burst-only window from the on-chip PMU), which are clean: 94.1% across every config and every resp-delay setting from 0 to 64 cycles. Newly captured CSVs (post-fix) should use the bus-side columns documented in the next section instead.

## 2.2	Picking the right efficiency number

The runner produces three independent throughput-ish numbers per config, and they answer different questions. Pick the one that matches the question being asked:

| Metric | Window | What it answers | Use it for |
| --- | --- | --- | --- |
| datapath_E2E_pct | first→last productive beat (on-chip PMU) | “When the engine is actively bursting, what fraction of cycles are productive?” | Engine capability ceiling — independent of descriptor pacing and host overhead. |
| bus_e2e_util_pct | full FPGA timer window (kick → done) | “Of every aclk cycle from kick to done, what fraction was a productive beat?” | True bus utilization for the workload (includes inter-descriptor gaps). |
| throughput_MBps (wall) | host wall clock, time.time() around kick→poll | “How long did the whole DMA take from the host’s perspective?” | End-to-end latency including UART poll cadence — not a bus metric. |

Quoting datapath_E2E_pct as “the engine’s efficiency” is the defensible headline. Quoting bus_e2e_util_pct is the defensible “workload as configured” number. Quoting throughput / theoretical_max where throughput is wall-clock and theoretical_max is single-bus peak is the wrong answer to almost every question.

## 2.3	Columns (current runner)

Run-identification:

- date, time: wall clock of the run

- config, channels, descriptors, desc_KB: workload axes

- mb_moved: net bytes the DMA moved end-to-end (R then W, so the on-bus traffic is 2× this).

- resp_delay_cycles (sweep only): per-beat R+B hold injected by axi_response_delay.

Bus-side (sourced from the on-chip harness timer + axi_bus_meter counters — no UART overhead, no wall-clock contamination):

- bus_time_s: cycles_total / aclk_hz. Span from descriptor kick to write-side done. The exact FPGA-side DMA duration.

- bus_throughput_MBps: mb_moved / bus_time_s. Net-bytes-moved divided by FPGA time. Compare to bus_max_net_moved_MBps.

- bus_max_one_dir_MBps: theoretical ceiling for one direction — aclk_hz * DATA_WIDTH_BYTES. At 100 MHz + 128b that’s 1600 MB/s.

- bus_max_net_moved_MBps: theoretical ceiling for net bytes moved end-to-end. Half of bus_max_one_dir_MBps because the DMA reads and writes each byte; both directions contend for the same engine budget. ~800 MB/s at the default operating point.

- bus_e2e_util_pct: productive_beats / cycles_total. Of every aclk in the FPGA timer window, what fraction was a productive beat. This is the bus efficiency number that’s robust against UART, poll cadence, and the wrong-denominator trap.

- datapath_{R,W,E2E}_pct: harness-reported steady-state productive% inside the first→last beat window per side (methodology def 2.1). Ignores any inter-descriptor gaps and warm-up cost. Headline metric for “what is the engine capable of when it’s running.”

- {R,W}_{prod,bp,starv}_pct: four-bucket breakdown of the data-phase window (methodology Section 3). prod + bp + starv + idle = 100%.

Host-side (kept for transparency; do not use as efficiency metrics):

- dma_time_s: time.time() - t0 across kick → poll-completion. Includes every UART CSR round-trip and the host’s poll sleep.

- throughput_MBps (wall): mb_moved / dma_time_s. Asymptotes upward as transfer size grows because the per-DMA host overhead amortizes — that’s a property of the host, not the engine.

## 2.4	Methodology notes

- The on-chip timer counts aclk cycles between descriptor kick and the moment done latches (write-side beat count reaches TIMER_EXPECTED_BEATS). The bus meters freeze at the same done signal, so the bucket sums equal the timer window exactly. That’s why dividing productive by either cycles_total (E2E) or the bucket sum (datapath) is consistent — the same window is being divided differently.

- dma_time_s was contaminated by a 50 ms host poll sleep until the runner cleanup; that’s now 1 ms by default and reconfigurable via --poll-interval-ms. Even at 1 ms, prefer bus_time_s for any efficiency math.

- For non-100 MHz bitstreams, pass --aclk-mhz so bus_time_s and the bus throughput ceiling are computed against the right clock.

## 2.5	Reproduce

cd flows-stream-bridge/host

# Full 40-config matrix (default 1ms poll, 100 MHz aclk):
python3 run_characterization.py --port /dev/ttyUSB2 --output matrix.json

# resp-delay axis (4 configs x 8 delays = 32 runs):
python3 run_characterization.py --port /dev/ttyUSB2 \
    --configs 1desc_1ch_1MB 16desc_1ch_1MB 16desc_4ch_1MB 16desc_8ch_1MB \
    --resp-delays 0,1,2,4,8,16,32,64 \
    --output respdelay.json

# Override host poll cadence / clock (e.g. for a 200 MHz build):
python3 run_characterization.py --port /dev/ttyUSB2 \
    --poll-interval-ms 0.5 --aclk-mhz 200 --output fast.json

The runner emits JSON; convert it to the column set above with:

cd flows-stream-bridge/host
python3 perf_json_to_csv.py ../../reports/perf/matrix.json \
    --out ../../reports/perf/matrix.csv --date 2026-06-15 --time 13:58:32

perf_json_to_csv.py maps one CSV row per result dict (bus-side columns first, host-side last). The --date/--time flags only stamp the run-identification columns; all measurements come from the JSON.