RTL Design Sherpa

CELL Micro-Architecture Specification 0.12

October 19, 2026

Table of Contents

1 Cell Mas Index	9

2 Document Information	10

2.1 Overview	10

2.1.1 CELL Micro-Architecture Specification	10

2.1.2 Document Purpose	10

2.1.3 Intended Audience	10

2.2 Design Notes	11

2.2.1 Revision History	11

2.3 References	13

2.3.1 Related Documents	13

3 Design Philosophy and Scope	14

3.1 What This Block Is	14

3.2 Scope and Deferrals	14

3.3 MAS Decisions at a Glance	15

3.4 Relationship to the ALU	16

4 Module Tree	18

4.1 Purpose	18

4.2 Module Tree	18

4.3 Block Intent (one paragraph each)	18

4.3.1 delta_mesh_cell	18

4.3.2 cell_rx	18

4.3.3 cell_cfg_regs	19

4.3.4 cell_mem_in	19

4.3.5 cell_mem_out	19

4.3.6 cell_credit	19

4.3.7 cell_packetizer	19

4.3.8 cell_exec_fsm	19

4.4 Parameters	20

4.5 Clock and Reset	20

5 Datapath Figures	22

5.1 Whole-Cell Datapath	22

5.1.1 Figure 2.1: Whole-cell datapath	22

5.2 Credit Loop	23

5.2.1 Figure 2.2: Credit loop	23

5.3 ALU Datapath Focus	24

5.3.1 Figure 2.3: ALU datapath focus	24

6 External and Internal Interfaces	26

6.1 Cell Boundary	26

6.1.1 Mesh-Facing Ports	26

6.1.2 Local Port Contract	27

6.1.3 AXI4-Lite Register Interface	27

6.1.4 Observability	27

6.2 Internal Interfaces	28

6.2.1 cell_rx → cell_mem_in	28

6.2.2 cell_rx → cell_cfg_regs	28

6.2.3 cell_cfg_regs → datapath	29

6.2.4 cell_mem_in → cell_exec_fsm	30

6.2.5 cell_exec_fsm → cell_mem_out	30

6.2.6 cell_exec_fsm → cell_credit	30

6.2.7 cell_credit → node (credit VC)	31

6.2.8 cell_mem_out → cell_packetizer	31

6.2.9 cell_packetizer → node (data network)	31

6.2.10 cell_exec_fsm ↔ ALU	31

7 Cycle Behavior: Fill Path as Built	32

7.1 Slice-1 Fill Path	32

7.1.1 Head-Beat Reception	32

7.1.2 Coordinate Match	32

7.1.3 Demux Decision	32

7.1.4 Payload Write Timing	33

7.1.5 Occupancy Accounting	33

7.2 Expected Slice-2/3 Cycle Behavior	33

8 Credit Logic	35

8.1 MAS Decision: Memory Sizing and Advertisement	35

8.2 MAS Decision: Credit Batch Size and Timeout	35

8.3 Per-Source Owed Counters	35

8.4 Credit Packet Format	36

8.5 Boot Advertisement Packet	36

8.6 Sender-Generic Returns	37

8.7 Implementation Note: Counter Bank Sizing	37

9 Packetizer and Execution FSM	38

9.1 Packetizer	38

9.1.1 MAS Decision: Batching and Flush Conditions	38

9.1.2 MAS Decision: Strap Counter Initialization	38

9.1.3 Header Build	38

9.1.4 Destination and Tag Programming	39

9.1.5 Strap Accounting	39

9.2 Execution FSM	40

9.2.1 Pop → Issue → Push Loop	40

9.2.2 MAS Decision: Operand-Source Mux	40

9.2.3 IP/OP Pointer Behavior	40

9.2.4 Gatling as the Cell ALU Sequencer (owner ruling 2026-10-19)	41

9.2.5 ALU Hookup	41

9.2.6 Issue Tag and Same-Tag Alignment (ratified spec — RTL pending)	42

9.2.7 MAC Accumulation	42

9.2.8 Soft Reset	42

10 Verification Plan	43

10.1 House Test Grid	43

10.2 Existing Tests	43

10.3 Planned Slice-2 Credit-Loop Test	43

10.4 Planned Slice-3 ALU-Run Test	44

10.5 Verification Hooks	44

10.6 Assertions and Checks	45

11 Register Map and MAS Decisions	46

11.1 Register Map	46

11.2 Bit Fields	47

11.2.1 CTRL (0x00)	47

11.2.2 STATUS (0x04) — read-only	48

11.2.3 FUNC_SEL (0x08)	48

11.2.4 OP_SRC (0x34)	48

11.2.5 IRQ_STATUS / IRQ_EN	49

11.3 MAS Decisions Revisited	49

11.4 RTL Naming Divergence (slice 1)	50

12 References	51

12.1 Normative Specifications	51

12.2 RTL Sources	51

12.3 Verification	52

12.4 House Style and Build	52

List of Figures

Figure 2.1: Whole-cell datapath	22

Figure 2.2: Credit loop	23

Figure 2.3: ALU datapath focus	24

List of Tables

Table 1: Document information	10

Table 2: Revision history	11

Table 3: Table 1.1: Summary of MAS decisions	15

Table 4: Table 2.1: cell_top parameters	20

Table 5: Table 3.1: Mesh-facing AXI4-Stream ports	26

Table 6: Table 3.2: Observability outputs	27

Table 7: Table 3.3: Input-memory write port	28

Table 8: Table 3.4: Config-stream register interface	28

Table 9: Table 3.5: Register outputs to datapath	29

Table 10: Table 3.6: Input-memory read port	30

Table 11: Table 3.7: Result-memory write port	30

Table 12: Table 3.8: Consumed-line credit strobe	30

Table 13: Table 3.9: Result-memory read port	31

Table 14: Table 4.1: Slice-1 head-beat dispatch	32

Table 15: Table 4.2: Result-packet header fields	38

Table 16: Table 4.3: OP_SRC encoding	40

Table 17: Table 4.4: ALU↔FSM seam signals	41

Table 18: Table 5.1: House test grid	43

Table 19: Table 5.2: Verification hooks	44

Table 20: Table 6.1: Cell register map	46

Table 21: Table 6.2: CTRL bit fields	47

Table 22: Table 6.3: STATUS bit fields	48

Table 23: Table 6.4: FUNC_SEL fields	48

Table 24: Table 6.5: OP_SRC fields	48

Table 25: Table 6.6: IRQ fields	49

Table 26: Table 6.7: MAS decisions and their hardware homes	49

# 1	Cell Mas Index

Generated: 2026-10-19

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	Overview

### 2.1.1	CELL Micro-Architecture Specification

Document information

| Property | Value |
| --- | --- |
| Document Title | CELL Micro-Architecture Specification |
| Version | 0.12 |
| Date | 2026-10-19 |
| Status | Draft — slice-1 RTL built; slice-2/3 modules specified |
| Classification | Open Source - MIT License |

### 2.1.2	Document Purpose

This Micro-Architecture Specification (MAS) documents how the compute cell is built and how the remaining slice-2/3 work completes it:

- The slice-1 module tree (cell_top, cell_rx, cell_cfg_regs, cell_mem_in, cell_mem_out) as it exists today

- The slice-2/3 modules (cell_credit, cell_packetizer, cell_exec_fsm) that are specified here

- The MAS decisions that close the PRD §12 open questions

- The cycle-level behavior of the fill path as built and the expected consume/emit path

- The verification plan for the credit loop and ALU-run tests

For what the cell must do and why, see the cell PRD (normative for the cell product). For the packet, credit, and coordinate protocols the cell consumes, see TILE_SPEC §11. For the ALU block itself, see the alu-infer PRD and alu-infer MAS.

### 2.1.3	Intended Audience

- RTL designers implementing the slice-2/3 modules inside cell/rtl/

- Verification engineers extending cell/dv/tests/test_cell.py

- Design reviewers checking the future implementation against intent

- Students reading the cell datapath top to bottom

## 2.2	Design Notes

### 2.2.1	Revision History

Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.11 | 2026-10-18 | RTL Design Sherpa | Beat-chained header rule (owner ruling): the 128-bit tile header occupies the first ceil(128/DW) payload beats, LSB-first; every link keeps its own width. Local port stays at delta DATA_WIDTH (64b) — supersedes 0.10’s 128-bit local-port widening. TILE_SPEC v1.6+ binding. |
| 0.12 | 2026-10-19 | RTL Design Sherpa | (addendum) Owner rulings 2026-10-18/19: local-port width stays at delta DATA_WIDTH (64b) with beat-chained header; strap terminology replaces grants in registers/PRD; DEST reserved, FLUSH_DEST/FLUSH_TAG are the operational drain destination; credit-class encoding added; gatling sequences the cell ALU. Slice-1 RTL identifiers retain pre-ruling grant names (noted in ch06). |
| 0.11 | 2026-10-18 | RTL Design Sherpa | (same-day addendum) ALU-focus datapath figure (fig 2.3, no delta side) + issue-tag / same-tag-alignment MAS decision in ch04 §4.4 — ratified owner ruling; tag assign/FIFO/align gate not yet in alu-infer RTL. |
| 0.10 | 2026-10-18 | RTL Design Sherpa | First issue. Slice-1 RTL built; slice-2/3 specified. Closes PRD §12 open questions with MAS decisions: IN_DEPTH=OUT_DEPTH=32 lines; CREDIT_BATCH_N default 1; 64-cycle partial-batch credit timeout; EMIT_BATCH_X flush on batch-full or CTRL.flush-and-send; strap counter default 16; local-port data width widens to 128 bits to carry the §11.6 tile header. |

## 2.3	References

### 2.3.1	Related Documents

- cell PRD (../../../PRD.md) — product definition, normative for the cell

- TILE_SPEC §11 (../../../../tile/docs/TILE_SPEC.md) — packet header, transport, credits/grants, coordinates

- alu-infer PRD (../../../../alu-infer/PRD.md) — normative definition of the ALU block

- alu-infer MAS (../../../../alu-infer/docs/alu_infer_mas/alu_infer_mas_index.md) — implementation-intent book for the ALU block

- delta_mesh_cell (../../../../../fabric-gen-ip/delta/rtl/delta_mesh_cell.sv) — the node the cell instantiates

Last Updated: 2026-10-19

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Design Philosophy and Scope

## 3.1	What This Block Is

The cell is the tile’s compute unit: sixteen of them make a tile. Each cell contains exactly one delta_mesh_cell node, instantiated inside cell_top, with its local port (port index 8, P_LOCAL) facing the cell’s internal datapath and its eight mesh-facing ports passing through to the cell boundary. That containment is the entire cell↔delta relationship: the node is governed by delta’s own HAS/MAS, and everything else in the cell is the cell’s own design, speaking the tile-level contracts of TILE_SPEC §11.

The cell’s job is to turn delta-delivered operand lines into result lines that leave back through the delta mesh toward RAPIDS:

- Program. cfg packets write the §7 registers — FUNC_SEL, OP_SRC, batching knobs, and the persistent operands A–D, T.

- Fill. mem packets write operand lines into the input memory, each line tagged with the packet’s source coordinate.

- Consume. The execution FSM pops OP_LINES input lines per invocation, issues them to the ALU, and pushes result lines into the result memory.

- Credit return. As each input line is consumed, the credit logic returns credits to the recorded source.

- Emit. The packetizer batches result flits into packets and sends them toward the gatling-programmed drain destination, spending the cell’s TILE_LAND_FLITS strap counter.

Everything — data, config, and programming — arrives in-band on delta (PRD §3; TILE_SPEC §11.13; alu-infer PRD §5).

## 3.2	Scope and Deferrals

In scope for this MAS:

- The slice-1 RTL as built: cell_top, cell_rx, cell_cfg_regs, cell_mem_in, cell_mem_out

- The slice-2/3 modules specified here: cell_credit, cell_packetizer, cell_exec_fsm

- The ALU↔FSM seam (the ALU itself is specified in the alu-infer MAS)

- The credit-loop and result-packetization behavior

- The verification plan for the credit-loop and ALU-run tests

Out of scope or deferred (PRD §10–11):

- Tetrad/root node internals and the tetrad control core

- RAPIDS internals and the CXS migration sequencing

- Fabric-level Z stacking

- Cell-destined result sending (DEST override) pending the tile-spec cell→cell grant extension

- Loading A–D from the input memory

- Wider-than-32-bit lines and element widths other than bf16

## 3.3	MAS Decisions at a Glance

This book closes the PRD §12 open questions with the following implementation decisions, each marked “MAS decision” with rationale in the chapter where it lives:

Table 1.1: Summary of MAS decisions

| Decision | Value | Rationale |
| --- | --- | --- |
| Input memory depth (IN_DEPTH) | 32 lines | Matches result memory depth; gives a 32-flit advertisement today; simple power-of-two addressing |
| Result memory depth (OUT_DEPTH) | 32 lines | Sized to absorb one full input run’s results at the default OP_LINES=1; deeper than the default TILE_LAND_FLITS strap so the memory is not the emit bottleneck |
| Line width (LINE_W) | 32 bits (2×bf16) | bf16 element model per alu-infer PRD §2; slice-3 may pack two lines per 64-bit flit |
| Boot advertisement | 32 flits | One 32-bit line per 64-bit payload flit today; the value is IN_DEPTH expressed in flits |
| Credit batch size (CREDIT_BATCH_N) | Default 1 | Exact per-line behavior; configurable for lower fabric traffic |
| Partial-batch credit timeout | 64 cycles | Bounded drain latency without adding credit traffic at N=1 |
| Result batch size (EMIT_BATCH_X) | Configurable, default 4 | Amortizes header overhead while respecting the 16-flit strap |
| Packetizer flush conditions | Batch-full OR CTRL.flush-and-send | Simple, deterministic; idle-timeout and end-of-job flush are deferred |
| Strap counter default | 16 flits | Mirrors the Hive default TILE_LAND_FLITS; restored by RAPIDS credit/add returns |
| Local-port data width (slice 2/3) | Delta DATA_WIDTH (64b today) | Beat-chained header rule (owner ruling 2026-10-18): the 128-bit tile header spans the first ceil(128/DW) payload beats, LSB-first; no widening, no width conversion. Supersedes the 0.10 draft’s 128-bit local port |

## 3.4	Relationship to the ALU

The cell owns the memories, the FSM, the packetizer, and the credit logic. The ALU block — alu-infer — is the cell’s compute engine and is governed by its own PRD and MAS. The cell MAS cites the alu-infer contracts at the FSM seam and does not restate the ALU internals.

Last Updated: 2026-10-18

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Module Tree

## 4.1	Purpose

cell_top is the compute cell top-level. It instantiates one delta_mesh_cell node and the cell’s own datapath blocks. The module tree below shows the slice-1 blocks that are built and the slice-2/3 blocks that this MAS specifies.

## 4.2	Module Tree

cell_top (built, slice 1)
├── delta_mesh_cell (black box — fabric-gen-ip/delta)
├── cell_rx (built, slice 1) — header parse / demux from the node's local port
├── cell_cfg_regs (built, slice 1) — AXIL slave + in-band cfg-stream register file
├── cell_mem_in (built, slice 1) — input operand memory with per-line source tag
├── cell_mem_out (built, slice 1) — result memory
├── cell_credit (specified, slice 2) — per-source owed counters + credit packet builder
├── cell_packetizer (specified, slice 2) — result-flit batching + §11.5 header builder
└── cell_exec_fsm (specified, slice 3) — pop → issue → push loop + ALU hookup

The ALU block itself (alu-infer) is instantiated inside cell_exec_fsm (or as a sibling driven by it); its specification is owned by the alu-infer PRD and MAS.

## 4.3	Block Intent (one paragraph each)

### 4.3.1	delta_mesh_cell

The network attachment. The cell instantiates one node; its local port (P_LOCAL = 8) delivers traffic to cell_rx, and its eight mesh ports pass through to the cell boundary. The node is a black box owned by fabric-gen-ip/delta; its HAS/MAS govern it. The cell depends only on the node’s local-port AXI4-Stream contract and on the node’s ability to route packets by destination coordinate.

### 4.3.2	cell_rx

Header parse and demux for the node’s local port. Slice 1 already parses a provisional 64-bit TUSER packing (because the instantiated delta node exposes a 64-bit TUSER) and routes mem payloads to the input memory, cfg writes to the register file, and credit packets to a counted TODO drop. Slice 2/3 reworks cell_rx to the §11.6 transport: delta’s route header rides TUSER, and the 128-bit tile header is the first data beat. Every input-memory line is tagged with the packet’s source coordinate so credit returns can be addressed correctly.

### 4.3.3	cell_cfg_regs

AXI4-Lite slave plus in-band cfg-stream register file per PRD §7. The register map is defined in Chapter 6. The block exposes register values to the execution FSM and packetizer, generates the one-cycle soft_rst pulse, and assembles STATUS and IRQ_STATUS.

### 4.3.4	cell_mem_in

Input operand memory. Sized to IN_DEPTH lines; each line carries a source-coordinate tag. It has one write port (from cell_rx) and one read port (from cell_exec_fsm). The read pointer IP is owned by the FSM; occupancy is exposed for STATUS.

### 4.3.5	cell_mem_out

Result memory. Sized to OUT_DEPTH lines; one write port (from cell_exec_fsm) and one read port (from cell_packetizer). The write pointer OP is owned by the FSM; occupancy is exposed for STATUS.

### 4.3.6	cell_credit

Credit-return logic. Maintains per-source owed counters keyed by the source coordinate recorded on each consumed input line. Batches returns to N = CREDIT_BATCH_N consumed lines per source; drains a partial batch after a 64-cycle timeout. Builds header-only credit/add packets addressed to the recorded source and sends them back through the node on the credit VC. Sends the boot credit/set advertisement to level 3 carrying the input memory’s size in flits.

### 4.3.7	cell_packetizer

Result drain and packetizer. Batches result flits up to EMIT_BATCH_X; flushes on batch-full or on CTRL.flush-and-send. Builds the §11.5 tile header with destination and tag supplied by the gatling-programmed drain command, and sends the packet through the node only while the TILE_LAND_FLITS strap counter covers the whole packet. Spends the strap counter per flit sent (header included) and restores it as RAPIDS returns credit/add packets.

### 4.3.8	cell_exec_fsm

Execution FSM and ALU hookup. Runs the pop→issue→push loop: pops OP_LINES input lines per invocation (advancing IP), presents them to the ALU’s valid/ready elastic pipeline, and pushes each retired result line into the result memory (advancing OP). Selects operand sources per OP_SRC (input-memory pop or one of A/B/C/D; T backs the threshold immediate). Clears IP/OP on CTRL.soft_reset.

## 4.4	Parameters

Table 2.1: cell_top parameters

| Parameter | Default | Meaning |
| --- | --- | --- |
| LINE_W | 32 | Line width in bits; one line is two bf16 lanes |
| COORD_W | 4 | Coordinate sub-field width; coordinate is {ext, level, X, Y, Z} |
| POS_X/Y/Z | 0 | This cell’s position in the tile grid |
| IN_DEPTH | 32 | Input memory depth in lines |
| OUT_DEPTH | 32 | Result memory depth in lines |
| AXIL_ADDR_WIDTH | 32 | AXIL address width |
| AXIL_DATA_WIDTH | 32 | AXIL data width |
| DATA_WIDTH | 64 | Cell↔delta local-port data width. Stays at the delta node’s DATA_WIDTH (64 today); the 128-bit §11.6 tile header is beat-chained LSB-first across ceil(128/DATA_WIDTH) beats. No local-port widening. |
| USER_WIDTH | 64 | Delta route header width; unchanged until delta widens it |
| NUM_VC | 2 | Number of delta virtual channels |

## 4.5	Clock and Reset

Single clock domain, active-low async aresetn (house ALWAYS_FF_RST), matching the tile’s single-clock-domain decision (TILE_SPEC §9). Reset clears the memories, registers, pointers, and credit/strap state; CTRL.soft_reset clears only IP, OP, IRQ_STATUS, and selected datapath state.

Last Updated: 2026-10-19

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Datapath Figures

## 5.1	Whole-Cell Datapath

### 5.1.1	Figure 2.1: Whole-cell datapath

Figure 2.1: Whole-cell datapath

The figure follows the house-style rule that three flows must be traceable at a glance:

- Programming/config flows down the center: cfg packets enter through the delta node, are demuxed by cell_rx, and write the config registers; the registers in turn configure the FSM, the ALU, and the packetizer.

- Operand fill flows down the left: mem payloads are written into the input memory and are later popped by the execution FSM.

- Results and credit returns flow back up the right: result lines move from the ALU into the result memory, through the packetizer, and back out through the delta node; consumed-line credits return on their own dashed path up to the node.

The boundary bar is the delta node edge. Every arrow is labeled with the transaction class it carries; sideband/config arrows are dashed.

## 5.2	Credit Loop

### 5.2.1	Figure 2.2: Credit loop

Figure 2.2: Credit loop

The credit-loop figure shows the sender-generic return path:

- An input line arrives from a source and is tagged with that source’s coordinate.

- The execution FSM consumes the line and strobes cell_credit.

- cell_credit increments a per-source owed counter and, when the batch threshold CREDIT_BATCH_N is reached (or the 64-cycle timeout expires), sends a header-only credit/add packet addressed to the recorded source.

- The packet travels back through the delta node to the source, which restores its per-destination credit counter.

The path never shares an arrow with result data; credits are a separate sideband VC.

## 5.3	ALU Datapath Focus

### 5.3.1	Figure 2.3: ALU datapath focus

Figure 2.3: ALU datapath focus

The ALU-focus figure drops the delta/network side and traces the three cell-internal flows around alu_infer_top:

- Operand fill (left): cell_mem_in pops (IP++) and the persistent A·B·C·D/T registers feed the execution FSM, which owns the OP_SRC operand mux. Config (OP_SRC, IP/OP, CTRL) arrives dashed from §7.

- Issue (center, heavy pen): the FSM issues the bundle (op_line_a/b, func_sel, acc_ctrl, threshold_imm) with alu_valid; the tag assign gives the invocation group its 8-bit tag and the tag FIFO tracks in-flight groups (ratified spec — RTL pending). Rows multiply in bf16 exactly, the fp32 MAC accumulates per-lane under acc_ctrl, and the align gate holds result_valid until every same-tag op’s valid has asserted.

- Result flow (right): result_line returns up its own route through the ALU boundary bar (result_valid/result_ready, in-order, II=1 while draining), and the FSM pushes the accepted lines down to cell_mem_out (OP++).

The single boundary bar is the alu_infer_top edge; no flow shares an arrow with another.

Last Updated: 2026-10-18

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	External and Internal Interfaces

## 6.1	Cell Boundary

### 6.1.1	Mesh-Facing Ports

cell_top exposes the eight mesh-facing ports of the instantiated delta_mesh_cell node:

Table 3.1: Mesh-facing AXI4-Stream ports

| Port | Direction | Width | Purpose |
| --- | --- | --- | --- |
| s_axis_tdata[8] | in | DATA_WIDTH | Data from mesh neighbors |
| s_axis_tvalid[8] | in | 1 | Valid from mesh neighbors |
| s_axis_tready[8] | out | 1 | Ready to mesh neighbors |
| s_axis_tlast[8] | in | 1 | Packet last from mesh neighbors |
| s_axis_tuser[8] | in | USER_WIDTH | Delta route header from mesh neighbors |
| m_axis_tdata[8] | out | DATA_WIDTH | Data to mesh neighbors |
| m_axis_tvalid[8] | out | 1 | Valid to mesh neighbors |
| m_axis_tready[8] | in | 1 | Ready from mesh neighbors |
| m_axis_tlast[8] | out | 1 | Packet last to mesh neighbors |
| m_axis_tuser[8] | out | USER_WIDTH | Delta route header to mesh neighbors |
| s_credit[8] | in | NUM_VC | Per-VC credit levels from neighbors |
| m_credit[8] | out | NUM_VC | Per-VC credit levels to neighbors |

These ports are pass-through wiring in cell_top; the node performs all routing. Cell-bound traffic must be addressed to the cell’s external ID {ext=1, level=0, X, Y, Z} and ride delta QoS ≠ 0 so the node does not consume it as its own config traffic (TILE_SPEC §11.2).

### 6.1.2	Local Port Contract

The cell’s internal datapath sees only the node’s local port (P_LOCAL = 8). Slice 1 used a provisional 64-bit TUSER packing for the tile header because the instantiated delta node exposes 64-bit TUSER.

MAS decision: Slice 2/3 moves to the TILE_SPEC §11.6 transport binding, with the beat-chained header rule (owner ruling 2026-10-18): every link keeps its own data width, and the 128-bit tile header occupies the first ceil(128/DW) payload beats, LSB-first.

- The delta route header rides TUSER of the head beat, as delta owns it.

- The 128-bit tile header is payload: on the 64-bit local port it spans beats 0–1 (tdata[63:0] = header [63:0], next beat’s tdata[63:0] = header [127:64]), then payload beats follow. On a 128-bit-or-wider link it is beat 0 alone.

- The cell’s local-port data width stays at the delta node’s DATA_WIDTH (64 bits today) — no widening, no width conversion anywhere. The demux reassembles the two header beats; the packetizer emits the header across two beats.

This supersedes slice-1’s provisional 64-bit TUSER packing and the earlier MAS 0.10 draft’s 128-bit local-port widening.

### 6.1.3	AXI4-Lite Register Interface

The cell exposes a house-standard AXI4-Lite slave for debug and local control. All registers are also writable from the in-band cfg stream. The register map is in Chapter 6.

### 6.1.4	Observability

Table 3.2: Observability outputs

| Signal | Width | Purpose |
| --- | --- | --- |
| o_err_drop_cnt | 32 | Dropped-head count from the delta node |
| o_todo_credit_drop_cnt | 32 | Slice-1 TODO counter for unhandled credit packets |
| o_todo_cfg_read_drop_cnt | 32 | Slice-1 TODO counter for unhandled cfg read replies |
| o_irq | 1 | Interrupt output (level) |
| MonBus (AXI4-Lite) | - | The cell’s one MonBus stream, merged at the tetrad with the other three cells and the tetrad node (tile spec §5). Not yet in the RTL |

## 6.2	Internal Interfaces

### 6.2.1	cell_rx → cell_mem_in

Table 3.3: Input-memory write port

| Signal | Width | Direction | Purpose |
| --- | --- | --- | --- |
| mem_wr_valid | 1 | rx → mem | Write valid |
| mem_wr_ready | 1 | mem → rx | Write ready |
| mem_wr_addr | log2(IN_DEPTH) | rx → mem | Line index |
| mem_wr_data | LINE_W | rx → mem | Line data |
| mem_wr_src_coord | COORD_BITS | rx → mem | Source coordinate tag |

### 6.2.2	cell_rx → cell_cfg_regs

Table 3.4: Config-stream register interface

| Signal | Width | Direction | Purpose |
| --- | --- | --- | --- |
| cfg_wr_valid | 1 | rx → regs | In-band cfg write valid |
| cfg_wr_ready | 1 | regs → rx | In-band cfg write ready |
| cfg_wr_addr | 8 | rx → regs | Register offset |
| cfg_wr_data | 32 | rx → regs | Register value |
| cfg_rd_valid | 1 | rx → regs | In-band cfg read request valid (slice-2 completion) |
| cfg_rd_addr | 8 | rx → regs | Register offset |
| cfg_rd_data | 32 | regs → rx | Read data (slice-2 completion) |
| cfg_rd_ready | 1 | rx → regs | Read request accepted (slice-2 completion) |

### 6.2.3	cell_cfg_regs → datapath

The register block drives the following register values:

Table 3.5: Register outputs to datapath

| Signal | Width | Purpose |
| --- | --- | --- |
| r_ctrl | 32 | CTRL register |
| r_func_sel | 32 | FUNC_SEL register |
| r_op_lines | 32 | OP_LINES register |
| r_emit_batch_x | 32 | EMIT_BATCH_X register |
| r_credit_batch_n | 32 | CREDIT_BATCH_N register |
| r_irq_en / r_irq_status | 32 | IRQ enables and status |
| r_a / r_b / r_c / r_d / r_t | 32 | Persistent ALU operands and threshold |
| r_op_src | 32 | Operand-source mux selection |
| r_dest | 32 | Deferred cell-destined destination |
| r_ip / r_op | 32 | Input/result memory pointers |
| o_soft_rst | 1 | One-cycle soft-reset pulse |

### 6.2.4	cell_mem_in → cell_exec_fsm

Table 3.6: Input-memory read port

| Signal | Width | Direction | Purpose |
| --- | --- | --- | --- |
| mem_in_rd_valid | 1 | fsm → mem | Read request valid |
| mem_in_rd_ready | 1 | mem → fsm | Read request ready |
| mem_in_rd_addr | log2(IN_DEPTH) | fsm → mem | Read address |
| mem_in_rd_data | LINE_W | mem → fsm | Read data (registered) |
| mem_in_rd_src_coord | COORD_BITS | mem → fsm | Source tag of read line |
| mem_in_empty | 1 | mem → fsm/reg | Memory empty |
| mem_in_full | 1 | mem → fsm/reg | Memory full |
| mem_in_occupancy | log2(IN_DEPTH+1) | mem → fsm/reg | Line occupancy |

### 6.2.5	cell_exec_fsm → cell_mem_out

Table 3.7: Result-memory write port

| Signal | Width | Direction | Purpose |
| --- | --- | --- | --- |
| mem_out_wr_valid | 1 | fsm → mem | Write valid |
| mem_out_wr_ready | 1 | mem → fsm | Write ready |
| mem_out_wr_addr | log2(OUT_DEPTH) | fsm → mem | Write address |
| mem_out_wr_data | LINE_W | fsm → mem | Result line |

### 6.2.6	cell_exec_fsm → cell_credit

Table 3.8: Consumed-line credit strobe

| Signal | Width | Direction | Purpose |
| --- | --- | --- | --- |
| credit_strobe | 1 | fsm → credit | One input line consumed |
| credit_src_coord | COORD_BITS | fsm → credit | Source coordinate of consumed line |

### 6.2.7	cell_credit → node (credit VC)

cell_credit builds header-only credit/add and credit/set packets and injects them into the node’s local-port credit sideband. The packet format is defined in TILE_SPEC §11.5 and §11.9.

### 6.2.8	cell_mem_out → cell_packetizer

Table 3.9: Result-memory read port

| Signal | Width | Direction | Purpose |
| --- | --- | --- | --- |
| pkt_rd_valid | 1 | pkt → mem | Read request valid |
| pkt_rd_ready | 1 | mem → pkt | Read request ready |
| pkt_rd_addr | log2(OUT_DEPTH) | pkt → mem | Read address |
| pkt_rd_data | LINE_W | mem → pkt | Result line |
| mem_out_empty | 1 | mem → pkt | Memory empty |
| mem_out_occupancy | log2(OUT_DEPTH+1) | mem → pkt | Line occupancy |

### 6.2.9	cell_packetizer → node (data network)

cell_packetizer builds write-format packets (ptype=mem, op=0) with the §11.5 tile header and sends them through the node’s local port. The destination and tag are supplied by the gatling-programmed drain command; the source coordinate is the cell’s external ID.

### 6.2.10	cell_exec_fsm ↔ ALU

The ALU interface is defined in the alu-infer MAS; the cell MAS cites it in Chapter 4.3.

Last Updated: 2026-10-18

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Cycle Behavior: Fill Path as Built

## 7.1	Slice-1 Fill Path

The slice-1 RTL already implements the receive and store side of the cell. This section documents its cycle behavior so that the slice-2/3 work can be checked against it.

### 7.1.1	Head-Beat Reception

A packet arrives on the node’s local port as a sequence of AXI4-Stream beats. On the head beat:

- s_user[11:0] carries the destination {X, Y, Z}.

- s_user[13:12] carries the QoS class; it must be 2'b01 (non-zero) so the node does not consume the packet as its own config traffic.

- s_user[29:15] carries the source coordinate {ext, level, X, Y, Z}.

- s_user[32:30] carries ptype: cfg=0, mem=2, credit=3, …

- s_user[33] carries op (write/read or credit add/set).

- s_user[49:34] carries a compacted 16-bit length.

- s_user[59:50] carries a compacted 10-bit target address.

- s_user[63:60] carries the 4-bit tag.

This 64-bit TUSER packing is provisional and exists only because the instantiated delta node exposes a 64-bit TUSER. It is superseded in slice 2/3 by the TILE_SPEC §11.6 two-header model.

### 7.1.2	Coordinate Match

cell_rx compares the destination {X, Y, Z} against its own POS_X/Y/Z. The ext bit and QoS are not part of the match; QoS is already required to be non-zero so the node did not consume the packet.

### 7.1.3	Demux Decision

Table 4.1: Slice-1 head-beat dispatch

| ptype | op | Action |
| --- | --- | --- |
| mem | 0 (write) | Accept payload beats into cell_mem_in starting at target_addr; increment mem_wr_ptr per beat |
| cfg | 0 (write) | Drive cfg_wr_valid to cell_cfg_regs with target_addr[7:0] and s_data[31:0] |
| cfg | 1 (read) | Counted TODO drop in slice 1; slice 2 builds a reply packet |
| credit | any | Counted TODO drop in slice 1; slice 2 routes to cell_credit |
| other | any | Drop head; increment o_err_drop_cnt via inc_err_drop |

### 7.1.4	Payload Write Timing

For a mem write packet:

- Cycle 0: head beat accepted; in_pkt latches pkt_src_coord, pkt_ptype, pkt_target; mem_wr_ptr clears.

- Cycle N: each payload beat with s_valid && s_ready && mem_wr_ready writes mem_data[addr] and mem_tag[addr] and increments mem_wr_ptr, where addr = pkt_target + mem_wr_ptr.

- The last payload beat (s_last) clears in_pkt.

cell_mem_in exposes wr_ready = !o_full; cell_rx backpressures the local port while a mem payload is in flight and the memory is full.

### 7.1.5	Occupancy Accounting

cell_mem_in increments its occupancy counter on every accepted write and decrements it on every accepted read. The read side is held idle in slice 1 (mem_in_rd_valid = 1'b0 in cell_top), so occupancy only rises during fill tests.

## 7.2	Expected Slice-2/3 Cycle Behavior

Slice 2/3 keeps the same fill timing but changes the header transport:

- The head beat’s TUSER carries only the delta route header.

- The 128-bit tile header is payload, beat-chained per the owner ruling 2026-10-18: on the 64-bit local port, beat 0 carries tdata = header[63:0] and beat 1 carries tdata = header[127:64]; payload beats follow. The demux reassembles the header across the two beats before dispatching on ptype.

- Payload beats carry one 32-bit line per 64-bit flit today; slice 3 packs two lines per 64-bit flit.

The demux still records the source coordinate from the tile header’s source field (not delta’s hop-source route header) and tags each input-memory line with it.

Last Updated: 2026-10-18

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Credit Logic

## 8.1	MAS Decision: Memory Sizing and Advertisement

MAS decision: IN_DEPTH = 32 lines and OUT_DEPTH = 32 lines.

Rationale: A 32-line depth is a power of two, matches the default credit-strap scale, and gives simple 5-bit pointers. The result memory is intentionally deeper than the default TILE_LAND_FLITS strap (16 flits) so that the landing-space strap, not the result memory, is the emit bottleneck. The input memory’s flit capacity is the cell’s boot advertisement value.

With LINE_W = 32 and a 64-bit payload flit today, one input line occupies one payload flit (low 32 bits). Therefore the boot advertisement is:

advertisement flits = IN_DEPTH = 32 flits

Note: Slice 3 may pack two 32-bit lines per 64-bit payload flit; if that packing is adopted, the advertisement value must be re-derived from the new flit count (IN_DEPTH / 2 flits) while the line count stays 32. Until then, the advertisement is 32 flits.

## 8.2	MAS Decision: Credit Batch Size and Timeout

MAS decision: Per-source credit batching uses N = CREDIT_BATCH_N consumed lines per source, with a default of N = 1. A partial batch that has not reached N drains after a 64-cycle timeout.

Rationale: N = 1 gives the simplest exact-per-line behavior, which is what the slice-2 credit-loop test will verify. Making N configurable lets later builds trade return latency for lower credit-packet traffic. The 64-cycle timeout bounds the latency of a trailing partial batch without adding any traffic at N = 1.

## 8.3	Per-Source Owed Counters

cell_credit maintains a bank of owed counters indexed by the recorded source coordinate. Each counter has:

- owed[src]: number of consumed lines not yet returned to this source

- timer[src]: cycles since the last return to this source (valid when owed is non-zero and below the batch threshold)

On every consumed-line strobe from cell_exec_fsm:

- owed[src] += 1

- If owed[src] >= CREDIT_BATCH_N, send one credit/add packet for CREDIT_BATCH_N flits, decrement owed[src] by CREDIT_BATCH_N, and clear the timer.

- Otherwise, load the timer with the timeout value (or reset it if it was already running).

Every cycle, for each source with 0 < owed[src] < CREDIT_BATCH_N:

- If timer[src] == 0, send one credit/add packet for owed[src] flits and clear owed[src].

- Else, timer[src] -= 1.

The timeout value is 64 cycles. The timer is loaded only when a new partial batch begins; additional consumed lines in the same partial batch do not reset the timer (they will be included in the same return when it fires).

## 8.4	Credit Packet Format

A consumed-line credit return is a header-only credit packet with:

- ptype = credit (3)

- op = 0 (add)

- length = number of flits returned (CREDIT_BATCH_N or the residual owed count)

- destination = the recorded source coordinate of the consumed line

- source = this cell’s external ID

- tag = echoed or zero (the sink does not store credit packets)

- target_address[1:0] = credit_class = 0 (sink space)

Credit packets ride the credit VC and need no grant (TILE_SPEC §11.9). TILE_SPEC §11.5 encodes the credit class in the low two bits of the target address: 0 = sink-space credit, 1 = descriptor-slot credit. Consumed-line returns and the boot advertisement below are sink-space credits (class 0).

## 8.5	Boot Advertisement Packet

After reset, cell_credit sends one credit/set packet to level = 3 (the tile I/O / RAPIDS) carrying the input memory’s full size in flits:

- op = 1 (set)

- length = IN_DEPTH flits (32 today)

- destination = {ext=0, level=3, X/Y/Z = tile I/O}

- source = this cell’s external ID

- target_address[1:0] = credit_class = 0 (sink space)

This advertisement is the cell’s “alive” signal; RAPIDS’s external sink-space counter for this cell starts at 0 and is filled by this packet (TILE_SPEC §11.10).

## 8.6	Sender-Generic Returns

The return is sender-generic: a credit goes to the source coordinate recorded on the input-memory line, not to a hard-coded RAPIDS ID. A cell that streamed into this cell’s input memory therefore receives the credits, not RAPIDS, exactly as TILE_SPEC §11.9 requires.

## 8.7	Implementation Note: Counter Bank Sizing

The number of legal source coordinates in a tile is bounded by the credit-counter index table in TILE_SPEC §11.9. A cell therefore needs at most enough per-source owed counters to cover every possible source that may send to it. A practical implementation allocates one counter per external ID (RAPIDS + peer cells + tetrad cores) that the tile can address, lazily allocating on first strobe if area must be minimized.

Last Updated: 2026-10-19

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Packetizer and Execution FSM

## 9.1	Packetizer

### 9.1.1	MAS Decision: Batching and Flush Conditions

MAS decision: The packetizer batches result flits up to EMIT_BATCH_X and flushes on either of two conditions:

- The batch accumulator holds EMIT_BATCH_X flits.

- CTRL.flush-and-send is written (bit 2 of CTRL).

An idle-timeout flush and an end-of-job marker flush are documented as deferred (PRD §12; PRD §11). This keeps slice-2 behavior deterministic and testable: a testbench can either fill a batch or force a flush.

### 9.1.2	MAS Decision: Strap Counter Initialization

MAS decision: The packetizer maintains a strap counter initialized from a cell parameter TILE_LAND_FLITS with a default of 16 flits. The counter is spent per flit sent (header included) and is restored by RAPIDS credit/add returns.

Rationale: The default 16 matches the Hive-set TILE_LAND_FLITS default in TILE_SPEC §11.9. The counter is private to this cell and never goes negative; a packet is presented only while the counter covers the entire packet, header included.

### 9.1.3	Header Build

The packetizer builds write-format packets (ptype=mem, op=0) with the TILE_SPEC §11.5 tile header as the first payload beat(s). The header fields are:

Table 4.2: Result-packet header fields

| Field | Value |
| --- | --- |
| Destination coordinate | Gatling-programmed drain destination (see below) |
| Source coordinate | This cell’s external ID {ext=1, level=0, X, Y, Z} |
| ptype | mem (2) |
| op | 0 (write) |
| Length | Payload bytes that follow |
| Target address | Programmed by the drain command (memory address in the sink channel) |
| Tag | Programmed sink channel |

MAS decision on transport: The packetizer uses the TILE_SPEC §11.6 two-header model with the beat-chained header rule (owner ruling 2026-10-18 — every link keeps its own width):

- The delta route header is placed in TUSER of the head beat.

- The 128-bit tile header is placed in the first ceil(128/DW) payload beats, LSB-first — on the 64-bit local port: beat 0 = header[63:0], beat 1 = header[127:64].

- Payload beats follow.

The local port does not widen. This supersedes slice-1’s provisional 64-bit TUSER packing and the 0.10 draft’s 128-bit local-port widening.

### 9.1.4	Destination and Tag Programming

The cell does not choose the result destination. Gatling (the tetrad engine) commands the drain and tells the cell where to send it (TILE_SPEC §11.14; PRD §6.3).

Precedence: The operational drain destination is held in FLUSH_DEST (offset 0x54) and the sink-channel tag in FLUSH_TAG (offset 0x58). These are programmed by gatling (via cfg write or an extended CTRL flush command) and are what the packetizer uses when CTRL.flush-and-send is written. The legacy DEST register (offset 0x38) is reserved for the deferred cell→cell result path; it is not used for rapids-bound drains. DEST becomes operational only after the tile-spec cell→cell credit/grant extension lands (TILE_SPEC §10); until then, result traffic is RAPIDS-bound and FLUSH_DEST/FLUSH_TAG take precedence.

Rationale: This gives gatling a concrete mechanism to command a flush without requiring a new packet type or a sideband interface, while keeping the DEST register available for the future peer-cell path.

### 9.1.5	Strap Accounting

Each packet costs one header flit plus one flit per payload line today. The packetizer checks:

strap_counter >= 1 + payload_flits

before presenting tvalid. It decrements the counter by one per flit sent. RAPIDS returns credits as it drains the cell’s landing space; the returns are credit/add packets that increment the strap counter.

## 9.2	Execution FSM

### 9.2.1	Pop → Issue → Push Loop

cell_exec_fsm runs the compute loop:

- Pop. When enabled and not busy, pop OP_LINES input lines from cell_mem_in, advancing IP by one per line. Capture the source coordinate of the first line (all lines of one invocation are expected to come from the same source; if they differ, the first line’s source is used for credit return).

- Issue. Present the operand lines to the ALU and assert the issue valid. Wait for ready; hold operands and func_sel stable while ready is low.

- Push. When the ALU asserts result_valid and the FSM asserts result_ready, push the result line into cell_mem_out, advancing OP by one.

The loop is pipelined: while one invocation computes, the FSM can pop and issue the next, as long as the ALU’s elastic pipeline accepts it (II=1 while draining).

### 9.2.2	MAS Decision: Operand-Source Mux

MAS decision: The operand-source mux is configured by OP_SRC:

Table 4.3: OP_SRC encoding

| OP_SRC value | op_line_a source | op_line_b source |
| --- | --- | --- |
| 0 | input-memory pop | input-memory pop |
| 1 | A | A |
| 2 | B | B |
| 3 | C | C |
| 4 | D | D |

The two sides are controlled by independent 3-bit fields in OP_SRC: sel_a[2:0] and sel_b[2:0]. Reset value is 0 (both operands from memory). The T register supplies threshold_imm for opcode 6 (compare/threshold).

### 9.2.3	IP/OP Pointer Behavior

IP and OP are 32-bit registers that auto-increment on pop and push respectively. They wrap at IN_DEPTH and OUT_DEPTH:

IP_next = (IP + 1) mod IN_DEPTH
OP_next = (OP + 1) mod OUT_DEPTH

CTRL.soft_reset clears both pointers to 0 and clears IRQ_STATUS. Software may also seed them through AXIL or cfg writes.

### 9.2.4	Gatling as the Cell ALU Sequencer (owner ruling 2026-10-19)

MAS decision: The slice-3 cell_exec_fsm remains the standalone bring-up driver for the cell, preserving the same POP → issue → PUSH client contract. In the tetrad-integrated tile, gatling (the tetrad barrel-core engine) sequences the cell’s alu-infer ALU directly:

- POP reads operand line(s) from the cell’s input memory and presents them to the ALU issue handshake.

- PUSH retires result line(s) from the ALU and writes them into the cell’s result memory.

- FLUSH programs FLUSH_DEST/FLUSH_TAG and triggers the packetizer drain.

- CFG/ECTX/KICK/WAIT carry tetrad-level context and control.

Gatling never originates config traffic that belongs in the cell’s cfg stream; cell programming remains in-band per the standing owner ruling 2026-10-17. The gatling PRD v0.2 owns the Xcell instruction encodings (CFG, POP, PUSH, ECTX, FLUSH, KICK, WAIT).

### 9.2.5	ALU Hookup

The FSM drives the ALU’s elastic valid/ready interface per the alu-infer MAS:

Table 4.4: ALU↔FSM seam signals

| Signal | Direction | Purpose |
| --- | --- | --- |
| alu_valid | FSM → ALU | Issue valid |
| alu_ready | ALU → FSM | Issue ready |
| op_line_a | FSM → ALU | Operand A line |
| op_line_b | FSM → ALU | Operand B line |
| threshold_imm | FSM → ALU | bf16 threshold immediate (from T) |
| func_sel | FSM → ALU | {opcode, sub-op, reserved} |
| acc_ctrl | FSM → ALU | CLEAR / ACCUMULATE / DRAIN for MAC |
| result_line | ALU → FSM | Result line |
| result_valid | ALU → FSM | Result valid |
| result_ready | FSM → ALU | Result ready |

The function/opcode table is defined in alu-infer PRD §3 and implemented per the alu-infer MAS. The cell MAS does not restate it.

### 9.2.6	Issue Tag and Same-Tag Alignment (ratified spec — RTL pending)

MAS decision: When the scheduler loads a new function into the ALU pipeline, an 8-bit tag is assigned, common to every op of that invocation group that must complete together. A tag FIFO tracks the in-flight groups. Each op’s result_valid asserts only when every result_valid of ops carrying the same tag has asserted — so ops that were scheduled together finish together, and the elastic pipeline’s per-op readiness is gated to the group boundary.

Rationale (owner ruling): if two ops are scheduled in the same clock they should finish at the same time, and the scheduler programs the valid/ready behavior it wants (including feedback cases) through this tag grouping rather than through pipeline-depth matching. This takes care of the elastic alignment naturally: the group is atomic at the result boundary even though individual ops may have different internal latencies.

Status: ratified with the alu-infer work at rev 0.50; not yet in the alu-infer RTL or the alu-infer MAS — the tag assign/FIFO/align gate of Figure 2.3 is the specified target, not the current implementation.

### 9.2.7	MAC Accumulation

MAC behavior is inherited from the alu-infer contract: the FMA computes the bf16 product exactly in fp32, accumulates per-lane in fp32, and rounds to bf16 once per result-line presentation. The FSM sequences acc_ctrl (CLEAR at layer boundaries, ACCUMULATE for steps, DRAIN for the final values) without changing the ALU interface.

### 9.2.8	Soft Reset

CTRL.soft_reset (bit 1) pulses o_soft_rst for one cycle. The FSM responds by clearing IP and OP and dropping any in-flight issue request. The ALU’s reset behavior is defined in the alu-infer MAS.

Last Updated: 2026-10-19

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Verification Plan

## 10.1	House Test Grid

The cell testbench follows the house gate / func / full grid:

Table 5.1: House test grid

| Level | Scope | Typical seeds | Runtime target |
| --- | --- | --- | --- |
| gate | Smoke: elaboration, reset, register reset values, basic fill | 1 seed | < 1 min |
| func | Directed feature tests across seeds | 8 seeds | ~5 min |
| full | Longer randomized runs, corner cases, coverage closure | 32+ seeds | ~30 min |

## 10.2	Existing Tests

cell/dv/tests/test_cell.py already covers slice-1 behavior:

- Elaboration + reset + register reset values over AXIL

- AXIL and cfg-stream register access

- Input-memory fill from a ptype=mem packet

- Bad-packet drop behavior

The delta node inside cell_top is treated as a black box; slice-1 simulation drives the local rx port through the test_mode injection hook (dut.test_rx_*) so the testbench can supply cell traffic without fighting the node’s continuous driver.

## 10.3	Planned Slice-2 Credit-Loop Test

The slice-2 test verifies the credit return path independently of the datapath:

- Drive mem packets into the cell from multiple recorded sources (using the test-mode injection hook or a BFM on the local port).

- Force the execution FSM to consume lines without running the ALU (or run a passthrough opcode).

- Check that cell_credit emits credit/add packets whose destination equals the recorded source coordinate and whose length matches the configured batching (CREDIT_BATCH_N or residual).

- Verify the 64-cycle partial-batch timeout by sending a partial batch and waiting for the automatic drain.

- Verify the boot credit/set advertisement to level=3 carrying IN_DEPTH flits.

The test forces straps so that result emission does not gate credit returns.

## 10.4	Planned Slice-3 ALU-Run Test

The slice-3 test verifies the pop→issue→push loop end to end:

- Program FUNC_SEL, OP_SRC, OP_LINES, and persistent operands A–D, T via cfg or AXIL.

- Fill the input memory with known operand lines.

- Enable the FSM and run until STATUS.busy clears or an IRQ fires.

- Read the result memory (via the packetizer drain or AXIL window).

- Check results against the alu-infer golden model per opcode.

Coverage: opcode (8) × special-value classes × lane position × accumulator clear/hold/drain corners.

## 10.5	Verification Hooks

Table 5.2: Verification hooks

| Hook | Use |
| --- | --- |
| test_rx_* / test_mode | Inject cell traffic without instantiating the delta mesh |
| inc_lines_in/out, inc_credits, inc_pkts | Count events independently of datapath |
| o_todo_credit_drop_cnt / o_todo_cfg_read_drop_cnt | Confirm slice-1 stubs are replaced in slice 2 |
| STATUS / IRQ_STATUS | Observe empty/full, error, and strap-starved states |

## 10.6	Assertions and Checks

- Credit return must match lines consumed, per source, within the batch timeout.

- Strap counter must never go negative; packetizer tvalid implies sufficient strap for the whole packet.

- ALU results retire in issue order.

- IP and OP wrap correctly at their respective depths.

Last Updated: 2026-10-19

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Register Map and MAS Decisions

## 11.1	Register Map

The v1 register map is identical to PRD §7, with two MAS-added operational registers for the gatling-programmed drain command.

Table 6.1: Cell register map

| Offset | Register | Notes |
| --- | --- | --- |
| 0x00 | CTRL | enable, soft reset, flush-and-send |
| 0x04 | STATUS | busy, input-empty, result-nonempty, strap-nonzero (RTL alias status_grant_nonzero) |
| 0x08 | FUNC_SEL | opcode (5b) + sub-op (2b) + reserved (2b) |
| 0x0C | OP_LINES | input lines per function invocation |
| 0x10 | EMIT_BATCH_X | flits per result packet |
| 0x14 | CREDIT_BATCH_N | consumed lines per credit packet per source |
| 0x18 | IRQ_EN | done, error, strap-starved (RTL alias IRQ_GRANT_STARVED_BIT), credit-timeout enables |
| 0x1C | IRQ_STATUS | write-1-clear status |
| 0x20 | A | persistent operand A |
| 0x24 | B | persistent operand B |
| 0x28 | C | persistent operand C |
| 0x2C | D | persistent operand D |
| 0x30 | T | bf16 threshold immediate |
| 0x34 | OP_SRC | operand-source mux: sel_a[2:0], sel_b[2:0] |
| 0x38 | DEST | deferred cell-destined destination |
| 0x3C | IP | input-memory read pointer |
| 0x40 | OP | result-memory write pointer |
| 0x44 | CNT_LINES_IN | lines received |
| 0x48 | CNT_LINES_OUT | results produced |
| 0x4C | CNT_CREDITS | credit packets returned |
| 0x50 | CNT_PKTS | result packets sent |
| 0x54 | FLUSH_DEST | MAS-added: gatling-programmed drain destination coordinate |
| 0x58 | FLUSH_TAG | MAS-added: gatling-programmed sink-channel tag |

## 11.2	Bit Fields

### 11.2.1	CTRL (0x00)

Table 6.2: CTRL bit fields

| Bit | Name | Purpose |
| --- | --- | --- |
| 0 | EN | Enable execution FSM |
| 1 | SOFT_RST | Soft reset: clears IP, OP, IRQ_STATUS; pulse lasts one cycle |
| 2 | FLUSH | Packetizer flush-and-send: emit whatever is buffered now |

### 11.2.2	STATUS (0x04) — read-only

Table 6.3: STATUS bit fields

| Bit | Name | Purpose |
| --- | --- | --- |
| 0 | BUSY | FSM busy (slice 3) |
| 1 | INPUT_EMPTY | Input memory empty |
| 2 | RESULT_NONEMPTY | Result memory not empty |
| 3 | STRAP_NONZERO | Strap counter > 0. Slice-1 RTL still names this bit STATUS_GRANT_NONZERO_BIT / status_grant_nonzero; the doc term is canonical. |

### 11.2.3	FUNC_SEL (0x08)

Table 6.4: FUNC_SEL fields

| Bits | Field | Purpose |
| --- | --- | --- |
| 4:0 | opcode | ALU opcode (alu-infer PRD §3) |
| 6:5 | sub_op | Sub-opcode, e.g. min vs max, compare vs threshold |
| 8:7 | reserved | Reserved under the bf16-only model |

### 11.2.4	OP_SRC (0x34)

Table 6.5: OP_SRC fields

| Bits | Field | Encoding |
| --- | --- | --- |
| 2:0 | sel_a | 0=memory pop, 1=A, 2=B, 3=C, 4=D |
| 5:3 | sel_b | 0=memory pop, 1=A, 2=B, 3=C, 4=D |
| 31:6 | reserved | 0 |

### 11.2.5	IRQ_STATUS / IRQ_EN

Table 6.6: IRQ fields

| Bit | Name | Purpose |
| --- | --- | --- |
| 0 | DONE | Computation/drain complete |
| 1 | ERROR | Dropped packet or fatal protocol error |
| 2 | STRAP_STARVED | Packetizer has data but no strap. Slice-1 RTL still names this bit IRQ_GRANT_STARVED_BIT; the doc term is canonical. |
| 3 | CREDIT_TIMEOUT | Partial-batch credit timer fired (informational) |

## 11.3	MAS Decisions Revisited

Table 6.7: MAS decisions and their hardware homes

| Decision | Where in this book | Register / parameter |
| --- | --- | --- |
| Input/result memory depth = 32 lines | Chapter 4.2 | IN_DEPTH, OUT_DEPTH parameters |
| Boot advertisement = 32 flits | Chapter 4.2 | Derived from IN_DEPTH |
| Credit batch default = 1 | Chapter 4.2 | CREDIT_BATCH_N register |
| Partial-batch timeout = 64 cycles | Chapter 4.2 | Hard-coded in cell_credit |
| Result batch flush conditions | Chapter 4.3 | EMIT_BATCH_X, CTRL.FLUSH |
| Strap default = 16 flits | Chapter 4.3 | TILE_LAND_FLITS parameter |
| Local-port width stays at delta DATA_WIDTH (64 bits today) | Chapter 3, Chapter 4.3 | DATA_WIDTH parameter |
| Gatling-programmed drain | Chapter 4.3 | FLUSH_DEST, FLUSH_TAG registers |

## 11.4	RTL Naming Divergence (slice 1)

Slice-1 RTL (cell_pkg.sv, cell_cfg_regs.sv) predates the 2026-10-18 ruling that renamed result-path grants to straps. The canonical MAS/PRD terms are strap counter, STRAP_NONZERO, and STRAP_STARVED; the slice-1 RTL identifiers remain:

- STATUS_GRANT_NONZERO_BIT / status_grant_nonzero for STATUS bit 3

- IRQ_GRANT_STARVED_BIT for IRQ_STATUS/IRQ_EN bit 2

A future RTL pass (slice 2/3) should rename these to the strap terms. Until then, the register map above states the canonical names and notes the RTL aliases.

Last Updated: 2026-10-19

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	References

## 12.1	Normative Specifications

- cell PRD — projects/components/compute-eng-ip/cell/docs/PRD.md.
The governing product definition. The MAS implements PRD §12 open questions and cites §§3, 6, 7 throughout.

- TILE_SPEC §11 — projects/components/compute-eng-ip/tile/docs/TILE_SPEC.md.
Defines the packet header (§11.5), transport binding (§11.6), payload types (§11.2), destination coordinate (§11.3), end-to-end credits and straps (§§11.9–11.10), and result flush (§11.14).

- alu-infer PRD §3 — projects/components/compute-eng-ip/alu-infer/PRD.md.
Normative ALU function set, element model (bf16), and MAC accumulation semantics.

- alu-infer MAS — projects/components/compute-eng-ip/alu-infer/docs/alu_infer_mas/alu_infer_mas_index.md.
Implementation-intent book for the ALU block: module tree, function rows, accumulator, ALU↔FSM interface, and cycle behavior.

## 12.2	RTL Sources

- cell_top.sv — projects/components/compute-eng-ip/cell/rtl/cell_top.sv.
Slice-1 top-level: delta node, rx demux, registers, memories.

- cell_rx.sv — projects/components/compute-eng-ip/cell/rtl/cell_rx.sv.
Slice-1 header parse/demux with provisional 64-bit TUSER packing.

- cell_cfg_regs.sv — projects/components/compute-eng-ip/cell/rtl/cell_cfg_regs.sv.
Slice-1 AXIL + cfg-stream register file.

- cell_mem_in.sv — projects/components/compute-eng-ip/cell/rtl/cell_mem_in.sv.
Slice-1 input memory with per-line source tag.

- cell_mem_out.sv — projects/components/compute-eng-ip/cell/rtl/cell_mem_out.sv.
Slice-1 result memory.

- cell_pkg.sv — projects/components/compute-eng-ip/cell/rtl/cell_pkg.sv.
Shared types, coordinate layout, register offsets, and slice-1 compact header offsets.

- delta_mesh_cell.sv — projects/components/fabric-gen-ip/delta/rtl/delta_mesh_cell.sv.
The node the cell instantiates; defines the 9-port unpacked-array interface and P_LOCAL = 8.

## 12.3	Verification

- test_cell.py — projects/components/compute-eng-ip/cell/dv/tests/test_cell.py.
Existing slice-1 cocotb test suite; the base for the slice-2/3 credit-loop and ALU-run tests.

## 12.4	House Style and Build

- Block diagrams handbook — vault/handbook/authoring/block-diagrams.md.
Binding house style for the whole-cell and credit-loop figures.

- alu-infer MAS PDF generator — projects/components/compute-eng-ip/alu-infer/docs/generate_mas_pdf.sh.
The build script this MAS generator is modeled on.

Last Updated: 2026-10-18