Table of Contents

# Converter Index

Generated: 2025-10-26

# Converters Component Specification

Version: 1.2 Status: Production Ready Last Updated: 2025-10-26

## Document Overview

This specification provides comprehensive documentation for the Converters component, which includes data width conversion and protocol conversion modules for seamless system integration.

Document Structure: - Chapter 1: Overview and Architecture - Chapter 2: Data Width Converters - Chapter 3: Protocol Converters - Chapter 4: Usage Examples and Integration

## Quick Navigation

### Chapter 1: Overview

- 01_introduction.md - Component overview and key features

- 02_architecture.md - System architecture and module hierarchy

- 03_design_philosophy.md - Design decisions and rationale

### Chapter 2: Data Width Converters

- 01_overview.md - Data width conversion overview

- 02_axi_data_upsize.md - Narrow→Wide accumulator

- 03_axi_data_dnsize.md - Wide→Narrow splitter

- 04_dual_buffer_mode.md - High-performance dual-buffer mode

- 05_axi4_dwidth_converter_wr.md - Full write path converter

- 06_axi4_dwidth_converter_rd.md - Full read path converter

### Chapter 3: Protocol Converters

- 01_overview.md - Protocol conversion overview

- 02_axi4_to_apb.md - AXI4-to-APB bridge

- 03_peakrdl_adapter.md - PeakRDL adapter

### Chapter 4: Usage Examples

- 01_configuration_guide.md - Parameter configuration guide

- 02_common_scenarios.md - Common use case examples

- 03_integration_patterns.md - System integration patterns

## Component Summary

### Data Width Converters

Generic Building Blocks: - axi_data_upsize - Accumulates N narrow beats into 1 wide beat (100% throughput) - axi_data_dnsize - Splits 1 wide beat into N narrow beats (80% or 100% throughput)

Full AXI4 Converters: - axi4_dwidth_converter_wr - Complete write path (AW + W + B channels) - axi4_dwidth_converter_rd - Complete read path (AR + R channels)

Block Diagrams:

Figure 1: axi_data_upsize - Narrow-to-Wide Accumulator

Figure 2: axi_data_dnsize - Wide-to-Narrow Splitter (Single Buffer, 80% throughput)

Figure 3: axi_data_dnsize - Wide-to-Narrow Splitter (Dual Buffer, 100% throughput)

### Protocol Converters

Available Converters: - axi4_to_apb_convert - Full AXI4-to-APB protocol bridge - axi4_to_apb_shim - Simplified AXI4-to-APB wrapper - peakrdl_to_cmdrsp - PeakRDL register to command/response adapter

Block Diagrams:

Figure 4: axi4_to_apb_convert - Protocol Conversion

Figure 5: peakrdl_to_cmdrsp - Register Interface Adapter

FSM Diagram:

Figure 6: AXI4-to-APB State Machine

## Performance Summary

| Module | Configuration | Throughput | Area | Use Case |
| --- | --- | --- | --- | --- |
| axi_data_upsize | Single buffer | 100% | 1× | Narrow→Wide (always optimal) |
| axi_data_dnsize | Single buffer | 80% | 1× | Wide→Narrow (area-efficient) |
| axi_data_dnsize | Dual buffer | 100% | 2× | Wide→Narrow (high-performance) |
| axi4_dwidth_converter_wr | With upsize | 100% | Standard | Full write path |
| axi4_dwidth_converter_rd | With dual dnsize | 100% | +100% | Full read path |
| axi4_to_apb | Protocol FSM | Sequential | Small | Protocol conversion |

## Key Features

### Data Width Conversion

- Flexible width ratios (any integer multiple: 2:1, 4:1, 8:1, 16:1, etc.)

- Configurable sideband handling (concatenate or broadcast)

- Optional burst tracking for LAST signal generation

- Dual-buffer mode for 100% throughput (downsize only)

- Generic building blocks for custom converters

### Protocol Conversion

- Full AXI4-to-APB protocol translation

- Address width adaptation (64-bit→32-bit)

- Data width adaptation (configurable)

- Error response mapping (PSLVERR→BRESP/RRESP)

- PeakRDL register interface decoupling

## Documentation Assets

### Graphviz Block Diagrams

Located in assets/graphviz/: - axi_data_upsize.gv/.png - axi_data_dnsize_single.gv/.png - axi_data_dnsize_dual.gv/.png - axi4_dwidth_converter_wr.gv/.png - axi4_dwidth_converter_rd.gv/.png - axi4_to_apb.gv/.png - peakrdl_adapter.gv/.png

See assets/graphviz/README.md for regeneration instructions.

### PlantUML FSM Diagrams

Located in assets/puml/: - axi4_to_apb_fsm.puml/.png

See assets/puml/README.md for regeneration instructions.

## Related Documentation

Component-Level: - ../../README.md - Component quick start guide - ../../DUAL_BUFFER_IMPLEMENTATION.md - Dual-buffer feature deep dive - ../../ANALYSIS_APB_CONVERTER.md - APB converter analysis

Repository-Level: - RTL source: projects/components/converters/rtl/ - Test suite: projects/components/converters/dv/tests/ - Usage examples: Chapter 4 of this specification

## Version History

| Version | Date | Description |
| --- | --- | --- |
| 1.2 | 2025-10-26 | Added comprehensive specification with block diagrams |
| 1.1 | 2025-10-25 | Added dual-buffer mode for axi_data_dnsize |
| 1.0 | 2025-10-24 | Initial release with generic and full converters |

## Document Conventions

Notation: - parameter_name - RTL parameters and signals - Bold - Important concepts and module names - Italic - Figure captions and references

Diagrams: - Light blue - Input interfaces - Light coral - Output interfaces - Light yellow - Control logic - Light gray - Data path elements - Arrows - Data/control flow

Author: RTL Design Sherpa Project Maintained By: Converters Component Team Last Review: 2025-10-26

# Chapter 1: Overview - Introduction

## Purpose

The Converters component provides essential data width conversion and protocol conversion modules that enable seamless integration between components with different data widths or communication protocols.

## Problem Statement

Modern SoC designs frequently encounter two integration challenges:

- Data Width Mismatch

  - CPU: 64-bit data bus

  - DDR Controller: 512-bit data bus

  - PCIe Endpoint: 128-bit data bus

  - Result: Direct connection impossible

- Protocol Incompatibility

  - AXI4 masters (CPU, DMA)

  - APB peripherals (UART, GPIO, SPI)

  - Custom register interfaces (PeakRDL)

  - Result: Protocol bridge required

## Solution Approach

### Data Width Converters

Generic Building Blocks: - axi_data_upsize - Accumulates narrow beats into wide beats - axi_data_dnsize - Splits wide beats into narrow beats

Full AXI4 Integration: - axi4_dwidth_converter_wr - Write path converter (AW + W + B) - axi4_dwidth_converter_rd - Read path converter (AR + R)

### Protocol Converters

- axi4_to_apb_convert - Full AXI4-to-APB bridge

- peakrdl_to_cmdrsp - Register interface adapter

## Key Benefits

### Reusability

- Generic modules work with any width ratio

- Configurable parameters for custom use cases

- Self-contained building blocks

### Performance

- Upsize: 100% throughput (single buffer)

- Downsize: 80% (single buffer) or 100% (dual buffer)

- Minimal latency overhead

### Flexibility

- Configurable sideband handling

- Optional burst tracking

- Dual-buffer mode for high-performance paths

## Component Scope

### In Scope

- Integer width ratios (2:1, 4:1, 8:1, 16:1, etc.)

- AXI4 and APB protocol support

- Configurable throughput vs area trade-offs

- Generic building blocks for custom converters

### Out of Scope

- Non-integer width ratios (e.g., 3:2 conversion)

- AXI4-Stream protocol (different component)

- Complex buffering beyond dual-buffer

- Clock domain crossing (use separate CDC modules)

## Target Applications

- CPU-to-DDR Integration - 64-bit CPU to 512-bit memory controller

- DMA Engines - Variable width data movers

- Mixed Protocol Systems - AXI4 to APB peripheral buses

- FPGA Fabric Interfaces - Width matching for IP integration

- Register Access - PeakRDL to custom control protocols

## Document Organization

This specification is organized into four chapters:

- Overview (This chapter) - Purpose, scope, and architecture

- Data Width Converters - Generic and full AXI4 modules

- Protocol Converters - AXI4-to-APB and PeakRDL adapters

- Usage Examples - Configuration, scenarios, and integration

Next: 02_architecture.md - System architecture and module hierarchy

# Chapter 2: Data Width Converters - Overview

## Introduction

Data width converters enable communication between components with mismatched data bus widths, a common challenge in modern SoC designs where different IP blocks operate at different data widths.

## Module Hierarchy

Data Width Converters
├── Generic Building Blocks
│   ├── axi_data_upsize.sv      - Narrow→Wide accumulator
│   └── axi_data_dnsize.sv      - Wide→Narrow splitter
│
└── Full AXI4 Converters
    ├── axi4_dwidth_converter_wr.sv - Write path (AW + W + B)
    └── axi4_dwidth_converter_rd.sv - Read path (AR + R)

## Generic Building Blocks

### axi_data_upsize - Narrow→Wide Accumulator

Purpose: Accumulate N narrow beats into 1 wide beat

Key Features: - 100% throughput (single buffer sufficient) - Configurable sideband modes (concatenate or OR) - Supports any integer width ratio - Minimal area overhead

Block Diagram:

AXI Data Upsize

Documentation: 02_axi_data_upsize.md

### axi_data_dnsize - Wide→Narrow Splitter

Purpose: Split 1 wide beat into N narrow beats

Key Features: - Two modes: Single buffer (80%) or Dual buffer (100%) throughput - Configurable sideband modes (slice or broadcast) - Optional burst tracking for LAST generation - Trade-off: throughput vs area

Block Diagrams:

Single Buffer Mode - 80% throughput, 1× area

Dual Buffer Mode - 100% throughput, 2× area

Documentation: 03_axi_data_dnsize.md | 04_dual_buffer_mode.md

## Full AXI4 Converters

### axi4_dwidth_converter_wr - Write Path Converter

Purpose: Complete write path conversion with AW, W, and B channels

Features: - Address phase management - Burst length adjustment - Skid buffers for flow control - Integrates axi_data_upsize for data conversion

Block Diagram:

AXI4 Write Converter

Documentation: 05_axi4_dwidth_converter_wr.md

### axi4_dwidth_converter_rd - Read Path Converter

Purpose: Complete read path conversion with AR and R channels

Features: - Address phase management - Burst length adjustment - Burst-aware RLAST generation - Integrates axi_data_dnsize for data conversion

Block Diagram:

AXI4 Read Converter

Documentation: 06_axi4_dwidth_converter_rd.md

## Throughput Comparison

| Module | Mode | Throughput | Area | When to Use |
| --- | --- | --- | --- | --- |
| axi_data_upsize | Single buffer | 100% | 1× | All narrow→wide conversions |
| axi_data_dnsize | Single buffer | 80% | 1× | Area-constrained designs |
| axi_data_dnsize | Dual buffer | 100% | 2× | High-bandwidth requirements |

Recommendation: - Upsize: Always use single buffer (100% throughput at minimal cost) - Downsize: Choose based on system requirements: - Single buffer for area-constrained or non-continuous traffic - Dual buffer for high-performance DMA or continuous streaming

## Common Configuration Examples

### Example 1: 64→512 bit (Write Path)

axi_data_upsize #(
    .NARROW_WIDTH(64),
    .WIDE_WIDTH(512),
    .NARROW_SB_WIDTH(8),   // WSTRB
    .WIDE_SB_WIDTH(64),
    .SB_OR_MODE(0)         // Concatenate
) u_upsize (...);

### Example 2: 512→128 bit (Read Path, High Performance)

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(2),     // RRESP
    .NARROW_SB_WIDTH(2),
    .SB_BROADCAST(1),      // Broadcast
    .DUAL_BUFFER(1)        // 100% throughput
) u_dnsize (...);

Next Sections: - 02_axi_data_upsize.md - Detailed upsize module documentation - 03_axi_data_dnsize.md - Detailed dnsize module documentation - 04_dual_buffer_mode.md - Dual-buffer architecture deep dive

# Chapter 3: Protocol Converters - Overview

## Introduction

Protocol converters enable communication between components using different communication protocols, essential for integrating diverse IP blocks in complex SoC designs.

## Available Converters

### 1. AXI4-to-APB Bridge

- Module: axi4_to_apb_convert.sv

- Purpose: Full protocol translation from AXI4 to APB

- Features: Address width adaptation, state machine control, error mapping

### 2. PeakRDL Adapter

- Module: peakrdl_to_cmdrsp.sv

- Purpose: Register interface to command/response protocol

- Features: Protocol decoupling, single-cycle commands, pipelined responses

## AXI4-to-APB Bridge

### Overview

Converts AXI4 master transactions (from CPU, DMA) to APB peripheral accesses.

Key Challenges: - Protocol differences (5-channel AXI4 vs 2-phase APB) - Address width mismatch (64-bit AXI4 vs 32-bit APB) - Burst support (AXI4 bursts → sequential APB transactions) - Error response mapping (PSLVERR → BRESP/RRESP)

### Block Diagram

AXI4-to-APB Converter

### State Machine

The converter uses a state machine to manage protocol translation:

AXI4-to-APB FSM

States: - IDLE - Wait for AXI4 transaction - READ - Process AXI4 read → APB read - WRITE - Process AXI4 write → APB write

### Use Cases

- CPU to APB Peripherals - Main processor accessing GPIO, UART, SPI

- DMA to Configuration Registers - DMA controller configuring peripherals

- Mixed Protocol Systems - Integrating AXI4 fabric with legacy APB devices

### Documentation

See 02_axi4_to_apb.md for detailed specification.

## PeakRDL Adapter

### Overview

Converts PeakRDL-generated register interface to a custom command/response protocol, enabling protocol decoupling and flexible register implementations.

Key Features: - APB-style register interface (input) - Command/response handshake (output) - Configurable address and data widths - Single-cycle command issue

### Block Diagram

PeakRDL Adapter

### Interface Types

Register Interface (APB-style): - reg_addr[ADDR_WIDTH-1:0] - Register address - reg_wdata[DATA_WIDTH-1:0] - Write data - reg_write - Write enable - reg_read - Read enable - reg_rdata[DATA_WIDTH-1:0] - Read data - reg_error - Error flag

Command/Response Protocol: - Command: valid/ready handshake with addr, data, write flag - Response: valid/ready handshake with data, error flag

### Use Cases

- PeakRDL Register Blocks - Decoupling register interface from implementation

- Custom Control Logic - Flexible register access mechanism

- Testbench Stimulus - Command-driven register access in verification

### Documentation

See 03_peakrdl_adapter.md for detailed specification.

## Comparison

| Feature | AXI4-to-APB | PeakRDL Adapter |
| --- | --- | --- |
| Input Protocol | AXI4 (5 channels) | APB-style register |
| Output Protocol | APB (2 phases) | Command/response |
| Complexity | High (FSM, burst handling) | Low (pass-through) |
| Use Case | System interconnect | Register decoupling |
| Performance | Sequential per APB | Single-cycle cmd |

## Design Considerations

### When to Use Protocol Converters

Use AXI4-to-APB when: - Integrating AXI4 masters with APB peripherals - Building CPU-to-peripheral bridges - System has mixed protocol requirements

Use PeakRDL adapter when: - Decoupling register interface from implementation - Need flexible register access protocol - Building custom control/configuration logic

### Integration Guidelines

- Address Map Planning - Ensure non-overlapping regions

- Error Handling - Map error responses appropriately

- Performance Analysis - Consider latency impact

- Testing Strategy - Verify protocol compliance

Next Sections: - 02_axi4_to_apb.md - Detailed AXI4-to-APB specification - 03_peakrdl_adapter.md - Detailed PeakRDL adapter specification

# Converters Graphviz Block Diagrams

This directory contains Graphviz source files (.gv) and generated PNG diagrams for Converters component modules.

## Files

### Data Width Converters

Generic Building Blocks: - axi_data_upsize.gv - Narrow→Wide accumulator (128→512 example) - axi_data_dnsize_single.gv - Wide→Narrow splitter, single buffer (512→128, 80% throughput) - axi_data_dnsize_dual.gv - Wide→Narrow splitter, dual buffer (512→128, 100% throughput)

Full AXI4 Converters: - axi4_dwidth_converter_wr.gv - Write path converter (AW + W + B channels) - axi4_dwidth_converter_rd.gv - Read path converter (AR + R channels)

### Protocol Converters

- axi4_to_apb.gv - AXI4-to-APB protocol bridge

- peakrdl_adapter.gv - PeakRDL register interface to command/response adapter

### Generated Images (.png)

All diagrams are available as PNG files: - axi_data_upsize.png - axi_data_dnsize_single.png - axi_data_dnsize_dual.png - axi4_dwidth_converter_wr.png - axi4_dwidth_converter_rd.png - axi4_to_apb.png - peakrdl_adapter.png

## Regenerating Diagrams

### Prerequisites

- Graphviz installed (sudo apt install graphviz on Ubuntu/Debian)

- dot command available in PATH

### Generate All Diagrams

# From this directory
make all

# Or manually
dot -Tpng axi_data_upsize.gv -o axi_data_upsize.png
dot -Tpng axi_data_dnsize_single.gv -o axi_data_dnsize_single.png
# ... etc

### Generate Single Diagram

# Upsize converter
dot -Tpng axi_data_upsize.gv -o axi_data_upsize.png

# Downsize single buffer
dot -Tpng axi_data_dnsize_single.gv -o axi_data_dnsize_single.png

# Downsize dual buffer
dot -Tpng axi_data_dnsize_dual.gv -o axi_data_dnsize_dual.png

### Generate SVG (for web/docs)

dot -Tsvg axi_data_upsize.gv -o axi_data_upsize.svg
# ... for all diagrams

### Generate PDF (for documentation)

dot -Tpdf axi_data_upsize.gv -o axi_data_upsize.pdf
# ... for all diagrams

## Diagram Contents

### axi_data_upsize.gv

Shows: - Narrow input interface (128-bit beats) - Beat counter (0→3 for 4:1 ratio) - 512-bit data buffer accumulation - Wide output interface (512-bit beats) - Sideband handling (WSTRB concatenation or RRESP OR) - 100% throughput annotation

Use Case: Understanding narrow-to-wide data accumulation

### axi_data_dnsize_single.gv

Shows: - Wide input interface (512-bit beats) - Single 512-bit buffer - 4:1 multiplexer for 128-bit slices - Output counter (0→3) - Narrow output interface (128-bit beats) - Timing diagram showing 80% throughput (1-cycle gap) - Sideband handling (WSTRB slice or RRESP broadcast)

Use Case: Understanding wide-to-narrow splitting (area-efficient mode)

### axi_data_dnsize_dual.gv

Shows: - Wide input interface (512-bit beats) - Dual ping-pong buffers (Buffer 0 and Buffer 1) - Buffer select logic (write/read muxes) - Independent counters for each buffer - Output multiplexing - Timing diagram showing 100% throughput (no gaps) - Ping-pong operation (one reads while other writes)

Use Case: Understanding dual-buffer high-performance mode

### axi4_dwidth_converter_wr.gv

Shows: - AXI4 slave interface (narrow - 64-bit) - AW/W/B channel handling - Address calculator and burst length adjuster - Skid buffers for flow control - axi_data_upsize integration - AXI4 master interface (wide - 512-bit) - Example transaction (8 narrow beats → 1 wide beat)

Use Case: Understanding full write path conversion

### axi4_dwidth_converter_rd.gv

Shows: - AXI4 slave interface (wide - 512-bit) - AR/R channel handling - Address calculator and burst length adjuster - Burst tracker for RLAST generation - axi_data_dnsize integration (dual buffer) - AXI4 master interface (narrow - 64-bit) - Continuous streaming timing (100% throughput)

Use Case: Understanding full read path conversion

### axi4_to_apb.gv

Shows: - AXI4 slave interface (64-bit addr, 32-bit data) - Protocol conversion state machine - Write path (AW + W → APB write) - Read path (AR → APB read) - Address width conversion (64→32 bit) - Response converter (PSLVERR → BRESP/RRESP) - APB master interface - FSM state descriptions - Example timing diagrams

Use Case: Understanding AXI4-to-APB protocol conversion

### peakrdl_adapter.gv

Shows: - PeakRDL register interface (APB-style) - Command generator - Response handler - Command/response interface (custom protocol) - Write transaction example - Read transaction example - Error propagation

Use Case: Understanding PeakRDL adapter protocol decoupling

## Diagram Style

Colors: - Light blue - Input interfaces (slave/narrow/wide) - Light coral/pink - Output interfaces (master/narrow/wide) - Light yellow - Control logic and notes - Light gray - Data path elements - Light green - Features/highlights - Light cyan - Examples - Orange - Warnings/limitations

Nodes: - Rounded boxes - Functional blocks - Cylinders - Registers/buffers - Diamonds - Decision/control points - Trapezoids - Multiplexers - Note boxes - Annotations, examples, timing diagrams - Records - Interfaces with multiple signals

Edges: - Bold solid - Primary data flow - Dashed - Control signals - Colored - Protocol-specific paths (blue=write, green=read)

## Adding New Diagrams

To add a new converter diagram:

- Create new .gv file:

- cp axi_data_upsize.gv my_new_converter.gv

- Edit the new file:

  - Update title and module name

  - Adjust nodes for converter functionality

  - Update data flow connections

  - Add timing/example annotations

- Update Makefile SOURCES list:

- SOURCES = ... \
          my_new_converter.gv

- Generate PNG:

- make all

- Link from documentation:

- ![My Converter Block Diagram](../assets/graphviz/my_new_converter.png)

## Referenced In Documentation

These diagrams are referenced in: - converter_index.md - Main specification index - ch02_data_width_converters/*.md - Data width converter chapters - ch03_protocol_converters/*.md - Protocol converter chapters - ch04_usage_examples/*.md - Usage and integration examples

## Tools

Graphviz Version: Any recent version (tested with 2.43+)

Alternative Viewers: - Online: http://www.webgraphviz.com/ (paste .gv content) - VS Code: Install “Graphviz Preview” extension - Command line: xdg-open <diagram>.png (Linux)

## Notes

- PNG files are version controlled for easy documentation viewing

- Source .gv files are the authoritative source

- Regenerate PNGs after any .gv file changes

- Keep diagrams up-to-date with module implementations

- Use consistent color scheme across all diagrams

Last Updated: 2025-10-26 Maintainer: RTL Design Sherpa Project

# Converters PlantUML FSM Diagrams

This directory contains PlantUML source files (.puml) and generated PNG diagrams for Converters FSM documentation.

## Files

### Source Files (.puml)

- axi4_to_apb_fsm.puml - APB state machine for AXI4-to-APB protocol converter

### Generated Images (.png)

- axi4_to_apb_fsm.png - FSM diagram for AXI4-to-APB converter

## FSM Overview

### AXI4-to-APB Converter FSM

States: - IDLE - Wait for AXI4 transaction (AR or AW) - READ - Process AXI4 AR transaction → APB read - WRITE - Process AXI4 AW+W transaction → APB write

Key Features: - Read priority over write when both pending - Burst support (continues in READ/WRITE until all beats complete) - APB protocol phases (Setup → Access → Wait) - Error response handling (PSLVERR → BRESP/RRESP)

Transitions: - IDLE → READ: AXI4 read request (ARVALID && ARREADY) - IDLE → WRITE: AXI4 write request (AWVALID && AWREADY) - READ → IDLE: APB read complete (PREADY && all beats done) - WRITE → IDLE: APB write complete (PREADY && all beats done) - Self-loops: Continue burst transfers

## Note on Other Converters

Data width converters (axi_data_upsize, axi_data_dnsize) do not have FSMs: - They use simple counter-based control logic - Upsize: Beat counter (0→N-1) to accumulate narrow beats - Downsize: Output counter (0→N-1) to split wide beats - No complex state transitions needed

Only protocol converters have FSMs: - AXI4-to-APB: Protocol translation requires state machine - PeakRDL adapter: Simple passthrough, no FSM needed

## Regenerating Diagrams

### Prerequisites

- PlantUML installed

- Java runtime (required by PlantUML)

### Install PlantUML

# Ubuntu/Debian
sudo apt install plantuml

# Or download JAR
wget https://github.com/plantuml/plantuml/releases/download/v1.2023.13/plantuml-1.2023.13.jar

### Generate PNG from PlantUML

# Using installed plantuml
plantuml axi4_to_apb_fsm.puml

# Or using downloaded JAR
java -jar plantuml.jar axi4_to_apb_fsm.puml

# Output: axi4_to_apb_fsm.png

### Generate SVG (for web)

plantuml -tsvg axi4_to_apb_fsm.puml

### Generate PDF (for documentation)

plantuml -tpdf axi4_to_apb_fsm.puml

## FSM Style

Colors: - Default state colors (light blue/yellow as per PlantUML defaults) - Transition arrows with labels showing conditions

Annotations: - Entry actions described in state boxes - Transition conditions on arrows - Notes explaining protocol sequences

Format: - Standard UML state machine notation - Clear state names matching RTL enumeration - Transition guards showing trigger conditions

## Referenced In Documentation

These diagrams are referenced in: - ch03_protocol_converters/02_axi4_to_apb.md - AXI4-to-APB converter chapter - converter_index.md - Main specification index

## Adding New FSM Diagrams

If future converters include FSMs:

- Create new .puml file:

- @startuml my_converter_fsm
title My Converter FSM

[*] --> IDLE
IDLE --> ACTIVE : trigger
ACTIVE --> IDLE : done

@enduml

- Generate PNG:

- plantuml my_converter_fsm.puml

- Link from documentation:

- ![My Converter FSM](../assets/puml/my_converter_fsm.png)

## Tools

PlantUML Version: Any recent version (tested with 1.2023+)

Alternative Renderers: - Online: http://www.plantuml.com/plantuml/uml/ (paste .puml content) - VS Code: Install “PlantUML” extension - IntelliJ IDEA: Built-in PlantUML support

## Notes

- PNG files are version controlled for easy viewing

- Source .puml files are the authoritative source

- Regenerate PNGs after any .puml changes

- Keep FSM diagrams synchronized with RTL state definitions

Last Updated: 2025-10-26 Maintainer: RTL Design Sherpa Project

# Converters - Data Width and Protocol Conversion Modules

Status: Production Ready Version: 1.2 Last Updated: 2025-10-25

## Quick Start

The Converters component provides both data width conversion and protocol conversion modules, enabling seamless integration between components with different data widths or protocols.

### Key Features

Data Width Converters: - Bidirectional Conversion - Upsize (narrow→wide) and Downsize (wide→narrow) - Flexible Width Ratios - Any integer ratio (2:1, 4:1, 8:1, 16:1, etc.) - Sideband Support - Configurable handling for WSTRB (slice) and RRESP (broadcast) - Burst Tracking - Optional burst-aware LAST signal generation (read path) - High Throughput - Optional dual-buffer mode for 100% throughput (downsize) - Generic Building Blocks - Reusable axi_data_upsize and axi_data_dnsize modules

Protocol Converters: - AXI4-to-APB Bridge - Full AXI4 to APB protocol conversion with address/data width adaptation - PeakRDL Adapter - Convert PeakRDL register interface to custom command/response protocol

### Performance Summary

| Module | Mode | Throughput | Area | Use Case |
| --- | --- | --- | --- | --- |
| axi_data_upsize | Single buffer | 100% | 1× | Narrow→Wide (always optimal) |
| axi_data_dnsize | Single buffer | 80% | 1× | Wide→Narrow (area-efficient) |
| axi_data_dnsize | Dual buffer | 100% | 2× | Wide→Narrow (high-performance) |

## Architecture Overview

### Component Hierarchy

Converters Component
├── Data Width Converters:
│   ├── Generic Building Blocks:
│   │   ├── axi_data_upsize.sv      - Narrow→Wide accumulator (100% throughput)
│   │   ├── axi_data_dnsize.sv      - Wide→Narrow splitter (80% or 100% throughput)
│   │   └── Key Features:
│   │       ├── Configurable sideband handling
│   │       ├── Optional burst tracking
│   │       └── Dual-buffer mode (dnsize only)
│   │
│   └── Full AXI4 Converters:
│       ├── axi4_dwidth_converter_wr.sv  - Write path converter (AW + W + B channels)
│       ├── axi4_dwidth_converter_rd.sv  - Read path converter (AR + R channels)
│       └── Integration:
│           ├── Skid buffers for flow control
│           ├── Address phase management
│           └── Response path handling
│
└── Protocol Converters:
    ├── axi4_to_apb_convert.sv   - AXI4-to-APB bridge (full protocol conversion)
    ├── axi4_to_apb_shim.sv      - AXI4-to-APB adapter (simplified wrapper)
    └── peakrdl_to_cmdrsp.sv     - PeakRDL adapter (register→command/response)

### Data Flow Examples

Write Path Downsize (512→128 bits):

┌────────────────────────────────────────────────────┐
│ Slave Interface (512-bit)                          │
│   ┌──────────┐                                     │
│   │ AW       │ ← 1 address                         │
│   │ W        │ ← 1 wide beat (512-bit + 64-bit WSTRB)
│   │ B        │ → 1 response                        │
│   └──────────┘                                     │
│       ↓                                            │
│   ┌──────────────────┐                             │
│   │ axi_data_dnsize  │ (DUAL_BUFFER=1)             │
│   │  512→128 split   │                             │
│   └──────────────────┘                             │
│       ↓                                            │
│   ┌──────────┐                                     │
│   │ Master Interface (128-bit)                     │
│   │ AW       │ → 1 address                         │
│   │ W        │ → 4 narrow beats (128-bit + 16-bit WSTRB each)
│   │ B        │ ← 1 response                        │
│   └──────────┘                                     │
└────────────────────────────────────────────────────┘

Read Path Upsize (128→512 bits):

┌────────────────────────────────────────────────────┐
│ Master Interface (512-bit)                         │
│   ┌──────────┐                                     │
│   │ AR       │ → 1 address                         │
│   │ R        │ ← 1 wide beat (512-bit + 2-bit RRESP)
│   └──────────┘                                     │
│       ↑                                            │
│   ┌──────────────────┐                             │
│   │ axi_data_upsize  │ (TRACK_BURSTS=1)            │
│   │  128→512 accum   │                             │
│   └──────────────────┘                             │
│       ↑                                            │
│   ┌──────────┐                                     │
│   │ Slave Interface (128-bit)                      │
│   │ AR       │ ← 1 address                         │
│   │ R        │ → 4 narrow beats (128-bit + 2-bit RRESP each)
│   └──────────┘                                     │
└────────────────────────────────────────────────────┘

## Module Documentation

### 1. axi_data_upsize.sv - Narrow→Wide Accumulator

Purpose: Accumulates multiple narrow beats into a single wide beat

Throughput: 100% (single buffer sufficient)

Key Parameters:

parameter int NARROW_WIDTH    = 32;      // Input data width
parameter int WIDE_WIDTH      = 128;     // Output data width (must be integer multiple)
parameter int NARROW_SB_WIDTH = 4;       // Narrow sideband width (WSTRB: WIDTH/8)
parameter int WIDE_SB_WIDTH   = 16;      // Wide sideband width (WSTRB: WIDTH/8)
parameter int SB_OR_MODE      = 0;       // 0=concatenate (WSTRB), 1=OR (RRESP)

Sideband Modes: - SB_OR_MODE=0 (Concatenate): For WSTRB - assemble strobe bits narrow[0].wstrb = 4'b1111 → wide.wstrb[3:0]   narrow[1].wstrb = 4'b1100 → wide.wstrb[7:4]   narrow[2].wstrb = 4'b0011 → wide.wstrb[11:8]   narrow[3].wstrb = 4'b1111 → wide.wstrb[15:12]   Result: wide.wstrb = 16'b1111_0011_1100_1111

- SB_OR_MODE=1 (OR Together): For RRESP - propagate errors

- narrow[0].rresp = 2'b00 (OK)
narrow[1].rresp = 2'b10 (SLVERR)
narrow[2].rresp = 2'b00 (OK)
narrow[3].rresp = 2'b00 (OK)
Result: wide.rresp = 2'b10 (SLVERR - any error propagates)

Usage Example:

axi_data_upsize #(
    .NARROW_WIDTH(128),
    .WIDE_WIDTH(512),
    .NARROW_SB_WIDTH(16),  // WSTRB: 128/8 = 16
    .WIDE_SB_WIDTH(64),    // WSTRB: 512/8 = 64
    .SB_OR_MODE(0)         // Concatenate WSTRB
) u_upsize (
    .aclk             (aclk),
    .aresetn          (aresetn),

    // Narrow input
    .narrow_valid     (s_wvalid),
    .narrow_ready     (s_wready),
    .narrow_data      (s_wdata),
    .narrow_sideband  (s_wstrb),
    .narrow_last      (s_wlast),

    // Wide output
    .wide_valid       (m_wvalid),
    .wide_ready       (m_wready),
    .wide_data        (m_wdata),
    .wide_sideband    (m_wstrb),
    .wide_last        (m_wlast)
);

### 2. axi_data_dnsize.sv - Wide→Narrow Splitter

Purpose: Splits single wide beat into multiple narrow beats

Throughput: - Single-buffer mode (DUAL_BUFFER=0): 80% (1-cycle gap per wide beat) - Dual-buffer mode (DUAL_BUFFER=1): 100% (continuous streaming)

Key Parameters:

parameter int WIDE_WIDTH        = 512;     // Input data width
parameter int NARROW_WIDTH      = 128;     // Output data width (must be integer divisor)
parameter int WIDE_SB_WIDTH     = 2;       // Wide sideband width (RRESP: 2 bits)
parameter int NARROW_SB_WIDTH   = 2;       // Narrow sideband width
parameter int SB_BROADCAST      = 1;       // 1=broadcast (RRESP), 0=slice (WSTRB)
parameter int TRACK_BURSTS      = 0;       // 1=track bursts for LAST, 0=simple passthrough
parameter int BURST_LEN_WIDTH   = 8;       // Burst length counter width
parameter int DUAL_BUFFER       = 0;       // 1=dual buffer (100% throughput, 2× area)
                                           // 0=single buffer (80% throughput, 1× area)

Sideband Modes: - SB_BROADCAST=1: Broadcast same value to all narrow beats (RRESP) wide.rresp = 2'b10 (SLVERR)   → narrow[0].rresp = 2'b10   → narrow[1].rresp = 2'b10   → narrow[2].rresp = 2'b10   → narrow[3].rresp = 2'b10

- SB_BROADCAST=0: Slice into narrow portions (WSTRB)

- wide.wstrb = 64'h000F_F0FF_00FF_FFFF
→ narrow[0].wstrb[15:0]  = 16'hFFFF
→ narrow[1].wstrb[15:0]  = 16'h00FF
→ narrow[2].wstrb[15:0]  = 16'hF0FF
→ narrow[3].wstrb[15:0]  = 16'h000F

Burst Tracking Mode: - TRACK_BURSTS=0: Pass wide_last to last narrow beat (simple mode) - TRACK_BURSTS=1: Generate LAST on final beat of entire burst (read path)

Dual-Buffer Mode (NEW in v1.1):

Single-buffer mode achieves 80% throughput due to 1-cycle gap when transitioning between wide beats:

Cycle: 1    2    3    4    5    6    7    8    9   10
Wide:  [====BEAT_0====]  WAIT [====BEAT_1====]  WAIT
Narrow: n0   n1   n2   n3  IDLE  n0   n1   n2   n3  IDLE
                            ↑ 1-cycle dead time

Dual-buffer mode eliminates the gap by ping-ponging between two buffers:

Cycle: 1    2    3    4    5    6    7    8    9
Wide:  [====BEAT_0====][====BEAT_1====][====BEAT_2====]
Narrow: n0   n1   n2   n3   n0   n1   n2   n3   n0
Buffer: BUF0 reading────┘   BUF1 reading────┘   BUF0...
        BUF1 writing────────┘   BUF0 writing────┘

When to Use Dual-Buffer Mode: - High-bandwidth DMA engines with continuous streaming - Performance-critical data paths where 100% utilization required - Sufficient area budget (~2× increase for buffer registers)

When to Use Single-Buffer Mode: - Area-constrained designs - Throughput requirements <100% - Natural gaps in traffic from upstream/downstream

Usage Example (Single-Buffer):

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(2),      // RRESP: 2 bits
    .NARROW_SB_WIDTH(2),
    .SB_BROADCAST(1),       // Broadcast RRESP
    .TRACK_BURSTS(1),       // Track bursts for LAST
    .BURST_LEN_WIDTH(8),
    .DUAL_BUFFER(0)         // Single buffer (80% throughput, area-efficient)
) u_dnsize (
    .aclk             (aclk),
    .aresetn          (aresetn),

    // Burst control (TRACK_BURSTS=1)
    .burst_len        (arlen),
    .burst_start      (arvalid && arready),

    // Wide input
    .wide_valid       (s_rvalid),
    .wide_ready       (s_rready),
    .wide_data        (s_rdata),
    .wide_sideband    (s_rresp),
    .wide_last        (s_rlast),

    // Narrow output
    .narrow_valid     (m_rvalid),
    .narrow_ready     (m_rready),
    .narrow_data      (m_rdata),
    .narrow_sideband  (m_rresp),
    .narrow_last      (m_rlast)
);

Usage Example (Dual-Buffer - High Performance):

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(64),     // WSTRB: 512/8 = 64
    .NARROW_SB_WIDTH(16),   // WSTRB: 128/8 = 16
    .SB_BROADCAST(0),       // Slice WSTRB
    .TRACK_BURSTS(0),       // Simple mode for write path
    .DUAL_BUFFER(1)         // Dual buffer (100% throughput, 2× area)
) u_dnsize_hp (
    .aclk             (aclk),
    .aresetn          (aresetn),

    // Burst control (unused in TRACK_BURSTS=0)
    .burst_len        (8'd0),
    .burst_start      (1'b0),

    // Wide input
    .wide_valid       (s_wvalid),
    .wide_ready       (s_wready),
    .wide_data        (s_wdata),
    .wide_sideband    (s_wstrb),
    .wide_last        (s_wlast),

    // Narrow output
    .narrow_valid     (m_wvalid),
    .narrow_ready     (m_wready),
    .narrow_data      (m_wdata),
    .narrow_sideband  (m_wstrb),
    .narrow_last      (m_wlast)
);

### 3. Full AXI4 Converters

axi4_dwidth_converter_wr.sv - Complete write path converter (AW + W + B)

axi4_dwidth_converter_rd.sv - Complete read path converter (AR + R)

These integrate the generic building blocks with: - Address phase management - Skid buffers for flow control - Response path handling - Full AXI4 protocol compliance

## Protocol Converters

### 4. AXI4-to-APB Bridge (axi4_to_apb_convert.sv)

Purpose: Full protocol conversion from AXI4 to APB with address/data width adaptation

Key Features: - Converts AXI4 read/write transactions to APB protocol - Handles address width conversion (AXI4 64-bit → APB 32-bit) - Data width adaptation (configurable) - State machine for AXI→APB protocol translation - Error response handling (SLVERR/DECERR)

Usage Example:

axi4_to_apb_convert #(
    .S_AXI_ADDR_WIDTH(64),
    .S_AXI_DATA_WIDTH(32),
    .M_APB_ADDR_WIDTH(32),
    .M_APB_DATA_WIDTH(32)
) u_axi_apb_bridge (
    .aclk           (clk),
    .aresetn        (rst_n),

    // AXI4 Slave Interface
    .s_axi_awaddr   (s_awaddr),
    .s_axi_awvalid  (s_awvalid),
    .s_axi_awready  (s_awready),
    // ... (full AXI4 AW/W/B/AR/R channels)

    // APB Master Interface
    .m_apb_paddr    (m_paddr),
    .m_apb_psel     (m_psel),
    .m_apb_penable  (m_penable),
    .m_apb_pwrite   (m_pwrite),
    .m_apb_pwdata   (m_pwdata),
    .m_apb_pready   (m_pready),
    .m_apb_prdata   (m_prdata),
    .m_apb_pslverr  (m_pslverr)
);

Use Cases: - Connecting AXI4 masters to APB peripherals - CPU to APB peripheral bus bridges - System integration with mixed protocols

### 5. PeakRDL-to-CmdRsp Adapter (peakrdl_to_cmdrsp.sv)

Purpose: Convert PeakRDL-generated register interface to custom command/response protocol

Key Features: - Converts APB-style register interface to command/response handshake - Supports read/write operations - Configurable command/response data widths - Single-cycle command issue - Pipelined response handling

Usage Example:

peakrdl_to_cmdrsp #(
    .ADDR_WIDTH(16),
    .DATA_WIDTH(32)
) u_peakrdl_adapter (
    .clk            (clk),
    .rst_n          (rst_n),

    // PeakRDL Register Interface (APB-style)
    .reg_addr       (reg_addr),
    .reg_wdata      (reg_wdata),
    .reg_write      (reg_write),
    .reg_read       (reg_read),
    .reg_rdata      (reg_rdata),
    .reg_error      (reg_error),

    // Command/Response Interface
    .cmd_valid      (cmd_valid),
    .cmd_ready      (cmd_ready),
    .cmd_addr       (cmd_addr),
    .cmd_data       (cmd_data),
    .cmd_write      (cmd_write),

    .rsp_valid      (rsp_valid),
    .rsp_ready      (rsp_ready),
    .rsp_data       (rsp_data),
    .rsp_error      (rsp_error)
);

Use Cases: - Interfacing PeakRDL register blocks to custom control logic - Register access through command/response protocol - Decoupling register interface from implementation

## Configuration Examples

### Example 1: Write Path Downsize (128→32 bits)

Use Case: ARM Cortex-M7 (128-bit AXI) → APB Bridge (32-bit)

axi_data_dnsize #(
    .WIDE_WIDTH(128),
    .NARROW_WIDTH(32),
    .WIDE_SB_WIDTH(16),     // WSTRB: 128/8 = 16
    .NARROW_SB_WIDTH(4),    // WSTRB: 32/8 = 4
    .SB_BROADCAST(0),       // Slice WSTRB
    .TRACK_BURSTS(0),       // Write path: simple mode
    .DUAL_BUFFER(0)         // Area-efficient
) u_wr_dnsize (
    // ... ports
);

Result: 1 wide W beat (128-bit) → 4 narrow W beats (32-bit each)

### Example 2: Read Path Upsize (256→512 bits)

Use Case: DDR4 Controller (512-bit) ← FPGA Fabric (256-bit)

axi_data_upsize #(
    .NARROW_WIDTH(256),
    .WIDE_WIDTH(512),
    .NARROW_SB_WIDTH(2),    // RRESP: 2 bits
    .WIDE_SB_WIDTH(2),      // RRESP: 2 bits
    .SB_OR_MODE(1)          // OR together RRESP
) u_rd_upsize (
    // ... ports
);

Result: 2 narrow R beats (256-bit) → 1 wide R beat (512-bit)

### Example 3: High-Performance Write Downsize (512→128 bits)

Use Case: DMA Engine (512-bit) → PCIe Endpoint (128-bit), continuous streaming

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(64),     // WSTRB: 512/8 = 64
    .NARROW_SB_WIDTH(16),   // WSTRB: 128/8 = 16
    .SB_BROADCAST(0),       // Slice WSTRB
    .TRACK_BURSTS(0),       // Write path: simple mode
    .DUAL_BUFFER(1)         // HIGH-PERFORMANCE: 100% throughput
) u_wr_dnsize_hp (
    // ... ports
);

Result: 100% throughput (no dead cycles), 2× area cost

## Testing

### Test Organization

projects/components/converters/dv/tests/
├── test_axi_data_upsize.py       - Generic upsize module tests
├── test_axi_data_dnsize.py       - Generic dnsize module tests (16 configs)
├── test_axi4_dwidth_converter_wr.py  - Full write converter tests
└── test_axi4_dwidth_converter_rd.py  - Full read converter tests

### Test Configurations (axi_data_dnsize.py)

Single-Buffer Tests (DUAL_BUFFER=0): 1. 128→32 WSTRB slice (simple mode) 2. 256→64 WSTRB slice (simple mode) 3. 128→32 RRESP broadcast (simple mode) 4. 256→64 RRESP broadcast (simple mode) 5. 128→32 RRESP broadcast (burst tracking) 6. 256→64 RRESP broadcast (burst tracking) 7. 512→128 RRESP broadcast (burst tracking) 8. 128→64 no sideband (simple mode)

Dual-Buffer Tests (DUAL_BUFFER=1): 9-16. Same configurations as above with DUAL_BUFFER=1

### Running Tests

# Run all converter tests
cd $REPO_ROOT/projects/components/converters/dv/tests
make run-all-parallel          # FUNC level, 48 workers

# Run specific module tests
make run-dnsize-func           # Downsize tests
make run-upsize-func           # Upsize tests

# Run with different test levels
make run-all-gate-parallel     # Quick smoke test
make run-all-func-parallel     # Functional coverage (default)
make run-all-full-parallel     # Comprehensive validation

# Individual test
pytest test_axi_data_dnsize.py::test_axi_data_dnsize[128to32_wstrb_slice_simple_DUAL] -v

## Quality Assurance

### Lint Checks

# Run all lint tools
cd $REPO_ROOT/projects/components/converters/rtl
make lint-all

# Individual tools
make verilator    # Verilator lint
make verible      # Verible style check
make yosys        # Yosys synthesis check

# View status
make status

### Expected Lint Results

axi_data_upsize.sv and axi_data_dnsize.sv: - Clean compilation (warnings only for unused signals in certain parameter configurations) - Warnings are benign (e.g., narrow_sideband unused when NARROW_SB_WIDTH=0)

**axi4_dwidth_converter_*.sv:** - Requires rtl/amba/gaxi/ include path for skid buffer modules - PINCONNECTEMPTY warnings are expected (unused count outputs)

## Documentation

### Available Documentation

- README.md (this file) - Quick start and overview

- GENERIC_MODULES_USAGE_GUIDE.md - Detailed parameter guide for upsize/dnsize

- DUAL_BUFFER_IMPLEMENTATION.md - Comprehensive dual-buffer feature documentation

- ANALYSIS_APB_CONVERTER.md - APB protocol converter analysis

### Key Resources

For Understanding Parameters:

cat $REPO_ROOT/projects/components/converters/rtl/GENERIC_MODULES_USAGE_GUIDE.md

For Dual-Buffer Mode:

cat $REPO_ROOT/projects/components/converters/DUAL_BUFFER_IMPLEMENTATION.md

For APB Integration:

cat $REPO_ROOT/projects/components/converters/ANALYSIS_APB_CONVERTER.md

## Quick Commands

# Setup environment
source $REPO_ROOT/env_python

# Run all tests (parallel)
cd $REPO_ROOT/projects/components/converters/dv/tests
make run-all-parallel

# Lint all RTL
cd $REPO_ROOT/projects/components/converters/rtl
make lint-all

# View test status
cd $REPO_ROOT/projects/components/converters/dv/tests
make status

# Clean all artifacts
make clean-all

## Design Decisions

### Why No Dual-Buffer for Upsize?

axi_data_upsize already achieves 100% throughput with single buffer: - Can accept narrow beat while outputting wide beat simultaneously - The || wide_ready term in narrow_ready enables pipelining - No benefit from dual buffering

### Why Optional Dual-Buffer for Dnsize?

Trade-off between area and performance: - Single-buffer: 80% throughput, 1× area (good for most use cases) - Dual-buffer: 100% throughput, 2× area (high-performance paths) - User can select based on system requirements

### Why Separate Upsize/Dnsize Modules?

Promotes reuse and flexibility: - Can be used independently in custom converters - Write converter: upsize + dnsize combination - Read converter: dnsize + upsize combination - Other use cases: data width matching, FIFO interfaces, etc.

## Known Limitations

- Width Ratio Constraint: WIDE_WIDTH must be exact integer multiple of NARROW_WIDTH

- Alignment: Full converters require aligned addresses (handled by full converter modules)

- Burst Length: Burst tracking mode supports AXI4 burst lengths (BURST_LEN_WIDTH=8)

- Sideband Width: Must match data width ratios for slice mode

## Future Enhancements

- Configurable Buffer Depth: Allow >2 buffers for higher throughput

- Performance Counters: Monitor stalls, utilization, throughput

- Power Gating: Disable unused buffer in dual-buffer mode

- Credit-Based Flow Control: Integration with upstream credit systems

## Version History

- v1.1 (2025-10-25): Added dual-buffer mode for axi_data_dnsize (100% throughput)

- v1.0 (2025-10-24): Initial release with generic modules and full converters

## Related Components

- STREAM - DMA engine using width converters for descriptor fetch

- RAPIDS - Accelerator with width conversion on data paths

- APB HPET - APB peripheral using narrow interfaces

- Bridge - Protocol converters with width adaptation

Author: RTL Design Sherpa Project Last Updated: 2025-10-25

# Dual-Buffer Implementation for axi_data_dnsize

Date: 2025-10-25 Purpose: Document the dual-buffer feature implementation for high-throughput data width conversion

## Executive Summary

Added optional dual-buffer mode to axi_data_dnsize.sv module to achieve 100% throughput for continuous data streams.

Key Results: - Single-buffer mode (DUAL_BUFFER=0): 80% throughput, 1-cycle dead time per wide beat - Dual-buffer mode (DUAL_BUFFER=1): 100% throughput, zero dead cycles - Area cost: Approximately 2× buffer registers (+100% overhead) - Compatibility: Fully backward compatible, works with all existing test configurations

## Architecture Overview

### Single-Buffer Mode (DUAL_BUFFER=0)

Structure:

┌─────────────────┐
│  r_data_buffer  │  ← ONE wide beat buffered
│  (WIDE_WIDTH)   │
└─────────────────┘
     ↓
  Split into
  narrow beats

Throughput Limitation: - Must wait for ALL narrow beats to complete before accepting next wide beat - 1-cycle gap when transitioning between wide beats - Throughput = 4/(4+1) = 80% for 4:1 ratio

### Dual-Buffer Mode (DUAL_BUFFER=1)

Structure:

┌─────────────┐
│  r_buffer_0  │  ← Splitting into narrow beats
│              │
└─────────────┘
     ↓ (reading)

┌─────────────┐
│  r_buffer_1  │  ← Accepting NEXT wide beat
│              │
└─────────────┘
     ↑ (writing)

Ping-Pong Operation: 1. Buffer 0 splits while Buffer 1 accepts new data 2. When Buffer 0 completes, swap: Buffer 1 splits, Buffer 0 accepts 3. Continuous operation with no dead cycles

Throughput: 100% (4/4) for any ratio

## Implementation Details

### New Parameter

parameter int DUAL_BUFFER = 0,  // 0=single buffer (80% throughput, area efficient)
                                // 1=dual buffer (100% throughput, 2× area)

### State Variables

Single-Buffer Mode:

logic [WIDE_WIDTH-1:0]          r_data_buffer;
logic [WIDE_SB_PORT_WIDTH-1:0]  r_sideband_buffer;
logic                           r_wide_buffered;
logic                           r_last_buffered;

Dual-Buffer Mode:

logic [WIDE_WIDTH-1:0]          r_buffer_0, r_buffer_1;
logic [WIDE_SB_PORT_WIDTH-1:0]  r_sb_buffer_0, r_sb_buffer_1;
logic                           r_last_buffer_0, r_last_buffer_1;
logic                           r_buffer_0_valid, r_buffer_1_valid;
logic                           r_read_buffer;  // 0=reading buf0, 1=reading buf1

### Write Path (DUAL_BUFFER=1)

if (wide_valid && wide_ready) begin
    if (!gen_dual_buffer.r_buffer_0_valid) begin
        // Write to buffer 0
        gen_dual_buffer.r_buffer_0 <= wide_data;
        gen_dual_buffer.r_buffer_0_valid <= 1'b1;
    end else begin
        // Write to buffer 1 (must be empty if wide_ready=1)
        gen_dual_buffer.r_buffer_1 <= wide_data;
        gen_dual_buffer.r_buffer_1_valid <= 1'b1;
    end
end

Key: Always write to the empty buffer.

### Read Path (DUAL_BUFFER=1)

// Read from current buffer
if (current_buffer_valid && narrow_ready) begin
    if (is_last_narrow_beat) begin
        // Clear current buffer's valid flag
        if (gen_dual_buffer.r_read_buffer)
            gen_dual_buffer.r_buffer_1_valid <= 1'b0;
        else
            gen_dual_buffer.r_buffer_0_valid <= 1'b0;

        // Swap to other buffer if it has data
        if (other_buffer_valid)
            gen_dual_buffer.r_read_buffer <= ~gen_dual_buffer.r_read_buffer;
    end
end

Key: On last narrow beat, clear current buffer and swap to other if available.

### Ready Logic

Single-Buffer:

assign wide_ready = !r_wide_buffered || (narrow_ready && w_last_narrow_beat);

- Ready when buffer empty OR sending last narrow beat (1-cycle early)

Dual-Buffer:

assign wide_ready = !r_buffer_0_valid || !r_buffer_1_valid;

- Ready when at least one buffer is empty

- Allows continuous acceptance

## Burst Tracking Integration

Dual-buffer mode fully supports burst tracking (TRACK_BURSTS=1):

Challenge: Track burst position across buffer swaps

Solution: Shared burst counter applies to whichever buffer is currently being read:

// Burst tracking state (shared between buffers)
logic [BURST_LEN_WIDTH-1:0] r_slave_beat_count;
logic [BURST_LEN_WIDTH-1:0] r_slave_total_beats;
logic                       r_burst_active;

// Applies to current read buffer
if (((r_slave_beat_count + 1'b1) >= r_slave_total_beats)) begin
    // Last narrow beat of entire burst
    // Clear current buffer and end burst
end

Result: LAST signal correctly generated on final beat of burst, regardless of buffer swapping.

## Resource Impact

### Area Analysis (128-bit Wide, 32-bit Narrow, 16-bit Sideband)

| Resource | Single Buffer | Dual Buffer | Overhead |
| --- | --- | --- | --- |
| Data registers | 128 bits | 256 bits | +128 bits |
| Sideband registers | 16 bits | 32 bits | +16 bits |
| LAST flags | 1 bit | 2 bits | +1 bit |
| Valid flags | 1 bit | 2 bits | +1 bit |
| Read selector | - | 1 bit | +1 bit |
| Total FFs | 146 | 292 | +146 (+100%) |

Control Logic: ~30% increase (buffer selection, swap logic)

Overall: Expect ~100% area increase for dual-buffer mode.

### Performance Impact

| Mode | Throughput (4:1 Ratio) | Cycles/Wide Beat | Utilization |
| --- | --- | --- | --- |
| Single-Buffer | 80% | 5 (4 active + 1 dead) | 4/5 |
| Dual-Buffer | 100% | 4 (4 active + 0 dead) | 4/4 |

Improvement: +25% throughput (from 80% to 100%)

## Test Coverage

### Test Configurations Added

All existing test configurations now have dual-buffer variants:

Single-Buffer Tests (DUAL_BUFFER=0): - 128to32_wstrb_slice_simple - 256to64_wstrb_slice_simple - 128to32_rresp_broadcast_simple - 256to64_rresp_broadcast_simple - 128to32_rresp_burst_track - 256to64_rresp_burst_track - 512to128_rresp_burst_track - 128to64_no_sideband_simple

Dual-Buffer Tests (DUAL_BUFFER=1): - Same configurations with “_DUAL” suffix - Total: 16 test configurations

### Verification Results

Tested: - ✓ Basic data splitting (all ratios) - ✓ Sideband handling (broadcast and slice modes) - ✓ Burst tracking with LAST generation - ✓ Backpressure handling - ✓ Continuous streaming

Result: All tests PASS for both single and dual-buffer modes.

## Usage Recommendations

### When to Use Single-Buffer Mode (DUAL_BUFFER=0)

✓ Use when: - Area is critical - Throughput requirements are <100% - Source/sink have natural gaps in traffic - Design is already bottlenecked elsewhere

Example: Write path downsize where upstream has occasional pauses

### When to Use Dual-Buffer Mode (DUAL_BUFFER=1)

✓ Use when: - Maximum throughput required - Continuous streaming data - Sufficient area budget - Critical path in high-performance system

Example: High-bandwidth DMA engine with continuous transfers

## Code Example

### Instantiation: Single-Buffer Mode (Area Efficient)

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(64),      // WSTRB
    .NARROW_SB_WIDTH(16),    // WSTRB
    .SB_BROADCAST(0),        // Slice mode
    .TRACK_BURSTS(0),        // Simple mode
    .DUAL_BUFFER(0)          // Single buffer (80% throughput)
) u_dnsize (
    .aclk(clk),
    .aresetn(rst_n),
    // ... ports
);

### Instantiation: Dual-Buffer Mode (High Throughput)

axi_data_dnsize #(
    .WIDE_WIDTH(512),
    .NARROW_WIDTH(128),
    .WIDE_SB_WIDTH(64),
    .NARROW_SB_WIDTH(16),
    .SB_BROADCAST(0),
    .TRACK_BURSTS(0),
    .DUAL_BUFFER(1)          // Dual buffer (100% throughput)
) u_dnsize (
    .aclk(clk),
    .aresetn(rst_n),
    // ... ports
);

## Design Decisions

### Why Not Dual-Buffer for Upsize?

axi_data_upsize already achieves 100% throughput with single buffer: - Can accept narrow beat while outputting wide beat simultaneously - The || wide_ready term in narrow_ready enables overlap - No benefit from dual buffering

Conclusion: Dual-buffer only needed for dnsize module.

### Why Use Generate Blocks?

Rationale: - Complete separation of single vs. dual-buffer logic - No runtime overhead (compile-time selection) - Easier to verify each mode independently - Cleaner code structure

### Why Not Make Dual-Buffer the Default?

Considerations: - 100% area overhead is significant - Many use cases don’t need 100% throughput - Single-buffer is simpler and well-tested - User can opt-in when needed

Decision: Default to DUAL_BUFFER=0, user explicitly enables when needed.

## Lessons Learned

### SystemVerilog Constraints

Issue: Cannot declare logic variables inside procedural blocks

// ILLEGAL:
always_ff @(posedge clk) begin
    logic temp;  // ← NOT ALLOWED
    temp = ...;
end

Solution: Inline expressions or move declarations outside block

### Generate Block Naming

Best Practice: Use hierarchical naming for generate block signals:

gen_dual_buffer.r_buffer_0_valid  // Clear which mode
gen_single_buffer.r_wide_buffered // Clear which mode

### Buffer Swap Logic

Key Insight: Swap only when: 1. Current buffer completes (last narrow beat sent) 2. Other buffer has valid data waiting

This prevents unnecessary swaps and maintains correct ordering.

## Future Enhancements

### Potential Improvements

- Configurable Buffer Depth: Allow >2 buffers for even higher throughput

- Credit-Based Flow Control: Integrate with upstream credit system

- Performance Counters: Monitor buffer utilization, stalls

- Power Gating: Disable unused buffer when not needed

### Alternative Architectures

Skid Buffer Approach: - Insert skid buffers on outputs instead of dual data buffers - Lower area overhead - Similar throughput improvement

FIFO Approach: - Replace buffers with small FIFOs - More flexible depth control - Higher area cost

## Summary

The dual-buffer implementation successfully provides an optional high-throughput mode for the axi_data_dnsize module:

✓ Functionality: Proven correct through comprehensive testing ✓ Performance: 100% throughput vs. 80% for single-buffer ✓ Compatibility: Fully backward compatible ✓ Flexibility: User-selectable via parameter ✓ Robustness: Works with all modes (broadcast, slice, burst tracking)

Trade-off: ~100% area increase for +25% throughput improvement

Recommendation: Use dual-buffer mode for performance-critical paths with continuous data flow.

## Related Files

- projects/components/converters/rtl/axi_data_dnsize.sv - RTL implementation

- projects/components/converters/dv/tests/test_axi_data_dnsize.py - Test suite

- projects/components/converters/dv/tbclasses/axi_data_dnsize_tb.py - Testbench class

- projects/components/converters/USAGE.md - Usage documentation

- projects/components/converters/ANALYSIS_APB_CONVERTER.md - APB converter analysis

Author: RTL Design Sherpa Date: 2025-10-25

# Analysis: APB Converter vs. Generic Data Width Converters

Date: 2025-10-25 Purpose: Analyze whether axi4_to_apb_convert.sv could benefit from generic axi_data_upsize and axi_data_dnsize modules

## Executive Summary

Finding: The APB converter implements data width conversion using identical algorithmic patterns to our generic modules, but in a fundamentally different usage model that makes direct integration impractical.

Recommendation: Do NOT refactor the APB converter to use generic modules. The tight coupling between protocol conversion and data width conversion requires the current inline implementation.

Value: The analysis validates that our generic module algorithms are correct - an independent implementation arrived at the same solution.

## Data Width Conversion Patterns Found

### 1. WRITE Path (Dnsize: Wide AXI → Narrow APB)

Location: axi4_to_apb_convert.sv lines 334, 337, 461

Implementation:

// Line 334 - Extract narrow data slice
w_apb_cmd_pkt_pwdata = (axi2abpratio == 1) ? r_s_axi_wdata[APBDW-1:0] :
                        r_s_axi_wdata[r_axi_wr_data_pointer*APBDW +: APBDW];

// Line 337 - Extract narrow strobe slice
w_apb_cmd_pkt_pstrb = (axi2abpratio == 1) ? r_s_axi_wstrb[APBSW-1:0] :
                       r_s_axi_wstrb[r_axi_wr_data_pointer*APBSW +: APBSW];

// Line 461 - Increment pointer through wide beat
w_axi_wr_data_pointer = r_axi_wr_data_pointer + 1;

Pattern: IDENTICAL to axi_data_dnsize module (line 210, 220) - Extract slice using pointer: data[ptr*WIDTH +: WIDTH] - Increment pointer for next narrow beat - Wrap at ratio-1

### 2. READ Path (Upsize: Narrow APB → Wide AXI)

Location: axi4_to_apb_convert.sv lines 283, 366

Implementation:

// Line 283 - Accumulate narrow data into wide shift register
r_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] <= r_apb_rsp_pkt_prdata;

// Line 366 - Combinational accumulation (alternative path)
w_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] = r_apb_rsp_pkt_prdata;

// Lines 284-287 - Increment and wrap pointer
r_axi_rsp_data_pointer <= r_axi_rsp_data_pointer + 1;
if (r_axi_rsp_data_pointer == PTR_WIDTH'(axi2abpratio-1)) begin
    r_axi_rsp_data_pointer <= 'b0;
end

Pattern: IDENTICAL to axi_data_upsize module (line 158) - Accumulate into shift register at pointer position - Increment pointer for next narrow beat - Complete wide beat when pointer reaches ratio-1

### 3. Pointer Management

APB Converter has THREE separate pointers: - r_axi_wr_data_pointer - WRITE path dnsize (lines 315-317) - r_axi_rd_data_pointer - READ path dnsize (lines 309-311) - r_axi_rsp_data_pointer - READ response upsize (lines 284-287)

Generic Modules have ONE pointer each: - axi_data_dnsize: r_beat_ptr for splitting wide→narrow - axi_data_upsize: r_beat_ptr for accumulating narrow→wide

## Key Architectural Differences

### Generic Modules (axi_data_upsize / axi_data_dnsize)

Usage Model: Complete Beat Processing - axi_data_upsize: Wait for ALL narrow beats → output ONE complete wide beat - axi_data_dnsize: Accept ONE wide beat → output ALL narrow beats in sequence - Pure streaming pipeline with valid/ready handshaking - Standalone operation - can be inserted in any data path

State Machine: - Simple: Buffer empty/full state - No burst tracking (optional feature) - Focus solely on data width conversion

Example Flow (128→32 upsize):

1. Receive narrow beat 0 (bits [31:0])   → accumulate, ptr=1
2. Receive narrow beat 1 (bits [63:32])  → accumulate, ptr=2
3. Receive narrow beat 2 (bits [95:64])  → accumulate, ptr=3
4. Receive narrow beat 3 (bits [127:96]) → accumulate, OUTPUT wide beat, ptr=0

### APB Converter (axi4_to_apb_convert.sv)

Usage Model: Incremental Processing Within Protocol State Machine - Converts incrementally as APB transactions complete - Does NOT wait for complete wide beats - Interleaves conversion with protocol translation

State Machine: - Complex: IDLE, READ, WRITE states - Manages burst counters (r_burst_count) - Handles AXI burst to multiple APB transactions - FIRST/LAST packet generation - FIFO management for side channel data

Example Flow (128→32 write path):

1. Receive AXI AWADDR + WDATA[127:0] (full wide beat)
2. Enter WRITE state
3. APB transaction 0: Extract WDATA[31:0],   ptr=1, send to APB
4. APB transaction 1: Extract WDATA[63:32],  ptr=2, send to APB
5. APB transaction 2: Extract WDATA[95:64],  ptr=3, send to APB
6. APB transaction 3: Extract WDATA[127:96], ptr=0, complete AXI beat
7. If burst continues, repeat for next AXI beat

Critical Difference: Conversion happens within the protocol state machine, one APB transaction at a time.

## Refactoring Analysis

### Option 1: Keep Current Inline Implementation ✓ RECOMMENDED

Pros: - ✓ Already working and tested - ✓ Tight integration with protocol state machine - ✓ No additional latency - ✓ Optimal resource usage - ✓ Clear code flow for incremental conversion

Cons: - ⚠ Code duplication of conversion patterns - ⚠ Complex state machine mixing protocol + data concerns - ⚠ Harder to verify conversion logic independently

Verdict: Best choice. The tight coupling is actually necessary for the protocol conversion requirements.

### Option 2: Refactor to Use Generic Modules ✗ NOT RECOMMENDED

Conceptual Architecture:

AXI4 (DW-bit) → axi_data_dnsize → AXI4 (APBDW-bit) → axi4_to_apb_convert → APB (APBDW-bit)
                                      ↑
                              Width-matched AXI

Why This DOESN’T Work:

- Incremental Processing Conflict:

  - Generic modules process complete beats

  - APB converter processes incrementally with state machine

  - No clean insertion point for pipeline stages

- State Machine Coupling:

  - Conversion is tightly coupled with burst management

  - Pointer increments synchronized with APB transaction completions

  - FIRST/LAST generation depends on both protocol and conversion state

- Complexity Increase:

  - Would need complex glue logic to bridge generic modules to state machine

  - Additional FIFOs to buffer partial conversions

  - More difficult to reason about system behavior

- Resource Overhead:

  - Extra registers for generic module state

  - Additional control logic for coordination

  - Larger design for no functional benefit

- Verification Burden:

  - More complex interactions to verify

  - Harder to achieve coverage

  - Debugging becomes more difficult

Verdict: Refactoring would make the design more complex without providing any benefits.

## Validation: Independent Verification of Generic Module Algorithms

Finding: The APB converter independently arrived at identical data width conversion algorithms.

Validation Points:

- Slice Extraction (Dnsize):

  - APB converter line 334: r_s_axi_wdata[r_axi_wr_data_pointer*APBDW +: APBDW]

  - Generic dnsize line 210: r_data_buffer[r_beat_ptr*NARROW_WIDTH +: NARROW_WIDTH]

  - ✓ IDENTICAL pattern

- Accumulation (Upsize):

  - APB converter line 283: r_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] <= data

  - Generic upsize line 158: r_data_accumulator[r_beat_ptr*NARROW_WIDTH +: NARROW_WIDTH] <= data

  - ✓ IDENTICAL pattern

- Pointer Management:

  - Both use same increment and wrap logic

  - Both use $clog2(RATIO) for pointer width

  - Both wrap at ratio-1

  - ✓ IDENTICAL pattern

Significance: Two independent implementations converging on the same solution provides strong evidence that our generic module algorithms are correct and optimal.

## Code Sections Reference

### APB Converter Data Width Conversion Code

Parameters:

// Line 39
parameter int AXI2APBRATIO = DW / APBDW,
parameter int PTR_WIDTH = $clog2(AXI2APBRATIO),

State Variables:

// Line 168
logic [DW-1:0] r_axi_data_shift, w_axi_data_shift;

// Lines 171-173 - THREE separate pointers
logic [PTR_WIDTH-1:0] r_axi_rd_data_pointer, w_axi_rd_data_pointer;
logic [PTR_WIDTH-1:0] r_axi_wr_data_pointer, w_axi_wr_data_pointer;
logic [PTR_WIDTH-1:0] r_axi_rsp_data_pointer, w_axi_rsp_data_pointer;

WRITE Path (Dnsize):

// Line 334 - Extract data slice
w_apb_cmd_pkt_pwdata = (axi2abpratio == 1) ? r_s_axi_wdata[APBDW-1:0] :
                        r_s_axi_wdata[r_axi_wr_data_pointer*APBDW +: APBDW];

// Line 337 - Extract strobe slice
w_apb_cmd_pkt_pstrb = (axi2abpratio == 1) ? r_s_axi_wstrb[APBSW-1:0] :
                       r_s_axi_wstrb[r_axi_wr_data_pointer*APBSW +: APBSW];

// Lines 315-317 - Increment and wrap
r_axi_wr_data_pointer <= r_axi_wr_data_pointer + 1;
if (r_axi_wr_data_pointer == PTR_WIDTH'(axi2abpratio-1))
    r_axi_wr_data_pointer <= 'b0;

READ Response Path (Upsize):

// Line 283 - Accumulate into shift register
r_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] <= r_apb_rsp_pkt_prdata;

// Lines 284-287 - Increment and wrap
r_axi_rsp_data_pointer <= r_axi_rsp_data_pointer + 1'b1;
if (r_axi_rsp_data_pointer == PTR_WIDTH'(axi2abpratio-1)) begin
    r_axi_rsp_data_pointer <= 'b0;
end

// Line 366 - Combinational accumulation (alternative path)
w_axi_data_shift[r_axi_rsp_data_pointer*APBDW +: APBDW] = r_apb_rsp_pkt_prdata;

## Conclusion

### Summary

The APB converter implements data width conversion using patterns identical to our generic modules, but in a fundamentally different architectural context:

- Generic modules: Standalone streaming pipeline stages processing complete beats

- APB converter: Inline conversion within protocol state machine processing incrementally

### Recommendations

- Do NOT refactor APB converter to use generic modules

  - Current inline implementation is optimal for its use case

  - Refactoring would increase complexity without benefits

- Keep generic modules separate for their intended use cases:

  - Pure data width conversion in streaming pipelines

  - Protocol-agnostic width adaptation

  - Reusable components for future designs

- Document the relationship between the two implementations:

  - Both use same fundamental algorithms (validation)

  - Different usage models require different architectures

  - Not all width conversion scenarios suit pipeline approach

- Consider future opportunities:

  - Other protocol converters may benefit from generic modules

  - AXI-to-AXI width converters are prime candidates

  - Future designs should evaluate if pipeline model fits

### Value Delivered

- ✓ Validation - Independent implementation confirms algorithm correctness

- ✓ Clarity - Understand why inline vs. pipeline architectures differ

- ✓ Documentation - Captured design rationale for future reference

- ✓ Guidance - When to use generic modules vs. inline conversion

## Related Files

- /mnt/data/github/rtldesignsherpa/projects/components/converters/rtl/axi_data_upsize.sv

- /mnt/data/github/rtldesignsherpa/projects/components/converters/rtl/axi_data_dnsize.sv

- /mnt/data/github/rtldesignsherpa/projects/components/converters/rtl/axi4_to_apb_convert.sv

- /mnt/data/github/rtldesignsherpa/projects/components/converters/USAGE.md

Author: RTL Design Sherpa Date: 2025-10-25