RTL Design Sherpa

HIVE Micro-Architecture Specification 0.1

October 12, 2026

Table of Contents

1 Hive Mas Index	11

2 Document Information	12

2.1 HIVE Micro-Architecture Specification	12

2.2 Document Purpose	12

2.3 References	12

2.4 Terminology	13

2.5 Revision History	13

3 Design Philosophy and Machine Organization	15

3.1 Philosophy	15

3.2 Machine Organization	15

3.3 What Is Imported vs. What Is Built	15

4 Clocking and Reset	17

4.1 Clock Domains	17

4.2 CDC Structure	17

4.3 Reset Architecture	18

4.4 Reset Sequencing	19

4.5 Soft Reset	19

4.6 Clock Gating (Optional)	19

5 hive_top	20

5.1 Composition	20

5.2 Structural Rules	20

6 hive_c_controller (Block 1)	22

6.1 Submodules	22

6.2 Interfaces	23

6.3 Open Items	23

7 serv_monitor (Block 2)	24

7.1 Submodules	24

7.2 Interfaces	25

7.3 Open Items	25

8 control_network (Block 3)	26

8.1 Submodules	26

8.2 Performance Verification Hooks	26

9 config_manager (Block 4)	28

9.1 Submodules	28

9.2 Interfaces	29

10 Common Utilities	30

11 Internal Packages and Wiring	31

11.1 Package Organization	31

11.2 Key Internal Interfaces	31

11.3 Wiring Conventions	32

12 The Descriptor Injection Cycle, Step by Step	33

13 The Monitoring Cycle, Step by Step	34

13.1 Steady State (every serv_clk cycle)	34

13.2 Periodic Report	34

13.3 Congestion Alert (urgent path)	34

13.4 Error Path	34

14 The Context Switch Sequence, Cycle by Cycle	36

15 Reset and Boot Behavior	38

16 Verification Hooks and Observability	39

16.1 Observation Bus	39

16.2 Protocol Checkers (immediate assertions)	39

16.3 Formal Properties	39

16.4 Testbench Structure (cocotb, rds-dv framework)	40

17 Parameters, Defines, and Filelists	41

17.1 Hardware Parameters (includes/hive_config.svh)	41

17.2 Software Knobs (sw/common/hive_config.h)	41

17.3 Planned Filelist Layout	41

17.4 Scaling Knobs	42

18 HIVE Hardware Architecture Specification Index	43

18.1 Document Organization	43

18.1.1 Front Matter	43

18.1.2 Chapter 1: Introduction	43

18.1.3 Chapter 2: System Overview	43

18.1.4 Chapter 3: Subsystem Architecture	43

18.1.5 Chapter 4: Interfaces	43

18.1.6 Chapter 5: Programming Model	43

18.1.7 Chapter 6: Performance	44

18.1.8 Chapter 7: Integration and Verification	44

18.2 Related Documentation	44

19 Product Requirements Document (PRD)	45

19.1 HIVE - Hierarchical Intelligent Vector Environment	45

19.2 1. Executive Summary	45

19.2.1 1.1 Quick Stats	45

19.2.2 1.2 Subsystem Goals	45

19.3 2. Documentation Structure	45

19.3.1 HIVE Hardware Architecture Specification (HAS) — external contract	45

19.3.2 HIVE Micro-Architecture Specification (MAS) — implementation intent	46

19.3.3 Other Documentation	46

19.4 3. Architecture Overview	46

19.4.1 3.1 Top-Level Block Diagram	46

19.4.2 3.2 Resource Budget (NexysA7 100T)	47

19.5 4. Key Features	48

19.5.1 4.1 Hierarchical RISC-V Architecture	48

19.5.2 4.2 Distributed Monitoring	49

19.5.3 4.3 Network Reconfiguration	49

19.5.4 4.4 RAPIDS Integration	49

19.6 5. Interfaces	50

19.6.1 5.1 External Interfaces	50

19.6.2 5.2 Virtual Tile Mapping	50

19.7 6. Use Cases	50

19.7.1 6.1 Descriptor Scheduling	50

19.7.2 6.2 Congestion Management	51

19.7.3 6.3 Network Reconfiguration	51

19.8 7. Test Coverage	51

19.8.1 7.1 Current Status	51

19.9 8. Performance Targets	51

19.9.1 8.1 Latency Targets	51

19.9.2 8.2 Monitoring Overhead	52

19.10 9. Integration Guidelines	52

19.10.1 9.1 HIVE + RAPIDS + Delta Integration	52

19.11 10. Development Status	53

19.11.1 10.1 Current Phase	53

19.11.2 10.2 Roadmap	53

19.12 11. Educational Value	53

19.13 13. Documentation Generation	54

19.13.1 13.1 Generating PDF/DOCX from the Specifications	54

19.14 14. References	54

19.14.1 14.1 Internal Documentation	54

19.14.2 14.2 Related Subsystems	54

19.14.3 14.3 External References	54

19.15 Navigation	55

20 RAPIDS Beats Architecture MAS	56

20.1 Document Organization	56

20.1.1 Front Matter	56

20.1.2 Chapter 1: Overview	56

20.1.3 Chapter 2: FUB (Functional Unit Blocks)	56

20.1.4 Chapter 3: Macro (Integration Blocks)	56

20.1.5 Chapter 4: Interfaces	57

20.2 Quick Reference	57

20.2.1 FUB Modules (fub_beats/)	57

20.2.2 Macro Modules (macro_beats/)	58

20.3 Architecture Comparison: RAPIDS vs STREAM	59

20.4 Clock and Reset Summary	60

20.4.1 Clock Domains	60

20.4.2 Reset Signals	60

20.5 Interface Summary	61

20.5.1 External Interfaces	61

20.5.2 Internal Buses	61

20.6 Related Documentation	61

20.7 Specification Conventions	62

20.7.1 Signal Naming	62

20.7.2 Parameter Naming	62

20.7.3 Figure and Table Numbering	62

21 Delta Micro-Architecture Specification Index	63

21.1 Overview	63

21.1.1 Key Features	63

21.1.2 Companion Specifications	63

21.2 Navigation	64

21.2.1 Front Matter	64

21.2.2 Chapter 1: Introduction	64

21.2.3 Chapter 2: Block Descriptions	64

21.2.4 Chapter 3: Cycle-Level Behavior	64

21.2.5 Chapter 4: Generated RTL	64

21.2.6 Chapter 5: Verification	64

22 RISC-V Falcon Suite	65

22.1 The ladder	65

22.1.1 Adjacent: hive-serv	66

22.2 Goals per rung	66

22.3 Packaging (per rung, the MC-line treatment)	67

22.4 References and layout	68

List of Figures

No figures in this document.

List of Tables

Table 1: Reference Documents	12

Table 2: Terminology and Definitions	13

Table 3: Document Revision History	13

Table 4: Imported vs. built	15

Table 5: Clock domains	17

Table 6: Domain resets	18

Table 7: Subordinate instances	20

Table 8: Utility inventory	30

Table 9: Package layout	31

Table 10: Checker inventory	39

Table 11: FUB Module Summary	57

Table 12: Macro Module Summary	58

Table 13: RAPIDS Beats vs STREAM Comparison	59

Table 14: Clock Domains	60

Table 15: Reset Signals	60

Table 16: External Interfaces	61

Table 17: Internal Buses	61

# 1	Hive Mas Index

Generated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	HIVE Micro-Architecture Specification

Document Number: HIVE-MAS-001 Version: 0.1 Status: Draft (pre-RTL — implementation intent) Classification: Open Source - MIT License

## 2.2	Document Purpose

This Micro-Architecture Specification (MAS) defines the intended internal implementation of the HIVE subsystem at block level: the module hierarchy, the internal interfaces between blocks, the cycle-level behavior of the key operations, the verification hooks the RTL will expose, and the configuration surface (parameters, defines, filelists). It is written for the RTL implementer and for verification engineers building block-level testbenches.

The MAS assumes the external contracts of the companion HAS — packet formats, register map, interface classes — and does not restate them except where an internal design decision depends on the boundary detail.

Status note: no RTL exists yet. Everything in this document is implementation intent to seed RTL development; where a decision is still open, it is marked as an open item rather than resolved silently.

## 2.3	References

Reference Documents

| ID | Document | Description |
| --- | --- | --- |
| [REF-1] | HIVE HAS v0.1 (docs/hive_has/) | Hardware Architecture Specification (external contract) |
| [REF-2] | HIVE PRD (PRD.md) | Product requirements and roadmap |
| [REF-3] | VexRiscv documentation (https://github.com/SpinalHDL/VexRiscv) | HIVE-C core generator; plugin/config reference |
| [REF-4] | SERV documentation (https://github.com/olofk/serv) | Monitor core; RF/WB interfaces |
| [REF-5] | Delta MAS (../../../../../fabric-gen-ip/delta/docs/delta_mas/delta_mas_index.md) | Router/NI internals HIVE configures and monitors |
| [REF-6] | Repo CDC library (rtl/cdc/) | Async FIFO / synchronizer building blocks for the four clock domains |

## 2.4	Terminology

Terminology and Definitions

| Term | Definition |
| --- | --- |
| FUB | Functional Unit Block — a leaf RTL module |
| Context memory | BRAM holding the four routing contexts per tile |
| Injection FIFO | Per-source AXIS TX buffering ahead of the Delta ingress |
| Quiesce | Drain-and-hold state a router enters before a context switch |
| Tap | Read-only observation port from a router into its SERV monitor |
| Wrapper | SV module that instantiates an external core (VexRiscv/SERV) plus glue |

## 2.5	Revision History

Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 2026-10-12 | seang | Initial MAS release; migrates the implementation content of the retired v0.3 hive_specification.md into block-level implementation intent |

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Design Philosophy and Machine Organization

## 3.1	Philosophy

HIVE’s micro-architecture follows three rules, in priority order:

- Hierarchy mirrors control. Every block’s structure reflects the control hierarchy: one hive_c_controller at the top, sixteen identical serv_monitor instances below it, one control_network binding them, one config_manager orchestrating the fabric they all touch. A reader should be able to draw the HAS block diagram from the RTL tree alone.

- Reuse the repo’s fabric. Clock crossing uses rtl/cdc/ primitives; AXIS buffering uses the repo GAXI FIFO/skid library; arbitration uses the repo round-robin arbiter. HIVE’s own RTL is only what is genuinely new: descriptor generation, monitor glue, control framing, context switching.

- Observability is a feature, not a debug afterthought. Every block exposes the signals the Ocelus monitor system (see rtl/amba/ocelus/) and the cocotb testbenches need, on dedicated observation ports that are part of the module interface — never tapped ad hoc from internals.

## 3.2	Machine Organization

hive_top                              # top-level integration
├── hive_c_controller/                # Block 1
├── serv_monitor/        (16×)        # Block 2
├── control_network/                  # Block 3
├── config_manager/                   # Block 4
└── common/                           # shared utilities (or repo imports)

Block details are specified in Chapter 2. The four clock domains and their crossings are specified in the next section of this chapter; CDC structure constrains several block interfaces, so it comes first.

## 3.3	What Is Imported vs. What Is Built

Imported vs. built

| Item | Source | Why |
| --- | --- | --- |
| HIVE-C CPU | VexRiscv (SpinalHDL, generated) | RV32IM 5-stage core; not reinvented |
| SERV monitor CPU | SERV (olofk, RTL) | Bit-serial RV32I; smallest monitor that can run real trigger logic |
| AXIS async FIFOs | repo GAXI / rtl/common | Gray-pointer CDC, already verified |
| Reset synchronizers | rtl/cdc/ | Async-assert/sync-deassert, house style |
| Descriptors, taps, triggers, framing, contexts | HIVE RTL (new) | The actual product of this component |

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Clocking and Reset

## 4.1	Clock Domains

Clock domains

| Domain | Clock | Frequency | Components |
| --- | --- | --- | --- |
| 1 | hive_c_clk | 100 MHz (PoC), 150+ MHz (optimized) | VexRiscv core, I/D memories, descriptor generation, control-network master, CDA AXIS master |
| 2 | serv_clk | 50-100 MHz | All 16 SERV cores, traffic counters, trigger logic, injection FIFOs, control-network slaves |
| 3 | delta_clk | 100 MHz | Delta routers, NI, router configuration registers (Delta-owned; HIVE writes them) |
| 4 | host_clk | 50-100 MHz, external | UART, AXI4-Lite slave, command parser |

serv_clk is shared by all 16 monitors — one domain, sixteen instances. delta_clk may be phase-aligned with hive_c_clk in the PoC but must not be assumed synchronous; all crossings use the async FIFOs below regardless.

## 4.2	CDC Structure

Every crossing is a repo-library dual-clock FIFO or synchronizer; there are no combinational crossings.

hive_c_clk → delta_clk    CDA descriptors:      256-bit × 8 entries
delta_clk → hive_c_clk    PKT_STATUS return:    128-bit × 16 entries (aggregates all SERVs)
serv_clk → delta_clk      Per-SERV injection:   128-bit × 4 entries × 16
delta_clk → serv_clk      Per-SERV config in:   128-bit × 4 entries × 16
host_clk → hive_c_clk     AXIL data:            32-bit × 8 entries; control: dual-flop sync
hive_c_clk → serv_clk     Control commands:     32-bit × 2 entries per SERV (low bandwidth)
serv_clk → hive_c_clk     Status uplink:        32-bit × 4 entries per SERV, round-robin arbitrated

: CDC inventory

FIFO depths follow the house rule: ≥ 4 entries minimum to bound full/empty oscillation; deeper where aggregation justifies it (status return holds a full round of 16 SERV reports).

## 4.3	Reset Architecture

Global reset (system_rst_n): active-low, asynchronous assertion, synchronous deassertion to each domain. Minimum assertion: 16 cycles of the slowest clock (serv_clk).

Domain resets, each synchronized from system_rst_n:

Domain resets

| Reset | Domain | Scope | Notes |
| --- | --- | --- | --- |
| hive_c_rst_n | hive_c_clk | VexRiscv, memories, control master | Memory init required after reset |
| serv_rst_n[15:0] | serv_clk | Individual monitors | Independently controllable: debug, recovery, reconfig |
| delta_rst_n | delta_clk | Routers, NI, config logic | Must drain in-flight packets first (asserted by config manager, not the host) |
| host_rst_n | host_clk | UART, AXIL slave, parser | Async to HIVE-C; commands require handshake |

Reset synchronizer (all domains, house pattern — async assert, 2-stage sync release):

module reset_synchronizer (
    input  logic clk,
    input  logic async_rst_n,
    output logic sync_rst_n
);
    logic [1:0] sync_chain;
    always_ff @(posedge clk or negedge async_rst_n) begin
        if (!async_rst_n) sync_chain <= 2'b00;
        else              sync_chain <= {sync_chain[0], 1'b1};
    end
    assign sync_rst_n = sync_chain[1];
endmodule

## 4.4	Reset Sequencing

Assertion: system_rst_n low → all domain resets assert asynchronously → FSMs to IDLE/RESET → FIFO pointers clear → config registers to reset values.

Deassertion (staged): host interface → HIVE-C → Delta Network → SERV monitors (last). After deassert, HIVE-C loads firmware, Delta enters Context 0 (systolic), SERVs enter monitoring mode; system-ready after ~100 cycles of the slowest clock. See Chapter 4.4 for the boot-time behavior description.

## 4.5	Soft Reset

The soft-reset register (HAS §4.3) maps to serv_rst_n[15:0], delta_rst_n, control-network reset, and hive_c_rst_n respectively. Soft resets are synchronous to the target domain and re-use the same staged sequencing; a HIVE-C self-reset (bit 0) is legal but leaves the network unsupervised until firmware re-initializes — firmware sets a watchdog policy for it.

## 4.6	Clock Gating (Optional)

- SERV gating: gate serv_clk per monitor when idle; ~5% system power; 1-2 cycle wake-up. Not in the PoC bring-up plan.

- Router gating: per Delta’s own power plan (Delta MAS); HIVE only asserts tile-disable via PKT_CONFIG.

- HIVE-C gating: rejected — must stay responsive to interrupts.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	hive_top

Top-level integration of the four blocks. hive_top owns the external port list (HAS Chapter 4), the parameter surface (Chapter 6 of this book), and the instantiation of everything below. It contains no logic of its own beyond parameter propagation and the reset staging tree.

## 5.1	Composition

Subordinate instances

| Instance | Module | Count | Role |
| --- | --- | --- | --- |
| u_hive_c | hive_c_controller | 1 | Master controller (Block 1) |
| u_serv[k] | serv_monitor | SERV_COUNT (16) | Per-tile monitors (Block 2) |
| u_ctrl_net | control_network | 1 | HIVE-C ↔ SERV star (Block 3) |
| u_cfg_mgr | config_manager | 1 | Contexts and switching (Block 4) |

## 5.2	Structural Rules

- hive_top does not create clocks; it takes the four domain clocks and system_rst_n from the system top (PLL/BUFG decisions live there).

- Every CDC FIFO listed in §1.2 is instantiated in the sending block and documented in that block’s interface section; hive_top only routes the clock/reset pairs to them.

- The Delta-facing AXIS ports of hive_c_controller and each serv_monitor cross into delta_clk inside the sender, so hive_top presents only delta_clk-domain AXIS toward Delta — Delta never sees a foreign clock.

- All monitor observation ports (Ocelus tap-outs, per Chapter 5) are exported up through hive_top as a flat observability bus so system integration can attach the monitor system without touching block internals.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	hive_c_controller (Block 1)

The master controller: VexRiscv “Small” (RV32IM) plus the glue that turns firmware decisions into CDA packets, control-network frames, and register access. Clock domain: hive_c_clk.

hive_c_controller
├── vexriscv_wrapper.sv      # VexRiscv instantiation + debug/irq plumbing
├── instruction_mem.sv       # 32 KB tightly-coupled I-mem (BRAM)
├── data_mem.sv              # 32 KB tightly-coupled D-mem (BRAM)
├── descriptor_gen.sv        # CDA descriptor assembly + injection queue
├── axis4_master.sv          # AXIS TX (CDA) into delta_clk CDC FIFO
├── ctrl_net_master.sv       # Control-network command serializer
├── axil_master.sv           # AXI4-Lite master window (RAPIDS/Delta/SERV cfg)
└── host_if.sv               # UART / AXI4-Lite slave command interface

: HIVE-C module tree

## 6.1	Submodules

vexriscv_wrapper. Instantiates the generated VexRiscv core with the “Small” configuration. Exposes JTAG or UART-based firmware load (open item: loader transport — the falcon suite’s kestrel loader pattern is the preferred precedent), machine timer for the statistics interval, and the interrupt aggregation from interrupt.c’s hardware sources (RAPIDS completion, control-network alert, host command).

instruction_mem / data_mem. 32 KB each, single-cycle tightly-coupled BRAM. Dual-ported where the loader needs a debug port; the core port always wins arbitration.

descriptor_gen. Turns a firmware-written descriptor record (written into a small command mailbox in data-memory space) into a 256-bit CDA beat: field assembly per HAS §4.2, injection-FIFO push, and backpressure hold on tready. Holds up to MAX_INFLIGHT_DESCRIPTORS outstanding descriptors; full is a firmware-visible stall flag, not an error.

axis4_master. Drives the CDA AXIS interface into the 256×8 async FIFO that crosses to delta_clk. Asserts tuser=2'b01, tdest=16, tlast every beat. Single outstanding packet at a time (one descriptor = one beat).

ctrl_net_master. Serializes control-network commands from firmware: command + target mask + payload into 32-bit frames, multicast capable for broadcasts (CONFIG_PREPARE, SET_ROUTING_MODE, threshold updates).

axil_master. The 0x4000_0000 register window (HAS §4.3). Address decode for the RAPIDS, Delta, SERV-status, and config regions; a lightweight request/response queue so firmware never stalls on AXIL handshake.

host_if. UART and/or AXI4-Lite slave command channel (synthesis option). Normalizes both to the same internal command FIFO consumed by firmware.

## 6.2	Interfaces

External: host interface, CDA AXIS master (into CDC), control-network master, AXIL master. Internal: memory buses (core-local), mailbox from firmware view, interrupt lines in.

## 6.3	Open Items

- Firmware load transport (JTAG vs. UART vs. preloaded BRAM image) — decision deferred to Phase 1; kestrel loader precedent preferred.

- Whether the descriptor mailbox lives in data-mem address space or a dedicated register file — implementation choice pending area numbers.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	serv_monitor (Block 2)

One per Delta tile; sixteen instances. The SERV core runs the trigger logic in firmware; the surrounding glue provides the counters, taps, and injection path the core alone cannot. Clock domain: serv_clk (all instances share the domain, never communicate directly).

serv_monitor
├── serv_wrapper.sv        # SERV core instantiation + bus glue
├── local_mem.sv           # 2 KB I-mem + 2 KB D-mem
├── traffic_counter.sv     # Packet/cycle counters from the router tap
├── trigger_logic.sv       # Threshold compare + event detect (independent of core)
├── axis_injector.sv       # AXIS TX (PKT_CONFIG / PKT_STATUS) into CDC FIFO
└── ctrl_net_slave.sv      # Control-network slave + mailbox to core

: SERV monitor module tree

## 7.1	Submodules

serv_wrapper. Instantiates the SERV core. Exposes a read-only tile-ID register at core address 0xF000_0000 (firmware reads its own identity) and routes the data bus to local memory, the injection FIFO aperture, the counter bank, and the control mailbox.

traffic_counter. The counter bank behind the router tap: pkt_rx_count[4], pkt_tx_count[4], pkt_local_inject, pkt_local_consume, buffer_overflow_count, stall_cycles — 32-bit wrapping, saturating-clear on read when Reset Statistics Counters is issued. buffer_occupancy[4], congestion_flag, and error_flags are wired combinationally from the tap for low-latency trigger evaluation.

trigger_logic. Hardware threshold comparator running in parallel with the core: flags occupancy > cfg_threshold (default 75% of buffer depth) and error conditions immediately, so congestion detection meets the < 10 cycle target even when the bit-serial core is mid-instruction. The core consumes trigger flags as interrupts/polled events and decides the packet response; trigger_logic never injects packets by itself (firmware owns all packet content — this split keeps policy in software, detection in hardware).

axis_injector. TX FIFO aperture (128-bit × 4 entries) plus the AXIS master into the per-SERV serv_clk → delta_clk CDC FIFO. Firmware writes words, then commits with TVALID; the injector appends tid, tuser, tdest, tlast per the commit record.

ctrl_net_slave. Receives 32-bit control frames addressed to this monitor (HAS Chapter 4 command set), updates threshold/config registers, raises core interrupts, and queues uplink status frames for the control network’s round-robin arbiter.

## 7.2	Interfaces

Router tap in (from the tile router, delta_clk — synchronized at the tap boundary inside traffic_counter; the tap is observation-only), control-network slave port, AXIS TX into CDC, observation port out.

## 7.3	Open Items

- Tap synchronization: per-direction dual-flop sync vs. async capture — to be settled with Delta’s tap definition (Delta MAS).

- Whether trigger_logic also owns the periodic-report timer or the core does (core timer saves logic, hardware timer saves instruction budget — Phase 1 experiment).

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	control_network (Block 3)

The star between HIVE-C and the sixteen monitors. Low bandwidth, hard latency and fairness requirements: commands in < 10 cycles, uplink aggregation across all 16 < 20 cycles, no slave starves. The block spans two clock domains — master side in hive_c_clk, slave side in serv_clk — with the CDC FIFOs below as the only crossing.

control_network
├── control_packetizer.sv   # Command/status frame encode + decode
├── control_router.sv       # Star fan-out: address decode, multicast
├── control_arbiter.sv      # Round-robin uplink arbitration (16 → 1)
└── cdc_fifo_* (repo lib)   # Per-direction async FIFOs (see §1.2 depths)

: Control network module tree

## 8.1	Submodules

control_packetizer. Frames are 32-bit (HAS §4.1 contract):

Bits [31:24]: Command / status opcode
Bits [23:16]: Target SERV id (command) or source SERV id (status);
              8'hFF = broadcast (commands only)
Bits [15:0]:  Payload (threshold values, context id, error code, ...)

Command and status share the opcode space with direction implied by the port. Multicast is mask-free: broadcast reaches all slaves; selective multicast is done by issuing multiple unicast frames.

control_router. Pure fan-out — one master port to sixteen slave ports, address-decoded, no buffering beyond the CDC FIFOs. In-order per slave; commands to different slaves may be in flight simultaneously.

control_arbiter. Round-robin over the sixteen uplink FIFOs, non-preemptive per frame, urgent flag honored as next-grant priority (an alert frame marked urgent wins the grant after the current frame). Fairness is structural: equal weights, one frame per grant; starvation is impossible by construction and asserted formally.

## 8.2	Performance Verification Hooks

Grant sequence, per-slave service interval, and FIFO occupancy are exported to the observation bus; the < 20-cycle all-16 aggregation target is checked by a cocotb scoreboard counting grant cycles across a full round.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	config_manager (Block 4)

Owns the four virtual routing contexts and the atomic-switch orchestration. Lives in hive_c_clk with a delta_clk register interface; the PKT_CONFIG broadcast path carries context changes to the routers.

config_manager
├── config_registers.sv     # Mode select, status, soft-reset, error sticky
├── context_memory.sv       # 4 contexts × per-tile routing tables (BRAM)
└── switch_controller.sv    # PREPARE / SET_ROUTING_MODE / ACTIVATE FSM

: Config manager module tree

## 9.1	Submodules

context_memory. Four contexts × 16 tiles × routing-table entry (BRAM; 2 BRAM blocks in the budget). Contexts 0-2 hold hardwired-mode metadata only (the routing functions are hardwired in Delta’s routers); Context 3 holds full per-tile tables via ROUTE_TABLE_n writes. Reset default: Context 0 loaded/selected.

config_registers. The HIVE-side configuration space (HAS §4.3): active context, per-context valid bits, switch-status RO bits, error sticky flags, soft-reset outputs. Exposed to HIVE-C firmware over the AXIL window and to the observation bus.

switch_controller. The three-phase FSM (HAS §5.4):

IDLE ──CONFIG_PREPARE──► DRAIN ──quiesce_confirmed──► SWITCH
                                                        │ SET_ROUTING_MODE broadcast
                                                        ▼
                                              ACTIVE ◄── CONFIG_ACTIVATE

- DRAIN: broadcasts prepare, then polls router STATUS registers (or receives SERV tap confirmation) until all participating routers report active_packets == 0. Timeout → error sticky, abort.

- SWITCH: emits one PKT_CONFIG broadcast (command 0x0000_0001, payload = context id, tile mask = all). The broadcast is a single packet consumed by all routers in the same cycle — that simultaneity is what makes the switch atomic.

- Hardware interlock (open item): the HAS requires firmware to verify quiesce before SET_ROUTING_MODE; a hardware interlock in switch_controller that refuses the mode update until active_packets == 0 everywhere is the preferred hardening and should be added before Phase 3 bring-up.

## 9.2	Interfaces

AXIL register window (from hive_c_controller), PKT_CONFIG injection request to HIVE-C’s CDA/config injection path (the manager does not own an AXIS master; it borrows the HIVE-C injection path or a SERV flood — implementation decision in Phase 2), observation bus out.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Common Utilities

Shared helpers used by more than one block. Rule: a utility lives in HIVE’s common/ only when the repo library has no equivalent; otherwise it is imported, not copied.

Utility inventory

| Module | Source | Used by | Notes |
| --- | --- | --- | --- |
| gaxi_fifo_async / AXIS async FIFO | repo GAXI library (rtl/common) | All CDC paths | Gray-pointer, house-verified; depths per §1.2 |
| reset_synchronizer | rtl/cdc/ | Every domain | Pattern in §1.3 |
| round_robin_arbiter | repo common | control_network | Generic N-way; N=16 for the uplink |
| credit_counter | repo common (Delta-shared semantics) | Injection path | Credit accounting if Delta ingress requires it (confirm against Delta MAS) |
| axis_fifo (sync) | repo GAXI | Injection FIFOs | serv_monitor TX aperture |

Parameter consistency: SERV_COUNT, buffer depths, and FIFO widths come from the single hive_config.svh include (Chapter 6); no block hardcodes its own copy.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Internal Packages and Wiring

## 11.1	Package Organization

One package per block plus a top-level config include; no cross-imports from leaf packages upward:

Package layout

| File | Contents |
| --- | --- |
| includes/hive_config.svh | All parameter defaults (Chapter 6); single source of truth |
| includes/hive_ctrl_net_pkg.sv | Control-network opcodes, frame struct, urgent-flag encoding |
| includes/hive_desc_pkg.sv | CDA descriptor struct, field masks, PKT_* encodings |
| includes/hive_cfg_pkg.sv | Context ids, config register offsets, error codes |
| includes/hive_obs_pkg.sv | Observation-bus record layout (Chapter 5) |

Structs (not raw bit vectors) cross block boundaries for every interface wider than a few signals — descriptor records, control frames, observation records — so port lists stay readable and compile-time checked.

## 11.2	Key Internal Interfaces

Descriptor mailbox → descriptor_gen (in hive_c_controller): firmware writes a descriptor_t record; descriptor_gen pulses done on handshake and raises stall while the injection queue is full.

Control network frame (§2.4): the 32-bit frame is the only master-to-slave and slave-to-master payload; widening to 64-bit is an open area item if the statistics payload overflows 16 bits regularly.

Config-manager injection request: switch_controller raises inject_req with a completed PKT_CONFIG record; the HIVE-C injection path (or SERV flood, Phase-2 decision) carries it into Delta.

Observation bus (Chapter 5): a flat, hive_c_clk-domain record stream exported through hive_top; each block drives its slice; slices are concatenated, not arbitrated (fixed layout in hive_obs_pkg).

## 11.3	Wiring Conventions

- AXIS signals are named axis_<role>_<signal> with role ∈ {cda, stat, cfg, inj}; AXI4-Lite as axil_<signal>.

- Resets are *_rst_n, active low, synchronized per §1.3; no block generates its own reset from system_rst_n directly.

- Every inter-block interface documents its clock domain at the port; a port pair in different domains must name the CDC element in the block header comment.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	The Descriptor Injection Cycle, Step by Step

The defining HIVE-C operation: one descriptor from firmware decision to RAPIDS FIFO. Target: < 30 cycles hive_c_clk → RAPIDS accept.

Cycle  Action
  1    Firmware completes a descriptor_t write to the mailbox; doorbell pulse.
  2    descriptor_gen pops the record; field assembly (HAS §4.2); tuser/tdest/last fixed.
  3    Record pushed to injection queue; queue occupancy +1 (observation).
 4-6   Injection queue at head; CDC FIFO (hive_c_clk side) accepts; tvalid/tready handshake.
 7-10  Gray-pointer cross to delta_clk side; FIFO output registered.
11-20  Delta traversal toward virtual tile 16 (router hops; ~2/hop, edge-attached).
21-24  RAPIDS ingress handshake; descriptor written to the descriptor FIFO.
       RAPIDS descriptor engine acks (tready) — injection latency target met.

Ordering: descriptors leave descriptor_gen in mailbox order; the CDC FIFO and Delta preserve that order per stream, which is the ordering property the formal inventory asserts (descriptor FIFO ordering, HAS §7).

Backpressure: if RAPIDS’ descriptor FIFO fills, tready deasserts and the CDC FIFO absorbs the stall; firmware sees stall only when the injection queue itself fills (MAX_INFLIGHT_DESCRIPTORS outstanding) — a policy signal to the scheduler, never a protocol error.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	The Monitoring Cycle, Step by Step

One SERV monitor’s steady-state loop and its alert path.

## 13.1	Steady State (every serv_clk cycle)

- traffic_counter samples the router tap: per-direction packet strobes increment pkt_rx/tx_count; occupancy strobes update buffer_occupancy.

- trigger_logic compares occupancy against cfg_threshold; the congestion line is combinationally current with the tap (this is what meets the < 10-cycle detection target — no firmware round trip).

- The SERV core polls (or is interrupted by) trigger_logic and the counter bank; on the periodic-report interval it snapshots metrics into the uplink frame.

## 13.2	Periodic Report

- Core writes a status record into ctrl_net_slave’s uplink mailbox.

- control_arbiter grants this slave’s turn (round-robin; bounded wait ≤ 16 frames).

- Frame crosses serv_clk → hive_c_clk (4-entry FIFO), is consumed by ctrl_net_master, and lands in the SERV-status AXIL window for firmware.

## 13.3	Congestion Alert (urgent path)

occupancy > threshold        (combinational, tap-current)
  → trigger_logic flags core (≤ 1 cycle)
  → core assembles PKT_STATUS 0x04 (alert) — or forwards a hardware
    pre-latched snapshot if configured (≤ 5 cycles to tvalid)
  → axis_injector commits: tuser=2'b11, tid = tile id
  → CDC into delta_clk; routed to HIVE-C (tdest=17) or control network
    urgent slot, per runtime configuration

## 13.4	Error Path

Parity/protocol-violation strobes set sticky error_flags and interrupt the core immediately; the core emits status type 0x08 and, per policy, may assert the local injection-halt line (its own aperture only — it cannot silence the compute element, which is Delta’s path).

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	The Context Switch Sequence, Cycle by Cycle

The config_manager FSM (§2.5) driven by HIVE-C firmware. Typical budget: 10-20 cycles; never more than 25.

Cycle  Action
  1    switch_controller receives CONFIG_PREPARE (AXIL write from firmware).
  2    Prepare broadcast issued on the control network; SERVs stop local
       injection and (if in data path) assert quiesce request to routers.
 3-8   Drain: routers flush in-flight packets (5-10 cycles). switch_controller
       polls STATUS.active_packets for all participating tiles.
  9    All-zero confirmed (or hardware interlock satisfied — see §2.5 open item).
 10    SET_ROUTING_MODE PKT_CONFIG assembled (payload = context id,
       tile mask = all); injected on the HIVE-C config path.
 11    Broadcast consumed by every router in the same cycle (atomic point:
       MODE_SELECT registers update simultaneously).
 12    config_registers.switch_status = ACTIVE_CONTEXT.
 13    CONFIG_ACTIVATE broadcast; routers resume under the new routing.
14-20  Margin for slower drain; beyond 25 cycles the FSM raises
       switch-timeout sticky error and requires firmware rollback (HAS §5.4).

Why the atomic point works: every router updates MODE_SELECT on the same broadcast beat. There is no skew window in which different routers hold different contexts — the switch is a single point in time (cycle 11 above), not an interval. This is the property the formal inventory asserts as “context switch atomicity.”

Quiesce verification: the FSM polls active_packets == 0 per router; with the §2.5 hardware interlock installed, SET_ROUTING_MODE is refused by hardware until the condition holds, making the protocol safe even against buggy firmware.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Reset and Boot Behavior

From system_rst_n release to a supervising HIVE system.

Phase  Action
  A    Staged deassertion (§1.3): host → HIVE-C → Delta → SERVs.
  B    HIVE-C fetches first instructions; firmware init: AXIL windows,
       interrupt enables, injection queue clear.
  C    Firmware loads Context 0 default (systolic) into the config space;
       config_manager reports ACTIVE_CONTEXT = 0.
  D    SERVs release, run monitor firmware init, begin tap sampling and
       periodic reporting.
  E    System-ready after ~100 cycles of the slowest clock (serv_clk).
       host_if accepts commands; CDA injection enabled.

Boot invariants: - No CDA packet may enter Delta before phase C completes (injection queue held in reset-clear until firmware enables it — a formal-friendly one-bit condition). - Routers must be in Context 0 from first ingress; a router that sees a packet while MODE_SELECT is undefined raises ERROR_FLAGS (protocol violation path, HAS §4.2). - SERVs that have not finished init must not block the control network uplink (their arbiter slot is granted and frames are simply empty/NAK — fairness holds even during boot).

Soft-reset behavior mirrors the staged sequence for the affected domain only; a SERV soft reset is invisible to Delta traffic (its tap is passive), and a Delta soft reset drains through the config_manager quiesce path rather than raw reset assertion wherever in-flight packets exist.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Verification Hooks and Observability

## 16.1	Observation Bus

Every block drives a fixed slice of a flat observation record (hive_obs_pkg), exported through hive_top in hive_c_clk domain. Slices include: injection queue occupancy/stall, descriptor_gen doorbell events, control-network grant sequence and per-slave service intervals, trigger_logic raw lines, switch_controller FSM state, and CDC FIFO full/empty flags. The repo’s Ocelus monitor system attaches to this bus for protocol and coverage collection in system simulations — no internal signals are probed ad hoc.

## 16.2	Protocol Checkers (immediate assertions)

Checker inventory

| Check | Where |
| --- | --- |
| tuser/tdest legal combinations (HAS §2 validation rules) | Delta ingress of each HIVE stream |
| No CDA before boot phase C | descriptor_gen output |
| Descriptor order preserved | CDC FIFO output vs. enqueue |
| Broadcast simultaneity (same cycle all routers) | switch_controller observation |
| TID integrity (tid == tile_id on CE PKT_DATA) | Delta NI (owned by Delta; HIVE TB asserts it) |
| Quiesce-before-switch | switch_controller (with interlock) |

## 16.3	Formal Properties

The HAS §7 formal inventory maps to properties owned here:

- Router deadlock freedom — Delta-owned; HIVE testbenches assume it.

- Descriptor FIFO ordering — asserted on the injection queue + CDC FIFO chain.

- Context switch atomicity — no packet observation between SET_ROUTING_MODE and CONFIG_ACTIVATE on any router tap.

- TID integrity — cross-asserted with Delta NI.

- Quiesce completeness — active_packets == 0 on all participating routers before the atomic point.

## 16.4	Testbench Structure (cocotb, rds-dv framework)

- BFMs: AXIS masters/slaves from cocotb-framework for the Delta-facing streams; AXIL BFM for the register window; a lightweight control-network frame BFM for the star.

- Reference: firmware-level golden models for descriptor generation and scheduling; spike is not applicable (no ISA verification — the cores are imported and verified by their own suites).

- Levels: unit per FUB (HAS §7 example), integration per block pair, system per HAS Chapter 7. Coverage targets: 100% line, 95%+ branch; functional coverage points derive from this book’s behavior chapters (one covergroup per §4 sequence’s decision points).

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Parameters, Defines, and Filelists

## 17.1	Hardware Parameters (includes/hive_config.svh)

// System shape
parameter int MESH_SIZE = 4;           // NxN mesh (SERV_COUNT must equal MESH_SIZE^2)
parameter int SERV_COUNT = 16;         // number of SERV monitors
// Delta-shared fabric parameters (must match Delta's configuration)
parameter int ROUTER_BUFFER_DEPTH = 8; // flits per input VC
parameter int NUM_VIRTUAL_CHANNELS = 2;
parameter int FLIT_WIDTH = 128;        // bits
// Configuration manager
parameter int NUM_CONTEXTS = 4;        // routing configurations
// Monitoring
parameter int SERV_MEM_SIZE = 2048;    // bytes per SERV (I + D)
parameter int CONGESTION_THRESHOLD = 6;// trigger at 75% of buffer depth
parameter int STATS_REPORT_INTERVAL = 1000; // cycles
// Performance knobs
parameter bit ENABLE_PIPELINE_BYPASS = 1;
parameter bit ENABLE_ADAPTIVE_ROUTING = 0;  // future: load balancing

: Hardware parameter surface

Rules: SERV_COUNT == MESH_SIZE**2 is a static assertion in hive_top. Delta-shared parameters must be passed down identically to the Delta instantiation (one config include at system top, not two).

## 17.2	Software Knobs (sw/common/hive_config.h)

#define MAX_INFLIGHT_DESCRIPTORS  8
#define PRIORITY_LEVELS           4
#define AGING_THRESHOLD           1000  // cycles
#define DESCRIPTOR_BATCH_SIZE     4
#define STATS_AGGREGATION_PERIOD  10000 // cycles

## 17.3	Planned Filelist Layout

hive-serv/
├── rtl/
│   ├── includes/            # hive_config.svh + packages (above)
│   ├── top/hive_top.sv
│   ├── hive_c_controller/
│   ├── serv_monitor/
│   ├── control_network/
│   ├── config_manager/
│   └── common/
├── dv/
│   ├── testplans/           # arrive with RTL (kestrel testplan convention)
│   └── tests/               # cocotb, one directory per FUB + integration
├── models/                  # SimPy performance models (HAS ch.6 targets)
└── sw/
    ├── hive_c_fw/           # main.c, rapids_mgr.c, ... (HAS ch.5)
    └── serv_fw/             # monitor image (hand-optimized assembly)

: Planned repository layout

## 17.4	Scaling Knobs

MESH_SIZE/SERV_COUNT scale the PoC (4×4) toward the Kintex-7 8×8 expansion (HAS §7): RTL is generate-based over SERV_COUNT, and the control-network arbiter is a generic N-way. NUM_CONTEXTS grows to 8 in the expansion; context_memory sizing derives from it.

Last Updated: 2026-10-12

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	HIVE Hardware Architecture Specification Index

Hierarchical Intelligent Vector Environment

Version: 0.1 Date: 2026-10-12 Purpose: System-level hardware architecture specification for the HIVE distributed control and monitoring subsystem (1 VexRiscv master + 16 SERV monitors) coordinating RAPIDS and Delta.

## 18.1	Document Organization

Note: All chapters linked below for automated document generation.

### 18.1.1	Front Matter

- Document Information

### 18.1.2	Chapter 1: Introduction

- Purpose and Scope

### 18.1.3	Chapter 2: System Overview

- System Overview and Context

### 18.1.4	Chapter 3: Subsystem Architecture

- Subsystem Architecture - the four blocks, inter-block communication, resource budget

### 18.1.5	Chapter 4: Interfaces

- Top-Level Interfaces

- Packet Formats and Routing Rules

- Memory Map and Register Spaces

### 18.1.6	Chapter 5: Programming Model

- HIVE-C Firmware Architecture

- Descriptor Scheduling

- SERV Monitor Programming

- Network Reconfiguration Workflow

### 18.1.7	Chapter 6: Performance

- Performance Characteristics

### 18.1.8	Chapter 7: Integration and Verification

- Integration and Verification

## 18.2	Related Documentation

- HIVE MAS - Micro-Architecture Specification (block-level implementation intent)

- HIVE PRD - Product requirements and use cases

- HIVE CLAUDE - AI assistance guide for this subsystem

- RAPIDS Beats HAS - the DMA engine HIVE-C controls

- Delta HAS - the network fabric HIVE configures

- RISC-V Falcon Suite README - suite overview and hive-serv placement beside the ladder

Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Product Requirements Document (PRD)

## 19.1	HIVE - Hierarchical Intelligent Vector Environment

Version: 0.2 Date: 2026-10-12 Status: Specification phase — HAS/MAS v0.1 published; no RTL yet Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 19.2	1. Executive Summary

The Hierarchical Intelligent Vector Environment (HIVE) is a distributed control and monitoring subsystem that coordinates the RAPIDS DMA engine and Delta Network. It demonstrates hierarchical RISC-V processor architecture, distributed monitoring, and dynamic network reconfiguration.

### 19.2.1	1.1 Quick Stats

- Architecture: 4 major blocks (HIVE-C, SERV monitors, Control network, Config manager)

- Processors: 1 VexRiscv (master) + 16 SERV (monitors) = 17 RISC-V cores

- Interfaces: Control Network (internal), AXIS (to Delta Network)

- Resource Budget: 6,100 LUTs HIVE control plane (14,100 including the Delta NoC), 30 BRAM on NexysA7 100T

- Status: Specification phase; HAS/MAS v0.1 published 2026-10-12

### 19.2.2	1.2 Subsystem Goals

- Primary: Demonstrate hierarchical control system design patterns

- Secondary: Enable dynamic network reconfiguration and distributed monitoring

- Tertiary: Educational reference for RISC-V integration and NoC control

## 19.3	2. Documentation Structure

This PRD provides a high-level overview. Detailed specifications are maintained separately:

### 19.3.1	HIVE Hardware Architecture Specification (HAS) — external contract

Location: projects/components/riscv-ip/hive-serv/docs/hive_has/

- Index - HAS structure

- System overview, subsystem architecture (4 blocks), interfaces & packet formats, programming model, performance targets, integration & verification

### 19.3.2	HIVE Micro-Architecture Specification (MAS) — implementation intent

Location: projects/components/riscv-ip/hive-serv/docs/hive_mas/

- Index - MAS structure

- Block-level module trees, clocking/reset, internal interfaces, cycle-level behavior of the key operations, verification hooks, parameters/filelists

### 19.3.3	Other Documentation

- README - Quick start and integration guide (to be created)

- CLAUDE - AI assistance guide for this subsystem

- Tasks - none tracked. The vault area was deleted 2026-09-27: hive has no .sv, no tests and no filelists, so its 24 open items described work that had not begun. They are in git history at vault/Tasks/projects/components/riscv-ip/hive-serv/.

History note: the earlier docs/hive_spec/ + docs/hive_specification.md drafts (v0.3) were migrated into the HAS/MAS and retired 2026-10-12; they remain in git history.

## 19.4	3. Architecture Overview

### 19.4.1	3.1 Top-Level Block Diagram

HIVE System Architecture
┌─────────────────────────────────────────────────────────────┐
│                     HIVE Control Plane                      │
│  ┌────────────────────────────────────────────────────┐    │
│  │   Block 1: HIVE-C Master Controller (VexRiscv)    │    │
│  │   - Global coordination                            │    │
│  │   - Descriptor scheduling                          │    │
│  │   - Performance aggregation                        │    │
│  └──────────────────┬─────────────────────────────────┘    │
│                     │ Block 3: Control Network              │
│          ┌──────────┴──────────┬──────────────┐           │
│          ▼          ▼          ▼              ▼           │
│  ┌──────────┐ ┌──────────┐ ... ┌──────────┐ (16×)        │
│  │ Block 2: │ │ Block 2: │     │ Block 2: │              │
│  │ SERV-0   │ │ SERV-1   │     │ SERV-15  │              │
│  │ Monitor  │ │ Monitor  │     │ Monitor  │              │
│  └────┬─────┘ └────┬─────┘     └────┬─────┘              │
│       │            │                 │                     │
│       │  Block 4: Configuration Manager                    │
│       │  - Virtual context storage                         │
│       │  - Topology switching                              │
│       └────────────┴──────────────────┘                    │
└─────────────────────────────────────────────────────────────┘
         │ AXIS Packets (CDA, PKT_CONFIG, PKT_STATUS)
         ▼
    Delta Network (4×4 Mesh) + RAPIDS DMA Engine

See: docs/hive_has/ch02_system_overview/01_system_overview.md and docs/hive_has/ch03_architecture/01_subsystem_architecture.md for the architecture; docs/hive_mas/ for block internals.

### 19.4.2	3.2 Resource Budget (NexysA7 100T)

FPGA Target: Xilinx Artix-7 XC7A100T-1CSG324C

| Component | LUTs | FFs | DSPs | BRAM (36Kb) | Notes |
| --- | --- | --- | --- | --- | --- |
| Block 1: HIVE-C Controller | 1,400 | 900 | 0 | 8 | VexRiscv “Small” config |
| Block 2: SERV Monitors (16×) | 2,000 | 2,624 | 0 | 16 | 125 LUTs per SERV |
| Block 3: Control Network | 1,200 | 800 | 0 | 4 | HIVE-C ↔ SERV comm |
| Block 4: Configuration Manager | 1,500 | 1,200 | 0 | 2 | Virtual contexts |
| HIVE control plane subtotal | 6,100 | 5,524 | 0 | 30 | 9.6% LUT, 22% BRAM |
| Delta Network 4×4 mesh (16 routers) | 8,000 | 6,000 | 0 | 32 | Delta-owned; see Delta HAS |
| HIVE + NoC total | 14,100 | 11,524 | 0 | 62 | 22% LUT, 46% BRAM |
| Available for Compute/RAPIDS | 49,300 | 115,276 | 240 | 73 | Remaining resources |
| Total Budget | 63,400 | 126,800 | 240 | 135 | 100% |

Correction note (2026-10-12): earlier tables presented 14,100 LUTs as the “HIVE subtotal” while omitting the Delta NoC row; the NoC’s 8,000 LUTs were silently included. The table now separates the HIVE control plane from the Delta NoC. The 22%-LUT / 46%-BRAM occupancy figures refer to the combined total.

Key Design Constraints: - HIVE consumes zero DSP blocks (all 240 DSPs available for compute) - HIVE’s own memory footprint is 30 BRAM blocks (22%); combined with the NoC it is 62 (46%) - HIVE uses 9.6% of LUTs on its own (22% combined with the NoC) - Leaves 78% of logic resources for RAPIDS and Delta Network compute elements

## 19.5	4. Key Features

### 19.5.1	4.1 Hierarchical RISC-V Architecture

| Feature | Status | Description |
| --- | --- | --- |
| VexRiscv master (HIVE-C) | note | RV32IM, 5-stage pipeline, 32KB I-mem + 32KB D-mem |
| SERV monitors (16×) | note | RV32I bit-serial, 2KB I-mem + 2KB D-mem each |
| Control network | note | Star topology, round-robin arbitration |
| Independent firmware | note | Separate binaries for HIVE-C and SERV |

### 19.5.2	4.2 Distributed Monitoring

| Feature | Status | Description |
| --- | --- | --- |
| Per-tile traffic counters | note | Packet counts per direction (N/S/E/W) |
| Buffer occupancy tracking | note | Real-time FIFO fill levels |
| Congestion detection | note | Programmable thresholds, immediate alerts |
| Error detection | note | Parity errors, protocol violations |
| Periodic reporting | note | Configurable interval (default 1000 cycles) |

### 19.5.3	4.3 Network Reconfiguration

| Feature | Status | Description |
| --- | --- | --- |
| Virtual configuration contexts | note | 4 pre-loaded routing modes |
| Atomic context switching | note | Deterministic latency (<25 cycles) |
| Quiesce protocol | note | Drain in-flight packets before switch |
| Broadcast configuration | note | PKT_CONFIG packets to all tiles |

### 19.5.4	4.4 RAPIDS Integration

| Feature | Status | Description |
| --- | --- | --- |
| Inband descriptor injection | note | CDA packets via Delta Network to RAPIDS |
| Low-latency delivery | note | ~10-20 cycles vs. 100+ for memory-mapped |
| AXIS master interface | note | Direct injection into Delta Network |
| Completion tracking | note | PKT_STATUS packets from RAPIDS |

## 19.6	5. Interfaces

### 19.6.1	5.1 External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| Host Interface | Slave | 32-bit | UART/AXI4-Lite for host commands |
| AXIS (HIVE-C TX) | Master | 256-bit | CDA packet injection to RAPIDS |
| AXIS (SERV TX, 16×) | Master | 128-bit | PKT_CONFIG, PKT_STATUS injection |
| AXIS (HIVE-C RX) | Slave | 128-bit | PKT_STATUS from network |
| Control Network | Internal | 32-bit | HIVE-C ↔ SERV communication |

See: docs/hive_has/ch04_interfaces/ for the complete interface, packet-format, and register-map specifications.

### 19.6.2	5.2 Virtual Tile Mapping

| Virtual Tile | Entity | Connection |
| --- | --- | --- |
| Tile 16 | RAPIDS DMA | Edge-attached to the 4×4 mesh |
| Tile 17 | HIVE-C Controller | Edge-attached to the 4×4 mesh |

## 19.7	6. Use Cases

### 19.7.1	6.1 Descriptor Scheduling

Scenario: HIVE-C receives high-level transfer command from host

Flow: 1. Host sends command via UART/AXI4-Lite to HIVE-C 2. HIVE-C generates 256-bit CDA descriptor 3. HIVE-C injects CDA packet via AXIS master (TUSER=2’b01, TDEST=16) 4. Delta Network routes to RAPIDS (Virtual Tile 16) 5. RAPIDS descriptor engine processes 6. Completion reported via PKT_STATUS to HIVE-C

### 19.7.2	6.2 Congestion Management

Scenario: SERV monitor detects buffer congestion on tile 7

Flow: 1. SERV-7 monitors router buffer occupancy 2. Threshold exceeded (buffer_occupancy > cfg_threshold) 3. SERV-7 generates PKT_STATUS alert packet 4. Alert routed to HIVE-C (Virtual Tile 17) 5. HIVE-C aggregates alerts, initiates mitigation 6. HIVE-C may trigger network reconfiguration

### 19.7.3	6.3 Network Reconfiguration

Scenario: Switch from packet-switched mesh to tree reduction mode

Flow: 1. HIVE-C issues CONFIG_PREPARE command via control network 2. All SERV monitors flush in-flight packets 3. Routers enter quiescent state 4. HIVE-C broadcasts SET_ROUTING_MODE (PKT_CONFIG, context=2) 5. All tiles simultaneously update routing mode 6. HIVE-C issues CONFIG_ACTIVATE command 7. Network resumes with tree reduction routing

## 19.8	7. Test Coverage

### 19.8.1	7.1 Current Status

Overall: Specification phase - implementation pending

| Component | Spec Status | Implementation Status |
| --- | --- | --- |
| HIVE-C Controller | HAS/MAS v0.1 | Pending |
| SERV Monitors | HAS/MAS v0.1 | Pending |
| Control Network | HAS/MAS v0.1 | Pending |
| Configuration Manager | HAS/MAS v0.1 | Pending |
| Integration | HAS/MAS v0.1 | Pending |

## 19.9	8. Performance Targets

### 19.9.1	8.1 Latency Targets

- Descriptor Injection: < 30 cycles (HIVE-C → RAPIDS)

- Command Response: < 1 µs @ 100 MHz

- Monitoring Aggregation: < 100 cycles for all 16 SERV status reads

- Context Switch: < 25 cycles (including quiesce)

### 19.9.2	8.2 Monitoring Overhead

- SERV Monitor Overhead: < 5% of compute time

- Congestion Detection Latency: < 10 cycles

- Periodic Reporting Interval: 1000 cycles (configurable)

See: docs/hive_has/ch06_performance/01_performance.md for the full target set and the tradeoff tables behind them.

## 19.10	9. Integration Guidelines

### 19.10.1	9.1 HIVE + RAPIDS + Delta Integration

// Top-level integration example (architectural port names)
module system_top (
    input  logic        clk_100mhz,
    input  logic        rst_n,
    // Host interface
    input  logic [31:0] host_cmd,
    output logic [31:0] host_resp,
    // External memory (for RAPIDS)
    // ... AXI4 memory interface ...
);

    // HIVE instance
    hive_top u_hive (
        .aclk           (clk_100mhz),
        .aresetn        (rst_n),
        .host_cmd       (host_cmd),
        .host_resp      (host_resp),
        .axis_cda_*     (hive_to_delta_cda),
        .axis_status_*  (delta_to_hive_status)
    );

    // Delta Network instance
    delta_network_4x4 u_delta (
        .aclk           (clk_100mhz),
        .aresetn        (rst_n),
        .tile16_*       (rapids_interface),  // RAPIDS virtual tile
        .tile17_*       (hive_interface)     // HIVE-C virtual tile
    );

    // RAPIDS instance
    rapids_top u_rapids (
        .aclk           (clk_100mhz),
        .aresetn        (rst_n),
        .axis_cda_*     (rapids_interface),  // From Delta
        .m_axi_*        (memory_interface)   // To external memory
    );

endmodule

See: docs/hive_has/ch07_integration/01_integration_and_verification.md for the full integration and verification strategy.

## 19.11	10. Development Status

### 19.11.1	10.1 Current Phase

Phase: Specification (HAS/MAS v0.1 complete, 2026-10-12)

- Hardware Architecture Specification: complete (7 chapters)

- Micro-Architecture Specification: complete (6 chapters)

- RTL: pending (roadmap in the HAS, Chapter 7)

See: nothing tracked – the vault area was deleted 2026-09-27 (unstarted); the former items are in git history.

### 19.11.2	10.2 Roadmap

Near-Term (Q4 2025): - HIVE-C RTL implementation (Phase 1, per HAS ch.7 roadmap) - SERV monitor wrapper implementation - Control network implementation

Long-Term (2026+): - Configuration manager implementation - HIVE-C firmware development - SERV firmware development - Integration with RAPIDS and Delta - FPGA deployment and testing

## 19.12	11. Educational Value

HIVE demonstrates: - Hierarchical processor architecture (master + agents) - Distributed monitoring and control - RISC-V processor integration (VexRiscv, SERV) - Inband control packet injection - Dynamic network reconfiguration - Resource budget management - Control network design patterns

Target Audience: - Advanced RTL designers - Network-on-chip architects - Embedded systems engineers - RISC-V integration specialists

## 19.13	13. Documentation Generation

### 19.13.1	13.1 Generating PDF/DOCX from the Specifications

Tool: bin/md_to_docx.py (repo root) via the wrapper scripts:

cd projects/components/riscv-ip/hive-serv/docs
./generate_has_pdf.sh            # HIVE_HAS_v0.1.docx / .pdf
./generate_mas_pdf.sh            # HIVE_MAS_v0.1.docx / .pdf
# Optional revision override:
./generate_has_pdf.sh --rev 0.2

The scripts check dependencies (python-docx, PyYAML, pandoc, LibreOffice), then invoke md_to_docx.py with the book’s index, styles yaml, title page, ToC, and page breaks. Built books are checked in alongside their sources.

Requirements: Python 3.6+, pandoc, LibreOffice (PDF export), and the repo’s bin/md_to_docx.py.

## 19.14	14. References

### 19.14.1	14.1 Internal Documentation

- HIVE HAS: docs/hive_has/ ← Primary external-contract reference

- HIVE MAS: docs/hive_mas/ ← Primary implementation-intent reference

- Master PRD: /PRD.md

- Repository Guide: /CLAUDE.md

### 19.14.2	14.2 Related Subsystems

- RAPIDS: projects/components/dma-ip/rapids/ - DMA engine controlled by HIVE-C

- Delta Network: projects/components/fabric-gen-ip/delta/ - 4×4 mesh NoC

- AMBA/Ocelus: rtl/amba/ocelus/ - Monitor infrastructure for observability

- Common: rtl/common/ - Building blocks

### 19.14.3	14.3 External References

- VexRiscv: https://github.com/SpinalHDL/VexRiscv

- SERV: https://github.com/olofk/serv

- RISC-V ISA: https://riscv.org/technical/specifications/

- AXI4-Stream: ARM IHI 0051A

Document Version: 0.2 Last Updated: 2026-10-12 Review Cycle: Monthly during active development Owner: RTL Design Sherpa Project

## 19.15	Navigation

- ← Back to Root: /PRD.md

- Hardware Architecture Specification: docs/hive_has/hive_has_index.md

- Micro-Architecture Specification: docs/hive_mas/hive_mas_index.md

- Quick Start: README.md (to be created)

- AI Guidance: CLAUDE.md

- Tasks: none tracked (vault area deleted 2026-09-27, unstarted)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	RAPIDS Beats Architecture MAS

Version: 0.7 Date: 2026-07-13 Purpose: Module Architecture Specification for RAPIDS Phase 1 “Beats” Architecture

## 20.1	Document Organization

Note: All chapters linked below for automated document generation.

### 20.1.1	Front Matter

- Document Information

### 20.1.2	Chapter 1: Overview

- Architecture Overview

- Top-Level Port List

- Clocks and Reset

### 20.1.3	Chapter 2: FUB (Functional Unit Blocks)

Control Path: - Scheduler - Descriptor Engine

AXI Engines: - AXI Read Engine - AXI Write Engine

Flow Control: - Beats Alloc Control - Beats Drain Control - Beats Latency Bridge

Control Engines: - Control-Read Engine - Control-Write Engine

### 20.1.4	Chapter 3: Macro (Integration Blocks)

Scheduler Integration: - Beats Scheduler Group - Beats Scheduler Group Array

Sink Data Path (Network to Memory): - Sink Data Path - Sink Data Path AXIS - Sink SRAM Controller

Source Data Path (Memory to Network): - Source Data Path - Source Data Path AXIS - Source SRAM Controller

Top-Level Integration: - RAPIDS Core Beats - RAPIDS Registers - RAPIDS Config Block - RAPIDS Beats Top

### 20.1.5	Chapter 4: Interfaces

- Interface Overview

- AXI4 Interface Specification

- AXIS Interface Specification

- MonBus Interface Specification

## 20.2	Quick Reference

### 20.2.1	FUB Modules (fub_beats/)

FUB Module Summary

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| scheduler_beats | scheduler_beats.sv | Transfer coordinator, descriptor processing | Implemented |
| descriptor_engine_beats | descriptor_engine_beats.sv | Descriptor fetch/parse (256-bit) | Implemented |
| axi_read_engine_beats | axi_read_engine_beats.sv | AXI read master, streaming pipeline | Implemented |
| axi_write_engine_beats | axi_write_engine_beats.sv | AXI write master, streaming pipeline | Implemented |
| alloc_ctrl_beats | alloc_ctrl_beats.sv | Space allocation tracking (virtual FIFO); kept and tested, no longer in the SRAM path since bdf4e0dff | Implemented |
| drain_ctrl_beats | drain_ctrl_beats.sv | Data availability tracking (virtual FIFO); kept and tested, no longer in the SRAM path | Implemented |
| latency_bridge_beats | latency_bridge_beats.sv | Registered-FIFO to valid/ready skid bridge; kept and tested, no longer in the SRAM path | Implemented |
| ctrlrd_engine | ctrlrd_engine.sv | Control-read consumer gate (poll-until-match, retry budget) | Implemented |
| ctrlwr_engine | ctrlwr_engine.sv | Control-write producer doorbell (single-beat write) | Implemented |

### 20.2.2	Macro Modules (macro_beats/)

Macro Module Summary

| Module | File | Purpose | Status |
| --- | --- | --- | --- |
| scheduler_group_beats | scheduler_group_beats.sv | Scheduler + Descriptor Engine wrapper | Implemented |
| scheduler_group_array_beats | scheduler_group_array_beats.sv | 8-channel scheduler array with arbitration | Implemented |
| snk_data_path_beats | snk_data_path_beats.sv | Network-to-memory path integration | Implemented |
| snk_data_path_axis_beats | snk_data_path_axis_beats.sv | AXIS variant of sink path | Implemented |
| src_data_path_beats | src_data_path_beats.sv | Memory-to-network path integration | Implemented |
| src_data_path_axis_beats | src_data_path_axis_beats.sv | AXIS variant of source path | Implemented |
| snk_sram_controller_beats | snk_sram_controller_beats.sv | Sink SRAM: naming wrapper around STREAM’s sram_controller (one instance, per-channel FIFOs inside it) | Implemented |
| src_sram_controller_beats | src_sram_controller_beats.sv | Source SRAM: naming wrapper around STREAM’s sram_controller | Implemented |
| rapids_src_beats / rapids_snk_beats | rapids_src_beats.sv, rapids_snk_beats.sv | One half each: scheduler array + data path + monbus merge | Implemented |
| rapids_core_beats | rapids_core_beats.sv | Both halves + the core monbus arbiter | Implemented |
| rapids_regs | regs/generated/rtl/rapids_regs.sv (generated from rtl/macro_beats/rapids_regs.rdl) | PeakRDL register block (base + MON regfile @ 0x1000) | Implemented |
| rapids_config_block | rapids_config_block.sv | Maps register hwif_out to core/monitor cfg_* | Implemented |
| rapids_beats_top | rapids_beats_top.sv | Top-level: APB slave, kickoff, monitors, MonBus AXI-Lite group | Implemented |

## 20.3	Architecture Comparison: RAPIDS vs STREAM

The “beats” architecture is a Phase 1 implementation of RAPIDS that shares concepts with STREAM but targets network-to-memory (and vice versa) transfers rather than memory-to-memory.

RAPIDS Beats vs STREAM Comparison

| Feature | STREAM | RAPIDS Beats |
| --- | --- | --- |
| Primary Use | Memory-to-memory DMA | Network-to-memory accelerator |
| Data Paths | Single bidirectional | Separate sink/source paths |
| Network Interface | None | AXIS master/slave |
| SRAM Buffering | Shared buffer | Separate sink/source buffers |
| Descriptor Format | 256-bit | 256-bit (compatible) |
| Channel Count | 8 | 8 |
| Flow Control | alloc_ctrl/drain_ctrl | STREAM stream_alloc_ctrl/stream_drain_ctrl inside the shared sram_controller (alloc_ctrl_beats/drain_ctrl_beats kept, out of the path) |

## 20.4	Clock and Reset Summary

### 20.4.1	Clock Domains

Clock Domains

| Clock | Frequency | Usage |
| --- | --- | --- |
| aclk | 100-500 MHz | Primary - all RAPIDS logic, AXI/AXIS interfaces |

### 20.4.2	Reset Signals

Reset Signals

| Reset | Polarity | Type | Usage |
| --- | --- | --- | --- |
| aresetn | Active-low | Async assert, sync deassert | Primary - all RAPIDS logic |

See: Clocks and Reset for complete timing specifications

## 20.5	Interface Summary

### 20.5.1	External Interfaces

External Interfaces

| Interface | Type | Width | Purpose |
| --- | --- | --- | --- |
| AXI4 (Descriptor) | Master | 256-bit | Descriptor fetch |
| AXI4 (Sink Write) | Master | 512-bit (param) | Sink data write to memory |
| AXI4 (Source Read) | Master | 512-bit (param) | Source data read from memory |
| AXIS (Sink) | Slave | 512-bit (param) | Network data ingress |
| AXIS (Source) | Master | 512-bit (param) | Network data egress |
| MonBus | Master | 64-bit | Monitor packet output |

### 20.5.2	Internal Buses

Internal Buses

| Interface | Width | Purpose |
| --- | --- | --- |
| MonBus | 64-bit | Internal monitoring bus |
| Descriptor Bus | 256-bit | Descriptor distribution |
| SRAM Data Bus | 512-bit | SRAM read/write data |

## 20.6	Related Documentation

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

- RAPIDS Beats HAS - Architecture specification (the original pre-beats rapids_spec/ tree was retired)

## 20.7	Specification Conventions

### 20.7.1	Signal Naming

- Clock: aclk

- Reset: aresetn (active-low)

- Valid/Ready: Standard AXI/AXIS handshake

- Registers: r_ prefix (e.g., r_state, r_counter)

- Wires: w_ prefix (e.g., w_next_state, w_grant)

### 20.7.2	Parameter Naming

- Uppercase: NUM_CHANNELS, DATA_WIDTH, ADDR_WIDTH

- Derived: CHAN_WIDTH = $clog2(NUM_CHANNELS)

### 20.7.3	Figure and Table Numbering

- Figures: Figure X.Y.Z: Title where X=chapter, Y=section, Z=figure number

- Tables: Caption line : Table Title after table markdown

Last Updated: 2026-07-13 Maintained By: RAPIDS Architecture Team

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Delta Micro-Architecture Specification Index

Component: Delta (AXI4-Stream Crossbar Generator) Version: 1.0 Date: 2026-10-12 Purpose: Detailed micro-architecture specification for the generated Delta RTL

## 21.1	Overview

Delta is a Python generator that emits a single self-contained SystemVerilog module per configuration — a flat M×N AXI4-Stream crossbar routing s_axis_* masters to m_axis_* slaves. This is the internals document: the one you open when you need to know what the generated RTL actually does, block by block, rather than what the high-level spec says.

### 21.1.1	Key Features

- Flat crossbar only: Tree generation was retired on 2026-10-12.

- TDEST-based routing: No address map; the destination slave is taken directly from s_axis_tdest.

- Packet-atomic round-robin arbitration: A granted master holds the slave until TLAST.

- Combinational data mux: The grant is the registered stage; payload passes through combinatorially.

- Optional richness: TSTRB byte qualifiers, and AMBA 5 TWAKEUP with the IHI 0051B ch. 5 odd-parity check signals (TDATACHK … TWAKEUPCHK), checked at every port with sticky error flags (default off).

- Optional counters: Per-master and per-slave packet counters (disable with --no-counters).

- Formal assertions: ifdef FORMAL checks for one-hot grants and in-range TDEST.

### 21.1.2	Companion Specifications

- Delta HAS - Hardware Architecture Specification (high-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

## 21.2	Navigation

### 21.2.1	Front Matter

- Document Information

### 21.2.2	Chapter 1: Introduction

- Overview

### 21.2.3	Chapter 2: Block Descriptions

- Request Decode

- Per-Slave Round-Robin Arbiter

- Crossbar Multiplexer

- TREADY Steering

- TWAKEUP Steering (AMBA5)

- Interface Check Signals (AMBA5)

- Packet Counters

- Assertions

### 21.2.4	Chapter 3: Cycle-Level Behavior

- Multi-Master Contention

- Packet Interleave Prevention

### 21.2.5	Chapter 4: Generated RTL

- Module Structure

- Parameters and Logic Implications

### 21.2.6	Chapter 5: Verification

- Test Strategy

Last Updated: 2026-10-12 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	RISC-V Falcon Suite

Five independent, clean-slate RISC-V cores whose primary product is education, in the same spirit as the pumice/scoria/andesite memory-controller line. Each rung is a complete, verified, documented design small enough to hold in your head; the rungs ascend in microarchitectural complexity so the diff between neighbors is the lesson. Rungs do not share RTL — each core is the cleanest possible example of its level, not the cheapest to produce. Educational benefit over IP; no rung chases performance or benchmarks.

Design spec (source of truth for scope, ISA, verification, packaging): docs/superpowers/specs/2026-10-06-riscv-falcon-suite-design.md.

## 22.1	The ladder

Each rung carries a raptor-codename, real falcons first (species in strict size order, tracking complexity). The falcon run ends at gyrfalcon — the largest living species — so the top rung steps deliberately to mythology: garuda, the giant divine raptor, king of birds, one past the largest falcon. The codename is the RTL identifier prefix and the module/package prefix; the DIRECTORY is the compound <codename>-<isa> form — kestrel-rv32i, not kestrel. The bare codename is how the core is referred to in prose (“kestrel sees the whole machine in one cycle”).

| Directory | Codename | ISA | Microarchitecture | Status |
| --- | --- | --- | --- | --- |
| kestrel-rv32i/ | kestrel | RV32I | single-cycle | RTL complete; rv32ui battery 42/42 with spike lockstep; riscv-formal 42/42; doc book v0.1 published; board loader sim-verified |
| merlin-rv32i/ | merlin | RV32I | 5-stage in-order pipeline | Planned; structure only |
| peregrine-rv32im/ | peregrine | RV32IM | advanced in-order: predictor, traps, L1 caches, AXI master | Planned; structure only |
| gyrfalcon-rv32ima/ | gyrfalcon | RV32IM baseline; gyrfalcon-A flavor RV32IMA | out-of-order capstone: rename, ROB, reservation stations; two flavors — baseline and gyrfalcon-A (A_EXT=1: LR/SC + AMO atomics study) | Planned; structure only |
| garuda-rv32imaf/ | garuda | RV32IMAF | gyrfalcon’s OoO plus an FPU: FP register file + rename, fcsr precise flags, non-pipelined long-latency units | Planned; structure only (penciled in; beyond the 2026-10-06 design spec, which pins four rungs) |

### 22.1.1	Adjacent: hive-serv

hive-serv/ — the SERV-based compute cluster (1 VexRiscv control plane + 16 SERV monitor cores). Not a falcon-ladder rung: it uses RISC-V cores rather than teaching one, so it lives beside the ladder rather than on it. Re-homed from compute-eng-ip/hive 2026-10-10 (it was retired 2026-09-27 as an unstarted component; the re-home revives it under the falcon suite). HAS/MAS v0.1 published 2026-10-12 (hive-serv/docs/hive_has/, hive-serv/docs/hive_mas/) — no RTL yet.

## 22.2	Goals per rung

kestrel — see everything. Single-cycle RV32I: PC, register file, ALU, immediate generator, control as an explicit decode truth table, a plainly documented combinational-read memory contract. The hovering falcon: the whole machine is visible in one cycle — including its cost, since the critical path is the machine, which is exactly the motivation for merlin. Delivers: the ISA (R/I/S/B/U/J formats, the full base integer set), the datapath, honest control logic.

merlin — flow it. The classic IF/ID/EX/MEM/WB pipeline on the deliberately identical RV32I ISA, so students diff behavior, not ISA. Forwarding from EX/MEM and MEM/WB with explicit priority, load-use interlock stall, branches resolved in EX predict-not-taken with flush. Delivers: pipeline registers, RAW hazards, forwarding priority, the load-use bubble, control-hazard flushes, why branch resolution stage matters.

peregrine — speed it up. Advanced in-order RV32IM: iterative multicycle multiply/divide (the structural-hazard lesson), BTB + 2-bit bimodal prediction with mispredict recovery, machine-mode CSRs with precise exceptions and CLINT-shaped interrupts, split blocking write-back L1 caches behind an AXI4 master on the repo fabric with MonBus visibility. Delivers: speculation and recovery, precise exceptions, the memory hierarchy, bus protocol — and ties the whole repo together (memory controllers, cache IP line, AMBA fabric).

gyrfalcon — hide everything. Out-of-order capstone in two flavors (added 2026-10-11): the baseline — peregrine’s RV32IM exactly — and gyrfalcon-A (A_EXT=1): LR/SC reservations, store-conditional failure, and AMOs, the atomicity study — so every peregrine battery still carries in both. Dual-issue dispatch, explicit rename map + free list, 32-entry ROB with in-order commit, three reservation stations, wakeup/select over common data buses, speculative dispatch with checkpoint-restore rename rollback. Delivers: register renaming and WAR/WAW elimination, the ROB as an undo log, out-of-order completion with in-order retirement — the honest, complete OOO story — and the atomicity study: reservations, SC failure, AMOs, acquire/release as memory-model operations.

garuda — float it. The RV32IMAF delta on gyrfalcon’s OoO: a 32-entry FP register file with its own rename, an fcsr whose five accrued exception flags (NV/DZ/OF/UF/NX) stay precise-exception-clean through the ROB, and the repo’s ieee754 fp32 arithmetic behind the reservation stations — multi-cycle FMUL/FADD and the non-pipelined Goldschmidt FDIV / Newton-rsqrt FSQRT (9–11 cycle class) as the long-latency-unit lesson. Rounding hard-wired RNE; any other requested mode raises NV, keeping the CSR story honest without a dynamic rounding-mode datapath. Delivers: long-latency execution scheduling, FP architectural state, and the F-extension ISA delta (F regs, fcsr, FMV/FCVT/FSGNJ/FCLASS) — consuming the math library’s math_ieee754_2008_fp32_* blocks end-to-end.

## 22.3	Packaging (per rung, the MC-line treatment)

- Doc book — Simplified RV32I: kestrel and successors, Memory Notes branding, built through bin/md_to_docx.py to DOCX/PDF, following the Simplified DFI books’ precedent. Each book lives in its rung’s docs/; the kestrel edition is published at kestrel-rv32i/docs/simplified_rv32i/.

- Formal — riscv-formal-style ISA compliance where it pays: full on kestrel, retire-interface on merlin, targeted properties (exception precision, recovery, in-order commit, deadlock freedom) on peregrine/gyrfalcon.

- Golden model — spike ISS lockstep in the rds-dv cocotb framework; the classic rv32ui/rv32um riscv-tests batteries as the compliance gate.

- Drill app — bin/apps/core_drills (ddr_drills architecture): pipeline stage-viewer with forwarding arrows, hazard quiz, branch-predictor drill, and a rename/ROB viewer. App design gets its own doc once merlin’s RTL is fixed.

## 22.4	References and layout

Common primary sources (the RISC-V ISA manuals) live in references/ and are cited by chapter, not page.

Per-rung skeleton follows the dma-ip/stream component layout convention: rtl/, docs/, dv/ (cocotb + lockstep testbenches). Shared suite doctrine lands in docs/ as it is written. Per-rung PRD.md/CLAUDE.md arrive with each rung’s implementation plan.