Memory Notes

Simplified RV32I: kestrel Simplified edition - the single-cycle RV32I core, rung 1 of the falcon suite

October 23, 2026

Table of Contents

1 Simplified Rv32I Index	11

2 Introduction	12

2.1 What this book is	12

2.2 What kestrel is	13

2.3 Why single-cycle comes first	14

2.4 How to read this book	15

3 The Falcon Ladder	16

3.1 The suite	16

3.1.1 Figure 1.1: The falcon ladder, smallest to largest	17

3.2 What each rung teaches	18

3.3 Why kestrel is rung 1	19

4 Conventions	20

4.1 Numbers and identifiers	20

4.2 Register names	21

4.3 Halt versus trap	22

4.4 RVFI	23

4.5 Citations	24

5 Instruction Formats	25

5.1 Six formats, one word	25

5.2 How kestrel decodes a format	26

5.3 Immediate assembly	27

5.4 Shift immediates	28

6 The Instruction Set	29

6.1 The thirty-seven	29

6.1.1 Integer register-register (R-type)	29

6.1.2 Integer register-immediate (I-type)	29

6.1.3 Loads and stores	30

6.1.4 Branches	31

6.1.5 Jumps and upper immediates	31

6.2 Reserved encodings	33

7 The System Layer: Stubs, NOPs, and Halts	34

7.1 What RV32I surrounds itself with	34

7.2 FENCE and FENCE.I retire as NOPs	35

7.3 ECALL and EBREAK halt	36

7.4 MRET falls through	37

7.5 The CSR class retires as a bounded stub	38

7.6 Halt causes, collected	39

8 Datapath Overview	40

8.1 The machine in one picture	40

8.1.1 Figure 3.1: kestrel block diagram	40

8.2 The module list	41

8.3 The state inventory	42

8.4 Reading the chapters that follow	43

9 One Cycle, End to End	44

9.1 The clock in one sentence	44

9.2 Fetch and decode	45

9.3 Operands and immediates	46

9.4 Execute	47

9.5 Next-PC selection	48

9.6 Memory access	49

9.7 Writeback	50

9.8 Two worked examples	51

10 Control State	52

10.1 Why there is no FSM	52

10.2 The halt holding register	53

10.3 No retry bit, no merge register	54

10.4 The retirement counter	55

10.5 What this buys	56

11 The RVFI Retire Interface	57

11.1 Retirement as the observation channel	57

11.2 The signal table	58

11.3 The three gating rules	60

11.4 Memory-field packing	61

11.5 Why this pays	62

12 The Control Bundle	63

12.1 Thirteen wires out of decode	63

12.2 The key: opcode, funct3, funct7	65

12.3 The default bundle	66

12.4 Reading the table	67

13 The Decode Table	68

13.1 Arithmetic and upper-immediate rows	69

13.2 Loads, stores, branches, jumps	70

13.3 System, fence, and the default	71

13.4 What the table teaches	72

14 The Memory Contract	73

14.1 The interface	73

14.2 Timing: one cycle, except when documented	75

14.3 What the contract deliberately excludes	76

15 Misaligned Data Accesses	77

15.1 What the spec permits	77

15.2 The rotation model	78

15.3 The misaligned halt	79

15.4 Why not hardware?	80

16 IALIGN and the Misaligned-Target Halt	81

16.1 The rule	81

16.2 Why decode cannot catch it	82

16.3 The writeback subtlety	83

16.4 The six halt causes, in context	84

16.5 The fence_i connection	85

17 Testbench and Golden Model	86

17.1 The harness	86

17.2 The golden interpreter	87

17.3 Programs and the hex pipeline	88

17.4 The run grammar	89

18 The rv32ui Battery	90

18.1 What the battery is	90

18.2 The verdict protocol	93

18.3 What the battery proves — and its seam	94

19 Spike Lockstep	95

19.1 What lockstep adds	95

19.2 The mechanics	96

19.3 ma_data, an expected halt	97

19.4 The mnstatus divergence	98

20 The riscv-formal Flow	99

20.1 What it is	99

20.2 kestrel’s setup	100

20.3 The checks and depths	101

20.4 The flow’s three honest workarounds	102

20.5 Results	103

21 The CEX Story: What Formal Caught That Forty-Two Tests Missed	104

21.1 The failure	105

21.2 Why the battery missed it	106

21.3 The fix	107

21.4 Aftermath	108

21.5 The lesson, stated once	109

22 What kestrel Costs	110

22.1 CPI 1, honestly stated	110

22.2 The critical path is the whole machine	111

22.3 The accounting, instruction by instruction	112

22.4 What would have to give	113

23 The Next Rung: merlin	114

23.1 The five-stage answer	114

23.1.1 Figure 7.1: kestrel’s cycle, cut into merlin’s stages	114

23.2 What appears when the cycle is cut	115

23.3 What carries over unchanged	116

23.4 Why the ladder stops here first	117

List of Figures

Figure 1.1: The falcon ladder, smallest to largest	17

Figure 3.1: kestrel block diagram	40

Figure 7.1: kestrel’s cycle, cut into merlin’s stages	114

List of Tables

Table 1: The falcon ladder	16

Table 2: The six RV32I instruction formats (spec Volume I, section 2.1.2)	25

Table 3: Immediate assembly per format (spec Volume I, section 2.1.3)	27

Table 4: OP group: ten register-register instructions	29

Table 5: OP-IMM group: nine register-immediate instructions	29

Table 6: LOAD and STORE groups: five loads, three stores	30

Table 7: BRANCH group: six conditional branches	31

Table 8: JAL, JALR, LUI, AUIPC	31

Table 9: The six halt causes (encoding centralized in common-rv32i/rtl/includes/rv32i_pkg.sv)	39

Table 10: kestrel’s six modules	41

Table 11: The complete kestrel state inventory, register file aside	42

Table 12: RVFI signals and their kestrel aggregation	58

Table 13: The thirteen decode outputs	63

Table 14: Decode rows — OP, OP-IMM, LUI, AUIPC	69

Table 15: Decode rows — LOAD, STORE, BRANCH, JAL, JALR	70

Table 16: Decode rows — SYSTEM, MISC-MEM, and the illegal default	71

Table 17: The kestrel memory interface	73

Table 18: Halt causes and their spec counterparts	84

Table 19: rv32ui battery at func level — 42/42 with lockstep; every test halts on ecall (cause 1) with gp = 1, except ma_data, whose expected halt is the misaligned load (cause 4)	90

Table 20: Critical-path content per instruction class	112

List of Waveforms

No waveforms in this document.

# 1	Simplified Rv32I Index

Generated: 2026-10-23

# 2	Introduction

## 2.1	What this book is

This book is the condensed study-notes edition of RV32I as implemented by kestrel, the single-cycle core at rung 1 of the falcon suite. It is not a replacement for the RISC-V specification — the specification (references/riscv-spec.pdf, Volume I: Unprivileged Architecture) is the authority, and this book cites it by chapter and section throughout. What this book adds is the mapping from spec prose to silicon: which decode rows kestrel actually implements, what its memory contract pins down, how its retire interface reports execution, and how it was verified hard enough to trust.

Everything here describes the as-built core in common-rv32i/rtl/includes/rv32i_pkg.sv, rtl/fub/kestrel_regfile.sv, common-rv32i/rtl/fub/rv32i_alu.sv, common-rv32i/rtl/fub/rv32i_imm_gen.sv, common-rv32i/rtl/fub/rv32i_decode.sv, and rtl/top/kestrel_core.sv. Where a design decision was made that the spec leaves open — and RV32I leaves several doors open — the decision is stated plainly and labeled as a decision, with the spec’s own permission cited.

## 2.2	What kestrel is

kestrel is a single-cycle RV32I core:

- One instruction retires per clock; the entire datapath completes in one cycle. (Behind block RAM a load takes a second clock while the memory holds the core; the core itself has no multi-cycle instruction.)

- Control is a combinational decode truth table, not a finite-state machine. There is no time dimension to control yet.

- The complete architectural state is the PC register, the 32-entry register file, and a handful of bookkeeping flops — three in total.

- There is no trap machinery. ECALL, EBREAK, illegal instructions, misaligned control-flow targets and misaligned loads/stores halt the core with a cause code on a dedicated port, and the halting instruction retires as an RVFI trap beat (see Chapters 3 and 5).

- Retirement is observed through a first-class RISC-V Formal Interface (RVFI) port bundle, so an external checker can diff the core against a golden model instruction by instruction.

The name follows the suite’s convention: falcon species in strict size order, one per rung. The kestrel is the smallest falcon — the hovering falcon — and kestrel the core is the smallest machine in the suite: the whole of it is visible in one cycle.

## 2.3	Why single-cycle comes first

A single-cycle core is the honest starting point for the ladder because it removes every confound. When one instruction occupies the entire clock, there are no pipeline registers to reason about, no hazards, no forwarding priority, no speculation, no precise-exception problem. The register file either read the right operands or it didn’t; the ALU either computed the right value or it didn’t; the truth table either decoded the right controls or it didn’t. Every verification failure points at exactly one combinational block.

That transparency is bought with a cost, and the cost is the subject of Chapter 7: the clock period must cover the slowest instruction’s path through the whole machine, so the single-cycle discipline taxes every other instruction. Rung 2 (merlin) exists precisely because of that tax. But you cannot appreciate what a pipeline buys until you have traced one full cycle by hand, and you cannot debug a pipeline’s hazard logic until you know what the correct answer was. kestrel is where the correct answer lives.

## 2.4	How to read this book

Chapters 2 through 5 are reference: the ISA, the datapath, the truth table, the memory contract. Chapter 6 is the verification record — read it even if you skip the RTL chapters, because it is also the story of what each verification method can and cannot see. Chapter 7 is the argument for the next rung.

RTL identifiers appear in code font exactly as they do in the sources (ls_misalign, rd_wen_eff, HALT_IALIGN), so statements in this book can be checked against the code mechanically.

Source: RISC-V Instruction Set Manual, Volume I, chapters 1-2 (specification edition riscv-spec.pdf); falcon-suite design spec docs/superpowers/specs/2026-10-06-riscv-falcon-suite-design.md

# 3	The Falcon Ladder

## 3.1	The suite

The falcon suite is a sequence of four RISC-V cores, one microarchitecture per rung, each rung a complete, verified, documented design small enough to hold in your head. Falcon species come in strict size order, and so do the cores: the smallest falcon is the simplest machine.

The falcon ladder

| Rung | Core | ISA | Microarchitecture |
| --- | --- | --- | --- |
| 1 | kestrel-rv32i | RV32I | Single-cycle; the whole machine in one clock |
| 2 | merlin-rv32i | RV32I | Five-stage in-order pipeline (IF/ID/EX/MEM/WB) |
| 3 | peregrine-rv32im | RV32IM | Advanced in-order: branch prediction, traps, L1 caches, AXI master |
| 4 | gyrfalcon-rv32ima | RV32IMA | Out-of-order: rename, reorder buffer, reservation stations |

### 3.1.1	Figure 1.1: The falcon ladder, smallest to largest

The falcon ladder: four rungs from single-cycle to out-of-order

The ladder is the point. Each rung exists to teach exactly one layer of computer-architecture machinery, and each rung’s verification story reuses the rung below it. kestrel’s decode truth table becomes merlin’s per-stage control; merlin’s forwarding network becomes the baseline peregrine speculates on top of; peregrine’s precise traps become the commit contract gyrfalcon’s reorder buffer enforces.

## 3.2	What each rung teaches

- kestrel (this book). Instruction formats, the datapath, control truth tables, the memory contract, and why single-cycle costs what it costs. Everything is visible in one cycle.

- merlin. Pipeline registers, RAW hazards, forwarding priority, the load-use bubble, and control-hazard flushes. The ISA is deliberately identical to kestrel’s, so students diff behavior, not ISA, on the way up.

- peregrine. Speculation and the memory hierarchy: a branch predictor, real trap handling with precise state, and L1 caches behind an AXI master. The ISA grows to RV32IM.

- gyrfalcon. Out-of-order execution: register renaming, a reorder buffer, and reservation stations — the machine that hides program order.

## 3.3	Why kestrel is rung 1

The suite’s design rule is that you meet each concept at the smallest machine that exhibits it. Single-cycle execution exhibits everything the ISA promises and nothing the microarchitecture adds — the ideal baseline. It is also the cheapest rung to verify exhaustively: a combinational core with an RVFI port drops directly into riscv-formal’s instruction-check flow, and its traces diff cleanly against both a golden interpreter and the spike emulator.

Two consequences shape this book. First, kestrel documents its simplifications honestly — the CSR stubs, the FENCE NOPs, the halt instead of trap — because rung 3 is where those simplifications get replaced by real machinery, and the suite wants the seam visible. Second, kestrel keeps the ISA surface identical to merlin’s so the rv32ui battery, the spike lockstep, and (adapted) the formal proofs all carry over. A rung-1 verification investment pays off at rung 2.

Source: falcon-suite design spec, “Naming and placement” and “Rung 1 — kestrel”; Patterson and Séquin, “RISC I: A Reduced Instruction Set VLSI Computer” (ISCA 1981) — the original see-everything machine (microarchitecture-papers.md, entry 1)

# 4	Conventions

## 4.1	Numbers and identifiers

- Hexadecimal values are written with a 0x prefix or SystemVerilog style (32'hFFFF_FFFE); binary encodings use SystemVerilog sized literals (7'b0110011).

- RTL signal and parameter names appear in code font exactly as in the sources (dmem_wstrb, RESET_ADDR), so every claim can be grepped.

- Instruction names (ADD, JALR) are uppercase; assembly mnemonics in prose are lowercase (add x3, x1, x2).

## 4.2	Register names

RV32I defines thirty-two 32-bit registers, x0 through x31 (spec Volume I, section 2.1.1). This book uses the ABI names where they aid recognition — x1 is ra (return address), x2 is sp, x3 is gp, x10 and x11 are a0/a1 — and the bare numbers elsewhere. One register is special: x0 always reads as zero, and writes to it are discarded. kestrel enforces both properties in the register file itself.

## 4.3	Halt versus trap

This book uses the two words the way the core uses its ports:

- A trap, per the spec (Volume I, section 1.6), is a control transfer to a handler. kestrel has no trap machinery and never traps.

- A halt is kestrel’s stop condition: the core freezes the PC, suppresses writeback, and raises a cause code on halt/halt_cause. ECALL, EBREAK, illegal instructions, misaligned control-flow targets and misaligned loads/stores all halt. On RVFI the halting instruction still retires — as a beat with rvfi_trap set — so observers see the architecturally final state.

When the spec mandates a trap and kestrel substitutes a halt, the text says so explicitly, because that substitution is a documented design decision with a verification consequence (Chapter 6).

## 4.4	RVFI

RVFI (RISC-V Formal Interface) is the retire-port convention from the riscv-formal project: one record per retired instruction carrying PC, instruction word, register read/write, and memory access details. kestrel’s RVFI aggregation rules are given in Chapter 3; the field-level semantics follow riscv-formal’s rvfi.md.

## 4.5	Citations

The specification is cited by chapter and section (Volume I, section 2.1.6), never by page — the pinned PDF is a rolling intermediate release whose pagination can drift. Research literature is cited by entry in references/microarchitecture-papers.md, the suite’s reading list, in the form (papers, entry N).

Source: RISC-V Instruction Set Manual, Volume I, sections 1.6 and 2.1.1; vendor/riscv-formal/docs/rvfi.md

# 5	Instruction Formats

## 5.1	Six formats, one word

Every RV32I instruction is exactly one 32-bit word (spec Volume I, sections 1.5 and 2.1.2). There is no compressed form at this rung: IALIGN=32, so instructions are always 4-byte aligned and the low two bits of the PC are always zero. That single fact is what makes kestrel’s instruction-side misaligned policy (Chapter 5) so crisp.

Six formats cover the thirty-seven instructions. Register fields are 5 bits; the opcode is 7 bits at [6:0]; funct3 and funct7 qualify the operation within major opcodes.

The six RV32I instruction formats (spec Volume I, section 2.1.2)

| Bits | 31:25 | 24:20 | 19:15 | 14:12 | 11:7 | 6:0 |
| --- | --- | --- | --- | --- | --- | --- |
| R | funct7 | rs2 | rs1 | funct3 | rd | opcode |
| I | imm[11:0] | rs1 | funct3 | rd | opcode |  |
| S | imm[11:5] | rs2 | rs1 | funct3 | imm[4:0] | opcode |
| B | imm[12¦10:5] | rs2 | rs1 | funct3 | imm[4:1¦11] | opcode |
| U | imm[31:12] | rd | opcode |  |  |  |
| J | imm[20¦10:1¦11¦19:12] | rd | opcode |  |  |  |

Formats with fewer than six fields pack the remaining bits into the immediate, which is why U and J rows above leave cells empty.

## 5.2	How kestrel decodes a format

kestrel’s rv32i_decode keys its truth table on the triple {opcode, funct3, funct7} — the exact fields that distinguish every base instruction. rv32i_imm_gen does not wait for it: the format is a function of the opcode alone, so it builds all five immediates in parallel and picks one by insn[6:2] (decode’s imm_sel output still exists but is unused). The full table is Chapter 4; the format layer is worth understanding on its own because the immediate encodings are the least obvious part of the ISA.

## 5.3	Immediate assembly

Only one of the six formats (I) has its immediate in one contiguous slice. The others scatter immediate bits around the register fields — an artifact of keeping register specifiers in fixed positions so the register file can read while the immediate is still assembling (spec Volume I, section 2.1.3, records this rationale). The built 32-bit signed immediate for each format:

Immediate assembly per format (spec Volume I, section 2.1.3)

| Format | Immediate assembly | Used by |
| --- | --- | --- |
| I | sext(insn[31:20]) | ADDI, loads, JALR, shifts-by-immediate |
| S | sext({insn[31:25], insn[11:7]}) | stores |
| B | sext({insn[31], insn[7], insn[30:25], insn[11:8], 1’b0}) | branches |
| U | {insn[31:12], 12’b0} | LUI, AUIPC |
| J | sext({insn[31], insn[19:12], insn[20], insn[30:21], 1’b0}) | JAL |

sext is sign-extension from bit 31 of the assembled value. B and J immediates append an explicit 0 in bit 0 because branch and jump targets are always 2-byte-aligned at minimum — and with IALIGN=32, kestrel requires 4.

rv32i_imm_gen implements exactly this table: five assemblies with the sign bits replicated by concatenation, and a unique case on the major opcode insn[6:2] picking one. U-type needs no sign extension work beyond the shift; the spec’s canonical form zero-extends the 20-bit field and places it in the upper bits, which is what the RTL does.

## 5.4	Shift immediates

RV32I encodes the shift amount for SLLI/SRLI/SRAI in the I-immediate’s low 5 bits (insn[24:20], shamt), with insn[30] selecting arithmetic versus logical right shift for the SR family and insn[31:25] required to be 0000000 (SLLI, SRLI) or 0100000 (SRAI). kestrel’s decode keys on the full funct7 field, so any other high field is an illegal instruction rather than a shift — matching the spec’s reserved-encoding rules (spec Volume I, section 2.1.4).

Source: RISC-V Instruction Set Manual, Volume I, sections 1.5, 2.1.2, 2.1.3, 2.1.4; common-rv32i/rtl/fub/rv32i_imm_gen.sv, common-rv32i/rtl/fub/rv32i_decode.sv

# 6	The Instruction Set

## 6.1	The thirty-seven

RV32I is thirty-seven instructions. kestrel implements all of them, plus documented behavior for the FENCE/FENCE.I and SYSTEM opcode classes that the base set surrounds itself with. The groups below follow the spec’s own order (Volume I, section 2.1); the right-hand column is the spec section each group is defined in.

### 6.1.1	Integer register-register (R-type)

OP group: ten register-register instructions

| Instruction | funct3/funct7 | Operation | Spec |
| --- | --- | --- | --- |
| ADD | 000 / 0000000 | rd = rs1 + rs2 | 2.1.4.2 |
| SUB | 000 / 0100000 | rd = rs1 - rs2 | 2.1.4.2 |
| SLL | 001 / 0000000 | rd = rs1 << rs2[4:0] | 2.1.4.2 |
| SLT | 010 / 0000000 | rd = (rs1 <s rs2) ? 1 : 0 | 2.1.4.2 |
| SLTU | 011 / 0000000 | rd = (rs1 <u rs2) ? 1 : 0 | 2.1.4.2 |
| XOR | 100 / 0000000 | rd = rs1 ^ rs2 | 2.1.4.2 |
| SRL | 101 / 0000000 | rd = rs1 >>u rs2[4:0] | 2.1.4.2 |
| SRA | 101 / 0100000 | rd = rs1 >>s rs2[4:0] | 2.1.4.2 |
| OR | 110 / 0000000 | rd = rs1 ¦ rs2 | 2.1.4.2 |
| AND | 111 / 0000000 | rd = rs1 & rs2 | 2.1.4.2 |

### 6.1.2	Integer register-immediate (I-type)

OP-IMM group: nine register-immediate instructions

| Instruction | funct3 | Operation | Spec |
| --- | --- | --- | --- |
| ADDI | 000 | rd = rs1 + sext(imm) | 2.1.4.1 |
| SLTI | 010 | rd = (rs1 <s sext(imm)) ? 1 : 0 | 2.1.4.1 |
| SLTIU | 011 | rd = (rs1 <u sext(imm)) ? 1 : 0 | 2.1.4.1 |
| XORI | 100 | rd = rs1 ^ sext(imm) | 2.1.4.1 |
| ORI | 110 | rd = rs1 ¦ sext(imm) | 2.1.4.1 |
| ANDI | 111 | rd = rs1 & sext(imm) | 2.1.4.1 |
| SLLI | 001, shamt | rd = rs1 << shamt | 2.1.4.1 |
| SRLI | 101, shamt, insn[30]=0 | rd = rs1 >>u shamt | 2.1.4.1 |
| SRAI | 101, shamt, insn[30]=1 | rd = rs1 >>s shamt | 2.1.4.1 |

SLTIU compares against the sign-extended immediate interpreted unsigned — a famously surprising detail; sltiu rd, rs1, -1 tests against 0xFFFFFFFF. kestrel implements it with the ALU’s unsigned less-than on the same sign-extended immediate everyone else gets, so the surprise is preserved exactly.

### 6.1.3	Loads and stores

LOAD and STORE groups: five loads, three stores

| Instruction | funct3 | Size | Sign | Spec |
| --- | --- | --- | --- | --- |
| LB | 000 | byte | signed | 2.1.6 |
| LH | 001 | halfword | signed | 2.1.6 |
| LW | 010 | word | n/a | 2.1.6 |
| LBU | 100 | byte | zero | 2.1.6 |
| LHU | 101 | halfword | zero | 2.1.6 |
| SB | 000 | byte | n/a | 2.1.6 |
| SH | 001 | halfword | n/a | 2.1.6 |
| SW | 010 | word | n/a | 2.1.6 |

Effective address is rs1 + sext(imm) for both groups. Loads sign-extend (LB, LH) or zero-extend (LBU, LHU) into rd; stores source rs2. The memory side of these eleven instructions — alignment policy, byte strobes, the misaligned-access halt — is the subject of Chapter 5.

### 6.1.4	Branches

BRANCH group: six conditional branches

| Instruction | funct3 | Condition (taken when) | Spec |
| --- | --- | --- | --- |
| BEQ | 000 | rs1 == rs2 | 2.1.5.2 |
| BNE | 001 | rs1 != rs2 | 2.1.5.2 |
| BLT | 100 | rs1 <s rs2 | 2.1.5.2 |
| BGE | 101 | rs1 >=s rs2 | 2.1.5.2 |
| BLTU | 110 | rs1 <u rs2 | 2.1.5.2 |
| BGEU | 111 | rs1 >=u rs2 | 2.1.5.2 |

Target is PC + sext(B-immediate), relative to the branch’s own address. Not-taken cost is identical to taken cost in a single-cycle machine — one cycle either way — which is one of the few places kestrel is easier than the spec’s mental model.

### 6.1.5	Jumps and upper immediates

JAL, JALR, LUI, AUIPC

| Instruction | Format | Operation | Spec |
| --- | --- | --- | --- |
| JAL | J | rd = pc+4; pc = pc + sext(imm) | 2.1.5.1 |
| JALR | I | rd = pc+4; pc = (rs1 + sext(imm)) & ~1 | 2.1.5.1 |
| LUI | U | rd = imm (upper 20 bits) | 2.1.4 |
| AUIPC | U | rd = pc + imm | 2.1.4 |

JALR masks the target’s bit 0 (JALR_ALIGN_MASK = 32'hFFFF_FFFE in the core); with IALIGN=32, a target whose bits [1:0] are not 00 raises the instruction-address-misaligned halt instead of executing (Chapter 5). AUIPC forms the first half of the standard auipc/jalr pair for position-independent long jumps; LUI never passes through the ALU in kestrel — the writeback mux takes the immediate directly.

## 6.2	Reserved encodings

Every encoding not listed above — branch funct3 2-3, load/store funct3 3 and 6-7, MISC-MEM funct3 2-7, SYSTEM funct3 100, SYSTEM imm12 outside ECALL/EBREAK/MRET, shift funct7 mismatches — is an illegal instruction in kestrel: decode falls to the default row and the core halts with cause 4'hF (Chapter 4 gives the table; Chapter 5 explains the halt). HINT encodings (base instructions writing x0, spec Volume I, section 2.1.9) execute as their base instruction; the x0 write is discarded, so the architectural effect is the NOP the spec recommends.

Source: RISC-V Instruction Set Manual, Volume I, sections 2.1.4, 2.1.5, 2.1.6, 2.1.9; common-rv32i/rtl/fub/rv32i_decode.sv, rtl/top/kestrel_core.sv

# 7	The System Layer: Stubs, NOPs, and Halts

## 7.1	What RV32I surrounds itself with

The thirty-seven base instructions sit inside two opcode classes that belong to the wider system: MISC-MEM (FENCE and friends, spec Volume I, section 2.1.7, with FENCE.I split into the Zifencei extension, section 4.1) and SYSTEM (ECALL, EBREAK, and the CSR instructions of Zicsr, section 4.2, with MRET defined in the privileged Volume II). kestrel has no memory ordering hardware, no CSR file, and no trap handler, so these classes retire through documented stubs and NOPs rather than real machinery. This chapter states exactly what that means — including what it does not mean.

## 7.2	FENCE and FENCE.I retire as NOPs

kestrel’s imem and dmem are separate, combinational-read ports on a single-cycle core: there is no write buffer to drain, no cache to invalidate, no instruction prefetch to refetch. Fence semantics are therefore vacuous at this rung, and decode retires FENCE (MISC-MEM funct3 000) and FENCE.I (funct3 001) as NOPs — one cycle each, no architectural effect. MISC-MEM funct3 2-7 are reserved and halt as illegal instructions.

One subtlety is worth pinning down because it surprises people: the core is Harvard, but the testbench memory is a single unified array that serves both fetch and load/store. That unification is a testbench choice, made so rv32ui-p-fence_i’s self-modifying code works with no coherence path (see Chapter 6). It is not a core property, and the book does not pretend otherwise: kestrel the core never sees its own stores land in the instruction stream unless the memory system chooses to show them.

## 7.3	ECALL and EBREAK halt

ECALL and EBREAK exist to transfer control to an execution environment (spec Volume I, section 2.1.8). kestrel has no execution environment beyond halt-and-observe, so both halt the core: ECALL with cause 4'h1, EBREAK with cause 4'h2. The halting instruction retires as an RVFI trap beat. In practice ECALL is the suite’s program-termination convention: every riscv-tests program ends in the p-env’s RVTEST_PASS/RVTEST_FAIL sequence, whose final ecall is how kestrel knows the program is done (Chapter 6 reads the verdict off the gp register’s last writeback).

## 7.4	MRET falls through

MRET (SYSTEM funct3 000, imm12 0x302) returns from a machine trap. It is meaningful only when traps exist — trap state, mepc, privilege transfers, all of which are rung-3 machinery (privileged Volume II, chapter 3). kestrel retires MRET as a documented NOP-with-fall-through: the PC advances to pc+4. That choice is not arbitrary: the riscv-tests p-environment points mepc at the instruction following its trap stub in the configurations kestrel runs, so pc+4 is exactly what the test expects. This is stated plainly so nobody mistakes it for trap support.

## 7.5	The CSR class retires as a bounded stub

The six Zicsr instruction forms — CSRRW, CSRRS, CSRRC and their immediate variants (SYSTEM funct3 001/010/011/101/110/111) — retire through a stub: csr_stub in decode raises rd_wen, and the writeback mux substitutes a hard zero. Reads return zero; writes drop on the floor. SYSTEM funct3 100 is reserved and halts as illegal.

Say it plainly: this is not CSR support. There is no CSR file, no mstatus, no mtvec, no counters, and nothing the instructions do has architectural effect beyond a zero written to rd. The stub exists for one bounded reason: the riscv-tests p-environment preamble executes about a dozen CSR instructions plus an MRET before every test body, and without the stub the battery cannot run at all. On the battery programs the only CSR read with rd != x0 is mhartid (value 0), which the stub matches. Anything needing real CSR state — trap handlers, counters, feature probing — is out of contract at this rung and fails loudly either as a zero where a nonzero was expected or as a program that halts where it should have trapped.

The honesty matters for the ladder: merlin inherits the same stub, and peregrine replaces it with a real machine-mode CSR file. The seam is deliberately visible in the decode table.

## 7.6	Halt causes, collected

The six halt causes (encoding centralized in common-rv32i/rtl/includes/rv32i_pkg.sv)

| Cause | Name | Raised by | Spec would say |
| --- | --- | --- | --- |
| 4'h1 | HALT_ECALL | decode, ECALL | environment call |
| 4'h2 | HALT_EBREAK | decode, EBREAK | breakpoint |
| 4'h3 | HALT_IALIGN | core, misaligned taken control transfer | instruction-address-misaligned trap |
| 4'h4 | HALT_LALIGN | core, misaligned load | load-address-misaligned trap |
| 4'h5 | HALT_SALIGN | core, misaligned store | store-address-misaligned trap |
| 4'hF | HALT_ILL | decode, default row | illegal instruction |

Causes 1, 2, and F come from decode; causes 3-5 can only be raised by the core, because they need the resolved next PC and branch decision, or the access address, which decode cannot see. Chapter 5 develops causes 3-5 and the alignment rules behind them.

Source: RISC-V Instruction Set Manual, Volume I, sections 2.1.7, 2.1.8, 4.1, 4.2; Volume II, chapter 3 (MRET, machine trap state); common-rv32i/rtl/fub/rv32i_decode.sv, rtl/top/kestrel_core.sv, common-rv32i/rtl/includes/rv32i_pkg.sv

# 8	Datapath Overview

## 8.1	The machine in one picture

kestrel is six SystemVerilog modules and one package. Fetch is a wire: imem_addr is driven directly by the PC register and imem_rdata is the instruction. Everything else — decode, immediate generation, register read, execute, memory access, writeback, next-PC selection, retirement reporting — is combinational logic wrapped around a handful of state elements.

### 8.1.1	Figure 3.1: kestrel block diagram

kestrel datapath block diagram

The diagram is drawn to match the RTL: the blocks are the module instances in kestrel_core.sv, and every named wire is a real signal in the source.

## 8.2	The module list

kestrel’s six modules

| Module | Role | State |
| --- | --- | --- |
| rv32i_pkg | Shared enums (alu_op_e, imm_sel_e) and the halt-cause encodings | none |
| kestrel_regfile | 32 x 32-bit registers, two combinational read ports, one synchronous write, x0 tied to zero | 32 words |
| rv32i_imm_gen | Builds the 32-bit immediate for the five formats | none |
| rv32i_alu | Ten operations: ADD SUB AND OR XOR SLL SRL SRA SLT SLTU | none |
| rv32i_decode | Combinational truth table from {opcode, funct3, funct7} to a thirteen-field control bundle | none |
| kestrel_core | Wires the leaves together: source muxes, branch comparator, next-PC mux, writeback mux, load/store rotation, halt, RVFI | 5 flops + counters |

## 8.3	The state inventory

Everything kestrel remembers, outside the register file:

The complete kestrel state inventory, register file aside

| Element | Width | Purpose |
| --- | --- | --- |
| pc | 32 | Program counter; resets to RESET_ADDR (default 0) |
| retire_count | 64 | RVFI retirement order: 0, 1, 2, … |
| halt_q | 1 | Halt holding register; latches any halt cause |

That is the whole machine. Three flops plus a 32-word register file is the entire sequential content of a core that passes the rv32ui battery and the riscv-formal ISA proofs — the single-cycle discipline buys its simplicity honestly, and this table is the receipt. There is no FSM anywhere: the halt bit is a plain holding flop, and Chapter 4 shows why no more control state exists.

The register file itself has no reset (2026-10-20): it is plain storage, so it fits in distributed RAM rather than 1,024 flops. A register is undefined until a program writes it, and keeps an earlier program’s value across a reload, so every program initializes each register it reads. The riscv-tests p-env does exactly that at its reset vector; the golden interpreter fails any test program that does not (Chapter 6). x0 is the one exception, and the core handles it by writing: entry 0 gets zero while the core is in reset and on every instruction that names x0 as its destination, so nothing on the read side has to special-case it.

## 8.4	Reading the chapters that follow

Chapter 3’s remaining sections trace one cycle end to end (section 2), catalog the state and its gating (section 3), and specify the RVFI retire interface (section 4). Chapter 4 opens the decode truth table itself.

Source: rtl/top/kestrel_core.sv, rtl/fub/kestrel_regfile.sv, common-rv32i/rtl/fub/rv32i_decode.sv, common-rv32i/rtl/fub/rv32i_alu.sv, common-rv32i/rtl/fub/rv32i_imm_gen.sv, common-rv32i/rtl/includes/rv32i_pkg.sv

# 9	One Cycle, End to End

## 9.1	The clock in one sentence

On every rising edge, kestrel writes one register-file entry (maybe), updates the PC (unless held), and moves its bookkeeping flops — while a combinational cloud standing between pc and the next edge computes everything the next cycle needs: the fetched instruction, its decode, the operands, the result, the memory access, the next PC, and the RVFI record for whatever retires this cycle.

The sections below follow the data in dependency order, not file order.

## 9.2	Fetch and decode

imem_addr = pc and insn = imem_rdata — the instruction memory is expected combinationally: address in at the cycle, word out in the same cycle (the contract is Chapter 5). Decode is the truth table of Chapter 4, keyed on {opcode, funct3, funct7}, producing the thirteen-field control bundle (alu_op, imm_sel, alu_src_a_pc, alu_src_b_imm, rd_wen, dmem_req, dmem_we, dmem_size, branch, jump, jalr, halt_cause, csr_stub) plus defaults of “no operation, no halt.”

## 9.3	Operands and immediates

The register file reads rs1 = insn[19:15] and rs2 = insn[24:20] combinationally; x0 reads zero because the file ties it. The immediate generator builds all five format immediates and picks one by the opcode alone, in parallel with the register read (decode’s imm_sel is unused). Two muxes select the ALU inputs: alu_src_a is the PC (for AUIPC and branch targets) or rs1_data; alu_src_b is the immediate or rs2_data.

## 9.4	Execute

The ALU computes one of ten operations. Its internal eq/lt/ltu flags are deliberately left unconnected at the instance: with src_a = PC for branches those flags would compare pc against imm, not rs1 against rs2. Branch conditions come from a dedicated comparator in the core — cmp_eq, cmp_lt (signed), cmp_ltu (unsigned) on the register data — with funct3 selecting the condition in a unique case. In a single-cycle machine this parallel evaluation is free: the branch decision and the pc+imm target are both ready when the next PC is chosen, so taken and not-taken branches cost the same cycle.

## 9.5	Next-PC selection

The next-PC mux has exactly four rows:

- JAL: pc + imm (J-immediate).

- JALR: (rs1_data + imm) & JALR_ALIGN_MASK with JALR_ALIGN_MASK = 32'hFFFF_FFFE — bit 0 cleared, per spec Volume I, section 2.1.5.1.

- Taken branch: pc + imm (B-immediate).

- Fallthrough: pc + 4.

When taken control flow resolves to an address whose low two bits are not 00, the core raises HALT_IALIGN instead of updating the PC — Chapter 5 covers the rule and why it lives in the core rather than decode.

## 9.6	Memory access

For loads and stores, dmem_addr is the word-aligned ALU result, dmem_wdata/dmem_wstrb are the store data and byte enables rotated into position, and read data is rotated back before size/sign selection implements LB/LBU/LH/LHU/LW. A naturally aligned access completes in the one cycle; a misaligned one halts and performs nothing. The full story — rotation math, the halt, RVFI encoding — is Chapter 5’s.

## 9.7	Writeback

rd_wdata is selected by a five-row priority mux:

- csr_stub — hard zero (the CSR stub, Chapter 2).

- LUI — the U-immediate, straight from the generator, never through the ALU.

- Load — the assembled, sign-extended read data.

- JAL/JALR — pc + 4, the link value.

- Everything else — alu_y.

The write enable is rd_wen_eff = rd_wen & ~halt_q & ~mem_wait & ~misalign_link & ~ls_misalign — equivalent to gating with halt: decode’s rd_wen qualified by halt (the misaligned-control-flow halt retires JAL/JALR encodings, and the misaligned-load halt retires loads, whose decode rd_wen is set, so the gate is halt itself, not decode) and by the memory wait of a block-RAM load. The register file discards writes to x0 independently.

## 9.8	Two worked examples

add x3, x1, x2: decode selects ADD with both source muxes on registers; the ALU’s alu_y passes the priority mux (row 5); x3’s entry is written on the next rising edge; next_pc = pc + 4.

bne x5, x6, -8 (a backward taken edge): the comparator’s ~cmp_eq is selected by funct3; the ALU simultaneously computes pc + imm; the next-PC mux row 3 wins; the PC register loads the target on the next edge and rvfi reports a beat with pc_wdata equal to that target.

Source: rtl/top/kestrel_core.sv (fetch, source muxes, ALU instance, comparator, next-PC mux, writeback mux); common-rv32i/rtl/fub/rv32i_decode.sv

# 10	Control State

## 10.1	Why there is no FSM

Classic processor control is a finite-state machine walking fetch, decode, execute, memory, writeback. kestrel doesn’t have those phases: they are spatial, not temporal. Fetch is a wire off the PC register; decode is a truth table; execute is an ALU; memory access is a combinational port; writeback is a mux feeding a clocked register file. There is nothing to sequence, so there is no FSM — and no microcode, no control ROM, no phase counter.

The one place where a naive design would reach for an FSM — halting — is handled by a single holding flop with a plain set condition.

## 10.2	The halt holding register

halt_q is a one-bit flop that latches any nonzero halt_cause_eff:

halt_q <= halt_q | halt_now;

halt_now is the combinational OR of decode’s causes (ecall, ebreak, illegal) and the core’s own misaligned-target and misaligned-access conditions, masked while the memory holds the core (mem_wait). The external halt output is halt_q | halt_now so the first halting cycle is visible immediately. Once latched:

- The PC freezes (pc <= (halt || mem_wait) ? pc : next_pc).

- Writeback is suppressed (rd_wen_eff on the held halt and the halts an rd-writing instruction can raise, rd_wb on halt).

- Retirement stops: rvfi_valid = rst_n & ~halt_q & ~mem_wait, and since halt_q latches on the halting cycle, the halting instruction itself is the last beat — a trap beat with rvfi_trap = halt_now.

- The testbench watches halt-hold: after the trap beat, four more cycles of halt raised, rvfi_valid low, rvfi_pc_rdata frozen (Chapter 6).

A holding register rather than a pulse matters for verification: an external observer sampling any time after the halt sees a stable, unambiguous stopped state with the cause still on the port.

## 10.3	No retry bit, no merge register

Until 2026-10-20 kestrel completed misaligned loads and stores in hardware: a cross-word access took two memory beats, remembered by a retry flop (ls_retry) and a 32-bit merge register (ls_rdata_lo) holding the first word’s bytes. On block RAM that retry logic sat on the critical path, so it was removed: a misaligned access now halts (Chapter 5), and those two flops left the inventory. No data is captured mid-instruction anywhere in kestrel.

## 10.4	The retirement counter

retire_count increments whenever rvfi_valid is high and backs rvfi_order, so retired instructions are numbered 0, 1, 2, … in program order forever. It is 64 bits wide — a 32-bit core will not wrap it — and it exists for exactly one consumer: riscv-formal’s consistency checks, which use the order field to relate beats across cycles.

## 10.5	What this buys

Three flops with one-line next-state equations are reviewable in minutes and provable in bounded model checking with shallow depths (Chapter 6’s formal checks run at depth 6 and cover reset plus memory wait states with headroom). The discipline the suite’s style brief imposes — datapath logic is truth tables and muxes; FSMs only where control is unavoidable — shows up here as the reason kestrel’s control can be exhaustively checked rather than merely simulated.

Source: rtl/top/kestrel_core.sv (halt register, PC register, retirement counter); falcon-suite RTL style brief

# 11	The RVFI Retire Interface

## 11.1	Retirement as the observation channel

kestrel was built RVFI-first: the retire port is not debug instrumentation bolted on afterward but the core’s defined observation channel, present from the first vertical slice. Every instruction that retires produces one beat; the halting instruction produces one final trap beat; nothing else produces anything. A cocotb testbench diffs these beats field by field against a golden interpreter, and riscv-formal relates them to an instruction-set model (Chapter 6). Both consumers need the aggregation rules below to be exact.

## 11.2	The signal table

RVFI signals and their kestrel aggregation

| Signal | Width | kestrel semantics |
| --- | --- | --- |
| rvfi_valid | 1 | High exactly on cycles a beat retires: rst_n & ~halt_q & ~mem_wait |
| rvfi_order | 64 | retire_count; 0, 1, 2, … over retired beats |
| rvfi_pc_rdata | 32 | The retiring instruction’s PC |
| rvfi_pc_wdata | 32 | next_pc — the architecturally next PC (the misaligned target on a cause-3 halt) |
| rvfi_insn | 32 | The instruction word |
| rvfi_trap | 1 | halt_now: high on the single halting trap beat |
| rvfi_rs1_addr / rvfi_rs2_addr | 5+5 | Register-file read ports (insn[19:15], insn[24:20]) |
| rvfi_rs1_rdata / rvfi_rs2_rdata | 32+32 | Read data; x0 reads zero naturally |
| rvfi_rd_addr / rvfi_rd_wdata | 5+32 | Writeback, when rd_wb; both zero otherwise |
| rvfi_mem_addr | 32 | The word address of a load/store, zero otherwise (and on a misaligned, halting access) |
| rvfi_mem_rmask / rvfi_mem_wmask | 4+4 | The byte lanes the access covers within that word |
| rvfi_mem_rdata / rvfi_mem_wdata | 32+32 | The raw word read (loads) / size-truncated store data shifted into its lanes, zero otherwise |

## 11.3	The three gating rules

Three qualifications turn the raw datapath signals into a clean retire stream:

- x0 reports zero. rd_wb = rd_wen & (insn[11:7] != 0) & ~halt, so a beat writing x0 reports rd_addr = 0 with rd_wdata = 0. This is not merely tidiness: riscv-formal requires rd_wdata == 0 whenever rd_addr == 0, and the early core failed exactly there until the rule was pinned. The register file independently discards the x0 write architecturally.

- A wait cycle is silent. While a block-RAM memory holds the core with mem_wait (a load’s first cycle), rvfi_valid is low: the instruction reports one beat, on the cycle it completes.

- Halt retires one trap beat, then silence. rvfi_trap = halt_now marks the halting instruction itself (ecall, ebreak, illegal, misaligned target, or misaligned load/store) — a beat with trap set, no rd write, no memory fields — after which the latched halt_q holds rvfi_valid low forever. The wrapper used by riscv-formal derives rvfi_halt =    rvfi_valid & halt, which matches the semantic exactly: the trap beat is the final retirement.

## 11.4	Memory-field packing

kestrel follows riscv-formal’s RISCV_FORMAL_ALIGNED_MEM convention: rvfi_mem_addr is the word address and the masks mark the byte lanes the access covers inside it, the same lanes the store strobes drive. A load reports the raw word; a store reports its value truncated to the access size and shifted into its lanes, not the raw register — leaking the upper bytes into the payload is a real bug class this encoding prevents. (When kestrel still completed misaligned accesses in hardware it reported the unaligned byte address with masks packed from bit 0, riscv-formal’s other convention.)

## 11.5	Why this pays

Because the rules are few and stated in RTL comments next to the assignments, the golden interpreter models the same rules in Python and the two can be diffed beat for beat over tens of thousands of instructions with zero tolerance. When a diff appears, it names the cycle, the PC, and the field — which is how the misaligned-jump defect of Chapter 6 was localized from a formal counterexample in minutes rather than debugged from waveforms.

Source: rtl/top/kestrel_core.sv (RVFI aggregation block); vendor/riscv-formal/docs/rvfi.md; dv/tbclasses/kestrel/rv32i_interpreter.py

# 12	The Control Bundle

## 12.1	Thirteen wires out of decode

rv32i_decode is a pure function from the 32-bit instruction word to a control bundle. It holds no state and no clock; the same word always produces the same bundle. The bundle’s fields:

The thirteen decode outputs

| Field | Type | Meaning when set / value |
| --- | --- | --- |
| alu_op | enum | ALU operation: ADD SUB AND OR XOR SLL SRL SRA SLT SLTU |
| imm_sel | enum | Immediate format to build: I S B U J |
| alu_src_a_pc | bit | ALU left input is the PC (else rs1) |
| alu_src_b_imm | bit | ALU right input is the immediate (else rs2) |
| rd_wen | bit | The instruction writes rd this cycle |
| dmem_req | bit | A data-memory access is in flight |
| dmem_we | bit | The access is a store (else a load) |
| dmem_size | 2 bits | Access size: 00 byte, 01 halfword, 10 word |
| branch | bit | Instruction is a conditional branch |
| jump | bit | Instruction is JAL |
| jalr | bit | Instruction is JALR |
| halt_cause | 4 bits | Halt encoding (Chapter 2), HALT_NONE when running |
| csr_stub | bit | SYSTEM CSR class: writeback a hard zero |

## 12.2	The key: opcode, funct3, funct7

The truth table is a single unique casez keyed on {opcode, funct3, funct7} — a 17-bit key. Every RV32I instruction is distinguished by those three fields alone, which is why the table needs no other instruction bits at the top level. (The SYSTEM privileged row then re-cases on insn[31:20], the imm12, to split ecall/ebreak/mret from the illegal encodings.)

Fields that carry immediate bits in some formats are keyed as don’t-cares: F7_DC = 7'b??????? matches OP-IMM arithmetic and every load, store, and branch row, since funct7 is immediate bits there. The shift rows are the exception — SLLI/SRLI/SRAI key funct7 fully, which is what makes mis-encoded shifts illegal rather than silently executed.

## 12.3	The default bundle

Before the case, decode assigns a default bundle: alu_op = ADD, imm_sel = I, every control bit clear, halt_cause = HALT_NONE. Each table row then sets only what it needs — most rows touch three or four fields. The default: row of the case changes exactly one field, halt_cause = HALT_ILL, which is the entire illegal-instruction mechanism. Keeping the default explicit means every encoding not claimed by a row halts, loudly, instead of executing as something plausible.

## 12.4	Reading the table

The next section gives the table in full, transcribed from common-rv32i/rtl/fub/rv32i_decode.sv. Notation is defined at the top of that section: the 17-bit key as opcode.funct3.funct7, any for don’t-care key fields, abbreviated bundle columns (alu, imm, aPC, bImm, rd), non-default values only (— means the default), and dmem sizes in bytes (B/H/W).

Source: common-rv32i/rtl/fub/rv32i_decode.sv, common-rv32i/rtl/includes/rv32i_pkg.sv

# 13	The Decode Table

The full unique casez ({opcode, funct3, funct7}) of common-rv32i/rtl/fub/rv32i_decode.sv, transcribed row for row. The key column is the 17-bit casez key written as opcode.funct3.funct7; any marks a don’t-care key field. Bundle columns show only non-default values and are abbreviated to keep the table readable — alu is alu_op, imm is imm_sel, aPC is alu_src_a_pc, bImm is alu_src_b_imm, rd is rd_wen, and dmem reports req, +we, and the size in bytes (B/H/W). — means the default value.

## 13.1	Arithmetic and upper-immediate rows

The ALU computes datapath arithmetic here; for LUI the writeback mux bypasses the ALU and takes the immediate directly (Chapter 3).

Decode rows — OP, OP-IMM, LUI, AUIPC

| opcode.funct3.funct7 (17-bit casez key) | Row | alu | imm | aPC | bImm | rd |
| --- | --- | --- | --- | --- | --- | --- |
| 0110011.000.0000000 | ADD | ADD | — | — | — | 1 |
| 0110011.000.0100000 | SUB | SUB | — | — | — | 1 |
| 0110011.001.0000000 | SLL | SLL | — | — | — | 1 |
| 0110011.010.0000000 | SLT | SLT | — | — | — | 1 |
| 0110011.011.0000000 | SLTU | SLTU | — | — | — | 1 |
| 0110011.100.0000000 | XOR | XOR | — | — | — | 1 |
| 0110011.101.0000000 | SRL | SRL | — | — | — | 1 |
| 0110011.101.0100000 | SRA | SRA | — | — | — | 1 |
| 0110011.110.0000000 | OR | OR | — | — | — | 1 |
| 0110011.111.0000000 | AND | AND | — | — | — | 1 |
| 0010011.000.any | ADDI | ADD | I | — | 1 | 1 |
| 0010011.010.any | SLTI | SLT | I | — | 1 | 1 |
| 0010011.011.any | SLTIU | SLTU | I | — | 1 | 1 |
| 0010011.100.any | XORI | XOR | I | — | 1 | 1 |
| 0010011.110.any | ORI | OR | I | — | 1 | 1 |
| 0010011.111.any | ANDI | AND | I | — | 1 | 1 |
| 0010011.001.0000000 | SLLI | SLL | I | — | 1 | 1 |
| 0010011.101.0000000 | SRLI | SRL | I | — | 1 | 1 |
| 0010011.101.0100000 | SRAI | SRA | I | — | 1 | 1 |
| 0110111.any.any | LUI | ADD | U | — | 1 | 1 |
| 0010111.any.any | AUIPC | ADD | U | 1 | 1 | 1 |

## 13.2	Loads, stores, branches, jumps

Every branch/jump row uses alu = ADD so the ALU produces pc + imm for the next-PC mux while the dedicated comparator decides taken-ness in parallel. JALR is the one jump whose ALU left input is rs1, not PC.

Decode rows — LOAD, STORE, BRANCH, JAL, JALR

| opcode.funct3.funct7 (17-bit casez key) | Row | imm | aPC | bImm | dmem | ctl |
| --- | --- | --- | --- | --- | --- | --- |
| 0000011.000.any | LB | I | — | 1 | req, B | — |
| 0000011.001.any | LH | I | — | 1 | req, H | — |
| 0000011.010.any | LW | I | — | 1 | req, W | — |
| 0000011.100.any | LBU | I | — | 1 | req, B | — |
| 0000011.101.any | LHU | I | — | 1 | req, H | — |
| 0100011.000.any | SB | S | — | 1 | req+we, B | — |
| 0100011.001.any | SH | S | — | 1 | req+we, H | — |
| 0100011.010.any | SW | S | — | 1 | req+we, W | — |
| 1100011.000.any | BEQ | B | 1 | 1 | — | branch |
| 1100011.001.any | BNE | B | 1 | 1 | — | branch |
| 1100011.100.any | BLT | B | 1 | 1 | — | branch |
| 1100011.101.any | BGE | B | 1 | 1 | — | branch |
| 1100011.110.any | BLTU | B | 1 | 1 | — | branch |
| 1100011.111.any | BGEU | B | 1 | 1 | — | branch |
| 1101111.any.any | JAL | J | 1 | 1 | — | jump |
| 1100111.000.any | JALR | I | — | 1 | — | jalr |

## 13.3	System, fence, and the default

Decode rows — SYSTEM, MISC-MEM, and the illegal default

| opcode | funct3 | imm12 | Row | Behavior (how the row retires) | halt_cause |
| --- | --- | --- | --- | --- | --- |
| 1110011 | 000 | 000 | ECALL | halt; RVFI trap beat | 4'h1 |
| 1110011 | 000 | 001 | EBREAK | halt; RVFI trap beat | 4'h2 |
| 1110011 | 000 | 302 | MRET | retire as NOP, pc+4 fall-through | — |
| 1110011 | 000 | other | — | illegal | 4'hF |
| 1110011 | 001/010/011/101/110/111 | any | CSRRW/S/C + imm forms | csr_stub: zero readback, writes dropped | — |
| 1110011 | 100 | any | — | reserved, illegal | 4'hF |
| 0001111 | 000 | any | FENCE | retire as NOP | — |
| 0001111 | 001 | any | FENCE.I | retire as NOP | — |
| 0001111 | 010-111 | any | — | reserved, illegal | 4'hF |
| any unclaimed | — | — | default | illegal instruction | 4'hF |

## 13.4	What the table teaches

Three design rulings visible in the table deserve explicit mention:

- Branch funct3 010/011, load/store funct3 011 and 110/111, MISC-MEM funct3 2-7, SYSTEM funct3 100, shift funct7 mismatches: all illegal. RV32I leaves these encodings reserved (spec Volume I, sections 2.1.4 and 2.1.6); kestrel halts on them rather than guessing. A reserved encoding today may become an instruction tomorrow, and silently executing it is how compatibility bugs are born.

- The CSR class is one stub row-set, not six special cases. All six Zicsr forms share csr_stub = 1; rd_wen = 1 and nothing else; the writeback mux owns the zero. The table stays small because the stub is honest about there being nothing to decode.

- Cause 3 appears nowhere in this table. HALT_IALIGN is raised only in kestrel_core, which can see the resolved next PC and the branch decision. Decode provably cannot know — a branch’s target alignment depends on the register operands it never evaluates. Chapter 5 gives the condition.

Source: common-rv32i/rtl/fub/rv32i_decode.sv (the unique casez in full), common-rv32i/rtl/includes/rv32i_pkg.sv (HALT_* encodings), rtl/top/kestrel_core.sv (misaligned-target halt)

# 14	The Memory Contract

## 14.1	The interface

kestrel talks to memory through two ports that are deliberately minimal. There is no bus fabric, no wait states, no byte enables on the instruction side, and no burst machinery — at this rung the memory system is expected to keep up with the core, and the contract says so out loud.

The kestrel memory interface

| Port | Direction | Width | Contract |
| --- | --- | --- | --- |
| imem_addr | out | 32 | Instruction fetch address, driven by the PC |
| imem_rdata | in | 32 | Combinational read data: the word at imem_addr, valid in the same cycle |
| dmem_req | out | 1 | A load or store is accessing data memory this cycle |
| dmem_we | out | 1 | The access is a store |
| dmem_addr | out | 32 | Word-aligned byte address ({alu_y[31:2], 2'b00}) |
| dmem_wstrb | out | 4 | Per-byte write enables, rotated into position for stores |
| dmem_wdata | out | 32 | Store data, rotated into position |
| dmem_rdata | in | 32 | Combinational read data for loads |

Three properties define the contract:

- Harvard at the core. Instruction fetch and data access are separate ports backed by whatever the integrator provides. The core never issues a data access to the instruction memory or vice versa, and it never sees coherence between them.

- Combinational reads, or an explicit wait. Both memories answer in the same cycle the address is presented. This is the load-bearing simplification of the single-cycle design: an instruction’s full path — fetch, decode, register read, ALU, data read, writeback — must fit in one clock. A synchronous-read memory (block RAM) may instead hold the core with mem_wait, during which nothing in the core changes. kestrel_mem_bram does that for a load’s data read only: it fetches one clock early at the core’s imem_addr_next (the PC after the coming edge), so the RAM’s output register acts as the instruction register and every other instruction still takes one clock.

- Word-wide buses with byte strobes. Data memory is 32 bits wide; sub-word stores rotate the data and strobe into the addressed byte lanes, and sub-word loads rotate read data back before size/sign selection. The testbench memory honors dmem_wstrb per byte, which is the contract that pins the rotation convention (Chapter 6).

## 14.2	Timing: one cycle, except when documented

An instruction that touches memory completes in exactly one clock; a misaligned access halts instead (next section of this chapter). There are no other exceptions beyond the memory’s own mem_wait: no arbitration stalls, no refresh interference. The PC holds only for halts and while mem_wait is high. Everything else advances every cycle.

RESET_ADDR (default 32'h0000_0000, a module parameter) selects the fetch address out of reset; the testbench also parameterizes it to prove the core starts anywhere.

## 14.3	What the contract deliberately excludes

- No instruction-side data access. Self-modifying code is invisible to the core unless the memory system unifies the ports. The kestrel testbench does exactly that — one array serving both fetch and load/store — as a testbench choice so rv32ui-p-fence_i runs without a coherence path. That unification is documented here so no integrator mistakes it for a core feature.

- No misaligned instruction fetch. IALIGN=32: fetch addresses are always 4-byte aligned, and the low PC bits are guaranteed by the next-PC logic plus the halt of section 3 of this chapter.

- No memory protection, ordering, or atomicity. RVWMO (spec Volume I, chapter 3) is satisfied vacuously by a core that issues at most one access at a time and never reorders; there is nothing to model.

Source: rtl/top/kestrel_core.sv (port list, L/S datapath); dv/tb/kestrel_tb_top.sv (unified testbench memory, wstrb-honoring store port); falcon-suite RTL style brief

# 15	Misaligned Data Accesses

## 15.1	What the spec permits

RV32I does not require misaligned load/store support, but it explicitly permits it: an execution environment may handle misaligned accesses in hardware, invisibly trap and emulate them, or trap and refuse, and it may treat accesses to side-effecting regions differently (spec Volume I, section 2.1.6). Naturally aligned accesses are guaranteed atomic; misaligned accesses are explicitly not required to be atomic. The design space is open, and kestrel takes the plainest corner of it: a misaligned load or store halts the core. Trap-and-refuse is the spec’s own option; kestrel has no trap machinery, so, like every other trap, it halts (cause 4 for a load, 5 for a store).

This is a data policy. Instruction-side misalignment is never handled in hardware by anything, and section 3 of this chapter covers that halt.

## 15.2	The rotation model

Naturally aligned sub-word accesses are implemented by byte-lane rotation. For an access at byte offset o with size bytes s:

- Offset: o = rs1[1:0] + imm[1:0], the address’s low bits from their own two-bit add, so the rotation and the halt test do not wait for the 32-bit address.

- Store: dmem_wdata = rs2_data << (8*o) — the value slides into the addressed lanes; dmem_wstrb = size_mask << o — the byte enables slide with it. The memory merges enabled bytes into the word.

- Load: rdata_shifted = dmem_rdata >> (8*o) — the addressed bytes slide down to bit 0, and a final case implements LB (sign-extend from bit 7), LBU/LHU (zero-extend), LH, LW.

Bytes are always aligned. Halves are aligned at offsets 0 and 2, words at 0. Every aligned access fits inside one word, so the rotation is the entire mechanism and the access completes in the single cycle.

## 15.3	The misaligned halt

A half at an odd offset, or a word at any nonzero offset, raises ls_misalign. The access performs nothing: the store strobes are forced to zero, the load does not write rd, and the instruction retires as the usual RVFI trap beat with empty memory fields, then the core holds (section 3). rv32ui-p-ma_data, which needs hardware support, halts on its first misaligned load; spike, which traps misaligned accesses too, takes trap_load_address_misaligned at the same instruction, so the lockstep still lines up (Chapter 6).

## 15.4	Why not hardware?

Until 2026-10-20 kestrel did complete misaligned accesses in hardware: in-word ones by rotation, cross-word ones in two beats, with a retry flop, a 32-bit merge register for the first word’s bytes, and the PC held for the extra cycle. On block RAM that logic sat on the critical path — it gated the register write and the PC hold, between the address add and the register file — and it was most of kestrel’s control state. Dropping it leaves a rotation and one compare, which is the smaller and simpler design a rung-1 core should be. A program that needs misaligned access can split it into aligned byte or half accesses, as compilers already do.

Source: RISC-V Instruction Set Manual, Volume I, section 2.1.6; rtl/top/kestrel_core.sv (L/S rotation and the misaligned halt); dv/tests/programs/ls_misaligned.s, ls_misalign_store.s, ls_chain.s

# 16	IALIGN and the Misaligned-Target Halt

## 16.1	The rule

With IALIGN=32 — no compressed instructions — every instruction fetch must be 4-byte aligned. The spec (Volume I, section 2.1.5) therefore requires an instruction-address-misaligned exception when taken control flow — a taken branch, JAL, or JALR — targets an address whose low two bits are not 00. RISC-V’s permission to handle data misalignment in hardware does not extend here (kestrel halts on misaligned data anyway, section 2): executing at a misaligned instruction address is never an option.

## 16.2	Why decode cannot catch it

The condition needs the resolved next PC: for JALR it depends on a register value, and for every branch it depends on the taken decision. Decode sees neither — it produces the control bundle, not the data. kestrel’s rv32i_pkg comments state the ownership directly: causes 1, 2, and F belong to decode; causes 3-5 belong to the core. In kestrel_core.sv:

misalign_target = (jump | jalr | branch_taken) & (next_pc[1:0] != 2'b00);

— OR-ed into the halt path as HALT_IALIGN (4'h3), through the same holding register as every other halt: trap beat, writeback suppressed, PC frozen, cause on the port.

## 16.3	The writeback subtlety

JAL and JALR set decode’s rd_wen — they are link instructions, and a taken JAL/JALR that does align must write pc+4. The misaligned case must not. Rather than teaching decode about alignment, the core gates both the register write and the RVFI report with halt itself: rd_wen_eff = rd_wen & ~halt_q & ~mem_wait & ~misalign_link & ~ls_misalign (the halts an rd-writing JAL/JALR or load can raise; equal to gating with halt, without the branch comparator on the path) and rd_wb = rd_wen & (insn[11:7] != 0) & ~halt. The trap beat reports the misaligned target on rvfi_pc_wdata (it is next_pc, and next_pc is the misaligned address), which is exactly what a real trap’s state would record as the faulting target — a detail the formal model checks explicitly.

## 16.4	The six halt causes, in context

Halt causes and their spec counterparts

| Cause | Encoding | Raised in | Spec’s name for the event |
| --- | --- | --- | --- |
| ECALL | 4'h1 | decode | environment-call exception |
| EBREAK | 4'h2 | decode | breakpoint exception |
| misaligned control-flow target | 4'h3 | core | instruction-address-misaligned exception |
| misaligned load | 4'h4 | core | load-address-misaligned exception |
| misaligned store | 4'h5 | core | store/AMO-address-misaligned exception |
| illegal encoding | 4'hF | decode | illegal-instruction exception |

Every row is a case where RV32I mandates a trap and kestrel — having no trap machinery — substitutes a halt with the cause on a port. The RVFI trap beat keeps the event architecturally visible: riscv-formal’s checks require exactly this reporting, and the suite’s later rungs replace the halt with real trap delivery. This table is the seam.

## 16.5	The fence_i connection

rv32ui-p-fence_i is the one battery test whose program modifies its own instruction stream between fetches. The core’s Harvard contract says nothing about store-to-fetch visibility, so the testbench provides it: one unified memory array serves imem_rdata and dmem_rdata, and FENCE.I’s NOP retirement is sufficient because there is no cache to invalidate and no prefetch to refetch. The test passing is evidence of the testbench’s coherence choice, not of a core feature — and the book says so because the distinction matters the moment a real cache enters at rung 3.

Source: RISC-V Instruction Set Manual, Volume I, sections 1.5, 2.1.5; common-rv32i/rtl/includes/rv32i_pkg.sv (cause encoding and ownership comment), rtl/top/kestrel_core.sv (misalign_target, writeback gating), common-rv32i/rtl/fub/rv32i_decode.sv; task-9 report (the formal CEX that pinned this)

# 17	Testbench and Golden Model

## 17.1	The harness

kestrel’s testbench is deliberately thin — the core is the design under test and RVFI is the only observation channel it needs. kestrel_tb_top wraps the core in a behavioral 64 KiB word memory with combinational reads, loaded at time 0 from +imem=<hex> (and +dmem=<hex>) plusargs via $readmemh. The memory is unified: one array serves fetch and load/store, the testbench choice that lets rv32ui-p-fence_i’s self-modifying code work with no coherence path (Chapter 5). The store port honors dmem_wstrb per byte — the contract that pins the rotated-strobe convention.

Two additions serve the battery (Chapter 6’s next sections): a backdoor write port (tb_mem_we/tb_mem_addr/tb_mem_wdata, one word per cycle, deliberately not reset-gated) so all 42 vendor images stream through one Verilator build, and a tohost watch on the store port.

The cocotb side (kestrel_tb.py) owns the clock/reset choreography and the RVFI capture, and it enforces a sampling contract that took one real bug to learn: reset is released right after a posedge and beats are sampled on falling edges. Post-edge release plus negedge sampling captures every retiring instruction exactly once — including the first RESET_ADDR beat, which lives only in the partial cycle before the first posedge and was entirely missed by the naive post-edge sampling. The halting cycle is sampled too: the trap beat (rvfi_valid=1, rvfi_trap=1) is recorded, after which rvfi_valid stays low.

After halt, the TB samples four more cycles pinning halt-hold: halt stays raised, rvfi_valid stays low, rvfi_pc_rdata frozen.

## 17.2	The golden interpreter

rv32i_interpreter.py is a 476-line Python model of RV32I that emits the same RVFI beat format as the core — one record per retired instruction, none for anything that doesn’t retire, plus the halt state. Its memory model mirrors the core’s conventions exactly: word-based, rotated-strobe stores, the cause-4/5 halt on a misaligned load/store, the CSR stub’s zero writeback, MRET fall-through, FENCE NOPs, and the cause-3 halt with the misaligned target on pc_wdata. When the RTL and the model disagree, the diff is a named field at a named beat at a named PC — minutes to localize, not a waveform archeology session.

The diff is total: every sampled beat is compared field by field across the entire RVFI bundle — pc, insn, order, trap, rs addresses and data, rd address and data, and all four memory channels. There is no sampled subset and no tolerance.

## 17.3	Programs and the hex pipeline

Hand-written assembly programs (focus, rv32ui_ops, branch/jump vectors, load/store vectors, system tests) are built by a small script:

as -> objdump (provenance) -> objcopy -O verilog -> normalize_hex.py -> <name>.hex

objcopy -O verilog emits bytes with byte-addressed @ records, which the testbench’s word arrays cannot load directly (an @80000000 record would index out of bounds), so normalize_hex.py packs little-endian words and emits sparse word-indexed records, with a --base option that subtracts the battery images’ 0x80000000 link address. Vendor riscv-tests hexes go through the same normalizer.

## 17.4	The run grammar

Every test runs at two Verilator build levels through the rds-dv framework, sources always from the filelists:

make -C projects/components/riscv-ip/kestrel-rv32i/dv/tests run-<testroot>-<gate|func>[-serial]

gate is the fast smoke level; func runs the full matrix. The battery, lockstep, and formal layers of the next sections all sit on top of this harness and reuse its contracts rather than re-deriving them.

Source: dv/tb/kestrel_tb_top.sv, dv/tbclasses/kestrel/kestrel_tb.py, dv/tbclasses/kestrel/rv32i_interpreter.py, dv/tests/programs/build_progs.sh, dv/tests/programs/normalize_hex.py; task-5 report (sampling contract, fail-first evidence)

# 18	The rv32ui Battery

## 18.1	What the battery is

The rv32ui-p-* programs from the riscv-tests suite are the classic RISC-V architectural conformance battery: 42 self-checking RV32I tests, each linking the same p-environment preamble and ending in RVTEST_PASS/RVTEST_FAIL. kestrel runs all 42 at func level in a single Verilator build — the runner asserts reset, backdoor-loads the normalized image, releases reset, runs to halt, and scores — and three of them (simple, add, addi) again at gate as a smoke level. The summary table is the record (beats = RVFI retire beats; records = spike commit-log records, lockstep-checked as section 3 describes):

rv32ui battery at func level — 42/42 with lockstep; every test halts on ecall (cause 1) with gp = 1, except ma_data, whose expected halt is the misaligned load (cause 4)

| Test (rv32ui-p-*) | Beats (RVFI retire) | Spike records (commit log) | Result (battery verdict) |
| --- | --- | --- | --- |
| add | 500 | 5507 | PASS / lockstep ok |
| addi | 277 | 5284 | PASS / lockstep ok |
| and | 520 | 5527 | PASS / lockstep ok |
| andi | 233 | 5240 | PASS / lockstep ok |
| auipc | 93 | 5100 | PASS / lockstep ok |
| beq | 326 | 5333 | PASS / lockstep ok |
| bge | 344 | 5351 | PASS / lockstep ok |
| bgeu | 369 | 5376 | PASS / lockstep ok |
| blt | 326 | 5333 | PASS / lockstep ok |
| bltu | 351 | 5358 | PASS / lockstep ok |
| bne | 326 | 5333 | PASS / lockstep ok |
| fence_i | 331 | 5338 | PASS / lockstep ok |
| jal | 90 | 5097 | PASS / lockstep ok |
| jalr | 150 | 5157 | PASS / lockstep ok |
| lb | 288 | 5295 | PASS / lockstep ok |
| lbu | 288 | 5295 | PASS / lockstep ok |
| ld_st | 998 | 6005 | PASS / lockstep ok |
| lh | 304 | 5311 | PASS / lockstep ok |
| lhu | 313 | 5320 | PASS / lockstep ok |
| lui | 100 | 5107 | PASS / lockstep ok |
| lw | 318 | 5325 | PASS / lockstep ok |
| ma_data | 76 | 5083 | PASS (expected halt: cause 4 at the first misaligned load) / lockstep ok |
| or | 523 | 5530 | PASS / lockstep ok |
| ori | 240 | 5247 | PASS / lockstep ok |
| sb | 489 | 5496 | PASS / lockstep ok |
| sh | 542 | 5549 | PASS / lockstep ok |
| simple | 76 | 5083 | PASS / lockstep ok |
| sll | 528 | 5535 | PASS / lockstep ok |
| slli | 276 | 5283 | PASS / lockstep ok |
| slt | 494 | 5501 | PASS / lockstep ok |
| slti | 272 | 5279 | PASS / lockstep ok |
| sltiu | 272 | 5279 | PASS / lockstep ok |
| sltu | 494 | 5501 | PASS / lockstep ok |
| sra | 547 | 5554 | PASS / lockstep ok |
| srai | 291 | 5298 | PASS / lockstep ok |
| srl | 541 | 5548 | PASS / lockstep ok |
| srli | 285 | 5292 | PASS / lockstep ok |
| st_ld | 518 | 5525 | PASS / lockstep ok |
| sub | 492 | 5499 | PASS / lockstep ok |
| sw | 549 | 5556 | PASS / lockstep ok |
| xor | 522 | 5529 | PASS / lockstep ok |
| xori | 242 | 5249 | PASS / lockstep ok |

Gate level reruns simple, add, and addi — 3/3 pass, lockstep not applicable at that level by design.

## 18.2	The verdict protocol

The p-environment reports through a tohost mailbox: RVTEST_PASS stores 1 to tohost (via the trap-vector path, since a real core traps the terminating ecall), and the fesvr front end converts a tohost write into the emulator’s exit code. kestrel works differently, and the difference is deliberate:

- The core halts on the terminating ecall (cause 1) by design, before any trap-vector sw gp, tohost can execute. The DUT never writes tohost — so the store port is watched for one anyway (the run contract is run-to-halt, tohost-write, or timeout, whichever first).

- The verdict is gp’s last RVFI writeback — x3 is exactly the register the p-environment would have stored, so its final value is the same information: 1 means pass, an odd value greater than 1 is a fail code.

- tohost is discovered per image, not hardcoded: a pure-Python ELF32 symbol reader finds it at 0x80001000 in 41 images and 0x80002000 in ld_st (its larger data section pushes .tohost past the usual address); fromhost follows at tohost + 0x40.

- Spike’s exit code independently confirms tohost == 1 for every test on the lockstep side (next section), so the battery’s verdict does not rest on its own convention alone.

## 18.3	What the battery proves — and its seam

Forty-two self-checking programs with a second emulator agreeing on every retired instruction is strong evidence of ISA conformance. Its seam is the p-environment preamble: every test runs the same ~12 CSR instructions and an MRET before its body, which is why the bounded CSR stub of Chapter 2 exists at all. And as Chapter 6’s final section records, forty-two directed tests still missed a real core bug that formal methods found in an afternoon — the battery tells you the programs you thought of pass; it says nothing about the ones you didn’t.

Source: task-8 report (battery table, tohost map, verdict protocol); dv/tbclasses/kestrel/rv32ui_battery.py, dv/tests/test_kestrel_rv32ui.py; logs/rv32ui_battery/rv32ui_battery_summary_func.txt

# 19	Spike Lockstep

## 19.1	What lockstep adds

The golden interpreter checks kestrel against kestrel’s own understanding of RV32I — same author, same conventions, same blind spots. Spike, the RISC-V emulator, is an independent implementation: different code base, different authors, years of field use. Lockstepping the two means an disagreement is almost certainly kestrel’s bug, and the suite pins spike 1.1.0 for reproducibility. Each battery test runs spike on the same ELF and the runner walks the two retirement streams together.

## 19.2	The mechanics

Spike is invoked as spike --isa=RV32I -l --log=<file> <elf> — the -l must accompany --log; --log alone redirects a disabled log and produces an empty file. This pinned build’s commit line carries pc and instruction encoding only:

core   0: 0x0000000080000000 (0x00000297) auipc   t0, 0x0

There are no register-write or memory-access payloads in this build (checked against the source: processor_t::disasm prints pc, insn, and disassembly only). Consequently the lockstep is layered: the golden interpreter’s full-field beat diff owns rd/rs/mem correctness, and spike owns the (pc, insn) retirement stream, the exception records, and the exit code — an independent check that the right instructions executed in the right order. Four format details are pinned by investigation:

- Boot prefix. fesvr’s boot ROM runs 5 instructions at 0x1000 before jumping to the ELF entry; the diff skips records until the first commit at e_entry.

- Executed N times lines. Spike collapses back-to-back identical (pc, insn) commits; the parser expands them before the walk.

- Exception records. core   0: exception <kind>, epc 0x<pc> (plus a tval line) interleave with commits; the trapping instruction still gets its own commit line first. The walk verifies each such record against the just-matched core beat and spike’s resume pc.

- Termination. The core’s final beat is the ecall trap beat; spike continues with the ecall commit and trap_user_ecall at the halt PC, then the p-environment’s write_tohost tail (length varies per run — not diffed). The exit code is the verdict.

## 19.3	ma_data, an expected halt

kestrel halts on a misaligned load or store (Chapter 5), and so does the pinned spike 1.1.0 in its own way: without the compile-time RISCV_ENABLE_MISALIGNED it traps them. On rv32ui-p-ma_data both stop at the same instruction, the first misaligned load: kestrel with cause 4, spike with trap_load_address_misaligned (and exit code 156, since the test never reaches its pass path). The battery lists ma_data as an expected halt: it passes when kestrel halts with cause 4 and the lockstep ends on that spike trap at the halt PC. Until 2026-10-20, when kestrel completed misaligned accesses in hardware, the battery used a side-by-side --enable-misaligned rebuild of the same spike commit instead.

## 19.4	The mnstatus divergence

Every test’s p-environment preamble executes csrwi mnstatus, 8 — a Smrnmi CSR, illegal in RV32I per spike’s default extension set. Spike traps it and resumes at mtvec, which the p-environment points at the next instruction; kestrel’s CSR stub simply retires it (Chapter 2). The 1:1 stream walk stays aligned because spike logs the trapping instruction’s commit, the exception record is verified against the just-matched core beat and spike’s resume pc, and the event is counted and reported as 1 stub trap(s) [trap_illegal_instruction] per test. Any other mid-stream exception is a hard failure — none ever occurred.

Source: task-8 report (spike format notes 1-8, battery table); dv/tbclasses/kestrel/rv32ui_battery.py (spike runner, log parser, lockstep_diff)

# 20	The riscv-formal Flow

## 20.1	What it is

riscv-formal is the open-source RISC-V formal-verification framework: per-instruction specification models written as Verilog (a BEQ model, an LW model, …), paired with checkers that relate a core’s RVFI beats to the model’s expected behavior on every cycle, discharged by bounded model checking. Where simulation shows the programs you thought of behave, riscv-formal makes claims about every stimulus up to a proof depth — including the perverse ones no test author would write.

## 20.2	kestrel’s setup

formal/kestrel/riscv-formal/ holds a thin rvfi_wrapper around kestrel_core in the shape riscv-formal’s reference cores use (picorv32’s), a checks.cfg, and a Makefile driving genchecks and sby. The wrapper is worth understanding because it is small:

- RISCV_FORMAL_ALIGNED_MEM is defined (2026-10-20): memory is reported as a word address with byte-lane masks, and the instruction models require rvfi_trap on every misaligned load and store, which is kestrel’s halt.

- Memory inputs (imem_rdata, dmem_rdata) are free rvformal_rand_reg values — there is no memory array. The checkers relate the core’s reported rvfi_mem channels and writeback to the ISA model, so random read data is exactly the right stimulus, and no imem-stability assumption is needed even under free mem_wait stalls.

- The core lacks rvfi_halt/rvfi_intr/rvfi_mode/rvfi_ixl ports, so the wrapper generates them: rvfi_halt = rvfi_valid & halt (for a core with no trap handler, the Task-8 trap beat is the final retirement — precisely rvfi_halt semantics), rvfi_intr = 0, rvfi_mode = M, rvfi_ixl = 32. RVFI_CONN is not used.

- RESET_ADDR stays at the core default: a parameter is an elaboration constant and cannot be made symbolic without touching RTL; the checks are address-agnostic, so no proof is weakened.

## 20.3	The checks and depths

genchecks generates 42 checks from isa_rv32i.txt: the 37 instruction models plus five consistency checks — reg (register file integrity), pc_fwd/pc_bwd (PC monotonicity both directions), liveness (something eventually retires), and unique (no duplicate retirement orders). This pinned genchecks emits no mem/causal/ill/hang/ cover checks; load/store semantics are covered by the instruction models plus reg. Depths: instruction and consistency checks at 6 — enough for reset plus a few memory wait states with headroom — with liveness/unique at trigger 6, check 30 per the picorv32 configuration. Solver: z3 via smtbmc.

## 20.4	The flow’s three honest workarounds

None weakens a proof; all are recorded because the next rung inherits the flow:

- sv2v pre-conversion. This yosys (0.62+95) cannot parse the RTL’s module-body import rv32i_pkg::*;, so the Makefile converts the RTL into plain Verilog before genchecks runs; the wrapper is still read as SystemVerilog.

- const rand reg workaround. The same yosys rejects const rand reg (used by five consistency checkers), and this build has no system command to patch copies at run time. The Makefile therefore builds the checks directory as a symlink farm around a sed-patched copy of rvfi_macros.vh (const rand reg → (* anyconst *) reg, verified semantically identical); the vendor tree is untouched.

- genchecks’ working-directory contract. This pin’s genchecks reads its config from the current directory and requires cwd to sit exactly two levels below the riscv-formal root; the Makefile runs it from formal/kestrel/riscv-formal accordingly.

## 20.5	Results

The first clean build proved the flow end to end and returned 34/42 passing — the eight failures all counterexamples of one genuine core bug, told in the next section. After the fix, a full rebuild reports 42/42 PASS (z3/smtbmc) — and the pre-fix 34/42 failure had itself reproduced identically across three full runs including one from make clean, so the green verdict is stable state, not a lucky run:

export PATH=/mnt/data/tools/oss-cad-suite/bin:/tmp/sv2v/sv2v-Linux:$PATH
make -C formal/kestrel/riscv-formal checks   # generate + run all 42 sby tasks

Per-check evidence lives in checks/<check>/status (PASS/FAIL with step counts) plus logs and VCD counterexamples under checks/<check>/.

Source: task-9 report and fix round 1 (check list, wrapper notes, flow deviations, depths, results); formal/kestrel/riscv-formal/ (formal_kestrel.sv, checks.cfg, Makefile)

# 21	The CEX Story: What Formal Caught That Forty-Two Tests Missed

This section is the verification chapter’s lesson, and it deserves its own space: the riscv-formal counterexamples found a real, architecturally visible core bug that the entire directed battery, two lockstep methods, and code review had all missed. The suite keeps this story on record because it is the strongest argument for the ladder’s verification doctrine — simulation shows what you thought of; proof covers what you didn’t.

## 21.1	The failure

The first full formal run returned 34/42 PASS, 8 FAIL — the six branch models plus jal and jalr, every one counterexampled the same way at the same line: rvfi_insn_check.sv:178, assert(spec_trap == trap), at check step 6. The evidence chain, shown for insn_beq_ch0, is identical for all eight:

- sby: failed assertion ... at rvfi_insn_check.sv:178.6 step 6.

- That line compares the model’s expected trap against the core’s reported rvfi_trap. The checker assumed a valid BEQ beat; the ISA model says it traps; the core reported no trap.

- The VCD at the check step shows spec_valid=1, spec_trap=1,    rvfi_trap=0, with retired instruction 0x980001E3 — a beq x0, x0 (opcode 1100011, funct3 000), taken by construction, whose B-immediate targets 0xFFFFF996: an address with next_pc[1:0] = 2. (The sibling checks carry their own stimuli — insn_bne_ch0’s trace, for instance, retires 0x80121163, a taken bne to another misaligned target.)

- The model source (vendor/riscv-formal/insns/insn_beq.v) computes spec_trap = (next_pc[1:0] != 0) || !misa_ok for IALIGN=32: any taken control transfer to a non-4-aligned target must trap. The JAL and JALR models carry the identical condition.

- rvfi.md is unambiguous: “rvfi_trap must also be set for … a jump instruction that jumps to a misaligned instruction.”

- The spec (Volume I, section 2.1.5) names the event: instruction-address-misaligned.

kestrel at the time silently jumped: pc <= pc + imm (or the JALR target masked to ~1) with rvfi_trap = 0, then fetched and executed at the misaligned address. A taken branch to address 0x2 executed garbage as if nothing were wrong.

## 21.2	Why the battery missed it

Forty-two directed tests, a golden interpreter, and a second emulator all passed — because not one of them ever targeted a misaligned instruction address. The rv32ui programs are linked code; every branch and jump target is assembler-controlled and 4-byte aligned by construction. The bug lived exactly where directed tests cannot reach: in the space of operands and targets no test author writes. Formal verification’s contribution is not diligence but coverage of the absurd: the model checker gleefully retires BEQ with every immediate, including the ones that land on next_pc[1:0] != 00, and it found the hole on depth 6 of an unconstrained search.

## 21.3	The fix

The fix is small, follows the core’s existing idiom, and is the HALT_IALIGN material of Chapter 5: halt_cause encoding moves into rv32i_pkg as the single source of truth with the new HALT_IALIGN = 4'h3; kestrel_core computes misalign_target = (jump | jalr | branch_taken) & (next_pc[1:0] != 2'b00) and OR-s it into the existing halt path — same holding register, same trap beat, same PC freeze. The one delicate point is the link register: decode keeps rd_wen = 1 for JAL/JALR, so both the register write and the RVFI report are gated by halt itself, not by decode’s rd_wen. The golden interpreter models the same halt (cause 3, pc_wdata equal to the misaligned target), and a directed program — system_misalign_jmp — pins it: an aligned JAL regression (link value pc+4 retires normally), then jal x0, .+2 must halt with cause 3, trap beat pc_wdata = 0x12, no register write.

## 21.4	Aftermath

Re-run against the patched RTL: riscv-formal 42/42 PASS, the rv32ui battery 42/42 PASS with spike lockstep intact (no rv32ui test targets a misaligned PC, so the battery stayed green as predicted), and every pre-existing regression — core, branch, load/store, decode suites at gate and func — green. The task’s mandated commit message reported the intermediate 34/42 honestly rather than claiming green; the fix round is the commit that earns the claim.

## 21.5	The lesson, stated once

Each method in this chapter sees a different slice of behavior. The golden interpreter sees the same conventions the RTL was written with. Spike sees the same programs, independently implemented. The battery sees real compiled code. riscv-formal sees everything the ISA permits up to a depth — and at this core’s scale (depth 6, minutes per check) that “everything” is cheap enough to run on every RTL change. The ladder’s doctrine follows directly: directed tests for the common case, lockstep for independence, and formal proofs as the gate that keeps honest optimizations honest.

Source: task-9 report (counterexample chain, proposed fix) and fix round 1 (fix, directed test, re-run evidence); common-rv32i/rtl/includes/rv32i_pkg.sv, rtl/top/kestrel_core.sv; dv/tests/programs/system_misalign_jmp.s

# 22	What kestrel Costs

## 22.1	CPI 1, honestly stated

kestrel retires one instruction per clock — CPI 1 — for every instruction in the ISA. Two honesty clauses attach to that claim. First, behind block RAM a load occupies two clocks (the memory holds the core while its data read lands), so the average is 1 plus the load fraction; with combinational memories it is exactly 1. Second, CPI 1 is per instruction, and instructions are not the unit anyone asked for — programs are. The single-cycle discipline taxes the fast instructions to pay for the slow ones, and the tax collector is the clock period.

## 22.2	The critical path is the whole machine

In a single-cycle design, the clock period must span the slowest instruction’s path through every block, because every block sits between the same two edges. Trace kestrel’s worst case, an ALU instruction, and count what the clock must cover:

- The PC register updates and presents imem_addr.

- The instruction memory reads combinationally; the instruction word settles.

- Decode ({opcode, funct3, funct7} case) and the immediate generator settle in parallel with the register file’s combinational reads.

- The source muxes feed the ALU; the ALU result settles. (For a load, the address then crosses the data memory, and the read data rotates and sign-extends into the writeback mux.)

- The next-PC mux selects; for branches, the dedicated comparator’s decision must also settle to drive that selection.

- The writeback data reaches the register file’s data input and the PC data reaches the PC register, both meeting setup before the edge.

Every one of those stages — two memories, a truth table, a register file, an ALU, three muxes, a comparator — is on the critical path for some instruction, so the clock is limited by their sum, not their slowest member. The ADD that could have run at the ALU’s speed is clocked at the LW’s speed, and both are clocked below what a leaner machine could do. Patterson and Séquin’s RISC I made this trade deliberately in 1981: one instruction, one cycle, clock be damned, because a machine whose every instruction completes in one cycle is a machine you can reason about completely (papers, entry 1). Kestrel inherits the reasoning and, this chapter argues, should not inherit the clock.

## 22.3	The accounting, instruction by instruction

Critical-path content per instruction class

| Instruction class | Path the clock must span |
| --- | --- |
| ALU (OP/OP-IMM) | imem + decode + regfile + ALU + writeback setup |
| Branch | imem + decode + regfile + comparator + next-PC mux + PC setup |
| Load | imem + decode + regfile + ALU + dmem + rotate/sign + writeback setup |
| Store | imem + decode + regfile + ALU + dmem + strobe setup |
| JAL/JALR | imem + decode + regfile (JALR) + add + next-PC mux + PC setup |

The table’s lesson: there is no clock frequency at which this machine is fast, only a frequency at which it is correct, and that frequency is set by the longest row — almost always the load, which crosses both memories and the datapath between them.

## 22.4	What would have to give

The single-cycle tax has exactly one root cause: every stage serves every instruction every cycle. Relax that and the frequency rises, but the reasoning burden rises with it — which is the trade the ladder is built to teach. The moment work is spread across multiple cycles, the stages need boundaries (pipeline registers), the boundaries create overlaps (hazards), and the hazards need policy (forwarding, stalls, flushes). That is not a defect of the next design; it is the price of the frequency, and merlin exists to teach how to pay it deliberately.

Source: rtl/top/kestrel_core.sv (datapath structure); Patterson and Séquin, “RISC I” (ISCA 1981) and Hennessy et al., “MIPS: A Microprocessor Architecture” (MICRO 1982) — papers entries 1 and 3; Flynn, “Very High-Speed Computing Systems” (1966) for the throughput-versus-clock framing (papers, entry 2)

# 23	The Next Rung: merlin

## 23.1	The five-stage answer

merlin, rung 2 of the falcon suite, keeps kestrel’s ISA bit-for-bit and changes the time dimension: instead of one cycle per instruction, an instruction flows through five stages — IF, ID, EX, MEM, WB — with pipeline registers between them, so five instructions are in flight at once and the clock only has to span one stage. The same ADD that paid for the load’s path in kestrel pays for the ALU’s alone in merlin; that is the entire economic argument for pipelining, and it is also where every new problem comes from.

### 23.1.1	Figure 7.1: kestrel’s cycle, cut into merlin’s stages

From one kestrel cycle to merlin’s five stages

The figure draws the correspondence explicitly: kestrel’s combinational cloud between two PC edges is merlin’s five stage boundaries. Nothing new is computed at rung 2; the computation is the same — it is overlapped.

## 23.2	What appears when the cycle is cut

Each new mechanism is the direct consequence of one cut:

- Pipeline registers (IF/ID, ID/EX, EX/MEM, MEM/WB). The boundary between stages becomes state, which is the first real state kestrel’s control has ever had.

- RAW hazards and forwarding. An instruction in EX may need the result of the instruction ahead of it, still in MEM or WB. merlin forwards EX/MEM and MEM/WB results back into EX with an explicit priority — the forwarding matrix is Chapter 1 of merlin’s book.

- Load-use interlock. A load’s data isn’t available until the end of MEM, one cycle too late for the dependent instruction in EX. The interlock stalls one cycle — the first stall the suite has ever had.

- Control hazards. Branches resolve in EX under merlin’s predict-not-taken policy; a taken branch flushes the two fetched instructions behind it. Kestrel’s taken-branch timing freedom (one cycle either way) becomes a two-instruction penalty and a flush path.

- The same verification, adapted. The RVFI aggregation that kestrel drives combinationally per cycle becomes merlin’s retirement logic, and the riscv-formal checks are adapted to a retire interface; the battery, the lockstep harness, and the golden interpreter carry over because the ISA does not change.

## 23.3	What carries over unchanged

The investment kestrel already made transfers directly: the decode truth table (Chapter 4) is still the complete specification of what each instruction does — merlin just evaluates it in different stages; the register file and ALU leaves are reused as-is; the memory contract’s word-wide, byte-strobed data port is unchanged; the halt causes survive as the core’s stop contract; and the documentation style — decisions stated plainly, spec citations by chapter — is the suite’s doctrine, not kestrel’s habit. Even the CSR stub carries forward unchanged: merlin has no more CSR state than kestrel.

## 23.4	Why the ladder stops here first

kestrel is complete on its own terms — a verified RV32I core with a documented contract and a proof-backed verification record. What it is not is fast, and it cannot be made fast without becoming something else. The suite’s argument is that “something else” is best understood as a sequence of one-concept-at-a-time changes, each motivated by a cost the previous rung made visible. Chapter 7’s cost accounting is the motivation; merlin is the first payment. When its book exists, it will cite this one the same way this one cites the specification — by chapter, with the seams labeled.

Source: falcon-suite design spec, “Rung 2 — merlin”; Hennessy et al., “MIPS” (papers, entry 3) for the stage partition; Flynn (papers, entry 2) for throughput versus latency