RTL Design Sherpa

Bridge Hardware Architecture Specification 1.5

October 18, 2026

Table of Contents

1 Bridge Has Index	20

2 Document Information	21

2.1 Bridge Hardware Architecture Specification	21

2.2 Revision History	22

2.3 Document Purpose	31

2.4 Intended Audience	32

2.5 Related Documents	33

3 Purpose and Scope	34

3.1 What is Bridge?	34

3.2 The Problem Bridge Solves	35

3.3 Scope	36

3.4 Out of Scope	37

4 Document Conventions	38

4.1 Terminology	38

4.2 Signal Naming	39

4.3 Diagrams	40

4.3.1 Figure 1.1: Block Diagram Legend	40

4.4 Code Examples	41

4.5 Parameter Notation	42

5 Definitions and Acronyms	43

5.1 Acronyms	43

5.2 Definitions	45

6 Use Cases	46

6.1 Primary Applications	46

6.1.1 SoC Interconnects	46

6.1.2 Memory Systems	46

6.1.3 Accelerator Systems	46

6.2 Example: RAPIDS Accelerator	47

6.2.1 Figure 2.1: RAPIDS System Topology	47

6.2.2 Configuration	47

6.2.3 Benefits	47

6.3 Rapid Prototyping	49

7 Key Features	50

7.1 Configuration-Driven Generation	50

7.1.1 CSV/TOML Configuration	50

7.1.2 Example Configuration	50

7.2 Multi-Protocol Support	51

7.2.1 Protocol Features	52

7.3 Channel-Specific Masters	53

7.3.1 Write-Only Masters (wr)	53

7.3.2 Read-Only Masters (rd)	53

7.3.3 Full Masters (rw)	53

7.3.4 Resource Savings	53

7.4 Automatic Converters	54

7.4.1 Width Conversion	54

7.4.2 Protocol Conversion	54

7.5 Flexible Topology	55

7.5.1 Scalability	55

7.5.2 Mixed Configurations	55

8 System Context	56

8.1 Bridge in SoC Architecture	56

8.1.1 Figure 2.2: Bridge System Context	56

8.2 Interface Boundaries	57

8.2.1 Master-Side (Upstream)	57

8.2.2 Slave-Side (Downstream)	57

8.3 Address Space	58

8.3.1 Address Map Organization	58

8.3.2 Address Decode	58

8.4 Clock and Reset	59

8.4.1 Clock Domain	59

8.4.2 Reset	59

8.5 Dependencies	60

8.5.1 Required Infrastructure	60

8.5.2 Optional Features	60

8.6 References	61

9 Block Diagram	62

9.1 Bridge Architecture Overview	62

9.1.1 Figure 3.1: Bridge Block Diagram	62

9.2 Functional Blocks	63

9.2.1 Master Adapter	63

9.2.2 Crossbar Core	63

9.2.3 Slave Adapter	63

9.3 Data Flow	64

9.3.1 Figure 3.2: Write Path	64

9.3.2 Figure 3.3: Read Path	64

9.4 Scalability	65

9.4.1 Topology Parameters	65

9.4.2 Resource Scaling	65

9.5 References	66

10 Data Flow	67

10.1 Transaction Flow Overview	67

10.1.1 Write Transaction Flow	67

10.1.2 Read Transaction Flow	67

10.2 Channel Independence	68

10.2.1 AXI4 Channel Separation	68

10.2.2 Parallel Operation	68

10.3 Bridge ID and ID Widening	69

10.3.1 Bridge ID (BID) Concept	69

10.3.2 IDs are widened inside the fabric	69

10.4 Response Routing	70

10.4.1 B Channel (Write Response)	70

10.4.2 R Channel (Read Data)	71

10.4.3 The requirement the FIFO form creates	71

10.5 Ordering: what the fabric actually guarantees	72

10.6 References	73

11 Protocol Support	74

11.1 Supported Protocols	74

11.1.1 AXI4 Full	74

11.1.2 AXI4-Lite	74

11.1.3 AXI3	74

11.1.4 APB (Advanced Peripheral Bus)	75

11.2 Protocol Conversion Matrix	76

11.3 Automatic Conversion Shims at Slave Boundary	78

11.4 Automatic Conversion at the Master Boundary	79

11.5 AXI4 to APB Conversion	81

11.5.1 Conversion Process	81

11.5.2 Figure 3.4: AXI4 to APB Conversion Sequence	81

11.5.3 Burst Handling	81

11.5.4 Timing Considerations	81

11.6 Width Conversion	82

11.6.1 Upsize (Narrow to Wide)	82

11.6.2 Figure 3.5: Width Upsize Conversion	82

11.6.3 Downsize (Wide to Narrow)	83

11.6.4 Figure 3.6: Width Downsize Conversion	83

11.7 Channel-Specific Protocol	84

11.7.1 Write-Only Masters	84

11.7.2 Read-Only Masters	84

11.8 ACE and Coherency — Explicitly Out of Scope	85

11.9 References	86

12 AXI4 Interface Overview	87

12.1 Interface Organization	87

12.1.1 Master-Side Interfaces	87

12.1.2 Slave-Side Interfaces	87

12.2 Signal Naming Convention	89

12.2.1 Custom Prefixes	89

12.2.2 ID Width Extension	89

12.3 Channel-Specific Interfaces	90

12.3.1 Write-Only Master (wr)	90

12.3.2 Read-Only Master (rd)	90

12.4 Monitor Bus Output (when variants includes mon)	91

12.5 Handshake Protocol	92

12.5.1 Valid/Ready Handshake	92

12.5.2 Backpressure	92

12.6 Burst Support	93

12.6.1 Supported Burst Types	93

12.6.2 Burst Length	93

13 APB Interface	94

13.1 Overview	94

13.2 Ports	95

13.2.1 Signal Definition	95

13.2.2 Signal Naming	95

13.3 Functional Description	96

13.3.1 AXI4 to APB Conversion	96

13.3.2 Burst Handling	96

13.3.3 Error Mapping	96

13.4 Timing	97

13.5 Waveforms	98

13.5.1 Write Transaction	98

13.5.2 Read Transaction	99

13.6 APB Requester Ports	100

13.7 Design Notes	101

14 Clock and Reset	102

14.1 Overview	102

14.2 Ports	103

14.2.1 Clock	103

14.2.2 Reset	103

14.3 Functional Description	104

14.3.1 One Fabric Clock	104

14.3.2 Clock Domain Crossing: CDC Slave Ports	104

14.3.3 Clock Domain Crossing: CDC Master Ports	105

14.3.4 Reset Behavior	107

14.3.5 Reset Effects	107

14.3.6 Outstanding Transactions	107

14.3.7 Reset Release	107

14.4 Timing	108

14.4.1 Setup and Hold	108

14.4.2 Clock-to-Output	108

14.5 Usage Example	109

14.5.1 Recommended Constraints	109

14.6 Design Notes	110

14.6.1 External CDC Required	110

14.6.2 CDC Recommendations	110

15 AXI5 and APB5 Interfaces (AMBA5 Support)	111

15.1 Overview	111

15.2 Parameters	112

15.2.1 Declaring AMBA5 Ports	112

15.3 Ports	113

15.3.1 AXI5 Port Surface	113

15.3.2 APB5 Slave Surface	114

15.3.3 AXI5-Lite Master Surface	114

15.3.4 APB5 Master Surface	115

15.3.5 AXI5-Lite Slave Surface	116

15.4 Functional Description	117

15.4.1 Interop Matrix	119

16 Unmapped-Address Handling	120

16.1 Overview	120

16.1.1 Why the bridge answers an address no slave owns	120

16.2 Ports	121

16.2.1 Status, interrupt and clearing	121

16.3 Functional Description	122

16.3.1 Behaviour	122

16.3.2 Register access (cfg regblock builds)	122

16.3.3 When it can never fire	123

16.3.4 What this does not do	123

16.4 Navigation	124

17 Wishbone B4 Interface	125

17.1 Overview	125

17.2 Ports	126

17.2.1 Signal Definition	126

17.2.2 Signal Naming	127

17.3 Functional Description	128

17.3.1 Slave port: AXI4 to Wishbone	128

17.3.2 Master port: Wishbone to AXI4	128

17.3.3 Rules the validator enforces	128

17.4 Design Notes	129

18 Slave Watchdog	130

18.1 Overview	130

18.2 Ports	131

18.3 Functional Description	132

18.3.1 When it trips	132

18.3.2 What it does then	132

18.3.3 Measured	132

18.4 Design Notes	133

19 Throughput Characteristics	134

19.1 Overview	134

19.2 Functional Description	135

19.2.1 Peak Throughput	135

19.2.2 Formula	135

19.2.3 Measured (2026-09-11)	135

19.2.4 On Silicon: the Genesys 2 Fabric Benchmark (2026-10-16)	136

19.2.5 Factors Affecting Throughput	137

19.2.6 Width-Converted Paths (measured)	138

19.2.7 Latency Against Offered Load (measured)	138

19.2.8 Bandwidth Characterization (measured)	139

19.2.9 Multi-Master Scaling	143

19.2.10 Burst Efficiency	143

19.2.11 Width Conversion Impact	143

19.2.12 Protocol Conversion Impact	144

19.3 Design Notes	145

19.4 References	146

20 Latency Analysis	147

20.1 Overview	147

20.2 Timing	148

20.2.1 Address Path Latency	148

20.2.2 Data Path Latency	148

20.2.3 Response Path Latency	148

20.2.4 Loaded Write Latency (measured)	150

20.2.5 End-to-End Latency	150

20.2.6 Width Conversion Latency	151

20.2.7 Protocol Conversion Latency	151

20.3 Design Notes	153

20.3.1 Design Recommendations	153

20.4 References	154

21 Resources	155

21.1 Overview	155

21.2 Measured Utilization and Timing	156

21.2.1 Scaling	158

21.3 Design Notes	159

21.3.1 Resource Reduction	159

21.3.2 Performance/Resource Trade-off	159

21.4 References	160

22 System Requirements	161

22.1 Overview	161

22.1.1 Clock Infrastructure	161

22.1.2 Reset Infrastructure	161

22.1.3 Power	161

22.2 Parameters	162

22.2.1 Address Map	162

22.2.2 Connectivity	162

22.3 Ports	163

22.3.1 Master Requirements	163

22.3.2 Slave Requirements	163

22.4 Timing	164

22.4.1 Setup/Hold	164

22.4.2 Recommended Margins	164

22.5 Testing	165

22.5.1 Pre-Integration Checks	165

22.5.2 Integration Checks	165

22.5.3 Post-Integration Checks	165

22.6 Assumptions and Known Limitations	166

22.6.1 Response-Ordering Assumption	166

22.6.2 Buffer and CAM Depths	166

22.7 References	167

23 Parameter Configuration	168

23.1 Overview	168

23.2 Parameters	169

23.2.1 Core Parameters	169

23.2.2 Registered Crossbar (xbar_pipeline)	169

23.2.3 Derived Parameters	175

23.2.4 Per-Port Configuration	176

23.2.5 Top-Level Monitor Configuration	178

23.2.6 Parameter Validation	179

23.3 Usage Example	180

23.3.1 TOML Configuration	180

23.3.2 CSV Connectivity	180

23.3.3 Simple 2x2	180

23.3.4 Mixed Protocol	181

24 Verification Strategy	182

24.1 Overview	182

24.2 Testing	183

24.2.1 Verification Levels	183

24.2.2 Test Infrastructure	184

24.2.3 Test Execution Model	185

24.2.4 Coverage Goals	186

24.2.5 Debug Features	186

24.2.6 Regression Testing	187

24.2.7 Known Limitations	187

25 Synthesis and Implementation	189

25.1 Overview	189

25.2 The Flow	190

25.2.1 Where it lives	190

25.2.2 How the bridge is constrained	191

25.2.3 Reading the summary line	191

25.3 Measured Results	193

25.3.1 What the first run found	193

25.3.2 The reference set	193

25.3.3 Reading the rows	195

25.4 Design Notes	197

25.4.1 What moves the numbers	197

25.4.2 Before a board build	197

26 AXI3 Integration	198

26.1 Boundary-Conversion Architecture	198

26.2 Converter-Sourced Constraints	199

26.3 Validation Evidence	201

26.3.1 Configuration constraints	201

26.4 References	202

List of Figures

Figure 1.1: Block Diagram Legend	40

Figure 2.1: RAPIDS System Topology	47

Figure 2.2: Bridge System Context	56

Figure 3.1: Bridge Block Diagram	62

Figure 3.2: Write Path	64

Figure 3.3: Read Path	64

Figure 3.4: AXI4 to APB Conversion Sequence	81

Figure 3.5: Width Upsize Conversion	82

Figure 3.6: Width Downsize Conversion	83

Figure 4.1: APB Write Transfer (Zero Wait States)	98

Figure 4.2: APB Read Transfer (One Wait State)	99

Figure 4.3: Clock Distribution	104

Figure 4.4: Reset Timing	107

Figure 4.5: Multi-Clock CDC	110

List of Tables

Table 1: Document information	21

Table 2: Revision history	22

Table 3: Common Terminology	38

Table 4: Signal Naming Prefixes	39

Table 5: Block Diagram Symbols	40

Table 6: Parameter Format Example	42

Table 7: Acronyms and Abbreviations	43

Table 8: Supported Protocols	51

Table 9: Channel-Specific Resource Savings	53

Table 10: Address Map Organization	58

Table 11: Topology Parameter Scaling	65

Table 12: AXI4 Channel Summary	68

Table 13: Response Routing	70

Table 14: Protocol Conversion Matrix	76

Table 15: Slave-port protocol values	78

Table 16: Master-port protocol values	79

Table 17: Master-Side Interface Channels	87

Table 18: Slave-Side Interface Channels	88

Table 19: Monitor Bus Output	91

Table 20: Supported Burst Types	93

Table 21: APB Signal Definitions	95

Table 22: AXI4 to APB Burst Conversion	96

Table 23: APB to AXI4 Error Mapping	96

Table 24: Clock Requirements	103

Table 25: Reset Signal	103

Table 26: CDC slave port clock pins (cdc = true)	105

Table 27: CDC master port clock pins (cdc = true)	105

Table 28: Reset Effects	107

Table 29: Timing Constraints	108

Table 30: Clock-to-Output Delays	108

Table 31: AXI5 Port Surface	113

Table 32: APB5 Slave Surface	114

Table 33: AXI5-Lite sideband groups at an axil5 master boundary	115

Table 34: AXI5-Lite sideband groups at an axil5 slave boundary	116

Table 35: Interop Matrix	119

Table 36: Unmapped-address status and interrupt ports	121

Table 37: Subtractive-slave response behaviour	122

Table 38: Unmapped-address status registers	122

Table 39: Wishbone B4 Signal Definitions	126

Table 40: Wishbone termination to AXI4 response	128

Table 41: Slave watchdog status (one per slave with timeout_cycles > 0)	131

Table 42: Peak Throughput by Data Width	135

Table 43: Measured throughput, direct 32-bit AXI4 paths	135

Table 44: The bridge against the vendor fabrics on the Genesys 2	136

Table 45: Factors Affecting Throughput	137

Table 46: Width-converted paths, bridge_4x4_rw (FULL level)	138

Table 47: Accepted throughput and latency against offered load, bridge_2x2_rw (GATE level, 24 reads per master per point)	138

Table 48: Bandwidth characterization, test_bridge_perf_char (FUNC level, 2026-10-18)	140

Table 49: 8-beat read rate against transactions in flight, bridge_2x2_rw (FUNC level)	142

Table 50: Burst Efficiency Comparison	143

Table 51: AXI4 vs APB Protocol Impact	144

Table 52: Address Path Latency	148

Table 53: Data Path Latency	148

Table 54: Response Path Latency	148

Table 55: Write latency under load, bridge_2x2_rw, 32-bit direct path	150

Table 56: Upsize Latency by Ratio	151

Table 57: Downsize Latency by Ratio	151

Table 58: AXI4 to APB Conversion Latency	151

Table 59: Measured utilization and timing, Artix-7 100T -1 at 10 ns (HAS 6.4 flow)	156

Table 60: Measured utilization and timing, Kintex-7 325T -2 at 6.667 ns	157

Table 61: Resource Scaling	158

Table 62: Performance/Resource Trade-offs	159

Table 63: Clock Requirements	161

Table 64: Reset Requirements	161

Table 65: Power Requirements	161

Table 66: Address Map Requirements	162

Table 67: Connectivity Requirements	162

Table 68: Timing Requirements	164

Table 69: Recommended Timing Margins	164

Table 70: Bridge Core Parameters	169

Table 71: Crossbar options (bridge TASK-006, was BRIDGE-017)	169

Table 72: Per-port options: CDC (bridge TASK-006, GitHub #119 L2), latency, outstanding limit	172

Table 73: Monitor preset	174

Table 74: Derived Parameters	176

Table 75: Master Port Configuration	176

Table 76: Slave Port Configuration	177

Table 77: Top-Level Monitor Configuration	178

Table 78: Generator Validation Checks	179

Table 79: Unit Level Test Focus	183

Table 80: Integration Test Coverage	183

Table 81: System Level Tests	183

Table 82: Test Categories	184

Table 83: Functional Coverage Goals	186

Table 84: Code Coverage Goals	186

Table 85: Debug Signals	187

Table 86: CI Test Schedule	187

Table 87: Configuration Test Matrix	187

Table 88: Known Test Gaps	187

Table 89: Simulator Support Status	188

Table 90: Synthesis flow knobs	190

Table 91: summary.csv columns	191

Table 92: The CAM ordering-count scan, before and after (2026-09-11)	193

Table 93: Reference set on the Artix-7 100T -1 (Nexys A7), constrained at 10 ns	193

Table 94: Reference set on the Kintex-7 325T -2 (Genesys 2), constrained at 6.667 ns	194

# 1	Bridge Has Index

Generated: 2026-10-18

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	Bridge Hardware Architecture Specification

Document information

| Property | Value |
| --- | --- |
| Document Title | Bridge Hardware Architecture Specification |
| Version | 1.5 |
| Date | 2026-10-18 |
| Status | Released |
| Classification | Open Source - MIT License |

## 2.2	Revision History

Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-01-03 | RTL Design Sherpa | Initial release - restructured from single spec |
| 1.1 | 2026-06-04 | RTL Design Sherpa | Content sync to RTL state at 2026-06-04. Documents (1) the 64→128-bit monbus packet migration with new 64-bit side-band timestamp wire — referenced via docs/markdown/rtl-amba/includes/ocelus_package_spec.md; (2) per-port + global SV-parameter monitor methodology with mandatory use_monitor TOML field and variants = ["no","mon"] generator selector; (3) generator-emitted AXI4↔AXIL conversion at the slave boundary (was external); (4) new AXIL→wider-slave master-side alignment converters (axil_to_axi4_wide_align_{rd,wr}); (5) verification-strategy rewrite to protocol-BFM-only TBs with memory-backed slaves, serial-only execution, per-module cocotb function names, boundary_probe (renamed from address_decode) tests; (6) note of the independent W-tracker FIFO + split AW/W-ready MUX fix (commit d24bd617) that closed an interleaved-master deadlock. |
| 1.2 | 2026-09-08 | RTL Design Sherpa | Correctness and voice pass. Four external qc rounds (65/54/53/57 findings, all triaged against rtl/generated and the generator) removed the planned-but-never-built ID-table/CAM out-of-order scheme, the fixed 64-bit internal path, response arbitration/FIFOs and the AXI4-Lite-as-future-work status from the text; annotated the TOML keys the loader has never known and its three-tier unknown-key behaviour; replaced asserted latency figures with measured ones. Four RTL defects found by the rounds are fixed and mutation-checked (bridge BUG-009 (was BRIDGE-011) response-FIFO overrun, axi4_subtractive_slave simultaneous AW+AR fault loss, axi5_atomic_filter duplicate B and B-before-WLAST). Prose then voice-passed with all correctness annotations preserved. |
| 1.3 | 2026-09-11 | RTL Design Sherpa | bridge TASK-004 (was BRIDGE-014): AXI5-Lite and APB/APB5 as master protocols (front-end converters); bridge TASK-005 (was BRIDGE-016) slave ID widening |
| 1.4 | 2026-09-11 | RTL Design Sherpa | bridge TASK-008 (was BRIDGE-019): Wishbone B4 on both sides of the fabric |
| 1.5 | 2026-09-11 | RTL Design Sherpa | bridge TASK-006 (was BRIDGE-017): measured throughput and latency, registered crossbar, QoS arbitration, CDC slave ports, out-of-context synthesis flow and measured resources, CAM ordering-count fix |
| 1.6 | 2026-09-13 | RTL Design Sherpa | bridge TASK-007 (was BRIDGE-018): native AXI5 fabric – Memory Tagging and read-data chunking through the structs |
| 1.7 | 2026-09-13 | RTL Design Sherpa | Document sync: one fabric clock plus CDC ports throughout, Wishbone best-effort stance, out-of-order chunk proof |
| 2.0 | 2026-10-12 | RTL Design Sherpa | Protocol-set correction: overview/summary bullets now state the full eight-protocol support (axi3/axi4/axi5/axil/axil5/apb/apb5/wb4 — per-port, slave or master side) verified against bin/bridge_pkg/config_validator.py; explicit “ACE coherency NOT supported” section added to ch03. Version strings aligned across index/front-matter/renders. |
| 1.1 (renumbered) | 2026-10-15 | RTL Design Sherpa | The bridge document set (GAS, HAS, MAS) is renumbered to one common rev, 1.1; rows above keep the numbers they were published under. Content: the one-target-per-master limitation is fixed (GitHub #119 L1) – per-ID target tracking ([bridge].id_track_bits), B/R response arbiters per master width path, the crossed-W hold; the generated TB carries its connectivity map and the generator emits the AXIL narrow-to-wide alignment test (GitHub #119 L8); per-port outstanding limits, max_outstanding (power of two 2..64, default 16), size every tracker – the fixed 16 removed (GitHub #119 L5); per-port latency modes (cut_all / cut_ax / cut_none) and honoured TOML skid depths (GitHub #119 L7); read-return-atomic and trace-echo ports no longer limited to one target (GitHub #119 L1b). |
| 1.2 | 2026-10-16 | RTL Design Sherpa | Slave-adapter CAM pipelining (GitHub #119): bridge_cam gains PIPELINE_ALLOC (registered insert with a pending-entry bypass; the full flag counts the pending entry) and PIPELINE_FREE (registered free; lookups see the post-free state), both on in every CAM-tracked slave adapter – MAS Figure 4.2. HAS 5.1 adds the Genesys 2 fabric-benchmark results against the Xilinx AXI Crossbar and SmartConnect; HAS 5.3 / 6.4 reference synthesis tables re-measured on the current generator. CDC on master ports (GitHub #119 L2): cdc = true on an AXI4 master puts axi4_cdc_{wr,rd} between the port and its timing wrapper, with <master>_aclk / <master>_aresetn on the top (HAS 4.5a, MAS 6.1); new fixture bridge_3x2_rw_mcdc and test_bridge_3x2_rw_mcdc_ratio. Generated TBs build their protocol checkers after reset. Read width converters are built with R_INTERLEAVE = 0 (no reassembly pool; it had grown every converter fixture past synthesis), so a slave may not interleave R beats across IDs (HAS 4.1, PRD FR-9). |
| 1.3 | 2026-10-16 | RTL Design Sherpa | GitHub #119 L3/L4. Protocol checkers on every port: each bridge gets <bridge>_checkers.sv, axi4_protocol_checker bound to every AXI4/AXI5 port on its own clock, in simulation and formal only; generated tests use it by default (HAS Verification Strategy, MAS 7.1, GAS 4.1). Slave watchdog: per-slave timeout_cycles puts axi4_slave_watchdog in front of an AXI4 slave, which answers everything DECERR once the slave is silent that long and raises <slave>_timeout (HAS Slave Watchdog, MAS 6.1, GAS 2.2). Measured width-converted path rates and latencies, and accepted throughput and latency against offered load (HAS 5.1). Write-converter B input registered and the master per-ID gate made shallower; HAS 5.3 / 6.4 re-measured. |
| 1.4 | 2026-10-18 | RTL Design Sherpa | Bandwidth limitations addressed. Shared slaves arbitrate at their pins from per-master request slots, so round-robin holds when the slave throttles (a master with two in flight: 0.34 -> 0.50 of the slave; MAS 2.2-2.4, GAS 4.4, HAS 5.1). A narrow master’s upsized path into a shared slave gets a rate-matching buffer (bridge_wr_rate_buf on writes, a FIFO on reads; rate_buffer, default 16 wide beats, 0 off), so the slave runs at its full rate behind it (0.80 -> 0.995 on writes, 0.93 -> 1.000 on reads; MAS 2.6, HAS 2.2, GAS 3.2). Width-converter fixes: the read upsizer’s bypass gate and the downsizer’s close/reopen of a partial wide beat, both formal-proven. New bandwidth characterization suite test_bridge_perf_char in the FUNC regression over nine fixture settings, CDC, AXI3 and AXI5 included, every figure floored (HAS 5.1, MAS 7.1). Open limits (W7-W9 floored at their measured level): W7 (an AXI3 master has one write in flight), W8 (the AXI3 slave converter’s issue rate and one-per-ID ordering), W9 (round-robin shares at a CDC slave) and W10 (the AXI3 slave converter uses SystemVerilog queues, which Vivado does not synthesize) (HAS 4.5a, 5.1, 6.5). |
| 1.5 | 2026-10-18 | RTL Design Sherpa | AXI3 limitations addressed. An AXI3 master’s writes overlap: axi3_to_axi4_wr passes every channel through (W7; was one write in flight, 0.111 beat/cycle single-beat). The AXI3 slave converter takes one request per cycle and passes unchopped requests straight to the pins, with up to 32 in flight across IDs (W8; was every other cycle, one per ID, four requests queued behind the arbiter). Its storage is gaxi_fifo_sync, so AXI3 slaves synthesize (W10). An AXI3 slave adapter is built like an AXI4 one: timing wrapper, per-ID tracking and request slots, with the converter last before the pins (W11). Every AXI3 path runs at 1.000 on 2+ beat bursts; round-robin at a shared AXI3 slave holds (HAS 5.1, 6.5; MAS 2.7; GAS 3.4). max_outstanding sizing rule with a measured in-flight sweep (HAS 5.1). W9 (CDC-slave round-robin) remains open, with the depth measurement that shows it needs slave-domain arbitration. |

## 2.3	Document Purpose

This Hardware Architecture Specification (HAS) is the high-level view of the Bridge component — the map you read before you integrate, not the schematic. It covers:

- System-level architecture and data flow

- Protocol support (AXI3, AXI4, AXI5, AXI4-Lite, AXI5-Lite, APB, APB5, Wishbone B4 — per-port, slave or master side; ACE coherency NOT supported)

- Interface definitions and requirements

- Performance characteristics

- Integration guidelines

For micro-architecture, FSM designs, and signal-level detail, see the companion Bridge Micro-Architecture Specification (MAS).

## 2.4	Intended Audience

- System architects defining interconnect topology

- Hardware engineers integrating Bridge into SoC designs

- Verification engineers planning test strategies

- Technical managers evaluating Bridge capabilities

## 2.5	Related Documents

- Bridge MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Purpose and Scope

## 3.1	What is Bridge?

Bridge is a Python-based multi-protocol crossbar generator. You describe the interconnect in human-readable configuration files; it hands you back parameterized SystemVerilog RTL. The generated AXI4 crossbars come with automatic support for:

- Protocol conversion - AXI3, AXI4, AXI5, AXI4-Lite, AXI5-Lite, APB, APB5, and Wishbone B4 interfaces, on slave or master ports (selected per port in the TOML; ACE coherency is NOT supported)

- Width conversion - Automatic upsize/downsize for data width mismatches

- Channel-specific masters - Write-only, read-only, or full AXI4 masters

## 3.2	The Problem Bridge Solves

A hand-written AXI4 crossbar is error-prone. Building one means:

- Writing 5 separate channel multiplexers (AW, W, B, AR, R)

- Implementing per-slave arbitration for each channel

- Routing responses back to the issuing master, including slaves that complete different IDs out of order

- Handling burst locking and interleaving constraints

- Inserting width converters for data width mismatches

- Inserting protocol converters for mixed systems — APB/APB5, AXI4-Lite/AXI5-Lite, AXI3, or Wishbone B4 peripherals behind an AXI4 fabric (and vice versa)

- Wiring hundreds of signals with consistent naming

Every item on that list is a place a bug can sit quietly until integration. I’ve chased most of them at least once.

Bridge automates all of it:

- Define ports and connectivity in configuration files

- Run the generator script

- Get production-ready SystemVerilog RTL

## 3.3	Scope

This specification covers:

- High-level architecture and block organization

- Supported protocols and conversion capabilities

- Interface requirements and signal conventions

- Performance characteristics and resource estimates

- Integration guidelines and verification strategy

## 3.4	Out of Scope

These live in the companion MAS:

- Detailed FSM state diagrams and transitions

- Signal-level timing diagrams

- Internal pipeline structures

- RTL implementation details

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Document Conventions

## 4.1	Terminology

Common Terminology

| Term | Definition |
| --- | --- |
| Master | AXI4 initiator that issues transactions |
| Slave | AXI4 target that responds to transactions |
| Crossbar | NxM interconnect connecting masters to slaves |
| Channel | AXI4 communication path (AW, W, B, AR, R) |
| Converter | Logic that transforms width or protocol |

## 4.2	Signal Naming

Bridge uses consistent signal prefixes — learn them once and every generated interface reads the same way:

Signal Naming Prefixes

| Prefix | Meaning |
| --- | --- |
| s_* | Slave-side interface (master port on bridge) |
| m_* | Master-side interface (slave port on bridge) |
| axi4_* | AXI4 protocol signals |
| apb_* | APB protocol signals |
| cfg_* | Configuration signals |
| dbg_* | Debug signals |

## 4.3	Diagrams

### 4.3.1	Figure 1.1: Block Diagram Legend

Block diagrams in this specification use these symbols:

Block Diagram Symbols

| Symbol | Meaning |
| --- | --- |
| White rectangle | External port or interface (masters at the top, slaves at the bottom) |
| Shaded rectangle | Functional block; the second line names the owning RTL |
| Solid arrow | A flow, labeled with what it carries: requests down, responses back up their own lane |
| Heavy arrow | The main flow out of a decision point, or a path on a clock other than aclk |
| Dashed arrow | Sideband: tracking writes, configuration, clocks |
| Dashed box | Optional or build-dependent block |
| Trapezium, wide edge up | A many-to-one pick (the per-slave arbiter): contenders enter the wide edge |

## 4.4	Code Examples

SystemVerilog examples are formatted like this:

// Module instantiation example
bridge_4x4 u_bridge (
    .aclk       (sys_clk),
    .aresetn    (sys_rst_n),
    // ... port connections
);

## 4.5	Parameter Notation

Parameter tables throughout the spec use this format:

Parameter Format Example

| Parameter | Type | Range | Default | Description |
| --- | --- | --- | --- | --- |
| NUM_MASTERS | int | 1-32 | 2 | Number of master ports |
| NUM_SLAVES | int | 1-256 | 2 | Number of slave ports |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Definitions and Acronyms

## 5.1	Acronyms

Acronyms and Abbreviations

| Acronym | Definition |
| --- | --- |
| AMBA | Advanced Microcontroller Bus Architecture |
| APB | Advanced Peripheral Bus |
| AR | AXI4 Read Address channel |
| AW | AXI4 Write Address channel |
| AXI | Advanced eXtensible Interface |
| AXI4 | AXI Protocol Version 4 |
| B | AXI4 Write Response channel |
| BID | Bridge ID (internal master identifier) |
| CAM | Content-Addressable Memory |
| CDC | Clock Domain Crossing |
| CSV | Comma-Separated Values |
| DECERR | Decode Error (AXI response) |
| DMA | Direct Memory Access |
| FIFO | First-In First-Out buffer |
| FSM | Finite State Machine |
| HAS | Hardware Architecture Specification |
| ID | Transaction Identifier |
| MAS | Micro-Architecture Specification |
| MUX | Multiplexer |
| NxM | N masters by M slaves (topology notation) |
| OOO | Out-of-Order |
| OOR | Out-of-Range |
| QoS | Quality of Service |
| R | AXI4 Read Data channel |
| RTL | Register-Transfer Level |
| SLVERR | Slave Error (AXI response) |
| SoC | System-on-Chip |
| TOML | Tom’s Obvious Minimal Language |
| W | AXI4 Write Data channel |

## 5.2	Definitions

Address Decode: Determining which slave receives a transaction, from its address.

Arbitration: Selecting which master gets a slave when several masters contend for it.

Bridge ID (BID): The issuing master’s index, width clog2(NUM_MASTERS). With more than one master it is both prepended to the AXI ID inside the fabric (slave-side IDs are {BRIDGE_ID, id}, wider than master-side ones) and carried as a sideband into the slave adapter’s response tracker (a bridge_cam or an in-order FIFO), which routes B and R back to the master.

Channel-Specific Master: A master that uses only a subset of AXI4 channels (write-only or read-only).

Crossbar: An NxM interconnect allowing any master to communicate with any connected slave.

Grant Locking: Mechanism that holds arbitration grant until transaction completes.

Out-of-Order Response: (defined for completeness; NOT supported by this bridge) Responses returned in different order than requests were issued.

Protocol Conversion: Transformation between different bus protocols (e.g., AXI4 to APB).

Response Routing: Directing B/R channel responses back to the originating master.

Round-Robin Arbitration: Fair arbitration where priority rotates among requesters.

Width Conversion: Transformation between different data bus widths (upsize or downsize).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Use Cases

## 6.1	Primary Applications

### 6.1.1	SoC Interconnects

In SoC designs, Bridge is the main interconnect fabric:

- Multi-core processor memory subsystems - Connect multiple CPU cores to shared memory

- Accelerator integration - Route GPU, DSP, or custom accelerator traffic

- DMA engine routing - Connect DMA controllers to memory and peripherals

- Peripheral bus bridges - Convert AXI4 to APB for low-speed peripherals

### 6.1.2	Memory Systems

Typical memory-system uses:

- Multi-port DDR controllers - Multiple masters accessing shared DDR

- SRAM buffer sharing - Shared packet buffers for networking

- Cache coherency interconnects - Connect cache controllers

- Memory-mapped I/O - Unified address space for peripherals

### 6.1.3	Accelerator Systems

Bridge supports custom accelerator architectures like RAPIDS:

- Descriptor write masters - Write control descriptors (write-only)

- Sink write masters - Write incoming data packets (write-only)

- Source read masters - Read outgoing data packets (read-only)

- CPU configuration - Full access for register configuration

## 6.2	Example: RAPIDS Accelerator

### 6.2.1	Figure 2.1: RAPIDS System Topology

RAPIDS System Topology

Bridge topology for RAPIDS accelerator with channel-specific masters.

### 6.2.2	Configuration

[bridge]
  name = "bridge_rapids"
  masters = [
    {name = "rapids_descr_wr", channels = "wr", data_width = 512},
    {name = "rapids_sink_wr", channels = "wr", data_width = 512},
    {name = "rapids_src_rd", channels = "rd", data_width = 512},
    {name = "cpu", channels = "rw", data_width = 64}
  ]
  slaves = [
    {name = "ddr", protocol = "axi4", data_width = 512},
    {name = "apb_periph", protocol = "apb", data_width = 32},
    {name = "sram", protocol = "axi4", data_width = 256}
  ]

### 6.2.3	Benefits

- 40% fewer channels for write-only masters (3 of 5: AW, W, B)

- 60% fewer channels for read-only masters (2 of 5: AR, R)

Those are CHANNEL counts, which is the honest way to state it – the saving in actual wires depends on the widths. At DW=64/AW=32/IW=4/UW=1 the five channels are roughly 295 bits, and dropping AR+R saves about 48% of them while dropping AW+W+B saves about 52%; at DW=512 it is nearer 47%/53%. Earlier revisions of this page quoted “39%” and “61%”, which match neither basis. - Automatic width conversion for 64b CPU to 512b DDR - Protocol conversion for APB peripheral access

## 6.3	Rapid Prototyping

Bridge accelerates design exploration:

- Quick topology exploration - Try different master/slave configurations

- Performance analysis - Evaluate arbitration and bandwidth

- Design space exploration - Compare resource usage

- Educational projects - Learn AXI4 protocol and interconnects

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Key Features

## 7.1	Configuration-Driven Generation

### 7.1.1	CSV/TOML Configuration

Everything about a Bridge instance is set in human-readable text files:

- ports configuration - Define each master and slave port

- connectivity matrix - Specify which masters connect to which slaves

- Version-control friendly - Text-based, easy to diff and review

### 7.1.2	Example Configuration

Port Definition (TOML):

masters = [
  {name = "cpu", prefix = "cpu_m_axi", data_width = 64, channels = "rw"},
  {name = "dma", prefix = "dma_m_axi", data_width = 256, channels = "rw"}
]
slaves = [
  {name = "ddr", prefix = "ddr_s_axi", data_width = 512, protocol = "axi4"},
  {name = "uart", prefix = "uart_apb", data_width = 32, protocol = "apb"}
]

Connectivity Matrix (CSV):

master\slave,ddr,uart
cpu,1,1
dma,1,0

## 7.2	Multi-Protocol Support

Supported Protocols

| Master Protocol | Slave Protocol | Conversion Type |
| --- | --- | --- |
| AXI4 | AXI4 | Direct or width convert |
| AXI4 | AXI4-Lite | Protocol downgrade |
| AXI4 | APB | Full protocol conversion |
| AXI5 | AXI4 / AXI4-Lite / APB | Base-subset interop; sideband terminates at the boundary (warning) |
| AXI5 | AXI5 (width-matched) | Native sideband pass-through; atomics (read-return classes need an rw port); poison |
| AXI4 / AXI5 | APB5 | APB4 conversion core + APB5 sideband surface |
| AXI4-Lite / AXI5-Lite | any | Lite requester: single-beat AXI4 inside; the AXI5-Lite ID and user group ride the fabric, the rest terminate at the boundary |
| APB / APB5 | any | APB requester: apb4_to_axi4 / apb5_to_axi4 front end, one AXI4 transaction per transfer, SLVERR and DECERR fold to PSLVERR |
| any | Wishbone B4 | axi4_to_wb4: one Wishbone transfer per AXI4 beat, ERR/RTY fold to SLVERR |
| Wishbone B4 | any | wb4_to_axi4 front end: single-beat AXI4 inside, SLVERR/DECERR terminate ERR |

AMBA5 ports are declared per port (protocol = "axi5" / "axil5" / "apb5" with an optional axi5_features list); the fabric stays AXI4 internally. Every protocol value is legal on a master port as well as a slave port (bridge TASK-004, was BRIDGE-014). See AXI5 and APB5 Interfaces.

### 7.2.1	Protocol Features

- AXI4 Full - Complete 5-channel implementation with bursts

- AXI4-Lite - Simplified single-beat transactions

- APB - Low-power peripheral access

- Wishbone B4 - The open FPGA bus, pipelined mode, on either side (bridge TASK-008, was BRIDGE-019)

## 7.3	Channel-Specific Masters

### 7.3.1	Write-Only Masters (wr)

Generate only write channels: - AW (Write Address) - W (Write Data) - B (Write Response)

Use case: DMA write engines, descriptor writers

### 7.3.2	Read-Only Masters (rd)

Generate only read channels: - AR (Read Address) - R (Read Data)

Use case: DMA read engines, source data fetchers

### 7.3.3	Full Masters (rw)

Generate all five channels for complete read/write capability.

Use case: CPU interfaces, configuration masters

### 7.3.4	Resource Savings

Channel-Specific Resource Savings

| Master Type | Channels | Signal Reduction |
| --- | --- | --- |
| Write-only (wr) | 3 of 5 | 40% fewer channels (~48% fewer wires at DW=64) |
| Read-only (rd) | 2 of 5 | 60% fewer channels (~52% fewer wires at DW=64) |
| Full (rw) | 5 of 5 | Baseline |

## 7.4	Automatic Converters

### 7.4.1	Width Conversion

Bridge automatically inserts width converters:

- Upsize - Narrow master to wide slave (e.g., 64b to 512b). Into a slave other masters share, a rate-matching buffer (rate_buffer, default 16 wide beats) keeps that slave at its full rate during the narrow master’s bursts (MAS 2.6)

- Downsize - Wide master to narrow slave (e.g., 512b to 32b)

- Per-path - Only where needed, not global conversion

### 7.4.2	Protocol Conversion

Bridge inserts protocol converters for mixed systems:

- AXI4 to APB - Full conversion with burst handling

- AXI4 to AXI4-Lite - Simplified conversion

## 7.5	Flexible Topology

### 7.5.1	Scalability

- Masters: 1-32 master ports

- Slaves: 1-256 slave ports

- Partial connectivity - Not all masters need access to all slaves

### 7.5.2	Mixed Configurations

- Mixed protocols - AXI4, AXI4-Lite, and APB slaves in same crossbar

- Mixed data widths - 32b, 64b, 128b, 256b, 512b

- Mixed channels - Write-only, read-only, and full masters

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	System Context

## 8.1	Bridge in SoC Architecture

### 8.1.1	Figure 2.2: Bridge System Context

Bridge System Context

Bridge as central interconnect connecting masters to slaves with protocol conversion.

## 8.2	Interface Boundaries

### 8.2.1	Master-Side (Upstream)

Bridge presents slave interfaces to upstream masters:

- Accepts AXI4 transactions from masters

- Provides flow control via ready signals

- Returns responses (B/R channels) to correct master

### 8.2.2	Slave-Side (Downstream)

Bridge presents master interfaces to downstream slaves:

- Issues AXI4/APB transactions to slaves

- Accepts responses from slaves

- Routes responses back through ID tracking

## 8.3	Address Space

### 8.3.1	Address Map Organization

Each slave occupies a configurable address region:

Address Map Organization

| Slave | Base Address | Address Range | Size |
| --- | --- | --- | --- |
| Slave 0 | base_addr from its own TOML entry | addr_range | Variable |
| Slave 1 | base_addr from its own TOML entry | addr_range | Variable |
| Slave N | base_addr from its own TOML entry | addr_range | Variable |

Windows are not derived from a single BASE and are not contiguous. Each slave carries its own base_addr, subject only to 4 KB alignment and non-overlap (Parameters). An earlier revision of this table showed BASE + addr_range_0 and “sum of previous ranges”, which no shipped config follows – bridge_mix_d places a 1 GB ddr at 0x0000_0000, a 4 KB doorbell at 0x4000_0000 and a 64 KB apb_periph at 0x4000_1000, and gaps between windows are legal (they decode-miss).

### 8.3.2	Address Decode

- Subtractive decode - the ranges are tested in order and the chain ends in an else, so the one-hot result is never all-zero

- One-hot result - exactly one slave selected per transaction, always

- Out-of-range - an address matching no slave range selects the internal subtractive slave, which completes the transaction with DECERR and 0xDEADBEEF read data rather than leaving it unanswered

Until 2026-09-07 the last bullet was aspirational: the decode chain had no else, so an unmapped address produced an all-zero select, no slave ever saw AWVALID/ARVALID, READY never rose, and the master waited forever. A hang is the worst available failure here because it destroys the evidence – there is no response to inspect, no error bit to read, and the offending address is whatever the master still has latched. See 4.5 for the status/interrupt path that now reports it.

## 8.4	Clock and Reset

### 8.4.1	Clock Domain

- One fabric clock - All Bridge logic and every master port synchronous to aclk

- CDC ports - An AXI4 slave or master declared cdc = true runs on its own <port>_aclk behind an async-FIFO crossing inside its adapter (HAS 4.5a); other ports share aclk

### 8.4.2	Reset

- Active-low asynchronous - Standard aresetn convention

- Full reset - All internal state cleared on reset

- Transaction safety - Outstanding transactions aborted on reset

## 8.5	Dependencies

### 8.5.1	Required Infrastructure

- AXI4-compliant masters and slaves

- Proper clock and reset distribution

- Address map configuration matching slave address ranges

### 8.5.2	Optional Features

- Width converters (only if widths mismatch)

- Protocol converters (only if protocols differ)

- Per-slave response tracker: bridge_cam by ID for multi-master AXI4/AXI5 slaves (or enable_ooo), an in-order bridge_id FIFO otherwise

## 8.6	References

- Jantsch & Tenhunen (eds.), Networks on Chip, Kluwer 2003 — the resource-network-interface (RNI) concept that Bridge’s master/slave adapters map to: a protocol-dependent wrapper plus a resource-independent crossbar side (ch5 §3.4.2). (Reference archive)

- Eschemann, The FPGA Programming Handbook, Packt 2023 — educational secondary source noting that an AXI interconnect’s cost scales with active masters, buffering, and whether it is a full crossbar (ch8 §8.3). (Reference archive)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Block Diagram

## 9.1	Bridge Architecture Overview

### 9.1.1	Figure 3.1: Bridge Block Diagram

Bridge Block Diagram

One master’s path through the generated bridge to one slave; every master and every slave repeats it. Write data drops down the left lane, requests take the center through the per-slave arbiter (the trapezium, where every master’s AW / AR contend), and B / R responses climb the right lane back to the master named by the returned bridge_id. The response tracker in the slave adapter is written at AW / AR accept (dashed) and read when the response returns.

## 9.2	Functional Blocks

### 9.2.1	Master Adapter

One per master (<master>_adapter), receives transactions from the upstream master:

- Skid buffers - axi4_slave_wr / axi4_slave_rd register every channel at the port

- Address decode - a one-hot slave select from each slave’s base_addr / addr_range; no hit selects the catch-all

- Width conversion - axi4_dwidth_converter_wr / _rd, one path per distinct slave width, only where the widths differ

- Fabric ID - the AXI ID is prefixed with the master’s BRIDGE_ID ({BRIDGE_ID, id}), so a slave port’s ID is wider than a master port’s

- Clock crossing - axi4_cdc_wr / _rd ahead of the skid buffers when the master port is cdc = true (GitHub #119 L2)

### 9.2.2	Crossbar Core

Central switching fabric (<bridge>_xbar):

- 5-channel routing - Independent AW, W, B, AR, R paths

- Arbitration - one AW and one AR arbiter per slave, round-robin (or arbitration = "qos"), grant held until the address handshake

- W steering - a W destination FIFO per master path and a W owner FIFO per slave keep each W burst with its AW

- Response muxes - B and R return to the master named by the bridge_id the slave adapter hands back

### 9.2.3	Slave Adapter

One per slave (<slave>_adapter), delivers transactions to the downstream slave:

- Response tracking - bridge_cam keyed by AXI ID when several masters share the slave, or an in-order bridge_id FIFO when one master does; written at AW / AR accept, read at B / last R

- Skid buffers or protocol conversion - axi4_master_wr / _rd, or axi4_to_apb4_shim, axi4_to_axil4_* / axi4_to_axil5_*, axi4_to_wb4

- Clock crossing - axi4_cdc_wr / _rd when the slave port is cdc = true

## 9.3	Data Flow

### 9.3.1	Figure 3.2: Write Path

Write Path Flow

### 9.3.2	Figure 3.3: Read Path

Read Path Flow

## 9.4	Scalability

### 9.4.1	Topology Parameters

Topology Parameter Scaling

| Parameter | Range | Impact |
| --- | --- | --- |
| NUM_MASTERS | 1-32 | Linear MUX growth |
| NUM_SLAVES | 1-256 | Linear decoder growth |
| DATA_WIDTH | 32-512 | Linear data path growth |
| ID_WIDTH | 1-16 | Scales the ID wires (slave side carries {BRIDGE_ID, id}) and the tracker CAM tags |

### 9.4.2	Resource Scaling

- Logic: O(M x N) for crossbar core

- Routing: O(M + N) for address/response paths

- Memory: O(M x outstanding) for ID tracking

## 9.5	References

- Dally & Towles, Principles and Practices of Interconnection Networks, Morgan Kaufmann 2004 — per-slave arbiters on AW and AR as the degenerate case of a separable allocator (ch18 §19.3), and skid buffers as buffered flow control (ch12). (Reference archive)

- Roy, Advanced Digital System Design, Springer 2023 — AXI permits W to trail AW independently; the W-owner FIFO records the AW grant so the data path follows the address decision (§18.2.3). (Reference archive)

- Duato/Yalamanchili/Lionel, Interconnection Networks, Morgan Kaufmann 2003 — deterministic routing preserves order only along a single path; safe out-of-order completion requires explicit ID-keyed tracking such as the CAM mode Bridge keeps available (ch3/4). (Reference archive)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Data Flow

## 10.1	Transaction Flow Overview

### 10.1.1	Write Transaction Flow

- Master issues AW - Address and control information

- Bridge decodes address - Determines target slave

- Arbitration - Grants access if multiple masters contend

- AW forwarded to slave - ID unchanged; the originating master is pushed onto the slave adapter’s bridge_id FIFO

- Master sends W data - Following the granted AW

- W forwarded to slave - Using same grant

- Slave returns B response - with the master’s own ID; the FIFO head selects the destination

- Bridge routes B to master - Using ID to find originator

### 10.1.2	Read Transaction Flow

- Master issues AR - Address and control information

- Bridge decodes address - Determines target slave

- Arbitration - Grants access if multiple masters contend

- AR forwarded to slave - ID unchanged; master recorded in the read bridge_id FIFO

- Slave returns R data - Potentially multiple beats

- Bridge routes R to master - Using ID to find originator

## 10.2	Channel Independence

### 10.2.1	AXI4 Channel Separation

Each AXI4 channel operates independently:

AXI4 Channel Summary

| Channel | Direction | Contents | Arbitration |
| --- | --- | --- | --- |
| AW | Master to Slave | Write address | Per-slave |
| W | Master to Slave | Write data | Follows AW grant |
| B | Slave to Master | Write response | ID-based routing |
| AR | Master to Slave | Read address | Per-slave |
| R | Slave to Master | Read data | ID-based routing |

### 10.2.2	Parallel Operation

- Read and write transactions can proceed simultaneously

- Different masters can access different slaves in parallel

- Same slave serializes access via arbitration

## 10.3	Bridge ID and ID Widening

### 10.3.1	Bridge ID (BID) Concept

Master ID:                 [MASTER_ID_WIDTH-1:0]
Bridge ID (master index):  BRIDGE_ID_WIDTH = clog2(NUM_MASTERS)
AXI ID on a slave port:    {BRIDGE_ID, master ID}, XBAR_ID_WIDTH bits

### 10.3.2	IDs are widened inside the fabric

With more than one master, each master adapter prepends its index to every request ID (bridge TASK-005), so two masters can never alias an ID at a slave. In bridge_2x2_rw:

input  logic [3:0]  cpu_m_axi_awid     // master side: 4 bits
output logic [4:0]  ddr_s_axi_awid     // slave side: {BRIDGE_ID, id} = 5 bits

The slave-side width is the widest master’s id_width plus $clog2(NUM_MASTERS) bits (XBAR_ID_WIDTH in <bridge>_pkg.sv); a slave declared narrower is rejected by the config validator. A single-master bridge prepends nothing. On B and R the master adapter returns only the low master-ID bits, so a master always sees its own ID.

## 10.4	Response Routing

The prefix bits are not decoded to route a response. Each slave adapter also receives the granted master’s index as a sideband (bridge_id), stores it at the address handshake, and returns it with the response (bid_bridge_id / rid_bridge_id); the crossbar’s B and R muxes select on that. Which store a slave gets is fixed at generation:

Response Routing

| Slave | Tracker | Order the slave may complete in |
| --- | --- | --- |
| AXI4 / AXI5 slave, more than one master | bridge_cam (u_wr_cam / u_rd_cam), tag = AXI ID, data = bridge_id, max_outstanding deep (default 16) | any order across IDs |
| any slave with enable_ooo = true | bridge_cam | any order across IDs |
| single-master bridge, apb / axil / wb shim slaves, internal subtractive slave | in-order bridge_id FIFO, max_outstanding deep (default 16) | request order across all IDs |

// CAM form (multi-master AXI4 slave)
bridge_cam #(.TAG_WIDTH(ID_WIDTH), .DATA_WIDTH(BRIDGE_ID_WIDTH),
             .DEPTH(16), .ALLOW_DUPLICATES(1),             // DEPTH: the slave's max_outstanding
             .PIPELINE_ALLOC(1), .PIPELINE_FREE(1)) u_wr_cam (  // registered insert / free (MAS 4.1)
    .allocate_tag  (xbar_ddr_axi_awid),   .allocate_data  (xbar_bridge_id_aw),
    .deallocate_tag(xbar_ddr_axi_bid),    .deallocate_data(bid_bridge_id), ...);

// FIFO form
wr_fifo[wr_ptr[...]] <= xbar_bridge_id_aw;      // push on AW accept
assign bid_bridge_id  = wr_fifo[rd_ptr[...]];   // route by FIFO head

### 10.4.1	B Channel (Write Response)

- Slave issues B carrying the fabric ID it was sent

- The slave adapter looks the BID up in its CAM (or pops its FIFO)

- The returned bridge_id selects the master

- The master adapter strips the prefix and forwards B

### 10.4.2	R Channel (Read Data)

- Slave issues R carrying the fabric ID

- The slave adapter’s read tracker names the master; the entry is freed on RLAST

- The master adapter strips the prefix

Either tracker holds 16 transactions; when it is full the slave’s AWREADY / ARREADY toward the crossbar drops (bridge BUG-009).

### 10.4.3	The requirement the FIFO form creates

A FIFO-tracked slave is routed by position, so it must return B/R in request order across ALL IDs. Those slaves are in order by construction (the protocol shims and the subtractive slave) or have a single master; a simulation-only check flags a violation (bridge BUG-008, was BRIDGE-010). A multi-master AXI4/AXI5 slave has no such requirement: the CAM routes by ID.

## 10.5	Ordering: what the fabric actually guarantees

Per-ID ordering is the slave’s AXI obligation; across IDs a CAM-tracked slave may complete in any order. The fabric adds one structural rule:

Per master – per-ID targets. A master may have transactions in flight at several slaves at once. The master adapter keeps a small table per ID ([bridge].id_track_bits, default 4: 16 entries indexed by the ID’s low bits) and aw_gate_ok/ar_gate_ok hold a new address phase only when its ID is already in flight to a DIFFERENT slave – the AXI same-ID ordering rule, enforced where it can be, before the request leaves. Responses from different slaves come back in whatever order the slaves produce them: the crossbar arbitrates among the slaves on each master width path, and the adapter among its paths. A new AW also waits while W is still owed to a different slave: W carries no ID and follows AW order, so two masters each owing W to two slaves could otherwise wait on each other at the slaves’ W ports. Read-return-atomic and trace-echo ports work the same way (their per-request state is held per ID too, MAS 2.1; GitHub #119 L1b). The rule was lifted 2026-10-15 (limitation fixed: GitHub #119 L1); before that every master held one target slave at a time.

Per FIFO-tracked slave – responses must come back in order. See “The requirement the FIFO form creates” above.

Worked example: master 0 issues a read to a slow slave 0 with ID 1, then a read to slave 1 with ID 2. The slave 1 response returns first. With ID 2 changed to ID 1, the second read is held at the master adapter until the first returns. test_bridge_2x2_rw_multitarget measures both cases.

## 10.6	References

- Dally & Towles, Principles and Practices of Interconnection Networks, Morgan Kaufmann 2004 — per-slave arbiters as a separable allocator degenerated to one resource per channel (ch18 §19.3), and skid buffers as buffered flow control (ch12). (Reference archive)

- Roy, Advanced Digital System Design, Springer 2023 — AXI’s lack of a strict timing relationship between AW and W, and the use of the stored transaction ID to correlate B back to AW (§18.2.3). (Reference archive)

- Duato/Yalamanchili/Lionel, Interconnection Networks, Morgan Kaufmann 2003 — deterministic routing preserves order only on a single path; out-of-order responses across IDs need explicit ID-keyed tables such as Bridge’s CAM mode (ch3/4). (Reference archive)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Protocol Support

## 11.1	Supported Protocols

### 11.1.1	AXI4 Full

Complete AMBA AXI4 protocol support:

- 5 channels: AW, W, B, AR, R

- Burst transactions: FIXED, INCR, WRAP

- Transaction IDs: Configurable width

- Out-of-order: across IDs at burst granularity for a CAM-tracked slave (multi-master AXI4/AXI5, or enable_ooo); FIFO-tracked slaves return in request order (bridge BUG-008, was BRIDGE-010). R beats of different IDs never interleave (HAS 4.1)

- Data widths: 32, 64, 128, 256, 512 bits

### 11.1.2	AXI4-Lite

Simplified AXI4 for register access:

- 5 channels: Same as AXI4

- Single-beat only: AWLEN/ARLEN = 0

- No IDs: Transaction ordering by channel

- Fixed width: Typically 32 or 64 bits

### 11.1.3	AXI3

Legacy AMBA AXI3 protocol support, added as a boundary conversion so the internal crossbar remains AXI4:

- 5 channels: AW, W, B, AR, R, with WID present on write data

- Burst length: 1–16 beats on the boundary; longer AXI4 bursts are chopped by the boundary converter

- Per-port data width: Same width as the paired AXI4 side; width adaptation happens inside the bridge behind the converter when necessary

- Lock mapping and interleaving constraints: set by the converter, not the crossbar — see HAS 6.5 AXI3 Integration

### 11.1.4	APB (Advanced Peripheral Bus)

Low-power peripheral protocol:

- Single channel: Combined address and data

- Simple handshake: PSEL, PENABLE, PREADY

- No bursts: Single transfer per transaction

- Low overhead: Minimal logic for slow peripherals

## 11.2	Protocol Conversion Matrix

Protocol Conversion Matrix

| Master | Slave | Conversion Type | Notes |
| --- | --- | --- | --- |
| AXI4 | AXI4 | Direct | Width conversion if needed |
| AXI4 | AXI4-Lite | Downgrade | Burst split to single beats |
| AXI4 | APB | Full convert | Channel and timing conversion |
| AXI4-Lite | AXI4 | Upgrade | Single-beat pass-through |
| AXI4-Lite | AXI4-Lite | Direct | Width conversion if needed |
| AXI4-Lite | APB | Convert | Simplified conversion |
| AXI5-Lite | any | Upgrade | As AXI4-Lite plus the AXI5-Lite ID; user rides the fabric, TRACE/POISON terminate at the boundary (no LOCK/LOOP/MPAM/MECID/NSAID on AXI5-Lite, IHI 0022L B2.1.5) |
| APB / APB5 | any | Full convert | apb{4,5}_to_axi4 front end, then as AXI4-Lite |
| any | Wishbone B4 | Full convert | axi4_to_wb4: one transfer per beat, in-order termination |
| Wishbone B4 | any | Full convert | wb4_to_axi4 front end, then as AXI4-Lite |
| AXI4 | AXI3 | Protocol convert | axi4_to_axi3 slave boundary; same data width only |
| AXI3 | AXI4 | Protocol convert | axi3_to_axi4 master boundary; same data width only |

## 11.3	Automatic Conversion Shims at Slave Boundary

When a slave port’s TOML entry names a protocol other than native AXI4 (e.g., protocol = "axi3", protocol = "axil" or protocol = "apb"), the generator emits the conversion shims between the crossbar core and that slave port. This happens once, at generation time; there is nothing to configure at runtime.

Supported Slave Protocols – these are the exact TOML values the generator accepts (config_validator.valid_protocols); anything else is a generation error, so spelling matters:

Slave-port protocol values

| protocol | Meaning | Shim emitted at the slave boundary |
| --- | --- | --- |
| axi4 | Native AXI4 | none |
| axi3 | AMBA AXI3 | axi4_to_axi3 |
| axi5 | AXI5, interop mode | axi5_master_{wr,rd} boundary wrappers |
| axil | AXI4-Lite | axi4_to_axil4_{rd,wr} |
| axil5 | AXI5-Lite | axi4_to_axil5_{rd,wr} |
| apb | APB3/APB4 | axi4_to_apb4_shim |
| apb5 | APB5 | axi4_to_apb5_shim |
| wb4 | Wishbone B4 (pipelined) | axi4_to_wb4 |

Note the spelling: it is axil, not axi4lite. Earlier revisions of this page used the latter, which the generator rejects.

The shim modules live in projects/components/utility-ip/converters/rtl/; the generator instantiates them into each generated top-level module. The slave port still presents the configured protocol (AXIL or APB) to the outside; inside, the crossbar core is uniformly AXI4.

## 11.4	Automatic Conversion at the Master Boundary

The same protocol values are legal on a master port (bridge TASK-004, was BRIDGE-014). The bridge presents the requester’s own protocol on the boundary – the AXI4-Lite signal set, the AXI5-Lite set with its sideband, or the APB completer set – and the master adapter converts to the AXI4 the crossbar speaks:

Master-port protocol values

| protocol | Boundary presents | Conversion in the master adapter |
| --- | --- | --- |
| axi4 | AXI4 | none (timing wrapper only) |
| axi3 | AXI3 | axi3_to_axi4, then the AXI4 timing wrapper |
| axi5 | AXI5 (minus REGION, plus enabled features) | axi5_slave_{wr,rd} boundary wrappers |
| axil | AXI4-Lite | AXI4 extras tied (AxLEN=0, INCR, WLAST=1, ID = placeholder); the wide-slave aligner toward wider slaves |
| axil5 | AXI5-Lite: ID + optional signals | as axil, but awid/arid -> the AXI4 ID and bid/rid back; user -> the 1-bit USER fields; TRACE/POISON terminated at the top |
| apb | APB4 completer | apb4_to_axi4, then the AXI4 timing wrapper |
| apb5 | APB5 completer (+ PAUSER/PWUSER/PWAKEUP in – all requester-driven, IHI 0024E 3.7.2 – PRUSER/PBUSER out) | apb5_to_axi4 (PAUSER[0]/PWUSER[0] -> USER), then the AXI4 timing wrapper |
| wb4 | Wishbone B4 completer (pipelined) | wb4_to_axi4, then the AXI4 timing wrapper; SLVERR/DECERR terminate ERR |

Rules the validator enforces on these ports: AXI4-Lite masters have id_width = 0 (no ID pins exist; the fabric ID is the master index, bridge TASK-005 (was BRIDGE-016)), AXI5-Lite masters id_width >= 1 (AWID/ARID are required on an AXI5-Lite Subordinate, IHI 0022L B2.1.5, GitHub #102 item 3), APB and Wishbone masters have addr_width = 32 (the requester addresses the whole fabric – an APB slave port’s PADDR is a window offset, a master’s is not), and axi5_features on an axil5 port may name only user, the one group with an AXI4 destination (exclusive, loop, mpam, mecid and nsaid are rejected as not existing on AXI5-Lite).

## 11.5	AXI4 to APB Conversion

### 11.5.1	Conversion Process

### 11.5.2	Figure 3.4: AXI4 to APB Conversion Sequence

AXI4 to APB Conversion

### 11.5.3	Burst Handling

- AXI4 bursts split into individual APB transfers

- WSTRB converted to per-byte enables

- B response generated after all beats complete

### 11.5.4	Timing Considerations

- APB is inherently slower (minimum 2 cycles per transfer)

- Pipeline stalls during APB access

- Consider separate APB crossbar for multiple slow peripherals

## 11.6	Width Conversion

### 11.6.1	Upsize (Narrow to Wide)

### 11.6.2	Figure 3.5: Width Upsize Conversion

Width Upsize

### 11.6.3	Downsize (Wide to Narrow)

### 11.6.4	Figure 3.6: Width Downsize Conversion

Width Downsize

## 11.7	Channel-Specific Protocol

### 11.7.1	Write-Only Masters

Only AW, W, B channels active:

- AR permanently deasserted

- R channel ignored (no routing logic)

- Reduced signal count and logic

### 11.7.2	Read-Only Masters

Only AR, R channels active:

- AW, W, B channels ignored

- No write arbitration for this master

- Reduced signal count and logic

## 11.8	ACE and Coherency — Explicitly Out of Scope

The bridge implements AMBA 5 AXI5 interop and sideband features, but it is not a coherence interconnect: there are no ACE snoop channels (AC/CR/CD), no ACVALID/ACREADY/CRVALID/CRREADY/CDVALID/CDREADY signals anywhere in the generated RTL, and cache-coherent masters are not supported. Coherent systems belong to the cache-ip domain (see the amber/jet/onxy work). This mirrors PRD.md: “No ACE protocol extensions (cache coherency).”

## 11.9	References

- Eschemann, The FPGA Programming Handbook, Packt 2023 — educational secondary source for AXI4 full and AXI4-Lite channel structure and handshake rules (ch8 §8.3). (Reference archive)

- Roy, Advanced Digital System Design, Springer 2023 — secondary source for AXI channel structure and headline protocol features (§18.2.2, §18.2.2.1). (Reference archive)

- Maaref, Architecting and Building High-Speed SoCs, Packt 2022 — AMBA-suite context and the evolution of AXI4 features such as QoS/Region (ch3 §4.2). (Reference archive)

- ARM, AMBA AXI and ACE Protocol Specification, ARM IHI 0022E — the authoritative AXI4 protocol definition. (Reference archive)

- ARM, AMBA 3 AXI Protocol Specification, ARM IHI 0022 — the AXI3 protocol authority; 4-bit AxLEN, 2-bit AxLOCK, write-data WID, absence of QoS/REGION. (Reference archive)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	AXI4 Interface Overview

## 12.1	Interface Organization

### 12.1.1	Master-Side Interfaces

Bridge presents slave interfaces to upstream masters. Each master port includes:

Master-Side Interface Channels

| Channel | Signals | Direction (to Bridge) |
| --- | --- | --- |
| AW | AWVALID, AWREADY, AWADDR, AWID, AWLEN, AWSIZE, AWBURST, … | Input |
| W | WVALID, WREADY, WDATA, WSTRB, WLAST | Input |
| B | BVALID, BREADY, BID, BRESP | Output |
| AR | ARVALID, ARREADY, ARADDR, ARID, ARLEN, ARSIZE, ARBURST, … | Input |
| R | RVALID, RREADY, RDATA, RID, RRESP, RLAST | Output |

### 12.1.2	Slave-Side Interfaces

Bridge presents master interfaces to downstream slaves. Each slave port includes:

Slave-Side Interface Channels

| Channel | Signals | Direction (from Bridge) |
| --- | --- | --- |
| AW | AWVALID, AWREADY, AWADDR, AWID, AWLEN, AWSIZE, AWBURST, … | Output |
| W | WVALID, WREADY, WDATA, WSTRB, WLAST | Output |
| B | BVALID, BREADY, BID, BRESP | Input |
| AR | ARVALID, ARREADY, ARADDR, ARID, ARLEN, ARSIZE, ARBURST, … | Output |
| R | RVALID, RREADY, RDATA, RID, RRESP, RLAST | Input |

## 12.2	Signal Naming Convention

### 12.2.1	Custom Prefixes

Each port uses a custom prefix for signal naming:

// Example: Master 0 with prefix "cpu_m_axi_"
input  logic [31:0] cpu_m_axi_awaddr,
input  logic [3:0]  cpu_m_axi_awid,
input  logic        cpu_m_axi_awvalid,
output logic        cpu_m_axi_awready,
// ... remaining AW signals

// Example: Slave 0 with prefix "ddr_s_axi_"
output logic [31:0] ddr_s_axi_awaddr,
output logic [4:0]  ddr_s_axi_awid,  // {BRIDGE_ID, id}: master width + clog2(NUM_MASTERS)
output logic        ddr_s_axi_awvalid,
input  logic        ddr_s_axi_awready,

### 12.2.2	ID Width Extension

With more than one master, slave-side IDs are WIDER than master-side:

Master ID Width: id_width of the master port (widest master = MASTER_ID_WIDTH)
Bridge ID Width: clog2(NUM_MASTERS)            (ID_PREFIX_WIDTH; 0 for one master)
Slave ID Width:  MASTER_ID_WIDTH + ID_PREFIX_WIDTH  (XBAR_ID_WIDTH)

bridge_2x2_rw: cpu_m_axi_awid [3:0]  ->  ddr_s_axi_awid [4:0]

Each master adapter sends {BRIDGE_ID, id} into the fabric and strips the prefix from B and R. A slave port’s declared id_width must be at least XBAR_ID_WIDTH; the validator rejects a narrower AXI3/AXI4/AXI5 slave. Responses are routed by the bridge_id sideband the slave adapter stores: a bridge_cam looked up by BID/RID for a multi-master AXI4/AXI5 slave (any completion order), or an in-order FIFO for single-master bridges and shim slaves (request order required, bridge BUG-008). See HAS 3.2.

Either way a slave returns each R burst contiguously: beats of different IDs must not interleave. The per-master-path R arbiters hold through RLAST, and the read width converters rely on it (they are built with R_INTERLEAVE = 0, without a reassembly pool, GitHub #119); the converter flags an interleaved beat in simulation.

## 12.3	Channel-Specific Interfaces

### 12.3.1	Write-Only Master (wr)

Only AW, W, B channels generated:

// Write Address Channel
input  logic [ADDR_WIDTH-1:0] descr_m_axi_awaddr,
input  logic [ID_WIDTH-1:0]   descr_m_axi_awid,
// ... full AW channel

// Write Data Channel
input  logic [DATA_WIDTH-1:0] descr_m_axi_wdata,
input  logic [STRB_WIDTH-1:0] descr_m_axi_wstrb,
// ... full W channel

// Write Response Channel
output logic [ID_WIDTH-1:0]   descr_m_axi_bid,
output logic [1:0]            descr_m_axi_bresp,
// ... full B channel

// NO AR or R channels

### 12.3.2	Read-Only Master (rd)

Only AR, R channels generated:

// Read Address Channel
input  logic [ADDR_WIDTH-1:0] src_m_axi_araddr,
input  logic [ID_WIDTH-1:0]   src_m_axi_arid,
// ... full AR channel

// Read Data Channel
output logic [DATA_WIDTH-1:0] src_m_axi_rdata,
output logic [ID_WIDTH-1:0]   src_m_axi_rid,
// ... full R channel

// NO AW, W, or B channels

## 12.4	Monitor Bus Output (when variants includes mon)

When monitor collection is enabled (via the bridge TOML variants list including "mon"), each bridge instance exposes a unified monitor interface at the bridge top level:

Monitor Bus Output

| Signal | Type | Width | Direction | Description |
| --- | --- | --- | --- | --- |
| s_mon_axil_* | AXIL Slave | 32-bit addr, 64-bit data | Input | Monitor packet read interface for CPU consumption |
| m_axil_mon_* | AXIL Master | 32-bit addr, 64-bit data | Output | Monitor packet write interface for bulk trace capture |
| stream_irq | logic | 1-bit | Output | Interrupt signal (asserted when error FIFO has records) |

Per-port collection comes from the axi4_master_{rd,wr}_monlite and axi4_slave_{rd,wr}_monlite wrappers (axi_monitor_lite; the full _mon wrappers are no longer used in generated bridges, 2026-09-26).

Packet Format: the canonical 128-bit packet layout and field definitions live in docs/markdown/rtl-amba/includes/ocelus_package_spec.md — see that reference for bit-field descriptions, protocol-type enumerations, and packet-type codes.

## 12.5	Handshake Protocol

### 12.5.1	Valid/Ready Handshake

All channels use AXI4 valid/ready handshake:

Transfer occurs when: VALID && READY

### 12.5.2	Backpressure

- Bridge asserts READY when able to accept

- Masters must hold VALID until READY

- Bridge does not drop transactions

## 12.6	Burst Support

### 12.6.1	Supported Burst Types

Supported Burst Types

| AWBURST/ARBURST | Type | Description |
| --- | --- | --- |
| 2’b00 | FIXED | Same address each beat |
| 2’b01 | INCR | Incrementing address |
| 2’b10 | WRAP | Wrapping at boundary |
| 2’b11 | Reserved | Not used |

### 12.6.2	Burst Length

- AWLEN/ARLEN: 8-bit field

- Length = AXLEN + 1 (1 to 256 beats)

- Burst does not cross 4KB boundary (AXI4 rule)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	APB Interface

## 13.1	Overview

APB is where the slow things live: UARTs, GPIO, configuration registers. This page covers the bridge’s APB surface — the signal set, the port-naming convention, transaction timing, and what the AXI4-to-APB conversion actually does.

## 13.2	Ports

### 13.2.1	Signal Definition

APB Signal Definitions

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| PSEL | 1 | Output | Slave select |
| PENABLE | 1 | Output | Transaction phase |
| PADDR | ADDR_WIDTH | Output | Address |
| PWRITE | 1 | Output | Write enable |
| PWDATA | DATA_WIDTH | Output | Write data |
| PSTRB | DATA_WIDTH/8 | Output | Byte strobes |
| PPROT | 3 | Output | Protection |
| PRDATA | DATA_WIDTH | Input | Read data |
| PSLVERR | 1 | Input | Slave error |
| PREADY | 1 | Input | Slave ready |

### 13.2.2	Signal Naming

APB slave ports use custom prefixes:

// Example: APB slave with prefix "uart_apb_"
output logic        uart_apb_psel,
output logic        uart_apb_penable,
output logic [31:0] uart_apb_paddr,
output logic        uart_apb_pwrite,
output logic [31:0] uart_apb_pwdata,
output logic [3:0]  uart_apb_pstrb,
output logic [2:0]  uart_apb_pprot,
input  logic [31:0] uart_apb_prdata,
input  logic        uart_apb_pslverr,
input  logic        uart_apb_pready

## 13.3	Functional Description

### 13.3.1	AXI4 to APB Conversion

When an AXI4 master accesses an APB slave, the bridge:

- Burst splitting - AXI4 bursts become multiple APB transfers

- Channel mapping - AW+W combined into PADDR+PWDATA

- Response generation - APB PSLVERR maps to AXI4 BRESP/RRESP

- Timing adaptation - Insert wait states as needed

### 13.3.2	Burst Handling

AXI4 to APB Burst Conversion

| AXI4 AWLEN | APB Transfers |
| --- | --- |
| 0 (1 beat) | 1 transfer |
| 1 (2 beats) | 2 transfers |
| N (N+1 beats) | N+1 transfers |

### 13.3.3	Error Mapping

APB to AXI4 Error Mapping

| APB PSLVERR | AXI4 BRESP/RRESP |
| --- | --- |
| 0 (OK) | 2’b00 (OKAY) |
| 1 (Error) | 2’b10 (SLVERR) |

## 13.4	Timing

Every APB transfer pays for a setup phase plus an access phase, so the floor is two cycles per transfer. There is no pipelining — transfers run sequentially — and no bursts: each beat requires the full handshake.

## 13.5	Waveforms

### 13.5.1	Write Transaction

#### 13.5.1.1	Figure 4.1: APB Write Transfer (Zero Wait States)

APB Write Transfer (Zero Wait States)

### 13.5.2	Read Transaction

#### 13.5.2.1	Figure 4.2: APB Read Transfer (One Wait State)

APB Read Transfer (One Wait State)

## 13.6	APB Requester Ports

Since bridge TASK-004 (was BRIDGE-014) an APB port can also be a master: protocol = "apb" or "apb5" in [[bridge.masters]]. The signal set is the same ten (fifteen for APB5) with the directions of the slave table reversed – the bridge is the completer, so PSEL, PENABLE, PADDR[31:0], PWRITE, PWDATA, PSTRB, PPROT are inputs and PREADY, PRDATA, PSLVERR outputs. PADDR is the full 32-bit fabric address (the validator insists on addr_width = 32); each transfer becomes one single-beat AXI4 transaction through apb4_to_axi4 / apb5_to_axi4, and both SLVERR and DECERR come back as PSLVERR. The requester is one-outstanding by nature of APB, so throughput is one transfer per fabric round trip.

## 13.7	Design Notes

- Use APB only for slow peripherals (UART, GPIO, config registers)

- Group APB slaves behind dedicated sub-crossbar

- Use AXI4/AXI4-Lite for higher-bandwidth slaves

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Clock and Reset

## 14.1	Overview

One clock, one reset. The AXI fabric runs in a single synchronous domain — the exception is an AXI4 port, master or slave, declared cdc = true, which runs on its own clock and gets its own discussion below — and the reset is asynchronous, active-low. This page covers what that means for your clock tree, your reset strategy, and your constraint file.

## 14.2	Ports

### 14.2.1	Clock

Clock Requirements

| Signal | Description | Requirements |
| --- | --- | --- |
| aclk | System clock | All interfaces synchronous |
| Frequency | Operating frequency | Design-dependent |
| Duty cycle | Clock duty cycle | 40-60% typical |

### 14.2.2	Reset

Reset Signal

| Signal | Description | Polarity |
| --- | --- | --- |
| aresetn | Async reset | Active-low |

## 14.3	Functional Description

### 14.3.1	One Fabric Clock

#### 14.3.1.1	Figure 4.3: Clock Distribution

Clock Distribution

The fabric itself runs on aclk. An AXI4 master or slave port may run on its own clock when declared cdc = true (next sections); every other port shares aclk.

### 14.3.2	Clock Domain Crossing: CDC Slave Ports

The FABRIC is one clock domain (aclk). Since bridge TASK-006 (was BRIDGE-017) a slave port can live in another: cdc = true on an AXI4 slave adds <slave>_aclk / <slave>_aresetn to the bridge top, and the slave adapter carries AW, W and AR to the port and B, R back through axi4_cdc_{wr,rd} – one Gray-pointer async FIFO per channel (rtl/amba/axi4/axi4_cdc.md) – downstream of its timing wrapper, which stays on aclk with its monitor and the bridge-id tracking. Everything upstream of the crossing is unchanged; the port itself runs entirely on its own clock, at one beat per cycle of the slower domain.

CDC slave port clock pins (cdc = true)

| Signal | Direction | Description |
| --- | --- | --- |
| <slave>_aclk | Input | the CDC slave port’s clock |
| <slave>_aresetn | Input | its active-low reset; release together with aresetn (a one-sided reset is not safe on the async FIFOs – handbook design/cdc.md) |

Limits: protocol = "axi4" (the crossing carries the AXI4 signal set; AXI5 sideband and the APB/AXIL/WB4 shims have no crossing yet – the validator rejects those), and the port keeps latency = "cut_all". Measured on bridge_2x2_rw_cdc (test_bridge_2x2_rw_cdc_ratio) with the slave clock at 3, 10 and 23 ns against a 10 ns fabric: every burst from both masters completes, the port’s beats appear on the slave clock, the sustained rate is set by the slower clock (>= 0.85 beat per cycle of it), and the same-clock port beside it is unaffected.

test_bridge_perf_char characterizes the port at 7 ns and at 13 ns (HAS 5.1, Bandwidth Characterization). Bursts of two beats or more run at the slower side’s full rate: 0.700 of the 7 ns slave’s cycles, which is the fabric’s one beat per 10 ns, and 1.000 of the 13 ns slave’s. The crossing adds about 13 fabric cycles to a round trip, two synchronizer crossings each way, so single-beat traffic needs more in flight to fill the port. One open limit (W9): the address FIFOs queue granted requests between the crossbar’s arbiter and the slave, so round-robin shares at a CDC slave follow each master’s outstanding count once the slave throttles.

### 14.3.3	Clock Domain Crossing: CDC Master Ports

The same crossing works on the request side (GitHub #119 L2): cdc = true on an AXI4 master adds <master>_aclk / <master>_aresetn to the bridge top, and the master adapter carries AW, W and AR from the port and B, R back to it through axi4_cdc_{wr,rd} AHEAD of its timing wrapper. The crossing’s write side runs on the port clock, its read side on aclk; the timing wrapper, its monitor, the address decode, the per-ID target tracking and the width paths all stay on aclk and see an ordinary same-clock master. A wr or rd port builds only the crossing for its direction. A transaction from a CDC master to a CDC slave crosses twice each way, once at each adapter.

CDC master port clock pins (cdc = true)

| Signal | Direction | Description |
| --- | --- | --- |
| <master>_aclk | Input | the CDC master port’s clock |
| <master>_aresetn | Input | its active-low reset; release together with aresetn |

The limits are the slave port’s: protocol = "axi4" and latency = "cut_all". An APB, AXI4-Lite, WB4, AXI3 or AXI5 requester on a foreign clock still needs a crossing outside the bridge. Measured on bridge_3x2_rw_mcdc (test_bridge_3x2_rw_mcdc_ratio: CDC masters cpu, rw, and lfsr, rd only, beside the same-clock dma, against the CDC slave ddr and the same-clock sram) with cpu_aclk at 3, 10 and 23 ns and lfsr_aclk 2 ns slower, against a 10 ns fabric:

- every burst completes and every word lands and reads back, including cpu into ddr, which crosses twice each way;

- the ports’ beats appear on their own clocks;

- the rate is set by the slower clock, not the crossing. cpu alone into sram moves 1.000 W beat per aclk cycle when it is the faster side and 0.906 per cpu_aclk cycle when it is the slower; lfsr reads at 1.000 per lfsr_aclk cycle when slower and 0.501 per 5 ns cycle (the 10 ns fabric) when faster. 0.906 is the requester BFM’s own gap between bursts, the same figure a same-clock master gives; behind the async FIFO the fabric takes 1.000;

- the same-clock dma into sram runs at 0.906 alone and 0.906 with cpu’s two-crossing traffic running beside it.

An apb/apb5 slave’s axi4_to_apb4_shim core is built around two async FIFOs and has its own pclk/presetn pins, but the generator ties them to aclk/aresetn (periph_adapter.sv in bridge_2x3_apb_req), and the bridge top has no APB clock port. A generated APB slave port therefore runs on aclk. For every port that is not cdc = true:

- All inputs sampled on aclk rising edge

- All outputs generated on aclk rising edge

- External CDC required if a non-AXI4 port uses a different clock

### 14.3.4	Reset Behavior

#### 14.3.4.1	Figure 4.4: Reset Timing

Reset Timing

Reset is active-low: the bridge is in reset whenever aresetn = 0.

### 14.3.5	Reset Effects

During reset (aresetn = 0):

Reset Effects

| Component | State |
| --- | --- |
| Arbiters | Cleared (no grants) |
| bridge_id FIFOs | Pointers cleared (no outstanding) |
| FIFOs | Emptied |
| Outputs | Deasserted (VALID = 0) |

### 14.3.6	Outstanding Transactions

Warning: reset discards transactions in flight:

- Master may see timeout (no response)

- Slave may receive partial transaction

- System must ensure quiescence before reset

### 14.3.7	Reset Release

After reset release (aresetn = 1):

- Bridge accepts new transactions after 1 cycle

- All state machines start in IDLE

- No residual grants or locks

## 14.4	Timing

### 14.4.1	Setup and Hold

All inputs must meet setup/hold relative to aclk:

Timing Constraints

| Constraint | Typical Value |
| --- | --- |
| Setup time | Process-dependent |
| Hold time | Process-dependent |

### 14.4.2	Clock-to-Output

All outputs appear after aclk rising edge:

Clock-to-Output Delays

| Path | Typical Delay |
| --- | --- |
| aclk to VALID | 1 cycle (registered) |
| aclk to READY | 1 cycle (registered) |
| aclk to DATA | 1 cycle (registered) |

READY is registered like the rest, which an earlier revision of this table gave as “Combinational”. Every external port terminates in a gaxi_skid_buffer inside its boundary wrapper – a master port’s awready is axi4_slave_wr.s_axi_awready, wired straight to the skid’s wr_ready, and that is assigned in an ALWAYS_FF_RST block. A master budgeting a combinational ready path against this bridge would be wrong by a register stage.

## 14.5	Usage Example

### 14.5.1	Recommended Constraints

# Example SDC constraints
create_clock -period 10 [get_ports aclk]  ;# 100 MHz

# Reset is async - synchronize externally
set_false_path -from [get_ports aresetn]

## 14.6	Design Notes

### 14.6.1	External CDC Required

For ports that are not cdc = true AXI4 ports, in systems with multiple clock domains:

#### 14.6.1.1	Figure 4.5: Multi-Clock CDC

Multi-Clock CDC

### 14.6.2	CDC Recommendations

- Use AXI4 async FIFO bridges

- Consider AXI4 clock converter IP

- Ensure proper gray-coding for pointers

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	AXI5 and APB5 Interfaces (AMBA5 Support)

## 15.1	Overview

The bridge is AXI4-shaped internally with every AXI5 feature the library wrappers carry riding alongside in the channel structs — since bridge TASK-007 (was BRIDGE-018) that includes Memory Tagging and read-data chunking, so there is no AXI5 signal an axi5 port can present that the fabric cannot carry natively to another axi5 port. Any master or slave port can be declared AMBA5. This chapter covers the external surfaces; the mechanism lives in the MAS (AMBA5 Boundary and Native Sideband).

## 15.2	Parameters

### 15.2.1	Declaring AMBA5 Ports

AMBA5 is declared per port, in the TOML:

[[bridge.masters]]
name = "cpu_wr"
protocol = "axi5"
axi5_features = ["trace", "atomic"]   # optional; empty = base AXI5
# ... standard fields unchanged

[[bridge.slaves]]
name = "periph5"
protocol = "apb5"                      # APB5 peripheral via the apb5 shim
channels = "rw"                        # APB rules unchanged (rw-only, 32-bit)

[[bridge.masters]]
name = "lite5"
protocol = "axil5"                     # AXI5-Lite requester (bridge TASK-004, was BRIDGE-014)
id_width = 1                           # AWID/ARID: required on an AXI5-Lite Subordinate
axi5_features = ["user"]               # the one group with an AXI4 destination

[[bridge.masters]]
name = "apb5m"
protocol = "apb5"                      # APB5 requester (bridge TASK-004, was BRIDGE-014)
id_width = 0
addr_width = 32                        # the requester addresses the whole fabric
channels = "rw"

## 15.3	Ports

### 15.3.1	AXI5 Port Surface

An axi5 port exposes the AXI4 signal set minus AW/ARREGION (AXI5 removed REGION) plus the enabled features’ sideband signals. Disabled features’ signals are not exposed at all — the boundary wrapper ties them off internally.

AXI5 Port Surface

| Feature | Signals added | Class |
| --- | --- | --- |
| nsaid | aw/arnsaid[3:0] | Droppable sideband |
| trace | aw/artrace, btrace, rtrace | Droppable sideband |
| mpam | aw/armpam[10:0] | Droppable sideband |
| mecid | aw/armecid[15:0] | Droppable sideband |
| unique | aw/aridunq in; bidunq, ridunq out (IHI 0022L A5.2) | Droppable sideband; the response indicators are regenerated at the master port (below) |
| wakeup | awakeup (one signal for both request channels, present on rd-only, wr-only and rw ports alike) | Not path sideband; see “AWAKEUP” below |
| poison | wpoison, rpoison | Connectivity-gated |
| atomic | awatop[5:0] | Connectivity-gated; read-return classes native on rw ports, DECERR on write-only ports |
| mte | aw/artagop[1:0], wtag, wtagupdate, btagmatch[1:0], rtag (tag buses are 4 bits per 16 bytes of data, wtagupdate one bit per 16 bytes; IHI 0022L A12.2 has no AWTAG, BTAG or RTAGMATCH, and BTAGMATCH encodes Table A12.14: 00 none, 01 separate, 10 fail, 11 pass) | Connectivity-gated; 128-bit ports or wider (bridge TASK-007, was BRIDGE-018) |
| chunking | archunken, rchunkv, rchunknum[3:0], rchunkstrb (one bit per 128 bits of data) | Droppable sideband; 128-bit ports or wider (bridge TASK-007, was BRIDGE-018) |

### 15.3.2	APB5 Slave Surface

An apb5 slave exposes the APB4 requester surface plus the APB5 sideband, mirroring rtl/amba/apb5/apb5_slave.sv pin-for-pin:

APB5 Slave Surface

| Direction | Signals |
| --- | --- |
| Requester → completer | APB4 set + PAUSER, PWUSER (driven '0 — nothing upstream sources them) + PWAKEUP (below) |
| Completer → requester | APB4 set + PRUSER, PBUSER (accepted and terminated) |

PWAKEUP is a requester output (IHI 0024E 3.7.2, Table 3-7). The bridge port used to take it as an input, following a library convention that had the direction backwards (GitHub #102 item 7). axi4_to_apb5_shim now drives it from a flop of PSEL: glitch-free, high from the cycle after the Setup phase and held through the PREADY cycle, so a completer that waits for PWAKEUP before PREADY always receives it, and low when no transfer is in progress. 3.7.2 permits assertion during or after PSEL; the recommended one-cycle lead is not available because the command queue that would predict a transfer is inside the shared APB4 conversion core. No *CHK check signals are exposed (the port is Check_Type = False).

The transfer protocol is unchanged from APB4, so the axi4_to_apb5_shim is a sideband wrapper over the APB4 conversion core; APB constraints (rw-only, 32-bit data) apply unchanged.

### 15.3.3	AXI5-Lite Master Surface

An axil5 master port (bridge TASK-004, was BRIDGE-014) exposes the AXI4-Lite requester set, the AXI5-Lite ID (awid/arid in, bid/rid out, id_width bits – required on an AXI5-Lite Subordinate, IHI 0022L B2.1.5, so id_width must be at least 1), and the whole AXI5-Lite optional set, every group whether or not it is enabled – the same rule as the slave surface, so the boundary keeps one shape. The directions flip relative to the slave table: requester-driven groups are bridge inputs, completer-driven groups bridge outputs.

AXI5-Lite sideband groups at an axil5 master boundary

| Group | Signals | Behaviour |
| --- | --- | --- |
| ID | awid, arid in; bid, rid out | reach the adapter’s AXI4 face, ride the fabric inside {master index, id} and come back unchanged |
| FORWARDED (when enabled) | awuser, wuser, aruser (user) | reach the adapter’s AXI4 face and ride the fabric to the slave |
| TERMINATED inputs | aw/artrace, wpoison; awuser/… when user is off | consumed at the bridge top (a reduction into an _unused wire), never floating |
| TERMINATED outputs | btrace, rtrace, rpoison; buser/ruser when user is off | driven '0 |
| Response USER | buser, ruser with user on | connected, but read 0: the master adapters tie response-side USER (see the PRD) |

### 15.3.4	APB5 Master Surface

An apb5 master port makes the bridge the APB5 completer: the APB4 set with the requester-driven signals as inputs and PREADY/PRDATA/PSLVERR as outputs, plus PAUSER, PWUSER, PWAKEUP in and PRUSER, PBUSER out, one bit each (the fabric USER width). PAUSER[0]/PWUSER[0] become awuser/aruser/wuser on the fabric – observable at an axil5 slave with user enabled. PWAKEUP is requester-driven (an input here, since the bridge is the completer) and is accepted and terminated: the bridge never waits for it. Behind the surface sits apb5_to_axi4 (apb4_to_axi4 for protocol = "apb"), then the same AXI4 timing wrapper an AXI4 master port gets; from there the port is an AXI4-Lite-shaped single-beat requester and takes the wide-slave aligner toward wider slaves. An unmapped address answers PSLVERR (the subtractive slave’s DECERR folds into APB’s one error bit).

### 15.3.5	AXI5-Lite Slave Surface

An axil5 slave gets axi4_to_axil5_{rd,wr} at the boundary: the AXI4-Lite conversion core plus the AXI5-Lite ID and optional signals. The signal set is IHI 0022L B2.1.5 / Table B2.2: AXI5-Lite has no AxLOCK (no exclusive access), AxMPAM, AxMECID, AxNSAID or xLOOP – the bridge carried all five until GitHub #102 item 3 – and it does carry AWID/BID and ARID/RID. Because the master side is AXI4, the rest splits three ways, and which group a signal falls in is the whole story of what an AXI4 front end can and cannot express.

AXI5-Lite sideband groups at an axil5 slave boundary

| Group | Signals | Behaviour |
| --- | --- | --- |
| ID | awid, arid (request); bid, rid (response) | The fabric ID ({master index, id}, the crossbar ID width), carried across the converter and returned unchanged. |
| FORWARDED | awuser, wuser, aruser (request); buser, ruser (response) | AXI4 has an equivalent, passed straight through and returned to the master. AXI4 AxLOCK has nowhere to go: an exclusive access completes as a normal one with OKAY. |
| TIED | awtrace, wpoison, artrace | AXI5-Lite optional signals with no AXI4 source. The port exists and is driven to '0 — never left floating. There is deliberately no ENABLE_ knob: there is nothing it could switch between. |
| TERMINATED | btrace, rtrace, rpoison | Completer-driven, with nothing on the AXI4 side to return them to. Accepted and dropped. |

## 15.4	Functional Description

Droppable sideband passes natively end-to-end when both ends of a path are AXI5, feature-enabled, and width-matched; on any other path it terminates at the fabric boundary with a generation-time warning.

Connectivity-gated features are a config error unless every connected path is native (AXI5 both ends, feature enabled both ends, data widths matched — dwidth converters cannot carry per-beat sideband). Dropping POISON silently would launder corrupted data; dropping ATOP would turn an atomic into a plain write; dropping a tag operation would turn a tagged write into an untagged one and a Match into a plain access, with the requester none the wiser — so mte is gated the same way.

Chunking is permission, not demand. ARCHUNKEN tells the completer it may return the burst in chunks; a completer that cannot simply returns ordered data with RCHUNKV low, and every chunking requester must accept that. So chunking is droppable: toward an AXI4 slave, or through a width converter, the enable terminates with a generation-time warning and the read completes unchunked. Toward an AXI5 slave that chunks, the enable and the chunk fields ride the structs natively; the fabric never looks inside them — it routes each R beat by ID and frees its tracking on RLAST exactly as for an ordered burst, so chunks may arrive in any order the completer chooses.

Both need a 128-bit bus. Tags are one per 16 bytes and chunks are 128 bits; the validator rejects mte or chunking on a narrower port (bridge_2x2_axi5_native and bridge_1x2_rd_axi5c are the 128-bit fixtures; MAS 7.1a lists what their tests check).

Atomics depend on the port having a read path. AWATOP = 01xxxx (AtomicStore) and plain writes forward natively on any atomic-enabled port. Read-return classes (AtomicLoad 10xxxx, AtomicSwap/Compare 11000x) answer on the R channel with the AW’s ID:

- On an rw master port they forward natively. The slave performs the operation and returns the location’s original data on R; the bridge routes that beat back by ID (axi5_atomic_rr_tracker at the slave adapter, an extra AR->R tracking slot at the master adapter). Every connected atomic slave must be rw – the validator rejects a write-only one, since it could never return the data.

- On a write-only master port there is nowhere to deliver the R beat, so the boundary’s axi5_atomic_filter answers those classes locally with DECERR – no slave-side AW handshake, no memory side effect. IHI 0022L A6.4.8 also requires the read data transfers to be sent even when the write response is an error; the filter generates them (GitHub #102 item 2), but a write-only port has no R channel to carry them, so the bridge accepts and discards them inside the filter (its s_rready is tied high, which also keeps its read-return queue from filling and stalling AW). The atomic therefore completes with DECERR on B only. A manager that issues a load-class atomic on a write-only port is outside the spec – such a port has no read data channel to receive the result – and must not issue them; the DECERR exists so a stray one cannot hang the port. To use load-class atomics, declare the port rw.

Unique-ID indicators are regenerated, not routed back. IHI 0022L A5.2 makes BIDUNQ equal the request’s AWIDUNQ, RIDUNQ equal ARIDUNQ, and an atomic’s RIDUNQ equal its AWIDUNQ. The request indicator is droppable sideband: an AXI4 slave, a dwidth converter, the subtractive DECERR slave and the atomic filter’s local response never see it. So the response indicators do not ride the fabric; each AXI5 master port with unique keeps one bit per ID, written at every AW (and AR, and read-return atomic AW) handshake and read by BID/RID. A5.2 forbids an AxIDUNQ=1 request while any same-ID request is outstanding, and any request reusing an outstanding AxIDUNQ=1 ID, so every outstanding transaction of one ID carries the same indicator and the per-ID bit is exact under any response order. On a write-only atomic port the table sits on the wrapper side of the filter, so a local DECERR carries the right bit.

AWAKEUP (IHI 0022L A14.1.1, feature wakeup). A master port accepts AWAKEUP and does not wait for it – the bridge is always clocked, and a Subordinate is permitted, not required, to wait. A wake-capable AXI5 slave port drives AWAKEUP as the OR of (a) every connected wake-capable master’s request – the master’s AWAKEUP OR’d with requests held in its boundary wrapper – so the slave can wake before a request reaches it, which is the A14.1.1 recommendation; the bridge cannot know which slave a request decodes to before it arrives, so every connected wake-capable slave sees it; and (b) the slave wrapper’s own held requests, so AWAKEUP is high on every cycle the port presents AWVALID, WVALID or ARVALID and stays high until the handshake, as A14.1.1 requires – including traffic from a master without AWAKEUP (A14.1.1’s “derive from AxVALID” fallback). Every term is a flop or a top-level AWAKEUP input, so the OR is glitch-free (A14.1). It falls when nothing is requested. The read and write halves of a port each produce a copy and the port ORs them: AWAKEUP is one signal for both request channels.

The tied group is the honest limit of an AXI4 front end. MPAM partition IDs, MECID encryption contexts and NSAID security IDs are properties of the originating master; a bridge whose masters speak AXI4 has nowhere to learn them, so an axi5 slave reached from an axi4 master sees zeros rather than invented values. If a design needs real MPAM or MECID at the slave, the master port has to be axi5, not axi4. (AXI5-Lite has no MPAM, MECID or NSAID at all.)

Sideband is held for the whole burst, not just the address beat. AXI4 presents AWID/AWUSER once, with AW; AXI5-Lite needs them stable across every beat that follows. The shims capture on the address handshake and hold:

wire w_aw_accept = s_axi_awvalid && s_axi_awready;
wire [AXI_USER_WIDTH-1:0] w_awuser_sel = w_aw_accept ? s_axi_awuser
                                                     : r_held_awuser;

A combinational passthrough here is a real defect rather than a style question: with two bursts in flight, burst A’s later beats would carry burst B’s AWUSER.

### 15.4.1	Interop Matrix

Interop Matrix

| Master Slave | axi4 | axi5 | apb / apb5 / axil | axil5 |
| --- | --- | --- | --- | --- |
| axi4 | native | base subset | via shim | via shim, sideband TIED to '0 |
| axi5 | sideband drops (warning) | native sideband when width-matched | sideband drops (warning) | forwarded where AXI4-expressible |
| axil / axil5 | single-beat AXI4 | single-beat AXI4 | via shim | ID and user forwarded end to end (axil5 master); rest tied |
| apb / apb5 | single-beat AXI4 | single-beat AXI4 | via shim (APB in, APB out) | PAUSER[0] visible as awuser (apb5 master) |

Connectivity-gated features tighten the axi5→axi5 cell: they require it.

# 16	Unmapped-Address Handling

## 16.1	Overview

### 16.1.1	Why the bridge answers an address no slave owns

A positively-decoded fabric answers only addresses that fall inside some slave’s range. An address in none of them selects nothing: the one-hot select is all-zero, no slave sees AWVALID/ARVALID, READY never rises, and the master waits forever.

A hang is the worst failure available here, because it destroys the evidence. There is no response to inspect, no error bit to read, and the offending address is whatever the master still has latched. A stray pointer in software becomes a wedged interconnect with nothing to point at the cause.

The bridge therefore ends its decode chain in an else – subtractive decode – and that else selects an internal slave that always answers.

## 16.2	Ports

### 16.2.1	Status, interrupt and clearing

The catch-all latches its status. Every bridge exposes:

Unmapped-address status and interrupt ports

| Port | Dir | Meaning |
| --- | --- | --- |
| unmapped_irq | out | sticky: set by the first unmapped access, held until cleared |
| unmapped_addr[31:0] | out | address of that first access |
| unmapped_count[7:0] | out | number of unmapped accesses, saturating at 255 |
| unmapped_clear | in | pulse to clear the flag and the count |

Three choices worth knowing about:

- Sticky, not a pulse. A one-cycle pulse is gone before software can look, and this is precisely the access nobody expected.

- The first address wins. A later fault does not overwrite it; the first one is usually what explains the rest.

- The address survives the clear, so a late reader still learns where the fault was after the flag is acknowledged. The count saturates rather than wrapping – “255+” is honest, a wrapped 3 is not.

## 16.3	Functional Description

### 16.3.1	Behaviour

Subtractive-slave response behaviour

| Write | every W beat is accepted; one B per AW |
| --- | --- |
| Read | AxLEN+1 beats, RLAST on the last |
| Response | DECERR on both, always |
| Read data | 0xDEADBEEF, replicated to the bus width |

DECERR rather than SLVERR is the AXI4 code for “no slave at this address”: nothing failed to service the request, there was nothing there to service it. There is no OKAY mode and no parameter to select one – a mode switch here is where lint, simulation and synthesis end up disagreeing about what the fabric does.

The read data is a recognisable pattern rather than zeros because zeros are indistinguishable from real memory. 0xDEADBEEF in a dump is a fault; 0 is a plausible value.

Write data is discarded. There is nowhere for it to go, and inventing a destination would be worse than saying so.

### 16.3.2	Register access (cfg regblock builds)

Bridges built with the cfg register block also expose the status over the cfg AXI4-Lite window. The port is s_cfg_axil_* – AXI4-Lite, not APB. An APB-attached CPU reaches it through an axi4_to_apb4_shim upstream of the bridge, which is an integration choice and not part of this module:

Unmapped-address status registers

| Register | Field | Access | Meaning |
| --- | --- | --- | --- |
| SUBTRACTIVE_STATUS | HIT[0] | RO | mirrors unmapped_irq |
|  | CLEAR[1] | W | write 1 to clear the flag and count |
|  | COUNT[15:8] | RO | mirrors unmapped_count |
| SUBTRACTIVE_ADDR | ADDR[31:0] | RO | mirrors unmapped_addr |

The sticky state lives in the RTL, not in the register block: one owner for one piece of state, so the two cannot disagree about whether a fault happened. These registers are a window onto it plus a clear. Clearing works from either the unmapped_clear pin or a CLEAR write; neither disables the other.

These registers are appended after every pre-existing register, so adding them did not move any existing offset. That is deliberate: a register map is an ABI. An earlier attempt inserted per-slave configuration for the catch-all midway through the map, which renumbered everything after it, silently moved the monitor-enable bits, and left the monitors switched off while every functional test still passed.

### 16.3.3	When it can never fire

The catch-all is the else of the decode chain, so it is reachable only if the slave ranges leave a gap. A map that tiles the whole address space has no unmapped address to catch, and the else is unreachable logic that synthesis removes – it costs nothing and reports nothing.

Most shipped variants are in exactly that position. Of the 22 generated bridges, 18 tile the space completely and 4 leave gaps. bridge_2x2_rw is typical of the first group: ddr takes <= 0x7FFF_FFFF and sram takes >= 0x8000_0000, between them the entire 32-bit range.

This is worth knowing before wiring unmapped_irq to anything: on a fully-tiled bridge it is tied low by construction, and a test that expects it to assert will wait forever. Check your own address map, not this table – it changes with the TOML.

The insurance is still worth having. A map is tiled today; the next slave added, or the next range narrowed, opens a gap, and the difference between “reports it” and “hangs the master” is decided at that moment rather than at integration time.

### 16.3.4	What this does not do

The catch-all does not make an unmapped access correct. It makes it visible and survivable: the master gets an error instead of stalling, and software gets an address instead of a puzzle. An address that reaches it is still a configuration or software fault.

## 16.4	Navigation

Previous: AXI5 and APB5 Interfaces

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Wishbone B4 Interface

## 17.1	Overview

Wishbone B4 is the open bus of the FPGA world – the fabric OpenCores cores speak and the one most soft CPUs ship with. Since bridge TASK-008 (was BRIDGE-019) a bridge port can be Wishbone on either side: a slave port with protocol = "wb4" makes the bridge the Wishbone requester toward an external completer; a master port with protocol = "wb4" makes the bridge the completer for an external Wishbone requester. The fabric between stays AXI4.

The mode is B4 pipelined – a new STB every clock while STALL is low, in-order termination – which is what rtl/amba/wb4 and the converters implement. B4 standard (“classic”) mode exists in the converters as a parameter but is not selectable from the TOML; the two modes do not mix on one bus, so a classic peer needs a classic bridge build.

## 17.2	Ports

### 17.2.1	Signal Definition

Wishbone B4 Signal Definitions

| Signal | Width | Slave port (bridge requests) | Master port (bridge completes) | Description |
| --- | --- | --- | --- | --- |
| CYC | 1 | Output | Input | Bus cycle in progress |
| STB | 1 | Output | Input | Transfer request |
| WE | 1 | Output | Input | 1 = write, 0 = read |
| ADR | ADDR_WIDTH | Output | Input | Byte address (the fabric address, unwindowed) |
| DAT_W | DATA_WIDTH | Output | Input | Write data (the spec’s DAT_O from the requester) |
| SEL | DATA_WIDTH/8 | Output | Input | Byte lane select = AXI WSTRB |
| CTI | 3 | Output | Input | Burst hint (carried, not acted on; CLASSIC when the bridge drives it) |
| BTE | 2 | Output | Input | Burst type extension (LINEAR when the bridge drives it) |
| STALL | 1 | Input | Output | Pipelined back-pressure |
| ACK | 1 | Input | Output | Normal termination |
| ERR | 1 | Input | Output | Error termination |
| RTY | 1 | Input | Output | Retry termination (never driven by the bridge) |
| DAT_R | DATA_WIDTH | Input | Output | Read data (the spec’s DAT_I at the requester) |

### 17.2.2	Signal Naming

Wishbone ports use the TOML prefix followed by the uppercase B4 name, the convention rtl/amba/wb4 and the RDS-DV BFMs share:

// Slave port, prefix "wbp_wb_" -- the bridge drives the bus
output logic        wbp_wb_CYC,
output logic        wbp_wb_STB,
output logic        wbp_wb_WE,
output logic [31:0] wbp_wb_ADR,
output logic [31:0] wbp_wb_DAT_W,
output logic [3:0]  wbp_wb_SEL,
output logic [2:0]  wbp_wb_CTI,
output logic [1:0]  wbp_wb_BTE,
input  logic        wbp_wb_STALL,
input  logic        wbp_wb_ACK,
input  logic        wbp_wb_ERR,
input  logic        wbp_wb_RTY,
input  logic [31:0] wbp_wb_DAT_R

A master port has the same thirteen with the directions reversed.

## 17.3	Functional Description

### 17.3.1	Slave port: AXI4 to Wishbone

axi4_to_wb4 (converters MAS, chapter 3) sits in the slave adapter: the AXI4-Lite decomposers turn each AXI4 burst into one single-beat transfer per beat, and axil4_to_wb4 turns those into Wishbone transfers, in order. WSTRB becomes SEL. Termination maps back as:

Wishbone termination to AXI4 response

| Wishbone | AXI4 response |
| --- | --- |
| ACK | OKAY |
| ERR | SLVERR |
| RTY | SLVERR (RTY_RESP; AXI has no retry) |

The monitored (mon) variant places the axi4_master_*_monlite wrappers between the crossbar and the converter, exactly as for APB slave ports.

### 17.3.2	Master port: Wishbone to AXI4

wb4_to_axi4 sits in the master adapter in front of the ordinary AXI4 timing wrapper: wb4_to_axil4 merges AXI’s two response channels back into Wishbone’s single in-order termination stream, and the wrapper promotes the AXI4-Lite face to AXI4 (AxLEN = 0, full-width AxSIZE, INCR, WLAST = 1, a constant ID – the fabric prepends the master index, bridge TASK-005 (was BRIDGE-016)). From the wrapper on, the port is an AXI4-Lite-shaped single-beat requester: decode, the width converters, the wide-slave aligner toward wider slaves and the response mux are the ones every other master uses. SLVERR and DECERR both terminate ERR – an unmapped address (the subtractive slave) is indistinguishable from a failing completer on this bus. RTY is never generated.

### 17.3.3	Rules the validator enforces

- channels = "rw" (one bus carries both directions), id_width = 0.

- data_width is 8, 16, 32 or 64.

- A master port has addr_width = 32: ADR is the full fabric address, where an APB or Wishbone slave port’s address is whatever the fabric forwards for its window.

## 17.4	Design Notes

- In-order by construction. Wishbone terminates in issue order and the bridge’s slave-side tracking is in-order for shim-converted slaves, so a Wishbone completer never needs the CAM. Two masters sharing a Wishbone completer are serialised by the crossbar’s arbitration and answered in that order.

- One transfer per beat. A 16-beat AXI4 burst is sixteen Wishbone transfers; the pipelined bus takes one per clock when the completer does not stall, so the cost is latency, not bandwidth.

- Burst hints are not generated. CTI/BTE are CLASSIC/LINEAR on a slave port; a Wishbone completer that needs registered-feedback bursts to perform will not see them from the bridge.

- Best effort, by decision (2026-09-13). The Wishbone ports are spec-legal and regress green, and that is the accepted state: forming pipelined Wishbone bursts from AXI bursts and putting a Wishbone slave in its own clock domain (cdc is AXI4-only) are not owed. They cost only Wishbone throughput nobody has asked for; a consumer that needs either files a task against that need.

Verified by dv/tests/test_bridge_2x2_wb4_paths.py on bridge_2x2_wb4 (error folding both ways, SEL lanes at both a Wishbone and a 64-bit AXI4 completer, burst decomposition, both requesters in flight at the Wishbone completer) plus the generated tests of that fixture.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Slave Watchdog

## 18.1	Overview

A slave that stops answering hangs every master with a transaction outstanding at it, and the masters behind those. A slave port declared with timeout_cycles = N (AXI4 slaves; 0, the default, means none) puts axi4_slave_watchdog (rtl/amba/axi4/) in front of it. While the slave answers, the watchdog is a zero-latency pass-through. If the slave is silent for N cycles with work outstanding, the watchdog cuts it off and completes everything itself with DECERR (GitHub #119 L3).

## 18.2	Ports

Slave watchdog status (one per slave with timeout_cycles > 0)

| Port | Dir | Meaning |
| --- | --- | --- |
| <slave>_timeout | out | sticky: the watchdog tripped; held until reset |

## 18.3	Functional Description

### 18.3.1	When it trips

The timer counts only while there is work outstanding (a transaction the slave accepted and has not finished, owed W beats, or a request it has not accepted) and the slave is the blocker: no B or R VALID from it and no handshake on any channel. Any handshake with the slave restarts the count. A slow slave that is alive never trips it, and a response held off because the bridge is not ready does not count against the slave.

### 18.3.2	What it does then

- The slave is cut off. It sees no VALID from then on (a request it had not accepted is withdrawn) and BREADY and RREADY high, so late responses drain and are dropped.

- Everything outstanding is answered DECERR. Each write gets one B once its W burst is complete; owed W beats are accepted and discarded. Each read gets its remaining beats, so a burst the slave had half returned is finished, not restarted, and RLAST lands on the right beat. Responses go out oldest-of-their-ID first, so per-ID order holds.

- New requests are answered the same way, as the subtractive slave answers an unmapped address: DECERR, and 0xDEADBEEF read data.

- No response is withdrawn. The trip is taken only in a cycle with no B or R VALID from the slave, so the masters never see a response vanish.

It tracks up to the slave’s max_outstanding transactions per direction, and the slave’s AWREADY/ARREADY seen by the fabric drop while that is full. It sits on aclk, between the slave adapter’s timing wrapper and the port, or the clock crossing on a cdc = true port.

### 18.3.3	Measured

bridge_2x2_rw_wdog (ddr behind a 64-cycle watchdog), test_bridge_2x2_rw_wdog_trip:

- a ddr answering 20 cycles late passes through untouched;

- with ddr hung on a write and a 4-beat read from two masters, both complete with DECERR 80 and 81 cycles after issue (64 plus the pipeline), and ddr_timeout rises;

- a new write and read to ddr are answered at once; sram, beside it, still reads back what was written;

- the protocol checkers on every port saw no violation throughout.

## 18.4	Design Notes

- Assumes W never leads its AW (the crossbar’s W steering guarantees it) and that the slave returns R bursts contiguously (PRD FR-9).

- Recovery is by reset. A slave that has been cut off is presumed dead; there is no path back to it short of reset.

- The AXI rule that VALID holds until READY is broken on the slave side when the trip withdraws a request the slave had not accepted. That is the slave the watchdog has declared dead.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Throughput Characteristics

## 19.1	Overview

What the fabric can move when everything goes right, and what eats into that number when it doesn’t. The short version: bursts and matched widths win; conversions and contention cost you.

## 19.2	Functional Description

### 19.2.1	Peak Throughput

One master driving one slave — no contention, no conversion:

Peak Throughput by Data Width

| Data Width | Frequency | Peak Throughput |
| --- | --- | --- |
| 32-bit | 100 MHz | 400 MB/s |
| 64-bit | 100 MHz | 800 MB/s |
| 128-bit | 100 MHz | 1.6 GB/s |
| 256-bit | 100 MHz | 3.2 GB/s |
| 512-bit | 100 MHz | 6.4 GB/s |

### 19.2.2	Formula

Peak Throughput = DATA_WIDTH (bits) × Frequency (Hz) / 8 bytes/bit

### 19.2.3	Measured (2026-09-11)

bridge_2x2_rw (two 32-bit AXI4 masters, two 32-bit AXI4 slaves, no width or protocol conversion), 16-beat INCR bursts, every BFM channel back-to-back, each figure computed over its own window from the first to the last beat at the port named. test_bridge_2x2_rw_perf asserts a floor under every row, so a change that costs bandwidth fails there instead of ageing this table.

Measured throughput, direct 32-bit AXI4 paths

| Traffic | Where measured | Beats / cycle | Notes |
| --- | --- | --- | --- |
| One master streaming reads | R at the master port | 1.00 | 128 bursts, 2048 beats in 2048 cycles |
| One master streaming writes | W at the slave port | 0.89-0.90 | 0 cycles of WREADY low at the master port: the ~2-cycle gap per burst is the requester re-arming W between bursts, not the fabric |
| Two masters streaming writes to one slave | W at the shared slave port | 1.00 | the port stays full; the other master’s beats fill the gaps above |
| – per-master share of the contended window | W at each master port | 0.499 / 0.501 | 128 bursts each (0.475 / 0.525 at 8 bursts: round-robin, window-length effects only) |
| Two masters to two different slaves | W at both slave ports | 1.78-1.80 | two independent paths; neither slows the other |

So the formula above holds for this fabric: a saturated port moves one beat per cycle, two masters on one port split it evenly, and paths to different slaves do not share anything. The write-stream row is the requester’s figure, not the bridge’s – the contention row is the bridge’s.

The current performance tests measure throughput floors under uniform-style patterns; a full saturation study (accepted traffic vs. applied load, latency vs. offered load) is tracked as bridge vault TASK-014.

### 19.2.4	On Silicon: the Genesys 2 Fabric Benchmark (2026-10-16)

The fabric benchmark (projects/fpga-systems/Genesys2/fabric-benchmark, PRD §8) compares a generated bridge, bridge_fb_data, with the Xilinx AXI Crossbar and SmartConnect. All three use the same port set and the same harness on the same board, at 128 bits and 100 MHz, with only the fabric instance changed. Every figure is the mean of 5 runs. Peak is 1525.9 MiB/s, one direction.

The bridge against the vendor fabrics on the Genesys 2

| Measure | Bridge | AXI Crossbar | SmartConnect |
| --- | --- | --- | --- |
| One DMA, 8 ch x 4 x 1 MB, MiB/s | 1525.8 | 1525.8 | 1525.8 |
| One DMA, 1 ch x 4 x 4 KB, MiB/s | 1416.6 | 1420.7 | 1358.5 |
| Two writers into one slave, both full rate, writer B limited to 4 outstanding: B’s share | 50.0% | 50.0% | 20.0% |
| The same, writer B limited to 2 outstanding: B’s share | 40.0% | 50.0% | 11.1% |

In steady state the bridge costs no bandwidth, and its per-transfer overhead matches the Xilinx crossbar. Under contention it is fairer than SmartConnect. One case is not at parity with the crossbar: a writer limited to 2 outstanding transactions gets 40% of a contended slave rather than 50%. The cause measured so far is a longer write round trip. STREAM’s AW-to-B time is 256-511 cycles through the bridge against 64-127 through the crossbar at the same throughput, which bounds a 2-outstanding writer. It is not the slave tracker’s depth: capping it at 4 changed nothing. Where the extra time goes is open (GitHub #119).

The fabric is also larger and slower than the crossbar. Placed and routed alone, out of context on the Genesys 2’s part, the bridge is 10,450 LUTs and 7,479 flops with an estimated Fmax of ~179 MHz; the AXI Crossbar is 2,688 LUTs and 2,190 flops at ~256 MHz; SmartConnect is 8,333 LUTs, 3,959 LUTRAM and 11,616 flops at ~268 MHz. The bridge’s estimate was ~148 MHz before the slave adapters’ CAM insert and free were registered (MAS 4.1); its worst path now runs from a master’s R skid through the response routing into the read CAM. All three close at the board’s 100 MHz.

### 19.2.5	Factors Affecting Throughput

Factors Affecting Throughput

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Arbitration contention | Reduces per-master throughput | Increase slave ports |
| Width conversion | the narrow side’s full rate; +4 to +12 cycles of read latency (measured below) | Match widths where latency matters |
| Protocol conversion | 2+ cycles for APB | Use AXI4 for high-bandwidth |
| Response routing | Minimal (pipelined) | N/A |

### 19.2.6	Width-Converted Paths (measured)

test_bridge_4x4_rw_width_perf (GitHub #119 L4): one master to one slave of bridge_4x4_rw at a time, 16-beat bursts at the master’s natural size, 16 bursts per direction, every word checked. Rates are beats per aclk cycle at each port. Read latency is the idle first beat, AR handshake to the first R handshake at the master port.

Width-converted paths, bridge_4x4_rw (FULL level)

| Path | Master → slave | Master W | Slave W | Master R | Slave R | Ideal (master) | Read latency (cycles) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| matched | 64b → 64b | 0.895 | 0.895 | 1.000 | 1.000 | 1.000 | 5 |
| upsize 4:1 | 32b → 128b | 0.895 | 0.226 | 0.945 | 0.259 | 1.000 | 9 |
| upsize 4:1 | 64b → 256b | 0.895 | 0.226 | 0.945 | 0.259 | 1.000 | 9 |
| downsize 2:1 | 64b → 32b | 0.498 | 0.972 | 0.501 | 1.000 | 0.500 | 11 |
| downsize 8:1 | 256b → 32b | 0.128 | 0.993 | 0.125 | 1.000 | 0.125 | 17 |

Every path runs at its narrower side’s full rate: an upsize moves a master beat per cycle, a downsize keeps the narrow slave busy every cycle. The 0.895 write figure is the requester BFM’s own gap between bursts; the same figure appears on the matched path. Conversion costs read latency: +4 cycles upsizing, +6 at 2:1 and +12 at 8:1 downsizing. A downsize has to gather a whole wide beat before the first one goes back.

### 19.2.7	Latency Against Offered Load (measured)

test_bridge_2x2_rw_load (GitHub #119 L4): open loop, a master starts one 4-beat read to ddr every gap cycles regardless of completions. Latency is from the call to the last beat at the requester, so it includes queueing in the requester. Peak at the ddr port is 1 beat per cycle.

Accepted throughput and latency against offered load, bridge_2x2_rw (GATE level, 24 reads per master per point)

| Traffic | Gap | Offered (beats/cycle) | Accepted | Mean latency | p95 latency |
| --- | --- | --- | --- | --- | --- |
| cpu alone | 16 | 0.250 | 0.258 | 10.0 | 10 |
| cpu alone | 8 | 0.500 | 0.511 | 10.0 | 10 |
| cpu alone | 5 | 0.800 | 0.807 | 10.0 | 10 |
| cpu alone | 4 | 1.000 | 1.000 | 10.0 | 10 |
| cpu alone | 3 | 1.333 | 1.000 | 21.5 | 31 |
| cpu alone | 2 | 2.000 | 1.000 | 41.8 | 52 |
| cpu + dma | 16 | 0.500 | 0.511 | 12.0 | 14 |
| cpu + dma | 8 | 1.000 | 1.000 | 12.0 | 14 |
| cpu + dma | 6 | 1.333 | 1.000 | 35.0 | 56 |
| cpu + dma | 4 | 2.000 | 1.000 | 58.0 | 98 |
| cpu + dma | 2 | 4.000 | 1.000 | 108.4 | 174 |

Accepted equals offered up to the slave’s one beat per cycle and holds there past it. Latency is flat until saturation (10 cycles alone, 12 with two masters contending for the same slave), then grows with the queue. The fabric adds no queueing of its own below saturation. (Accepted reads a little above offered at light load because the window runs first beat to last.)

### 19.2.8	Bandwidth Characterization (measured)

test_bridge_perf_char drives each fixture with pipelined requesters (VALID held high across transactions, so a gap at a port belongs to the fabric), a fixed number of transactions in flight and rotating IDs, and checks every word. It measures inside each phase’s own window and holds every figure to a floor: one beat per cycle where the shape can saturate, otherwise Little’s law for the transactions in flight against the fixture’s measured round trip. It runs in the FUNC regression. A port on its own clock is measured in that clock’s cycles. Peak is one beat per cycle of the port’s clock, or of the slower clock across a crossing.

Bandwidth characterization, test_bridge_perf_char (FUNC level, 2026-10-18)

| Fixture | What it adds | Streams, 2+ beats | Shared slave, both masters active | Single-beat round trip (one in flight) |
| --- | --- | --- | --- | --- |
| bridge_2x2_rw | the reference | 1.000 | 0.997 writes, 1.000 reads | write 8 / read 7 cycles |
| bridge_2x2_rw_pipe | registered crossbar | 1.000 | 0.996 writes, 1.000 reads | write 10 / read 8 |
| bridge_4x4_rw | mixed widths, rate buffers | 1.000 | 0.995 writes, 1.000 reads | 12 on the upsized paths |
| bridge_2x2_axi5 | AXI5 masters, AXI5 and AXI4 slaves | 1.000 | 0.997 writes, 1.000 reads | write 7 / read 6 |
| bridge_2x2_rw_cdc, slave 7 ns | slave faster than the 10 ns fabric | 0.700 at the slave (the fabric’s rate), 1.000 at the masters | 0.696 writes, 0.700 reads | ~13 fabric cycles |
| bridge_2x2_rw_cdc, slave 13 ns | slave slower than the fabric | 1.000 at the slave, 0.770 at the masters | 0.993 writes, 0.999 reads | ~12 slave cycles |
| bridge_2x2_axi3_masters | AXI3 masters, 32b and 64b | 1.000 | 0.995 writes, 1.000 reads | write 7 / read 6 |
| bridge_mix_e | AXI3 and AXI4 masters, AXI3, AXI4 and APB slaves | 1.000; single-beat writes AXI3 to AXI3 0.890 at 8 in flight | 0.997 writes, 1.000 reads | write 8 / read 6 |

Single-beat traffic is bounded by the round trip at a few in flight: eight in flight carry 8 / (round trip) beats per cycle, and 16 in flight saturate every same-clock path. That is latency, not lost bandwidth. A CDC port pays two synchronizer crossings each way. Eight single-beat transactions in flight reach 0.58 of the port’s peak with the slave at 7 ns and 0.62 at 13 ns. Bursts of two beats or more saturate the slower side.

#### 19.2.8.1	Limits fixed on 2026-10-18

- W7, AXI3 master writes one at a time. axi3_to_axi4_wr held AW from acceptance to B. Writes ran at beats / (beats + ~8): 0.111 single-beat, 0.674 at 16 beats. It now passes every channel through, so writes overlap; W bursts must still follow AW order (AXI3 write-interleave depth 1), and a simulation-only check flags any that do not.

- W8, the AXI3 slave converter. axi4_to_axi3_rd/wr issued at most one AR or AW every other cycle (single-beat reads 0.50), took one transaction per ID (same-ID single-beat reads 0.334), lost a W cycle per AXI3 burst, and queued four granted requests behind the arbiter (round-robin share 0.26). Now:

  - one request per cycle;

  - an unchopped request passes straight to the pins, so nothing is queued behind the arbiter (round-robin share 0.498 / 0.500);

  - up to 32 in flight across IDs; a chopped (17+ beat) request waits for its ID to drain;

  - W and B pass through when nothing is queued ahead.

- W10, the AXI3 slave converter did not synthesize. It kept its queues in SystemVerilog queues ([$:N]), which Vivado 2025.1 rejects (“[Synth 8-27] dollar-sign operator not supported”). They are gaxi_fifo_sync instances now.

- W11, no register on the AXI3 slave’s R and B. The converter faced the crossbar directly, so <slave>_rvalid reached <slave>_rready through the crossbar’s R arbiter (-0.741 ns at 10 ns). An AXI3 slave adapter is now built like an AXI4 one: the timing wrapper, per-ID CAM tracking when several masters share it, and the per-master request slots. The converter is the last stage before the pins. Responses are routed by ID through the tracker, as for AXI4, instead of by FIFO position.

Built on the Artix-7 at 10 ns: bridge_2x2_axi3_masters meets timing (+0.060 ns, 6884 LUTs); bridge_2x2_axi3_slaves (10636 LUTs) and bridge_mix_e (17158 LUTs) miss by 1.006 and 2.686 ns. Their worst paths are inside bridge_cam’s response-free logic, the per-ID tracker every shared slave uses, AXI4 included, not in the AXI3 port.

#### 19.2.8.2	Open

- W9, round-robin share at a CDC slave. The address channels’ async FIFOs (8 deep) sit between the arbiter and the slave’s pins. Granted requests queue there. Against a slave that takes two at a time, a master with two in flight gets 0.18-0.20 against one with eight. Bandwidth is not affected; the slave stays busy. The depth cannot be traded away. A crossing needs about six entries in flight to carry one request per cycle, and those are granted requests. Measured at 13 ns:

CDC depth against round-robin share and bandwidth, bridge_2x2_rw_cdc

| CDC depth | Shallow master’s share | 4-beat reads | Single-beat reads |
| --- | --- | --- | --- |
| 8 | 0.20 | 1.000 | 0.671 |
| 4 | 0.26 | 0.650 | 0.642 |
| 2 | 0.34 | 0.334 | 0.334 |

- A fair share needs the arbitration in the slave’s clock domain: per-master crossings for AW and AR, and for W, since W follows the AW pick.

#### 19.2.8.3	Outstanding Depth (max_outstanding)

Each port’s max_outstanding (default 16) bounds the transactions it keeps in flight. A stream saturates its port while

max_outstanding * beats >= beats + slave_latency + round_trip

On bridge_2x2_rw (round trip 6-7 cycles), 8-beat bursts reach 1.000 with 4 in flight at 16 cycles of slave latency and with 16 at 64 cycles:

8-beat read rate against transactions in flight, bridge_2x2_rw (FUNC level)

| Slave latency (cycles) | 1 in flight | 2 | 4 | 8 | 16 | 32 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.577 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 16 | 0.271 | 0.544 | 1.000 | 1.000 | 1.000 | 1.000 |
| 64 | 0.105 | 0.212 | 0.431 | 0.846 | 1.000 | 1.000 |

The default of 16 covers 8-beat bursts up to about 110 cycles of slave latency. Where it binds is single-beat traffic behind a slow slave: 16 in flight carry 16 / (1 + latency + round trip), so 0.22 behind a 64-cycle slave. Raise max_outstanding (2..64) on those ports only. It sizes every tracker on the path, and the per-ID CAM is already the fabric’s critical path on the Artix-7.

### 19.2.9	Multi-Master Scaling

Arbitration is a fair round-robin (measured above: 0.499 / 0.501 at the shared port; the sum stays at one beat per cycle). The pick is made at the slave’s pins from per-master request slots, so the round-robin holds when the slave throttles too: against a slave taking two transactions at a time, a master with 2 in flight gets 0.50 of it against another master’s 8 (it got 0.34 while granted requests queued behind the arbiter). A CDC slave still queues granted requests in its crossing, so there the share follows outstanding counts (W9 above). A slave that accepts many at once keeps its own queue, and the share inside it follows each master’s outstanding count. Per master, then:

Per-Master Throughput = Peak Throughput / Active Masters (to same slave)

#### 19.2.9.1	Example: 4 Masters, 2 Slaves

All 4 masters accessing Slave 0:
  Per-master = Peak / 4

2 masters to Slave 0, 2 masters to Slave 1:
  Per-master = Peak / 2 (full parallelism)

### 19.2.10	Burst Efficiency

Burst Efficiency Comparison

| Transaction Type | Overhead | Efficiency |
| --- | --- | --- |
| Single beat | 1 cycle address + 1 cycle data | 50% |
| 4-beat burst | 1 cycle address + 4 cycle data | 80% |
| 16-beat burst | 1 cycle address + 16 cycle data | 94% |
| 256-beat burst | 1 cycle address + 256 cycle data | 99.6% |

### 19.2.11	Width Conversion Impact

#### 19.2.11.1	Upsize (Narrow to Wide)

64-bit to 512-bit (8:1 ratio):
  Input: 8 beats × 64 bits = 512 bits
  Output: 1 beat × 512 bits = 512 bits

Throughput: Preserved (same data, fewer beats)
Latency: +1 cycle (packing delay)

#### 19.2.11.2	Downsize (Wide to Narrow)

512-bit to 64-bit (8:1 ratio):
  Input: 1 beat × 512 bits = 512 bits
  Output: 8 beats × 64 bits = 512 bits

Throughput: Preserved (same data, more beats)
Latency: +7 cycles (sequential output)

### 19.2.12	Protocol Conversion Impact

AXI4 vs APB Protocol Impact

| Metric | AXI4 Direct | AXI4 to APB |
| --- | --- | --- |
| Min cycles/transfer | 1 | 2 |
| Burst support | Yes | No (split) |
| Pipeline depth | 1+ | 1 |
| Peak throughput | DATA_WIDTH/cycle | DATA_WIDTH/2 cycles |

## 19.3	Design Notes

Bursts buy you efficiency:

- Use bursts whenever possible

- AXI4 supports up to 256 beats per burst

- APB does not support bursts (split internally)

And protocol choice matters more than most people expect:

- Keep high-bandwidth traffic on AXI4 paths

- Use APB only for low-bandwidth peripherals

- Group APB slaves to avoid impacting AXI4 traffic

## 19.4	References

- Duato/Yalamanchili/Lionel, Interconnection Networks, Morgan Kaufmann 2003 — the standard network-evaluation methodology plots accepted traffic vs. applied load and latency vs. applied load, normalized to network capacity (ch9). (Reference archive)

- Dally & Towles, Principles and Practices of Interconnection Networks, Morgan Kaufmann 2004 — uniform-random traffic is benign; permutation, hot-spot, and adversarial patterns are the stresses that reveal allocation limits (ch3 §3.2-3.3). (Reference archive)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Latency Analysis

## 20.1	Overview

Where the cycles go on the way through the bridge: address path, data path, response path, then the end-to-end best cases and the conversion costs. The headline numbers for a direct AXI4 path are measured, not estimated — the note after Table 5.7 explains how, and why an earlier version of that table was wrong.

The current latency measurements are best-case and lightly-loaded figures; a full latency-vs-offered-load sweep to find the saturation knee is tracked as bridge vault TASK-014.

## 20.2	Timing

### 20.2.1	Address Path Latency

Address Path Latency

| Stage | Cycles | Notes |
| --- | --- | --- |
| Master Adapter (skid) | 1 | Pipeline registration |
| Address Decode | 0 | Combinational |
| Arbitration | 0-1 | 0 if no contention, 1 if arbitrate |
| Slave Router | 1 | Pipeline registration: the slave adapter’s skid, or on a slave several masters share, the master’s request slot ahead of the pick |
| Total Address | 2-3 | Best/worst case |

### 20.2.2	Data Path Latency

Data Path Latency

| Stage | Cycles | Notes |
| --- | --- | --- |
| W follows AW | 0 | Same grant |
| Width conversion | 0-1 | 0 if match, 1 if convert |
| Protocol conversion | 0-2+ | 0 for AXI4, 2+ for APB |
| Total Data | 0-3+ | Best/worst case |

### 20.2.3	Response Path Latency

Response Path Latency

| Stage | Cycles | Notes |
| --- | --- | --- |
| Slave response | Variable | Slave-dependent |
| ID extraction | 0 | Combinational – routing is by FIFO position, no lookup |
| Response routing + master delivery | 2 | Registered; NOT a direct connection |
| Total Response | 2 + slave | Plus slave latency |

Measured. bridge_2x2_rw, cpu master to ddr slave, idle bridge:

| Path | Cycles |
| --- | --- |
| AW accepted at the master -> AWVALID at the slave port | 2 |
| B accepted at the slave port -> BVALID at the master | 2 |

Two skid stages each way: the master adapter’s axi4_slave_wr AW skid, then the slave adapter’s axi4_master_wr AW skid. The response path has the same structure, which is why the two figures match.

The response row previously read “Master delivery 0 (Direct connection)”, total 1. It is registered, and the figure is 2.

What this measures, and what an earlier revision of this note got wrong. These are PROPAGATION times – how long a beat takes to appear on the far side. An earlier revision quoted 4 cycles for the request path, measured accept-to-accept. That metric also counts however long the far side held READY low, so it describes the attached slave’s timing as much as the bridge’s, and it was not even stable: successive runs gave 4/2 and then 2/6. Propagation is 2/2 on every run.

test_bridge_2x2_rw_latency asserts both values EXACTLY, so adding or removing a pipeline stage fails the test rather than silently ageing this table.

Other configurations differ – width converters and the APB/AXIL shims add stages – so this is the figure for a direct AXI4 path, not a universal constant.

With the registered crossbar ([bridge] xbar_pipeline = true, bridge TASK-006 (was BRIDGE-017)) both figures are 3: one skid stage each way inside the xbar. test_bridge_2x2_rw_pipe_latency asserts 3/3 on bridge_2x2_rw_pipe exactly as the baseline test asserts 2/2, and test_bridge_2x2_rw_perf runs its four phases on both fixtures with the same floors – the stage costs a cycle of latency and no bandwidth.

### 20.2.4	Loaded Write Latency (measured)

test_bridge_2x2_rw_perf streams 16-beat write bursts from one master and pairs the k-th AW accepted at the master port with the k-th BVALID there:

Write latency under load, bridge_2x2_rw, 32-bit direct path

| Queue | AW accepted -> BVALID (cycles) |
| --- | --- |
| First burst, fabric empty | 23 = 16 W beats + the 2 + 2 skid stages + the slave’s turnaround |
| k bursts queued ahead | about 16 k + 7; maximum 323 at a 20-deep queue |

The queue exists because the bridge accepts AWs ahead of their W data – the master adapter’s AW skid, the slave adapter’s max_outstanding-entry (default 16) response-tracking FIFO and its AW skid together hold about 20 bursts – so a requester that issues many AWs before its data sees its B responses spaced by the data time, not by the fabric. Read latency does not queue the same way: the R stream is one beat per cycle end to end (Table 5.1a).

### 20.2.5	End-to-End Latency

#### 20.2.5.1	Write Transaction (Best Case)

Cycle 0: AW arrives at Bridge
Cycle 1: AW exits to Slave (skid buffer)
Cycle 2: Slave receives AW
Cycle 1: W arrives (same cycle as AW skid)
Cycle 2: W exits to Slave
Cycle 3: Slave receives W, generates B
Cycle 4: B routed back to Master

Total: 4 cycles (minimum)

#### 20.2.5.2	Read Transaction (Best Case)

Cycle 0: AR arrives at Bridge
Cycle 1: AR exits to Slave (skid buffer)
Cycle 2: Slave receives AR, generates R
Cycle 3: R routed back to Master

Total: 3 cycles (minimum)

#### 20.2.5.3	Contention Latency

When multiple masters contend for same slave:

Arbitration adds 1 cycle per contending master

Example: 4 masters requesting same slave
  Worst case: Wait for 3 other masters = 3 extra cycles
  Average case: Wait for 1.5 masters = 1-2 extra cycles

### 20.2.6	Width Conversion Latency

#### 20.2.6.1	Upsize Latency

Upsize Latency by Ratio

| Ratio | Extra Cycles | Notes |
| --- | --- | --- |
| 1:1 | 0 | No conversion |
| 2:1 | 0 | Pack on fly |
| 4:1 | 0 | Pack on fly |
| 8:1 | 0-1 | May need accumulation |

#### 20.2.6.2	Downsize Latency

Downsize Latency by Ratio

| Ratio | Extra Cycles | Notes |
| --- | --- | --- |
| 1:1 | 0 | No conversion |
| 1:2 | +1 | 2 output beats |
| 1:4 | +3 | 4 output beats |
| 1:8 | +7 | 8 output beats |

### 20.2.7	Protocol Conversion Latency

AXI4 to APB Conversion Latency

| Phase | Cycles |
| --- | --- |
| AXI4 AW decode | 1 |
| APB setup | 1 |
| APB access | 1+ (PREADY) |
| B generation | 1 |
| Total | 4+ cycles |

#### 20.2.7.1	APB Wait States

APB slaves may insert wait states:

PREADY deasserted: +1 cycle per wait
Typical UART/GPIO: 0-2 wait states
Slow peripherals: May have many wait states

## 20.3	Design Notes

### 20.3.1	Design Recommendations

- Match data widths - Avoid conversion latency

- Use AXI4 for fast paths - Avoid APB latency

- Minimize contention - Spread traffic across slaves

- Use bursts - Amortize address latency

- Pipeline responses - Don’t block on slow slaves

## 20.4	References

- Duato/Yalamanchili/Lionel, Interconnection Networks, Morgan Kaufmann 2003 — accepted traffic vs. applied load and latency vs. applied load are the standard evaluation plots for finding the saturation point (ch9). (Reference archive)

- Dally & Towles, Principles and Practices of Interconnection Networks, Morgan Kaufmann 2004 — uniform-random traffic is benign; permutation and hot-spot patterns expose the allocation limits that matter for worst-case latency (ch3 §3.2-3.3). (Reference archive)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Resources

## 21.1	Overview

What a generated bridge costs on an FPGA, measured. The numbers below come from the out-of-context synthesis and implementation flow in projects/components/fabric-gen-ip/bridge/fpga/ (HAS 6.4: how it constrains the bridge, how to repeat a run, how to read a line of reports/summary.csv), on the two parts this repository’s boards carry. Earlier revisions of this page carried hand estimates per component and per configuration; they did not reconcile with each other and had never been checked against a synthesis report, so they are gone. Anything not in the tables below is a configuration nobody has synthesized yet, and the flow takes minutes to answer for it.

## 21.2	Measured Utilization and Timing

Measured utilization and timing, Artix-7 100T -1 at 10 ns (HAS 6.4 flow)

| Bridge | LUTs | FFs | BRAM | WNS reg-to-reg (ns) | Fmax est. (MHz) | Worst logic levels |
| --- | --- | --- | --- | --- | --- | --- |
| bridge_2x2_axi5 | 5,120 | 4,013 | 0 | -0.11 | 98.9 | 10 |
| bridge_2x2_axi5_native | 8,252 | 6,773 | 0 | -1.20 | 89.3 | 11 |
| bridge_2x2_rw | 5,832 | 4,605 | 0 | -0.04 | 99.6 | 11 |
| bridge_2x2_rw_cdc | 5,184 | 4,021 | 0 | +0.07 | 100.7 | 11 |
| bridge_2x2_rw_pipe | 5,569 | 4,869 | 0 | +0.57 | 106.1 | 8 |
| bridge_2x2_rw_qos | 5,344 | 3,925 | 0 | -0.28 | 97.3 | 11 |
| bridge_2x2_rw_qos_pipe | 5,932 | 4,933 | 0 | -0.02 | 99.8 | 12 |
| bridge_4x4_rw | 40,239 | 38,241 | 0 | -3.01 | 76.9 | 15 |
| bridge_5x3_channels | 30,835 | 28,051 | 0 | -3.21 | 75.7 | 15 |
| bridge_mix_a | 7,994 | 6,623 | 0 | -2.10 | 82.6 | 13 |

Measured utilization and timing, Kintex-7 325T -2 at 6.667 ns

| Bridge | LUTs | FFs | BRAM | WNS reg-to-reg (ns) | Fmax est. (MHz) | Worst logic levels |
| --- | --- | --- | --- | --- | --- | --- |
| bridge_2x2_axi5 | 4,949 | 4,013 | 0 | +0.59 | 164.6 | 11 |
| bridge_2x2_axi5_native | 8,119 | 6,773 | 0 | +0.47 | 161.3 | 12 |
| bridge_2x2_rw | 5,682 | 4,605 | 0 | +0.56 | 163.8 | 10 |
| bridge_2x2_rw_cdc | 5,030 | 4,021 | 0 | +0.26 | 156.0 | 10 |
| bridge_2x2_rw_pipe | 5,564 | 4,869 | 0 | +1.14 | 180.8 | 9 |
| bridge_2x2_rw_qos | 5,176 | 3,925 | 0 | +0.48 | 161.6 | 11 |
| bridge_2x2_rw_qos_pipe | 5,886 | 4,933 | 0 | +0.53 | 163.0 | 11 |
| bridge_4x4_rw | 40,042 | 38,241 | 0 | -0.14 | 146.8 | 13 |
| bridge_5x3_channels | 30,712 | 28,051 | 0 | -0.68 | 136.1 | 15 |
| bridge_mix_a | 7,844 | 6,623 | 0 | +0.30 | 157.1 | 13 |

Fmax is an estimate from the register-to-register slack at the constrained period; the I/O budget and the reading of each row are in HAS 6.4. Re-measured 2026-10-16 (rev 1.3). The 2x2s, QoS included, are at 100 MHz on the Artix-7 and meet 150 MHz on the Kintex-7. The fixtures with width converters, unmeasurable until the read converters lost their reassembly pool, now miss by 2-3 ns on the Artix-7 and up to 0.7 ns on the Kintex-7, on wide response paths that xbar_pipeline registers.

### 21.2.1	Scaling

Resource Scaling

| Component | Scales with |
| --- | --- |
| Crossbar core | O(M x N) muxing, plus one arbiter per slave channel |
| Master adapters | O(M), each holding a full beat per channel in its skid buffers |
| Slave adapters | O(N), plus a converter and its monitor sandwich for every non-AXI4 or width-stepped port |
| bridge_id FIFOs | O(N x depth), small LUT arrays, no BRAM |
| Registered crossbar (xbar_pipeline) | one 2-deep skid per slave-side channel |
| CDC slave port (cdc) | one async FIFO per channel of that port |

Block RAM and DSP usage are zero in every configuration measured: the per-slave bridge_id FIFOs and the CAM are LUT arrays, and there is no arithmetic.

The current resource figures come from a small set of configurations driven with uniform-style traffic; a systematic resource-vs-load characterization across adversarial patterns is tracked as bridge vault TASK-014.

## 21.3	Design Notes

### 21.3.1	Resource Reduction

- Use channel-specific masters - a read-only or write-only master builds half the adapter and half the crossbar columns

- Match data widths - a width step costs a converter each way

- Use APB and AXI-Lite slaves sparingly - each carries a shim and a second monitor

- Limit outstanding transactions - shallower per-slave bridge_id FIFOs

### 21.3.2	Performance/Resource Trade-off

Performance/Resource Trade-offs

| Feature | Resource Impact | Performance Impact |
| --- | --- | --- |
| Deeper skid buffers | +registers | +timing margin |
| Deeper bridge_id FIFOs | +LUTs | +outstanding depth (no OOO support) |
| Pipeline stages | +registers | +frequency |
| Wider data paths | +routing | +throughput |

## 21.4	References

- Duato/Yalamanchili/Lionel, Interconnection Networks, Morgan Kaufmann 2003 — accepted traffic vs. applied load and latency vs. applied load are the standard network-evaluation plots; resource counts must be paired with those load sweeps to be meaningful (ch9). (Reference archive)

- Dally & Towles, Principles and Practices of Interconnection Networks, Morgan Kaufmann 2004 — uniform-random traffic is benign; permutation and hot-spot patterns are the standard stresses that reveal allocation and buffer-sizing limits which in turn drive resource growth (ch3 §3.2-3.3). (Reference archive)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	System Requirements

## 22.1	Overview

Everything your system owes the bridge before it will do its job: one clean clock, an asynchronous active-low reset, a single power domain, interfaces that follow the AXI4 rules, and an address map with no overlaps. Gaps in the map are allowed — the bridge answers those addresses itself — and the checklist at the end tells you how to prove all of it.

### 22.1.1	Clock Infrastructure

Clock Requirements

| Requirement | Specification |
| --- | --- |
| Clock source | One fabric clock (aclk) for the fabric; a cdc = true AXI4 master or slave port brings its own <port>_aclk (HAS 4.5a) |
| Clock quality | Low jitter, stable frequency |
| Clock distribution | Must reach all Bridge ports |

### 22.1.2	Reset Infrastructure

Reset Requirements

| Requirement | Specification |
| --- | --- |
| Reset type | Asynchronous, active-low (aresetn) |
| Reset synchronization | External synchronizer recommended |
| Reset distribution | Must reach all Bridge ports |

### 22.1.3	Power

Power Requirements

| Requirement | Specification |
| --- | --- |
| Power domains | Single domain (no power gating) |
| Voltage | FPGA/ASIC process-dependent |

## 22.2	Parameters

### 22.2.1	Address Map

Address Map Requirements

| Requirement | Specification |
| --- | --- |
| Slave ranges | Must not overlap |
| Range alignment | Power-of-2 recommended |
| Coverage | Gaps are permitted: an address in no slave range is claimed by the internal subtractive slave and answered with DECERR (see 4.5). It is still a fault – it is reported, not tolerated. |

### 22.2.2	Connectivity

Connectivity Requirements

| Requirement | Specification |
| --- | --- |
| Matrix completeness | All masters must access at least one slave |
| Reachability | Consider traffic patterns for performance |

## 22.3	Ports

### 22.3.1	Master Requirements

Masters connecting to Bridge must:

- Comply with AXI4 protocol - Valid/ready handshake

- Use correct ID width - Match configured ID_WIDTH

- Use correct data width - Match configured DATA_WIDTH

- Use correct address width - Match configured ADDR_WIDTH

- Stay within address range - Target valid slave addresses

### 22.3.2	Slave Requirements

Slaves connecting to Bridge must:

- Comply with AXI4/APB protocol - Based on configuration

- Accept the widened ID - a slave sees {master index, master id}: the widest master’s id_width plus $clog2(NUM_MASTERS) bits (none for a single master). Declare the slave’s id_width at least that wide; the validator refuses less.

- Match configured data width - Or use width conversion

- Respond to all transactions - No hanging

## 22.4	Timing

### 22.4.1	Setup/Hold

Timing Requirements

| Constraint | Requirement |
| --- | --- |
| Input setup | Meet FPGA/ASIC requirements |
| Input hold | Meet FPGA/ASIC requirements |
| Clock-to-output | Per process characterization |

### 22.4.2	Recommended Margins

Recommended Timing Margins

| Margin | Value | Purpose |
| --- | --- | --- |
| Setup margin | 10-20% | Variation tolerance |
| Hold margin | Positive | No hold violations |
| Clock uncertainty | Include in constraints | PLL/MMCM jitter |

## 22.5	Testing

### 22.5.1	Pre-Integration Checks

- RTL lint clean - No Verilator warnings

- Parameter validation - All parameters in valid range

- Address map review - no overlaps. Complete coverage is NOT required: a gap is claimed by the subtractive slave and answered with DECERR (4.5). Decide deliberately which you want – a gap that is meant to be a gap is fine, a gap you did not know about is a bug the fabric will now report.

- ID width calculation - Sufficient for NUM_MASTERS

### 22.5.2	Integration Checks

- Signal connectivity - All ports connected

- Width matching - DATA_WIDTH, ADDR_WIDTH, ID_WIDTH

- Protocol matching - AXI4 vs APB correctly configured

- Clock/reset connectivity - All ports share same domain

### 22.5.3	Post-Integration Checks

- Basic transaction test - Read/write to each slave

- Arbitration test - Multi-master contention

- Error handling test - unmapped-address response: DECERR + 0xDEADBEEF, unmapped_irq sticky, SUBTRACTIVE_STATUS.CLEAR clears it (see 4.5)

- Performance validation - Meet throughput targets

## 22.6	Assumptions and Known Limitations

### 22.6.1	Response-Ordering Assumption

Bridge routes B and R by a sideband master index stored per slave. For an AXI4/AXI5 slave of a multi-master bridge (or any slave with enable_ooo) the store is bridge_cam, looked up by the returned BID/RID, so the slave may complete different IDs in any order (ARM IHI 0022E §A4). Every other slave – single-master bridges, AXI4-Lite/APB/Wishbone shim slaves and the internal subtractive slave – uses an in-order FIFO and must return responses in request order, which those slaves do by construction.

### 22.6.2	Buffer and CAM Depths

All adapter skid depths, response-tracking FIFO depths, and CAM dimensions are generation-time constants. There is no runtime sizing API. A systematic sizing rule that relates buffer depth to credit-loop latency, outstanding-transaction count, and width-converter buffering is tracked as bridge vault TASK-016.

## 22.7	References

- ARM, AMBA AXI and ACE Protocol Specification, ARM IHI 0022E — AXI4 ordering rules allow out-of-order completion across IDs (§A4). (Reference archive)

- Maaref, Architecting and Building High-Speed SoCs, Packt 2022 — AMBA-suite context and the AXI ID ordering rules that Bridge’s FIFO mode simplifies (ch3 §4.2). (Reference archive)

- Eschemann, The FPGA Programming Handbook, Packt 2023 — educational secondary source describing out-of-order reads as a normal AXI4 feature (ch8 §8.3). (Reference archive)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Parameter Configuration

## 23.1	Overview

Every bridge the generator emits starts here: port counts, bus widths, per-port configuration, and the address map. This chapter collects the full parameter set — the core knobs, the widths the generator derives from them, the per-port fields, the monitor options, and the checks the generator runs before it accepts your configuration.

## 23.2	Parameters

### 23.2.1	Core Parameters

Bridge Core Parameters

| Parameter | Type | Range | Default | Description |
| --- | --- | --- | --- | --- |
| NUM_MASTERS | int | 1-32 | 2 | Number of master ports |
| NUM_SLAVES | int | 1-256 | 2 | Number of slave ports |
| ADDR_WIDTH | int | 12-64 | 32 | Address bus width |
| DATA_WIDTH | int | 32-512 | 64 | Data bus width |
| ID_WIDTH | int | 1-16 | 4 | Master ID width |
| USER_WIDTH | int | 1-16 | 1 | User signal width |

### 23.2.2	Registered Crossbar (xbar_pipeline)

Crossbar options (bridge TASK-006, was BRIDGE-017)

| TOML key ([bridge]) | Type | Default | Effect |
| --- | --- | --- | --- |
| xbar_pipeline | bool | false | true puts a 2-deep skid stage on every slave-side channel (AW, W, AR, B, R) inside the crossbar. Propagation 3/3 cycles instead of 2/2 (Table 5.7), throughput unchanged. For high-fanout or wide fabrics that miss timing on the arbiter-plus-mux or the response OR-merge cone. Measured (HAS 6.4): +0.57 ns against about 0 on the plain 2x2 at 100 MHz on the Artix-7. It registers the slave-side response paths, which is where the larger width-converting fixtures still miss. |
| arbitration | "rr" / "qos" | "rr" | Per-slave arbitration policy. "qos" grants the highest AxQOS plus aging (MAS 2.3, Per-Slave Request Arbitration); it decides among requests pending at the arbiter, so it shows at a slave that backpressures AW, not at one that accepts every AW on arrival; equals share round-robin. |
| qos_aging_shift | int 0..7 | 4 | With "qos": a waiting request gains one priority level every 2**shift cycles, so the worst wait for a QoS-0 request is 15 * 2**shift cycles. |
| id_track_bits | int 0..8 | 4 | Per-ID target tracking in every master adapter: 2**id_track_bits entries indexed by the ID’s low bits (never more bits than the port’s ID). A request waits only when its ID is in flight to a different slave; IDs sharing the index bits are treated as one ID, which is safe and at worst stalls needlessly. 0 orders every ID as one. Every port, read-return-atomic and trace-echo ports included (GitHub #119 L1b). |
| max_outstanding | int, power of two 2..64 | 16 | Outstanding limit per direction, the default for every port; a port overrides it with its own max_outstanding. A master holds at most this many AWs and ARs in flight; a slave tracks at most this many (its bridge_cam or in-order FIFO), and its AWREADY/ARREADY drop when full. Fixes the fixed 16 (GitHub #119 L5); measured exactly by test_bridge_2x2_rw_depth_limits. |

Per-port options: CDC (bridge TASK-006, GitHub #119 L2), latency, outstanding limit

| TOML key ([[bridge.slaves]]) | Type | Default | Effect |
| --- | --- | --- | --- |
| cdc | bool | false | true gives this AXI4 slave port its own clock: <name>_aclk / <name>_aresetn on the top, axi4_cdc_{wr,rd} between the adapter’s timing wrapper and the port (HAS 4.5a). The same key on a [[bridge.masters]] entry does the same for a master port, with the crossing ahead of its wrapper (GitHub #119 L2). protocol = "axi4" only. |
| latency | "cut_all" / "cut_ax" / "cut_none" | "cut_all" | Which channels of this port’s timing wrapper are registered (masters take the same key). cut_ax keeps AW/AR, cut_none makes the port boundary wires: 0 cycles. Same-clock axi4/axi5 ports only. GitHub #119 L7; test_bridge_2x2_rw_lat_modes measures every pair. |
| timeout_cycles | int, 0 or >= 2 | 0 | > 0 puts axi4_slave_watchdog in front of this AXI4 slave: after that many cycles of silence with work outstanding it answers everything with DECERR, and the top gains a sticky <name>_timeout (HAS “Slave Watchdog”, GitHub #119 L3). 0 = no watchdog. |
| max_outstanding | int, power of two 2..64 | [bridge].max_outstanding | This slave’s response-tracker depth (bridge_cam or in-order FIFO), and so its outstanding limit. The same key on a [[bridge.masters]] entry sets that master adapter’s limit. |

Monitor preset

| TOML key ([bridge]) | Type | Default | Effect |
| --- | --- | --- | --- |
| mon_preset | "lite" (also accepted: "error_only" / "functional" / "all" / "none") | "lite" | Accepted for compatibility; since 2026-09-26 every monitored port builds axi_monitor_lite through the *_monlite wrapper (error, timeout, completion and active-count threshold packets, same monbus, about a fifth of the full monitor’s LUTs), so the cone presets no longer select hardware. Which packets a port emits is the runtime cfg_*_enable pins. To remove a monitor use use_monitor = false on the port or use_no_monitors = true on the bridge. mon_add / mon_remove are validated and ignored. |

### 23.2.3	Derived Parameters

The generator works these out from the core set — you never write them yourself:

Derived Parameters

| Parameter | Formula | Example |
| --- | --- | --- |
| BRIDGE_ID_WIDTH | clog2(NUM_MASTERS), minimum 1 | 4 masters = 2 bits |
| MASTER_ID_WIDTH | widest master id_width | 4 |
| ID_PREFIX_WIDTH | clog2(NUM_MASTERS), 0 for one master | 4 masters = 2 bits |
| XBAR_ID_WIDTH | MASTER_ID_WIDTH + ID_PREFIX_WIDTH | 4 + 2 = 6 bits |
| STRB_WIDTH | DATA_WIDTH / 8 | 64b = 8 strobes |

BRIDGE_ID_WIDTH sizes the sideband master index stored in each slave adapter’s response tracker. XBAR_ID_WIDTH is the AXI ID width inside the fabric and on every AXI slave port, because each master adapter widens its IDs to {BRIDGE_ID, id}; a slave’s id_width must be at least this. All four are emitted into <bridge>_pkg.sv. There is no TOTAL_ID_WIDTH.

### 23.2.4	Per-Port Configuration

Each master and slave port carries its own field set in the configuration file.

#### 23.2.4.1	Master Port Configuration

Master Port Configuration

| Field | Type | Description |
| --- | --- | --- |
| name | string | Unique identifier |
| prefix | string | Signal prefix |
| channels | enum | “rw”, “wr”, or “rd” |
| data_width | int | Port data width |
| id_width | int | Port ID width |
| use_monitor | bool | Optional, defaults to true. Per-port monitor wrappers. Only meaningful on a mon variant; on a no variant there is nothing to disable. |

#### 23.2.4.2	Slave Port Configuration

Slave Port Configuration

| Field | Type | Description |
| --- | --- | --- |
| name | string | Unique identifier |
| prefix | string | Signal prefix |
| protocol | enum | axi4, axi5, axil, axil5, apb, apb5, wb4 – the exact values config_validator.valid_protocols accepts. Note axil, not axi4lite. |
| axi5_features | list | On an axi5 port: any of nsaid, trace, mpam, mecid, unique (droppable), chunking (droppable, 128-bit ports), wakeup (AWAKEUP, not path sideband), poison, atomic, mte (connectivity-gated; mte 128-bit ports). On an axil5 port: user only. Classes and rules in HAS 4.4. |
| data_width | int | Port data width |
| base_addr | hex | Base address (must be 4K-aligned) |
| addr_range | hex | Address range size (must be multiple of 4K) |
| use_monitor | bool | Optional, defaults to true. Per-port monitor wrappers. Only meaningful on a mon variant; on a no variant there is nothing to disable. |

#### 23.2.4.3	Slave Address Window Rules (MANDATORY)

Every slave must occupy a 4K-aligned address window that is a multiple of 4 KB:

Rule 1: Base Address Alignment
  base_addr & 0xFFF == 0x000
  
  Valid:   0x00000000, 0x00001000, 0x80000000, 0xFFFF0000
  Invalid: 0x00000001, 0x80000800, 0xC0000100

Rule 2: Range Alignment
  addr_range % 0x1000 == 0
  
  Valid:   0x1000 (4 KB), 0x2000 (8 KB), 0x10000 (64 KB)
  Invalid: 0x800 (2 KB), 0x1001, 0x7FFF

Rule 3: Non-Overlapping Windows
  All slaves must have non-overlapping address ranges
  
  Valid:   Slave0: 0x00000000-0x0FFFFFFF, Slave1: 0x10000000-0x1FFFFFFF
  Invalid: Slave0: 0x00000000-0x10000000, Slave1: 0x0FFFFFFF-0x1FFFFFFF

Why This Rule Exists:

- Real memory systems use 4K page boundaries (virtual memory, MMU)

- Linker scripts and memory maps assume 4K granularity

- Address decoders simplify to bit masks with 4K alignment

- Prevents ambiguity in address decode logic

There’s nothing arbitrary about the 4K requirement — it’s the granularity the rest of the system already assumes.

### 23.2.5	Top-Level Monitor Configuration

When monitor collection is desired, the bridge TOML configuration includes:

Top-Level Monitor Configuration

| Field | Type | Description |
| --- | --- | --- |
| variants | list | List of bridge variants to generate. Supported: ["no"] (no monitor), ["mon"] (monitor only), ["no", "mon"] (both). Default: ["no"] |

#### 23.2.5.1	Monitor Identifiers

Each monitored port gets a unique (UNIT_ID, AGENT_ID) pair for identification in monitor packets:

UNIT_ID Assignment:
  - UNIT_ID = 1: Master-side monitor wrappers (axi4_master_{rd,wr}_monlite)
  - UNIT_ID = 2: Slave-side monitor wrappers (axi4_slave_{rd,wr}_monlite)

AGENT_ID Assignment (per port):
  - AGENT_ID = (port_index << 4) | channel_bit
  - channel_bit: 0 = read channel (AR/R), 1 = write channel (AW/W/B)
  - Example: Master port 2, write direction → AGENT_ID = (2 << 4) | 1 = 0x21

#### 23.2.5.2	AXIL→Wider-Slave Master-Side Alignment

When an AXI4-Lite master connects to a wider AXI4 slave through the bridge (e.g., 32-bit AXIL master to 64-bit AXI4 slave), the generator emits a master-side alignment converter (the axil_to_axi4_wide_align_{rd,wr} modules) between the master adapter and the crossbar core. The converter handles partial-word alignment on the narrow side while preserving AXIL’s single-beat semantics; the protocol stays AXIL — protocol shims are a slave-boundary concern, applied separately.

### 23.2.6	Parameter Validation

#### 23.2.6.1	Generator Checks

The generator checks every configuration against these rules:

Generator Validation Checks

| Check | Error Condition |
| --- | --- |
| NUM_MASTERS | < 1 or > 32 |
| NUM_SLAVES | < 1 or > 256 |
| DATA_WIDTH | Not power of 2 |
| Address overlap | Slave ranges intersect |
| Connectivity | Master with no slaves |
| ID width | Insufficient for masters |

#### 23.2.6.2	Runtime Validation

Bridge includes optional assertions for:

- Address alignment

- Burst boundary crossing

- Protocol violations

- ID mismatch

## 23.3	Usage Example

A complete configuration is two parts: the TOML file that describes the bridge, and the CSV that describes the connectivity.

### 23.3.1	TOML Configuration

[bridge]
  name = "my_bridge"
  description = "Example bridge configuration"

  defaults.skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

  masters = [
    {name = "cpu", prefix = "cpu_m_axi", channels = "rw",
     id_width = 4, addr_width = 32, data_width = 64, user_width = 1},
    {name = "dma", prefix = "dma_m_axi", channels = "rw",
     id_width = 4, addr_width = 32, data_width = 256, user_width = 1}
  ]

  slaves = [
    {name = "ddr", prefix = "ddr_s_axi", protocol = "axi4",
     id_width = 6, addr_width = 32, data_width = 512, user_width = 1,
     base_addr = 0x80000000, addr_range = 0x40000000},
    {name = "uart", prefix = "uart_apb", protocol = "apb",
     addr_width = 12, data_width = 32,
     base_addr = 0x00000000, addr_range = 0x00001000}
  ]

### 23.3.2	CSV Connectivity

master\slave,ddr,uart
cpu,1,1
dma,1,0

Two configurations to use as starting points:

### 23.3.3	Simple 2x2

[bridge]
  name = "bridge_simple"
  masters = [
    {name = "m0", prefix = "m0_axi", data_width = 64},
    {name = "m1", prefix = "m1_axi", data_width = 64}
  ]
  slaves = [
    {name = "s0", prefix = "s0_axi", base_addr = 0x0, addr_range = 0x10000000},
    {name = "s1", prefix = "s1_axi", base_addr = 0x10000000, addr_range = 0x10000000}
  ]

### 23.3.4	Mixed Protocol

[bridge]
  name = "bridge_mixed"
  masters = [
    {name = "cpu", prefix = "cpu_axi", data_width = 64, channels = "rw"}
  ]
  slaves = [
    {name = "mem",  prefix = "mem_axi",  protocol = "axi4", data_width = 512,
     base_addr = 0x00000000, addr_range = 0x80000000},
    {name = "gpio", prefix = "gpio_apb", protocol = "apb",  data_width = 32,
     base_addr = 0x80000000, addr_range = 0x00010000},
    {name = "uart", prefix = "uart_apb", protocol = "apb",  data_width = 32,
     base_addr = 0x80010000, addr_range = 0x00010000}
  ]

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Verification Strategy

## 24.1	Overview

Bridge gets verified at three levels — unit, integration, and system — and all of it runs on CocoTB with protocol BFMs doing the driving. This chapter lays out the test infrastructure, the execution model, the coverage targets, the debug hooks, and the regression schedule, then closes with the gaps we know about.

## 24.2	Testing

### 24.2.1	Verification Levels

#### 24.2.1.1	Unit Level

Test individual Bridge components:

Unit Level Test Focus

| Component | Test Focus |
| --- | --- |
| Master Adapter | Skid buffer, sideband bridge_id (IDs pass through) |
| Slave Router | Address decode, arbitration |
| Width Converter | Upsize/downsize correctness |
| Protocol Converter | AXI4 to APB timing |
| Response Router | ID extraction, routing |

#### 24.2.1.2	Integration Level

Test complete Bridge functionality:

Integration Test Coverage

| Test Category | Coverage |
| --- | --- |
| Address routing | All master-slave paths |
| Arbitration | Contention, fairness |
| ID tracking | Response routing |
| Error handling | OOR, timeouts |
| Performance | Throughput, latency |

#### 24.2.1.3	System Level

Test Bridge in target system:

System Level Tests

| Test Category | Focus |
| --- | --- |
| End-to-end | CPU to memory |
| Multi-master | DMA + CPU |
| Mixed protocol | AXI4 + APB |
| Stress | Maximum traffic |

### 24.2.2	Test Infrastructure

#### 24.2.2.1	CocoTB Framework with Protocol-BFM-Only Driving

Bridge verification runs on CocoTB with protocol BFMs only: all test stimulus is driven through protocol BFMs (AXI4Master, AXI4Slave, etc.) with no direct DUT signal manipulation. Each slave port is backed by an in-memory model (MemoryModel), and checking is assertion-based (readback matches write) rather than routing-based.

from CocoTBFramework.components.axi4 import AXI4Master, AXI4Slave
from TBClasses.shared.tbbase import TBBase

class BridgeTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)
        self.masters = [AXI4Master(dut, dut.aclk, f"m{i}_axi") for i in range(NUM_MASTERS)]
        self.slaves = [MemoryModel(dut, dut.aclk, f"s{i}_axi") for i in range(NUM_SLAVES)]

@cocotb.test()
async def cocotb_test_basic_write(dut):
    tb = BridgeTB(dut)
    await tb.setup_clocks_and_reset()

    # Write transaction via BFM only
    await tb.masters[0].write(addr=0x1000, data=0xDEADBEEF)
    
    # Verify: readback matches write (memory-model assertion)
    readback = await tb.masters[0].read(addr=0x1000)
    assert readback == 0xDEADBEEF

#### 24.2.2.2	Test Categories

Test Categories

| Category | Description | Example |
| --- | --- | --- |
| Basic | Single transactions | Read/write |
| Burst | Multi-beat transactions | 16-beat burst |
| Concurrent | Multiple masters active | M0 + M1 |
| Boundary Probe | Address window edges | Top/mid/bottom of each slave page |
| Stress | Maximum utilization | Back-to-back |
| Error | Error conditions | SLVERR injection, timeout |
| Monitor | Monitor packet collection | Packet capture, IRQ assertion |

#### 24.2.2.3	Protocol Checkers on Every Port

Every generated bridge also gets <bridge>_checkers.sv: one SystemVerilog bind per external AXI4/AXI5 port, attaching formal/bridge/axi4_protocol_checker to that port on its own clock (GitHub #119 L3). It checks VALID held until READY, payload stable while VALID waits, VALID low after reset, and legal burst type and size. On a master port it checks the requester; on a slave port it checks the bridge, which no other checker in the suite does. <bridge>_checkers.f is the bridge filelist plus the checker and the binds. It is for simulation and formal only: nothing is added to the RTL or the synthesis filelist, which keeps assertions out of the RTL.

Generated tests build with it by default; BRIDGE_CHECKERS=0 builds the bare bridge. A failed check stops the simulation and fails the test. A mutation showed the checkers watch: wiring cpu’s AWREADY to 0 at its checker fails basic_connectivity on u_chk_cpu.aw_valid_stable. APB, AXI4-Lite, AXI3 and WB4 ports have no checker yet.

### 24.2.3	Test Execution Model

#### 24.2.3.1	Serial Execution

Bridge tests run serially only (no parallel execution via pytest-xdist). Each test function is named per-module to avoid cross-test cocotb function name pollution:

# Naming convention: test_bridge_{N}x{M}_{variant}_{scenario}
def test_bridge_1x2_rd_basic(request):        # Test name indicates 1 master, 2 slaves, read-only
def test_bridge_1x2_rd_monitor_smoke(request): # Monitor test on 1x2
def test_bridge_4x4_rw_concurrent(request):    # Concurrent test on 4x4

Cocotb test functions inside each file follow the same naming pattern but with cocotb_test_* prefix to avoid pytest collection conflicts.

#### 24.2.3.2	Boundary Probe Testing

Address-window boundary testing probes the top, middle, and bottom of each slave’s address window to catch address-decode logic errors at window edges:

for slave_idx, (base, window_size) in enumerate(slave_windows):
    # Probe bottom (base address)
    await master.write(base + 0x000, data)
    
    # Probe middle (arbitrary address within window)
    await master.write(base + window_size // 2, data)
    
    # Probe top (base + window_size - 1)
    await master.write(base + window_size - 1, data)

### 24.2.4	Coverage Goals

#### 24.2.4.1	Functional Coverage

Functional Coverage Goals

| Category | Target |
| --- | --- |
| Address paths | 100% master-slave combinations via boundary probes |
| Transaction types | Read, write, burst, error (SLVERR) |
| Monitor modes | Enabled and disabled |
| ID values | Full ID range (representative) |

#### 24.2.4.2	Code Coverage

Code Coverage Goals

| Metric | Target |
| --- | --- |
| Line coverage | >95% |
| Branch coverage | >90% |
| FSM coverage | 100% state transitions |
| Toggle coverage | >80% signals |

### 24.2.5	Debug Features

#### 24.2.5.1	Waveform Capture

Bridge tests support VCD/FST waveform dumps:

pytest test_bridge.py --vcd=waves.vcd
gtkwave waves.vcd

#### 24.2.5.2	Debug Signals

Bridge includes optional debug signals:

Debug Signals

| Signal | Description |
| --- | --- |
| dbg_aw_grant | Current AW grant |
| dbg_ar_grant | Current AR grant |
| dbg_outstanding | Outstanding transaction count |
| dbg_state | FSM states |

### 24.2.6	Regression Testing

#### 24.2.6.1	Continuous Integration

CI Test Schedule

| Stage | Tests | Frequency |
| --- | --- | --- |
| Pre-commit | Basic sanity | Each commit |
| Nightly | Full regression | Daily |
| Weekly | Performance | Weekly |

#### 24.2.6.2	Test Matrix

Configuration Test Matrix

| Config | Data Width | Protocol | Tested |
| --- | --- | --- | --- |
| 2x2 | 64 | AXI4 | Yes |
| 4x4 | 64 | AXI4 | Yes |
| 4x4 | 256 | AXI4 | Yes |
| 2x2 | 64 | AXI4+APB | Yes |
| RAPIDS | 512 | AXI4+APB | Yes |

### 24.2.7	Known Limitations

#### 24.2.7.1	Test Gaps

Known Test Gaps

| Gap | Status | Plan |
| --- | --- | --- |
| APB burst stress | Partial | Phase 3 |
| Width downsize stress | Partial | Planned |
| 32-master config | Not tested | Low priority |

#### 24.2.7.2	Simulator Support

Simulator Support Status

| Simulator | Status |
| --- | --- |
| Verilator | Fully supported |
| Icarus Verilog | Supported |
| Commercial (VCS, Questa) | Not tested |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Synthesis and Implementation

## 25.1	Overview

Everything before this page is about what a generated bridge does. This page is about what it costs and how fast it closes, measured rather than estimated: a worked Vivado flow that takes any generated bridge through synthesis, placement and routing as a stand-alone block, the numbers it produced for a reference set of configurations on the two parts this repository’s boards carry, and how to read and repeat them. Chapter 5.3 carries the resource tables; this chapter is the flow behind them.

## 25.2	The Flow

### 25.2.1	Where it lives

projects/components/fabric-gen-ip/bridge/fpga/ is an FPGA build directory in the repository’s standard shape (handbook fpga/cmn-infra/build-flows): a Makefile of variables that includes make/fpga_flow.mk, a tcl/ directory the flow discovers, and reports/. It builds no bitstream. Its one script, synth_only.tcl, is a non-project batch run: read the bridge’s filelist closure, synthesize the bridge as the top with no pins (Vivado’s out-of-context mode), constrain its clocks, place and route, and write reports.

Synthesis flow knobs

| Knob | Default | Meaning |
| --- | --- | --- |
| BRIDGE | bridge_2x2_rw | any generated fixture; the top module and the filelist follow from the name |
| PART | xc7a100tcsg324-1 | the Artix-7 on the Nexys A7; xc7k325tffg900-2 is the Kintex-7 on the Genesys 2 |
| CLK_NS | 10.0 | the period every clock is constrained to |

cd projects/components/fabric-gen-ip/bridge/fpga
make lint  BRIDGE=bridge_2x2_rw_cdc                 # verilator on the same closure, seconds
make synth BRIDGE=bridge_2x2_rw                     # one run, a few minutes
make synth BRIDGE=bridge_4x4_rw PART=xc7k325tffg900-2 CLK_NS=6.667
bin/synth_sweep.sh                                  # the reference set, both parts
bin/summary_table.py                                # reports/summary.csv as the tables below

Each run leaves reports/<bridge>__<part>/ (utilization after synthesis and after routing, a per-instance utilization breakdown, the timing summary, the twenty worst paths overall and register-to-register, the clock-interaction and CDC reports, and a summary.txt) and appends one line to reports/summary.csv. Two bridges, or one bridge on two parts, never overwrite each other. The flow’s build lock serializes Vivado runs in the directory, so the sweep is deliberately one run at a time.

### 25.2.2	How the bridge is constrained

The flow reads the constraints off the bridge’s port list, so a new fixture needs nothing written by hand:

- Clocks. Every port whose name ends in aclk gets a clock of CLK_NS. A single-domain bridge gets one; a bridge with CDC slave ports (HAS 4.5a) gets one per domain, and the domains are declared asynchronous to each other, which is exactly how a system would constrain it. The crossing itself is then checked by the CDC report, not by setup analysis across it.

- Resets. Every *aresetn input is a false path. Reset is asynchronous on assertion in every generated build (MAS 6.1) and deassertion is synchronized outside the bridge.

- Data I/O. Inputs arrive, and outputs must be valid, 30% of the period into the cycle, against the clock of the port they belong to. This is the ordinary out-of-context convention for a block whose neighbours are registered; it is not a property of the bridge, which is why the register-to-register slack is reported on its own and is the figure the tables below quote.

- Defines. XILINX is set so bridge_cam takes its distributed-RAM attributes; Vivado sets SYNTHESIS itself, which removes the simulation-only checkers in the crossbar and subtractive adapter.

### 25.2.3	Reading the summary line

summary.csv columns

| Column | What it is |
| --- | --- |
| luts, ffs, bram_tiles, dsps | post-route utilization of the whole bridge |
| wns_ns | worst setup slack over every path, I/O budget included |
| wns_reg2reg_ns | worst setup slack between registers inside the bridge |
| fmax_reg2reg_mhz | 1000 / (CLK_NS - wns_reg2reg_ns): the clock the internal paths would close at, estimated at this constraint |
| worst_logic_levels | LUT depth of the worst internal path |
| unrouted_nets | must be 0; anything else is a failed run, not a slow one |

The Fmax column is an estimate at the period you constrained. Vivado stops optimizing a path once it has margin, so a bridge that shows +2 ns of slack at 10 ns is not guaranteed to close at 8 ns until you run it at 8 ns. The handbook’s rule applies (fpga/cmn-infra/timing-closure): when a run does fail, look at the magnitude, the clock interaction and the logic levels, in that order, before touching the RTL.

## 25.3	Measured Results

### 25.3.1	What the first run found

The first bridge through this flow, bridge_2x2_rw on the Artix-7 at 10 ns, failed timing by 22 ns with every functional test green. The worst path ran from a master adapter’s tracking pointer, through the crossbar’s AR arbitration and mux, into the slave adapter’s CAM: 40 LUT levels, a 32 ns data path, 83% of it routing. The CAM’s Mode-2 allocate computed a newcomer’s ordering count as the largest count among the entries with the same ID, written as a loop over the 16 entries, and that loop synthesized to sixteen chained comparators hanging off the arbitrated ARID (MAS 4.1). The counts of one ID are always 0..k-1, so the value is the number of matching entries, a popcount; the entry to retire, the one match with count 0, is one-hot, so its index and stored master are OR reductions. With that change the same bridge meets 10 ns.

The CAM ordering-count scan, before and after (2026-09-11)

| bridge_2x2_rw, Artix-7 100T -1, 10 ns | Before | After |
| --- | --- | --- |
| LUTs | 5,824 | 4,615 |
| FFs | 3,253 | 3,253 |
| WNS register-to-register (ns) | -22.27 | +0.12 |
| Worst logic levels | 40 | 9 |
| Fmax estimate (MHz) | 31 | 101 |

No simulation, formal proof or lint had a view of this: the block was correct and its depth was invisible until synthesis. It is why the flow exists and why it runs in minutes.

### 25.3.2	The reference set

Ten generated fixtures cover the shapes the generator produces: the baseline 2x2, its registered-crossbar, QoS, QoS-plus-registered and CDC variants, a 4x4 with mixed widths, a shim-heavy mix, a 32-bit AXI5 pair, the 128-bit native-AXI5 2x2 with Memory Tagging and chunking on every port, and a 5x3 with channel-specific masters. Each is constrained at 100 MHz on the Artix-7 and 150 MHz on the Kintex-7, the clocks the Nexys A7 and Genesys 2 harnesses in this repository run their fabrics at. Every row has zero unrouted nets, zero block RAM and zero DSPs.

Reference set on the Artix-7 100T -1 (Nexys A7), constrained at 10 ns

| Bridge | LUTs | FFs | BRAM | WNS reg-to-reg (ns) | Fmax est. (MHz) | Worst logic levels |
| --- | --- | --- | --- | --- | --- | --- |
| bridge_2x2_axi5 | 5,120 | 4,013 | 0 | -0.11 | 98.9 | 10 |
| bridge_2x2_axi5_native | 8,252 | 6,773 | 0 | -1.20 | 89.3 | 11 |
| bridge_2x2_rw | 5,832 | 4,605 | 0 | -0.04 | 99.6 | 11 |
| bridge_2x2_rw_cdc | 5,184 | 4,021 | 0 | +0.07 | 100.7 | 11 |
| bridge_2x2_rw_pipe | 5,569 | 4,869 | 0 | +0.57 | 106.1 | 8 |
| bridge_2x2_rw_qos | 5,344 | 3,925 | 0 | -0.28 | 97.3 | 11 |
| bridge_2x2_rw_qos_pipe | 5,932 | 4,933 | 0 | -0.02 | 99.8 | 12 |
| bridge_4x4_rw | 40,239 | 38,241 | 0 | -3.01 | 76.9 | 15 |
| bridge_5x3_channels | 30,835 | 28,051 | 0 | -3.21 | 75.7 | 15 |
| bridge_mix_a | 7,994 | 6,623 | 0 | -2.10 | 82.6 | 13 |

Reference set on the Kintex-7 325T -2 (Genesys 2), constrained at 6.667 ns

| Bridge | LUTs | FFs | BRAM | WNS reg-to-reg (ns) | Fmax est. (MHz) | Worst logic levels |
| --- | --- | --- | --- | --- | --- | --- |
| bridge_2x2_axi5 | 4,949 | 4,013 | 0 | +0.59 | 164.6 | 11 |
| bridge_2x2_axi5_native | 8,119 | 6,773 | 0 | +0.47 | 161.3 | 12 |
| bridge_2x2_rw | 5,682 | 4,605 | 0 | +0.56 | 163.8 | 10 |
| bridge_2x2_rw_cdc | 5,030 | 4,021 | 0 | +0.26 | 156.0 | 10 |
| bridge_2x2_rw_pipe | 5,564 | 4,869 | 0 | +1.14 | 180.8 | 9 |
| bridge_2x2_rw_qos | 5,176 | 3,925 | 0 | +0.48 | 161.6 | 11 |
| bridge_2x2_rw_qos_pipe | 5,886 | 4,933 | 0 | +0.53 | 163.0 | 11 |
| bridge_4x4_rw | 40,042 | 38,241 | 0 | -0.14 | 146.8 | 13 |
| bridge_5x3_channels | 30,712 | 28,051 | 0 | -0.68 | 136.1 | 15 |
| bridge_mix_a | 7,844 | 6,623 | 0 | +0.30 | 157.1 | 13 |

### 25.3.3	Reading the rows

Re-measured 2026-10-16 (rev 1.3) on the generator of that date: per-ID target tracking (GitHub #119 L1), trackers sized by max_outstanding (L5), latency modes (L7), the registered CAM insert and free, the per-ID gate built from registered flags, and width converters without the read reassembly pool and with a registered B input. The 2026-09-11 rows these replace predate all of it. Rows within about 0.3 ns of zero move by that much from run to run with placement; read them as “at the target”.

- The 2x2s are at 100 MHz on the Artix-7 and 150 MHz on the Kintex-7. On the Artix-7 every 2x2 except the native-AXI5 one is within 0.3 ns of 10 ns, and bridge_2x2_rw_pipe closes with +0.57 ns. Every 2x2 meets 6.667 ns on the Kintex-7. bridge_2x2_rw grew from 4,615 LUTs and 3,253 FFs to about 5,800 and 4,600: the per-ID target tables, the CAM’s pending-insert and pending-free registers, and the gate’s registered flags.

- The September CAM-allocate path is gone, taken out by the registered CAM insert; no row’s worst path ends at a CAM allocate.

- QoS is back at 100 MHz. In September it missed by 2.91 ns (bridge_2x2_rw_qos). The per-ID gate the arbitration sees in series is now a 1-bit lookup and a one-hot AND (registered busy and full flags), so QoS and QoS behind the registered crossbar are at the target on the Artix-7 and close on the Kintex-7.

- The native-AXI5 2x2 misses the Artix-7 by 1.2 ns: 128-bit beats with every AMBA5 feature on every port, route-dominated. It meets the Kintex-7.

- Converter fixtures are measurable again and much closer. Until 2026-10-16, every read width converter carried a 16 x 272-beat reassembly pool (about 128k LUTs each). bridge_mix_a did not fit the Artix-7 and bridge_4x4_rw did not finish synthesis. With the pool gone (R_INTERLEAVE = 0, legal because slaves may not interleave R), the write converter’s B input registered and its CAM match one-hot:

  - bridge_4x4_rw went from -7.5 to -3.0 ns on the Artix-7 and is at the target on the Kintex-7 (-0.14 ns);

  - bridge_5x3_channels is -3.2 / -0.7 ns;

  - bridge_mix_a is -2.1 ns / met.

- What those three still miss on is wide-path routing. Their worst paths are route-dominated (78-83% route): the 256-bit R data return through a crossbar response mux into a master’s R skid (5x3), and a slave CAM’s pending free into its count array (mix_a). xbar_pipeline registers the response path, and none of these three fixtures uses it.

- Width steps set the size. The 4x4 with 32/64/128/256-bit ports is about seven times the 2x2: every converter carries a full wide beat each way, per path.

- No block RAM, no DSPs, anywhere. The CAMs, trackers and FIFOs are LUT arrays; there is no arithmetic.

## 25.4	Design Notes

### 25.4.1	What moves the numbers

- Data width sets the register count almost linearly: every skid buffer and every crossbar stage holds a full beat.

- Masters x slaves sets the crossbar’s mux width and the arbiter count; the master adapters and slave adapters scale with their own counts.

- xbar_pipeline adds one 2-deep skid per slave-side channel (MAS 2.3), paid in registers, and returns it as the shorter worst path.

- Protocol shims and width converters are the expensive ports: an APB or AXI-Lite slave carries a converter plus its own monitor sandwich, a width step carries a converter each way.

- CDC slave ports add one async FIFO per channel (HAS 4.5a); the cost is the FIFO storage, and the crossing does not enter the fabric’s critical path.

### 25.4.2	Before a board build

Run this flow first. It takes minutes, it needs no constraints file, and it answers “does this configuration fit and close on this part” before a harness, a clock wizard and a pinout are wrapped around it.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	AXI3 Integration

## 26.1	Boundary-Conversion Architecture

The bridge crossbar is AXI4 internally. An AXI3 port is therefore converted at the boundary, not natively carried through the fabric:

- AXI3 slave port (protocol = "axi3" on a slave): the adapter is an AXI4 slave adapter, with timing wrapper, per-ID tracking and request slots. Its AXI4 master face drives axi4_to_axi3, whose AXI3 master face presents the external slave pins.

- AXI3 master port (protocol = "axi3" on a master): the external AXI3 master drives axi3_to_axi4, whose AXI4 slave face feeds the ordinary AXI4 master timing wrapper and the rest of the adapter.

Both converters require the AXI3 data width and the paired AXI4 width to match; any width adaptation is the bridge’s internal dwidth_converter or axil_to_axi4_wide_align path behind the protocol converter, exactly as it is for native AXI4 ports.

## 26.2	Converter-Sourced Constraints

The bridge does not enforce AXI3 semantics itself — it instantiates the converters and relies on their contracts. The constraints below are taken from the converters MAS:

- No write-data interleaving across IDs. axi4_to_axi3_wr stamps every AXI3 W beat with its burst’s AWID and does not interleave W IDs. axi3_to_axi4_wr passes every channel through, so an AXI3 master’s writes overlap. Its W bursts must follow AW order (W may lead its AW). A simulation-only check, ASSERT_A3_WID_STABLE, fires if WID changes mid-burst or a burst’s WID is not the ID of the AW it pairs with.

- Egress bursts longer than 16 beats are chopped. axi4_to_axi3_rd and axi4_to_axi3_wr split AXI4 bursts into sequential AXI3 bursts of up to 16 beats each; INCR and FIXED bursts are advanced/regenerated, while WRAP bursts longer than 16 beats are deferred (assertion-rejected).

- QoS and REGION are absent on AXI3 pins. axi4_to_axi3 drops them; axi3_to_axi4 drives them to zero.

- Same-width converters only. AXI_DATA_WIDTH must be identical on both sides; the bridge places its existing width-adaptation logic behind the converter when the external AXI3 width differs from a target slave width.

- Lock mapping per the ARM AXI3 encoding (ARM IHI 0022: 00 normal, 01 exclusive, 10 locked), as fixed by bridge GitHub #98:

  - axi4_to_axi3: 1'b0 → 2'b00, 1'b1 → 2'b01.

  - axi3_to_axi4: 2'b01 → 1'b1; 2'b00, 2'b10 (locked — AXI4 dropped locked accesses), and the reserved 2'b11 → 1'b0.

- Bandwidth (measured, HAS 5.1). Bursts of two beats or more run at 1.000 through an AXI3 port in either direction, single-beat ones too behind an AXI3 master. An AXI3 slave takes one request per cycle. Up to 32 may be in flight across IDs, and a chopped (17+ beat) request waits for its ID to drain. An unchopped request passes straight to the pins, so round-robin at a shared AXI3 slave holds (0.498 / 0.500). (Fixed 2026-10-18: W7, one write at a time behind an AXI3 master; W8, the slave converter’s issue rate, one-per-ID ordering, a dead W cycle per burst and its request queue.)

- Building an AXI3 slave (W10 and W11 fixed, HAS 5.1). The converter’s storage is gaxi_fifo_sync, so an AXI3 slave synthesizes, and the timing wrapper registers its channels. On the Artix-7 at 10 ns the worst paths of bridge_2x2_axi3_slaves and bridge_mix_e are inside the shared per-ID tracker, as on any multi-master AXI4 slave, not in the AXI3 port.

- Responses are routed by ID. Responses from a shared AXI3 slave go through the per-ID tracker, as for AXI4. Until 2026-10-18 they were routed by FIFO position, which required the slave to answer in AW order across IDs. No test yet has an AXI3 slave reorder across IDs; the AXI4 reordering fixture (bridge_2x2_ooo) exercises the same tracker.

See converters MAS §3.15 AXI4 to AXI3 Converter (15_axi4_to_axi3.md) and §3.16 AXI3 to AXI4 Converter (16_axi3_to_axi4.md) for the full contracts.

## 26.3	Validation Evidence

The following generated configurations exercise AXI3 on both boundaries and pass the bridge GATE and FUNC regression suites:

- bridge_1x2_axi3

- bridge_2x2_axi3_masters

- bridge_2x2_axi3_slaves

- bridge_mix_e

### 26.3.1	Configuration constraints

The bridge validator enforces these AXI3-specific rules at configuration time:

- AXI3 port data widths must be one of {32, 64, 128, 256}.

- CDC on an AXI3 port is rejected in this version.

- AXI3 masters require id_width > 0.

- AXI3 slave id_width must be at least the fabric’s master-index-prefixed width (the same rule that applies to AXI4 slaves).

## 26.4	References

- ARM, AMBA 3 AXI Protocol Specification, ARM IHI 0022 — the AXI3 protocol authority; 4-bit AxLEN, 2-bit AxLOCK, write-data WID, absence of QoS/REGION. (Reference archive)

- ARM, AMBA AXI and ACE Protocol Specification, ARM IHI 0022E — the AXI4 protocol authority. (Reference archive)