RTL Design Sherpa

Pre-Bitstream Timing Prediction on FPGA Artix-7 Companion to the ASIC Methodology White Paper

June 12, 2026

Table of Contents

1 Readme Fpga	5

2 Pre-Bitstream Timing Prediction on FPGA (Artix-7)	5

2.1 Abstract	5

2.2 1. What changes between the ASIC paper and this one	6

2.3 2. The harness on FPGA	7

2.4 3. The Vivado flow	7

2.5 4. The spreadsheet	8

2.5.1 4.1 characterization	8

2.5.2 4.2 building blocks	9

2.6 5. The validation loop	10

2.6.1 5.1 Wrapper	10

2.6.2 5.2 Signature accumulator	11

2.6.3 5.3 On-board observation	11

2.6.4 5.4 Vivado STA contract	11

2.6.5 5.5 Quick command	11

2.6.6 5.6 The simpler sweep – when STA is enough	12

2.6.7 5.7 What v0.1 still doesn’t do	12

2.7 6. Recommended workflow	12

2.8 7. Limitations / intentional non-features	13

2.9 8. Navigation	13

List of Figures

No figures in this document.

List of Tables

No tables in this document.

# 1	Readme Fpga

Generated: 2026-08-14

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Pre-Bitstream Timing Prediction on FPGA (Artix-7)

The FPGA companion to the ASIC pre-synth methodology. Same harness, same nine FUBs, same spreadsheet shape – but the lookup numbers come from Vivado primitive datasheets (and eventually from real post-route reports off the Nexys A7), and the validation loop runs through an on-board clock sweep instead of an STA tool.

Version: 0.1 (initial publication; calibration runs pending) Last Updated: 2026-06-12 Companion to: README.md (ASIC / ASAP7 paper) Status: Vivado flow live; spreadsheet seeded with datasheet numbers; hardware validation loop planned, not built.

## 2.1	Abstract

This document carries the same purpose as the ASIC paper: predict and close timing before the bitstream build ever starts. On ASIC, that prediction is grounded in Yosys + ABC + OpenSTA per-gate-delay measurements. On FPGA the inputs are different but the workflow is the same:

- characterize the relevant Xilinx primitives (LUT6, CARRY4, DSP48, BRAM, MUXF7/F8, LUTRAM, SRL) at the speed grade you’ll ship at,

- record a per-primitive ps-per-level row on the characterization sheet,

- the building-blocks sheet looks those numbers up via VLOOKUP and reports a slack column for every block you might instantiate,

- a designer at the spreadsheet sees fail / pass per block as they tweak the planned clock frequency, before committing to a 10-30 minute Vivado place-and-route round trip.

The deliverable is work/artix7_characterization.xlsx, generated by work/build_fpga_characterization_xlsx.py. The Vivado flow lives under fpga/ – a Nexys A7-100T project that builds the same char_top harness, wrapped for a single board clock and eight observation LEDs.

This methodology is explicitly proactive. It is not a debugger to reach for after Vivado throws a negative WNS at you; it is a design-time budget table the designer consults before writing the first always_ff. The point is to flip the loop – characterize once per (device, speed grade), then build with a known-good budget – so place and route becomes a confirmation step rather than the first time anyone finds out the pipeline doesn’t close.

## 2.2	1. What changes between the ASIC paper and this one

| Topic | ASIC paper | FPGA paper |
| --- | --- | --- |
| Process | TSMC-class ASAP7 PDK (open-source) | Xilinx Artix-7 (xc7a100tcsg324-1 on Nexys A7-100T) |
| Toolchain | Yosys + slang + ABC + OpenSTA | Vivado (synth + impl + bitgen) |
| Corners | TT / FF / SS (Liberty PVT corners) | Speed grade -1 / -2 / -3 |
| Frequency range | 2.0 GHz - 3.0 GHz (where 7nm starts to choke) | 100 - 300 MHz (where Artix-7 typically choke) |
| Per-level primitives | NAND, XOR, MUX, CARRY, MULT, GRAY_CTR, QUEUE | LUT6, MUXF7, MUXF8, CARRY4, DSP48, LUTRAM, SRL |
| Mixed-bag proxy | MULT (multiplier tree) | DSP48 (DSP macro) |
| Reference single-point | INV, CLK_DIV | BRAM (RAMB36 sync read) |
| Tflop source | Tcq + Tsu derived from NAND chain probe | FDRE Tcq + Tsu (~400 ps -1, ~280 ps -3) |
| Wire delay model | per-metal-layer ps/mm (M2/M3 … M8/M9) | single FPGA-routing ps/hop (default 200 ps -1) |
| Ground truth | ABC stime -c per chain | Vivado post-route report_timing_summary |
| Validation loop | corner data already there (same Liberty) | on-board MMCM clock sweep + host-side golden model |

The spreadsheet’s building blocks sheet is identical RTL with FPGA- specific numbers swapped in. The stream sheet (per-FUB port walks) is intentionally omitted from this v0.1 – the per-port hand-walks port over 1:1 from the ASIC sheet, only the looked-up per-level numbers change.

## 2.3	2. The harness on FPGA

char_top is the same nine-FUB harness described in README.md Section 5. All anti-optimisation defences (LFSR-driven inputs, outputs routed to top-level ports, dont_touch attributes) work identically under Vivado.

For board-level use a thin wrapper trims the wide observation ports down to something the Nexys A7’s 8 user LEDs can drive:

- fpga/rtl/char_top_fpga.sv instantiates char_top with all 9 FUBs enabled and XOR-reduces each FUB’s output bus to a single bit.

- 8 LEDs drive one bit per FUB; the 9th FUB folds into LED[0] via an XOR with the NAND output so both chains reach a top-level port without losing an LED.

- A 100 MHz onboard oscillator (board pin E3) feeds clk; the CPU_RESETN button (C12) feeds rst_n; four switches (J15 / L16 / M13 / R15) provide the LFSR seed nibble.

Every chain still reaches an output port, so Vivado optimisation can’t delete anything – the FPGA equivalent of the ASIC scaffold.

## 2.4	3. The Vivado flow

The flow lives under fpga/, shaped like stream_characterization/flows-stream-bridge:

fpga/
├── Makefile                      project | synth | bitstream | program | clean
├── rtl/char_top_fpga.sv          thin wrapper around char_top
├── constraints/char_top_fpga.xdc Nexys A7-100T pin map + 100 MHz clock
└── tcl/
    ├── create_project.tcl        project gen from rtl/filelists/char_top.f
    ├── synth_only.tcl            synth + utilization_synth.txt
    ├── build_all.tcl             synth + impl + bitgen + reports
    ├── program_fpga.tcl          JTAG flash via Vivado hw_server
    └── filelist_utils.tcl        env-var + +incdir + nested -f support

Quick start (with REPO_ROOT exported via the repo’s env_python):

cd projects/NexysA7/timing_characterization/fpga
make help            # see all targets
make synth           # ~2 min, posts utilization + post-synth timing
make bitstream       # ~10-30 min, full flow + all reports
make utilization     # cat the latest utilization report
make timing          # cat the latest post-route timing summary
make program         # flash if board is plugged in

Reports land under fpga/reports/:

| File | Contents |
| --- | --- |
| utilization_synth.txt | post-synth LUT/FF/BRAM/DSP totals |
| utilization_impl.txt | post-route same |
| timing_summary.txt | per-path-group WNS, the calibration source |
| timing_worst.txt | top 50 worst paths |
| power.txt | Vivado’s power estimate |

The bitstream itself lands at fpga/bitstream/timing_char.bit.

## 2.5	4. The spreadsheet

work/artix7_characterization.xlsx is structured exactly like the ASIC companion. Two sheets:

### 2.5.1	4.1 characterization

Per-primitive ps-per-level rows for the seven primitives that scale with chain depth (LUT6, MUXF7, MUXF8, CARRY4, DSP48, LUTRAM, SRL), plus a single-point reference for BRAM access time. Speed grades -1 / -2 / -3 across the five-frequency sweep (100 / 150 / 200 / 250 / 300 MHz).

Sample TT-like numbers (Artix-7 -1 ballparks):

| primitive | per-level ps (-1) | notes |
| --- | --- | --- |
| LUT6 | 200 | single-LUT propagation |
| MUXF7 | 250 | F7 cascade |
| MUXF8 | 280 | F8 cascade |
| CARRY4 | 80 | per-bit CIN -> COUT (dedicated chain) |
| DSP48 | 2500 | PROXY for mixed-bag logic |
| LUTRAM | 600 | distributed RAM read |
| SRL | 400 | SRL16 / SRL32 shift |
| BRAM (ref) | 2400 | RAMB36 sync read, 1-cycle |

Tflop (FDRE Tcq + Tsu): 400 ps at -1, 340 ps at -2, 280 ps at -3.

The Design clocks table at the bottom is the single planning point. One row per clock the design uses:

| clock name | freq (MHz) | target sg | period | input 60% | output 40% | wire/hop |
| --- | --- | --- | --- | --- | --- | --- |
| clk_main | 100 | sg-1 | derived | derived | derived | 200 |
| (5 empty slots) |  |  |  |  |  |  |

Yellow cells are editable; everything else derives. The “wire/hop” column defaults higher than the ASIC sheet (200 ps -1) because FPGA routing dominates timing more than ASIC standard-cell routing.

### 2.5.2	4.2 building blocks

One row per reusable RTL block, with editable params and corner-aware target columns:

- gaxi_skid_buffer

- gaxi_fifo_sync (REG=0)

- gaxi_fifo_sync (REG=1, BRAM-backed)

- arbiter_round_robin

- axi4_master_rd

- axi4_master_wr

- BRAM (raw RAMB36)

Each row’s slack cell pulls from the Design clocks table via VLOOKUP – change clk_main’s freq or speed grade and slack flips per block. Negative slack renders bold red, three-decimal precision (0.000).

## 2.6	5. The validation loop

The ASIC paper has STA as the ground truth – Yosys + ABC + OpenSTA and the delay numbers come out of stime -c. On FPGA the equivalent is Vivado’s post-route report_timing_summary, which make bitstream already emits. The on-board sanity check that “does the spreadsheet match what the chip actually does at this clock?” is the MMCM sweep:

Honest assessment up front. On Artix-7 at the frequencies in this sweep (100–300 MHz), the FUBs are small enough that Vivado’s STA is generally trustworthy, and the simpler bitstream-sweep target (§5.6) gives you everything you need to calibrate the spreadsheet. The MMCM sweep adds little beyond that – it only earns its keep on the rare outlier path where STA mis-modelled the delay and silicon disagrees. Keep it as the “is it real?” gut check; do not treat it as the headline calibration loop. Most users should reach for make bitstream-sweep, not make bitstream-mmcm.

### 2.6.1	5.1 Wrapper

fpga/rtl/char_top_fpga_mmcm.sv wraps an MMCME2_BASE primitive with a VCO running at 1200 MHz, exposing four test clocks to four parallel char_top instances:

| domain | divide | test freq |
| --- | --- | --- |
| clk_test_0 | /8 | 150 MHz |
| clk_test_1 | /6 | 200 MHz |
| clk_test_2 | /5 | 240 MHz |
| clk_test_3 | /4 | 300 MHz |

Each instance gets the same LFSR seed nibble from SW[1:0], so all four domains are doing the same work – just at different clocks. Per-domain reset is held until the MMCM locks, then released through a 2-flop synchroniser into each test clock.

### 2.6.2	5.2 Signature accumulator

fpga/rtl/sig_accum.sv folds every char_top output bus (207 bits worth – nand, inverter, xor, carry, mult, mux, queue, clk_div, gray) into a 32-bit XOR signature in the source domain. A free-running snapshot register updates every 2^16 source-clock cycles and the value crosses into the 100 MHz observation domain via a 2-flop CDC synchroniser (Xilinx ASYNC_REG = "TRUE" attributes preserve the metastability margin).

### 2.6.3	5.3 On-board observation

Eight user LEDs show:

| LED | meaning |
| --- | --- |
| LED[0] | clk_test_0 (150 MHz) signature changed since last snapshot |
| LED[1] | clk_test_1 (200 MHz) signature changed |
| LED[2] | clk_test_2 (240 MHz) signature changed |
| LED[3] | clk_test_3 (300 MHz) signature changed |
| LED[7:4] | lower 4 bits of the signature for the selected clock |

SW[3:2] selects which domain drives LED[7:4] for closer inspection.

A failing clock domain’s signature freezes (LED stays solid one way or the other instead of flickering at a few Hz), so a quick glance at LED[3:0] tells you which test clocks the chip is keeping up with.

### 2.6.4	5.4 Vivado STA contract

fpga/constraints/char_top_fpga_mmcm.xdc declares each MMCM output as a generated clock and groups them as asynchronous to the 100 MHz observation clock, so make bitstream-mmcm produces a timing_summary.txt with one WNS row per clk_test_N group. Those WNS numbers are the calibration source: feed them back into work/build_fpga_characterization_xlsx.py’s PRIM_PS dict and regenerate the workbook.

### 2.6.5	5.5 Quick command

cd projects/NexysA7/timing_characterization/fpga
make bitstream-mmcm   # builds char_top_fpga_mmcm with 4 test clocks
make program          # flash the board
                      # watch LED[3:0] for clock-domain liveness

### 2.6.6	5.6 The simpler sweep – when STA is enough

The MMCM build is only worth the extra silicon when you want to observe on-chip behaviour at multiple clocks from one bitstream. If you only need Vivado’s STA opinion to feed the spreadsheet, the simpler flow is the bitstream-sweep target – it loops the regular char_top_fpga wrapper across a list of target periods, archives each run’s reports under reports/sweep_<F>MHz/, and rolls per-clock-group WNS into a single reports/sweep_wns.csv:

make bitstream-sweep FREQS_MHZ="100 150 200 250 300"
# ... runs N back-to-back full builds ...
cat reports/sweep_wns.csv

CSV columns: freq_mhz, period_ns, group, wns_ns, slack_status, utilization_luts, utilization_ffs, utilization_brams, utilization_dsps. That’s the direct input to recalibrating work/build_fpga_characterization_xlsx.py’s PRIM_PS dict.

Under the hood it’s just a TARGET_PERIOD_NS env var that fpga/tcl/clock_constraint.tcl reads at project-creation time and bakes into a fresh XDC fragment. No MMCM, no sig_accum, no on-chip behaviour – pure post-route Vivado STA.

### 2.6.7	5.7 What v0.1 still doesn’t do

- Host-side golden model. Both sweeps (MMCM and target-freq) report structural / STA signals. Neither asks “did the chip produce THE RIGHT data?” A UART or USB-blaster handshake out to the host’s LFSR reference model would close that gap.

- Spreadsheet auto-update. parse_timing_sweep.py produces sweep_wns.csv but doesn’t feed the values back into the workbook builder’s PRIM_PS dict; that’s still a copy-paste workflow.

## 2.7	6. Recommended workflow

- Plan. Open work/artix7_characterization.xlsx. On characterization, set clk_main freq + speed grade to your target. Flip to building blocks – any red rows are the blocks you’ll need to pipeline harder or use sparingly.

- Iterate the plan. Lower the freq until everything’s green (the design’s actual ceiling on this device); switch the speed grade to model a faster-graded part; bump wire/hop to model longer inter-CLB routes.

- Build. cd fpga; make synth for a quick utilisation check; make bitstream for the full place-and-route + reports.

- Calibrate. Read reports/timing_summary.txt after make bitstream. Compare the per-FUB WNS against what the spreadsheet predicted. Update work/build_fpga_characterization_xlsx.py’s PRIM_PS dict with the calibrated numbers and regenerate the workbook.

The methodology pays for itself the first time it tells you “your plan doesn’t fit at 300 MHz on -1 silicon” before you spend 30 minutes on a Vivado run to learn the same thing.

## 2.8	7. Limitations / intentional non-features

- Seed data are datasheet ballparks, not calibrated measurements. Treat the v0.1 workbook as a methodology demo; the per-LUT / per-CARRY4 numbers are reasonable to start a discussion but should be replaced with calibrated values from real Vivado runs.

- Wire delay is one number. ASIC has four metal layers in the workbook; FPGA’s interconnect is more uniform and the inter-CLB routing dominates. One ps/hop with a per-clock override is enough resolution for a SWAG.

- No MMCM sweep, no host golden model. The on-board validation loop in section 5 is sketched but not wired up.

- No power numbers. Vivado emits report_power; the workbook doesn’t yet model it.

## 2.9	8. Navigation

- ASIC companion paper: README.md

- Vivado flow Makefile: fpga/Makefile

- Board pin map: fpga/constraints/char_top_fpga.xdc

- FPGA wrapper RTL: fpga/rtl/char_top_fpga.sv

- FPGA spreadsheet: work/artix7_characterization.xlsx

- Spreadsheet generator: work/build_fpga_characterization_xlsx.py

- Components area: ../../components/README.md

- Repository guide: /CLAUDE.md

Version: 0.1 Last Updated: 2026-06-12 Maintained By: RTL Design Sherpa Project