RTL Design Sherpa

APB Crossbar Hardware Architecture Specification 1.0

January 3, 2026

Table of Contents

Apbx Xbar Has Index	19

APB Crossbar Hardware Architecture Specification Index	19

Document Organization	19

Front Matter	19

Chapter 1: Introduction	19

Chapter 2: System Overview	19

Chapter 3: Architecture	19

Chapter 4: Interfaces	20

Chapter 5: Performance	20

Chapter 6: Integration	20

Related Documentation	20

Document Information	20

APB Crossbar Hardware Architecture Specification	20

Document Purpose	21

References	21

Terminology	21

Revision History	22

Purpose and Scope	22

Document Purpose	22

Component Overview	23

Document Scope	23

In Scope	23

Out of Scope	23

Specification Level	23

Document Conventions	24

Notation	24

Signal Naming	24

Parameter Naming	25

Diagrams	25

Block Diagrams	25

Address Notation	25

Table Captions	25

Requirement References	25

Definitions and Acronyms	26

Acronyms	26

Terms	27

APB Protocol Signals	28

Use Cases	29

Primary Use Cases	29

UC-1: Simple Peripheral Interconnect (1x4)	29

UC-2: Multi-Master System (2x4)	29

UC-3: Protocol Conversion (1x1)	29

Variant Selection Guide	29

Key Features	30

Feature Summary	30

F1: Parametric MxN Configuration	30

F2: Automatic Address-Based Routing	30

F3: Round-Robin Arbitration	31

F4: Zero-Bubble Throughput	31

F5: Proven Building Blocks	31

Feature Comparison	31

Design Philosophy	32

Composition Over Complexity	32

Scalability	32

System Context	32

System Context Diagram	32

Figure 2.1: System Context Diagram	33

Typical Integration Topology	33

Master-Side Connections	33

Slave-Side Connections	33

Interface Boundaries	34

Input Boundary (Master-Side)	34

Output Boundary (Slave-Side)	34

Clock and Reset	34

System Integration Considerations	34

Address Map Planning	34

Multi-Master Considerations	34

Block Diagram	35

Top-Level Architecture	35

Figure 3.1: APB Crossbar Block Diagram	35

Functional Blocks	35

Master-Side Protocol Conversion	35

Internal Crossbar Fabric	36

Slave-Side Protocol Conversion	36

Module Hierarchy	36

Data Path Width	36

Data Flow	37

Transaction Flow Overview	37

Figure 3.2: Transaction Data Flow	37

Write Transaction Flow	37

Stage 1: Master Request	38

Stage 2: Protocol Conversion (Master-Side)	38

Stage 3: Address Decode	38

Stage 4: Arbitration (if needed)	38

Stage 5: Protocol Conversion (Slave-Side)	38

Stage 6: Response Return	38

Read Transaction Flow	38

Timing Summary	38

Back-to-Back Transactions	39

Address Mapping	39

Address Map Structure	39

Figure 3.3: Address Mapping Structure	40

Default Address Map	41

Address Decode Logic	41

Example Decode	41

Customizing Base Address	41

Address Map Constraints	42

Fixed Slave Region Size	42

Maximum Configuration	42

Address Width Requirement	42

Master-Side APB Interfaces	43

Interface Overview	43

Signal Definition	43

Signal Descriptions	43

PSEL (Input)	43

PENABLE (Input)	44

PADDR (Input)	44

PWRITE (Input)	44

PWDATA (Input)	44

PSTRB (Input)	44

PPROT (Input)	44

PRDATA (Output)	44

PSLVERR (Output)	44

PREADY (Output)	44

Transaction Timing	45

Uncontended Write (No Arbitration Wait)	45

Contended Transaction (Arbitration Wait)	45

Slave-Side APB Interfaces	45

Interface Overview	45

Signal Definition	45

Signal Behavior	46

Output Signals (Crossbar to Slave)	46

Input Signals (Slave to Crossbar)	46

Slave Response Handling	47

Wait States	47

Error Response	47

Multi-Slave Considerations	47

Address Overlap Prevention	47

Independent Operation	47

Clock and Reset	48

Clock Interface	48

Signal Definition	48

Clock Requirements	48

Reset Interface	48

Signal Definition	48

Reset Behavior	48

Reset Safe Values	49

Clock-Reset Relationship	49

Reset Assertion	49

Reset Deassertion	49

Reset Sequence Diagram	49

Integration Notes	49

Clock Domain Crossing	49

Clock Gating	50

Throughput Characteristics	50

Maximum Throughput	50

Single Master	50

Multi-Master (Uncontended)	50

Multi-Master (Contended)	51

Zero-Bubble Operation	51

Grant Persistence	51

Back-to-Back Timing	51

Throughput Factors	51

Factors That Reduce Throughput	51

Optimal Usage Pattern	52

Latency Analysis	52

Transaction Latency	52

Uncontended Access	52

Contended Access	52

Latency Breakdown	53

Forward Path (Master to Slave)	53

Response Path (Slave to Master)	53

Latency Timing Diagram	53

Best Case (No Waits, No Contention)	53

Worst Case (Contention + Slave Wait States)	54

Latency Optimization	54

Design Recommendations	54

Resource Estimates	55

FPGA Resource Usage	55

Pre-Generated Variants	55

Scaling Factors	55

Lines of Code	56

Power Considerations	56

Static Power	56

Dynamic Power	56

Power Optimization	56

Area Comparison	56

Custom Configuration Estimation	57

System Requirements	57

Hardware Requirements	57

Clock and Reset	57

APB Compliance	58

Software Requirements	58

Address Map Configuration	58

Driver Considerations	58

Constraints	59

Address Map Constraints	59

Transaction Constraints	59

Integration Checklist	59

Pre-Integration	59

Post-Integration	59

Parameter Configuration	60

Module Parameters	60

Core Parameters	60

Derived Parameters	60

Parameter Usage Examples	60

Default Configuration	60

Custom Base Address	61

64-bit Data Width	61

Wide Address	61

Parameter Validation	61

Address Width Check	61

Data Width Constraints	62

Custom Generation Parameters	62

Verification Strategy	63

Pre-Verified Component	63

Test Categories	63

Basic Connectivity	63

Address Decode Verification	63

Arbitration Testing	63

Stress Testing	63

Integration Testing Recommendations	63

Level 1: Basic Integration	63

Level 2: Functional Integration	64

Level 3: System Integration	64

Verification Collateral	64

Available Test Infrastructure	64

Running Verification	65

Known Limitations	65

Not Tested	65

Related Documentation	65

APB Crossbar Micro-Architecture Specification Index	66

Document Organization	66

Main Documentation	66

Chapter 1: Architecture	66

Chapter 2: Address Decode and Arbitration	66

Chapter 3: RTL Generator	66

Quick Navigation	66

For New Users	66

For Integration	67

Common Questions	67

Visual Assets	67

Architecture Diagrams	67

Timing Diagrams	67

Component Overview	68

Key Features	68

Pre-Generated Variants	68

Design Philosophy	68

Related Documentation	69

Companion Specifications	69

Project-Level	69

Test Infrastructure	69

RTL	69

Version History	69

Product Requirements Document (PRD)	70

APB Crossbar Generator	70

1. Executive Summary	70

1.1 Quick Stats	70

1.2 Project Goals	70

2. Key Design Principles	70

2.1 Architecture Philosophy	70

2.2 Design Trade-offs	71

3. Architecture Overview	71

3.1 Top-Level Block Diagram	71

3.2 Address Mapping	72

3.3 Arbitration Strategy	73

4. Available Variants	74

4.1 Pre-Generated Modules	74

4.2 Wrapper Modules	75

5. Functional Requirements	75

FR-1: Arbitrary MxN Configuration	75

FR-2: Round-Robin Arbitration	75

FR-3: Address-Based Routing	75

FR-4: Zero-Bubble Throughput	76

FR-5: Configurable Address Map	76

6. Interface Specifications	76

6.1 Master-Side Interface (APB Slave)	76

6.2 Slave-Side Interface (APB Master)	76

7. Parameter Configuration	77

8. Generator Usage	77

8.1 Pre-Generated Variants	77

8.2 Custom Generation	77

8.3 Generator Options	77

9. Performance Characteristics	78

9.1 Latency	78

9.2 Throughput	78

9.3 Resource Utilization (Estimated)	78

10. Verification Status	79

10.1 Test Coverage	79

10.2 Test Methodology	79

11. Integration Guide	79

11.1 Simple Integration (1to4)	79

11.2 Multi-Master Integration (2to4)	80

12. Design Constraints	81

12.1 Limitations	81

12.2 Assumptions	81

13. Future Enhancements	81

13.1 Potential Features	81

13.2 Generator Improvements	81

14. References	82

14.1 Internal Documentation	82

14.2 External Standards	82

14.3 Related Components	82

Claude Code Guide: APB Crossbar Component	82

Quick Context	83

Critical Rules for This Component	83

Rule #0: This is a GENERATOR, Not Just Modules	83

Rule #1: Address Map is Fixed Per-Slave	83

Rule #2: Know the Pre-Generated Variants	84

Rule #3: Generator Syntax	84

Architecture Quick Reference	85

Module Organization	85

Block Diagram (2×4 Example)	85

Common User Questions and Responses	86

Q: “How do I connect a CPU to 4 peripherals?”	86

Q: “I need CPU and DMA to access peripherals. Which crossbar?”	87

Q: “What if I need 3 masters and 8 slaves?”	87

Q: “How does arbitration work?”	88

Q: “Can I change the address map?”	88

Q: “How do I run tests?”	88

Q: “What’s the thin variant for?”	89

Integration Patterns	89

Pattern 1: Simple Peripheral Interconnect	89

Pattern 2: Multi-Master System	90

Pattern 3: Hierarchical Interconnect	90

Anti-Patterns to Catch	91

✗ Anti-Pattern 1: Generating When Pre-Generated Exists	91

✗ Anti-Pattern 2: Assuming Custom Per-Slave Sizes	91

✗ Anti-Pattern 3: Not Mentioning Address Map	92

✗ Anti-Pattern 4: Forgetting About Wrappers	92

Debugging Workflow	92

Issue: Address Not Routing Correctly	92

Issue: Arbitration Not Fair	93

Issue: Back-to-Back Transactions Stalling	93

Quick Commands	93

Remember	94

List of Figures

Figure 2.1: System Context Diagram	33

Figure 3.1: APB Crossbar Block Diagram	35

Figure 3.2: Transaction Data Flow	37

Figure 3.3: Address Mapping Structure	40

List of Tables

Table 1: Reference Documents	21

Table 2: Terminology and Definitions	21

Table 3: Document Revision History	22

Table 4: HAS vs MAS Coverage Comparison	24

Table 5: Signal Naming Conventions	24

Table 6: Parameter Naming Conventions	25

Table 7: Acronym Definitions	26

Table 8: Term Definitions	27

Table 9: APB Protocol Signal Definitions	28

Table 10: Variant Selection Guide	29

Table 11: Pre-Generated Variant Comparison	31

Table 12: Typical Master Connections	33

Table 13: Typical Slave Connections (1to4 example)	34

Table 14: Data Path Width Parameters	36

Table 15: Transaction Timing Summary	38

Table 16: Default Address Map (4-slave example)	41

Table 17: Address Decode Example	41

Table 18: Master Port Signal Definition	43

Table 19: Slave Port Signal Definition	46

Table 20: Clock Signal	48

Table 21: Reset Signal	48

Table 22: Reset Safe Values	49

Table 23: Single Master Throughput	50

Table 24: Contended Access Throughput	51

Table 25: Throughput Reduction Factors	51

Table 26: Uncontended Transaction Latency	52

Table 27: Arbitration Latency	52

Table 28: Forward Path Latency	53

Table 29: Response Path Latency	53

Table 30: Latency Optimization Recommendations	54

Table 31: FPGA Resource Estimates (32-bit)	55

Table 32: Resource Scaling Factors	55

Table 33: Source Code Metrics	56

Table 34: Power Optimization Techniques	56

Table 35: Relative Area Comparison	57

Table 36: Clock and Reset Requirements	57

Table 37: APB Compliance Requirements	58

Table 38: Driver Considerations	58

Table 39: Address Map Constraints	59

Table 40: Transaction Constraints	59

Table 41: Core Parameter Definition	60

Table 42: Derived Parameters	60

Table 43: Valid Data Width Configurations	62

Table 44: Generator Command Line Options	62

Table 45: Pre-Verification Status	63

Table 46: Level 1 Integration Tests	63

Table 47: Level 2 Integration Tests	64

Table 48: Level 3 Integration Tests	64

Table 49: Test Infrastructure	64

Table 50: Known Limitations	65

# Apbx Xbar Has Index

Generated: 2026-08-14

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# APB Crossbar Hardware Architecture Specification Index

Version: 1.0 Date: 2026-01-03 Purpose: High-level hardware architecture specification for APB Crossbar component

## Document Organization

Note: All chapters linked below for automated document generation.

### Front Matter

- Document Information

### Chapter 1: Introduction

- Purpose and Scope

- Document Conventions

- Definitions and Acronyms

### Chapter 2: System Overview

- Use Cases

- Key Features

- System Context

### Chapter 3: Architecture

- Block Diagram

- Data Flow

- Address Mapping

### Chapter 4: Interfaces

- Master-Side APB Interfaces

- Slave-Side APB Interfaces

- Clock and Reset

### Chapter 5: Performance

- Throughput Characteristics

- Latency Analysis

- Resource Estimates

### Chapter 6: Integration

- System Requirements

- Parameter Configuration

- Verification Strategy

## Related Documentation

- APB Crossbar MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

Last Updated: 2026-01-03 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Document Information

## APB Crossbar Hardware Architecture Specification

Document Number: APB-XBAR-HAS-001 Version: 1.0 Status: Production Ready Classification: Open Source - Apache 2.0 License

## Document Purpose

This Hardware Architecture Specification (HAS) provides a high-level architectural overview of the APB Crossbar component. It describes the system-level design, external interfaces, performance characteristics, and integration requirements without detailing internal implementation specifics.

Target Audience: - System architects evaluating APB Crossbar for SoC integration - Hardware engineers planning peripheral interconnect design - Software engineers developing drivers and firmware - Verification engineers planning system-level testing

Companion Documents: - APB Crossbar Micro-Architecture Specification (MAS) - Detailed block-level implementation - APB Crossbar Product Requirements Document (PRD) - Requirements and rationale

## References

Reference Documents

| ID | Document | Description |
| --- | --- | --- |
| [REF-1] | APB Crossbar MAS v1.0 | Micro-Architecture Specification |
| [REF-2] | APB Crossbar PRD | Product Requirements Document |
| [REF-3] | ARM AMBA APB | APB Protocol Specification (IHI 0024C) |

## Terminology

Terminology and Definitions

| Term | Definition |
| --- | --- |
| APB | Advanced Peripheral Bus - ARM AMBA low-power peripheral bus |
| Arbiter | Logic that selects between competing requests using round-robin |
| Crossbar | NxM interconnect fabric connecting multiple masters to multiple slaves |
| Master | APB initiator device that initiates read/write transactions |
| PSEL | APB peripheral select signal indicating slave selection |
| PENABLE | APB enable signal indicating data phase of transaction |
| PREADY | APB ready signal indicating slave response completion |
| Slave | APB target device that responds to master transactions |
| Transaction | Complete APB read or write operation (setup + data phase) |

## Revision History

Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-01-03 | seang | Initial HAS release |

Last Updated: 2026-01-03

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Purpose and Scope

## Document Purpose

This Hardware Architecture Specification (HAS) describes the APB Crossbar component at a system integration level. It provides sufficient detail for:

- System architects to evaluate the component for SoC integration

- Hardware engineers to plan interconnect topology and address mapping

- Software engineers to understand register access paths and address spaces

- Verification engineers to develop system-level test plans

## Component Overview

The APB Crossbar is a parametric MxN interconnect fabric that connects multiple APB masters to multiple APB slaves. It provides:

- Automatic address-based routing to appropriate slave

- Fair round-robin arbitration when multiple masters access the same slave

- Zero-bubble throughput for back-to-back transactions

- Scalable configuration from 1x1 to 16x16

## Document Scope

### In Scope

- System-level architecture and block organization

- External interface specifications (all APB signals)

- Address mapping and routing strategy

- Arbitration policy and fairness guarantees

- Performance characteristics (latency, throughput)

- Integration requirements and parameters

### Out of Scope

- Internal RTL implementation details (see MAS)

- Detailed timing diagrams for internal signals

- Synthesis and implementation guidance

- Test infrastructure details

## Specification Level

This HAS represents a black-box view of the APB Crossbar:

HAS vs MAS Coverage Comparison

| Aspect | HAS Coverage | MAS Coverage |
| --- | --- | --- |
| External ports | Complete | Complete |
| Functional behavior | Complete | Complete |
| Internal architecture | Overview only | Detailed |
| FSM states | - | Complete |
| Timing paths | Interface only | All paths |
| RTL structure | - | Complete |

Next: Document Conventions

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Document Conventions

## Notation

### Signal Naming

Signal Naming Conventions

| Convention | Meaning | Example |
| --- | --- | --- |
| m<i>_apb_* | Master-side APB signals (input to crossbar) | m0_apb_PSEL |
| s<j>_apb_* | Slave-side APB signals (output from crossbar) | s2_apb_PREADY |
| P* | APB protocol signal prefix | PADDR, PWRITE, PRDATA |
| <i>, <j> | Master/slave index placeholder | m0, s3 |

### Parameter Naming

Parameter Naming Conventions

| Convention | Meaning | Example |
| --- | --- | --- |
| UPPER_CASE | Module parameters | ADDR_WIDTH, DATA_WIDTH |
| NUM_* | Count parameters | NUM_MASTERS, NUM_SLAVES |
| *_WIDTH | Bit width parameters | DATA_WIDTH = 32 |
| BASE_* | Base address parameters | BASE_ADDR = 0x10000000 |

## Diagrams

### Block Diagrams

Block diagrams in this document use:

- Rectangles for functional blocks

- Arrows showing data flow direction

- Dashed lines for configuration/control paths

- Labels on connections indicating signal groups

### Address Notation

Addresses are shown in hexadecimal with the 0x prefix:

- 0x1000_0000 - Underscores separate 16-bit groups for readability

- [31:0] - Bit range notation (MSB:LSB)

- 64KB - Size notation using standard abbreviations

## Table Captions

Tables are numbered sequentially within each chapter. A colon (:) followed by the caption text appears below each table.

## Requirement References

When referencing requirements from the PRD:

- FR-n - Functional Requirement number n

- PR-n - Performance Requirement number n

- IR-n - Interface Requirement number n

Next: Definitions and Acronyms

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Definitions and Acronyms

## Acronyms

Acronym Definitions

| Acronym | Definition |
| --- | --- |
| AMBA | Advanced Microcontroller Bus Architecture |
| APB | Advanced Peripheral Bus |
| CPU | Central Processing Unit |
| DMA | Direct Memory Access |
| FSM | Finite State Machine |
| GPIO | General Purpose Input/Output |
| HAS | Hardware Architecture Specification |
| I2C | Inter-Integrated Circuit |
| LOC | Lines of Code |
| LUT | Lookup Table (FPGA resource) |
| MAS | Micro-Architecture Specification |
| PRD | Product Requirements Document |
| RTL | Register Transfer Level |
| SoC | System on Chip |
| SPI | Serial Peripheral Interface |
| UART | Universal Asynchronous Receiver/Transmitter |

## Terms

Term Definitions

| Term | Definition |
| --- | --- |
| Address Decode | Logic that determines which slave should receive a transaction based on address |
| Arbitration | Process of selecting one request when multiple masters compete for the same slave |
| Back-to-back | Consecutive transactions with no idle cycles between them |
| Crossbar | Interconnect topology allowing any master to communicate with any slave |
| Grant Persistence | Maintaining arbitration grant through entire transaction duration |
| Master | Device that initiates APB transactions (CPU, DMA, etc.) |
| Round-Robin | Arbitration scheme that rotates priority among requesters |
| Slave | Device that responds to APB transactions (peripherals) |
| Starvation | Condition where a requester is indefinitely denied access |
| Transaction | Complete APB read or write operation including setup and data phases |
| Zero-Bubble | No idle cycles inserted between consecutive transactions |

## APB Protocol Signals

APB Protocol Signal Definitions

| Signal | Direction | Description |
| --- | --- | --- |
| PCLK | Input | APB clock (all signals synchronous to rising edge) |
| PRESETn | Input | Active-low asynchronous reset |
| PADDR | Master to Slave | Address bus |
| PSEL | Master to Slave | Peripheral select (one per slave) |
| PENABLE | Master to Slave | Enable signal (high during data phase) |
| PWRITE | Master to Slave | Write/read direction (1=write, 0=read) |
| PWDATA | Master to Slave | Write data bus |
| PSTRB | Master to Slave | Write strobes (byte enables) |
| PPROT | Master to Slave | Protection attributes |
| PRDATA | Slave to Master | Read data bus |
| PREADY | Slave to Master | Ready signal (can extend transaction) |
| PSLVERR | Slave to Master | Slave error response |

Next: Chapter 2: System Overview

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Use Cases

## Primary Use Cases

### UC-1: Simple Peripheral Interconnect (1x4)

Scenario: Single CPU accessing multiple peripherals

A microcontroller needs to access UART, GPIO, Timer, and SPI peripherals through a single APB master interface.

Configuration: - 1 Master (CPU) - 4 Slaves (UART, GPIO, Timer, SPI) - Variant: apbx_xbar_1to4

Address Map:

0x1000_0000 - 0x1000_FFFF : UART  (64KB)
0x1001_0000 - 0x1001_FFFF : GPIO  (64KB)
0x1002_0000 - 0x1002_FFFF : Timer (64KB)
0x1003_0000 - 0x1003_FFFF : SPI   (64KB)

### UC-2: Multi-Master System (2x4)

Scenario: CPU and DMA both accessing peripherals

An SoC where both CPU and DMA controller need access to shared peripherals.

Configuration: - 2 Masters (CPU, DMA) - 4 Slaves (peripherals) - Variant: apbx_xbar_2to4

Key Behavior: - Round-robin arbitration when both masters access same peripheral - No master starvation guaranteed - Zero-bubble throughput on uncontended access

### UC-3: Protocol Conversion (1x1)

Scenario: APB timing boundary or clock domain preparation

Insert APB crossbar as a timing stage or preparation for clock domain crossing.

Configuration: - 1 Master - 1 Slave - Variant: apbx_xbar_1to1 or apbx_xbar_thin

Benefits: - Isolates master from slave timing - Provides consistent transaction buffering - Minimal resource overhead (thin variant)

## Variant Selection Guide

Variant Selection Guide

| Use Case | Recommended Variant | Rationale |
| --- | --- | --- |
| Simple peripheral bus | apbx_xbar_1to4 | Most common SoC configuration |
| CPU + DMA system | apbx_xbar_2to4 | Fair access for both masters |
| Timing isolation | apbx_xbar_thin | Minimal overhead passthrough |
| Multi-master arbitration | apbx_xbar_2to1 | Focus on arbitration only |
| Custom topology | Generator | Any MxN configuration |

Next: Key Features

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Key Features

## Feature Summary

The APB Crossbar provides the following key features:

### F1: Parametric MxN Configuration

- Support for any combination of M masters and N slaves

- Pre-generated variants for common configurations (1x1, 2x1, 1x4, 2x4)

- Python generator for custom configurations up to 16x16

- Single RTL source serves all configurations

### F2: Automatic Address-Based Routing

- Parallel address decode for all slaves

- Fixed 64KB address space per slave

- Configurable base address via BASE_ADDR parameter

- Zero-decode-latency routing

Address Calculation:

slave_index = (PADDR - BASE_ADDR) >> 16

### F3: Round-Robin Arbitration

- Independent arbiter per slave

- Fair rotation of master priority

- No master starvation guaranteed

- Grant persistence through transaction completion

### F4: Zero-Bubble Throughput

- Back-to-back transactions without idle cycles

- Grant held during entire transaction

- Immediate response routing

- Maximum APB bandwidth utilization

### F5: Proven Building Blocks

- Built from production-tested apb4_slave.sv and apb4_master.sv modules

- No new protocol logic - pure composition

- Each component independently verified

## Feature Comparison

Pre-Generated Variant Comparison

| Feature | apbx_xbar_1to1 | apbx_xbar_2to1 | apbx_xbar_1to4 | apbx_xbar_2to4 | apbx_xbar_thin |
| --- | --- | --- | --- | --- | --- |
| Masters | 1 | 2 | 1 | 4 | 1 |
| Slaves | 1 | 1 | 4 | 4 | 1 |
| Arbitration | No | Yes | No | Yes | No |
| Address Decode | No | No | Yes | Yes | No |
| Approximate LOC | 200 | 400 | 500 | 1000 | 150 |

## Design Philosophy

### Composition Over Complexity

The crossbar is built by composing well-tested building blocks:

- APB Slaves (M instances) - Convert incoming APB to internal cmd/rsp

- Arbiters (N instances) - Select winning master per slave

- Address Decode - Route commands to appropriate slave

- Response Routing - Return responses to originating masters

- APB Masters (N instances) - Convert internal cmd/rsp back to APB

### Scalability

Resource usage scales predictably: - M x N routing paths - N independent arbiters - Linear growth with configuration size

Next: System Context

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# System Context

## System Context Diagram

The following diagram shows the APB Crossbar in a typical SoC context:

### Figure 2.1: System Context Diagram

System Context Diagram

## Typical Integration Topology

### Master-Side Connections

The APB Crossbar connects to upstream APB masters:

Typical Master Connections

| Master Type | Typical Source | Connection Point |
| --- | --- | --- |
| CPU | AHB-to-APB Bridge | m0_apb_* |
| DMA | DMA Controller APB Port | m1_apb_* |
| Debug | Debug Access Port | m2_apb_* (if present) |

### Slave-Side Connections

The crossbar connects to downstream APB peripherals:

Typical Slave Connections (1to4 example)

| Slave Type | Address Offset | Connection Point |
| --- | --- | --- |
| UART | +0x0_0000 | s0_apb_* |
| GPIO | +0x1_0000 | s1_apb_* |
| Timer | +0x2_0000 | s2_apb_* |
| SPI | +0x3_0000 | s3_apb_* |

## Interface Boundaries

### Input Boundary (Master-Side)

Each master port presents a complete APB slave interface:

- Receives PSEL, PENABLE, PADDR, PWRITE, PWDATA, PSTRB, PPROT

- Returns PRDATA, PSLVERR, PREADY

### Output Boundary (Slave-Side)

Each slave port presents a complete APB master interface:

- Drives PSEL, PENABLE, PADDR, PWRITE, PWDATA, PSTRB, PPROT

- Receives PRDATA, PSLVERR, PREADY

### Clock and Reset

Single clock domain: - pclk - APB clock (all signals synchronous) - presetn - Active-low asynchronous reset

## System Integration Considerations

### Address Map Planning

The fixed 64KB per-slave allocation requires planning:

- Peripheral fit: Ensure each peripheral needs <= 64KB address space

- Base address: Set BASE_ADDR to avoid conflicts with other subsystems

- Address width: ADDR_WIDTH must accommodate BASE_ADDR + (NUM_SLAVES x 64KB)

### Multi-Master Considerations

When using multiple masters:

- Arbitration overhead: Expect 1-2 cycle delay on contended access

- Priority: All masters have equal priority (round-robin)

- Bandwidth: Shared access reduces per-master bandwidth

Next: Chapter 3: Architecture

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Block Diagram

## Top-Level Architecture

The APB Crossbar consists of three main functional layers:

### Figure 3.1: APB Crossbar Block Diagram

Block Diagram

## Functional Blocks

### Master-Side Protocol Conversion

Component: apb4_slave[M] instances (one per master)

Each APB slave instance: - Accepts incoming APB transactions from a master - Converts APB protocol to internal command/response format - Provides transaction buffering - Generates appropriate response timing

Interface: - Input: Complete APB slave signals from external master - Output: Internal cmd/rsp bus to crossbar fabric

### Internal Crossbar Fabric

Components: - Address Decode - Parallel decode logic - Per-Slave Arbiters - Round-robin arbiter per slave (N instances) - Response Routing - Registered response mux

The crossbar fabric: 1. Decodes transaction address to determine target slave 2. Arbitrates when multiple masters request the same slave 3. Routes commands to the selected slave 4. Routes responses back to the originating master

### Slave-Side Protocol Conversion

Component: apb4_master[N] instances (one per slave)

Each APB master instance: - Accepts internal cmd/rsp from crossbar fabric - Generates compliant APB transactions to the slave - Handles wait states via PREADY - Propagates error responses via PSLVERR

Interface: - Input: Internal cmd/rsp bus from crossbar fabric - Output: Complete APB master signals to external slave

## Module Hierarchy

apbx_xbar_MxN
+-- apb4_slave[0..M-1]          # Master-side conversion
+-- addr_decode                 # Parallel address decode
+-- arbiter[0..N-1]            # Per-slave round-robin
+-- response_mux               # Response routing
+-- apb4_master[0..N-1]         # Slave-side conversion

## Data Path Width

All data paths maintain consistent width:

Data Path Width Parameters

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| ADDR_WIDTH | 32 | 8-64 | Address bus width |
| DATA_WIDTH | 32 | 8-64 | Data bus width |
| STRB_WIDTH | 4 | 1-8 | Byte strobe width (DATA_WIDTH/8) |

Next: Data Flow

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Data Flow

## Transaction Flow Overview

### Figure 3.2: Transaction Data Flow

Data Flow Diagram

## Write Transaction Flow

A write transaction flows through the following stages:

### Stage 1: Master Request

The external master initiates an APB write: 1. Master asserts PSEL and PADDR with target address 2. Master sets PWRITE=1 and provides PWDATA 3. Next cycle: Master asserts PENABLE

### Stage 2: Protocol Conversion (Master-Side)

The apb4_slave module converts to internal format: 1. Captures transaction parameters (address, data, write) 2. Generates internal command 3. Routes to address decode

### Stage 3: Address Decode

Parallel decode determines target slave:

slave_index = (PADDR - BASE_ADDR) >> 16

### Stage 4: Arbitration (if needed)

If multiple masters target the same slave: 1. Arbiter selects one master (round-robin priority) 2. Other masters wait (PREADY low) 3. Grant held until transaction completes

### Stage 5: Protocol Conversion (Slave-Side)

The apb4_master module generates APB transaction: 1. Asserts PSEL to selected slave 2. Provides PADDR, PWRITE, PWDATA 3. Asserts PENABLE next cycle 4. Waits for slave PREADY

### Stage 6: Response Return

Response flows back to originating master: 1. Slave asserts PREADY (with optional PSLVERR) 2. apb4_master captures response 3. Response routed to correct apb4_slave 4. apb4_slave asserts PREADY to original master

## Read Transaction Flow

Read transactions follow the same flow with these differences:

- PWRITE=0 indicates read operation

- No PWDATA on forward path

- PRDATA returned on response path

## Timing Summary

Transaction Timing Summary

| Transaction Phase | Clock Cycles | Notes |
| --- | --- | --- |
| Setup phase | 1 | PSEL asserted, PENABLE low |
| Data phase | 1+ | PENABLE high, wait for PREADY |
| Decode | 0 | Combinational (parallel decode) |
| Arbitration | 0-1 | 0 if uncontended, 1 if contended |

## Back-to-Back Transactions

The crossbar supports zero-bubble back-to-back transactions:

- Master can assert new PSEL immediately after PREADY

- Grant persistence eliminates re-arbitration overhead

- Maximum throughput: 1 transaction per 2 APB cycles

Next: Address Mapping

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Address Mapping

## Address Map Structure

The APB Crossbar uses a fixed 64KB address region per slave:

### Figure 3.3: Address Mapping Structure

Address Mapping

## Default Address Map

With default BASE_ADDR = 0x1000_0000:

Default Address Map (4-slave example)

| Slave Index | Start Address | End Address | Size |
| --- | --- | --- | --- |
| 0 | 0x1000_0000 | 0x1000_FFFF | 64KB |
| 1 | 0x1001_0000 | 0x1001_FFFF | 64KB |
| 2 | 0x1002_0000 | 0x1002_FFFF | 64KB |
| 3 | 0x1003_0000 | 0x1003_FFFF | 64KB |
| … | … | … | … |
| N-1 | BASE + (N-1)*64KB | BASE + N*64KB - 1 | 64KB |

## Address Decode Logic

The target slave is determined by simple bit extraction:

offset = PADDR - BASE_ADDR
slave_index = offset[19:16]    // Bits 19:16 select slave (0-15)
local_addr = offset[15:0]      // Bits 15:0 are local address within slave

### Example Decode

For address 0x1002_3456 with BASE_ADDR = 0x1000_0000:

Address Decode Example

| Step | Calculation | Result |
| --- | --- | --- |
| 1. Compute offset | 0x1002_3456 - 0x1000_0000 | 0x0002_3456 |
| 2. Extract slave index | offset[19:16] | 0x2 (Slave 2) |
| 3. Extract local address | offset[15:0] | 0x3456 |

## Customizing Base Address

The BASE_ADDR parameter shifts the entire address map:

// Peripheral space at 0x4000_0000
apbx_xbar_1to4 #(
    .BASE_ADDR(32'h4000_0000)
) u_xbar (...);

// Resulting map:
// Slave 0: 0x4000_0000 - 0x4000_FFFF
// Slave 1: 0x4001_0000 - 0x4001_FFFF
// Slave 2: 0x4002_0000 - 0x4002_FFFF
// Slave 3: 0x4003_0000 - 0x4003_FFFF

## Address Map Constraints

### Fixed Slave Region Size

Each slave occupies exactly 64KB (0x10000 bytes). This is a current design constraint.

Implications: - Peripherals requiring more than 64KB need multiple slave ports - Peripherals using less than 64KB have unused address space

### Maximum Configuration

With 4-bit slave index extraction: - Maximum 16 slaves - Total address space: 16 x 64KB = 1MB

### Address Width Requirement

The ADDR_WIDTH parameter must be sufficient to address: - BASE_ADDR + (NUM_SLAVES x 64KB)

Example: For 4 slaves at BASE_ADDR = 0x8000_0000, need: - 0x8000_0000 + 0x4_0000 = 0x8004_0000 - Minimum ADDR_WIDTH: 32 bits

Next: Chapter 4: Interfaces

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Master-Side APB Interfaces

## Interface Overview

Each master port provides a complete APB slave interface to accept transactions from an external APB master (CPU, DMA, etc.).

## Signal Definition

For each master index i (0 to M-1):

Master Port Signal Definition

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| m<i>_apb_PSEL | 1 | Input | Peripheral select |
| m<i>_apb_PENABLE | 1 | Input | Transfer enable |
| m<i>_apb_PADDR | ADDR_WIDTH | Input | Address bus |
| m<i>_apb_PWRITE | 1 | Input | Write/read direction |
| m<i>_apb_PWDATA | DATA_WIDTH | Input | Write data bus |
| m<i>_apb_PSTRB | STRB_WIDTH | Input | Write strobes |
| m<i>_apb_PPROT | 3 | Input | Protection attributes |
| m<i>_apb_PRDATA | DATA_WIDTH | Output | Read data bus |
| m<i>_apb_PSLVERR | 1 | Output | Slave error response |
| m<i>_apb_PREADY | 1 | Output | Transfer ready |

## Signal Descriptions

### PSEL (Input)

Peripheral select signal. When asserted: - Indicates master wants to access the crossbar - Must remain asserted throughout the transaction - Combined with PADDR determines target slave

### PENABLE (Input)

Transfer enable signal. APB state machine: - PSEL=1, PENABLE=0: Setup phase - PSEL=1, PENABLE=1: Access phase (data transfer)

### PADDR (Input)

Address bus. Used for: - Target slave selection (upper bits) - Local address within slave (lower bits) - Must be stable while PSEL asserted

### PWRITE (Input)

Transfer direction: - PWRITE=1: Write transaction (data from master) - PWRITE=0: Read transaction (data to master)

### PWDATA (Input)

Write data bus: - Valid during write transactions - Sampled when PREADY and PENABLE both high - Width set by DATA_WIDTH parameter

### PSTRB (Input)

Write byte strobes: - One bit per byte of DATA_WIDTH - Indicates which bytes are valid in PWDATA - Ignored during read transactions

### PPROT (Input)

Protection attributes (3 bits): - Bit 0: Privileged/normal access - Bit 1: Secure/non-secure access - Bit 2: Instruction/data access

### PRDATA (Output)

Read data bus: - Valid during read transactions when PREADY high - Undefined during write transactions - Width matches DATA_WIDTH parameter

### PSLVERR (Output)

Slave error response: - Sampled when PREADY and PENABLE both high - PSLVERR=1 indicates error condition - Propagated from downstream slave

### PREADY (Output)

Transfer ready signal: - PREADY=1 allows transaction to complete - PREADY=0 inserts wait states - Reflects downstream slave PREADY (or arbitration wait)

## Transaction Timing

### Uncontended Write (No Arbitration Wait)

Cycle 1    Cycle 2    Cycle 3
PSEL   __|-----------|----------|____
PENABLE__|________---|----------|____
PADDR  --<  Address  >--
PWRITE --<  1        >--
PWDATA --<  Data     >--
PREADY _________|----------|____

### Contended Transaction (Arbitration Wait)

When another master holds the target slave:

Cycle 1    Cycle 2    Cycle 3    Cycle 4
PSEL   __|--------------------------|____
PENABLE__|_____________|--------------|____
PADDR  --<  Address                   >--
PREADY _______________________|------|____
                     ^ Wait for grant

Next: Slave-Side Interfaces

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Slave-Side APB Interfaces

## Interface Overview

Each slave port provides a complete APB master interface to drive transactions to an external APB slave (peripheral).

## Signal Definition

For each slave index j (0 to N-1):

Slave Port Signal Definition

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s<j>_apb_PSEL | 1 | Output | Peripheral select |
| s<j>_apb_PENABLE | 1 | Output | Transfer enable |
| s<j>_apb_PADDR | ADDR_WIDTH | Output | Address bus |
| s<j>_apb_PWRITE | 1 | Output | Write/read direction |
| s<j>_apb_PWDATA | DATA_WIDTH | Output | Write data bus |
| s<j>_apb_PSTRB | STRB_WIDTH | Output | Write strobes |
| s<j>_apb_PPROT | 3 | Output | Protection attributes |
| s<j>_apb_PRDATA | DATA_WIDTH | Input | Read data bus |
| s<j>_apb_PSLVERR | 1 | Input | Slave error response |
| s<j>_apb_PREADY | 1 | Input | Transfer ready |

## Signal Behavior

### Output Signals (Crossbar to Slave)

PSEL: Asserted when a transaction targets this slave. Remains high throughout the transaction.

PENABLE: Follows standard APB timing: - Low during setup phase - High during access phase

PADDR: Contains the full address from the master. The local address within the 64KB region is in bits [15:0].

PWRITE, PWDATA, PSTRB, PPROT: Passed through from the winning master without modification.

### Input Signals (Slave to Crossbar)

PRDATA: Captured by crossbar when both PREADY and PENABLE are high. Routed to originating master.

PSLVERR: Captured with PRDATA. Propagated to originating master as error indication.

PREADY: Controls transaction completion: - PREADY=1: Transaction completes this cycle - PREADY=0: Insert wait states

## Slave Response Handling

### Wait States

Slaves can insert wait states by holding PREADY low:

Cycle 1    Cycle 2    Cycle 3    Cycle 4
PSEL   __|-------------------------------|____
PENABLE__|____________|------------------|____
PREADY _____________________|-----------|____
                      ^ Wait states

The crossbar: - Holds master PREADY low while waiting - Does not timeout (assumes slaves always respond) - Passes through all wait states to master

### Error Response

When slave asserts PSLVERR:

- Crossbar captures PSLVERR with PRDATA

- Routes PSLVERR to originating master

- Master sees PSLVERR=1 when its PREADY goes high

No error handling or recovery - error indication only.

## Multi-Slave Considerations

### Address Overlap Prevention

Each slave port is mutually exclusive: - Only one slave receives PSEL for any given address - No overlapping address regions - Address decode is deterministic

### Independent Operation

Each slave operates independently: - Own PREADY timing - No synchronization between slaves - Concurrent transactions to different slaves allowed (different masters)

Next: Clock and Reset

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Clock and Reset

## Clock Interface

### Signal Definition

Clock Signal

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| pclk | 1 | Input | APB clock |

### Clock Requirements

Single Clock Domain: - All crossbar logic operates on pclk rising edge - All APB signals sampled/driven synchronous to pclk - No internal clock generation or division

Frequency: - No minimum frequency requirement - Maximum frequency determined by critical path - Typical: 100-250 MHz (depends on target technology)

Duty Cycle: - No specific duty cycle requirement - Standard 50% duty cycle recommended

## Reset Interface

### Signal Definition

Reset Signal

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| presetn | 1 | Input | Active-low asynchronous reset |

### Reset Behavior

Assertion (presetn goes low): - All internal state cleared immediately - All output signals driven to safe values - Master ports: PREADY=1, PRDATA=0, PSLVERR=0 - Slave ports: PSEL=0, PENABLE=0, PADDR=0, etc.

Deassertion (presetn goes high): - Internal state begins normal operation - Reset release synchronized to pclk internally - Ready to accept transactions immediately

### Reset Safe Values

Reset Safe Values

| Signal Group | Reset Value | Rationale |
| --- | --- | --- |
| Master PREADY | 1 | No false wait states |
| Master PRDATA | 0 | No false data |
| Master PSLVERR | 0 | No false errors |
| Slave PSEL | 0 | No false transactions |
| Slave PENABLE | 0 | No false enables |
| Arbiter grants | 0 | No master selected |

## Clock-Reset Relationship

### Reset Assertion

Reset can be asserted at any time: - Asynchronous assertion (immediate effect) - Synchronous or asynchronous deassertion - All in-flight transactions aborted

### Reset Deassertion

For clean operation: 1. Assert reset for minimum 2 pclk cycles 2. Deassert reset synchronous to pclk rising edge 3. Wait 1 cycle before first transaction

### Reset Sequence Diagram

pclk      _|~|_|~|_|~|_|~|_|~|_|~|_|~|_

presetn   ________|~~~~~~~~~~~~~~~~

Internal  xxxxxxxx|--- Valid State --
State

Ready for ________|--- Transactions --
Traffic

## Integration Notes

### Clock Domain Crossing

If masters or slaves are in different clock domains: - Insert appropriate CDC logic external to crossbar - Crossbar assumes single clock domain - Consider APB clock bridge if needed

### Clock Gating

The crossbar supports clock gating: - No internal activity when idle - Safe to gate pclk when no transactions pending - All registers use standard flip-flops (no clock gating cells)

Next: Chapter 5: Performance

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Throughput Characteristics

## Maximum Throughput

### Single Master

With a single master accessing any slave:

Single Master Throughput

| Metric | Value | Notes |
| --- | --- | --- |
| Minimum cycles per transaction | 2 | APB protocol minimum |
| Maximum transactions per cycle | 0.5 | 1 transaction / 2 cycles |
| Data throughput (32-bit @ 100MHz) | 200 MB/s | Theoretical maximum |
| Data throughput (32-bit @ 250MHz) | 500 MB/s | Theoretical maximum |

### Multi-Master (Uncontended)

When masters access different slaves: - Each master achieves single-master throughput - Aggregate throughput = M x single-master throughput - No arbitration overhead

### Multi-Master (Contended)

When masters compete for the same slave:

Contended Access Throughput

| Masters | Effective Throughput | Notes |
| --- | --- | --- |
| 2 | 50% per master | Round-robin sharing |
| 3 | 33% per master | Round-robin sharing |
| 4 | 25% per master | Round-robin sharing |

## Zero-Bubble Operation

### Grant Persistence

The crossbar implements grant persistence: - Once a master wins arbitration, it holds the grant - Grant released only when master releases PSEL - Enables back-to-back transactions without re-arbitration

### Back-to-Back Timing

Cycle 1    Cycle 2    Cycle 3    Cycle 4
       |  T1 setup|  T1 data |  T2 setup|  T2 data|
PSEL   --|--------|----------|----------|----------|--
PENABLE--|________|----------|__________|----------|--
PREADY  _|________|----------|__________|----------|__

Transaction T1 and T2 are consecutive with no idle cycles.

## Throughput Factors

### Factors That Reduce Throughput

Throughput Reduction Factors

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Slave wait states | Variable | Choose faster peripherals |
| Arbitration conflicts | 1+ cycle | Distribute access across slaves |
| Slave response time | Variable | Design slaves for quick response |

### Optimal Usage Pattern

For maximum throughput: 1. Distribute master access across different slaves 2. Minimize slave wait states 3. Use burst-like access patterns (same master, same slave) 4. Avoid rapid master switching on same slave

Next: Latency Analysis

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Latency Analysis

## Transaction Latency

### Uncontended Access

When a master has exclusive access to a slave:

Uncontended Transaction Latency

| Phase | Cycles | Description |
| --- | --- | --- |
| Setup | 1 | PSEL asserted, PENABLE low |
| Access | 1+ | PENABLE high, wait for PREADY |
| Total | 2+ | Minimum 2 cycles |

### Contended Access

When multiple masters compete for the same slave:

Arbitration Latency

| Scenario | Additional Latency | Description |
| --- | --- | --- |
| Win arbitration | 0 cycles | No delay |
| Lose to 1 master | 2+ cycles | Wait for one transaction |
| Lose to N masters | 2N+ cycles | Wait for N transactions |

## Latency Breakdown

### Forward Path (Master to Slave)

Forward Path Latency

| Component | Latency | Type |
| --- | --- | --- |
| apb4_slave capture | 0-1 cycle | Registered |
| Address decode | 0 cycles | Combinational |
| Arbitration | 0-1 cycle | Combinational + wait |
| apb4_master drive | 0-1 cycle | Registered |
| Typical Total | 1-2 cycles | From PSEL to slave PSEL |

### Response Path (Slave to Master)

Response Path Latency

| Component | Latency | Type |
| --- | --- | --- |
| Slave response | Variable | PREADY timing |
| apb4_master capture | 0-1 cycle | Registered |
| Response routing | 0 cycles | Combinational |
| apb4_slave drive | 0-1 cycle | Registered |
| Typical Total | 1-2 cycles | From slave PREADY to master PREADY |

## Latency Timing Diagram

### Best Case (No Waits, No Contention)

Master PSEL   ___|-----------|___
Master PENABLE___|_____------|___
               ^         ^
Slave PSEL    _____|---------|___
Slave PENABLE _____|___------|___
Slave PREADY  _________|-----|___
                       ^
Master PREADY _________|-----|___
                       ^
              Total: 2 cycles (APB minimum)

### Worst Case (Contention + Slave Wait States)

Master PSEL   ___|--------------------------|___
Master PENABLE___|________________----------|___
               ^  Arbitration    ^
               |  wait           |
Slave PSEL    ____________|---------------|___
Slave PENABLE ____________|______---------|___
Slave PREADY  _____________________|------|___
                                   ^
Master PREADY _____________________|------|___
                                   ^
              Total: 5+ cycles (depends on contention + slave)

## Latency Optimization

### Design Recommendations

Latency Optimization Recommendations

| Goal | Recommendation |
| --- | --- |
| Minimize contention | Use more slaves, spread access |
| Reduce arbitration wait | Lower master count per slave |
| Reduce slave latency | Design slaves with quick PREADY |

Next: Resource Estimates

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Resource Estimates

## FPGA Resource Usage

### Pre-Generated Variants

Estimated resource usage for common configurations (32-bit data width):

FPGA Resource Estimates (32-bit)

| Variant | LUTs | FFs | Description |
| --- | --- | --- | --- |
| apbx_xbar_thin | ~50 | ~20 | Minimal passthrough |
| apbx_xbar_1to1 | ~150 | ~80 | Full 1x1 with buffering |
| apbx_xbar_2to1 | ~300 | ~150 | 2 masters, arbitration |
| apbx_xbar_1to4 | ~400 | ~200 | 4 slaves, decode |
| apbx_xbar_2to4 | ~800 | ~400 | Full 2x4 crossbar |

### Scaling Factors

Resource usage scales approximately as:

Resource Scaling Factors

| Component | Scaling | Notes |
| --- | --- | --- |
| apb4_slave instances | M x base | Linear with masters |
| apb4_master instances | N x base | Linear with slaves |
| Arbiters | N x log2(M) | Per slave, sized for masters |
| Address decode | N | Comparators per slave |
| Response mux | N x M | Full crosspoint for responses |

## Lines of Code

Source code metrics for pre-generated variants:

Source Code Metrics

| Variant | Lines of Code | Modules | Complexity |
| --- | --- | --- | --- |
| apbx_xbar_thin | ~150 | 1 | Low |
| apbx_xbar_1to1 | ~200 | 3 | Low |
| apbx_xbar_2to1 | ~400 | 5 | Medium |
| apbx_xbar_1to4 | ~500 | 7 | Medium |
| apbx_xbar_2to4 | ~1000 | 11 | Medium-High |

## Power Considerations

### Static Power

Static power scales with: - Total register count (FFs) - Routing resources used - Technology node

### Dynamic Power

Dynamic power depends on: - Transaction rate (activity factor) - Data width (toggling bits) - Clock frequency

### Power Optimization

Power Optimization Techniques

| Technique | Benefit | Trade-off |
| --- | --- | --- |
| Clock gating | Reduce idle power | Adds gating logic |
| Lower data width | Fewer toggling bits | Reduced throughput |
| Fewer slaves | Less decode logic | Reduced flexibility |

## Area Comparison

Relative area comparison (normalized to apbx_xbar_1to1):

Relative Area Comparison

| Variant | Relative Area |
| --- | --- |
| apbx_xbar_thin | 0.3x |
| apbx_xbar_1to1 | 1.0x (baseline) |
| apbx_xbar_2to1 | 2.0x |
| apbx_xbar_1to4 | 2.5x |
| apbx_xbar_2to4 | 5.0x |

## Custom Configuration Estimation

For custom MxN configuration:

Estimated LUTs ~= 50 + (M * 75) + (N * 50) + (M * N * 10)
Estimated FFs  ~= 20 + (M * 40) + (N * 30)

Example: 4x8 crossbar - LUTs: 50 + (475) + (850) + (4810) = 50 + 300 + 400 + 320 = ~1070 - FFs: 20 + (440) + (830) = 20 + 160 + 240 = ~420

Next: Chapter 6: Integration

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# System Requirements

## Hardware Requirements

### Clock and Reset

Clock and Reset Requirements

| Requirement | Specification |
| --- | --- |
| Clock input | Single clock (pclk) |
| Clock frequency | No minimum, technology-dependent maximum |
| Reset input | Active-low asynchronous (presetn) |
| Reset duration | Minimum 2 clock cycles |

### APB Compliance

The crossbar requires APB-compliant masters and slaves:

APB Compliance Requirements

| Requirement | Description |
| --- | --- |
| Protocol version | APB4 (with PSTRB, PPROT) |
| Wait states | Supported (PREADY-based) |
| Error response | Supported (PSLVERR) |
| Slave timeout | Not handled internally |

## Software Requirements

### Address Map Configuration

Software must be configured with the correct address map:

// Example address definitions
#define PERIPHERAL_BASE     0x10000000
#define UART_BASE          (PERIPHERAL_BASE + 0x00000)
#define GPIO_BASE          (PERIPHERAL_BASE + 0x10000)
#define TIMER_BASE         (PERIPHERAL_BASE + 0x20000)
#define SPI_BASE           (PERIPHERAL_BASE + 0x30000)

### Driver Considerations

Driver Considerations

| Consideration | Recommendation |
| --- | --- |
| Polling vs interrupt | Both supported |
| Atomic operations | Not guaranteed across arbitration |
| Cache coherency | APB typically uncached |

## Constraints

### Address Map Constraints

Address Map Constraints

| Constraint | Value | Notes |
| --- | --- | --- |
| Slave region size | 64KB (fixed) | Cannot be changed |
| Maximum slaves | 16 | Generator limit |
| Maximum masters | 16 | Generator limit |
| Address alignment | Byte aligned | No alignment requirement |

### Transaction Constraints

Transaction Constraints

| Constraint | Description |
| --- | --- |
| Outstanding transactions | 1 per master |
| Transaction interleaving | Not supported |
| Burst transactions | Not supported |

## Integration Checklist

### Pre-Integration

- ☐ Verify BASE_ADDR does not conflict with other subsystems

- ☐ Confirm all slaves fit within 64KB regions

- ☐ Validate ADDR_WIDTH is sufficient for address map

- ☐ Check DATA_WIDTH matches system data width

### Post-Integration

- ☐ Verify address decode routing to correct slaves

- ☐ Test all master-to-slave paths

- ☐ Validate arbitration behavior (if multi-master)

- ☐ Check reset behavior

Next: Parameter Configuration

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Parameter Configuration

## Module Parameters

### Core Parameters

Core Parameter Definition

| Parameter | Type | Default | Range | Description |
| --- | --- | --- | --- | --- |
| ADDR_WIDTH | int | 32 | 8-64 | Address bus width in bits |
| DATA_WIDTH | int | 32 | 8, 16, 32, 64 | Data bus width in bits |
| BASE_ADDR | logic[31:0] | 0x10000000 | Any | Base address for slave map |

### Derived Parameters

These are calculated automatically:

Derived Parameters

| Parameter | Derivation | Description |
| --- | --- | --- |
| STRB_WIDTH | DATA_WIDTH / 8 | Byte strobe width |
| NUM_MASTERS | Fixed per variant | Number of master ports |
| NUM_SLAVES | Fixed per variant | Number of slave ports |

## Parameter Usage Examples

### Default Configuration

apbx_xbar_2to4 u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections
);
// Uses defaults: ADDR_WIDTH=32, DATA_WIDTH=32, BASE_ADDR=0x10000000

### Custom Base Address

apbx_xbar_2to4 #(
    .BASE_ADDR(32'h4000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections
);
// Slaves at 0x4000_0000, 0x4001_0000, 0x4002_0000, 0x4003_0000

### 64-bit Data Width

apbx_xbar_2to4 #(
    .DATA_WIDTH(64)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections with 64-bit data buses
);
// STRB_WIDTH automatically becomes 8

### Wide Address

apbx_xbar_2to4 #(
    .ADDR_WIDTH(40),
    .BASE_ADDR(40'h80_0000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),
    // ... port connections with 40-bit address buses
);

## Parameter Validation

### Address Width Check

Ensure ADDR_WIDTH can accommodate the full address range:

Required minimum: log2(BASE_ADDR + NUM_SLAVES * 64KB)

Example: 4 slaves at BASE_ADDR=0x1000_0000 - Max address: 0x1000_0000 + 4*0x10000 = 0x1004_0000 - Required: 29 bits minimum - ADDR_WIDTH=32 is sufficient

### Data Width Constraints

Valid Data Width Configurations

| DATA_WIDTH | STRB_WIDTH | Valid APB Widths |
| --- | --- | --- |
| 8 | 1 | Yes |
| 16 | 2 | Yes |
| 32 | 4 | Yes (most common) |
| 64 | 8 | Yes |

## Custom Generation Parameters

When using the Python generator for custom configurations:

python generate_xbars.py \
    --masters 3 \
    --slaves 6 \
    --base-addr 0x80000000 \
    --output apbx_xbar_3to6.sv

Generator Command Line Options

| Option | Description | Default |
| --- | --- | --- |
| –masters | Number of master ports | Required |
| –slaves | Number of slave ports | Required |
| –base-addr | Base address (hex) | 0x10000000 |
| –output | Output file path | Auto-generated |
| –thin | Generate minimal variant | False |

Next: Verification Strategy

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Verification Strategy

## Pre-Verified Component

The APB Crossbar is pre-verified with comprehensive tests:

Pre-Verification Status

| Variant | Test Transactions | Status |
| --- | --- | --- |
| apbx_xbar_1to1 | 100+ | Pass |
| apbx_xbar_2to1 | 130+ | Pass |
| apbx_xbar_1to4 | 200+ | Pass |
| apbx_xbar_2to4 | 350+ | Pass |

## Test Categories

### Basic Connectivity

Tests for each variant include: - Single read transaction to each slave - Single write transaction to each slave - Read-modify-write sequences - All address regions exercised

### Address Decode Verification

For multi-slave variants: - Verify each slave selected by correct address range - Boundary address testing - Invalid address behavior

### Arbitration Testing

For multi-master variants: - Round-robin fairness verification - Contention handling - Grant persistence validation - No starvation testing

### Stress Testing

High-intensity scenarios: - Back-to-back transactions - Random delays - Mixed read/write patterns - Concurrent multi-master access

## Integration Testing Recommendations

### Level 1: Basic Integration

Level 1 Integration Tests

| Test | Description | Pass Criteria |
| --- | --- | --- |
| Reset test | Apply reset, verify outputs | All outputs at safe values |
| Single transaction | Read from each slave | Correct data returned |
| Write verify | Write and readback | Data matches |

### Level 2: Functional Integration

Level 2 Integration Tests

| Test | Description | Pass Criteria |
| --- | --- | --- |
| All masters | Each master accesses each slave | No errors |
| Arbitration | Concurrent master access | Fair grant rotation |
| Error propagation | Force PSLVERR from slave | Error reaches master |

### Level 3: System Integration

Level 3 Integration Tests

| Test | Description | Pass Criteria |
| --- | --- | --- |
| CPU access | Software reads/writes peripherals | Correct behavior |
| DMA access | DMA transfers via crossbar | Data integrity |
| Mixed traffic | CPU and DMA concurrent | No deadlock, fair access |

## Verification Collateral

### Available Test Infrastructure

Test Infrastructure

| Item | Location | Description |
| --- | --- | --- |
| CocoTB tests | dv/tests/ | Python testbenches |
| Wave configs | dv/tests/GTKW/ | GTKWave configurations |
| Coverage | (implicit) | Functional coverage |

### Running Verification

# Run all crossbar tests
cd projects/components/apbx-xbar/dv/tests/
pytest test_apbx_xbar_*.py -v

# Run specific variant
pytest test_apbx_xbar_2to4.py -v

# Generate waveforms
pytest test_apbx_xbar_2to4.py --vcd=debug.vcd

# View waveforms
gtkwave debug.vcd

## Known Limitations

### Not Tested

Known Limitations

| Scenario | Reason | Recommendation |
| --- | --- | --- |
| Slave timeout | Not implemented | Add external watchdog |
| >16 masters/slaves | Generator limit | Custom modification needed |
| Non-64KB regions | Not supported | Use multiple crossbars |

End of Hardware Architecture Specification

## Related Documentation

- APB Crossbar MAS - Detailed implementation

- PRD.md - Product requirements

- README.md - Quick start guide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# APB Crossbar Micro-Architecture Specification Index

Component: APB Crossbar (MxN Interconnect) Version: 1.0 Date: 2026-01-03 Purpose: Detailed micro-architecture specification for APB Crossbar component

## Document Organization

This specification covers the APB Crossbar component - a parametric MxN interconnect for connecting multiple APB masters to multiple APB slaves with automatic address-based routing and round-robin arbitration.

### Main Documentation

README.md

## Chapter 1: Architecture

chapters/01_architecture.md

## Chapter 2: Address Decode and Arbitration

chapters/02_address_and_arbitration.md

## Chapter 3: RTL Generator

chapters/03_rtl_generator.md

## Quick Navigation

### For New Users

- Start with README.md for overview and quick start

- Read 01_architecture.md to understand the design

- Study 02_address_and_arbitration.md for operational details

- Reference 03_rtl_generator.md if custom configuration needed

### For Integration

- Pre-generated variants: See 01_architecture.md Section “Pre-Generated Variants”

- Custom generation: See 03_rtl_generator.md Section “Quick Start”

- Address mapping: See 02_address_and_arbitration.md Section “Address Decode”

- Arbitration behavior: See 02_address_and_arbitration.md Section “Arbitration”

### Common Questions

All answered in README.md Section “Common Questions”

## Visual Assets

All diagrams referenced in the documentation are available in:

- Source Files:

  - assets/graphviz/*.gv - Graphviz source diagrams

  - assets/wavedrom/*.json - WaveJSON timing diagrams

- Rendered Files:

  - assets/png/*.png - PNG format (for document generation)

### Architecture Diagrams

- APB Crossbar Architecture (2x4 Example)

  - Source: assets/graphviz/apbx_xbar_architecture.gv

  - Rendered: assets/png/apbx_xbar_architecture.png

- Address Decode Flow

  - Source: assets/graphviz/address_decode_flow.gv

  - Rendered: assets/png/address_decode_flow.png

### Timing Diagrams

- Round-Robin Arbitration

  - Source: assets/wavedrom/arbitration_round_robin.json

  - Rendered: assets/wavedrom/arbitration_round_robin.png

## Component Overview

### Key Features

- Parametric MxN Configuration: Any combination of M masters and N slaves (up to 16x16)

- Automatic Address Decode: 64KB per slave, simple offset-based routing

- Round-Robin Arbitration: Per-slave fair arbitration, no master starvation

- Zero-Bubble Throughput: Back-to-back transactions without idle cycles

- Grant Persistence: Hold grant through transaction completion

- RTL Generation: Python-based generator for custom configurations

### Pre-Generated Variants

| Module | M×N | Use Case |
| --- | --- | --- |
| apbx_xbar_1to1 | 1×1 | Passthrough, protocol conversion |
| apbx_xbar_2to1 | 2×1 | Multi-master arbitration |
| apbx_xbar_1to4 | 1×4 | Simple SoC peripheral bus |
| apbx_xbar_2to4 | 2×4 | Typical SoC with CPU+DMA |
| apbx_xbar_thin | 1×1 | Minimal overhead passthrough |

### Design Philosophy

Proven Building Blocks: - Built from production-tested apb4_slave and apb4_master modules - No new protocol logic - pure composition - Each component independently verified

Parametric Generation: - Generator creates any MxN configuration - Pre-generated common variants for fast integration - Custom variants generated on-demand

Clean Separation: - Master-side: APB slaves convert protocol → cmd/rsp - Internal: Arbitration + address decoding - Slave-side: APB masters convert cmd/rsp → protocol

## Related Documentation

### Companion Specifications

- APB Crossbar HAS - Hardware Architecture Specification (high-level)

### Project-Level

- PRD.md: ../../PRD.md - Complete product requirements document

- CLAUDE.md: ../../CLAUDE.md - AI assistant integration guide

- README.md: ../../README.md - Quick start guide

### Test Infrastructure

- Test Directory: ../../dv/tests/ - CocoTB + pytest test suite

- Test Results: All pre-generated variants 100% passing

### RTL

- Core Modules: ../../rtl/apbx_xbar_*.sv - Pre-generated crossbars

- Wrappers: ../../rtl/wrappers/ - Pre-configured wrappers

- Generator: ../../bin/generate_xbars.py - Python generator script

## Version History

Version 1.0 (2025-10-25): - Initial specification release - Complete visual documentation (3 diagrams) - Comprehensive generator documentation - All pre-generated variants verified (100% passing)

Last Updated: 2026-01-03 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Product Requirements Document (PRD)

## APB Crossbar Generator

Version: 1.0 Date: 2025-10-19 Status: Production Ready Owner: RTL Design Sherpa Project Parent Document: /PRD.md

## 1. Executive Summary

The APB Crossbar is a parametric APB interconnect generator that creates configurable MxN crossbar fabrics for connecting multiple APB masters to multiple APB slaves. Built using proven apb4_slave and apb4_master modules, the crossbar provides independent round-robin arbitration per slave and automatic address-based routing.

### 1.1 Quick Stats

- Modules: 5 pre-generated variants + generator for custom sizes

- Max Capacity: Up to 16x16 (configurable)

- Architecture: apb4_slave → arbitration + decode → apb4_master

- Status: Production ready, all tests passing

- Generator: Python-based parametric code generation

### 1.2 Project Goals

- Primary: Provide production-quality APB interconnect for SoC integration

- Secondary: Demonstrate parametric RTL generation best practices

- Tertiary: Support both fixed and dynamically-generated variants

## 2. Key Design Principles

### 2.1 Architecture Philosophy

Proven Building Blocks: - Uses apb4_slave.sv and apb4_master.sv from rtl/amba/apb4 - Each module independently tested and production-proven - Crossbar = composition of proven components

Parametric Generation: - Generator creates any MxN configuration - Pre-generated common variants (1to1, 2to1, 1to4, 2to4, thin) - Custom variants generated on-demand

Clean Separation: - Master-side: APB slaves convert protocol to cmd/rsp - Internal: Arbitration + address decoding - Slave-side: APB masters convert cmd/rsp back to protocol

### 2.2 Design Trade-offs

| Feature | Choice | Rationale |
| --- | --- | --- |
| Arbitration | Round-robin per slave | Fair, simple, predictable |
| Address Map | 64KB per slave | Sufficient for most peripherals |
| Grant Persistence | Hold through response | Zero-bubble throughput |
| Address Decode | Parallel decode | Low latency, simple logic |

## 3. Architecture Overview

### 3.1 Top-Level Block Diagram

APB Crossbar (M masters × N slaves)
┌─────────────────────────────────────────────────────────────────┐
│                                                                  │
│  Master-Side (APB slaves)                                        │
│  ┌──────────────┐  ┌──────────────┐       ┌──────────────┐     │
│  │ apb4_slave[0] │  │ apb4_slave[1] │  ...  │ apb4_slave[M-1]│     │
│  │ (Master 0)   │  │ (Master 1)   │       │ (Master M-1)  │     │
│  └──────┬───────┘  └──────┬───────┘       └──────┬────────┘     │
│         │                  │                      │              │
│         └──────────────────┴──────────────────────┘              │
│                            │                                     │
│                     cmd/rsp buses                                │
│                            │                                     │
│         ┌──────────────────┴──────────────────────┐              │
│         │                                          │              │
│  ┌──────▼──────┐  ┌──────────────┐  ┌────────────▼──┐          │
│  │ Address     │  │ Arbitration  │  │ Response       │          │
│  │ Decode      │  │ per Slave    │  │ Routing        │          │
│  │ (parallel)  │  │ (round-robin)│  │ (registered)   │          │
│  └──────┬──────┘  └──────┬───────┘  └────────┬───────┘          │
│         │                 │                   │                  │
│         └─────────────────┴───────────────────┘                  │
│                            │                                     │
│                     cmd/rsp buses                                │
│                            │                                     │
│         ┌──────────────────┴──────────────────────┐              │
│         │                  │                      │              │
│  ┌──────▼───────┐  ┌──────▼───────┐       ┌──────▼────────┐    │
│  │ apb4_master[0]│  │ apb4_master[1]│  ...  │ apb4_master[N-1]│    │
│  │ (Slave 0)    │  │ (Slave 1)    │       │ (Slave N-1)    │    │
│  └──────────────┘  └──────────────┘       └───────────────┘    │
│                                                                  │
│  Slave-Side (APB masters)                                        │
└──────────────────────────────────────────────────────────────────┘

Figure: APB Crossbar top-level architecture showing 2 masters connected to 4 slaves. Source: docs/apbx_xbar_mas/assets/graphviz/apbx_xbar_architecture.gv | SVG

### 3.2 Address Mapping

BASE_ADDR + 0x0000_0000 → 0x0000_FFFF : Slave 0 (64KB)
BASE_ADDR + 0x0001_0000 → 0x0001_FFFF : Slave 1 (64KB)
BASE_ADDR + 0x0002_0000 → 0x0002_FFFF : Slave 2 (64KB)
BASE_ADDR + 0x0003_0000 → 0x0003_FFFF : Slave 3 (64KB)
...
BASE_ADDR + 0x000F_0000 → 0x000F_FFFF : Slave 15 (64KB)

Default BASE_ADDR: 0x1000_0000

Address Decode Example:

The crossbar extracts the slave index from the upper bits of the address offset:

slave_index = (address - BASE_ADDR) >> 16  // Divide by 64KB (0x10000)

Figure: Address decode flow showing how address 0x10023456 routes to Slave 2. Source: docs/apbx_xbar_mas/assets/graphviz/address_decode_flow.gv | SVG

### 3.3 Arbitration Strategy

Per-Slave Round-Robin: - Each slave has independent arbiter - Master priority rotates after each grant - Grant held from command acceptance through response completion - No master can starve another master

Example:

Slave 0 accessed by M0, M1, M0 → Next grant goes to M1
Slave 1 accessed by M1, M1, M0 → Next grant goes to M0

Figure: Round-robin arbitration timing showing 2 masters competing for Slave 0. Master priority rotates after each grant to ensure fair access. Source: docs/apbx_xbar_mas/assets/wavedrom/arbitration_round_robin.json

## 4. Available Variants

### 4.1 Pre-Generated Modules

| Module | Masters | Slaves | Use Case | File Size |
| --- | --- | --- | --- | --- |
| apbx_xbar_1to1 | 1 | 1 | Protocol conversion, testing | ~200 LOC |
| apbx_xbar_2to1 | 2 | 1 | Multi-master arbitration | ~400 LOC |
| apbx_xbar_1to4 | 1 | 4 | Address decode, simple SoC | ~500 LOC |
| apbx_xbar_2to4 | 2 | 4 | Full crossbar, typical SoC | ~1000 LOC |
| apbx_xbar_thin | 1 | 1 | Minimal passthrough | ~150 LOC |

### 4.2 Wrapper Modules

Pre-configured wrappers for common topologies:

| Wrapper | Configuration | Purpose |
| --- | --- | --- |
| apbx_xbar_1to1_wrap | 1×1 | Simple connection |
| apbx_xbar_2to1_wrap | 2×1 | Dual-master arbitration |
| apbx_xbar_1to4_wrap | 1×4 | Single-master decode |
| apbx_xbar_2to4_wrap | 2×4 | Full SoC crossbar |
| apbx_xbar_wrap_m10_s10 | 10×10 | Large crossbar |
| apbx_xbar_thin_wrap_m10_s10 | 10×10 | Minimal version |

## 5. Functional Requirements

### FR-1: Arbitrary MxN Configuration

Priority: P0 (Critical) Status: ✓ Implemented and verified

Description: Generate crossbar for any M masters and N slaves (up to 16x16)

Verification: Generator creates valid RTL for all tested configurations

### FR-2: Round-Robin Arbitration

Priority: P0 (Critical) Status: ✓ Implemented and verified

Description: Fair per-slave arbitration with rotating master priority

Verification: Arbitration stress tests (130+ transactions for 2to1)

### FR-3: Address-Based Routing

Priority: P0 (Critical) Status: ✓ Implemented and verified

Description: Automatic slave selection based on address ranges

Verification: Address decode validation (200+ transactions for 1to4)

### FR-4: Zero-Bubble Throughput

Priority: P1 (High) Status: ✓ Implemented and verified

Description: Back-to-back transactions supported without idle cycles

Verification: Performance tests show consecutive transactions

### FR-5: Configurable Address Map

Priority: P1 (High) Status: ✓ Implemented and verified

Description: BASE_ADDR parameter sets base of address map

Verification: Tests with multiple BASE_ADDR values

## 6. Interface Specifications

### 6.1 Master-Side Interface (APB Slave)

Per-master APB slave interface:

input  logic                    m<i>_apb_PSEL
input  logic                    m<i>_apb_PENABLE
input  logic [ADDR_WIDTH-1:0]   m<i>_apb_PADDR
input  logic                    m<i>_apb_PWRITE
input  logic [DATA_WIDTH-1:0]   m<i>_apb_PWDATA
input  logic [STRB_WIDTH-1:0]   m<i>_apb_PSTRB
input  logic [2:0]              m<i>_apb_PPROT
output logic [DATA_WIDTH-1:0]   m<i>_apb_PRDATA
output logic                    m<i>_apb_PSLVERR
output logic                    m<i>_apb_PREADY

### 6.2 Slave-Side Interface (APB Master)

Per-slave APB master interface:

output logic                    s<j>_apb_PSEL
output logic                    s<j>_apb_PENABLE
output logic [ADDR_WIDTH-1:0]   s<j>_apb_PADDR
output logic                    s<j>_apb_PWRITE
output logic [DATA_WIDTH-1:0]   s<j>_apb_PWDATA
output logic [STRB_WIDTH-1:0]   s<j>_apb_PSTRB
output logic [2:0]              s<j>_apb_PPROT
input  logic [DATA_WIDTH-1:0]   s<j>_apb_PRDATA
input  logic                    s<j>_apb_PSLVERR
input  logic                    s<j>_apb_PREADY

## 7. Parameter Configuration

| Parameter | Type | Default | Range | Description |
| --- | --- | --- | --- | --- |
| ADDR_WIDTH | int | 32 | 1-64 | Address bus width |
| DATA_WIDTH | int | 32 | 8,16,32,64 | Data bus width |
| STRB_WIDTH | int | DATA_WIDTH/8 | - | Strobe width (auto-calc) |
| BASE_ADDR | logic[31:0] | 0x10000000 | Any | Base address for slave map |

## 8. Generator Usage

### 8.1 Pre-Generated Variants

Use existing modules directly:

# Available in projects/components/apbx-xbar/rtl/
apbx_xbar_1to1.sv
apbx_xbar_2to1.sv
apbx_xbar_1to4.sv
apbx_xbar_2to4.sv
apbx_xbar_thin.sv

### 8.2 Custom Generation

Generate custom MxN crossbar:

cd projects/components/apbx-xbar/bin/
python generate_xbars.py --masters 3 --slaves 6
python generate_xbars.py --masters 4 --slaves 8 --base-addr 0x80000000

### 8.3 Generator Options

python generate_xbars.py --help

Options:
  --masters M       Number of masters (1-16)
  --slaves N        Number of slaves (1-16)
  --base-addr ADDR  Base address (default: 0x10000000)
  --output FILE     Output file path
  --thin            Generate thin variant (minimal logic)

## 9. Performance Characteristics

### 9.1 Latency

| Path | Latency | Notes |
| --- | --- | --- |
| Command path | 1 cycle | APB slave → decode → APB master |
| Response path | 1 cycle | APB master → routing → APB slave |
| Total | 2 cycles min | APB protocol overhead |

### 9.2 Throughput

- Back-to-back transactions: Supported

- Zero-bubble overhead: Yes (with grant persistence)

- Maximum rate: 1 transaction per 2 APB cycles per master

### 9.3 Resource Utilization (Estimated)

| Configuration | LUTs | FFs | Notes |
| --- | --- | --- | --- |
| 1to1 | ~50 | ~20 | Passthrough |
| 2to1 | ~150 | ~80 | Arbitration |
| 1to4 | ~200 | ~100 | Address decode |
| 2to4 | ~400 | ~200 | Full crossbar |
| 10to10 | ~5K | ~2K | Large crossbar |

## 10. Verification Status

### 10.1 Test Coverage

| Test | Transactions | Status | Coverage |
| --- | --- | --- | --- |
| test_apbx_xbar_1to1 | 100+ | ✓ Pass | Basic connectivity |
| test_apbx_xbar_2to1 | 130+ | ✓ Pass | Arbitration stress |
| test_apbx_xbar_1to4 | 200+ | ✓ Pass | Address decode |
| test_apbx_xbar_2to4 | 350+ | ✓ Pass | Full crossbar stress |

Overall: 100% passing, >750 total transactions tested

### 10.2 Test Methodology

Test Structure: - CocoTB framework - Random transaction generation - Variable delay profiles - Address range validation - Arbitration fairness checks

Verification Points: - Correct address routing - Round-robin arbitration - Response integrity - Back-to-back transactions - Error propagation

## 11. Integration Guide

### 11.1 Simple Integration (1to4)

apbx_xbar_1to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master interface
    .m0_apb_PSEL(cpu_psel),
    .m0_apb_PENABLE(cpu_penable),
    .m0_apb_PADDR(cpu_paddr),
    .m0_apb_PWRITE(cpu_pwrite),
    .m0_apb_PWDATA(cpu_pwdata),
    .m0_apb_PSTRB(cpu_pstrb),
    .m0_apb_PPROT(cpu_pprot),
    .m0_apb_PRDATA(cpu_prdata),
    .m0_apb_PSLVERR(cpu_pslverr),
    .m0_apb_PREADY(cpu_pready),

    // Slave 0: UART (0x1000_0000 - 0x1000_FFFF)
    .s0_apb_PSEL(uart_psel),
    .s0_apb_PENABLE(uart_penable),
    // ... (similar connections)

    // Slave 1: GPIO (0x1001_0000 - 0x1001_FFFF)
    .s1_apb_PSEL(gpio_psel),
    // ... (similar connections)

    // Slave 2: Timer (0x1002_0000 - 0x1002_FFFF)
    // Slave 3: SPI (0x1003_0000 - 0x1003_FFFF)
);

### 11.2 Multi-Master Integration (2to4)

apbx_xbar_2to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master 0: CPU
    .m0_apb_PSEL(cpu_psel),
    // ... (full interface)

    // Master 1: DMA
    .m1_apb_PSEL(dma_psel),
    // ... (full interface)

    // Slaves 0-3: Peripherals
    // ... (slave connections)
);

## 12. Design Constraints

### 12.1 Limitations

| Constraint | Value | Rationale |
| --- | --- | --- |
| Max masters | 16 | Generator limit (configurable) |
| Max slaves | 16 | Generator limit (configurable) |
| Slave region size | 64KB | Fixed per slave |
| No slave disable | N/A | All slaves always active |
| No timeout | N/A | Assumes slaves always respond |

### 12.2 Assumptions

- Slave responses: All slaves respond eventually (no timeout handling)

- Address ranges: Slaves occupy 64KB regions starting at BASE_ADDR

- APB compliance: All masters and slaves follow APB protocol

- Single clock domain: All signals synchronous to pclk

## 13. Future Enhancements

### 13.1 Potential Features

- Configurable slave region sizes: Not just 64KB

- Slave enable/disable: Dynamic slave activation

- Timeout detection: Watchdog for hung slaves

- Transaction ordering: Optional in-order completion

- Priority arbitration: Weighted instead of round-robin

### 13.2 Generator Improvements

- HDL output formats: Support Verilog, VHDL, Chisel

- Automated testing: Generate tests alongside RTL

- Documentation generation: Auto-generate integration guides

- Synthesis scripts: Generate vendor-specific scripts

## 14. References

### 14.1 Internal Documentation

- README.md - Quick start guide

- CLAUDE.md - AI assistant guidelines

- bin/generate_xbars.py - Generator script

- dv/tests/ - Test implementations

### 14.2 External Standards

- AMBA APB Protocol v2.0 - ARM IHI 0024C

- APB4 Extensions - AMBA 5 specification

### 14.3 Related Components

- rtl/amba/apb4/apb4_slave.sv - Master-side building block

- rtl/amba/apb4/apb4_master.sv - Slave-side building block

- projects/components/apbx-xbar/bin/apbx_xbar_generator.py - Main generator

Document Version: 1.0 Last Review: 2025-10-19 Next Review: 2026-01-01 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# Claude Code Guide: APB Crossbar Component

Version: 1.0 Last Updated: 2025-10-19 Purpose: AI-specific guidance for working with APB Crossbar component

## Quick Context

What: APB Crossbar Generator - Parametric APB interconnect for connecting M masters to N slaves Status: ✓ Production Ready (all tests passing) Your Role: Help users generate, integrate, and customize APB crossbars

📖 Complete Specification: projects/components/apbx-xbar/PRD.md ← Always reference this for technical details

## Critical Rules for This Component

### Rule #0: This is a GENERATOR, Not Just Modules

IMPORTANT: APB Crossbar is both: 1. Pre-generated modules (1to1, 2to1, 1to4, 2to4, thin) in rtl/ 2. Python generator (bin/generate_xbars.py) for custom configurations

When users ask for crossbar: - ✓ Check if pre-generated variant exists first - ✓ Only suggest generation if custom size needed

Decision Tree:

User needs crossbar?
├─ 1x1, 2x1, 1x4, 2x4? → Use pre-generated module
├─ Thin/minimal? → Use apbx_xbar_thin
└─ Custom MxN? → Run generator script

### Rule #1: Address Map is Fixed Per-Slave

Each slave occupies 64KB region:

Slave 0: BASE_ADDR + 0x0000_0000 → 0x0000_FFFF
Slave 1: BASE_ADDR + 0x0001_0000 → 0x0001_FFFF
Slave 2: BASE_ADDR + 0x0002_0000 → 0x0002_FFFF
...

Users CANNOT change per-slave size (current limitation)

If user asks for different sizes:

✗ WRONG: "Let me modify the generator to support custom sizes per slave"

✓ CORRECT: "Current design uses fixed 64KB per slave. You can:
1. Use BASE_ADDR parameter to shift entire map
2. For custom sizes, modify generator's addr_offset calculation
3. Or use multiple crossbars with different BASE_ADDR values"

### Rule #2: Know the Pre-Generated Variants

Available in rtl/ directory:

| Module | M×N | Use Case | When to Recommend |
| --- | --- | --- | --- |
| apbx_xbar_1to1 | 1×1 | Passthrough | Protocol conversion, testing |
| apbx_xbar_2to1 | 2×1 | Arbitration | Multi-master to single peripheral |
| apbx_xbar_1to4 | 1×4 | Decode | Single CPU to multiple peripherals |
| apbx_xbar_2to4 | 2×4 | Full crossbar | CPU + DMA to peripherals |
| apbx_xbar_thin | 1×1 | Minimal | Low-overhead passthrough |

Always suggest pre-generated first!

### Rule #3: Generator Syntax

Generate custom crossbar:

cd projects/components/apbx-xbar/bin/
python generate_xbars.py --masters 3 --slaves 6

Options:

--masters M         Number of masters (1-16)
--slaves N          Number of slaves (1-16)
--base-addr ADDR    Base address (default: 0x10000000)
--output FILE       Output file path
--thin              Generate thin variant (minimal logic)

Example:

# Generate 3x8 crossbar with custom base
python generate_xbars.py --masters 3 --slaves 8 --base-addr 0x80000000

# Generate thin 5x5 variant
python generate_xbars.py --masters 5 --slaves 5 --thin

## Architecture Quick Reference

### Module Organization

projects/components/apbx-xbar/
├── rtl/
│   ├── apbx_xbar_1to1.sv        Core crossbar modules
│   ├── apbx_xbar_2to1.sv
│   ├── apbx_xbar_1to4.sv
│   ├── apbx_xbar_2to4.sv
│   ├── apbx_xbar_thin.sv
│   └── wrappers/               Pre-configured wrappers
│       ├── apbx_xbar_1to1_wrap.sv
│       ├── apbx_xbar_2to1_wrap.sv
│       ├── apbx_xbar_1to4_wrap.sv
│       ├── apbx_xbar_2to4_wrap.sv
│       ├── apbx_xbar_wrap.sv
│       ├── apbx_xbar_wrap_m10_s10.sv
│       └── apbx_xbar_thin_wrap_m10_s10.sv
├── bin/
│   └── generate_xbars.py       Generator script
├── dv/
│   └── tests/
│       ├── test_apbx_xbar_1to1.py
│       ├── test_apbx_xbar_2to1.py
│       ├── test_apbx_xbar_1to4.py
│       ├── test_apbx_xbar_2to4.py
│       └── GTKW/                Waveform configs
├── PRD.md                       Complete specification
├── CLAUDE.md                    This file
└── README.md                    Quick start guide

### Block Diagram (2×4 Example)

Master 0      Master 1
               │             │
               ▼             ▼
         ┌─────────────────────┐
         │  APB Slaves (M-side) │
         └──────────┬────────────┘
                    │
         ┌──────────┴────────────┐
         │ Address Decode        │
         │ + Arbitration         │
         │ + Response Routing    │
         └──────────┬────────────┘
                    │
         ┌──────────┴────────────┐
         │  APB Masters (S-side) │
         └──────┬────────┬───────┘
                │        │   (more)
           Slave 0   Slave 1 ... Slave 3

## Common User Questions and Responses

### Q: “How do I connect a CPU to 4 peripherals?”

A: Use the pre-generated 1to4 crossbar:

apbx_xbar_1to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)  // Start of peripheral space
) u_peripheral_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // CPU connection (master 0)
    .m0_apb_PSEL(cpu_psel),
    .m0_apb_PENABLE(cpu_penable),
    .m0_apb_PADDR(cpu_paddr),
    .m0_apb_PWRITE(cpu_pwrite),
    .m0_apb_PWDATA(cpu_pwdata),
    .m0_apb_PSTRB(cpu_pstrb),
    .m0_apb_PPROT(cpu_pprot),
    .m0_apb_PRDATA(cpu_prdata),
    .m0_apb_PSLVERR(cpu_pslverr),
    .m0_apb_PREADY(cpu_pready),

    // Peripheral 0: UART @ 0x1000_0000
    .s0_apb_PSEL(uart_psel),
    .s0_apb_PENABLE(uart_penable),
    // ... (full interface)

    // Peripheral 1: GPIO @ 0x1001_0000
    // Peripheral 2: Timer @ 0x1002_0000
    // Peripheral 3: SPI @ 0x1003_0000
);

Address map: - UART: 0x1000_0000 - 0x1000_FFFF - GPIO: 0x1001_0000 - 0x1001_FFFF - Timer: 0x1002_0000 - 0x1002_FFFF - SPI: 0x1003_0000 - 0x1003_FFFF

📖 See: PRD.md Section 11.1

### Q: “I need CPU and DMA to access peripherals. Which crossbar?”

A: Use the 2to4 crossbar:

apbx_xbar_2to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_soc_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master 0: CPU
    .m0_apb_PSEL(cpu_psel),
    // ... (full interface)

    // Master 1: DMA
    .m1_apb_PSEL(dma_psel),
    // ... (full interface)

    // Slaves 0-3: Peripherals
    // ... (slave connections)
);

Key feature: Round-robin arbitration per slave ensures fair access between CPU and DMA.

📖 See: PRD.md Section 11.2

### Q: “What if I need 3 masters and 8 slaves?”

A: Generate custom crossbar:

cd projects/components/apbx-xbar/bin/
python generate_xbars.py --masters 3 --slaves 8   # writes apbx_xbar_3to8.sv next to the script

# Check generated file, then move it into rtl/
ls -lh apbx_xbar_3to8.sv
mv apbx_xbar_3to8.sv ../rtl/

Then instantiate like any other crossbar:

apbx_xbar_3to8 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_custom_xbar (
    // ... connections for 3 masters and 8 slaves
);

📖 See: PRD.md Section 8

### Q: “How does arbitration work?”

A: Per-slave round-robin:

Each slave has its own arbiter that rotates master priority:

Example with 2 masters accessing Slave 0:

Transaction 1: M0 requests → M0 granted
Transaction 2: M0 and M1 request → M1 granted (rotated)
Transaction 3: M1 requests → M1 granted
Transaction 4: M0 and M1 request → M0 granted (rotated)

Key points: - Independent per slave: Each slave arbitrates independently - Fair: No master can starve another - Grant persistence: Once granted, master holds slave until response completes - Zero-bubble: Back-to-back transactions supported

📖 See: PRD.md Section 3.3

### Q: “Can I change the address map?”

A: BASE_ADDR parameter shifts entire map, but per-slave size is fixed:

// Default: 0x1000_0000
apbx_xbar_1to4 #(.BASE_ADDR(32'h1000_0000)) u_xbar1 (...);
// Slaves at: 0x1000_0000, 0x1001_0000, 0x1002_0000, 0x1003_0000

// Shifted: 0x8000_0000
apbx_xbar_1to4 #(.BASE_ADDR(32'h8000_0000)) u_xbar2 (...);
// Slaves at: 0x8000_0000, 0x8001_0000, 0x8002_0000, 0x8003_0000

Limitation: Each slave always occupies 64KB (0x10000 bytes)

Workaround for different sizes: 1. Use multiple crossbars with different BASE_ADDR 2. Modify generator’s addr_offset calculation 3. Use address masking in slaves

📖 See: PRD.md Section 3.2

### Q: “How do I run tests?”

A: Use pytest:

cd projects/components/apbx-xbar/dv/tests/

# Run specific test
pytest test_apbx_xbar_1to4.py -v

# Run all tests
pytest test_apbx_xbar_*.py -v

# Run with waveforms
pytest test_apbx_xbar_2to4.py --vcd=waves.vcd
gtkwave waves.vcd

Test coverage: - test_apbx_xbar_1to1: 100+ transactions (passthrough) - test_apbx_xbar_2to1: 130+ transactions (arbitration) - test_apbx_xbar_1to4: 200+ transactions (address decode) - test_apbx_xbar_2to4: 350+ transactions (full stress)

All tests passing ✓

📖 See: PRD.md Section 10

### Q: “What’s the thin variant for?”

A: Minimal overhead passthrough:

apbx_xbar_thin #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32)
) u_thin_xbar (
    // ... 1×1 connection
);

Use cases: - Protocol conversion - Timing boundary - Clock domain crossing preparation - Testing/debugging

Difference from apbx_xbar_1to1: - Fewer internal registers - Lower latency - Smaller area (~30% reduction) - No arbitration overhead

📖 See: README.md and PRD.md Section 4

## Integration Patterns

### Pattern 1: Simple Peripheral Interconnect

// CPU to 4 peripherals
apbx_xbar_1to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h1000_0000)
) u_periph_xbar (
    .pclk(sys_clk),
    .presetn(sys_rst_n),

    // CPU side
    .m0_apb_PSEL(cpu_apb_psel),
    .m0_apb_PENABLE(cpu_apb_penable),
    .m0_apb_PADDR(cpu_apb_paddr),
    .m0_apb_PWRITE(cpu_apb_pwrite),
    .m0_apb_PWDATA(cpu_apb_pwdata),
    .m0_apb_PSTRB(cpu_apb_pstrb),
    .m0_apb_PPROT(cpu_apb_pprot),
    .m0_apb_PRDATA(cpu_apb_prdata),
    .m0_apb_PSLVERR(cpu_apb_pslverr),
    .m0_apb_PREADY(cpu_apb_pready),

    // Peripherals
    .s0_apb_PSEL(uart_psel), /* ... full interface ... */
    .s1_apb_PSEL(gpio_psel), /* ... */
    .s2_apb_PSEL(timer_psel), /* ... */
    .s3_apb_PSEL(spi_psel) /* ... */
);

### Pattern 2: Multi-Master System

// CPU + DMA to peripherals
apbx_xbar_2to4 #(
    .ADDR_WIDTH(32),
    .DATA_WIDTH(32),
    .BASE_ADDR(32'h4000_0000)
) u_soc_xbar (
    .pclk(apb_clk),
    .presetn(apb_rst_n),

    // Master 0: CPU
    .m0_apb_PSEL(cpu_psel),
    /* ... full CPU interface ... */

    // Master 1: DMA Controller
    .m1_apb_PSEL(dma_psel),
    /* ... full DMA interface ... */

    // Slave 0-3: Memory-mapped peripherals
    .s0_apb_PSEL(mem_ctrl_psel), /* ... */
    .s1_apb_PSEL(uart_psel), /* ... */
    .s2_apb_PSEL(i2c_psel), /* ... */
    .s3_apb_PSEL(adc_psel) /* ... */
);

### Pattern 3: Hierarchical Interconnect

// Top-level crossbar
apbx_xbar_1to4 u_top_xbar (
    .m0_apb_* (cpu_apb_*),
    .s0_apb_* (periph_bus0_*),  // To sub-crossbar 0
    .s1_apb_* (periph_bus1_*),  // To sub-crossbar 1
    .s2_apb_* (mem_ctrl_*),     // Direct to memory controller
    .s3_apb_* (dma_ctrl_*)      // Direct to DMA
);

// Sub-crossbar 0 for low-speed peripherals
apbx_xbar_1to4 u_periph_xbar0 (
    .m0_apb_* (periph_bus0_*),
    .s0_apb_* (uart0_*),
    .s1_apb_* (gpio0_*),
    .s2_apb_* (i2c0_*),
    .s3_apb_* (spi0_*)
);

// Sub-crossbar 1 for high-speed peripherals
apbx_xbar_1to4 u_periph_xbar1 (
    .m0_apb_* (periph_bus1_*),
    .s0_apb_* (uart1_*),
    .s1_apb_* (timer_*),
    .s2_apb_* (pwm_*),
    .s3_apb_* (adc_*)
);

## Anti-Patterns to Catch

### ✗ Anti-Pattern 1: Generating When Pre-Generated Exists

✗ WRONG:
User: "I need a 2×4 crossbar"
You: "Let me generate that for you..."
python generate_xbars.py --masters 2 --slaves 4

✓ CORRECTED:
"Use the pre-generated apbx_xbar_2to4.sv in the rtl/ directory.
No generation needed!"

### ✗ Anti-Pattern 2: Assuming Custom Per-Slave Sizes

✗ WRONG:
User: "Can I make slave 0 256KB and slave 1 4KB?"
You: "Sure, let me modify the parameters..."

✓ CORRECTED:
"Current design uses fixed 64KB per slave. For custom sizes:
1. Modify generator's addr_offset calculation
2. Or use multiple crossbars with different BASE_ADDR
3. Or implement address masking in slaves"

### ✗ Anti-Pattern 3: Not Mentioning Address Map

✗ WRONG:
User: "How do I integrate the crossbar?"
You: *Shows port connections only*

✓ CORRECTED:
"Here's the integration with address map:
apbx_xbar_1to4 #(.BASE_ADDR(32'h1000_0000)) u_xbar (...);

Address map:
- Slave 0: 0x1000_0000 - 0x1000_FFFF
- Slave 1: 0x1001_0000 - 0x1001_FFFF
- Slave 2: 0x1002_0000 - 0x1002_FFFF
- Slave 3: 0x1003_0000 - 0x1003_FFFF"

### ✗ Anti-Pattern 4: Forgetting About Wrappers

✗ WRONG:
User: "I need a quick 10×10 crossbar"
You: "Run the generator with --masters 10 --slaves 10"

✓ CORRECTED:
"We have pre-configured wrappers in rtl/wrappers/:
- apbx_xbar_wrap_m10_s10.sv (full version)
- apbx_xbar_thin_wrap_m10_s10.sv (thin version)

Use those for faster integration!"

## Debugging Workflow

### Issue: Address Not Routing Correctly

Check in order: 1. ✓ BASE_ADDR parameter set correctly? 2. ✓ Address within slave’s 64KB region? 3. ✓ PSEL signal asserted by master? 4. ✓ All APB signals properly connected?

Calculate expected slave:

slave_index = (address - BASE_ADDR) >> 16  // Divide by 64KB

Debug commands:

pytest dv/tests/test_apbx_xbar_1to4.py --vcd=debug.vcd -v
gtkwave debug.vcd  # Check address decode logic

### Issue: Arbitration Not Fair

Symptoms: - One master dominates - Other masters starved

Check: 1. ✓ Arbiters instantiated per slave? 2. ✓ Round-robin logic correct? 3. ✓ Grant persistence working?

Verify with tests:

pytest dv/tests/test_apbx_xbar_2to1.py -v  # Arbitration stress test

### Issue: Back-to-Back Transactions Stalling

Check: 1. ✓ Grant persistence enabled? 2. ✓ Slaves responding with PREADY? 3. ✓ No unintended pipeline bubbles?

View waveforms:

pytest dv/tests/test_apbx_xbar_2to4.py --vcd=perf.vcd
gtkwave perf.vcd  # Check for idle cycles

## Quick Commands

# List available crossbars
ls projects/components/apbx-xbar/rtl/*.sv

# Generate custom crossbar
cd projects/components/apbx-xbar/bin/
python generate_xbars.py --masters 3 --slaves 6

# Run all tests
cd projects/components/apbx-xbar/dv/tests/
pytest test_apbx_xbar_*.py -v

# Run specific test with waveforms
pytest test_apbx_xbar_2to4.py --vcd=debug.vcd -v

# View waveforms
gtkwave debug.vcd

# Check documentation
cat projects/components/apbx-xbar/PRD.md
cat projects/components/apbx-xbar/README.md

## Remember

- 🔍 Check pre-generated first - Don’t generate unnecessarily

- 📍 Address map matters - Always mention BASE_ADDR + 64KB regions

- ⚖️ Fair arbitration - Round-robin per slave

- 🔗 Complete connections - All APB signals must be wired

- ✓ Tests available - 100% passing, comprehensive coverage

- 📚 Wrappers exist - Check rtl/wrappers/ for common configs

- 🎯 Generator limits - Up to 16×16 (configurable)

Version: 1.0 Last Updated: 2025-10-19 Maintained By: RTL Design Sherpa Project