RTL Design Sherpa

APB GPIO Micro-Architecture Specification 1.0

January 4, 2026

Table of Contents

1 Gpio Mas Index	18

2 APB GPIO - Overview	18

2.1 Introduction	18

2.2 Features	18

2.2.1 Core Functionality	18

2.2.2 Interrupt Capabilities	18

2.2.3 Atomic Operations	18

2.2.4 Clock Domain Crossing	19

2.3 Applications	19

2.3.1 Typical Use Cases	19

2.3.2 System Integration	19

2.4 Block Diagram	19

2.4.1 Figure 1.1: APB GPIO Block Diagram	19

2.5 Key Specifications	20

2.6 Register Summary	20

3 APB GPIO - Architecture	21

3.1 High-Level Block Diagram	21

3.1.1 Figure 1.2: APB GPIO Top-Level Architecture	21

3.2 Module Hierarchy	22

3.2.1 Figure 1.3: APB GPIO Module Hierarchy	22

3.3 Data Flow	23

3.3.1 Write Transaction Flow	23

3.3.2 Figure 1.4: Write Transaction Flow	23

3.3.3 Read Transaction Flow	24

3.3.4 Figure 1.5: Read Transaction Flow	24

3.3.5 Interrupt Flow	24

3.3.6 Figure 1.6: Interrupt Flow	24

3.4 Clock Domains	25

3.4.1 Synchronous Mode (CDC_ENABLE = 0)	25

3.4.2 Figure 1.7: Synchronous Mode Clock Domains	25

3.4.3 Asynchronous Mode (CDC_ENABLE = 1)	26

3.4.4 Figure 1.8: Asynchronous Mode Clock Domains	26

3.5 Parameterization	28

3.6 Resource Estimates	28

4 APB GPIO - Clocks and Reset	29

4.1 Clock Signals	29

4.1.1 pclk (APB Clock)	29

4.1.2 gpio_clk (GPIO Clock)	29

4.2 Reset Signals	29

4.2.1 presetn (APB Reset)	29

4.2.2 gpio_rstn (GPIO Reset)	29

4.3 Reset Behavior	29

4.3.1 Register Reset Values	29

4.3.2 Output Pin Behavior During Reset	30

4.4 Clock Domain Crossing	30

4.4.1 When CDC_ENABLE = 0	30

4.4.2 When CDC_ENABLE = 1	30

4.5 Input Synchronization	30

4.6 Timing Constraints	31

4.6.1 Synchronous Mode	31

4.6.2 Asynchronous Mode	31

5 APB GPIO - Acronyms and Terminology	31

5.1 Protocol Acronyms	31

5.2 Signal Acronyms	31

5.3 GPIO-Specific Terms	32

5.4 Interrupt Terms	32

5.5 Register Terms	32

6 APB GPIO - References	33

6.1 Internal Documentation	33

6.1.1 RTL Source Files	33

6.1.2 Related Specifications	33

6.2 External References	33

6.2.1 ARM AMBA Specifications	33

6.2.2 Industry Standards	33

6.3 Design References	34

6.3.1 Clock Domain Crossing	34

6.3.2 Interrupt Handling	34

7 APB GPIO - Block Descriptions Overview	35

7.1 Module Hierarchy	35

7.1.1 Figure 2.1: APB GPIO Module Hierarchy	35

7.2 Block Summary	35

7.3 Detailed Block Descriptions	35

7.3.1 1. APB Interface	35

7.3.2 2. Register File	35

7.3.3 3. GPIO Core	35

7.3.4 4. Interrupt Controller	36

7.3.5 5. CDC Logic	36

8 APB GPIO - APB Interface Block	36

8.1 Overview	36

8.2 Block Diagram	36

8.3 Interface Signals	36

8.3.1 APB Slave Interface	36

8.4 Operation	37

8.4.1 Read Transaction	37

8.4.2 Write Transaction	37

8.5 Timing Diagram	37

8.6 Implementation Notes	38

9 APB GPIO - Register File Block	38

9.1 Overview	38

9.2 Block Diagram	38

9.3 Generated Interface (HWIF)	38

9.3.1 Hardware-to-Software (hw2reg)	38

9.3.2 Software-to-Hardware (reg2hw)	38

9.4 Register Access	39

9.4.1 Byte Enable Support	39

9.4.2 Access Types	39

9.5 Implementation Notes	39

10 APB GPIO - GPIO Core Block	40

10.1 Overview	40

10.2 Block Diagram	40

10.3 Input Path	40

10.3.1 Synchronization	40

10.3.2 Input Register	40

10.4 Output Path	40

10.4.1 Output Register	40

10.4.2 Output Enable	41

10.4.3 External Signals	41

10.5 Direction Control	41

10.5.1 Per-Pin Configuration	41

10.5.2 Read-Back Behavior	41

10.6 Implementation Notes	41

11 APB GPIO - Interrupt Controller Block	42

11.1 Overview	42

11.2 Block Diagram	42

11.2.1 Figure 2.5: Interrupt Controller Block Diagram	42

11.3 Interrupt Modes	42

11.3.1 Edge-Triggered Mode	42

11.3.2 Level-Sensitive Mode	43

11.4 Edge Detection Logic	44

11.4.1 Figure 2.6: Edge Detection Logic	44

11.5 Interrupt Status	45

11.5.1 Status Register	45

11.5.2 Interrupt Enable	45

11.6 Aggregate IRQ Output	45

11.7 Interrupt Handling Flow	45

11.8 Implementation Notes	45

12 APB GPIO - CDC Logic Block	46

12.1 Overview	46

12.2 Block Diagram	46

12.2.1 Figure 2.7: CDC Logic Block Diagram	46

12.3 Configuration	46

12.3.1 Parameter: CDC_ENABLE	46

12.4 Clock Domains	47

12.4.1 When CDC_ENABLE = 0	47

12.4.2 Figure 2.8: Single Clock Domain (CDC Disabled)	47

12.4.3 When CDC_ENABLE = 1	47

12.4.4 Figure 2.9: Dual Clock Domain (CDC Enabled)	47

12.5 CDC Implementation	47

12.5.1 APB to GPIO Direction	47

12.5.2 GPIO to APB Direction	47

12.6 Synchronization Method	47

12.6.1 Control Signals	47

12.6.2 Multi-bit Data	47

12.7 Timing Considerations	48

12.7.1 Latency	48

12.7.2 Coherency	48

12.8 Reset Synchronization	48

13 APB GPIO - Interfaces Overview	48

13.1 External Interfaces	48

13.2 Interface Summary Diagram	49

13.3 Chapter Contents	49

13.3.1 APB Slave Interface	49

13.3.2 GPIO Pin Interface	49

13.3.3 Interrupt Interface	49

13.3.4 System Interface	49

14 APB GPIO - APB Slave Interface	50

14.1 Signal Description	50

14.1.1 APB Slave Signals	50

14.2 Protocol Compliance	50

14.2.1 APB3/APB4 Features	50

14.3 Timing	51

14.3.1 Zero Wait State	51

14.3.2 Timing Diagram	51

14.4 Address Decoding	52

14.4.1 Address Map	52

14.4.2 Byte Strobes	52

14.5 Error Handling	52

15 APB GPIO - GPIO Pin Interface	53

15.1 Signal Description	53

15.1.1 GPIO Signals	53

15.2 Pin Behavior	53

15.3 Synchronization	55

15.3.1 Input Synchronizer	55

15.3.2 Input Latency	55

15.4 Electrical Considerations	55

15.4.1 Output Characteristics	55

15.4.2 Input Characteristics	55

15.5 Timing Constraints	56

15.5.1 Output Path	56

15.5.2 Input Path	56

16 APB GPIO - Interrupt Interface	56

16.1 Signal Description	56

16.2 Interrupt Generation	56

16.2.1 Aggregate Logic	56

16.2.2 Per-Pin Configuration	56

16.3 Interrupt Modes	57

16.3.1 Edge-Triggered	57

16.3.2 Level-Sensitive	58

16.4 Interrupt Timing	58

16.5 Interrupt Handling	59

16.5.1 Software Sequence	59

16.5.2 Clearing Interrupts	59

16.6 Connection Guidelines	59

17 APB GPIO - System Interface	60

17.1 Clock Signals	60

17.1.1 pclk - APB Clock	60

17.1.2 gpio_clk - GPIO Clock	60

17.2 Reset Signals	60

17.2.1 presetn - APB Reset	60

17.2.2 gpio_rstn - GPIO Reset	60

17.3 Clock Configurations	61

17.4 Reset Sequence	61

17.4.1 Power-On Reset	61

17.4.2 Timing Diagram	62

17.5 Constraints	62

17.5.1 Clock Relationship	62

17.5.2 Reset Recovery	62

18 APB GPIO - Programming Model Overview	63

18.1 Register Summary	63

18.2 Chapter Contents	63

18.2.1 Basic Operations	63

18.2.2 Interrupt Configuration	63

18.2.3 Programming Examples	63

18.2.4 Software Considerations	64

19 APB GPIO - Basic Operations	64

19.1 Timing Diagrams	64

19.1.1 Direction Configuration	64

19.1.2 Output Write	65

19.1.3 Input Read	66

19.1.4 Input Synchronization	67

19.1.5 Atomic Operations	67

19.2 Initialization	68

19.2.1 Reset State	68

19.2.2 Enable GPIO	68

19.3 Output Operations	68

19.3.1 Configure as Output	68

19.3.2 Write Output Values	69

19.3.3 Toggle Outputs	69

19.3.4 Atomic Bit Operations	69

19.4 Input Operations	69

19.4.1 Configure as Input	69

19.4.2 Read Input Values	69

19.4.3 Read with Mask	69

19.5 Mixed I/O Configuration	69

19.5.1 Configure Mixed Directions	69

19.5.2 Read-Modify-Write Pattern	70

19.6 Output Enable Behavior	70

19.6.1 Hardware Interface	70

19.6.2 Glitch Considerations	70

20 APB GPIO - Interrupt Configuration	70

20.1 Interrupt Timing Diagrams	70

20.1.1 Rising Edge Interrupt	70

20.1.2 Falling Edge Interrupt	71

20.1.3 Level-Sensitive Interrupt	71

20.1.4 Interrupt Clear (W1C)	72

20.2 Interrupt Setup	73

20.2.1 1. Configure Interrupt Type	73

20.2.2 2. Configure Polarity	73

20.2.3 3. Configure Both-Edge Mode (Edge Mode Only)	73

20.2.4 4. Enable Interrupts	73

20.3 Interrupt Configuration Table	73

20.4 Complete Setup Examples	73

20.4.1 Rising Edge Interrupt	73

20.4.2 Both-Edge Interrupt	74

20.4.3 Active-Low Level Interrupt	74

20.5 Interrupt Handling	74

20.5.1 Check Interrupt Status	74

20.5.2 Clear Interrupts (Write-1-to-Clear)	74

20.5.3 Complete ISR Example	74

20.6 Level-Sensitive Considerations	75

20.6.1 Avoid Interrupt Storm	75

20.6.2 Masking During Handling	75

21 APB GPIO - Programming Examples	76

21.1 LED Control	76

21.1.1 Simple LED Blink	76

21.1.2 Multiple LED Control	76

21.2 Button Input	76

21.2.1 Simple Button Read	76

21.2.2 Button with Interrupt	77

21.3 DIP Switch Reading	77

21.3.1 8-Bit Switch Input	77

21.4 Parallel Data Interface	78

21.4.1 8-Bit Output Port	78

21.4.2 8-Bit Input Port with Ready	78

21.5 PWM-Style Output	79

21.5.1 Bit-Banged PWM (Low Frequency)	79

21.6 Wake-On-Change	79

21.6.1 Power Management Integration	79

22 APB GPIO - Software Considerations	80

22.1 Performance	80

22.1.1 Register Access Timing	80

22.1.2 Optimizing Access	80

22.2 Synchronization	81

22.2.1 Input Latency	81

22.2.2 Volatile Registers	81

22.2.3 Multi-Core Considerations	81

22.3 Interrupt Best Practices	81

22.3.1 Clear Before Return	81

22.3.2 Avoid Spurious Interrupts	81

22.3.3 Level-Sensitive Caution	82

22.4 Error Handling	82

22.4.1 No Hardware Errors	82

22.4.2 Software Validation	82

22.5 Debug Tips	83

22.5.1 Read-Back Verification	83

22.5.2 Loopback Testing	83

23 APB GPIO - Register Map	84

23.1 Register Summary	84

23.2 GPIO_CONTROL (0x000)	84

23.3 GPIO_DIRECTION (0x004)	85

23.4 GPIO_OUTPUT (0x008)	85

23.5 GPIO_INPUT (0x00C)	85

23.6 GPIO_INT_ENABLE (0x010)	85

23.7 GPIO_INT_TYPE (0x014)	86

23.8 GPIO_INT_POLARITY (0x018)	86

23.9 GPIO_INT_BOTH (0x01C)	86

23.10 GPIO_INT_STATUS (0x020)	87

23.11 Address Calculation	87

List of Figures

Figure 1.1: APB GPIO Block Diagram	19

Figure 1.2: APB GPIO Top-Level Architecture	21

Figure 1.3: APB GPIO Module Hierarchy	22

Figure 1.4: Write Transaction Flow	23

Figure 1.5: Read Transaction Flow	24

Figure 1.6: Interrupt Flow	24

Figure 1.7: Synchronous Mode Clock Domains	25

Figure 1.8: Asynchronous Mode Clock Domains	26

Figure 2.1: APB GPIO Module Hierarchy	35

Figure 2.5: Interrupt Controller Block Diagram	42

Figure 2.6: Edge Detection Logic	44

Figure 2.7: CDC Logic Block Diagram	46

Figure 2.8: Single Clock Domain (CDC Disabled)	47

Figure 2.9: Dual Clock Domain (CDC Enabled)	47

List of Tables

Table 1: Table 1.1: GPIO Parameters	28

Table 2: Table 1.2: Resource Estimates	28

Table 3: Table 2.1: Block Summary	35

Table 4: Table 2.5: Edge-Triggered Modes	42

Table 5: Table 2.6: Level-Sensitive Modes	43

Table 6: Table 2.7: CDC Enable Parameter	46

Table 7: Table 2.8: CDC Latency	48

List of Waveforms

No waveforms in this document.

# 1	Gpio Mas Index

Generated: 2026-08-14

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	APB GPIO - Overview

## 2.1	Introduction

The APB GPIO controller provides a 32-bit general-purpose I/O interface with APB bus connectivity. It enables software-controlled digital I/O with flexible interrupt generation capabilities.

## 2.2	Features

### 2.2.1	Core Functionality

- 32-bit bidirectional GPIO port

- Per-bit direction control (input/output)

- Per-bit output enable control

- Input synchronization for metastability protection

### 2.2.2	Interrupt Capabilities

- Per-bit interrupt enable

- Edge-triggered interrupts (rising, falling, or both)

- Level-triggered interrupts (high or low)

- Combined interrupt output (OR of all enabled sources)

- Write-1-to-clear interrupt status

### 2.2.3	Atomic Operations

- Atomic set (OR with mask)

- Atomic clear (AND with inverted mask)

- Atomic toggle (XOR with mask)

- No read-modify-write race conditions

### 2.2.4	Clock Domain Crossing

- Optional CDC support via CDC_ENABLE parameter

- Separate GPIO clock domain for async I/O

- Multi-stage input synchronization

## 2.3	Applications

### 2.3.1	Typical Use Cases

- LED control and status indication

- Push-button and switch inputs

- External device reset control

- Interrupt generation from external events

- Bit-banged serial protocols (I2C, SPI fallback)

- Debug signals and test points

### 2.3.2	System Integration

- Memory-mapped APB peripheral

- Single interrupt line to CPU/interrupt controller

- Direct connection to FPGA I/O pads via IOBUFs

- Compatible with standard GPIO software drivers

## 2.4	Block Diagram

### 2.4.1	Figure 1.1: APB GPIO Block Diagram

APB GPIO Block Diagram

## 2.5	Key Specifications

| Parameter | Value |
| --- | --- |
| GPIO Width | 32 bits (configurable) |
| APB Data Width | 32 bits |
| APB Address Width | 12 bits (4KB) |
| Sync Stages | 2 (configurable) |
| CDC Support | Optional |

## 2.6	Register Summary

| Address | Register | Description |
| --- | --- | --- |
| 0x000 | GPIO_CONTROL | Global enable and interrupt enable |
| 0x004 | GPIO_DIRECTION | Per-bit direction (1=output) |
| 0x008 | GPIO_OUTPUT | Output data value |
| 0x00C | GPIO_INPUT | Input data (read-only) |
| 0x010 | GPIO_INT_ENABLE | Per-bit interrupt enable |
| 0x014 | GPIO_INT_TYPE | Interrupt type (1=level, 0=edge) |
| 0x018 | GPIO_INT_POLARITY | Polarity (1=high/rising) |
| 0x01C | GPIO_INT_BOTH | Both-edge enable |
| 0x020 | GPIO_INT_STATUS | Interrupt status (W1C) |
| 0x024 | GPIO_RAW_INT | Raw interrupt (pre-mask) |
| 0x028 | GPIO_OUTPUT_SET | Atomic set |
| 0x02C | GPIO_OUTPUT_CLR | Atomic clear |
| 0x030 | GPIO_OUTPUT_TGL | Atomic toggle |

Next: 02_architecture.md - High-level architecture

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	APB GPIO - Architecture

## 3.1	High-Level Block Diagram

### 3.1.1	Figure 1.2: APB GPIO Top-Level Architecture

APB GPIO Architecture

## 3.2	Module Hierarchy

### 3.2.1	Figure 1.3: APB GPIO Module Hierarchy

APB GPIO Module Hierarchy

## 3.3	Data Flow

### 3.3.1	Write Transaction Flow

### 3.3.2	Figure 1.4: Write Transaction Flow

Write Transaction Flow

### 3.3.3	Read Transaction Flow

### 3.3.4	Figure 1.5: Read Transaction Flow

Read Transaction Flow

### 3.3.5	Interrupt Flow

### 3.3.6	Figure 1.6: Interrupt Flow

Interrupt Flow

## 3.4	Clock Domains

### 3.4.1	Synchronous Mode (CDC_ENABLE = 0)

### 3.4.2	Figure 1.7: Synchronous Mode Clock Domains

Synchronous Mode

In synchronous mode, all modules operate on the APB clock (pclk). Input synchronization remains active for external GPIO pins to prevent metastability.

### 3.4.3	Asynchronous Mode (CDC_ENABLE = 1)

### 3.4.4	Figure 1.8: Asynchronous Mode Clock Domains

Asynchronous Mode

In asynchronous mode, the APB clock domain handles protocol conversion while the GPIO clock domain handles all register and I/O operations. Skid buffers provide safe clock domain crossing.

## 3.5	Parameterization

Table 1.1: GPIO Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| GPIO_WIDTH | int | 32 | Number of GPIO pins |
| SYNC_STAGES | int | 2 | Input synchronizer stages |
| CDC_ENABLE | int | 0 | Enable clock domain crossing |
| SKID_DEPTH | int | 2 | CDC skid buffer depth |

## 3.6	Resource Estimates

Table 1.2: Resource Estimates

| Component | Flip-Flops | LUTs |
| --- | --- | --- |
| gpio_core | ~200 | ~300 |
| gpio_regs | ~400 | ~200 |
| gpio_config_regs | ~50 | ~100 |
| apb4_slave (no CDC) | ~20 | ~50 |
| apb4_slave_cdc | ~100 | ~150 |
| Total (no CDC) | ~670 | ~650 |
| Total (with CDC) | ~750 | ~750 |

Next: 03_clocks_and_reset.md - Clock and reset behavior

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	APB GPIO - Clocks and Reset

## 4.1	Clock Signals

### 4.1.1	pclk (APB Clock)

- Purpose: Primary APB bus clock

- Usage: APB protocol, register access

- Typical Frequency: 50-200 MHz

### 4.1.2	gpio_clk (GPIO Clock)

- Purpose: Optional separate GPIO clock domain

- Usage: Only when CDC_ENABLE=1

- Relationship: Can be asynchronous to pclk

## 4.2	Reset Signals

### 4.2.1	presetn (APB Reset)

- Type: Active-low asynchronous reset

- Scope: APB interface logic

- Behavior: Resets APB state machine, clears pending transactions

### 4.2.2	gpio_rstn (GPIO Reset)

- Type: Active-low asynchronous reset

- Scope: GPIO core logic

- Usage: Only when CDC_ENABLE=1

- Behavior: Resets GPIO outputs, interrupt state

## 4.3	Reset Behavior

### 4.3.1	Register Reset Values

| Register | Reset Value | Notes |
| --- | --- | --- |
| GPIO_CONTROL | 0x00000000 | GPIO disabled |
| GPIO_DIRECTION | 0x00000000 | All inputs |
| GPIO_OUTPUT | 0x00000000 | Outputs low |
| GPIO_INT_ENABLE | 0x00000000 | No interrupts |
| GPIO_INT_TYPE | 0x00000000 | Edge mode |
| GPIO_INT_POLARITY | 0x00000000 | Falling/low |
| GPIO_INT_BOTH | 0x00000000 | Single edge |
| GPIO_INT_STATUS | 0x00000000 | No pending |

### 4.3.2	Output Pin Behavior During Reset

During reset: - gpio_out[31:0] = 0 - gpio_oe[31:0] = 0 (all high-Z) - irq = 0

## 4.4	Clock Domain Crossing

### 4.4.1	When CDC_ENABLE = 0

- All logic runs on pclk

- gpio_clk input is ignored

- Connect gpio_clk = pclk for clean design

### 4.4.2	When CDC_ENABLE = 1

- APB interface uses pclk

- GPIO core uses gpio_clk

- Skid buffers handle CDC

- Both resets must be asserted together at power-on

## 4.5	Input Synchronization

GPIO inputs are always synchronized regardless of CDC setting:

Diagram 1

- SYNC_STAGES parameter controls depth (default: 2)

- Prevents metastability from external signal transitions

- Adds latency equal to SYNC_STAGES clock cycles

## 4.6	Timing Constraints

### 4.6.1	Synchronous Mode

- Standard single-clock timing

- All paths constrained to pclk

### 4.6.2	Asynchronous Mode

- Set false_path between pclk and gpio_clk domains

- Set max_delay for CDC paths

- Synchronizer FFs should have ASYNC_REG attribute

Next: 04_acronyms.md - Acronyms and terminology

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	APB GPIO - Acronyms and Terminology

## 5.1	Protocol Acronyms

| Acronym | Full Name | Description |
| --- | --- | --- |
| APB | Advanced Peripheral Bus | Low-power AMBA bus protocol |
| AMBA | Advanced Microcontroller Bus Architecture | ARM standard bus protocols |
| CDC | Clock Domain Crossing | Synchronization between clock domains |

## 5.2	Signal Acronyms

| Acronym | Full Name | Description |
| --- | --- | --- |
| CLK | Clock | Timing reference signal |
| RST | Reset | System initialization signal |
| OE | Output Enable | Tri-state buffer control |
| IRQ | Interrupt Request | Hardware interrupt signal |

## 5.3	GPIO-Specific Terms

| Term | Description |
| --- | --- |
| GPIO | General Purpose Input/Output - Configurable digital I/O pins |
| Pin | Individual GPIO signal (input or output) |
| Port | Group of 32 GPIO pins managed together |
| Direction | Input (0) or Output (1) configuration per pin |
| Polarity | Active-high or active-low signal interpretation |

## 5.4	Interrupt Terms

| Term | Description |
| --- | --- |
| Edge-triggered | Interrupt on signal transition |
| Level-sensitive | Interrupt while signal is at specified level |
| Rising edge | Low-to-high transition |
| Falling edge | High-to-low transition |
| Both edges | Either transition direction |

## 5.5	Register Terms

| Term | Description |
| --- | --- |
| RW | Read-Write register |
| RO | Read-Only register |
| W1C | Write-1-to-Clear register |
| HWIF | Hardware Interface (PeakRDL generated) |

Next: 05_references.md - Reference documents

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	APB GPIO - References

## 6.1	Internal Documentation

### 6.1.1	RTL Source Files

- rtl/gpio/apb4_gpio.sv - Main GPIO module

- rtl/gpio/gpio_regs.sv - PeakRDL-generated register file

- rtl/gpio/peakrdl/gpio_regs.rdl - Register description source

### 6.1.2	Related Specifications

- APB Protocol Specification (AMBA 3)

- RLB Integration Guide

## 6.2	External References

### 6.2.1	ARM AMBA Specifications

- AMBA 3 APB Protocol Specification

  - ARM IHI 0024E

  - Defines APB interface timing and protocol

### 6.2.2	Industry Standards

- GPIO Best Practices

  - Synchronization for external inputs

  - Interrupt handling patterns

  - Tri-state buffer management

## 6.3	Design References

### 6.3.1	Clock Domain Crossing

- Dual flip-flop synchronizer methodology

- Skid buffer for data path CDC

- Reset synchronization techniques

### 6.3.2	Interrupt Handling

- Edge detection circuits

- Interrupt aggregation patterns

- Software interrupt acknowledge flows

Next: Chapter 2: Block Descriptions

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	APB GPIO - Block Descriptions Overview

## 7.1	Module Hierarchy

### 7.1.1	Figure 2.1: APB GPIO Module Hierarchy

Module Hierarchy

## 7.2	Block Summary

Table 2.1: Block Summary

| Block | File | Description |
| --- | --- | --- |
| APB GPIO Top | apb4_gpio.sv | Top-level module with all GPIO functionality |
| Register File | apb4_gpio_regs.sv | PeakRDL-generated control/status registers |

## 7.3	Detailed Block Descriptions

### 7.3.1	1. APB Interface

Handles APB protocol conversion and register access.

See: 01_apb_interface.md

### 7.3.2	2. Register File

PeakRDL-generated registers for configuration and status.

See: 02_register_file.md

### 7.3.3	3. GPIO Core

Main GPIO functionality including I/O control and interrupts.

See: 03_gpio_core.md

### 7.3.4	4. Interrupt Controller

Edge detection, level sensing, and interrupt aggregation.

See: 04_interrupt_controller.md

### 7.3.5	5. CDC Logic

Optional clock domain crossing for asynchronous GPIO clock.

See: 05_cdc_logic.md

Next: 01_apb_interface.md - APB Interface details

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	APB GPIO - APB Interface Block

## 8.1	Overview

The APB interface provides the connection between the system APB bus and the GPIO register file.

## 8.2	Block Diagram

APB Interface Block

APB Interface Block

## 8.3	Interface Signals

### 8.3.1	APB Slave Interface

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_apb_psel | 1 | Input | Slave select |
| s_apb_penable | 1 | Input | Enable phase |
| s_apb_pwrite | 1 | Input | Write operation |
| s_apb_paddr | 12 | Input | Address bus |
| s_apb_pwdata | 32 | Input | Write data |
| s_apb_pstrb | 4 | Input | Byte strobes |
| s_apb_prdata | 32 | Output | Read data |
| s_apb_pready | 1 | Output | Ready response |
| s_apb_pslverr | 1 | Output | Error response |

## 8.4	Operation

### 8.4.1	Read Transaction

- Master asserts psel and paddr

- Master asserts penable on next cycle

- Slave returns prdata with pready

### 8.4.2	Write Transaction

- Master asserts psel, paddr, pwdata, pwrite

- Master asserts penable on next cycle

- Slave samples data with pready

## 8.5	Timing Diagram

_____       _____       _____
pclk  __/     \_____/     \_____/     \_____

         _________________
psel  __/                 \_________________

                   _______
penable __________|       |_________________

paddr   XXXXXXXXXX|  ADDR |XXXXXXXXXXXXXXXXX

prdata  XXXXXXXXXX|  DATA |XXXXXXXXXXXXXXXXX
                   _______
pready  __________|       |_________________

## 8.6	Implementation Notes

- Zero wait-state operation for all registers

- No error responses (pslverr always 0)

- 32-bit aligned access only

Next: 02_register_file.md - Register File

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	APB GPIO - Register File Block

## 9.1	Overview

The register file is generated by PeakRDL from apb4_gpio.rdl and provides hardware/software interface for GPIO control.

## 9.2	Block Diagram

Register File Block

Register File Block

## 9.3	Generated Interface (HWIF)

### 9.3.1	Hardware-to-Software (hw2reg)

| Signal | Width | Description |
| --- | --- | --- |
| gpio_input.data | 32 | Current GPIO input values |
| gpio_int_status.data | 32 | Interrupt status bits |

### 9.3.2	Software-to-Hardware (reg2hw)

| Signal | Width | Description |
| --- | --- | --- |
| gpio_control.enable | 1 | GPIO enable |
| gpio_direction.data | 32 | Pin direction (0=in, 1=out) |
| gpio_output.data | 32 | Output values |
| gpio_int_enable.data | 32 | Interrupt enable per pin |
| gpio_int_type.data | 32 | Interrupt type (0=edge, 1=level) |
| gpio_int_polarity.data | 32 | Polarity (0=fall/low, 1=rise/high) |
| gpio_int_both.data | 32 | Both edges enable |

## 9.4	Register Access

### 9.4.1	Byte Enable Support

All registers support byte-granular writes via pstrb: - pstrb[0] enables bits [7:0] - pstrb[1] enables bits [15:8] - pstrb[2] enables bits [23:16] - pstrb[3] enables bits [31:24]

### 9.4.2	Access Types

| Type | Read Behavior | Write Behavior |
| --- | --- | --- |
| RW | Returns current value | Updates register |
| RO | Returns hardware value | No effect |
| W1C | Returns current value | Clears bits where 1 written |

## 9.5	Implementation Notes

- Generated by PeakRDL regblock

- Synchronous to pclk domain

- All registers reset to 0

Next: 03_gpio_core.md - GPIO Core

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	APB GPIO - GPIO Core Block

## 10.1	Overview

The GPIO core handles input synchronization, output driving, and direction control for all 32 GPIO pins.

## 10.2	Block Diagram

GPIO Core Block

GPIO Core Block

## 10.3	Input Path

### 10.3.1	Synchronization

External inputs pass through a dual flip-flop synchronizer:

Diagram 2

- Prevents metastability from asynchronous inputs

- Configurable depth via SYNC_STAGES parameter

- Adds SYNC_STAGES clock cycles of latency

### 10.3.2	Input Register

Synchronized inputs are presented to software via GPIO_INPUT register.

## 10.4	Output Path

### 10.4.1	Output Register

Software writes to GPIO_OUTPUT register to set output values.

### 10.4.2	Output Enable

Direction register controls tri-state buffers: - direction[i] = 0: Pin is input (high-Z output) - direction[i] = 1: Pin is output (driven)

### 10.4.3	External Signals

| Signal | Width | Description |
| --- | --- | --- |
| gpio_out | 32 | Output data values |
| gpio_oe | 32 | Output enables (active high) |
| gpio_in | 32 | Input data values |

## 10.5	Direction Control

### 10.5.1	Per-Pin Configuration

Each pin independently configured:

if (direction[i]) begin
    // Output mode
    gpio_oe[i] = 1'b1;
    gpio_out[i] = output_reg[i];
end else begin
    // Input mode
    gpio_oe[i] = 1'b0;
    // gpio_out[i] = don't care
end

### 10.5.2	Read-Back Behavior

Reading GPIO_INPUT returns: - For input pins: External signal value (synchronized) - For output pins: External signal value (may differ from output_reg if open-drain)

## 10.6	Implementation Notes

- All 32 pins processed in parallel

- Zero-latency output updates

- Input synchronization always active

Next: 04_interrupt_controller.md - Interrupt Controller

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	APB GPIO - Interrupt Controller Block

## 11.1	Overview

The interrupt controller provides flexible interrupt generation for each GPIO pin with support for edge and level triggering.

## 11.2	Block Diagram

### 11.2.1	Figure 2.5: Interrupt Controller Block Diagram

Interrupt Controller Block

## 11.3	Interrupt Modes

### 11.3.1	Edge-Triggered Mode

GPIO_INT_TYPE[i] = 0

Table 2.5: Edge-Triggered Modes

| GPIO_INT_POLARITY | GPIO_INT_BOTH | Trigger Condition |
| --- | --- | --- |
| 0 | 0 | Falling edge only |
| 1 | 0 | Rising edge only |
| X | 1 | Both edges |

### 11.3.2	Level-Sensitive Mode

GPIO_INT_TYPE[i] = 1

Table 2.6: Level-Sensitive Modes

| GPIO_INT_POLARITY | Trigger Condition |
| --- | --- |
| 0 | Active low (interrupt while pin = 0) |
| 1 | Active high (interrupt while pin = 1) |

## 11.4	Edge Detection Logic

### 11.4.1	Figure 2.6: Edge Detection Logic

Edge Detection Logic

## 11.5	Interrupt Status

### 11.5.1	Status Register

- Each bit in GPIO_INT_STATUS corresponds to one pin

- Set when interrupt condition detected

- Cleared by writing 1 to the bit (W1C)

### 11.5.2	Interrupt Enable

- GPIO_INT_ENABLE[i] = 1 enables interrupt for pin i

- Disabled pins don’t affect irq output

- Status bits still set regardless of enable

## 11.6	Aggregate IRQ Output

irq = |(gpio_int_status & gpio_int_enable)

Single IRQ output is OR of all enabled, active interrupts.

## 11.7	Interrupt Handling Flow

- Hardware detects condition, sets status bit

- IRQ asserted to processor

- Software reads GPIO_INT_STATUS to identify source

- Software handles interrupt

- Software writes 1 to status bit to clear

- IRQ deasserts (if no other sources active)

## 11.8	Implementation Notes

- Edge detection uses synchronized input

- Level-sensitive interrupts re-trigger if not cleared

- Status bits latch until software clears

Next: 05_cdc_logic.md - CDC Logic

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	APB GPIO - CDC Logic Block

## 12.1	Overview

Optional clock domain crossing logic enables GPIO core to run on a separate clock from the APB interface.

## 12.2	Block Diagram

### 12.2.1	Figure 2.7: CDC Logic Block Diagram

CDC Logic Block

## 12.3	Configuration

### 12.3.1	Parameter: CDC_ENABLE

Table 2.7: CDC Enable Parameter

| Value | Behavior |
| --- | --- |
| 0 | Single clock domain, all logic on pclk |
| 1 | Dual clock domain, GPIO core on gpio_clk |

## 12.4	Clock Domains

### 12.4.1	When CDC_ENABLE = 0

### 12.4.2	Figure 2.8: Single Clock Domain (CDC Disabled)

CDC Disabled

### 12.4.3	When CDC_ENABLE = 1

### 12.4.4	Figure 2.9: Dual Clock Domain (CDC Enabled)

CDC Enabled

## 12.5	CDC Implementation

### 12.5.1	APB to GPIO Direction

Register values synchronized to gpio_clk domain: - gpio_direction - gpio_output - gpio_int_enable - gpio_int_type - gpio_int_polarity - gpio_int_both

### 12.5.2	GPIO to APB Direction

Status values synchronized to pclk domain: - gpio_input (synchronized input values) - gpio_int_status (interrupt status)

## 12.6	Synchronization Method

### 12.6.1	Control Signals

Dual flip-flop synchronizers for single-bit controls.

### 12.6.2	Multi-bit Data

Skid buffers with handshake protocol for register transfers.

## 12.7	Timing Considerations

### 12.7.1	Latency

Table 2.8: CDC Latency

| Path | Latency |
| --- | --- |
| Register write to GPIO output | 2-4 gpio_clk cycles |
| GPIO input to register read | 2-4 pclk cycles |
| Interrupt detection to IRQ | 2-4 pclk cycles |

### 12.7.2	Coherency

- No guaranteed atomicity across clock domains

- Software must handle potential inconsistencies

- Interrupt status always reflects gpio_clk domain

## 12.8	Reset Synchronization

Both resets must be asserted at power-on: 1. Assert both presetn and gpio_rstn 2. Release gpio_rstn first 3. Release presetn after gpio_clk domain stable

Back to: 00_overview.md - Block Descriptions Overview

Next Chapter: Chapter 3: Interfaces

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	APB GPIO - Interfaces Overview

## 13.1	External Interfaces

The APB GPIO module has the following external interfaces:

| Interface | Type | Description |
| --- | --- | --- |
| APB Slave | Bus | Configuration and status access |
| GPIO Pins | I/O | 32 general-purpose I/O pins |
| Interrupt | Signal | Aggregate interrupt output |
| Clocks/Reset | System | Clock and reset inputs |

## 13.2	Interface Summary Diagram

GPIO Interfaces

GPIO Interfaces

## 13.3	Chapter Contents

### 13.3.1	APB Slave Interface

Complete APB protocol interface for register access.

See: 01_apb4_slave.md

### 13.3.2	GPIO Pin Interface

External GPIO pin connections with tri-state control.

See: 02_gpio_pins.md

### 13.3.3	Interrupt Interface

Interrupt request output signal.

See: 03_interrupt.md

### 13.3.4	System Interface

Clock and reset signal requirements.

See: 04_system.md

Next: 01_apb4_slave.md - APB Slave Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	APB GPIO - APB Slave Interface

## 14.1	Signal Description

### 14.1.1	APB Slave Signals

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| pclk | 1 | I | APB clock |
| presetn | 1 | I | APB reset (active low) |
| s_apb_psel | 1 | I | Peripheral select |
| s_apb_penable | 1 | I | Enable phase |
| s_apb_pwrite | 1 | I | Write transaction |
| s_apb_paddr | 12 | I | Address bus |
| s_apb_pwdata | 32 | I | Write data |
| s_apb_pstrb | 4 | I | Byte strobes |
| s_apb_prdata | 32 | O | Read data |
| s_apb_pready | 1 | O | Ready response |
| s_apb_pslverr | 1 | O | Slave error |

## 14.2	Protocol Compliance

### 14.2.1	APB3/APB4 Features

| Feature | Support |
| --- | --- |
| PSEL | Yes |
| PENABLE | Yes |
| PWRITE | Yes |
| PADDR | 12-bit |
| PWDATA | 32-bit |
| PRDATA | 32-bit |
| PREADY | Yes (always 1) |
| PSLVERR | Yes (always 0) |
| PSTRB | Yes |
| PPROT | No |

## 14.3	Timing

### 14.3.1	Zero Wait State

All register accesses complete in minimum APB cycles: - Read: 2 cycles (setup + access) - Write: 2 cycles (setup + access)

### 14.3.2	Timing Diagram

___     ___     ___     ___
pclk    _|   |___|   |___|   |___|   |___

        _______________
psel   |               |_________________

                _______
penable ________|       |________________

paddr   --------|  A1   |----------------

pwdata  --------|  D1   |---------------- (write)

prdata  --------|  D1   |---------------- (read)
                _______
pready  ________|       |________________

## 14.4	Address Decoding

### 14.4.1	Address Map

| Address | Register | Access |
| --- | --- | --- |
| 0x000 | GPIO_CONTROL | RW |
| 0x004 | GPIO_DIRECTION | RW |
| 0x008 | GPIO_OUTPUT | RW |
| 0x00C | GPIO_INPUT | RO |
| 0x010 | GPIO_INT_ENABLE | RW |
| 0x014 | GPIO_INT_TYPE | RW |
| 0x018 | GPIO_INT_POLARITY | RW |
| 0x01C | GPIO_INT_BOTH | RW |
| 0x020 | GPIO_INT_STATUS | W1C |

### 14.4.2	Byte Strobes

Byte-granular writes supported: - pstrb[3:0] corresponds to pwdata[31:0] - Unselected bytes retain previous values

## 14.5	Error Handling

- No address decode errors (all addresses valid)

- No timeout errors

- pslverr always 0

Next: 02_gpio_pins.md - GPIO Pin Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	APB GPIO - GPIO Pin Interface

## 15.1	Signal Description

### 15.1.1	GPIO Signals

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| gpio_out | 32 | O | Output data values |
| gpio_oe | 32 | O | Output enables |
| gpio_in | 32 | I | Input data values |

## 15.2	Pin Behavior

Output Mode (direction[i] = 1)

- gpio_oe[i] = 1 (output enabled)

- gpio_out[i] = GPIO_OUTPUT register value

- Pin driven to output value

Input Mode (direction[i] = 0)

- gpio_oe[i] = 0 (tri-state)

- gpio_out[i] = don’t care

- External value captured via synchronizer

## 15.3	Synchronization

### 15.3.1	Input Synchronizer

All inputs pass through dual flip-flop synchronizer:

Diagram 5

- SYNC_STAGES parameter controls depth (default: 2)

- Prevents metastability from asynchronous inputs

- All 32 inputs synchronized in parallel

### 15.3.2	Input Latency

Input changes visible in GPIO_INPUT register after: - SYNC_STAGES cycles of gpio_clk (or pclk if CDC_ENABLE=0) - Plus APB read latency

## 15.4	Electrical Considerations

### 15.4.1	Output Characteristics

| Parameter | Description |
| --- | --- |
| Drive | Standard CMOS output |
| Slew | Defined by I/O cell |
| Protection | ESD per I/O cell design |

### 15.4.2	Input Characteristics

| Parameter | Description |
| --- | --- |
| Levels | CMOS compatible |
| Hysteresis | Optional per I/O cell |
| Pull-up/down | External to GPIO module |

## 15.5	Timing Constraints

### 15.5.1	Output Path

- gpio_out updates on clock edge

- gpio_oe updates on same clock edge

- No glitch-free guarantee during direction change

### 15.5.2	Input Path

- Setup/hold relative to synchronizer clock

- Metastability resolved by synchronizer

Next: 03_interrupt.md - Interrupt Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	APB GPIO - Interrupt Interface

## 16.1	Signal Description

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| irq | 1 | O | Interrupt request (active high) |

## 16.2	Interrupt Generation

### 16.2.1	Aggregate Logic

irq = |(GPIO_INT_STATUS[31:0] & GPIO_INT_ENABLE[31:0])

IRQ is asserted when any enabled interrupt source is active.

### 16.2.2	Per-Pin Configuration

Each GPIO pin can generate interrupts independently:

| Register | Function |
| --- | --- |
| GPIO_INT_ENABLE | Enable/disable per pin |
| GPIO_INT_TYPE | Edge (0) or Level (1) |
| GPIO_INT_POLARITY | Falling/Low (0) or Rising/High (1) |
| GPIO_INT_BOTH | Both edges (edge mode only) |
| GPIO_INT_STATUS | Current interrupt status |

## 16.3	Interrupt Modes

### 16.3.1	Edge-Triggered

GPIO_INT_TYPE[i] = 0

Diagram 6

- Captures transitions on synchronized input

- Status bit latches until cleared by software

- Both-edge mode ignores polarity setting

### 16.3.2	Level-Sensitive

GPIO_INT_TYPE[i] = 1

Diagram 7

- Continuously compares input to polarity

- Status follows input level

- Re-triggers if not cleared while active

## 16.4	Interrupt Timing

Edge-Triggered Latency

Total: 4 clock cycles typical

Level-Sensitive Latency

Total: 3 clock cycles typical

## 16.5	Interrupt Handling

### 16.5.1	Software Sequence

- IRQ asserts (hardware)

- CPU vectors to interrupt handler

- Read GPIO_INT_STATUS to identify sources

- Handle interrupt condition

- Write 1 to GPIO_INT_STATUS bits to clear

- IRQ deasserts if no other sources

### 16.5.2	Clearing Interrupts

| Mode | Clear Method |
| --- | --- |
| Edge | Write 1 to STATUS bit |
| Level | Clear source, then write 1 to STATUS |

## 16.6	Connection Guidelines

- Connect to interrupt controller input

- Active-high, level-sensitive recommended at controller

- Single IRQ covers all 32 GPIO pins

Next: 04_system.md - System Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	APB GPIO - System Interface

## 17.1	Clock Signals

### 17.1.1	pclk - APB Clock

| Parameter | Value |
| --- | --- |
| Purpose | APB interface clock |
| Frequency | 50-200 MHz typical |
| Domain | APB bus timing |

Used for: - APB protocol timing - Register file access - IRQ generation (single-clock mode)

### 17.1.2	gpio_clk - GPIO Clock

| Parameter | Value |
| --- | --- |
| Purpose | GPIO core clock |
| Frequency | Application dependent |
| Usage | Only when CDC_ENABLE=1 |

Used for: - Input synchronization - Output register updates - Interrupt detection

## 17.2	Reset Signals

### 17.2.1	presetn - APB Reset

| Parameter | Value |
| --- | --- |
| Polarity | Active low |
| Type | Asynchronous assert, synchronous deassert |
| Scope | APB interface logic |

Resets: - APB state machine - Register file - Response logic

### 17.2.2	gpio_rstn - GPIO Reset

| Parameter | Value |
| --- | --- |
| Polarity | Active low |
| Type | Asynchronous assert, synchronous deassert |
| Usage | Only when CDC_ENABLE=1 |

Resets: - GPIO output registers - Input synchronizers - Interrupt state

## 17.3	Clock Configurations

Single Clock Domain (CDC_ENABLE = 0)

gpio_clk: Tie to pclk or leave unconnected

Dual Clock Domain (CDC_ENABLE = 1)

## 17.4	Reset Sequence

### 17.4.1	Power-On Reset

- Assert both presetn and gpio_rstn low

- Clocks may be running or stopped

- Hold reset for minimum 2 clock cycles

- Release gpio_rstn first

- Wait 2 gpio_clk cycles

- Release presetn

- Wait 2 pclk cycles before APB access

### 17.4.2	Timing Diagram

________________________________________
pclk     |  |  |  |  |  |  |  |  |  |  |  |  |  |

          ________________________________________
gpio_clk |  |  |  |  |  |  |  |  |  |  |  |  |  |

                              _____________________
presetn  ____________________|

                        ___________________________
gpio_rstn ______________|

              |<- min 2 ->|<- 2 ->|<- 2 ->|
                 clocks    gclk    pclk

## 17.5	Constraints

### 17.5.1	Clock Relationship

| Mode | Constraint |
| --- | --- |
| CDC_ENABLE=0 | Single clock, no special constraints |
| CDC_ENABLE=1 | Set false_path between domains |

### 17.5.2	Reset Recovery

- Allow minimum 2 clock cycles after reset deassert

- First APB transaction may start on 3rd cycle

Back to: 00_overview.md - Interfaces Overview

Next Chapter: Chapter 4: Programming Model

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	APB GPIO - Programming Model Overview

## 18.1	Register Summary

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x000 | GPIO_CONTROL | RW | Global control |
| 0x004 | GPIO_DIRECTION | RW | I/O direction |
| 0x008 | GPIO_OUTPUT | RW | Output data |
| 0x00C | GPIO_INPUT | RO | Input data |
| 0x010 | GPIO_INT_ENABLE | RW | Interrupt enable |
| 0x014 | GPIO_INT_TYPE | RW | Edge/level select |
| 0x018 | GPIO_INT_POLARITY | RW | Interrupt polarity |
| 0x01C | GPIO_INT_BOTH | RW | Both edges |
| 0x020 | GPIO_INT_STATUS | W1C | Interrupt status |

## 18.2	Chapter Contents

### 18.2.1	Basic Operations

Fundamental GPIO read/write operations.

See: 01_basic_operations.md

### 18.2.2	Interrupt Configuration

Setting up and handling GPIO interrupts.

See: 02_interrupt_config.md

### 18.2.3	Programming Examples

Common use cases with code samples.

See: 03_examples.md

### 18.2.4	Software Considerations

Performance tips and best practices.

See: 04_software_notes.md

Next: 01_basic_operations.md - Basic Operations

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	APB GPIO - Basic Operations

## 19.1	Timing Diagrams

The following diagrams show the internal signal flow for basic GPIO operations.

### 19.1.1	Direction Configuration

When software writes to GPIO_DIRECTION, the direction register updates and controls the output enable for each pin.

GPIO Direction Write

The APB write completes in a single cycle. The direction register (r_gpio_direction) updates on the clock edge following PREADY, and the output enable (gpio_oe) reflects the new configuration immediately.

### 19.1.2	Output Write

Writing to GPIO_OUTPUT sets the output data register, which drives the external pins when direction is set to output.

GPIO Output Write

The write data flows through the APB interface to the output register. When gpio_oe[n] is high (output mode), gpio_out[n] drives the written value to the external pin.

### 19.1.3	Input Read

Reading GPIO_INPUT returns the synchronized input values from external pins.

GPIO Input Read

External inputs pass through a 2-stage synchronizer before being captured. The synchronized value (w_gpio_sync) is returned on s_apb_PRDATA during the APB read transaction.

### 19.1.4	Input Synchronization

All GPIO inputs pass through a 2-stage synchronizer to prevent metastability.

GPIO Input Sync

The synchronizer adds 2 clock cycles of latency. External asynchronous transitions on gpio_in propagate through sync_stage1 and sync_stage2 before appearing on the internal synchronized signal w_gpio_sync.

### 19.1.5	Atomic Operations

The SET, CLEAR, and TOGGLE registers provide atomic bit manipulation without read-modify-write races.

GPIO Atomic Operations

Three consecutive APB writes demonstrate: 1. GPIO_SET: Sets bits where write data is 1, leaves others unchanged 2. GPIO_CLEAR: Clears bits where write data is 1, leaves others unchanged 3. GPIO_TOGGLE: Inverts bits where write data is 1, leaves others unchanged

## 19.2	Initialization

### 19.2.1	Reset State

After reset, all registers are 0: - GPIO disabled - All pins configured as inputs - No interrupts enabled

### 19.2.2	Enable GPIO

// Enable GPIO controller
GPIO_CONTROL = 0x00000001;

## 19.3	Output Operations

### 19.3.1	Configure as Output

// Set pins 7:4 as outputs (bits = 1 for output)
GPIO_DIRECTION = 0x000000F0;

### 19.3.2	Write Output Values

// Set pins 7:4 to value 0101
GPIO_OUTPUT = 0x00000050;

### 19.3.3	Toggle Outputs

// Read current output, XOR to toggle
uint32_t current = GPIO_OUTPUT;
GPIO_OUTPUT = current ^ 0x000000F0;  // Toggle pins 7:4

### 19.3.4	Atomic Bit Operations

// Set specific bits (pins 5 and 7)
GPIO_OUTPUT |= 0x000000A0;

// Clear specific bits (pins 4 and 6)
GPIO_OUTPUT &= ~0x00000050;

## 19.4	Input Operations

### 19.4.1	Configure as Input

// Set pins 3:0 as inputs (bits = 0 for input)
GPIO_DIRECTION &= ~0x0000000F;

### 19.4.2	Read Input Values

// Read all inputs
uint32_t inputs = GPIO_INPUT;

// Check specific pin (pin 2)
if (inputs & 0x00000004) {
    // Pin 2 is high
}

### 19.4.3	Read with Mask

// Read only pins 3:0
uint32_t low_nibble = GPIO_INPUT & 0x0000000F;

## 19.5	Mixed I/O Configuration

### 19.5.1	Configure Mixed Directions

// Pins 31:16 = outputs, pins 15:0 = inputs
GPIO_DIRECTION = 0xFFFF0000;

### 19.5.2	Read-Modify-Write Pattern

// Change only pins 11:8 to outputs
uint32_t dir = GPIO_DIRECTION;
dir |= 0x00000F00;    // Set pins 11:8
GPIO_DIRECTION = dir;

## 19.6	Output Enable Behavior

### 19.6.1	Hardware Interface

When direction bit is set: - gpio_oe[i] = 1 (output enabled) - gpio_out[i] = GPIO_OUTPUT[i] value

When direction bit is clear: - gpio_oe[i] = 0 (high impedance) - gpio_out[i] = don’t care

### 19.6.2	Glitch Considerations

To avoid output glitches when switching direction: 1. Set GPIO_OUTPUT to desired value 2. Then change GPIO_DIRECTION

// Safe direction change to output
GPIO_OUTPUT = desired_value;    // Set value first
GPIO_DIRECTION |= pin_mask;     // Then enable output

Next: 02_interrupt_config.md - Interrupt Configuration

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	APB GPIO - Interrupt Configuration

## 20.1	Interrupt Timing Diagrams

The following diagrams illustrate GPIO interrupt detection and handling.

### 20.1.1	Rising Edge Interrupt

Edge-triggered interrupts detect transitions on input pins.

GPIO Rising Edge Interrupt

The detection sequence: 1. External input gpio_in[0] transitions from 0 to 1 2. 2-stage synchronizer captures the transition (w_gpio_sync) 3. Edge detector compares current vs. delayed value (r_gpio_sync_d) 4. Rising edge pulse (w_rising_edge) generated for one clock 5. Raw interrupt latched in r_raw_int[0] 6. Combined irq output asserts

### 20.1.2	Falling Edge Interrupt

Falling edge detection uses inverted polarity configuration.

GPIO Falling Edge Interrupt

With cfg_int_type[0]=0 (edge mode) and cfg_int_polarity[0]=0 (falling edge), the detector triggers on 1-to-0 transitions.

### 20.1.3	Level-Sensitive Interrupt

Level-sensitive interrupts track the input state directly.

GPIO Level Interrupt

Key differences from edge mode: - irq follows the input level (not latched) - No edge detection logic involved - Interrupt re-asserts if source not cleared before ISR exit

### 20.1.4	Interrupt Clear (W1C)

Write-1-to-Clear mechanism clears latched interrupts.

GPIO Interrupt Clear

The clear sequence: 1. r_raw_int[0] is active (edge was detected) 2. Software writes 0x01 to INT_STATUS register 3. W1C logic clears r_int_status[0] 4. irq deasserts

Note: For level-sensitive interrupts, the external source must be cleared first, otherwise the interrupt immediately re-asserts.

## 20.2	Interrupt Setup

### 20.2.1	1. Configure Interrupt Type

// Edge-triggered (0) or Level-sensitive (1)
// Pins 3:0 edge, pins 7:4 level
GPIO_INT_TYPE = 0x000000F0;

### 20.2.2	2. Configure Polarity

For edge mode: - 0 = falling edge - 1 = rising edge

For level mode: - 0 = active low - 1 = active high

// Rising edge / active high for pins 7:0
GPIO_INT_POLARITY = 0x000000FF;

### 20.2.3	3. Configure Both-Edge Mode (Edge Mode Only)

// Enable both edges for pin 0
GPIO_INT_BOTH = 0x00000001;

### 20.2.4	4. Enable Interrupts

// Enable interrupts on pins 7:0
GPIO_INT_ENABLE = 0x000000FF;

## 20.3	Interrupt Configuration Table

| INT_TYPE | INT_POLARITY | INT_BOTH | Trigger |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Falling edge |
| 0 | 1 | 0 | Rising edge |
| 0 | X | 1 | Both edges |
| 1 | 0 | X | Active low |
| 1 | 1 | X | Active high |

## 20.4	Complete Setup Examples

### 20.4.1	Rising Edge Interrupt

// Configure pin 5 for rising edge interrupt
GPIO_INT_TYPE &= ~(1 << 5);      // Edge mode
GPIO_INT_POLARITY |= (1 << 5);   // Rising edge
GPIO_INT_BOTH &= ~(1 << 5);      // Single edge
GPIO_INT_ENABLE |= (1 << 5);     // Enable

### 20.4.2	Both-Edge Interrupt

// Configure pin 3 for both-edge interrupt
GPIO_INT_TYPE &= ~(1 << 3);      // Edge mode
GPIO_INT_BOTH |= (1 << 3);       // Both edges
GPIO_INT_ENABLE |= (1 << 3);     // Enable

### 20.4.3	Active-Low Level Interrupt

// Configure pin 7 for active-low level interrupt
GPIO_INT_TYPE |= (1 << 7);       // Level mode
GPIO_INT_POLARITY &= ~(1 << 7);  // Active low
GPIO_INT_ENABLE |= (1 << 7);     // Enable

## 20.5	Interrupt Handling

### 20.5.1	Check Interrupt Status

uint32_t status = GPIO_INT_STATUS;

### 20.5.2	Clear Interrupts (Write-1-to-Clear)

// Clear specific interrupt (pin 5)
GPIO_INT_STATUS = (1 << 5);

// Clear all pending interrupts
GPIO_INT_STATUS = 0xFFFFFFFF;

### 20.5.3	Complete ISR Example

void gpio_isr(void) {
    // Read status
    uint32_t status = GPIO_INT_STATUS;

    // Handle each pending interrupt
    for (int i = 0; i < 32; i++) {
        if (status & (1 << i)) {
            handle_gpio_event(i);
        }
    }

    // Clear handled interrupts
    GPIO_INT_STATUS = status;
}

## 20.6	Level-Sensitive Considerations

### 20.6.1	Avoid Interrupt Storm

For level-sensitive interrupts, the source must be cleared before the status:

void level_sensitive_isr(void) {
    uint32_t status = GPIO_INT_STATUS;

    // For level-sensitive pins, handle source first
    if (status & LEVEL_PIN_MASK) {
        clear_external_source();  // Clear what's driving pin
    }

    // Then clear status
    GPIO_INT_STATUS = status;
}

### 20.6.2	Masking During Handling

// Temporarily disable while handling
uint32_t saved_enable = GPIO_INT_ENABLE;
GPIO_INT_ENABLE = 0;  // Disable all

// Handle interrupt source
handle_interrupt();

// Re-enable
GPIO_INT_ENABLE = saved_enable;

Next: 03_examples.md - Programming Examples

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	APB GPIO - Programming Examples

## 21.1	LED Control

### 21.1.1	Simple LED Blink

#define LED_PIN  (1 << 0)

void led_init(void) {
    GPIO_CONTROL = 1;            // Enable GPIO
    GPIO_DIRECTION |= LED_PIN;   // Set as output
}

void led_on(void) {
    GPIO_OUTPUT |= LED_PIN;
}

void led_off(void) {
    GPIO_OUTPUT &= ~LED_PIN;
}

void led_toggle(void) {
    GPIO_OUTPUT ^= LED_PIN;
}

### 21.1.2	Multiple LED Control

#define LED_MASK  0x000000FF  // LEDs on pins 7:0

void leds_write(uint8_t pattern) {
    uint32_t output = GPIO_OUTPUT;
    output = (output & ~LED_MASK) | pattern;
    GPIO_OUTPUT = output;
}

## 21.2	Button Input

### 21.2.1	Simple Button Read

#define BUTTON_PIN  (1 << 8)

void button_init(void) {
    GPIO_CONTROL = 1;              // Enable GPIO
    GPIO_DIRECTION &= ~BUTTON_PIN; // Set as input
}

bool button_pressed(void) {
    return (GPIO_INPUT & BUTTON_PIN) != 0;
}

### 21.2.2	Button with Interrupt

#define BUTTON_PIN  (1 << 8)

volatile bool button_event = false;

void button_init_irq(void) {
    GPIO_CONTROL = 1;
    GPIO_DIRECTION &= ~BUTTON_PIN;

    // Configure falling edge interrupt (button press)
    GPIO_INT_TYPE &= ~BUTTON_PIN;      // Edge mode
    GPIO_INT_POLARITY &= ~BUTTON_PIN;  // Falling edge
    GPIO_INT_BOTH &= ~BUTTON_PIN;      // Single edge
    GPIO_INT_ENABLE |= BUTTON_PIN;     // Enable
}

void button_isr(void) {
    if (GPIO_INT_STATUS & BUTTON_PIN) {
        button_event = true;
        GPIO_INT_STATUS = BUTTON_PIN;  // Clear
    }
}

## 21.3	DIP Switch Reading

### 21.3.1	8-Bit Switch Input

#define SWITCH_MASK  0x00FF0000  // Switches on pins 23:16
#define SWITCH_SHIFT 16

void switch_init(void) {
    GPIO_CONTROL = 1;
    GPIO_DIRECTION &= ~SWITCH_MASK;  // All inputs
}

uint8_t switch_read(void) {
    return (GPIO_INPUT & SWITCH_MASK) >> SWITCH_SHIFT;
}

## 21.4	Parallel Data Interface

### 21.4.1	8-Bit Output Port

#define DATA_MASK   0x000000FF  // Data on pins 7:0
#define STROBE_PIN  (1 << 8)    // Strobe on pin 8

void data_port_init(void) {
    GPIO_CONTROL = 1;
    GPIO_DIRECTION |= (DATA_MASK | STROBE_PIN);
    GPIO_OUTPUT &= ~STROBE_PIN;  // Strobe low
}

void data_write(uint8_t data) {
    uint32_t output = GPIO_OUTPUT;
    output = (output & ~DATA_MASK) | data;
    GPIO_OUTPUT = output;

    // Generate strobe pulse
    GPIO_OUTPUT |= STROBE_PIN;
    // Small delay if needed
    GPIO_OUTPUT &= ~STROBE_PIN;
}

### 21.4.2	8-Bit Input Port with Ready

#define DATA_MASK   0x000000FF  // Data on pins 7:0
#define READY_PIN   (1 << 8)    // Ready on pin 8

void data_input_init(void) {
    GPIO_CONTROL = 1;
    GPIO_DIRECTION &= ~(DATA_MASK | READY_PIN);

    // Interrupt on ready rising edge
    GPIO_INT_TYPE &= ~READY_PIN;
    GPIO_INT_POLARITY |= READY_PIN;
    GPIO_INT_ENABLE |= READY_PIN;
}

uint8_t data_read(void) {
    return GPIO_INPUT & DATA_MASK;
}

## 21.5	PWM-Style Output

### 21.5.1	Bit-Banged PWM (Low Frequency)

#define PWM_PIN  (1 << 0)

void pwm_init(void) {
    GPIO_CONTROL = 1;
    GPIO_DIRECTION |= PWM_PIN;
}

// Call from timer interrupt
void pwm_update(uint8_t duty, uint8_t *counter) {
    (*counter)++;
    if (*counter >= 100) *counter = 0;

    if (*counter < duty) {
        GPIO_OUTPUT |= PWM_PIN;
    } else {
        GPIO_OUTPUT &= ~PWM_PIN;
    }
}

## 21.6	Wake-On-Change

### 21.6.1	Power Management Integration

#define WAKE_PINS  0x0000000F  // Wake sources on pins 3:0

void wake_setup(void) {
    // Configure both-edge interrupts for wake pins
    GPIO_INT_TYPE &= ~WAKE_PINS;   // Edge mode
    GPIO_INT_BOTH |= WAKE_PINS;    // Both edges
    GPIO_INT_ENABLE |= WAKE_PINS;  // Enable

    // Clear any pending before sleep
    GPIO_INT_STATUS = WAKE_PINS;
}

void enter_sleep(void) {
    wake_setup();
    // Platform-specific sleep entry
    __WFI();  // Wait for interrupt
}

Next: 04_software_notes.md - Software Considerations

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	APB GPIO - Software Considerations

## 22.1	Performance

### 22.1.1	Register Access Timing

| Operation | APB Cycles | Notes |
| --- | --- | --- |
| Read | 2 | Setup + access |
| Write | 2 | Setup + access |
| Read-Modify-Write | 4 | Read + write |

### 22.1.2	Optimizing Access

Batch operations when possible:

// Inefficient - 4 separate writes
GPIO_OUTPUT |= (1 << 0);
GPIO_OUTPUT |= (1 << 1);
GPIO_OUTPUT |= (1 << 2);
GPIO_OUTPUT |= (1 << 3);

// Efficient - single write
GPIO_OUTPUT |= 0x0000000F;

Cache register values:

// Inefficient - 4 reads + 4 writes
for (int i = 0; i < 4; i++) {
    GPIO_DIRECTION |= (1 << i);
}

// Efficient - 1 read + 1 write
uint32_t dir = GPIO_DIRECTION;
dir |= 0x0000000F;
GPIO_DIRECTION = dir;

## 22.2	Synchronization

### 22.2.1	Input Latency

GPIO inputs have inherent latency: - SYNC_STAGES clock cycles (default 2) - Plus software polling/interrupt overhead

Account for latency in timing-critical code.

### 22.2.2	Volatile Registers

Always declare GPIO registers as volatile:

#define GPIO_INPUT  (*(volatile uint32_t *)0xFEC0700C)

### 22.2.3	Multi-Core Considerations

If multiple cores access GPIO:

// Use atomic operations or locks
spin_lock(&gpio_lock);
uint32_t val = GPIO_OUTPUT;
val |= new_bits;
GPIO_OUTPUT = val;
spin_unlock(&gpio_lock);

## 22.3	Interrupt Best Practices

### 22.3.1	Clear Before Return

Always clear interrupt status before ISR return:

void gpio_isr(void) {
    uint32_t status = GPIO_INT_STATUS;
    // Handle interrupts
    GPIO_INT_STATUS = status;  // Must clear!
}

### 22.3.2	Avoid Spurious Interrupts

Disable interrupts during configuration:

void reconfigure_interrupt(int pin) {
    uint32_t mask = (1 << pin);

    // Disable first
    GPIO_INT_ENABLE &= ~mask;

    // Reconfigure
    // ...

    // Clear any pending
    GPIO_INT_STATUS = mask;

    // Re-enable
    GPIO_INT_ENABLE |= mask;
}

### 22.3.3	Level-Sensitive Caution

Level interrupts can cause interrupt storms:

void level_isr(void) {
    // WRONG - will re-trigger immediately
    GPIO_INT_STATUS = status;

    // RIGHT - handle source first
    clear_external_interrupt_source();
    GPIO_INT_STATUS = status;
}

## 22.4	Error Handling

### 22.4.1	No Hardware Errors

GPIO controller doesn’t generate errors: - All addresses valid - pslverr always 0

### 22.4.2	Software Validation

Validate configuration in software:

bool gpio_set_direction(uint32_t pin, bool output) {
    if (pin >= 32) return false;

    if (output) {
        GPIO_DIRECTION |= (1 << pin);
    } else {
        GPIO_DIRECTION &= ~(1 << pin);
    }
    return true;
}

## 22.5	Debug Tips

### 22.5.1	Read-Back Verification

void gpio_debug(void) {
    printf("CONTROL:   0x%08X\n", GPIO_CONTROL);
    printf("DIRECTION: 0x%08X\n", GPIO_DIRECTION);
    printf("OUTPUT:    0x%08X\n", GPIO_OUTPUT);
    printf("INPUT:     0x%08X\n", GPIO_INPUT);
    printf("INT_STAT:  0x%08X\n", GPIO_INT_STATUS);
}

### 22.5.2	Loopback Testing

Connect output to input for self-test:

bool gpio_loopback_test(int out_pin, int in_pin) {
    GPIO_DIRECTION |= (1 << out_pin);
    GPIO_DIRECTION &= ~(1 << in_pin);

    // Test high
    GPIO_OUTPUT |= (1 << out_pin);
    delay_us(10);  // Allow synchronization
    if (!(GPIO_INPUT & (1 << in_pin))) return false;

    // Test low
    GPIO_OUTPUT &= ~(1 << out_pin);
    delay_us(10);
    if (GPIO_INPUT & (1 << in_pin)) return false;

    return true;
}

Back to: 00_overview.md - Programming Model Overview

Next Chapter: Chapter 5: Registers

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	APB GPIO - Register Map

## 23.1	Register Summary

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x000 | GPIO_CONTROL | RW | 0x00000000 | Global control |
| 0x004 | GPIO_DIRECTION | RW | 0x00000000 | Pin direction |
| 0x008 | GPIO_OUTPUT | RW | 0x00000000 | Output data |
| 0x00C | GPIO_INPUT | RO | - | Input data |
| 0x010 | GPIO_INT_ENABLE | RW | 0x00000000 | Interrupt enable |
| 0x014 | GPIO_INT_TYPE | RW | 0x00000000 | Interrupt type |
| 0x018 | GPIO_INT_POLARITY | RW | 0x00000000 | Interrupt polarity |
| 0x01C | GPIO_INT_BOTH | RW | 0x00000000 | Both-edge enable |
| 0x020 | GPIO_INT_STATUS | W1C | 0x00000000 | Interrupt status |

## 23.2	GPIO_CONTROL (0x000)

Global control register.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:1 | Reserved | RO | 0 | Reserved |
| 0 | ENABLE | RW | 0 | GPIO enable (1=enabled) |

## 23.3	GPIO_DIRECTION (0x004)

Pin direction control. Each bit controls one GPIO pin.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | DIR | RW | 0 | Direction per pin (0=input, 1=output) |

## 23.4	GPIO_OUTPUT (0x008)

Output data register. Values driven when pin configured as output.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | DATA | RW | 0 | Output values per pin |

## 23.5	GPIO_INPUT (0x00C)

Input data register. Reflects synchronized external pin values.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | DATA | RO | - | Input values per pin |

Note: Value depends on external signals, not reset.

## 23.6	GPIO_INT_ENABLE (0x010)

Interrupt enable register. Controls which pins can generate interrupts.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | IE | RW | 0 | Interrupt enable per pin (1=enabled) |

## 23.7	GPIO_INT_TYPE (0x014)

Interrupt type select. Chooses edge or level sensitivity.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | TYPE | RW | 0 | Type per pin (0=edge, 1=level) |

## 23.8	GPIO_INT_POLARITY (0x018)

Interrupt polarity select.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | POL | RW | 0 | Polarity per pin |

For edge mode: 0=falling, 1=rising For level mode: 0=active-low, 1=active-high

## 23.9	GPIO_INT_BOTH (0x01C)

Both-edge interrupt enable. Only applicable in edge mode.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | BOTH | RW | 0 | Both edges per pin (1=both edges) |

When set, GPIO_INT_POLARITY is ignored for that pin.

## 23.10	GPIO_INT_STATUS (0x020)

Interrupt status register. Shows pending interrupts.

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | STATUS | W1C | 0 | Interrupt pending per pin |

Access: Read returns current status. Write 1 clears the bit.

## 23.11	Address Calculation

For system address:

Register_Address = BASE_ADDR + WINDOW_OFFSET + Register_Offset

Where:
  BASE_ADDR = 0xFEC00000 (RLB base)
  WINDOW_OFFSET = 0x7000 (GPIO window)
  Register_Offset = value from table above

Example:
  GPIO_INPUT = 0xFEC00000 + 0x7000 + 0x00C = 0xFEC0700C

Back to: GPIO Specification Index