RTL Design Sherpa

KESTREL-RV32I Micro-Architecture Specification 0.1

October 7, 2026

Table of Contents

1 Kestrel Mas Index	13

2 Document Information	14

2.1 KESTREL-RV32I Micro-Architecture Specification	14

2.2 Document Purpose	15

2.3 References	16

2.4 Terminology	17

2.5 Revision History	18

3 Design Philosophy and Machine Organization	20

3.1 Design Philosophy	20

3.2 The Machine in One Picture	21

3.2.1 Figure 1.1: kestrel datapath block diagram	21

3.3 Module Inventory	22

3.4 State Inventory	24

3.5 Key Interfaces at a Glance	25

4 kestrel_core (Top)	26

4.1 Purpose	26

4.2 Interface	27

4.3 Internal Structure	28

4.3.1 Fetch	28

4.3.2 Source Muxes	28

4.3.3 Writeback Priority Mux	28

4.3.4 Next-PC Mux	29

4.3.5 Branch Comparator (the Ruling)	29

4.3.6 Load/Store Rotation Network	29

4.3.7 Halt and PC Registers	30

5 kestrel_decode	31

5.1 Purpose	31

5.2 Keying and Defaults	32

5.3 The Decode Table	33

5.3.1 Arithmetic and upper-immediate rows	33

5.3.2 Loads, stores, branches, jumps	34

5.3.3 System, fence, and the default	35

5.4 What the Table Teaches	37

6 kestrel_imm_gen	38

6.1 Purpose	38

6.2 Interface	39

6.3 Internal Structure	40

7 kestrel_alu	42

7.1 Purpose	42

7.2 Interface	43

7.3 Operations	44

7.4 The Flags Ruling	45

8 kestrel_regfile	46

8.1 Purpose	46

8.2 Interface	47

8.3 Internal Structure and Rules	48

9 kestrel_mem_loader	49

9.1 Purpose	49

9.1.1 Figure 2.1: Loader composition	49

9.2 Memory Arrays	50

9.3 CTRL / Run Register and Reset Sequencing	51

9.4 Write Side: Leaf Slave plus Two Holding Bits	52

9.5 Read Side: Combinational Beat through the AR Skid	53

9.6 Address Map, as Implemented	54

9.7 Debug Taps	55

10 kestrel_mem_bram	56

10.1 Purpose	56

10.2 The Array	57

10.3 The Sequencer (Port A)	58

10.4 AXIL Side (Port B)	59

10.5 Registers (addr[17] = 1)	60

10.6 Verification	61

11 Package Types and Top-Level Wiring	62

11.1 kestrel_pkg	62

11.1.1 ALU operation enum (alu_op_e, 4 bits)	62

11.1.2 Immediate-format enum (imm_sel_e, 3 bits)	62

11.1.3 Halt-cause encodings (localparam logic [3:0])	62

11.2 Top-Level Wiring (kestrel_core)	64

11.3 Reset Discipline	66

12 The Single Cycle, Step by Step	67

12.1 The Clock in One Sentence	67

12.2 1. Fetch and Decode	68

12.3 2. Operands and Immediates	69

12.4 3. Execute	70

12.5 4. Memory Access	71

12.6 5. Writeback	72

12.7 6. Next-PC Selection and the Edge	73

12.8 The Basic Cycle on the Pins	74

12.8.1 Waveform 4.1: Basic single-cycle execution — an aligned load followed by an ALU op	74

12.9 Branch and Jump Resolution	75

13 Cross-Word Misaligned Retry	76

13.1 When the Retry Fires	76

13.2 Beat-by-Beat Behavior	77

13.3 The Waveform	78

13.3.1 Waveform 4.2: Cross-word store retry — SH at byte offset 3	78

13.4 Worked Example: a Halfword Across the Boundary	79

13.5 Why Hardware, Why Two Cycles	80

14 Halt, IALIGN, and the System Layer	81

14.1 The Halt Holding Register	81

14.2 The IALIGN Halt (Cause 3)	82

14.3 The System Layer: Stubs, NOPs, and Halts	83

14.3.1 FENCE and FENCE.I retire as NOPs	83

14.3.2 MRET falls through	83

14.3.3 The CSR class retires as a bounded stub	83

14.4 Halt Timing Summary	84

15 RVFI Aggregation	85

15.1 Retirement as the Observation Channel	85

15.2 The Field Table	86

15.3 The Three Gating Rules	87

15.4 Memory-Field Packing	88

15.5 Why This Pays	89

16 Verification Hooks and Observability	90

16.1 The Three Testplan-Tracked Hooks	90

16.2 Supporting Hooks	92

16.3 The Consumer Stack	93

16.4 Cross-Reference to the Testplan Set	94

17 Parameters, Defines, and Filelists	95

17.1 Parameters	95

17.1.1 kestrel_core	95

17.1.2 kestrel_mem_loader	95

17.2 Defines and Includes	96

17.3 Filelists	97

17.4 Regenerating This Document	98

18 KESTREL-RV32I Hardware Architecture Specification Index	99

18.1 Document Organization	100

18.1.1 Front Matter	100

18.1.2 Chapter 1: Introduction	100

18.1.3 Chapter 2: System Overview	100

18.1.4 Chapter 3: Architecture	100

18.1.5 Chapter 4: Interfaces	100

18.1.6 Chapter 5: Performance	100

18.1.7 Chapter 6: Integration	100

18.2 Related Documentation	101

19 Simplified RV32I: kestrel	102

19.1 Overview	103

19.2 Document Structure	104

19.2.1 Chapter 1: Introduction	104

19.2.2 Chapter 2: The RV32I ISA Quick Reference	104

19.2.3 Chapter 3: The kestrel Datapath	104

19.2.4 Chapter 4: The Control Truth Table	104

19.2.5 Chapter 5: The Memory Contract	104

19.2.6 Chapter 6: Verification	105

19.2.7 Chapter 7: What kestrel Costs	105

19.2.8 Version History	105

20 RISC-V Falcon Suite	106

20.1 The ladder	107

20.1.1 Moved: hive-serv	108

20.1.2 Tetrad engine: gatling-rv32itetrad	108

20.2 Goals per rung	109

20.3 Packaging (per rung, the MC-line treatment)	110

20.4 References and layout	111

21 RISC-V Formal Interface (RVFI)	112

21.1 RVFI Specification	112

21.1.1 Instruction Metadata	112

21.1.2 Integer Register Read/Write	113

21.1.3 Program Counter	113

21.1.4 Memory Access	114

21.1.5 Alternative Arithmetic Operations	114

21.1.6 Control and Status Registers (CSRs)	115

21.1.7 Handling of Speculative Execution	115

21.2 RVFI TODOs and Requests for Comments	117

21.2.1 Support for fused instructions	117

21.2.2 Modelling of Floating-Point State	117

21.2.3 Modelling of Virtual Memory	117

21.2.4 Modelling of Atomic Memory Operations	119

21.2.5 Skipping instructions	119

22 kestrel-rv32i Testplans	121

22.1 Inventory	122

22.2 What each plan covers	123

22.3 JUnit naming convention (observed)	124

22.4 Rollup	125

22.5 Running the tests	126

22.6 Functional coverage (CORE-18)	127

List of Figures

Figure 1.1: kestrel datapath block diagram	21

Figure 2.1: Loader composition	49

List of Tables

Table 1: Reference Documents	16

Table 2: Terminology and Definitions	17

Table 3: Document Revision History	18

Table 4: Module inventory	22

Table 5: The complete core state inventory, register file aside	24

Table 6: Internal interface summary (full wiring in Chapter 3)	25

Table 7: Writeback priority mux	28

Table 8: The thirteen decode outputs	31

Table 9: Decode rows — OP, OP-IMM, LUI, AUIPC	33

Table 10: Decode rows — LOAD, STORE, BRANCH, JAL, JALR	34

Table 11: Decode rows — SYSTEM, MISC-MEM, and the illegal default	35

Table 12: kestrel_imm_gen interface	39

Table 13: Immediate assembly per format	40

Table 14: kestrel_alu interface	43

Table 15: ALU operations (shamt = b[4:0])	44

Table 16: kestrel_regfile interface	47

Table 17: Loader map implementation facts	54

Table 18: Loader and block-RAM wrapper compared	56

Table 19: The fetch / execute / load sequencer	58

Table 20: kestrel_mem_bram registers	60

Table 21: alu_op_e enumerators	62

Table 22: imm_sel_e enumerators	62

Table 23: HALT_* encodings — the single source of truth shared by decode and core	62

Table 24: kestrel_core internal wiring map	64

Table 25: Control-transfer resolution	75

Table 26: The two beats of a cross-word access (o = addr[1:0])	77

Table 27: Halt timing at the boundary	84

Table 28: RVFI signals and their kestrel aggregation	86

Table 29: The three testplan-tracked hooks	90

Table 30: Supporting loader hooks	92

Table 31: Verification consumer stack	93

Table 32: kestrel_core parameters	95

Table 33: kestrel_mem_loader parameters	95

Table 34: Shared defines	96

Table 35: Filelists (KESTREL_ROOT = projects/components/riscv-ip/kestrel-rv32i; REPO_ROOT = RTLDesignSherpa checkout root)	97

Table 36: Version history	105

List of Waveforms

Waveform 4.1: Basic single-cycle execution — an aligned load followed by an ALU op	74

Waveform 4.2: Cross-word store retry — SH at byte offset 3	78

# 1	Kestrel Mas Index

Generated: 2026-10-20

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	KESTREL-RV32I Micro-Architecture Specification

Document Number: KESTREL-MAS-001 Version: 0.4 Status: Draft Classification: Open Source - MIT License

## 2.2	Document Purpose

This Micro-Architecture Specification (MAS) describes how the kestrel-rv32i core is built: one section per functional block covering its purpose, interface, internal structure, and key logic; the internal interfaces between blocks; the cycle-level behavior including the misaligned retry and halt timing; the verification hooks the design exposes on purpose; and the configuration surface. It is written for implementers and maintainers of the RTL, and for reviewers who need to check the design against its specification.

The external contract — ports as an integrator wires them, memory and halt behavior, loader programming — is the companion Hardware Architecture Specification (HAS). This document assumes it.

Target Audience: - RTL maintainers extending or fixing the core - Design reviewers checking implementation against intent - Verification engineers wiring checkers to the internal observability points - Students reading the RTL top to bottom

Companion Documents: - KESTREL-RV32I Hardware Architecture Specification (HAS) - External architecture and integration contract - Simplified RV32I: kestrel - Tutorial-study book covering the same ground as pedagogy

## 2.3	References

Reference Documents

| ID | Document | Description |
| --- | --- | --- |
| [REF-1] | KESTREL HAS v0.1 | Hardware Architecture Specification (companion) |
| [REF-2] | RISC-V Instruction Set Manual, Volume I: Unprivileged Architecture (projects/components/riscv-ip/references/riscv-spec.pdf) | ISA authority; cited by chapter/section as “unpriv §N” |
| [REF-3] | riscv-formal rvfi.md (vendor/riscv-formal/docs/rvfi.md) | RVFI retire-channel field semantics |
| [REF-4] | KESTREL testplans (dv/testplans/*.yaml) | V&V scenarios and coverage cross-reference |
| [REF-5] | falcon-suite RTL style brief | Datapath-as-truth-tables / FSMs-only-where-unavoidable discipline |

## 2.4	Terminology

Terminology and Definitions

| Term | Definition |
| --- | --- |
| Beat | One memory access cycle; a cross-word access uses two beats |
| Control bundle | The thirteen-field decode output steering the datapath each cycle |
| FUB | Functional Unit Block - leaf RTL module |
| Halt holding register | halt_q — one bit that latches any halt cause and holds it |
| Retry bit | ls_retry — one bit of control state sequencing the cross-word retry |
| Rotator | The byte-lane shift network implementing rotated strobes/data for L/S |
| Trap beat | The single RVFI beat of a halting instruction (rvfi_trap=1) |

## 2.5	Revision History

Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 2026-10-07 | seang | Initial MAS release; documents the as-built RTL (Tasks 1-11) including the Task-9 IALIGN halt and the Task-11 AXIL board loader |
| 0.2 | 2026-10-20 | seang | Block diagrams redrawn to the repo element key (v3): class fills, double-bordered registered blocks, green return paths and a generated key under each figure. No technical content changed. |
| 0.3 | 2026-10-20 | seang | Section (h2) headings now open a new page, the pipeline default. No technical content changed. |
| 0.4 | 2026-10-20 | seang | mem_wait core input and kestrel_mem_bram (block RAM, registered read, CPI 2/3 for loads; registers for run/stop, status, cycles, instret, gp): new Ch. 2.7; core chapter notes the wait. rv32ui 42/42 through it with spike lockstep; riscv-formal 42/42 with a free mem_wait. |

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Design Philosophy and Machine Organization

## 3.1	Design Philosophy

Three commitments shape the microarchitecture, and every block chapter shows them landing:

- The datapath is FSM-free. Classic processor control walks fetch/decode/execute/memory/writeback as states; in kestrel those phases are spatial, not temporal. Fetch is a wire off the PC register; decode is a truth table; execute is an ALU; memory access is a combinational port; writeback is a mux feeding a clocked register file. There is nothing to sequence, so there is no FSM — and no microcode, no control ROM, no phase counter. The two places a naive design would reach for an FSM — halting and the misaligned retry — are single holding flops with plain set/clear conditions.

- Control is an explicit truth table. Decode is one unique casez keyed on {opcode, funct3, funct7} producing a thirteen-field control bundle. The same word always produces the same bundle; the table is small enough to read top to bottom and is transcribed in the kestrel_decode chapter.

- RVFI is the observation channel. The retire port is not debug instrumentation bolted on afterward; it is the defined observation channel, present from the first vertical slice, and its aggregation rules are stated next to the assignments in the RTL and again in Chapter 4.

The discipline the suite’s style brief imposes — datapath logic is truth tables and muxes; FSMs only where control is unavoidable — is what makes this core reviewable in one sitting and provable at shallow model-checking depth (riscv-formal runs at depth 6 and covers reset plus a two-cycle memory access with headroom).

## 3.2	The Machine in One Picture

### 3.2.1	Figure 1.1: kestrel datapath block diagram

kestrel datapath block diagram

The diagram is drawn to match the RTL: the blocks are the module instances in kestrel_core.sv, and every named wire is a real signal in the source. State elements are the PC register, the register file, and three small holding registers described below; everything else is combinational.

## 3.3	Module Inventory

Module inventory

| Module | File | Role | State |
| --- | --- | --- | --- |
| kestrel_pkg | rtl/includes/kestrel_pkg.sv | Shared enums (alu_op_e, imm_sel_e) and the halt-cause encodings | none |
| kestrel_decode | rtl/fub/kestrel_decode.sv | Combinational truth table from {opcode, funct3, funct7} to the control bundle | none |
| kestrel_imm_gen | rtl/fub/kestrel_imm_gen.sv | Builds the 32-bit immediate for the five formats | none |
| kestrel_alu | rtl/fub/kestrel_alu.sv | Ten operations: ADD SUB AND OR XOR SLL SRL SRA SLT SLTU | none |
| kestrel_regfile | rtl/fub/kestrel_regfile.sv | 32 x 32-bit registers, two combinational read ports, one synchronous write, x0 discarded | 32 words |
| kestrel_core | rtl/top/kestrel_core.sv | Wires the leaves together: source muxes, branch comparator, next-PC mux, writeback mux, L/S rotation, halt, RVFI | 5 elements below |
| kestrel_mem_loader | rtl/fub/kestrel_mem_loader.sv | Optional board glue: memories + AXIL slave + run control (see its chapter) | run bit, holding regs |

## 3.4	State Inventory

Everything kestrel remembers, outside the register file and (when integrated) the memories:

The complete core state inventory, register file aside

| Element | Width | Purpose | Next-state equation |
| --- | --- | --- | --- |
| pc | 32 | Program counter; resets to RESET_ADDR | pc <= halt ? pc : (ls_first ? pc : next_pc) |
| retire_count | 64 | RVFI retirement order: 0, 1, 2, … | +1 when rvfi_valid |
| halt_q | 1 | Halt holding register; latches any halt cause | halt_q <= halt_q \¦ halt_now |
| ls_retry | 1 | Cross-word L/S retry-cycle flag | ls_retry <= ls_first |
| ls_rdata_lo | 32 | First-word read data captured for the retry-cycle merge | captures ls_rdata_shifted when ls_first |

Five state elements with one-line next-state equations are reviewable in minutes. The retry bit and the halt bit are control state only; ls_rdata_lo is the only place data is captured mid-instruction — a pipeline register in the most minimal sense.

## 3.5	Key Interfaces at a Glance

Internal interface summary (full wiring in Chapter 3)

| Interface | Width | Producer → Consumer |
| --- | --- | --- |
| Control bundle | 13 fields | kestrel_decode → muxes, regfile, L/S datapath |
| imm | 32 | kestrel_imm_gen → source-B mux, writeback (LUI) |
| alu_y | 32 | kestrel_alu → dmem address, writeback |
| Branch flags | eq/lt/ltu | dedicated comparator → branch-cond mux → next-PC mux |
| next_pc | 32 | next-PC mux → PC register and rvfi_pc_wdata |
| rd_wdata | 32 | writeback mux → register file |
| RVFI bundle | 17 signals | aggregation assigns → top-level ports |

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	kestrel_core (Top)

## 4.1	Purpose

kestrel_core wires the four datapath leaves together and owns everything that is not a leaf: source muxes, the dedicated branch comparator, the next-PC mux, the writeback priority mux, the load/store rotation network with its retry state, the halt holding register, and the RVFI aggregation assigns. It is the only module in the core with architectural state besides the register file.

## 4.2	Interface

Ports are specified exhaustively in the HAS (Chapter 4 there). Internally the top exposes no hierarchy below the four leaf instances (u_decode, u_imm_gen, u_regfile, u_alu) plus its own always-blocks; the loader, when used, sits outside this module.

## 4.3	Internal Structure

### 4.3.1	Fetch

imem_addr = pc and insn = imem_rdata — fetch is a wire. The instruction is expected combinationally (the memory contract, HAS Ch. 4), or in the first cycle mem_wait is low: while the memory system holds mem_wait high, every state update in this chapter (PC, register write, halt latch, retry bits, retire counter) is suppressed, rvfi_valid is low and the combinational halt is masked (Ch. 2.7); opcode = insn[6:0] and funct3 = insn[14:12] fan out to decode and the L/S size/sign logic.

### 4.3.2	Source Muxes

alu_src_a = alu_src_a_pc ? pc : rs1_data selects PC (AUIPC, branch targets) or rs1. alu_src_b = alu_src_b_imm ? imm : rs2_data selects the immediate or rs2. Register-file read addresses come straight from the instruction: rs1 = insn[19:15], rs2 = insn[24:20], rd = insn[11:7].

### 4.3.3	Writeback Priority Mux

rd_wdata is selected by a five-row priority mux:

Writeback priority mux

| Priority | Condition | Value |
| --- | --- | --- |
| 1 | csr_stub | 32'd0 — the CSR stub’s zero writeback |
| 2 | opcode == OPCODE_LUI | imm — LUI never passes through the ALU |
| 3 | ls_load | ls_load_data — assembled, size/sign-corrected read data |
| 4 | jump \¦\¦ jalr | pc + 4 — the link value |
| 5 | default | alu_y |

The write enable is rd_wen_eff = rd_wen & ~ls_first & ~halt: decode’s rd_wen qualified by the cross-word retry’s first cycle (a partial word must not commit early) and by halt (the cause-3 halt retires JAL/JALR encodings whose decode rd_wen is set, so the gate is halt itself, not decode). The register file independently discards writes to x0.

### 4.3.4	Next-PC Mux

Four rows, in priority order: JAL/JALR (jump || jalr) → jalr ? (rs1_data + imm) & 32'hFFFF_FFFE : pc + imm; taken branch (branch_taken) → pc + imm; default → pc + 4. JALR_ALIGN_MASK = 32'hFFFF_FFFE clears target bit 0 per unpriv §2.1.5.1. In a single-cycle machine the branch decision and the pc + imm target settle in parallel, so taken and not-taken branches cost the same cycle.

### 4.3.5	Branch Comparator (the Ruling)

The ALU exports eq/lt/ltu flags, and the core’s ALU instance deliberately leaves them unconnected (.eq (), .lt (), .ltu ()). The reason is a reviewed design ruling: for branches the ALU’s left input is the PC (it is simultaneously computing pc + imm for the next-PC mux), so its internal flags would compare pc against imm, not rs1 against rs2 — silently wrong branch decisions. Branch conditions therefore come from a dedicated comparator on the register data:

- cmp_eq = (rs1_data == rs2_data)

- cmp_lt = ($signed(rs1_data) < $signed(rs2_data))

- cmp_ltu = (rs1_data < rs2_data)

- branch_cond selected from cmp_* by funct3 in a unique case (BEQ→eq, BNE→~eq, BLT→lt, BGE→~lt, BLTU→ltu, BGEU→~ltu, default→0)

- branch_taken = branch & branch_cond

In a single-cycle machine this parallel evaluation is free: both the ALU result and the comparator decision are ready when the next PC is chosen.

### 4.3.6	Load/Store Rotation Network

The L/S datapath implements the hardware-misaligned policy (unpriv §2.1.6 permits hardware handling; the plan records it as a decision). For an access at byte offset o = alu_y[1:0] with size s bytes:

- Size decode: dmem_size (00 byte, 01 half, 10 word) → ls_size_bytes (1/2/4), ls_size_mask (0001/0011/1111), ls_data_mask (0xFF/0xFFFF/0xFFFFFFFF).

- Split arithmetic: ls_tail_size = 4 - o; ls_head_size = s - ls_tail_size; ls_crossing = ls_active & (o + s > 4); ls_first = ls_crossing & ~ls_retry; ls_second = ls_retry; ls_single = ls_active & ~ls_crossing.

- Address: dmem_addr = ls_second ? {alu_y[31:2] + 1, 2'b00} : {alu_y[31:2], 2'b00} — beat 2 issues at the next word.

- Store rotation (the store rotator): beat 1 writes rs2_data << (8*o) with strobe ls_size_mask << o; beat 2 writes rs2_data >> (8*ls_tail_size) with strobe ls_head_strobe (a head-size decode: 0/1/2/3/4 bytes → 0000/0001/0011/0111/1111).

- Load rotation (the load rotator): beat 1 captures ls_rdata_lo <= dmem_rdata >> (8*o); beat 2 assembles (dmem_rdata & ls_head_mask) << (8*ls_tail_size) | (ls_rdata_lo & ls_tail_mask). Single-beat loads use (dmem_rdata >> (8*o)) & ls_data_mask.

- Size/sign select: ls_load_data from ls_raw_rdata per funct3: LB sign-extends from bit 7, LBU zero-extends, LH/LHU from bit 15, LW passes through.

dmem_wstrb = ls_store ? (ls_second ? ls_head_strobe : ls_size_mask << o) : 4'b0000. Bytes that would rotate past bit 31 on beat 1 are dropped by construction — beat 2 carries them. Full cycle-level behavior and the waveform are in Chapter 4.

### 4.3.7	Halt and PC Registers

The halt path (halt_q, misalign_target, halt_cause_eff, halt_now) and the PC register (pc <= halt ? pc : (ls_first ? pc : next_pc)) with the retry/merge/counter flops complete the state. Their behavior is specified in Chapter 4.

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	kestrel_decode

## 5.1	Purpose

kestrel_decode is a pure function from the 32-bit instruction word to the control bundle. It holds no state and no clock; the same word always produces the same bundle. The bundle’s thirteen fields:

The thirteen decode outputs

| Field | Type | Meaning when set / value |
| --- | --- | --- |
| alu_op | alu_op_e | ALU operation: ADD SUB AND OR XOR SLL SRL SRA SLT SLTU |
| imm_sel | imm_sel_e | Immediate format to build: I S B U J |
| alu_src_a_pc | bit | ALU left input is the PC (else rs1) |
| alu_src_b_imm | bit | ALU right input is the immediate (else rs2) |
| rd_wen | bit | The instruction writes rd this cycle |
| dmem_req | bit | A data-memory access is in flight |
| dmem_we | bit | The access is a store (else a load) |
| dmem_size | 2 bits | Access size: 00 byte, 01 halfword, 10 word |
| branch | bit | Instruction is a conditional branch |
| jump | bit | Instruction is JAL |
| jalr | bit | Instruction is JALR |
| halt_cause | 4 bits | Halt encoding from kestrel_pkg; HALT_NONE when running |
| csr_stub | bit | SYSTEM CSR class: writeback a hard zero |

## 5.2	Keying and Defaults

The truth table is a single unique casez keyed on {opcode, funct3, funct7} — a 17-bit key. Every RV32I instruction is distinguished by those three fields alone; the SYSTEM privileged row re-cases on insn[31:20] (imm12) to split ECALL/EBREAK/MRET from illegal encodings. Fields that carry immediate bits are keyed as don’t-cares (F7_DC = 7'b???????) for OP-IMM arithmetic, loads, stores, and branches; the shift rows key funct7 fully, which is what makes mis-encoded shifts illegal rather than silently executed.

Before the case, decode assigns a default bundle: alu_op = ADD, imm_sel = I, every control bit clear, halt_cause = HALT_NONE. Each row then sets only what it needs. The default: row changes exactly one field, halt_cause = HALT_ILL — the entire illegal-instruction mechanism. Every encoding not claimed by a row halts, loudly, instead of executing as something plausible.

## 5.3	The Decode Table

Transcribed row for row from rtl/fub/kestrel_decode.sv. The key column is the 17-bit casez key written as opcode.funct3.funct7; any marks a don’t-care key field. Bundle columns show only non-default values; alu is alu_op, imm is imm_sel, aPC is alu_src_a_pc, bImm is alu_src_b_imm, rd is rd_wen; dmem reports req, +we, and size in bytes (B/H/W); — means the default.

### 5.3.1	Arithmetic and upper-immediate rows

The ALU computes datapath arithmetic here; for LUI the writeback mux bypasses the ALU and takes the immediate directly.

Decode rows — OP, OP-IMM, LUI, AUIPC

| opcode.funct3.funct7 (17-bit casez key) | Row | alu | imm | aPC | bImm | rd |
| --- | --- | --- | --- | --- | --- | --- |
| 0110011.000.0000000 | ADD | ADD | — | — | — | 1 |
| 0110011.000.0100000 | SUB | SUB | — | — | — | 1 |
| 0110011.001.0000000 | SLL | SLL | — | — | — | 1 |
| 0110011.010.0000000 | SLT | SLT | — | — | — | 1 |
| 0110011.011.0000000 | SLTU | SLTU | — | — | — | 1 |
| 0110011.100.0000000 | XOR | XOR | — | — | — | 1 |
| 0110011.101.0000000 | SRL | SRL | — | — | — | 1 |
| 0110011.101.0100000 | SRA | SRA | — | — | — | 1 |
| 0110011.110.0000000 | OR | OR | — | — | — | 1 |
| 0110011.111.0000000 | AND | AND | — | — | — | 1 |
| 0010011.000.any | ADDI | ADD | I | — | 1 | 1 |
| 0010011.010.any | SLTI | SLT | I | — | 1 | 1 |
| 0010011.011.any | SLTIU | SLTU | I | — | 1 | 1 |
| 0010011.100.any | XORI | XOR | I | — | 1 | 1 |
| 0010011.110.any | ORI | OR | I | — | 1 | 1 |
| 0010011.111.any | ANDI | AND | I | — | 1 | 1 |
| 0010011.001.0000000 | SLLI | SLL | I | — | 1 | 1 |
| 0010011.101.0000000 | SRLI | SRL | I | — | 1 | 1 |
| 0010011.101.0100000 | SRAI | SRA | I | — | 1 | 1 |
| 0110111.any.any | LUI | ADD | U | — | 1 | 1 |
| 0010111.any.any | AUIPC | ADD | U | 1 | 1 | 1 |

### 5.3.2	Loads, stores, branches, jumps

Every branch/jump row uses alu = ADD so the ALU produces pc + imm for the next-PC mux while the dedicated comparator decides taken-ness in parallel. JALR is the one jump whose ALU left input is rs1, not PC.

Decode rows — LOAD, STORE, BRANCH, JAL, JALR

| opcode.funct3.funct7 (17-bit casez key) | Row | imm | aPC | bImm | dmem | ctl |
| --- | --- | --- | --- | --- | --- | --- |
| 0000011.000.any | LB | I | — | 1 | req, B | — |
| 0000011.001.any | LH | I | — | 1 | req, H | — |
| 0000011.010.any | LW | I | — | 1 | req, W | — |
| 0000011.100.any | LBU | I | — | 1 | req, B | — |
| 0000011.101.any | LHU | I | — | 1 | req, H | — |
| 0100011.000.any | SB | S | — | 1 | req+we, B | — |
| 0100011.001.any | SH | S | — | 1 | req+we, H | — |
| 0100011.010.any | SW | S | — | 1 | req+we, W | — |
| 1100011.000.any | BEQ | B | 1 | 1 | — | branch |
| 1100011.001.any | BNE | B | 1 | 1 | — | branch |
| 1100011.100.any | BLT | B | 1 | 1 | — | branch |
| 1100011.101.any | BGE | B | 1 | 1 | — | branch |
| 1100011.110.any | BLTU | B | 1 | 1 | — | branch |
| 1100011.111.any | BGEU | B | 1 | 1 | — | branch |
| 1101111.any.any | JAL | J | 1 | 1 | — | jump |
| 1100111.000.any | JALR | I | — | 1 | — | jalr |

### 5.3.3	System, fence, and the default

Decode rows — SYSTEM, MISC-MEM, and the illegal default

| opcode | funct3 | imm12 | Row | Behavior (how the row retires) | halt_cause |
| --- | --- | --- | --- | --- | --- |
| 1110011 | 000 | 000 | ECALL | halt; RVFI trap beat | 4'h1 |
| 1110011 | 000 | 001 | EBREAK | halt; RVFI trap beat | 4'h2 |
| 1110011 | 000 | 302 | MRET | retire as NOP, pc+4 fall-through | — |
| 1110011 | 000 | other | — | illegal | 4'hF |
| 1110011 | 001/010/011/101/110/111 | any | CSRRW/S/C + imm forms | csr_stub: zero readback, writes dropped | — |
| 1110011 | 100 | any | — | reserved, illegal | 4'hF |
| 0001111 | 000 | any | FENCE | retire as NOP | — |
| 0001111 | 001 | any | FENCE.I | retire as NOP | — |
| 0001111 | 010-111 | any | — | reserved, illegal | 4'hF |
| any unclaimed | — | — | default | illegal instruction | 4'hF |

## 5.4	What the Table Teaches

Three design rulings visible in the table deserve explicit mention:

- Reserved encodings halt rather than guess. Branch funct3 010/011, load/store funct3 011 and 110/111, MISC-MEM funct3 2-7, SYSTEM funct3 100, shift funct7 mismatches: all illegal (unpriv §§2.1.4, 2.1.6 reserve these). A reserved encoding today may become an instruction tomorrow; silently executing it is how compatibility bugs are born.

- The CSR class is one stub row-set, not six special cases. All six Zicsr forms share csr_stub = 1; rd_wen = 1 and nothing else; the writeback mux owns the zero. The table stays small because the stub is honest about there being nothing to decode.

- Cause 3 appears nowhere in this table. HALT_IALIGN is raised only in kestrel_core, which can see the resolved next PC and the branch decision. Decode provably cannot know — a branch’s target alignment depends on the register operands it never evaluates.

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	kestrel_imm_gen

## 6.1	Purpose

kestrel_imm_gen builds the 32-bit sign- or zero-extended immediate for the five RV32I instruction formats, selected by imm_sel from the control bundle. It is pure combinational logic — a five-row case on bit fields of insn.

## 6.2	Interface

kestrel_imm_gen interface

| Port | Direction | Width | Description |
| --- | --- | --- | --- |
| insn | Input | 32 | The instruction word (only immediate-bearing bits are used) |
| sel | Input | imm_sel_e | Format select from decode: I S B U J |
| imm | Output | 32 | The assembled immediate |

## 6.3	Internal Structure

The immediate assembly follows the ISA’s format definitions exactly (unpriv §2.1):

Immediate assembly per format

| sel | Assembly | Formats using it |
| --- | --- | --- |
| I | {{20{insn[31]}}, insn[31:20]} | ADDI/SLTI/SLTIU/XORI/ORI/ANDI, loads, JALR |
| S | {{20{insn[31]}}, insn[31:25], insn[11:7]} | stores |
| B | {{20{insn[31]}}, insn[7], insn[30:25], insn[11:8], 1'b0} | branches |
| U | {insn[31:12], 12'b0} | LUI, AUIPC |
| J | {{12{insn[31]}}, insn[19:12], insn[20], insn[30:21], 1'b0} | JAL |

Notes for maintainers:

- B and J immediates append the implicit low zero (unpriv §2.1) in logic; no consumer may re-shift.

- U is zero-extended by construction (no sign bits) — LUI writes it directly, AUIPC adds it to the PC.

- SLTIU’s famously surprising semantics (compare against the sign-extended immediate interpreted unsigned, unpriv §2.1.4.1) require no special handling here: the immediate is the same sign-extended value every I-format consumer gets; the ALU’s unsigned less-than on it preserves the surprise exactly.

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	kestrel_alu

## 7.1	Purpose

kestrel_alu performs the datapath arithmetic for the core: ten operations selected by alu_op, operating on the source-mux outputs. It also computes eq/lt/ltu flags — which the core deliberately does not use for branches (see the ruling below).

## 7.2	Interface

kestrel_alu interface

| Port | Direction | Width | Description |
| --- | --- | --- | --- |
| op | Input | alu_op_e | Operation select |
| a | Input | 32 | Left operand (PC or rs1, via the core’s source mux) |
| b | Input | 32 | Right operand (immediate or rs2) |
| y | Output | 32 | Result |
| eq / lt / ltu | Output | 1 / 1 / 1 | Flags: equality, signed less-than, unsigned less-than — unconnected at the core instance |

## 7.3	Operations

ALU operations (shamt = b[4:0])

| alu_op | y | Notes |
| --- | --- | --- |
| ADD | a + b | Also computes branch/JAL targets when a = pc |
| SUB | a - b |  |
| AND / OR / XOR | bitwise |  |
| SLL | a << shamt | shamt = b[4:0] (unpriv §2.1.4.1: shift amounts use 5 bits) |
| SRL | a >> shamt | logical |
| SRA | $signed(a) >>> shamt | arithmetic |
| SLT | {{31{1'b0}}, lt} | sets rd to 0/1 |
| SLTU | {{31{1'b0}}, ltu} | unsigned compare, notably for SLTIU’s sign-extended-then-unsigned immediate |

The shift-by-b[4:0] matches the ISA’s shamt semantics exactly — and because the decode table keys SLLI/SRLI/SRAI fully on funct7, encodings with shamt[5] set (reserved in RV32I) never reach this module as shifts; they halt as illegal in decode.

## 7.4	The Flags Ruling

The flags compare a against b: eq = (a == b), lt = ($signed(a) < $signed(b)), ltu = (a < b). For branch instructions the core drives a = pc (the ALU is simultaneously computing pc + imm), so these flags would compare pc against imm — silently wrong. The core’s instance leaves eq/lt/ltu unconnected and branches decide on the dedicated rs1/rs2 comparator in the top. The ALU flags remain correct for SLT/SLTU’s internal use (lt/ltu feed the SLT/SLTU rows) and are available to any future consumer whose a/b are the intended operands. This was a reviewed ruling (Task 4 review, SDD ledger), recorded here so nobody “fixes” the dangling pins.

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	kestrel_regfile

## 8.1	Purpose

kestrel_regfile is the 32 x 32-bit general-purpose register file: two combinational read ports, one synchronous write port, and the architectural x0 rule enforced in hardware. Reads are combinational because the single-cycle design cannot afford a read latency; the write commits on the clock edge.

## 8.2	Interface

kestrel_regfile interface

| Port | Direction | Width | Description |
| --- | --- | --- | --- |
| clk / rst_n | Input | 1 / 1 | Clock; active-low reset clears all 32 entries |
| rs1_addr / rs1_data | Input / Output | 5 / 32 | Read port 1 (combinationally regs[rs1_addr]) |
| rs2_addr / rs2_data | Input / Output | 5 / 32 | Read port 2 |
| rd_addr / rd_data / rd_wen | Input / Input / Input | 5 / 32 / 1 | Write port; committed on the edge when rd_wen and rd_addr != 0 |

## 8.3	Internal Structure and Rules

- Storage is logic [31:0] regs [32], reset to all zeros (so no register carries architectural garbage out of reset, and pre-load observability is deterministic).

- The x0 rule lives here: writes with rd_addr == 0 are discarded by the write condition rd_wen && (rd_addr != X0_ADDR); reads of x0 return the reset/stored zero naturally.

- The core additionally gates rd_wen (into rd_wen_eff) so that the retry’s first beat and halted cycles never reach this port — see the kestrel_core chapter.

- No write-forwarding exists: a write commits on the edge and is readable the next cycle. This is exactly the single-cycle contract (a store is visible to the next cycle’s load through the memory; a register write is visible to the next instruction through the register file).

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	kestrel_mem_loader

## 9.1	Purpose

kestrel_mem_loader is the optional board glue (HAS Ch. 4 specifies its external contract; this section covers how it is built). It contains: two 16384-word distributed-RAM arrays (imem, dmem), a CTRL/run register, and the write-port merge logic — wrapped in the repo’s skid-buffered AXIL leaf slaves, composed exactly as rtl/amba/shared/sdpram_slave_axil_axil.sv composes them. Mutual exclusion between loader and core is by protocol; there is no contention arbiter and no FSM in the datapath.

### 9.1.1	Figure 2.1: Loader composition

kestrel_mem_loader internal composition

## 9.2	Memory Arrays

- (* ram_style = "distributed" *) logic [31:0] imem [0:MEM_WORDS-1] (and dmem) — the attribute keeps reads combinational, which the single-cycle contract requires; a block RAM’s synchronous read would break it.

- One write port each, merging two clients per byte lane in a single clocked process: the loader (ld_wr_*, gated by !run_q) and the core (core_wr_*, gated by run_q && dmem_req && dmem_wstrb != 0). Both are additionally fenced during reset assertion.

- Under the unified map each array carries up to three combinational read ports (core fetch, core L/S, AXIL readback); distributed-RAM read replication absorbs that in synthesis.

- Power-up content is undefined in hardware; the simulation model zero-initializes so pre-load reads are deterministic. An image is always loaded before run.

## 9.3	CTRL / Run Register and Reset Sequencing

run_q is a sticky bit (set by a CTRL write with wstrb[0] & wdata[0], cleared only by reset). core_rst_n is literally run_q: low in load mode (the core is held, parks its fetch at RESET_ADDR, retires nothing), high from the commit cycle of the run write. Power-on default is load mode.

## 9.4	Write Side: Leaf Slave plus Two Holding Bits

The axil4_slave_wr leaf terminates the AW/W/B protocol into FUB nets. The wrapper’s entire write-side state is two holding bits plus the run register:

- wr_addr_q / wr_addr_pending_q: the AW address holds until its W beat fires (fub_awready = !wr_addr_pending_q || w_fire; a same-cycle swap lets a new AW in as the old W fires).

- b_pending_q: at most one B outstanding (fub_wready = wr_addr_pending_q && !b_pending_q); B returns OKAY one holding bit deep and the leaf’s B skid buffers it against the master.

Loader-write qualification: ld_wr_fire = w_fire && !wr_addr_q[CTRL_BIT] && !run_q — memory writes land only in load mode; CTRL writes never touch the arrays. Run-mode loader writes are dropped in logic with B still OKAY (testplan scenario ML-09).

## 9.5	Read Side: Combinational Beat through the AR Skid

The axil4_slave_rd leaf supplies the AR/R handshake; the wrapper’s response is a single-cycle combinational beat: fub_arready = fub_rready (AR accepted exactly when the R skid can take the beat), fub_rvalid = fub_arvalid, fub_rresp = OKAY. The read mux is:

fub_rdata = fub_araddr[CTRL_BIT] ? {31'b0, run_q}
          : fub_araddr[DMEM_BIT] ? dmem[fub_araddr[WORD_IDX_W-1:0]]
          :                        imem[fub_araddr[WORD_IDX_W-1:0]];

Core-side reads select the array the same way: imem_rdata = imem_addr[DMEM_BIT] ? dmem[imem_addr[13:0]] : imem[imem_addr[13:0]] (and likewise for dmem_rdata), which is what makes one flat image coherent across fetch, L/S, and AXIL readback.

## 9.6	Address Map, as Implemented

Loader map implementation facts

| Item | Implementation |
| --- | --- |
| Array word index | addr[13:0] of the presenting port’s byte address (WORD_IDX_W = $clog2(MEM_WORDS) = 14), AXIL and core alike |
| Array select | addr[16]: 0 = imem, 1 = dmem, on every port (unified map) |
| CTRL decode | addr[17] = 1 → CTRL register at 0x0002_0000 |

Two consequences the maintainer must not “tidy” without a plan-level decision: byte-address bits [15:14] are not decoded, so each region’s 64 KB byte window aliases onto its array every 0x4000 bytes; and because the core presents only word-aligned addresses, its window is the 4096 word-aligned slots (16 KB of image) per array — the delivery testbench streams word k to AXIL byte address 4*k, which is the convention that keeps loader and core views identical. The module header’s addr[15:2] wording describes intent; the implemented addr[13:0] slice above is the as-built truth this specification documents.

## 9.7	Debug Taps

o_dbg_busy_wr / o_dbg_busy_rd are the leaf slaves’ busy outputs, exposed for the AXIL backpressure checks (scenario ML-08).

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	kestrel_mem_bram

## 10.1	Purpose

kestrel_mem_bram is the block-RAM alternative to kestrel_mem_loader (owner, 2026-10-20): a board-sized memory with registered reads behind the same core ports and the same AXIL address map. A single-cycle core cannot wait for a synchronous read on its own, so the wrapper holds it with mem_wait while a read lands. Every instruction takes two clocks and a load three; the ISA is untouched (HAS Ch. 4, memory contract property 2).

Loader and block-RAM wrapper compared

|  | kestrel_mem_loader | kestrel_mem_bram |
| --- | --- | --- |
| Storage | 2 x 64 KB distributed RAM | 1 x 128 KB block RAM, true dual port |
| Core reads | combinational | registered, core held with mem_wait |
| CPI | 1 (2 for a cross-word access) | 2, 3 for a load (plus the retry beat) |
| Word index | byte-address bits [13:0] (aliases, see Ch. 2.6) | addr[16:2], no aliasing |
| Run control | sticky run | run writable both ways: 0 returns to load mode |
| Status | run | halted, cause, cycles, instret, last gp write |

## 10.2	The Array

One logic [31:0] r_mem [32768] with ram_style = "block", written per byte lane and read through an output register on each port (the inference template Vivado maps to RAMB36 in true-dual-port mode; 32 tiles). Port A serves the core, port B the AXIL loader. Code, data and .tohost share one address space, as in every other kestrel memory (fetch and L/S coherent, so rv32ui-p-fence_i runs).

## 10.3	The Sequencer (Port A)

Three states, one register of instruction word:

The fetch / execute / load sequencer

| State | Port A | mem_wait | Leaves to |
| --- | --- | --- | --- |
| FETCH | read at imem_addr (the PC) | 1 | EXEC |
| EXEC | imem_rdata = the read; present dmem_addr; a store writes at the end of the cycle | 1 for a load, else 0 | LOAD for a load, else FETCH |
| LOAD | hold; dmem_rdata = the read, imem_rdata = the captured word | 0 | FETCH |

The instruction word is captured at the end of EXEC so it stays valid through LOAD, where the same port’s output now holds the load data. A store commits in EXEC, the cycle the core retires it, so the next FETCH already sees it. A cross-word access is two core cycles (HAS Ch. 4 retry), each going round the sequencer. While the core is in load mode the sequencer parks in FETCH.

## 10.4	AXIL Side (Port B)

The write side is the loader’s: the axil4_slave_wr leaf, an AW holding register and a B holding bit. Memory writes are accepted only in load mode. The read side takes two stages, because port B’s read is registered: AR is accepted into a wait cycle, the array (or register) data is copied into a holding register, and R is offered from there and held under backpressure. AR is refused in a cycle where a loader write owns port B.

## 10.5	Registers (addr[17] = 1)

kestrel_mem_bram registers

| addr[4:2] | Name | Access |
| --- | --- | --- |
| 0 | CTRL/STATUS | W bit 0 run (1 releases the core, 0 returns to load mode); R {cause[7:4], halted[1], run[0]} |
| 1 | CYCLES | clocks since run was set, frozen at halt |
| 2 | INSTRET | retired instructions (RVFI beats) |
| 3 | GP | last value written to x3 – the riscv-tests verdict, since kestrel halts on the final ecall before the tohost store |
| 4 | BUILD_ID | 0x4B42524D (“KBRM”) |

Setting run from 0 clears halted, cause and the counters.

## 10.6	Verification

- dv/tests/test_kestrel_mem_bram.py: the rv32ui battery through this path with the golden-interpreter diff and spike lockstep (42/42 at func), the registers (status, counters, CPI 2..3, stop / reload / rerun) and AXIL backpressure on the two-stage read. A wrapper mutant (no instruction hold through LOAD) fails 12 of the 42 images.

- formal/kestrel/riscv-formal: every check with mem_wait driven freely; a core mutant (register write not gated by mem_wait) fails reg in one second.

- On the Genesys 2: projects/fpga-systems/Genesys2/riscv-ip/kestrel, the same host programs over UART in sim and on the board.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Package Types and Top-Level Wiring

## 11.1	kestrel_pkg

kestrel_pkg (rtl/includes/kestrel_pkg.sv) is the shared type and encoding source. It contains no state and no modules; every kestrel filelist pulls it in with -f, and consumers never name the package path directly.

### 11.1.1	ALU operation enum (alu_op_e, 4 bits)

alu_op_e enumerators

| Enumerator | Operation |
| --- | --- |
| ADD SUB | add / subtract |
| AND OR XOR | bitwise |
| SLL SRL SRA | shifts (shamt = b[4:0]) |
| SLT SLTU | set-less-than, signed / unsigned |

### 11.1.2	Immediate-format enum (imm_sel_e, 3 bits)

imm_sel_e enumerators

| Enumerator | Format | Assembly |
| --- | --- | --- |
| I | I-type | {{20{insn[31]}}, insn[31:20]} |
| S | S-type | {{20{insn[31]}}, insn[31:25], insn[11:7]} |
| B | B-type | {{20{insn[31]}}, insn[7], insn[30:25], insn[11:8], 1'b0} |
| U | U-type | {insn[31:12], 12'b0} |
| J | J-type | {{12{insn[31]}}, insn[19:12], insn[20], insn[30:21], 1'b0} |

### 11.1.3	Halt-cause encodings (localparam logic [3:0])

HALT_* encodings — the single source of truth shared by decode and core

| Name | Value | Raised in | Meaning |
| --- | --- | --- | --- |
| HALT_NONE | 4'h0 | — | running |
| HALT_ECALL | 4'h1 | kestrel_decode | ECALL |
| HALT_EBREAK | 4'h2 | kestrel_decode | EBREAK |
| HALT_IALIGN | 4'h3 | kestrel_core | taken control transfer to a non-4-aligned target |
| HALT_ILL | 4'hF | kestrel_decode | illegal encoding (default row) |

The package comment records the ownership split: causes 1, 2, and F belong to decode; cause 3 belongs to the core because the condition needs the resolved next PC and branch decision that decode cannot see.

## 11.2	Top-Level Wiring (kestrel_core)

The instance map every maintainer should be able to redraw from memory:

kestrel_core internal wiring map

| Net(s) | Driver → Load | Notes |
| --- | --- | --- |
| insn = imem_rdata | fetch → decode, imm_gen, regfile addrs, RVFI | opcode = insn[6:0], funct3 = insn[14:12] |
| Control bundle (13) | u_decode → muxes, regfile, L/S datapath | keyed on {opcode, funct3, funct7} |
| imm | u_imm_gen → alu_src_b mux, writeback (LUI) |  |
| rs1_data / rs2_data | u_regfile → source muxes, comparator, store-data rotators | read addrs from insn[19:15] / insn[24:20] |
| alu_y | u_alu → dmem address, writeback mux | ALU flags unconnected (ruling, kestrel_alu chapter) |
| cmp_eq/cmp_lt/cmp_ltu → branch_cond → branch_taken | core comparator → next-PC mux | funct3 selects the condition |
| next_pc | next-PC mux → PC register, rvfi_pc_wdata | 4-row priority mux |
| rd_wdata / rd_wen_eff | writeback mux + gates → u_regfile | x0 discard inside the file |
| L/S nets (ls_*, dmem_*) | core rotators → dmem pins | retry state ls_retry, merge register ls_rdata_lo |
| Halt nets (halt_*, misalign_target) | core → halt ports, halt_q | halt_cause_eff = dec_halt_cause \¦ (misalign_target ? HALT_IALIGN : HALT_NONE) |
| RVFI bundle (17) | core aggregation assigns → top ports | rules in Chapter 4 |

## 11.3	Reset Discipline

All sequential logic uses the shared reset_defs.svh macros (ALWAYS_FF_RST, RST_ASSERTED): asynchronous assert, synchronous deassert, active-low rst_n. The register file clears to zeros; the PC clears to RESET_ADDR; halt_q, ls_retry, ls_rdata_lo, and retire_count clear to zero. The loader applies the same macros with aresetn, and its core_rst_n output is the core’s actual reset in loader integrations.

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	The Single Cycle, Step by Step

## 12.1	The Clock in One Sentence

On every rising edge, kestrel writes one register-file entry (maybe), updates the PC (unless held), and moves its bookkeeping flops — while a combinational cloud standing between pc and the next edge computes everything the next cycle needs: the fetched instruction, its decode, the operands, the result, the memory access, the next PC, and the RVFI record for whatever retires this cycle.

The sections below follow the data in dependency order through a fully general cycle.

## 12.2	1. Fetch and Decode

imem_addr = pc and insn = imem_rdata — the instruction memory answers in the same cycle. Decode is the truth table of the kestrel_decode chapter, producing the thirteen-field control bundle plus defaults of “no operation, no halt.”

## 12.3	2. Operands and Immediates

The register file reads rs1 = insn[19:15] and rs2 = insn[24:20] combinationally; x0 reads zero because the file ties it. The immediate generator builds one of the five format immediates under imm_sel. Two muxes select the ALU inputs: alu_src_a is the PC (for AUIPC and branch targets) or rs1_data; alu_src_b is the immediate or rs2_data.

## 12.4	3. Execute

The ALU computes one of ten operations. For branches, the dedicated comparator evaluates cmp_eq/cmp_lt/cmp_ltu on rs1_data/rs2_data in parallel, with funct3 selecting the condition in a unique case. Both the ALU’s target (pc + imm, since alu_src_a = pc on branch rows) and the taken decision are ready when the next PC is chosen — taken and not-taken branches cost the same cycle.

## 12.5	4. Memory Access

For loads and stores, dmem_addr is the word-aligned ALU result ({alu_y[31:2], 2'b00} — or the next word on a retry beat), dmem_wdata/dmem_wstrb are the rotated store data and byte enables, and read data is rotated back before size/sign selection implements LB/LBU/LH/LHU/LW. Aligned and in-word-misaligned accesses complete in this one cycle; a cross-word access holds the PC for a retry cycle and issues a second beat (next section).

## 12.6	5. Writeback

rd_wdata is selected by the five-row priority mux (CSR stub zero → LUI immediate → load data → link pc+4 → ALU result); rd_wen_eff = rd_wen & ~ls_first & ~halt qualifies it; the write commits on the edge.

## 12.7	6. Next-PC Selection and the Edge

The next-PC mux selects among JAL/JALR target, taken-branch target, and pc + 4. The PC register loads next_pc on the edge — unless halt (frozen) or ls_first (held for the retry beat). rvfi_pc_wdata = next_pc reports the architecturally next PC on the retire beat.

## 12.8	The Basic Cycle on the Pins

### 12.8.1	Waveform 4.1: Basic single-cycle execution — an aligned load followed by an ALU op

Waveform 4.1: basic single-cycle execution — an aligned load followed by an ALU op

Two back-to-back retiring instructions. During cycle n the LW’s full path — imem read, decode, register read, ALU address, dmem read, rotate/sign-extend, writeback mux — completes between the same two edges, and the beat retires (rvfi_valid=1) with the load’s memory fields. The ADDI in cycle n+1 retires just the same: CPI 1 with no bubbles, because the whole machine is combinational between the PC and the edge.

## 12.9	Branch and Jump Resolution

Control-transfer resolution

| Transfer | Target computation | Decision source | Cost |
| --- | --- | --- | --- |
| BEQ/BNE/BLT/BGE/BLTU/BGEU | pc + imm(B) — ALU row alu_src_a_pc=1 | dedicated comparator, funct3-selected | 1 cycle, taken or not |
| JAL | pc + imm(J) | unconditional (jump=1) | 1 cycle |
| JALR | (rs1_data + imm) & 32'hFFFF_FFFE | unconditional (jalr=1) | 1 cycle |
| Taken transfer to next_pc[1:0] != 00 | computed but not committed | misalign_target → halt cause 0x3 | halts (next section) |

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	Cross-Word Misaligned Retry

## 13.1	When the Retry Fires

The L/S datapath completes any access with addr[1:0] + size <= 4 in the single cycle — every aligned access and every misaligned access that stays inside one word. When addr[1:0] + size > 4 the access straddles two words, one 32-bit beat cannot carry it, and the retry machinery engages:

- ls_crossing = ls_active & ((ls_offset + ls_size_bytes) > 4)

- ls_first = ls_crossing & ~ls_retry — the first beat is in flight

- ls_second = ls_retry — the retry beat

- ls_retry <= ls_first — one bit of control state; no FSM

## 13.2	Beat-by-Beat Behavior

The two beats of a cross-word access (o = addr[1:0])

| Aspect | Beat 1 (ls_first) | Beat 2 (ls_second) |
| --- | --- | --- |
| dmem_addr | {alu_y[31:2], 2'b00} — word containing the start | {alu_y[31:2] + 1, 2'b00} — the next word |
| Store data | rs2_data << (8*o) | rs2_data >> (8*(4-o)) |
| Store strobe | size_mask << o (tail bytes) | ls_head_strobe (head bytes only) |
| Load path | ls_rdata_lo <= dmem_rdata >> (8*o) | ls_raw_rdata = (dmem_rdata & head_mask) << (8*(4-o)) \¦ (ls_rdata_lo & tail_mask) |
| PC | held (pc <= ls_first ? pc : next_pc) | advances on the edge |
| rvfi_valid | low — no partial beat leaks | high — the access retires once, with assembled data |

Everything else in the core is unaware the retry happened: dmem_req (and dmem_we for stores) simply stays high across both beats while the address steps by one word.

## 13.3	The Waveform

### 13.3.1	Waveform 4.2: Cross-word store retry — SH at byte offset 3

Waveform 4.2: cross-word store retry — SH at byte offset 3

A store-halfword at byte offset 3 (the SH@3 case): beat 1 writes the tail byte of word W with strobe 4'b1000; beat 2 writes the head byte of word W+4 with strobe 4'b0001, data right-rotated by the tail size. dmem_req/dmem_we stay high across both beats, ls_retry marks the retry cycle, the PC holds for one extra cycle, and the instruction retires exactly once — rvfi_valid high only on the final cycle.

## 13.4	Worked Example: a Halfword Across the Boundary

lh x2, 7(x10) with x10 = 0x1000 reads the halfword at 0x1007 — one byte at offset 3 of word 0x1000 and one at offset 0 of word 0x1004.

- Beat 1: dmem_addr = 0x1000, read data >> 24, captured into ls_rdata_lo — the tail byte.

- Beat 2: dmem_addr = 0x1004, head byte merged at bits [15:8]; the halfword is sign-extended from bit 15 into x2.

- RVFI: one beat, mem_addr = 0x1007, rmask = 0011 packed from the addressed address, mem_rdata holding the assembled halfword in bits [15:0].

A directed test with distinct nonzero bytes on both sides of the boundary (ls_misaligned) pins this end to end; a first-cut implementation that silently truncated bytes rotating past bit 31 reported 0x00FE where the architectural answer was 0xCAFE — caught because the golden interpreter models the full two-word semantics and the lockstep diff tolerates no slop.

## 13.5	Why Hardware, Why Two Cycles

Trapping to software emulation needs the trap machinery kestrel deliberately does not have, and emulating in a trap stub would make the misaligned case a hundred cycles instead of two. Hardware handling keeps every access at fixed cost — one cycle in-word, two crossing — which is the only cost model a single-cycle core can afford to reason about. The price is integration: a misaligned word access performs two word reads/writes and is not atomic, which the spec never promised anyway (unpriv §2.1.6). Instruction-side misalignment is not permitted this latitude — the next section covers the halt.

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Halt, IALIGN, and the System Layer

## 14.1	The Halt Holding Register

halt_q is a one-bit flop that latches any nonzero halt_cause_eff:

halt_cause_eff = dec_halt_cause | (misalign_target ? HALT_IALIGN : HALT_NONE)
halt_now       = |halt_cause_eff
halt_q         <= halt_q | halt_now
halt           = halt_q | halt_now        // visible on the first halting cycle

Decode raises HALT_ECALL (1), HALT_EBREAK (2), and HALT_ILL (F); the core raises HALT_IALIGN (3). Once latched:

- The PC freezes (pc <= halt ? pc : ...).

- Writeback is suppressed (rd_wen_eff and the RVFI rd_wb gate both use halt).

- Retirement stops: rvfi_valid = rst_n & ~halt_q & ~ls_first, and since halt_q latches on the halting cycle, the halting instruction itself is the last beat — a trap beat with rvfi_trap = halt_now.

A holding register rather than a pulse matters for verification: an external observer sampling any time after the halt sees a stable, unambiguous stopped state with the cause still on the port. The testbench samples four post-halt cycles pinning halt raised, rvfi_valid low, rvfi_pc_rdata frozen.

## 14.2	The IALIGN Halt (Cause 3)

With IALIGN=32 — no compressed instructions — every fetch must be 4-byte aligned, and unpriv §2.1.5 requires an instruction-address-misaligned exception when taken control flow targets an address whose low two bits are not 00. RISC-V’s permission to handle data misalignment in hardware does not extend here.

The condition, visible only in the core:

misalign_target = (jump | jalr | branch_taken) & (next_pc[1:0] != 2'b00)

Why decode cannot catch it: for JALR the target depends on a register value; for every branch it depends on the taken decision. Decode sees neither — it produces the control bundle, not the data. kestrel_pkg states the ownership directly: cause 3 belongs to the core.

The writeback subtlety. JAL/JALR set decode’s rd_wen — they are link instructions, and a taken JAL/JALR that does align must write pc+4. The misaligned case must not. Rather than teaching decode about alignment, the core gates both the register write and the RVFI report with halt itself: rd_wen_eff = rd_wen & ~ls_first & ~halt, rd_wb = rd_wen & (insn[11:7] != 0) & ~halt. The trap beat reports the misaligned target on rvfi_pc_wdata (it is next_pc), exactly what a real trap would record as the faulting address — a detail riscv-formal’s checks verify explicitly.

## 14.3	The System Layer: Stubs, NOPs, and Halts

The thirty-seven base instructions sit inside two opcode classes that belong to the wider system: MISC-MEM (FENCE and friends, unpriv §2.1.7; FENCE.I is Zifencei, §4.1) and SYSTEM (ECALL, EBREAK, and the Zicsr CSR instructions, §4.2; MRET is defined in the privileged Volume II). kestrel has no memory-ordering hardware, no CSR file, and no trap handler, so these classes retire through documented stubs and NOPs. Exactly what that means — including what it does not mean:

### 14.3.1	FENCE and FENCE.I retire as NOPs

kestrel’s imem and dmem are separate, combinational-read ports on a single-cycle core: there is no write buffer to drain, no cache to invalidate, no instruction prefetch to refetch. Fence semantics are vacuous at this rung, and decode retires FENCE (MISC-MEM funct3 000) and FENCE.I (funct3 001) as NOPs — one cycle each, no architectural effect. MISC-MEM funct3 2-7 are reserved and halt as illegal. (The one battery test whose program modifies its own instruction stream, rv32ui-p-fence_i, works because the testbench memory is a single unified array serving both ports — a testbench coherence choice, not a core feature.)

### 14.3.2	MRET falls through

MRET (SYSTEM funct3 000, imm12 0x302) returns from a machine trap. It is meaningful only when traps exist — trap state, mepc, privilege transfers, all rung-3 machinery. kestrel retires MRET as a documented NOP-with-fall-through: the PC advances to pc+4. That choice is not arbitrary: the riscv-tests p-environment points mepc at the instruction following its trap stub in the configurations kestrel runs, so pc+4 is exactly what the test expects. This is stated plainly so nobody mistakes it for trap support.

### 14.3.3	The CSR class retires as a bounded stub

The six Zicsr forms (SYSTEM funct3 001/010/011/101/110/111) retire through csr_stub: decode raises csr_stub and rd_wen, and the writeback mux substitutes a hard zero. Reads return zero; writes drop on the floor. SYSTEM funct3 100 is reserved and halts as illegal.

Say it plainly: this is not CSR support. There is no CSR file, no mstatus, no mtvec, no counters, and nothing the instructions do has architectural effect beyond a zero written to rd. The stub exists for one bounded reason: the riscv-tests p-environment preamble executes about a dozen CSR instructions plus an MRET before every test body, and without the stub the battery cannot run at all. On the battery programs the only CSR read with rd != x0 is mhartid (value 0), which the stub matches. Anything needing real CSR state — trap handlers, counters, feature probing — is out of contract at this rung and fails loudly either as a zero where a nonzero was expected or as a program that halts where it should have trapped. The honesty matters for the ladder: merlin inherits the same stub, and peregrine replaces it with a real machine-mode CSR file. The seam is deliberately visible in the decode table.

## 14.4	Halt Timing Summary

Halt timing at the boundary

| Cycle | halt | rvfi_valid | rvfi_trap | PC | Notes |
| --- | --- | --- | --- | --- | --- |
| Last running cycle | 0 | 1 | 0 | advances | normal retirement |
| Halting cycle | 1 (immediate) | 1 | 1 | frozen from this edge | trap beat; cause on halt_cause; no rd/mem fields |
| All later cycles | 1 | 0 | 0 | frozen | halt_q holds; cause held on the port |

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	RVFI Aggregation

## 15.1	Retirement as the Observation Channel

Every instruction that retires produces one beat; the halting instruction produces one final trap beat; nothing else produces anything. The cocotb testbench diffs these beats field by field against a golden interpreter, and riscv-formal relates them to an instruction-set model. Both consumers need the aggregation rules below to be exact — they are stated next to the assignments in the RTL and restated here.

## 15.2	The Field Table

RVFI signals and their kestrel aggregation

| Signal | Width | kestrel aggregation |
| --- | --- | --- |
| rvfi_valid | 1 | rst_n & ~halt_q & ~ls_first — high exactly on retiring cycles |
| rvfi_order | 64 | retire_count; 0, 1, 2, … over retired beats |
| rvfi_pc_rdata | 32 | The retiring instruction’s PC |
| rvfi_pc_wdata | 32 | next_pc — the architecturally next PC (the misaligned target on a cause-3 halt) |
| rvfi_insn | 32 | The instruction word |
| rvfi_trap | 1 | halt_now: high on the single halting trap beat |
| rvfi_rs1_addr / rvfi_rs2_addr | 5+5 | Register-file read ports (insn[19:15], insn[24:20]) |
| rvfi_rs1_rdata / rvfi_rs2_rdata | 32+32 | Read data; x0 reads zero naturally |
| rvfi_rd_addr / rvfi_rd_wdata | 5+32 | Writeback, when rd_wb; both zero otherwise |
| rvfi_mem_addr | 32 | The unaligned byte address of a load/store (alu_y); zero otherwise |
| rvfi_mem_rmask / rvfi_mem_wmask | 4+4 | Byte lanes touched, packed from bit 0 relative to rvfi_mem_addr (a word access reports 4'b1111 regardless of offset) |
| rvfi_mem_rdata / rvfi_mem_wdata | 32+32 | Assembled access data (loads) / size-truncated store data (rs2_data & ls_data_mask); zero otherwise |

## 15.3	The Three Gating Rules

Three qualifications turn the raw datapath signals into a clean retire stream:

- x0 reports zero. rd_wb = rd_wen & (insn[11:7] != 0) & ~halt, so a beat writing x0 reports rd_addr = 0 with rd_wdata = 0. This is not merely tidiness: riscv-formal requires rd_wdata == 0 whenever rd_addr == 0, and the early core failed exactly there until the rule was pinned. The register file independently discards the x0 write architecturally.

- The retry’s first cycle is silent. During the first beat of a cross-word access, rvfi_valid is low (ls_first masks it): no partial beat leaks out. The access reports one beat on its final cycle, with the full assembled data.

- Halt retires one trap beat, then silence. rvfi_trap = halt_now marks the halting instruction itself — a beat with trap set, no rd write, no memory fields — after which the latched halt_q holds rvfi_valid low forever. The wrapper used by riscv-formal derives rvfi_halt = rvfi_valid & halt, which matches the semantics exactly: the trap beat is the final retirement. (rvfi_intr = 0, rvfi_mode = M, rvfi_ixl = 32 are generated in the wrapper, not the core.)

## 15.4	Memory-Field Packing

For loads and stores, rvfi_mem_addr is the unaligned byte address and the masks count byte lanes from bit 0 relative to that address — the riscv-formal byte-i convention of rvfi.md. A store’s rvfi_mem_wdata is the store value truncated to the access size (rs2_data & ls_data_mask), not the raw register; leaking the upper bytes into the payload is a real bug class this encoding exists to prevent, and one kestrel fixed when the battery’s ma_data-shaped vectors arrived. For a cross-word access the single retire beat reports the full-size mask with the assembled data — the retry’s two physical beats collapse into one architectural record.

## 15.5	Why This Pays

Because the rules are few and stated in RTL comments next to the assignments, the golden interpreter models the same rules in Python and the two can be diffed beat for beat over tens of thousands of instructions with zero tolerance. When a diff appears, it names the cycle, the PC, and the field — which is how the misaligned-jump defect (found by riscv-formal as a counterexample the 42-test battery missed) was localized in minutes rather than debugged from waveforms.

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Verification Hooks and Observability

## 16.1	The Three Testplan-Tracked Hooks

kestrel was built RVFI-first, and the verification architecture hangs three observable hooks off the boundary. Each maps to committed testplan scenarios:

The three testplan-tracked hooks

| Hook | Signals | What it proves | Testplan anchor |
| --- | --- | --- | --- |
| RVFI retire channel | 17 rvfi_* ports | Architectural correctness: every retired beat diffed full-field against the golden interpreter rv32i_interpreter.py (pc, insn, order, trap, rs/rd, all four memory channels — no sampled subset, no tolerance); riscv-formal’s 42 checks relate the same beats to ISA models | kestrel_core CORE-01..CORE-05, CORE-14; kestrel_mem_loader ML-07 |
| Halt observability | halt, halt_cause | Program end, cause classification, and the post-hold: the halting trap beat, then at least four cycles of halt raised with rvfi_valid low and the PC frozen | kestrel_core CORE-12, CORE-13 |
| Data-port watch | dmem_req, dmem_addr, dmem_wstrb, dmem_wdata | The memory contract on the pins: rotated strobes land the right bytes (TB per-byte merge), retry addressing steps one word, the tohost mailbox is observed on the store port, store-to-load visibility is single-cycle | kestrel_core CORE-08..CORE-11; loader ML-02/ML-05/ML-06 |

## 16.2	Supporting Hooks

Supporting loader hooks

| Hook | Signals | Use |
| --- | --- | --- |
| Loader reset tap | core_rst_n | Load-mode isolation: with rst_n released but CTRL.run still 0, the core is held, its PC parks at RESET_ADDR, and loader writes become visible on the core-side read port (ML-04) |
| Leaf busy taps | o_dbg_busy_wr, o_dbg_busy_rd | AXIL backpressure: B/R must hold stable under stalled bready/rready with busy asserted (ML-08) |
| Run-mode write rejection | AXIL write + readback | A loader write in run mode is dropped in logic with B OKAY — the halted core’s ECALL word must not be poisoned (ML-09) |

## 16.3	The Consumer Stack

Verification consumer stack

| Layer | Mechanism | Scope |
| --- | --- | --- |
| Golden interpreter lockstep | rv32i_interpreter.py emits the same RVFI beat format as the core; the cocotb harness diffs full-field, zero tolerance | Every golden-trace test, plus the constrained-random fuzz (5 gate / 25 func / 200 full streams) |
| Spike lockstep | spike --isa=RV32I -l commit stream walked 1:1 against the core’s (pc, insn) beats; exception records verified per beat; exit code confirms tohost==1 | Battery and fuzz at func level, using the hardware-misaligned spike 1.1.0 build |
| rv32ui battery | 42 self-checking riscv-tests programs run to the ECALL halt; verdict = x3’s last writeback | 3/3 gate, 42/42 func and full |
| riscv-formal | 42 bounded-model-checking proofs (37 instruction models + reg/pc_fwd/pc_bwd/liveness/unique) against the RVFI port, depth 6, z3/smtbmc | Every RTL change; the gate that caught the misaligned-jump bug 42 directed tests missed |

## 16.4	Cross-Reference to the Testplan Set

The six plans in dv/testplans/ are the V&V cross-reference for both this MAS and the HAS: kestrel_core_testplan.yaml (18 scenarios — ISA classes, branches, loads/stores incl. the cross-word retry, system layer, battery, lockstep, fuzz, coverage closure at 94.9% line), kestrel_mem_loader_testplan.yaml (ML-01..ML-09), and per-FUB plans (kestrel_alu, kestrel_decode, kestrel_imm_gen, kestrel_regfile) whose directed rows pin each block’s unit behavior against the chapters of this document. Coverage closure is measured across all cells (Verilator line coverage through the central cov_utils hook); regenerate with the commands recorded in each plan.

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Parameters, Defines, and Filelists

## 17.1	Parameters

### 17.1.1	kestrel_core

kestrel_core parameters

| Parameter | Type | Default | Verified values | Description |
| --- | --- | --- | --- | --- |
| RESET_ADDR | logic [31:0] | 32'h0000_0000 | 0x0, 0x100, 0x8000_0000 | Reset fetch address; the first retired beat’s pc_rdata equals it (testplan scenario CORE-03) |

### 17.1.2	kestrel_mem_loader

kestrel_mem_loader parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| AXIL_ADDR_WIDTH | int | 18 | AXIL slave address width; bits [17:0] of the map |
| AXIL_DATA_WIDTH | int | 32 | One 32-bit word per beat |
| MEM_WORDS | int | 16384 | Words per array (64 KB imem + 64 KB dmem) |
| SKID_DEPTH_AW / SKID_DEPTH_W / SKID_DEPTH_B | int | 2 / 2 / 2 | axil4_slave_wr skid depths |
| SKID_DEPTH_AR / SKID_DEPTH_R | int | 2 / 4 | axil4_slave_rd skid depths |

Only the default loader parameterization is exercised by the testplans (single build config; recorded as verified, no sweep claimed). RESET_ADDR coverage is recorded per value in the core testplan.

## 17.2	Defines and Includes

Shared defines

| Include | Used by | Purpose |
| --- | --- | --- |
| reset_defs.svh (via -f $REPO_ROOT/rtl/amba/filelists/reset_defs.f) | kestrel_core, kestrel_regfile, kestrel_mem_loader | ALWAYS_FF_RST / RST_ASSERTED macros: asynchronous assert, synchronous deassert, active-low |

No define configuration surface exists beyond this; there are no feature ifdefs in the kestrel RTL.

## 17.3	Filelists

Filelists (KESTREL_ROOT = projects/components/riscv-ip/kestrel-rv32i; REPO_ROOT = RTLDesignSherpa checkout root)

| Filelist | Contents | Needs |
| --- | --- | --- |
| rtl/filelists/kestrel_pkg.f | kestrel_pkg alone | $KESTREL_ROOT |
| rtl/filelists/fub/kestrel_alu.f | ALU + package | $KESTREL_ROOT |
| rtl/filelists/fub/kestrel_decode.f | Decode + package | $KESTREL_ROOT |
| rtl/filelists/fub/kestrel_imm_gen.f | Immediate generator + package | $KESTREL_ROOT |
| rtl/filelists/fub/kestrel_regfile.f | Register file + reset_defs + package | $KESTREL_ROOT, $REPO_ROOT |
| rtl/filelists/fub/kestrel_mem_loader.f | Loader + reset_defs + package + axil4_slave_wr/axil4_slave_rd closures | $KESTREL_ROOT, $REPO_ROOT |
| rtl/filelists/top/kestrel_core.f | Core top + all core FUB closures | $KESTREL_ROOT, $REPO_ROOT |
| rtl/filelists/kestrel_all.f | Master closure: everything above | $KESTREL_ROOT, $REPO_ROOT |
| dv/filelists/kestrel_tb.f | kestrel_tb_top + kestrel_loader_tb_top on kestrel_all.f | $KESTREL_ROOT |

Compile-order and closure discipline: each list is complete for its module (package, includes, leaf dependencies); kestrel_all.f includes everything with -f and never hand-lists sources. The filelists are the supported compile interface.

## 17.4	Regenerating This Document

# from projects/components/riscv-ip/kestrel-rv32i/docs
./generate_mas_pdf.sh --rev 0.1     # KESTREL_MAS_v0.1.docx / .pdf
./generate_has_pdf.sh --rev 0.1     # KESTREL_HAS_v0.1.docx / .pdf
python3 gen_kestrel_figures.py      # regenerate figures + waveform PNGs

The block and composition figures are graphviz sources in kestrel_{has,mas}/assets/graphviz/, rendered by each directory’s regenerate_all_graphviz.sh. gen_kestrel_figures.py (this directory) copies the falcon-ladder figure and renders the wavedrom sources in kestrel_mas/assets/wavedrom/*.json via wavedrom-cli + inkscape; both build scripts only consume the committed PNGs.

Last Updated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	KESTREL-RV32I Hardware Architecture Specification Index

Version: 0.4 Date: 2026-10-20 Purpose: High-level hardware architecture specification for the kestrel-rv32i single-cycle RV32I core

## 18.1	Document Organization

Note: All chapters linked below for automated document generation.

### 18.1.1	Front Matter

- Document Information

### 18.1.2	Chapter 1: Introduction

- Purpose and Scope

### 18.1.3	Chapter 2: System Overview

- System Overview and Context

### 18.1.4	Chapter 3: Architecture

- ISA Scope

- Programmer-Visible Machine State

### 18.1.5	Chapter 4: Interfaces

- kestrel_core Port Specification

- Memory Contract

- Halt and Trap Behavior

- kestrel_mem_loader and AXIL Programming Interface

### 18.1.6	Chapter 5: Performance

- Performance Characteristics

### 18.1.7	Chapter 6: Integration

- Integration Guide

## 18.2	Related Documentation

- KESTREL MAS - Micro-Architecture Specification (block-level implementation)

- Simplified RV32I: kestrel - Tutorial-study book this core is documented by

- RISC-V Falcon Suite README - Suite overview and rung ladder

- RISC-V Unprivileged ISA - ISA authority (cited by chapter)

- Testplans - V&V cross-reference (six plans)

Last Updated: 2026-10-07 Maintained By: RTL Design Sherpa Project

# 19	Simplified RV32I: kestrel

Source: The RISC-V Instruction Set Manual, Volume I: Unprivileged Architecture (riscv-spec.pdf, chapter-cited; this book cites chapters and sections, not pages) and the falcon-suite design documentation Edition: Simplified for the kestrel-rv32i single-cycle core, rung 1 of the falcon suite. Derived from condensed study notes; emphasis is on what the as-built kestrel core actually does and verifies, with the RISC-V specification cited as the authority. Version: 0.2 Last Updated: 2026-10-20 Status: Draft. Personal study notes.

## 19.1	Overview

RV32I is the RISC-V base integer instruction set: thirty-seven instructions, six instruction formats, thirty-one general-purpose registers plus the hard-wired zero register, and a program counter. It is small enough to hold in your head and rich enough to run real compiled code. That combination makes it the canonical teaching ISA — and the foundation rung of the falcon suite.

kestrel is the suite’s rung-1 core: a single-cycle RV32I implementation in which one instruction retires per clock and the whole machine — fetch, decode, register read, execute, memory access, writeback — completes inside that clock. There is no pipeline, no hazard logic, no trap machinery, and almost no control state. What there is, is a datapath you can trace by hand and a decode truth table you can read top to bottom.

This book condenses RV32I into the material needed to understand kestrel: the instruction set itself (Chapter 2), the datapath (Chapter 3), the control truth table (Chapter 4), the memory contract including the hardware-misaligned policy (Chapter 5), the verification story — the rv32ui battery, spike lockstep, and the riscv-formal proof that caught a bug forty-two directed tests missed (Chapter 6) — and what the single-cycle discipline costs, which is exactly what motivates rung 2, merlin (Chapter 7).

## 19.2	Document Structure

### 19.2.1	Chapter 1: Introduction

- 01_introduction.md - What this book is, what kestrel is, how to read the as-built core

- 02_the_falcon_ladder.md - The four-rung suite, why kestrel is rung 1, what each rung teaches

- 03_conventions.md - Notation, register names, halt-versus-trap terminology, citations

### 19.2.2	Chapter 2: The RV32I ISA Quick Reference

- 01_formats.md - The six instruction formats and how immediates are assembled

- 02_the_instruction_set.md - The thirty-seven base instructions, group by group

- 03_system_layer.md - What kestrel does with FENCE, ECALL/EBREAK, MRET, and the CSR class

### 19.2.3	Chapter 3: The kestrel Datapath

- 01_overview.md - Block-level tour, the module list, the complete state inventory

- 02_a_cycle_in_detail.md - One clock, end to end: fetch, decode, execute, writeback

- 03_control_state.md - The five state elements and why there is no FSM

- 04_rvfi.md - The RVFI retire interface and how kestrel aggregates it

### 19.2.4	Chapter 4: The Control Truth Table

- 01_the_control_bundle.md - The thirteen decode outputs and the default bundle

- 02_the_decode_table.md - The full decode case as a table, every row

### 19.2.5	Chapter 5: The Memory Contract

- 01_the_contract.md - The Harvard interface, word-wide buses, combinational reads

- 02_misaligned_data.md - The hardware-misaligned decision: rotation, the retry cycle, RVFI encoding

- 03_ialign_and_halt.md - IALIGN=32, the instruction-side misaligned halt, the four halt causes

### 19.2.6	Chapter 6: Verification

- 01_testbench_and_golden.md - The cocotb testbench, sampling contract, golden interpreter lockstep

- 02_rv32ui_battery.md - The 42-test rv32ui-p-* battery, results table, verdict protocol

- 03_spike_lockstep.md - Spike 1.1.0 lockstep, the misaligned-build note, stream-diff details

- 04_riscv_formal.md - The riscv-formal flow: wrapper, checks, depths, 42/42

- 05_the_cex_story.md - The misaligned-jump counterexample: a real bug the battery missed

### 19.2.7	Chapter 7: What kestrel Costs

- 01_what_kestrel_costs.md - CPI 1 and its price: the whole machine on the critical path

- 02_merlin_teaser.md - The five-stage answer and what carries over unchanged

### 19.2.8	Version History

Version history

| Version | Date | Change |
| --- | --- | --- |
| 0.1 | 2026-10-06 | First draft |
| 0.2 | 2026-10-20 | Section (h2) headings now open a new page, the pipeline default. No technical content changed. |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	RISC-V Falcon Suite

Five independent, clean-slate RISC-V cores whose primary product is education, in the same spirit as the pumice/scoria/andesite memory-controller line. Each rung is a complete, verified, documented design small enough to hold in your head; the rungs ascend in microarchitectural complexity so the diff between neighbors is the lesson. Rungs do not share RTL — each core is the cleanest possible example of its level, not the cheapest to produce. Educational benefit over IP; no rung chases performance or benchmarks.

Design spec (source of truth for scope, ISA, verification, packaging): docs/superpowers/specs/2026-10-06-riscv-falcon-suite-design.md.

## 20.1	The ladder

Each rung carries a raptor-codename, real falcons first (species in strict size order, tracking complexity). The falcon run ends at gyrfalcon — the largest living species — so the top rung steps deliberately to mythology: garuda, the giant divine raptor, king of birds, one past the largest falcon. The codename is the RTL identifier prefix and the module/package prefix; the DIRECTORY is the compound <codename>-<isa> form — kestrel-rv32i, not kestrel. The bare codename is how the core is referred to in prose (“kestrel sees the whole machine in one cycle”).

| Directory | Codename | ISA | Microarchitecture | Status |
| --- | --- | --- | --- | --- |
| kestrel-rv32i/ | kestrel | RV32I | single-cycle | RTL complete; rv32ui battery 42/42 with spike lockstep; riscv-formal 42/42; doc book v0.1 published; board loader sim-verified |
| merlin-rv32i/ | merlin | RV32I | 5-stage in-order pipeline | Planned; structure only |
| peregrine-rv32im/ | peregrine | RV32IM | advanced in-order: predictor, traps, L1 caches, AXI master | Planned; structure only |
| gyrfalcon-rv32ima/ | gyrfalcon | RV32IM baseline; gyrfalcon-A flavor RV32IMA | out-of-order capstone: rename, ROB, reservation stations; two flavors — baseline and gyrfalcon-A (A_EXT=1: LR/SC + AMO atomics study) | Planned; structure only |
| garuda-rv32imaf/ | garuda | RV32IMAF | gyrfalcon’s OoO plus an FPU: FP register file + rename, fcsr precise flags, non-pipelined long-latency units | Planned; structure only (penciled in; beyond the 2026-10-06 design spec, which pins four rungs) |

### 20.1.1	Moved: hive-serv

hive-serv — the SERV-based control/monitoring system (1 VexRiscv control plane + 16 SERV monitor cores) — left this area 2026-10-13 for compute-eng-ip/hive-serv: it uses RISC-V cores rather than teaching one, and it is the control half of the compute family (its other half is the cell CGRA), not a falcon-ladder rung. This directory is the ladder again, nothing more.

### 20.1.2	Tetrad engine: gatling-rv32itetrad

gatling-rv32itetrad/ is not a ladder rung either — it is the compute family’s tetrad engine (owner PRD 2026-10-17): a barrel RV32I core with the Xcell extension (CFG/POP/PUSH/ECTX, custom-0), one engine per tetrad with four contexts — the four cells — sharing one F/D/X/W datapath. It replaces the cell PRD’s ALU + execution-FSM block (“the program is the config”) and answers the kestrel-for-the-compute-FSM question in kestrel’s own lineage: same RV32I contract and lockstep flow, inverted pipeline lesson — N contexts hide one shared datapath’s latency instead of stages hiding latency for one stream. Open decisions and its interface to the bf16 alu-infer block are tracked in vault TASK-001.

## 20.2	Goals per rung

kestrel — see everything. Single-cycle RV32I: PC, register file, ALU, immediate generator, control as an explicit decode truth table, a plainly documented combinational-read memory contract. The hovering falcon: the whole machine is visible in one cycle — including its cost, since the critical path is the machine, which is exactly the motivation for merlin. Delivers: the ISA (R/I/S/B/U/J formats, the full base integer set), the datapath, honest control logic.

merlin — flow it. The classic IF/ID/EX/MEM/WB pipeline on the deliberately identical RV32I ISA, so students diff behavior, not ISA. Forwarding from EX/MEM and MEM/WB with explicit priority, load-use interlock stall, branches resolved in EX predict-not-taken with flush. Delivers: pipeline registers, RAW hazards, forwarding priority, the load-use bubble, control-hazard flushes, why branch resolution stage matters.

peregrine — speed it up. Advanced in-order RV32IM: iterative multicycle multiply/divide (the structural-hazard lesson), BTB + 2-bit bimodal prediction with mispredict recovery, machine-mode CSRs with precise exceptions and CLINT-shaped interrupts, split blocking write-back L1 caches behind an AXI4 master on the repo fabric with MonBus visibility. Delivers: speculation and recovery, precise exceptions, the memory hierarchy, bus protocol — and ties the whole repo together (memory controllers, cache IP line, AMBA fabric).

gyrfalcon — hide everything. Out-of-order capstone in two flavors (added 2026-10-11): the baseline — peregrine’s RV32IM exactly — and gyrfalcon-A (A_EXT=1): LR/SC reservations, store-conditional failure, and AMOs, the atomicity study — so every peregrine battery still carries in both. Dual-issue dispatch, explicit rename map + free list, 32-entry ROB with in-order commit, three reservation stations, wakeup/select over common data buses, speculative dispatch with checkpoint-restore rename rollback. Delivers: register renaming and WAR/WAW elimination, the ROB as an undo log, out-of-order completion with in-order retirement — the honest, complete OOO story — and the atomicity study: reservations, SC failure, AMOs, acquire/release as memory-model operations.

garuda — float it. The RV32IMAF delta on gyrfalcon’s OoO: a 32-entry FP register file with its own rename, an fcsr whose five accrued exception flags (NV/DZ/OF/UF/NX) stay precise-exception-clean through the ROB, and the repo’s ieee754 fp32 arithmetic behind the reservation stations — multi-cycle FMUL/FADD and the non-pipelined Goldschmidt FDIV / Newton-rsqrt FSQRT (9–11 cycle class) as the long-latency-unit lesson. Rounding hard-wired RNE; any other requested mode raises NV, keeping the CSR story honest without a dynamic rounding-mode datapath. Delivers: long-latency execution scheduling, FP architectural state, and the F-extension ISA delta (F regs, fcsr, FMV/FCVT/FSGNJ/FCLASS) — consuming the math library’s math_ieee754_2008_fp32_* blocks end-to-end.

## 20.3	Packaging (per rung, the MC-line treatment)

- Doc book — Simplified RV32I: kestrel and successors, Memory Notes branding, built through bin/md_to_docx.py to DOCX/PDF, following the Simplified DFI books’ precedent. Each book lives in its rung’s docs/; the kestrel edition is published at kestrel-rv32i/docs/simplified_rv32i/.

- Formal — riscv-formal-style ISA compliance where it pays: full on kestrel, retire-interface on merlin, targeted properties (exception precision, recovery, in-order commit, deadlock freedom) on peregrine/gyrfalcon.

- Golden model — spike ISS lockstep in the rds-dv cocotb framework; the classic rv32ui/rv32um riscv-tests batteries as the compliance gate.

- Drill app — bin/apps/core_drills (ddr_drills architecture): pipeline stage-viewer with forwarding arrows, hazard quiz, branch-predictor drill, and a rename/ROB viewer. App design gets its own doc once merlin’s RTL is fixed.

## 20.4	References and layout

Common primary sources (the RISC-V ISA manuals) live in references/ and are cited by chapter, not page.

Per-rung skeleton follows the dma-ip/stream component layout convention: rtl/, docs/, dv/ (cocotb + lockstep testbenches). Shared suite doctrine lands in docs/ as it is written. Per-rung PRD.md/CLAUDE.md arrive with each rung’s implementation plan.

# 21	RISC-V Formal Interface (RVFI)

## 21.1	RVFI Specification

In the following specification the term XLEN refers to the width of an x register in bits, as described in the RISC-V ISA specification. The term NRET refers to the maximum number of instructions that the core under test can retire in one cycle. If more than one of the retired instruction writes the same register, the channel with the highest index contains the instruction that wins the conflict. The term ILEN refers to the maximum instruction width for the processor under test.

The Interface consists only of output signals. Each signal is a concatenation of NRET values of constant width, effectively creating NRET channels. For simplicity, the following descriptions refer to one such channel. For example, we refer to rvfi_valid as a 1-bit signal, not a NRET-bits signal.

### 21.1.1	Instruction Metadata

output [NRET        - 1 : 0] rvfi_valid
output [NRET *   64 - 1 : 0] rvfi_order
output [NRET * ILEN - 1 : 0] rvfi_insn
output [NRET        - 1 : 0] rvfi_trap
output [NRET        - 1 : 0] rvfi_halt
output [NRET        - 1 : 0] rvfi_intr
output [NRET * 2    - 1 : 0] rvfi_mode
output [NRET * 2    - 1 : 0] rvfi_ixl

When the core retires an instruction, it asserts the rvfi_valid signal and uses the signals described below to output the details of the retired instruction. The signals below are only valid during such a cycle and can be driven to arbitrary values in a cycle in which rvfi_valid is not asserted.

The rvfi_order field must be set to the instruction index. No indices must be used twice and there must be no gaps. Instructions may be retired in a reordered fashion, as long as causality is preserved (register and memory write operations must be retired before the read operations that depend on them).

rvfi_insn is the instruction word for the retired instruction. In case of an instruction with fewer than ILEN bits, the upper bits of this output must be all zero. For compressed instructions the compressed instruction word must be output on this port. For fused instructions the complete fused instruction sequence must be output.

rvfi_trap must be set for an instruction that cannot be decoded as a legal instruction, such as 0x00000000.

In addition, rvfi_trap must be set for a misaligned memory read or write in PMAs that don’t allow misaligned access, or other memory access violations. rvfi_trap must also be set for a jump instruction that jumps to a misaligned instruction.

The signal rvfi_halt must be set when the instruction is the last instruction that the core retires before halting execution. It should not be set for an instruction that triggers a trap condition if the CPU reacts to the trap by executing a trap handler. This signal enables verification of liveness properties.

rvfi_intr must be set for the first instruction that is part of a trap handler, i.e. an instruction that has a rvfi_pc_rdata that does not match the rvfi_pc_wdata of the previous instruction.

rvfi_mode must be set to the current privilege level, using the following encoding: 0=U-Mode, 1=S-Mode, 2=Reserved, 3=M-Mode

Finally rvfi_ixl must be set to the value of MXL/SXL/UXL in the current privilege level, using the following encoding: 1=32, 2=64

### 21.1.2	Integer Register Read/Write

output [NRET *    5 - 1 : 0] rvfi_rs1_addr
output [NRET *    5 - 1 : 0] rvfi_rs2_addr
output [NRET * XLEN - 1 : 0] rvfi_rs1_rdata
output [NRET * XLEN - 1 : 0] rvfi_rs2_rdata

rvfi_rs1_addr and rvfi_rs2_addr are the decoded rs1 and rs1 register addresses for the retired instruction. For an instruction that reads no rs1/rs2 register, this output can have an arbitrary value. However, if this output is nonzero then rvfi_rs1_rdata must carry the value stored in that register in the pre-state.

rvfi_rs1_rdata/rvfi_rs2_rdata is the value of the x register addressed by rs1/rs2 before execution of this instruction. This output must be zero when rs1/rs2 is zero.

output [NRET *    5 - 1 : 0] rvfi_rd_addr
output [NRET * XLEN - 1 : 0] rvfi_rd_wdata

rvfi_rd_addr is the decoded rd register address for the retired instruction. For an instruction that writes no rd register, this output must always be zero.

rvfi_rd_wdata is the value of the x register addressed by rd after execution of this instruction. This output must be zero when rd is zero.

### 21.1.3	Program Counter

output [NRET * XLEN - 1 : 0] rvfi_pc_rdata
output [NRET * XLEN - 1 : 0] rvfi_pc_wdata

This is the program counter (pc) before (rvfi_pc_rdata) and after (rvfi_pc_wdata) execution of this instruction. I.e. this is the address of the retired instruction and the address of the next instruction.

### 21.1.4	Memory Access

output [NRET * XLEN   - 1 : 0] rvfi_mem_addr
output [NRET * XLEN/8 - 1 : 0] rvfi_mem_rmask
output [NRET * XLEN/8 - 1 : 0] rvfi_mem_wmask
output [NRET * XLEN   - 1 : 0] rvfi_mem_rdata
output [NRET * XLEN   - 1 : 0] rvfi_mem_wdata

For memory operations (rvfi_mem_rmask and/or rvfi_mem_wmask are non-zero), rvfi_mem_addr holds the accessed memory location.

When the define RISCV_FORMAL_ALIGNED_MEM is set, the address must have a 4-byte alignment for XLEN=32 and an 8-byte alignment for XLEN=64. When the define is not set, then the address must point directly to the LSB or the word / half word / byte that is accessed.

rvfi_mem_rmask is a bitmask that specifies which bytes in rvfi_mem_rdata contain valid read data from rvfi_mem_addr.

rvfi_mem_wmask is a bitmask that specifies which bytes in rvfi_mem_wdata contain valid data that is written to rvfi_mem_addr.

rvfi_mem_rdata is the pre-state data read from rvfi_mem_addr. rvfi_mem_rmask specifies which bytes are valid.

rvfi_mem_wdata is the post-state data written to rvfi_mem_addr. rvfi_mem_wmask specifies which bytes are valid.

When RISCV_FORMAL_ALIGNED_MEM is set then riscv-formal assumes that unaligned memory access causes a trap.

### 21.1.5	Alternative Arithmetic Operations

Some arithmetic operations (such as multiplication and division) are beyond to practical capabilities of even modern hardware model checkers. In order to still be able to verify things like bypassing for the arithmetic units performing those operations we define a set of alternative arithmetic operations. When the define RISCV_FORMAL_ALTOPS is set riscv-formal will expect the processor under test to implement those alternative operations instead.

Commutative operations (like multiplication) are replaced with addition followed by applying XOR with a bitmask that indicates the type of the operation. Noncommutative operations (like division) are replaced with subtraction followed by applying XOR with a bitmask that indicates the type of the operation.

The bitmasks are 64 bits wide. RV32 implementations only use the lower 32 bits of the bitmasks. The *W instructions in RV64 (suchg as MULW) are implemented by adding or subtracting the lower 32 bits of the operands, then XORing with the lower 32 bits of the bitmask, then sign extending the result to 64 bits.

#### 21.1.5.1	Integer Multiply/Divide Instructions

| Operation | Add/Sub | Bitmask |
| --- | --- | --- |
| MUL | Add | 0x2cdf52a55876063e |
| MULH | Add | 0x15d01651f6583fb7 |
| MULHSU | Sub | 0xea3969edecfbe137 |
| MULHU | Add | 0xd13db50d949ce5e8 |
| DIV | Sub | 0x29bbf66f7f8529ec |
| DIVU | Sub | 0x8c629acb10e8fd70 |
| REM | Sub | 0xf5b7d8538da68fa5 |
| REMU | Sub | 0xbc4402413138d0e1 |

### 21.1.6	Control and Status Registers (CSRs)

For each supported CSR there are four additional output ports:

output [NRET * XLEN - 1 : 0] rvfi_csr_<csrname>_rmask
output [NRET * XLEN - 1 : 0] rvfi_csr_<csrname>_wmask
output [NRET * XLEN - 1 : 0] rvfi_csr_<csrname>_rdata
output [NRET * XLEN - 1 : 0] rvfi_csr_<csrname>_wdata

The rmask and wmask ports specify which bits of rdata and wdata are valid.

It is always valid for an instruction to activate more rmask/rdata bits than required by the instruction, as long as the reported bits correctly reflect the machine state.

If reading a CSR has side effects, those side effects are not triggered by raised rmask bits but by the type of the instruction.

The Verilog define RISCV_FORMAL_CSR_<CSRNAME> must be set for each CSR traced via RVFI by the core under test.

See RISC-V Formal CSR Sematics for the exact semantics of CSR values output via RVFI.

### 21.1.7	Handling of Speculative Execution

Out-of-order cores that execute speculatively can commit speculative instructions on RVFI.

Rollbacks must be output via the rollback interface, that is enabled when RISCV_FORMAL_ROLLBACK is defined:

output [ 0 : 0] rvfi_rollback_valid
output [63 : 0] rvfi_rollback_order

All RVFI packets output prior to the cycle with asserted rvfi_rollback_valid with a rvfi_order field of greater or equal to rvfi_rollback_order are invalidated by a rollback event.

RVFI packets output in the same cycle as rvfi_rollback_valid are already part of the new instruction stream re-starting at the instruction number indicated in rvfi_rollback_order.

## 21.2	RVFI TODOs and Requests for Comments

The following section contains notes on future extensions to RVFI. They will come part of the spec as soon as there is at least one core that implements the feature, and a matching formal check that utilises the feature. In many cases the additional ports will only be used (and expected from the core) when additional to-be-defined RISCV_FORMAL_* Verilog defines are set.

### 21.2.1	Support for fused instructions

Fused instructions are simply handled as larger instructions in RVFI. Additional rvfi_rs* ports (or even rvfi_rd* ports) may be added to accommodate the fused instructions.

No instruction models for fused instructions have been created yet.

Alternatively fused instructions may be output as individual instructions in separate RVFI channels.

### 21.2.2	Modelling of Floating-Point State

The following is the proposed RVFI extension for floating point ISAs:

output [NRET *    5 - 1 : 0] rvfi_frs1_addr
output [NRET *    5 - 1 : 0] rvfi_frs2_addr
output [NRET *    5 - 1 : 0] rvfi_frs3_addr
output [NRET *    5 - 1 : 0] rvfi_frd_addr
output [NRET        - 1 : 0] rvfi_frs1_rvalid
output [NRET        - 1 : 0] rvfi_frs2_rvalid
output [NRET        - 1 : 0] rvfi_frs3_rvalid
output [NRET        - 1 : 0] rvfi_frd_wvalid
output [NRET * FLEN - 1 : 0] rvfi_frs1_rdata
output [NRET * FLEN - 1 : 0] rvfi_frs2_rdata
output [NRET * FLEN - 1 : 0] rvfi_frs3_rdata
output [NRET * FLEN - 1 : 0] rvfi_frd_wdata
output [NRET * XLEN - 1 : 0] rvfi_csr_fcsr_rmask
output [NRET * XLEN - 1 : 0] rvfi_csr_fcsr_wmask
output [NRET * XLEN - 1 : 0] rvfi_csr_fcsr_rdata
output [NRET * XLEN - 1 : 0] rvfi_csr_fcsr_wdata

Since f0 is not a zero register, additional *_[rw]valid signals are required to indicate if frs1, frs2, frs3, and frd and their corresponding pre- or post-values are valid.

Alternative arithmetic operations (RISCV_FORMAL_ALTOPS) will be defined for all non-trivial floating point operations.

### 21.2.3	Modelling of Virtual Memory

For processors with support for S-mode and virtual memory we define the following additional RVFI signals for data load/stores:

output [NRET *   64 - 1 : 0] rvfi_mem_paddr
output [NRET * XLEN - 1 : 0] rvfi_mem_pte0
output [NRET * XLEN - 1 : 0] rvfi_mem_pte1
output [NRET * XLEN - 1 : 0] rvfi_mem_pte2
output [NRET * XLEN - 1 : 0] rvfi_mem_pte3

And the following additional RVFI signals for instruction fetches:

output [NRET *   64 - 1 : 0] rvfi_pc_paddr
output [NRET * XLEN - 1 : 0] rvfi_pc_pte0
output [NRET * XLEN - 1 : 0] rvfi_pc_pte1
output [NRET * XLEN - 1 : 0] rvfi_pc_pte2
output [NRET * XLEN - 1 : 0] rvfi_pc_pte3

And we require that the satp CSR is observable through RVFI:

output [NRET * XLEN - 1 : 0] rvfi_csr_satp_rmask
output [NRET * XLEN - 1 : 0] rvfi_csr_satp_wmask
output [NRET * XLEN - 1 : 0] rvfi_csr_satp_rdata
output [NRET * XLEN - 1 : 0] rvfi_csr_satp_wdata

The rvfi_mem_paddr field carries the physical address of the memory access. The rvfi_mem_pte[0123] fields carry the values of the page table entries used to convert rvfi_mem_addr to rvfi_mem_paddr. Unused rvfi_mem_pte[0123] fields must always be set to zero.

For memory accesses in M-mode, or with satp.MODE=0, rvfi_mem_paddr must have the same value as rvfi_mem_addr and all four rvfi_mem_pte[0123] fields must be set to zero.

For example in Sv32 mode, modulo missing fences, rvfi_mem_pte1 must carry the value of the 32-bit word at the following memory location:

pt1 = rvfi_csr_satp_rdata & 0x003fffff
vpn1 = (rvfi_mem_addr >> 22) & 0x3ff
pte1_addr = (pt1 << 12) | (vpn1 << 2)

And rvfi_mem_pte0 must carry the value of the 32-bit word at the following memory location (or zero if pte1.X or pte1.R or !pte1.V):

pt0 = rvfi_mem_pte1 >> 10
vpn0 = (rvfi_mem_addr >> 12) & 0x3ff
pte0_addr = (pt0 << 12) | (vpn0 << 2)

Finally, rvfi_mem_paddr must be set to the following address:

ppn = rvfi_mem_pte0 >> 10
offset = rvfi_mem_addr & 0xfff
rvfi_mem_paddr = (ppn << 12) | offset

### 21.2.4	Modelling of Atomic Memory Operations

AMO instructions (AMOSWAP.W, etc.) can be modelled using the existing rvfi_mem_* interface by asserting bits in both rvfi_mem_rmask and rvfi_mem_wmask.

There is also no extension to the RVFI port necessary to accommodate the LR, SC, FENCE and FENCE.I instructions.

Verification of this instructions for a single-core systems can be done using the RVFI port only. A strategy must be defined to verify their correct behavior in multicore systems.

For atomic instructions with rd = x0 a core might have no way of knowing the old or new value of the memory location. For those situations we add an additional RVFI output port:

output [NRET          - 1 : 0] rvfi_mem_extamo

When rvfi_mem_extamo is set, rvfi_mem_wdata carries the rs2 value used with the atomic instruction instead of the new value in the memory location. rvfi_mem_rmask is all-zeros in this case.

### 21.2.5	Skipping instructions

Consider the following sequence of instructions:

....
    add t0,t1,t2
    beqz t3,label
    sub t0,t1,t3
label:
    ....

When t3 has a non-zero value the processor could decide not to schedule the add instruction because its value is never going to be used. In this case the processor would be unable to produce a valid RVFI trace for the instruction sequence.

An additional signal can be added to RVFI that can be used to mark such instructions:

output [NRET        - 1 : 0] rvfi_skip

When rvfi_skip is high the core may output arbitrary data on the *_rdata and *_wdata ports (excluding rvfi_pc_rdata and rvfi_pc_wdata). The register values written by such intrustions may only be observed by other skipped instructions. An additional formal proof must be added to check this property.

Memory operations (rvfi_mem_rmask and/or rvfi_mem_wmask are non-zero) can not be skipped.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	kestrel-rv32i Testplans

YAML testplans mapping every kestrel FUB and the core top to the cocotb/pytest tests that exercise them. Format mirrors the pumice/retro_legacy_blocks testplan convention so the same coverage rollup applies (bin/cov_utils/functional_coverage_tracker.py).

## 22.1	Inventory

| Testplan | Module | Test file(s) | Scenarios | Verified | Not impl. |
| --- | --- | --- | --- | --- | --- |
| kestrel_regfile_testplan.yaml | kestrel_regfile.sv | test_kestrel_regfile.py | 5 | 5 | 0 |
| kestrel_alu_testplan.yaml | kestrel_alu.sv | test_kestrel_decode.py | 5 | 5 | 0 |
| kestrel_imm_gen_testplan.yaml | kestrel_imm_gen.sv | test_kestrel_decode.py | 6 | 6 | 0 |
| kestrel_decode_testplan.yaml | kestrel_decode.sv | test_kestrel_decode.py | 12 | 12 | 0 |
| kestrel_mem_loader_testplan.yaml | kestrel_mem_loader.sv | test_kestrel_mem_loader.py | 9 | 9 | 0 |
| kestrel_core_testplan.yaml | kestrel_core.sv | test_kestrel_{core,branch,ls,rv32ui,mem_loader,fuzz}.py | 18 | 17 | 1 |
| Total |  |  | 55 | 54 | 1 |

Not-implemented scenarios and why:

- CORE-18 — functional coverage closure / cover model (Phase C).

## 22.2	What each plan covers

- RF (kestrel_regfile): reset-to-zero, write/readback on both ports, read-during-write old-value, x0 discard, independent port addressing (attributed to the core-level golden trace, RF-05).

- ALU (kestrel_alu): exhaustive 10-op x 16-value x 16-value golden-model sweep including shamt=b[4:0], signed/unsigned compare flags. The flags are unconnected inside kestrel_core by design (branch comparator is separate) — verified at unit level only.

- IG (kestrel_imm_gen): I/S/B/U/J immediate assembly and sign extension with directed ±vectors; B/J LSB-zero rule; plus a SEED-driven randomized sweep (625 vectors, opcode+funct3 anchored per format class) against the Python imm_golden.

- DEC (kestrel_decode): the 48-encoding golden truth table (37 canonical RV32I + mret + fence/fence.i + ecall/ebreak + all 6 CSR-stub forms), illegal / reserved encodings landing on halt cause 0xF, ecall/ebreak causes 1/2. Halt cause 3 (IALIGN) is decode-invisible by design and lives in the CORE plan (CORE-13).

- ML (kestrel_mem_loader): AXIL window over both 64 KB regions, per-byte wstrb merge, CTRL.run readback/release sequencing, load-mode isolation (core_rst_n hold, parked PC, no retirement), core↔dmem cross-mux both directions, run-mode observability through the AXIL port, AXIL backpressure (bready/rready held low — bvalid/rvalid hold, busy taps, no lost beat), run-mode loader-write rejection (post-CTRL.run poison write leaves the arrays untouched, B still OKAY), and the golden-trace board path (battery images streamed over AXIL with spike lockstep).

- CORE (kestrel_core): ISA classes via golden trace, branches/jumps (incl. IALIGN halt cause 3), loads/stores incl. the cross-word 2-cycle retry, the system layer (FENCE NOPs, ECALL/EBREAK halt+hold, illegal halt, trap-beat shape), RVFI invariants, the 42-image rv32ui battery, spike lockstep, and the CORE-17 constrained-random fuzz (seeded streams vs the interpreter full-field diff plus spike lockstep at every level). Phase C gap (coverage closure) is tracked as not_implemented.

## 22.3	JUnit naming convention (observed)

Kestrel’s pytest wrappers launch one Verilator build per (cocotb testcase, test level) cell; each cell writes a cocotb JUnit XML whose testcase name is the cocotb test function name, with the pytest module as classname:

<testcase name="cocotb_test_kestrel_core_focus" classname="test_kestrel_core" .../>

So test_function strings in these plans quote the cocotb_test_* node names exactly (e.g. cocotb_test_kestrel_branch_all, cocotb_test_kestrel_rv32ui_battery). They match the tracker by exact name — there are no pytest-parametrized [case-level] node names in the cocotb XML.

Results XML layout: the wrappers set COCOTB_RESULTS_FILE to dv/tests/logs/results_<testcase>_<level>.xml. Both dv/tests/logs/ and dv/tests/local_sim_build/ are gitignored — nothing is checked in; every run regenerates them. The per-cell cocotb XMLs also land one level down at dv/tests/local_sim_build/<testcase>_<level>/<rand>_results.xml with identical testcase names, so to feed the tracker from a previous local run, flatten its XMLs first:

cd projects/components/riscv-ip/kestrel-rv32i/dv/tests
find local_sim_build -name '*_results.xml' -exec cp {} logs/ \;

## 22.4	Rollup

# From the repo root, after a regression (or after flattening a previous
# local run's XMLs into dv/tests/logs/ as above):
python3 bin/cov_utils/functional_coverage_tracker.py \
    --testplans-dir projects/components/riscv-ip/kestrel-rv32i/dv/testplans \
    --report \
    --results-dir projects/components/riscv-ip/kestrel-rv32i/dv/tests/logs

The tracker exists for per-scenario status verification: it matches each YAML’s test_function strings against the JUnit XML testcase names and reports whether the referenced cocotb tests actually ran and passed. Its functional-coverage percentage shows 0% for these plans — by design, as in the pumice house: these YAMLs carry no coverage_points blocks, so the tool has no cover data to score. The scenario-level rollup for kestrel is each YAML’s implied_coverage block: scenario_tracked counts status: verified scenarios and implied_percentage = tracked / total_scenarios. Per-scenario covers_lines attribution from Verilator .dat files is the pumice update_testplan_coverage.py workflow and is deliberately not wired in kestrel yet (CORE-18 owns the cover model).

## 22.5	Running the tests

cd projects/components/riscv-ip/kestrel-rv32i/dv/tests
make list                     # discovered test roots
make run-all-func-parallel    # FUNC regression (gate+func cells)
make run-all-full-parallel    # FULL regression (gate+func+full cells)

REG_LEVEL semantics: GATE runs gate cells only, FUNC adds func, FULL adds full. The rv32ui battery reads TEST_LEVEL inside the sim (gate = 3-image smoke, func = 42 images + spike lockstep, full = 42 images without lockstep).

## 22.6	Functional coverage (CORE-18)

Collection is wired through the central cov_utils conftest hook (dv/tests/conftest.py) — the same pattern every other component uses. To regenerate:

make clean-all
COVERAGE=1 make run-all-full
make coverage-report

Measured 2026-10-07 across all 60 cells (including the CORE-17 fuzz at full level, 200 streams):

| Metric | Result | House target | Status |
| --- | --- | --- | --- |
| Line (Verilator) | 94.9% | 80% | PASS |
| Line — kestrel_core | 100.0% | — | — |
| Line — kestrel_decode | 95.0% | — | — |
| Line — kestrel_mem_loader | 90.1% | — | — |

Toggle collection is enabled (--coverage-toggle); the house merge_testlevel_coverage.py report tracks line only — toggle numbers are ad-hoc from coverage_data/merged/latest_merged_coverage.dat via verilator_coverage. Known toggle laggards are data-path width toggles exercised thinly by directed tests; the fuzz’s random patterns are the planned ratchet. Coverage artifacts (coverage_data/, coverage_reports/, local_sim_build/, logs/) are gitignored by design; the numbers above are the committed record, regenerable with the commands in this section.