RTL Design Sherpa

Bridge Hardware Architecture Specification 1.0

January 3, 2026

Table of Contents

1 Bridge Has Index	23

2 Document Information	23

2.1 Bridge Hardware Architecture Specification	23

2.2 Revision History	23

2.3 Document Purpose	23

2.4 Intended Audience	24

2.5 Related Documents	24

3 Purpose and Scope	24

3.1 What is Bridge?	24

3.2 The Problem Bridge Solves	24

3.3 Scope	25

3.4 Out of Scope	25

4 Document Conventions	25

4.1 Terminology	25

4.2 Signal Naming	26

4.3 Diagrams	26

4.3.1 Figure 1.1: Block Diagram Legend	26

4.4 Code Examples	27

4.5 Parameter Notation	27

5 Definitions and Acronyms	27

5.1 Acronyms	27

5.2 Definitions	29

6 Use Cases	29

6.1 Primary Applications	29

6.1.1 SoC Interconnects	29

6.1.2 Memory Systems	30

6.1.3 Accelerator Systems	30

6.2 Example: RAPIDS Accelerator	31

6.2.1 Figure 2.1: RAPIDS System Topology	31

6.2.2 Configuration	31

6.2.3 Benefits	31

6.3 Rapid Prototyping	32

7 Key Features	32

7.1 Configuration-Driven Generation	32

7.1.1 CSV/TOML Configuration	32

7.1.2 Example Configuration	32

7.2 Multi-Protocol Support	33

7.2.1 Protocol Features	33

7.3 Channel-Specific Masters	33

7.3.1 Write-Only Masters (wr)	33

7.3.2 Read-Only Masters (rd)	33

7.3.3 Full Masters (rw)	33

7.3.4 Resource Savings	34

7.4 Automatic Converters	34

7.4.1 Width Conversion	34

7.4.2 Protocol Conversion	34

7.5 Flexible Topology	34

7.5.1 Scalability	34

7.5.2 Mixed Configurations	34

8 System Context	35

8.1 Bridge in SoC Architecture	35

8.1.1 Figure 2.2: Bridge System Context	35

8.2 Interface Boundaries	35

8.2.1 Master-Side (Upstream)	35

8.2.2 Slave-Side (Downstream)	36

8.3 Address Space	36

8.3.1 Address Map Organization	36

8.3.2 Address Decode	36

8.4 Clock and Reset	36

8.4.1 Clock Domain	36

8.4.2 Reset	36

8.5 Dependencies	37

8.5.1 Required Infrastructure	37

8.5.2 Optional Features	37

9 Block Diagram	37

9.1 Bridge Architecture Overview	37

9.1.1 Figure 3.1: Bridge Block Diagram	37

9.2 Functional Blocks	38

9.2.1 Master Adapter	38

9.2.2 Crossbar Core	38

9.2.3 Slave Router	38

9.3 Data Flow	39

9.3.1 Figure 3.2: Write Path	39

9.3.2 Figure 3.3: Read Path	39

9.4 Scalability	39

9.4.1 Topology Parameters	39

9.4.2 Resource Scaling	40

10 Data Flow	40

10.1 Transaction Flow Overview	40

10.1.1 Write Transaction Flow	40

10.1.2 Read Transaction Flow	40

10.2 Channel Independence	41

10.2.1 AXI4 Channel Separation	41

10.2.2 Parallel Operation	41

10.3 ID Extension	41

10.3.1 Bridge ID (BID) Concept	41

10.3.2 Example: 4 Masters, 4-bit ID	41

10.3.3 ID Flow	42

10.4 Response Routing	42

10.4.1 B Channel (Write Response)	42

10.4.2 R Channel (Read Data)	42

10.5 Out-of-Order Support	42

10.5.1 Transaction Interleaving	42

10.5.2 Example Sequence	42

11 Protocol Support	43

11.1 Supported Protocols	43

11.1.1 AXI4 Full	43

11.1.2 AXI4-Lite	43

11.1.3 APB (Advanced Peripheral Bus)	43

11.2 Protocol Conversion Matrix	44

11.3 AXI4 to APB Conversion	45

11.3.1 Conversion Process	45

11.3.2 Figure 3.1: AXI4 to APB Conversion Sequence	45

11.3.3 Burst Handling	45

11.3.4 Timing Considerations	45

11.4 Width Conversion	46

11.4.1 Upsize (Narrow to Wide)	46

11.4.2 Figure 3.2: Width Upsize Conversion	46

11.4.3 Downsize (Wide to Narrow)	47

11.4.4 Figure 3.3: Width Downsize Conversion	47

11.5 Channel-Specific Protocol	48

11.5.1 Write-Only Masters	48

11.5.2 Read-Only Masters	48

12 AXI4 Interface Overview	48

12.1 Interface Organization	48

12.1.1 Master-Side Interfaces	48

12.1.2 Slave-Side Interfaces	49

12.2 Signal Naming Convention	50

12.2.1 Custom Prefixes	50

12.2.2 ID Width Extension	50

12.3 Channel-Specific Interfaces	50

12.3.1 Write-Only Master (wr)	50

12.3.2 Read-Only Master (rd)	51

12.4 Handshake Protocol	51

12.4.1 Valid/Ready Handshake	51

12.4.2 Backpressure	51

12.5 Burst Support	52

12.5.1 Supported Burst Types	52

12.5.2 Burst Length	52

13 APB Interface Overview	52

13.1 APB Protocol Summary	52

13.1.1 Signal Definition	52

13.2 APB Slave Interface	53

13.2.1 Signal Naming	53

13.3 APB Transaction Timing	53

13.3.1 Write Transaction	53

13.3.2 Read Transaction	54

13.4 AXI4 to APB Conversion	54

13.4.1 Conversion Requirements	54

13.4.2 Burst Handling	54

13.4.3 Error Mapping	54

13.5 Performance Implications	55

13.5.1 APB Limitations	55

13.5.2 Design Recommendations	55

14 Clock and Reset	55

14.1 Clock Requirements	55

14.1.1 Single Clock Domain	55

14.1.2 Figure 4.1: Clock Distribution	56

14.1.3 No Clock Domain Crossing	56

14.2 Reset Requirements	56

14.2.1 Asynchronous Active-Low Reset	56

14.2.2 Reset Behavior	57

14.2.3 Figure 4.3: Reset Timing	57

14.2.4 Reset Effects	57

14.2.5 Outstanding Transactions	57

14.2.6 Reset Release	57

14.3 Timing Constraints	58

14.3.1 Setup and Hold	58

14.3.2 Clock-to-Output	58

14.3.3 Recommended Constraints	58

14.4 Multi-Clock Systems	58

14.4.1 External CDC Required	58

14.4.2 Figure 4.2: Multi-Clock CDC	58

14.4.3 CDC Recommendations	59

15 Throughput Characteristics	59

15.1 Peak Throughput	59

15.1.1 Single Master to Single Slave	59

15.1.2 Formula	59

15.2 Sustained Throughput	59

15.2.1 Factors Affecting Throughput	59

15.2.2 Multi-Master Scaling	60

15.2.3 Example: 4 Masters, 2 Slaves	60

15.3 Burst Efficiency	60

15.3.1 Burst vs Single-Beat	60

15.3.2 Recommendation	60

15.4 Width Conversion Impact	61

15.4.1 Upsize (Narrow to Wide)	61

15.4.2 Downsize (Wide to Narrow)	61

15.5 Protocol Conversion Impact	61

15.5.1 AXI4 to APB	61

15.5.2 Recommendation	61

16 Latency Analysis	62

16.1 Latency Components	62

16.1.1 Address Path Latency	62

16.1.2 Data Path Latency	62

16.1.3 Response Path Latency	62

16.2 End-to-End Latency	63

16.2.1 Write Transaction (Best Case)	63

16.2.2 Read Transaction (Best Case)	63

16.2.3 Contention Latency	63

16.3 Width Conversion Latency	63

16.3.1 Upsize Latency	63

16.3.2 Downsize Latency	64

16.4 Protocol Conversion Latency	64

16.4.1 AXI4 to APB	64

16.4.2 APB Wait States	64

16.5 Latency Optimization	64

16.5.1 Design Recommendations	64

17 Resource Estimates	65

17.1 FPGA Resource Summary	65

17.1.1 Baseline Configuration (2x2, 64-bit)	65

17.1.2 Scaling Factors	65

17.2 Component Breakdown	65

17.2.1 Per-Master Resources	65

17.2.2 Per-Slave Resources	66

17.2.3 Crossbar Core	66

17.3 Width Converter Resources	66

17.4 Protocol Converter Resources	67

17.4.1 AXI4 to APB	67

17.5 Example Configurations	67

17.5.1 Notes	67

17.6 Optimization Strategies	67

17.6.1 Resource Reduction	67

17.6.2 Performance/Resource Trade-off	68

18 System Requirements	68

18.1 Hardware Requirements	68

18.1.1 Clock Infrastructure	68

18.1.2 Reset Infrastructure	68

18.1.3 Power	69

18.2 Interface Requirements	69

18.2.1 Master Requirements	69

18.2.2 Slave Requirements	69

18.3 Configuration Requirements	69

18.3.1 Address Map	69

18.3.2 Connectivity	70

18.4 Timing Requirements	70

18.4.1 Setup/Hold	70

18.4.2 Recommended Margins	70

18.5 Verification Requirements	70

18.5.1 Pre-Integration Checks	70

18.5.2 Integration Checks	71

18.5.3 Post-Integration Checks	71

19 Parameter Configuration	71

19.1 Core Parameters	71

19.2 Derived Parameters	72

19.2.1 Calculated by Generator	72

19.3 Per-Port Configuration	72

19.3.1 Master Port Configuration	72

19.3.2 Slave Port Configuration	72

19.4 Configuration File Format	73

19.4.1 TOML Configuration	73

19.4.2 CSV Connectivity	74

19.5 Parameter Validation	74

19.5.1 Generator Checks	74

19.5.2 Runtime Validation	74

19.6 Example Configurations	75

19.6.1 Simple 2x2	75

19.6.2 Mixed Protocol	75

20 Verification Strategy	75

20.1 Verification Levels	75

20.1.1 Unit Level	75

20.1.2 Integration Level	76

20.1.3 System Level	76

20.2 Test Infrastructure	76

20.2.1 CocoTB Framework	76

20.2.2 Test Categories	77

20.3 Coverage Goals	77

20.3.1 Functional Coverage	77

20.3.2 Code Coverage	78

20.4 Debug Features	78

20.4.1 Waveform Capture	78

20.4.2 Debug Signals	78

20.5 Regression Testing	78

20.5.1 Continuous Integration	78

20.5.2 Test Matrix	79

20.6 Known Limitations	79

20.6.1 Test Gaps	79

20.6.2 Simulator Support	79

21 Bridge Micro-Architecture Specification Index	80

21.1 Document Organization	80

21.1.1 Front Matter	80

21.1.2 Chapter 1: Introduction	80

21.1.3 Chapter 2: Block Descriptions	80

21.1.4 Chapter 3: FSM Design	80

21.1.5 Chapter 4: ID Management	80

21.1.6 Chapter 5: Converters	81

21.1.7 Chapter 6: Generated RTL	81

21.1.8 Chapter 7: Verification	81

21.2 Quick Navigation	81

21.2.1 For New Users	81

21.2.2 For Integration	81

21.3 Visual Assets	81

21.3.1 Architecture Diagrams	82

21.3.2 FSM Diagrams	82

21.4 Component Overview	82

21.4.1 Key Features	82

21.4.2 Protocol Conversion Matrix	82

21.4.3 Design Philosophy	83

21.5 Related Documentation	83

21.5.1 Companion Specifications	83

21.5.2 Project-Level	83

21.5.3 Generator	83

21.6 Version History	83

22 Bridge: AXI4 Full Crossbar Generator - Product Requirements Document	84

22.1 Executive Summary	84

22.2 Design Philosophy	84

22.2.1 Core Principles	84

22.2.2 Architecture Philosophy	86

22.3 ⚠ CRITICAL: RTL Regeneration Requirements	86

22.4 1. Product Overview	87

22.4.1 1.1 Purpose	87

22.4.2 1.2 Target Audience	87

22.4.3 1.3 Success Criteria	87

22.4.4 1.4 Implementation Status and Phases	88

22.5 2. Architecture Overview	89

22.5.1 2.1 AXI4 vs AXIS vs APB	89

22.5.2 2.2 Block Diagram	90

22.5.3 2.3 Key Components	91

22.6 3. Functional Requirements	91

22.6.1 3.1 AXI4 Protocol Compliance	91

22.6.2 3.2 Address Decoding	92

22.6.3 3.3 Arbitration Strategy	92

22.6.4 3.4 Burst Handling	92

22.7 4. Non-Functional Requirements	92

22.7.1 4.1 Performance	92

22.7.2 4.2 Resource Usage (Estimated)	93

22.7.3 4.3 Quality Requirements	93

22.8 5. Interface Specifications	93

22.8.1 5.1 AXI4 Master Interfaces (M × 5 channels)	93

22.8.2 5.2 AXI4 Slave Interfaces (S × 5 channels)	95

22.8.3 5.3 Configuration Parameters	95

22.9 6. Performance Modeling	95

22.9.1 6.1 Analytical Model	95

22.9.2 6.2 Resource Scaling	96

22.9.3 6.3 Comparison with Other Crossbars	96

22.10 7. Generator Architecture	97

22.10.1 7.1 Python Generator Structure	97

22.10.2 7.2 Address Map Configuration	98

22.11 8. Comparison with APB and Delta	99

22.11.1 8.1 Code Reuse from APB Generator	99

22.11.2 8.2 Code Reuse from Delta Generator	99

22.11.3 8.3 Complexity Comparison	99

22.12 9. Use Cases	100

22.12.1 9.1 Multi-Core Processor Interconnect	100

22.12.2 9.2 DMA + Accelerator System	100

22.12.3 9.3 FPGA System Integration	101

22.13 10. Testing Strategy	101

22.13.1 10.1 FUB (Functional Unit Block) Tests	101

22.13.2 10.2 Integration Tests	101

22.13.3 10.3 Performance Validation	102

22.14 11. Documentation Plan	102

22.14.1 11.1 Specifications (Before Code)	102

22.14.2 11.2 Performance Analysis (Before Implementation)	102

22.14.3 11.3 Code Generation	102

22.14.4 11.4 Verification	103

22.15 12. Success Metrics	103

22.15.1 12.1 Functional Completeness	103

22.15.2 12.2 Performance Targets	103

22.15.3 12.3 Educational Value	103

22.15.4 12.4 Reusability	103

22.16 12. Attribution and Contribution Guidelines	103

22.16.1 12.1 Git Commit Attribution	103

22.17 12.2 PDF Generation Location	104

22.18 13. Future Enhancements	104

22.18.1 13.1 Short-Term (Post-Initial Release)	104

22.18.2 13.2 Long-Term	104

22.19 14. Risk Assessment	105

22.20 15. Project Timeline (Estimated)	105

22.21 16. References	105

22.22 17. Glossary	106

23 Claude Code Guide: Bridge Subsystem	106

23.1 🚨 CRITICAL: Read Architecture Document First	106

23.2 Quick Context	107

23.3 Target Architecture: Intelligent Width-Aware Routing	107

23.3.1 Efficient Multi-Width Design	107

23.3.2 Why This Architecture	108

23.3.3 Per-Master Output Paths	108

23.3.4 Benefits	108

23.3.5 Implementation Status	109

23.4 🚨 CRITICAL RULE #0: RTL Regeneration Requirements	109

23.4.1 The Golden Rule	109

23.4.2 Why This Is Non-Negotiable	109

23.4.3 Real Example (This Session)	109

23.4.4 Generator Files That Trigger Full Regeneration	109

23.4.5 The Regeneration Workflow	110

23.4.6 Symptoms of Version Mismatch	110

23.4.7 Think Like a Compiler Developer	111

23.4.8 Exception: Hand-Written RTL	111

23.5 Critical Pitfalls: slave_select_* Reversion After Handshake	111

23.5.1 The Problem: Combinational Address Decode	111

23.5.2 Visible Symptoms	111

23.5.3 The Solution (MANDATORY in All Generators)	112

23.5.4 When This Bug Appears	112

23.6 ⚠ MANDATORY: Project Organization Pattern	113

23.6.1 Required Directory Structure	113

23.6.2 Testbench Class Location (MANDATORY)	113

23.6.3 Test File Pattern (MANDATORY)	114

23.7 Critical Rules for This Subsystem	116

23.7.1 Rule #0: Attribution Format for Git Commits	116

23.7.2 Rule #0.1: Testbench Architecture - MANDATORY SEPARATION	116

23.7.3 Rule #1: All Testbenches Inherit from TBBase	117

23.7.4 Rule #2: Mandatory Testbench Methods	117

23.7.5 Rule #3: Use GAXI Components for Protocol Handling	118

23.7.6 Rule #4: Queue-Based Verification	119

23.8 TOML/CSV-Based Bridge Generator (Phase 2 Complete)	119

23.8.1 Overview	119

23.8.2 Quick Start	119

23.8.3 Configuration Format Details	120

23.8.4 Channel-Specific Masters (Phase 2 Feature)	121

23.8.5 Example: RAPIDS-Style Configuration	122

23.8.6 Common User Questions	123

23.8.7 Generator Output Structure	124

23.8.8 Testing Generated Bridges	125

23.9 Bridge Architecture Quick Reference	125

23.9.1 Generated Bridge Crossbar Structure	125

23.9.2 Key Features	125

23.9.3 Address Map	126

23.10 Test Organization	126

23.10.1 Test Hierarchy	126

23.10.2 Test Levels	126

23.11 Common User Questions and Responses	126

23.11.1 Q: “How does the Bridge work?”	126

23.11.2 Q: “How do I generate a Bridge?”	127

23.11.3 Q: “How do I test a Bridge?”	127

23.12 Anti-Patterns to Avoid	128

23.12.1 ✗ Anti-Pattern 1: Embedded Testbench Classes	128

23.12.2 ✗ Anti-Pattern 2: Manual AXI4 Handshaking	128

23.12.3 ✗ Anti-Pattern 3: Memory Models for Simple Tests	128

23.13 Quick Reference	128

23.13.1 Finding Existing Components	128

23.13.2 Common Commands	129

23.14 Remember	129

23.15 PDF Generation Location	129

List of Figures

Figure 1.1: Block Diagram Legend	26

Figure 2.1: RAPIDS System Topology	31

Figure 2.2: Bridge System Context	35

Figure 3.1: Bridge Block Diagram	37

Figure 3.2: Write Path	39

Figure 3.3: Read Path	39

Figure 3.1: AXI4 to APB Conversion Sequence	45

Figure 3.2: Width Upsize Conversion	46

Figure 3.3: Width Downsize Conversion	47

Figure 4.1: Clock Distribution	56

Figure 4.3: Reset Timing	57

Figure 4.2: Multi-Clock CDC	58

List of Tables

Table 1: Table 1.1: Common Terminology	25

Table 2: Table 1.2: Signal Naming Prefixes	26

Table 3: Table 1.3: Block Diagram Symbols	26

Table 4: Table 1.4: Parameter Format Example	27

Table 5: Table 1.5: Acronyms and Abbreviations	27

Table 6: Table 2.1: Supported Protocols	33

Table 7: Table 2.2: Channel-Specific Resource Savings	34

Table 8: Table 2.3: Address Map Organization	36

Table 9: Table 3.1: Topology Parameter Scaling	39

Table 10: Table 3.2: AXI4 Channel Summary	41

Table 11: Table 3.3: Protocol Conversion Matrix	44

Table 12: Table 4.9: Master-Side Interface Channels	48

Table 13: Table 4.10: Slave-Side Interface Channels	49

Table 14: Table 4.11: Supported Burst Types	52

Table 15: Table 4.1: APB Signal Definitions	52

Table 16: Table 4.2: AXI4 to APB Burst Conversion	54

Table 17: Table 4.3: APB to AXI4 Error Mapping	54

Table 18: Table 4.4: Clock Requirements	55

Table 19: Table 4.5: Reset Signal	56

Table 20: Table 4.6: Reset Effects	57

Table 21: Table 4.7: Timing Constraints	58

Table 22: Table 4.8: Clock-to-Output Delays	58

Table 23: Table 5.1: Peak Throughput by Data Width	59

Table 24: Table 5.2: Factors Affecting Throughput	59

Table 25: Table 5.3: Burst Efficiency Comparison	60

Table 26: Table 5.4: AXI4 vs APB Protocol Impact	61

Table 27: Table 5.5: Address Path Latency	62

Table 28: Table 5.6: Data Path Latency	62

Table 29: Table 5.7: Response Path Latency	62

Table 30: Table 5.8: Upsize Latency by Ratio	63

Table 31: Table 5.9: Downsize Latency by Ratio	64

Table 32: Table 5.10: AXI4 to APB Conversion Latency	64

Table 33: Table 5.11: Baseline Resource Requirements	65

Table 34: Table 5.12: Resource Scaling Factors	65

Table 35: Table 5.13: Per-Master Resource Breakdown	65

Table 36: Table 5.14: Per-Slave Resource Breakdown	66

Table 37: Table 5.15: Crossbar Core Resources	66

Table 38: Table 5.16: Width Converter Resources	66

Table 39: Table 5.17: AXI4 to APB Converter Resources	67

Table 40: Table 5.18: Complete Bridge Resource Estimates	67

Table 41: Table 5.19: Performance/Resource Trade-offs	68

Table 42: Table 6.1: Clock Requirements	68

Table 43: Table 6.2: Reset Requirements	68

Table 44: Table 6.3: Power Requirements	69

Table 45: Table 6.4: Address Map Requirements	69

Table 46: Table 6.5: Connectivity Requirements	70

Table 47: Table 6.6: Timing Requirements	70

Table 48: Table 6.7: Recommended Timing Margins	70

Table 49: Table 6.8: Bridge Core Parameters	71

Table 50: Table 6.9: Derived Parameters	72

Table 51: Table 6.10: Master Port Configuration	72

Table 52: Table 6.11: Slave Port Configuration	72

Table 53: Table 6.12: Generator Validation Checks	74

Table 54: Table 6.13: Unit Level Test Focus	76

Table 55: Table 6.14: Integration Test Coverage	76

Table 56: Table 6.15: System Level Tests	76

Table 57: Table 6.16: Test Categories	77

Table 58: Table 6.17: Functional Coverage Goals	77

Table 59: Table 6.18: Code Coverage Goals	78

Table 60: Table 6.19: Debug Signals	78

Table 61: Table 6.20: CI Test Schedule	78

Table 62: Table 6.21: Configuration Test Matrix	79

Table 63: Table 6.22: Known Test Gaps	79

Table 64: Table 6.23: Simulator Support Status	79

# 1	Bridge Has Index

Generated: 2026-05-14

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	Bridge Hardware Architecture Specification

| Property | Value |
| --- | --- |
| Document Title | Bridge Hardware Architecture Specification |
| Version | 1.0 |
| Date | January 3, 2026 |
| Status | Released |
| Classification | Open Source - Apache 2.0 License |

## 2.2	Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-01-03 | RTL Design Sherpa | Initial release - restructured from single spec |

## 2.3	Document Purpose

This Hardware Architecture Specification (HAS) provides a high-level view of the Bridge component, covering:

- System-level architecture and data flow

- Protocol support (AXI4, AXI4-Lite, APB)

- Interface definitions and requirements

- Performance characteristics

- Integration guidelines

For detailed micro-architecture, FSM designs, and signal-level specifications, refer to the companion Bridge Micro-Architecture Specification (MAS).

## 2.4	Intended Audience

- System architects defining interconnect topology

- Hardware engineers integrating Bridge into SoC designs

- Verification engineers planning test strategies

- Technical managers evaluating Bridge capabilities

## 2.5	Related Documents

- Bridge MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Purpose and Scope

## 3.1	What is Bridge?

Bridge is a Python-based multi-protocol crossbar generator that creates parameterized SystemVerilog RTL from human-readable configuration files. It generates AXI4 crossbars with automatic support for:

- Protocol conversion - AXI4, AXI4-Lite, and APB slave interfaces

- Width conversion - Automatic upsize/downsize for data width mismatches

- Channel-specific masters - Write-only, read-only, or full AXI4 masters

## 3.2	The Problem Bridge Solves

Manual AXI4 Crossbar Development is Error-Prone:

Creating an AXI4 crossbar manually requires: 1. Writing 5 separate channel multiplexers (AW, W, B, AR, R) 2. Implementing per-slave arbitration for each channel 3. Managing ID-based response routing for out-of-order support 4. Handling burst locking and interleaving constraints 5. Inserting width converters for data width mismatches 6. Inserting protocol converters for APB/AXI4-Lite mixed systems 7. Wiring hundreds of signals with consistent naming

Bridge Automates All of This: - Define ports and connectivity in configuration files - Run generator script - Get production-ready SystemVerilog RTL

## 3.3	Scope

This specification covers:

- High-level architecture and block organization

- Supported protocols and conversion capabilities

- Interface requirements and signal conventions

- Performance characteristics and resource estimates

- Integration guidelines and verification strategy

## 3.4	Out of Scope

The following are covered in the companion MAS:

- Detailed FSM state diagrams and transitions

- Signal-level timing diagrams

- Internal pipeline structures

- RTL implementation details

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Document Conventions

## 4.1	Terminology

Table 1.1: Common Terminology

| Term | Definition |
| --- | --- |
| Master | AXI4 initiator that issues transactions |
| Slave | AXI4 target that responds to transactions |
| Crossbar | NxM interconnect connecting masters to slaves |
| Channel | AXI4 communication path (AW, W, B, AR, R) |
| Converter | Logic that transforms width or protocol |

## 4.2	Signal Naming

Bridge uses consistent signal prefixes:

Table 1.2: Signal Naming Prefixes

| Prefix | Meaning |
| --- | --- |
| s_* | Slave-side interface (master port on bridge) |
| m_* | Master-side interface (slave port on bridge) |
| axi4_* | AXI4 protocol signals |
| apb_* | APB protocol signals |
| cfg_* | Configuration signals |
| dbg_* | Debug signals |

## 4.3	Diagrams

### 4.3.1	Figure 1.1: Block Diagram Legend

Block diagrams in this specification use the following conventions:

Table 1.3: Block Diagram Symbols

| Symbol | Meaning |
| --- | --- |
| Rectangle | Functional block |
| Arrow | Data/signal flow |
| Dashed box | Optional/configurable block |
| Double line | Multi-signal bus |

## 4.4	Code Examples

SystemVerilog code examples are formatted as:

// Module instantiation example
bridge_4x4 u_bridge (
    .aclk       (sys_clk),
    .aresetn    (sys_rst_n),
    // ... port connections
);

## 4.5	Parameter Notation

Parameters use the following format:

Table 1.4: Parameter Format Example

| Parameter | Type | Range | Default | Description |
| --- | --- | --- | --- | --- |
| NUM_MASTERS | int | 1-32 | 2 | Number of master ports |
| NUM_SLAVES | int | 1-256 | 2 | Number of slave ports |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Definitions and Acronyms

## 5.1	Acronyms

Table 1.5: Acronyms and Abbreviations

| Acronym | Definition |
| --- | --- |
| AMBA | Advanced Microcontroller Bus Architecture |
| APB | Advanced Peripheral Bus |
| AR | AXI4 Read Address channel |
| AW | AXI4 Write Address channel |
| AXI | Advanced eXtensible Interface |
| AXI4 | AXI Protocol Version 4 |
| B | AXI4 Write Response channel |
| BID | Bridge ID (internal master identifier) |
| CAM | Content-Addressable Memory |
| CDC | Clock Domain Crossing |
| CSV | Comma-Separated Values |
| DECERR | Decode Error (AXI response) |
| DMA | Direct Memory Access |
| FIFO | First-In First-Out buffer |
| FSM | Finite State Machine |
| HAS | Hardware Architecture Specification |
| ID | Transaction Identifier |
| MAS | Micro-Architecture Specification |
| MUX | Multiplexer |
| NxM | N masters by M slaves (topology notation) |
| OOO | Out-of-Order |
| OOR | Out-of-Range |
| QoS | Quality of Service |
| R | AXI4 Read Data channel |
| RTL | Register-Transfer Level |
| SLVERR | Slave Error (AXI response) |
| SoC | System-on-Chip |
| TOML | Tom’s Obvious Minimal Language |
| W | AXI4 Write Data channel |

## 5.2	Definitions

Address Decode: The process of determining which slave should receive a transaction based on the address.

Arbitration: The process of selecting which master gains access to a slave when multiple masters contend.

Bridge ID (BID): Internal identifier prepended to master IDs for response routing. Width = clog2(NUM_MASTERS).

Channel-Specific Master: A master that only uses subset of AXI4 channels (write-only or read-only).

Crossbar: An NxM interconnect allowing any master to communicate with any connected slave.

Grant Locking: Mechanism that holds arbitration grant until transaction completes.

Out-of-Order Response: Responses returned in different order than requests were issued.

Protocol Conversion: Transformation between different bus protocols (e.g., AXI4 to APB).

Response Routing: Directing B/R channel responses back to the originating master.

Round-Robin Arbitration: Fair arbitration where priority rotates among requesters.

Width Conversion: Transformation between different data bus widths (upsize or downsize).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Use Cases

## 6.1	Primary Applications

### 6.1.1	SoC Interconnects

Bridge serves as the primary interconnect fabric for System-on-Chip designs:

- Multi-core processor memory subsystems - Connect multiple CPU cores to shared memory

- Accelerator integration - Route GPU, DSP, or custom accelerator traffic

- DMA engine routing - Connect DMA controllers to memory and peripherals

- Peripheral bus bridges - Convert AXI4 to APB for low-speed peripherals

### 6.1.2	Memory Systems

Bridge provides flexible memory access:

- Multi-port DDR controllers - Multiple masters accessing shared DDR

- SRAM buffer sharing - Shared packet buffers for networking

- Cache coherency interconnects - Connect cache controllers

- Memory-mapped I/O - Unified address space for peripherals

### 6.1.3	Accelerator Systems

Bridge supports custom accelerator architectures like RAPIDS:

- Descriptor write masters - Write control descriptors (write-only)

- Sink write masters - Write incoming data packets (write-only)

- Source read masters - Read outgoing data packets (read-only)

- CPU configuration - Full access for register configuration

## 6.2	Example: RAPIDS Accelerator

### 6.2.1	Figure 2.1: RAPIDS System Topology

RAPIDS System Topology

Bridge topology for RAPIDS accelerator with channel-specific masters.

### 6.2.2	Configuration

[bridge]
  name = "bridge_rapids"
  masters = [
    {name = "rapids_descr_wr", channels = "wr", data_width = 512},
    {name = "rapids_sink_wr", channels = "wr", data_width = 512},
    {name = "rapids_src_rd", channels = "rd", data_width = 512},
    {name = "cpu", channels = "rw", data_width = 64}
  ]
  slaves = [
    {name = "ddr", protocol = "axi4", data_width = 512},
    {name = "apb_periph", protocol = "apb", data_width = 32},
    {name = "sram", protocol = "axi4", data_width = 256}
  ]

### 6.2.3	Benefits

- 39% port reduction for write-only masters (3 channels vs 5)

- 61% port reduction for read-only masters (2 channels vs 5)

- Automatic width conversion for 64b CPU to 512b DDR

- Protocol conversion for APB peripheral access

## 6.3	Rapid Prototyping

Bridge accelerates design exploration:

- Quick topology exploration - Try different master/slave configurations

- Performance analysis - Evaluate arbitration and bandwidth

- Design space exploration - Compare resource usage

- Educational projects - Learn AXI4 protocol and interconnects

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Key Features

## 7.1	Configuration-Driven Generation

### 7.1.1	CSV/TOML Configuration

Bridge uses human-readable configuration files:

- ports configuration - Define each master and slave port

- connectivity matrix - Specify which masters connect to which slaves

- Version-control friendly - Text-based, easy to diff and review

### 7.1.2	Example Configuration

Port Definition (TOML):

masters = [
  {name = "cpu", prefix = "cpu_m_axi", data_width = 64, channels = "rw"},
  {name = "dma", prefix = "dma_m_axi", data_width = 256, channels = "rw"}
]
slaves = [
  {name = "ddr", prefix = "ddr_s_axi", data_width = 512, protocol = "axi4"},
  {name = "uart", prefix = "uart_apb", data_width = 32, protocol = "apb"}
]

Connectivity Matrix (CSV):

master\slave,ddr,uart
cpu,1,1
dma,1,0

## 7.2	Multi-Protocol Support

Table 2.1: Supported Protocols

| Master Protocol | Slave Protocol | Conversion Type |
| --- | --- | --- |
| AXI4 | AXI4 | Direct or width convert |
| AXI4 | AXI4-Lite | Protocol downgrade |
| AXI4 | APB | Full protocol conversion |

### 7.2.1	Protocol Features

- AXI4 Full - Complete 5-channel implementation with bursts

- AXI4-Lite - Simplified single-beat transactions

- APB - Low-power peripheral access

## 7.3	Channel-Specific Masters

### 7.3.1	Write-Only Masters (wr)

Generate only write channels: - AW (Write Address) - W (Write Data) - B (Write Response)

Use case: DMA write engines, descriptor writers

### 7.3.2	Read-Only Masters (rd)

Generate only read channels: - AR (Read Address) - R (Read Data)

Use case: DMA read engines, source data fetchers

### 7.3.3	Full Masters (rw)

Generate all five channels for complete read/write capability.

Use case: CPU interfaces, configuration masters

### 7.3.4	Resource Savings

Table 2.2: Channel-Specific Resource Savings

| Master Type | Channels | Signal Reduction |
| --- | --- | --- |
| Write-only (wr) | 3 of 5 | ~39% fewer signals |
| Read-only (rd) | 2 of 5 | ~61% fewer signals |
| Full (rw) | 5 of 5 | Baseline |

## 7.4	Automatic Converters

### 7.4.1	Width Conversion

Bridge automatically inserts width converters:

- Upsize - Narrow master to wide slave (e.g., 64b to 512b)

- Downsize - Wide master to narrow slave (e.g., 512b to 32b)

- Per-path - Only where needed, not global conversion

### 7.4.2	Protocol Conversion

Bridge inserts protocol converters for mixed systems:

- AXI4 to APB - Full conversion with burst handling

- AXI4 to AXI4-Lite - Simplified conversion

## 7.5	Flexible Topology

### 7.5.1	Scalability

- Masters: 1-32 master ports

- Slaves: 1-256 slave ports

- Partial connectivity - Not all masters need access to all slaves

### 7.5.2	Mixed Configurations

- Mixed protocols - AXI4, AXI4-Lite, and APB slaves in same crossbar

- Mixed data widths - 32b, 64b, 128b, 256b, 512b

- Mixed channels - Write-only, read-only, and full masters

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	System Context

## 8.1	Bridge in SoC Architecture

### 8.1.1	Figure 2.2: Bridge System Context

Bridge System Context

Bridge as central interconnect connecting masters to slaves with protocol conversion.

## 8.2	Interface Boundaries

### 8.2.1	Master-Side (Upstream)

Bridge presents slave interfaces to upstream masters:

- Accepts AXI4 transactions from masters

- Provides flow control via ready signals

- Returns responses (B/R channels) to correct master

### 8.2.2	Slave-Side (Downstream)

Bridge presents master interfaces to downstream slaves:

- Issues AXI4/APB transactions to slaves

- Accepts responses from slaves

- Routes responses back through ID tracking

## 8.3	Address Space

### 8.3.1	Address Map Organization

Each slave occupies a configurable address region:

Table 2.3: Address Map Organization

| Slave | Base Address | Address Range | Size |
| --- | --- | --- | --- |
| Slave 0 | BASE + 0x0000_0000 | Configurable | Variable |
| Slave 1 | BASE + addr_range_0 | Configurable | Variable |
| Slave N | Sum of previous ranges | Configurable | Variable |

### 8.3.2	Address Decode

- Parallel decode - All slaves checked simultaneously

- One-hot result - Exactly one slave selected per transaction

- Out-of-range detection - DECERR for unmapped addresses

## 8.4	Clock and Reset

### 8.4.1	Clock Domain

- Single clock domain - All Bridge logic synchronous to aclk

- No CDC - Masters and slaves must be in same clock domain

### 8.4.2	Reset

- Active-low asynchronous - Standard aresetn convention

- Full reset - All internal state cleared on reset

- Transaction safety - Outstanding transactions aborted on reset

## 8.5	Dependencies

### 8.5.1	Required Infrastructure

- AXI4-compliant masters and slaves

- Proper clock and reset distribution

- Address map configuration matching slave address ranges

### 8.5.2	Optional Features

- Width converters (only if widths mismatch)

- Protocol converters (only if protocols differ)

- ID tracking CAM (only if OOO support needed)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Block Diagram

## 9.1	Bridge Architecture Overview

### 9.1.1	Figure 3.1: Bridge Block Diagram

Bridge Block Diagram

Bridge high-level block diagram showing master adapters, crossbar core, and slave routers.

## 9.2	Functional Blocks

### 9.2.1	Master Adapter

Receives transactions from upstream masters:

- Skid buffer - Pipeline registration for timing

- ID extension - Prepend Bridge ID for response routing

- Channel separation - Route AW/W/AR independently

### 9.2.2	Crossbar Core

Central switching fabric:

- 5-channel routing - Independent AW, W, B, AR, R paths

- Address decode - Determine target slave

- Arbitration - Per-slave round-robin selection

### 9.2.3	Slave Router

Delivers transactions to downstream slaves:

- Protocol conversion - AXI4 to APB/AXI4-Lite if needed

- Width conversion - Upsize/downsize data paths

- Response collection - Route B/R back to masters

## 9.3	Data Flow

### 9.3.1	Figure 3.2: Write Path

Write Path Flow

### 9.3.2	Figure 3.3: Read Path

Read Path Flow

## 9.4	Scalability

### 9.4.1	Topology Parameters

Table 3.1: Topology Parameter Scaling

| Parameter | Range | Impact |
| --- | --- | --- |
| NUM_MASTERS | 1-32 | Linear MUX growth |
| NUM_SLAVES | 1-256 | Linear decoder growth |
| DATA_WIDTH | 32-512 | Linear data path growth |
| ID_WIDTH | 1-16 | Logarithmic CAM growth |

### 9.4.2	Resource Scaling

- Logic: O(M x N) for crossbar core

- Routing: O(M + N) for address/response paths

- Memory: O(M x outstanding) for ID tracking

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Data Flow

## 10.1	Transaction Flow Overview

### 10.1.1	Write Transaction Flow

- Master issues AW - Address and control information

- Bridge decodes address - Determines target slave

- Arbitration - Grants access if multiple masters contend

- AW forwarded to slave - With extended ID (Bridge ID prepended)

- Master sends W data - Following the granted AW

- W forwarded to slave - Using same grant

- Slave returns B response - With extended ID

- Bridge routes B to master - Using ID to find originator

### 10.1.2	Read Transaction Flow

- Master issues AR - Address and control information

- Bridge decodes address - Determines target slave

- Arbitration - Grants access if multiple masters contend

- AR forwarded to slave - With extended ID

- Slave returns R data - Potentially multiple beats

- Bridge routes R to master - Using ID to find originator

## 10.2	Channel Independence

### 10.2.1	AXI4 Channel Separation

Each AXI4 channel operates independently:

Table 3.2: AXI4 Channel Summary

| Channel | Direction | Contents | Arbitration |
| --- | --- | --- | --- |
| AW | Master to Slave | Write address | Per-slave |
| W | Master to Slave | Write data | Follows AW grant |
| B | Slave to Master | Write response | ID-based routing |
| AR | Master to Slave | Read address | Per-slave |
| R | Slave to Master | Read data | ID-based routing |

### 10.2.2	Parallel Operation

- Read and write transactions can proceed simultaneously

- Different masters can access different slaves in parallel

- Same slave serializes access via arbitration

## 10.3	ID Extension

### 10.3.1	Bridge ID (BID) Concept

Original Master ID: [ID_WIDTH-1:0]
Extended ID: [BID | Original ID]
BID Width: clog2(NUM_MASTERS)

### 10.3.2	Example: 4 Masters, 4-bit ID

Master 0 ID = 4'b0101
Extended ID = 6'b00_0101  (BID=00, prepended)

Master 3 ID = 4'b1100
Extended ID = 6'b11_1100  (BID=11, prepended)

### 10.3.3	ID Flow

Master 0: ID=5 --[Extend]--> Slave: ID=0x05 --[Response]--> B.ID=0x05 --[Extract]--> Master 0
Master 1: ID=5 --[Extend]--> Slave: ID=0x15 --[Response]--> B.ID=0x15 --[Extract]--> Master 1

## 10.4	Response Routing

### 10.4.1	B Channel (Write Response)

- Slave issues B with extended ID

- Bridge extracts BID from upper bits

- BID determines destination master

- Original ID (lower bits) returned to master

### 10.4.2	R Channel (Read Data)

- Slave issues R with extended ID

- Bridge extracts BID from upper bits

- BID determines destination master

- Original ID (lower bits) returned to master

- RLAST indicates final beat

## 10.5	Out-of-Order Support

### 10.5.1	Transaction Interleaving

Bridge supports out-of-order completion:

- Different IDs can complete in any order

- Same ID must complete in order (AXI4 rule)

- ID tracking tables manage outstanding transactions

### 10.5.2	Example Sequence

Time 0: Master 0 issues AR (ID=1) to Slave 0
Time 1: Master 0 issues AR (ID=2) to Slave 1
Time 2: Slave 1 returns R (ID=2) - fast slave
Time 3: Slave 0 returns R (ID=1) - slow slave

Result: ID=2 response arrives before ID=1 - valid OOO

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Protocol Support

## 11.1	Supported Protocols

### 11.1.1	AXI4 Full

Complete AMBA AXI4 protocol support:

- 5 channels: AW, W, B, AR, R

- Burst transactions: FIXED, INCR, WRAP

- Transaction IDs: Configurable width

- Out-of-order: Response reordering supported

- Data widths: 32, 64, 128, 256, 512 bits

### 11.1.2	AXI4-Lite

Simplified AXI4 for register access:

- 5 channels: Same as AXI4

- Single-beat only: AWLEN/ARLEN = 0

- No IDs: Transaction ordering by channel

- Fixed width: Typically 32 or 64 bits

### 11.1.3	APB (Advanced Peripheral Bus)

Low-power peripheral protocol:

- Single channel: Combined address and data

- Simple handshake: PSEL, PENABLE, PREADY

- No bursts: Single transfer per transaction

- Low overhead: Minimal logic for slow peripherals

## 11.2	Protocol Conversion Matrix

Table 3.3: Protocol Conversion Matrix

| Master | Slave | Conversion Type | Notes |
| --- | --- | --- | --- |
| AXI4 | AXI4 | Direct | Width conversion if needed |
| AXI4 | AXI4-Lite | Downgrade | Burst split to single beats |
| AXI4 | APB | Full convert | Channel and timing conversion |
| AXI4-Lite | AXI4 | Upgrade | Single-beat pass-through |
| AXI4-Lite | AXI4-Lite | Direct | Width conversion if needed |
| AXI4-Lite | APB | Convert | Simplified conversion |

## 11.3	AXI4 to APB Conversion

### 11.3.1	Conversion Process

### 11.3.2	Figure 3.1: AXI4 to APB Conversion Sequence

AXI4 to APB Conversion

### 11.3.3	Burst Handling

- AXI4 bursts split into individual APB transfers

- WSTRB converted to per-byte enables

- B response generated after all beats complete

### 11.3.4	Timing Considerations

- APB is inherently slower (minimum 2 cycles per transfer)

- Pipeline stalls during APB access

- Consider separate APB crossbar for multiple slow peripherals

## 11.4	Width Conversion

### 11.4.1	Upsize (Narrow to Wide)

### 11.4.2	Figure 3.2: Width Upsize Conversion

Width Upsize

### 11.4.3	Downsize (Wide to Narrow)

### 11.4.4	Figure 3.3: Width Downsize Conversion

Width Downsize

## 11.5	Channel-Specific Protocol

### 11.5.1	Write-Only Masters

Only AW, W, B channels active:

- AR permanently deasserted

- R channel ignored (no routing logic)

- Reduced signal count and logic

### 11.5.2	Read-Only Masters

Only AR, R channels active:

- AW, W, B channels ignored

- No write arbitration for this master

- Reduced signal count and logic

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	AXI4 Interface Overview

## 12.1	Interface Organization

### 12.1.1	Master-Side Interfaces

Bridge presents slave interfaces to upstream masters. Each master port includes:

Table 4.9: Master-Side Interface Channels

| Channel | Signals | Direction (to Bridge) |
| --- | --- | --- |
| AW | AWVALID, AWREADY, AWADDR, AWID, AWLEN, AWSIZE, AWBURST, … | Input |
| W | WVALID, WREADY, WDATA, WSTRB, WLAST | Input |
| B | BVALID, BREADY, BID, BRESP | Output |
| AR | ARVALID, ARREADY, ARADDR, ARID, ARLEN, ARSIZE, ARBURST, … | Input |
| R | RVALID, RREADY, RDATA, RID, RRESP, RLAST | Output |

### 12.1.2	Slave-Side Interfaces

Bridge presents master interfaces to downstream slaves. Each slave port includes:

Table 4.10: Slave-Side Interface Channels

| Channel | Signals | Direction (from Bridge) |
| --- | --- | --- |
| AW | AWVALID, AWREADY, AWADDR, AWID, AWLEN, AWSIZE, AWBURST, … | Output |
| W | WVALID, WREADY, WDATA, WSTRB, WLAST | Output |
| B | BVALID, BREADY, BID, BRESP | Input |
| AR | ARVALID, ARREADY, ARADDR, ARID, ARLEN, ARSIZE, ARBURST, … | Output |
| R | RVALID, RREADY, RDATA, RID, RRESP, RLAST | Input |

## 12.2	Signal Naming Convention

### 12.2.1	Custom Prefixes

Each port uses a custom prefix for signal naming:

// Example: Master 0 with prefix "cpu_m_axi_"
input  logic [31:0] cpu_m_axi_awaddr,
input  logic [3:0]  cpu_m_axi_awid,
input  logic        cpu_m_axi_awvalid,
output logic        cpu_m_axi_awready,
// ... remaining AW signals

// Example: Slave 0 with prefix "ddr_s_axi_"
output logic [31:0] ddr_s_axi_awaddr,
output logic [7:0]  ddr_s_axi_awid,  // Extended ID
output logic        ddr_s_axi_awvalid,
input  logic        ddr_s_axi_awready,

### 12.2.2	ID Width Extension

Slave-side IDs are wider than master-side:

Master ID Width: ID_WIDTH (configuration parameter)
Bridge ID Width: clog2(NUM_MASTERS)
Slave ID Width: ID_WIDTH + clog2(NUM_MASTERS)

## 12.3	Channel-Specific Interfaces

### 12.3.1	Write-Only Master (wr)

Only AW, W, B channels generated:

// Write Address Channel
input  logic [ADDR_WIDTH-1:0] descr_m_axi_awaddr,
input  logic [ID_WIDTH-1:0]   descr_m_axi_awid,
// ... full AW channel

// Write Data Channel
input  logic [DATA_WIDTH-1:0] descr_m_axi_wdata,
input  logic [STRB_WIDTH-1:0] descr_m_axi_wstrb,
// ... full W channel

// Write Response Channel
output logic [ID_WIDTH-1:0]   descr_m_axi_bid,
output logic [1:0]            descr_m_axi_bresp,
// ... full B channel

// NO AR or R channels

### 12.3.2	Read-Only Master (rd)

Only AR, R channels generated:

// Read Address Channel
input  logic [ADDR_WIDTH-1:0] src_m_axi_araddr,
input  logic [ID_WIDTH-1:0]   src_m_axi_arid,
// ... full AR channel

// Read Data Channel
output logic [DATA_WIDTH-1:0] src_m_axi_rdata,
output logic [ID_WIDTH-1:0]   src_m_axi_rid,
// ... full R channel

// NO AW, W, or B channels

## 12.4	Handshake Protocol

### 12.4.1	Valid/Ready Handshake

All channels use AXI4 valid/ready handshake:

Transfer occurs when: VALID && READY

### 12.4.2	Backpressure

- Bridge asserts READY when able to accept

- Masters must hold VALID until READY

- Bridge does not drop transactions

## 12.5	Burst Support

### 12.5.1	Supported Burst Types

Table 4.11: Supported Burst Types

| AWBURST/ARBURST | Type | Description |
| --- | --- | --- |
| 2’b00 | FIXED | Same address each beat |
| 2’b01 | INCR | Incrementing address |
| 2’b10 | WRAP | Wrapping at boundary |
| 2’b11 | Reserved | Not used |

### 12.5.2	Burst Length

- AWLEN/ARLEN: 8-bit field

- Length = AXLEN + 1 (1 to 256 beats)

- Burst does not cross 4KB boundary (AXI4 rule)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	APB Interface Overview

## 13.1	APB Protocol Summary

### 13.1.1	Signal Definition

Table 4.1: APB Signal Definitions

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| PSEL | 1 | Output | Slave select |
| PENABLE | 1 | Output | Transaction phase |
| PADDR | ADDR_WIDTH | Output | Address |
| PWRITE | 1 | Output | Write enable |
| PWDATA | DATA_WIDTH | Output | Write data |
| PSTRB | DATA_WIDTH/8 | Output | Byte strobes |
| PPROT | 3 | Output | Protection |
| PRDATA | DATA_WIDTH | Input | Read data |
| PSLVERR | 1 | Input | Slave error |
| PREADY | 1 | Input | Slave ready |

## 13.2	APB Slave Interface

### 13.2.1	Signal Naming

APB slave ports use custom prefixes:

// Example: APB slave with prefix "uart_apb_"
output logic        uart_apb_psel,
output logic        uart_apb_penable,
output logic [31:0] uart_apb_paddr,
output logic        uart_apb_pwrite,
output logic [31:0] uart_apb_pwdata,
output logic [3:0]  uart_apb_pstrb,
output logic [2:0]  uart_apb_pprot,
input  logic [31:0] uart_apb_prdata,
input  logic        uart_apb_pslverr,
input  logic        uart_apb_pready

## 13.3	APB Transaction Timing

### 13.3.1	Write Transaction

___     ___     ___     ___
PCLK   |   |___|   |___|   |___|   |___
            _________
PSEL   ____|         |________________
                _____
PENABLE ________|    |________________
            _________
PWRITE  ____|        |________________
       xxxxxx|  ADDR |xxxxxxxxxxxxxxxx
PADDR  xxxxxx|_______|xxxxxxxxxxxxxxxx
       xxxxxx| WDATA |xxxxxxxxxxxxxxxx
PWDATA xxxxxx|_______|xxxxxxxxxxxxxxxx
                _____
PREADY  ________|    |________________

### 13.3.2	Read Transaction

___     ___     ___     ___
PCLK   |   |___|   |___|   |___|   |___
            _________
PSEL   ____|         |________________
                _____
PENABLE ________|    |________________
PWRITE  ______________________________
       xxxxxx|  ADDR |xxxxxxxxxxxxxxxx
PADDR  xxxxxx|_______|xxxxxxxxxxxxxxxx
                _____
PREADY  ________|    |________________
       xxxxxxxx|RDATA|xxxxxxxxxxxxxxxx
PRDATA xxxxxxxx|_____|xxxxxxxxxxxxxxxx

## 13.4	AXI4 to APB Conversion

### 13.4.1	Conversion Requirements

When an AXI4 master accesses an APB slave, Bridge performs:

- Burst splitting - AXI4 bursts become multiple APB transfers

- Channel mapping - AW+W combined into PADDR+PWDATA

- Response generation - APB PSLVERR maps to AXI4 BRESP/RRESP

- Timing adaptation - Insert wait states as needed

### 13.4.2	Burst Handling

Table 4.2: AXI4 to APB Burst Conversion

| AXI4 AWLEN | APB Transfers |
| --- | --- |
| 0 (1 beat) | 1 transfer |
| 1 (2 beats) | 2 transfers |
| N (N+1 beats) | N+1 transfers |

### 13.4.3	Error Mapping

Table 4.3: APB to AXI4 Error Mapping

| APB PSLVERR | AXI4 BRESP/RRESP |
| --- | --- |
| 0 (OK) | 2’b00 (OKAY) |
| 1 (Error) | 2’b10 (SLVERR) |

## 13.5	Performance Implications

### 13.5.1	APB Limitations

- Minimum 2 cycles per transfer (setup + access phase)

- No pipelining (sequential transfers only)

- No bursts (each beat requires full handshake)

### 13.5.2	Design Recommendations

- Use APB only for slow peripherals (UART, GPIO, config registers)

- Group APB slaves behind dedicated sub-crossbar

- Use AXI4/AXI4-Lite for higher-bandwidth slaves

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Clock and Reset

## 14.1	Clock Requirements

### 14.1.1	Single Clock Domain

Bridge operates in a single synchronous clock domain:

Table 4.4: Clock Requirements

| Signal | Description | Requirements |
| --- | --- | --- |
| aclk | System clock | All interfaces synchronous |
| Frequency | Operating frequency | Design-dependent |
| Duty cycle | Clock duty cycle | 40-60% typical |

### 14.1.2	Figure 4.1: Clock Distribution

Clock Distribution

All masters and slaves must be in the same clock domain as Bridge.

### 14.1.3	No Clock Domain Crossing

Bridge does not include CDC logic:

- All inputs sampled on aclk rising edge

- All outputs generated on aclk rising edge

- External CDC required if masters/slaves use different clocks

## 14.2	Reset Requirements

### 14.2.1	Asynchronous Active-Low Reset

Table 4.5: Reset Signal

| Signal | Description | Polarity |
| --- | --- | --- |
| aresetn | Async reset | Active-low |

### 14.2.2	Reset Behavior

### 14.2.3	Figure 4.3: Reset Timing

Reset Timing

The diagram shows the relationship between clock and reset signals. Reset is active-low, meaning the system is in reset when aresetn = 0.

### 14.2.4	Reset Effects

During reset (aresetn = 0):

Table 4.6: Reset Effects

| Component | State |
| --- | --- |
| Arbiters | Cleared (no grants) |
| ID tables | Cleared (no outstanding) |
| FIFOs | Emptied |
| Outputs | Deasserted (VALID = 0) |

### 14.2.5	Outstanding Transactions

Warning: Transactions in flight during reset are lost:

- Master may see timeout (no response)

- Slave may receive partial transaction

- System must ensure quiescence before reset

### 14.2.6	Reset Release

After reset release (aresetn = 1):

- Bridge accepts new transactions after 1 cycle

- All state machines start in IDLE

- No residual grants or locks

## 14.3	Timing Constraints

### 14.3.1	Setup and Hold

All inputs must meet setup/hold relative to aclk:

Table 4.7: Timing Constraints

| Constraint | Typical Value |
| --- | --- |
| Setup time | Process-dependent |
| Hold time | Process-dependent |

### 14.3.2	Clock-to-Output

All outputs appear after aclk rising edge:

Table 4.8: Clock-to-Output Delays

| Path | Typical Delay |
| --- | --- |
| aclk to VALID | 1 cycle (registered) |
| aclk to READY | Combinational |
| aclk to DATA | 1 cycle (registered) |

### 14.3.3	Recommended Constraints

# Example SDC constraints
create_clock -period 10 [get_ports aclk]  ;# 100 MHz

# Reset is async - synchronize externally
set_false_path -from [get_ports aresetn]

## 14.4	Multi-Clock Systems

### 14.4.1	External CDC Required

For systems with multiple clock domains:

### 14.4.2	Figure 4.2: Multi-Clock CDC

Multi-Clock CDC

### 14.4.3	CDC Recommendations

- Use AXI4 async FIFO bridges

- Consider AXI4 clock converter IP

- Ensure proper gray-coding for pointers

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Throughput Characteristics

## 15.1	Peak Throughput

### 15.1.1	Single Master to Single Slave

Table 5.1: Peak Throughput by Data Width

| Data Width | Frequency | Peak Throughput |
| --- | --- | --- |
| 32-bit | 100 MHz | 400 MB/s |
| 64-bit | 100 MHz | 800 MB/s |
| 128-bit | 100 MHz | 1.6 GB/s |
| 256-bit | 100 MHz | 3.2 GB/s |
| 512-bit | 100 MHz | 6.4 GB/s |

### 15.1.2	Formula

Peak Throughput = DATA_WIDTH (bits) × Frequency (Hz) / 8 bytes/bit

## 15.2	Sustained Throughput

### 15.2.1	Factors Affecting Throughput

Table 5.2: Factors Affecting Throughput

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Arbitration contention | Reduces per-master throughput | Increase slave ports |
| Width conversion | 1-cycle penalty per direction | Match widths where possible |
| Protocol conversion | 2+ cycles for APB | Use AXI4 for high-bandwidth |
| Response routing | Minimal (pipelined) | N/A |

### 15.2.2	Multi-Master Scaling

With fair round-robin arbitration:

Per-Master Throughput = Peak Throughput / Active Masters (to same slave)

### 15.2.3	Example: 4 Masters, 2 Slaves

All 4 masters accessing Slave 0:
  Per-master = Peak / 4

2 masters to Slave 0, 2 masters to Slave 1:
  Per-master = Peak / 2 (full parallelism)

## 15.3	Burst Efficiency

### 15.3.1	Burst vs Single-Beat

Table 5.3: Burst Efficiency Comparison

| Transaction Type | Overhead | Efficiency |
| --- | --- | --- |
| Single beat | 1 cycle address + 1 cycle data | 50% |
| 4-beat burst | 1 cycle address + 4 cycle data | 80% |
| 16-beat burst | 1 cycle address + 16 cycle data | 94% |
| 256-beat burst | 1 cycle address + 256 cycle data | 99.6% |

### 15.3.2	Recommendation

- Use bursts whenever possible

- AXI4 supports up to 256 beats per burst

- APB does not support bursts (split internally)

## 15.4	Width Conversion Impact

### 15.4.1	Upsize (Narrow to Wide)

64-bit to 512-bit (8:1 ratio):
  Input: 8 beats × 64 bits = 512 bits
  Output: 1 beat × 512 bits = 512 bits

Throughput: Preserved (same data, fewer beats)
Latency: +1 cycle (packing delay)

### 15.4.2	Downsize (Wide to Narrow)

512-bit to 64-bit (8:1 ratio):
  Input: 1 beat × 512 bits = 512 bits
  Output: 8 beats × 64 bits = 512 bits

Throughput: Preserved (same data, more beats)
Latency: +7 cycles (sequential output)

## 15.5	Protocol Conversion Impact

### 15.5.1	AXI4 to APB

Table 5.4: AXI4 vs APB Protocol Impact

| Metric | AXI4 Direct | AXI4 to APB |
| --- | --- | --- |
| Min cycles/transfer | 1 | 2 |
| Burst support | Yes | No (split) |
| Pipeline depth | 1+ | 1 |
| Peak throughput | DATA_WIDTH/cycle | DATA_WIDTH/2 cycles |

### 15.5.2	Recommendation

- Keep high-bandwidth traffic on AXI4 paths

- Use APB only for low-bandwidth peripherals

- Group APB slaves to avoid impacting AXI4 traffic

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Latency Analysis

## 16.1	Latency Components

### 16.1.1	Address Path Latency

Table 5.5: Address Path Latency

| Stage | Cycles | Notes |
| --- | --- | --- |
| Master Adapter (skid) | 1 | Pipeline registration |
| Address Decode | 0 | Combinational |
| Arbitration | 0-1 | 0 if no contention, 1 if arbitrate |
| Slave Router | 1 | Pipeline registration |
| Total Address | 2-3 | Best/worst case |

### 16.1.2	Data Path Latency

Table 5.6: Data Path Latency

| Stage | Cycles | Notes |
| --- | --- | --- |
| W follows AW | 0 | Same grant |
| Width conversion | 0-1 | 0 if match, 1 if convert |
| Protocol conversion | 0-2+ | 0 for AXI4, 2+ for APB |
| Total Data | 0-3+ | Best/worst case |

### 16.1.3	Response Path Latency

Table 5.7: Response Path Latency

| Stage | Cycles | Notes |
| --- | --- | --- |
| Slave response | Variable | Slave-dependent |
| ID extraction | 0 | Combinational |
| Response routing | 1 | Pipeline registration |
| Master delivery | 0 | Direct connection |
| Total Response | 1 + slave | Plus slave latency |

## 16.2	End-to-End Latency

### 16.2.1	Write Transaction (Best Case)

Cycle 0: AW arrives at Bridge
Cycle 1: AW exits to Slave (skid buffer)
Cycle 2: Slave receives AW
Cycle 1: W arrives (same cycle as AW skid)
Cycle 2: W exits to Slave
Cycle 3: Slave receives W, generates B
Cycle 4: B routed back to Master

Total: 4 cycles (minimum)

### 16.2.2	Read Transaction (Best Case)

Cycle 0: AR arrives at Bridge
Cycle 1: AR exits to Slave (skid buffer)
Cycle 2: Slave receives AR, generates R
Cycle 3: R routed back to Master

Total: 3 cycles (minimum)

### 16.2.3	Contention Latency

When multiple masters contend for same slave:

Arbitration adds 1 cycle per contending master

Example: 4 masters requesting same slave
  Worst case: Wait for 3 other masters = 3 extra cycles
  Average case: Wait for 1.5 masters = 1-2 extra cycles

## 16.3	Width Conversion Latency

### 16.3.1	Upsize Latency

Table 5.8: Upsize Latency by Ratio

| Ratio | Extra Cycles | Notes |
| --- | --- | --- |
| 1:1 | 0 | No conversion |
| 2:1 | 0 | Pack on fly |
| 4:1 | 0 | Pack on fly |
| 8:1 | 0-1 | May need accumulation |

### 16.3.2	Downsize Latency

Table 5.9: Downsize Latency by Ratio

| Ratio | Extra Cycles | Notes |
| --- | --- | --- |
| 1:1 | 0 | No conversion |
| 1:2 | +1 | 2 output beats |
| 1:4 | +3 | 4 output beats |
| 1:8 | +7 | 8 output beats |

## 16.4	Protocol Conversion Latency

### 16.4.1	AXI4 to APB

Table 5.10: AXI4 to APB Conversion Latency

| Phase | Cycles |
| --- | --- |
| AXI4 AW decode | 1 |
| APB setup | 1 |
| APB access | 1+ (PREADY) |
| B generation | 1 |
| Total | 4+ cycles |

### 16.4.2	APB Wait States

APB slaves may insert wait states:

PREADY deasserted: +1 cycle per wait
Typical UART/GPIO: 0-2 wait states
Slow peripherals: May have many wait states

## 16.5	Latency Optimization

### 16.5.1	Design Recommendations

- Match data widths - Avoid conversion latency

- Use AXI4 for fast paths - Avoid APB latency

- Minimize contention - Spread traffic across slaves

- Use bursts - Amortize address latency

- Pipeline responses - Don’t block on slow slaves

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Resource Estimates

## 17.1	FPGA Resource Summary

### 17.1.1	Baseline Configuration (2x2, 64-bit)

Table 5.11: Baseline Resource Requirements

| Resource | Count | Notes |
| --- | --- | --- |
| LUTs | ~2,000-3,000 | Logic and routing |
| Registers | ~1,500-2,000 | Pipeline stages |
| Block RAM | 0-1 | Optional ID CAM |
| DSP | 0 | No arithmetic |

### 17.1.2	Scaling Factors

Table 5.12: Resource Scaling Factors

| Component | Scaling |
| --- | --- |
| Crossbar core | O(M x N) |
| Master adapters | O(M) |
| Slave routers | O(N) |
| ID tracking | O(M x Outstanding) |
| Width converters | O(per-path ratio) |

## 17.2	Component Breakdown

### 17.2.1	Per-Master Resources

Table 5.13: Per-Master Resource Breakdown

| Component | LUTs | Registers |
| --- | --- | --- |
| Skid buffer | ~50-100 | ~DATA_WIDTH |
| ID extension | ~20 | ~ID_WIDTH |
| Channel mux | ~100 | ~50 |
| Per Master Total | ~200-300 | ~DATA_WIDTH + 100 |

### 17.2.2	Per-Slave Resources

Table 5.14: Per-Slave Resource Breakdown

| Component | LUTs | Registers |
| --- | --- | --- |
| Address decode | ~50 | 0 |
| Arbiter | ~100-200 | ~50 |
| Response mux | ~100 | ~50 |
| Per Slave Total | ~250-350 | ~100 |

### 17.2.3	Crossbar Core

Table 5.15: Crossbar Core Resources

| Component | LUTs | Registers |
| --- | --- | --- |
| AW mux (N-way) | ~100 x N | ~50 x N |
| W mux (N-way) | ~100 x N | ~50 x N |
| B demux (M-way) | ~50 x M | ~50 x M |
| AR mux (N-way) | ~100 x N | ~50 x N |
| R demux (M-way) | ~50 x M | ~50 x M |

## 17.3	Width Converter Resources

Table 5.16: Width Converter Resources

| Ratio | LUTs | Registers | Notes |
| --- | --- | --- | --- |
| 1:2 upsize | ~200 | ~DATA_IN | Pack logic |
| 1:4 upsize | ~300 | ~DATA_IN | Pack logic |
| 1:8 upsize | ~400 | ~DATA_IN | Pack logic |
| 2:1 downsize | ~200 | ~DATA_OUT | Split logic |
| 4:1 downsize | ~300 | ~DATA_OUT | Split logic |
| 8:1 downsize | ~400 | ~DATA_OUT | Split logic |

## 17.4	Protocol Converter Resources

### 17.4.1	AXI4 to APB

Table 5.17: AXI4 to APB Converter Resources

| Component | LUTs | Registers |
| --- | --- | --- |
| FSM | ~100 | ~20 |
| Burst counter | ~50 | ~8 |
| Data buffer | ~100 | ~DATA_WIDTH |
| Response logic | ~50 | ~10 |
| Total | ~300 | ~DATA_WIDTH + 50 |

## 17.5	Example Configurations

Table 5.18: Complete Bridge Resource Estimates

| Config | Masters | Slaves | Data | LUTs | Registers |
| --- | --- | --- | --- | --- | --- |
| 2x2 Basic | 2 | 2 | 64 | ~2,500 | ~1,500 |
| 4x4 Standard | 4 | 4 | 64 | ~5,000 | ~3,000 |
| 4x4 Wide | 4 | 4 | 256 | ~8,000 | ~5,000 |
| 8x8 Large | 8 | 8 | 128 | ~12,000 | ~8,000 |
| RAPIDS (4x3) | 4 | 3 | 512 | ~10,000 | ~6,000 |

### 17.5.1	Notes

- Estimates include all converters and adapters

- Actual results vary by synthesis tool and FPGA family

- Block RAM usage depends on ID table implementation

- DSP usage is zero (no multiplication/division)

## 17.6	Optimization Strategies

### 17.6.1	Resource Reduction

- Use channel-specific masters - 40-60% port reduction

- Match data widths - Avoid converter logic

- Use APB sparingly - Reduces AXI4 overhead

- Limit outstanding transactions - Smaller ID tables

### 17.6.2	Performance/Resource Trade-off

Table 5.19: Performance/Resource Trade-offs

| Feature | Resource Impact | Performance Impact |
| --- | --- | --- |
| Deeper skid buffers | +registers | +timing margin |
| Larger ID tables | +BRAM | +OOO depth |
| Pipeline stages | +registers | +frequency |
| Wider data paths | +routing | +throughput |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	System Requirements

## 18.1	Hardware Requirements

### 18.1.1	Clock Infrastructure

Table 6.1: Clock Requirements

| Requirement | Specification |
| --- | --- |
| Clock source | Single clock domain (aclk) |
| Clock quality | Low jitter, stable frequency |
| Clock distribution | Must reach all Bridge ports |

### 18.1.2	Reset Infrastructure

Table 6.2: Reset Requirements

| Requirement | Specification |
| --- | --- |
| Reset type | Asynchronous, active-low (aresetn) |
| Reset synchronization | External synchronizer recommended |
| Reset distribution | Must reach all Bridge ports |

### 18.1.3	Power

Table 6.3: Power Requirements

| Requirement | Specification |
| --- | --- |
| Power domains | Single domain (no power gating) |
| Voltage | FPGA/ASIC process-dependent |

## 18.2	Interface Requirements

### 18.2.1	Master Requirements

Masters connecting to Bridge must:

- Comply with AXI4 protocol - Valid/ready handshake

- Use correct ID width - Match configured ID_WIDTH

- Use correct data width - Match configured DATA_WIDTH

- Use correct address width - Match configured ADDR_WIDTH

- Stay within address range - Target valid slave addresses

### 18.2.2	Slave Requirements

Slaves connecting to Bridge must:

- Comply with AXI4/APB protocol - Based on configuration

- Accept extended IDs - ID_WIDTH + BID_WIDTH

- Match configured data width - Or use width conversion

- Respond to all transactions - No hanging

## 18.3	Configuration Requirements

### 18.3.1	Address Map

Table 6.4: Address Map Requirements

| Requirement | Specification |
| --- | --- |
| Slave ranges | Must not overlap |
| Range alignment | Power-of-2 recommended |
| Coverage | All master addresses must map to slaves or OOR |

### 18.3.2	Connectivity

Table 6.5: Connectivity Requirements

| Requirement | Specification |
| --- | --- |
| Matrix completeness | All masters must access at least one slave |
| Reachability | Consider traffic patterns for performance |

## 18.4	Timing Requirements

### 18.4.1	Setup/Hold

Table 6.6: Timing Requirements

| Constraint | Requirement |
| --- | --- |
| Input setup | Meet FPGA/ASIC requirements |
| Input hold | Meet FPGA/ASIC requirements |
| Clock-to-output | Per process characterization |

### 18.4.2	Recommended Margins

Table 6.7: Recommended Timing Margins

| Margin | Value | Purpose |
| --- | --- | --- |
| Setup margin | 10-20% | Variation tolerance |
| Hold margin | Positive | No hold violations |
| Clock uncertainty | Include in constraints | PLL/MMCM jitter |

## 18.5	Verification Requirements

### 18.5.1	Pre-Integration Checks

- RTL lint clean - No Verilator warnings

- Parameter validation - All parameters in valid range

- Address map review - No overlaps, complete coverage

- ID width calculation - Sufficient for NUM_MASTERS

### 18.5.2	Integration Checks

- Signal connectivity - All ports connected

- Width matching - DATA_WIDTH, ADDR_WIDTH, ID_WIDTH

- Protocol matching - AXI4 vs APB correctly configured

- Clock/reset connectivity - All ports share same domain

### 18.5.3	Post-Integration Checks

- Basic transaction test - Read/write to each slave

- Arbitration test - Multi-master contention

- Error handling test - OOR address response

- Performance validation - Meet throughput targets

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Parameter Configuration

## 19.1	Core Parameters

Table 6.8: Bridge Core Parameters

| Parameter | Type | Range | Default | Description |
| --- | --- | --- | --- | --- |
| NUM_MASTERS | int | 1-32 | 2 | Number of master ports |
| NUM_SLAVES | int | 1-256 | 2 | Number of slave ports |
| ADDR_WIDTH | int | 12-64 | 32 | Address bus width |
| DATA_WIDTH | int | 32-512 | 64 | Data bus width |
| ID_WIDTH | int | 1-16 | 4 | Master ID width |
| USER_WIDTH | int | 1-16 | 1 | User signal width |

## 19.2	Derived Parameters

### 19.2.1	Calculated by Generator

Table 6.9: Derived Parameters

| Parameter | Formula | Example |
| --- | --- | --- |
| BID_WIDTH | clog2(NUM_MASTERS) | 4 masters = 2 bits |
| TOTAL_ID_WIDTH | ID_WIDTH + BID_WIDTH | 4 + 2 = 6 bits |
| STRB_WIDTH | DATA_WIDTH / 8 | 64b = 8 strobes |

## 19.3	Per-Port Configuration

### 19.3.1	Master Port Configuration

Table 6.10: Master Port Configuration

| Field | Type | Description |
| --- | --- | --- |
| name | string | Unique identifier |
| prefix | string | Signal prefix |
| channels | enum | “rw”, “wr”, or “rd” |
| data_width | int | Port data width |
| id_width | int | Port ID width |

### 19.3.2	Slave Port Configuration

Table 6.11: Slave Port Configuration

| Field | Type | Description |
| --- | --- | --- |
| name | string | Unique identifier |
| prefix | string | Signal prefix |
| protocol | enum | “axi4”, “axi4lite”, “apb” |
| data_width | int | Port data width |
| base_addr | hex | Base address (must be 4K-aligned) |
| addr_range | hex | Address range size (must be multiple of 4K) |

#### 19.3.2.1	Slave Address Window Rules (MANDATORY)

Every slave must occupy a 4K-aligned address window that is a multiple of 4 KB:

Rule 1: Base Address Alignment
  base_addr & 0xFFF == 0x000
  
  Valid:   0x00000000, 0x00001000, 0x80000000, 0xFFFF0000
  Invalid: 0x00000001, 0x80000800, 0xC0000100

Rule 2: Range Alignment
  addr_range % 0x1000 == 0
  
  Valid:   0x1000 (4 KB), 0x2000 (8 KB), 0x10000 (64 KB)
  Invalid: 0x800 (2 KB), 0x1001, 0x7FFF

Rule 3: Non-Overlapping Windows
  All slaves must have non-overlapping address ranges
  
  Valid:   Slave0: 0x00000000-0x0FFFFFFF, Slave1: 0x10000000-0x1FFFFFFF
  Invalid: Slave0: 0x00000000-0x10000000, Slave1: 0x0FFFFFFF-0x1FFFFFFF

Why This Rule Exists: - Real memory systems use 4K page boundaries (virtual memory, MMU) - Linker scripts and memory maps assume 4K granularity - Address decoders simplify to bit masks with 4K alignment - Prevents ambiguity in address decode logic

## 19.4	Configuration File Format

### 19.4.1	TOML Configuration

[bridge]
  name = "my_bridge"
  description = "Example bridge configuration"

  defaults.skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

  masters = [
    {name = "cpu", prefix = "cpu_m_axi", channels = "rw",
     id_width = 4, addr_width = 32, data_width = 64, user_width = 1},
    {name = "dma", prefix = "dma_m_axi", channels = "rw",
     id_width = 4, addr_width = 32, data_width = 256, user_width = 1}
  ]

  slaves = [
    {name = "ddr", prefix = "ddr_s_axi", protocol = "axi4",
     id_width = 6, addr_width = 32, data_width = 512, user_width = 1,
     base_addr = 0x80000000, addr_range = 0x40000000},
    {name = "uart", prefix = "uart_apb", protocol = "apb",
     addr_width = 12, data_width = 32,
     base_addr = 0x00000000, addr_range = 0x00001000}
  ]

### 19.4.2	CSV Connectivity

master\slave,ddr,uart
cpu,1,1
dma,1,0

## 19.5	Parameter Validation

### 19.5.1	Generator Checks

Table 6.12: Generator Validation Checks

| Check | Error Condition |
| --- | --- |
| NUM_MASTERS | < 1 or > 32 |
| NUM_SLAVES | < 1 or > 256 |
| DATA_WIDTH | Not power of 2 |
| Address overlap | Slave ranges intersect |
| Connectivity | Master with no slaves |
| ID width | Insufficient for masters |

### 19.5.2	Runtime Validation

Bridge includes optional assertions for:

- Address alignment

- Burst boundary crossing

- Protocol violations

- ID mismatch

## 19.6	Example Configurations

### 19.6.1	Simple 2x2

[bridge]
  name = "bridge_simple"
  masters = [
    {name = "m0", prefix = "m0_axi", data_width = 64},
    {name = "m1", prefix = "m1_axi", data_width = 64}
  ]
  slaves = [
    {name = "s0", prefix = "s0_axi", base_addr = 0x0, addr_range = 0x10000000},
    {name = "s1", prefix = "s1_axi", base_addr = 0x10000000, addr_range = 0x10000000}
  ]

### 19.6.2	Mixed Protocol

[bridge]
  name = "bridge_mixed"
  masters = [
    {name = "cpu", prefix = "cpu_axi", data_width = 64, channels = "rw"}
  ]
  slaves = [
    {name = "mem", prefix = "mem_axi", protocol = "axi4", data_width = 512},
    {name = "gpio", prefix = "gpio_apb", protocol = "apb", data_width = 32},
    {name = "uart", prefix = "uart_apb", protocol = "apb", data_width = 32}
  ]

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Verification Strategy

## 20.1	Verification Levels

### 20.1.1	Unit Level

Test individual Bridge components:

Table 6.13: Unit Level Test Focus

| Component | Test Focus |
| --- | --- |
| Master Adapter | Skid buffer, ID extension |
| Slave Router | Address decode, arbitration |
| Width Converter | Upsize/downsize correctness |
| Protocol Converter | AXI4 to APB timing |
| Response Router | ID extraction, routing |

### 20.1.2	Integration Level

Test complete Bridge functionality:

Table 6.14: Integration Test Coverage

| Test Category | Coverage |
| --- | --- |
| Address routing | All master-slave paths |
| Arbitration | Contention, fairness |
| ID tracking | Response routing |
| Error handling | OOR, timeouts |
| Performance | Throughput, latency |

### 20.1.3	System Level

Test Bridge in target system:

Table 6.15: System Level Tests

| Test Category | Focus |
| --- | --- |
| End-to-end | CPU to memory |
| Multi-master | DMA + CPU |
| Mixed protocol | AXI4 + APB |
| Stress | Maximum traffic |

## 20.2	Test Infrastructure

### 20.2.1	CocoTB Framework

Bridge uses CocoTB for verification:

from CocoTBFramework.components.axi4 import AXI4Master, AXI4Slave

@cocotb.test()
async def test_basic_write(dut):
    tb = BridgeTB(dut)
    await tb.setup_clocks_and_reset()

    # Write transaction
    await tb.master[0].write(addr=0x1000, data=0xDEADBEEF)

    # Verify
    assert tb.slave[0].mem[0x1000] == 0xDEADBEEF

### 20.2.2	Test Categories

Table 6.16: Test Categories

| Category | Description | Example |
| --- | --- | --- |
| Basic | Single transactions | Read/write |
| Burst | Multi-beat transactions | 16-beat burst |
| Concurrent | Multiple masters active | M0 + M1 |
| Stress | Maximum utilization | Back-to-back |
| Error | Error conditions | OOR address |

## 20.3	Coverage Goals

### 20.3.1	Functional Coverage

Table 6.17: Functional Coverage Goals

| Category | Target |
| --- | --- |
| Address paths | 100% master-slave combinations |
| Transaction types | Read, write, burst |
| ID values | Full ID range |
| Data patterns | Walking 1s/0s, random |

### 20.3.2	Code Coverage

Table 6.18: Code Coverage Goals

| Metric | Target |
| --- | --- |
| Line coverage | >95% |
| Branch coverage | >90% |
| FSM coverage | 100% state transitions |
| Toggle coverage | >80% signals |

## 20.4	Debug Features

### 20.4.1	Waveform Capture

Bridge tests support VCD/FST waveform dumps:

pytest test_bridge.py --vcd=waves.vcd
gtkwave waves.vcd

### 20.4.2	Debug Signals

Bridge includes optional debug signals:

Table 6.19: Debug Signals

| Signal | Description |
| --- | --- |
| dbg_aw_grant | Current AW grant |
| dbg_ar_grant | Current AR grant |
| dbg_outstanding | Outstanding transaction count |
| dbg_state | FSM states |

## 20.5	Regression Testing

### 20.5.1	Continuous Integration

Table 6.20: CI Test Schedule

| Stage | Tests | Frequency |
| --- | --- | --- |
| Pre-commit | Basic sanity | Each commit |
| Nightly | Full regression | Daily |
| Weekly | Performance | Weekly |

### 20.5.2	Test Matrix

Table 6.21: Configuration Test Matrix

| Config | Data Width | Protocol | Tested |
| --- | --- | --- | --- |
| 2x2 | 64 | AXI4 | Yes |
| 4x4 | 64 | AXI4 | Yes |
| 4x4 | 256 | AXI4 | Yes |
| 2x2 | 64 | AXI4+APB | Yes |
| RAPIDS | 512 | AXI4+APB | Yes |

## 20.6	Known Limitations

### 20.6.1	Test Gaps

Table 6.22: Known Test Gaps

| Gap | Status | Plan |
| --- | --- | --- |
| APB burst stress | Partial | Phase 3 |
| Width downsize stress | Partial | Planned |
| 32-master config | Not tested | Low priority |

### 20.6.2	Simulator Support

Table 6.23: Simulator Support Status

| Simulator | Status |
| --- | --- |
| Verilator | Fully supported |
| Icarus Verilog | Supported |
| Commercial (VCS, Questa) | Not tested |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Bridge Micro-Architecture Specification Index

Component: Bridge (Multi-Protocol Crossbar Generator) Version: 1.0 Date: 2026-01-03 Purpose: Detailed micro-architecture specification for Bridge component

## 21.1	Document Organization

This specification covers the Bridge component - a CSV-driven generator that creates AXI4 crossbars with automatic width conversion, protocol conversion (AXI4, AXI4-Lite, APB), and channel-specific master support (read-only, write-only, or full).

### 21.1.1	Front Matter

- Document Information

### 21.1.2	Chapter 1: Introduction

- Overview

### 21.1.3	Chapter 2: Block Descriptions

- Master Adapter

- Slave Router

- Crossbar Core

- Arbitration

- ID Management

- Width Conversion

- Protocol Conversion

- Response Routing

- Error Handling

### 21.1.4	Chapter 3: FSM Design

- Arbiter FSMs

- Transaction Tracking

### 21.1.5	Chapter 4: ID Management

- CAM Architecture

- ID Tracking Tables

### 21.1.6	Chapter 5: Converters

- Width Converters

- APB Converters

### 21.1.7	Chapter 6: Generated RTL

- Module Structure

- Signal Naming

### 21.1.8	Chapter 7: Verification

- Test Strategy

- Debug Guide

## 21.2	Quick Navigation

### 21.2.1	For New Users

- Start with Overview for understanding Bridge capabilities

- Read Block Diagram to understand the architecture

- Study Arbitration for operational details

- Reference Module Structure for generated RTL

### 21.2.2	For Integration

- Protocol support: See Protocol Conversion for AXI4/APB/AXI-Lite

- Width handling: See Width Conversion for data width mismatches

- ID management: See ID Management for transaction tracking

- Error handling: See Error Handling for OOR and timeout

## 21.3	Visual Assets

All diagrams referenced in the documentation are available in:

- Source Files:

  - assets/graphviz/*.gv - Graphviz source diagrams

  - assets/puml/*.puml - PlantUML FSM diagrams

- Rendered Files:

  - assets/graphviz/*.png - Rendered block diagrams

  - assets/puml/*.png - Rendered FSM diagrams

### 21.3.1	Architecture Diagrams

- Master Adapter - assets/graphviz/master_adapter.png

- Slave Router - assets/graphviz/slave_router.png

- Crossbar Core - assets/graphviz/crossbar_core.png

- ID Management - assets/graphviz/id_management.png

- Width Conversion - assets/graphviz/width_conversion.png

- Protocol Conversion - assets/graphviz/protocol_conversion_apb.png

- Response Routing - assets/graphviz/response_routing.png

- Error Handling - assets/graphviz/error_handling.png

### 21.3.2	FSM Diagrams

- AW Arbiter FSM - assets/puml/aw_arbiter_fsm.png

- AR Arbiter FSM - assets/puml/ar_arbiter_fsm.png

## 21.4	Component Overview

### 21.4.1	Key Features

- Multi-Protocol Support: AXI4, AXI4-Lite, and APB slave conversion

- CSV-Driven Configuration: Human-readable port and connectivity definitions

- Channel-Specific Masters: Write-only (wr), read-only (rd), or full (rw) support

- Automatic Width Conversion: Upsize/downsize for data width mismatches

- Out-of-Order Support: ID-based response routing with CAM tracking

- Custom Signal Prefixes: Unique prefixes per port for clean integration

### 21.4.2	Protocol Conversion Matrix

| Master Protocol | Slave Protocol | Conversion |
| --- | --- | --- |
| AXI4 | AXI4 | Direct or width convert |
| AXI4 | AXI4-Lite | Protocol downgrade |
| AXI4 | APB | Full protocol conversion |

### 21.4.3	Design Philosophy

Efficient Multi-Width Routing: - Direct connections where widths match (zero conversion overhead) - Width converters only where needed - No fixed internal crossbar width

Resource Optimization: - Channel-specific masters reduce port count by 40-60% - Per-path width converters instead of global conversion - Minimal logic for matching-width connections

## 21.5	Related Documentation

### 21.5.1	Companion Specifications

- Bridge HAS - Hardware Architecture Specification (high-level)

### 21.5.2	Project-Level

- PRD.md: ../../PRD.md - Complete product requirements document

- CLAUDE.md: ../../CLAUDE.md - AI assistant integration guide

### 21.5.3	Generator

- Generator: ../../bin/bridge_csv_generator.py - CSV-based generator script

- Test Configs: ../../bin/test_configs/ - Example TOML/CSV configurations

## 21.6	Version History

Version 1.0 (2026-01-03): - Initial specification release - Restructured from single spec to HAS/MAS format - Complete block-level documentation - FSM and ID management details

Last Updated: 2026-01-03 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Bridge: AXI4 Full Crossbar Generator - Product Requirements Document

Project: Bridge Version: 2.1 Status: 🟢 Phase 2 Complete - Simplified Architecture with Hard Limits Created: 2025-10-18 Last Updated: 2025-11-02

## 22.1	Executive Summary

Bridge is a Python-based AXI4 crossbar generator that produces simple, performant SystemVerilog RTL for connecting multiple AXI4 masters to multiple AXI4 slaves. The name follows the infrastructure theme - bridges connect different regions, enabling communication across divides, just like crossbars connect masters and slaves.

Design Philosophy: A simple AMBA fabric that is performant, but makes no attempt to support all features. We enforce hard limits (8-bit ID width, 64-bit address width) to eliminate unnecessary complexity, focusing only on what real hardware needs.

Key Differentiator from Delta: - Delta: AXI-Stream crossbar (streaming data, single channel, simple routing) - Bridge: AXI4 full crossbar (memory-mapped, 5 channels, burst support, ID-based routing)

Key Differentiator from Commercial IP: - Commercial AXI4 Crossbars: Feature-complete, support every AXI4 edge case, complex configurability - Bridge: Simple and performant, supports common use cases, hard limits for simplicity

Target Use Case: High-performance memory-mapped interconnects for multi-core processors, accelerators, and memory controllers where simplicity and performance matter more than spec compliance.

## 22.2	Design Philosophy

Vision: A simple AMBA fabric that is performant, but makes no attempt to support all features.

### 22.2.1	Core Principles

1. Simplicity Over Completeness - Focus on common use cases, not edge cases - Implement what’s needed for real hardware, not AXI4 spec compliance theater - Prefer straightforward implementations over complex feature sets

2. Performance First - Direct connections where possible (matching widths = zero overhead) - Minimal conversion logic in critical paths - Optimize for throughput and latency, not feature count

3. Hard Limits for Simplicity

We enforce uniform widths on certain parameters to eliminate complexity:

| Parameter | Hard Limit | Rationale |
| --- | --- | --- |
| ID Width | 8 bits (fixed) | Eliminates ID width conversion logic. 256 unique IDs is sufficient for all realistic use cases. |
| Address Width | 64 bits (fixed) | Eliminates address width conversion. 64-bit addressing covers all memory-mapped systems. |
| Data Width | Variable (32b-512b) | Width converters ONLY for data. Commonly needed for bandwidth optimization. |

Why This Works: - ID width conversion adds complexity with minimal benefit (nobody needs >256 outstanding transactions per master) - Address width conversion is rarely needed (peripheral addresses fit in 32-bit, but using 64-bit everywhere is simpler) - Data width conversion is the ONLY width conversion that provides real value (bandwidth matching)

4. What We DON’T Support (By Design)

Features intentionally excluded for simplicity: - ✗ Variable ID width per port - ✗ Variable address width per port - ✗ AXI4-Lite protocol variant (use standard AXI4 with len=0) - ✗ ACE protocol extensions (cache coherency) - ✗ AXI5 features - ✗ Complete AXI4 sideband signal support (QoS, Region, User signals declared but not routed)

5. What We DO Support

Core AXI4 features that matter: - ✓ Full 5-channel AXI4 protocol (AW, W, B, AR, R) - ✓ Burst transactions (INCR, WRAP, FIXED) - ✓ Out-of-order completion via transaction IDs - ✓ Multiple outstanding transactions per master - ✓ Channel-specific masters (write-only, read-only, read-write) - ✓ Data width conversion (32b ↔ 64b ↔ 128b ↔ 256b ↔ 512b) - ✓ Configurable M×N topology (1-32 masters, 1-256 slaves) - ✓ Fair round-robin arbitration per slave

### 22.2.2	Architecture Philosophy

Target: Intelligent width-aware routing, not fixed-width crossbar

OLD Approach (What We’re Moving Away From):

Master (64b) → Upsize(64→256) → Fixed 256b Crossbar → Downsize(256→64) → Slave (64b)

- Two conversions for same-width connections (wasteful!)

- Artificial bandwidth bottleneck

- Unnecessary logic and latency

NEW Approach (Target Architecture):

Master (64b) → Direct Connection → Slave (64b)    [0 conversions!]
Master (512b) → Conv(512→64) → Slave (64b)         [1 conversion]

- Per-master paths to each unique slave width it connects to

- Direct paths where widths match (zero overhead)

- Converters only where actually needed

- Router selects appropriate path based on address decode

Result: Minimal conversion logic, maximum performance, pragmatic simplicity.

## 22.3	⚠ CRITICAL: RTL Regeneration Requirements

ALL generated RTL files MUST be deleted and regenerated together whenever ANY generator code changes.

Why This Matters: - Generated RTL files may have interdependencies (bridges, wrappers, integrators) - Generator code changes can create version mismatches between files - Partial regeneration creates subtle incompatibilities that cause test failures - Even “small innocuous” generator changes can have cascading effects

The Rule:

# ✗ WRONG - Partial regeneration
./bridge_generator.py --masters 5 --slaves 3 --output ../rtl/
# Only regenerates bridge_axi4_flat_5x3.sv
# Other files (wrappers, integrators) now mismatched!

# ✓ CORRECT - Full regeneration
rm ../rtl/bridge_*.sv                    # Delete ALL generated bridges
rm ../rtl/bridge_wrapper_*.sv            # Delete ALL generated wrappers
./regenerate_all_bridges.sh              # Regenerate everything together

Generator Files That Trigger Full Regeneration: - bridge_generator.py - Main bridge generator - bridge_csv_generator.py - CSV-based generator - bridge_address_arbiter.py - Address decode logic - bridge_channel_router.py - Channel routing logic - bridge_response_router.py - Response routing logic - bridge_amba_integrator.py - AMBA component integration - bridge_wrapper_generator.py - Wrapper generation - Any Python file in projects/components/bridge/bin/

Symptoms of Version Mismatch: - Tests that previously passed now fail - Simulation errors about missing signals - Mismatched port widths or counts - Address decode routing to wrong slaves

Think of Generated RTL Like Compiled Code: When you update a compiler, you don’t selectively recompile - you rebuild everything. When you update a generator, you don’t selectively regenerate - you regenerate everything.

## 22.4	1. Product Overview

### 22.4.1	1.1 Purpose

Bridge provides automated generation of AXI4 crossbar infrastructure with: - Python code generation - Parameterized RTL generation (similar to APB/Delta) - Performance modeling - Analytical + simulation validation - Flat topology - Full M×N interconnect matrix - ID-based routing - Out-of-order transaction support - Burst optimization - Pipelined burst transfers

### 22.4.2	1.2 Target Audience

Primary Users: - RTL designers building SoC interconnect - System architects evaluating interconnect topologies - Verification engineers needing crossbar testbenches - Students learning AXI4 protocol and interconnects

Educational Focus: - Demonstrates AXI4 protocol complexity - Shows arbitration strategies for memory-mapped busses - Teaches ID-based transaction tracking - Illustrates burst optimization techniques

### 22.4.3	1.3 Success Criteria

Functional: - [x] Generates working AXI4 crossbar RTL (bridge_generator.py) - [x] Generates CSV-configured bridges (bridge_csv_generator.py) - [x] Passes Verilator lint - [x] Supports 1-32 masters, 1-256 slaves - [x] Handles out-of-order completion via IDs (bridge_cam.sv) - [x] Supports burst lengths 1-256 beats - [x] Channel-specific masters (wr/rd/rw) for resource optimization (Phase 2) - [ ] APB converter integration (Phase 3 pending)

Performance: - [x] Latency ≤ 3 cycles for single-beat transactions - [x] Throughput: All M×N paths can transfer concurrently - [x] Performance models implemented (bridge_model.py - V1 Flat) - [ ] Fmax ≥ 300 MHz on UltraScale+ FPGAs (pending synthesis validation)

Quality: - [x] Complete specifications (PRD) before code - [x] Performance models validate requirements (bridge_model.py) - [x] All generated RTL Verilator verified - [x] Integration examples provided (CSV examples) - [x] Comprehensive documentation (BRIDGE_CURRENT_STATE.md, BRIDGE_ARCHITECTURE_DIAGRAMS.md)

### 22.4.4	1.4 Implementation Status and Phases

Unified Generator (bridge_generator.py):

The bridge generator now supports both TOML/CSV configuration and legacy array-indexed modes:

Configuration Modes: 1. TOML/CSV Mode (Preferred) - TOML port configuration (bridge_name.toml) - CSV connectivity matrix (bridge_name_connectivity.csv) - Custom port prefixes (rapids_m_axi_, cpu_m_axi_, etc.) - Interface wrapper integration (timing isolation) - Mixed protocols (AXI4 + APB + AXI4-Lite slaves) - Channel-specific masters (wr/rd/rw) - Status: ✓ Phase 2 complete, Phase 3 pending

- Legacy CSV Mode (Backwards Compatible)

  - Separate ports.csv and connectivity.csv files

  - Migration path to TOML format

  - See test_configs/README.md for conversion guide

Phase Status:

Phase 1: CSV Configuration ✓ COMPLETE - CSV parser (ports.csv, connectivity.csv) - Port generation with custom prefixes - Converter identification logic - Basic crossbar instantiation

Phase 2: Channel-Specific Masters ✓ COMPLETE (2025-10-26) - Added channels field to PortSpec (rw/wr/rd) - Conditional port generation based on channels - Resource Optimization: - wr (write-only): AW, W, B channels only → 39% port reduction - rd (read-only): AR, R channels only → 61% port reduction - rw (full): All 5 channels - Width converter awareness (only generate needed converters) - Example: 4-master bridge saves 35% signals with optimized channels

Phase 3: APB Converter Integration ⏳ PENDING - AXI2APB converter module (create or integrate existing) - APB signal packing/unpacking - APB converter instantiation in generated bridges - Width converter + APB converter chaining - End-to-end testing with APB slaves - Status: Placeholders in generated code with detailed TODO comments

Additional Resources: - bridge_model.py - Performance modeling (V1 Flat implemented) - bridge_cam.sv - Transaction ID tracking for OOO support - See BRIDGE_CURRENT_STATE.md for detailed review - See docs/BRIDGE_ARCHITECTURE_DIAGRAMS.md for visual architecture

## 22.5	2. Architecture Overview

### 22.5.1	2.1 AXI4 vs AXIS vs APB

| Feature | APB | AXI-Stream (Delta) | AXI4 (Bridge) |
| --- | --- | --- | --- |
| Protocol Type | Simple register bus | Streaming data | Memory-mapped burst |
| Channels | 1 (address + data) | 1 (data stream) | 5 (AW, W, B, AR, R) |
| Request Generation | Address range decode | TDEST decode | Address range decode |
| Arbitration | Per-slave, per-cycle | Per-slave, packet-locked | Per-slave, per-address-phase |
| Out-of-Order | No (sequential) | No (streaming order) | Yes (ID-based) |
| Burst Support | No | Packet (via TLAST) | Yes (AWLEN/ARLEN) |
| Complexity | Low | Medium | High |
| Latency | 1-2 cycles | 2 cycles | 2-3 cycles |
| Use Case | Control registers | Data streaming | Memory-mapped I/O |

Bridge Complexity Sources: 1. 5 independent channels requiring separate arbitration 2. ID-based routing for out-of-order completion 3. Burst handling with interleaving constraints 4. Write response tracking (match AW with B channel) 5. Address decode + ID muxing for response routing

### 22.5.2	2.2 Block Diagram

┌──────────────────────────────────────────────────────────────────────┐
│                          BRIDGE AXI4 CROSSBAR                        │
└──────────────────────────────────────────────────────────────────────┘

Masters (M)                                                    Slaves (S)
┌─────────┐                                                   ┌─────────┐
│ Master 0│ ─AW──┐                                      ┌─AW──│ Slave 0 │
│  (CPU)  │ ─W───┤                                      ├─W───│ (DDR)   │
│         │ ─B───┤                                      ├─B───│         │
│         │ ─AR──┤                                      ├─AR──│         │
│         │ ─R───┤                                      ├─R───│         │
└─────────┘      │                                      │     └─────────┘
                 │                                      │
┌─────────┐      │   ┌──────────────────────────┐      │     ┌─────────┐
│ Master 1│ ─────┼───│  Request Generation      │──────┼─────│ Slave 1 │
│  (GPU)  │      │   │  (Address Range Decode)  │      │     │ (SRAM)  │
└─────────┘      │   └──────────────────────────┘      │     └─────────┘
                 │             │                        │
┌─────────┐      │   ┌──────────────────────────┐      │     ┌─────────┐
│ Master 2│ ─────┼───│  Arbitration Matrix      │──────┼─────│ Slave 2 │
│  (DMA)  │      │   │  (5 separate arbiters    │      │     │ (PCIE)  │
└─────────┘      │   │   per slave: AW,W,B,AR,R)│      │     └─────────┘
                 │   └──────────────────────────┘      │
┌─────────┐      │             │                        │     ┌─────────┐
│ Master 3│ ─────┴───│  Data/Addr Multiplexing  │──────┴─────│ Slave 3 │
│ (Accel) │          │  (ID-based routing)      │            │ (Periph)│
└─────────┘          └──────────────────────────┘            └─────────┘
                               │
                     ┌──────────────────────────┐
                     │  Transaction Tracking    │
                     │  (ID tables for OoO)     │
                     └──────────────────────────┘

### 22.5.3	2.3 Key Components

1. Request Generation - Address range decode for each slave - Generates M×S request matrix per channel (AW, AR) - Similar to APB but more complex (2 address channels)

2. Per-Slave Arbitration - 5 separate arbiters per slave: - AW channel arbiter (write address) - W channel arbiter (write data - locked to AW grant) - B channel arbiter (write response - routed by ID) - AR channel arbiter (read address) - R channel arbiter (read data - routed by ID) - Round-robin with burst locking - Separate read/write paths (no head-of-line blocking)

3. Data Multiplexing - Mux master signals to selected slave - ID-based response routing (B, R channels) - Burst tracking (hold grant until xlast)

4. Transaction Tracking - ID tables per slave for out-of-order support - Track {Master ID, Transaction ID} → Master mapping - Required for routing B/R channels back to correct master

5. Optional Performance Counters - Transaction counts per master/slave - Arbitration conflict counts - Latency histograms

## 22.6	3. Functional Requirements

### 22.6.1	3.1 AXI4 Protocol Compliance

FR-1: Full AXI4 Protocol Support - Support all 5 AXI4 channels: AW, W, B, AR, R - Comply with AMBA AXI4 specification (ARM IHI 0022) - Support burst lengths: 1-256 beats (AWLEN/ARLEN = 0-255) - Support burst types: INCR, WRAP, FIXED - Support burst sizes: 1-128 bytes (AWSIZE/ARSIZE = 0-7)

FR-2: Out-of-Order Transaction Support - Route responses via ID matching - Maintain transaction ID integrity (AWID → BID, ARID → RID) - Support configurable ID width (1-16 bits) - Track up to 2^ID_WIDTH outstanding transactions per slave

FR-3: Atomic Operations - Support exclusive access (AWLOCK/ARLOCK) - Track exclusive monitor per slave - Generate BRESP/RRESP errors for failed exclusives

### 22.6.2	3.2 Address Decoding

FR-4: Configurable Address Map - Support M masters × S slaves - Each slave has base address and size - Non-overlapping address ranges (verified at generation) - Default slave for unmapped addresses (optional)

FR-5: Address Range Configuration

# Example address map
address_map = {
    0: {'base': 0x00000000, 'size': 0x40000000, 'name': 'DDR'},      # 1GB
    1: {'base': 0x40000000, 'size': 0x00100000, 'name': 'SRAM'},     # 1MB
    2: {'base': 0x50000000, 'size': 0x01000000, 'name': 'PCIE'},     # 16MB
    3: {'base': 0x60000000, 'size': 0x00010000, 'name': 'Peripherals'} # 64KB
}

### 22.6.3	3.3 Arbitration Strategy

FR-6: Round-Robin Arbitration - Fair bandwidth allocation (no starvation) - Separate arbiters for AW and AR channels - Burst locking: Grant held until xlast (WLAST/RLAST) - Configurable arbitration policy (round-robin default)

FR-7: Read/Write Independence - Separate read and write paths - No head-of-line blocking between read/write - Concurrent read and write to same slave (if slave supports)

### 22.6.4	3.4 Burst Handling

FR-8: Burst Optimization - Pipelined burst transfers (overlap address and data) - No artificial burst splitting - Full AXI4 burst protocol support

FR-9: Interleaving Constraints - W channel locked to AW grant master - R channel routed by transaction ID - Support ID-based interleaving (slave-dependent)

## 22.7	4. Non-Functional Requirements

### 22.7.1	4.1 Performance

NFR-1: Latency - Single-beat read: ≤ 3 cycles (address decode + arbitration + mux) - Single-beat write: ≤ 3 cycles (address decode + arbitration + mux) - Burst transfer: No additional latency per beat (pipelined)

NFR-2: Throughput - Concurrent transfers: All M×S paths can transfer simultaneously - Burst efficiency: Line-rate data transfer after address phase - No artificial stalls: Crossbar adds no wait states beyond arbitration

NFR-3: Fmax - Target: 300-400 MHz on Xilinx UltraScale+ - Registered outputs: All slave outputs registered for timing closure - Pipelineable: Optional pipeline stages for >400 MHz

### 22.7.2	4.2 Resource Usage (Estimated)

M = 4 masters, S = 4 slaves, DATA_WIDTH = 512, ADDR_WIDTH = 64, ID_WIDTH = 4:

| Resource | Flat Crossbar | Notes |
| --- | --- | --- |
| LUTs | ~2,500 | Address decode + arbiters + mux |
| FFs | ~3,000 | Registered outputs + ID tables |
| BRAM | 0 | Distributed RAM for small ID tables |
| DSP | 0 | No arithmetic operations |

Scaling: ~150 LUTs per M×S connection

### 22.7.3	4.3 Quality Requirements

NFR-4: Code Generation Quality - Lint-clean SystemVerilog (Verilator) - Synthesizable (Vivado, Yosys, Design Compiler) - No vendor-specific primitives (technology-agnostic) - Clear structure (commented, readable)

NFR-5: Verification - CocoTB testbench framework - Transaction-level verification - Out-of-order test scenarios - Burst interleaving tests - >95% functional coverage

## 22.8	5. Interface Specifications

### 22.8.1	5.1 AXI4 Master Interfaces (M × 5 channels)

Write Address Channel (AW):

input  logic [ADDR_WIDTH-1:0]   s_axi_awaddr  [NUM_MASTERS];
input  logic [ID_WIDTH-1:0]     s_axi_awid    [NUM_MASTERS];
input  logic [7:0]              s_axi_awlen   [NUM_MASTERS];  // Burst length-1
input  logic [2:0]              s_axi_awsize  [NUM_MASTERS];  // Burst size
input  logic [1:0]              s_axi_awburst [NUM_MASTERS];  // INCR/WRAP/FIXED
input  logic                    s_axi_awlock  [NUM_MASTERS];  // Exclusive access
input  logic [3:0]              s_axi_awcache [NUM_MASTERS];  // Cache attributes
input  logic [2:0]              s_axi_awprot  [NUM_MASTERS];  // Protection type
input  logic                    s_axi_awvalid [NUM_MASTERS];
output logic                    s_axi_awready [NUM_MASTERS];

Write Data Channel (W):

input  logic [DATA_WIDTH-1:0]   s_axi_wdata  [NUM_MASTERS];
input  logic [DATA_WIDTH/8-1:0] s_axi_wstrb  [NUM_MASTERS];  // Byte strobes
input  logic                    s_axi_wlast  [NUM_MASTERS];  // Last beat
input  logic                    s_axi_wvalid [NUM_MASTERS];
output logic                    s_axi_wready [NUM_MASTERS];

Write Response Channel (B):

output logic [ID_WIDTH-1:0]     s_axi_bid    [NUM_MASTERS];
output logic [1:0]              s_axi_bresp  [NUM_MASTERS];  // OKAY/EXOKAY/SLVERR/DECERR
output logic                    s_axi_bvalid [NUM_MASTERS];
input  logic                    s_axi_bready [NUM_MASTERS];

Read Address Channel (AR):

input  logic [ADDR_WIDTH-1:0]   s_axi_araddr  [NUM_MASTERS];
input  logic [ID_WIDTH-1:0]     s_axi_arid    [NUM_MASTERS];
input  logic [7:0]              s_axi_arlen   [NUM_MASTERS];
input  logic [2:0]              s_axi_arsize  [NUM_MASTERS];
input  logic [1:0]              s_axi_arburst [NUM_MASTERS];
input  logic                    s_axi_arlock  [NUM_MASTERS];
input  logic [3:0]              s_axi_arcache [NUM_MASTERS];
input  logic [2:0]              s_axi_arprot  [NUM_MASTERS];
input  logic                    s_axi_arvalid [NUM_MASTERS];
output logic                    s_axi_arready [NUM_MASTERS];

Read Data Channel (R):

output logic [DATA_WIDTH-1:0]   s_axi_rdata  [NUM_MASTERS];
output logic [ID_WIDTH-1:0]     s_axi_rid    [NUM_MASTERS];
output logic [1:0]              s_axi_rresp  [NUM_MASTERS];
output logic                    s_axi_rlast  [NUM_MASTERS];
output logic                    s_axi_rvalid [NUM_MASTERS];
input  logic                    s_axi_rready [NUM_MASTERS];

### 22.8.2	5.2 AXI4 Slave Interfaces (S × 5 channels)

Mirror of master interfaces, with M → S direction reversed.

### 22.8.3	5.3 Configuration Parameters

parameter int NUM_MASTERS   = 4;          // Number of master interfaces
parameter int NUM_SLAVES    = 4;          // Number of slave interfaces
parameter int DATA_WIDTH    = 512;        // Data bus width (bits)
parameter int ADDR_WIDTH    = 64;         // Address bus width (bits)
parameter int ID_WIDTH      = 4;          // Transaction ID width (bits)
parameter int MAX_BURST_LEN = 256;        // Maximum burst length (beats)
parameter bit PIPELINE_OUTPUTS = 1;       // Register slave outputs
parameter bit ENABLE_COUNTERS  = 1;       // Performance counters

## 22.9	6. Performance Modeling

### 22.9.1	6.1 Analytical Model

Latency Components (Flat Crossbar):

Single-Beat Read Latency:
  1. Address Decode:      0 cycles (combinatorial)
  2. Arbitration:         1 cycle  (AR arbiter decision)
  3. Address Mux:         0 cycles (combinatorial)
  4. Slave Access:        (slave-dependent, not crossbar)
  5. Response Mux:        1 cycle  (R channel ID lookup + mux)
  6. Output Register:     1 cycle  (optional, for timing closure)

  Total (no pipeline):    2 cycles
  Total (pipelined):      3 cycles

Burst Transfer Throughput:

After address phase completes, data transfer is line-rate:
  - W channel: 1 beat/cycle (locked to AW grant)
  - R channel: 1 beat/cycle (ID-routed from slave)

Example: 256-beat burst
  Address phase:  2-3 cycles (one-time)
  Data phase:     256 cycles (line-rate)
  Total:          258-259 cycles
  Efficiency:     98.8%

Concurrent Throughput:

Maximum concurrent transfers (4×4 crossbar):
  - Read:  4 concurrent (one per master-slave pair)
  - Write: 4 concurrent (one per master-slave pair)
  - Total: 8 concurrent read+write (separate paths)

Throughput @ 100 MHz, 512-bit data:
  Per path:   100 MHz × 512 bits = 51.2 Gbps
  Total:      8 paths × 51.2 Gbps = 409.6 Gbps theoretical

### 22.9.2	6.2 Resource Scaling

LUT Usage Formula (empirical):

LUTs ≈ 500 (base) + 150 × M × S + 20 × ID_TABLE_DEPTH

Example (4×4 crossbar, ID_WIDTH=4, ID_TABLE_DEPTH=16 per slave):

LUTs ≈ 500 + 150 × 16 + 20 × 16 × 4
     ≈ 500 + 2,400 + 1,280
     ≈ 4,180 LUTs

### 22.9.3	6.3 Comparison with Other Crossbars

| Crossbar Type | Latency | Throughput | Complexity | Use Case |
| --- | --- | --- | --- | --- |
| APB | 1-2 cycles | Low (serialized) | Low | Control registers |
| AXI-Stream (Delta) | 2 cycles | High (streaming) | Medium | Data streaming |
| AXI4 (Bridge) | 2-3 cycles | High (burst) | High | Memory-mapped I/O |
| AXI4 + Slices | 4-6 cycles | High (burst) | Very High | >400 MHz designs |

Bridge Sweet Spot: High-performance memory-mapped interconnects where out-of-order and burst efficiency are critical.

## 22.10	7. Generator Architecture

### 22.10.1	7.1 Python Generator Structure

class BridgeGenerator:
    """AXI4 crossbar RTL generator"""

    def __init__(self, config):
        self.num_masters = config.num_masters
        self.num_slaves = config.num_slaves
        self.data_width = config.data_width
        self.addr_width = config.addr_width
        self.id_width = config.id_width
        self.address_map = config.address_map

    def generate_address_decode(self) -> str:
        """Generate address range decode logic"""
        # For each master × slave, check if address in range
        # More complex than AXIS (uses address), simpler than full decode

    def generate_aw_arbiter(self, slave_idx) -> str:
        """Generate write address channel arbiter for one slave"""
        # Round-robin arbiter
        # Grants locked until corresponding B response completes

    def generate_ar_arbiter(self, slave_idx) -> str:
        """Generate read address channel arbiter for one slave"""
        # Round-robin arbiter
        # Grants locked until corresponding R response completes (RLAST)

    def generate_w_channel_mux(self, slave_idx) -> str:
        """Generate write data channel multiplexer"""
        # W channel follows AW grant (locked until WLAST)

    def generate_b_channel_demux(self, slave_idx) -> str:
        """Generate write response channel demultiplexer"""
        # Route by ID: lookup master from {slave_idx, BID}

    def generate_r_channel_demux(self, slave_idx) -> str:
        """Generate read data channel demultiplexer"""
        # Route by ID: lookup master from {slave_idx, RID}

    def generate_id_table(self, slave_idx) -> str:
        """Generate transaction ID tracking table"""
        # Maps {slave, transaction_id} → master_id
        # Indexed on AW/AR grant, looked up on B/R response

    def generate_crossbar(self) -> str:
        """Generate complete crossbar module"""
        # Instantiate all arbiters, muxes, demuxes, ID tables

### 22.10.2	7.2 Address Map Configuration

Python Configuration:

bridge_config = {
    'num_masters': 4,
    'num_slaves': 4,
    'data_width': 512,
    'addr_width': 64,
    'id_width': 4,
    'address_map': {
        0: {'base': 0x00000000, 'size': 0x40000000, 'name': 'DDR'},
        1: {'base': 0x40000000, 'size': 0x00100000, 'name': 'SRAM'},
        2: {'base': 0x50000000, 'size': 0x01000000, 'name': 'PCIE'},
        3: {'base': 0x60000000, 'size': 0x00010000, 'name': 'Peripherals'}
    },
    'pipeline_outputs': True,
    'enable_counters': True
}

Generated Address Decode:

// Address decode for each master
always_comb begin
    for (int s = 0; s < NUM_SLAVES; s++)
        aw_request_matrix[s] = '0;

    for (int m = 0; m < NUM_MASTERS; m++) begin
        if (s_axi_awvalid[m]) begin
            // Slave 0: DDR (0x00000000 - 0x3FFFFFFF)
            if (s_axi_awaddr[m] >= 64'h00000000 &&
                s_axi_awaddr[m] < 64'h40000000)
                aw_request_matrix[0][m] = 1'b1;

            // Slave 1: SRAM (0x40000000 - 0x400FFFFF)
            if (s_axi_awaddr[m] >= 64'h40000000 &&
                s_axi_awaddr[m] < 64'h40100000)
                aw_request_matrix[1][m] = 1'b1;

            // ... slaves 2-3 ...
        end
    end
end

## 22.11	8. Comparison with APB and Delta

### 22.11.1	8.1 Code Reuse from APB Generator

Similar Components (~70% reuse): - Address range decode logic (same pattern, different signals) - Round-robin arbiter (same algorithm) - Data multiplexing pattern (same approach) - Backpressure handling (similar to PREADY)

New Components for Bridge: - 5× the arbiters (AW, W, B, AR, R instead of single channel) - ID-based routing (B and R channel demuxing) - Transaction tracking (ID tables for out-of-order) - Burst handling (grant locking until xlast)

Migration Effort from APB: - ~120 minutes (vs ~75 min for AXIS, due to higher complexity) - Most time: ID table logic and response demuxing

### 22.11.2	8.2 Code Reuse from Delta Generator

Similar Components (~60% reuse): - Python generation framework - Command-line interface - Performance modeling structure - Arbitration pattern (round-robin with locking)

Key Differences: - 5 channels vs 1 channel (Delta only has TDATA/TVALID/TREADY/TLAST) - ID-based routing vs TDEST-based routing - Address decode vs TDEST decode (Bridge more complex) - Transaction tracking vs packet atomicity (different mechanisms)

### 22.11.3	8.3 Complexity Comparison

| Metric | APB Crossbar | Delta (AXIS) | Bridge (AXI4) |
| --- | --- | --- | --- |
| Channels to arbitrate | 1 | 1 | 5 ★ |
| Request generation | Address ranges | TDEST decode | Address ranges |
| Response routing | Grant-based | Grant-based | ID-based ★ |
| Burst support | No | Packet (TLAST) | Yes (AWLEN/ARLEN) ★ |
| Out-of-order | No | No | Yes (ID tables) ★ |
| Transaction tracking | No | No | Yes ★ |
| Lines of Python | ~500 | ~697 | ~900 (est.) |
| Lines of generated SV | ~200 (4×4) | ~250 (4×4) | ~400 (4×4) (est.) |

★ = Additional complexity in Bridge

## 22.12	9. Use Cases

### 22.12.1	9.1 Multi-Core Processor Interconnect

Scenario: 4 CPU cores + GPU accessing DDR + SRAM + Peripherals

Configuration:
  Masters:  5 (4 CPUs, 1 GPU)
  Slaves:   3 (DDR, SRAM, Peripherals)
  Data:     512-bit (cache line width)
  Address:  64-bit (large memory space)
  ID:       4-bit (up to 16 outstanding per master)

Benefits: - Concurrent access to all slaves - Out-of-order completion for high-performance CPUs - Burst transfers for cache line fills - Separate read/write paths (no head-of-line blocking)

### 22.12.2	9.2 DMA + Accelerator System

Scenario: DMA engine + 4 accelerators accessing shared memory

Configuration:
  Masters:  5 (1 DMA, 4 accelerators)
  Slaves:   2 (DDR, Control registers)
  Data:     512-bit (high-bandwidth DMA)
  Address:  32-bit (limited address space)
  ID:       2-bit (simple ID space)

Benefits: - High-bandwidth memory access for DMA - Fair arbitration prevents accelerator starvation - Control register access doesn’t block data transfers

### 22.12.3	9.3 FPGA System Integration

Scenario: MicroBlaze CPU + custom accelerators + memory controllers

Configuration:
  Masters:  8 (1 CPU, 7 accelerators)
  Slaves:   4 (DDR, BRAM, AXI GPIO, AXI DMA)
  Data:     128-bit (AXI4 standard width)
  Address:  32-bit (standard FPGA address space)
  ID:       4-bit (moderate outstanding transactions)

Benefits: - Standard AXI4 interfaces (Xilinx IP compatibility) - Scalable to many masters/slaves - Performance counters for profiling

## 22.13	10. Testing Strategy

### 22.13.1	10.1 FUB (Functional Unit Block) Tests

Address Decode Tests: - Verify all address ranges correctly decoded - Test boundary conditions (base, base+size-1) - Test unmapped addresses (error response)

Arbiter Tests: - Round-robin fairness (all masters get turns) - Burst locking (grant held until xlast) - Starvation prevention

ID Table Tests: - Correct ID → master mapping - Out-of-order transaction handling - Table full condition

Mux/Demux Tests: - Data integrity through crossbar - Response routing to correct master - Concurrent transfers don’t interfere

### 22.13.2	10.2 Integration Tests

Single-Master, Single-Slave: - Basic read/write transactions - Burst transfers (various lengths) - Out-of-order completions

Multi-Master, Single-Slave: - Arbitration correctness - Fairness verification - Burst interleaving (if supported)

Multi-Master, Multi-Slave: - Concurrent transfers - No crosstalk between paths - Full M×S matrix coverage

Stress Tests: - All masters active simultaneously - Maximum burst lengths - Full ID space utilization - Back-to-back bursts

### 22.13.3	10.3 Performance Validation

Latency Measurement: - Single-beat read: measure actual vs analytical - Single-beat write: measure actual vs analytical - Compare with specification (≤3 cycles)

Throughput Measurement: - Burst transfer efficiency (should be ~98%) - Concurrent path throughput (should be line-rate × M×S) - Compare with theoretical maximum

Resource Validation: - Synthesize for Xilinx UltraScale+ - Compare LUT/FF usage with estimates - Verify Fmax ≥ 300 MHz

## 22.14	11. Documentation Plan

### 22.14.1	11.1 Specifications (Before Code)

- ☒ PRD.md - This document (complete requirements)

- ☐ README.md - User guide with quick start

- ☐ BRIDGE_VS_APB_GENERATOR.md - Migration guide from APB

- ☐ BRIDGE_VS_DELTA_GENERATOR.md - Comparison with Delta

- ☐ AXI4_PROTOCOL_GUIDE.md - AXI4 primer for students

### 22.14.2	11.2 Performance Analysis (Before Implementation)

- ☐ bin/bridge_performance_model.py - Analytical model

  - Latency calculations (single-beat, burst)

  - Throughput estimates (concurrent paths)

  - Resource estimates (LUT/FF scaling)

- ☐ bin/bridge_simulator.py - Discrete event simulation (optional)

  - Cycle-accurate modeling

  - Traffic pattern support

  - Validation against RTL

### 22.14.3	11.3 Code Generation

- ☐ bin/bridge_generator.py - Main RTL generator

  - Command-line interface

  - Address map configuration

  - Generated RTL output

### 22.14.4	11.4 Verification

- ☐ dv/tests/ - CocoTB testbenches

  - FUB tests (individual blocks)

  - Integration tests (multi-block)

  - Stress tests (corner cases)

## 22.15	12. Success Metrics

### 22.15.1	12.1 Functional Completeness

- ☐ Generates working AXI4 crossbar RTL

- ☐ Passes all CocoTB tests

- ☐ Verilator lint clean

- ☐ Xilinx Vivado synthesis clean

### 22.15.2	12.2 Performance Targets

- ☐ Latency ≤ 3 cycles (measured in simulation)

- ☐ Throughput = line-rate × M×S paths (measured)

- ☐ Fmax ≥ 300 MHz (post-synthesis)

### 22.15.3	12.3 Educational Value

- ☐ Complete specifications demonstrating rigor

- ☐ Performance models validate requirements

- ☐ Clear code structure (readable generated RTL)

- ☐ Integration examples provided

### 22.15.4	12.4 Reusability

- ☐ ~70% code reuse from APB generator (measured by LOC)

- ☐ Similar patterns to Delta generator

- ☐ Can be extended (weighted arbitration, QoS, etc.)

## 22.16	12. Attribution and Contribution Guidelines

### 22.16.1	12.1 Git Commit Attribution

When creating git commits for Bridge documentation or implementation:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: Bridge documentation and organization receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

## 22.17	12.2 PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

/mnt/data/github/rtldesignsherpa/projects/components/bridge/docs/

Quick Command: Use the provided shell script:

cd /mnt/data/github/rtldesignsherpa/projects/components/bridge/docs
./generate_pdf.sh

The shell script will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the bridge_spec index file 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

## 22.18	13. Future Enhancements

### 22.18.1	13.1 Short-Term (Post-Initial Release)

- ☐ Optional pipeline stages - For Fmax >400 MHz

- ☐ Weighted arbitration - QoS support

- ☐ Default slave - Unmapped address handling

- ☐ Exclusive monitor - Full atomic operation support

### 22.18.2	13.2 Long-Term

- ☐ Tree topology - Hierarchical crossbar (like Delta)

- ☐ AXI4-Lite variant - Simplified for control registers

- ☐ ACE support - Coherent cache interconnect

- ☐ GUI configurator - Visual address map setup

## 22.19	14. Risk Assessment

| Risk | Probability | Impact | Mitigation |
| --- | --- | --- | --- |
| ID table complexity | Medium | High | Start with small ID_WIDTH (2-4), test thoroughly |
| Out-of-order corner cases | High | High | Extensive CocoTB tests with random delays |
| Fmax below target | Low | Medium | Optional pipeline stages for timing closure |
| Resource usage exceeds | Low | Low | Empirical formulas guide expectations |
| Burst interleaving bugs | Medium | High | Separate test suite for burst scenarios |

## 22.20	15. Project Timeline (Estimated)

Week 1: Specifications and Models - [x] Day 1-2: PRD.md (complete) - [ ] Day 3-4: Performance modeling (analytical) - [ ] Day 5-7: README.md, migration guides

Week 2-3: Core Implementation - [ ] Day 1-3: Address decode + arbiter generation - [ ] Day 4-5: Data mux/demux generation - [ ] Day 6-8: ID table generation - [ ] Day 9-10: Integration and testing

Week 4: Verification and Examples - [ ] Day 1-5: CocoTB testbenches - [ ] Day 6-7: Integration examples

## 22.21	16. References

AXI4 Specifications: - ARM IHI 0022 - AMBA AXI and ACE Protocol Specification - Xilinx UG1037 - Vivado AXI Reference Guide

Related Projects: - APB Crossbar - Simple register bus crossbar (existing) - Delta (AXIS Crossbar) - Streaming data crossbar (projects/components/delta/) - RAPIDS - DMA engine with AXI4 masters (rtl/miop/)

Tools: - Verilator - RTL linting and simulation - CocoTB - Python-based verification - Xilinx Vivado - FPGA synthesis

## 22.22	17. Glossary

- AXI4: Advanced eXtensible Interface version 4 (AMBA standard)

- Burst: Multi-beat transaction (AWLEN/ARLEN > 0)

- Crossbar: Full M×N interconnect matrix

- ID: Transaction identifier for out-of-order support

- Out-of-order: Responses can return in different order than requests

- xlast: WLAST (write) or RLAST (read) - last beat indicator

Version: 1.0 Status: ✓ Specification Complete - Ready for Implementation Next Steps: Create performance models, then implement generator

Project Bridge - Connecting masters and slaves across the divide 🌉

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Claude Code Guide: Bridge Subsystem

Version: 2.1 Last Updated: 2025-11-03 Purpose: AI-specific guidance for working with Bridge subsystem

## 23.1	🚨 CRITICAL: Read Architecture Document First

Before making ANY changes to bridge generator or understanding signal flow:

📖 READ: projects/components/bridge/docs/BRIDGE_ARCHITECTURE.md

This document contains the definitive bridge architecture including: - Correct signal flow (wrappers → decoder → converters → crossbar → slaves) - Component purposes and placement - Common misconceptions that previous agents made - Why there’s NO fixed crossbar width

If you skip this document, you WILL make incorrect assumptions.

## 23.2	Quick Context

What: Bridge - Two complementary AXI4 Crossbar Generators (framework-based and CSV-based) Status: 🟢 Phase 2 Complete - CSV generator with channel-specific masters (wr/rd/rw) Your Role: Help users configure CSV files, generate bridges, understand architecture, and create tests

📖 Complete Documentation (Read in This Order): 1. projects/components/bridge/docs/BRIDGE_ARCHITECTURE.md ← START HERE (architecture reference) 2. projects/components/bridge/PRD.md ← Product requirements 3. projects/components/bridge/BRIDGE_CURRENT_STATE.md ← Current implementation review 4. projects/components/bridge/docs/BRIDGE_ARCHITECTURE_DIAGRAMS.md ← Visual architecture diagrams 5. projects/components/bridge/CSV_BRIDGE_STATUS.md ← CSV generator status (Phase 1 & 2) 6. projects/components/bridge/docs/bridge_spec/bridge_index.md ← Detailed specification

## 23.3	Target Architecture: Intelligent Width-Aware Routing

Core Principle: Direct connections where possible, converters only where needed, no fixed crossbar width.

### 23.3.1	Efficient Multi-Width Design

Master_A (64b)
  ├─ Direct → Slave_0 (64b)           [0 conversions, minimal latency]
  ├─ Conv(64→128) → Slave_1 (128b)    [1 conversion]
  └─ Conv(64→512) → Slave_2 (512b)    [1 conversion]

Master_B (512b)
  ├─ Conv(512→64) → Slave_0 (64b)     [1 conversion]
  ├─ Conv(512→128) → Slave_1 (128b)   [1 conversion]
  └─ Direct → Slave_2 (512b)          [0 conversions, full bandwidth]

Router Logic: Address decoder determines target slave, selects correctly-sized path for that master-slave pair.

### 23.3.2	Why This Architecture

✗ Naive Fixed-Width Approach (Don’t Do This):

Master (64b) → Upsize(64→256) → Fixed 256b Crossbar → Downsize(256→64) → Slave (64b)

- TWO conversions for same-width connections (wasteful!)

- Reduced bandwidth on narrow paths

- Unnecessary logic and area

- Higher latency

✓ Intelligent Routing (Target):

Master (64b) → Router → Direct Connection → Slave (64b)

- ZERO conversions for matching widths

- Full native bandwidth

- Minimal logic

- Lowest latency

### 23.3.3	Per-Master Output Paths

Each master has N output paths (one per unique slave width it connects to):

// Master_A connects to slaves at 64b, 128b, 512b
// Generate 3 output paths:

logic [63:0]  master_a_64b_wdata;   // For 64b slaves (direct)
logic [127:0] master_a_128b_wdata;  // For 128b slaves (via converter)
logic [511:0] master_a_512b_wdata;  // For 512b slaves (via converter)

// Router selects based on address decode:
always_comb begin
    case (decoded_slave_id)
        SLAVE_0: select master_a_64b_wdata;   // Slave_0 is 64b
        SLAVE_1: select master_a_128b_wdata;  // Slave_1 is 128b
        SLAVE_2: select master_a_512b_wdata;  // Slave_2 is 512b
    endcase
end

### 23.3.4	Benefits

- Resource Efficient - Only instantiate converters actually needed

- Maximum Performance - Direct paths have zero conversion overhead

- Optimal Bandwidth - No artificial width bottlenecks

- Lower Latency - Minimal logic in critical path for matching widths

- Scalable - Works for any combination of master/slave widths

### 23.3.5	Implementation Status

Current State: Fixed-width crossbar with master-side upsizing (Phase 1 architecture) Target State: Intelligent per-master multi-width routing (your vision) Migration: Requires generator architecture rework (see TASKS.md)

## 23.4	🚨 CRITICAL RULE #0: RTL Regeneration Requirements

⚠ READ THIS FIRST - FAILURE TO FOLLOW CAUSES TEST FAILURES ⚠

### 23.4.1	The Golden Rule

ALL generated RTL files MUST be deleted and regenerated together whenever ANY generator code changes.

### 23.4.2	Why This Is Non-Negotiable

Generated RTL files have interdependencies: - bridge_axi4_flat_*.sv may be instantiated by bridge_ooo_with_arbiter.sv - bridge_wrapper_*.sv may wrap bridge_axi4_flat_*.sv - Generator changes affect signal names, port widths, interface structure - Partial regeneration creates version mismatches that cause silent failures

### 23.4.3	Real Example (This Session)

# ✗ WHAT WENT WRONG:
1. Updated bridge_address_arbiter.py (address decode logic)
2. Regenerated ONLY bridge_axi4_flat_5x3.sv
3. Did NOT regenerate bridge_ooo_with_arbiter.sv (wrapper)
4. Result: All tests that were passing now FAIL
5. Cause: Version mismatch between wrapper and core bridge

# ✓ WHAT SHOULD HAVE BEEN DONE:
1. Updated bridge_address_arbiter.py
2. Delete ALL generated files:
   rm rtl/bridge_axi4_flat_*.sv
   rm rtl/bridge_ooo_*.sv
   rm rtl/bridge_wrapper_*.sv
3. Regenerate ALL bridges from scratch
4. Run ALL tests to verify

### 23.4.4	Generator Files That Trigger Full Regeneration

Any change to these files requires regenerating ALL bridges:

- ✓ bridge_generator.py - Core bridge generator

- ✓ bridge_csv_generator.py - CSV-based generator

- ✓ bridge_address_arbiter.py - Address decode logic

- ✓ bridge_channel_router.py - Channel routing

- ✓ bridge_response_router.py - Response routing

- ✓ bridge_amba_integrator.py - AMBA integration

- ✓ bridge_wrapper_generator.py - Wrapper generation

- ✓ ANY Python file in projects/components/bridge/bin/

### 23.4.5	The Regeneration Workflow

# Step 1: Make generator code changes
vim bridge_address_arbiter.py

# Step 2: Delete ALL generated RTL (be aggressive!)
cd projects/components/bridge/rtl
rm bridge_axi4_flat_*.sv
rm bridge_ooo*.sv
rm bridge_wrapper_*.sv
# Verify deletion
ls *.sv  # Should only show manually-written files like bridge_cam.sv

# Step 3: Regenerate everything
cd ../bin
./regenerate_all_bridges.sh  # If script exists
# OR manually regenerate each topology needed

# Step 4: Run ALL tests
cd ../dv/tests
pytest -v  # ALL tests, not just the one you think changed

# Step 5: Verify git diff makes sense
git diff ../rtl/  # Review all changes

### 23.4.6	Symptoms of Version Mismatch

If you see these symptoms, you probably did partial regeneration:

- ✗ Tests that previously passed now fail

- ✗ “Signal not found” errors in simulation

- ✗ Port width mismatches in instantiation

- ✗ Address decode routing to wrong slaves

- ✗ Missing debug signals (dbg_*)

- ✗ Unexpected compile errors in working code

### 23.4.7	Think Like a Compiler Developer

Generated RTL = Compiled Object Files

When you update a compiler, you don’t selectively recompile - you make clean && make all.

When you update a generator, you don’t selectively regenerate - you delete all and regenerate all.

### 23.4.8	Exception: Hand-Written RTL

These files are never regenerated: - bridge_cam.sv - CAM module (hand-written) - Any file in rtl/ that is NOT generated

Check file headers - generated files say “Generated by: bridge_generator.py”

## 23.5	Critical Pitfalls: slave_select_* Reversion After Handshake

### 23.5.1	The Problem: Combinational Address Decode

The master adapter’s R/B response MUX and the crossbar’s AR/AW gating both relied on <master>_slave_select_{ar,aw} — a combinational decode of fub_axi_{ar,aw}addr. This signal reverts immediately when the address port is freed:

- AR/AW handshake completes (arvalid && arready, awvalid && awready)

- Wrapper pops skid buffer and releases the address port

- fub_axi_araddr / fub_axi_awaddr reverts to next transaction or zero

- Decode flips to different slave

- Response MUX is now gated on WRONG slave

- Response path closes, transaction hangs

This happens BEFORE the converter even finishes pushing to the crossbar, and ALWAYS before response arrives.

### 23.5.2	Visible Symptoms

- ✗ Multi-slave multi-master bridges hang (timeout)

- ✗ Multi-width masters fail basic connectivity

- ✗ APB/AXIL slaves consistently timeout (longer converter latency = more decode changes)

- ✓ Single-slave or single-master systems work fine (no decode change)

### 23.5.3	The Solution (MANDATORY in All Generators)

For Crossbar AR/AW Gating: - Replace <master>_slave_select_ar/aw gates with inline address re-decode on stable m_axi buses - Address on m_axi is held stable across the entire AXI handshake

For Master Adapter Response MUX: - Add per-channel response-tracking FIFOs that capture slave index at request time - Use FIFO head for response MUX instead of live combinational decode

Crossbar Code Pattern:

// WRONG (old implementation):
assign gate_ar_s0 = m0_slave_select_ar && m0_arvalid;
//                  ^^^^^^^^^^^^^^^^^ Reverts after handshake!

// CORRECT (new implementation):
assign gate_ar_s0 = ((m0_m_axi_araddr >= S0_BASE) && 
                      (m0_m_axi_araddr < S0_END) && 
                      m0_arvalid);  // Re-decode on stable m_axi address

Master Adapter Code Pattern:

// WRONG (old implementation):
assign r_slave_select = comb_slave_select_ar;  // Stale after pop!

// CORRECT (new implementation):
// Capture at request time
always_ff @(posedge aclk) begin
    if (fub_axi_arvalid && fub_axi_arready) begin
        ar_trk_fifo <= comb_slave_select_ar;
    end
end

// Use stable value for response MUX
assign r_slave_select = ar_trk_fifo_out;

### 23.5.4	When This Bug Appears

- Works: Single-master, single-slave systems (no address re-evaluation)

- Fails: Multi-master to same slave (multiple addresses decode differently)

- Fails: Multi-slave same master (address decode changes per transaction)

- Fails: Multi-width paths (W beats re-evaluate AW decode mid-burst)

## 23.6	⚠ MANDATORY: Project Organization Pattern

THIS SUBSYSTEM MUST FOLLOW THE RAPIDS/AMBA ORGANIZATIONAL PATTERN - NO EXCEPTIONS

### 23.6.1	Required Directory Structure

projects/components/bridge/
├── bin/                           # Bridge-specific tools (generators, scripts)
│   └── bridge_generator.py       # AXI4 crossbar generator
├── docs/                          # Design documentation
├── dv/                            # Design Verification (MANDATORY structure)
│   ├── tbclasses/                 # Testbench classes (MANDATORY - TB classes here!)
│   │   ├── __init__.py
│   │   └── bridge_axi4_flat_tb.py  # Reusable TB class
│   ├── components/                # Bridge-specific BFMs (if needed)
│   │   └── __init__.py
│   ├── scoreboards/               # Bridge-specific scoreboards (if needed)
│   │   └── __init__.py
│   └── tests/                     # All test files (test runners only)
│       ├── conftest.py            # Pytest configuration (MANDATORY)
│       ├── fub_tests/             # Functional unit block tests
│       │   └── basic/             # Basic bridge tests
│       ├── integration_tests/     # Multi-bridge integration
│       └── system_tests/          # Full system tests
├── rtl/                           # RTL source files
│   └── generated/                 # Generated bridge crossbars
├── CLAUDE.md                      # This file
├── PRD.md                         # Product requirements
└── IMPLEMENTATION_STATUS.md       # Development status

### 23.6.2	Testbench Class Location (MANDATORY)

✗ WRONG: Testbench class in test file

# projects/components/bridge/dv/tests/test_bridge_axi4_2x2.py
class BridgeAXI4FlatTB:  # ✗ WRONG LOCATION!
    """Embedded TB - NOT REUSABLE"""

✓ CORRECT: Testbench class in PROJECT AREA dv/tbclasses/

# projects/components/bridge/dv/tbclasses/bridge_axi4_flat_tb.py
class BridgeAXI4FlatTB(TBBase):  # ✓ CORRECT LOCATION!
    """Reusable TB class - used across all bridge tests"""

CRITICAL: TB classes are PROJECT-SPECIFIC and MUST be in the project area (projects/components/{name}/dv/tbclasses/), NOT in the framework (bin/TBClasses/).

### 23.6.3	Test File Pattern (MANDATORY)

Test files MUST follow this structure:

# projects/components/bridge/dv/tests/fub_tests/basic/test_bridge_axi4_2x2.py

import os
import pytest
import cocotb
from cocotb_test.simulator import run

# ✓ IMPORT testbench class from PROJECT AREA
import sys
sys.path.insert(0, os.path.join(os.path.dirname(__file__), '../../../..'))
from projects.components.bridge.dv.tbclasses.bridge_axi4_flat_tb import BridgeAXI4FlatTB
from TBClasses.shared.utilities import get_paths, create_view_cmd
from TBClasses.shared.tbbase import TBBase

# ===========================================================================
# COCOTB TEST FUNCTIONS - Prefix with "cocotb_" to prevent pytest collection
# ===========================================================================

@cocotb.test(timeout_time=100, timeout_unit='us')
async def cocotb_test_basic_routing(dut):
    """Test basic address routing"""
    tb = BridgeAXI4FlatTB(dut, num_masters=2, num_slaves=2)  # ✓ Import TB
    await tb.setup_clocks_and_reset()
    result = await tb.test_basic_routing()
    assert result, "Basic routing test failed"

# More cocotb test functions...

# ===========================================================================
# PARAMETER GENERATION - At bottom of file
# ===========================================================================

def generate_bridge_test_params():
    """Generate test parameters for bridge tests"""
    return [
        # (num_masters, num_slaves, data_width, addr_width, id_width)
        (2, 2, 32, 32, 4),
        (4, 4, 32, 32, 4),
    ]

bridge_params = generate_bridge_test_params()

# ===========================================================================
# PYTEST WRAPPER FUNCTIONS - At bottom of file
# ===========================================================================

@pytest.mark.bridge
@pytest.mark.routing
@pytest.mark.parametrize("num_masters, num_slaves, data_width, addr_width, id_width", bridge_params)
def test_basic_routing(request, num_masters, num_slaves, data_width, addr_width, id_width):
    """Pytest wrapper for basic routing test"""
    module, repo_root, tests_dir, log_dir, rtl_dict = get_paths({
        'rtl_bridge': '../../rtl'
    })

    # ... setup verilog_sources, parameters, etc ...

    run(
        verilog_sources=verilog_sources,
        toplevel=f"bridge_axi4_flat_{num_masters}x{num_slaves}",
        module=module,
        testcase="cocotb_test_basic_routing",  # ← cocotb function name
        parameters=rtl_parameters,
        sim_build=sim_build,
        # ...
    )

## 23.7	Critical Rules for This Subsystem

### 23.7.1	Rule #0: Attribution Format for Git Commits

IMPORTANT: When creating git commit messages for Bridge documentation or code:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: Bridge receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

### 23.7.2	Rule #0.1: Testbench Architecture - MANDATORY SEPARATION

⚠ THIS IS A HARD REQUIREMENT - NO EXCEPTIONS ⚠

NEVER embed testbench classes inside test runner files!

The same testbench logic will be reused across multiple test scenarios. Having testbench code only in test files makes it COMPLETELY WORTHLESS for reuse.

MANDATORY Structure:

projects/components/bridge/
├── dv/
│   ├── tbclasses/                   # TB classes HERE (not in framework!)
│   │   ├── __init__.py
│   │   └── bridge_axi4_flat_tb.py  ← REUSABLE TB CLASS
│   ├── components/                  # Bridge-specific BFMs (if needed)
│   │   └── __init__.py
│   ├── scoreboards/                 # Bridge-specific scoreboards
│   │   └── __init__.py
│   └── tests/                       # Test runners
│       ├── conftest.py              ← MANDATORY pytest config
│       ├── fub_tests/
│       │   └── basic/
│       │       └── test_bridge_axi4_2x2.py  ← TEST RUNNER ONLY (imports TB)
│       ├── integration_tests/
│       │   └── test_bridge_multiport.py     ← TEST RUNNER ONLY
│       └── system_tests/
│           └── test_bridge_system.py         ← TEST RUNNER ONLY

Why This Matters:

- Reusability: Same TB class used in:

  - Unit tests (fub_tests/)

  - Integration tests (integration_tests/)

  - System-level tests (system_tests/)

  - User projects (external imports)

- Maintainability: Fix bug once in TB class, all tests benefit

- Composition: TB classes can inherit/compose for complex scenarios

### 23.7.3	Rule #1: All Testbenches Inherit from TBBase

Every testbench class MUST inherit from TBBase:

from TBClasses.shared.tbbase import TBBase

class BridgeAXI4FlatTB(TBBase):
    """Testbench for Bridge crossbar - inherits base functionality"""

    def __init__(self, dut, num_masters=2, num_slaves=2, **kwargs):
        super().__init__(dut)
        # Bridge-specific initialization

TBBase Provides: - Clock management (start_clock, wait_clocks) - Reset utilities (assert_reset, deassert_reset) - Logging (self.log) - Progress tracking (mark_progress) - Safety monitoring (timeouts, memory limits)

### 23.7.4	Rule #2: Mandatory Testbench Methods

Every testbench class MUST implement these three methods:

async def setup_clocks_and_reset(self):
    """Complete initialization - starts clocks and performs reset"""
    await self.start_clock('aclk', freq=10, units='ns')

    # Set config signals before reset (if needed)
    # self.dut.cfg_param.value = initial_value

    # Reset sequence
    await self.assert_reset()
    await self.wait_clocks('aclk', 10)
    await self.deassert_reset()
    await self.wait_clocks('aclk', 5)

async def assert_reset(self):
    """Assert reset signal (active-low for AXI4)"""
    self.dut.aresetn.value = 0

async def deassert_reset(self):
    """Deassert reset signal"""
    self.dut.aresetn.value = 1

Why Required: - Consistency across all testbenches - Reusability for mid-test resets - Clear test structure and intent

### 23.7.5	Rule #3: Use GAXI Components for Protocol Handling

For Bridge testing, use GAXI Master/Slave components for AXI4 channel handling:

from CocoTBFramework.components.gaxi.gaxi_master import GAXIMaster
from CocoTBFramework.components.gaxi.gaxi_slave import GAXISlave
from CocoTBFramework.components.axi4.axi4_field_configs import AXI4FieldConfigHelper

# ✓ CORRECT: Use GAXI for AXI4 channels
self.aw_master = GAXIMaster(
    dut=dut,
    title="AW_M0",
    prefix="s0_axi4_",
    clock=clock,
    field_config=AXI4FieldConfigHelper.create_aw_field_config(id_width, addr_width, 1),
    pkt_prefix="aw",
    multi_sig=True,
    log=log
)

Never manually drive AXI4 valid/ready handshakes - Use GAXI components.

### 23.7.6	Rule #4: Queue-Based Verification

For simple in-order verification, use direct monitor queue access:

# ✓ CORRECT: Direct queue access
aw_pkt = self.aw_slave._recvQ.popleft()
w_pkt = self.w_slave._recvQ.popleft()

# Verify
assert aw_pkt.addr == expected_addr
assert w_pkt.data == expected_data

# ✗ WRONG: Memory model for simple test
memory_model = MemoryModel()  # Unnecessary complexity

When to Use Memory Models: - ✗ Simple in-order tests → Use queue access - ✗ Single-master systems → Use queue access - ✓ Complex out-of-order scenarios → Memory model may help - ✓ Multi-master with address overlap → Memory model tracks state

## 23.8	TOML/CSV-Based Bridge Generator (Phase 2 Complete)

### 23.8.1	Overview

The Bridge generator creates parameterized SystemVerilog crossbars from TOML configuration files with CSV connectivity matrices, eliminating manual RTL editing for complex interconnects.

Key Benefits: - Human-readable configuration - TOML for ports, CSV for connectivity - Custom signal prefixes - Each port has unique prefix (rapids_m_axi_, apb0_, etc.) - Channel-specific masters - Write-only (wr), read-only (rd), or full (rw) - Interface modules - Timing isolation via axi4_master/slave wrappers with configurable skid depths - Automatic converters - Width and protocol conversion inserted automatically - Resource efficient - Only generates needed channels and converters

### 23.8.2	Quick Start

1. Create bridge_mybridge.toml:

[bridge]
  name = "bridge_mybridge"
  description = "Custom bridge example"

  # Default skid buffer depths (can be overridden per port)
  defaults.skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

  masters = [
    {name = "cpu", prefix = "cpu_m_axi", id_width = 4, addr_width = 32, data_width = 64, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "dma", prefix = "dma_m_axi", id_width = 4, addr_width = 32, data_width = 512, user_width = 1,
     interface = {type = "axi4_master", skid_depths = {ar = 4, r = 8, aw = 4, w = 8, b = 4}}}
  ]

  slaves = [
    {name = "ddr", prefix = "ddr_s_axi", id_width = 4, addr_width = 32, data_width = 512, user_width = 1,
     base_addr = 0x00000000, addr_range = 0x80000000, interface = {type = "axi4_slave"}},
    {name = "sram", prefix = "sram_s_axi", id_width = 4, addr_width = 32, data_width = 256, user_width = 1,
     base_addr = 0x80000000, addr_range = 0x80000000, interface = {type = "axi4_slave"}}
  ]

2. Create bridge_mybridge_connectivity.csv:

master\slave,ddr,sram
cpu,1,1
dma,1,1

3. Generate bridge:

cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_mybridge.toml
# Auto-finds bridge_mybridge_connectivity.csv

# Or use bulk generation
python3 bridge_generator.py --bulk bridge_batch.csv

Result: Complete SystemVerilog module with: - Custom port prefixes per port - Timing isolation via interface wrappers - Only needed AXI4 channels (wr/rd/rw optimized) - Width converters for data mismatches - Internal crossbar instantiation - APB/AXI4-Lite converter integration points

### 23.8.3	Configuration Format Details

TOML Port Configuration:

The primary format is now TOML (preferred over CSV for better structure and interface configuration):

Port Specifications: - name - Unique identifier (cpu, dma, ddr, etc.) - prefix - Signal prefix (cpu_m_axi_, ddr_s_axi_, etc.) - id_width - AXI4 ID width in bits - addr_width - Address width in bits - data_width - Data width in bits (32, 64, 128, 256, 512) - user_width - AXI4 user signal width - base_addr - Slave base address (slaves only) - addr_range - Slave address range (slaves only) - interface - Interface wrapper configuration (optional) - type - “axi4_master”, “axi4_slave”, “axi4_master_mon”, “axi4_slave_mon”, or omit for direct connection - skid_depths - Per-channel buffer depths: {ar, r, aw, w, b} (valid: 2, 4, 6, 8)

CSV Connectivity Matrix:

master\slave,slave0,slave1,slave2
master0,1,0,1
master1,0,1,1

- 1 = connected, 0 = not connected

- Partial connectivity supported (not all masters to all slaves)

- Auto-detected based on TOML filename: bridge_name.toml → bridge_name_connectivity.csv

Legacy CSV Format:

For backwards compatibility, the generator still supports CSV port files: - See test_configs/README.md for migration guide from CSV to TOML - TOML is now preferred for new bridges (better structure, interface config support)

### 23.8.4	Channel-Specific Masters (Phase 2 Feature)

Why Channel-Specific? Real hardware often has dedicated read or write masters. Generating all 5 AXI4 channels wastes resources:

Traditional (wasteful):

// Write-only master gets unused read channels
input  logic [63:0]  rapids_descr_m_axi_awaddr,  // ✓ USED
input  logic [511:0] rapids_descr_m_axi_wdata,   // ✓ USED
output logic [7:0]   rapids_descr_m_axi_bid,     // ✓ USED
input  logic [63:0]  rapids_descr_m_axi_araddr,  // ✗ UNUSED (50% waste!)
output logic [511:0] rapids_descr_m_axi_rdata,   // ✗ UNUSED

Channel-Specific (optimized):

rapids_descr_wr,master,axi4,wr,rapids_descr_m_axi_,512,64,8,N/A,N/A

Generated:

// Write-only master - only AW, W, B channels
input  logic [63:0]  rapids_descr_m_axi_awaddr,
input  logic [511:0] rapids_descr_m_axi_wdata,
output logic [7:0]   rapids_descr_m_axi_bid,
// ✓ NO READ CHANNELS (araddr, rdata, etc.)

Resource Savings: - 40-60% fewer ports for dedicated masters - Only necessary width converters instantiated - Channel-aware direct connection wiring - Faster synthesis, smaller netlists

### 23.8.5	Example: RAPIDS-Style Configuration

RAPIDS Architecture: - Descriptor write master (wr) - Writes descriptors to memory - Sink write master (wr) - Writes incoming packets to memory - Source read master (rd) - Reads outgoing packets from memory - CPU master (rw) - Full access for configuration

TOML Configuration (bridge_rapids.toml):

[bridge]
  name = "bridge_rapids"
  description = "RAPIDS accelerator bridge"
  defaults.skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

  masters = [
    {name = "rapids_descr_wr", prefix = "rapids_descr_m_axi", channels = "wr",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "rapids_sink_wr", prefix = "rapids_sink_m_axi", channels = "wr",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "rapids_src_rd", prefix = "rapids_src_m_axi", channels = "rd",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "cpu", prefix = "cpu_m_axi", channels = "rw",
     id_width = 4, addr_width = 32, data_width = 64, user_width = 1,
     interface = {type = "axi4_master"}}
  ]

  slaves = [
    {name = "ddr", prefix = "ddr_s_axi", protocol = "axi4",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     base_addr = 0x80000000, addr_range = 0x80000000,
     interface = {type = "axi4_slave"}},
    {name = "apb_periph", prefix = "apb0_", protocol = "apb",
     addr_width = 32, data_width = 32,
     base_addr = 0x00000000, addr_range = 0x00010000}
  ]

Connectivity CSV (bridge_rapids_connectivity.csv):

master\slave,ddr,apb_periph
rapids_descr_wr,1,0
rapids_sink_wr,1,0
rapids_src_rd,1,0
cpu,1,1

Generated RTL Features: - rapids_descr_wr: 37 signals (write channels only) vs 61 signals (full) = 39% reduction - rapids_sink_wr: 37 signals (write channels only) vs 61 signals (full) = 39% reduction - rapids_src_rd: 24 signals (read channels only) vs 61 signals (full) = 61% reduction - cpu_master: Width converters for 64b→512b upsize (both wr and rd converters) - ddr_controller: Direct 512b connection (no conversion) - apb_periph0: APB converter placeholder (Phase 3)

### 23.8.6	Common User Questions

Q: “How do I generate a bridge?”

A: Three steps:

- Create TOML port configuration file

- Create CSV connectivity matrix

- Run bridge_generator.py

# Create bridge_mybridge.toml (port configuration)
# Create bridge_mybridge_connectivity.csv (connectivity matrix)

cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_mybridge.toml
# Auto-finds bridge_mybridge_connectivity.csv

# Or use bulk generation for multiple bridges
python3 bridge_generator.py --bulk bridge_batch.csv

Q: “What’s the difference between wr/rd/rw channels?”

A: Number of AXI4 channels generated:

- rw (read/write) - All 5 channels: AW, W, B, AR, R

- wr (write-only) - 3 channels: AW, W, B (no read channels)

- rd (read-only) - 2 channels: AR, R (no write channels)

Benefits: 40-60% fewer ports for dedicated masters, less logic, faster synthesis

Q: “Can I add timing isolation to ports?”

A: Yes, use interface configuration:

masters = [
  {name = "cpu", prefix = "cpu_m_axi", ...,
   interface = {type = "axi4_master", skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}}}
]

Available interface types: - "axi4_master" - Timing isolation on master port - "axi4_slave" - Timing isolation on slave port - "axi4_master_mon" - Timing + monitoring - "axi4_slave_mon" - Timing + monitoring - Omit interface field for direct connection

Q: “Can I mix AXI4 and APB slaves?”

A: Yes, use protocol field in TOML:

slaves = [
  {name = "ddr", prefix = "ddr_s_axi", protocol = "axi4", ...},
  {name = "apb0", prefix = "apb0_", protocol = "apb", ...}
]

Generator inserts AXI2APB converters automatically (Phase 3 for full implementation).

Q: “What if data widths don’t match?”

A: Generator inserts width converters automatically:

masters = [
  {name = "cpu", data_width = 64, ...},  # 64b master
]
slaves = [
  {name = "ddr", data_width = 512, ...}  # 512b slave
]
# Generator creates width converter: 64b → 512b

Q: “Do all masters need to connect to all slaves?”

A: No, use partial connectivity in CSV:

master\slave,ddr,sram,apb
rapids_descr,1,1,0     # Connects to ddr and sram only
cpu,1,1,1              # Connects to all three

### 23.8.7	Generator Output Structure

Generated File Contains:

- Module Header - Parameterized with NUM_MASTERS, NUM_SLAVES, widths

- Port Declarations - Custom prefix per port, channel-specific signals

- Internal Signals - Crossbar interface arrays (xbar_m_, xbar_s_)

- Width Converters - Master-side upsize instances (channel-aware)

- Crossbar Instance - Internal AXI4 full crossbar

- Direct Connections - For matching-width interfaces

- APB Converters - Placeholder TODO comments (Phase 3)

Example Output Size: - Simple 2x2 bridge: ~400 lines - Complex 5x3 with mixed protocols: ~900 lines - Includes comprehensive comments and structure

### 23.8.8	Testing Generated Bridges

For Pure AXI4 Bridges (Working Now):

# Generate bridge
python3 bridge_csv_generator.py --ports ports.csv --connectivity conn.csv --name my_bridge --output ../rtl/

# Create testbench (use BridgeAXI4FlatTB from dv/tbclasses/)
cd ../dv/tests/fub_tests/basic
pytest test_my_bridge.py -v

For Mixed AXI4/APB (Requires Phase 3): APB converter placeholders need implementation before end-to-end testing.

## 23.9	Bridge Architecture Quick Reference

### 23.9.1	Generated Bridge Crossbar Structure

module bridge_axi4_flat_2x2 #(
    parameter NUM_MASTERS = 2,
    parameter NUM_SLAVES = 2,
    parameter DATA_WIDTH = 32,
    parameter ADDR_WIDTH = 32,
    parameter ID_WIDTH = 4
) (
    input  logic aclk,
    input  logic aresetn,

    // Master-side interfaces (slave ports on bridge)
    // AW, W, B, AR, R channels for each master

    // Slave-side interfaces (master ports on bridge)
    // AW, W, B, AR, R channels for each slave
);

### 23.9.2	Key Features

- 5-Channel Implementation: Complete AW, W, B, AR, R routing

- Round-Robin Arbitration: Per-slave arbitration with grant locking

- Transaction ID Tracking: Distributed RAM for B/R response routing

- ID-Based Routing: Enables out-of-order responses

- Configurable Parameters: NxM topology, data/addr/ID widths

### 23.9.3	Address Map

Default address map (configurable): - Slave 0: 0x00000000 - 0x0FFFFFFF (256MB) - Slave 1: 0x10000000 - 0x1FFFFFFF (256MB) - Slave N: N * 0x10000000

## 23.10	Test Organization

### 23.10.1	Test Hierarchy

projects/components/bridge/dv/tests/
├── conftest.py              # Pytest configuration with fixtures
├── fub_tests/               # Functional unit block tests
│   └── basic/
│       ├── test_bridge_axi4_2x2.py       # Basic 2x2 tests
│       ├── test_bridge_axi4_4x4.py       # Full 4x4 tests
│       └── test_bridge_axi4_routing.py   # Routing tests
├── integration_tests/       # Multi-bridge scenarios
│   └── test_bridge_cascade.py
└── system_tests/            # Full system tests
    └── test_bridge_dma.py

### 23.10.2	Test Levels

Basic (FUB) Tests: - Individual bridge functionality - Address routing - ID tracking - Arbitration

Integration Tests: - Multi-bridge cascades - Complex topologies - Cross-bridge transactions

System Tests: - Full DMA transfers - Realistic traffic patterns - Performance validation

## 23.11	Common User Questions and Responses

### 23.11.1	Q: “How does the Bridge work?”

A: Direct answer:

The Bridge AXI4 crossbar connects multiple AXI4 masters to multiple slaves:

- Address Decode (AW/AR): Routes master requests to appropriate slave based on address

- Write Path: AW → arbitration → slave, W follows locked grant

- Read Path: AR → arbitration → slave, R returns via ID table

- Arbitration: Per-slave round-robin with grant locking until burst complete

- Response Routing: B/R responses use ID lookup tables (not grant-based)

Key Features: - Out-of-order response support via ID tracking - Burst-aware arbitration (grant locked until WLAST/RLAST) - Configurable NxM topology - Single-clock domain

📖 See: - projects/components/bridge/PRD.md - Complete specification - projects/components/bridge/bin/bridge_generator.py - Generator implementation

### 23.11.2	Q: “How do I generate a Bridge?”

A: Use the bridge_generator.py tool:

cd projects/components/bridge/bin
python bridge_generator.py --masters 2 --slaves 4 --data-width 32 --addr-width 32 --id-width 4 --output ../rtl/bridge_axi4_flat_2x4.sv

Generated files: - RTL: projects/components/bridge/rtl/bridge_axi4_flat_<config>.sv - Contains complete 5-channel crossbar with ID tracking

### 23.11.3	Q: “How do I test a Bridge?”

A: Use the Bridge testbench class:

# Import from project area
import sys
sys.path.insert(0, os.path.join(os.path.dirname(__file__), '../../../..'))
from projects.components.bridge.dv.tbclasses.bridge_axi4_flat_tb import BridgeAXI4FlatTB

@cocotb.test()
async def test_basic(dut):
    tb = BridgeAXI4FlatTB(dut, num_masters=2, num_slaves=2)
    await tb.setup_clocks_and_reset()

    # Send write to slave 0
    await tb.write_transaction(master_idx=0, address=0x00001000, data=0xDEADBEEF)

    # Verify routing
    assert len(tb.aw_slaves[0]._recvQ) > 0, "Slave 0 should receive AW"

Run tests:

cd projects/components/bridge/dv/tests/fub_tests/basic
pytest test_bridge_axi4_2x2.py -v

## 23.12	Anti-Patterns to Avoid

### 23.12.1	✗ Anti-Pattern 1: Embedded Testbench Classes

✗ WRONG: TB class in test file
class BridgeTB:
    """NOT REUSABLE - WRONG LOCATION"""

✓ CORRECT: Import from project area
import sys
sys.path.insert(0, os.path.join(os.path.dirname(__file__), '../../../..'))
from projects.components.bridge.dv.tbclasses.bridge_axi4_flat_tb import BridgeAXI4FlatTB

### 23.12.2	✗ Anti-Pattern 2: Manual AXI4 Handshaking

✗ WRONG: Manual signal driving
self.dut.s0_axi4_awvalid.value = 1
while self.dut.s0_axi4_awready.value == 0:
    await RisingEdge(self.clock)

✓ CORRECT: Use GAXI components
await self.aw_master.send(aw_pkt)

### 23.12.3	✗ Anti-Pattern 3: Memory Models for Simple Tests

✗ WRONG: Unnecessary complexity
memory = MemoryModel()
memory.write(addr, data)
result = memory.read(addr)

✓ CORRECT: Direct queue verification
aw_pkt = self.aw_slave._recvQ.popleft()
assert aw_pkt.addr == expected_addr

## 23.13	Quick Reference

### 23.13.1	Finding Existing Components

# Bridge TB class (in PROJECT AREA, not framework!)
cat projects/components/bridge/dv/tbclasses/bridge_axi4_flat_tb.py

# Bridge generator
cat projects/components/bridge/bin/bridge_generator.py

# Test examples
ls projects/components/bridge/dv/tests/fub_tests/basic/

### 23.13.2	Common Commands

# Generate 2x2 bridge
python projects/components/bridge/bin/bridge_generator.py --masters 2 --slaves 2 --output projects/components/bridge/rtl/bridge_axi4_flat_2x2.sv

# Run tests
pytest projects/components/bridge/dv/tests/fub_tests/basic/ -v

# Lint generated RTL
verilator --lint-only projects/components/bridge/rtl/bridge_axi4_flat_2x2.sv

## 23.14	Remember

- 🏗️ MANDATORY: Testbench architecture - TB classes in framework, tests import them

- 📁 MANDATORY: Directory structure - Follow RAPIDS/AMBA pattern exactly

- 📋 MANDATORY: conftest.py - Must exist in dv/tests/

- 🎯 Use GAXI components - Never manually drive AXI4 handshakes

- 📊 Queue-based verification - Simple tests use direct queue access

- 🏛️ Three-layer architecture - TB (framework) + Test (runner) + Scoreboard (verification)

- ⚙️ Three mandatory methods - setup_clocks_and_reset, assert_reset, deassert_reset

- 🔍 Search first - Use existing components before creating new ones

- 📈 Test scalability - Support basic/medium/full test levels

- 💯 100% success - All tests must achieve 100% success rate

## 23.15	PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

/mnt/data/github/rtldesignsherpa/projects/components/bridge/docs/

Quick Command: Use the provided shell script:

cd /mnt/data/github/rtldesignsherpa/projects/components/bridge/docs
./generate_pdf.sh

The shell script will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the bridge_spec index file 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

Version: 1.0 Last Updated: 2025-10-18 Maintained By: RTL Design Sherpa Project