RTL Design Sherpa

Bridge Micro-Architecture Specification 1.0

January 3, 2026

Table of Contents

1 Bridge Mas Index	37

2 Document Information	37

2.1 Bridge Micro-Architecture Specification	37

2.2 Revision History	37

2.3 Document Purpose	37

2.4 Intended Audience	38

2.5 Related Documents	38

3 Overview	38

3.1 Bridge Micro-Architecture	38

3.2 Key Capabilities	38

3.2.1 Multi-Protocol Support	38

3.2.2 Channel-Specific Masters	39

3.2.3 Automatic Converters	39

3.3 Architecture Summary	39

3.3.1 Block Organization	39

3.3.2 Signal Flow	40

3.4 Document Organization	40

3.5 Related Documentation	40

4 2.1 Master Adapter	40

4.1 2.1.1 Purpose and Function	41

4.2 2.1.2 Block Diagram	41

4.2.1 Figure 2.1: Master Adapter Architecture	41

4.3 2.1.3 Channel Specialization	42

4.3.1 Read-Only Masters	42

4.3.2 Write-Only Masters	42

4.3.3 Read-Write Masters	42

4.4 2.1.4 Skid Buffer Architecture	42

4.4.1 Registered Forward Path	43

4.4.2 Single-Cycle Backpressure Response	43

4.4.3 Pipeline Depth	43

4.5 2.1.5 Bridge ID Management	43

4.5.1 ID Width Calculation	43

4.5.2 ID Injection (Request Path)	43

4.5.3 ID Stripping (Response Path)	44

4.6 2.1.6 Protocol Normalization	44

4.6.1 Valid Address Ranges	44

4.6.2 Burst Constraints	44

4.6.3 Signal Defaults	45

4.7 2.1.6 Response Path Slave Selection Tracking	45

4.7.1 Problem: Stale Slave Select Decode	45

4.7.2 Solution: Per-Channel Response-Tracking FIFOs	45

4.8 2.1.7 Interface Specifications	45

4.8.1 External Master Interface (per master)	45

4.8.2 Internal Crossbar Interface (per master)	46

4.9 2.1.8 Resource Utilization	46

4.9.1 Per-Master Adapter Resources (Typical)	46

4.9.2 Scaling Considerations	46

4.10 2.1.9 Timing Characteristics	46

4.10.1 Latency	46

4.10.2 Throughput	47

4.10.3 Critical Paths	47

4.11 2.1.10 Configuration Parameters	47

4.11.1 Per-Master Parameters (from TOML/CSV)	47

4.11.2 Global Parameters (affect all adapters)	47

4.12 2.1.11 Debug and Observability	47

4.12.1 Recommended Debug Signals	47

4.12.2 Common Issues and Debug	48

4.13 2.1.12 Future Enhancements	48

4.13.1 Planned Features	48

4.13.2 Under Consideration	48

5 2.2 Slave Router	49

5.1 2.2.1 Purpose and Function	49

5.2 2.2.2 Block Diagram	49

5.2.1 Figure 2.2: Slave Router Architecture	49

5.3 2.2.3 Address Decoding Algorithm	50

5.3.1 Configuration-Based Address Maps	50

5.3.2 Decoding Priority	50

5.3.3 Range Checking Logic	50

5.3.4 Power-of-Two Optimization	51

5.4 2.2.4 Request Routing	51

5.4.1 AR Channel Routing	51

5.4.2 AW/W Channel Routing	51

5.4.3 Write Data Tracking FSM	52

5.5 2.2.5 Out-of-Range Handling	52

5.5.1 Detection	52

5.5.2 Error Response Generation	52

5.5.3 Debug Data Patterns	53

5.6 2.2.6 Default Slave Support	53

5.6.1 Configuration	53

5.6.2 Behavior	53

5.7 2.2.7 Address Aliasing	53

5.7.1 Multiple Slaves, Same Address	53

5.7.2 Intentional Aliasing Use-Cases	54

5.8 2.2.8 Configuration Parameters	54

5.8.1 Per-Router Parameters	54

5.8.2 Global Parameters	54

5.9 2.2.9 Resource Utilization	54

5.9.1 Per-Router Resources (Typical)	54

5.9.2 Scaling Considerations	55

5.10 2.2.10 Timing Characteristics	55

5.10.1 Decode Latency	55

5.10.2 Throughput	55

5.10.3 Critical Paths	55

5.11 2.2.11 Debug and Observability	55

5.11.1 Recommended Debug Signals	55

5.11.2 Common Issues and Debug	56

5.12 2.2.12 Verification Considerations	56

5.12.1 Address Decode Tests	56

5.12.2 Write Tracking Tests	56

5.12.3 OOR Error Tests	56

5.13 2.2.13 Performance Optimization	57

5.13.1 Techniques	57

5.14 2.2.14 Future Enhancements	57

5.14.1 Planned Features	57

5.14.2 Under Consideration	57

6 2.3 Crossbar Core	58

6.1 2.3.1 Purpose and Function	58

6.2 2.3.2 Block Diagram	59

6.2.1 Figure 2.3: Crossbar Core Architecture	59

6.3 2.3.3 Connectivity Matrix	61

6.3.1 Full N×M Crossbar	61

6.3.2 Request Path Multiplexing	61

6.3.3 Response Path Demultiplexing	61

6.4 2.3.4 Request Path Architecture	62

6.4.1 Per-Slave Request Arbitration	62

6.4.2 Request Multiplexers with Stable Address Gating	62

6.4.3 AW→W Burst Tracking FIFO	63

6.4.4 Backpressure Propagation	63

6.5 2.3.5 Response Path Architecture	64

6.5.1 Bridge ID Extraction	64

6.5.2 CAM-Based Routing (Optional)	64

6.5.3 Response Demultiplexers	64

6.5.4 Multi-Slave Response Merging	65

6.6 2.3.6 AXI Ordering Requirements	65

6.6.1 Read-After-Write (RAW) Ordering	65

6.6.2 Write-After-Write (WAW) Ordering	65

6.6.3 Out-of-Order (OOO) Completion	66

6.7 2.3.7 Resource Utilization	66

6.7.1 Crossbar Core Resources	66

6.7.2 Scaling with Masters and Slaves	66

6.7.3 Optimization Techniques	66

6.8 2.3.8 Timing Characteristics	67

6.8.1 Latency	67

6.8.2 Throughput	67

6.9 2.3.9 Critical Paths	67

6.9.1 Common Critical Paths	67

6.9.2 Mitigation Strategies	67

6.10 2.3.10 Configuration Parameters	68

6.10.1 Crossbar Parameters (from TOML)	68

6.11 2.3.11 Debug and Observability	68

6.11.1 Recommended Debug Signals	68

6.11.2 Performance Counters	68

6.12 2.3.12 Common Issues and Debug	68

6.13 2.3.13 Verification Considerations	69

6.13.1 Functional Tests	69

6.13.2 Corner Cases	69

6.13.3 Protocol Compliance	69

6.14 2.3.14 Future Enhancements	69

6.14.1 Planned Features	69

6.14.2 Under Consideration	69

7 2.4 Arbitration	70

7.1 2.4.1 Purpose and Function	70

7.2 2.4.2 Arbitration Architecture	70

7.2.1 Per-Slave, Per-Channel Arbiters	70

7.2.2 Block Diagram	71

7.3 2.4.3 Round-Robin Arbitration	72

7.3.1 Algorithm	72

7.3.2 Implementation	72

7.3.3 Characteristics	73

7.4 2.4.4 Fixed-Priority Arbitration	73

7.4.1 Algorithm	73

7.4.2 Implementation	73

7.4.3 Characteristics	74

7.4.4 Starvation Prevention	74

7.5 2.4.5 Weighted Arbitration	74

7.5.1 Algorithm	74

7.5.2 Implementation	74

7.5.3 Characteristics	75

7.6 2.4.6 Burst Handling	75

7.6.1 Grant Locking	75

7.6.2 Implementation	76

7.6.3 Fairness Considerations	76

7.7 2.4.7 Resource Utilization	76

7.7.1 Per-Arbiter Resources	76

7.7.2 Scaling with Master Count	77

7.8 2.4.8 Timing Characteristics	77

7.8.1 Arbitration Latency	77

7.8.2 Critical Paths	77

7.9 2.4.9 Configuration Parameters	78

7.9.1 Arbiter Configuration (TOML)	78

7.10 2.4.10 Debug and Observability	78

7.10.1 Recommended Debug Signals	78

7.10.2 Common Issues and Debug	79

7.11 2.4.11 Verification Considerations	79

7.11.1 Test Scenarios	79

7.11.2 Corner Cases	79

7.12 2.4.12 Performance Impact	80

7.12.1 Arbiter Choice Impact	80

7.12.2 Arbiter Efficiency	80

7.13 2.4.13 Future Enhancements	80

7.13.1 Planned Features	80

7.13.2 Under Consideration	80

8 2.5 ID Management	81

8.1 2.5.1 Purpose and Function	81

8.2 2.5.2 Bridge ID Concept	81

8.2.1 Problem Statement	81

8.2.2 Solution: Bridge ID Injection	82

8.3 2.5.3 Block Diagram	83

8.3.1 Figure 2.5: ID Management Architecture	83

8.4 2.5.4 Bridge ID Width Calculation	85

8.4.1 Formula	85

8.4.2 Examples	85

8.4.3 ID Width Growth	85

8.5 2.5.4a Multi-Master OR-Merge Gating	85

8.6 2.5.5 ID Injection (Request Path)	86

8.6.1 Read Address Channel (AR)	86

8.6.2 Write Address Channel (AW)	86

8.6.3 Injection Timing	86

8.7 2.5.6 ID Extraction (Response Path)	87

8.7.1 Read Data Channel (R)	87

8.7.2 Write Response Channel (B)	87

8.8 2.5.7 Content Addressable Memory (CAM)	87

8.8.1 When to Use CAM	87

8.8.2 CAM Structure	88

8.8.3 CAM Operations	88

8.9 2.5.8 CAM Implementation	88

8.9.1 Parallel CAM (Fast, Resource-Intensive)	88

8.9.2 Sequential CAM (Slow, Resource-Efficient)	89

8.10 2.5.9 Outstanding Transaction Limits	90

8.10.1 Configuration	90

8.10.2 Enforcement	90

8.10.3 Sizing Guidelines	90

8.11 2.5.10 Resource Utilization	90

8.11.1 ID Injection/Extraction Only	90

8.11.2 With CAM	90

8.11.3 Scaling	91

8.12 2.5.11 Timing Characteristics	91

8.12.1 ID Injection Latency	91

8.12.2 ID Extraction/Routing Latency	91

8.12.3 End-to-End Impact	91

8.13 2.5.12 Configuration Parameters	92

8.13.1 ID Management Configuration (TOML)	92

8.14 2.5.13 Debug and Observability	92

8.14.1 Recommended Debug Signals	92

8.14.2 Performance Counters	92

8.15 2.5.14 Common Issues and Debug	93

8.16 2.5.15 Verification Considerations	93

8.16.1 Test Scenarios	93

8.17 2.5.16 Future Enhancements	94

8.17.1 Planned Features	94

8.17.2 Under Consideration	94

9 2.6 Width Conversion	94

9.1 2.6.1 Purpose and Function	94

9.2 2.6.2 Internal Data Width	95

9.2.1 64-Bit Standard	95

9.2.2 Width Conversion Locations	95

9.3 2.6.3 Block Diagram	96

9.3.1 Figure 2.6: Width Conversion Architecture	96

9.4 2.6.4 Upsizing (Narrow to Wide)	97

9.4.1 Overview	97

9.4.2 Write Upsizing	97

9.4.3 Read Upsizing	97

9.4.4 Strobe Mapping (Upsizing)	98

9.5 2.6.5 Downsizing (Wide to Narrow)	98

9.5.1 Overview	98

9.5.2 Write Downsizing	98

9.5.3 Read Downsizing	98

9.5.4 Strobe Mapping (Downsizing)	99

9.6 2.6.6 Address Alignment	99

9.6.1 Address Adjustment for Width	99

9.6.2 Unaligned Access Handling	99

9.7 2.6.7 Burst Length Adjustment	100

9.7.1 Length Calculation	100

9.7.2 Odd Burst Lengths	100

9.8 2.6.8 Resource Utilization	100

9.8.1 Per-Converter Resources	100

9.8.2 Scaling	101

9.9 2.6.9 Timing Characteristics	101

9.9.1 Latency	101

9.9.2 Throughput	101

9.10 2.6.10 Configuration Parameters	102

9.10.1 Width Conversion Configuration (TOML)	102

9.11 2.6.11 Debug and Observability	102

9.11.1 Recommended Debug Signals	102

9.11.2 Common Issues and Debug	103

9.12 2.6.12 Verification Considerations	103

9.12.1 Test Scenarios	103

9.13 2.6.13 Performance Considerations	103

9.13.1 Conversion Overhead	103

9.13.2 Optimization Strategies	104

9.14 2.6.14 Future Enhancements	104

9.14.1 Planned Features	104

9.14.2 Under Consideration	104

10 2.7 Protocol Conversion	104

10.1 2.7.1 Purpose and Function	105

10.2 2.7.2 Supported Protocols	105

10.2.1 Current Support (Phase 2)	105

10.2.2 Current Limitation: AXI4-Lite Conversion	105

10.2.3 Future Support (Phase 2+)	105

10.3 2.7.3 Conversion Architecture Overview	106

10.3.1 Figure 2.7.1: Protocol Conversion Architecture	106

10.4 2.7.4 Block Diagram	106

10.4.1 Figure 2.7: Protocol Conversion Architecture	106

10.5 2.7.5 AXI4-Lite to AXI4 Conversion (Master-Side)	107

10.5.1 AXI4-Lite Protocol Overview	107

10.5.2 Conversion Requirements	107

10.5.3 Signal Mapping	107

10.5.4 Implementation	109

10.5.5 Resource Utilization	111

10.5.6 Performance Impact	111

10.5.7 Configuration	112

10.5.8 Common Issues and Debug	112

10.6 2.7.6 AXI4 to APB Conversion (Slave-Side)	112

10.6.1 APB Protocol Overview	112

10.6.2 APB Signals	112

10.6.3 APB State Machine	113

10.6.4 Read Transaction Conversion	113

10.6.5 Write Transaction Conversion	114

10.6.6 Burst Handling	114

10.6.7 Response Mapping	115

10.7 2.7.5 Implementation	115

10.7.1 AXI4-to-APB Converter FSM	115

10.7.2 Address Generation	116

10.8 2.7.6 Resource Utilization	117

10.8.1 APB Converter Resources	117

10.8.2 Scaling	117

10.9 2.7.7 Timing Characteristics	117

10.9.1 Latency	117

10.9.2 Throughput	117

10.10 2.7.8 Configuration Parameters	118

10.10.1 Protocol Conversion Configuration (TOML)	118

10.11 2.7.9 Debug and Observability	118

10.11.1 Recommended Debug Signals	118

10.11.2 Common Issues and Debug	118

10.12 2.7.10 Verification Considerations	119

10.12.1 Test Scenarios	119

10.13 2.7.11 Performance Considerations	119

10.13.1 When to Use APB	119

10.13.2 APB vs. AXI4 Comparison	119

10.14 2.7.12 Mixed Protocol Bridges	120

10.14.1 Example Configuration	120

10.14.2 Routing Optimization	120

10.15 2.7.13 Master-Side vs Slave-Side Conversion	121

10.15.1 Comparison	121

10.15.2 When to Use Each	121

10.16 2.7.14 Future Protocol Support	121

10.16.1 Planned Features	121

10.16.2 Under Consideration	121

10.17 2.7.14 Best Practices	122

10.17.1 Design Recommendations	122

10.17.2 Performance Tips	122

11 2.8 Response Routing	122

11.1 2.8.1 Purpose and Function	123

11.2 2.8.2 Block Diagram	124

11.2.1 Figure 2.8: Response Routing Architecture	124

11.3 2.8.3 Stable Response Routing via FIFO-Tracked Slave Select	125

11.3.1 Architecture: FIFO-Tracked Master Routing	125

11.3.2 BID Extraction	125

11.3.3 CAM-Based Extraction	126

11.4 2.8.4 Response Demultiplexing	126

11.4.1 Per-Master Response Paths	126

11.4.2 Demux Implementation	126

11.5 2.8.5 Multi-Slave Response Merging	127

11.5.1 Problem Statement	127

11.5.2 Response Arbitration	127

11.5.3 Response Buffering	128

11.6 2.8.6 Backpressure Management	128

11.6.1 Ready Signal Routing	128

11.6.2 Stall Conditions	129

11.7 2.8.7 Response Path Latency	129

11.7.1 End-to-End R Channel	129

11.7.2 End-to-End B Channel	129

11.8 2.8.8 Error Response Routing	130

11.8.1 Slave Error Responses	130

11.8.2 Out-of-Range Error Responses	130

11.9 2.8.9 Resource Utilization	130

11.9.1 Response Router Resources	130

11.9.2 Scaling	131

11.10 2.8.10 Configuration Parameters	131

11.10.1 Response Routing Configuration (TOML)	131

11.11 2.8.11 Debug and Observability	131

11.11.1 Recommended Debug Signals	131

11.11.2 Performance Counters	132

11.12 2.8.12 Common Issues and Debug	132

11.13 2.8.13 Verification Considerations	132

11.13.1 Test Scenarios	132

11.14 2.8.14 Performance Optimization	133

11.14.1 Techniques	133

11.15 2.8.15 Advanced Features	134

11.15.1 Response Reordering (Future)	134

11.15.2 Response Coalescing (Future)	134

11.15.3 Response QoS (Future)	134

11.16 2.8.16 Timing Considerations	134

11.16.1 Critical Paths	134

11.16.2 Optimization Strategies	135

11.17 2.8.17 Future Enhancements	135

11.17.1 Planned Features	135

11.17.2 Under Consideration	135

12 2.9 Error Handling	135

12.1 2.9.1 Purpose and Function	135

12.2 2.9.2 Error Categories	136

12.2.1 Transaction Errors	136

12.2.2 System Errors	136

12.3 2.9.3 Block Diagram	137

12.3.1 Figure 2.9: Error Handling Architecture	137

12.4 2.9.4 Out-of-Range Address Handling	137

12.4.1 Detection	137

12.4.2 Error Response Generation	138

12.4.3 Configurable Data Patterns	139

12.5 2.9.5 Protocol Violation Handling	139

12.5.1 Common Violations	139

12.5.2 Protocol Checker Implementation	139

12.6 2.9.6 Timeout Detection	140

12.6.1 Per-Transaction Watchdog	140

12.6.2 Timeout Configuration	141

12.7 2.9.7 Error Logging and Reporting	142

12.7.1 Error Status Register	142

12.7.2 Error History Buffer	142

12.7.3 Error Interrupts	143

12.8 2.9.8 Resource Utilization	143

12.8.1 Error Handling Resources	143

12.9 2.9.9 Configuration Parameters	143

12.9.1 Error Handling Configuration (TOML)	143

12.10 2.9.10 Debug and Observability	144

12.10.1 Recommended Debug Signals	144

12.11 2.9.11 Common Issues and Debug	144

12.12 2.9.12 Verification Considerations	145

12.12.1 Test Scenarios	145

12.13 2.9.13 Recovery Mechanisms	145

12.13.1 Automatic Recovery	145

12.13.2 Manual Recovery	146

12.14 2.9.14 Safety and Reliability	146

12.14.1 Error Containment	146

12.14.2 Fail-Safe Behavior	146

12.14.3 Error Injection (Debug/Test)	146

12.15 2.9.15 Future Enhancements	147

12.15.1 Planned Features	147

12.15.2 Under Consideration	147

12.15.3 Bridge Arbiter Finite State Machines	147

12.15.4 Figure 5.3: AW Arbiter FSM	148

12.15.5 Figure 5.4: AR Arbiter FSM	149

13 Transaction Tracking FSMs	155

13.1 Overview	155

13.2 Write Data Tracking FSM	155

13.2.1 Purpose	155

13.2.2 States	156

13.2.3 Figure 3.1: Write Data Tracking FSM	156

13.2.4 Implementation	156

13.3 Response Pending Counter	157

13.3.1 Purpose	157

13.3.2 Implementation	157

13.4 Burst Counter FSM	158

13.4.1 Purpose	158

13.4.2 States	158

13.4.3 Implementation	158

13.5 Related Documentation	158

14 CAM Architecture	159

14.1 Overview	159

14.2 CAM Purpose	159

14.2.1 Transaction Tracking	159

14.2.2 Response Routing	159

14.3 CAM Structure	159

14.3.1 Figure 4.1: CAM Entry Format	159

14.3.2 CAM Sizing	159

14.3.3 Example Configuration	159

14.4 CAM Operations	160

14.4.1 Insertion (AR/AW Acceptance)	160

14.4.2 Lookup (R/B Response)	160

14.4.3 Deletion (Response Complete)	160

14.5 Implementation Options	160

14.5.1 Distributed RAM CAM	160

14.5.2 Block RAM CAM	161

14.6 CAM Overflow Handling	161

14.6.1 Prevention	161

14.6.2 Error Response	161

14.7 Related Documentation	161

15 ID Tracking Tables	162

15.1 Overview	162

15.2 Table Structure	162

15.2.1 Per-Slave ID Table	162

15.2.2 Figure 4.2: ID Table Structure	163

15.2.3 Table Sizing	165

15.3 ID Extension	165

15.3.1 Master-Side Extension	165

15.3.2 Slave-Side Presentation	165

15.4 ID Extraction	165

15.4.1 Response Parsing	165

15.5 Table Operations	166

15.5.1 Allocation (AR/AW Phase)	166

15.5.2 Lookup (R/B Phase)	166

15.5.3 Deallocation (Response Complete)	166

15.6 Multi-ID Considerations	167

15.6.1 Same External ID, Different Masters	167

15.6.2 Same Extended ID (Error)	167

15.7 Resource Utilization	167

15.7.1 Table Resources	167

15.8 Related Documentation	167

16 Width Converters	168

16.1 Overview	168

16.2 Upsize Converter	168

16.2.1 Purpose	168

16.2.2 Figure 5.1: Upsize Converter (64-bit to 512-bit)	168

16.2.3 Implementation	168

16.2.4 Burst Length Conversion	169

16.3 Downsize Converter	169

16.3.1 Purpose	169

16.3.2 Figure 5.2: Downsize Converter (512-bit to 64-bit)	170

16.3.3 Implementation	170

16.4 Strobe Handling	171

16.4.1 Upsize Strobe Packing	171

16.4.2 Downsize Strobe Splitting	171

16.5 Resource Utilization	172

16.5.1 Upsize Converter (64 to 512)	172

16.5.2 Downsize Converter (512 to 64)	172

16.6 Related Documentation	172

17 APB Converters	172

17.1 Overview	172

17.2 Conversion Requirements	172

17.2.1 Protocol Differences	172

17.2.2 Conversion Strategy	173

17.3 Write Conversion	173

17.3.1 Figure 5.3: AXI4 Write to APB Write	173

17.3.2 State Machine	174

17.4 Read Conversion	175

17.4.1 Figure 5.4: AXI4 Read to APB Read	175

17.4.2 Read State Machine	175

17.5 Burst Handling	176

17.5.1 Burst to Single Conversion	176

17.5.2 Address Calculation	176

17.6 Error Handling	176

17.6.1 APB Error to AXI4 Response	176

17.6.2 Error Propagation	176

17.7 Performance Considerations	177

17.7.1 Throughput Impact	177

17.7.2 Latency Impact	177

17.8 Resource Utilization	177

17.8.1 APB Converter Resources	177

17.9 Related Documentation	177

18 Module Structure	178

18.1 Generated RTL Overview	178

18.2 Top-Level Module	178

18.2.1 Module Declaration	178

18.2.2 Port Organization	178

18.3 Internal Structure	179

18.3.1 Component Instantiation	179

18.4 Signal Naming Convention	179

18.4.1 Master-Side Signals (External)	179

18.4.2 Slave-Side Signals (External)	180

18.4.3 Internal Signals	180

18.5 Generated File Structure	180

18.5.1 Single-File Output	180

18.5.2 Multi-File Output (Optional)	181

18.6 Parameterization	181

18.6.1 Compile-Time Parameters	181

18.6.2 Per-Port Parameters	181

18.7 Related Documentation	181

19 Signal Naming	182

19.1 Naming Convention Overview	182

19.2 External Port Naming	182

19.2.1 Pattern	182

19.2.2 Components	182

19.2.3 Master Port Examples	182

19.2.4 Slave Port Examples	183

19.3 APB Port Naming	183

19.3.1 Pattern	183

19.3.2 APB Signal Examples	183

19.4 Internal Signal Naming	183

19.4.1 Crossbar Signals	183

19.4.2 Arbitration Signals	183

19.4.3 ID Signals	184

19.5 Debug Signal Naming	184

19.5.1 Pattern	184

19.5.2 Debug Signal Examples	184

19.6 Verification Pattern Matching	184

19.6.1 Factory Pattern Support	184

19.6.2 Pattern Rules	185

19.7 Related Documentation	185

20 Test Strategy	185

20.1 Overview	185

20.2 Test Categories	185

20.2.1 Unit Tests	185

20.2.2 Integration Tests	186

20.3 Test Parameterization	186

20.3.1 Configuration Matrix	186

20.3.2 Protocol Combinations	186

20.4 Factory-Based Testing	187

20.4.1 BFM Instantiation	187

20.4.2 Transaction Generation	187

20.5 Coverage Goals	188

20.5.1 Functional Coverage	188

20.5.2 Code Coverage	188

20.6 Test Execution	188

20.6.1 Running Tests	188

20.6.2 Test Levels	189

20.7 Related Documentation	189

21 Debug Guide	189

21.1 Overview	189

21.2 Debug Signal Access	189

21.2.1 Enabling Debug Outputs	189

21.2.2 Debug Signal Categories	190

21.3 Common Issues	190

21.3.1 Issue: Transaction Timeout	190

21.3.2 Issue: Wrong Response Routing	190

21.3.3 Issue: Data Corruption	191

21.4 Waveform Analysis	191

21.4.1 Generating Waveforms	191

21.4.2 Key Signals to Observe	191

21.4.3 Timing Analysis	192

21.5 Assertion Failures	192

21.5.1 Built-in Assertions	192

21.5.2 Handling Assertion Failures	192

21.6 Performance Debugging	193

21.6.1 Throughput Issues	193

21.6.2 Latency Issues	193

21.7 Related Documentation	193

22 Bridge Hardware Architecture Specification Index	193

22.1 Document Organization	193

22.1.1 Front Matter	194

22.1.2 Chapter 1: Introduction	194

22.1.3 Chapter 2: System Overview	194

22.1.4 Chapter 3: Architecture	194

22.1.5 Chapter 4: Interfaces	194

22.1.6 Chapter 5: Performance	194

22.1.7 Chapter 6: Integration	194

22.2 Related Documentation	194

23 Bridge: AXI4 Full Crossbar Generator - Product Requirements Document	195

23.1 Executive Summary	195

23.2 Design Philosophy	196

23.2.1 Core Principles	196

23.2.2 Architecture Philosophy	197

23.3 ⚠ CRITICAL: RTL Regeneration Requirements	197

23.4 1. Product Overview	198

23.4.1 1.1 Purpose	198

23.4.2 1.2 Target Audience	198

23.4.3 1.3 Success Criteria	199

23.4.4 1.4 Implementation Status and Phases	199

23.5 2. Architecture Overview	200

23.5.1 2.1 AXI4 vs AXIS vs APB	200

23.5.2 2.2 Block Diagram	201

23.5.3 2.3 Key Components	202

23.6 3. Functional Requirements	203

23.6.1 3.1 AXI4 Protocol Compliance	203

23.6.2 3.2 Address Decoding	203

23.6.3 3.3 Arbitration Strategy	203

23.6.4 3.4 Burst Handling	203

23.7 4. Non-Functional Requirements	204

23.7.1 4.1 Performance	204

23.7.2 4.2 Resource Usage (Estimated)	204

23.7.3 4.3 Quality Requirements	204

23.8 5. Interface Specifications	205

23.8.1 5.1 AXI4 Master Interfaces (M × 5 channels)	205

23.8.2 5.2 AXI4 Slave Interfaces (S × 5 channels)	206

23.8.3 5.3 Configuration Parameters	206

23.9 6. Performance Modeling	206

23.9.1 6.1 Analytical Model	206

23.9.2 6.2 Resource Scaling	207

23.9.3 6.3 Comparison with Other Crossbars	207

23.10 7. Generator Architecture	208

23.10.1 7.1 Python Generator Structure	208

23.10.2 7.2 Address Map Configuration	209

23.11 8. Comparison with APB and Delta	210

23.11.1 8.1 Code Reuse from APB Generator	210

23.11.2 8.2 Code Reuse from Delta Generator	210

23.11.3 8.3 Complexity Comparison	210

23.12 9. Use Cases	211

23.12.1 9.1 Multi-Core Processor Interconnect	211

23.12.2 9.2 DMA + Accelerator System	211

23.12.3 9.3 FPGA System Integration	212

23.13 10. Testing Strategy	212

23.13.1 10.1 FUB (Functional Unit Block) Tests	212

23.13.2 10.2 Integration Tests	212

23.13.3 10.3 Performance Validation	213

23.14 11. Documentation Plan	213

23.14.1 11.1 Specifications (Before Code)	213

23.14.2 11.2 Performance Analysis (Before Implementation)	213

23.14.3 11.3 Code Generation	213

23.14.4 11.4 Verification	214

23.15 12. Success Metrics	214

23.15.1 12.1 Functional Completeness	214

23.15.2 12.2 Performance Targets	214

23.15.3 12.3 Educational Value	214

23.15.4 12.4 Reusability	214

23.16 12. Attribution and Contribution Guidelines	215

23.16.1 12.1 Git Commit Attribution	215

23.17 12.2 PDF Generation Location	215

23.18 13. Future Enhancements	215

23.18.1 13.1 Short-Term (Post-Initial Release)	215

23.18.2 13.2 Long-Term	216

23.19 14. Risk Assessment	216

23.20 15. Project Timeline (Estimated)	216

23.21 16. References	217

23.22 17. Glossary	217

24 Claude Code Guide: Bridge Subsystem	217

24.1 🚨 CRITICAL: Read Architecture Document First	218

24.2 Quick Context	218

24.3 Target Architecture: Intelligent Width-Aware Routing	218

24.3.1 Efficient Multi-Width Design	218

24.3.2 Why This Architecture	219

24.3.3 Per-Master Output Paths	219

24.3.4 Benefits	219

24.3.5 Implementation Status	220

24.4 🚨 CRITICAL RULE #0: RTL Regeneration Requirements	220

24.4.1 The Golden Rule	220

24.4.2 Why This Is Non-Negotiable	220

24.4.3 Real Example (This Session)	220

24.4.4 Generator Files That Trigger Full Regeneration	221

24.4.5 The Regeneration Workflow	221

24.4.6 Symptoms of Version Mismatch	221

24.4.7 Think Like a Compiler Developer	222

24.4.8 Exception: Hand-Written RTL	222

24.5 Critical Pitfalls: slave_select_* Reversion After Handshake	222

24.5.1 The Problem: Combinational Address Decode	222

24.5.2 Visible Symptoms	222

24.5.3 The Solution (MANDATORY in All Generators)	223

24.5.4 When This Bug Appears	223

24.6 ⚠ MANDATORY: Project Organization Pattern	224

24.6.1 Required Directory Structure	224

24.6.2 Testbench Class Location (MANDATORY)	224

24.6.3 Test File Pattern (MANDATORY)	225

24.7 Critical Rules for This Subsystem	227

24.7.1 Rule #0: Attribution Format for Git Commits	227

24.7.2 Rule #0.1: Testbench Architecture - MANDATORY SEPARATION	227

24.7.3 Rule #1: All Testbenches Inherit from TBBase	228

24.7.4 Rule #2: Mandatory Testbench Methods	228

24.7.5 Rule #3: Use GAXI Components for Protocol Handling	229

24.7.6 Rule #4: Queue-Based Verification	230

24.8 TOML/CSV-Based Bridge Generator (Phase 2 Complete)	230

24.8.1 Overview	230

24.8.2 Quick Start	230

24.8.3 Configuration Format Details	231

24.8.4 Channel-Specific Masters (Phase 2 Feature)	232

24.8.5 Example: RAPIDS-Style Configuration	233

24.8.6 Common User Questions	234

24.8.7 Generator Output Structure	235

24.8.8 Testing Generated Bridges	236

24.9 Bridge Architecture Quick Reference	236

24.9.1 Generated Bridge Crossbar Structure	236

24.9.2 Key Features	236

24.9.3 Address Map	237

24.10 Test Organization	237

24.10.1 Test Hierarchy	237

24.10.2 Test Levels	237

24.11 Common User Questions and Responses	237

24.11.1 Q: “How does the Bridge work?”	237

24.11.2 Q: “How do I generate a Bridge?”	238

24.11.3 Q: “How do I test a Bridge?”	238

24.12 Anti-Patterns to Avoid	239

24.12.1 ✗ Anti-Pattern 1: Embedded Testbench Classes	239

24.12.2 ✗ Anti-Pattern 2: Manual AXI4 Handshaking	239

24.12.3 ✗ Anti-Pattern 3: Memory Models for Simple Tests	239

24.13 Quick Reference	240

24.13.1 Finding Existing Components	240

24.13.2 Common Commands	240

24.14 Remember	240

24.15 PDF Generation Location	241

List of Figures

Figure 2.1: Master Adapter Architecture	41

Figure 2.2: Slave Router Architecture	49

Figure 2.3: Crossbar Core Architecture	59

Figure 2.5: ID Management Architecture	83

Figure 2.6: Width Conversion Architecture	96

Figure 2.7.1: Protocol Conversion Architecture	106

Figure 2.7: Protocol Conversion Architecture	106

Figure 2.8: Response Routing Architecture	124

Figure 2.9: Error Handling Architecture	137

Figure 5.3: AW Arbiter FSM	148

Figure 5.4: AR Arbiter FSM	149

Figure 3.1: Write Data Tracking FSM	156

Figure 4.1: CAM Entry Format	159

Figure 4.2: ID Table Structure	163

Figure 5.1: Upsize Converter (64-bit to 512-bit)	168

Figure 5.2: Downsize Converter (512-bit to 64-bit)	170

Figure 5.3: AXI4 Write to APB Write	173

Figure 5.4: AXI4 Read to APB Read	175

List of Tables

Table 1: Table 1.1: Supported Protocols	38

Table 2: Table 1.2: Channel-Specific Master Types	39

Table 3: Table 1.3: Document Organization	40

Table 4: Table 5.3: Burst Length Conversion Examples	169

Table 5: Table 5.1: AXI4 vs APB Protocol Comparison	172

Table 6: Table 5.2: APB to AXI4 Error Mapping	176

Table 7: Table 7.1: Test Parameter Matrix	186

Table 8: Table 7.2: Test Level Definitions	189

Table 9: Table 7.3: Debug Signal Categories	190

List of Waveforms

No waveforms in this document.

# 1	Bridge Mas Index

Generated: 2026-05-14

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	Bridge Micro-Architecture Specification

| Property | Value |
| --- | --- |
| Document Title | Bridge Micro-Architecture Specification |
| Version | 1.0 |
| Date | January 3, 2026 |
| Status | Released |
| Classification | Open Source - Apache 2.0 License |

## 2.2	Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-01-03 | RTL Design Sherpa | Initial release - restructured from single spec |

## 2.3	Document Purpose

This Micro-Architecture Specification (MAS) provides detailed implementation information for the Bridge component, covering:

- Block-level architecture and internal structure

- FSM state diagrams and transition tables

- ID management and CAM architecture

- Signal-level interface details

- Width and protocol converter implementation

- Timing diagrams and debug information

For high-level architecture, system integration, and performance overview, refer to the companion Bridge Hardware Architecture Specification (HAS).

## 2.4	Intended Audience

- RTL designers implementing or modifying Bridge

- Verification engineers developing testbenches

- System integrators debugging Bridge behavior

- Technical staff requiring implementation details

## 2.5	Related Documents

- Bridge HAS - Hardware Architecture Specification (high-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Overview

## 3.1	Bridge Micro-Architecture

This document describes the internal micro-architecture of Bridge, a multi-protocol AXI4 crossbar generator. Bridge creates parameterized SystemVerilog RTL from CSV/TOML configuration files.

## 3.2	Key Capabilities

### 3.2.1	Multi-Protocol Support

Bridge supports three AMBA protocols:

Table 1.1: Supported Protocols

| Protocol | Use Case | Conversion |
| --- | --- | --- |
| AXI4 Full | High-bandwidth memory | Direct or width convert |
| AXI4-Lite | Register access | Protocol downgrade |
| APB | Low-speed peripherals | Full conversion |

### 3.2.2	Channel-Specific Masters

Bridge generates optimized port interfaces:

Table 1.2: Channel-Specific Master Types

| Type | Channels | Signals | Use Case |
| --- | --- | --- | --- |
| Full (rw) | AW, W, B, AR, R | 100% | CPU, config master |
| Write-only (wr) | AW, W, B | ~60% | DMA write engine |
| Read-only (rd) | AR, R | ~40% | DMA read engine |

### 3.2.3	Automatic Converters

Bridge inserts converters automatically:

- Width converters - Handle data width mismatches

- Protocol converters - AXI4 to APB conversion

- Per-path optimization - Only where needed

## 3.3	Architecture Summary

### 3.3.1	Block Organization

Bridge NxM
├── Master Adapters (M instances)
│   ├── Skid buffers
│   └── ID extension
├── Crossbar Core
│   ├── Address decode
│   ├── Per-slave arbiters
│   └── Channel muxes
├── Slave Routers (N instances)
│   ├── Protocol converters
│   └── Width converters
└── Response Routing
    ├── ID extraction
    └── Master demux

### 3.3.2	Signal Flow

Master → Adapter → Decode → Arbiter → Mux → Converter → Slave
                                              ↓
Master ← Demux ← ID Extract ← Response ← Converter ← Slave

## 3.4	Document Organization

Table 1.3: Document Organization

| Chapter | Content |
| --- | --- |
| Ch 2 | Block Descriptions |
| Ch 3 | FSM Design |
| Ch 4 | ID Management |
| Ch 5 | Converters |
| Ch 6 | Generated RTL |
| Ch 7 | Verification |

## 3.5	Related Documentation

- Bridge HAS - High-level architecture and integration

- PRD.md - Product requirements

- Generator source - bin/bridge_csv_generator.py

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	2.1 Master Adapter

The Master Adapter is a per-master module that provides timing isolation, protocol normalization, and request preparation for the crossbar interconnect. Each master port in the bridge has its own dedicated adapter instance.

## 4.1	2.1.1 Purpose and Function

The Master Adapter serves several critical functions:

- Timing Isolation: Inserts pipeline registers (skid buffers) to break combinatorial paths from master to crossbar

- Channel Specialization: Separates read-only, write-only, and read-write masters for optimal resource utilization

- Bridge ID Injection: Adds internal tracking IDs to transactions for response routing

- Protocol Normalization: Ensures all transactions meet crossbar assumptions and constraints

- Backpressure Management: Handles ready/valid handshaking with single-cycle latency

## 4.2	2.1.2 Block Diagram

### 4.2.1	Figure 2.1: Master Adapter Architecture

Master Adapter Architecture

The figure shows Master Adapter architecture with per-master skid buffers, bridge ID injection/stripping, and channel specialization for rd/wr/rw configurations.

## 4.3	2.1.3 Channel Specialization

The Bridge generator creates three types of master adapters based on the channel usage specified in the configuration:

### 4.3.1	Read-Only Masters

Channels: AR (Address Read), R (Read Data)

Configuration: "rd" or "read"
RTL Generated: adapter_master_rd_<id>.sv

Optimizations:
- No write address channel logic
- No write data channel logic
- No write response channel logic
- Reduced arbitration participation (read path only)

### 4.3.2	Write-Only Masters

Channels: AW (Address Write), W (Write Data), B (Write Response)

Configuration: "wr" or "write"
RTL Generated: adapter_master_wr_<id>.sv

Optimizations:
- No read address channel logic
- No read data channel logic
- Reduced arbitration participation (write path only)

### 4.3.3	Read-Write Masters

Channels: AR, R, AW, W, B (Full AXI4 interface)

Configuration: "rw" or "readwrite"
RTL Generated: adapter_master_rw_<id>.sv

Full Functionality:
- All five AXI4 channels implemented
- Participates in both read and write arbitration
- Maximum flexibility but larger resource footprint

## 4.4	2.1.4 Skid Buffer Architecture

Each AXI4 channel passes through a skid buffer for timing isolation. The skid buffer implements:

### 4.4.1	Registered Forward Path

// Simplified skid buffer concept
always_ff @(posedge clk) begin
    if (!rst_n) begin
        valid_reg <= 1'b0;
    end else if (!valid_reg || ready_out) begin
        valid_reg <= valid_in;
        data_reg  <= data_in;
    end
end

### 4.4.2	Single-Cycle Backpressure Response

- When downstream is not ready, accepts one additional beat into holding register

- Signals ready_in = 0 in same cycle to upstream

- No combinatorial paths between upstream and downstream

### 4.4.3	Pipeline Depth

- Default: 1 stage (skid buffer)

- Configurable: Up to 8 stages for high-frequency designs

- Trade-off: Latency vs. timing closure

Performance Impact: - Adds 1 cycle latency per channel - Enables higher clock frequencies - Prevents critical paths through crossbar

## 4.5	2.1.5 Bridge ID Management

### 4.5.1	ID Width Calculation

BID_WIDTH = clog2(num_masters)

Examples:
- 2 masters  → BID_WIDTH = 1
- 4 masters  → BID_WIDTH = 2
- 8 masters  → BID_WIDTH = 3
- 16 masters → BID_WIDTH = 4

### 4.5.2	ID Injection (Request Path)

For each master adapter, a unique constant Bridge ID is appended to the AXI transaction ID:

AR Channel:
  Internal ARID = {MASTER_BID[BID_WIDTH-1:0], External ARID[ARID_WIDTH-1:0]}
  Internal ARID Width = BID_WIDTH + ARID_WIDTH

AW Channel:
  Internal AWID = {MASTER_BID[BID_WIDTH-1:0], External AWID[AWID_WIDTH-1:0]}
  Internal AWID Width = BID_WIDTH + AWID_WIDTH

Example: 4-master system, external ARID_WIDTH = 4

Master 0: BID = 2'b00, External ID = 4'h3 → Internal ID = 6'b00_0011
Master 1: BID = 2'b01, External ID = 4'h3 → Internal ID = 6'b01_0011
Master 2: BID = 2'b10, External ID = 4'h5 → Internal ID = 6'b10_0101
Master 3: BID = 2'b11, External ID = 4'hA → Internal ID = 6'b11_1010

### 4.5.3	ID Stripping (Response Path)

The Bridge ID is removed from responses before returning to the master:

R Channel:
  External RID = Internal RID[ARID_WIDTH-1:0]
  Bridge routes based on Internal RID[BID_WIDTH+ARID_WIDTH-1:ARID_WIDTH]

B Channel:
  External BID = Internal BID[AWID_WIDTH-1:0]
  Bridge routes based on Internal BID[BID_WIDTH+AWID_WIDTH-1:AWID_WIDTH]

This ensures: - Master sees original transaction IDs - Bridge internally tracks which master originated each transaction - Responses route back to correct master even with ID reuse across masters

## 4.6	2.1.6 Protocol Normalization

The Master Adapter enforces several protocol requirements:

### 4.6.1	Valid Address Ranges

- Checks addresses against slave address maps

- Flags out-of-range accesses for error handling

- Prevents deadlocks from illegal addresses

### 4.6.2	Burst Constraints

- Validates ARLEN/AWLEN vs. 4KB boundary rules

- Ensures burst types are supported (FIXED, INCR, WRAP)

- Checks SIZE vs. DATA_WIDTH compatibility

### 4.6.3	Signal Defaults

- Provides default values for unused signals

- Ensures ARCACHE/AWCACHE have legal values

- Sets ARPROT/AWPROT based on configuration

## 4.7	2.1.6 Response Path Slave Selection Tracking

### 4.7.1	Problem: Stale Slave Select Decode

The master adapter decodes incoming address to determine target slave immediately (combinational). When the wrapper’s skid buffer pops and fub_axi_awaddr/fub_axi_araddr reverts, the decode changes even though the converter is still in flight pushing the request to the crossbar. This causes the response MUX to route read/write responses from the wrong slave.

### 4.7.2	Solution: Per-Channel Response-Tracking FIFOs

Added separate FIFOs for AR and AW channels that capture the slave index (comb_slave_select_ar/aw) at the moment of the FUB-level handshake:

// At request time (when fub_axi_arvalid && fub_axi_arready)
ar_trk_push <= comb_slave_select_ar;  // Capture decoded slave index

// At response time (when m_axi_rvalid && m_axi_rready && m_axi_rlast)
r_slave_select <= ar_trk_pop;  // Stable slave index for response MUX

Benefits: - R/B responses route correctly even after address reverts - Supports multi-slave masters spanning different widths - Stable path for both data width and target slave

Implementation: - One FIFO per (master, AW/AR channel) - Width: clog2(NUM_SLAVES) bits (4 bits typical for 16-slave systems) - Depth: Matches maximum outstanding transactions (8-16 typical)

## 4.8	2.1.7 Interface Specifications

### 4.8.1	External Master Interface (per master)

Input Channels (from master):
- ARVALID, ARREADY, ARADDR, ARBURST, ARCACHE, ARID, ARLEN, 
  ARLOCK, ARPROT, ARQOS, ARSIZE

- AWVALID, AWREADY, AWADDR, AWBURST, AWCACHE, AWID, AWLEN,
  AWLOCK, AWPROT, AWQOS, AWSIZE

- WVALID, WREADY, WDATA, WSTRB, WLAST

Output Channels (to master):
- RVALID, RREADY, RDATA, RID, RLAST, RRESP

- BVALID, BREADY, BID, BRESP

### 4.8.2	Internal Crossbar Interface (per master)

Output Channels (to crossbar):
- AR* signals + Internal ARID (wider)
- AW* signals + Internal AWID (wider)  
- W* signals (unchanged)

Input Channels (from crossbar):
- R* signals + Internal RID (wider)
- B* signals + Internal BID (wider)

## 4.9	2.1.8 Resource Utilization

### 4.9.1	Per-Master Adapter Resources (Typical)

Read-Write Master (64-bit data, 32-bit addr, 4-bit ID):

Logic Elements:  ~200-400 (depending on ID width and features)
Registers:       ~150-250 (pipeline stages + control)
Block RAM:       0 (CAM is in crossbar core)

Breakdown:
- Skid buffers (5 channels × ~30 regs each): ~150 regs
- ID manipulation logic: ~50 LEs
- Control FSMs: ~100 LEs
- Routing logic: ~50 LEs

Read-Only Master: ~60% of read-write resources
Write-Only Master: ~65% of read-write resources

### 4.9.2	Scaling Considerations

Resource usage scales with: - Number of masters (linear scaling, one adapter per master) - ID width (logarithmic: +BID_WIDTH bits per transaction) - Pipeline depth (linear: deeper pipelines = more registers) - Data width (linear: wider data = wider skid buffers)

## 4.10	2.1.9 Timing Characteristics

### 4.10.1	Latency

Request Path (Master → Crossbar): - Minimum: 1 cycle (skid buffer) - With 4-stage pipeline: 4 cycles - With ID injection: +0 cycles (combinatorial within stage)

Response Path (Crossbar → Master): - Minimum: 1 cycle (skid buffer) - With 4-stage pipeline: 4 cycles - With ID stripping: +0 cycles (combinatorial within stage)

Total Round-Trip Overhead: 2-8 cycles (depending on pipeline depth)

### 4.10.2	Throughput

- Maximum: 1 transaction per cycle per channel (fully pipelined)

- Backpressure: Propagates with 1-cycle latency

- Burst Performance: No degradation; bursts flow continuously when ready

### 4.10.3	Critical Paths

Typical critical paths (if skid buffers not used): - Master ARVALID → Crossbar arbitration logic - Crossbar grant → Master ARREADY - Slave RDATA → Master RDATA

Solution: Skid buffers break all these paths at the cost of 1-cycle latency.

## 4.11	2.1.10 Configuration Parameters

### 4.11.1	Per-Master Parameters (from TOML/CSV)

[[masters]]
name = "cpu"
channels = "rw"              # "rd", "wr", or "rw"
arid_width = 4               # External ID width
awid_width = 4               # Can differ from ARID
addr_width = 32              # Address bus width
data_width = 64              # Data bus width
pipeline_depth = 1           # Skid buffer stages (1-8)

### 4.11.2	Global Parameters (affect all adapters)

[bridge]
internal_data_width = 64     # Crossbar data width
enable_width_conversion = true
bid_width = 2                # Calculated: clog2(num_masters)

## 4.12	2.1.11 Debug and Observability

### 4.12.1	Recommended Debug Signals

For ILA or waveform capture:

- Master adapter input valid/ready (all channels)
- Master adapter output valid/ready (all channels)
- Bridge ID assignments per transaction
- Pipeline stage occupancy
- Backpressure events (ready = 0 while valid = 1)

### 4.12.2	Common Issues and Debug

Symptom: Master hangs waiting for READY
Check: - Is adapter receiving grants from arbiters? - Is slave responding to requests? - Is response path clear?

Symptom: Incorrect data returned to master
Check: - Bridge ID extraction logic - ID width mismatches - Response routing in crossbar

Symptom: Timing violations
Check: - Increase pipeline_depth parameter - Verify clock frequency vs. design complexity - Check for combinatorial loops

## 4.13	2.1.12 Future Enhancements

### 4.13.1	Planned Features

- Dynamic Pipeline Depth: Adjust depth based on operating frequency

- FIFO Mode: Deeper buffering (16-256 entries) for burst-intensive masters

- QoS Support: Priority-based arbitration hints

- Performance Counters: Transaction counts, stall cycles, utilization metrics

### 4.13.2	Under Consideration

- Clock Domain Crossing: Per-master clock domains with async FIFOs

- Width Conversion at Adapter: Move width logic to adapters for distributed conversion

- Outstanding Transaction Tracking: Windowing for improved OOO performance

Related Sections: - Section 2.3: Crossbar Core (interconnect architecture) - Section 2.4: Arbitration (how adapters compete for slaves) - Section 2.5: ID Management (CAM structures for response routing) - Section 3.2: Master Port Interface (signal-level specifications)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	2.2 Slave Router

The Slave Router is responsible for address decoding and routing master requests to the appropriate slave. Each master has its own dedicated slave router instance that examines addresses and directs transactions to one of the configured slaves or generates error responses for out-of-range addresses.

## 5.1	2.2.1 Purpose and Function

The Slave Router performs the following critical functions:

- Address Decoding: Matches request addresses against configured slave address ranges

- Request Routing: Directs AR/AW/W channels to the selected slave

- Out-of-Range Detection: Identifies addresses that don’t map to any slave

- Error Response Generation: Creates DECERR responses for invalid addresses

- Default Slave Support: Routes unmapped addresses to optional default slave

## 5.2	2.2.2 Block Diagram

### 5.2.1	Figure 2.2: Slave Router Architecture

Slave Router Architecture

Slave router architecture showing address decoding, routing matrix, and out-of-range detection for AW and AR channels.

## 5.3	2.2.3 Address Decoding Algorithm

### 5.3.1	Configuration-Based Address Maps

Each slave is configured with:

[[slaves]]
name = "memory"
base_address = 0x4000_0000
size = 0x1000_0000        # 256 MB
default = false           # Not a default slave

The router generates address ranges:

Start Address = base_address
End Address   = base_address + size - 1

### 5.3.2	Decoding Priority

When multiple slaves have overlapping address ranges, the router uses first-match priority:

Priority Order = Order in configuration file (top to bottom)

Example:
  Slave 0: 0x0000_0000 - 0x0FFF_FFFF  (checked first)
  Slave 1: 0x1000_0000 - 0x1FFF_FFFF  (checked second)
  Slave 2: 0x2000_0000 - 0x2FFF_FFFF  (checked third)
  
Address 0x1000_5000 → Matches Slave 1

### 5.3.3	Range Checking Logic

For each address, the router performs:

// Simplified address decode logic
logic [NUM_SLAVES-1:0] slave_match;

for (int i = 0; i < NUM_SLAVES; i++) begin
    slave_match[i] = (addr >= SLAVE_BASE[i]) && 
                     (addr <= SLAVE_END[i]);
end

// Priority encode: Select first matching slave
logic [$clog2(NUM_SLAVES)-1:0] slave_select;
always_comb begin
    slave_select = 0;
    for (int i = 0; i < NUM_SLAVES; i++) begin
        if (slave_match[i]) begin
            slave_select = i;
            break;  // First match wins
        end
    end
end

// Out-of-range detection
logic oor = ~(|slave_match);  // No slaves matched

### 5.3.4	Power-of-Two Optimization

For slaves with power-of-two sizes starting at aligned addresses, simplified decode:

// Optimized decode for: base=0x8000_0000, size=0x1000_0000 (256MB)
// Mask off lower bits: Only check upper bits
logic match = (addr[31:28] == 4'h8);  // Much simpler than range check

The generator automatically detects and applies this optimization.

## 5.4	2.2.4 Request Routing

### 5.4.1	AR Channel Routing

Read address routing flow: 1. Decode: Determine target slave from ARADDR 2. Check OOR: If no slave matches, flag for error response 3. Route: Send AR transaction to selected slave’s arbiter 4. Track: Remember routing decision for R channel responses

ARADDR → Decoder → Slave Selection → AR to Slave Arbiter
                    ↓
                 If OOR → Error Response Generator

### 5.4.2	AW/W Channel Routing

Write transactions require coordinated routing:

- AW Phase:

  - Decode AWADDR to determine target slave

  - Route AW transaction to selected slave’s arbiter

  - Store routing decision for subsequent W beats

- W Phase:

  - Follow AW routing (W channel has no address)

  - Route all W beats to same slave as AW

  - Continue until WLAST = 1

AWADDR → Decoder → Slave Selection → AW to Slave Arbiter
                                      ↓
                                   Store Routing
                                      ↓
W[0..N] ────────────────────────────→ W to Same Slave
(until WLAST)

### 5.4.3	Write Data Tracking FSM

State Machine for W Channel Routing:

IDLE:
  - Wait for AWVALID && AWREADY
  - On handshake: Store target slave, goto WRITING
  
WRITING:
  - Route W beats to stored slave
  - On WVALID && WREADY && WLAST: goto IDLE
  
Error Handling:
  - If AW was OOR: Discard W beats, generate BRESP error

## 5.5	2.2.5 Out-of-Range Handling

### 5.5.1	Detection

An address is out-of-range (OOR) if:

∀ slaves[i]: (address < base[i]) OR (address > end[i])

Unless a default slave is configured.

### 5.5.2	Error Response Generation

When OOR is detected:

Read Transactions (AR → R):

1. Accept ARVALID (assert ARREADY) 
2. Do NOT forward to any slave
3. Generate R response internally:
   - RID = Original ARID (with BID preserved)
   - RDATA = 0xBADDCAFE_DEADBEEF (debug pattern) or all zeros
   - RRESP = 2'b11 (DECERR)
   - RLAST = 1 (for each beat if burst)
4. Latency: Typically 2-3 cycles after AR acceptance

Write Transactions (AW/W → B):

1. Accept AWVALID (assert AWREADY)
2. Accept all W beats until WLAST (sink the data)
3. Do NOT forward to any slave
4. Generate B response internally:
   - BID = Original AWID (with BID preserved)
   - BRESP = 2'b11 (DECERR)
5. Latency: Typically 2-3 cycles after WLAST

### 5.5.3	Debug Data Patterns

For OOR read responses, configurable data patterns aid debugging:

Option 1: All zeros (default)
  RDATA = 64'h0000_0000_0000_0000

Option 2: Debug signature
  RDATA = 64'hBADD_CAFE_DEAD_BEEF

Option 3: Address echo (LSBs)
  RDATA = {32'hBADD_ADDR, ARADDR[31:0]}

Option 4: Master/Slave ID indicator  
  RDATA = {8'hEE, Master_ID[7:0], Slave_ID[7:0], ARADDR[47:0]}

## 5.6	2.2.6 Default Slave Support

### 5.6.1	Configuration

[[slaves]]
name = "error_responder"
base_address = 0x0        # Ignored for default slave
size = 0x0                # Ignored for default slave
default = true            # Catch-all for unmapped addresses

### 5.6.2	Behavior

When a default slave is configured: - Addresses that don’t match any specific slave → Routed to default slave - Default slave typically returns DECERR but with configurable response - Useful for prototyping (accept all addresses initially) - Can implement memory-mapped debug register for address capture

Note: Only ONE default slave allowed per bridge.

## 5.7	2.2.7 Address Aliasing

### 5.7.1	Multiple Slaves, Same Address

If configuration has overlapping ranges:

[[slaves]]
name = "fast_cache"
base_address = 0x8000_0000
size = 0x1000_0000

[[slaves]]
name = "slow_memory"
base_address = 0x8000_0000  # Same base!
size = 0x4000_0000

Result: First-match priority applies. All accesses go to fast_cache. The slow_memory range 0x9000_0000-0xBFFF_FFFF is unreachable from this master.

Warning: Generator can optionally flag this as error in DRC mode.

### 5.7.2	Intentional Aliasing Use-Cases

Legitimate uses of overlapping ranges: 1. Cache hierarchy: Small fast cache shadows larger slow memory 2. Memory remapping: Different views of same physical memory 3. Peripheral mirroring: Register block appears at multiple addresses

## 5.8	2.2.8 Configuration Parameters

### 5.8.1	Per-Router Parameters

# Router behavior is defined by slave configurations

[[slaves]]
name = "ddr_memory"
base_address = 0x8000_0000
size = 0x4000_0000           # 1 GB
default = false
oor_data_pattern = 0xDEAD    # Pattern for OOR reads

### 5.8.2	Global Parameters

[bridge]
enable_default_slave = false      # Allow default slave
strict_address_decode = true      # Flag overlapping ranges as errors
oor_response_latency = 2          # Cycles for OOR error response (2-4)

## 5.9	2.2.9 Resource Utilization

### 5.9.1	Per-Router Resources (Typical)

4-slave configuration (32-bit address):

Logic Elements:  ~150-200 LEs
Registers:       ~50 regs
Block RAM:       0

Breakdown:
- Address comparators (4 slaves × ~30 LEs): ~120 LEs
- Priority encoder: ~20 LEs
- W channel FSM: ~30 regs, ~20 LEs
- OOR error generator: ~20 regs, ~10 LEs

8-slave configuration:

Logic Elements:  ~250-350 LEs (scales roughly with slave count)
Registers:       ~60 regs

### 5.9.2	Scaling Considerations

Resource usage scales with: - Number of slaves: Linear (each slave adds comparator logic) - Address width: Minimal impact (wider comparators, but same structure) - Optimizations: Power-of-two sizes reduce logic significantly

## 5.10	2.2.10 Timing Characteristics

### 5.10.1	Decode Latency

Combinatorial Decode (Default): - Address → Slave selection: 0 cycles (combinatorial) - Critical path: ARADDR → Slave arbiter request - May limit maximum frequency in large systems

Registered Decode (Optional): - Address → Slave selection: 1 cycle (registered) - Adds latency but breaks critical path - Recommended for >8 slaves or >300 MHz operation

### 5.10.2	Throughput

- Maximum: 1 transaction per cycle per master

- No blocking: Router does not stall; arbiters handle conflicts

- Pipelining: AR and AW decode in parallel (independent paths)

### 5.10.3	Critical Paths

Typical critical paths: - ARADDR → Address comparators → Priority encoder → Arbiter request - w/4 slaves: ~10-15 logic levels (FPGA-dependent) - w/8 slaves: ~12-18 logic levels

Mitigation: - Enable registered decode mode (+1 cycle latency) - Use power-of-two slave sizes (simplified compare) - Synthesizer optimization directives

## 5.11	2.2.11 Debug and Observability

### 5.11.1	Recommended Debug Signals

- Address decode outputs (which slave matched)
- OOR flags (per channel)
- Default slave hit counter
- W channel FSM state
- Routing decision storage (for W tracking)

### 5.11.2	Common Issues and Debug

Symptom: Reads return all zeros
Check: - Is address out-of-range? - Check slave base/size configuration - Verify address decode logic in waveform

Symptom: Write data goes to wrong slave
Check: - W channel tracking FSM state - Did AW routing complete before W started? - Check for AWVALID/AWREADY handshake

Symptom: Unexpected DECERR responses
Check: - Address decode configuration - Overlapping slave ranges (wrong priority) - Off-by-one in size calculations

## 5.12	2.2.12 Verification Considerations

### 5.12.1	Address Decode Tests

Critical test scenarios: 1. Boundary conditions: base_address, base_address + size - 1 2. Just out-of-range: base_address - 1, base_address + size 3. Each slave: Verify routing to correct slave 4. Overlapping ranges: Verify priority encoding 5. Default slave: Unmapped addresses route correctly

### 5.12.2	Write Tracking Tests

W channel FSM testing: 1. Simple write: Single AW, single W (WLAST=1) 2. Burst write: AW with AWLEN=7, eight W beats 3. Back-to-back writes: New AW before previous W completes 4. Interleaved masters: Multiple masters writing simultaneously (if supported)

### 5.12.3	OOR Error Tests

Test: Read from unmapped address
- Send AR to 0xFFFF_FFFF (assuming unmapped)
- Verify R response with RRESP = DECERR
- Verify RID matches ARID
- Check response latency

Test: Write to unmapped address
- Send AW to invalid address
- Send W data
- Verify B response with BRESP = DECERR
- Verify BID matches AWID

## 5.13	2.2.13 Performance Optimization

### 5.13.1	Techniques

1. Registered Decode (High Frequency)

Trade-off: +1 cycle latency for timing closure
Best for: >8 slaves, >300 MHz targets

2. Simplified Address Decode (Power-of-2)

Optimization: Mask-based compare instead of range check
Best for: Slaves with aligned, power-of-2 sizes
Savings: ~50% reduction in comparator logic

3. Parallel Decode (Low Logic)

Implementation: Separate decoders per address bit-field
Best for: Many non-overlapping slaves
Savings: Reduces logic depth for priority encoding

4. CAM-Based Decode (Many Slaves)

Trade-off: Uses block RAM for address lookup table
Best for: >16 slaves, non-contiguous ranges
Note: Requires RAM resources

## 5.14	2.2.14 Future Enhancements

### 5.14.1	Planned Features

- Dynamic Address Remapping: Runtime-configurable slave ranges

- Transaction Filtering: Block certain address ranges per-master

- Priority Hints: QoS-based prioritization (beyond first-match)

- Address Translation: Offset/mask transformations before slave routing

### 5.14.2	Under Consideration

- Multi-region Slaves: Slave spans multiple non-contiguous ranges

- Secure Address Spaces: Per-master access control lists

- Debug Address Capture: Log invalid addresses to register

Related Sections: - Section 2.1: Master Adapter (upstream from router) - Section 2.3: Crossbar Core (downstream arbitration) - Section 2.4: Arbitration (how routed requests compete) - Section 3.3: Slave Port Interface (target of routed requests)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	2.3 Crossbar Core

The Crossbar Core is the central interconnect fabric that enables any-to-any connectivity between masters and slaves. It contains the arbitration logic, transaction routing, and response path management that form the heart of the bridge.

## 6.1	2.3.1 Purpose and Function

The Crossbar Core performs the following critical functions:

- Full Connectivity: Provides complete N×M master-to-slave interconnect matrix

- Independent Arbitration: Per-slave arbiters allow parallel access to different slaves

- Response Routing: Directs slave responses back to originating masters using Bridge IDs

- Transaction Ordering: Maintains AXI ordering requirements within address dependencies

- Backpressure Management: Handles flow control across multiple concurrent transactions

## 6.2	2.3.2 Block Diagram

### 6.2.1	Figure 2.3: Crossbar Core Architecture

Crossbar Core Architecture

Crossbar core architecture showing complete M×S switching fabric with address decode, per-slave arbitration, data path multiplexing, and ID-based response routing.

## 6.3	2.3.3 Connectivity Matrix

### 6.3.1	Full N×M Crossbar

The bridge implements a non-blocking crossbar where: - N masters can each access different slaves simultaneously - Conflicts occur only when multiple masters target the same slave - Independent read and write paths increase parallelism

Example: 4 masters, 3 slaves

S0   S1   S2
    M0   ●    ●    ●
    M1   ●    ●    ●
    M2   ●    ●    ●
    M3   ●    ●    ●

● = Connection available

### 6.3.2	Request Path Multiplexing

For each slave, requests from all masters are multiplexed:

Slave 0 Request Inputs:
  - M0 → S0 (AR, AW, W channels)
  - M1 → S0 (AR, AW, W channels)
  - M2 → S0 (AR, AW, W channels)
  - M3 → S0 (AR, AW, W channels)
  
Arbiter selects one master per channel per cycle
  → Grants propagate through MUX
  → Selected master's transaction forwarded to slave

### 6.3.3	Response Path Demultiplexing

Responses from slaves are demultiplexed back to masters:

Slave 0 Response Outputs (R, B channels):
  - Extract Bridge ID from RID/BID
  - Route to corresponding master (M0, M1, M2, or M3)
  - Strip Bridge ID before delivering to master adapter
  
Uses CAM (Content Addressable Memory) for fast ID lookup

## 6.4	2.3.4 Request Path Architecture

### 6.4.1	Per-Slave Request Arbitration

Each slave has independent arbiters for: 1. AR Channel: Read address arbitration (all masters competing) 2. AW/W Channels: Write address/data arbitration (all masters competing)

This separation allows: - Simultaneous read and write to same slave (if slave supports it) - Independent grant decisions for AR vs. AW channels - Better throughput for mixed read/write workloads

### 6.4.2	Request Multiplexers with Stable Address Gating

After arbitration, multiplexers select granted master’s signals. AR/AW gating uses inline address re-decode on the stable m_axi address bus (held stable across the full handshake):

// Inline address re-decode for stable gating (replaces slave_select_* gating)
// The address on m_axi is held stable from fub_axi handshake through m_axi handshake
always_comb begin
    // Re-decode each master's address to determine target slave
    m0_targets_s0 = (m0_m_axi_araddr >= S0_BASE) && (m0_m_axi_araddr < S0_END);
    m1_targets_s0 = (m1_m_axi_araddr >= S0_BASE) && (m1_m_axi_araddr < S0_END);
    m2_targets_s0 = (m2_m_axi_araddr >= S0_BASE) && (m2_m_axi_araddr < S0_END);
    m3_targets_s0 = (m3_m_axi_araddr >= S0_BASE) && (m3_m_axi_araddr < S0_END);
end

// Gate with valid signals
assign m0_ar_s0_valid = m0_arvalid && m0_targets_s0;
assign m1_ar_s0_valid = m1_arvalid && m1_targets_s0;
assign m2_ar_s0_valid = m2_arvalid && m2_targets_s0;
assign m3_ar_s0_valid = m3_arvalid && m3_targets_s0;

// Simplified AR channel MUX for Slave 0
always_comb begin
    case (ar_grant_s0)
        2'b00: begin  // Master 0 granted
            s0_arvalid = m0_arvalid;
            s0_araddr  = m0_m_axi_araddr;  // Stable across m_axi handshake
            s0_arid    = m0_arid;    // Includes Bridge ID
            // ... other AR signals
        end
        2'b01: begin  // Master 1 granted
            s0_arvalid = m1_arvalid;
            s0_araddr  = m1_m_axi_araddr;  // Stable across m_axi handshake
            s0_arid    = m1_arid;
            // ... other AR signals
        end
        // ... cases for M2, M3
    endcase
end

Why this matters: The address on m_axi_* is held stable for the entire AXI handshake (arvalid && arready). This allows address re-decoding to determine the target slave even as the master’s internal address state changes.

### 6.4.3	AW→W Burst Tracking FIFO

For masters with multiple slave connections, W beats must follow the slave that the AW was driven to. A per-(master, slave) FIFO tracks which slave the AW was routed to:

// Track which slave the AW was driven to
always_ff @(posedge aclk or negedge aresetn) begin
    if (!aresetn) begin
        aw_trk_slave_id <= '0;
        aw_trk_valid <= 1'b0;
    end else if (m0_awvalid && m0_m_axi_awready && s_selected_aw == S0) begin
        aw_trk_slave_id <= S0;  // Track that AW went to slave 0
        aw_trk_valid <= 1'b1;
    end else if (m0_wvalid && m0_m_axi_wready && m0_m_axi_wlast) begin
        aw_trk_valid <= 1'b0;  // AW->W burst complete
    end
end

// Gate W path based on tracked slave
logic w_to_s0, w_to_s1, w_to_s2;
always_comb begin
    w_to_s0 = m0_wvalid && (aw_trk_slave_id == S0);
    w_to_s1 = m0_wvalid && (aw_trk_slave_id == S1);
    w_to_s2 = m0_wvalid && (aw_trk_slave_id == S2);
end

Why this matters: Without tracking, W beats would re-evaluate address decode which may have stale or different values, routing to the wrong width converter or slave.

### 6.4.4	Backpressure Propagation

Ready signals flow back from slave through arbiter to granted master:

Slave S0 ARREADY → Arbiter → Granted Master ARREADY
                           ↘ Other Masters ARREADY = 0

This ensures: - Only granted master sees READY from slave - Non-granted masters see READY = 0 (backpressure) - No combinatorial loops in ready path (registered arbitration)

## 6.5	2.3.5 Response Path Architecture

### 6.5.1	Bridge ID Extraction

Response routing uses the Bridge ID embedded in transaction IDs:

R Channel:
  Slave RID = {BID[BID_WIDTH-1:0], Original_ID[ID_WIDTH-1:0]}
  Extract: Master_Index = BID
  Route R response to Master[Master_Index]

B Channel:
  Slave BID = {BID[BID_WIDTH-1:0], Original_ID[ID_WIDTH-1:0]}
  Extract: Master_Index = BID
  Route B response to Master[Master_Index]

### 6.5.2	CAM-Based Routing (Optional)

For large master counts or OOO responses, a CAM tracks outstanding transactions:

CAM Entry Structure:
  - Internal ID (with BID)
  - Master index
  - Transaction type (read/write)
  - Timestamp (for timeout detection)

Lookup:
  Input: RID or BID from slave
  Output: Master index for routing
  Latency: 1 cycle (registered CAM)

Benefit: Handles complex scenarios like ID reordering, burst interleaving

### 6.5.3	Response Demultiplexers

Based on extracted Bridge ID, responses are routed:

// Simplified R channel DEMUX from Slave 0
logic [BID_WIDTH-1:0] master_id;
assign master_id = s0_rid[TOTAL_ID_WIDTH-1:ID_WIDTH];  // Extract BID

always_comb begin
    // Default: No masters receive response
    m0_rvalid = 1'b0;
    m1_rvalid = 1'b0;
    m2_rvalid = 1'b0;
    m3_rvalid = 1'b0;
    
    // Route to indicated master
    case (master_id)
        2'b00: begin
            m0_rvalid = s0_rvalid;
            m0_rdata  = s0_rdata;
            m0_rid    = s0_rid[ID_WIDTH-1:0];  // Strip BID
            // ... other R signals
        end
        2'b01: begin
            m1_rvalid = s0_rvalid;
            // ... route to M1
        end
        // ... M2, M3
    endcase
end

### 6.5.4	Multi-Slave Response Merging

When multiple slaves can respond simultaneously, arbitration ensures: - Only one slave’s response delivered per master per cycle - Fair arbitration if multiple slaves have responses for same master - No response loss (responses queued until master ready)

## 6.6	2.3.6 AXI Ordering Requirements

The crossbar maintains AXI ordering rules:

### 6.6.1	Read-After-Write (RAW) Ordering

Rule: Read from address must see data from earlier write to same address

Crossbar Behavior: - Ordering enforced at slave level (slave handles RAW within itself) - Crossbar does NOT reorder transactions to same slave from same master - Different masters to same slave: No ordering guaranteed (slave must handle)

### 6.6.2	Write-After-Write (WAW) Ordering

Rule: Writes to overlapping addresses must complete in issue order

Crossbar Behavior: - Same master to same slave: Order preserved by arbiter (FIFO grant queue) - Different masters to same slave: Slave responsible for write ordering

### 6.6.3	Out-of-Order (OOO) Completion

Allowed: - Read responses can return out-of-order (different RIDs) - Reads to different slaves can complete in any order - Writes to different slaves can complete in any order

Crossbar Support: - Bridge ID tracking enables OOO response routing - Master sees responses in slave-determined order - Multi-master OOO requires careful slave design

## 6.7	2.3.7 Resource Utilization

### 6.7.1	Crossbar Core Resources

4 masters × 3 slaves configuration (64-bit data, 32-bit addr):

Logic Elements:  ~2000-3500 LEs
Registers:       ~800-1200 regs
Block RAM:       0-4 KB (if CAM used)

Breakdown per slave:
- Arbiter (4 masters, RR):        ~200 LEs, ~50 regs
- Request MUX (AR/AW/W):          ~400 LEs, ~100 regs
- Response DEMUX (R/B):           ~300 LEs, ~80 regs
- Control FSMs:                   ~100 LEs, ~50 regs

Total for 3 slaves: 3 × 1000 LEs = ~3000 LEs
Plus routing overhead: +500 LEs

### 6.7.2	Scaling with Masters and Slaves

Linear scaling: - Adding 1 master: +~500 LEs per slave (new arbiter input) - Adding 1 slave: +~1000 LEs (complete new slave port)

Example: 8 masters × 6 slaves

Estimated:  ~12,000 LEs, ~3000 regs
Block RAM:  ~8 KB (for CAM if enabled)

### 6.7.3	Optimization Techniques

- Read-Only/Write-Only Masters: Reduces arbiter complexity by 40-50%

- Power-of-Two Master Count: Simplifies BID width and routing logic

- Pipeline Stages: Trading latency for frequency (deeper pipelines)

## 6.8	2.3.8 Timing Characteristics

### 6.8.1	Latency

Request Path (Master → Slave): - Arbiter decision: 1 cycle (registered) - MUX selection: 0 cycles (combinatorial) or +1 cycle (registered) - Total: 1-2 cycles through crossbar

Response Path (Slave → Master): - BID extraction: 0 cycles (combinatorial) - DEMUX routing: 0 cycles (combinatorial) or +1 cycle (registered) - Total: 0-1 cycles through crossbar

End-to-End (Master adapter → Slave adapter): - Adapters: 2 cycles (skid buffers) - Router: 0-1 cycles (decode) - Crossbar: 1-2 cycles (arbitration + MUX) - Total: 3-5 cycles minimum latency

### 6.8.2	Throughput

Best Case (no conflicts): - Each master to different slave: N transactions/cycle (N = master count) - Maximum aggregate bandwidth: N × data_width bits/cycle

Arbitration Limits: - Multiple masters to one slave: 1 transaction/cycle to that slave - Other slaves remain available for parallel access

Burst Performance: - Once granted, bursts flow at 1 beat/cycle - Grant held until burst completes (RLAST or WLAST)

## 6.9	2.3.9 Critical Paths

### 6.9.1	Common Critical Paths

- Arbiter Request → Grant:

  - All masters’ VALID signals → Arbiter logic → Grant decision

  - Depth: ~5-8 logic levels for 4 masters

- Grant → Ready Backpressure:

  - Slave READY → Arbiter → Selected master READY

  - Depth: ~4-6 logic levels

- Response Demux:

  - Slave RDATA/RID → BID extraction → Master select → Master RDATA

  - Depth: ~6-10 logic levels for 64-bit data

### 6.9.2	Mitigation Strategies

- Registered MUX/DEMUX: +1 cycle latency, breaks paths

- Pipelined Arbitration: Multi-cycle arbiter for >8 masters

- Hierarchical Crossbar: For >16 masters, use tree structure

## 6.10	2.3.10 Configuration Parameters

### 6.10.1	Crossbar Parameters (from TOML)

[bridge]
num_masters = 4
num_slaves = 3
internal_data_width = 64
arbiter_type = "round_robin"       # "round_robin", "fixed_priority", "weighted"
registered_mux = false             # true = +1 cycle, better timing
registered_demux = false           # true = +1 cycle, better timing
enable_cam = false                 # true = CAM-based routing
cam_depth = 16                     # Outstanding transactions tracked

## 6.11	2.3.11 Debug and Observability

### 6.11.1	Recommended Debug Signals

Per Slave:
- Arbiter grants (which master granted)
- Arbiter requests (which masters requesting)
- Request MUX outputs (VALID, READY, ADDR, ID)
- Response DEMUX inputs (VALID, READY, DATA, ID)

Global:
- Active transactions count
- Stall counters (arbiter conflicts)
- BID extraction errors
- CAM hit/miss (if enabled)

### 6.11.2	Performance Counters

Useful metrics for profiling:

- Transactions per slave (read, write separate)
- Arbiter conflict rate (requests denied due to grant  contention)
- Average grant latency
- Utilization per master (% cycles active)
- Utilization per slave (% cycles busy)

## 6.12	2.3.12 Common Issues and Debug

Symptom: Master hangs with VALID=1, READY=0
Check: - Is another master holding grant to this slave? - Is arbiter logic functioning (check grant signals)? - Is slave responding with READY?

Symptom: Response goes to wrong master
Check: - Bridge ID values (verify correct BID per master) - CAM contents (if used) - BID extraction logic (check bit positions)

Symptom: Throughput lower than expected
Check: - Arbiter conflicts (multiple masters to same slave?) - Pipeline depth (excessive latency reducing effective bandwidth?) - Burst efficiency (are bursts being granted properly?)

## 6.13	2.3.13 Verification Considerations

### 6.13.1	Functional Tests

- Single Master to Each Slave: Verify basic connectivity

- All Masters to One Slave: Stress arbiter fairness

- All Masters to All Slaves: Maximum parallelism test

- OOO Responses: Issue transactions with different latencies

- Burst Interleaving: Multiple masters with bursts to same slave

### 6.13.2	Corner Cases

- Back-to-back grants (no idle cycles)
- Grant held for maximum burst length (256 beats)
- Rapid master switching (each gets 1 transaction then switches)
- Response while request in progress (pipelining)
- All masters requesting same slave simultaneously

### 6.13.3	Protocol Compliance

- AXI4 protocol checker at each master/slave interface

- Verify no READY → VALID dependencies (AXI violation)

- Check ID preservation through crossbar (modulo Bridge ID)

- Verify LAST signal handling

## 6.14	2.3.14 Future Enhancements

### 6.14.1	Planned Features

- Weighted Round-Robin: QoS support with configurable priorities

- Slave-Side Arbitration Policies: Per-slave arbiter configuration

- Grant Prediction: Speculative grant for lower latency

- Congestion Control: Throttling to prevent hotspots

### 6.14.2	Under Consideration

- Partial Crossbar: Configurable master-to-slave connectivity (not full mesh)

- Multi-Tier Hierarchy: For 32+ masters/slaves

- Virtual Channels: Separate channels for different traffic classes

- Register Slicing: Automatic pipeline insertion for timing

Related Sections: - Section 2.1: Master Adapter (request sources) - Section 2.2: Slave Router (address decode before arbitration) - Section 2.4: Arbitration (detailed arbiter algorithms) - Section 2.5: ID Management (CAM structures, Bridge ID tracking) - Section 3.1: Top-Level Integration (crossbar instantiation)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	2.4 Arbitration

Arbitration is the mechanism by which the bridge decides which master gains access to a slave when multiple masters simultaneously request the same slave. Each slave has dedicated arbiters for its read and write channels, enabling fair and efficient resource allocation.

## 7.1	2.4.1 Purpose and Function

The arbiter performs the following critical functions:

- Conflict Resolution: Selects one master when multiple masters request same slave

- Fairness: Ensures all masters eventually get access (starvation-free)

- Throughput Optimization: Minimizes idle cycles and maximizes utilization

- Grant Management: Maintains grants for burst transactions

- Priority Enforcement: Supports priority-based access policies (optional)

## 7.2	2.4.2 Arbitration Architecture

### 7.2.1	Per-Slave, Per-Channel Arbiters

Each slave has two independent arbiters:

Slave 0:
  - AR Arbiter (Read Address Channel)
    * Inputs: M0_ARVALID, M1_ARVALID, M2_ARVALID, M3_ARVALID
    * Output: Grant[1:0] → Selects one master
    
  - AW Arbiter (Write Address Channel)
    * Inputs: M0_AWVALID, M1_AWVALID, M2_AWVALID, M3_AWVALID
    * Output: Grant[1:0] → Selects one master
    * Coordinates with W channel data flow

Independence: AR and AW arbiters operate independently, allowing simultaneous read and write to same slave (if slave supports full-duplex operation).

### 7.2.2	Block Diagram

ARBITER (per slave, per channel)
                    
    From Masters                                  To Slave
    (via Router)                                  (via MUX)
         │                                            │
         │   ┌────────────────────────────────────┐  │
         │   │                                    │  │
    M0───┼──>│  Request                           │  │
 (VALID) │   │  Capture   ┌──────────────────┐   │  │
         │   │            │                  │   │  │
    M1───┼──>│            │  Arbitration     │   │  │
 (VALID) │   │            │  Logic           │───┼──┼──> GRANT[1:0]
         │   │            │  • Round-Robin   │   │  │    (Master Select)
    M2───┼──>│            │  • Fixed Prio    │   │  │
 (VALID) │   │            │  • Weighted      │   │  │
         │   │            │                  │   │  │
    M3───┼──>│            └──────────────────┘   │  │
 (VALID) │   │                     │             │  │
         │   │            ┌────────▼─────────┐   │  │
    S────┼───│<───────────┤  Burst Tracking  │   │  │
(READY)  │   │            │  (Hold Grant)    │   │  │
         │   │            └──────────────────┘   │  │
    S────┼───│<───────────┐                      │  │
(LAST)   │   │            │  Grant held until   │  │
         │   │            │  LAST asserted       │  │
         │   │            └──────────────────────┘  │
         │   │                                    │  │
         │   └────────────────────────────────────┘  │
         │                                            │

## 7.3	2.4.3 Round-Robin Arbitration

### 7.3.1	Algorithm

The Round-Robin (RR) arbiter cycles through masters in order, giving each master one opportunity to access the slave:

Priority Order (rotates after each grant):
  Cycle 0: M0 > M1 > M2 > M3
  Cycle 1: M1 > M2 > M3 > M0  (M0 was granted, now lowest priority)
  Cycle 2: M2 > M3 > M0 > M1  (M1 was granted)
  Cycle 3: M3 > M0 > M1 > M2  (M2 was granted)
  Cycle 4: M0 > M1 > M2 > M3  (M3 was granted, cycle repeats)

### 7.3.2	Implementation

// Simplified Round-Robin arbiter (4 masters)
logic [1:0] last_grant;      // Which master was granted last
logic [3:0] request;         // Current requests (VALID signals)
logic [1:0] grant;           // Grant decision

always_ff @(posedge clk) begin
    if (!rst_n) begin
        last_grant <= 2'b00;
    end else if (|request && slave_ready) begin
        // Update last_grant when transaction completes
        if (grant_accepted) begin
            last_grant <= grant;
        end
    end
end

always_comb begin
    // Default: No grant
    grant = 2'b00;
    
    // Priority encode starting after last grant
    case (last_grant)
        2'b00: begin  // Last grant to M0, try M1, M2, M3, M0
            if (request[1])      grant = 2'b01;
            else if (request[2]) grant = 2'b10;
            else if (request[3]) grant = 2'b11;
            else if (request[0]) grant = 2'b00;
        end
        2'b01: begin  // Last grant to M1, try M2, M3, M0, M1
            if (request[2])      grant = 2'b10;
            else if (request[3]) grant = 2'b11;
            else if (request[0]) grant = 2'b00;
            else if (request[1]) grant = 2'b01;
        end
        // ... similar for M2, M3
    endcase
end

### 7.3.3	Characteristics

Fairness: Excellent - Each master gets equal access over time
Latency: Bounded - Maximum wait = (N-1) × transaction_time
Throughput: Optimal - No idle cycles when requests pending
Starvation: None - Every master eventually gets grant

Best Use Cases: - Peers with similar priority - General-purpose interconnects - Default choice for most bridges

## 7.4	2.4.4 Fixed-Priority Arbitration

### 7.4.1	Algorithm

The Fixed-Priority arbiter always grants to the highest-priority requesting master:

Priority Order (never changes):
  M0 > M1 > M2 > M3  (M0 always highest priority)

Grant Decision:
  if (M0_VALID) → Grant = M0
  else if (M1_VALID) → Grant = M1
  else if (M2_VALID) → Grant = M2
  else if (M3_VALID) → Grant = M3

### 7.4.2	Implementation

// Fixed-priority arbiter (4 masters)
logic [3:0] request;
logic [1:0] grant;

always_comb begin
    // Priority encoder: M0 highest, M3 lowest
    if (request[0])      grant = 2'b00;  // M0 highest priority
    else if (request[1]) grant = 2'b01;
    else if (request[2]) grant = 2'b10;
    else if (request[3]) grant = 2'b11;  // M3 lowest priority
    else                 grant = 2'b00;  // Default (no requests)
end

### 7.4.3	Characteristics

Fairness: Poor - Low-priority masters can starve
Latency: Variable - High-priority: minimal, Low-priority: unbounded
Throughput: Optimal - No idle cycles when requests pending
Starvation: Possible for low-priority masters

Best Use Cases: - Real-time systems with priority requirements - CPU (high) vs. DMA (low) scenarios - Time-critical masters need guaranteed access

### 7.4.4	Starvation Prevention

Optional enhancement: Priority aging

- Track cycles since last grant per master
- Temporarily boost priority of starved masters
- Reset age counter after grant

Example:
  M0 age = 100 cycles → Boost M0 above normal priority
  M1 age = 5 cycles   → Normal priority

## 7.5	2.4.5 Weighted Arbitration

### 7.5.1	Algorithm

The Weighted arbiter assigns different access rates to masters based on weights:

Configuration:
  M0 weight = 4  (50% of bandwidth)
  M1 weight = 2  (25% of bandwidth)
  M2 weight = 1  (12.5% of bandwidth)
  M3 weight = 1  (12.5% of bandwidth)

Grant Sequence (repeating pattern):
  M0, M0, M0, M0, M1, M1, M2, M3  (8 cycles total)
  M0 gets 4/8, M1 gets 2/8, M2 gets 1/8, M3 gets 1/8

### 7.5.2	Implementation

// Weighted arbiter using credit counter
logic [7:0] credits [0:3];  // Credit counters per master
logic [1:0] grant;

always_ff @(posedge clk) begin
    if (!rst_n) begin
        credits[0] <= 8'd4;  // M0 weight
        credits[1] <= 8'd2;  // M1 weight
        credits[2] <= 8'd1;  // M2 weight
        credits[3] <= 8'd1;  // M3 weight
    end else if (grant_accepted) begin
        // Decrement granted master's credits
        credits[grant] <= credits[grant] - 1;
        
        // Reload when all credits exhausted
        if (all_credits_zero) begin
            credits[0] <= 8'd4;
            credits[1] <= 8'd2;
            credits[2] <= 8'd1;
            credits[3] <= 8'd1;
        end
    end
end

always_comb begin
    // Grant to master with highest credits and active request
    grant = select_highest_credit_with_request(credits, request);
end

### 7.5.3	Characteristics

Fairness: Configurable - Proportional to weights
Latency: Bounded - Depends on weight ratio
Throughput: Near-optimal - Small overhead for credit tracking
Starvation: None - All masters get share per weight

Best Use Cases: - QoS requirements (guaranteed bandwidth) - Mixed workload (video + CPU + DMA) - Service-level agreements

## 7.6	2.4.6 Burst Handling

### 7.6.1	Grant Locking

Once granted, a master holds the grant until its burst completes:

AR Channel Burst:
  1. Master asserts ARVALID with ARLEN = 7 (8-beat burst)
  2. Arbiter grants to this master
  3. Grant held until slave returns RLAST
  4. Other masters blocked during this time

AW/W Channel Burst:
  1. Master asserts AWVALID with AWLEN = 15 (16-beat burst)
  2. Arbiter grants to this master
  3. Grant held until master asserts WLAST
  4. Other masters blocked during W data transfer

### 7.6.2	Implementation

// Burst grant locking
logic grant_locked;
logic [1:0] locked_master;

always_ff @(posedge clk) begin
    if (!rst_n) begin
        grant_locked <= 1'b0;
    end else begin
        if (grant_accepted && !last_beat) begin
            // Lock grant for burst
            grant_locked <= 1'b1;
            locked_master <= grant;
        end else if (last_beat) begin
            // Release grant when burst completes
            grant_locked <= 1'b0;
        end
    end
end

assign grant = grant_locked ? locked_master : arbiter_decision;

### 7.6.3	Fairness Considerations

Long bursts can block other masters: - Maximum AXI burst: 256 beats - At 1 cycle/beat: Up to 256 cycles blocked - Impacts latency for other masters

Mitigation: - Limit burst lengths in master configuration - Use burst interleaving (complex, not implemented in Phase 1) - Monitor arbiter stall counters

## 7.7	2.4.7 Resource Utilization

### 7.7.1	Per-Arbiter Resources

Round-Robin (4 masters):

Logic Elements:  ~150 LEs
Registers:       ~40 regs

Breakdown:
- Priority encoder:        ~80 LEs
- Last grant tracking:     ~10 regs
- Request capture:         ~20 regs
- Grant FSM:               ~40 LEs, ~10 regs

Fixed-Priority (4 masters):

Logic Elements:  ~80 LEs
Registers:       ~20 regs

Simpler than RR (no rotation logic)

Weighted (4 masters):

Logic Elements:  ~200 LEs
Registers:       ~60 regs

Additional credit counters and comparison logic

### 7.7.2	Scaling with Master Count

Resource usage scales approximately O(N²) where N = master count:

2 masters:  ~50 LEs
4 masters:  ~150 LEs
8 masters:  ~400 LEs
16 masters: ~1200 LEs

The N² scaling comes from: - Priority encoder: Compares all pairs of masters - Grant MUX: N-to-1 multiplexing increases with N

## 7.8	2.4.8 Timing Characteristics

### 7.8.1	Arbitration Latency

Single-Cycle Arbitration (default):

Request → Grant Decision: 1 cycle
Grant Decision → MUX Output: 0 cycles (combinatorial)
Total: 1 cycle

Multi-Cycle Arbitration (high frequency):

Request → Grant Decision: 2-3 cycles (pipelined)
Improves timing at cost of latency

### 7.8.2	Critical Paths

Typical critical paths in arbiter: 1. Request collection: All master VALID signals → Arbiter input 2. Priority encode: Compare all requests → Grant decision 3. Grant propagate: Grant → MUX select → Slave interface

Path Depth: - 4 masters: ~6-8 logic levels - 8 masters: ~8-12 logic levels - 16 masters: ~12-16 logic levels

## 7.9	2.4.9 Configuration Parameters

### 7.9.1	Arbiter Configuration (TOML)

[bridge]
arbiter_type = "round_robin"    # "round_robin", "fixed_priority", "weighted"

[bridge.arbitration]
pipeline_stages = 1              # 1-3 (more = better timing, higher latency)
enable_priority_aging = false    # Prevent starvation in fixed-priority
aging_threshold = 1000           # Cycles before priority boost

# Weighted arbitration weights (only if arbiter_type = "weighted")
[[bridge.arbitration.weights]]
master = "cpu"
weight = 4                       # Relative weight (1-255)

[[bridge.arbitration.weights]]
master = "dma"
weight = 2

[[bridge.arbitration.weights]]
master = "periph"
weight = 1

## 7.10	2.4.10 Debug and Observability

### 7.10.1	Recommended Debug Signals

Per Arbiter:
- Request vector (all masters requesting)
- Grant decision (which master granted)
- Grant locked status (burst in progress)
- Last grant (for RR rotation tracking)

Performance Counters:
- Grants per master (utilization)
- Denied requests per master (contention)
- Average grant latency per master
- Arbiter conflict cycles (multiple requests, one grant)

### 7.10.2	Common Issues and Debug

Symptom: Master never gets grant (starvation)
Check: - Arbiter type (fixed-priority can starve low-priority masters) - Other masters continuously requesting? - Enable priority aging if using fixed-priority

Symptom: Unfair access (one master dominates)
Check: - Arbiter type (should be round-robin for fairness) - Burst lengths (long bursts block others) - Request patterns (one master requesting more frequently)

Symptom: Low throughput despite requests
Check: - Arbiter conflicts (too many masters to one slave) - Pipeline depth (excessive arbitration latency) - Burst efficiency (short bursts waste cycles)

## 7.11	2.4.11 Verification Considerations

### 7.11.1	Test Scenarios

- Fair Access Test (Round-Robin):

- All masters continuously request same slave
- Verify each master gets equal grants over 1000 cycles
- Expected: Each of 4 masters gets ~250 grants

- Priority Test (Fixed-Priority):

- High-priority master requests intermittently
- Low-priority masters request continuously
- Verify high-priority always wins when requesting

- Burst Locking Test:

- Master issues 16-beat burst
- Other masters request during burst
- Verify grant held until LAST
- Verify other masters blocked during burst

- Weighted Bandwidth Test:

- Configure M0=4, M1=2, M2=1, M3=1
- All masters request continuously for 10000 cycles
- Verify grants proportional: M0≈50%, M1≈25%, M2≈12.5%, M3≈12.5%

### 7.11.2	Corner Cases

- Single master requesting (trivial grant)
- All masters requesting simultaneously (worst-case arbitration)
- Grant released and new grant same cycle (back-to-back)
- Master drops request after arbiter latency (stale grant)
- Maximum length burst (256 beats blocking)

## 7.12	2.4.12 Performance Impact

### 7.12.1	Arbiter Choice Impact

Round-Robin: - Pros: Fair, starvation-free, predictable latency - Cons: Cannot prioritize time-critical masters - Use: General-purpose, peer masters

Fixed-Priority: - Pros: Guaranteed low latency for high-priority masters - Cons: Low-priority can starve, unpredictable for low-priority - Use: Real-time, priority-sensitive systems

Weighted: - Pros: Configurable bandwidth allocation, QoS support - Cons: More complex, small overhead - Use: Mixed workload, SLA requirements

### 7.12.2	Arbiter Efficiency

Assuming all masters request same slave continuously:

Best Case: 100% efficiency (one master)
  - No arbitration conflicts
  - Zero idle cycles

Typical Case: 25-50% per-master efficiency (4 masters, round-robin)
  - Each master gets ~25% of time grants
  - Load balancing to other slaves improves

Worst Case: Heavy contention
  - 4 masters to 1 slave: 25% efficiency per master
  - Solution: Add slaves or use more slaves in parallel

## 7.13	2.4.13 Future Enhancements

### 7.13.1	Planned Features

- Dynamic Weight Adjustment: Runtime-configurable weights via registers

- QoS Classes: Multiple priority levels with configurable policies

- Deadline-Based Arbitration: Grant based on transaction deadlines

- Predictive Arbitration: Speculative grants to reduce latency

### 7.13.2	Under Consideration

- Multi-Level Arbitration: Hierarchical for >16 masters

- Token Bucket: Rate limiting per master

- History-Based: Learn access patterns and optimize grants

- ECC Protection: For arbitration state (safety-critical systems)

Related Sections: - Section 2.3: Crossbar Core (arbiter integration) - Section 2.1: Master Adapter (request sources) - Section 2.5: ID Management (transaction tracking during grants) - Chapter 6: Performance (arbiter impact on throughput)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	2.5 ID Management

ID Management is the system by which the bridge tracks outstanding transactions and routes responses back to the originating master. This is accomplished through Bridge ID injection, Content Addressable Memory (CAM) structures, and ID translation logic.

## 8.1	2.5.1 Purpose and Function

The ID management system performs the following critical functions:

- Transaction Tracking: Maintains association between requests and originating masters

- Response Routing: Directs responses to correct master using embedded IDs

- Out-of-Order Support: Handles responses returning in different order than issued

- ID Space Isolation: Prevents ID conflicts between multiple masters

- Burst Management: Tracks multi-beat bursts through the bridge

## 8.2	2.5.2 Bridge ID Concept

### 8.2.1	Problem Statement

Without ID management, the bridge cannot determine which master originated a transaction:

Scenario: Two masters with overlapping IDs
  Master 0: Issues read with ARID = 4'h5
  Master 1: Issues read with ARID = 4'h5 (same ID!)
  
When slave responds with RID = 4'h5, which master gets the response?
  → Ambiguous! Need additional information.

### 8.2.2	Solution: Bridge ID Injection

The bridge adds a Bridge ID (BID) to each transaction ID:

Internal ID = {Bridge_ID, Original_ID}

Master 0, ARID = 4'h5:
  Internal ARID = {2'b00, 4'h5} = 6'b00_0101
  
Master 1, ARID = 4'h5:
  Internal ARID = {2'b01, 4'h5} = 6'b01_0101
  
Now responses with RID = 6'b00_0101 → Route to Master 0
                    RID = 6'b01_0101 → Route to Master 1

## 8.3	2.5.3 Block Diagram

### 8.3.1	Figure 2.5: ID Management Architecture

ID Management Architecture

ID management architecture showing CAM and FIFO modes for transaction tracking and response routing.

## 8.4	2.5.4 Bridge ID Width Calculation

### 8.4.1	Formula

BID_WIDTH = clog2(NUM_MASTERS)

Where clog2(x) = ceiling(log2(x))

### 8.4.2	Examples

2 masters:  clog2(2)  = 1 bit   (BID: 0, 1)
3 masters:  clog2(3)  = 2 bits  (BID: 00, 01, 10)
4 masters:  clog2(4)  = 2 bits  (BID: 00, 01, 10, 11)
5 masters:  clog2(5)  = 3 bits  (BID: 000-100)
8 masters:  clog2(8)  = 3 bits  (BID: 000-111)
16 masters: clog2(16) = 4 bits  (BID: 0000-1111)

### 8.4.3	ID Width Growth

Configuration: 4 masters, external ARID_WIDTH = 4

Internal ARID_WIDTH = BID_WIDTH + External ARID_WIDTH
                    = 2 + 4
                    = 6 bits

Slave sees 6-bit IDs
Master sees 4-bit IDs
Bridge translates between them

## 8.5	2.5.4a Multi-Master OR-Merge Gating

When multiple masters drive address and ID signals toward a slave, an OR-merge combines them. To prevent idle masters with stale addresses from corrupting the routing tag, every OR-merge term is gated on both the slave_select condition AND the valid signal:

// Wrong (deprecated):
assign s0_bridge_id_aw = m0_bridge_id_aw | m1_bridge_id_aw | m2_bridge_id_aw;
//                       ↓ Master 1 contributes even if awvalid=0 and address is stale!

// Correct:
assign s0_bridge_id_aw = (m0_slave_select_aw && m0_awvalid ? m0_bridge_id_aw : '0) |
                        (m1_slave_select_aw && m1_awvalid ? m1_bridge_id_aw : '0) |
                        (m2_slave_select_aw && m2_awvalid ? m2_bridge_id_aw : '0);

Why this matters: An idle master with stale address (fub_axi_awaddr = 0) still decodes to slave 0 if that’s the address map. Without valid gating, its zero bridge_id corrupts the slave’s routing tag via OR-merge. When the slave responds, the B-response gets routed to the idle master instead of the actual requester.

Implementation: - All AW/AR/W data signals gated by {slave_select && <channel_valid>} - bridge_id_aw/ar signals gated by corresponding <channel>valid - Prevents any idle master from affecting the OR-merge result

## 8.6	2.5.5 ID Injection (Request Path)

### 8.6.1	Read Address Channel (AR)

// ID injection at master adapter
logic [BID_WIDTH-1:0] master_bid;         // Constant per master
logic [ARID_WIDTH-1:0] external_arid;     // From master
logic [TOTAL_ARID_WIDTH-1:0] internal_arid;  // To crossbar

assign master_bid = MASTER_INDEX;  // M0=0, M1=1, M2=2, M3=3
assign internal_arid = {master_bid, external_arid};

// Example: Master 2, ARID = 4'h7
// internal_arid = {2'b10, 4'h7} = 6'b10_0111

### 8.6.2	Write Address Channel (AW)

// ID injection for write transactions
logic [BID_WIDTH-1:0] master_bid;
logic [AWID_WIDTH-1:0] external_awid;
logic [TOTAL_AWID_WIDTH-1:0] internal_awid;

assign internal_awid = {master_bid, external_awid};

// Note: AWID and ARID widths can differ per master

### 8.6.3	Injection Timing

Combinatorial (default): - ID concatenation done in same cycle as VALID assertion - Zero latency overhead - May contribute to critical path in high-frequency designs

Registered (optional): - Add pipeline register after ID injection - +1 cycle latency - Breaks critical path

## 8.7	2.5.6 ID Extraction (Response Path)

### 8.7.1	Read Data Channel (R)

// ID extraction at crossbar response router
logic [TOTAL_RID_WIDTH-1:0] internal_rid;   // From slave
logic [BID_WIDTH-1:0] extracted_bid;        // Upper bits
logic [RID_WIDTH-1:0] external_rid;         // Lower bits

// Extract Bridge ID
assign extracted_bid = internal_rid[TOTAL_RID_WIDTH-1:RID_WIDTH];

// Strip Bridge ID for master
assign external_rid = internal_rid[RID_WIDTH-1:0];

// Route based on BID
always_comb begin
    case (extracted_bid)
        2'b00: master0_rvalid = slave_rvalid;
        2'b01: master1_rvalid = slave_rvalid;
        2'b10: master2_rvalid = slave_rvalid;
        2'b11: master3_rvalid = slave_rvalid;
    endcase
end

### 8.7.2	Write Response Channel (B)

// Similar extraction for write responses
logic [TOTAL_BID_WIDTH-1:0] internal_bid;
logic [BID_WIDTH-1:0] extracted_bid;
logic [BID_WIDTH-1:0] external_bid;

assign extracted_bid = internal_bid[TOTAL_BID_WIDTH-1:BID_WIDTH];
assign external_bid = internal_bid[BID_WIDTH-1:0];

// Route to originating master

## 8.8	2.5.7 Content Addressable Memory (CAM)

### 8.8.1	When to Use CAM

Simple Configurations (No CAM needed): - Single master (BID unnecessary) - Direct ID mapping suffices - Lower resource usage

Complex Configurations (CAM beneficial): - Many masters (>8) - Out-of-order responses common - Multiple outstanding transactions per master - ID reordering within slave

### 8.8.2	CAM Structure

Entry Format:
┌────────────┬──────────────┬─────────────┬───────────┬───────┐
│ Internal ID│ Master Index │ Transaction │ Timestamp │ Valid │
│  (6-12 bits│  (2-4 bits)  │    Type     │ (counter) │ (1b)  │
│            │              │   (R/W)     │           │       │
└────────────┴──────────────┴─────────────┴───────────┴───────┘
     10b           3b             1b          16b         1b

Total per entry: ~30 bits
CAM depth: 16-64 entries typical

### 8.8.3	CAM Operations

Allocation (Request Path):

1. Master issues AR/AW transaction
2. Find free CAM entry (Valid = 0)
3. Write entry:
   - Internal ID = {BID, External_ID}
   - Master Index = BID
   - Transaction Type = Read or Write
   - Timestamp = Current cycle count
   - Valid = 1
4. Forward transaction to slave

Lookup (Response Path):

1. Slave returns R/B response with Internal ID
2. CAM lookup: Search for matching Internal ID
3. Extract Master Index from matching entry
4. Route response to Master[Master Index]
5. If LAST: Deallocate entry (Valid = 0)

Associative Search: - All entries checked in parallel - 1-cycle lookup (registered CAM) - Match found → Returns master index - No match → Error condition (protocol violation)

## 8.9	2.5.8 CAM Implementation

### 8.9.1	Parallel CAM (Fast, Resource-Intensive)

// Parallel CAM structure (16 entries)
typedef struct packed {
    logic valid;
    logic [TOTAL_ID_WIDTH-1:0] internal_id;
    logic [BID_WIDTH-1:0] master_idx;
    logic txn_type;  // 0=read, 1=write
    logic [15:0] timestamp;
} cam_entry_t;

cam_entry_t cam [0:15];

// Parallel search
logic [15:0] match;
logic [3:0] match_idx;

for (genvar i = 0; i < 16; i++) begin
    assign match[i] = cam[i].valid && 
                      (cam[i].internal_id == response_id);
end

// Priority encode first match
assign match_idx = find_first_set(match);
assign routed_master = cam[match_idx].master_idx;

### 8.9.2	Sequential CAM (Slow, Resource-Efficient)

// Sequential CAM search (multi-cycle)
logic [3:0] search_idx;
logic searching;

always_ff @(posedge clk) begin
    if (search_start) begin
        search_idx <= 0;
        searching <= 1'b1;
    end else if (searching) begin
        if (cam[search_idx].valid && 
            cam[search_idx].internal_id == response_id) begin
            // Match found
            routed_master <= cam[search_idx].master_idx;
            searching <= 1'b0;
        end else if (search_idx == 15) begin
            // No match found (error)
            searching <= 1'b0;
            error <= 1'b1;
        end else begin
            search_idx <= search_idx + 1;
        end
    end
end

Trade-off: - Parallel: 1-cycle, ~2000 LEs for 16 entries - Sequential: 16 cycles, ~200 LEs for 16 entries

## 8.10	2.5.9 Outstanding Transaction Limits

### 8.10.1	Configuration

[bridge]
enable_cam = true
cam_depth = 16              # Max outstanding transactions

[[masters]]
name = "cpu"
max_outstanding_reads = 8   # Per-master limit
max_outstanding_writes = 8

### 8.10.2	Enforcement

When CAM full:

1. Master issues new request
2. Check CAM for free entry
3. If full:
   - ARREADY/AWREADY = 0 (backpressure)
   - Wait for response to free entry
4. When entry freed (RLAST/BVALID):
   - Accept new request

### 8.10.3	Sizing Guidelines

CAM Depth = Σ(max_outstanding per master) + Safety margin

Example: 4 masters, 4 outstanding each
  Minimum CAM depth = 4 × 4 = 16 entries
  Recommended depth = 20 entries (25% margin)

## 8.11	2.5.10 Resource Utilization

### 8.11.1	ID Injection/Extraction Only

Per Master (no CAM):

Logic Elements:  ~20-50 LEs
Registers:       ~10 regs

Simple bit concatenation and extraction
Minimal overhead

### 8.11.2	With CAM

CAM Resources (16 entries, 6-bit IDs):

Parallel CAM:
  Logic Elements:  ~2000 LEs
  Registers:       ~500 regs
  Block RAM:       0 (distributed)

BRAM-Based CAM:
  Logic Elements:  ~500 LEs
  Registers:       ~100 regs
  Block RAM:       1-2 KB

Sequential CAM:
  Logic Elements:  ~200 LEs
  Registers:       ~100 regs
  Block RAM:       0

### 8.11.3	Scaling

CAM Depth      Parallel      BRAM         Sequential
16 entries     ~2000 LEs     ~500 LEs     ~200 LEs
32 entries     ~4000 LEs     ~800 LEs     ~250 LEs
64 entries     ~8000 LEs     ~1200 LEs    ~300 LEs

## 8.12	2.5.11 Timing Characteristics

### 8.12.1	ID Injection Latency

Combinatorial (default): - 0 cycles overhead - Part of adapter pipeline stage

Registered: - +1 cycle latency - Breaks timing path

### 8.12.2	ID Extraction/Routing Latency

Direct Decode (no CAM): - 0 cycles (combinatorial BID extraction)

Parallel CAM: - 1 cycle (registered search)

Sequential CAM: - 1-N cycles (where N = CAM depth) - Average: N/2 cycles

### 8.12.3	End-to-End Impact

Configuration: No CAM, combinatorial ID management
  Request:  Master → +0 cycles → Crossbar
  Response: Slave → +0 cycles → Master
  Total: 0 cycles overhead

Configuration: Parallel CAM, registered
  Request:  Master → +1 cycle (allocation) → Crossbar
  Response: Slave → +1 cycle (lookup) → Master
  Total: 2 cycles overhead

## 8.13	2.5.12 Configuration Parameters

### 8.13.1	ID Management Configuration (TOML)

[bridge]
num_masters = 4
enable_cam = false              # Use CAM for ID tracking
cam_type = "parallel"           # "parallel", "bram", "sequential"
cam_depth = 16                  # Outstanding transaction capacity

[bridge.id_management]
registered_injection = false    # Register ID injection (+1 cycle)
registered_extraction = false   # Register ID extraction (+1 cycle)
enable_timeout = true           # Detect hung transactions
timeout_cycles = 10000          # Cycles before timeout error

[[masters]]
name = "cpu"
arid_width = 4                  # External ID width
awid_width = 4
max_outstanding_reads = 8       # CAM allocation limit
max_outstanding_writes = 8

## 8.14	2.5.13 Debug and Observability

### 8.14.1	Recommended Debug Signals

ID Injection:
- Master BID assignments (constant per master)
- External IDs (from masters)
- Internal IDs (after injection)

ID Extraction:
- Internal IDs (from slaves)
- Extracted BIDs
- External IDs (to masters)

CAM (if enabled):
- CAM occupancy (number of valid entries)
- Allocation/deallocation events
- Search hits/misses
- Timeout events

### 8.14.2	Performance Counters

- Total transactions tracked
- CAM hit rate
- CAM miss rate (should be 0)
- Average CAM occupancy
- Peak CAM occupancy
- Timeout errors
- ID width overhead (bits added per transaction)

## 8.15	2.5.14 Common Issues and Debug

Symptom: Response goes to wrong master
Check: - BID assignment (verify each master has unique BID) - ID width calculation (BID_WIDTH correct?) - Bit slicing in extraction logic - CAM contents (if used)

Symptom: CAM full, transactions stalling
Check: - CAM depth vs. outstanding transaction count - Are responses being received? (slave responsive?) - Timeout threshold (too short?) - Leaked entries (not deallocated after LAST)

Symptom: CAM miss errors
Check: - Slave returning incorrect IDs - ID corruption in transit - Race condition (deallocation before response complete)

## 8.16	2.5.15 Verification Considerations

### 8.16.1	Test Scenarios

- Basic ID Injection/Extraction:

- Single master, single transaction
- Verify BID added correctly
- Verify BID stripped before response delivery
- Check external ID unchanged

- Multi-Master ID Isolation:

- Multiple masters with same external IDs
- Verify responses route to correct master
- Check BID uniqueness prevents collisions

- CAM Capacity:

- Issue max_outstanding transactions
- Verify backpressure when CAM full
- Issue responses to free entries
- Verify new transactions accepted

- Out-of-Order Responses:

- Issue transactions: ID=1, ID=2, ID=3
- Return responses: ID=3, ID=1, ID=2
- Verify CAM correctly routes each

- CAM Timeout:

- Issue transaction
- Slave doesn't respond within timeout_cycles
- Verify timeout error signaled
- Verify CAM entry deallocated (or flagged)

## 8.17	2.5.16 Future Enhancements

### 8.17.1	Planned Features

- Dynamic CAM Depth: Runtime adjustment based on utilization

- Per-Master CAM Partition: Guaranteed entries per master

- ID Compression: Reduce internal ID width for slaves with limited ID support

- Transaction Ordering: CAM tracks issue order for reordering enforcement

### 8.17.2	Under Consideration

- Multi-Level CAM: Hierarchical for large transaction counts

- TCAM Support: Partial ID matching (wildcards)

- Error Injection: Debug mode to test error handling

- CAM Mirrors: Redundant CAMs for fail-safe operation

Related Sections: - Section 2.1: Master Adapter (BID injection location) - Section 2.3: Crossbar Core (BID extraction, response routing) - Section 2.8: Response Routing (detailed routing logic) - Section 3.2: Master Port Interface (ID width specifications)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	2.6 Width Conversion

Width Conversion is the mechanism by which the bridge adapts between masters and slaves with different data bus widths. The bridge uses a 64-bit internal data path, with conversion logic at the master and slave interfaces to support narrower or wider external widths.

## 9.1	2.6.1 Purpose and Function

Width conversion performs the following critical functions:

- Data Path Adaptation: Converts between different data widths (8, 16, 32, 64, 128, 256 bits)

- Burst Splitting: Divides wide transactions into multiple narrow transactions

- Burst Merging: Combines multiple narrow beats into fewer wide beats

- Strobe Mapping: Translates write strobes across width boundaries

- Address Alignment: Adjusts addresses for width differences

## 9.2	2.6.2 Internal Data Width

### 9.2.1	64-Bit Standard

The bridge uses 64-bit (8-byte) internal data width for the crossbar:

Rationale:
- Balance between resource usage and performance
- Common width for modern embedded processors
- Efficient for both 32-bit and 64-bit masters
- Reasonably sized multiplexers in crossbar

### 9.2.2	Width Conversion Locations

Master (32-bit) → [Upsizer] → Crossbar (64-bit) → [Downsizer] → Slave (32-bit)
Master (64-bit) → [No conversion] → Crossbar (64-bit) → [No conversion] → Slave (64-bit)
Master (128-bit) → [Downsizer] → Crossbar (64-bit) → [Upsizer] → Slave (128-bit)

## 9.3	2.6.3 Block Diagram

### 9.3.1	Figure 2.6: Width Conversion Architecture

Width Conversion Architecture

Width conversion architecture showing data upsizing and downsizing with beat count adjustment and strobe handling.

## 9.4	2.6.4 Upsizing (Narrow to Wide)

### 9.4.1	Overview

Upsizing converts narrow data to wide data by buffering multiple narrow beats into a single wide beat.

Example: 32-bit master → 64-bit crossbar
  Master beat 0: 32'hAAAA_BBBB (bytes 3:0)
  Master beat 1: 32'hCCCC_DDDD (bytes 7:4)
  → Crossbar beat: 64'hCCCC_DDDD_AAAA_BBBB

### 9.4.2	Write Upsizing

Burst of 4 (32-bit) → Burst of 2 (64-bit):

Master AW: AWADDR = 0x1000, AWLEN = 3, AWSIZE = 2 (4 bytes)
Master W beats:
  W[0]: WDATA = 0xAAAA_BBBB, WSTRB = 4'b1111, WLAST = 0
  W[1]: WDATA = 0xCCCC_DDDD, WSTRB = 4'b1111, WLAST = 0
  W[2]: WDATA = 0xEEEE_FFFF, WSTRB = 4'b1111, WLAST = 0
  W[3]: WDATA = 0x1111_2222, WSTRB = 4'b1111, WLAST = 1

Crossbar AW: AWADDR = 0x1000, AWLEN = 1, AWSIZE = 3 (8 bytes)
Crossbar W beats:
  W[0]: WDATA = 0xCCCC_DDDD_AAAA_BBBB, WSTRB = 8'b1111_1111, WLAST = 0
  W[1]: WDATA = 0x1111_2222_EEEE_FFFF, WSTRB = 8'b1111_1111, WLAST = 1

### 9.4.3	Read Upsizing

Burst of 8 (32-bit) → Burst of 4 (64-bit):

Master AR: ARADDR = 0x2000, ARLEN = 7, ARSIZE = 2 (4 bytes)

Crossbar AR: ARADDR = 0x2000, ARLEN = 3, ARSIZE = 3 (8 bytes)

Crossbar R beats:
  R[0]: RDATA = 0x1111_2222_3333_4444, RLAST = 0
  R[1]: RDATA = 0x5555_6666_7777_8888, RLAST = 0
  R[2]: RDATA = 0x9999_AAAA_BBBB_CCCC, RLAST = 0
  R[3]: RDATA = 0xDDDD_EEEE_FFFF_0000, RLAST = 1

Master R beats (split from crossbar):
  R[0]: RDATA = 0x3333_4444, RLAST = 0  (from R[0] low)
  R[1]: RDATA = 0x1111_2222, RLAST = 0  (from R[0] high)
  R[2]: RDATA = 0x7777_8888, RLAST = 0  (from R[1] low)
  R[3]: RDATA = 0x5555_6666, RLAST = 0  (from R[1] high)
  R[4]: RDATA = 0xBBBB_CCCC, RLAST = 0  (from R[2] low)
  R[5]: RDATA = 0x9999_AAAA, RLAST = 0  (from R[2] high)
  R[6]: RDATA = 0xFFFF_0000, RLAST = 0  (from R[3] low)
  R[7]: RDATA = 0xDDDD_EEEE, RLAST = 1  (from R[3] high)

### 9.4.4	Strobe Mapping (Upsizing)

32-bit WSTRB → 64-bit WSTRB:

Beat 0 (lower 32 bits):
  32-bit WSTRB = 4'b1010 → 64-bit WSTRB = 8'b0000_1010

Beat 1 (upper 32 bits):
  32-bit WSTRB = 4'b1111 → 64-bit WSTRB = 8'b1111_0000

Combined:
  64-bit WSTRB = 8'b1111_1010

## 9.5	2.6.5 Downsizing (Wide to Narrow)

### 9.5.1	Overview

Downsizing converts wide data to narrow data by splitting a single wide beat into multiple narrow beats.

Example: 128-bit master → 64-bit crossbar
  Master beat: 128'h1111_2222_3333_4444_5555_6666_7777_8888
  → Crossbar beat 0: 64'h5555_6666_7777_8888 (lower)
  → Crossbar beat 1: 64'h1111_2222_3333_4444 (upper)

### 9.5.2	Write Downsizing

Burst of 2 (128-bit) → Burst of 4 (64-bit):

Master AW: AWADDR = 0x3000, AWLEN = 1, AWSIZE = 4 (16 bytes)
Master W beats:
  W[0]: WDATA = 0xAAAA...(128 bits), WSTRB = 16'hFFFF, WLAST = 0
  W[1]: WDATA = 0xBBBB...(128 bits), WSTRB = 16'hFFFF, WLAST = 1

Crossbar AW: AWADDR = 0x3000, AWLEN = 3, AWSIZE = 3 (8 bytes)
Crossbar W beats:
  W[0]: WDATA = 0xAAAA_low(64), WSTRB = 8'hFF, WLAST = 0
  W[1]: WDATA = 0xAAAA_high(64), WSTRB = 8'hFF, WLAST = 0
  W[2]: WDATA = 0xBBBB_low(64), WSTRB = 8'hFF, WLAST = 0
  W[3]: WDATA = 0xBBBB_high(64), WSTRB = 8'hFF, WLAST = 1

### 9.5.3	Read Downsizing

Burst of 1 (128-bit) → Burst of 2 (64-bit):

Master AR: ARADDR = 0x4000, ARLEN = 0, ARSIZE = 4 (16 bytes)

Crossbar AR: ARADDR = 0x4000, ARLEN = 1, ARSIZE = 3 (8 bytes)

Crossbar R beats:
  R[0]: RDATA = 0x1111_2222_3333_4444, RLAST = 0
  R[1]: RDATA = 0x5555_6666_7777_8888, RLAST = 1

Master R beat (merged):
  R[0]: RDATA = 0x5555_6666_7777_8888_1111_2222_3333_4444, RLAST = 1

### 9.5.4	Strobe Mapping (Downsizing)

128-bit WSTRB → 64-bit WSTRB (split):

128-bit WSTRB = 16'hF0F0_0FF0

Beat 0 (bytes 7:0):
  64-bit WSTRB = 8'b0000_1111_1111_0000 = 8'h0F0 → 8'hF0

Beat 1 (bytes 15:8):
  64-bit WSTRB = 8'b1111_0000_1111_0000 = 8'hF0F0 >> 8 → 8'hF0

## 9.6	2.6.6 Address Alignment

### 9.6.1	Address Adjustment for Width

When changing widths, addresses must align to the new width:

32-bit (4-byte) aligned address: 0x1004
Upsized to 64-bit (8-byte): 0x1000 (round down to 8-byte boundary)

Narrow access at 0x1004 within 64-bit word:
  - Byte offset = 0x1004 & 0x7 = 4
  - Data at bytes [7:4] of 64-bit word
  - Lower bytes [3:0] unused (WSTRB = 8'b1111_0000)

### 9.6.2	Unaligned Access Handling

Master: 32-bit, unaligned access at 0x1002
Problem: Not aligned to 4-byte boundary

Options:
1. Error response (strict mode)
2. Round down address, use WSTRB for actual bytes
3. Split into multiple aligned accesses

Bridge default: Option 2 (use WSTRB)

## 9.7	2.6.7 Burst Length Adjustment

### 9.7.1	Length Calculation

Formula:
  New_Length = (Old_Length + 1) × (Old_Width / New_Width) - 1

Example: Upsizing 32→64
  Old: AWLEN = 7 (8 beats), WIDTH = 32
  New: AWLEN = (7+1) × (32/64) - 1 = 8 × 0.5 - 1 = 3 (4 beats)

Example: Downsizing 128→64
  Old: ARLEN = 3 (4 beats), WIDTH = 128
  New: ARLEN = (3+1) × (128/64) - 1 = 4 × 2 - 1 = 7 (8 beats)

### 9.7.2	Odd Burst Lengths

When burst doesn’t divide evenly:

Example: 3 beats of 32-bit → 64-bit
  3 beats × 4 bytes = 12 bytes total
  12 bytes / 8 bytes per beat = 1.5 beats
  
Solution:
  - Beat 0: Full 64-bit (8 bytes)
  - Beat 1: Partial 64-bit (4 bytes, WSTRB = 8'b0000_1111)
  - AWLEN = 1 (2 beats)

## 9.8	2.6.8 Resource Utilization

### 9.8.1	Per-Converter Resources

32→64 Upsizer:

Logic Elements:  ~300 LEs
Registers:       ~100 regs (buffering + FSM)
Block RAM:       0

Breakdown:
- Data buffer (32 bits): ~32 regs
- Strobe buffer: ~4 regs
- Beat counter: ~8 regs
- Control FSM: ~50 LEs, ~20 regs
- MUX logic: ~150 LEs

64→32 Downsizer:

Logic Elements:  ~250 LEs
Registers:       ~120 regs
Block RAM:       0

Similar structure but includes split logic

128→64 Downsizer:

Logic Elements:  ~400 LEs
Registers:       ~180 regs
Block RAM:       0

Larger data paths, more complex MUX

### 9.8.2	Scaling

Resource usage scales primarily with: - Width ratio: 2:1 vs. 4:1 vs. 8:1 conversion - Data width: 128-bit vs. 256-bit buffers - Buffering depth: Single vs. multi-beat buffering

## 9.9	2.6.9 Timing Characteristics

### 9.9.1	Latency

Upsizing (combining beats): - Buffering latency: 1-2 cycles (wait for N narrow beats) - First beat output: After receiving required narrow beats - Example: 32→64 requires 2 master beats before 1 crossbar beat

Downsizing (splitting beats): - Minimal latency: 1 cycle (register stage) - First beat output: Immediately (split from wide beat) - Example: 128→64 outputs first 64-bit beat immediately

### 9.9.2	Throughput

Upsizing Throughput:

32→64 upsizing:
  Master: 32 bits/cycle = 4 bytes/cycle
  Crossbar: 64 bits per 2 cycles = 4 bytes/cycle (same)
  No throughput loss

Downsizing Throughput:

128→64 downsizing:
  Master: 128 bits/cycle = 16 bytes/cycle
  Crossbar: 64 bits/cycle = 8 bytes/cycle
  Throughput halved (crossbar becomes bottleneck)

## 9.10	2.6.10 Configuration Parameters

### 9.10.1	Width Conversion Configuration (TOML)

[bridge]
internal_data_width = 64        # Crossbar width
enable_width_conversion = true  # Allow width differences

[[masters]]
name = "cpu_32bit"
data_width = 32                 # Narrower than crossbar (upsizing)
addr_width = 32

[[masters]]
name = "dma_64bit"
data_width = 64                 # Matches crossbar (no conversion)
addr_width = 32

[[masters]]
name = "gpu_128bit"
data_width = 128                # Wider than crossbar (downsizing)
addr_width = 36

[[slaves]]
name = "memory_64bit"
data_width = 64                 # Matches crossbar (no conversion)

[[slaves]]
name = "periph_32bit"
data_width = 32                 # Narrower than crossbar (downsizing)

## 9.11	2.6.11 Debug and Observability

### 9.11.1	Recommended Debug Signals

Upsizer:
- Beat accumulator (partial wide beat being assembled)
- Beat counter (position in burst)
- Buffer valid (accumulated beats ready)
- Strobe accumulation

Downsizer:
- Beat splitter state (which sub-beat currently outputting)
- Remaining beats to output
- Source data register

### 9.11.2	Common Issues and Debug

Symptom: Data corruption after width conversion
Check: - Byte ordering (endianness) - Strobe mapping (correct bytes enabled) - Address alignment - Burst length calculation

Symptom: Stalls after width conversion
Check: - Buffer depths (upsizer needs buffering) - Backpressure propagation - LAST signaling

Symptom: Incorrect burst lengths
Check: - Length calculation formula - Odd burst handling - SIZE field matching width

## 9.12	2.6.12 Verification Considerations

### 9.12.1	Test Scenarios

- Power-of-2 Width Ratios:

- 32→64 (2:1)
- 64→128 (1:2)
- 32→128 (4:1)

- Various Burst Lengths:

- Single beat (ALEN=0)
- Even bursts (ALEN=3, 7, 15)
- Odd bursts (ALEN=1, 5, 9)

- Sub-Word Accesses:

- Byte writes (WSTRB with individual bytes)
- Half-word, word accesses
- Unaligned accesses

- Mixed Widths:

- 32-bit master → 64-bit crossbar → 32-bit slave
- 128-bit master → 64-bit crossbar → 32-bit slave
- All width combinations

## 9.13	2.6.13 Performance Considerations

### 9.13.1	Conversion Overhead

Upsizing Overhead: - Buffering latency: +1-2 cycles - Throughput maintained - Best for burst-oriented masters

Downsizing Overhead: - Split latency: +1 cycle - Throughput reduced by width ratio - Can bottleneck wide masters

### 9.13.2	Optimization Strategies

- Match Common Widths: Design masters/slaves to match internal width (64-bit)

- Burst Sizing: Use appropriate burst lengths for width ratios

- Parallel Paths: Multiple 32-bit masters can aggregate to 64-bit crossbar bandwidth

- Selective Conversion: Only convert where necessary

## 9.14	2.6.14 Future Enhancements

### 9.14.1	Planned Features

- Configurable Internal Width: Support 32, 64, 128, 256-bit crossbar

- Multi-Beat Buffering: Deeper upsizing buffers for smoother flow

- Byte Rotation: Handle unaligned access more efficiently

- Pipelined Conversion: Multi-stage for high frequency

### 9.14.2	Under Consideration

- Asymmetric Widths: Different widths for read vs. write paths

- Sparse Data Optimization: Skip unused bytes in conversions

- Tagged Data: Metadata preservation through width conversion

- Zero-Copy Bypass: Direct routing for matching widths

Related Sections: - Section 2.1: Master Adapter (upsizing location) - Section 2.3: Crossbar Core (internal data width) - Section 3.2: Master Port Interface (width specifications) - Section 3.3: Slave Port Interface (width specifications)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	2.7 Protocol Conversion

Protocol Conversion enables the bridge to interface with slaves using different bus protocols. While the bridge internally uses AXI4, it can convert to simpler protocols like APB (Advanced Peripheral Bus) for low-bandwidth peripheral access.

## 10.1	2.7.1 Purpose and Function

Protocol conversion performs the following critical functions:

- Protocol Translation: Converts AXI4 transactions to target protocol (e.g., APB)

- Handshake Mapping: Translates ready/valid to protocol-specific handshakes

- Burst Decomposition: Breaks AXI bursts into single-beat target transactions

- Response Mapping: Converts protocol-specific responses back to AXI responses

- Timing Adaptation: Handles different timing requirements between protocols

## 10.2	2.7.2 Supported Protocols

### 10.2.1	Current Support (Phase 2)

AXI4 (Native): - Full AXI4 protocol - No conversion required - Maximum performance

AXI4-Lite (Master-Side): - Simplified AXI4 subset - Single-beat transactions only (ARLEN=0, AWLEN=0) - Converts to full AXI4 for crossbar - Common for control/status registers

APB (Slave-Side): - APB3 and APB4 support - For low-bandwidth peripherals - Simplified handshaking

### 10.2.2	Current Limitation: AXI4-Lite Conversion

IMPORTANT: AXI4-Lite slaves are currently treated as full AXI4 slaves internally. The bridge does NOT perform real AXI4-Lite protocol conversion (single-beat restriction, no ARLEN/AWLEN, etc.). AXIL slaves: - Use the standard AXI4 timing wrapper (same as AXI4 slaves) - Expose full 5-channel AXI4 interface at the bridge boundary - Are documented as “axil” for user reference only

Real AXI4-Lite protocol enforcement (burst length restriction, size field handling) is deferred to a future phase. Currently, AXIL slaves behave identically to AXI4 slaves from the bridge’s perspective.

### 10.2.3	Future Support (Phase 2+)

- Real AXI4-Lite Conversion: Enforce single-beat constraint, remove unused signals

- AHB: Advanced High-performance Bus

- Wishbone: Open-source bus standard

- Custom: User-defined protocols

## 10.3	2.7.3 Conversion Architecture Overview

### 10.3.1	Figure 2.7.1: Protocol Conversion Architecture

Protocol Conversion Architecture

The diagram shows the complete protocol conversion flow: AXI4-Lite masters are adapted to full AXI4 before entering the crossbar, while APB peripherals receive AXI4 transactions through a dedicated converter.

## 10.4	2.7.4 Block Diagram

### 10.4.1	Figure 2.7: Protocol Conversion Architecture

Protocol Conversion Architecture

Protocol conversion showing AXI4 to APB conversion with state machine, burst decomposition, and response mapping.

## 10.5	2.7.5 AXI4-Lite to AXI4 Conversion (Master-Side)

### 10.5.1	AXI4-Lite Protocol Overview

AXI4-Lite is a simplified subset of AXI4 designed for simple control/status register access:

Key Differences from Full AXI4:
- FIXED burst length: ARLEN/AWLEN always = 0 (single beat)
- FIXED burst size: No SIZE field, always full data width
- FIXED burst type: No BURST field, always INCR
- NO exclusive access: No LOCK support
- NO unaligned transfers: Address must be aligned
- Simpler ID: Typically 1-4 bits (fewer outstanding transactions)

Similarities to AXI4:
- Same 5 channels: AR, R, AW, W, B
- Same valid/ready handshaking
- Same response codes: OKAY, SLVERR, DECERR, EXOKAY
- Same data widths: 32 or 64 bits typically

### 10.5.2	Conversion Requirements

To adapt AXI4-Lite masters to the full AXI4 crossbar, the adapter must:

- Add Missing Signals: Provide default values for burst-related signals

- Validate Constraints: Ensure single-beat assumption holds

- Pass-Through Simplicity: Most signals connect directly

### 10.5.3	Signal Mapping

// AXI4-Lite to AXI4 Signal Mapping

// AR Channel (Read Address)
// AXI4-Lite Input          AXI4 Crossbar Output
axi4lite_arvalid      →     axi4_arvalid
axi4lite_arready      ←     axi4_arready
axi4lite_araddr       →     axi4_araddr
axi4lite_arprot       →     axi4_arprot
axi4lite_arid (opt)   →     axi4_arid

// Added by adapter (constants):
                            axi4_arlen    = 8'h00      // Always 1 beat
                            axi4_arsize   = log2(DW/8) // Full width
                            axi4_arburst  = 2'b01      // INCR
                            axi4_arlock   = 1'b0       // No lock
                            axi4_arcache  = 4'b0000    // Device non-buf
                            axi4_arqos    = 4'h0       // No QoS
                            axi4_arregion = 4'h0       // Region 0

// R Channel (Read Data)
// AXI4 Crossbar Input      AXI4-Lite Output
axi4_rvalid           →     axi4lite_rvalid
axi4_rready           ←     axi4lite_rready
axi4_rdata            →     axi4lite_rdata
axi4_rresp            →     axi4lite_rresp
axi4_rid (opt)        →     axi4lite_rid (opt)

// Discarded by adapter:
axi4_rlast                  // Always 1 for single beat

// AW Channel (Write Address)
axi4lite_awvalid      →     axi4_awvalid
axi4lite_awready      ←     axi4_awready
axi4lite_awaddr       →     axi4_awaddr
axi4lite_awprot       →     axi4_awprot
axi4lite_awid (opt)   →     axi4_awid

// Added by adapter:
                            axi4_awlen    = 8'h00
                            axi4_awsize   = log2(DW/8)
                            axi4_awburst  = 2'b01
                            axi4_awlock   = 1'b0
                            axi4_awcache  = 4'b0000
                            axi4_awqos    = 4'h0
                            axi4_awregion = 4'h0

// W Channel (Write Data)
axi4lite_wvalid       →     axi4_wvalid
axi4lite_wready       ←     axi4_wready
axi4lite_wdata        →     axi4_wdata
axi4lite_wstrb        →     axi4_wstrb

// Added by adapter:
                            axi4_wlast    = 1'b1       // Always last

// B Channel (Write Response)
axi4_bvalid           →     axi4lite_bvalid
axi4_bready           ←     axi4lite_bready
axi4_bresp            →     axi4lite_bresp
axi4_bid (opt)        →     axi4lite_bid (opt)

### 10.5.4	Implementation

The AXI4-Lite adapter is extremely simple, primarily providing constant values:

// AXI4-Lite to AXI4 Adapter (simplified)
module axi4lite_to_axi4_adapter #(
    parameter ADDR_WIDTH = 32,
    parameter DATA_WIDTH = 64,
    parameter ID_WIDTH = 4         // Optional, often 0 for AXI4-Lite
) (
    input logic clk,
    input logic rst_n,
    
    // AXI4-Lite Master Interface
    input  logic                     lite_arvalid,
    output logic                     lite_arready,
    input  logic [ADDR_WIDTH-1:0]    lite_araddr,
    input  logic [2:0]               lite_arprot,
    input  logic [ID_WIDTH-1:0]      lite_arid,    // Optional
    
    output logic                     lite_rvalid,
    input  logic                     lite_rready,
    output logic [DATA_WIDTH-1:0]    lite_rdata,
    output logic [1:0]               lite_rresp,
    output logic [ID_WIDTH-1:0]      lite_rid,     // Optional
    
    input  logic                     lite_awvalid,
    output logic                     lite_awready,
    input  logic [ADDR_WIDTH-1:0]    lite_awaddr,
    input  logic [2:0]               lite_awprot,
    input  logic [ID_WIDTH-1:0]      lite_awid,    // Optional
    
    input  logic                     lite_wvalid,
    output logic                     lite_wready,
    input  logic [DATA_WIDTH-1:0]    lite_wdata,
    input  logic [DATA_WIDTH/8-1:0]  lite_wstrb,
    
    output logic                     lite_bvalid,
    input  logic                     lite_bready,
    output logic [1:0]               lite_bresp,
    output logic [ID_WIDTH-1:0]      lite_bid,     // Optional
    
    // Full AXI4 Crossbar Interface
    output logic                     axi4_arvalid,
    input  logic                     axi4_arready,
    output logic [ADDR_WIDTH-1:0]    axi4_araddr,
    output logic [7:0]               axi4_arlen,
    output logic [2:0]               axi4_arsize,
    output logic [1:0]               axi4_arburst,
    output logic                     axi4_arlock,
    output logic [3:0]               axi4_arcache,
    output logic [2:0]               axi4_arprot,
    output logic [3:0]               axi4_arqos,
    output logic [3:0]               axi4_arregion,
    output logic [ID_WIDTH-1:0]      axi4_arid,
    
    input  logic                     axi4_rvalid,
    output logic                     axi4_rready,
    input  logic [DATA_WIDTH-1:0]    axi4_rdata,
    input  logic [1:0]               axi4_rresp,
    input  logic                     axi4_rlast,
    input  logic [ID_WIDTH-1:0]      axi4_rid,
    
    // ... (AW, W, B channels similar)
);

    // AR Channel: Pass-through with constants
    assign axi4_arvalid  = lite_arvalid;
    assign lite_arready  = axi4_arready;
    assign axi4_araddr   = lite_araddr;
    assign axi4_arprot   = lite_arprot;
    assign axi4_arid     = lite_arid;
    
    // Constants for single-beat burst
    assign axi4_arlen    = 8'h00;                    // 1 beat
    assign axi4_arsize   = $clog2(DATA_WIDTH/8);     // Full width
    assign axi4_arburst  = 2'b01;                    // INCR
    assign axi4_arlock   = 1'b0;                     // No lock
    assign axi4_arcache  = 4'b0000;                  // Device non-buf
    assign axi4_arqos    = 4'h0;                     // No QoS
    assign axi4_arregion = 4'h0;                     // Region 0
    
    // R Channel: Pass-through, ignore rlast
    assign lite_rvalid = axi4_rvalid;
    assign axi4_rready = lite_rready;
    assign lite_rdata  = axi4_rdata;
    assign lite_rresp  = axi4_rresp;
    assign lite_rid    = axi4_rid;
    // axi4_rlast ignored (always 1 for single beat)
    
    // AW Channel: Similar to AR
    assign axi4_awvalid  = lite_awvalid;
    assign lite_awready  = axi4_awready;
    assign axi4_awaddr   = lite_awaddr;
    assign axi4_awprot   = lite_awprot;
    assign axi4_awid     = lite_awid;
    assign axi4_awlen    = 8'h00;
    assign axi4_awsize   = $clog2(DATA_WIDTH/8);
    assign axi4_awburst  = 2'b01;
    assign axi4_awlock   = 1'b0;
    assign axi4_awcache  = 4'b0000;
    assign axi4_awqos    = 4'h0;
    assign axi4_awregion = 4'h0;
    
    // W Channel: Pass-through, add wlast
    assign axi4_wvalid = lite_wvalid;
    assign lite_wready = axi4_wready;
    assign axi4_wdata  = lite_wdata;
    assign axi4_wstrb  = lite_wstrb;
    assign axi4_wlast  = 1'b1;                       // Always last
    
    // B Channel: Pass-through
    assign lite_bvalid = axi4_bvalid;
    assign axi4_bready = lite_bready;
    assign lite_bresp  = axi4_bresp;
    assign lite_bid    = axi4_bid;

endmodule

### 10.5.5	Resource Utilization

AXI4-Lite Adapter Resources:

Logic Elements:  ~50-100 LEs (minimal, mostly wiring)
Registers:       ~50 regs (if skid buffers added)
Block RAM:       0

Breakdown:
- Signal pass-through: ~20 LEs (buffering)
- Constant generation: ~10 LEs
- Optional skid buffers: ~50 regs (for timing)

Note: Most implementations are purely combinatorial wire
      assignments with optional pipeline registers.

### 10.5.6	Performance Impact

Latency: - Zero-latency (combinatorial) if no pipeline stages - 1-2 cycles if skid buffers added for timing - No protocol conversion overhead

Throughput: - 1 transaction per cycle (same as native AXI4) - No degradation for single-beat transactions - Limited by AXI4-Lite’s single-beat constraint

### 10.5.7	Configuration

[[masters]]
name = "control_processor"
protocol = "axi4lite"          # Specify simplified protocol
channels = "rw"                # Full read-write
arid_width = 0                 # Often no ID in AXI4-Lite
awid_width = 0
addr_width = 32
data_width = 32                # Typically 32 or 64 bits

### 10.5.8	Common Issues and Debug

Issue 1: Burst Detected on AXI4-Lite

Symptom: ARLEN/AWLEN != 0 on AXI4-Lite interface
Cause: Master not properly configured as AXI4-Lite
Check: Verify master only issues single-beat transactions

Issue 2: Unaligned Addresses

Symptom: ARADDR/AWADDR not aligned to data width
Cause: AXI4-Lite requires full-width aligned access
Check: Address[log2(DW/8)-1:0] should be zero

Issue 3: rlast/wlast Handling

Symptom: Master expects rlast/wlast but doesn't have them
Cause: True AXI4-Lite interface omits these signals
Solution: Adapter provides wlast=1 to crossbar, strips rlast

## 10.6	2.7.6 AXI4 to APB Conversion (Slave-Side)

### 10.6.1	APB Protocol Overview

APB is a simple, low-power bus protocol:

Characteristics:
- Single address phase
- Single data phase
- No burst support (one transfer per operation)
- Minimal logic
- Low power consumption
- Suitable for peripherals: UARTs, timers, GPIOs

### 10.6.2	APB Signals

Address Phase:
  PADDR[N-1:0]  - Address bus
  PSEL          - Slave select
  PENABLE       - Enable (2nd cycle of transfer)
  PWRITE        - Write direction (1=write, 0=read)
  
Data Phase (Write):
  PWDATA[N-1:0] - Write data
  PSTRB[N/8-1:0]- Write strobes (APB4 only)
  
Data Phase (Read):
  PRDATA[N-1:0] - Read data
  
Response:
  PREADY        - Slave ready (can extend transfer)
  PSLVERR       - Slave error (APB3+)

### 10.6.3	APB State Machine

APB requires a 2-phase handshake:

IDLE:
  - Wait for AXI request (ARVALID or AWVALID)
  - PSEL = 0, PENABLE = 0
  
SETUP:
  - Assert PSEL = 1
  - Drive PADDR, PWRITE, PWDATA (if write)
  - PENABLE = 0
  - Duration: 1 cycle
  
ACCESS:
  - Assert PENABLE = 1
  - Wait for PREADY = 1
  - Capture PRDATA (if read) or PSLVERR
  - Can extend multiple cycles if PREADY = 0
  
Complete:
  - De-assert PSEL, PENABLE
  - Return to IDLE or SETUP (if more beats)

### 10.6.4	Read Transaction Conversion

AXI4 Read:

Cycle 0: ARVALID=1, ARADDR=0x100, ARLEN=3 (4 beats)
Cycle 1: ARREADY=1
Cycles 2-5: R beats returning

Converted to APB (4 separate APB reads):

Beat 0:
  Cycle 0: PSEL=1, PENABLE=0, PADDR=0x100, PWRITE=0 (SETUP)
  Cycle 1: PSEL=1, PENABLE=1, wait PREADY (ACCESS)
  Cycle 2: PREADY=1, capture PRDATA → First R beat

Beat 1:
  Cycle 3: PSEL=1, PENABLE=0, PADDR=0x104 (SETUP)
  Cycle 4: PSEL=1, PENABLE=1, wait PREADY (ACCESS)
  Cycle 5: PREADY=1, capture PRDATA → Second R beat
  
... (beats 2 and 3 similar)

Latency: 2-3 cycles per beat (SETUP + ACCESS + ready)

### 10.6.5	Write Transaction Conversion

AXI4 Write:

Cycle 0: AWVALID=1, AWADDR=0x200, AWLEN=1 (2 beats)
Cycle 1: AWREADY=1
Cycle 1: WVALID=1, WDATA=0xAAAA_BBBB, WLAST=0
Cycle 2: WREADY=1
Cycle 2: WVALID=1, WDATA=0xCCCC_DDDD, WLAST=1
Cycle 3: WREADY=1
Cycle 4: BVALID=1, BRESP=OKAY

Converted to APB (2 separate APB writes):

Beat 0:
  Cycle 0: PSEL=1, PENABLE=0, PADDR=0x200, PWRITE=1, PWDATA=0xAAAA_BBBB
  Cycle 1: PSEL=1, PENABLE=1, wait PREADY
  Cycle 2: PREADY=1 → First write complete

Beat 1:
  Cycle 3: PSEL=1, PENABLE=0, PADDR=0x204, PWRITE=1, PWDATA=0xCCCC_DDDD
  Cycle 4: PSEL=1, PENABLE=1, wait PREADY
  Cycle 5: PREADY=1 → Second write complete, return B

### 10.6.6	Burst Handling

APB does not support bursts, so:

AXI Burst: AWLEN = 15 (16 beats)
→ 16 separate APB transfers
→ Address increments per AWBURST type:
   - INCR: Addr += SIZE each beat
   - WRAP: Wrapping within boundary
   - FIXED: Same address each beat

### 10.6.7	Response Mapping

APB → AXI Response Translation:

PREADY=1, PSLVERR=0 → RRESP/BRESP = 2'b00 (OKAY)
PREADY=1, PSLVERR=1 → RRESP/BRESP = 2'b10 (SLVERR)

Timeout (PREADY stuck at 0):
  After N cycles → RRESP/BRESP = 2'b11 (DECERR)

## 10.7	2.7.5 Implementation

### 10.7.1	AXI4-to-APB Converter FSM

// Simplified AXI4-to-APB converter
typedef enum logic [2:0] {
    IDLE,
    AR_SETUP,
    AR_ACCESS,
    AW_SETUP,
    W_ACCESS,
    B_RESPONSE
} state_t;

state_t state, next_state;

always_ff @(posedge clk) begin
    if (!rst_n) state <= IDLE;
    else state <= next_state;
end

always_comb begin
    next_state = state;
    
    case (state)
        IDLE: begin
            if (arvalid) next_state = AR_SETUP;
            else if (awvalid) next_state = AW_SETUP;
        end
        
        AR_SETUP: begin
            next_state = AR_ACCESS;  // 1 cycle SETUP
        end
        
        AR_ACCESS: begin
            if (pready) begin
                if (more_beats) next_state = AR_SETUP;  // Next beat
                else next_state = IDLE;
            end
        end
        
        AW_SETUP: begin
            if (wvalid) next_state = W_ACCESS;
        end
        
        W_ACCESS: begin
            if (pready) begin
                if (!wlast) next_state = AW_SETUP;  // Next beat
                else next_state = B_RESPONSE;
            end
        end
        
        B_RESPONSE: begin
            if (bready) next_state = IDLE;
        end
    endcase
end

// APB signal generation
assign psel = (state != IDLE);
assign penable = (state == AR_ACCESS || state == W_ACCESS);
assign pwrite = (state == AW_SETUP || state == W_ACCESS);

### 10.7.2	Address Generation

// Address increment for burst
logic [ADDR_WIDTH-1:0] current_addr;
logic [7:0] beat_count;

always_ff @(posedge clk) begin
    if (state == IDLE) begin
        current_addr <= arvalid ? araddr : awaddr;
        beat_count <= 0;
    end else if ((state == AR_ACCESS || state == W_ACCESS) && pready) begin
        beat_count <= beat_count + 1;
        
        case (burst_type)
            2'b01: current_addr <= current_addr + (1 << size);  // INCR
            2'b10: current_addr <= wrap_address(current_addr);  // WRAP
            2'b00: current_addr <= current_addr;                // FIXED
        endcase
    end
end

assign paddr = current_addr;

## 10.8	2.7.6 Resource Utilization

### 10.8.1	APB Converter Resources

Per APB Slave Interface:

Logic Elements:  ~400 LEs
Registers:       ~150 regs
Block RAM:       0

Breakdown:
- FSM control:           ~100 LEs, ~20 regs
- Address generation:    ~80 LEs, ~40 regs
- Burst counter:         ~50 LEs, ~20 regs
- Response accumulation: ~80 LEs, ~30 regs
- Data path MUX:         ~90 LEs, ~40 regs

### 10.8.2	Scaling

Adding APB slaves: - Linear scaling: +~400 LEs per APB slave - Shared address decoder logic - Independent per-slave FSMs

## 10.9	2.7.7 Timing Characteristics

### 10.9.1	Latency

APB Read Latency (per beat):

Best case: 2 cycles (SETUP + ACCESS with PREADY=1)
Typical: 3-5 cycles (if slave extends with PREADY=0)
Worst case: Configurable timeout (e.g., 1000 cycles)

For 8-beat AXI burst:
  Total: 8 × 3 = 24 cycles typical

APB Write Latency (per beat):

Similar to read: 2-5 cycles per beat

### 10.9.2	Throughput

Severely Limited:

APB: ~0.3-0.5 transactions/cycle (due to 2-3 cycle protocol)
AXI4: 1 transaction/cycle (burst mode)

APB suitable only for low-bandwidth peripherals

## 10.10	2.7.8 Configuration Parameters

### 10.10.1	Protocol Conversion Configuration (TOML)

[[slaves]]
name = "uart_peripheral"
protocol = "apb"            # "axi4", "apb", "ahb" (future)
base_address = 0xF000_0000
size = 0x1000
data_width = 32
apb_timeout = 1000          # Cycles before timeout error

[[slaves]]
name = "ddr_memory"
protocol = "axi4"           # Native, no conversion
base_address = 0x8000_0000
size = 0x4000_0000
data_width = 64

## 10.11	2.7.9 Debug and Observability

### 10.11.1	Recommended Debug Signals

APB Converter:
- FSM state
- APB phase (SETUP, ACCESS)
- Beat counter (progress through burst)
- PREADY timeout counter
- Response accumulation (for burst)

APB Bus:
- PSEL, PENABLE, PWRITE
- PADDR, PWDATA, PRDATA
- PREADY, PSLVERR

### 10.11.2	Common Issues and Debug

Symptom: APB slave not responding (timeout)
Check: - PREADY signal (stuck at 0?) - APB slave clock/reset - PSEL assertion - Timeout threshold

Symptom: Data corruption on APB
Check: - SETUP phase duration (should be 1 cycle) - PENABLE assertion timing - Data sampling on correct cycle

Symptom: Burst to APB takes too long
Check: - Burst length (consider limiting ARLEN/AWLEN) - APB slave response time (PREADY) - Alternative: Use AXI4 slave instead

## 10.12	2.7.10 Verification Considerations

### 10.12.1	Test Scenarios

- Single APB Transfer:

- AXI ARLEN=0 (1 beat) → 1 APB read
- Verify SETUP → ACCESS sequence
- Check PREADY handling

- APB Burst Decomposition:

- AXI AWLEN=7 (8 beats) → 8 APB writes
- Verify address increment
- Check each beat completes before next

- APB PREADY Extension:

- Slave holds PREADY=0 for N cycles
- Verify converter waits
- Check no data corruption

- APB Error Response:

- Slave asserts PSLVERR
- Verify mapped to AXI SLVERR
- Check error propagated to master

- APB Timeout:

- Slave never asserts PREADY
- Verify timeout after N cycles
- Check DECERR response

## 10.13	2.7.11 Performance Considerations

### 10.13.1	When to Use APB

Good Use Cases: - Low-speed peripherals (UART, GPIO, timers) - Infrequent accesses - Simple register interfaces - Power-sensitive designs

Poor Use Cases: - High-bandwidth devices - Burst-intensive masters - Performance-critical paths - Memory interfaces

### 10.13.2	APB vs. AXI4 Comparison

Feature          APB              AXI4
Complexity       Simple           Complex
Throughput       Low (~0.3/cyc)   High (1/cyc burst)
Latency/beat     2-5 cycles       1 cycle
Burst Support    No               Yes (up to 256)
Resources        ~400 LEs         Native (no converter)
Power            Very low         Moderate
Use Case         Peripherals      Memory, DMA

## 10.14	2.7.12 Mixed Protocol Bridges

### 10.14.1	Example Configuration

# Bridge with mixed protocols
[bridge]
num_masters = 2
num_slaves = 3

[[masters]]
name = "cpu"
protocol = "axi4"

[[masters]]
name = "dma"
protocol = "axi4"

[[slaves]]
name = "ddr_memory"
protocol = "axi4"          # High bandwidth

[[slaves]]
name = "sram"
protocol = "axi4"          # Medium bandwidth

[[slaves]]
name = "peripherals"
protocol = "apb"           # Low bandwidth, simple

### 10.14.2	Routing Optimization

CPU → DDR Memory: AXI4-to-AXI4 (native, fast)
CPU → Peripherals: AXI4-to-APB (converted, slower)
DMA → SRAM: AXI4-to-AXI4 (native, fast)
DMA → Peripherals: AXI4-to-APB (rare, acceptable slowdown)

## 10.15	2.7.13 Master-Side vs Slave-Side Conversion

### 10.15.1	Comparison

Feature              Master-Side (AXI4-Lite)    Slave-Side (APB)
──────────────────────────────────────────────────────────────────
Complexity           Very Simple                Complex
Resource Usage       ~50-100 LEs                ~400 LEs
Latency Added        0-1 cycles                 2-5 cycles/beat
Throughput Impact    None                       Severe (3x slower)
Burst Handling       Single beat only           Decompose to singles
State Machine        None (combinatorial)       Multi-state FSM
Buffering Required   Optional (timing)          Essential
Use Case             Control registers          Peripherals

### 10.15.2	When to Use Each

AXI4-Lite Master Adapter: - Simple control/status register interfaces - Low-complexity masters (MCUs, simple CPUs) - Minimal resource overhead acceptable - No burst performance needed

APB Slave Converter: - Legacy peripheral integration - Very simple slave devices (GPIO, timers) - Low-bandwidth acceptable - Power optimization critical

## 10.16	2.7.14 Future Protocol Support

### 10.16.1	Planned Features

AXI4-Lite: - Subset of AXI4 - No burst support (ALEN=0 always) - Simpler than full AXI4 - Common for control registers

AHB (AMBA High-performance Bus): - More capable than APB - Pipeline support - Burst support - Suitable for moderate-bandwidth peripherals

Wishbone: - Open-source bus standard - Common in FPGA designs - Multiple addressing modes - Configurable data widths

### 10.16.2	Under Consideration

- Custom Protocol: User-defined through configuration

- Stream Interface: AXI4-Stream for data streaming

- PCIe TLP: For PCIe endpoint integration

- CHI: ARM’s Coherent Hub Interface

## 10.17	2.7.14 Best Practices

### 10.17.1	Design Recommendations

- Limit APB Burst Lengths: Configure masters to use short bursts to APB slaves

- Proper Timeouts: Set realistic timeout values for APB slaves

- Protocol Matching: Use native AXI4 where possible, APB only when necessary

- Address Map Planning: Group APB peripherals together for efficient decoding

- Width Matching: Match APB data width to peripheral requirements

### 10.17.2	Performance Tips

Inefficient: 256-beat AXI burst → APB
  256 beats × 3 cyc/beat = 768 cycles

Better: Limit to 4-beat bursts → APB
  64 bursts of 4 beats each
  Still long, but more manageable

Best: Use AXI4-Lite slave for registers
  No burst, but native protocol

Related Sections: - Section 2.3: Crossbar Core (protocol integration point) - Section 3.3: Slave Port Interface (APB signal specifications) - Chapter 4: Programming (configuring protocol conversion) - Appendix A: Generator Deep Dive (protocol converter generation)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	2.8 Response Routing

Response Routing is the mechanism by which slave responses (read data and write acknowledgments) are directed back to the originating master. This system uses Bridge IDs, demultiplexing logic, and optional CAM structures to ensure responses reach the correct destination.

## 11.1	2.8.1 Purpose and Function

Response routing performs the following critical functions:

- Response Direction: Routes R and B channel responses to correct master

- ID-Based Routing: Uses extracted Bridge IDs to determine destination

- Multi-Slave Merging: Combines responses from multiple slaves to single master

- Flow Control: Manages backpressure from master on response channels

- Error Propagation: Ensures error responses reach originating master

## 11.2	2.8.2 Block Diagram

### 11.2.1	Figure 2.8: Response Routing Architecture

Response Routing Architecture

Response routing architecture showing B and R channel merging from slaves and ID-based demultiplexing to masters.

## 11.3	2.8.3 Stable Response Routing via FIFO-Tracked Slave Select

### 11.3.1	Architecture: FIFO-Tracked Master Routing

For multi-slave masters, responses are routed based on a stable slave-select value captured at request time, not the current address decode:

// At AW/AR time: Capture slave index
assign aw_slave_select = address_decode(fub_axi_awaddr);
always_ff @(posedge aclk) begin
    if (fub_axi_awvalid && fub_axi_awready) begin
        aw_trk_fifo <= aw_slave_select;  // Capture now
    end
end

// At B time: Use captured slave from FIFO (not current decode)
assign b_slave_select = aw_trk_fifo_out;  // Stable value
logic [NUM_MASTERS-1:0] master_bvalid;
always_comb begin
    master_bvalid = '0;
    master_bvalid[extracted_bid] = slave_bvalid && (b_slave_select == current_slave);
end

Benefits: - B-responses route to correct master even after address reverts - Handles multi-slave masters spanning different widths - Prevents response loss due to stale address decodes

### 11.3.2	BID Extraction

For ID-based routing, Bridge ID is extracted from response:

// Extract Bridge ID from response ID
logic [TOTAL_RID_WIDTH-1:0] slave_rid;     // From slave
logic [BID_WIDTH-1:0] extracted_bid;       // Master index
logic [RID_WIDTH-1:0] external_rid;        // To master

// Upper bits = Bridge ID, lower bits = original ID
assign extracted_bid = slave_rid[TOTAL_RID_WIDTH-1:RID_WIDTH];
assign external_rid = slave_rid[RID_WIDTH-1:0];

// Route response based on BID (combined with slave select tracking above)
logic [NUM_MASTERS-1:0] master_rvalid;
always_comb begin
    master_rvalid = '0;  // Default: no master selected
    master_rvalid[extracted_bid] = slave_rvalid && (r_slave_select == current_slave);
end

### 11.3.3	CAM-Based Extraction

For complex configurations with OOO or ID reordering:

// CAM lookup for response routing
logic [TOTAL_RID_WIDTH-1:0] response_id;
logic [BID_WIDTH-1:0] master_index;
logic cam_hit;

// Search CAM for matching transaction
assign {cam_hit, master_index} = cam_lookup(response_id);

// Route response to found master
if (cam_hit) begin
    master_rvalid[master_index] = slave_rvalid;
end else begin
    // Error: No matching transaction (protocol violation)
    error_response();
end

## 11.4	2.8.4 Response Demultiplexing

### 11.4.1	Per-Master Response Paths

Each master has dedicated response channels from the demultiplexer:

Master 0 Response:
  - M0_RVALID, M0_RREADY, M0_RDATA, M0_RID, M0_RRESP, M0_RLAST
  - M0_BVALID, M0_BREADY, M0_BID, M0_BRESP

Source: Responses from any slave with BID=0

### 11.4.2	Demux Implementation

// Response demultiplexer (4 masters, 3 slaves)
// Combines responses from all slaves, routes to correct master

// Slave responses (multiple sources)
logic s0_rvalid, s1_rvalid, s2_rvalid;
logic [63:0] s0_rdata, s1_rdata, s2_rdata;
logic [5:0] s0_rid, s1_rid, s2_rid;  // Includes BID

// Master responses (multiple destinations)
logic m0_rvalid, m1_rvalid, m2_rvalid, m3_rvalid;
logic [63:0] m0_rdata, m1_rdata, m2_rdata, m3_rdata;
logic [3:0] m0_rid, m1_rid, m2_rid, m3_rid;  // BID stripped

// Demux logic per slave
for (genvar s = 0; s < 3; s++) begin
    logic [1:0] bid;
    assign bid = slave_rid[s][5:4];  // Extract BID
    
    always_comb begin
        case (bid)
            2'b00: begin
                if (slave_rvalid[s] && !m0_busy) begin
                    m0_rvalid = 1'b1;
                    m0_rdata = slave_rdata[s];
                    m0_rid = slave_rid[s][3:0];  // Strip BID
                end
            end
            2'b01: /* Route to M1 */
            2'b10: /* Route to M2 */
            2'b11: /* Route to M3 */
        endcase
    end
end

## 11.5	2.8.5 Multi-Slave Response Merging

### 11.5.1	Problem Statement

When multiple slaves can respond to same master simultaneously:

Scenario:
  Master 0 has outstanding transactions to Slave 0 and Slave 1
  Both slaves return R responses in same cycle
  
Problem: Master 0 can only accept one R response per cycle
Solution: Arbitrate between slave responses

### 11.5.2	Response Arbitration

// Arbitrate between multiple slave responses for same master
logic [2:0] slave_has_response;  // Which slaves have responses for M0
logic [1:0] selected_slave;      // Which slave wins arbitration

// Detect responses destined for M0
for (genvar s = 0; s < 3; s++) begin
    assign slave_has_response[s] = slave_rvalid[s] && 
                                   (extract_bid(slave_rid[s]) == 2'b00);
end

// Round-robin arbiter for fairness
always_ff @(posedge clk) begin
    if (!rst_n) begin
        selected_slave <= 0;
    end else if (m0_rready && (|slave_has_response)) begin
        // Rotate selection for fairness
        if (slave_has_response[selected_slave])
            ; // Keep current
        else
            selected_slave <= find_next_requesting_slave();
    end
end

// MUX selected slave's response to master
assign m0_rvalid = slave_has_response[selected_slave];
assign m0_rdata = slave_rdata[selected_slave];
assign m0_rid = strip_bid(slave_rid[selected_slave]);

### 11.5.3	Response Buffering

Optional: Add FIFOs to buffer pending responses:

Purpose:
  - Prevent response loss during arbitration conflicts
  - Allow slaves to return responses even if master busy
  - Smooth out bursty response patterns

Configuration:
  per_master_response_fifo_depth = 4-16 entries

Trade-off:
  + No response loss
  + Better throughput
  - Additional resources (~200 LEs per FIFO)
  - +1-2 cycle latency

## 11.6	2.8.6 Backpressure Management

### 11.6.1	Ready Signal Routing

Master RREADY/BREADY must reach correct slave:

// Route master ready back to slaves
logic m0_rready;  // From master
logic [2:0] slave_rready;  // To slaves

// Only selected slave sees master's ready
for (genvar s = 0; s < 3; s++) begin
    assign slave_rready[s] = (selected_slave == s) ? m0_rready : 1'b0;
end

// Non-selected slaves see ready = 0 (backpressure)

### 11.6.2	Stall Conditions

Response path can stall when:

1. Master not ready (M_RREADY = 0)
   → Selected slave stalls (S_RREADY = 0)
   → Other slaves buffer or stall

2. Arbitration conflict (multiple slaves ready)
   → Non-selected slaves stalled
   → Selected slave proceeds

3. Response FIFO full
   → Slave stalled until space available

## 11.7	2.8.7 Response Path Latency

### 11.7.1	End-to-End R Channel

Slave R response → Bridge → Master R channel

Components:
1. Slave response valid
2. BID extraction (0 cycles, combinatorial)
3. Demux routing (0-1 cycles)
4. Response arbitration (1 cycle if conflict)
5. Optional FIFO (0-1 cycles)
6. Master adapter (1 cycle, skid buffer)

Total: 2-4 cycles typical

### 11.7.2	End-to-End B Channel

Similar to R channel:
  Slave B → Extraction → Demux → Arbitration → Master B
  Total: 2-4 cycles

## 11.8	2.8.8 Error Response Routing

### 11.8.1	Slave Error Responses

When slave returns error:

RRESP = 2'b10 (SLVERR) or 2'b11 (DECERR)
BRESP = 2'b10 (SLVERR) or 2'b11 (DECERR)

Routing:
  - Extract BID normally
  - Route to originating master
  - Preserve error code
  - Master sees error response

### 11.8.2	Out-of-Range Error Responses

When router generates error for OOR address:

Process:
1. Router captures OOR request
2. Generates internal error response
   - Uses cached request ID (with BID)
   - Sets RRESP/BRESP = DECERR
3. Injects into response path
4. Routes to master using BID

## 11.9	2.8.9 Resource Utilization

### 11.9.1	Response Router Resources

4 masters, 3 slaves (64-bit data):

Logic Elements:  ~1500-2000 LEs
Registers:       ~400-600 regs
Block RAM:       0 (unless FIFOs enabled)

Breakdown:
- BID extraction (3 slaves):     ~150 LEs
- Demux logic (4 masters):        ~600 LEs, ~150 regs
- Response arbitration:           ~300 LEs, ~100 regs
- Backpressure routing:           ~200 LEs, ~50 regs
- Control FSMs:                   ~250 LEs, ~100 regs

Optional FIFOs (4 × 8 entries):  +800 LEs, +2KB BRAM

### 11.9.2	Scaling

Resource usage scales with:
- Number of masters: Linear (one demux path per master)
- Number of slaves: Linear (one extraction per slave)
- Data width: Linear (wider data = wider demux)
- FIFO depth: Linear (deeper = more BRAM)

## 11.10	2.8.10 Configuration Parameters

### 11.10.1	Response Routing Configuration (TOML)

[bridge]
num_masters = 4
num_slaves = 3

[bridge.response_routing]
enable_response_fifos = false       # Buffer responses per master
fifo_depth = 8                      # Entries per FIFO
response_arbiter_type = "round_robin"  # "round_robin", "fixed_priority"
registered_demux = false            # Register demux output (+1 cycle)

# Per-master response credits (optional)
[[masters]]
name = "cpu"
max_response_credits = 16           # Outstanding responses allowed

## 11.11	2.8.11 Debug and Observability

### 11.11.1	Recommended Debug Signals

BID Extraction:
- Slave RID/BID (from each slave)
- Extracted BID (master index)
- External RID/BID (to master, BID stripped)

Response Demux:
- Demux select signals (which master per slave)
- Response valid per master
- Response valid per slave

Arbitration:
- Conflict detection (multiple responses for same master)
- Arbiter grant (which slave wins)
- Stall counters

FIFOs (if enabled):
- FIFO occupancy per master
- FIFO full/empty flags
- FIFO overflow errors

### 11.11.2	Performance Counters

- Responses routed per master
- Arbitration conflicts (cycles with multiple responses to same master)
- Response stall cycles (master not ready)
- Average response latency per master
- FIFO overflow counts
- CAM hits/misses (if CAM enabled)

## 11.12	2.8.12 Common Issues and Debug

Symptom: Response not reaching master
Check: - BID extraction (correct bit slicing?) - Master index (BID → master mapping) - Demux select logic - Backpressure (is master READY?)

Symptom: Response goes to wrong master
Check: - BID values (verify each master has unique BID) - BID width calculation (clog2 correct?) - CAM contents (if used) - ID corruption in transit

Symptom: Response ordering incorrect
Check: - CAM lookup (should preserve order) - Arbitration fairness (round-robin working?) - Multiple outstanding transactions per master

Symptom: Response loss
Check: - FIFO overflows (enable FIFOs if needed) - Dropped responses during arbitration conflicts - Protocol violations (slave not following AXI rules)

## 11.13	2.8.13 Verification Considerations

### 11.13.1	Test Scenarios

- Single Master, Single Slave:

- Basic routing test
- Verify BID extraction and stripping
- Check response reaches correct master

- Multiple Masters, Single Slave:

- Interleaved responses
- Verify each response routes to correct master
- Check BID uniqueness prevents collisions

- Single Master, Multiple Slaves:

- Simultaneous responses from multiple slaves
- Verify arbitration fairness
- Check no response loss

- All Masters, All Slaves:

- Maximum stress test
- All masters issuing to all slaves
- Responses returning in various orders
- Verify correct routing under heavy load

- OOO Responses:

- Issue transactions: ID=1, ID=2, ID=3
- Return responses: ID=3, ID=1, ID=2
- Verify CAM correctly routes each
- Check response order at master

- Backpressure Test:

- Master asserts RREADY=0
- Verify slave stalls (RREADY=0 propagated)
- Master asserts RREADY=1
- Verify responses flow

## 11.14	2.8.14 Performance Optimization

### 11.14.1	Techniques

1. Registered Demux:

Trade-off: +1 cycle latency for timing closure
Best for: High-frequency designs, many masters

2. Response FIFOs:

Trade-off: +2KB BRAM, +1-2 cycle latency
Benefit: Prevent response loss, smooth conflicts
Best for: High-throughput systems

3. Distributed Arbitration:

Implementation: Per-master arbiters instead of global
Benefit: Reduced logic depth, better timing
Best for: >8 masters

4. Priority Response Routing:

Implementation: High-priority masters get preference in conflicts
Benefit: Guaranteed low latency for critical masters
Best for: Real-time systems

## 11.15	2.8.15 Advanced Features

### 11.15.1	Response Reordering (Future)

Allow bridge to reorder responses for efficiency:

Scenario:
  Master issues: Req A (slow slave), Req B (fast slave)
  Fast slave responds first
  Bridge can deliver B before A (if IDs different)

Benefit: Reduced latency for fast responses
Requirement: CAM to track outstanding in-order constraints

### 11.15.2	Response Coalescing (Future)

Combine multiple responses into fewer beats:

Scenario:
  Multiple single-beat reads to narrow slave
  Bridge coaleses into fewer wide beats
  
Benefit: Reduced overhead
Complexity: High (requires buffering and alignment)

### 11.15.3	Response QoS (Future)

Prioritize responses based on master QoS:

Feature: High-QoS masters get response priority
Implementation: Weighted arbitration in response path
Use case: RT masters need bounded response latency

## 11.16	2.8.16 Timing Considerations

### 11.16.1	Critical Paths

Common critical paths in response routing:

1. BID extraction → Demux select → Master RDATA
   Depth: ~8-12 logic levels
   
2. Slave RVALID → Arbitration → Master RVALID
   Depth: ~6-10 logic levels
   
3. Master RREADY → Backpressure → Slave RREADY
   Depth: ~5-8 logic levels

### 11.16.2	Optimization Strategies

- Register demux outputs (+1 cycle, breaks path)
- Pipeline arbitration (+1-2 cycles, multi-stage)
- Use CAM for complex ID scenarios (parallel lookup)
- Limit response buffering depth (less MUX levels)

## 11.17	2.8.17 Future Enhancements

### 11.17.1	Planned Features

- Dynamic Response Buffering: Adjust FIFO depth based on utilization

- Response Ordering Hints: Masters specify ordering requirements

- Multi-Cycle Arbitration: Pipelined for >16 masters

- Response Compression: Pack narrow responses into wide beats

### 11.17.2	Under Consideration

- Response Speculation: Predictive routing before full ID available

- Virtual Response Channels: Separate paths for different QoS classes

- Response Mirroring: Duplicate responses for redundancy

- Cross-Clock Response: Async response paths with CDC

Related Sections: - Section 2.3: Crossbar Core (response path architecture) - Section 2.5: ID Management (BID extraction details) - Section 2.4: Arbitration (response arbitration algorithms) - Section 3.2: Master Port Interface (response channel signals)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	2.9 Error Handling

Error Handling encompasses all mechanisms by which the bridge detects, reports, and recovers from error conditions. This includes protocol violations, out-of-range addresses, timeout conditions, and configuration errors.

## 12.1	2.9.1 Purpose and Function

Error handling performs the following critical functions:

- Error Detection: Identifies violations and abnormal conditions

- Error Response Generation: Creates appropriate AXI error responses

- Error Reporting: Logs and signals errors for debug

- Graceful Degradation: Maintains system stability during errors

- Recovery Support: Enables system recovery after errors

## 12.2	2.9.2 Error Categories

### 12.2.1	Transaction Errors

Out-of-Range (OOR) Addresses:

Master requests address not mapped to any slave
Response: DECERR (Decode Error)
Action: Bridge generates error response internally

Protocol Violations:

Master violates AXI4 protocol rules
Examples:
  - VALID deasserted while READY=0
  - Incorrect burst parameters
  - ID width mismatches
Response: Configurable (error log, SLVERR, or ignore)

Slave Errors:

Slave returns SLVERR or DECERR response
Action: Bridge forwards error to master unchanged

### 12.2.2	System Errors

Timeout Conditions:

Slave fails to respond within configured timeout
Response: DECERR after timeout expires
Action: Force transaction completion

CAM Overflow:

Too many outstanding transactions (CAM full)
Response: Backpressure master (ARREADY/AWREADY=0)
Action: Wait for responses to free entries

Configuration Errors:

Invalid bridge configuration detected at generation time
Examples:
  - Overlapping slave address ranges
  - Invalid data width combinations
  - Illegal parameter values
Response: Generation error, fail early

## 12.3	2.9.3 Block Diagram

### 12.3.1	Figure 2.9: Error Handling Architecture

Error Handling System

Error handling system showing detection (OOR, protocol, timeout, CAM), generation, and logging subsystems.

## 12.4	2.9.4 Out-of-Range Address Handling

### 12.4.1	Detection

Router identifies OOR when no slave address range matches:

// Address decode with OOR detection
logic [NUM_SLAVES-1:0] slave_match;
logic oor_detected;

for (genvar i = 0; i < NUM_SLAVES; i++) begin
    assign slave_match[i] = (addr >= SLAVE_BASE[i]) && 
                            (addr <= SLAVE_END[i]);
end

assign oor_detected = ~(|slave_match) && ~default_slave_en;

### 12.4.2	Error Response Generation

Read OOR Response:

// Generate R response for OOR read
always_ff @(posedge clk) begin
    if (ar_oor_detected && arvalid && arready) begin
        // Capture transaction details
        oor_arid <= arid;
        oor_arlen <= arlen;
        oor_burst_count <= 0;
        oor_generating <= 1'b1;
    end else if (oor_generating) begin
        // Generate R beats
        rvalid <= 1'b1;
        rdata <= OOR_DATA_PATTERN;  // Configurable pattern
        rid <= oor_arid;
        rresp <= 2'b11;  // DECERR
        rlast <= (oor_burst_count == oor_arlen);
        
        if (rready) begin
            oor_burst_count <= oor_burst_count + 1;
            if (rlast) oor_generating <= 1'b0;
        end
    end
end

Write OOR Response:

// Generate B response for OOR write
always_ff @(posedge clk) begin
    if (aw_oor_detected && awvalid && awready) begin
        // Capture write ID
        oor_awid <= awid;
        oor_w_count <= 0;
        oor_sinking_w <= 1'b1;
    end else if (oor_sinking_w) begin
        // Sink W data (accept and discard)
        wready <= 1'b1;
        
        if (wvalid && wlast) begin
            oor_sinking_w <= 1'b0;
            oor_gen_bresp <= 1'b1;
        end
    end else if (oor_gen_bresp) begin
        // Generate B response
        bvalid <= 1'b1;
        bid <= oor_awid;
        bresp <= 2'b11;  // DECERR
        
        if (bready) begin
            oor_gen_bresp <= 1'b0;
        end
    end
end

### 12.4.3	Configurable Data Patterns

[bridge.error_handling]
oor_read_data_pattern = 0xDEADCAFE  # Pattern for OOR reads
oor_response_latency = 2            # Cycles to generate response

## 12.5	2.9.5 Protocol Violation Handling

### 12.5.1	Common Violations

VALID Dropped While READY=0:

AXI4 Rule: Once VALID asserted, must remain until READY
Violation: Master deasserts VALID before handshake
Detection: Track VALID history per channel
Action: Log error, optionally assert error signal

Incorrect Burst Parameters:

Violations:
  - SIZE > DATA_WIDTH
  - LEN > 255
  - Address not aligned to SIZE
  - Burst crosses 4KB boundary
Detection: Parameter checking logic
Action: SLVERR response or reject transaction

ID Width Mismatch:

Violation: Master uses ID wider than configured
Detection: Check ID values against expected width
Action: Truncate or error (configurable)

### 12.5.2	Protocol Checker Implementation

// AXI4 protocol checker (simplified)
module axi_protocol_checker #(
    parameter ID_WIDTH = 4,
    parameter ADDR_WIDTH = 32,
    parameter DATA_WIDTH = 64
) (
    input logic clk, rst_n,
    // AXI signals
    input logic arvalid, arready,
    input logic [ADDR_WIDTH-1:0] araddr,
    input logic [7:0] arlen,
    input logic [2:0] arsize,
    // Error outputs
    output logic protocol_error,
    output logic [7:0] error_code
);

    // Check: VALID held until READY
    logic arvalid_prev;
    always_ff @(posedge clk) begin
        arvalid_prev <= arvalid;
        if (arvalid_prev && !arvalid && !arready) begin
            protocol_error <= 1'b1;
            error_code <= 8'h01;  // VALID_DROPPED
        end
    end
    
    // Check: SIZE <= DATA_WIDTH
    always_comb begin
        if (arvalid && (arsize > $clog2(DATA_WIDTH/8))) begin
            protocol_error = 1'b1;
            error_code = 8'h02;  // INVALID_SIZE
        end
    end
    
    // Check: Address alignment
    always_comb begin
        if (arvalid && (araddr[arsize-1:0] != 0)) begin
            protocol_error = 1'b1;
            error_code = 8'h03;  // UNALIGNED_ADDR
        end
    end
    
    // Additional checks...
endmodule

## 12.6	2.9.6 Timeout Detection

### 12.6.1	Per-Transaction Watchdog

// Timeout detector for outstanding transactions
typedef struct packed {
    logic valid;
    logic [TOTAL_ID_WIDTH-1:0] id;
    logic [15:0] timestamp;
    logic is_read;  // 1=read, 0=write
} timeout_entry_t;

timeout_entry_t watchdog [0:15];  // Track up to 16 transactions
logic [15:0] global_timer;

always_ff @(posedge clk) begin
    if (!rst_n) begin
        global_timer <= 0;
    end else begin
        global_timer <= global_timer + 1;
        
        // Check for timeouts
        for (int i = 0; i < 16; i++) begin
            if (watchdog[i].valid) begin
                logic [15:0] elapsed;
                elapsed = global_timer - watchdog[i].timestamp;
                
                if (elapsed > TIMEOUT_THRESHOLD) begin
                    // Timeout detected
                    timeout_error <= 1'b1;
                    timeout_id <= watchdog[i].id;
                    timeout_type <= watchdog[i].is_read ? "READ" : "WRITE";
                    
                    // Force completion with DECERR
                    force_error_response(watchdog[i]);
                    watchdog[i].valid <= 1'b0;
                end
            end
        end
    end
end

### 12.6.2	Timeout Configuration

[bridge.error_handling]
enable_timeout = true
timeout_cycles = 10000         # Max cycles before timeout
timeout_action = "decerr"      # "decerr", "slverr", or "hang"
per_slave_timeout = [          # Optional per-slave overrides
    {name = "slow_periph", timeout = 50000},
    {name = "fast_mem", timeout = 1000}
]

## 12.7	2.9.7 Error Logging and Reporting

### 12.7.1	Error Status Register

Error Status Register (Read/Clear):
┌──────┬──────────────────────────────────────┐
│ Bits │ Description                          │
├──────┼──────────────────────────────────────┤
│ 0    │ OOR Read Error                       │
│ 1    │ OOR Write Error                      │
│ 2    │ Protocol Violation                   │
│ 3    │ Timeout Error                        │
│ 4    │ CAM Overflow                         │
│ 5    │ Slave Error (SLVERR received)        │
│ 6    │ Decode Error (DECERR received)       │
│ 7    │ ID Match Error                       │
│15:8  │ Reserved                             │
│31:16 │ Error Count (saturating)             │
└──────┴──────────────────────────────────────┘

### 12.7.2	Error History Buffer

// Circular buffer for error history
typedef struct packed {
    logic [7:0] error_type;
    logic [31:0] address;
    logic [TOTAL_ID_WIDTH-1:0] id;
    logic [31:0] timestamp;
    logic [7:0] master_index;
    logic [7:0] slave_index;
} error_entry_t;

error_entry_t error_log [0:15];  // 16-entry history
logic [3:0] error_wr_ptr;

always_ff @(posedge clk) begin
    if (error_detected) begin
        error_log[error_wr_ptr] <= {
            error_type,
            error_addr,
            error_id,
            global_timer,
            error_master,
            error_slave
        };
        error_wr_ptr <= error_wr_ptr + 1;
    end
end

### 12.7.3	Error Interrupts

Optional interrupt generation:

Interrupt Enable Register:
  - OOR_INT_EN
  - TIMEOUT_INT_EN
  - PROTOCOL_INT_EN
  - etc.

Interrupt when enabled error occurs
Clear on status register read or explicit clear

## 12.8	2.9.8 Resource Utilization

### 12.8.1	Error Handling Resources

Logic Elements:  ~800-1200 LEs
Registers:       ~400-600 regs
Block RAM:       0-2 KB (if error logging enabled)

Breakdown:
- OOR detection/response:     ~300 LEs, ~100 regs
- Protocol checkers:           ~250 LEs, ~50 regs
- Timeout watchers:            ~200 LEs, ~150 regs
- Error logging:               ~150 LEs, ~100 regs
- Status registers:            ~100 LEs, ~50 regs

Optional error log (16 entries): +2KB BRAM

## 12.9	2.9.9 Configuration Parameters

### 12.9.1	Error Handling Configuration (TOML)

[bridge.error_handling]
# OOR handling
enable_oor_errors = true
oor_response_type = "decerr"      # "decerr", "slverr"
oor_read_pattern = 0xDEADCAFE     # Data pattern for OOR reads

# Timeout detection
enable_timeout = true
timeout_cycles = 10000
timeout_response = "decerr"

# Protocol checking
enable_protocol_checker = true
protocol_checker_mode = "log"     # "log", "error", "ignore"

# Error logging
enable_error_log = true
error_log_depth = 16              # Entries in circular buffer

# Error reporting
enable_error_interrupts = false
error_status_register_addr = 0xFFFF_FFF0  # Optional mapped register

## 12.10	2.9.10 Debug and Observability

### 12.10.1	Recommended Debug Signals

Error Detection:
- OOR flags (per master)
- Protocol violation flags
- Timeout counters
- CAM occupancy

Error Response:
- Error response generation FSM states
- Active error responses
- Error data patterns

Error Logging:
- Error count
- Last error type
- Last error address
- Error log write pointer

## 12.11	2.9.11 Common Issues and Debug

Symptom: Unexpected DECERR responses
Check: - Address map configuration (gaps?) - Slave address ranges (overlaps?) - Default slave configuration - Timeout thresholds (too short?)

Symptom: System hangs on certain addresses
Check: - Timeout detection enabled? - Slave responsiveness - Protocol violations upstream - CAM overflow

Symptom: Error log filling quickly
Check: - What errors are occurring? - Master behavior (bad addresses?) - Slave configuration - Timeout thresholds

## 12.12	2.9.12 Verification Considerations

### 12.12.1	Test Scenarios

- OOR Address Tests:

- Read from unmapped address
- Write to unmapped address
- Burst to partially mapped range
- Verify DECERR response

- Timeout Tests:

- Slave never responds
- Verify timeout after N cycles
- Check error logged
- Verify forced completion

- Protocol Violation Tests:

- Drop VALID before READY
- Invalid burst parameters
- Misaligned addresses
- Verify detection and response

- Error Recovery Tests:

- Error occurs
- System continues operating
- Subsequent transactions succeed
- Error status readable

- Error Logging Tests:

- Generate multiple errors
- Verify all logged
- Check circular buffer wrap
- Verify timestamps

## 12.13	2.9.13 Recovery Mechanisms

### 12.13.1	Automatic Recovery

OOR Errors:
  - Generate error response
  - Continue normal operation
  - No intervention needed

Timeout Errors:
  - Force transaction completion
  - Free CAM entry
  - Allow new transactions

Protocol Violations:
  - Log error
  - Complete/reject transaction  
  - Continue with next transaction

### 12.13.2	Manual Recovery

CAM Stuck:
  - Software reset via control register
  - Clear CAM entries
  - Restart affected masters

Persistent Errors:
  - Read error log
  - Identify root cause
  - Reconfigure or reset subsystem

## 12.14	2.9.14 Safety and Reliability

### 12.14.1	Error Containment

Goal: Prevent error propagation

Mechanisms:
- Isolate error to originating master
- Don't corrupt other masters' transactions
- Maintain crossbar integrity
- Log for post-mortem analysis

### 12.14.2	Fail-Safe Behavior

On Critical Error:
1. Generate safe error response (DECERR)
2. Log error details
3. Assert error signal (optional)
4. Continue operation if possible
5. Halt only if configured (safety-critical mode)

### 12.14.3	Error Injection (Debug/Test)

[bridge.debug]
enable_error_injection = true

[[bridge.debug.error_injection]]
type = "oor"
trigger_address = 0xBAD_ADDR
action = "decerr"

[[bridge.debug.error_injection]]
type = "timeout"
trigger_after_n_cycles = 100
target_slave = 2

## 12.15	2.9.15 Future Enhancements

### 12.15.1	Planned Features

- Error Recovery FSM: Automatic recovery from certain error conditions

- Error Rate Limiting: Prevent error storm from misbehaving master

- Detailed Error Telemetry: Capture full transaction context on error

- Error Prediction: Detect patterns indicating impending failures

### 12.15.2	Under Consideration

- ECC Protection: Error correction for internal state

- Redundant Checking: Duplicate checkers for safety-critical

- Watchdog Timers: System-level health monitoring

- Error Severity Levels: Classify errors by impact

Related Sections: - Section 2.2: Slave Router (OOR detection) - Section 2.5: ID Management (CAM overflow) - Section 2.8: Response Routing (error response paths) - Chapter 5: Verification (error testing strategies)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### 12.15.3	Bridge Arbiter Finite State Machines

#### 12.15.3.1	Overview

The Bridge AXI4 crossbar implements independent per-slave arbitration using simple 2-state finite state machines (FSMs). Each slave has two dedicated arbiters:

- AW Arbiter - Write Address channel arbitration

- AR Arbiter - Read Address channel arbitration

Total FSM Count: 2 × NUM_SLAVES

These arbiters provide fair, round-robin access to slave resources, preventing master starvation while maintaining simple, predictable behavior.

#### 12.15.3.2	AW Channel Arbiter FSM

Purpose: Arbitrate write address requests from multiple masters to a single slave

States: 2 (IDLE, GRANT_ACTIVE)

Algorithm: Round-robin with grant locking

### 12.15.4	Figure 5.3: AW Arbiter FSM

AW Arbiter FSM

State Descriptions:

IDLE State: - Entry Actions: - aw_grant_matrix[s] = 0 - No grant issued - aw_grant_active[s] = 0 - Arbiter inactive - Behavior: - Monitor aw_request_matrix[s] for master requests - Track round-robin priority via aw_last_grant[s] - Remain in IDLE until at least one master requests access

GRANT_ACTIVE State: - Entry Actions: - aw_grant_matrix[s][m] = 1 - Issue grant to winning master - aw_grant_active[s] = 1 - Mark arbiter as active - Behavior: - Hold grant until AW handshake completes (AWVALID && AWREADY) - W channel data multiplexing follows AW grant - B channel response uses ID table (not grant-based)

State Transitions:

| From | To | Condition | Description |
| --- | --- | --- | --- |
| IDLE | GRANT_ACTIVE | \¦aw_request_matrix[s]\¦ > 0 | At least one master requesting access |
| GRANT_ACTIVE | IDLE | m_axi_awvalid[s] && m_axi_awready[s] | AW handshake complete |

#### 12.15.4.1	AR Channel Arbiter FSM

Purpose: Arbitrate read address requests from multiple masters to a single slave

States: 2 (IDLE, GRANT_ACTIVE)

Algorithm: Round-robin with grant locking (same as AW)

### 12.15.5	Figure 5.4: AR Arbiter FSM

AR Arbiter FSM

State Descriptions:

IDLE State: - Entry Actions: - ar_grant_matrix[s] = 0 - No grant issued - ar_grant_active[s] = 0 - Arbiter inactive - Behavior: - Monitor ar_request_matrix[s] for master requests - Track round-robin priority via ar_last_grant[s] - Independent from AW arbiter state

GRANT_ACTIVE State: - Entry Actions: - ar_grant_matrix[s][m] = 1 - Issue grant to winning master - ar_grant_active[s] = 1 - Mark arbiter as active - Behavior: - Hold grant until AR handshake completes (ARVALID && ARREADY) - R channel data multiplexing follows AR grant - R channel response uses ID table for routing

State Transitions:

| From | To | Condition | Description |
| --- | --- | --- | --- |
| IDLE | GRANT_ACTIVE | \¦ar_request_matrix[s]\¦ > 0 | At least one master requesting read |
| GRANT_ACTIVE | IDLE | m_axi_arvalid[s] && m_axi_arready[s] | AR handshake complete |

#### 12.15.5.1	Round-Robin Arbitration Algorithm

Algorithm Description:

Both AW and AR arbiters use the same fair round-robin arbitration algorithm:

Pseudocode:
1. Start search from (last_grant[s] + 1) % NUM_MASTERS
2. Search cyclically through all masters
3. Select first master with pending request
4. Update last_grant[s] = winning_master
5. Issue grant

Example with 4 Masters:

Initial state: last_grant[0] = 0

Request Pattern:
  Master 0: request
  Master 1: request
  Master 2: no request
  Master 3: request

Arbitration Sequence:
  Grant 1: Master 1 (start from master 1, first requester)
          last_grant = 1

  Grant 2: Master 3 (start from master 2, find master 3)
          last_grant = 3

  Grant 3: Master 0 (start from master 0, first requester)
          last_grant = 0

  Grant 4: Master 1 (start from master 1, first requester)
          last_grant = 1

Fairness Properties:

- No Starvation - Every requesting master will eventually receive grant

- Priority Rotation - Priority shifts after each grant

- Predictable Latency - Maximum wait time = (NUM_MASTERS - 1) × grant_duration

- Equal Opportunity - All masters treated equally

#### 12.15.5.2	Grant Locking Mechanism

Purpose: Prevent grant changes mid-transaction

AW Grant Locking:

// AW grant locked until address handshake completes
always_ff @(posedge aclk or negedge aresetn) begin
    if (!aresetn) begin
        aw_grant_active[s] <= 1'b0;
        aw_grant_matrix[s] <= '0;
    end else begin
        if (aw_grant_active[s]) begin
            // GRANT_ACTIVE state: Hold grant until handshake
            if (m_axi_awvalid[s] && m_axi_awready[s]) begin
                aw_grant_active[s] <= 1'b0;  // Return to IDLE
                aw_grant_matrix[s] <= '0;
            end
        end else if (|aw_request_matrix[s]) begin
            // IDLE state: Arbitrate if requests pending
            aw_grant_matrix[s] <= round_robin_select(aw_request_matrix[s]);
            aw_grant_active[s] <= 1'b1;  // Enter GRANT_ACTIVE
        end
    end
end

AR Grant Locking:

// AR grant locked until address handshake completes
always_ff @(posedge aclk or negedge aresetn) begin
    if (!aresetn) begin
        ar_grant_active[s] <= 1'b0;
        ar_grant_matrix[s] <= '0;
    end else begin
        if (ar_grant_active[s]) begin
            // GRANT_ACTIVE state: Hold grant until handshake
            if (m_axi_arvalid[s] && m_axi_arready[s]) begin
                ar_grant_active[s] <= 1'b0;  // Return to IDLE
                ar_grant_matrix[s] <= '0;
            end
        end else if (|ar_request_matrix[s]) begin
            // IDLE state: Arbitrate if requests pending
            ar_grant_matrix[s] <= round_robin_select(ar_request_matrix[s]);
            ar_grant_active[s] <= 1'b1;  // Enter GRANT_ACTIVE
        end
    end
end

Why Grant Locking Matters:

- Protocol Compliance - AXI4 spec requires stable AWID/ARID during handshake

- Data Integrity - W channel must follow granted master’s AW request

- Response Routing - B/R channels need consistent transaction tracking

- Simplicity - Single grant active per slave prevents mux conflicts

#### 12.15.5.3	Independent Read/Write Arbitration

Key Design Feature: AW and AR arbiters operate completely independently

Benefits:

- No Head-of-Line Blocking:

  - Read requests don’t wait for write completion

  - Write requests don’t wait for read completion

- Parallel Operation:

  - Master 0 can write to Slave 0 (AW path)

  - Master 1 can read from Slave 0 (AR path)

  - Both transactions proceed simultaneously

- Resource Efficiency:

  - Full utilization of read/write bandwidth

  - No artificial serialization

Example Scenario:

Time T0: Master 0 issues write to Slave 0
         - AW arbiter grants to Master 0
         - AW_GRANT_ACTIVE = 1
         - W data follows

Time T1: Master 1 issues read to Slave 0 (during Master 0 write)
         - AR arbiter grants to Master 1 (independent!)
         - AR_GRANT_ACTIVE = 1
         - R data returns

Result: Read and write happen in parallel - no blocking

#### 12.15.5.4	FSM Instance Breakdown by Configuration

2×2 Configuration (2 masters, 2 slaves): - Slave 0 AW Arbiter: 1 FSM - Slave 0 AR Arbiter: 1 FSM - Slave 1 AW Arbiter: 1 FSM - Slave 1 AR Arbiter: 1 FSM - Total: 4 FSMs

4×4 Configuration (4 masters, 4 slaves): - 4 slaves × 2 arbiters per slave = 8 FSMs

8×8 Configuration (8 masters, 8 slaves): - 8 slaves × 2 arbiters per slave = 16 FSMs

Scalability: - FSM count scales linearly with NUM_SLAVES - Arbitration complexity scales with NUM_MASTERS (search time) - Synthesis impact minimal for up to 16×16 configurations

#### 12.15.5.5	Performance Characteristics

Latency:

- Best Case: 1 clock cycle (IDLE → GRANT_ACTIVE → IDLE with immediate handshake)

- Worst Case: (NUM_MASTERS) clock cycles (round-robin search + contention)

- Average Case: ~(NUM_MASTERS / 2) clock cycles

Throughput:

- Back-to-Back Grants: Possible if different masters (no wait)

- Same Master Repeated: 1 cycle minimum between grants (FSM state change)

Fairness:

- Guaranteed: Every master gets grant within (NUM_MASTERS - 1) arbitration cycles

- No Priority: All masters treated equally (can be extended for QoS)

#### 12.15.5.6	Comparison with Other Crossbar Arbiters

| Feature | Bridge Arbiter | APB Crossbar | Commercial Tools |
| --- | --- | --- | --- |
| States | 2 (IDLE, GRANT_ACTIVE) | 1 (passthrough) | 3-5 (complex) |
| Algorithm | Round-robin | N/A (APB is 1:1) | Priority, QoS, weighted |
| Grant Locking | Yes (until handshake) | N/A | Burst-aware locking |
| Read/Write Indep | Yes (2 FSMs/slave) | N/A | Yes |
| Complexity | Low | Minimal | High |
| Predictability | High (round-robin) | N/A | Medium (priority-based) |

Simplicity Trade-off:

- Advantages:

  - Easy to verify (2-state FSM)

  - Predictable latency

  - Fair resource allocation

- Limitations (future enhancements):

  - No Quality-of-Service (QoS) support

  - No priority levels

  - No weighted arbitration

#### 12.15.5.7	Future Enhancements (Phase 3+)

QoS Support:

// Master QoS priority field (AXI4 optional)
input logic [3:0] s_axi_awqos [NUM_MASTERS];

// Arbitration considers QoS priority
function automatic [NUM_MASTERS-1:0] qos_arbitrate(
    input logic [NUM_MASTERS-1:0] requests,
    input logic [3:0] qos [NUM_MASTERS]
);
    // Higher QoS value = higher priority
    // Round-robin among same QoS level
endfunction

Weighted Arbitration:

// Master weight configuration
parameter int MASTER_WEIGHTS [NUM_MASTERS] = '{1, 2, 1, 4};

// Grant frequency proportional to weight
// Master 3 gets 4× more grants than Master 0

Burst-Aware Locking:

// Lock grant until entire burst completes
// Currently: Lock until address handshake
// Enhanced: Lock until WLAST (write) or RLAST (read)

See Also: - 1.1 - Introduction - Bridge overview - 3.1 - Module Structure - Generated RTL organization - 3.3 - Crossbar Core - Internal crossbar instantiation

Reference: - ARM AMBA AXI4 Specification (IHI 0022E) - Section A7 (Arbitration)

Next: 3.3 - Crossbar Core

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	Transaction Tracking FSMs

## 13.1	Overview

Bridge uses several FSMs to track outstanding transactions and ensure correct response routing. These FSMs work in conjunction with the ID management CAM to maintain transaction state.

## 13.2	Write Data Tracking FSM

### 13.2.1	Purpose

Track W channel data beats to ensure correct routing after AW handshake.

### 13.2.2	States

| State | Description |
| --- | --- |
| IDLE | Waiting for AW handshake |
| WRITING | Routing W beats to target slave |

### 13.2.3	Figure 3.1: Write Data Tracking FSM

Write Data Tracking FSM

### 13.2.4	Implementation

typedef enum logic [0:0] {
    W_IDLE    = 1'b0,
    W_WRITING = 1'b1
} w_state_t;

w_state_t r_w_state;
logic [$clog2(NUM_SLAVES)-1:0] r_w_target;

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_w_state <= W_IDLE;
        r_w_target <= '0;
    end else begin
        case (r_w_state)
            W_IDLE: begin
                if (awvalid && awready) begin
                    r_w_state <= W_WRITING;
                    r_w_target <= decoded_slave;
                end
            end
            W_WRITING: begin
                if (wvalid && wready && wlast) begin
                    r_w_state <= W_IDLE;
                end
            end
        endcase
    end
end

## 13.3	Response Pending Counter

### 13.3.1	Purpose

Track number of outstanding transactions per master for flow control.

### 13.3.2	Implementation

logic [7:0] r_outstanding [NUM_MASTERS];

// Increment on AR/AW acceptance
always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        for (int i = 0; i < NUM_MASTERS; i++)
            r_outstanding[i] <= '0;
    end else begin
        for (int m = 0; m < NUM_MASTERS; m++) begin
            logic inc = (ar_accepted[m] || aw_accepted[m]);
            logic dec = (r_delivered[m] || b_delivered[m]);

            case ({inc, dec})
                2'b10: r_outstanding[m] <= r_outstanding[m] + 1;
                2'b01: r_outstanding[m] <= r_outstanding[m] - 1;
                default: r_outstanding[m] <= r_outstanding[m];
            endcase
        end
    end
end

// Backpressure when outstanding limit reached
assign accept_ar[m] = (r_outstanding[m] < MAX_OUTSTANDING);
assign accept_aw[m] = (r_outstanding[m] < MAX_OUTSTANDING);

## 13.4	Burst Counter FSM

### 13.4.1	Purpose

Track burst beat count for responses spanning multiple cycles.

### 13.4.2	States

| State | Description |
| --- | --- |
| IDLE | No burst in progress |
| COUNTING | Tracking burst beats |

### 13.4.3	Implementation

logic [7:0] r_burst_count;
logic r_burst_active;

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_burst_count <= '0;
        r_burst_active <= 1'b0;
    end else begin
        if (!r_burst_active && rvalid && rready) begin
            // Start new burst
            r_burst_active <= !rlast;
            r_burst_count <= rlast ? '0 : 8'd1;
        end else if (r_burst_active && rvalid && rready) begin
            // Continue burst
            r_burst_count <= r_burst_count + 1;
            r_burst_active <= !rlast;
        end
    end
end

## 13.5	Related Documentation

- Arbiter FSMs - Address channel arbitration

- ID Management - CAM for response tracking

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	CAM Architecture

## 14.1	Overview

Bridge uses Content-Addressable Memory (CAM) structures to track outstanding transactions and enable out-of-order response routing. The CAM allows fast lookup of master origin based on transaction ID.

## 14.2	CAM Purpose

### 14.2.1	Transaction Tracking

The CAM stores: - Transaction ID (key) - Originating master index (value) - Transaction metadata (optional)

### 14.2.2	Response Routing

When a response arrives: 1. Extract transaction ID from response 2. CAM lookup returns originating master 3. Route response to correct master

## 14.3	CAM Structure

### 14.3.1	Figure 4.1: CAM Entry Format

CAM Entry Format

### 14.3.2	CAM Sizing

CAM Depth = MAX_OUTSTANDING × NUM_MASTERS
CAM Width = TOTAL_ID_WIDTH + BID_WIDTH + METADATA_WIDTH

### 14.3.3	Example Configuration

4 masters, 4-bit external ID, max 16 outstanding per master:
  BID_WIDTH = clog2(4) = 2
  TOTAL_ID_WIDTH = 4 + 2 = 6
  CAM Depth = 16 × 4 = 64 entries
  CAM Width = 6 + 2 + 8 = 16 bits

## 14.4	CAM Operations

### 14.4.1	Insertion (AR/AW Acceptance)

// On AR handshake
if (arvalid && arready) begin
    cam_write_en <= 1'b1;
    cam_write_id <= {bid, arid};  // Key
    cam_write_master <= master_idx;  // Value
end

### 14.4.2	Lookup (R/B Response)

// On R response from slave
logic [BID_WIDTH-1:0] response_master;
logic cam_hit;

cam_lookup(slave_rid, response_master, cam_hit);

if (cam_hit) begin
    route_response_to(response_master);
end else begin
    // Error: Unknown transaction
end

### 14.4.3	Deletion (Response Complete)

// On RLAST or B response
if ((rvalid && rready && rlast) || (bvalid && bready)) begin
    cam_delete_en <= 1'b1;
    cam_delete_id <= response_id;
end

## 14.5	Implementation Options

### 14.5.1	Distributed RAM CAM

For small outstanding counts (< 16 entries):

// Parallel comparator-based CAM
logic [DEPTH-1:0] valid;
logic [ID_WIDTH-1:0] id_table [DEPTH];
logic [BID_WIDTH-1:0] master_table [DEPTH];

// Parallel compare for lookup
logic [DEPTH-1:0] match;
for (genvar i = 0; i < DEPTH; i++) begin
    assign match[i] = valid[i] && (id_table[i] == lookup_id);
end

### 14.5.2	Block RAM CAM

For larger outstanding counts (> 16 entries):

// Sequential search CAM (saves logic)
logic [$clog2(DEPTH)-1:0] search_idx;
logic searching;

always_ff @(posedge clk) begin
    if (start_lookup) begin
        searching <= 1'b1;
        search_idx <= 0;
    end else if (searching) begin
        if (match_found || search_idx == DEPTH-1)
            searching <= 1'b0;
        else
            search_idx <= search_idx + 1;
    end
end

## 14.6	CAM Overflow Handling

### 14.6.1	Prevention

// Backpressure when CAM full
assign ar_ready = !cam_full && downstream_ready;
assign aw_ready = !cam_full && downstream_ready;

### 14.6.2	Error Response

If CAM overflow would occur: 1. Assert backpressure (ARREADY/AWREADY = 0) 2. Wait for responses to free entries 3. Never drop transactions

## 14.7	Related Documentation

- ID Tracking - ID table implementation

- Response Routing - Using CAM for routing

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	ID Tracking Tables

## 15.1	Overview

ID tracking tables maintain the mapping between extended IDs (with Bridge ID) and originating masters. These tables enable correct response routing in multi-master systems.

## 15.2	Table Structure

### 15.2.1	Per-Slave ID Table

Each slave has its own ID tracking table:

### 15.2.2	Figure 4.2: ID Table Structure

ID Table Structure

### 15.2.3	Table Sizing

Entries = MAX_OUTSTANDING_PER_SLAVE
Width = TOTAL_ID_WIDTH + clog2(NUM_MASTERS) + 1 (valid)

## 15.3	ID Extension

### 15.3.1	Master-Side Extension

When transaction enters Bridge:

// Extend ID with Bridge ID (master index)
logic [TOTAL_ID_WIDTH-1:0] extended_id;
assign extended_id = {master_bid, external_id};

// Example: Master 2, external ID = 4'hA
// master_bid = 2'b10, external_id = 4'b1010
// extended_id = 6'b10_1010

### 15.3.2	Slave-Side Presentation

Extended ID goes to slave:

assign s_axi_arid = extended_id;  // 6 bits to slave
assign s_axi_awid = extended_id;  // 6 bits to slave

## 15.4	ID Extraction

### 15.4.1	Response Parsing

When response returns from slave:

// Extract Bridge ID and external ID
logic [BID_WIDTH-1:0] bridge_id;
logic [ID_WIDTH-1:0] external_id;

assign bridge_id = slave_rid[TOTAL_ID_WIDTH-1:ID_WIDTH];
assign external_id = slave_rid[ID_WIDTH-1:0];

// Route to master based on bridge_id
assign m_rvalid[bridge_id] = s_rvalid;
assign m_rid[bridge_id] = external_id;  // Strip Bridge ID

## 15.5	Table Operations

### 15.5.1	Allocation (AR/AW Phase)

always_ff @(posedge clk) begin
    if (arvalid && arready) begin
        // Find free entry
        for (int i = 0; i < DEPTH; i++) begin
            if (!valid[i]) begin
                id_table[i] <= extended_arid;
                master_table[i] <= master_idx;
                valid[i] <= 1'b1;
                break;
            end
        end
    end
end

### 15.5.2	Lookup (R/B Phase)

// Combinational lookup
logic [NUM_MASTERS-1:0] master_onehot;
always_comb begin
    master_onehot = '0;
    for (int i = 0; i < DEPTH; i++) begin
        if (valid[i] && (id_table[i] == response_id)) begin
            master_onehot[master_table[i]] = 1'b1;
        end
    end
end

### 15.5.3	Deallocation (Response Complete)

always_ff @(posedge clk) begin
    if (rvalid && rready && rlast) begin
        // Find and invalidate matching entry
        for (int i = 0; i < DEPTH; i++) begin
            if (valid[i] && (id_table[i] == response_rid)) begin
                valid[i] <= 1'b0;
                break;
            end
        end
    end
end

## 15.6	Multi-ID Considerations

### 15.6.1	Same External ID, Different Masters

Master 0 issues ID=5 → Extended: 00_0101
Master 1 issues ID=5 → Extended: 01_0101
Master 2 issues ID=5 → Extended: 10_0101

All three can be outstanding simultaneously!
The extended ID ensures uniqueness.

### 15.6.2	Same Extended ID (Error)

This cannot happen: - Bridge ID is unique per master - Extended ID = {unique BID, external ID} - Same extended ID implies same master + same external ID - AXI4 requires unique IDs per master for outstanding transactions

## 15.7	Resource Utilization

### 15.7.1	Table Resources

4 masters, 4-bit external ID, 16 outstanding per slave:

Per-slave table:
  Entries: 16
  Width: 6 + 2 + 1 = 9 bits
  Total: 16 × 9 = 144 bits

4 slaves:
  Total: 4 × 144 = 576 bits (~72 bytes)

Implementation:
  Distributed RAM: ~150 LEs
  Block RAM: 1 BRAM (minimum)

## 15.8	Related Documentation

- CAM Architecture - CAM implementation details

- Response Routing - Using tables for routing

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Width Converters

## 16.1	Overview

Width converters handle data width mismatches between masters and slaves. Bridge automatically inserts upsizers (narrow to wide) and downsizers (wide to narrow) as needed.

## 16.2	Upsize Converter

### 16.2.1	Purpose

Convert narrow master data to wide slave interface.

### 16.2.2	Figure 5.1: Upsize Converter (64-bit to 512-bit)

Upsize Converter

### 16.2.3	Implementation

module width_upsize #(
    parameter IN_WIDTH = 64,
    parameter OUT_WIDTH = 512
) (
    input  logic clk, rst_n,

    // Narrow input
    input  logic                    s_valid,
    output logic                    s_ready,
    input  logic [IN_WIDTH-1:0]     s_data,
    input  logic                    s_last,

    // Wide output
    output logic                    m_valid,
    input  logic                    m_ready,
    output logic [OUT_WIDTH-1:0]    m_data,
    output logic                    m_last
);

    localparam RATIO = OUT_WIDTH / IN_WIDTH;

    logic [OUT_WIDTH-1:0] r_buffer;
    logic [$clog2(RATIO)-1:0] r_count;

    always_ff @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            r_count <= '0;
            r_buffer <= '0;
        end else if (s_valid && s_ready) begin
            // Pack input into buffer
            r_buffer[r_count * IN_WIDTH +: IN_WIDTH] <= s_data;

            if (s_last || r_count == RATIO - 1) begin
                r_count <= '0;
            end else begin
                r_count <= r_count + 1;
            end
        end
    end

    // Output when buffer full or last beat
    assign m_valid = (r_count == RATIO - 1) || s_last;
    assign m_data = r_buffer;
    assign s_ready = !m_valid || m_ready;

endmodule

### 16.2.4	Burst Length Conversion

Table 5.3: Burst Length Conversion Examples

| Master AWLEN | Master Beats | Slave AWLEN | Slave Beats |
| --- | --- | --- | --- |
| 7 (8 beats) | 8 × 64b | 0 (1 beat) | 1 × 512b |
| 15 (16 beats) | 16 × 64b | 1 (2 beats) | 2 × 512b |
| 255 (256 beats) | 256 × 64b | 31 (32 beats) | 32 × 512b |

## 16.3	Downsize Converter

### 16.3.1	Purpose

Convert wide master data to narrow slave interface.

### 16.3.2	Figure 5.2: Downsize Converter (512-bit to 64-bit)

Downsize Converter

### 16.3.3	Implementation

module width_downsize #(
    parameter IN_WIDTH = 512,
    parameter OUT_WIDTH = 64
) (
    input  logic clk, rst_n,

    // Wide input
    input  logic                    s_valid,
    output logic                    s_ready,
    input  logic [IN_WIDTH-1:0]     s_data,
    input  logic                    s_last,

    // Narrow output
    output logic                    m_valid,
    input  logic                    m_ready,
    output logic [OUT_WIDTH-1:0]    m_data,
    output logic                    m_last
);

    localparam RATIO = IN_WIDTH / OUT_WIDTH;

    logic [IN_WIDTH-1:0] r_buffer;
    logic [$clog2(RATIO)-1:0] r_count;
    logic r_active;

    always_ff @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            r_count <= '0;
            r_buffer <= '0;
            r_active <= 1'b0;
        end else begin
            if (!r_active && s_valid) begin
                // Load new wide word
                r_buffer <= s_data;
                r_active <= 1'b1;
                r_count <= '0;
            end else if (r_active && m_ready) begin
                if (r_count == RATIO - 1) begin
                    r_active <= 1'b0;
                end else begin
                    r_count <= r_count + 1;
                end
            end
        end
    end

    // Output from buffer slice
    assign m_valid = r_active;
    assign m_data = r_buffer[r_count * OUT_WIDTH +: OUT_WIDTH];
    assign m_last = r_active && (r_count == RATIO - 1);
    assign s_ready = !r_active;

endmodule

## 16.4	Strobe Handling

### 16.4.1	Upsize Strobe Packing

// Pack 8-byte strobes into 64-byte strobes
logic [7:0] in_strb;   // 64-bit input
logic [63:0] out_strb; // 512-bit output

always_ff @(posedge clk) begin
    if (s_valid && s_ready) begin
        out_strb[r_count * 8 +: 8] <= in_strb;
    end
end

### 16.4.2	Downsize Strobe Splitting

// Split 64-byte strobes into 8-byte strobes
logic [63:0] in_strb;  // 512-bit input
logic [7:0] out_strb;  // 64-bit output

assign out_strb = in_strb[r_count * 8 +: 8];

## 16.5	Resource Utilization

### 16.5.1	Upsize Converter (64 to 512)

Registers: ~520 (buffer + control)
Logic: ~200 LEs (packing + control)

### 16.5.2	Downsize Converter (512 to 64)

Registers: ~520 (buffer + control)
Logic: ~150 LEs (selection + control)

## 16.6	Related Documentation

- Width Conversion Block - Block-level description

- APB Converters - Protocol conversion

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	APB Converters

## 17.1	Overview

APB converters transform AXI4 transactions into APB protocol. This is necessary when high-performance masters need to access low-speed peripherals.

## 17.2	Conversion Requirements

### 17.2.1	Protocol Differences

Table 5.1: AXI4 vs APB Protocol Comparison

| Feature | AXI4 | APB |
| --- | --- | --- |
| Channels | 5 (AW, W, B, AR, R) | 1 (combined) |
| Bursts | Up to 256 beats | None (single) |
| Pipelining | Yes | No |
| Min cycles | 1 per beat | 2 per transfer |

### 17.2.2	Conversion Strategy

- Split AXI4 bursts into individual APB transfers

- Serialize AW+W into APB write sequence

- Convert AR into APB read sequence

- Generate AXI4 responses from APB completions

## 17.3	Write Conversion

### 17.3.1	Figure 5.3: AXI4 Write to APB Write

AXI4 to APB Write

### 17.3.2	State Machine

typedef enum logic [2:0] {
    IDLE,
    AW_ACCEPT,
    W_ACCEPT,
    APB_SETUP,
    APB_ACCESS,
    B_GENERATE
} apb_wr_state_t;

apb_wr_state_t r_state;

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_state <= IDLE;
    end else begin
        case (r_state)
            IDLE: if (awvalid) r_state <= AW_ACCEPT;

            AW_ACCEPT: begin
                if (awready) r_state <= W_ACCEPT;
            end

            W_ACCEPT: begin
                if (wvalid && wready) r_state <= APB_SETUP;
            end

            APB_SETUP: r_state <= APB_ACCESS;

            APB_ACCESS: begin
                if (pready) begin
                    if (r_beat_count == r_awlen)
                        r_state <= B_GENERATE;
                    else
                        r_state <= W_ACCEPT;  // Next beat
                end
            end

            B_GENERATE: begin
                if (bready) r_state <= IDLE;
            end
        endcase
    end
end

## 17.4	Read Conversion

### 17.4.1	Figure 5.4: AXI4 Read to APB Read

AXI4 to APB Read

### 17.4.2	Read State Machine

typedef enum logic [2:0] {
    IDLE,
    AR_ACCEPT,
    APB_SETUP,
    APB_ACCESS,
    R_GENERATE
} apb_rd_state_t;

## 17.5	Burst Handling

### 17.5.1	Burst to Single Conversion

AXI4 Burst (AWLEN=7, 8 beats):
  Beat 0: AWADDR + 0×SIZE
  Beat 1: AWADDR + 1×SIZE
  Beat 2: AWADDR + 2×SIZE
  ...
  Beat 7: AWADDR + 7×SIZE

APB Sequence:
  Transfer 0: PADDR = AWADDR + 0×SIZE
  Transfer 1: PADDR = AWADDR + 1×SIZE
  ...
  Transfer 7: PADDR = AWADDR + 7×SIZE

### 17.5.2	Address Calculation

// Calculate APB address for each beat
logic [ADDR_WIDTH-1:0] apb_addr;
logic [7:0] beat_count;

always_comb begin
    case (r_burst_type)
        FIXED: apb_addr = r_base_addr;
        INCR:  apb_addr = r_base_addr + (beat_count << r_size);
        WRAP:  apb_addr = wrap_address(r_base_addr, beat_count, r_size, r_len);
    endcase
end

## 17.6	Error Handling

### 17.6.1	APB Error to AXI4 Response

Table 5.2: APB to AXI4 Error Mapping

| APB PSLVERR | AXI4 Response |
| --- | --- |
| 0 (OK) | OKAY (2’b00) |
| 1 (Error) | SLVERR (2’b10) |

### 17.6.2	Error Propagation

// Latch first error in burst
logic r_error_seen;

always_ff @(posedge clk) begin
    if (r_state == APB_ACCESS && pready && pslverr)
        r_error_seen <= 1'b1;
    if (r_state == IDLE)
        r_error_seen <= 1'b0;
end

// Final response reflects any errors
assign bresp = r_error_seen ? 2'b10 : 2'b00;

## 17.7	Performance Considerations

### 17.7.1	Throughput Impact

AXI4 alone: 1 beat per cycle (peak)
AXI4 to APB: 1 beat per 2+ cycles (minimum)

Throughput reduction: ~50% or more

### 17.7.2	Latency Impact

AXI4 read: 1-2 cycles (typical)
AXI4-to-APB read: 4+ cycles minimum
  - 1 cycle: AR accept
  - 1 cycle: APB setup
  - 1+ cycles: APB access (PREADY)
  - 1 cycle: R generate

## 17.8	Resource Utilization

### 17.8.1	APB Converter Resources

Logic Elements: ~300-400 LEs
Registers: ~100-150 regs
Block RAM: 0

Breakdown:
- State machine: ~50 LEs, ~20 regs
- Address calc: ~100 LEs, ~50 regs
- Beat counter: ~30 LEs, ~8 regs
- Data buffering: ~100 LEs, ~DATA_WIDTH regs

## 17.9	Related Documentation

- Protocol Conversion Block - Block description

- Width Converters - Data width conversion

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Module Structure

## 18.1	Generated RTL Overview

Bridge generator creates parameterized SystemVerilog modules from configuration files. The generated RTL follows a consistent structure for all topologies.

## 18.2	Top-Level Module

### 18.2.1	Module Declaration

module bridge_4x3 #(
    parameter int NUM_MASTERS = 4,
    parameter int NUM_SLAVES = 3,
    parameter int ADDR_WIDTH = 32,
    parameter int DATA_WIDTH = 64,
    parameter int ID_WIDTH = 4
) (
    // Clock and reset
    input  logic aclk,
    input  logic aresetn,

    // Master interfaces (M0-M3)
    // ... master port signals ...

    // Slave interfaces (S0-S2)
    // ... slave port signals ...
);

### 18.2.2	Port Organization

Module Ports:
├── Clock/Reset
│   ├── aclk
│   └── aresetn
├── Master Ports (per master)
│   ├── AW Channel (if rw or wr)
│   ├── W Channel (if rw or wr)
│   ├── B Channel (if rw or wr)
│   ├── AR Channel (if rw or rd)
│   └── R Channel (if rw or rd)
└── Slave Ports (per slave)
    ├── AW Channel
    ├── W Channel
    ├── B Channel
    ├── AR Channel
    └── R Channel

## 18.3	Internal Structure

### 18.3.1	Component Instantiation

// Generated module internal structure

// Master adapters (per master)
master_adapter_rw u_m0_adapter (...);
master_adapter_wr u_m1_adapter (...);
master_adapter_rd u_m2_adapter (...);
master_adapter_rw u_m3_adapter (...);

// Address decoders (per master)
address_decoder u_m0_decoder (...);
address_decoder u_m1_decoder (...);
address_decoder u_m2_decoder (...);
address_decoder u_m3_decoder (...);

// Per-slave arbiters
arbiter_aw u_s0_aw_arb (...);
arbiter_ar u_s0_ar_arb (...);
arbiter_aw u_s1_aw_arb (...);
arbiter_ar u_s1_ar_arb (...);
arbiter_aw u_s2_aw_arb (...);
arbiter_ar u_s2_ar_arb (...);

// Width converters (as needed)
width_upsize_64_512 u_m0_s0_upsizer (...);

// Protocol converters (as needed)
axi4_to_apb u_s2_apb_conv (...);

// Response routing
response_router u_resp_router (...);

## 18.4	Signal Naming Convention

### 18.4.1	Master-Side Signals (External)

{prefix}_aw{signal}    - Write address channel
{prefix}_w{signal}     - Write data channel
{prefix}_b{signal}     - Write response channel
{prefix}_ar{signal}    - Read address channel
{prefix}_r{signal}     - Read data channel

Example (prefix = "cpu_m_axi"):
  cpu_m_axi_awvalid
  cpu_m_axi_awready
  cpu_m_axi_awaddr
  cpu_m_axi_wdata
  cpu_m_axi_bvalid

### 18.4.2	Slave-Side Signals (External)

{prefix}_aw{signal}    - Write address channel
{prefix}_w{signal}     - Write data channel
{prefix}_b{signal}     - Write response channel
{prefix}_ar{signal}    - Read address channel
{prefix}_r{signal}     - Read data channel

Example (prefix = "ddr_s_axi"):
  ddr_s_axi_awvalid
  ddr_s_axi_awready
  ddr_s_axi_awaddr
  ddr_s_axi_wdata
  ddr_s_axi_bvalid

### 18.4.3	Internal Signals

// Crossbar internal signals
xbar_m{N}_aw_{signal}  - Master N to crossbar AW
xbar_m{N}_ar_{signal}  - Master N to crossbar AR
xbar_s{N}_aw_{signal}  - Crossbar to slave N AW
xbar_s{N}_ar_{signal}  - Crossbar to slave N AR

// Arbitration signals
grant_aw_s{N}[M-1:0]   - AW grants for slave N
grant_ar_s{N}[M-1:0]   - AR grants for slave N

// ID tracking signals
id_table_s{N}[...]     - ID table for slave N

## 18.5	Generated File Structure

### 18.5.1	Single-File Output

bridge_{name}.sv
├── Module declaration
├── Parameter section
├── Port declarations
├── Internal signal declarations
├── Master adapter instances
├── Address decoder instances
├── Arbiter instances
├── Converter instances
├── Response router instance
└── Debug signals (optional)

### 18.5.2	Multi-File Output (Optional)

bridge_{name}/
├── bridge_{name}.sv        - Top-level wrapper
├── master_adapter_m0.sv    - Master 0 adapter
├── master_adapter_m1.sv    - Master 1 adapter
├── address_decoder.sv      - Shared decoder
├── arbiter_aw.sv           - AW arbiter
├── arbiter_ar.sv           - AR arbiter
├── width_upsize_64_512.sv  - Width converter
├── axi4_to_apb.sv          - Protocol converter
└── response_router.sv      - Response routing

## 18.6	Parameterization

### 18.6.1	Compile-Time Parameters

// Core parameters
parameter int NUM_MASTERS = 4;
parameter int NUM_SLAVES = 3;
parameter int ADDR_WIDTH = 32;
parameter int DATA_WIDTH = 64;
parameter int ID_WIDTH = 4;

// Derived parameters
localparam int BID_WIDTH = $clog2(NUM_MASTERS);
localparam int TOTAL_ID_WIDTH = ID_WIDTH + BID_WIDTH;
localparam int STRB_WIDTH = DATA_WIDTH / 8;

### 18.6.2	Per-Port Parameters

// Generated per-port widths
localparam int M0_DATA_WIDTH = 64;
localparam int M1_DATA_WIDTH = 256;
localparam int S0_DATA_WIDTH = 512;
localparam int S1_DATA_WIDTH = 32;  // APB

## 18.7	Related Documentation

- Signal Naming - Detailed naming conventions

- Generator Usage - How to run the generator

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Signal Naming

## 19.1	Naming Convention Overview

Bridge uses consistent signal naming to enable pattern-based verification and clear integration.

## 19.2	External Port Naming

### 19.2.1	Pattern

{prefix}_{channel}{signal}

### 19.2.2	Components

| Component | Description | Example |
| --- | --- | --- |
| prefix | User-defined port prefix | cpu_m_axi, ddr_s_axi |
| channel | AXI4 channel code | aw, w, b, ar, r |
| signal | Signal within channel | valid, ready, addr, data |

### 19.2.3	Master Port Examples

// Master with prefix "cpu_m_axi"
input  logic        cpu_m_axi_awvalid,
output logic        cpu_m_axi_awready,
input  logic [31:0] cpu_m_axi_awaddr,
input  logic [3:0]  cpu_m_axi_awid,
input  logic [7:0]  cpu_m_axi_awlen,
input  logic [2:0]  cpu_m_axi_awsize,
input  logic [1:0]  cpu_m_axi_awburst,
input  logic        cpu_m_axi_awlock,
input  logic [3:0]  cpu_m_axi_awcache,
input  logic [2:0]  cpu_m_axi_awprot,
input  logic [3:0]  cpu_m_axi_awqos,
input  logic        cpu_m_axi_awuser,

### 19.2.4	Slave Port Examples

// Slave with prefix "ddr_s_axi"
output logic        ddr_s_axi_awvalid,
input  logic        ddr_s_axi_awready,
output logic [31:0] ddr_s_axi_awaddr,
output logic [5:0]  ddr_s_axi_awid,   // Extended ID
output logic [7:0]  ddr_s_axi_awlen,
output logic [2:0]  ddr_s_axi_awsize,
// ...

## 19.3	APB Port Naming

### 19.3.1	Pattern

{prefix}p{signal}

### 19.3.2	APB Signal Examples

// APB slave with prefix "uart_apb_"
output logic        uart_apb_psel,
output logic        uart_apb_penable,
output logic [11:0] uart_apb_paddr,
output logic        uart_apb_pwrite,
output logic [31:0] uart_apb_pwdata,
output logic [3:0]  uart_apb_pstrb,
output logic [2:0]  uart_apb_pprot,
input  logic [31:0] uart_apb_prdata,
input  logic        uart_apb_pslverr,
input  logic        uart_apb_pready,

## 19.4	Internal Signal Naming

### 19.4.1	Crossbar Signals

// Master to crossbar
logic        xbar_m0_awvalid;
logic        xbar_m0_awready;
logic [31:0] xbar_m0_awaddr;

// Crossbar to slave
logic        xbar_s0_awvalid;
logic        xbar_s0_awready;
logic [31:0] xbar_s0_awaddr;

### 19.4.2	Arbitration Signals

// Request/grant matrices
logic [NUM_MASTERS-1:0] aw_request_s0;  // AW requests to slave 0
logic [NUM_MASTERS-1:0] aw_grant_s0;    // AW grants from slave 0

// Arbiter state
logic aw_grant_active_s0;               // Grant locked
logic [$clog2(NUM_MASTERS)-1:0] aw_last_grant_s0;  // Round-robin state

### 19.4.3	ID Signals

// Extended IDs (internal)
logic [TOTAL_ID_WIDTH-1:0] xbar_m0_awid;  // BID + external ID
logic [TOTAL_ID_WIDTH-1:0] xbar_s0_bid;   // Response with BID

// External IDs (to master)
logic [ID_WIDTH-1:0] cpu_m_axi_bid;       // Stripped ID

## 19.5	Debug Signal Naming

### 19.5.1	Pattern

dbg_{category}_{description}

### 19.5.2	Debug Signal Examples

// Arbitration debug
output logic [NUM_MASTERS-1:0] dbg_aw_grant_s0,
output logic [NUM_MASTERS-1:0] dbg_ar_grant_s0,

// Transaction debug
output logic [7:0] dbg_outstanding_m0,
output logic [7:0] dbg_outstanding_m1,

// State machine debug
output logic [1:0] dbg_aw_state_s0,
output logic [1:0] dbg_ar_state_s0,

## 19.6	Verification Pattern Matching

### 19.6.1	Factory Pattern Support

Signal naming enables automatic BFM connection:

# CocoTB/GAXI pattern matching
master = AXI4Master(
    dut=dut,
    prefix="cpu_m_axi_",  # Matches all cpu_m_axi_* signals
    clock=dut.aclk
)

### 19.6.2	Pattern Rules

- Prefix must end with underscore or be directly followed by channel code

- Channel codes are lowercase (aw, w, b, ar, r)

- Signal names match AXI4 specification (valid, ready, addr, data, etc.)

## 19.7	Related Documentation

- Module Structure - Overall RTL organization

- Verification - Pattern-based testing

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Test Strategy

## 20.1	Overview

Bridge verification uses CocoTB-based testing with parameterized configurations. The test infrastructure supports all bridge topologies and protocol combinations.

## 20.2	Test Categories

### 20.2.1	Unit Tests

Per-block verification:

Unit Test Coverage:
├── Master Adapter Tests
│   ├── ID extension verification
│   ├── Channel routing
│   └── Backpressure handling
├── Address Decoder Tests
│   ├── Address mapping
│   ├── Multi-region support
│   └── Default slave routing
├── Arbiter Tests
│   ├── Round-robin fairness
│   ├── Grant/lock behavior
│   └── Multi-master contention
└── Converter Tests
    ├── Width up/down sizing
    ├── APB protocol conversion
    └── Burst handling

### 20.2.2	Integration Tests

Full bridge verification:

Integration Test Matrix:
├── 2x2 Configuration
│   ├── Basic read/write
│   ├── Concurrent transactions
│   └── ID collision avoidance
├── 4x4 Configuration
│   ├── Full arbitration coverage
│   ├── Width conversion paths
│   └── Mixed protocol targets
└── NxM Configurations
    ├── Asymmetric topologies
    ├── Channel-specific masters
    └── Protocol mixing

## 20.3	Test Parameterization

### 20.3.1	Configuration Matrix

Table 7.1: Test Parameter Matrix

| Parameter | Test Values | Purpose |
| --- | --- | --- |
| NUM_MASTERS | 2, 4, 8 | Scalability |
| NUM_SLAVES | 2, 3, 4 | Routing coverage |
| DATA_WIDTH | 32, 64, 128, 256 | Width paths |
| ID_WIDTH | 4, 6, 8 | ID space |
| OUTSTANDING | 4, 8, 16 | Depth stress |

### 20.3.2	Protocol Combinations

# Test protocol matrix
PROTOCOL_COMBOS = [
    {"masters": ["axi4"], "slaves": ["axi4"]},
    {"masters": ["axi4"], "slaves": ["axi4", "apb"]},
    {"masters": ["axi4", "axil"], "slaves": ["axi4", "apb"]},
]

## 20.4	Factory-Based Testing

### 20.4.1	BFM Instantiation

from CocoTBFramework.components.gaxi import GAXIMaster, GAXISlave

class BridgeTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)

        # Create masters using factory pattern
        self.masters = []
        for i in range(NUM_MASTERS):
            prefix = f"m{i}_axi_"
            master = GAXIMaster(
                dut=dut,
                prefix=prefix,
                clock=dut.aclk,
                reset=dut.aresetn
            )
            self.masters.append(master)

        # Create slaves
        self.slaves = []
        for i in range(NUM_SLAVES):
            prefix = f"s{i}_axi_"
            slave = GAXISlave(
                dut=dut,
                prefix=prefix,
                clock=dut.aclk,
                reset=dut.aresetn
            )
            self.slaves.append(slave)

### 20.4.2	Transaction Generation

async def test_concurrent_writes(self):
    """Test multiple masters writing simultaneously."""

    # Generate transactions for each master
    tasks = []
    for i, master in enumerate(self.masters):
        addr = 0x1000 + (i * 0x100)
        data = [0xDEAD0000 + i] * 4
        task = cocotb.start_soon(
            master.write(addr=addr, data=data, size=2)
        )
        tasks.append(task)

    # Wait for all to complete
    for task in tasks:
        await task

    # Verify arbitration occurred correctly
    self.verify_arbitration_order()

## 20.5	Coverage Goals

### 20.5.1	Functional Coverage

Coverage Targets:
├── Protocol Coverage
│   ├── All burst types (FIXED, INCR, WRAP)
│   ├── All sizes (1, 2, 4, 8, ... bytes)
│   └── All response types
├── Routing Coverage
│   ├── Every master to every slave path
│   ├── Default slave routing
│   └── Error response paths
├── Arbitration Coverage
│   ├── All grant combinations
│   ├── Priority scenarios
│   └── Lock sequences
└── Corner Cases
    ├── Maximum outstanding
    ├── ID exhaustion
    └── Backpressure saturation

### 20.5.2	Code Coverage

Code Coverage Targets:
├── Line Coverage: >95%
├── Branch Coverage: >90%
├── FSM State Coverage: 100%
└── Toggle Coverage: >85%

## 20.6	Test Execution

### 20.6.1	Running Tests

# Run all bridge tests
cd projects/components/bridge/dv/tests
pytest -v

# Run specific configuration
pytest test_bridge_4x4.py -v

# Run with coverage
pytest --cov=bridge -v

# Run with waveforms
WAVES=1 pytest test_bridge_2x2.py -v

### 20.6.2	Test Levels

Table 7.2: Test Level Definitions

| Level | Duration | Coverage | Use Case |
| --- | --- | --- | --- |
| basic | ~30s | Smoke test | Quick verification |
| medium | ~90s | Core paths | Development |
| full | ~180s | Comprehensive | Pre-commit |

## 20.7	Related Documentation

- Debug Guide - Debugging failed tests

- Signal Naming - Pattern matching for BFMs

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Debug Guide

## 21.1	Overview

This guide provides debugging strategies for Bridge verification failures. Common issues and their resolution approaches are documented.

## 21.2	Debug Signal Access

### 21.2.1	Enabling Debug Outputs

# Enable debug signals in test
@cocotb.test()
async def test_with_debug(dut):
    # Access debug signals
    grant_s0 = dut.dbg_aw_grant_s0.value
    outstanding_m0 = dut.dbg_outstanding_m0.value
    state_s0 = dut.dbg_aw_state_s0.value

### 21.2.2	Debug Signal Categories

Table 7.3: Debug Signal Categories

| Category | Signals | Purpose |
| --- | --- | --- |
| Arbitration | dbg_*_grant_s* | Grant state per slave |
| Outstanding | dbg_outstanding_m* | Transaction depth per master |
| State | dbg_*_state_s* | FSM states per channel |
| ID Tracking | dbg_id_table_* | CAM contents |

## 21.3	Common Issues

### 21.3.1	Issue: Transaction Timeout

Symptoms: - Test hangs waiting for response - TimeoutError after configured timeout

Debug Steps:

# 1. Check if request was accepted
await RisingEdge(dut.aclk)
print(f"AWVALID: {dut.m0_axi_awvalid.value}")
print(f"AWREADY: {dut.m0_axi_awready.value}")

# 2. Check arbitration state
print(f"AW Grant S0: {dut.dbg_aw_grant_s0.value}")

# 3. Check outstanding count
print(f"Outstanding M0: {dut.dbg_outstanding_m0.value}")

# 4. Check slave response
print(f"BVALID: {dut.s0_axi_bvalid.value}")
print(f"BREADY: {dut.s0_axi_bready.value}")

Common Causes: - Arbiter locked by another master - Outstanding limit reached - Slave not responding - ID table full

### 21.3.2	Issue: Wrong Response Routing

Symptoms: - Response arrives at wrong master - BID mismatch errors

Debug Steps:

# Check ID extension
print(f"Outgoing AWID: {dut.xbar_m0_awid.value}")
print(f"Slave received ID: {dut.s0_axi_awid.value}")

# Check response ID
print(f"Slave BID: {dut.s0_axi_bid.value}")
print(f"Routed to master: {dut.dbg_resp_master.value}")

Common Causes: - ID extension incorrect - CAM lookup failure - ID width mismatch

### 21.3.3	Issue: Data Corruption

Symptoms: - Read data doesn’t match written data - WSTRB/data alignment issues

Debug Steps:

# Check width conversion
print(f"Master data width: {dut.M0_DATA_WIDTH.value}")
print(f"Slave data width: {dut.S0_DATA_WIDTH.value}")

# Check strobe packing
print(f"Master WSTRB: {dut.m0_axi_wstrb.value}")
print(f"Slave WSTRB: {dut.s0_axi_wstrb.value}")

# Check address alignment
print(f"AWADDR: {dut.m0_axi_awaddr.value}")
print(f"AWSIZE: {dut.m0_axi_awsize.value}")

Common Causes: - Width converter byte lane error - Unaligned access handling - Burst boundary crossing

## 21.4	Waveform Analysis

### 21.4.1	Generating Waveforms

# Enable VCD dump
WAVES=1 pytest test_bridge_2x2.py::test_basic -v

# View with GTKWave
gtkwave sim_build/dump.vcd

### 21.4.2	Key Signals to Observe

Waveform Signal Groups:
├── Clock/Reset
│   └── aclk, aresetn
├── Master 0 AW Channel
│   └── m0_axi_awvalid, m0_axi_awready, m0_axi_awaddr, m0_axi_awid
├── Slave 0 AW Channel
│   └── s0_axi_awvalid, s0_axi_awready, s0_axi_awaddr, s0_axi_awid
├── Arbitration
│   └── dbg_aw_grant_s0, dbg_ar_grant_s0
└── Response
    └── s0_axi_bvalid, m0_axi_bvalid, dbg_resp_master

### 21.4.3	Timing Analysis

Expected Transaction Timing:
     ┌───┐   ┌───┐   ┌───┐   ┌───┐   ┌───┐
aclk ┘   └───┘   └───┘   └───┘   └───┘   └───

     ┌───────────────────┐
awv  ┘                   └───────────────────

             ┌───────────┐
awr  ────────┘           └───────────────────

                         ┌───────────────────
wval ────────────────────┘

Time:  0     1     2     3     4     5

## 21.5	Assertion Failures

### 21.5.1	Built-in Assertions

// Protocol assertions in generated RTL
assert property (@(posedge aclk) disable iff (!aresetn)
    s_awvalid && !s_awready |=> s_awvalid
) else $error("AW handshake violation");

assert property (@(posedge aclk) disable iff (!aresetn)
    s_wvalid && s_wlast |-> s_awid == r_current_awid
) else $error("W data ID mismatch");

### 21.5.2	Handling Assertion Failures

- Locate assertion in RTL - Search for error message

- Check signal history - Review 10-20 cycles before failure

- Identify root cause - Usually protocol or timing issue

- Fix test or RTL - Depending on where bug exists

## 21.6	Performance Debugging

### 21.6.1	Throughput Issues

# Measure transaction throughput
start_time = cocotb.utils.get_sim_time('ns')

for i in range(100):
    await master.write(addr=0x1000 + i*4, data=[i])

end_time = cocotb.utils.get_sim_time('ns')
throughput = 100 / (end_time - start_time) * 1e9  # tx/sec
print(f"Throughput: {throughput:.2f} transactions/sec")

### 21.6.2	Latency Issues

# Measure single transaction latency
start = cocotb.utils.get_sim_time('ns')
await master.read(addr=0x1000)
latency = cocotb.utils.get_sim_time('ns') - start
print(f"Read latency: {latency} ns")

## 21.7	Related Documentation

- Test Strategy - Overall test approach

- Arbiter FSMs - FSM state details

- ID Tracking - ID table operation

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Bridge Hardware Architecture Specification Index

Version: 1.0 Date: 2026-01-03 Purpose: High-level hardware architecture specification for Bridge component

## 22.1	Document Organization

Note: All chapters linked below for automated document generation.

### 22.1.1	Front Matter

- Document Information

### 22.1.2	Chapter 1: Introduction

- Purpose and Scope

- Document Conventions

- Definitions and Acronyms

### 22.1.3	Chapter 2: System Overview

- Use Cases

- Key Features

- System Context

### 22.1.4	Chapter 3: Architecture

- Block Diagram

- Data Flow

- Protocol Support

### 22.1.5	Chapter 4: Interfaces

- AXI4 Interface Overview

- APB Interface Overview

- Clock and Reset

### 22.1.6	Chapter 5: Performance

- Throughput Characteristics

- Latency Analysis

- Resource Estimates

### 22.1.7	Chapter 6: Integration

- System Requirements

- Parameter Configuration

- Verification Strategy

## 22.2	Related Documentation

- Bridge MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

Last Updated: 2026-01-03 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Bridge: AXI4 Full Crossbar Generator - Product Requirements Document

Project: Bridge Version: 2.1 Status: 🟢 Phase 2 Complete - Simplified Architecture with Hard Limits Created: 2025-10-18 Last Updated: 2025-11-02

## 23.1	Executive Summary

Bridge is a Python-based AXI4 crossbar generator that produces simple, performant SystemVerilog RTL for connecting multiple AXI4 masters to multiple AXI4 slaves. The name follows the infrastructure theme - bridges connect different regions, enabling communication across divides, just like crossbars connect masters and slaves.

Design Philosophy: A simple AMBA fabric that is performant, but makes no attempt to support all features. We enforce hard limits (8-bit ID width, 64-bit address width) to eliminate unnecessary complexity, focusing only on what real hardware needs.

Key Differentiator from Delta: - Delta: AXI-Stream crossbar (streaming data, single channel, simple routing) - Bridge: AXI4 full crossbar (memory-mapped, 5 channels, burst support, ID-based routing)

Key Differentiator from Commercial IP: - Commercial AXI4 Crossbars: Feature-complete, support every AXI4 edge case, complex configurability - Bridge: Simple and performant, supports common use cases, hard limits for simplicity

Target Use Case: High-performance memory-mapped interconnects for multi-core processors, accelerators, and memory controllers where simplicity and performance matter more than spec compliance.

## 23.2	Design Philosophy

Vision: A simple AMBA fabric that is performant, but makes no attempt to support all features.

### 23.2.1	Core Principles

1. Simplicity Over Completeness - Focus on common use cases, not edge cases - Implement what’s needed for real hardware, not AXI4 spec compliance theater - Prefer straightforward implementations over complex feature sets

2. Performance First - Direct connections where possible (matching widths = zero overhead) - Minimal conversion logic in critical paths - Optimize for throughput and latency, not feature count

3. Hard Limits for Simplicity

We enforce uniform widths on certain parameters to eliminate complexity:

| Parameter | Hard Limit | Rationale |
| --- | --- | --- |
| ID Width | 8 bits (fixed) | Eliminates ID width conversion logic. 256 unique IDs is sufficient for all realistic use cases. |
| Address Width | 64 bits (fixed) | Eliminates address width conversion. 64-bit addressing covers all memory-mapped systems. |
| Data Width | Variable (32b-512b) | Width converters ONLY for data. Commonly needed for bandwidth optimization. |

Why This Works: - ID width conversion adds complexity with minimal benefit (nobody needs >256 outstanding transactions per master) - Address width conversion is rarely needed (peripheral addresses fit in 32-bit, but using 64-bit everywhere is simpler) - Data width conversion is the ONLY width conversion that provides real value (bandwidth matching)

4. What We DON’T Support (By Design)

Features intentionally excluded for simplicity: - ✗ Variable ID width per port - ✗ Variable address width per port - ✗ AXI4-Lite protocol variant (use standard AXI4 with len=0) - ✗ ACE protocol extensions (cache coherency) - ✗ AXI5 features - ✗ Complete AXI4 sideband signal support (QoS, Region, User signals declared but not routed)

5. What We DO Support

Core AXI4 features that matter: - ✓ Full 5-channel AXI4 protocol (AW, W, B, AR, R) - ✓ Burst transactions (INCR, WRAP, FIXED) - ✓ Out-of-order completion via transaction IDs - ✓ Multiple outstanding transactions per master - ✓ Channel-specific masters (write-only, read-only, read-write) - ✓ Data width conversion (32b ↔ 64b ↔ 128b ↔ 256b ↔ 512b) - ✓ Configurable M×N topology (1-32 masters, 1-256 slaves) - ✓ Fair round-robin arbitration per slave

### 23.2.2	Architecture Philosophy

Target: Intelligent width-aware routing, not fixed-width crossbar

OLD Approach (What We’re Moving Away From):

Master (64b) → Upsize(64→256) → Fixed 256b Crossbar → Downsize(256→64) → Slave (64b)

- Two conversions for same-width connections (wasteful!)

- Artificial bandwidth bottleneck

- Unnecessary logic and latency

NEW Approach (Target Architecture):

Master (64b) → Direct Connection → Slave (64b)    [0 conversions!]
Master (512b) → Conv(512→64) → Slave (64b)         [1 conversion]

- Per-master paths to each unique slave width it connects to

- Direct paths where widths match (zero overhead)

- Converters only where actually needed

- Router selects appropriate path based on address decode

Result: Minimal conversion logic, maximum performance, pragmatic simplicity.

## 23.3	⚠ CRITICAL: RTL Regeneration Requirements

ALL generated RTL files MUST be deleted and regenerated together whenever ANY generator code changes.

Why This Matters: - Generated RTL files may have interdependencies (bridges, wrappers, integrators) - Generator code changes can create version mismatches between files - Partial regeneration creates subtle incompatibilities that cause test failures - Even “small innocuous” generator changes can have cascading effects

The Rule:

# ✗ WRONG - Partial regeneration
./bridge_generator.py --masters 5 --slaves 3 --output ../rtl/
# Only regenerates bridge_axi4_flat_5x3.sv
# Other files (wrappers, integrators) now mismatched!

# ✓ CORRECT - Full regeneration
rm ../rtl/bridge_*.sv                    # Delete ALL generated bridges
rm ../rtl/bridge_wrapper_*.sv            # Delete ALL generated wrappers
./regenerate_all_bridges.sh              # Regenerate everything together

Generator Files That Trigger Full Regeneration: - bridge_generator.py - Main bridge generator - bridge_csv_generator.py - CSV-based generator - bridge_address_arbiter.py - Address decode logic - bridge_channel_router.py - Channel routing logic - bridge_response_router.py - Response routing logic - bridge_amba_integrator.py - AMBA component integration - bridge_wrapper_generator.py - Wrapper generation - Any Python file in projects/components/bridge/bin/

Symptoms of Version Mismatch: - Tests that previously passed now fail - Simulation errors about missing signals - Mismatched port widths or counts - Address decode routing to wrong slaves

Think of Generated RTL Like Compiled Code: When you update a compiler, you don’t selectively recompile - you rebuild everything. When you update a generator, you don’t selectively regenerate - you regenerate everything.

## 23.4	1. Product Overview

### 23.4.1	1.1 Purpose

Bridge provides automated generation of AXI4 crossbar infrastructure with: - Python code generation - Parameterized RTL generation (similar to APB/Delta) - Performance modeling - Analytical + simulation validation - Flat topology - Full M×N interconnect matrix - ID-based routing - Out-of-order transaction support - Burst optimization - Pipelined burst transfers

### 23.4.2	1.2 Target Audience

Primary Users: - RTL designers building SoC interconnect - System architects evaluating interconnect topologies - Verification engineers needing crossbar testbenches - Students learning AXI4 protocol and interconnects

Educational Focus: - Demonstrates AXI4 protocol complexity - Shows arbitration strategies for memory-mapped busses - Teaches ID-based transaction tracking - Illustrates burst optimization techniques

### 23.4.3	1.3 Success Criteria

Functional: - [x] Generates working AXI4 crossbar RTL (bridge_generator.py) - [x] Generates CSV-configured bridges (bridge_csv_generator.py) - [x] Passes Verilator lint - [x] Supports 1-32 masters, 1-256 slaves - [x] Handles out-of-order completion via IDs (bridge_cam.sv) - [x] Supports burst lengths 1-256 beats - [x] Channel-specific masters (wr/rd/rw) for resource optimization (Phase 2) - [ ] APB converter integration (Phase 3 pending)

Performance: - [x] Latency ≤ 3 cycles for single-beat transactions - [x] Throughput: All M×N paths can transfer concurrently - [x] Performance models implemented (bridge_model.py - V1 Flat) - [ ] Fmax ≥ 300 MHz on UltraScale+ FPGAs (pending synthesis validation)

Quality: - [x] Complete specifications (PRD) before code - [x] Performance models validate requirements (bridge_model.py) - [x] All generated RTL Verilator verified - [x] Integration examples provided (CSV examples) - [x] Comprehensive documentation (BRIDGE_CURRENT_STATE.md, BRIDGE_ARCHITECTURE_DIAGRAMS.md)

### 23.4.4	1.4 Implementation Status and Phases

Unified Generator (bridge_generator.py):

The bridge generator now supports both TOML/CSV configuration and legacy array-indexed modes:

Configuration Modes: 1. TOML/CSV Mode (Preferred) - TOML port configuration (bridge_name.toml) - CSV connectivity matrix (bridge_name_connectivity.csv) - Custom port prefixes (rapids_m_axi_, cpu_m_axi_, etc.) - Interface wrapper integration (timing isolation) - Mixed protocols (AXI4 + APB + AXI4-Lite slaves) - Channel-specific masters (wr/rd/rw) - Status: ✓ Phase 2 complete, Phase 3 pending

- Legacy CSV Mode (Backwards Compatible)

  - Separate ports.csv and connectivity.csv files

  - Migration path to TOML format

  - See test_configs/README.md for conversion guide

Phase Status:

Phase 1: CSV Configuration ✓ COMPLETE - CSV parser (ports.csv, connectivity.csv) - Port generation with custom prefixes - Converter identification logic - Basic crossbar instantiation

Phase 2: Channel-Specific Masters ✓ COMPLETE (2025-10-26) - Added channels field to PortSpec (rw/wr/rd) - Conditional port generation based on channels - Resource Optimization: - wr (write-only): AW, W, B channels only → 39% port reduction - rd (read-only): AR, R channels only → 61% port reduction - rw (full): All 5 channels - Width converter awareness (only generate needed converters) - Example: 4-master bridge saves 35% signals with optimized channels

Phase 3: APB Converter Integration ⏳ PENDING - AXI2APB converter module (create or integrate existing) - APB signal packing/unpacking - APB converter instantiation in generated bridges - Width converter + APB converter chaining - End-to-end testing with APB slaves - Status: Placeholders in generated code with detailed TODO comments

Additional Resources: - bridge_model.py - Performance modeling (V1 Flat implemented) - bridge_cam.sv - Transaction ID tracking for OOO support - See BRIDGE_CURRENT_STATE.md for detailed review - See docs/BRIDGE_ARCHITECTURE_DIAGRAMS.md for visual architecture

## 23.5	2. Architecture Overview

### 23.5.1	2.1 AXI4 vs AXIS vs APB

| Feature | APB | AXI-Stream (Delta) | AXI4 (Bridge) |
| --- | --- | --- | --- |
| Protocol Type | Simple register bus | Streaming data | Memory-mapped burst |
| Channels | 1 (address + data) | 1 (data stream) | 5 (AW, W, B, AR, R) |
| Request Generation | Address range decode | TDEST decode | Address range decode |
| Arbitration | Per-slave, per-cycle | Per-slave, packet-locked | Per-slave, per-address-phase |
| Out-of-Order | No (sequential) | No (streaming order) | Yes (ID-based) |
| Burst Support | No | Packet (via TLAST) | Yes (AWLEN/ARLEN) |
| Complexity | Low | Medium | High |
| Latency | 1-2 cycles | 2 cycles | 2-3 cycles |
| Use Case | Control registers | Data streaming | Memory-mapped I/O |

Bridge Complexity Sources: 1. 5 independent channels requiring separate arbitration 2. ID-based routing for out-of-order completion 3. Burst handling with interleaving constraints 4. Write response tracking (match AW with B channel) 5. Address decode + ID muxing for response routing

### 23.5.2	2.2 Block Diagram

┌──────────────────────────────────────────────────────────────────────┐
│                          BRIDGE AXI4 CROSSBAR                        │
└──────────────────────────────────────────────────────────────────────┘

Masters (M)                                                    Slaves (S)
┌─────────┐                                                   ┌─────────┐
│ Master 0│ ─AW──┐                                      ┌─AW──│ Slave 0 │
│  (CPU)  │ ─W───┤                                      ├─W───│ (DDR)   │
│         │ ─B───┤                                      ├─B───│         │
│         │ ─AR──┤                                      ├─AR──│         │
│         │ ─R───┤                                      ├─R───│         │
└─────────┘      │                                      │     └─────────┘
                 │                                      │
┌─────────┐      │   ┌──────────────────────────┐      │     ┌─────────┐
│ Master 1│ ─────┼───│  Request Generation      │──────┼─────│ Slave 1 │
│  (GPU)  │      │   │  (Address Range Decode)  │      │     │ (SRAM)  │
└─────────┘      │   └──────────────────────────┘      │     └─────────┘
                 │             │                        │
┌─────────┐      │   ┌──────────────────────────┐      │     ┌─────────┐
│ Master 2│ ─────┼───│  Arbitration Matrix      │──────┼─────│ Slave 2 │
│  (DMA)  │      │   │  (5 separate arbiters    │      │     │ (PCIE)  │
└─────────┘      │   │   per slave: AW,W,B,AR,R)│      │     └─────────┘
                 │   └──────────────────────────┘      │
┌─────────┐      │             │                        │     ┌─────────┐
│ Master 3│ ─────┴───│  Data/Addr Multiplexing  │──────┴─────│ Slave 3 │
│ (Accel) │          │  (ID-based routing)      │            │ (Periph)│
└─────────┘          └──────────────────────────┘            └─────────┘
                               │
                     ┌──────────────────────────┐
                     │  Transaction Tracking    │
                     │  (ID tables for OoO)     │
                     └──────────────────────────┘

### 23.5.3	2.3 Key Components

1. Request Generation - Address range decode for each slave - Generates M×S request matrix per channel (AW, AR) - Similar to APB but more complex (2 address channels)

2. Per-Slave Arbitration - 5 separate arbiters per slave: - AW channel arbiter (write address) - W channel arbiter (write data - locked to AW grant) - B channel arbiter (write response - routed by ID) - AR channel arbiter (read address) - R channel arbiter (read data - routed by ID) - Round-robin with burst locking - Separate read/write paths (no head-of-line blocking)

3. Data Multiplexing - Mux master signals to selected slave - ID-based response routing (B, R channels) - Burst tracking (hold grant until xlast)

4. Transaction Tracking - ID tables per slave for out-of-order support - Track {Master ID, Transaction ID} → Master mapping - Required for routing B/R channels back to correct master

5. Optional Performance Counters - Transaction counts per master/slave - Arbitration conflict counts - Latency histograms

## 23.6	3. Functional Requirements

### 23.6.1	3.1 AXI4 Protocol Compliance

FR-1: Full AXI4 Protocol Support - Support all 5 AXI4 channels: AW, W, B, AR, R - Comply with AMBA AXI4 specification (ARM IHI 0022) - Support burst lengths: 1-256 beats (AWLEN/ARLEN = 0-255) - Support burst types: INCR, WRAP, FIXED - Support burst sizes: 1-128 bytes (AWSIZE/ARSIZE = 0-7)

FR-2: Out-of-Order Transaction Support - Route responses via ID matching - Maintain transaction ID integrity (AWID → BID, ARID → RID) - Support configurable ID width (1-16 bits) - Track up to 2^ID_WIDTH outstanding transactions per slave

FR-3: Atomic Operations - Support exclusive access (AWLOCK/ARLOCK) - Track exclusive monitor per slave - Generate BRESP/RRESP errors for failed exclusives

### 23.6.2	3.2 Address Decoding

FR-4: Configurable Address Map - Support M masters × S slaves - Each slave has base address and size - Non-overlapping address ranges (verified at generation) - Default slave for unmapped addresses (optional)

FR-5: Address Range Configuration

# Example address map
address_map = {
    0: {'base': 0x00000000, 'size': 0x40000000, 'name': 'DDR'},      # 1GB
    1: {'base': 0x40000000, 'size': 0x00100000, 'name': 'SRAM'},     # 1MB
    2: {'base': 0x50000000, 'size': 0x01000000, 'name': 'PCIE'},     # 16MB
    3: {'base': 0x60000000, 'size': 0x00010000, 'name': 'Peripherals'} # 64KB
}

### 23.6.3	3.3 Arbitration Strategy

FR-6: Round-Robin Arbitration - Fair bandwidth allocation (no starvation) - Separate arbiters for AW and AR channels - Burst locking: Grant held until xlast (WLAST/RLAST) - Configurable arbitration policy (round-robin default)

FR-7: Read/Write Independence - Separate read and write paths - No head-of-line blocking between read/write - Concurrent read and write to same slave (if slave supports)

### 23.6.4	3.4 Burst Handling

FR-8: Burst Optimization - Pipelined burst transfers (overlap address and data) - No artificial burst splitting - Full AXI4 burst protocol support

FR-9: Interleaving Constraints - W channel locked to AW grant master - R channel routed by transaction ID - Support ID-based interleaving (slave-dependent)

## 23.7	4. Non-Functional Requirements

### 23.7.1	4.1 Performance

NFR-1: Latency - Single-beat read: ≤ 3 cycles (address decode + arbitration + mux) - Single-beat write: ≤ 3 cycles (address decode + arbitration + mux) - Burst transfer: No additional latency per beat (pipelined)

NFR-2: Throughput - Concurrent transfers: All M×S paths can transfer simultaneously - Burst efficiency: Line-rate data transfer after address phase - No artificial stalls: Crossbar adds no wait states beyond arbitration

NFR-3: Fmax - Target: 300-400 MHz on Xilinx UltraScale+ - Registered outputs: All slave outputs registered for timing closure - Pipelineable: Optional pipeline stages for >400 MHz

### 23.7.2	4.2 Resource Usage (Estimated)

M = 4 masters, S = 4 slaves, DATA_WIDTH = 512, ADDR_WIDTH = 64, ID_WIDTH = 4:

| Resource | Flat Crossbar | Notes |
| --- | --- | --- |
| LUTs | ~2,500 | Address decode + arbiters + mux |
| FFs | ~3,000 | Registered outputs + ID tables |
| BRAM | 0 | Distributed RAM for small ID tables |
| DSP | 0 | No arithmetic operations |

Scaling: ~150 LUTs per M×S connection

### 23.7.3	4.3 Quality Requirements

NFR-4: Code Generation Quality - Lint-clean SystemVerilog (Verilator) - Synthesizable (Vivado, Yosys, Design Compiler) - No vendor-specific primitives (technology-agnostic) - Clear structure (commented, readable)

NFR-5: Verification - CocoTB testbench framework - Transaction-level verification - Out-of-order test scenarios - Burst interleaving tests - >95% functional coverage

## 23.8	5. Interface Specifications

### 23.8.1	5.1 AXI4 Master Interfaces (M × 5 channels)

Write Address Channel (AW):

input  logic [ADDR_WIDTH-1:0]   s_axi_awaddr  [NUM_MASTERS];
input  logic [ID_WIDTH-1:0]     s_axi_awid    [NUM_MASTERS];
input  logic [7:0]              s_axi_awlen   [NUM_MASTERS];  // Burst length-1
input  logic [2:0]              s_axi_awsize  [NUM_MASTERS];  // Burst size
input  logic [1:0]              s_axi_awburst [NUM_MASTERS];  // INCR/WRAP/FIXED
input  logic                    s_axi_awlock  [NUM_MASTERS];  // Exclusive access
input  logic [3:0]              s_axi_awcache [NUM_MASTERS];  // Cache attributes
input  logic [2:0]              s_axi_awprot  [NUM_MASTERS];  // Protection type
input  logic                    s_axi_awvalid [NUM_MASTERS];
output logic                    s_axi_awready [NUM_MASTERS];

Write Data Channel (W):

input  logic [DATA_WIDTH-1:0]   s_axi_wdata  [NUM_MASTERS];
input  logic [DATA_WIDTH/8-1:0] s_axi_wstrb  [NUM_MASTERS];  // Byte strobes
input  logic                    s_axi_wlast  [NUM_MASTERS];  // Last beat
input  logic                    s_axi_wvalid [NUM_MASTERS];
output logic                    s_axi_wready [NUM_MASTERS];

Write Response Channel (B):

output logic [ID_WIDTH-1:0]     s_axi_bid    [NUM_MASTERS];
output logic [1:0]              s_axi_bresp  [NUM_MASTERS];  // OKAY/EXOKAY/SLVERR/DECERR
output logic                    s_axi_bvalid [NUM_MASTERS];
input  logic                    s_axi_bready [NUM_MASTERS];

Read Address Channel (AR):

input  logic [ADDR_WIDTH-1:0]   s_axi_araddr  [NUM_MASTERS];
input  logic [ID_WIDTH-1:0]     s_axi_arid    [NUM_MASTERS];
input  logic [7:0]              s_axi_arlen   [NUM_MASTERS];
input  logic [2:0]              s_axi_arsize  [NUM_MASTERS];
input  logic [1:0]              s_axi_arburst [NUM_MASTERS];
input  logic                    s_axi_arlock  [NUM_MASTERS];
input  logic [3:0]              s_axi_arcache [NUM_MASTERS];
input  logic [2:0]              s_axi_arprot  [NUM_MASTERS];
input  logic                    s_axi_arvalid [NUM_MASTERS];
output logic                    s_axi_arready [NUM_MASTERS];

Read Data Channel (R):

output logic [DATA_WIDTH-1:0]   s_axi_rdata  [NUM_MASTERS];
output logic [ID_WIDTH-1:0]     s_axi_rid    [NUM_MASTERS];
output logic [1:0]              s_axi_rresp  [NUM_MASTERS];
output logic                    s_axi_rlast  [NUM_MASTERS];
output logic                    s_axi_rvalid [NUM_MASTERS];
input  logic                    s_axi_rready [NUM_MASTERS];

### 23.8.2	5.2 AXI4 Slave Interfaces (S × 5 channels)

Mirror of master interfaces, with M → S direction reversed.

### 23.8.3	5.3 Configuration Parameters

parameter int NUM_MASTERS   = 4;          // Number of master interfaces
parameter int NUM_SLAVES    = 4;          // Number of slave interfaces
parameter int DATA_WIDTH    = 512;        // Data bus width (bits)
parameter int ADDR_WIDTH    = 64;         // Address bus width (bits)
parameter int ID_WIDTH      = 4;          // Transaction ID width (bits)
parameter int MAX_BURST_LEN = 256;        // Maximum burst length (beats)
parameter bit PIPELINE_OUTPUTS = 1;       // Register slave outputs
parameter bit ENABLE_COUNTERS  = 1;       // Performance counters

## 23.9	6. Performance Modeling

### 23.9.1	6.1 Analytical Model

Latency Components (Flat Crossbar):

Single-Beat Read Latency:
  1. Address Decode:      0 cycles (combinatorial)
  2. Arbitration:         1 cycle  (AR arbiter decision)
  3. Address Mux:         0 cycles (combinatorial)
  4. Slave Access:        (slave-dependent, not crossbar)
  5. Response Mux:        1 cycle  (R channel ID lookup + mux)
  6. Output Register:     1 cycle  (optional, for timing closure)

  Total (no pipeline):    2 cycles
  Total (pipelined):      3 cycles

Burst Transfer Throughput:

After address phase completes, data transfer is line-rate:
  - W channel: 1 beat/cycle (locked to AW grant)
  - R channel: 1 beat/cycle (ID-routed from slave)

Example: 256-beat burst
  Address phase:  2-3 cycles (one-time)
  Data phase:     256 cycles (line-rate)
  Total:          258-259 cycles
  Efficiency:     98.8%

Concurrent Throughput:

Maximum concurrent transfers (4×4 crossbar):
  - Read:  4 concurrent (one per master-slave pair)
  - Write: 4 concurrent (one per master-slave pair)
  - Total: 8 concurrent read+write (separate paths)

Throughput @ 100 MHz, 512-bit data:
  Per path:   100 MHz × 512 bits = 51.2 Gbps
  Total:      8 paths × 51.2 Gbps = 409.6 Gbps theoretical

### 23.9.2	6.2 Resource Scaling

LUT Usage Formula (empirical):

LUTs ≈ 500 (base) + 150 × M × S + 20 × ID_TABLE_DEPTH

Example (4×4 crossbar, ID_WIDTH=4, ID_TABLE_DEPTH=16 per slave):

LUTs ≈ 500 + 150 × 16 + 20 × 16 × 4
     ≈ 500 + 2,400 + 1,280
     ≈ 4,180 LUTs

### 23.9.3	6.3 Comparison with Other Crossbars

| Crossbar Type | Latency | Throughput | Complexity | Use Case |
| --- | --- | --- | --- | --- |
| APB | 1-2 cycles | Low (serialized) | Low | Control registers |
| AXI-Stream (Delta) | 2 cycles | High (streaming) | Medium | Data streaming |
| AXI4 (Bridge) | 2-3 cycles | High (burst) | High | Memory-mapped I/O |
| AXI4 + Slices | 4-6 cycles | High (burst) | Very High | >400 MHz designs |

Bridge Sweet Spot: High-performance memory-mapped interconnects where out-of-order and burst efficiency are critical.

## 23.10	7. Generator Architecture

### 23.10.1	7.1 Python Generator Structure

class BridgeGenerator:
    """AXI4 crossbar RTL generator"""

    def __init__(self, config):
        self.num_masters = config.num_masters
        self.num_slaves = config.num_slaves
        self.data_width = config.data_width
        self.addr_width = config.addr_width
        self.id_width = config.id_width
        self.address_map = config.address_map

    def generate_address_decode(self) -> str:
        """Generate address range decode logic"""
        # For each master × slave, check if address in range
        # More complex than AXIS (uses address), simpler than full decode

    def generate_aw_arbiter(self, slave_idx) -> str:
        """Generate write address channel arbiter for one slave"""
        # Round-robin arbiter
        # Grants locked until corresponding B response completes

    def generate_ar_arbiter(self, slave_idx) -> str:
        """Generate read address channel arbiter for one slave"""
        # Round-robin arbiter
        # Grants locked until corresponding R response completes (RLAST)

    def generate_w_channel_mux(self, slave_idx) -> str:
        """Generate write data channel multiplexer"""
        # W channel follows AW grant (locked until WLAST)

    def generate_b_channel_demux(self, slave_idx) -> str:
        """Generate write response channel demultiplexer"""
        # Route by ID: lookup master from {slave_idx, BID}

    def generate_r_channel_demux(self, slave_idx) -> str:
        """Generate read data channel demultiplexer"""
        # Route by ID: lookup master from {slave_idx, RID}

    def generate_id_table(self, slave_idx) -> str:
        """Generate transaction ID tracking table"""
        # Maps {slave, transaction_id} → master_id
        # Indexed on AW/AR grant, looked up on B/R response

    def generate_crossbar(self) -> str:
        """Generate complete crossbar module"""
        # Instantiate all arbiters, muxes, demuxes, ID tables

### 23.10.2	7.2 Address Map Configuration

Python Configuration:

bridge_config = {
    'num_masters': 4,
    'num_slaves': 4,
    'data_width': 512,
    'addr_width': 64,
    'id_width': 4,
    'address_map': {
        0: {'base': 0x00000000, 'size': 0x40000000, 'name': 'DDR'},
        1: {'base': 0x40000000, 'size': 0x00100000, 'name': 'SRAM'},
        2: {'base': 0x50000000, 'size': 0x01000000, 'name': 'PCIE'},
        3: {'base': 0x60000000, 'size': 0x00010000, 'name': 'Peripherals'}
    },
    'pipeline_outputs': True,
    'enable_counters': True
}

Generated Address Decode:

// Address decode for each master
always_comb begin
    for (int s = 0; s < NUM_SLAVES; s++)
        aw_request_matrix[s] = '0;

    for (int m = 0; m < NUM_MASTERS; m++) begin
        if (s_axi_awvalid[m]) begin
            // Slave 0: DDR (0x00000000 - 0x3FFFFFFF)
            if (s_axi_awaddr[m] >= 64'h00000000 &&
                s_axi_awaddr[m] < 64'h40000000)
                aw_request_matrix[0][m] = 1'b1;

            // Slave 1: SRAM (0x40000000 - 0x400FFFFF)
            if (s_axi_awaddr[m] >= 64'h40000000 &&
                s_axi_awaddr[m] < 64'h40100000)
                aw_request_matrix[1][m] = 1'b1;

            // ... slaves 2-3 ...
        end
    end
end

## 23.11	8. Comparison with APB and Delta

### 23.11.1	8.1 Code Reuse from APB Generator

Similar Components (~70% reuse): - Address range decode logic (same pattern, different signals) - Round-robin arbiter (same algorithm) - Data multiplexing pattern (same approach) - Backpressure handling (similar to PREADY)

New Components for Bridge: - 5× the arbiters (AW, W, B, AR, R instead of single channel) - ID-based routing (B and R channel demuxing) - Transaction tracking (ID tables for out-of-order) - Burst handling (grant locking until xlast)

Migration Effort from APB: - ~120 minutes (vs ~75 min for AXIS, due to higher complexity) - Most time: ID table logic and response demuxing

### 23.11.2	8.2 Code Reuse from Delta Generator

Similar Components (~60% reuse): - Python generation framework - Command-line interface - Performance modeling structure - Arbitration pattern (round-robin with locking)

Key Differences: - 5 channels vs 1 channel (Delta only has TDATA/TVALID/TREADY/TLAST) - ID-based routing vs TDEST-based routing - Address decode vs TDEST decode (Bridge more complex) - Transaction tracking vs packet atomicity (different mechanisms)

### 23.11.3	8.3 Complexity Comparison

| Metric | APB Crossbar | Delta (AXIS) | Bridge (AXI4) |
| --- | --- | --- | --- |
| Channels to arbitrate | 1 | 1 | 5 ★ |
| Request generation | Address ranges | TDEST decode | Address ranges |
| Response routing | Grant-based | Grant-based | ID-based ★ |
| Burst support | No | Packet (TLAST) | Yes (AWLEN/ARLEN) ★ |
| Out-of-order | No | No | Yes (ID tables) ★ |
| Transaction tracking | No | No | Yes ★ |
| Lines of Python | ~500 | ~697 | ~900 (est.) |
| Lines of generated SV | ~200 (4×4) | ~250 (4×4) | ~400 (4×4) (est.) |

★ = Additional complexity in Bridge

## 23.12	9. Use Cases

### 23.12.1	9.1 Multi-Core Processor Interconnect

Scenario: 4 CPU cores + GPU accessing DDR + SRAM + Peripherals

Configuration:
  Masters:  5 (4 CPUs, 1 GPU)
  Slaves:   3 (DDR, SRAM, Peripherals)
  Data:     512-bit (cache line width)
  Address:  64-bit (large memory space)
  ID:       4-bit (up to 16 outstanding per master)

Benefits: - Concurrent access to all slaves - Out-of-order completion for high-performance CPUs - Burst transfers for cache line fills - Separate read/write paths (no head-of-line blocking)

### 23.12.2	9.2 DMA + Accelerator System

Scenario: DMA engine + 4 accelerators accessing shared memory

Configuration:
  Masters:  5 (1 DMA, 4 accelerators)
  Slaves:   2 (DDR, Control registers)
  Data:     512-bit (high-bandwidth DMA)
  Address:  32-bit (limited address space)
  ID:       2-bit (simple ID space)

Benefits: - High-bandwidth memory access for DMA - Fair arbitration prevents accelerator starvation - Control register access doesn’t block data transfers

### 23.12.3	9.3 FPGA System Integration

Scenario: MicroBlaze CPU + custom accelerators + memory controllers

Configuration:
  Masters:  8 (1 CPU, 7 accelerators)
  Slaves:   4 (DDR, BRAM, AXI GPIO, AXI DMA)
  Data:     128-bit (AXI4 standard width)
  Address:  32-bit (standard FPGA address space)
  ID:       4-bit (moderate outstanding transactions)

Benefits: - Standard AXI4 interfaces (Xilinx IP compatibility) - Scalable to many masters/slaves - Performance counters for profiling

## 23.13	10. Testing Strategy

### 23.13.1	10.1 FUB (Functional Unit Block) Tests

Address Decode Tests: - Verify all address ranges correctly decoded - Test boundary conditions (base, base+size-1) - Test unmapped addresses (error response)

Arbiter Tests: - Round-robin fairness (all masters get turns) - Burst locking (grant held until xlast) - Starvation prevention

ID Table Tests: - Correct ID → master mapping - Out-of-order transaction handling - Table full condition

Mux/Demux Tests: - Data integrity through crossbar - Response routing to correct master - Concurrent transfers don’t interfere

### 23.13.2	10.2 Integration Tests

Single-Master, Single-Slave: - Basic read/write transactions - Burst transfers (various lengths) - Out-of-order completions

Multi-Master, Single-Slave: - Arbitration correctness - Fairness verification - Burst interleaving (if supported)

Multi-Master, Multi-Slave: - Concurrent transfers - No crosstalk between paths - Full M×S matrix coverage

Stress Tests: - All masters active simultaneously - Maximum burst lengths - Full ID space utilization - Back-to-back bursts

### 23.13.3	10.3 Performance Validation

Latency Measurement: - Single-beat read: measure actual vs analytical - Single-beat write: measure actual vs analytical - Compare with specification (≤3 cycles)

Throughput Measurement: - Burst transfer efficiency (should be ~98%) - Concurrent path throughput (should be line-rate × M×S) - Compare with theoretical maximum

Resource Validation: - Synthesize for Xilinx UltraScale+ - Compare LUT/FF usage with estimates - Verify Fmax ≥ 300 MHz

## 23.14	11. Documentation Plan

### 23.14.1	11.1 Specifications (Before Code)

- ☒ PRD.md - This document (complete requirements)

- ☐ README.md - User guide with quick start

- ☐ BRIDGE_VS_APB_GENERATOR.md - Migration guide from APB

- ☐ BRIDGE_VS_DELTA_GENERATOR.md - Comparison with Delta

- ☐ AXI4_PROTOCOL_GUIDE.md - AXI4 primer for students

### 23.14.2	11.2 Performance Analysis (Before Implementation)

- ☐ bin/bridge_performance_model.py - Analytical model

  - Latency calculations (single-beat, burst)

  - Throughput estimates (concurrent paths)

  - Resource estimates (LUT/FF scaling)

- ☐ bin/bridge_simulator.py - Discrete event simulation (optional)

  - Cycle-accurate modeling

  - Traffic pattern support

  - Validation against RTL

### 23.14.3	11.3 Code Generation

- ☐ bin/bridge_generator.py - Main RTL generator

  - Command-line interface

  - Address map configuration

  - Generated RTL output

### 23.14.4	11.4 Verification

- ☐ dv/tests/ - CocoTB testbenches

  - FUB tests (individual blocks)

  - Integration tests (multi-block)

  - Stress tests (corner cases)

## 23.15	12. Success Metrics

### 23.15.1	12.1 Functional Completeness

- ☐ Generates working AXI4 crossbar RTL

- ☐ Passes all CocoTB tests

- ☐ Verilator lint clean

- ☐ Xilinx Vivado synthesis clean

### 23.15.2	12.2 Performance Targets

- ☐ Latency ≤ 3 cycles (measured in simulation)

- ☐ Throughput = line-rate × M×S paths (measured)

- ☐ Fmax ≥ 300 MHz (post-synthesis)

### 23.15.3	12.3 Educational Value

- ☐ Complete specifications demonstrating rigor

- ☐ Performance models validate requirements

- ☐ Clear code structure (readable generated RTL)

- ☐ Integration examples provided

### 23.15.4	12.4 Reusability

- ☐ ~70% code reuse from APB generator (measured by LOC)

- ☐ Similar patterns to Delta generator

- ☐ Can be extended (weighted arbitration, QoS, etc.)

## 23.16	12. Attribution and Contribution Guidelines

### 23.16.1	12.1 Git Commit Attribution

When creating git commits for Bridge documentation or implementation:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: Bridge documentation and organization receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

## 23.17	12.2 PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

/mnt/data/github/rtldesignsherpa/projects/components/bridge/docs/

Quick Command: Use the provided shell script:

cd /mnt/data/github/rtldesignsherpa/projects/components/bridge/docs
./generate_pdf.sh

The shell script will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the bridge_spec index file 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

## 23.18	13. Future Enhancements

### 23.18.1	13.1 Short-Term (Post-Initial Release)

- ☐ Optional pipeline stages - For Fmax >400 MHz

- ☐ Weighted arbitration - QoS support

- ☐ Default slave - Unmapped address handling

- ☐ Exclusive monitor - Full atomic operation support

### 23.18.2	13.2 Long-Term

- ☐ Tree topology - Hierarchical crossbar (like Delta)

- ☐ AXI4-Lite variant - Simplified for control registers

- ☐ ACE support - Coherent cache interconnect

- ☐ GUI configurator - Visual address map setup

## 23.19	14. Risk Assessment

| Risk | Probability | Impact | Mitigation |
| --- | --- | --- | --- |
| ID table complexity | Medium | High | Start with small ID_WIDTH (2-4), test thoroughly |
| Out-of-order corner cases | High | High | Extensive CocoTB tests with random delays |
| Fmax below target | Low | Medium | Optional pipeline stages for timing closure |
| Resource usage exceeds | Low | Low | Empirical formulas guide expectations |
| Burst interleaving bugs | Medium | High | Separate test suite for burst scenarios |

## 23.20	15. Project Timeline (Estimated)

Week 1: Specifications and Models - [x] Day 1-2: PRD.md (complete) - [ ] Day 3-4: Performance modeling (analytical) - [ ] Day 5-7: README.md, migration guides

Week 2-3: Core Implementation - [ ] Day 1-3: Address decode + arbiter generation - [ ] Day 4-5: Data mux/demux generation - [ ] Day 6-8: ID table generation - [ ] Day 9-10: Integration and testing

Week 4: Verification and Examples - [ ] Day 1-5: CocoTB testbenches - [ ] Day 6-7: Integration examples

## 23.21	16. References

AXI4 Specifications: - ARM IHI 0022 - AMBA AXI and ACE Protocol Specification - Xilinx UG1037 - Vivado AXI Reference Guide

Related Projects: - APB Crossbar - Simple register bus crossbar (existing) - Delta (AXIS Crossbar) - Streaming data crossbar (projects/components/delta/) - RAPIDS - DMA engine with AXI4 masters (rtl/miop/)

Tools: - Verilator - RTL linting and simulation - CocoTB - Python-based verification - Xilinx Vivado - FPGA synthesis

## 23.22	17. Glossary

- AXI4: Advanced eXtensible Interface version 4 (AMBA standard)

- Burst: Multi-beat transaction (AWLEN/ARLEN > 0)

- Crossbar: Full M×N interconnect matrix

- ID: Transaction identifier for out-of-order support

- Out-of-order: Responses can return in different order than requests

- xlast: WLAST (write) or RLAST (read) - last beat indicator

Version: 1.0 Status: ✓ Specification Complete - Ready for Implementation Next Steps: Create performance models, then implement generator

Project Bridge - Connecting masters and slaves across the divide 🌉

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Claude Code Guide: Bridge Subsystem

Version: 2.1 Last Updated: 2025-11-03 Purpose: AI-specific guidance for working with Bridge subsystem

## 24.1	🚨 CRITICAL: Read Architecture Document First

Before making ANY changes to bridge generator or understanding signal flow:

📖 READ: projects/components/bridge/docs/BRIDGE_ARCHITECTURE.md

This document contains the definitive bridge architecture including: - Correct signal flow (wrappers → decoder → converters → crossbar → slaves) - Component purposes and placement - Common misconceptions that previous agents made - Why there’s NO fixed crossbar width

If you skip this document, you WILL make incorrect assumptions.

## 24.2	Quick Context

What: Bridge - Two complementary AXI4 Crossbar Generators (framework-based and CSV-based) Status: 🟢 Phase 2 Complete - CSV generator with channel-specific masters (wr/rd/rw) Your Role: Help users configure CSV files, generate bridges, understand architecture, and create tests

📖 Complete Documentation (Read in This Order): 1. projects/components/bridge/docs/BRIDGE_ARCHITECTURE.md ← START HERE (architecture reference) 2. projects/components/bridge/PRD.md ← Product requirements 3. projects/components/bridge/BRIDGE_CURRENT_STATE.md ← Current implementation review 4. projects/components/bridge/docs/BRIDGE_ARCHITECTURE_DIAGRAMS.md ← Visual architecture diagrams 5. projects/components/bridge/CSV_BRIDGE_STATUS.md ← CSV generator status (Phase 1 & 2) 6. projects/components/bridge/docs/bridge_spec/bridge_index.md ← Detailed specification

## 24.3	Target Architecture: Intelligent Width-Aware Routing

Core Principle: Direct connections where possible, converters only where needed, no fixed crossbar width.

### 24.3.1	Efficient Multi-Width Design

Master_A (64b)
  ├─ Direct → Slave_0 (64b)           [0 conversions, minimal latency]
  ├─ Conv(64→128) → Slave_1 (128b)    [1 conversion]
  └─ Conv(64→512) → Slave_2 (512b)    [1 conversion]

Master_B (512b)
  ├─ Conv(512→64) → Slave_0 (64b)     [1 conversion]
  ├─ Conv(512→128) → Slave_1 (128b)   [1 conversion]
  └─ Direct → Slave_2 (512b)          [0 conversions, full bandwidth]

Router Logic: Address decoder determines target slave, selects correctly-sized path for that master-slave pair.

### 24.3.2	Why This Architecture

✗ Naive Fixed-Width Approach (Don’t Do This):

Master (64b) → Upsize(64→256) → Fixed 256b Crossbar → Downsize(256→64) → Slave (64b)

- TWO conversions for same-width connections (wasteful!)

- Reduced bandwidth on narrow paths

- Unnecessary logic and area

- Higher latency

✓ Intelligent Routing (Target):

Master (64b) → Router → Direct Connection → Slave (64b)

- ZERO conversions for matching widths

- Full native bandwidth

- Minimal logic

- Lowest latency

### 24.3.3	Per-Master Output Paths

Each master has N output paths (one per unique slave width it connects to):

// Master_A connects to slaves at 64b, 128b, 512b
// Generate 3 output paths:

logic [63:0]  master_a_64b_wdata;   // For 64b slaves (direct)
logic [127:0] master_a_128b_wdata;  // For 128b slaves (via converter)
logic [511:0] master_a_512b_wdata;  // For 512b slaves (via converter)

// Router selects based on address decode:
always_comb begin
    case (decoded_slave_id)
        SLAVE_0: select master_a_64b_wdata;   // Slave_0 is 64b
        SLAVE_1: select master_a_128b_wdata;  // Slave_1 is 128b
        SLAVE_2: select master_a_512b_wdata;  // Slave_2 is 512b
    endcase
end

### 24.3.4	Benefits

- Resource Efficient - Only instantiate converters actually needed

- Maximum Performance - Direct paths have zero conversion overhead

- Optimal Bandwidth - No artificial width bottlenecks

- Lower Latency - Minimal logic in critical path for matching widths

- Scalable - Works for any combination of master/slave widths

### 24.3.5	Implementation Status

Current State: Fixed-width crossbar with master-side upsizing (Phase 1 architecture) Target State: Intelligent per-master multi-width routing (your vision) Migration: Requires generator architecture rework (see TASKS.md)

## 24.4	🚨 CRITICAL RULE #0: RTL Regeneration Requirements

⚠ READ THIS FIRST - FAILURE TO FOLLOW CAUSES TEST FAILURES ⚠

### 24.4.1	The Golden Rule

ALL generated RTL files MUST be deleted and regenerated together whenever ANY generator code changes.

### 24.4.2	Why This Is Non-Negotiable

Generated RTL files have interdependencies: - bridge_axi4_flat_*.sv may be instantiated by bridge_ooo_with_arbiter.sv - bridge_wrapper_*.sv may wrap bridge_axi4_flat_*.sv - Generator changes affect signal names, port widths, interface structure - Partial regeneration creates version mismatches that cause silent failures

### 24.4.3	Real Example (This Session)

# ✗ WHAT WENT WRONG:
1. Updated bridge_address_arbiter.py (address decode logic)
2. Regenerated ONLY bridge_axi4_flat_5x3.sv
3. Did NOT regenerate bridge_ooo_with_arbiter.sv (wrapper)
4. Result: All tests that were passing now FAIL
5. Cause: Version mismatch between wrapper and core bridge

# ✓ WHAT SHOULD HAVE BEEN DONE:
1. Updated bridge_address_arbiter.py
2. Delete ALL generated files:
   rm rtl/bridge_axi4_flat_*.sv
   rm rtl/bridge_ooo_*.sv
   rm rtl/bridge_wrapper_*.sv
3. Regenerate ALL bridges from scratch
4. Run ALL tests to verify

### 24.4.4	Generator Files That Trigger Full Regeneration

Any change to these files requires regenerating ALL bridges:

- ✓ bridge_generator.py - Core bridge generator

- ✓ bridge_csv_generator.py - CSV-based generator

- ✓ bridge_address_arbiter.py - Address decode logic

- ✓ bridge_channel_router.py - Channel routing

- ✓ bridge_response_router.py - Response routing

- ✓ bridge_amba_integrator.py - AMBA integration

- ✓ bridge_wrapper_generator.py - Wrapper generation

- ✓ ANY Python file in projects/components/bridge/bin/

### 24.4.5	The Regeneration Workflow

# Step 1: Make generator code changes
vim bridge_address_arbiter.py

# Step 2: Delete ALL generated RTL (be aggressive!)
cd projects/components/bridge/rtl
rm bridge_axi4_flat_*.sv
rm bridge_ooo*.sv
rm bridge_wrapper_*.sv
# Verify deletion
ls *.sv  # Should only show manually-written files like bridge_cam.sv

# Step 3: Regenerate everything
cd ../bin
./regenerate_all_bridges.sh  # If script exists
# OR manually regenerate each topology needed

# Step 4: Run ALL tests
cd ../dv/tests
pytest -v  # ALL tests, not just the one you think changed

# Step 5: Verify git diff makes sense
git diff ../rtl/  # Review all changes

### 24.4.6	Symptoms of Version Mismatch

If you see these symptoms, you probably did partial regeneration:

- ✗ Tests that previously passed now fail

- ✗ “Signal not found” errors in simulation

- ✗ Port width mismatches in instantiation

- ✗ Address decode routing to wrong slaves

- ✗ Missing debug signals (dbg_*)

- ✗ Unexpected compile errors in working code

### 24.4.7	Think Like a Compiler Developer

Generated RTL = Compiled Object Files

When you update a compiler, you don’t selectively recompile - you make clean && make all.

When you update a generator, you don’t selectively regenerate - you delete all and regenerate all.

### 24.4.8	Exception: Hand-Written RTL

These files are never regenerated: - bridge_cam.sv - CAM module (hand-written) - Any file in rtl/ that is NOT generated

Check file headers - generated files say “Generated by: bridge_generator.py”

## 24.5	Critical Pitfalls: slave_select_* Reversion After Handshake

### 24.5.1	The Problem: Combinational Address Decode

The master adapter’s R/B response MUX and the crossbar’s AR/AW gating both relied on <master>_slave_select_{ar,aw} — a combinational decode of fub_axi_{ar,aw}addr. This signal reverts immediately when the address port is freed:

- AR/AW handshake completes (arvalid && arready, awvalid && awready)

- Wrapper pops skid buffer and releases the address port

- fub_axi_araddr / fub_axi_awaddr reverts to next transaction or zero

- Decode flips to different slave

- Response MUX is now gated on WRONG slave

- Response path closes, transaction hangs

This happens BEFORE the converter even finishes pushing to the crossbar, and ALWAYS before response arrives.

### 24.5.2	Visible Symptoms

- ✗ Multi-slave multi-master bridges hang (timeout)

- ✗ Multi-width masters fail basic connectivity

- ✗ APB/AXIL slaves consistently timeout (longer converter latency = more decode changes)

- ✓ Single-slave or single-master systems work fine (no decode change)

### 24.5.3	The Solution (MANDATORY in All Generators)

For Crossbar AR/AW Gating: - Replace <master>_slave_select_ar/aw gates with inline address re-decode on stable m_axi buses - Address on m_axi is held stable across the entire AXI handshake

For Master Adapter Response MUX: - Add per-channel response-tracking FIFOs that capture slave index at request time - Use FIFO head for response MUX instead of live combinational decode

Crossbar Code Pattern:

// WRONG (old implementation):
assign gate_ar_s0 = m0_slave_select_ar && m0_arvalid;
//                  ^^^^^^^^^^^^^^^^^ Reverts after handshake!

// CORRECT (new implementation):
assign gate_ar_s0 = ((m0_m_axi_araddr >= S0_BASE) && 
                      (m0_m_axi_araddr < S0_END) && 
                      m0_arvalid);  // Re-decode on stable m_axi address

Master Adapter Code Pattern:

// WRONG (old implementation):
assign r_slave_select = comb_slave_select_ar;  // Stale after pop!

// CORRECT (new implementation):
// Capture at request time
always_ff @(posedge aclk) begin
    if (fub_axi_arvalid && fub_axi_arready) begin
        ar_trk_fifo <= comb_slave_select_ar;
    end
end

// Use stable value for response MUX
assign r_slave_select = ar_trk_fifo_out;

### 24.5.4	When This Bug Appears

- Works: Single-master, single-slave systems (no address re-evaluation)

- Fails: Multi-master to same slave (multiple addresses decode differently)

- Fails: Multi-slave same master (address decode changes per transaction)

- Fails: Multi-width paths (W beats re-evaluate AW decode mid-burst)

## 24.6	⚠ MANDATORY: Project Organization Pattern

THIS SUBSYSTEM MUST FOLLOW THE RAPIDS/AMBA ORGANIZATIONAL PATTERN - NO EXCEPTIONS

### 24.6.1	Required Directory Structure

projects/components/bridge/
├── bin/                           # Bridge-specific tools (generators, scripts)
│   └── bridge_generator.py       # AXI4 crossbar generator
├── docs/                          # Design documentation
├── dv/                            # Design Verification (MANDATORY structure)
│   ├── tbclasses/                 # Testbench classes (MANDATORY - TB classes here!)
│   │   ├── __init__.py
│   │   └── bridge_axi4_flat_tb.py  # Reusable TB class
│   ├── components/                # Bridge-specific BFMs (if needed)
│   │   └── __init__.py
│   ├── scoreboards/               # Bridge-specific scoreboards (if needed)
│   │   └── __init__.py
│   └── tests/                     # All test files (test runners only)
│       ├── conftest.py            # Pytest configuration (MANDATORY)
│       ├── fub_tests/             # Functional unit block tests
│       │   └── basic/             # Basic bridge tests
│       ├── integration_tests/     # Multi-bridge integration
│       └── system_tests/          # Full system tests
├── rtl/                           # RTL source files
│   └── generated/                 # Generated bridge crossbars
├── CLAUDE.md                      # This file
├── PRD.md                         # Product requirements
└── IMPLEMENTATION_STATUS.md       # Development status

### 24.6.2	Testbench Class Location (MANDATORY)

✗ WRONG: Testbench class in test file

# projects/components/bridge/dv/tests/test_bridge_axi4_2x2.py
class BridgeAXI4FlatTB:  # ✗ WRONG LOCATION!
    """Embedded TB - NOT REUSABLE"""

✓ CORRECT: Testbench class in PROJECT AREA dv/tbclasses/

# projects/components/bridge/dv/tbclasses/bridge_axi4_flat_tb.py
class BridgeAXI4FlatTB(TBBase):  # ✓ CORRECT LOCATION!
    """Reusable TB class - used across all bridge tests"""

CRITICAL: TB classes are PROJECT-SPECIFIC and MUST be in the project area (projects/components/{name}/dv/tbclasses/), NOT in the framework (bin/TBClasses/).

### 24.6.3	Test File Pattern (MANDATORY)

Test files MUST follow this structure:

# projects/components/bridge/dv/tests/fub_tests/basic/test_bridge_axi4_2x2.py

import os
import pytest
import cocotb
from cocotb_test.simulator import run

# ✓ IMPORT testbench class from PROJECT AREA
import sys
sys.path.insert(0, os.path.join(os.path.dirname(__file__), '../../../..'))
from projects.components.bridge.dv.tbclasses.bridge_axi4_flat_tb import BridgeAXI4FlatTB
from TBClasses.shared.utilities import get_paths, create_view_cmd
from TBClasses.shared.tbbase import TBBase

# ===========================================================================
# COCOTB TEST FUNCTIONS - Prefix with "cocotb_" to prevent pytest collection
# ===========================================================================

@cocotb.test(timeout_time=100, timeout_unit='us')
async def cocotb_test_basic_routing(dut):
    """Test basic address routing"""
    tb = BridgeAXI4FlatTB(dut, num_masters=2, num_slaves=2)  # ✓ Import TB
    await tb.setup_clocks_and_reset()
    result = await tb.test_basic_routing()
    assert result, "Basic routing test failed"

# More cocotb test functions...

# ===========================================================================
# PARAMETER GENERATION - At bottom of file
# ===========================================================================

def generate_bridge_test_params():
    """Generate test parameters for bridge tests"""
    return [
        # (num_masters, num_slaves, data_width, addr_width, id_width)
        (2, 2, 32, 32, 4),
        (4, 4, 32, 32, 4),
    ]

bridge_params = generate_bridge_test_params()

# ===========================================================================
# PYTEST WRAPPER FUNCTIONS - At bottom of file
# ===========================================================================

@pytest.mark.bridge
@pytest.mark.routing
@pytest.mark.parametrize("num_masters, num_slaves, data_width, addr_width, id_width", bridge_params)
def test_basic_routing(request, num_masters, num_slaves, data_width, addr_width, id_width):
    """Pytest wrapper for basic routing test"""
    module, repo_root, tests_dir, log_dir, rtl_dict = get_paths({
        'rtl_bridge': '../../rtl'
    })

    # ... setup verilog_sources, parameters, etc ...

    run(
        verilog_sources=verilog_sources,
        toplevel=f"bridge_axi4_flat_{num_masters}x{num_slaves}",
        module=module,
        testcase="cocotb_test_basic_routing",  # ← cocotb function name
        parameters=rtl_parameters,
        sim_build=sim_build,
        # ...
    )

## 24.7	Critical Rules for This Subsystem

### 24.7.1	Rule #0: Attribution Format for Git Commits

IMPORTANT: When creating git commit messages for Bridge documentation or code:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: Bridge receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

### 24.7.2	Rule #0.1: Testbench Architecture - MANDATORY SEPARATION

⚠ THIS IS A HARD REQUIREMENT - NO EXCEPTIONS ⚠

NEVER embed testbench classes inside test runner files!

The same testbench logic will be reused across multiple test scenarios. Having testbench code only in test files makes it COMPLETELY WORTHLESS for reuse.

MANDATORY Structure:

projects/components/bridge/
├── dv/
│   ├── tbclasses/                   # TB classes HERE (not in framework!)
│   │   ├── __init__.py
│   │   └── bridge_axi4_flat_tb.py  ← REUSABLE TB CLASS
│   ├── components/                  # Bridge-specific BFMs (if needed)
│   │   └── __init__.py
│   ├── scoreboards/                 # Bridge-specific scoreboards
│   │   └── __init__.py
│   └── tests/                       # Test runners
│       ├── conftest.py              ← MANDATORY pytest config
│       ├── fub_tests/
│       │   └── basic/
│       │       └── test_bridge_axi4_2x2.py  ← TEST RUNNER ONLY (imports TB)
│       ├── integration_tests/
│       │   └── test_bridge_multiport.py     ← TEST RUNNER ONLY
│       └── system_tests/
│           └── test_bridge_system.py         ← TEST RUNNER ONLY

Why This Matters:

- Reusability: Same TB class used in:

  - Unit tests (fub_tests/)

  - Integration tests (integration_tests/)

  - System-level tests (system_tests/)

  - User projects (external imports)

- Maintainability: Fix bug once in TB class, all tests benefit

- Composition: TB classes can inherit/compose for complex scenarios

### 24.7.3	Rule #1: All Testbenches Inherit from TBBase

Every testbench class MUST inherit from TBBase:

from TBClasses.shared.tbbase import TBBase

class BridgeAXI4FlatTB(TBBase):
    """Testbench for Bridge crossbar - inherits base functionality"""

    def __init__(self, dut, num_masters=2, num_slaves=2, **kwargs):
        super().__init__(dut)
        # Bridge-specific initialization

TBBase Provides: - Clock management (start_clock, wait_clocks) - Reset utilities (assert_reset, deassert_reset) - Logging (self.log) - Progress tracking (mark_progress) - Safety monitoring (timeouts, memory limits)

### 24.7.4	Rule #2: Mandatory Testbench Methods

Every testbench class MUST implement these three methods:

async def setup_clocks_and_reset(self):
    """Complete initialization - starts clocks and performs reset"""
    await self.start_clock('aclk', freq=10, units='ns')

    # Set config signals before reset (if needed)
    # self.dut.cfg_param.value = initial_value

    # Reset sequence
    await self.assert_reset()
    await self.wait_clocks('aclk', 10)
    await self.deassert_reset()
    await self.wait_clocks('aclk', 5)

async def assert_reset(self):
    """Assert reset signal (active-low for AXI4)"""
    self.dut.aresetn.value = 0

async def deassert_reset(self):
    """Deassert reset signal"""
    self.dut.aresetn.value = 1

Why Required: - Consistency across all testbenches - Reusability for mid-test resets - Clear test structure and intent

### 24.7.5	Rule #3: Use GAXI Components for Protocol Handling

For Bridge testing, use GAXI Master/Slave components for AXI4 channel handling:

from CocoTBFramework.components.gaxi.gaxi_master import GAXIMaster
from CocoTBFramework.components.gaxi.gaxi_slave import GAXISlave
from CocoTBFramework.components.axi4.axi4_field_configs import AXI4FieldConfigHelper

# ✓ CORRECT: Use GAXI for AXI4 channels
self.aw_master = GAXIMaster(
    dut=dut,
    title="AW_M0",
    prefix="s0_axi4_",
    clock=clock,
    field_config=AXI4FieldConfigHelper.create_aw_field_config(id_width, addr_width, 1),
    pkt_prefix="aw",
    multi_sig=True,
    log=log
)

Never manually drive AXI4 valid/ready handshakes - Use GAXI components.

### 24.7.6	Rule #4: Queue-Based Verification

For simple in-order verification, use direct monitor queue access:

# ✓ CORRECT: Direct queue access
aw_pkt = self.aw_slave._recvQ.popleft()
w_pkt = self.w_slave._recvQ.popleft()

# Verify
assert aw_pkt.addr == expected_addr
assert w_pkt.data == expected_data

# ✗ WRONG: Memory model for simple test
memory_model = MemoryModel()  # Unnecessary complexity

When to Use Memory Models: - ✗ Simple in-order tests → Use queue access - ✗ Single-master systems → Use queue access - ✓ Complex out-of-order scenarios → Memory model may help - ✓ Multi-master with address overlap → Memory model tracks state

## 24.8	TOML/CSV-Based Bridge Generator (Phase 2 Complete)

### 24.8.1	Overview

The Bridge generator creates parameterized SystemVerilog crossbars from TOML configuration files with CSV connectivity matrices, eliminating manual RTL editing for complex interconnects.

Key Benefits: - Human-readable configuration - TOML for ports, CSV for connectivity - Custom signal prefixes - Each port has unique prefix (rapids_m_axi_, apb0_, etc.) - Channel-specific masters - Write-only (wr), read-only (rd), or full (rw) - Interface modules - Timing isolation via axi4_master/slave wrappers with configurable skid depths - Automatic converters - Width and protocol conversion inserted automatically - Resource efficient - Only generates needed channels and converters

### 24.8.2	Quick Start

1. Create bridge_mybridge.toml:

[bridge]
  name = "bridge_mybridge"
  description = "Custom bridge example"

  # Default skid buffer depths (can be overridden per port)
  defaults.skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

  masters = [
    {name = "cpu", prefix = "cpu_m_axi", id_width = 4, addr_width = 32, data_width = 64, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "dma", prefix = "dma_m_axi", id_width = 4, addr_width = 32, data_width = 512, user_width = 1,
     interface = {type = "axi4_master", skid_depths = {ar = 4, r = 8, aw = 4, w = 8, b = 4}}}
  ]

  slaves = [
    {name = "ddr", prefix = "ddr_s_axi", id_width = 4, addr_width = 32, data_width = 512, user_width = 1,
     base_addr = 0x00000000, addr_range = 0x80000000, interface = {type = "axi4_slave"}},
    {name = "sram", prefix = "sram_s_axi", id_width = 4, addr_width = 32, data_width = 256, user_width = 1,
     base_addr = 0x80000000, addr_range = 0x80000000, interface = {type = "axi4_slave"}}
  ]

2. Create bridge_mybridge_connectivity.csv:

master\slave,ddr,sram
cpu,1,1
dma,1,1

3. Generate bridge:

cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_mybridge.toml
# Auto-finds bridge_mybridge_connectivity.csv

# Or use bulk generation
python3 bridge_generator.py --bulk bridge_batch.csv

Result: Complete SystemVerilog module with: - Custom port prefixes per port - Timing isolation via interface wrappers - Only needed AXI4 channels (wr/rd/rw optimized) - Width converters for data mismatches - Internal crossbar instantiation - APB/AXI4-Lite converter integration points

### 24.8.3	Configuration Format Details

TOML Port Configuration:

The primary format is now TOML (preferred over CSV for better structure and interface configuration):

Port Specifications: - name - Unique identifier (cpu, dma, ddr, etc.) - prefix - Signal prefix (cpu_m_axi_, ddr_s_axi_, etc.) - id_width - AXI4 ID width in bits - addr_width - Address width in bits - data_width - Data width in bits (32, 64, 128, 256, 512) - user_width - AXI4 user signal width - base_addr - Slave base address (slaves only) - addr_range - Slave address range (slaves only) - interface - Interface wrapper configuration (optional) - type - “axi4_master”, “axi4_slave”, “axi4_master_mon”, “axi4_slave_mon”, or omit for direct connection - skid_depths - Per-channel buffer depths: {ar, r, aw, w, b} (valid: 2, 4, 6, 8)

CSV Connectivity Matrix:

master\slave,slave0,slave1,slave2
master0,1,0,1
master1,0,1,1

- 1 = connected, 0 = not connected

- Partial connectivity supported (not all masters to all slaves)

- Auto-detected based on TOML filename: bridge_name.toml → bridge_name_connectivity.csv

Legacy CSV Format:

For backwards compatibility, the generator still supports CSV port files: - See test_configs/README.md for migration guide from CSV to TOML - TOML is now preferred for new bridges (better structure, interface config support)

### 24.8.4	Channel-Specific Masters (Phase 2 Feature)

Why Channel-Specific? Real hardware often has dedicated read or write masters. Generating all 5 AXI4 channels wastes resources:

Traditional (wasteful):

// Write-only master gets unused read channels
input  logic [63:0]  rapids_descr_m_axi_awaddr,  // ✓ USED
input  logic [511:0] rapids_descr_m_axi_wdata,   // ✓ USED
output logic [7:0]   rapids_descr_m_axi_bid,     // ✓ USED
input  logic [63:0]  rapids_descr_m_axi_araddr,  // ✗ UNUSED (50% waste!)
output logic [511:0] rapids_descr_m_axi_rdata,   // ✗ UNUSED

Channel-Specific (optimized):

rapids_descr_wr,master,axi4,wr,rapids_descr_m_axi_,512,64,8,N/A,N/A

Generated:

// Write-only master - only AW, W, B channels
input  logic [63:0]  rapids_descr_m_axi_awaddr,
input  logic [511:0] rapids_descr_m_axi_wdata,
output logic [7:0]   rapids_descr_m_axi_bid,
// ✓ NO READ CHANNELS (araddr, rdata, etc.)

Resource Savings: - 40-60% fewer ports for dedicated masters - Only necessary width converters instantiated - Channel-aware direct connection wiring - Faster synthesis, smaller netlists

### 24.8.5	Example: RAPIDS-Style Configuration

RAPIDS Architecture: - Descriptor write master (wr) - Writes descriptors to memory - Sink write master (wr) - Writes incoming packets to memory - Source read master (rd) - Reads outgoing packets from memory - CPU master (rw) - Full access for configuration

TOML Configuration (bridge_rapids.toml):

[bridge]
  name = "bridge_rapids"
  description = "RAPIDS accelerator bridge"
  defaults.skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

  masters = [
    {name = "rapids_descr_wr", prefix = "rapids_descr_m_axi", channels = "wr",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "rapids_sink_wr", prefix = "rapids_sink_m_axi", channels = "wr",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "rapids_src_rd", prefix = "rapids_src_m_axi", channels = "rd",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "cpu", prefix = "cpu_m_axi", channels = "rw",
     id_width = 4, addr_width = 32, data_width = 64, user_width = 1,
     interface = {type = "axi4_master"}}
  ]

  slaves = [
    {name = "ddr", prefix = "ddr_s_axi", protocol = "axi4",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     base_addr = 0x80000000, addr_range = 0x80000000,
     interface = {type = "axi4_slave"}},
    {name = "apb_periph", prefix = "apb0_", protocol = "apb",
     addr_width = 32, data_width = 32,
     base_addr = 0x00000000, addr_range = 0x00010000}
  ]

Connectivity CSV (bridge_rapids_connectivity.csv):

master\slave,ddr,apb_periph
rapids_descr_wr,1,0
rapids_sink_wr,1,0
rapids_src_rd,1,0
cpu,1,1

Generated RTL Features: - rapids_descr_wr: 37 signals (write channels only) vs 61 signals (full) = 39% reduction - rapids_sink_wr: 37 signals (write channels only) vs 61 signals (full) = 39% reduction - rapids_src_rd: 24 signals (read channels only) vs 61 signals (full) = 61% reduction - cpu_master: Width converters for 64b→512b upsize (both wr and rd converters) - ddr_controller: Direct 512b connection (no conversion) - apb_periph0: APB converter placeholder (Phase 3)

### 24.8.6	Common User Questions

Q: “How do I generate a bridge?”

A: Three steps:

- Create TOML port configuration file

- Create CSV connectivity matrix

- Run bridge_generator.py

# Create bridge_mybridge.toml (port configuration)
# Create bridge_mybridge_connectivity.csv (connectivity matrix)

cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_mybridge.toml
# Auto-finds bridge_mybridge_connectivity.csv

# Or use bulk generation for multiple bridges
python3 bridge_generator.py --bulk bridge_batch.csv

Q: “What’s the difference between wr/rd/rw channels?”

A: Number of AXI4 channels generated:

- rw (read/write) - All 5 channels: AW, W, B, AR, R

- wr (write-only) - 3 channels: AW, W, B (no read channels)

- rd (read-only) - 2 channels: AR, R (no write channels)

Benefits: 40-60% fewer ports for dedicated masters, less logic, faster synthesis

Q: “Can I add timing isolation to ports?”

A: Yes, use interface configuration:

masters = [
  {name = "cpu", prefix = "cpu_m_axi", ...,
   interface = {type = "axi4_master", skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}}}
]

Available interface types: - "axi4_master" - Timing isolation on master port - "axi4_slave" - Timing isolation on slave port - "axi4_master_mon" - Timing + monitoring - "axi4_slave_mon" - Timing + monitoring - Omit interface field for direct connection

Q: “Can I mix AXI4 and APB slaves?”

A: Yes, use protocol field in TOML:

slaves = [
  {name = "ddr", prefix = "ddr_s_axi", protocol = "axi4", ...},
  {name = "apb0", prefix = "apb0_", protocol = "apb", ...}
]

Generator inserts AXI2APB converters automatically (Phase 3 for full implementation).

Q: “What if data widths don’t match?”

A: Generator inserts width converters automatically:

masters = [
  {name = "cpu", data_width = 64, ...},  # 64b master
]
slaves = [
  {name = "ddr", data_width = 512, ...}  # 512b slave
]
# Generator creates width converter: 64b → 512b

Q: “Do all masters need to connect to all slaves?”

A: No, use partial connectivity in CSV:

master\slave,ddr,sram,apb
rapids_descr,1,1,0     # Connects to ddr and sram only
cpu,1,1,1              # Connects to all three

### 24.8.7	Generator Output Structure

Generated File Contains:

- Module Header - Parameterized with NUM_MASTERS, NUM_SLAVES, widths

- Port Declarations - Custom prefix per port, channel-specific signals

- Internal Signals - Crossbar interface arrays (xbar_m_, xbar_s_)

- Width Converters - Master-side upsize instances (channel-aware)

- Crossbar Instance - Internal AXI4 full crossbar

- Direct Connections - For matching-width interfaces

- APB Converters - Placeholder TODO comments (Phase 3)

Example Output Size: - Simple 2x2 bridge: ~400 lines - Complex 5x3 with mixed protocols: ~900 lines - Includes comprehensive comments and structure

### 24.8.8	Testing Generated Bridges

For Pure AXI4 Bridges (Working Now):

# Generate bridge
python3 bridge_csv_generator.py --ports ports.csv --connectivity conn.csv --name my_bridge --output ../rtl/

# Create testbench (use BridgeAXI4FlatTB from dv/tbclasses/)
cd ../dv/tests/fub_tests/basic
pytest test_my_bridge.py -v

For Mixed AXI4/APB (Requires Phase 3): APB converter placeholders need implementation before end-to-end testing.

## 24.9	Bridge Architecture Quick Reference

### 24.9.1	Generated Bridge Crossbar Structure

module bridge_axi4_flat_2x2 #(
    parameter NUM_MASTERS = 2,
    parameter NUM_SLAVES = 2,
    parameter DATA_WIDTH = 32,
    parameter ADDR_WIDTH = 32,
    parameter ID_WIDTH = 4
) (
    input  logic aclk,
    input  logic aresetn,

    // Master-side interfaces (slave ports on bridge)
    // AW, W, B, AR, R channels for each master

    // Slave-side interfaces (master ports on bridge)
    // AW, W, B, AR, R channels for each slave
);

### 24.9.2	Key Features

- 5-Channel Implementation: Complete AW, W, B, AR, R routing

- Round-Robin Arbitration: Per-slave arbitration with grant locking

- Transaction ID Tracking: Distributed RAM for B/R response routing

- ID-Based Routing: Enables out-of-order responses

- Configurable Parameters: NxM topology, data/addr/ID widths

### 24.9.3	Address Map

Default address map (configurable): - Slave 0: 0x00000000 - 0x0FFFFFFF (256MB) - Slave 1: 0x10000000 - 0x1FFFFFFF (256MB) - Slave N: N * 0x10000000

## 24.10	Test Organization

### 24.10.1	Test Hierarchy

projects/components/bridge/dv/tests/
├── conftest.py              # Pytest configuration with fixtures
├── fub_tests/               # Functional unit block tests
│   └── basic/
│       ├── test_bridge_axi4_2x2.py       # Basic 2x2 tests
│       ├── test_bridge_axi4_4x4.py       # Full 4x4 tests
│       └── test_bridge_axi4_routing.py   # Routing tests
├── integration_tests/       # Multi-bridge scenarios
│   └── test_bridge_cascade.py
└── system_tests/            # Full system tests
    └── test_bridge_dma.py

### 24.10.2	Test Levels

Basic (FUB) Tests: - Individual bridge functionality - Address routing - ID tracking - Arbitration

Integration Tests: - Multi-bridge cascades - Complex topologies - Cross-bridge transactions

System Tests: - Full DMA transfers - Realistic traffic patterns - Performance validation

## 24.11	Common User Questions and Responses

### 24.11.1	Q: “How does the Bridge work?”

A: Direct answer:

The Bridge AXI4 crossbar connects multiple AXI4 masters to multiple slaves:

- Address Decode (AW/AR): Routes master requests to appropriate slave based on address

- Write Path: AW → arbitration → slave, W follows locked grant

- Read Path: AR → arbitration → slave, R returns via ID table

- Arbitration: Per-slave round-robin with grant locking until burst complete

- Response Routing: B/R responses use ID lookup tables (not grant-based)

Key Features: - Out-of-order response support via ID tracking - Burst-aware arbitration (grant locked until WLAST/RLAST) - Configurable NxM topology - Single-clock domain

📖 See: - projects/components/bridge/PRD.md - Complete specification - projects/components/bridge/bin/bridge_generator.py - Generator implementation

### 24.11.2	Q: “How do I generate a Bridge?”

A: Use the bridge_generator.py tool:

cd projects/components/bridge/bin
python bridge_generator.py --masters 2 --slaves 4 --data-width 32 --addr-width 32 --id-width 4 --output ../rtl/bridge_axi4_flat_2x4.sv

Generated files: - RTL: projects/components/bridge/rtl/bridge_axi4_flat_<config>.sv - Contains complete 5-channel crossbar with ID tracking

### 24.11.3	Q: “How do I test a Bridge?”

A: Use the Bridge testbench class:

# Import from project area
import sys
sys.path.insert(0, os.path.join(os.path.dirname(__file__), '../../../..'))
from projects.components.bridge.dv.tbclasses.bridge_axi4_flat_tb import BridgeAXI4FlatTB

@cocotb.test()
async def test_basic(dut):
    tb = BridgeAXI4FlatTB(dut, num_masters=2, num_slaves=2)
    await tb.setup_clocks_and_reset()

    # Send write to slave 0
    await tb.write_transaction(master_idx=0, address=0x00001000, data=0xDEADBEEF)

    # Verify routing
    assert len(tb.aw_slaves[0]._recvQ) > 0, "Slave 0 should receive AW"

Run tests:

cd projects/components/bridge/dv/tests/fub_tests/basic
pytest test_bridge_axi4_2x2.py -v

## 24.12	Anti-Patterns to Avoid

### 24.12.1	✗ Anti-Pattern 1: Embedded Testbench Classes

✗ WRONG: TB class in test file
class BridgeTB:
    """NOT REUSABLE - WRONG LOCATION"""

✓ CORRECT: Import from project area
import sys
sys.path.insert(0, os.path.join(os.path.dirname(__file__), '../../../..'))
from projects.components.bridge.dv.tbclasses.bridge_axi4_flat_tb import BridgeAXI4FlatTB

### 24.12.2	✗ Anti-Pattern 2: Manual AXI4 Handshaking

✗ WRONG: Manual signal driving
self.dut.s0_axi4_awvalid.value = 1
while self.dut.s0_axi4_awready.value == 0:
    await RisingEdge(self.clock)

✓ CORRECT: Use GAXI components
await self.aw_master.send(aw_pkt)

### 24.12.3	✗ Anti-Pattern 3: Memory Models for Simple Tests

✗ WRONG: Unnecessary complexity
memory = MemoryModel()
memory.write(addr, data)
result = memory.read(addr)

✓ CORRECT: Direct queue verification
aw_pkt = self.aw_slave._recvQ.popleft()
assert aw_pkt.addr == expected_addr

## 24.13	Quick Reference

### 24.13.1	Finding Existing Components

# Bridge TB class (in PROJECT AREA, not framework!)
cat projects/components/bridge/dv/tbclasses/bridge_axi4_flat_tb.py

# Bridge generator
cat projects/components/bridge/bin/bridge_generator.py

# Test examples
ls projects/components/bridge/dv/tests/fub_tests/basic/

### 24.13.2	Common Commands

# Generate 2x2 bridge
python projects/components/bridge/bin/bridge_generator.py --masters 2 --slaves 2 --output projects/components/bridge/rtl/bridge_axi4_flat_2x2.sv

# Run tests
pytest projects/components/bridge/dv/tests/fub_tests/basic/ -v

# Lint generated RTL
verilator --lint-only projects/components/bridge/rtl/bridge_axi4_flat_2x2.sv

## 24.14	Remember

- 🏗️ MANDATORY: Testbench architecture - TB classes in framework, tests import them

- 📁 MANDATORY: Directory structure - Follow RAPIDS/AMBA pattern exactly

- 📋 MANDATORY: conftest.py - Must exist in dv/tests/

- 🎯 Use GAXI components - Never manually drive AXI4 handshakes

- 📊 Queue-based verification - Simple tests use direct queue access

- 🏛️ Three-layer architecture - TB (framework) + Test (runner) + Scoreboard (verification)

- ⚙️ Three mandatory methods - setup_clocks_and_reset, assert_reset, deassert_reset

- 🔍 Search first - Use existing components before creating new ones

- 📈 Test scalability - Support basic/medium/full test levels

- 💯 100% success - All tests must achieve 100% success rate

## 24.15	PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

/mnt/data/github/rtldesignsherpa/projects/components/bridge/docs/

Quick Command: Use the provided shell script:

cd /mnt/data/github/rtldesignsherpa/projects/components/bridge/docs
./generate_pdf.sh

The shell script will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the bridge_spec index file 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

📖 See: bin/md_to_docx.py for complete implementation details

Version: 1.0 Last Updated: 2025-10-18 Maintained By: RTL Design Sherpa Project