RTL Design Sherpa

DDR4 / LPDDR4 Family Controller Micro-Architecture Specification 0.1

October 3, 2026

Table of Contents

1 Andesite Mas Index	11

2 Document Information	12

2.1 andesite DDR4/LPDDR4 Family Controller Micro-Architecture Specification	12

2.2 Revision History	12

2.3 Document Purpose	17

2.4 Intended Audience	17

2.5 Related Documents	17

3 Block Inventory	19

3.1 Referenced, not rewritten: the inherited set	20

4 What Changes vs scoria	22

4.1 One paragraph per chapter page	24

5 Command Formatter (andesite_dfi_cmd_formatter)	26

5.1 Purpose	26

5.2 Parameters	26

5.3 Interface	27

5.4 Microarchitecture internals	29

5.4.1 DDR4 command truth table	29

5.4.2 Activate form	30

5.4.3 CA parity	30

5.4.4 NEW LPDDR4 CA submodule	30

5.4.5 Inline CKE and per-command CA cycles (LPDDR4)	31

5.5 FSM policy	31

5.6 Timing	31

5.7 Notes	32

5.8 P1 RTL interface (landed 2026-10-04)	32

6 Init Sequencer (andesite_init_sequencer)	33

6.1 Purpose	33

6.2 Parameters	33

6.3 Interface	34

6.4 Microarchitecture internals	38

6.4.1 DDR4 FSM state sequence	38

6.4.2 LPDDR4 FSM state sequence	39

6.4.3 Sequencing rules	39

6.5 FSM policy	39

6.6 Timing	39

6.7 Notes	40

6.8 CA parity error recovery	40

6.9 Gear-down programming (P1 interpretation, 2026-10-04)	41

7 Mode Register (andesite_mode_register)	42

7.1 Purpose	42

7.2 Parameters	42

7.3 Interface	43

7.4 Microarchitecture internals	44

7.4.1 DDR4 MR semantics	44

7.4.2 LPDDR4 MR set	45

7.4.3 MR coupling table	46

7.5 FSM policy	46

7.6 Timing	47

7.7 Notes	47

8 Address Mapper (andesite_addr_mapper)	48

8.1 Purpose	48

8.2 Parameters	48

8.3 Interface	49

8.4 Microarchitecture internals	50

8.4.1 Decode structure	50

8.4.2 Scheduler-visible outputs	50

8.4.3 Hashing	50

8.5 FSM policy	51

8.6 Timing	51

8.7 Notes	51

9 Scheduler and Command Arbiter (andesite_scheduler_layer + andesite_cmd_arbiter)	52

9.1 Purpose	52

9.2 Parameters	52

9.3 Interface	53

9.3.1 Host-side command ports	53

9.3.2 Bank-group timing ports	55

9.3.3 Maintenance request/grant ports	56

9.4 Macro init/policy interface (P1 rewiring, landed 2026-10-05)	56

9.5 Microarchitecture internals	58

9.5.1 FR-FCFS admission with L/S awareness	58

9.5.2 Global timers	59

9.5.3 Maintenance request/grant semantics	59

9.5.4 LPDDR4 degeneration	60

9.6 FSM policy	60

9.7 Timing	60

9.8 Notes	61

9.9 cmd_history_checker’s DDR4 growth (landed 2026-10-04)	61

10 Refresh Controller (andesite_refresh_ctrl)	62

10.1 Purpose	62

10.2 Parameters	62

10.3 Interface	62

10.4 Microarchitecture internals	66

10.4.1 Inherited mechanism	66

10.4.2 LPDDR4 delta: controller-directed per-bank refresh	67

10.5 FSM policy	67

10.6 Timing	67

10.7 Notes	67

11 ZQ Calibration Controller (andesite_zq_ctrl)	69

11.1 Purpose	69

11.2 Parameters	69

11.3 Interface	69

11.3.1 Common ports (the landed DDR4 core)	69

11.3.2 LPDDR4-only ports (the landed MPC submodule)	72

11.4 Microarchitecture internals	73

11.4.1 DDR4 path: inherited from scoria	73

11.4.2 LPDDR4 path: new MPC submodule	73

11.4.3 Scheduler interface	73

11.5 FSM policy	74

11.6 Timing	74

11.7 Notes	74

12 ODT Controller (andesite_odt_ctrl)	75

12.1 Purpose	75

12.2 Parameters	75

12.3 Interface	76

12.4 Microarchitecture internals	78

12.4.1 Policy state table	78

12.4.2 Command-stream tap	78

12.4.3 Pin timing	78

12.4.4 The init seam	79

12.4.5 LPDDR4 delta	79

12.4.6 Telemetry	79

12.5 FSM policy	79

12.6 Timing	79

12.7 Notes	80

13 Training Interfaces (andesite_wrlvl_ifc, andesite_rdlvl_ifc, andesite_ca_train_ifc)	81

13.1 Purpose	81

13.1.1 The three family rules	81

13.2 Parameters	81

13.3 Interface	82

13.3.1 DFI training signal groups	82

13.3.2 CSR / host side	83

13.4 Microarchitecture internals	87

13.4.1 wrlvl_ifc: write leveling, MODIFIED	87

13.4.2 rdlvl_ifc: MPR read leveling, NEW	87

13.4.3 ca_train_ifc: LPDDR4 CA/WDQ training, NEW	87

13.4.4 Four-state telemetry	88

13.5 FSM policy	88

13.6 Timing	89

13.7 Notes	89

14 DFI Datapath (andesite_dfi_cmd_path, andesite_dfi_rd_aligner, andesite_dfi_wr_serializer, andesite_dfi_layer)	90

14.1 Purpose	90

14.2 Parameters	90

14.3 Interface	91

14.4 Microarchitecture internals	92

14.4.1 dfi_cmd_path: widened command bus	92

14.4.2 dfi_rd_aligner: DBI on the read path	92

14.4.3 dfi_wr_serializer: DBI on the write path	93

14.4.4 dfi_layer: the 4.0 control surface	93

14.4.5 LPDDR4 delta	93

14.5 FSM policy	94

14.6 Timing	94

14.7 Notes	94

15 DFI 4.0 Pin-Level Table	95

15.1 Command and address pins	95

15.2 Gear-down	99

15.3 Training and leveling pins	99

15.4 Data-path pins (DBI and the inherited surface)	102

15.5 Study corrections (2026-10-04, andesite TASK-005)	106

15.6 What this table does not do	106

16 Core Signal Contracts	108

16.1 What a contract is here	108

16.2 Pre-RTL citation posture	108

16.3 Contract: DDR4 command decode (the formatter’s core)	109

16.4 Contract: maintenance request/grant	110

16.5 Contract: training four-state telemetry	110

16.6 Regenerating the workbook	111

List of Figures

No figures in this document.

List of Tables

Table 1: Table 0.1: Related documents	17

Table 2: Table 1.1: MAS block inventory — the changed and new blocks, one page each	19

Table 3: Table 1.2: The inheritance list — referenced to scoria’s books, with the where and the why	20

Table 4: Table 1.3: The marking set, copied from the HAS	22

Table 5: Table 2.1: Command formatter parameters	26

Table 6: Table 2.2: Command formatter ports	27

Table 7: Table 2.2.1: Init sequencer build parameters	33

Table 8: Table 2.2.2: Init sequencer ports	34

Table 9: Table 2.5: Mode register parameters	42

Table 10: Table 2.6: Mode register ports	43

Table 11: Table 2.7: DDR4 MR0-MR6 field semantics	45

Table 12: Table 2.8: Mode-register coupling	46

Table 13: Table 2.9: Address mapper parameters	48

Table 14: Table 2.10: Address mapper ports	49

Table 15: Table 2.5.1: Scheduler parameters	52

Table 16: Table 2.6.1: Refresh controller parameters	62

Table 17: Table 2.6.2: Refresh controller ports	62

Table 18: Table 2.7.1: ZQ controller parameters	69

Table 19: Table 2.7.2: ZQ controller ports	72

Table 20: Table 2.8.1: odt_ctrl parameters	75

Table 21: Table 2.8.2: odt_ctrl ports	76

Table 22: Table 2.9.1: Training-interface parameters	81

Table 23: Table 2.10.1: DFI datapath parameters	90

Table 24: Table 2.10.2: DFI datapath signal groups	92

Table 25: Table 3.1: DFI 4.0 command and address pins	95

Table 26: Table 3.2: Gear-down entry	99

Table 27: Table 3.3: Training and leveling pins	99

Table 28: Table 3.4: Data-path and inherited pins	102

Table 29: Table 4.1: Anchor map — the kmap generator cites these file:line pairs	108

# 1	Andesite Mas Index

Generated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	andesite DDR4/LPDDR4 Family Controller Micro-Architecture Specification

| Property | Value |
| --- | --- |
| Document Title | andesite DDR4/LPDDR4 Family Controller Micro-Architecture Specification |
| Version | 0.5 (draft) |
| Date | October 5, 2026 |
| Status | v0.5 with the integration pass landed (2026-10-05): the scheduler macro carries the P1 rewiring, dfi_cmd_path exists, and the training/DFI/AXI4 layer assemblies plus andesite_core are built and suite-green (248 tests at the DFI 4.0 boundary). This book expands the owner-reviewed HAS’s changed and new blocks to signal level. Every claim is inherited from a named scoria source, cited to JESD79-4 / JESD209-4 / DFI 4.0 with the §TBC(TASK-005) discipline, or recorded as an open question in the HAS ch06 |
| Classification | Open Source - MIT License |

## 2.2	Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 0.5 | 2026-10-05 | RTL Design Sherpa | TASK-016 integration pass, signal level. Scheduler macro: P1 rewiring (DDR4-only init, cmd_ack = valid&&ready qualified on MRS/ZQCL, zq_cal_start_o, retired dfi_init_*/t_*_wait_i/mr_wr_o ports); command stream widened to {ap,col,row,bg,bank,rank,op} with cmd_bg_o (CMD_W grew by $clog2(NUM_BG)); the init request’s duplicate-accept fix is a combinational gate on the accept cycle. dfi_cmd_path (new): A10 case-split address routing (RD/RDA/WR/WRA merge ap<<10, PREA forces A10, PRE clears, MRS/ZQCL keep the row-field payload). andesite_training_layer (new): the three carried training interfaces behind a maintenance command channel mirroring the init-source ack-pulse pattern. andesite_dfi_layer (new): CDC word packings WD_DW={last,strb,dbi,data} and RD_DW={last,resp,dbi,data} so write-DBI and read-DBI cross registered with their data; dfi_init_complete_i drives pinit_complete_i. andesite_axi4_layer (carried-modified): scoria rename-carry, byte-parity + logic-parity gates green. andesite_core (new top): four-layer wiring per the ch03 wiring table. Suites: fub 225, macro 19, core top 4 (248 total); master lint 53 modules; registry PASS. DV-framework drift recorded: the BFM behavior dispatch samples cs_n/error/init/update unconditionally — the init-slice TB carries phy_dfi_cs_n + idle handshake pins. |
| 0.1 | 2026-10-03 | RTL Design Sherpa | First draft, written before any RTL exists. Carries the HAS’s changed/new blocks one level down: per-block interface tables, FSM policy, mechanism detail, and the contract anchors the generated kmap book cites. Inherited-unchanged blocks are referenced to scoria’s books, not rewritten. |
| 0.4 | 2026-10-04 | RTL Design Sherpa | P3 gate: the modified/new tier is landed. 30 design files: 15 carried-inherited rename-only (13 active + the powerdown_ctrl/dfi_signal_pack dormant pair), 8 carried-modified (addr_mapper +bg, global_timers +L/S, cmd_arbiter +L/S admission, refresh_ctrl +FGR, zq_ctrl +MPC submodule, mem_cmd_scheduler carrying the FGR/MPC/recovery consumer paths, dfi_wr_serializer/dfi_rd_aligner +DBI), 4 new (odt_ctrl, rdlvl_ifc, ca_train_ifc, zq_mpc_lpddr4), and the 3 P1 blocks (dfi_cmd_formatter, init_sequencer +TASK-006 recovery FSM, mode_register). The marking-pair gate (HAS reuse table vs this book’s What-Changes table) is GREEN at 23=23. The kmap CITES for the address-decode and MR-semantics fences now point at RTL anchor blocks in addr_mapper/mode_register (drift gate live, byte-identical reruns). dfi_cmd_path + dfi_layer remain per the plan: cmd_path rides the T9 integration pass; the layer is P4. The scheduler macro’s init/mode_register rewiring and its macro-level suite land with that same pass. |
| 0.3 | 2026-10-04 | RTL Design Sherpa | P2 carry reconciliation: the inheritance table’s ten active blocks exist as andesite_* with scoria’s port lists (carry-diff gates prove rename/import/header-only deltas); cmd_history_checker carries the DDR4 L/S growth named on the scheduler page. The dormant pair exists uninstantiated. rd_intake/wr_intake land with P3’s addr_mapper (they instantiate it). |
| 0.2 | 2026-10-04 | RTL Design Sherpa | P1 RTL reconciliation: Tables 2.2/2.2.2 (formatter, init_sequencer) and the mode_register interface now match the landed modules — v4.0 dfi_cs naming, 5-bit op_i carrying OP_MPC, 3-bit MR index on cmd_bank, 6-bit MR addresses, csr_mrN_image init payloads. Deferred ports (dfi_alert_n, ca_o/ca_valid_o) are recorded as follow-ons (TASK-006 / LPDDR4 breadth), not stubs. The kmap book’s command-decode citations re-pointed at the RTL (drift gate live); per-command boolean SOP verdicts land with P3. |

## 2.3	Document Purpose

This Micro-Architecture Specification (MAS) is the implementation-level view of the andesite controller — the HAS (../andesite_has/) taken one level down. It is the document an RTL author reads before writing SystemVerilog and a verification engineer reads before writing checkers.

It covers:

- each changed or new block’s interface, parameters, and internal mechanism

- the FSM policy for every block: where a state machine is required, where one is forbidden (no training search loops, per the D2 precedent), and what replaces FSMs elsewhere

- the DFI 4.0 pin-level table the HAS’s interface chapter promises

- the core signal contracts whose anchors the generated kmap book (../kmaps/) verifies by citation

## 2.4	Intended Audience

- RTL implementers writing the changed and new blocks

- Verification engineers building the DFI 4.0 BFM checks and the formal spacing properties the HAS ch06 names

- Architects confirming that a block-level decision matches the HAS’s marking and cause

## 2.5	Related Documents

Table 0.1: Related documents

| Document | Location | Content |
| --- | --- | --- |
| Hardware Architecture Specification | ../andesite_has/andesite_has_index.md | the owner-reviewed v0.1 architecture this document expands; bindings between the two follow the HAS |
| Bootstrap design spec | ../../../../../../docs/superpowers/specs/2026-10-03-andesite-bootstrap-design.md | the tranche’s scope decisions (§6 is this book’s plan) |
| Family docs | ../../../../docs/ | shared-core design (mem_ctrl_pkg) and family doctrine |
| scoria HAS | ../../../scoria-ddr3-lpddr3/docs/scoria_has/ | the reuse pool; authoritative for inherited-unchanged blocks |
| scoria design requirements | ../../../scoria-ddr3-lpddr3/docs/design-requirements.md | the delta-analysis method; TASK-001 mode detail (§6) |
| Kmap generator + workbook | ../kmaps/ | generated command-encoding tables citing this book’s anchors |
| Family README | ../../../../../README.md | family overview and per-IP status |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Block Inventory

The MAS documents the changed and new blocks below, one page per chapter in ch02_blocks/. Each page follows the house FUB-page skeleton: header metadata, purpose, parameters, interface, microarchitecture internals, FSM policy, timing, and notes — the skeleton the bch MAS established and the pumice MAS before it. Markings are copied from the HAS, not re-derived; a marking here that disagrees with the HAS is a defect here.

Table 1.1: MAS block inventory — the changed and new blocks, one page each

| Block | Marking | MAS page | HAS pointer |
| --- | --- | --- | --- |
| dfi_cmd_formatter (+ NEW LPDDR4 CA submodule) | MODIFIED / NEW | ch02_blocks/01_cmd_formatter.md | Ch 3.2, 3.6 |
| init_sequencer | MODIFIED | ch02_blocks/02_init_sequencer.md | Ch 3.2 |
| mode_register | MODIFIED | ch02_blocks/03_mode_register.md | Ch 3.2 |
| addr_mapper | MODIFIED | ch02_blocks/04_addr_mapper.md | Ch 3.1 |
| scheduler_layer + cmd_arbiter | MODIFIED | ch02_blocks/05_scheduler.md | Ch 3.1 |
| refresh_ctrl | MODIFIED | ch02_blocks/06_refresh_ctrl.md | Ch 3.4 |
| zq_ctrl (+ NEW LPDDR4 MPC submodule) | INHERITED / NEW | ch02_blocks/07_zq_ctrl.md | Ch 3.2 |
| odt_ctrl | NEW | ch02_blocks/08_odt_ctrl.md | Ch 3.5 |
| training interfaces (wrlvl_ifc, rdlvl_ifc, ca_train_ifc) | MODIFIED / NEW | ch02_blocks/09_training.md | Ch 3.3 |
| DFI datapath (dfi_cmd_path, dfi_rd_aligner, dfi_wr_serializer, dfi_layer) | MODIFIED | ch02_blocks/10_dfi_datapath.md | Ch 3.5, 4.1 |

Ten pages, matching the ten areas the HAS’s Chapter 3 argues: the command encoding, init, mode registers, address mapping, scheduling, refresh, ZQ, ODT, training, and the DFI datapath. The kmap book’s citations point into these pages’ mechanism sections; where a page names an encoding or a decode equation, that line is an anchor.

## 3.1	Referenced, not rewritten: the inherited set

The inherited-unchanged blocks are not MAS pages. Each is specified by scoria’s book and its RTL header, and this MAS states where and why:

Table 1.2: The inheritance list — referenced to scoria’s books, with the where and the why

| scoria module | Marking | Where its specification lives | Why not rewritten here |
| --- | --- | --- | --- |
| scoria_top, scoria_top_geared, scoria_core, scoria_axi4_ifc | INHERITED | scoria HAS Ch 2.3 / 4.2 and the rtl/top/ + rtl/macro/ headers | structure and host side unchanged; andesite’s deltas do not reach them |
| bank_timer / bank_timers, page_policy, rd_cmd_cam, wr_data_cam | INHERITED | scoria HAS Ch 2.3 and the rtl/fub/ headers | the scheduling mechanism is unchanged; only what the arbiter asks of it (the L/S pairs) is new, and that belongs to the scheduler page |
| rd_intake, wr_intake, wr_splitter, rd_return_ring, axi_burst_chopper | INHERITED | scoria HAS Ch 2.3 / 4.2 | the host-side front end is untouched by this generation |
| dfi_cdc | INHERITED | scoria HAS Ch 2.3 / 4.1 | the CDC shape transfers to DFI 4.0 unchanged (HAS Ch 4.1) |
| cmd_history_checker | INHERITED | scoria HAS Ch 2.3 | mechanism inherited; its DDR4 spacing-parameter growth is named on the scheduler page, where the parameters are defined |
| powerdown_ctrl, dfi_signal_pack | DORMANT | andesite HAS Ch 3.1 | carried dormant with the waking condition named there; a dormant block has no mechanism news to specify |

The rule both tables enforce: the MAS expands the HAS’s changed/new blocks; it does not re-derive, re-mark, or silently restate anything the HAS and scoria’s books already settle.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	What Changes vs scoria

The complete marking set, copied from the HAS (Ch 2.3 and Table 3.1) rather than re-derived. This table and the HAS carry the same pairs; the Task-10 consistency gate diffs them mechanically.

Table 1.3: The marking set, copied from the HAS

| scoria module | Marking | What changes, one line |
| --- | --- | --- |
| scoria_top | INHERITED | structure unchanged |
| scoria_top_geared | INHERITED | the width-gearing wrapper, unchanged |
| scoria_core | INHERITED | core assembly unchanged |
| scoria_axi4_ifc | INHERITED | the AXI4 host side is untouched by this generation |
| scoria_mem_cmd_scheduler | MODIFIED | L/S admission alongside scoria’s ZQ maintenance admission |
| scoria_dfi_layer | MODIFIED | DFI 4.0 control surface and DBI wires |
| scoria_addr_mapper | MODIFIED | bank-group decode: BG0/BG1 between chip select, bank and row |
| scoria_cmd_arbiter | MODIFIED | L/S-aware issue on the same-group / cross-group split |
| scoria_global_timers | MODIFIED | tCCD_L/S and tRRD_L/S as first-class counters |
| scoria_dfi_cmd_formatter | MODIFIED | ACT_n fifth pin, BG0/BG1, CA parity; NEW submodule: LPDDR4 6-bit CA path |
| scoria_init_sequencer | MODIFIED | DDR4 reset procedure, MR3-MR6-MR5-MR4-MR2-MR1-MR0 order, gear-down entry, parity enable |
| scoria_mode_register | MODIFIED | MR0-MR6 field maps per memtype |
| scoria_zq_ctrl | INHERITED | DDR4’s ZQCS/ZQCL exactly as built; NEW submodule: LPDDR4 MPC calibration |
| scoria_refresh_ctrl | MODIFIED | FGR 1x/2x/4x on the inherited elastic/TCR/placement base |
| scoria_dfi_cmd_path | MODIFIED | ACT_n, BG and parity wires ride the command path |
| scoria_dfi_rd_aligner | MODIFIED | DBI read path |
| scoria_dfi_wr_serializer | MODIFIED | DBI write path |
| scoria_wrlvl_ifc | MODIFIED | training-interface family grows; contract and no-search rule unchanged |
| scoria_powerdown_ctrl | DORMANT | carried dormant, waking condition in HAS Ch 3.1 |
| scoria_dfi_signal_pack | DORMANT | carried dormant, waking condition in HAS Ch 3.1 |
| (new) odt_ctrl | NEW | dynamic ODT: RTT_NOM/WR/PARK and the ODT latency family |
| (new) rdlvl_ifc | NEW | MPR-based read-leveling interface, no search loop |
| (new) ca_train_ifc | NEW | LPDDR4 CA/WDQ training interface, no search loop |

## 4.1	One paragraph per chapter page

Command formatter (MODIFIED / NEW). The wire-level truth table changes shape: ACT becomes a five-pin command, BG0/BG1 appear on the activate’s address pins, and parity rides the command stream. The LPDDR4 side is a new 6-bit double-data-rate CA encoding, two cycles per command — a submodule so the DDR4 path stays reviewable on its own. Mechanism detail: Ch 2.1.

Init sequencer (MODIFIED). The FSM grows DDR4’s reset-procedure states and gear-down entry, and programs parity enable in the MR order. The order remains JEDEC’s, now MR3-MR6-MR5-MR4-MR2-MR1-MR0; the MAS page lists the states and the waits by name, with values per the HAS’s Q1 recording. Mechanism detail: Ch 2.2.

Mode register (MODIFIED). MR0-MR6 for DDR4 and the LPDDR4 MR set, field maps per memtype, with the MAS page carrying the field-level tables the HAS deliberately leaves to this book. Mechanism detail: Ch 2.3.

Address mapper (MODIFIED). One bounded change: the decode grows BG0/BG1 between chip select, bank and row. The decode equations are the kmap book’s first address-map targets. Mechanism detail: Ch 2.4.

Scheduler and arbiter (MODIFIED). L/S-aware issue: the arbiter consults same-group versus cross-group spacing, the global timers carry the long/short pairs, and maintenance admission (refresh, ZQ, ODT turnarounds) rides the inherited request/grant channel. LPDDR4 degenerates L = S gracefully. Mechanism detail: Ch 2.5.

Refresh controller (MODIFIED). FGR 1x/2x/4x via the MR3 image, interval arithmetic scaled per factor, and LPDDR4’s controller-named per-bank scheduling replacing the rotor. The elastic/TCR/placement policy base is inherited intact. Mechanism detail: Ch 2.6.

ZQ controller (INHERITED / NEW). The DDR4 core is scoria’s, unchanged. The new submodule sequences LPDDR4 calibration through MPC. Mechanism detail: Ch 2.7.

ODT controller (NEW). The policy block: RTT_NOM/WR/PARK selection on the command-stream tap, ODT pin latency enforcement, rank coupling for multi-rank builds. LPDDR4’s MR-programmed termination is init-side, not this page’s. Mechanism detail: Ch 2.8.

Training interfaces (MODIFIED / NEW). wrlvl_ifc keeps its contract; rdlvl_ifc and ca_train_ifc are new pages in the same family — handshake, MR/MPC path, windows, four-state telemetry, no search loops anywhere. Mechanism detail: Ch 2.9.

DFI datapath (MODIFIED). dfi_cmd_path widens for ACT_n/BG/parity; the aligner and serializer pass DBI through; the layer presents the 4.0 control surface. The CDC and the pack stage are inherited (the latter dormant). Mechanism detail: Ch 2.10.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 5	Command Formatter (andesite_dfi_cmd_formatter)

Module: andesite_dfi_cmd_formatter.sv Location: rtl/fub/ (planned) Category: FUB Parent: dfi_cmd_path / dfi_layer Status: specified — no RTL exists (HAS v0.1 posture)

## 5.1	Purpose

dfi_cmd_formatter is the last combinational step before the command stream hits the DFI pins. It takes the scheduler’s abstract opcode and address fields and turns them into the wire pattern the DRAM expects: ACT_n, RAS_n, CAS_n, WE_n, chip-select, bank, bank-group and address for DDR4; the 6-bit double-data-rate CA bus for LPDDR4. The block doesn’t sequence anything — it just encodes. Sequence lives in the scheduler and the init_sequencer, the same split pumice and scoria used.

The andesite delta is the DDR4 fifth pin and bank-group wiring, plus a NEW LPDDR4 CA submodule that sits beside the DDR4 path so each stays reviewable on its own. Parity generation also lands here, gated by init and returned on dfi_alert_n.

## 5.2	Parameters

Table 2.1: Command formatter parameters

| Parameter | Type | Range | Default | Meaning | Source |
| --- | --- | --- | --- | --- | --- |
| ADDR_WIDTH | int | 18 | 18 | row/column address width presented to formatter | design point |
| BANK_WIDTH | int | 2 | 2 | bank address width (DDR4); 3 for LPDDR4 | design point |
| BG_WIDTH | int | 2 | 2 | bank-group width (DDR4 only) | design point |
| RANK_WIDTH | int | 1 | 1 | rank/chip-select width | design point |
| CA_WIDTH | int | 6 | 6 | LPDDR4 CA bus width | JESD209-4 |
| PARITY_EN | bit | 0/1 | 0 | CA parity generation enable (dynamic after init) | CSR |

## 5.3	Interface

Table 2.2: Command formatter ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | in | 1 | controller clock; DFI 1:4 gear ratio means one command beat per four DRAM clocks §TBC(TASK-005) |
| rst_n | in | 1 | active-low synchronous reset |
| op_i | in | 5 | internal opcode, andesite_pkg::dram_op_e: OP_NOP..OP_ZQCL carried from scoria plus OP_MPC; the 5-bit enum carries the LPDDR4 MPC opcode selection |
| bank_i | in | BANK_WIDTH | bank address from the mapper |
| bg_i | in | BG_WIDTH | bank-group address from the mapper (DDR4) |
| row_i | in | ADDR_WIDTH | row address, valid with OP_ACT |
| col_i | in | ADDR_WIDTH | column address, valid with OP_RD/OP_WR |
| rank_i | in | RANK_WIDTH | chip select / rank target |
| memtype_i | in | 3 | family memtype enum from andesite_pkg |
| parity_en_i | in | 1 | enable CA parity generation after init |
| dfi_act_n | out | 1 | DFI 4.0 activate command pin §TBC(TASK-005) |
| dfi_ras_n | out | 1 | DFI row-address strobe command pin |
| dfi_cas_n | out | 1 | DFI column-address strobe command pin |
| dfi_we_n | out | 1 | DFI write-enable command pin |
| dfi_bank | out | BANK_WIDTH | DFI bank address |
| dfi_bg | out | BG_WIDTH | DFI bank-group address (DDR4) |
| dfi_address | out | ADDR_WIDTH | DFI address bus |
| dfi_cs | out | RANKS | DFI chip-select, active low; v4.0 catalog name (no *_cs_n) |
| dfi_cke | out | 1 | DFI clock-enable; DDR4-scoped (LPDDR4 carries CKE inline on the CA bus) |
| dfi_parity_in | out | 1 | DFI CA parity output to PHY §TBC(TASK-005) |

## 5.4	Microarchitecture internals

### 5.4.1	DDR4 command truth table

The DDR4 encoding follows the JESD79-4 four-pin command convention, with ACT_n as the fifth pin for activate. The truth table is the citation anchor for the block; don’t renumber it.

CS_n=0:
  ACT_n  RAS_n  CAS_n  WE_n    Command
    1      1      1      1     NOP
    0      1      1      1     ACT
    1      1      0      1     RD
    1      1      0      0     WR
    0      0      0      0     MRS
    0      0      0      1     REF
    1      0      1      0     PRE     (A10=0: PRE per bank; A10=1: PREA)
    1      1      1      0     ZQ      (A10=0: ZQCS; A10=1: ZQCL)

RD/WR auto-precharge variants (RDA/WRA) reuse the RD/WR pin encoding with
the AP address bit set (A10=1) -- the auto-precharge select is an address
input, not a pin variant, so the kmap book treats AP as a table input.

CS_n=1: DES (deselect)

CKE-qualified entries: SRE (self-refresh entry), SRX (self-refresh exit)
LPDDR4-only command: MPC (multipurpose command), see CA submodule below

: Table 2.3: DDR4 command truth table (citation anchor). Pin encodings are stated per the JESD79-4 command-truth-table convention and are confirmed at the HAS Q1 cold-storage read before RTL; the kmap book’s DDR4 table (andesite TASK-004) minimizes its qualifier SOPs from exactly this anchor.

A10 inside PRE selects precharge-all (PREA); A10 inside ZQ selects ZQCL versus ZQCS; A10 inside RD/WR selects the auto-precharge variants (RDA/WRA). During ACT, A10 is not a variant-select bit; it carries row address bit 10 like any other activate address pin. BG0 and BG1 ride the address bus during ACT.

### 5.4.2	Activate form

The activate command packs the row and bank-group fields onto the DFI address and bank-group outputs:

{ ACT_n=0, dfi_bank[1:0]=BA[1:0], dfi_bg[1:0]=BG[1:0], dfi_address[17:0]=row[17:0] }

: Figure 2.1: DDR4 activate field packing

### 5.4.3	CA parity

Parity is generated in the formatter from the command pins plus the address/bank/bank-group bits that travel on the same DRAM clock edge. The insertion point is the registered pipeline stage that drives dfi_parity_in. Generation only runs when parity_en_i is asserted; that bit is cleared at reset and set by the init_sequencer once the MR sequence has programmed parity mode. This is the same init-side enable dependency scoria used for DLL-related gating.

A parity error returned on dfi_alert_n is a DFI 4.0 alert event (§3.5.7 command-address parity, §4.11 alert handling). The formatter logs the raw dfi_alert_n pulse; the sequencer owns the recovery policy and the MR re-programming path, described in ch02_blocks/02_init_sequencer.md §“CA parity error recovery”.

### 5.4.4	NEW LPDDR4 CA submodule

LPDDR4 uses a 6-bit double-data-rate CA bus, two cycles per command (RD/WR); ACT is issued as an ACT-1/ACT-2 pair. The submodule takes the internal opcode, bank, and MPC/MRW payload and emits the two-cycle CA beat pair. Each command is therefore a 12-bit symbol split across two half-rates. The exact CA encodings are pinned by the kmap book’s LPDDR4 CA-bus table in docs/kmaps/generated/ (andesite TASK-004); this page does not fabricate them.

Command class -> CA encoding source
  NOP      -> kmap table
  ACT      -> kmap table
  RD       -> kmap table
  WR       -> kmap table
  MPC      -> kmap table
  MRW      -> kmap table
  MRR      -> kmap table
  REFab    -> kmap table
  REFpb    -> kmap table (bank carried in command)

: Table 2.4: LPDDR4 CA encoding placeholder (kmap book anchor)

The submodule is combinational encoding plus the two-cycle registered shifter. The first cycle drives ca_o with CA[5:0]; the second cycle drives ca_o with CA[11:6]. ca_valid_o marks both cycles.

### 5.4.5	Inline CKE and per-command CA cycles (LPDDR4)

LPDDR4 removed the dedicated CKE pin — JESD209-4E carries the CKE-equivalent state inline on the CA bus, and the LPDDR4 research index lists this as one of the big architectural breaks from LPDDR3. The dfi_cke output of this formatter is therefore DDR4-scoped only; for LPDDR4, the CA submodule encodes CKE-equivalent states (power-down entry/exit, self-refresh entry/exit) as CA-bus transactions. That ties directly to the HAS Ch 3.1 dormant-pair story: powerdown_ctrl and dfi_signal_pack are DDR4-scoped this edition, while LPDDR4 low-power states live in the CA encoding.

The per-command CA cycle structure also differs from the flat “two cycles per command” wording in the purpose section. RD and WR span two CA cycles, but ACT is not a single two-cycle command — it is issued as an ACT-1 / ACT-2 pair on two separate clocks. The bit-level CA layouts for every command class are per JESD209-4E cold-storage confirmation; the kmap book’s LPDDR4 CA table (docs/kmaps/generated/02_lpddr4_ca_command_table.md) is the authoritative placeholder and stays TBC until that confirmation is done.

## 5.5	FSM policy

There is no FSM. The formatter is combinational encoding followed by a registered pipeline stage; the LPDDR4 submodule adds a two-cycle shift register for the second CA beat. This is the pumice/scoria precedent: encode here, sequence elsewhere.

## 5.6	Timing

All command-to-command spacing is enforced upstream by global_timers and the scheduler. The formatter introduces one controller-clock pipeline delay from decoded inputs to DFI outputs. LPDDR4 commands consume two controller clocks on the CA bus, so the downstream command accept handshake reflects that stretch. Specific JEDEC values (tCCD_L, tCCD_S, tMRD, tMOD, the ODT latency family) are runtime CSRs, initialised from the JESD79-4/JESD209-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1).

## 5.7	Notes

- Keep the DDR4 and LPDDR4 paths separate at the top level of this FUB. Merging them makes review harder and gains nothing — the memtype select is one mux at the output boundary.

- dfi_alert_n is asynchronous at the PHY; sample it synchronously inside this block and export a one-cycle pulse.

- LPDDR4’s REFpb command names the bank explicitly in the CA encoding, which is why the refresh controller’s per-bank scheduling lands naturally on this path.

## 5.8	P1 RTL interface (landed 2026-10-04)

Table 2.2 above now matches the landed andesite_dfi_cmd_formatter: v4.0 dfi_cs naming (no *_cs_n), a 5-bit op_i carrying OP_MPC in the enum (the standalone mpc_op_i port was retired with it), and a scalar dfi_cke (DDR4-scoped; LPDDR4’s inline CKE rides the CA bus). The dfi_alert_n input and the ca_o/ca_valid_o pair are deferred on purpose — they land with the andesite TASK-006 recovery FSM and the LPDDR4 breadth track respectively, not as untested stubs. The CA_WIDTH and PARITY_EN rows of Table 2.1 are design-point parameters; the RTL carries them as the CA-bus width constant and the parity_en_i port.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 6	Init Sequencer (andesite_init_sequencer)

Module: andesite_init_sequencer.sv Location: rtl/init/ Category: control / initialization Parent: andesite_core / andesite_top Status: specified — no RTL exists (HAS v0.1 posture)

## 6.1	Purpose

init_sequencer owns the power-up handshake with the DRAM. It drives RESET#, raises CKE, programs the mode-register set in JEDEC order, issues ZQCL, waits out tDLLK and tZQinit, and optionally performs the gear-down entry sequence. Until it reaches READY, the sequencer holds the command bus: no host traffic reaches the formatter. After READY, it goes idle but stays available for a firmware-triggered re-initialization.

The block is MODIFIED from scoria because DDR4 adds the RESET# pin, seven mode registers, gear-down, and CA parity enable; LPDDR4 needs its own bus-reset, MRW order, and MPC-carried calibration handoff.

## 6.2	Parameters

Table 2.2.1: Init sequencer build parameters

| Parameter | Type | Range | Default | Meaning |
| --- | --- | --- | --- | --- |
| MEMTYPE | memtype_e | DDR4, LPDDR4 | build | selects the init branch and the MR set |
| NUM_CHANNELS | int | 1..2 | 1 | LPDDR4 channel count; DDR4 uses 1 |
| GEARDOWN_SUPPORTED | bit | 0,1 | 1 | when 1, the FSM includes the gear-down entry states |

All timing values are runtime CSRs, not parameters. That’s the family rule: a timing compiled in cannot be swept, and a controller that cannot be swept cannot be characterized.

## 6.3	Interface

Table 2.2.2: Init sequencer ports

| Signal | Direction | Width | Description |  |
| --- | --- | --- | --- | --- |
| clk | in | 1 | controller clock |  |
| reset_n | in | 1 | active-low reset |  |
| csr_init_trigger | in | 1 | firmware pulse: drop back to POWER_ON_RESET and re-run |  |
| csr_memtype | in | 3 | runtime copy of MEMTYPE for the FSM branch |  |
| csr_geardown_en | in | 1 | enable gear-down entry after ZQCL |  |
| csr_parity_en | in | 1 | enable CA parity through the MR sequence |  |
| tinit1_csr | in | TINIT_WIDTH | tINIT1 wait, runtime CSR |  |
| tinit3_csr | in | TINIT_WIDTH | tINIT3 wait, runtime CSR |  |
| tinit4_csr | in | TINIT_WIDTH | tINIT4 wait, runtime CSR |  |
| tdllk_csr | in | TINIT_WIDTH | tDLLK wait, runtime CSR |  |
| tzqinit_csr | in | TINIT_WIDTH | tZQinit wait, runtime CSR |  |
| tmrd_csr | in | TINIT_WIDTH | tMRD wait, runtime CSR |  |
| tmod_csr | in | TINIT_WIDTH | tMOD wait, runtime CSR |  |
| mr_image_out | out | packed MR vector | current MR value for mode_register |  |
| mr_load | out | 1 | pulse: mode_register captures mr_image_out |  |
| reset_n_out | out | 1 | RESET# pin to top/PHY, active low |  |
| cke_out | out | 1 | CKE pin, registered from activation until init complete |  |
| cmd_req | out | 1 | request to the formatter/scheduler for the next init command |  |
| cmd_ack | in | 1 | grant: the command is issued this cycle |  |
| cmd_op | out | opcode | init command opcode (MRS/MRW/ZQCL/MPC/etc.) |  |
| cmd_bank | out | 3 | bank address (2 bits used); the MRS path carries the 3-bit MR index (0-6) |  |
| cmd_addr | out | address width | MR index / MRW address field |  |
| zq_cal_start | out | 1 | pulse to zq_ctrl to start ZQ calibration |  |
| gear_down_entry | out | 1 | pulse to PHY/formatter to enter gear-down §TBC(TASK-005) |  |
| parity_enable_out | out | 1 | parity is now active; formatter begins counting |  |
| init_done | out | 1 | READY reached, normal operation may begin |  |
| init_err | out | 1 | sequencing or timeout error, latched until re-init |  |
| ca_train_start | out | 1 | LPDDR4 handoff to ca_train_ifc when training is required |  |
| parity_alert_i | in | 1 | the formatter’s logged CA-parity alert pulse; the recovery FSM’s entry event |  |
| recovery_interval_i | in | 16 | JEDEC-named recovery interval for RESENDING, runtime CSR |  |
| csr_telem_clear_i | in | 1 | explicit firmware clear of the recovery telemetry |  |
| retract_req_o | out | 1 | maintenance-class request asking the scheduler to withdraw the suspect command |  |
| retract_ack_i | in | 1 | scheduler grant for the retract; request-and-wait, like cmd_req/cmd_ack |  |
| obs_recovery_state_o | out | 2 | recovery sub-FSM state: 0 IDLE, 1 ALERT_SEEN, 2 RESENDING |  |
| obs_alerts_seen_o | out | 16 | saturating telemetry: alerts seen |  |
| obs_cmds_dropped_o | out | 16 | saturating telemetry: suspect commands dropped |  |
| obs_cmds_resent_o | out | 16 | saturating telemetry: commands released for re-issue |  |

The cmd_req/cmd_ack pair is the same request-and-wait discipline the scheduler uses for maintenance traffic. Init owns the bus, but it still waits for the formatter to be ready.

## 6.4	Microarchitecture internals

### 6.4.1	DDR4 FSM state sequence

The DDR4 branch is one flat FSM. Gear-down entry is folded as states, not a nested machine.

POWER_ON_RESET
    -> RESET_ASSERT        (drive RESET# low; wait tINIT1)
    -> RESET_DEASSERT_WAIT  (RESET# high; wait tINIT3)
    -> CKE_ENABLE           (raise CKE)
    -> CKE_WAIT            (wait tINIT4)
    -> MRS_MR3
    -> MRS_MR6
    -> MRS_MR5
    -> MRS_MR4
    -> MRS_MR2
    -> MRS_MR1
    -> MRS_MR0
    -> ZQCL
    -> DLLK_ZQINIT_WAIT    (wait tDLLK and tZQinit)
    -> [GEARDOWN_ENTRY if csr_geardown_en: program MR3 gear-down,
        issue entry sequence, wait sync]
    -> READY

### 6.4.2	LPDDR4 FSM state sequence

LPDDR4 uses its own branch. The exact MRW order follows JESD209-4; the table below captures the structural stages.

POWER_ON_RESET
    -> BUS_RESET            (reset over CA bus or reset pin per JESD209-4)
    -> INIT_WAIT            (JESD209-4 initialization wait)
    -> MRW_SEQUENCE        (JESD209-4 order: device feature, ODT,
                            drive-strength, DQ-ODT, and remaining MRs)
    -> MPC_ZQ_CALIBRATION  (issue MPC ZQCal start / latch via formatter CA path)
    -> [CA_TRAINING_HANDOFF if required: ca_train_start to ca_train_ifc]
    -> READY

### 6.4.3	Sequencing rules

The MR order is a citation, not a design choice. pumice’s EMRS3-first correction is the family precedent, and andesite follows it: MR3 first, then MR6, MR5, MR4, MR2, MR1, MR0 last. MR0 carries the DLL reset only; CA parity latency/mode is programmed in MR5, so parity enable rides the MR5 program step. MR0 must wait until the bus is stable.

Parity enable is ordered after bus stability for the same reason. Once parity_enable_out rises, the formatter’s parity counter starts counting from that point; enabling it earlier would count commands that were not parity-protected.

CKE is continuously registered from the CKE_ENABLE state until init completes. Dropping it mid-sequence is an error and is flagged by init_err.

## 6.5	FSM policy

This block has one FSM and it is the init FSM. There are no nested FSMs and no sub-state machines hidden inside the MR sequence. Gear-down entry is a sequence of states in the same FSM; the PHY/formatter handshake is a single output pulse plus a wait state.

The MR states themselves are simple: load the next MR image, pulse mr_load, assert cmd_req, wait for cmd_ack, then wait tMRD or tMOD as required before the next state.

## 6.6	Timing

Every wait state counts from a CSR-loaded value. The values are runtime CSRs, initialised from the JESD79-4/JESD209-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1).

| Wait | Symbol | Loaded from | Counted in |
| --- | --- | --- | --- |
| Reset assertion | tINIT1 | tinit1_csr | MC clocks |
| Reset deassert to CKE | tINIT3 | tinit3_csr | MC clocks |
| CKE to first MRS/MRW | tINIT4 | tinit4_csr | MC clocks |
| Between MRS commands | tMRD | tmrd_csr | MC clocks |
| MRS to non-MRS command | tMOD | tmod_csr | MC clocks |
| DLL lock + ZQ init | tDLLK, tZQinit | tdllk_csr, tzqinit_csr | MC clocks, both must expire |

The counter reload value for each state is captured when the state is entered, so a CSR written mid-wait does not affect the running count. That’s the same reload-only rule the refresh controller uses for tREFI derate.

Ordering checks — for example, that MR3 is issued before MR0 — are assertions in DV, not in RTL. That’s the family rule: the FSM encodes the order, and DV proves the FSM never strays.

## 6.7	Notes

- LPDDR4 calibration handoff: MPC_ZQ_CALIBRATION does not perform the calibration itself. It issues the MPC opcodes through the formatter’s LPDDR4 CA submodule and then raises init_done. If the board requires CA training, the FSM can hand off to ca_train_ifc before READY.

- Re-initialization: A firmware pulse on csr_init_trigger forces the FSM back to POWER_ON_RESET. The init_done output falls immediately so the scheduler stops admitting host commands.

- Gear-down and parity are independent CSR selects. Either, both, or neither may be enabled; the FSM states exist for both paths.

- DFI 4.0 gear-down handshake: The gear_down_entry pulse crosses to the PHY through the DFI 4.0 control surface §TBC(TASK-005). The exact DFI signal names are public and live in the formatter chapter.

## 6.8	CA parity error recovery

Recovery from a CA parity alert is handled by a small sub-FSM that sits beside the init FSM and never enters the bank machine. The formatter logs the raw dfi_alert_n pulse (see ch02_blocks/01_cmd_formatter.md §“CA parity”); that logged pulse is the entry event for this recovery FSM. DFI 4.0 alert and parity semantics are §3.5.7 and §4.11.

IDLE -> ALERT_SEEN -> RESENDING -> IDLE

- IDLE — waiting. The recovery FSM is transparent; normal scheduler grants pass through unchanged.

- ALERT_SEEN — on the cycle the formatter’s logged pulse arrives, the command currently in the grant stream is marked suspect and dropped. The FSM asserts a retract sideband to the scheduler, asking it to withdraw the suspect command and re-issue it from its request queue. The scheduler’s request-never-preempts rule is preserved: the recovery request is just another maintenance-class request that must wait for the scheduler’s grant, exactly like the init sequencer’s cmd_req/cmd_ack pair.

- RESENDING — the FSM holds the JEDEC-named recovery interval, counting down, then returns to IDLE. It never drives a command out of RESENDING: release-to-scheduler, not re-issue — the scheduler re-issues the dropped command from its own request queue once the retract clears. The interval value is a runtime CSR loaded from the JESD79-4 speed bin at CSR-derivation time; no numeric constant is compiled into the RTL.

Telemetry is kept in small saturating counters: alerts seen, commands dropped, and commands re-issued. These counters are visible to firmware and are reset only by controller reset or an explicit firmware clear.

The recovery FSM does not trigger a full re-initialization. A single parity event drops one command and retransmits it; only a sustained or uncorrectable pattern would escalate to firmware-assisted MR5 re-programming or init re-run. That escalation policy is recorded as open question Q4 in the HAS, not hard-wired here.

## 6.9	Gear-down programming (P1 interpretation, 2026-10-04)

The fence’s “program MR3 gear-down” step rides the MR image: the FSM issues the normal MR3 MRS with whatever image firmware configured (csr_mr3_image), then the entry pulse and the sync wait follow ZQ-init. The gear-down bit’s position in MR3 is Q1 (JESD79-4 cold-storage read) – the RTL makes no bit-position claim; the P1 test asserts the configured bit reaches the bus, wiring-only.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 7	Mode Register (andesite_mode_register)

Module: andesite_mode_register.sv Location: rtl/fub/ (planned) Category: FUB Parent: init_sequencer / dfi_cmd_formatter / training interfaces Status: specified — no RTL exists (HAS v0.1 posture)

## 7.1	Purpose

mode_register holds the MR images per memtype and feeds them to the formatter in the order the init_sequencer demands. It also services runtime MR reads and writes — firmware training paths re-touch MR3 for MPR access and MR1 for write leveling, and the LPDDR4 MR set gets similar treatment during CA and WDQ training. The block is a set of registers plus a mux; it doesn’t own the programming order. Order is the init_sequencer’s property, and this page cites that split rather than duplicating it.

## 7.2	Parameters

Table 2.5: Mode register parameters

| Parameter | Type | Range | Default | Meaning | Source |
| --- | --- | --- | --- | --- | --- |
| DDR4_MR_COUNT | int | 7 | 7 | MR0-MR6 for DDR4 | JESD79-4 |
| LPDDR4_MR_COUNT | int | TBD | TBD | LPDDR4 MR space; exact count per JESD209-4 | Q1 |
| DATA_WIDTH | int | 16 | 16 | MR payload width (max across both memtypes) | design point |
| RANK_WIDTH | int | 1 | 1 | rank select for per-rank MR images | design point |

## 7.3	Interface

Table 2.6: Mode register ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | in | 1 | controller clock |
| rst_n | in | 1 | active-low synchronous reset |
| memtype_i | in | 3 | family memtype enum from andesite_pkg |
| rank_i | in | RANK_WIDTH | rank target for MR access |
| wr_en_i | in | 1 | write strobe from init_sequencer or firmware |
| wr_addr_i | in | 6 | MR index (0-6 for DDR4; LPDDR4 index per JESD209-4, 64-entry working bound) |
| wr_data_i | in | DATA_WIDTH | MR write payload |
| rd_en_i | in | 1 | read strobe for firmware readback |
| rd_addr_i | in | 6 | MR index for readback |
| rd_data_o | out | DATA_WIDTH | MR readback data |
| mr_sel_o | out | 6 | MR index to formatter for MRS/MRW command |
| mr_data_o | out | DATA_WIDTH | MR payload to formatter for MRS/MRW command |
| mpr_page_o | out | 2 | MR3 MPR page select to rdlvl_ifc |
| fgr_factor_o | out | 2 | MR3 FGR 1x/2x/4x select to refresh_ctrl |
| rtt_nom_o | out | 3 | MR1 RTT_NOM value to odt_ctrl |
| rtt_wr_o | out | 3 | MR2 RTT_WR value to odt_ctrl |
| rtt_park_o | out | 3 | MR5 RTT_PARK value to odt_ctrl |
| rd_dbi_en_o | out | 1 | MR5 read DBI enable to datapath |
| wr_dbi_en_o | out | 1 | MR5 write DBI enable to datapath |
| ca_parity_lat_o | out | 2 | MR5 CA parity latency/mode to formatter |
| wrlvl_en_o | out | 1 | MR1 write-leveling enable to wrlvl_ifc |
| lpddr4_odt_o | out | TBD | LPDDR4 ODT/DQ-ODT programming to PHY/odt_ctrl |

## 7.4	Microarchitecture internals

### 7.4.1	DDR4 MR semantics

The DDR4 mode-register set expands scoria’s four registers to seven. The table below is semantics-binding; bit positions per JESD79-4 are confirmed at Q1 cold-storage read.

Table 2.7: DDR4 MR0-MR6 field semantics

| Register | Fields (semantics-binding) |
| --- | --- |
| MR0 | Burst length (fixed 8 or on-the-fly 4/8), read burst type (sequential/interleaved), CAS latency select, DLL reset bit, write recovery |
| MR1 | DLL enable, additive latency (AL), RTT_NOM, write-leveling enable, TDQS enable, output driver impedance |
| MR2 | CAS write latency (CWL), RTT_WR, write CRC mode bits (inert this edition per HAS Ch 3.1), LP ASR |
| MR3 | MPR operation and page select, FGR refresh factor (1x/2x/4x), gear-down mode, MPR read format |
| MR4 | Temperature status, preamble, CAL (command address latency) |
| MR5 | Read DBI enable, write DBI enable, RTT_PARK, data-mask enable, CA parity latency/mode (A[2:0]), parity persistent-error (A9), parity error status (A4) |
| MR6 | VrefDQ training range and value, tCCD_L select |

Bit map per JESD79-4 MRn (confirmed at Q1 cold-storage read); the maps above are semantics-binding.

### 7.4.2	LPDDR4 MR set

LPDDR4 doesn’t use the same MRS command as DDR4. Its MRs are written by MRW over the 6-bit CA bus and read back by MRR. The page owns the LPDDR4 write image alongside DDR4’s. LPDDR4’s MRs cover ODT and DQ-ODT programming, drive strength, CA training patterns, refresh-related bits, and the vendor-specific area. Exact LPDDR4 MR bit maps per JESD209-4 (Q1). The coupling is looser than DDR4’s because termination is MR-programmed rather than ODT-pin driven, but the register-block structure is the same: one image per rank, one write port, one readback port.

### 7.4.3	MR coupling table

Table 2.8: Mode-register coupling

| MR / Field | Consumer | What it controls |
| --- | --- | --- |
| MR3 FGR select | refresh_ctrl | Refresh interval scaling: 1x, 2x or 4x |
| MR3 MPR page | rdlvl_ifc | MPR read-leveling page and format |
| MR2 RTT_WR | odt_ctrl | Write termination for DDR4 |
| MR5 RTT_PARK | odt_ctrl | Idle termination for DDR4 |
| MR5 RD/WR DBI | datapath (dfi_rd_aligner, dfi_wr_serializer) | Data Bus Inversion enable |
| MR5 CA parity | dfi_cmd_formatter | Parity latency and mode |
| MR1 write leveling | wrlvl_ifc | Write-leveling entry/exit |
| MR1 RTT_NOM | odt_ctrl | Nominal termination for DDR4 |
| MR2 CWL | datapath / scheduler | CAS write latency (value flows to write-data timing) |
| MR6 VrefDQ | rdlvl_ifc / firmware | VrefDQ training range and value |
| LPDDR4 ODT/DQ-ODT | PHY / odt_ctrl | LPDDR4 MR-programmed termination |

## 7.5	FSM policy

There is no FSM. The block is registers, a write port, a readback mux, and field fanout to the consumers above. Programming order is the init_sequencer’s property (HAS Ch 3.2). Runtime MRW/MRR requests from firmware are single-cycle register updates; the formatter turns mr_sel_o and mr_data_o into the actual bus command.

## 7.6	Timing

MR writes and reads complete in one controller clock. The formatter and sequencer enforce the JEDEC command intervals (tMRD, tMOD) around MRS and MRW commands. Those values are runtime CSRs, initialised from the JESD79-4/JESD209-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1).

## 7.7	Notes

- Keep the DDR4 and LPDDR4 image arrays in separate address spaces selected by memtype_i. Don’t try to map them onto one flat array — the field widths and meanings differ enough that a unified map invites mistakes.

- The MR3 FGR factor and MPR page are the two fields most likely to be rewritten at runtime; expose them as named CSR fields even if the rest of the MR image is write-only at init.

- LPDDR4’s MR-programmed termination means the odt_ctrl block is DDR4-scoped for this edition; the LPDDR4 ODT values still live here and fan out to the PHY configuration path.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 8	Address Mapper (andesite_addr_mapper)

Module: andesite_addr_mapper.sv Location: rtl/fub/ (planned) Category: FUB Parent: scheduler_layer / cmd_arbiter Status: carried from scoria and landed — the tables below name the landed port lists

## 8.1	Purpose

addr_mapper turns the system address into the rank, bank-group, bank, row and column fields the scheduler and formatter need. The andesite change is one bounded addition: bank group (BG0/BG1) sits between chip select and bank. scoria’s decode was rank, bank, row, column; andesite inserts bank group above bank for DDR4. LPDDR4 has no bank groups, so the group field is constant — degenerate, not special-cased.

## 8.2	Parameters

Table 2.9: Address mapper parameters

| Parameter | Type | Range | Default | Meaning | Source |
| --- | --- | --- | --- | --- | --- |
| AXI_ADDR_WIDTH | int | 32..48 | 32 | host/system address width | design point |
| NUM_RANKS | int | 1, 2, 4 | 1 | ranks per channel | design point |
| NUM_BANKS | int | 4, 8 | 8 | banks per rank | design point |
| BG_WIDTH | int | 0..2 | 2 | bank-group bits (0 for LPDDR4) | design point |
| ROW_WIDTH | int | 14..18 | 14 | row bits | design point |
| COL_WIDTH | int | 10..12 | 10 | column bits (without burst offset) | design point |
| BYTE_OFFSET_WIDTH | int | 3..5 | 3 | log2 of the beat byte size (8 B -> 3) | design point |

## 8.3	Interface

Table 2.10: Address mapper ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| — | — | — | (no clock port: the mapper is pure combinational, carried from scoria) |
| — | — | — | (no reset port for the same reason) |
| axi_addr_i | in | AXI_ADDR_WIDTH | incoming system address |
| bank_lsb_i | in | 5 | bank field LSB in the word address (field-boundary knob) |
| rank_o | out | $clog2(NUM_RANKS) | decoded rank / chip select |
| bg_o | out | BG_WIDTH | decoded bank group (constant for LPDDR4) |
| bank_o | out | $clog2(NUM_BANKS) | decoded bank |
| row_o | out | ROW_WIDTH | decoded row |
| col_o | out | COL_WIDTH | decoded column (burst offset stripped) |
| hash_en_i | in | 1 | bank XOR-hash enable (carried from pumice; HASH_EN was never a parameter) |
| hash_seed_i | in | 8 | hash seed |

## 8.4	Microarchitecture internals

### 8.4.1	Decode structure

The decode is parameterized and runtime-programmable via the ADDR_MAP-style CSR. Field boundaries reset to the design-point geometry and can be moved by firmware for characterization.

sys_addr -> { cs[CS-1:0],
              bg[BG-1:0]       (DDR4: BG0/BG1; LPDDR4: constant 0),
              bank[BANK-1:0]   (DDR4: 2 bits; LPDDR4: 3 bits),
              row[ROW-1:0],
              col[COL-1:0] }

Field boundaries are runtime CSRs (ADDR_MAP-style), reset to the design point's
geometry; hash-en/hash-seed carried from pumice if enabled.

: Figure 2.2: Address decode structure (citation anchor)

memtype_i is deliberately absent: the decode is geometry-parameterized, not memtype-branched (the LPDDR4 degeneration is bg_o tied to zero by the consumer, per the note below the table). The ADDR_MAP-style field boundary is the bank_lsb_i knob, runtime-set, exactly as scoria’s mapper. The bg_o delta is the andesite change; every other port is scoria’s.

### 8.4.2	Scheduler-visible outputs

The mapper computes the group identity the arbiter gates on. For each candidate command, the scheduler sees {rank, bg, bank} and uses it for the same-group / cross-group checks described in ch02_blocks/05_scheduler.md. LPDDR4 has no bank groups, so bg_o is constant zero and the same-group check collapses to rank+bank — graceful degeneration, not a special case. The long/short timing pairs (tCCD_L/tCCD_S, tRRD_L/tRRD_S) still apply structurally; LPDDR4 simply makes L equal S.

### 8.4.3	Hashing

Address hashing is inherited from pumice and carried in scoria. When hash_en_i is set, a subset of the row/column bits is XOR-scrambled with hash_seed_i before the final field extraction. This spreads row-buffer contention across banks. The hash function is unchanged from the landed pumice implementation; andesite only widens the address path to include bank group.

## 8.5	FSM policy

There is no FSM. The block is pure combinational decode plus optional registered outputs to the scheduler. The fixed-form caution applies: this module inherits scoria’s tested decode and extends it only in the named dimension — bank group between chip select and bank. No new hashing modes, no new address rearrangements.

## 8.6	Timing

Decode is combinational. Registered outputs add one controller-clock delay if needed to meet scheduler timing. Geometry is build-time per HAS Ch 2.4/5; field-boundary CSRs are runtime.

## 8.7	Notes

- The bg_o width is 2 for DDR4 and 0 for LPDDR4 at the parameter level. If zero-width ports are awkward in your RTL style, tie the LPDDR4 path to constant zero inside the module and keep the port width at 2 — but don’t make the scheduler special-case it.

- The design point fixes DDR4-1600 x8 with 4 bank groups × 4 banks and LPDDR4-1600 x16 with 8 ungrouped banks per channel. Reset the ADDR_MAP CSRs to those geometries so firmware doesn’t have to.

- scoria’s decode equations are the starting point; the kmap book’s first address-map targets are the verification targets for andesite.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 9	Scheduler and Command Arbiter (andesite_scheduler_layer + andesite_cmd_arbiter)

Module: andesite_scheduler_layer.sv, andesite_cmd_arbiter.sv Location: rtl/macro/ (scheduler), rtl/fub/ (arbiter) Category: scheduling / arbitration Parent: andesite_core Status: carried from scoria and landed — arbiter carries the L/S delta; the tables below name the landed port lists. The macro’s init/mode-register instances are rewired to the P1 blocks (macro-integration pass, 2026-10-05): the P1 init_sequencer is self-timed and DDR4-only, and the P1 mode_register policy outputs are the macro’s observability contract.

## 9.1	Purpose

The scheduler picks which ready command goes to the formatter each cycle. It inherits scoria’s FR-FCFS arbiter and its maintenance request/grant channel. The andesite delta is L/S-aware issue: DDR4 bank groups add a same-group versus cross-group split to the command-spacing checks, and the global timers grow the tCCD_L/S and tRRD_L/S pairs. LPDDR4 has no bank groups, so the long and short checks collapse to one.

The scheduler never invents a new arbitration philosophy. It takes scoria’s mechanism and teaches it about bank groups.

## 9.2	Parameters

Table 2.5.1: Scheduler parameters

| Parameter | Type | Range | Default | Meaning |
| --- | --- | --- | --- | --- |
| NUM_RANKS | int | 1+ | 1 | ranks per channel |
| NUM_BANKS | int | 8..16 | 8 | banks per rank |
| NUM_BG | int | 1..4 | 4 | bank groups; 1 is the LPDDR4 degeneration (the L/S delta parameter) |
| NUM_ENTRIES | int | 8..32 | 8 | read and write CAM depth |
| AGE_WIDTH | int | — | 16 | age-counter width |

NUM_BG = 1 is how LPDDR4 degenerates gracefully. There is no if (memtype == LPDDR4) branch in the issue check; the group index is constant and the L/S counters collapse by construction.

## 9.3	Interface

### 9.3.1	Host-side command ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| aclk | in | 1 | controller clock |
| aresetn | in | 1 | active-low reset |
| init_done_i | in | 1 | scheduler admits no host commands until high |
| rd_sch_valid_i | in | NUM_ENTRIES | read CAM per-entry valid (the CAMs live in the intake/AXI interface) |
| wr_sch_valid_i | in | NUM_ENTRIES | write CAM per-entry valid |
| rd_sch_bank_i | in | NUM_ENTRIES*BKW | read candidate bank per entry; rd_sch_row_i/rd_sch_col_i carry the rest of the payload |
| wr_sch_bank_i | in | NUM_ENTRIES*BKW | write candidate bank per entry; wr_sch_row_i/wr_sch_col_i alongside |
| rd_sch_older_i | in | NUM_ENTRIES^2 | read CAM age-order matrix; the write side is wr_sch_older_i |
| rd_issue_valid_o | out | 1 | read command wins this cycle; rd_issue_slot_o names the winning entry |
| wr_commit_valid_o | out | 1 | write command wins this cycle; wr_commit_slot_o names the winning entry |
| cmd_valid_o | out | 1 | issued command strobe toward the formatter path; cmd_op_o, cmd_rank_o, cmd_bank_o, cmd_row_o, cmd_col_o, cmd_ap_o carry it, cmd_ready_i back-pressures |
| cmd_bg_o | out | $clog2(NUM_BG) | bank group of the issued command (the arbiter’s registered-pick group, bank[BKW-1 -: BGW]); rides the widened command word {ap,col,row,bg,bank,rank,op} to the DFI formatter |

### 9.3.2	Bank-group timing ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| tccd_ok_i | in | 1 | tCCD base spacing ok (cross-group; the LPDDR4 degenerate check) |
| tccd_l_ok_i | in | 1 | tCCD_L same-group spacing ok — the andesite L/S delta input |
| trrd_ok_i | in | 1 | tRRD_S-class ACT-to-ACT ok, per rank |
| trrd_l_ok_i | in | 1 | tRRD_L same-group ACT-to-ACT ok — L/S delta input |
| tfaw_ok_i | in | 1 | tFAW four-activate window ok, per rank |
| twtr_ok_i | in | 1 | write-to-read turnaround ok; trtw_ok_i is the read-to-write partner |
| bank_act_ready_i | in | NUM_RANKS x NUM_BANKS | per-bank timer readiness (activate); bank_rdwr_ready_i, bank_pre_ready_i and the _la_i lookahead variants ride alongside |

The L/S counters live in the global timers; the arbiter consumes the ok flags. The bank group of the last-issued command is tracked inside the arbiter (r_last_col_bg, r_last_act_bg), not on a port — there is no last_cmd_group signal.

### 9.3.3	Maintenance request/grant ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| refresh_req_i | in | 1 | refresh wants the bus (from refresh_ctrl) |
| refresh_grant_o | out | 1 | bus granted to refresh |
| refresh_drain_i | in | 1 | refresh drain in progress; CAMs hold |
| refresh_kind_i | in | 1 | 0 = all-bank REF, 1 = per-bank (LPDDR4) |
| refresh_bank_i | in | BKW | per-bank target mirror (the rotor) |
| zq_req_i | in | 1 | ZQ calibration wants the bus (from zq_ctrl) |
| zq_grant_o | out | 1 | bus granted to ZQ |

The carried RTL keeps scoria’s two request/grant pairs — one for refresh, one for ZQ — rather than one unified maint_req/maint_tag channel; the unified tagged channel stays the conceptual model, and the formatter-facing tag is derived downstream. The ODT turnaround request lands with odt_ctrl (Ch 2.8). The rule is inherited from scoria: request and wait, never preempt.

Table 2.5.2: Scheduler interface

## 9.4	Macro init/policy interface (P1 rewiring, landed 2026-10-05)

The scheduler macro owns the P1 init_sequencer and mode_register instances. The scoria DDR3-era connections (mc_clk-style pins, the dfi_init_start/dfi_init_complete PHY handshake, and the CL/CWL/BL shadow outputs) retired with the rewiring — the P1 sequencer is self-timed (tINIT* CSRs; any non-DDR4 memtype latches init_err) and the P1 register never carried latency shadows.

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| t_init_wait_i | in | 16 | tINIT1 — RESET# low time |
| t_rp_wait_i | in | 8 | tINIT3 — RESET# release to CKE (zero-extended) |
| t_cke_wait_i | in | 16 | tINIT4 — CKE to first MRS |
| t_mrd_wait_i | in | 8 | tMRD — MRS-to-MRS gap (zero-extended) |
| t_mod_wait_i | in | 16 | tMOD — last MRS to ZQCL |
| t_dll_wait_i / t_zqinit_wait_i | in | 16 | tDLLK / tZQinit |
| mr0_i … mr6_i | in | 16 ×7 | DDR4 MR images; the P1 FSM writes MR3, MR6, MR5, MR4, MR2, MR1, MR0 |
| dram_reset_n_o / cke_o | out | 1 | DRAM RESET# / CKE pins, driven by the P1 init FSM |
| init_done_o / init_err_o | out | 1 | init complete / unsupported-memtype or watchdog error |
| zq_cal_start_o | out | 1 | status strobe marking the init FSM’s ZQCL issue (the ZQCL itself rides the command stream) |
| gear_down_entry_o / ca_train_start_o / parity_enable_o | out | 1 | P1 init-FSM status outputs (parity latches on at MR5 per the init sequence) |
| rtt_nom_o / rtt_wr_o / rtt_park_o | out | 3 | ODT policy images (the odt_ctrl observability contract) |
| rd_dbi_en_o / wr_dbi_en_o | out | 1 | MR5 read/write DBI enables (the DFI datapath consumers) |
| mpr_page_o / fgr_factor_o / ca_parity_lat_o / lpddr4_odt_o | out | 2 / 2 / 2 / 3 | MR3 MPR page, FGR factor, CA-parity latency, LPDDR4 ODT image |
| wrlvl_en_o | out | 1 | MR1[7] — write-leveling mode authority (to the training layer) |

Init commands (MRS / ZQCL) reach the DRAM through the ordinary command stream: the P1 cmd_req/cmd_ack handshake is honored by the arbiter’s accept pulse (cmd_ack = accept qualified by OP_MRS/OP_ZQCL), and the arbiter forwards the init payload (cmd_addr low bits) on the stream’s row field, exactly as the carried scoria forwarding did. The init MR index rides cmd_bank into the mode-register store’s write address.

## 9.5	Microarchitecture internals

### 9.5.1	FR-FCFS admission with L/S awareness

scoria’s FR-FCFS arbiter is unchanged in spirit. Each candidate command is scored by age and readiness, and the highest scorer that passes the timing checks is granted. The andesite change is inside the timing check.

issue_ok(cmd) = bank_timers.ok(cmd)
                AND global_timers.ok(cmd, group(cmd), group(last_cmd))

group(cmd) is the bank group of the candidate’s bank. For a read or write command, the global-timer check now depends on whether the candidate targets the same bank group as the last-issued command:

- Same group: tCCD_L and tRRD_L apply.

- Different group: tCCD_S and tRRD_S apply.

The check also covers the outstanding same-group spacing obligations a burst of commands to one group leaves behind; the last-issued group registers (r_last_col_bg, r_last_act_bg) and the L/S ok flags carry that state into the admission check.

### 9.5.2	Global timers

The global timer block inherits scoria’s counters and adds four new ones. All are runtime CSRs.

| Counter | JEDEC symbol | Applies |
| --- | --- | --- |
| tCCD_L | tCCD_L | same bank group |
| tCCD_S | tCCD_S | different bank groups |
| tRRD_L | tRRD_L | same bank group, activate-to-activate |
| tRRD_S | tRRD_S | different bank groups, activate-to-activate |

These join the inherited tCCD, tRAS, tRC, tRP, tRCD, tWTR, tRTW, tWR, and the read/write turnaround set. The values are runtime CSRs, initialised from the JESD79-4/JESD209-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1).

### 9.5.3	Maintenance request/grant semantics

The carried RTL keeps scoria’s channel shape: two request/grant pairs into the arbiter, one for refresh (refresh_req_i/refresh_grant_o) and one for ZQ (zq_req_i/zq_grant_o). The original spec sketch on this page was one unified maint_req with a maint_tag source identifier; the carry ruled that the inherited pairs are the truth this tier, and the tag survives only as the conceptual model the formatter-side maintenance encoding derives from.

| Source | Arbiter channel | Action when granted |
| --- | --- | --- |
| refresh_ctrl | refresh_req_i/refresh_grant_o | issue REF or LPDDR4 per-bank refresh |
| zq_ctrl | zq_req_i/zq_grant_o | issue ZQCS, ZQCL, or LPDDR4 MPC ZQ calibration |
| andesite_training_layer | trn_cmd_req_i/trn_cmd_grant_o | issue training MRS or MPC; request-and-wait, all-banks-idle two-step shape like refresh |
| odt_ctrl | lands with Ch 2.8 | issue the ODT turnaround command or NOP-with-ODT |

The rule is inherited from scoria: request and wait, never preempt. A maintenance source raises its req and holds it until grant arrives. It cannot yank the bus away from an in-flight host command.

### 9.5.4	LPDDR4 degeneration

LPDDR4 has no bank groups, so NUM_BANK_GROUPS = 1 and group(cmd) is always zero. The same-group and cross-group checks collapse into a single check per pair because there is only one group to compare against. The L/S counters still exist, but they are loaded with the same value. This must fall out of the parameterization, not out of an if (memtype == LPDDR4) branch.

This is worth stating plainly: LPDDR4 is not a special case in the arbiter. It is the NUM_BANK_GROUPS = 1 corner of the same code.

## 9.6	FSM policy

The arbiter is priority logic plus CAMs, not an FSM. That is inherited from scoria. The scheduler’s grant sequencing — one command per cycle, maintenance highest, FR-FCFS among host commands — is the same mechanism with the L/S inputs added.

The global timers are counters, not a state machine. Each timer decrements every cycle and asserts its ok flag at zero.

## 9.7	Timing

One grant per cycle, as inherited. The new counters add combinational delay to the admission check but no new pipeline stages. The critical path is:

- Read CAM and write CAM produce candidates.

- Bank timers check per-bank readiness.

- Global timers check L/S spacing; the last-issued group registers select the long or short window.

- FR-FCFS priority selects the winner.

- Grant is registered and the command is issued.

The counter values are runtime CSRs, initialised from the JESD79-4/JESD209-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1).

## 9.8	Notes

- Why pairs, not a unified tagged channel: the carried RTL is scoria’s two request/grant pairs, kept byte-behavioral. The scheduler doesn’t need to know what kind of maintenance it is granting; it only needs to know that maintenance beats host traffic and that only one source wins per cycle. A unified maint_req/maint_tag channel stays on the books as the conceptual model, not the landed interface.

- L/S vs the old tCCD: The inherited tCCD is kept as a fallback for modes where bank-group information is not available, but the primary check is the L/S pair. DV must prove that tCCD_L and tCCD_S subsume the old tCCD correctly for DDR4 and collapse to it for LPDDR4.

- Maintenance starvation: Because maintenance never preempts, a pathological stream of host commands could delay refresh or ZQ. The refresh controller’s credit window and the ZQ controller’s overdue counter handle this at the source; the scheduler’s only obligation is fair arbitration once the request is raised.

## 9.9	cmd_history_checker’s DDR4 growth (landed 2026-10-04)

The checker’s spacing-parameter set now carries the long/short pairs the bank-group scheduler defines: T_CCD_L/T_CCD_S (CAS-to-CAS within / across a bank group) and T_RRD_L/T_RRD_S (ACT-to-ACT), all runtime parameters inert at 0, plus a cmd_bg_i bank-group command input. scoria’s nine checks are byte-identical; checks (10)-(13) are additive. The ported suite proves each new check fires with its own fatal text (fires_tccd_l/s, fires_trrd_l/s) and that legal streams at both minima stay silent (legal_ls_pairs_at_the_minimum).

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 10	Refresh Controller (andesite_refresh_ctrl)

Module: andesite_refresh_ctrl.sv Location: rtl/fub/ Category: maintenance / refresh Parent: andesite_core Status: carried from scoria and landed, FGR delta included — the ports table names the landed list; the fence below is the design citation (the generator diffs it)

## 10.1	Purpose

refresh_ctrl keeps the DRAM rows alive. It inherits scoria’s verified base, specified in projects/components/mem-ctrl-ip/scoria-ddr3-lpddr3/docs/scoria_has/ch03_architecture/04_refresh.md and proven in formal/scoria/refresh_ctrl: the tREFI interval counter, all-bank refresh, JEDEC’s plus-or-minus-eight postpone/pull-in credit window, and the TASK-001 policy modes (A: elastic refresh, B: temperature-compensated refresh, C: placement discipline). The andesite deltas are DDR4 fine-granularity refresh and LPDDR4 controller-directed per-bank refresh.

The block is MODIFIED, not rewritten. Every inherited property transfers with the mechanism.

## 10.2	Parameters

Table 2.6.1: Refresh controller parameters

| Parameter | Type | Range | Default | Meaning |
| --- | --- | --- | --- | --- |
| NUM_BANKS | int | 8..16 | 8 | banks per rank |
| BA_W | int | — | $clog2(NUM_BANKS) | bank-address width (drives refresh_bank_o) |

## 10.3	Interface

Table 2.6.2: Refresh controller ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mc_clk | in | 1 | controller clock |
| mc_rst_n | in | 1 | active-low reset |
| enable_i | in | 1 | refresh engine enable |
| t_refi_i | in | 16 | tREFI reload value, runtime CSR; the FGR delta divides it by the factor (below) |
| refi_reload_i | in | 1 | reload the countdown immediately (DV/bring-up pulse; tie 0 in production) |
| trefi_pb_i | in | 16 | per-bank tREFI for the LPDDR4 REFpb path |
| refresh_burst_i | in | 4 | refreshes owed per request cycle (the JEDEC burst) |
| refpb_mode_i | in | 1 | 0 = all-bank REF, 1 = controller-directed per-bank refresh |
| demand_i | in | 1 | demand indication from the scheduler side |
| elastic_en_i | in | 1 | Mode A elastic refresh enable; pullin_idle_streak_i (8) and postpone_demand_streak_i (7) ride as its thresholds |
| tcr_en_i | in | 1 | Mode B temperature-compensated refresh enable; trefi_derate_i (2) is its derate select, reload-only as inherited |
| fgr_factor_i | in | 2 | DDR4 FGR factor from the MR3 image: 0 = 1x, 1 = 2x, 2 = 4x; an illegal encoding clamps to the 1x row (the kmap FGR select table — deliberately not the Mode B 3→2 clamp) |
| t_rfc_1x_i | in | 16 | tRFC at 1x density, runtime CSR; t_rfc_2x_i and t_rfc_4x_i carry the 2x/4x densities — the factor selects among them on refresh_trfc_o |
| postpone_limit_i | in | 4 | credit-window postpone bound (JEDEC +8); pullin_limit_i is the pull-in bound |
| refresh_req_o | out | 1 | request to the scheduler; held until refresh_grant_i — request-and-wait, never preempt |
| refresh_grant_i | in | 1 | scheduler grant |
| grant_was_pb_i | in | 1 | the granted command was per-bank |
| refresh_drain_active_o | out | 1 | drains in-flight commands before the refresh issues |
| refresh_kind_o | out | 1 | 0 = all-bank, 1 = per-bank |
| refresh_bank_o | out | BA_W | per-bank refresh target (the rotor output) |
| pending_refreshes_o | out | 4 | credit-window occupancy (postpones owed) |
| obs_refi_cnt_o | out | 16 | interval-countdown observability; obs_drain_remaining_o, obs_bank_rotor_o, obs_grants_total_o, obs_pullin_credit_o, obs_postpone_events_o, obs_pullin_events_o complete the telemetry set |
| refresh_trfc_o | out | 16 | tRFC_active = tRFC(fgr), the factor-selected recovery window; consumed by the recovery owner (the arbiter) when the macro rewiring lands |

The FGR delta is reload-only: a factor change mid-interval takes effect on the next reload, never the running counter — the same posture Mode B carries. The factor and the per-density tRFC CSRs are ports, not parameters.

## 10.4	Microarchitecture internals

### 10.4.1	Inherited mechanism

The base mechanism is scoria’s refresh_ctrl, proven in formal/scoria/refresh_ctrl. It carries:

- A tREFI interval counter.

- The JEDEC ±8 postpone/pull-in credit window.

- Mode A elastic streaks: pullin_idle_streak reset to 16, postpone_demand_streak reset to 1.

- Mode B TCR derate applied to the counter reload only.

- REF_STATS telemetry.

- The sixteen-cycle idle confirmation, because CAM occupancy blinks off between bursts and those micro-gaps must not be treated as idle.

The credit ceiling is ±8 in every mode. Mode A and Mode B touch scheduling policy, not the credit arithmetic. ### DDR4 delta: fine-granularity refresh

DDR4’s MR3 selects 1x, 2x, or 4x refresh granularity. The controller scales its interval arithmetic by the FGR factor.

fgr_factor = {1x:1, 2x:2, 4x:4}   (MR3 image, runtime CSR)
tREFI_effective = tREFI / fgr_factor   (counter reload value)
tRFC_active = tRFC(fgr)                 (CSR per density)
credits window scales with the density bookkeeping — the ±8 ceiling is unchanged.

fgr_factor_i is the runtime copy of the MR3 field that init_sequencer programmed (0 = 1x, 1 = 2x, 2 = 4x; illegal clamps to the 1x row per the kmap FGR table — deliberately not the Mode B 3→2 clamp). The tREFI counter reloads with tREFI_effective. The per-refresh recovery window is tRFC_active, selected from the t_rfc_1x_i, t_rfc_2x_i, or t_rfc_4x_i CSR by the factor and exposed on refresh_trfc_o for the recovery owner; the scaling is reload-only, so a factor change mid-interval takes effect on the next reload.

The credit-window bookkeeping scales with the density accounting, but the ±8 ceiling is unchanged. Postponing 8 refreshes at 4x granularity is not the same as postponing 8 refreshes at 1x; the bookkeeping must track effective refreshes, not raw commands.

### 10.4.2	LPDDR4 delta: controller-directed per-bank refresh

LPDDR4 changes who names the bank. For LPDDR2/3, REFpb carried no bank address and scoria’s rotor mirrored the device’s fixed counter. JESD209-4 lets the controller put the bank on the CA bus, so the rotor is replaced by explicit bank scheduling.

This edition implements a round-robin default. The policy layer that scoria’s TASK-001 built — CSR-select, reset-off, telemetry-instrumented — is exactly where occupancy-aware schemes from andesite TASK-001’s DARP survey would hang. Round-robin is the commodity choice; advanced schemes are left as future policy hooks.

The scheduling policy hook picks the next bank. On the carried block the hook is internal: a round-robin rotor advances the per-bank pointer and drives refresh_bank_o (observable on obs_bank_rotor_o), with no bank-busy input — the busy-masked, occupancy-aware variants are the future policy this page leaves hooked, not a port the inherited engine has. Future policies can replace the hook without touching the refresh engine.

## 10.5	FSM policy

The refresh engine’s FSM is inherited in shape: request/grant, issue, then track the busy interval. The FGR factor and per-bank selection are inputs, not new states.

Mode selects are data, not control. Whether the current refresh is 1x or 4x, all-bank or per-bank, round-robin or occupancy-aware — these choices change the values loaded into counters and the bank address driven to the scheduler. They do not change the state machine’s skeleton. That’s why the FSM stays inherited.

## 10.6	Timing

The tREFI counter reloads with tREFI_effective, a runtime CSR derived from the JESD79-4/JESD209-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1). tRFC_active is selected from the FGR-specific CSR.

The Mode B TCR derate applies to the reload value only: a mid-interval CSR change takes effect on the next reload, not on the running counter. That’s the same reload-only rule inherited from scoria.

## 10.7	Notes

- Formal obligation: Retention must be re-derived per FGR factor and for the per-bank arithmetic. Carrying the 1x all-bank proof forward unchanged would prove the wrong thing while appearing green. The obligation is recorded here and in HAS Ch 6 item 5.

- Refresh is an ordinary bank occupant: Bank timers own the banks during refresh. A refresh request is not a special bypass; it waits its turn in the scheduler like any other command.

- Why round-robin only: Occupancy-aware per-bank refresh is implementable now that the controller names the bank, but it is a policy change, not a mechanism change. The policy hook exists; the survey’s schemes stay in andesite TASK-001.

- Telemetry: REF_STATS carries over. ref_stats_fgr_factor and ref_stats_perbank_bank join it so a refresh sweep is measurable in-system rather than inferred.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 11	ZQ Calibration Controller (andesite_zq_ctrl)

Module: andesite_zq_ctrl.sv + andesite_zq_mpc_lpddr4.sv (both landed) Location: rtl/fub/ Category: maintenance / calibration Parent: andesite_core Status: DDR4 core carried from scoria and landed; the LPDDR4 MPC submodule is landed per the FSM below; ports tables reconciled to the landed port lists

## 11.1	Purpose

zq_ctrl issues periodic ZQ calibration. For DDR4, the block is scoria’s module carried unchanged: interval CSR, request-and-wait grant, tZQCS enforced by holding the next interval, issued-calibration counter, and CSR-selectable placement policy. For LPDDR4, calibration moves onto the MPC command, so a new submodule sequences it through the formatter’s LPDDR4 CA path.

The DDR4 side is INHERITED. The LPDDR4 side is NEW. Both share one maintenance request port to the scheduler with a source tag.

## 11.2	Parameters

Table 2.7.1: ZQ controller parameters

| Parameter | Type | Range | Default | Meaning |
| --- | --- | --- | --- | --- |
| — | — | — | — | (no parameters; the interval and overdue widths are fixed on the ports — 32-bit interval, 13-bit overdue) |

## 11.3	Interface

### 11.3.1	Common ports (the landed DDR4 core)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mc_clk | in | 1 | controller clock |
| mc_rst_n | in | 1 | active-low reset |
| enable_i | in | 1 | calibration engine enable (ZQ_CFG.zq_enable) |
| memtype_i | in | 3 | memtype_e: selects the calibration path — MEMTYPE_LPDDR4 hands expiries to the MPC submodule; anything else runs the inherited DDR4 core |
| t_zqcs_interval_i | in | 32 | calibration interval, runtime CSR; 0 = off (shared by both paths) |
| t_zqcs_i | in | 16 | tZQCS recovery, runtime CSR (DDR4 path) |
| placement_i | in | 2 | placement policy select (Mode C, DDR4 path) |
| overdue_max_i | in | 13 | defer-under-demand threshold (DDR4 path) |
| demand_i | in | 1 | demand indication from the scheduler side |
| zq_req_o | out | 1 | request to the scheduler; held until zq_grant_i — request-and-wait, never preempt (muxed between the core and the submodule) |
| zq_grant_i | in | 1 | scheduler grant |
| obs_busy_o | out | 1 | a calibration is in flight (post-grant window, either path) |
| obs_zqcs_total_o | out | 16 | issued-calibration counter (shared: counts either path’s completion) |
| obs_interval_cnt_o | out | 32 | interval countdown observability (shared by both paths) |
| obs_overdue_o | out | 1 | overdue flag (DDR4 placement-policy telemetry; the LPDDR4 path does not defer, so it stays low there) |

The carried DDR4 core exposes no DDR4-only ports beyond this set: ZQCS is its only calibration flavour, and the init-time long calibration is sequenced by the scheduler macro’s t_zqinit_wait_i window, not by this block. The original spec sketch’s init_zqcl_req/init_zqcl_done handshake and the unified maint_req/maint_tag channel are not how the carried RTL works — the core requests calibration on zq_req_o and waits on zq_grant_i.

### 11.3.2	LPDDR4-only ports (the landed MPC submodule)

The submodule is andesite_zq_mpc_lpddr4 (its own FUB file); zq_ctrl instantiates it and memtype-muxes the scheduler pair and the busy observable — a combinational passthrough of the two registered sources, so each path keeps single-register latency.

Table 2.7.2: ZQ controller ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| t_zq_i | in | 16 | tZQ calibration latency, runtime CSR (JESD209-4 speed-bin derived); waited out in MPC_WAIT |
| mpc_opcode_i | in | 6 | MPC opcode image from the CSR. The encodings are TBC(JESD209-4): the submodule drives the image and never decodes it — no MPC opcode encodings are invented anywhere in andesite |
| mpc_issuing_o | out | 1 | high in MPC_ISSUE; the formatter samples the image with the grant cycle |
| mpc_op_o | out | 6 | the opcode image presented while issuing (follows mpc_opcode_i combinationally) |

## 11.4	Microarchitecture internals

### 11.4.1	DDR4 path: inherited from scoria

The DDR4 core is scoria’s zq_ctrl, unchanged. It implements:

- Interval CSR: countdown to the next calibration event.

- Request-and-wait grant: it asks the scheduler and waits; it never preempts.

- tZQCS enforcement: the next interval is held until the recovery time expires.

- Issued-calibration counter: tracks how many calibrations have gone out.

- CSR-selectable placement policy: scoria TASK-001 Mode C, baseline versus defer-under-demand with overdue_max.

For the internal details, see scoria’s HAS Ch 3.2 and the landed RTL in projects/components/mem-ctrl-ip/scoria-ddr3-lpddr3/rtl/zq/. This MAS page does not re-specify what already exists; it cites it.

### 11.4.2	LPDDR4 path: new MPC submodule

LPDDR4 carries calibration on the MPC command, not dedicated ZQCS/ZQCL opcodes. The new submodule issues the ZQCal start and ZQCal latch opcodes through the formatter’s LPDDR4 CA submodule.

MPC_IDLE
    -> MPC_ISSUE   (assert cmd_req, wait for scheduler grant, drive MPC ZQCal start)
    -> MPC_WAIT    (wait tZQ latched from CSR)
    -> MPC_DONE    (drive calibration complete, reload interval counter)
    -> MPC_IDLE

tZQ here is the LPDDR4 ZQ calibration latency, the t_zq_i runtime CSR loaded from JESD209-4 at CSR-derivation time. The submodule rides the core’s shared interval counter and telemetry: obs_zqcs_total_o counts either path’s completion, obs_interval_cnt_o is the live countdown, and MPC_DONE is what reloads the shared interval.

The MPC submodule depends on the LPDDR4 CA path in dfi_cmd_formatter (Ch 2.1). It does not build its own CA encoder; it drives the MPC opcode and lets the formatter place it on the 6-bit CA bus.

### 11.4.3	Scheduler interface

zq_ctrl presents its calibration request on the carried pair, zq_req_o/zq_grant_i, alongside refresh’s own pair. The scheduler does not special-case ZQ versus refresh; each source waits its turn. See 05_scheduler.md for the maintenance request/grant table.

## 11.5	FSM policy

The inherited DDR4 core keeps whatever FSM it already has. This page does not invent a new one for it.

The LPDDR4 MPC submodule is a new small FSM with the four states listed above: MPC_IDLE, MPC_ISSUE, MPC_WAIT, MPC_DONE. It is not nested under the DDR4 core; it is a sibling path selected by MEMTYPE.

## 11.6	Timing

For DDR4, tZQCS and tZQCL are runtime CSRs enforced by the inherited core. For LPDDR4, tZQ is a runtime CSR, initialised from the JESD209-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1), and the MPC submodule waits it out in MPC_WAIT.

The interval counter reloads when calibration completes. A mid-interval CSR change takes effect on the next reload, following the same reload-only rule used elsewhere.

## 11.7	Notes

- Init-time ZQCL: the carried core has no init handshake. The init-time long calibration is sequenced by the scheduler macro’s t_zqinit_wait_i window (and the init MRS/ZQ chain in the macro); this block owns periodic ZQCS from t_zqcs_interval_i. When the MPC submodule lands it takes over the LPDDR4 side of that chain.

- Telemetry sharing: obs_zqcs_total_o, obs_interval_cnt_o, and obs_overdue_o are the core’s counters. The MPC submodule will share them when it lands; the two paths never operate at once because the memtype selects one build, so there is no arbitration conflict.

- Placement policy: the overdue_max_i/placement_i policy is inherited for DDR4 and carried conceptually for LPDDR4, but this edition only exercises the default path for LPDDR4 MPC calibration. Future policy hooks use the same CSR surface.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 12	ODT Controller (andesite_odt_ctrl)

Module: andesite_odt_ctrl (landed, NEW) Location: projects/components/mem-ctrl-ip/andesite-ddr4-lpddr4/rtl/fub/ Category: termination policy / rank control Parent: andesite_top / scheduler boundary Status: landed, NEW — the ports table names the landed list; the fence below is the design citation

## 12.1	Purpose

odt_ctrl turns DDR4’s termination schedule into a per-rank ODT pin policy. The DRAM exposes three termination values — RTT_NOM, RTT_WR, and RTT_PARK — and a set of latencies that say when each one takes effect. This block watches the scheduler’s grant stream, decides which rank should terminate how, and drives the ODT pins with the right delay. It is a tap, not an arbiter; it never stalls a command to make a termination point.

## 12.2	Parameters

Table 2.8.1: odt_ctrl parameters

| Parameter | Type | Range | Default | Meaning | PRD |
| --- | --- | --- | --- | --- | --- |
| NUM_RANKS | int | 1..4 | 1 | ranks sharing the DQ bus | A1 |
| RTT_NOM_IMG | CSR field | vendor-defined | TBD | MR1 RTT_NOM image for this board | A2 |
| RTT_WR_IMG | CSR field | vendor-defined | TBD | MR2 RTT_WR image for this board | A3 |
| RTT_PARK_IMG | CSR field | vendor-defined | TBD | MR5 RTT_PARK image for this board | A4 |
| ODTLon_CSR | CSR | speed-bin range | TBD | assertion latency from command to ODT active | A5 |
| ODTLoff_CSR | CSR | speed-bin range | TBD | de-assertion latency from command to ODT inactive | A6 |
| ODT_TURN_CSR | CSR | speed-bin range | TBD | write-to-read ODT turnaround value | A7 |
| tADC_CSR | CSR | speed-bin range | TBD | command-to-ODT-change delay for transitions | A8 |

The numeric values behind these CSRs come from the selected JESD79-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1).

## 12.3	Interface

Table 2.8.2: odt_ctrl ports

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mc_clk | in | 1 | controller clock |
| mc_rst_n | in | 1 | active-low reset |
| init_done_i | in | 1 | ownership transfers from init_sequencer at this edge; init_odt_i is sampled as the starting pin value |
| init_odt_i | in | NUM_RANKS | ODT value held by init_sequencer until handoff |
| grant_valid_i | in | 1 | the grant tap is valid this cycle |
| grant_op_i | in | 5 | granted command (dram_op_e: RD/RDA/WR/WRA decode; anything else holds state) |
| grant_rank_i | in | RKW | rank selected by the grant |
| odt_pin_o | out | NUM_RANKS | per-rank ODT pin toward the PHY/DRAM |
| odtlon_i | in | 8 | assertion latency, command to ODT active |
| odtloff_i | in | 8 | de-assertion latency, burst to ODT inactive |
| odt_turn_i | in | 8 | write-to-read ODT turnaround |
| tadc_i | in | 8 | command-to-ODT-change delay (also the handoff settle) |
| rtt_nom_img_i | in | 3 | MR1 RTT_NOM image — a value contract (the RTT values are init-side); rtt_wr_img_i/rtt_park_img_i (MR2/MR5) ride alongside |
| hist_state_o | out | 2 per rank | per-rank ODT policy state (packed) |
| trans_count_o | out | 8 per rank | per-rank transition counters (packed) |

## 12.4	Microarchitecture internals

### 12.4.1	Policy state table

The termination policy is per-rank. Each rank is in one of four policy states, selected from the grant stream (the landed RTL names:

state        | RTT applied   | entered when
PARK         | RTT_PARK      | no rank selected (park policy)
RD_NOM       | RTT_NOM       | a read granted to another rank (this rank terminates)
RD_SELF      | ODT off       | a read granted to this rank (its own ODT stays off)
WR           | RTT_WR        | a write granted to this rank
(per-rank; values are the MR1/MR2/MR5 images, CSR-programmed)

RD_NOM means a read granted to another rank makes this rank present RTT_NOM. RD_SELF means a read granted to this rank keeps its own ODT off. WR means a write granted to a rank makes that rank present RTT_WR. A single-rank design point exercises PARK and WR (RD_SELF is the read case at one rank); the multi-rank coupling is already in the structure and is not special-cased away.

### 12.4.2	Command-stream tap

odt_ctrl consumes the scheduler’s grant stream: which rank won, and whether it won with a read or a write. That is all the block needs. It does not arbitrate, it does not back-pressure, and it does not modify the command stream. The HAS requires this explicitly (HAS Ch 3.5, requirement 3): ODT transitions never stall commands.

### 12.4.3	Pin timing

ODT assertion and de-assertion are delayed from the command by the ODTL family. ODTLon governs how many clocks after a write command the ODT pin asserts; ODTLoff governs how many clocks after the burst the pin releases. The write-to-read ODT turnaround is the gap when switching from a write termination picture to a read termination picture, and tADC is the command-to- ODT-change interval used for rank-to-rank transitions. All four are runtime CSRs, named by their JEDEC symbols, initialised from the JESD79-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1).

A small counter per rank loads the relevant latency when the grant decodes to a state change and counts down to the pin edge. The counter can be shared between assertion and release because the block never has overlapping state machines on the same rank in this policy.

### 12.4.4	The init seam

Until init_done, the ODT pin belongs to init_sequencer. The init sequence programs MR1/MR2/MR5 and holds ODT per the JEDEC initialization constraint; after the final mode-register writes it waits at least tMOD before raising init_done. At that cycle ownership transfers to odt_ctrl: the block samples init_odt as its starting pin value and from then on drives the pin from its own policy state. The handoff is a named, bounded seam.

### 12.4.5	LPDDR4 delta

LPDDR4 has no ODT pin. DQ ODT is programmed in the LPDDR4 mode-register set at init and held static this edition — it lives in mode_register’s LPDDR4 maps and in the init sequence, not in odt_ctrl. The dynamic machinery above is DDR4-scoped; the block’s structure keeps that scoping explicit rather than pretending a pin LPDDR4 doesn’t have.

### 12.4.6	Telemetry

odt_ctrl records state history and transition counts so a termination policy sweep is measurable (HAS Table 3.7). The telemetry exposes:

- a per-rank rolling history of the policy state (IDLE / RD / WR)

- counters for IDLE→RD, IDLE→WR, RD→IDLE, WR→IDLE, and RD↔WR transitions

- a sticky “PARK never left” indicator for the single-rank case

## 12.5	FSM policy

The block permits either a small policy FSM per rank or a counter-plus-compare implementation. The states are IDLE, RD, and WR as shown in the policy table. A pure combinational decode of grant_cmd is not sufficient: the ODTL delays need a time reference, so every output change is launched from a loaded counter, not from the decode alone. In practice this means the FSM’s next- state logic feeds a counter load, and the pin flips on counter expiration.

## 12.6	Timing

- ODTLon: runtime CSR, assertion delay from write command to ODT active.

- ODTLoff: runtime CSR, de-assertion delay after burst end to ODT inactive.

- Write-to-read ODT turnaround: runtime CSR, gap between write and read ODT pictures.

- tADC: runtime CSR, command-to-ODT-change interval for transitions.

- tMOD: enforced at the init seam by init_sequencer before init_done rises.

- All numeric values are runtime CSRs, initialised from the JESD79-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1).

## 12.7	Notes

- The single-rank design point exercises IDLE and WR only; RD is reachable in multi-rank builds but the structure is the same.

- Rank coupling is built in, not bolted on: the “other rank terminates” rule is just the per-rank state table evaluated for every rank each cycle.

- No command ever waits for odt_ctrl. If a latency cannot be met because the scheduler changed its mind, the block follows the most recent grant; the scheduler’s timing rules must already guarantee that the DRAM can absorb the command.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 13	Training Interfaces (andesite_wrlvl_ifc, andesite_rdlvl_ifc, andesite_ca_train_ifc)

Module: andesite_wrlvl_ifc (MODIFIED, carried and landed), andesite_rdlvl_ifc (NEW, landed), andesite_ca_train_ifc (NEW, landed) Location: projects/components/mem-ctrl-ip/andesite-ddr4-lpddr4/rtl/fub/ Category: training control / PHY handshake Parent: andesite_training_layer / andesite_top Status: all three landed; the tables below name the landed port lists

## 13.1	Purpose

These three blocks expose DRAM training as controller-side sequencing engines. They handle the DFI handshake, the mode-register or MPC path, the JEDEC timing windows, and the result capture. The actual delay-line search lives in firmware — the controller provides the interface, not the algorithm. This page collects the family because all three share the same handshake discipline, the same telemetry framing, and the same no-search rule.

### 13.1.1	The three family rules

- No search loop in hardware. Anywhere. The blocks handshake, sequence, window, and report; firmware walks the delays.

- Four-state telemetry. The host must distinguish never attempted, converged, timed out, and no result in window. A detector that has never fired is not evidence of anything — that’s the pumice CRC lesson, and this family applies it to training.

- CSR-defined timeouts with distinct status. Where a JEDEC window’s maximum is controller-dependent, the CSR defines it with a distinct status bit. A host that waits forever for a result it will never get looks the same as a broken link.

## 13.2	Parameters

Table 2.9.1: Training-interface parameters

| Parameter | Type | Range | Default | Meaning | PRD |
| --- | --- | --- | --- | --- | --- |
| NUM_CS | int | 1..8 | 1 | number of independent chip selects (ranks or channels) | B1 |
| CSW | int | — | $clog2(NUM_CS) (min 1) | chip-select index width (cs_sel_i) | B1 |

The JEDEC timing windows arrive as runtime-CSR ports on the landed wrlvl_ifc — t_wldqsen_i, t_wlmrd_i, t_wlo_i, t_wloe_i (write leveling, B2) — initialised from the JESD79-4 or JESD209-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1). The per-interface timeout is t_wlmrd_max_i (B5). The rdlvl/CA-training windows (B3, B4) land with those interfaces.

## 13.3	Interface

### 13.3.1	DFI training signal groups

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_phylvl_req_cs_n_o | out | NUM_CS | controller requests per-CS write leveling (DFI leveling handshake, active-low per CS) |
| dfi_phylvl_ack_cs_n_i | in | NUM_CS | PHY acknowledges the training request per CS |
| dfi_phy_wrlvl_cs_n_o | out | NUM_CS | write-leveling chip-select to the PHY |
| dfi_wrlvl_strobe_o | out | 1 | DQS strobe the PHY forwards for the prime-DQ sample |
| dfi_phy_rdlvl_cs_n_o | out | NUM_CS | read-leveling chip-select, driven in rdlvl_ifc’s HANDSHAKE |
| dfi_phylvl_req_cs_n_i | in | NUM_CS | PHY read-leveling request per CS (rdlvl_ifc handshake input) |

The controller initiates the handshake — the host owns the delay sweep (design decision D2), so there is no PHY-driven request pin on wrlvl_ifc; rdlvl_ifc consumes the PHY’s request and raises its own ack (dfi_phylvl_ack_cs_n_o is shared by both interfaces’ handshakes on the DFI bus, one training kind at a time).

Table 2.9.2: DFI 4.0 training signals

### 13.3.2	CSR / host side

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wrlvl_en_i | in | 1 | enter write-leveling mode (the MR1[7] path) |
| strobe_i | in | 1 | host strobe — one DQS edge per pulse; the host owns the delay sweep |
| cs_sel_i | in | CSW | chip select under training |
| t_wldqsen_i | in | 16 | tWLDQSEN window |
| t_wlmrd_i | in | 16 | tWLMRD window |
| t_wlmrd_max_i | in | 16 | tWLMRD timeout (0 = no timeout — the distinct timeout status) |
| t_wlo_i | in | 16 | tWLO window |
| t_wloe_i | in | 16 | tWLOE window |
| prime_dq_i | in | 1 | sampled prime DQ returned by the PHY |
| result_valid_o | out | 1 | a sample completed |
| result_o | out | 1 | captured prime-DQ value |
| obs_attempts_o | out | 16 | enable/strobe count — how many times the interface ran |
| obs_flips_o | out | 16 | result-bit transitions observed across the sweep |
| obs_timeout_o | out | 1 | t_wlmrd_max_i expired (distinct status) |
| obs_ever_done_o | out | 1 | converged at least once |
| obs_state_o | out | 3 | FSM-state observability |
| rdlvl_en_i | in | 1 | start one read-leveling sweep (a delay setting) |
| csr_mr3_mpr_enter_i | in | 16 | MR3 image with MPR enable + pattern select (the RTL makes no MPR-bit-position claim — HAS Q1); csr_mr3_mpr_exit_i is the disable image |
| t_mpr_enter_i | in | 16 | MRW ack to valid training pattern; t_mpr_exit_i / t_mpr_readout_i are the exit and capture windows |
| t_rdlvl_timeout_i | in | 16 | read-leveling handshake bound (0 = none) |
| mpr_pattern_i | in | 1 | observed DQ, latched per chip select in CAPTURE |
| cmd_req_o | out | 1 | MRW/MPC request path (shared shape with the init sequencer) |
| cmd_ack_i | in | 1 | formatter/scheduler grant for the request |
| cmd_op_o | out | 5 | OP_MRS (rdlvl_ifc) / OP_MPC (ca_train_ifc) |
| cmd_bank_o | out | 3 | MR index (3 = MR3 on the rdlvl_ifc MRW path) |
| cmd_addr_o | out | 16 | the MR3 image on the MRW path |
| ca_train_en_i | in | 1 | start one CA-training round; wdq_cal_en_i starts one WDQ-calibration round |
| csr_mpc_ca_enter_i | in | 6 | MPC opcode image for CA-training entry (CSR; encodings TBC(JESD209-4) — driven, never decoded); csr_mpc_ca_exit_i, csr_mpc_wdq_enter_i, csr_mpc_wdq_exit_i are the sibling images |
| t_ca_train_i | in | 16 | CA sample window; t_wdq_cal_i is the WDQ window; t_ca_timeout_i bounds the MPC handshake |
| ca_sample_i | in | 1 | CA observation during SAMPLE; wdq_sample_i is the WDQ observation |
| mpc_op_o | out | 6 | the opcode image presented on the MPC issue path |
| chan_sel_i | in | 1 | LPDDR4 x16 channel select (channels train independently) |

The telemetry discipline is the landed wrlvl_*/obs_* shape above, one counter set per interface.

Table 2.9.3: Host/CSR training interface

## 13.4	Microarchitecture internals

### 13.4.1	wrlvl_ifc: write leveling, MODIFIED

The contract is inherited from scoria unchanged. scoria’s HAS Ch 3.3 / its book owns the original contract: DFI leveling handshake, MR1 path in and out of write-leveling mode, the tWLMRD, tWLDQSEN, tWLO, and tWLOE windows as runtime CSRs, prime-DQ result capture, and the four-state telemetry. The marking is MODIFIED only because the shared training machinery — handshake discipline, window enforcement, and telemetry framing — is factored so all three interfaces ride it. What wrlvl_ifc owes the host is exactly what it owed before.

### 13.4.2	rdlvl_ifc: MPR read leveling, NEW

DDR4’s fly-by topology skews DQS against CK on the read side too. MR3’s MPR (multi-purpose register) makes the DRAM drive a known training pattern onto DQ, so the controller can observe its read capture timing without the memory array in the loop. The sequence is:

MRW(MR3, MPR enable + pattern select)
  -> [DFI read-leveling handshake per DFI v4.0]
  -> drive MPR pattern
  -> host reads captured values
  -> MRW(MPR disable)

The DFI read-leveling handshake uses dfi_phylvl_req_cs_n, dfi_phylvl_ack_cs_n, and dfi_phy_rdlvl_cs_n, with dfi_lvl_pattern carrying the training pattern, per DFI v4.0 §TBC(TASK-005). The MPR readout interval and its pre/post-pattern windows are runtime CSRs, named per JESD79-4, initialised from the speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1). Result capture is per chip select.

### 13.4.3	ca_train_ifc: LPDDR4 CA/WDQ training, NEW

LPDDR4’s CA bus needs training DDR4 doesn’t: LPDDR4 samples CA against CK, and both edges matter. The write-DQ path also needs its own calibration step. JESD209-4 carries both through MPC training opcodes, which is why this block sits beside zq_ctrl’s MPC submodule and reuses the formatter’s LPDDR4 CA path. The two flows are:

enter CA training (MPC)
  -> sample CA against CK
  -> host reads sample
  -> adjust
  -> exit CA training (MPC)

and

enter WDQ calibration (MPC)
  -> drive/training pattern on WDQ
  -> host reads captured value
  -> adjust
  -> exit WDQ calibration (MPC)

State is per channel because LPDDR4’s two x16 channels train independently. The training intervals are runtime CSRs, initialised from the JESD209-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1). Whether CA training is mandatory before operation is board-dependent; the init chapter records it as a conditional step, but the interface exists either way.

### 13.4.4	Four-state telemetry

Every interface reports the same 2-bit status, plus three counters:

status encoding:
  2'b00 = never attempted
  2'b01 = converged
  2'b10 = timed out
  2'b11 = no result in window

counters (per interface):
  attempts  : how many times enable was asserted
  results   : how many times a valid capture completed
  timeouts  : how many times the timeout CSR expired before convergence

A detector that has never fired is not evidence of anything. The “never attempted” state exists so firmware can tell the difference between “I haven’t tried” and “I tried and failed”.

## 13.5	FSM policy

Each interface keeps a minimal handshake FSM. For rdlvl_ifc the states are:

IDLE -> MPR_ENTRY -> HANDSHAKE -> CAPTURE -> MPR_EXIT -> REPORT -> IDLE

- IDLE: waiting for rdlvl_enable.

- MPR_ENTRY: issue MRW(MR3, MPR enable + pattern select) and wait the MPR entry interval.

- HANDSHAKE: drive dfi_phylvl_ack_cs_n and dfi_phy_rdlvl_cs_n until the PHY acknowledges, per DFI v4.0 §TBC(TASK-005).

- CAPTURE: latch the observed MPR pattern per chip select.

- MPR_EXIT: issue MRW(MR3, MPR disable) and wait the MPR exit interval.

- REPORT: update the four-state telemetry and counters, then return to IDLE.

There are no search states. The host walks the delay line by asserting enable again with a new delay setting. The same shape applies to wrlvl_ifc and ca_train_ifc, with the command path switched from MR3 to MR1 and from MPR to MPC respectively.

## 13.6	Timing

- tWLMRD: runtime CSR, MR1-write to DQS assertion for write leveling.

- tWLDQSEN: runtime CSR, DQS enable window for write leveling.

- tWLO: runtime CSR, DQS-to-ODT window observed during write leveling.

- tWLOE: runtime CSR, DQS enable-to-DQ window observed during write leveling.

- MPR readout interval: runtime CSR, MR3 entry to valid training pattern.

- MPR pre/post-pattern intervals: runtime CSRs, margins around the MPR drive.

- CA/WDQ training intervals: runtime CSRs per JESD209-4.

- tMOD: runtime CSR, wait after any MRW before the next valid command; used at MPR entry/exit and at write-leveling MR1 entry/exit.

- All numeric values are runtime CSRs, initialised from the JESD79-4 or JESD209-4 speed bin at CSR-derivation time (HAS Ch 5; numeric constants are HAS open question Q1).

## 13.7	Notes

- wrlvl_ifc is MODIFIED only by factoring; its visible contract is the same as scoria’s.

- rdlvl_ifc never decides a delay. It only exposes the MPR pattern so firmware can decide.

- ca_train_ifc is LPDDR4-only in practice, but the structure is generic; the MPC issuer is reused with zq_ctrl.

- The three FUBs instantiate in andesite_training_layer, which owns the DFI training pins, the one-active mux, and the maintenance-class command channel (trn_cmd_*) into the scheduler. init_sequencer adds training-entry states after ZQ, mode_register carries MR3 MPR fields and LPDDR4 training MRs, and dfi_cmd_formatter’s LPDDR4 CA submodule issues the MPC opcodes. No datapath block changes for training.

RTL Design Sherpa · Learning Hardware Design Through Practice  Documentation Index · MIT License

# 14	DFI Datapath (andesite_dfi_cmd_path, andesite_dfi_rd_aligner, andesite_dfi_wr_serializer, andesite_dfi_layer)

Module: andesite_dfi_cmd_path, andesite_dfi_rd_aligner, andesite_dfi_wr_serializer, andesite_dfi_layer Location: projects/components/mem-ctrl-ip/andesite-ddr4-lpddr4/rtl/ Category: DFI 4.0 boundary / datapath Parent: andesite_top / PHY interface Status: implemented (TASK-016 t6)

## 14.1	Purpose

This page covers four modules because the DFI 4.0 delta touches them as one layer. The command path widens for DDR4’s extra pins; the read aligner and write serializer grow DBI handling; and the DFI layer presents the new control surface. The CDC (dfi_cdc) and dfi_signal_pack stay inherited and dormant per the HAS, so they don’t get their own MAS page.

## 14.2	Parameters

Table 2.10.1: DFI datapath parameters

| Parameter | Type | Range | Default | Meaning | PRD |
| --- | --- | --- | --- | --- | --- |
| NUM_RANKS | int | 1..4 | 1 | rank count for chip-select fanout | C3 |
| NUM_BANKS | int | — | 8 | bank count | — |
| NUM_BG | int | — | 4 | bank-group count | — |
| ROW_WIDTH | int | — | 14 | row address width | — |
| COL_WIDTH | int | — | 10 | column address width | — |
| ADDR_WIDTH | int | — | 18 | DFI address bus width | — |
| DFI_RATE | int | 1,2,4 | 2 | DFI frequency ratio | — |
| DRAM_BEAT_WIDTH | int | PHY-defined | 64 | DRAM beat DQ width | C1 |
| DFI_DATA_WIDTH | int | DRAM_BEAT_WIDTH * DFI_RATE | 128 | DQ width at the DFI boundary | C1 |
| CMD_FIFO_DEPTH | int | power of 2 | 8 | command CDC FIFO depth | — |
| WD_FIFO_DEPTH | int | power of 2 | 16 | write-data CDC FIFO depth | — |
| RD_FIFO_DEPTH | int | power of 2 | 32 | read-data CDC FIFO depth | — |
| RD_MAX_OUTSTANDING | int | — | 16 | read-aligner outstanding-read depth | — |
| RD_EN_CYC | int | — | 4 | dfi_rddata_en window width (DFI cycles) | — |
| BL_WORDS | int | — | RD_EN_CYC | DFI words captured per read | — |

## 14.3	Interface

The full pin-level inventory of the DFI 4.0 boundary lives in ch03_interfaces/01_dfi40_pins.md; this page describes the modules behind the signals, not the signal list itself. The cross-reference is intentional — that page owns the table so this one doesn’t duplicate it.

Table 2.10.2: DFI datapath signal groups

| Signal group | Source block | DFI 4.0 clause suffix |
| --- | --- | --- |
| Command pins (dfi_act_n*, dfi_bg*, dfi_parity_in*, etc.) | dfi_cmd_path / dfi_cmd_formatter | §TBC(TASK-005) |
| Read data + DBI | dfi_rd_aligner | §TBC(TASK-005) |
| Write data + DBI | dfi_wr_serializer | §TBC(TASK-005) |
| Training handshakes | Owned by andesite_training_layer, not routed here | §TBC(TASK-005) |

## 14.4	Microarchitecture internals

### 14.4.1	dfi_cmd_path: widened command bus

dfi_cmd_path inherits scoria’s registered pipeline and widens the command word to carry the DDR4 additions:

inherited: dfi_cs_n, dfi_ras_n, dfi_cas_n, dfi_we_n, dfi_address, dfi_bank
added:     dfi_act_n, dfi_bg[1:0], dfi_parity_in

The added pins are driven from dfi_cmd_formatter: dfi_act_n is the fifth command pin, dfi_bg[1:0] carry the bank-group decode from addr_mapper, and dfi_parity_in carries the CA parity bit. For LPDDR4 these pins leave over the formatter’s LPDDR4 CA submodule instead; dfi_cmd_path simply passes the wider container through.

### 14.4.2	dfi_rd_aligner: DBI on the read path

The read aligner presents dfi_rddata_dbi alongside the read data. DBI is enabled per-byte from the MR5 read-DBI image: when enabled, the DRAM inverts bytes whose DBI bit is set at send time, and the receiver (PHY/controller) restores them. The aligner reports the inversion mask to the upper layers aligned to the same beat as the data. Read DBI is MR5-programmed and independent of write DBI.

per-byte behavior:
  if MR5[read DBI enable] == 1:
     dfi_rddata_dbi[n] reports whether byte n was inverted
  else:
     dfi_rddata_dbi[n] is zero / don't-care

### 14.4.3	dfi_wr_serializer: DBI on the write path

The write serializer applies dfi_wrdata_dbi to the write data before it reaches the PHY. When MR5 write-DBI is enabled, each byte whose DBI bit is set is inverted on the way out; when disabled, the DBI vector is ignored. Write CRC is explicitly out of scope this edition (HAS Ch 3.1; inert in MR2) — the serializer carries no CRC lane, no CRC state machine, and no stub. It is named here so nobody wires a placeholder.

per-byte behavior:
  if MR5[write DBI enable] == 1:
     byte_out[n] = dfi_wrdata[n] ^ dfi_wrdata_dbi[n]
  else:
     byte_out[n] = dfi_wrdata[n]

### 14.4.4	dfi_layer: the 4.0 control surface

andesite_dfi_layer is a structural assembly in rtl/macro/:

andesite_dfi_cdc        : ctl_clk <-> dfi_clk async FIFOs (cmd, wrdata, rddata)
andesite_dfi_cmd_path   : widened command word -> DFI 4.0 pin vector + fire strobes
andesite_dfi_wr_serializer : drive dfi_wrdata/en/mask at t_phy_wrlat
andesite_dfi_rd_aligner    : capture dfi_rddata and push into the read CDC FIFO

andesite_dfi_signal_pack stays dormant per the HAS and is not instantiated. Training pins (write-leveling, read-leveling, CA/WDQ training) live on andesite_training_layer, which owns the DFI training pins and their PHY clock-domain crossing — they are deliberately not carried through the DFI datapath layer.

The command word is the widened scheduler container {ap,col,row,bg,bank,rank,op}; the CDC command FIFO width is parameterized so the layer passes the full word without narrowing.

The single-rank design point drives the DFI v3.1 CS-qualified data lanes dfi_wrdata_cs_o and dfi_rddata_cs_o to constant zero, matching scoria’s v3.1 precedent. Only dfi_cs_o (from dfi_cmd_path) is active.

dfi_cke_o is the registered cke_i input, the real CKE pin generated by the P1 init sequencer. The P1 formatter’s own dfi_cke output is a DDR4 placeholder and is intentionally not forwarded.

dfi_init_start_o is the CDC’s registered copy of init_busy_i. There is no dfi_error / dfi_error_info at this layer: scoria never had them here, and they are not added in andesite (the MAS claim is reconciled here).

### 14.4.5	LPDDR4 delta

The datapath structure is memory-type neutral. Only the command pin mapping changes: for LPDDR4, the command word leaves through the formatter’s 6-bit CA submodule as a double-data-rate CA bus, two cycles per command. The read and write data paths are unchanged. LPDDR4’s two x16 channels are a build parameter the datapath doesn’t care about — per-channel state lives in ca_train_ifc and the refresh path, not here.

## 14.5	FSM policy

There are no FSMs in this layer. dfi_cmd_path, dfi_rd_aligner, and dfi_wr_serializer are registered pipelines and muxing, inherited shape. The DBI enable qualification is combinational on the MR5 image. dfi_layer itself is a routing and registration shell; the only “state” is the CDC FIFOs in the inherited dfi_cdc, and those keep scoria’s elastic-buffer policy.

## 14.6	Timing

- The command path is a registered pipeline; latency is one DFI clock per stage, matching scoria’s verified shape.

- The read aligner and write serializer add no FSM latency beyond the inherited pipeline; DBI inversion is combinational on the MR5 image.

- Gear-down entry timing is owned by init_sequencer; dfi_layer only presents the handshake pins, per DFI 4.0 §TBC(TASK-005).

- All DFI 4.0 clause references are suffixed §TBC(TASK-005) because the specification is acquired and studied under andesite TASK-005.

## 14.7	Notes

- The DFI 4.0 _pN phase notation is a generalization, not a new signal. The 1:4 frequency ratio remains a defined mode of the newer revision, per DFI 4.0 §TBC(TASK-005).

- Write CRC is out of scope this edition. The datapath has no CRC lane and no stub; if the condition in HAS Ch 3.1 changes, the change starts with the MR2 image and a new datapath specification.

- The full pin table belongs to ch03_interfaces/01_dfi40_pins.md; this page intentionally cross-references rather than duplicates it.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	DFI 4.0 Pin-Level Table

This page pins the boundary the HAS’s Chapter 4 names: every signal HAS Table 4.1 lists appears here as a pin-level row — name, direction, clock domain, reset value, and the source block that drives or consumes it. The HAS carries the argument; this table is what the DV framework’s DFI signal catalog checks against when the BFM exists (andesite TASK-005).

Citation discipline, restated from the HAS because it matters more here: the DFI 4.0 specification is on disk (/mnt/data/github/dfi-specs/DDR_PHY_Interface_Specification_v4_0.pdf) and was studied on 2026-10-04. Clause references below are real DFI 4.0 section numbers; any remaining §TBC(TASK-005) marks a claim the study could not confirm. The “behavior” column states what andesite does, not what a clause number says.

Reset convention for the whole table: DFI outputs reset to the de-asserted, command-inactive level (chip selects high, command pins high, data enables low); DFI inputs are qualified by their valid/enable companions and need no reset value. Per-signal reset conventions of DFI 4.0 itself are listed in Table 3, “DFI Signal Requirements” (§2.2.3 DFI Signals).

## 15.1	Command and address pins

Table 3.1: DFI 4.0 command and address pins

| Pin | Dir | Clock | Reset | Source block | Behavior | Citation |
| --- | --- | --- | --- | --- | --- | --- |
| dfi_cs | out | DFI ctrl | 1 | cmd_formatter | chip select; rank select from rank_i (v4.0 rename of dfi_cs_n) | §3.1 Control Interface |
| dfi_act_n | out | DFI ctrl | 1 | cmd_formatter | activate select; 0 marks the ACT form | §3.1 Control Interface |
| dfi_ras_n | out | DFI ctrl | 1 | cmd_formatter | command pin; A16 during non-ACT commands | §3.1 Control Interface |
| dfi_cas_n | out | DFI ctrl | 1 | cmd_formatter | command pin; A15 during non-ACT commands | §3.1 Control Interface |
| dfi_we_n | out | DFI ctrl | 1 | cmd_formatter | command pin; A14 during non-ACT commands | §3.1 Control Interface |
| dfi_bank | out | DFI ctrl | 0 | cmd_formatter | bank address (DDR4: BA[1:0]; LPDDR4 holds idle) | §3.1 Control Interface |
| dfi_bg | out | DFI ctrl | 0 | cmd_formatter | bank-group address BG[1:0], DDR4 only | §3.1 Control Interface |
| dfi_address | out | DFI ctrl | 0 | cmd_formatter | address bus; row on ACT, column on RD/WR, MR data on MRS | §3.1 Control Interface |
| dfi_cke | out | DFI ctrl | 0 | cmd_formatter | clock enable; ownership held by init until READY (HAS Ch 3.2 seam) | §3.1 Control Interface |
| dfi_reset_n | out | DFI ctrl | 0 | init_sequencer | DRAM RESET#, driven through the init window; presented under its DFI name at the boundary (scoria convention, HAS Ch 4.1) | §3.1 Control Interface |
| dfi_parity_in | out | DFI ctrl | 0 | cmd_formatter | CA parity generation point; qualified by parity_en_i | §3.5.7 CRC and CA Parity |
| dfi_alert_n | in | DFI ctrl | — | NEW alert handling at the boundary | parity/training alert return from PHY; logged by the formatter, recovery owned by the sequencer (HAS Ch 3.2) | §3.5.7 CRC and CA Parity |
| ca_o[5:0] | out | DFI ctrl (DDR) | 0 | cmd_formatter LPDDR4 CA submodule | LPDDR4 CA bus, double-data-rate, two cycles per command | §3.14 Channels for LPDDR4 memories |
| ca_valid_o | out | DFI ctrl | 0 | cmd_formatter LPDDR4 CA submodule | marks both CA beat cycles | andesite internal qualifier |

dfi_cid (chip ID) is not driven at NUM_RANKS = 1; it is presented constant, the same disposition scoria recorded for the v3.1 data-phase chip selects (HAS Ch 4.1). A multi-rank build drives it from the granted command’s rank (§3.1 Control Interface, §4.3 3DS Stack Support).

## 15.2	Gear-down

Table 3.2: Gear-down entry

| Pin | Dir | Clock | Reset | Source block | Behavior | Citation |
| --- | --- | --- | --- | --- | --- | --- |
| dfi_geardown_en | out | DFI ctrl | idle | init_sequencer | DDR4 geardown enable; when asserted the MC changes CA signals only every other DFI PHY clock (§4.18) | §3.13 Geardown Mode |

## 15.3	Training and leveling pins

Table 3.3: Training and leveling pins

| Pin | Dir | Clock | Reset | Source block | Behavior | Citation |
| --- | --- | --- | --- | --- | --- | --- |
| dfi_phy_wrlvl_cs | out | DFI ctrl | 1 | training_layer | selects write leveling (v4.0 rename of dfi_phy_wrlvl_cs_n) | §3.6 DFI Training Interface |
| dfi_phy_rdlvl_cs | out | DFI ctrl | 1 | training_layer | selects read leveling; unused by scoria, driven by rdlvl_ifc (v4.0 rename of dfi_phy_rdlvl_cs_n) | §3.6.1 Read Training Operation |
| dfi_phy_rdlvl_gate_cs | out | DFI ctrl | 1 | training_layer | read-gate leveling select; presented, exercised per BFM capability (HAS Q3) (v4.0 rename of dfi_phy_rdlvl_gate_cs_n) | §3.6.1 Read Training Operation |
| dfi_phy_calvl_cs | out | DFI ctrl | 1 | training_layer | selects CA training; driven by ca_train_ifc (v4.0 rename of dfi_phy_calvl_cs_n) | §3.6.4 CA Training Operation |
| dfi_lvl_pattern | out | DFI ctrl | 0 | training_layer | training pattern select | §3.6.6 dfi_lvl_pattern Encoding |
| dfi_lvl_periodic | out | DFI ctrl | 0 | training_layer | periodic-training control | §3.6.7 Periodic Training Flag |
| dfi_calvl_en | out | DFI ctrl | 0 | training_layer | MC enables CA training logic | §3.6.4 CA Training Operation |
| dfi_calvl_req | in | DFI ctrl | — | training_layer | PHY requests CA training | §3.6.4 CA Training Operation |
| dfi_calvl_capture | out | DFI ctrl | 0 | training_layer | CA-training capture strobe from MC to PHY | §3.6.4 CA Training Operation |
| dfi_calvl_resp | in | DFI ctrl | — | training_layer | CA-training response from PHY | §3.6.4 CA Training Operation |
| dfi_calvl_ca_sel | out | DFI ctrl | 0 | training_layer | CA foreground pattern strobe | §3.6.4 CA Training Operation |
| dfi_calvl_data | out | DFI data phases | 0 | training_layer | LPDDR4 CA VREF training data (per slice) | §3.6.4 CA Training Operation |
| dfi_calvl_done | out | DFI data phases | 0 | training_layer | LPDDR4 CA VREF training done | §3.6.4 CA Training Operation |
| dfi_calvl_result | in | DFI data phases | — | training_layer | LPDDR4 CA VREF training result from PHY | §3.6.4 CA Training Operation |
| dfi_calvl_strobe | out | DFI data phases | 0 | training_layer | LPDDR4 CA VREF training strobe | §3.6.4 CA Training Operation |

## 15.4	Data-path pins (DBI and the inherited surface)

Table 3.4: Data-path and inherited pins

| Pin | Dir | Clock | Reset | Source block | Behavior | Citation |
| --- | --- | --- | --- | --- | --- | --- |
| dfi_wrdata / dfi_wrdata_mask | out | DFI data phases | 0 | dfi_datapath | write data and mask/DBI; dfi_wrdata_mask becomes write DBI when DBI is enabled and phydbi_mode = 0; phase-multiplied _pN, inherited | §3.2.1 Write Data Mask/Write DBI |
| dfi_wrdata_en / dfi_wrdata_cs | out | DFI data phases | 0 | dfi_datapath | write data enable; per-phase chip select driven constant at one rank (scoria convention; v4.0 rename of dfi_wrdata_cs_n) | §3.2.2 Write Data Chip Select |
| dfi_rddata / dfi_rddata_valid | in | DFI data phases | — | dfi_datapath | read data and valid; inherited capture | §3.3.5 Read Data Signals and Parameters |
| dfi_rddata_dbi | in | DFI data phases | — | dfi_datapath | read DBI, per byte; presented alongside dfi_rddata | §3.3.1 Read DBI |
| dfi_rddata_cs | out | DFI data phases | 1 | dfi_datapath | per-data-phase chip select, constant at one rank (inherited; v4.0 rename of dfi_rddata_cs_n) | §3.3.2 Read Data Chip Select |
| dfi_error / dfi_error_info | in | DFI ctrl | — | boundary (alert handling) | error indication and info; scoria’s surface, DDR4 CA-parity detail now used (HAS Ch 4.1) | §3.8 Error Interface |
| dfi_init_start | out | DFI ctrl | active | init_sequencer via the DFI layer | MC init/frequency-change request; generated at the DFI layer from the P1 init FSM’s busy/done (the P1 sequencer is self-timed and emits no PHY handshake of its own) | §3.5.1 Initialization |
| dfi_init_complete | in | DFI ctrl | — | boundary | PHY init complete / frequency-change acknowledge; driven by the PHY, not consumed at this design point (self-timed init) | §3.5.1 Initialization |
| dfi_lp_ctrl_req / dfi_lp_data_req | out | DFI ctrl | 0 | dormant (powerdown_ctrl pair) | exposed per the timeout-and-report rule; nothing consumes them at this design point (HAS Ch 3.1) | §3.7 Low Power Control Interface |

## 15.5	Study corrections (2026-10-04, andesite TASK-005)

- Active-low chip-select names are v3.1, not v4.0. DFI 4.0 renames dfi_cs_n → dfi_cs, dfi_wrdata_cs_n → dfi_wrdata_cs, dfi_rddata_cs_n → dfi_rddata_cs, and all training chip selects from dfi_phy_*_cs_n to dfi_phy_*_cs (§3.1, §3.2.2, §3.3.2, §3.6). The original table used v3.1 names; this edition uses v4.0 names.

- dfi_phylvl_req_cs_n / dfi_phylvl_ack_cs_n are not DFI 4.0 signals. They were defined in v3.1 as PHY-requested training handshakes and are superseded in v4.0 by PHY Independent Mode (§3.6.8). They have been removed from this v4.0 pin table.

- dfi_wrdata_dbi does not exist. Write DBI reuses dfi_wrdata_mask when DBI is enabled and phydbi_mode = 0 (§3.2.1). The separate dfi_wrdata_dbi pin has been removed.

- dfi_ca_capture was wrong in name and direction. The DFI 4.0 signal is dfi_calvl_capture, it is MC-to-PHY (out), and the PHY-to-MC observation returns are dfi_calvl_resp and dfi_calvl_result (§3.6.4).

- dfi_init is not a DFI signal name. The spec defines dfi_init_start (MC-to-PHY) and dfi_init_complete (PHY-to-MC) (§3.5.1). Both are now listed separately.

## 15.6	What this table does not do

It does not restate the inherited datapath’s phase structure (the _pN buses and the 1:4 ratio are argued in HAS Ch 4.1 and owned by dfi_datapath). DFI 4.0 clause numbers are now cited above; any remaining §TBC(TASK-005) marks a claim the study could not confirm.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Core Signal Contracts

## 16.1	What a contract is here

A signal contract in this repo is a machine-checkable statement of what a signal is allowed to do: a term list with its defining file:line citation, the invariants between terms, and a decision table whose forbidden rows are marked ILLEGAL. The canonical methodology is vault/handbook/design/signal-contracts-and-kmaps.md; the shared generator machinery is bin/kmaps/, and the andesite kmap book (../kmaps/, andesite TASK-004) builds its contract sheets from the anchors this chapter names.

## 16.2	Pre-RTL citation posture

At MAS v0.1 there is no RTL, so every contract below cites the MAS ch02 page where the intended expression is written verbatim — the fenced anchors. When the first RTL lands, each citation is re-pointed at the corresponding .sv file and line, and the generator’s citation gate then enforces that the workbook and the RTL agree; a drift fails the run. The anchor map:

Table 4.1: Anchor map — the kmap generator cites these file:line pairs

| Contract domain | Defining anchors (file:line, at v0.1) |
| --- | --- |
| DDR4 command decode | 01_cmd_formatter.md — truth table fence |
| Activate field packing | 01_cmd_formatter.md — activate form fence |
| LPDDR4 CA encodings | 01_cmd_formatter.md — placeholder table, pinned by the kmap book |
| Init FSM ordering | 02_init_sequencer.md — DDR4 and LPDDR4 state-list fences |
| Address decode structure | 04_addr_mapper.md — decode fence |
| Scheduler admission | 05_scheduler.md — admission-rule fence |
| FGR interval arithmetic | 06_refresh_ctrl.md — interval fence |
| MPC calibration FSM | 07_zq_ctrl.md — MPC FSM fence |
| ODT policy states | 08_odt_ctrl.md — policy-state fence |
| Training flows and telemetry | 09_training.md — MPR/CA/WDQ flow fences, four-state encoding fence |
| DFI datapath pin set and DBI | 10_dfi_datapath.md — pin-set and DBI fences |
| LPDDR4 CA encodings | generated/02_lpddr4_ca_command_table.md — TBC JESD209-4 |
| Address decode | generated/03_addr_decode_maps.md — design-point decode maps |
| MR programming | generated/04_mr_programming_maps.md — MR0-MR6 / LPDDR4 MRW |
| ODT truth | generated/05_odt_truth_table.md — termination policy |
| FGR select | generated/06_fgr_refresh_map.md — refresh granularity |

The line numbers are captured at authoring time by the generator’s citation gate; the table above names the fences, and verify_citations pins the exact lines on every run.

## 16.3	Contract: DDR4 command decode (the formatter’s core)

Terms. ACT_n, RAS_n, CAS_n, WE_n (DFI command pins, source 01_cmd_formatter.md); CS_n (select); AP (the auto-precharge address bit, A10).

Invariants (each renders ILLEGAL rows impossible, per the truth-table anchor):

- Exactly one of the anchored encodings holds whenever CS_n = 0; the fence’s rows are exhaustive and mutually exclusive.

- ACT_n = 0 marks exactly three rows in the anchor: ACT (with RAS_n=1,    CAS_n=1, WE_n=1), MRS (RAS_n=0, CAS_n=0, WE_n=0) and REF (RAS_n=0,    CAS_n=0, WE_n=1) — no other pin combination may carry ACT_n = 0.

- AP is an address input: it never appears in the pin-level table; the variants (RDA/WRA, PREA, ZQCL) are the RD/WR/PRE/ZQ rows qualified by AP = 1.

- Parity (dfi_parity_in) is generated only when parity_en_i = 1, and parity_en_i may only be set after the init sequence has programmed parity mode (HAS Ch 3.2 dependency).

Decision table. One row per (command, AP) combination the controller issues; forbidden rows — commands outside the anchored set, or any command while parity_en_i disagrees with the MR image — are ILLEGAL. The minimized qualifier SOPs for this table are the kmap book’s DDR4 command table (andesite TASK-004 output); the workbook and this contract must always cite the same anchor lines.

## 16.4	Contract: maintenance request/grant

Terms. req/gnt between refresh_ctrl, zq_ctrl (both cores), odt_ctrl turnaround notices, and the scheduler (source 05_scheduler.md).

Invariants.

- A maintenance source raises req and waits; it never asserts its command onto the bus without gnt (family doctrine 2, quoted in full in the family docs).

- gnt is exclusive: at most one maintenance source holds the bus in any cycle.

- A refresh of either granularity and a ZQ calibration of either form are ordinary occupants of the banks they touch: the bank timers own those banks for the operation’s duration (HAS Ch 3.4 requirement 4).

Decision table. One row per (source, state) pair; rows where two sources hold gnt, or a source issues without gnt, are ILLEGAL.

## 16.5	Contract: training four-state telemetry

Terms. The 2-bit status and counters per interface (source 09_training.md).

Invariants.

- The status is exactly one of {never attempted, converged, timed out, no result in window}; a detector that has never fired is not evidence of anything, so “never attempted” must be representable and is the reset state.

- A timeout is distinguishable from a no-result: the CSR-defined timeout with its distinct status bit is the family’s rule (HAS Ch 3.3), and the two states may not share an encoding.

- No interface state performs a search: the state lists contain no delay-walk states, and the host-visible counters only count attempts, results, and timeouts.

Decision table. One row per (interface, status) pair; any row pairing a search behaviour with a legal status is ILLEGAL.

## 16.6	Regenerating the workbook

cd projects/components/mem-ctrl-ip/andesite-ddr4-lpddr4
python3 docs/kmaps/gen_andesite_kmaps.py

The generator writes docs/kmaps/andesite_cmd_kmaps.xlsx and the docs/kmaps/generated/*.md renderings. Do not hand-edit the .xlsx; it is a build artifact of the generator, and the citation gate fails the run if any file:line citation drifts from this book.

For every contract the verdict is NOT CHECKED until RTL exists — the correct pre-RTL posture, identical to the bch MAS’s. The verdict becomes meaningful once rtl_sop can be diffed against the derived minimal cover.