RTL Design Sherpa

STREAM Characterization Nexys A7 Project Guide 0.90

July 2026

Table of Contents

1 Document Information	8

1.1 References	8

1.2 Terminology	8

1.3 Revision History	9

2 Overview	9

2.1 What it characterizes	9

2.2 The flow matrix	10

2.3 Board and tools	10

2.4 Status	11

3 How It Works	11

3.1 The spine (primary flow)	11

3.1.1 Figure 2.1: STREAM Characterization Harness Spine	11

3.2 Bridge slaves and base addresses	11

3.3 Key RTL blocks	12

3.4 The instrumentation harness	13

4 Test Methodology	13

4.1 What is under test	13

4.2 What is measured, and how	14

4.2.1 Figure 3.1: Measurement Timer Windows	15

4.2.2 Figure 3.2: Response-Delay Latency Model	16

4.3 Workloads	16

4.4 The oracle	16

4.5 Measurement pitfalls	16

5 Build and Run	17

5.1 Make targets	17

5.2 Host tools (flows-stream-bridge/host/)	18

5.3 Port autodetect and identity	18

6 Harness CSR Configuration	18

6.1 Address map and by-name access	18

6.1.1 Waveform 5.1: Descriptor Load and Kick	19

6.2 harness_csr — control / status (base 0x0001_0000)	19

6.3 Timer and the latency knob	20

6.4 CRC, kick, and observability	21

6.5 STREAM monitor window (0x1000)	22

6.6 Descriptor format (desc_ram @ 0x0002_0000)	22

7 Simulation / Silicon Equivalence	22

7.1 The testbench	22

7.2 Test levels	23

7.3 Sibling flows	23

8 Troubleshooting	23

8.1 Current state	23

8.2 Known issues	24

8.3 Cannot find the board	24

8.4 Not simulatable	24

8.5 Board fit vs sim geometry	24

8.6 Which README to trust	25

List of Figures

Figure 2.1: STREAM Characterization Harness Spine	11

Figure 3.1: Measurement Timer Windows	15

Figure 3.2: Response-Delay Latency Model	16

List of Tables

Table 1: Reference documents	8

Table 2: Revision history	9

Table 3: Characterization axes and reports	9

Table 4: The flow matrix	10

Table 5: Board and tool requirements	10

Table 6: Bridge slave address map	11

Table 7: Key RTL blocks	12

Table 8: What is measured	14

Table 9: Characterization workloads	16

Table 10: flows-stream-bridge make targets	17

Table 11: harness_csr control / status	19

Table 12: Timer and memory-latency registers	20

Table 13: CRC, kick, and observability registers	21

Table 14: cocotb test levels	23

Table 15: Known issues and workarounds	24

List of Waveforms

Waveform 5.1: Descriptor Load and Kick	19

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 1	Document Information

This is the operator / developer guide for the Nexys A7 STREAM Characterization project (projects/NexysA7/stream_characterization). It explains what the project characterizes, how the harness works, how to build / simulate / program / run it, and how the harness CSR is configured.

The device under test is the in-house STREAM scatter-gather DMA (stream_top_ch8, from projects/components/dmas/stream/). This guide documents how the harness drives and measures it — not the DMA core’s internals. The STREAM core’s own architecture specifications (HAS/MAS) live under projects/components/dmas/stream/docs/.

## 1.1	References

Reference documents

| Source | Title |
| --- | --- |
| RTL Design Sherpa | README.md, PORT_MAP.md, DMA_UTILIZATION_MEASUREMENT.md |
| RTL Design Sherpa | flows-stream-bridge/host/ADDRESS_MAP.md (bridge + CSR map) |
| RTL Design Sherpa | reports/{perf,compression,area,ext_addressing}/README.md |
| RTL Design Sherpa | bin/TBClasses/harness/ (shared UART-char collateral) |
| ARM | AMBA AXI4 / AXI4-Lite / APB Protocol Specifications |

## 1.2	Terminology

Flow

One cell of the characterization matrix — a {DMA} × {bridge} pairing built as a flows-<dma>-<bridge>/ directory.

Harness CSR

The instrumentation register block (harness_csr) that arms the pattern generators, latency model, bus meters, and CRC checkers around the DUT.

MonBus

STREAM’s internal 64-bit monitoring bus; its packets are drained to debug_sram (bulk trace) and stream_err (IRQ FIFO), and optionally compressed.

Observer

axi4_dma_observer — non-perturbing valid/ready snoops on the DMA payload buses that feed the utilization buckets and the compression counters.

## 1.3	Revision History

Revision history

| Version | Date | Notes |
| --- | --- | --- |
| 0.90 | 2026-07-18 | Initial project guide |

# 2	Overview

## 2.1	What it characterizes

This project measures the in-house STREAM scatter-gather DMA on a Digilent Nexys A7-100T (Artix-7 xc7a100tcsg324-1, 100 MHz), driven entirely over one USB-UART link, across four axes:

Characterization axes and reports

| Axis | Report | Headline |
| --- | --- | --- |
| Performance / utilization | reports/perf/ | 100% datapath utilization across the 40-config matrix (~1525 MB/s, 128-bit datapath) |
| MonBus trace compression | reports/compression/ | 63.0% (64-bit codec), 75.7% (half-beat 32-bit) reduction |
| Area | reports/area/ | bare DMA OOC vs an open-source iDMA comparison |
| Extended addressing | reports/ext_addressing/ | row/row ≈ 1.5 GB/s vs col/col ≈ 0.26–0.32 GB/s |

## 2.2	The flow matrix

The project is framed as a 2×2 comparison — {STREAM in-house, Vivado MCDMA} × {in-house bridge, Vivado bridge} — so every cell is measured through the same shared instrumentation harness:

The flow matrix

| Flow | DUT | Status |
| --- | --- | --- |
| flows-stream-bridge/ | STREAM (stream_top_ch8) + in-house bridge | Primary, most complete — cocotb gate + full FPGA sweeps |
| flows-vivado-mcdma/ | Vivado AXI-MCDMA IP + in-house bridge | Skeleton (blinks LED; FPGA-only — VHDL IP can’t Verilate) |
| flows-idma-bridge/ | PULP iDMA | Area + datapath-perf cosim only (feeds the iDMA comparison) |
| flows-stream-vivado-bridge/, flows-vivado-mcdma-vivado-bridge/ | future cells | Listed in make help; not built |

This guide focuses on the primary flows-stream-bridge flow; the others are inventoried in Chapter 7.

## 2.3	Board and tools

Board and tool requirements

| Item | Detail |
| --- | --- |
| Board | Digilent Nexys A7-100T (xc7a100tcsg324-1), 100 MHz |
| Build | Vivado 2025.1 |
| Sim | Verilator + cocotb / cocotb_test / pytest |
| Host | Python 3 + pyserial over UART (115200 8N1) |

## 2.4	Status

The primary stream-bridge flow is working — cocotb gate + full FPGA sweeps are collected, with data in the perf (v1.4) and compression (v1.3) writeups (progress/STATUS = 90-DONE). flows-vivado-mcdma is a skeleton; flows-idma-bridge is area + cosim only.

# 3	How It Works

## 3.1	The spine (primary flow)

The host drives one UART link into a generated 1→N AXI4-Lite/APB fabric that fans out to the DUT config, the harness CSR, and the trace/error memories:

### 3.1.1	Figure 2.1: STREAM Characterization Harness Spine

STREAM characterization harness spine

Source: 01_harness_spine.mmd

## 3.2	Bridge slaves and base addresses

From host/ADDRESS_MAP.md and PORT_MAP.md (the current design; the older flows-stream-bridge/README.md describes a superseded 5-slave decoder — trust the address-map docs):

Bridge slave address map

| Base | Size | Slave | Width | Protocol |
| --- | --- | --- | --- | --- |
| 0x0000_0000 | 4 KB | stream_apb (STREAM config) | 32b | APB |
| 0x0001_0000 | 4 KB | harness_csr | 32b | AXIL |
| 0x0002_0000 | 64 KB | desc_ram (32 B/desc) | 256b | AXIL |
| 0x0003_0000 | 4 KB | stream_err (MonBus IRQ-FIFO drain) | 64b | AXIL |
| 0x0004_0000 | 256 KB | debug_sram (MonBus bulk trace) | 64b | AXIL |
| 0x0008_0000 | 4 KB | dma_axil (tied off, reserved MCDMA) | 32b | AXIL |
| 0x000c_0000 | 64 KB | bridge_trace_sram (bridge MonBus trace) | 64b | AXIL |

Every MonBus-carrying AXI path is 64-bit. The descriptor read path is a direct 256-bit AXI4 (m_axi_desc ↔ desc_ram, bypassing the bridge); the DMA payload masters m_axi_rd / m_axi_wr are 128-bit into the pattern-gen / CRC-check slaves.

## 3.3	Key RTL blocks

Key RTL blocks

| File | Role |
| --- | --- |
| flows-stream-bridge/rtl/stream_char_top.sv | FPGA pin-level top: instantiates the harness, led_status_driver, seven_seg_4digit. |
| flows-stream-bridge/rtl/stream_char_harness.sv | The integration hub: uart_axil_bridge + generated bridge_stream_char_axil + harness_csr + desc_ram + debug_sram + axi_response_delay ×2 + axi4_dma_slaves + axi4_dma_observer + the DUT stream_top_ch8. |
| stream_char_framework/rtl/ | Shared framework RTL: harness_csr.sv (authoritative CSR), desc_ram.sv, debug_sram.sv, axi_response_delay.sv, led_status_driver.sv, seven_seg_4digit.sv, sram_chan_tracker.sv. |

## 3.4	The instrumentation harness

Around the DUT, the harness supplies stimulus, a memory-latency model, and non-perturbing observability:

- axi4_dma_slaves — an LFSR read-source pattern generator (feeds m_axi_rd) and a write-sink CRC checker (verifies m_axi_wr).

- axi_response_delay ×2 — a programmable memory-latency model on the read and write paths (the RESP_DELAY CSR, Chapter 5).

- axi4_dma_observer — non-perturbing valid/ready bus meters plus a burst histogram and the MonBus-compression observer, all read back through the harness CSR.

The DUT’s own MonBus monitors feed debug_sram; the monbus compressor / half-beat packer (USE_MON_COMPRESSION) drives the compression characterization.

# 4	Test Methodology

This chapter describes what is exercised on the FPGA block (Figure 2.1) and how utilization, throughput, compression, and integrity are measured. The full utilization definitions are in the project’s DMA_UTILIZATION_MEASUREMENT.md; this is the condensed operator view.

## 4.1	What is under test

The STREAM scatter-gather DMA (stream_top_ch8), driven by descriptors and measured on its 128-bit read/write payload buses. A programmable axi_response_delay model sits in front of the pattern-gen/CRC-check slaves so the DUT can be exercised against a range of emulated memory latencies.

## 4.2	What is measured, and how

What is measured

| Quantity | How it is obtained |
| --- | --- |
| Datapath utilization | axi4_dma_observer valid/ready buckets OBS_{RD,WR}_{PROD,BP,STARV,IDLE} over a timed window (see the four utilization definitions in DMA_UTILIZATION_MEASUREMENT.md) |
| Throughput | beats × bytes / window cycles, using TIMER_CYCLES and the first/last beat stamps (TIMER_{R,W}_{FIRST,LAST}) |
| Data integrity | per-channel CRC_RD_PER_CH* / CRC_WR_PER_CH* vs expected, with valid/match masks |
| MonBus compression | the compression observer counters (COMP_TIER*, overflow flags) — ratio of packed vs raw records |
| Area | out-of-context synthesis of the bare stream_top_ch8 (make area) |
| Extended addressing | row/col-major throughput (requires USE_ROW_COL_MAJOR_ADDRESSING) |

A measurement run: program STREAM by name over APB, build and load descriptors into desc_ram, set the memory-latency model (RESP_DELAY), kick via KICK_GO (Waveform 5.1), let the timer window run, then read the meters, timer, and CRCs. Monitor presets (perf-mon, debug-compl, …) select which MonBus cones are active.

The timer captures the edges of the window — kick, first/last AR/R and W/B — so a run’s throughput and latency are derived from a well-defined interval:

### 4.2.1	Figure 3.1: Measurement Timer Windows

Measurement timer windows

Source: 05_timer_windows.dot

The emulated memory latency is a per-beat timestamp FIFO — the RESP_DELAY CSR sets how long each beat dwells before it is released to the DUT:

### 4.2.2	Figure 3.2: Response-Delay Latency Model

Response-delay latency model

Source: 03_response_delay.dot

## 4.3	Workloads

Characterization workloads

| Axis | Values |
| --- | --- |
| Descriptors / channel | 1, 2, 4, 8, 16 |
| Channels | 1, 2, 4, 8 |
| Transfer size | 1 MB (per the 40-config matrix) |
| Response delay | 0, 128, … aclk cycles (memory-latency sweep) |

## 4.4	The oracle

Data integrity is judged by per-channel golden CRC on the write-sink checker vs the read-source pattern generator. The same by-name config, descriptor bytes, and CRC run in the UART-equivalence sim (Chapter 6).

## 4.5	Measurement pitfalls

- Sticky RESP_DELAY — a delay sweep leaves the CSR set; re-program before a clean matrix/size sweep or a later no-delay run silently under-performs.

- After a long mixed session some configs wedge; only a full make program clears it (a reset-completeness gap, not codec/data corruption).

- 16desc_* configs trip a benign trace.overflow — it bounds the debug trace, not the bus-meter counters.

# 5	Build and Run

Every Makefile requires REPO_ROOT, so source the environment first:

cd /path/to/RTLDesignSherpa && source env_python
cd projects/NexysA7/stream_characterization

## 5.1	Make targets

The top-level Makefile is a per-flow factory — targets take a flow suffix:

make bitstream-stream-bridge     # build the primary flow
make program-stream-bridge
make sim-stream-bridge
make all-bitstreams              # every built flow
make docs                        # render the reports
make help                        # full 2x2 matrix + targets

The primary flow (flows-stream-bridge/Makefile) has the detailed targets:

flows-stream-bridge make targets

| Target | What it does |
| --- | --- |
| make bitstream | regen-bridges + verify-sim + full P&R (~10–30 min) → bitstream/stream_char.bit |
| make program | JTAG flash |
| make sim | cocotb regression (dv/tests → run-all-full-parallel) |
| make area | out-of-context bare-stream_top_ch8 area report |
| make synth / make utilization / make timing | synth-only + reports |
| make regen-bridges / make regen-regs-nomon | regenerate the bridge / register collateral |
| make verify-sim | pre-bitstream gate (-k "ping or csr_read"); bypass with BITSTREAM_SKIP_VERIFY=1 |

The dv/tests/Makefile runs the cocotb suite by level: run-all-{gate,func,full}[-parallel][-wave] (TEST_LEVEL, 48-worker xdist, --reruns 3).

cd flows-stream-bridge
make bitstream
make program
make sim

## 5.2	Host tools (flows-stream-bridge/host/)

All default --port auto (autodetect) and --baud 115200.

run_characterization.py — the main sweep runner (40-config matrix: {1,2,4,8,16} desc/ch × 1–8 ch × 1 MB):

./run_characterization.py --port /dev/ttyUSB1 --channels 1 2 4 8 --size 1MB --csv fpga_suite.csv
./run_characterization.py --resp-delays 0,128 --compression --mon-config debug-compl

--configs, --phase, --resp-delays[-wr], --rd-prefetch, -o out.json, --dry-run; env XFER_BEATS=b sets burst size.

characterize.py — a lighter one-row-per-config sweep (cycles + MB/s). stream_ext_suite.py / stream_ext_char.py — extended row/col addressing (needs the DUT built with USE_ROW_COL_MAJOR_ADDRESSING=1). per_source_capture.py — per-source MonBus isolation for compression datasets. dump_status.py — quick sanity (prints BUILD_ID 0x53545243). dump_monbus_sram.py — dump the trace ring (--base 0x00040000 STREAM, 0x000c0000 bridge). By-name access helpers: stream_addrs.A() (STREAM APB), harness_addrs.H() (harness CSR), stream_device.Stream.

Exact reproduce commands live in the reports/perf/README.md and reports/compression/README.md appendices.

## 5.3	Port autodetect and identity

--port auto probes each /dev/ttyUSB* by writing the magic 0xC0FFEE5A to the harness SCRATCH CSR and reading it back; the board that echoes is the harness. Confirm it is alive with dump_status.py — the BUILD_ID at harness CSR 0x24 reads 0x5354_5243 (“STRC”).

# 6	Harness CSR Configuration

## 6.1	Address map and by-name access

The bridge fans out to the slaves in Chapter 2. This chapter covers the harness_csr slave at HARNESS_CSR_BASE = 0x0001_0000.

No PeakRDL RDL exists for the harness CSR. harness_csr.sv (stream_char_framework/rtl/) is authoritative; a mirror table in stream_char_framework/bin/gen_harness_regmap.py generates stream_char_framework/rtl/harness_csr_regmap.py. The host accesses it by name — host/harness_addrs.py H("NAME") resolves HARNESS_CSR_BASE + offset via RegisterMap; never hardcode offsets.

The STREAM DUT’s APB config, by contrast, is PeakRDL-generated (stream_regmap.py from stream_regs.rdl), accessed by name via host/stream_addrs.py A("NAME") — see the monitor-window note below.

### 6.1.1	Waveform 5.1: Descriptor Load and Kick

Descriptor load and kick

Source: 01_desc_kick.json

## 6.2	harness_csr — control / status (base 0x0001_0000)

harness_csr control / status

| Register | Offset | Field | Bits | Access | Description |
| --- | --- | --- | --- | --- | --- |
| CTRL | 0x00 | start | [0] | W | Pulse: start a measured run |
| CTRL | 0x00 | clear_stats | [1] | W | Pulse: clear meters + sticky errors |
| CTRL | 0x00 | freeze_trace | [2] | RW | Latch: freeze counters |
| CTRL | 0x00 | soft_reset | [3] | W | Pulse: soft reset |
| CTRL | 0x00 | cam_clear | [4] | W | Pulse: clear the error CAM |
| STATUS | 0x04 | stream_irq | [0] | R | STREAM interrupt |
| STATUS | 0x04 | any_error | [1] | R | Sticky error OR |
| STATUS | 0x04 | trace_overflow | [2] | R | Trace ring wrapped |
| STATUS | 0x04 | clear_busy | [3] | R | Clear in progress |
| DBG_WR_PTR | 0x08 | value | [31:0] | R | Trace write pointer (words) |
| DBG_OVERFLOW | 0x0C | value | [0] | R | Sticky trace overflow |
| CRC_RD_EXPECTED / CRC_WR_EXPECTED / CRC_WR_COMPUTED | 0x10 / 0x14 / 0x18 | value | [31:0] | R | Aggregate CRCs |
| CRC_MATCH | 0x1C | match / valid | [0] / [1] | R | CRC match |
| SCRATCH | 0x20 | value | [31:0] | RW | Ping / autodetect (magic 0xC0FFEE5A) |
| BUILD_ID | 0x24 | value | [31:0] | R | 0x5354_5243 (“STRC”) |

## 6.3	Timer and the latency knob

Timer and memory-latency registers

| Register | Offset | Field | Bits | Access | Description |
| --- | --- | --- | --- | --- | --- |
| TIMER_CTRL | 0x28 | clear | [0] | W | Pulse: clear the cycle counter |
| TIMER_STATUS | 0x2C | done / running / pass | [0] / [1] / [2] | R | Timer state |
| TIMER_CYCLES_LO/HI | 0x30 / 0x34 | value | [31:0] | R | 64-bit free-running cycles |
| TIMER_EXPECTED_BEATS | 0x38 | value | [31:0] | RW | Sink beat count gating “done” |
| RESP_DELAY | 0x3C | rd_delay | [15:0] | RW | Read memory latency (aclk cycles) |
| RESP_DELAY | 0x3C | wr_delay | [31:16] | RW | Write memory latency |
| TIMER_{R,W}_{FIRST,LAST}_{LO,HI} | 0x40–0x5C | value | [31:0] | R | First/last R/W beat timestamps |

## 6.4	CRC, kick, and observability

CRC, kick, and observability registers

| Register | Offset | Field | Bits | Description |
| --- | --- | --- | --- | --- |
| CRC_RD_PER_CH0..7 | 0x60–0x7C | value | [31:0] | Per-channel read CRC |
| CRC_WR_PER_CH0..7 | 0x80–0x9C | value | [31:0] | Per-channel write CRC |
| CRC_VALID_MASK / CRC_MATCH_MASK | 0xA0 / 0xA4 | value | [7:0] | Per-channel masks |
| CH0..3_KICK_ADDR | 0xB0–0xBC | value | [31:0] | Kick shadow addr (ch 0–3) |
| KICK_GO | 0xC0 | mask | [7:0] | W: pulse a HW kick per set bit |
| CH4..7_KICK_ADDR | 0xC4–0xD0 | value | [31:0] | Kick shadow (ch 4–7) — split around KICK_GO |
| DESC_*_HS | 0xD4–0xFC | value | [31:0] | Descriptor SRAM / AXI handshake counters |
| OBS_RD_* / OBS_WR_* | 0x100–0x11C | value | [31:0] | DMA observer aggregate buckets |
| OBS_HIST_SEL/DATA/TOTAL | 0x120–0x128 | value | [31:0] | Burst-count histogram |
| COMP_TIER1_A/B/C, COMP_TIER0, COMP_*_OVF | 0x1E0–0x1FC | value | [31:0] | MonBus-compression observer readback |

The kick-address block is split around KICK_GO (ch 0–3 below, ch 4–7 above), so a naive BASE + 4*ch mislands channel 4 — use the host’s kick_addr_csr(ch) helper.

## 6.5	STREAM monitor window (0x1000)

STREAM’s PeakRDL regmap places the AXI monitor register block at APB offset 0x1000+ (stream_mon_regs.rdl @ 0x1000). The perf-counter readbacks (RDMON_PERF_*, WRMON_PERF_*, DAXMON_PERF_*) live there and are read by name via stream_addrs.A() — hardcoding this window broke a path once, so the by-name access is mandatory. Monitor presets (perf-mon, debug-basic, debug-compl, debug-all, debug-core) are built by host/mon_configs.py.

## 6.6	Descriptor format (desc_ram @ 0x0002_0000)

32 B / 256-bit per descriptor, 8 × 32-bit words: src_addr lo/hi (w0/1), dst_addr lo/hi (w2/3), length in beats (w4), next_ptr (w5, 0 = end), ctrl (w6: valid[0], interrupt[1], last[2], error[3], channel_id[7:4], priority[15:8], stamp[31:16]), reserved (w7). Built by host/descriptor_builder.py.

# 7	Simulation / Silicon Equivalence

The primary flows-stream-bridge flow has a real UART-equivalence cocotb sim: the testbench drives the same uart_axil_bridge RTL over the same ASCII W/R byte stream the FPGA host uses, and the extended-addressing tests run the same Python host programs. Only the bottom transport differs.

## 7.1	The testbench

- TB: flows-stream-bridge/dv/tbclasses/stream_char_tb.py (StreamCharTB). Drives i_uart_rx / o_uart_tx via the CocoTBFramework UARTMaster / UARTMonitor BFMs; UART_BAUD is raised (CLKS_PER_BIT = 8) for sim speed — bit-timing only, the byte stream is unchanged. Registers are addressed by name (stream_addrs.A, harness_addrs.H).

- Backend model: the real harness RTL, elaborated in Verilator — axi4_dma_slaves (LFSR pattern-gen source + CRC-check sink), axi_response_delay, axi4_dma_observer. Descriptors are built by the shared host/descriptor_builder.py.

- Running the actual host program in sim: the ext-addressing tests execute stream_ext_suite.run_suite / stream_ext_char under await cocotb.external(program)(), with a _Bridge shim wrapping cocotb.function(uart_write / uart_read) — the shared harness methodology (a cocotb.function transport, not a free-running pump). This is the same-source silicon-equivalence path.

## 7.2	Test levels

dv/tests/test_stream_char.py runs by TEST_LEVEL:

cocotb test levels

| Level | Cocotb tests |
| --- | --- |
| gate | ping |
| func | + desc_load, csr_read, apb_config, desc_perf, rw_perf, obs_equiv, dma_1ch, dma_2ch |
| full | + dma_3ch … dma_Nch, compress_char |

Plus test_stream_char_ext_suite / test_stream_char_ext_char (the run-the-host-program tests) and host-side unit tests (host/test_harness_regmap.py, test_stream_device.py, test_mon_configs.py, …). Sim params (BASE_RTL_PARAMS): NUM_CHANNELS = 4 (down from 8 for Artix BRAM), DATA_WIDTH = 128, DESC_RAM_ENTRIES = 128, DEBUG_SRAM_WORDS = 4096, USE_MON_COMPRESSION = 1. make bitstream runs a verify-sim gate (-k "ping or csr_read") so a broken elaboration is caught before a long build.

## 7.3	Sibling flows

- flows-idma-bridge has a cocotb cosim (make perf / make desc-overhead) but not the UART harness.

- flows-vivado-mcdma has no cocotb — the MCDMA IP is VHDL and Verilator cannot simulate it, so that flow is FPGA-only (rationale in its dv/README.md).

The shared collateral in bin/TBClasses/harness/ (byte_channel.py, uart_register_map.py, cocotb_axil_bridge.py with make_uart_channel, device.py, harness.py) is the basis this TB builds on.

# 8	Troubleshooting

## 8.1	Current state

The primary stream-bridge flow is working: cocotb gate + full FPGA sweeps are collected, with data in the perf (v1.4) and compression (v1.3) writeups (flows-stream-bridge/progress/STATUS = 90-DONE). flows-vivado-mcdma is a skeleton (blinks an LED, drives no transfers); flows-idma-bridge is area + cosim only.

## 8.2	Known issues

Known issues and workarounds

| Symptom | Cause / workaround |
| --- | --- |
| Configs hang after a long mixed session | State-accumulation wedge (4–7 active channels under debug-compl): AXI_RD/WR_COMPLETE = 0, and a per-run soft/cluster reset does not clear it — only make program (full reprogram) does. Reset-completeness gap, not data corruption. Tracked in rtl/amba/KNOWN_ISSUES/axi_monitor_blockready_hang_partial_channels.md. |
| A later no-delay run silently under-performs | Sticky RESP_DELAY: a --resp-delays sweep leaves the CSR set. Re-program before a matrix/size sweep. |
| 16desc_* configs trip trace.overflow | Benign — bounds the 2048-beat debug trace, not the bus-meter counters. |
| run_characterization.py -o crashes on save | -o paths are relative to host/; use an absolute path or ../../reports/perf. |

## 8.3	Cannot find the board

The USB-UART re-enumerates on replug. Use --port auto (SCRATCH magic 0xC0FFEE5A) or a stable /dev/serial/by-id/... path — never a hardcoded /dev/ttyUSB*. Confirm alive with dump_status.py → BUILD_ID = 0x5354_5243.

## 8.4	Not simulatable

The Vivado MCDMA IP is VHDL — Verilator cannot simulate it, so MCDMA validation is FPGA-only. The full iDMA end-to-end (desc64 + backend) is not built (width mismatch); its two halves are characterized separately for the area comparison.

## 8.5	Board fit vs sim geometry

NUM_CHANNELS is reduced 8 → 4 for sim (Artix BRAM); the as-built characterization bitstream is 8-channel with monitors on (the perf build). Timing closes at +0.031 ns WNS after trimming monitors to perf-only.

## 8.6	Which README to trust

The dated flows-stream-bridge/README.md (“Phase 1”, 2026-04) describes an earlier 5-slave axil_decode_5s / axil2apb decoder. The current design uses the generated bridge_stream_char_axil; trust PORT_MAP.md and host/ADDRESS_MAP.md (July 2026), and the regmap generator, over that README.