RTL Design Sherpa

Bridge Hardware Architecture Specification 1.1

August 14, 2026

Table of Contents

1 Bridge Has Index	24

2 Document Information	25

2.1 Bridge Hardware Architecture Specification	26

2.2 Revision History	27

2.3 Document Purpose	29

2.4 Intended Audience	30

2.5 Related Documents	31

3 Purpose and Scope	32

3.1 What is Bridge?	33

3.2 The Problem Bridge Solves	34

3.3 Scope	35

3.4 Out of Scope	36

4 Document Conventions	37

4.1 Terminology	38

4.2 Signal Naming	39

4.3 Diagrams	40

4.3.1 Figure 1.1: Block Diagram Legend	40

4.4 Code Examples	41

4.5 Parameter Notation	42

5 Definitions and Acronyms	43

5.1 Acronyms	44

5.2 Definitions	46

6 Use Cases	47

6.1 Primary Applications	48

6.1.1 SoC Interconnects	48

6.1.2 Memory Systems	48

6.1.3 Accelerator Systems	48

6.2 Example: RAPIDS Accelerator	49

6.2.1 Figure 2.1: RAPIDS System Topology	49

6.2.2 Configuration	49

6.2.3 Benefits	49

6.3 Rapid Prototyping	51

7 Key Features	52

7.1 Configuration-Driven Generation	53

7.1.1 CSV/TOML Configuration	53

7.1.2 Example Configuration	53

7.2 Multi-Protocol Support	54

7.2.1 Protocol Features	54

7.3 Channel-Specific Masters	55

7.3.1 Write-Only Masters (wr)	55

7.3.2 Read-Only Masters (rd)	55

7.3.3 Full Masters (rw)	55

7.3.4 Resource Savings	55

7.4 Automatic Converters	56

7.4.1 Width Conversion	56

7.4.2 Protocol Conversion	56

7.5 Flexible Topology	57

7.5.1 Scalability	57

7.5.2 Mixed Configurations	57

8 System Context	58

8.1 Bridge in SoC Architecture	59

8.1.1 Figure 2.2: Bridge System Context	59

8.2 Interface Boundaries	60

8.2.1 Master-Side (Upstream)	60

8.2.2 Slave-Side (Downstream)	60

8.3 Address Space	61

8.3.1 Address Map Organization	61

8.3.2 Address Decode	61

8.4 Clock and Reset	62

8.4.1 Clock Domain	62

8.4.2 Reset	62

8.5 Dependencies	63

8.5.1 Required Infrastructure	63

8.5.2 Optional Features	63

9 Block Diagram	64

9.1 Bridge Architecture Overview	65

9.1.1 Figure 3.1: Bridge Block Diagram	65

9.2 Functional Blocks	66

9.2.1 Master Adapter	66

9.2.2 Crossbar Core	66

9.2.3 Slave Router	66

9.3 Data Flow	67

9.3.1 Figure 3.2: Write Path	67

9.3.2 Figure 3.3: Read Path	67

9.4 Scalability	68

9.4.1 Topology Parameters	68

9.4.2 Resource Scaling	68

10 Data Flow	69

10.1 Transaction Flow Overview	70

10.1.1 Write Transaction Flow	70

10.1.2 Read Transaction Flow	70

10.2 Channel Independence	71

10.2.1 AXI4 Channel Separation	71

10.2.2 Parallel Operation	71

10.3 ID Extension	72

10.3.1 Bridge ID (BID) Concept	72

10.3.2 Example: 4 Masters, 4-bit ID	72

10.4 Response Routing	73

10.4.1 B Channel (Write Response)	73

10.4.2 R Channel (Read Data)	73

10.5 Out-of-Order Support	74

10.5.1 Transaction Interleaving	74

10.5.2 Example Sequence	74

11 Protocol Support	75

11.1 Supported Protocols	76

11.1.1 AXI4 Full	76

11.1.2 AXI4-Lite	76

11.1.3 APB (Advanced Peripheral Bus)	76

11.2 Protocol Conversion Matrix	77

11.3 Automatic Conversion Shims at Slave Boundary	78

11.4 AXI4 to APB Conversion	79

11.4.1 Conversion Process	79

11.4.2 Figure 3.1: AXI4 to APB Conversion Sequence	79

11.4.3 Burst Handling	79

11.4.4 Timing Considerations	79

11.5 Width Conversion	80

11.5.1 Upsize (Narrow to Wide)	80

11.5.2 Figure 3.2: Width Upsize Conversion	80

11.5.3 Downsize (Wide to Narrow)	81

11.5.4 Figure 3.3: Width Downsize Conversion	81

11.6 Channel-Specific Protocol	82

11.6.1 Write-Only Masters	82

11.6.2 Read-Only Masters	82

12 AXI4 Interface Overview	83

12.1 Interface Organization	84

12.1.1 Master-Side Interfaces	84

12.1.2 Slave-Side Interfaces	84

12.2 Signal Naming Convention	86

12.2.1 Custom Prefixes	86

12.2.2 ID Width Extension	86

12.3 Channel-Specific Interfaces	87

12.3.1 Write-Only Master (wr)	87

12.3.2 Read-Only Master (rd)	87

12.4 Monitor Bus Output (when variants includes mon)	88

12.5 Handshake Protocol	89

12.5.1 Valid/Ready Handshake	89

12.5.2 Backpressure	89

12.6 Burst Support	90

12.6.1 Supported Burst Types	90

12.6.2 Burst Length	90

13 APB Interface Overview	91

13.1 APB Protocol Summary	92

13.1.1 Signal Definition	92

13.2 APB Slave Interface	93

13.2.1 Signal Naming	93

13.3 APB Transaction Timing	94

13.3.1 Write Transaction	94

13.3.2 Read Transaction	94

13.4 AXI4 to APB Conversion	95

13.4.1 Conversion Requirements	95

13.4.2 Burst Handling	95

13.4.3 Error Mapping	95

13.5 Performance Implications	96

13.5.1 APB Limitations	96

13.5.2 Design Recommendations	96

14 Clock and Reset	97

14.1 Clock Requirements	98

14.1.1 Single Clock Domain	98

14.1.2 Figure 4.1: Clock Distribution	98

14.1.3 No Clock Domain Crossing	98

14.2 Reset Requirements	99

14.2.1 Asynchronous Active-Low Reset	99

14.2.2 Reset Behavior	99

14.2.3 Figure 4.3: Reset Timing	99

14.2.4 Reset Effects	99

14.2.5 Outstanding Transactions	99

14.2.6 Reset Release	100

14.3 Timing Constraints	101

14.3.1 Setup and Hold	101

14.3.2 Clock-to-Output	101

14.3.3 Recommended Constraints	101

14.4 Multi-Clock Systems	102

14.4.1 External CDC Required	102

14.4.2 Figure 4.2: Multi-Clock CDC	102

14.4.3 CDC Recommendations	102

15 AXI5 and APB5 Interfaces (AMBA5 Support)	103

15.1 Declaring AMBA5 Ports	104

15.2 AXI5 Port Surface	105

15.3 APB5 Slave Surface	106

15.4 Interop Matrix	107

16 Throughput Characteristics	108

16.1 Peak Throughput	109

16.1.1 Single Master to Single Slave	109

16.1.2 Formula	109

16.2 Sustained Throughput	110

16.2.1 Factors Affecting Throughput	110

16.2.2 Multi-Master Scaling	110

16.2.3 Example: 4 Masters, 2 Slaves	110

16.3 Burst Efficiency	111

16.3.1 Burst vs Single-Beat	111

16.3.2 Recommendation	111

16.4 Width Conversion Impact	112

16.4.1 Upsize (Narrow to Wide)	112

16.4.2 Downsize (Wide to Narrow)	112

16.5 Protocol Conversion Impact	113

16.5.1 AXI4 to APB	113

16.5.2 Recommendation	113

17 Latency Analysis	114

17.1 Latency Components	115

17.1.1 Address Path Latency	115

17.1.2 Data Path Latency	115

17.1.3 Response Path Latency	115

17.2 End-to-End Latency	116

17.2.1 Write Transaction (Best Case)	116

17.2.2 Read Transaction (Best Case)	116

17.2.3 Contention Latency	116

17.3 Width Conversion Latency	117

17.3.1 Upsize Latency	117

17.3.2 Downsize Latency	117

17.4 Protocol Conversion Latency	118

17.4.1 AXI4 to APB	118

17.4.2 APB Wait States	118

17.5 Latency Optimization	119

17.5.1 Design Recommendations	119

18 Resource Estimates	120

18.1 FPGA Resource Summary	121

18.1.1 Baseline Configuration (2x2, 64-bit)	121

18.1.2 Scaling Factors	121

18.2 Component Breakdown	122

18.2.1 Per-Master Resources	122

18.2.2 Per-Slave Resources	122

18.2.3 Crossbar Core	122

18.3 Width Converter Resources	123

18.4 Protocol Converter Resources	124

18.4.1 AXI4 to APB	124

18.5 Example Configurations	125

18.5.1 Notes	125

18.6 Optimization Strategies	126

18.6.1 Resource Reduction	126

18.6.2 Performance/Resource Trade-off	126

19 System Requirements	127

19.1 Hardware Requirements	128

19.1.1 Clock Infrastructure	128

19.1.2 Reset Infrastructure	128

19.1.3 Power	128

19.2 Interface Requirements	129

19.2.1 Master Requirements	129

19.2.2 Slave Requirements	129

19.3 Configuration Requirements	130

19.3.1 Address Map	130

19.3.2 Connectivity	130

19.4 Timing Requirements	131

19.4.1 Setup/Hold	131

19.4.2 Recommended Margins	131

19.5 Verification Requirements	132

19.5.1 Pre-Integration Checks	132

19.5.2 Integration Checks	132

19.5.3 Post-Integration Checks	132

20 Parameter Configuration	133

20.1 Core Parameters	134

20.2 Derived Parameters	135

20.2.1 Calculated by Generator	135

20.3 Per-Port Configuration	136

20.3.1 Master Port Configuration	136

20.3.2 Slave Port Configuration	136

20.4 Top-Level Monitor Configuration	138

20.4.1 Monitor Identifiers	138

20.4.2 AXIL→Wider-Slave Master-Side Alignment	138

20.5 Configuration File Format	139

20.5.1 TOML Configuration	139

20.5.2 CSV Connectivity	139

20.6 Parameter Validation	140

20.6.1 Generator Checks	140

20.6.2 Runtime Validation	140

20.7 Example Configurations	141

20.7.1 Simple 2x2	141

20.7.2 Mixed Protocol	141

21 Verification Strategy	142

21.1 Verification Levels	143

21.1.1 Unit Level	143

21.1.2 Integration Level	143

21.1.3 System Level	143

21.2 Test Infrastructure	144

21.2.1 CocoTB Framework with Protocol-BFM-Only Driving	144

21.2.2 Test Categories	144

21.3 Test Execution Model	146

21.3.1 Serial Execution	146

21.3.2 Boundary Probe Testing	146

21.4 Coverage Goals	147

21.4.1 Functional Coverage	147

21.4.2 Code Coverage	147

21.5 Debug Features	148

21.5.1 Waveform Capture	148

21.5.2 Debug Signals	148

21.6 Regression Testing	149

21.6.1 Continuous Integration	149

21.6.2 Test Matrix	149

21.7 Known Limitations	150

21.7.1 Test Gaps	150

21.7.2 Simulator Support	150

22 Bridge Micro-Architecture Specification Index	151

22.1 Document Organization	152

22.1.1 Front Matter	152

22.1.2 Chapter 1: Introduction	152

22.1.3 Chapter 2: Block Descriptions	152

22.1.4 Chapter 3: FSM Design	152

22.1.5 Chapter 4: ID Management	152

22.1.6 Chapter 5: Converters	152

22.1.7 Chapter 6: Generated RTL	153

22.1.8 Chapter 7: Verification	153

22.2 Quick Navigation	154

22.2.1 For New Users	154

22.2.2 For Integration	154

22.3 Visual Assets	155

22.3.1 Architecture Diagrams	155

22.3.2 FSM Diagrams	155

22.4 Component Overview	156

22.4.1 Key Features	156

22.4.2 Protocol Conversion Matrix	156

22.4.3 Design Philosophy	156

22.5 Related Documentation	157

22.5.1 Companion Specifications	157

22.5.2 Project-Level	157

22.5.3 Generator	157

22.6 Version History	158

23 Bridge: AXI4 Full Crossbar Generator - Product Requirements Document	159

23.1 Executive Summary	160

23.2 Design Philosophy	161

23.2.1 Core Principles	161

23.2.2 Architecture Philosophy	162

23.3 CRITICAL: RTL Regeneration Requirements	163

23.4 1. Product Overview	164

23.4.1 1.1 Purpose	164

23.4.2 1.2 Target Audience	164

23.4.3 1.3 Success Criteria	164

23.4.4 1.4 Implementation Status and Phases	164

23.5 2. Architecture Overview	166

23.5.1 2.1 AXI4 vs AXIS vs APB	166

23.5.2 2.2 Block Diagram	167

23.5.3 2.3 Key Components	168

23.6 3. Functional Requirements	169

23.6.1 3.1 AXI4 Protocol Compliance	169

23.6.2 3.2 Address Decoding	169

23.6.3 3.3 Arbitration Strategy	169

23.6.4 3.4 Burst Handling	170

23.7 4. Non-Functional Requirements	171

23.7.1 4.1 Performance	171

23.7.2 4.2 Resource Usage (Estimated)	171

23.7.3 4.3 Quality Requirements	171

23.8 5. Interface Specifications	172

23.8.1 5.1 AXI4 Master Interfaces (M × 5 channels)	172

23.8.2 5.2 AXI4 Slave Interfaces (S × 5 channels)	173

23.8.3 5.3 Configuration Parameters	173

23.9 6. Performance Modeling	174

23.9.1 6.1 Analytical Model	174

23.9.2 6.2 Resource Scaling	174

23.9.3 6.3 Comparison with Other Crossbars	175

23.10 7. Generator Architecture	176

23.10.1 7.1 Python Generator Structure	176

23.10.2 7.2 Address Map Configuration	177

23.11 8. Comparison with APB and Delta	179

23.11.1 8.1 Code Reuse from APB Generator	179

23.11.2 8.2 Code Reuse from Delta Generator	179

23.11.3 8.3 Complexity Comparison	179

23.12 9. Use Cases	181

23.12.1 9.1 Multi-Core Processor Interconnect	181

23.12.2 9.2 DMA + Accelerator System	181

23.12.3 9.3 FPGA System Integration	181

23.13 10. Testing Strategy	182

23.13.1 10.1 FUB (Functional Unit Block) Tests	182

23.13.2 10.2 Integration Tests	182

23.13.3 10.3 Performance Validation	182

23.14 11. Documentation Plan	183

23.14.1 11.1 Specifications (Before Code)	183

23.14.2 11.2 Performance Analysis (Before Implementation)	183

23.14.3 11.3 Code Generation	183

23.14.4 11.4 Verification	183

23.15 12. Success Metrics	184

23.15.1 12.1 Functional Completeness	184

23.15.2 12.2 Performance Targets	184

23.15.3 12.3 Educational Value	184

23.15.4 12.4 Reusability	184

23.16 12. Attribution and Contribution Guidelines	185

23.16.1 12.1 Git Commit Attribution	185

23.17 12.2 PDF Generation Location	186

23.18 13. Future Enhancements	187

23.18.1 13.1 Short-Term (Post-Initial Release)	187

23.18.2 13.2 Long-Term	187

23.19 14. Risk Assessment	188

23.20 15. Project Timeline (Estimated)	189

23.21 16. References	190

23.22 17. Glossary	191

24 Claude Code Guide: Bridge Subsystem	192

24.1 CRITICAL: Read Architecture Documents First	193

24.2 Quick Context	194

24.3 Target Architecture: Intelligent Width-Aware Routing	195

24.3.1 Efficient Multi-Width Design	195

24.3.2 Why This Architecture	195

24.3.3 Per-Master Output Paths	195

24.3.4 Benefits	196

24.3.5 Implementation Status	196

24.4 CRITICAL RULE #0: RTL Regeneration Requirements	197

24.4.1 The Golden Rule	197

24.4.2 Why This Is Non-Negotiable	197

24.4.3 Real Example (Historical Session)	197

24.4.4 Generator Files That Trigger Full Regeneration	197

24.4.5 The Regeneration Workflow	197

24.4.6 Symptoms of Version Mismatch	198

24.4.7 Think Like a Compiler Developer	198

24.4.8 Exception: Hand-Written RTL	198

24.5 Critical Pitfalls: slave_select_* Reversion After Handshake	199

24.5.1 The Problem: Combinational Address Decode	199

24.5.2 Visible Symptoms	199

24.5.3 The Solution (MANDATORY in All Generators)	199

24.5.4 When This Bug Appears	200

24.6 MANDATORY: Project Organization Pattern	201

24.6.1 Required Directory Structure	201

24.6.2 Testbench Class Location (MANDATORY)	201

24.6.3 Test File Pattern (MANDATORY)	202

24.7 Critical Rules for This Subsystem	204

24.7.1 Rule #0: Attribution Format for Git Commits	204

24.7.2 Rule #0.1: Testbench Architecture - MANDATORY SEPARATION	204

24.7.3 Rule #1: All Testbenches Inherit from TBBase	205

24.7.4 Rule #2: Mandatory Testbench Methods	205

24.7.5 Rule #3: Use GAXI Components for Protocol Handling	206

24.7.6 Rule #4: Queue-Based Verification	206

24.8 TOML/CSV-Based Bridge Generator (Phase 2 Complete)	208

24.8.1 Overview	208

24.8.2 Quick Start	208

24.8.3 Configuration Format Details	209

24.8.4 Channel-Specific Masters (Phase 2 Feature)	210

24.8.5 Example: RAPIDS-Style Configuration	210

24.8.6 Common User Questions	211

24.8.7 Generator Output Structure	213

24.8.8 Testing Generated Bridges	214

24.9 Bridge Architecture Quick Reference	215

24.9.1 Generated Bridge Crossbar Structure	215

24.9.2 Key Features	215

24.9.3 Address Map	215

24.10 Test Organization	216

24.10.1 Test Hierarchy	216

24.10.2 Test Levels	216

24.11 Common User Questions and Responses	217

24.11.1 Q: “How does the Bridge work?”	217

24.11.2 Q: “How do I generate a Bridge?”	217

24.11.3 Q: “How do I test a Bridge?”	217

24.12 Anti-Patterns to Avoid	219

24.12.1 Anti-Pattern 1: Embedded Testbench Classes	219

24.12.2 Anti-Pattern 2: Manual AXI4 Handshaking	219

24.12.3 Anti-Pattern 3: Memory Models for Simple Tests	219

24.13 Quick Reference	220

24.13.1 Finding Existing Components	220

24.13.2 Common Commands	220

24.14 Remember	221

24.15 PDF Generation Location	222

List of Figures

Figure 1.1: Block Diagram Legend	40

Figure 2.1: RAPIDS System Topology	49

Figure 2.2: Bridge System Context	59

Figure 3.1: Bridge Block Diagram	65

Figure 3.2: Write Path	67

Figure 3.3: Read Path	67

Figure 3.1: AXI4 to APB Conversion Sequence	79

Figure 3.2: Width Upsize Conversion	80

Figure 3.3: Width Downsize Conversion	81

Figure 4.1: Clock Distribution	98

Figure 4.3: Reset Timing	99

Figure 4.2: Multi-Clock CDC	102

List of Tables

Table 1: Table 1.1: Common Terminology	38

Table 2: Table 1.2: Signal Naming Prefixes	39

Table 3: Table 1.3: Block Diagram Symbols	40

Table 4: Table 1.4: Parameter Format Example	42

Table 5: Table 1.5: Acronyms and Abbreviations	44

Table 6: Table 2.1: Supported Protocols	54

Table 7: Table 2.2: Channel-Specific Resource Savings	55

Table 8: Table 2.3: Address Map Organization	61

Table 9: Table 3.1: Topology Parameter Scaling	68

Table 10: Table 3.2: AXI4 Channel Summary	71

Table 11: Table 3.3: Protocol Conversion Matrix	77

Table 12: Table 4.9: Master-Side Interface Channels	84

Table 13: Table 4.10: Slave-Side Interface Channels	85

Table 14: Table 4.11: Supported Burst Types	90

Table 15: Table 4.1: APB Signal Definitions	92

Table 16: Table 4.2: AXI4 to APB Burst Conversion	95

Table 17: Table 4.3: APB to AXI4 Error Mapping	95

Table 18: Table 4.4: Clock Requirements	98

Table 19: Table 4.5: Reset Signal	99

Table 20: Table 4.6: Reset Effects	99

Table 21: Table 4.7: Timing Constraints	101

Table 22: Table 4.8: Clock-to-Output Delays	101

Table 23: Table 5.1: Peak Throughput by Data Width	109

Table 24: Table 5.2: Factors Affecting Throughput	110

Table 25: Table 5.3: Burst Efficiency Comparison	111

Table 26: Table 5.4: AXI4 vs APB Protocol Impact	113

Table 27: Table 5.5: Address Path Latency	115

Table 28: Table 5.6: Data Path Latency	115

Table 29: Table 5.7: Response Path Latency	115

Table 30: Table 5.8: Upsize Latency by Ratio	117

Table 31: Table 5.9: Downsize Latency by Ratio	117

Table 32: Table 5.10: AXI4 to APB Conversion Latency	118

Table 33: Table 5.11: Baseline Resource Requirements	121

Table 34: Table 5.12: Resource Scaling Factors	121

Table 35: Table 5.13: Per-Master Resource Breakdown	122

Table 36: Table 5.14: Per-Slave Resource Breakdown	122

Table 37: Table 5.15: Crossbar Core Resources	122

Table 38: Table 5.16: Width Converter Resources	123

Table 39: Table 5.17: AXI4 to APB Converter Resources	124

Table 40: Table 5.18: Complete Bridge Resource Estimates	125

Table 41: Table 5.19: Performance/Resource Trade-offs	126

Table 42: Table 6.1: Clock Requirements	128

Table 43: Table 6.2: Reset Requirements	128

Table 44: Table 6.3: Power Requirements	128

Table 45: Table 6.4: Address Map Requirements	130

Table 46: Table 6.5: Connectivity Requirements	130

Table 47: Table 6.6: Timing Requirements	131

Table 48: Table 6.7: Recommended Timing Margins	131

Table 49: Table 6.8: Bridge Core Parameters	134

Table 50: Table 6.9: Derived Parameters	135

Table 51: Table 6.10: Master Port Configuration	136

Table 52: Table 6.11: Slave Port Configuration	136

Table 53: Table 6.12: Generator Validation Checks	140

Table 54: Table 6.13: Unit Level Test Focus	143

Table 55: Table 6.14: Integration Test Coverage	143

Table 56: Table 6.15: System Level Tests	143

Table 57: Table 6.16: Test Categories	144

Table 58: Table 6.17: Functional Coverage Goals	147

Table 59: Table 6.18: Code Coverage Goals	147

Table 60: Table 6.19: Debug Signals	148

Table 61: Table 6.20: CI Test Schedule	149

Table 62: Table 6.21: Configuration Test Matrix	149

Table 63: Table 6.22: Known Test Gaps	150

Table 64: Table 6.23: Simulator Support Status	150

# 1	Bridge Has Index

Generated: 2026-08-14

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	Bridge Hardware Architecture Specification

| Property | Value |
| --- | --- |
| Document Title | Bridge Hardware Architecture Specification |
| Version | 1.1 |
| Date | June 4, 2026 |
| Status | Released |
| Classification | Open Source - Apache 2.0 License |

## 2.2	Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-01-03 | RTL Design Sherpa | Initial release - restructured from single spec |
| 1.1 | 2026-06-04 | RTL Design Sherpa | Content sync to RTL state at 2026-06-04. Documents (1) the 64→128-bit monbus packet migration with new 64-bit side-band timestamp wire — referenced via docs/markdown/rtl-amba/includes/monitor_package_spec.md; (2) per-port + global SV-parameter monitor methodology with mandatory use_monitor TOML field and variants = ["no","mon"] generator selector; (3) generator-emitted AXI4↔AXIL conversion at the slave boundary (was external); (4) new AXIL→wider-slave master-side alignment converters (axil_to_axi4_wide_align_{rd,wr}); (5) verification-strategy rewrite to protocol-BFM-only TBs with memory-backed slaves, serial-only execution, per-module cocotb function names, boundary_probe (renamed from address_decode) tests; (6) note of the independent W-tracker FIFO + split AW/W-ready MUX fix (commit d24bd617) that closed an interleaved-master deadlock. |

## 2.3	Document Purpose

This Hardware Architecture Specification (HAS) is the high-level view of the Bridge component. It covers:

- System-level architecture and data flow

- Protocol support (AXI4, AXI4-Lite, APB)

- Interface definitions and requirements

- Performance characteristics

- Integration guidelines

For micro-architecture, FSM designs, and signal-level detail, see the companion Bridge Micro-Architecture Specification (MAS).

## 2.4	Intended Audience

- System architects defining interconnect topology

- Hardware engineers integrating Bridge into SoC designs

- Verification engineers planning test strategies

- Technical managers evaluating Bridge capabilities

## 2.5	Related Documents

- Bridge MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Purpose and Scope

## 3.1	What is Bridge?

Bridge is a Python-based multi-protocol crossbar generator that turns human-readable configuration files into parameterized SystemVerilog RTL. The generated AXI4 crossbars come with automatic support for:

- Protocol conversion - AXI4, AXI4-Lite, and APB slave interfaces

- Width conversion - Automatic upsize/downsize for data width mismatches

- Channel-specific masters - Write-only, read-only, or full AXI4 masters

## 3.2	The Problem Bridge Solves

A hand-written AXI4 crossbar is error-prone. Building one means:

- Writing 5 separate channel multiplexers (AW, W, B, AR, R)

- Implementing per-slave arbitration for each channel

- Managing ID-based response routing for out-of-order support

- Handling burst locking and interleaving constraints

- Inserting width converters for data width mismatches

- Inserting protocol converters for APB/AXI4-Lite mixed systems

- Wiring hundreds of signals with consistent naming

Bridge automates all of it:

- Define ports and connectivity in configuration files

- Run the generator script

- Get production-ready SystemVerilog RTL

## 3.3	Scope

This specification covers:

- High-level architecture and block organization

- Supported protocols and conversion capabilities

- Interface requirements and signal conventions

- Performance characteristics and resource estimates

- Integration guidelines and verification strategy

## 3.4	Out of Scope

These live in the companion MAS:

- Detailed FSM state diagrams and transitions

- Signal-level timing diagrams

- Internal pipeline structures

- RTL implementation details

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Document Conventions

## 4.1	Terminology

Table 1.1: Common Terminology

| Term | Definition |
| --- | --- |
| Master | AXI4 initiator that issues transactions |
| Slave | AXI4 target that responds to transactions |
| Crossbar | NxM interconnect connecting masters to slaves |
| Channel | AXI4 communication path (AW, W, B, AR, R) |
| Converter | Logic that transforms width or protocol |

## 4.2	Signal Naming

Bridge uses consistent signal prefixes:

Table 1.2: Signal Naming Prefixes

| Prefix | Meaning |
| --- | --- |
| s_* | Slave-side interface (master port on bridge) |
| m_* | Master-side interface (slave port on bridge) |
| axi4_* | AXI4 protocol signals |
| apb_* | APB protocol signals |
| cfg_* | Configuration signals |
| dbg_* | Debug signals |

## 4.3	Diagrams

### 4.3.1	Figure 1.1: Block Diagram Legend

Block diagrams in this specification use these symbols:

Table 1.3: Block Diagram Symbols

| Symbol | Meaning |
| --- | --- |
| Rectangle | Functional block |
| Arrow | Data/signal flow |
| Dashed box | Optional/configurable block |
| Double line | Multi-signal bus |

## 4.4	Code Examples

SystemVerilog examples are formatted like this:

// Module instantiation example
bridge_4x4 u_bridge (
    .aclk       (sys_clk),
    .aresetn    (sys_rst_n),
    // ... port connections
);

## 4.5	Parameter Notation

Parameter tables throughout the spec use this format:

Table 1.4: Parameter Format Example

| Parameter | Type | Range | Default | Description |
| --- | --- | --- | --- | --- |
| NUM_MASTERS | int | 1-32 | 2 | Number of master ports |
| NUM_SLAVES | int | 1-256 | 2 | Number of slave ports |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Definitions and Acronyms

## 5.1	Acronyms

Table 1.5: Acronyms and Abbreviations

| Acronym | Definition |
| --- | --- |
| AMBA | Advanced Microcontroller Bus Architecture |
| APB | Advanced Peripheral Bus |
| AR | AXI4 Read Address channel |
| AW | AXI4 Write Address channel |
| AXI | Advanced eXtensible Interface |
| AXI4 | AXI Protocol Version 4 |
| B | AXI4 Write Response channel |
| BID | Bridge ID (internal master identifier) |
| CAM | Content-Addressable Memory |
| CDC | Clock Domain Crossing |
| CSV | Comma-Separated Values |
| DECERR | Decode Error (AXI response) |
| DMA | Direct Memory Access |
| FIFO | First-In First-Out buffer |
| FSM | Finite State Machine |
| HAS | Hardware Architecture Specification |
| ID | Transaction Identifier |
| MAS | Micro-Architecture Specification |
| MUX | Multiplexer |
| NxM | N masters by M slaves (topology notation) |
| OOO | Out-of-Order |
| OOR | Out-of-Range |
| QoS | Quality of Service |
| R | AXI4 Read Data channel |
| RTL | Register-Transfer Level |
| SLVERR | Slave Error (AXI response) |
| SoC | System-on-Chip |
| TOML | Tom’s Obvious Minimal Language |
| W | AXI4 Write Data channel |

## 5.2	Definitions

Address Decode: Determining which slave receives a transaction, from its address.

Arbitration: Selecting which master gets a slave when several masters contend for it.

Bridge ID (BID): Internal identifier prepended to master IDs for response routing. Width = clog2(NUM_MASTERS).

Channel-Specific Master: A master that only uses subset of AXI4 channels (write-only or read-only).

Crossbar: An NxM interconnect allowing any master to communicate with any connected slave.

Grant Locking: Mechanism that holds arbitration grant until transaction completes.

Out-of-Order Response: Responses returned in different order than requests were issued.

Protocol Conversion: Transformation between different bus protocols (e.g., AXI4 to APB).

Response Routing: Directing B/R channel responses back to the originating master.

Round-Robin Arbitration: Fair arbitration where priority rotates among requesters.

Width Conversion: Transformation between different data bus widths (upsize or downsize).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Use Cases

## 6.1	Primary Applications

### 6.1.1	SoC Interconnects

Bridge acts as the main interconnect fabric in SoC designs:

- Multi-core processor memory subsystems - Connect multiple CPU cores to shared memory

- Accelerator integration - Route GPU, DSP, or custom accelerator traffic

- DMA engine routing - Connect DMA controllers to memory and peripherals

- Peripheral bus bridges - Convert AXI4 to APB for low-speed peripherals

### 6.1.2	Memory Systems

Typical memory-system uses:

- Multi-port DDR controllers - Multiple masters accessing shared DDR

- SRAM buffer sharing - Shared packet buffers for networking

- Cache coherency interconnects - Connect cache controllers

- Memory-mapped I/O - Unified address space for peripherals

### 6.1.3	Accelerator Systems

Bridge supports custom accelerator architectures like RAPIDS:

- Descriptor write masters - Write control descriptors (write-only)

- Sink write masters - Write incoming data packets (write-only)

- Source read masters - Read outgoing data packets (read-only)

- CPU configuration - Full access for register configuration

## 6.2	Example: RAPIDS Accelerator

### 6.2.1	Figure 2.1: RAPIDS System Topology

RAPIDS System Topology

Bridge topology for RAPIDS accelerator with channel-specific masters.

### 6.2.2	Configuration

[bridge]
  name = "bridge_rapids"
  masters = [
    {name = "rapids_descr_wr", channels = "wr", data_width = 512},
    {name = "rapids_sink_wr", channels = "wr", data_width = 512},
    {name = "rapids_src_rd", channels = "rd", data_width = 512},
    {name = "cpu", channels = "rw", data_width = 64}
  ]
  slaves = [
    {name = "ddr", protocol = "axi4", data_width = 512},
    {name = "apb_periph", protocol = "apb", data_width = 32},
    {name = "sram", protocol = "axi4", data_width = 256}
  ]

### 6.2.3	Benefits

- 39% port reduction for write-only masters (3 channels vs 5)

- 61% port reduction for read-only masters (2 channels vs 5)

- Automatic width conversion for 64b CPU to 512b DDR

- Protocol conversion for APB peripheral access

## 6.3	Rapid Prototyping

Bridge accelerates design exploration:

- Quick topology exploration - Try different master/slave configurations

- Performance analysis - Evaluate arbitration and bandwidth

- Design space exploration - Compare resource usage

- Educational projects - Learn AXI4 protocol and interconnects

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Key Features

## 7.1	Configuration-Driven Generation

### 7.1.1	CSV/TOML Configuration

Bridge is configured with human-readable text files:

- ports configuration - Define each master and slave port

- connectivity matrix - Specify which masters connect to which slaves

- Version-control friendly - Text-based, easy to diff and review

### 7.1.2	Example Configuration

Port Definition (TOML):

masters = [
  {name = "cpu", prefix = "cpu_m_axi", data_width = 64, channels = "rw"},
  {name = "dma", prefix = "dma_m_axi", data_width = 256, channels = "rw"}
]
slaves = [
  {name = "ddr", prefix = "ddr_s_axi", data_width = 512, protocol = "axi4"},
  {name = "uart", prefix = "uart_apb", data_width = 32, protocol = "apb"}
]

Connectivity Matrix (CSV):

master\slave,ddr,uart
cpu,1,1
dma,1,0

## 7.2	Multi-Protocol Support

Table 2.1: Supported Protocols

| Master Protocol | Slave Protocol | Conversion Type |
| --- | --- | --- |
| AXI4 | AXI4 | Direct or width convert |
| AXI4 | AXI4-Lite | Protocol downgrade |
| AXI4 | APB | Full protocol conversion |
| AXI5 | AXI4 / AXI4-Lite / APB | Base-subset interop; sideband terminates at the boundary (warning) |
| AXI5 | AXI5 (width-matched) | Native sideband pass-through; store-class atomics; poison |
| AXI4 / AXI5 | APB5 | APB4 conversion core + APB5 sideband surface |

AMBA5 ports are declared per port (protocol = "axi5" / "apb5" with an optional axi5_features list); the fabric stays AXI4 internally. See AXI5 and APB5 Interfaces.

### 7.2.1	Protocol Features

- AXI4 Full - Complete 5-channel implementation with bursts

- AXI4-Lite - Simplified single-beat transactions

- APB - Low-power peripheral access

## 7.3	Channel-Specific Masters

### 7.3.1	Write-Only Masters (wr)

Generate only write channels: - AW (Write Address) - W (Write Data) - B (Write Response)

Use case: DMA write engines, descriptor writers

### 7.3.2	Read-Only Masters (rd)

Generate only read channels: - AR (Read Address) - R (Read Data)

Use case: DMA read engines, source data fetchers

### 7.3.3	Full Masters (rw)

Generate all five channels for complete read/write capability.

Use case: CPU interfaces, configuration masters

### 7.3.4	Resource Savings

Table 2.2: Channel-Specific Resource Savings

| Master Type | Channels | Signal Reduction |
| --- | --- | --- |
| Write-only (wr) | 3 of 5 | ~39% fewer signals |
| Read-only (rd) | 2 of 5 | ~61% fewer signals |
| Full (rw) | 5 of 5 | Baseline |

## 7.4	Automatic Converters

### 7.4.1	Width Conversion

Bridge automatically inserts width converters:

- Upsize - Narrow master to wide slave (e.g., 64b to 512b)

- Downsize - Wide master to narrow slave (e.g., 512b to 32b)

- Per-path - Only where needed, not global conversion

### 7.4.2	Protocol Conversion

Bridge inserts protocol converters for mixed systems:

- AXI4 to APB - Full conversion with burst handling

- AXI4 to AXI4-Lite - Simplified conversion

## 7.5	Flexible Topology

### 7.5.1	Scalability

- Masters: 1-32 master ports

- Slaves: 1-256 slave ports

- Partial connectivity - Not all masters need access to all slaves

### 7.5.2	Mixed Configurations

- Mixed protocols - AXI4, AXI4-Lite, and APB slaves in same crossbar

- Mixed data widths - 32b, 64b, 128b, 256b, 512b

- Mixed channels - Write-only, read-only, and full masters

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	System Context

## 8.1	Bridge in SoC Architecture

### 8.1.1	Figure 2.2: Bridge System Context

Bridge System Context

Bridge as central interconnect connecting masters to slaves with protocol conversion.

## 8.2	Interface Boundaries

### 8.2.1	Master-Side (Upstream)

Bridge presents slave interfaces to upstream masters:

- Accepts AXI4 transactions from masters

- Provides flow control via ready signals

- Returns responses (B/R channels) to correct master

### 8.2.2	Slave-Side (Downstream)

Bridge presents master interfaces to downstream slaves:

- Issues AXI4/APB transactions to slaves

- Accepts responses from slaves

- Routes responses back through ID tracking

## 8.3	Address Space

### 8.3.1	Address Map Organization

Each slave occupies a configurable address region:

Table 2.3: Address Map Organization

| Slave | Base Address | Address Range | Size |
| --- | --- | --- | --- |
| Slave 0 | BASE + 0x0000_0000 | Configurable | Variable |
| Slave 1 | BASE + addr_range_0 | Configurable | Variable |
| Slave N | Sum of previous ranges | Configurable | Variable |

### 8.3.2	Address Decode

- Parallel decode - All slaves checked simultaneously

- One-hot result - Exactly one slave selected per transaction

- Out-of-range detection - DECERR for unmapped addresses

## 8.4	Clock and Reset

### 8.4.1	Clock Domain

- Single clock domain - All Bridge logic synchronous to aclk

- No CDC - Masters and slaves must be in same clock domain

### 8.4.2	Reset

- Active-low asynchronous - Standard aresetn convention

- Full reset - All internal state cleared on reset

- Transaction safety - Outstanding transactions aborted on reset

## 8.5	Dependencies

### 8.5.1	Required Infrastructure

- AXI4-compliant masters and slaves

- Proper clock and reset distribution

- Address map configuration matching slave address ranges

### 8.5.2	Optional Features

- Width converters (only if widths mismatch)

- Protocol converters (only if protocols differ)

- ID tracking CAM (only if OOO support needed)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Block Diagram

## 9.1	Bridge Architecture Overview

### 9.1.1	Figure 3.1: Bridge Block Diagram

Bridge Block Diagram

Bridge high-level block diagram showing master adapters, crossbar core, and slave routers.

## 9.2	Functional Blocks

### 9.2.1	Master Adapter

Receives transactions from upstream masters:

- Skid buffer - Pipeline registration for timing

- ID extension - Prepend Bridge ID for response routing

- Channel separation - Route AW/W/AR independently

### 9.2.2	Crossbar Core

Central switching fabric:

- 5-channel routing - Independent AW, W, B, AR, R paths

- Address decode - Determine target slave

- Arbitration - Per-slave round-robin selection

### 9.2.3	Slave Router

Delivers transactions to downstream slaves:

- Protocol conversion - AXI4 to APB/AXI4-Lite if needed

- Width conversion - Upsize/downsize data paths

- Response collection - Route B/R back to masters

## 9.3	Data Flow

### 9.3.1	Figure 3.2: Write Path

Write Path Flow

### 9.3.2	Figure 3.3: Read Path

Read Path Flow

## 9.4	Scalability

### 9.4.1	Topology Parameters

Table 3.1: Topology Parameter Scaling

| Parameter | Range | Impact |
| --- | --- | --- |
| NUM_MASTERS | 1-32 | Linear MUX growth |
| NUM_SLAVES | 1-256 | Linear decoder growth |
| DATA_WIDTH | 32-512 | Linear data path growth |
| ID_WIDTH | 1-16 | Logarithmic CAM growth |

### 9.4.2	Resource Scaling

- Logic: O(M x N) for crossbar core

- Routing: O(M + N) for address/response paths

- Memory: O(M x outstanding) for ID tracking

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	Data Flow

## 10.1	Transaction Flow Overview

### 10.1.1	Write Transaction Flow

- Master issues AW - Address and control information

- Bridge decodes address - Determines target slave

- Arbitration - Grants access if multiple masters contend

- AW forwarded to slave - With extended ID (Bridge ID prepended)

- Master sends W data - Following the granted AW

- W forwarded to slave - Using same grant

- Slave returns B response - With extended ID

- Bridge routes B to master - Using ID to find originator

### 10.1.2	Read Transaction Flow

- Master issues AR - Address and control information

- Bridge decodes address - Determines target slave

- Arbitration - Grants access if multiple masters contend

- AR forwarded to slave - With extended ID

- Slave returns R data - Potentially multiple beats

- Bridge routes R to master - Using ID to find originator

## 10.2	Channel Independence

### 10.2.1	AXI4 Channel Separation

Each AXI4 channel operates independently:

Table 3.2: AXI4 Channel Summary

| Channel | Direction | Contents | Arbitration |
| --- | --- | --- | --- |
| AW | Master to Slave | Write address | Per-slave |
| W | Master to Slave | Write data | Follows AW grant |
| B | Slave to Master | Write response | ID-based routing |
| AR | Master to Slave | Read address | Per-slave |
| R | Slave to Master | Read data | ID-based routing |

### 10.2.2	Parallel Operation

- Read and write transactions can proceed simultaneously

- Different masters can access different slaves in parallel

- Same slave serializes access via arbitration

## 10.3	ID Extension

### 10.3.1	Bridge ID (BID) Concept

Original Master ID: [ID_WIDTH-1:0]
Extended ID: [BID | Original ID]
BID Width: clog2(NUM_MASTERS)

### 10.3.2	Example: 4 Masters, 4-bit ID

Master 0 ID = 4'b0101
Extended ID = 6'b00_0101  (BID=00, prepended)

Master 3 ID = 4'b1100
Extended ID = 6'b11_1100  (BID=11, prepended)

ID Flow

## 10.4	Response Routing

### 10.4.1	B Channel (Write Response)

- Slave issues B with extended ID

- Bridge extracts BID from upper bits

- BID determines destination master

- Original ID (lower bits) returned to master

### 10.4.2	R Channel (Read Data)

- Slave issues R with extended ID

- Bridge extracts BID from upper bits

- BID determines destination master

- Original ID (lower bits) returned to master

- RLAST indicates final beat

## 10.5	Out-of-Order Support

### 10.5.1	Transaction Interleaving

Bridge supports out-of-order completion:

- Different IDs can complete in any order

- Same ID must complete in order (AXI4 rule)

- ID tracking tables manage outstanding transactions

### 10.5.2	Example Sequence

Time 0: Master 0 issues AR (ID=1) to Slave 0
Time 1: Master 0 issues AR (ID=2) to Slave 1
Time 2: Slave 1 returns R (ID=2) - fast slave
Time 3: Slave 0 returns R (ID=1) - slow slave

Result: ID=2 response arrives before ID=1 - valid OOO

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	Protocol Support

## 11.1	Supported Protocols

### 11.1.1	AXI4 Full

Complete AMBA AXI4 protocol support:

- 5 channels: AW, W, B, AR, R

- Burst transactions: FIXED, INCR, WRAP

- Transaction IDs: Configurable width

- Out-of-order: Response reordering supported

- Data widths: 32, 64, 128, 256, 512 bits

### 11.1.2	AXI4-Lite

Simplified AXI4 for register access:

- 5 channels: Same as AXI4

- Single-beat only: AWLEN/ARLEN = 0

- No IDs: Transaction ordering by channel

- Fixed width: Typically 32 or 64 bits

### 11.1.3	APB (Advanced Peripheral Bus)

Low-power peripheral protocol:

- Single channel: Combined address and data

- Simple handshake: PSEL, PENABLE, PREADY

- No bursts: Single transfer per transaction

- Low overhead: Minimal logic for slow peripherals

## 11.2	Protocol Conversion Matrix

Table 3.3: Protocol Conversion Matrix

| Master | Slave | Conversion Type | Notes |
| --- | --- | --- | --- |
| AXI4 | AXI4 | Direct | Width conversion if needed |
| AXI4 | AXI4-Lite | Downgrade | Burst split to single beats |
| AXI4 | APB | Full convert | Channel and timing conversion |
| AXI4-Lite | AXI4 | Upgrade | Single-beat pass-through |
| AXI4-Lite | AXI4-Lite | Direct | Width conversion if needed |
| AXI4-Lite | APB | Convert | Simplified conversion |

## 11.3	Automatic Conversion Shims at Slave Boundary

When a slave port’s TOML entry names a protocol other than native AXI4 (e.g., protocol = "axi4lite" or protocol = "apb"), the generator emits the conversion shims between the crossbar core and that slave port. This happens once, at generation time; there is nothing to configure at runtime.

Supported Slave Protocols: - axi4: Native AXI4 (no shim required) - axi4lite: AXI4-Lite protocol (emits axi4_to_axil4_{rd,wr} shims) - apb: APB3/APB4 protocol (emits axi4_to_axil4 shim followed by internal AXIL-to-APB bridge)

The shim modules live in projects/components/converters/rtl/; the generator instantiates them into each generated top-level module. The slave port still presents the configured protocol (AXIL or APB) to the outside; inside, the crossbar core is uniformly AXI4.

## 11.4	AXI4 to APB Conversion

### 11.4.1	Conversion Process

### 11.4.2	Figure 3.1: AXI4 to APB Conversion Sequence

AXI4 to APB Conversion

### 11.4.3	Burst Handling

- AXI4 bursts split into individual APB transfers

- WSTRB converted to per-byte enables

- B response generated after all beats complete

### 11.4.4	Timing Considerations

- APB is inherently slower (minimum 2 cycles per transfer)

- Pipeline stalls during APB access

- Consider separate APB crossbar for multiple slow peripherals

## 11.5	Width Conversion

### 11.5.1	Upsize (Narrow to Wide)

### 11.5.2	Figure 3.2: Width Upsize Conversion

Width Upsize

### 11.5.3	Downsize (Wide to Narrow)

### 11.5.4	Figure 3.3: Width Downsize Conversion

Width Downsize

## 11.6	Channel-Specific Protocol

### 11.6.1	Write-Only Masters

Only AW, W, B channels active:

- AR permanently deasserted

- R channel ignored (no routing logic)

- Reduced signal count and logic

### 11.6.2	Read-Only Masters

Only AR, R channels active:

- AW, W, B channels ignored

- No write arbitration for this master

- Reduced signal count and logic

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	AXI4 Interface Overview

## 12.1	Interface Organization

### 12.1.1	Master-Side Interfaces

Bridge presents slave interfaces to upstream masters. Each master port includes:

Table 4.9: Master-Side Interface Channels

| Channel | Signals | Direction (to Bridge) |
| --- | --- | --- |
| AW | AWVALID, AWREADY, AWADDR, AWID, AWLEN, AWSIZE, AWBURST, … | Input |
| W | WVALID, WREADY, WDATA, WSTRB, WLAST | Input |
| B | BVALID, BREADY, BID, BRESP | Output |
| AR | ARVALID, ARREADY, ARADDR, ARID, ARLEN, ARSIZE, ARBURST, … | Input |
| R | RVALID, RREADY, RDATA, RID, RRESP, RLAST | Output |

### 12.1.2	Slave-Side Interfaces

Bridge presents master interfaces to downstream slaves. Each slave port includes:

Table 4.10: Slave-Side Interface Channels

| Channel | Signals | Direction (from Bridge) |
| --- | --- | --- |
| AW | AWVALID, AWREADY, AWADDR, AWID, AWLEN, AWSIZE, AWBURST, … | Output |
| W | WVALID, WREADY, WDATA, WSTRB, WLAST | Output |
| B | BVALID, BREADY, BID, BRESP | Input |
| AR | ARVALID, ARREADY, ARADDR, ARID, ARLEN, ARSIZE, ARBURST, … | Output |
| R | RVALID, RREADY, RDATA, RID, RRESP, RLAST | Input |

## 12.2	Signal Naming Convention

### 12.2.1	Custom Prefixes

Each port uses a custom prefix for signal naming:

// Example: Master 0 with prefix "cpu_m_axi_"
input  logic [31:0] cpu_m_axi_awaddr,
input  logic [3:0]  cpu_m_axi_awid,
input  logic        cpu_m_axi_awvalid,
output logic        cpu_m_axi_awready,
// ... remaining AW signals

// Example: Slave 0 with prefix "ddr_s_axi_"
output logic [31:0] ddr_s_axi_awaddr,
output logic [7:0]  ddr_s_axi_awid,  // Extended ID
output logic        ddr_s_axi_awvalid,
input  logic        ddr_s_axi_awready,

### 12.2.2	ID Width Extension

Slave-side IDs are wider than master-side:

Master ID Width: ID_WIDTH (configuration parameter)
Bridge ID Width: clog2(NUM_MASTERS)
Slave ID Width: ID_WIDTH + clog2(NUM_MASTERS)

## 12.3	Channel-Specific Interfaces

### 12.3.1	Write-Only Master (wr)

Only AW, W, B channels generated:

// Write Address Channel
input  logic [ADDR_WIDTH-1:0] descr_m_axi_awaddr,
input  logic [ID_WIDTH-1:0]   descr_m_axi_awid,
// ... full AW channel

// Write Data Channel
input  logic [DATA_WIDTH-1:0] descr_m_axi_wdata,
input  logic [STRB_WIDTH-1:0] descr_m_axi_wstrb,
// ... full W channel

// Write Response Channel
output logic [ID_WIDTH-1:0]   descr_m_axi_bid,
output logic [1:0]            descr_m_axi_bresp,
// ... full B channel

// NO AR or R channels

### 12.3.2	Read-Only Master (rd)

Only AR, R channels generated:

// Read Address Channel
input  logic [ADDR_WIDTH-1:0] src_m_axi_araddr,
input  logic [ID_WIDTH-1:0]   src_m_axi_arid,
// ... full AR channel

// Read Data Channel
output logic [DATA_WIDTH-1:0] src_m_axi_rdata,
output logic [ID_WIDTH-1:0]   src_m_axi_rid,
// ... full R channel

// NO AW, W, or B channels

## 12.4	Monitor Bus Output (when variants includes mon)

When monitor collection is enabled (via the bridge TOML variants list including "mon"), each bridge instance exposes a unified monitor interface at the bridge top level:

| Signal | Type | Width | Direction | Description |
| --- | --- | --- | --- | --- |
| s_mon_axil_* | AXIL Slave | 32-bit addr, 64-bit data | Input | Monitor packet read interface for CPU consumption |
| m_axil_mon_* | AXIL Master | 32-bit addr, 64-bit data | Output | Monitor packet write interface for bulk trace capture |
| stream_irq | logic | 1-bit | Output | Interrupt signal (asserted when error FIFO has records) |

Per-port collection comes from the axi4_master_{rd,wr}_mon and axi4_slave_{rd,wr}_mon wrappers; a tree of monbus_arbiter instances funnels them into a single monbus_axil_group at the bridge top. The group provides a 64-bit free-running timestamp counter, sampled at each packet arrival, and exposes both a slave interface (CPU read access) and a master interface (bulk DMA writes to system memory).

Packet Format: the canonical 128-bit packet layout and field definitions live in docs/markdown/rtl-amba/includes/monitor_package_spec.md — see that reference for bit-field descriptions, protocol-type enumerations, and packet-type codes.

## 12.5	Handshake Protocol

### 12.5.1	Valid/Ready Handshake

All channels use AXI4 valid/ready handshake:

Transfer occurs when: VALID && READY

### 12.5.2	Backpressure

- Bridge asserts READY when able to accept

- Masters must hold VALID until READY

- Bridge does not drop transactions

## 12.6	Burst Support

### 12.6.1	Supported Burst Types

Table 4.11: Supported Burst Types

| AWBURST/ARBURST | Type | Description |
| --- | --- | --- |
| 2’b00 | FIXED | Same address each beat |
| 2’b01 | INCR | Incrementing address |
| 2’b10 | WRAP | Wrapping at boundary |
| 2’b11 | Reserved | Not used |

### 12.6.2	Burst Length

- AWLEN/ARLEN: 8-bit field

- Length = AXLEN + 1 (1 to 256 beats)

- Burst does not cross 4KB boundary (AXI4 rule)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	APB Interface Overview

## 13.1	APB Protocol Summary

### 13.1.1	Signal Definition

Table 4.1: APB Signal Definitions

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| PSEL | 1 | Output | Slave select |
| PENABLE | 1 | Output | Transaction phase |
| PADDR | ADDR_WIDTH | Output | Address |
| PWRITE | 1 | Output | Write enable |
| PWDATA | DATA_WIDTH | Output | Write data |
| PSTRB | DATA_WIDTH/8 | Output | Byte strobes |
| PPROT | 3 | Output | Protection |
| PRDATA | DATA_WIDTH | Input | Read data |
| PSLVERR | 1 | Input | Slave error |
| PREADY | 1 | Input | Slave ready |

## 13.2	APB Slave Interface

### 13.2.1	Signal Naming

APB slave ports use custom prefixes:

// Example: APB slave with prefix "uart_apb_"
output logic        uart_apb_psel,
output logic        uart_apb_penable,
output logic [31:0] uart_apb_paddr,
output logic        uart_apb_pwrite,
output logic [31:0] uart_apb_pwdata,
output logic [3:0]  uart_apb_pstrb,
output logic [2:0]  uart_apb_pprot,
input  logic [31:0] uart_apb_prdata,
input  logic        uart_apb_pslverr,
input  logic        uart_apb_pready

## 13.3	APB Transaction Timing

### 13.3.1	Write Transaction

___     ___     ___     ___
PCLK   |   |___|   |___|   |___|   |___
            _________
PSEL   ____|         |________________
                _____
PENABLE ________|    |________________
            _________
PWRITE  ____|        |________________
       xxxxxx|  ADDR |xxxxxxxxxxxxxxxx
PADDR  xxxxxx|_______|xxxxxxxxxxxxxxxx
       xxxxxx| WDATA |xxxxxxxxxxxxxxxx
PWDATA xxxxxx|_______|xxxxxxxxxxxxxxxx
                _____
PREADY  ________|    |________________

### 13.3.2	Read Transaction

___     ___     ___     ___
PCLK   |   |___|   |___|   |___|   |___
            _________
PSEL   ____|         |________________
                _____
PENABLE ________|    |________________
PWRITE  ______________________________
       xxxxxx|  ADDR |xxxxxxxxxxxxxxxx
PADDR  xxxxxx|_______|xxxxxxxxxxxxxxxx
                _____
PREADY  ________|    |________________
       xxxxxxxx|RDATA|xxxxxxxxxxxxxxxx
PRDATA xxxxxxxx|_____|xxxxxxxxxxxxxxxx

## 13.4	AXI4 to APB Conversion

### 13.4.1	Conversion Requirements

When an AXI4 master accesses an APB slave, the bridge:

- Burst splitting - AXI4 bursts become multiple APB transfers

- Channel mapping - AW+W combined into PADDR+PWDATA

- Response generation - APB PSLVERR maps to AXI4 BRESP/RRESP

- Timing adaptation - Insert wait states as needed

### 13.4.2	Burst Handling

Table 4.2: AXI4 to APB Burst Conversion

| AXI4 AWLEN | APB Transfers |
| --- | --- |
| 0 (1 beat) | 1 transfer |
| 1 (2 beats) | 2 transfers |
| N (N+1 beats) | N+1 transfers |

### 13.4.3	Error Mapping

Table 4.3: APB to AXI4 Error Mapping

| APB PSLVERR | AXI4 BRESP/RRESP |
| --- | --- |
| 0 (OK) | 2’b00 (OKAY) |
| 1 (Error) | 2’b10 (SLVERR) |

## 13.5	Performance Implications

### 13.5.1	APB Limitations

- Minimum 2 cycles per transfer (setup + access phase)

- No pipelining (sequential transfers only)

- No bursts (each beat requires full handshake)

### 13.5.2	Design Recommendations

- Use APB only for slow peripherals (UART, GPIO, config registers)

- Group APB slaves behind dedicated sub-crossbar

- Use AXI4/AXI4-Lite for higher-bandwidth slaves

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Clock and Reset

## 14.1	Clock Requirements

### 14.1.1	Single Clock Domain

Bridge operates in a single synchronous clock domain:

Table 4.4: Clock Requirements

| Signal | Description | Requirements |
| --- | --- | --- |
| aclk | System clock | All interfaces synchronous |
| Frequency | Operating frequency | Design-dependent |
| Duty cycle | Clock duty cycle | 40-60% typical |

### 14.1.2	Figure 4.1: Clock Distribution

Clock Distribution

All masters and slaves must be in the same clock domain as Bridge.

### 14.1.3	No Clock Domain Crossing

Bridge does not include CDC logic:

- All inputs sampled on aclk rising edge

- All outputs generated on aclk rising edge

- External CDC required if masters/slaves use different clocks

## 14.2	Reset Requirements

### 14.2.1	Asynchronous Active-Low Reset

Table 4.5: Reset Signal

| Signal | Description | Polarity |
| --- | --- | --- |
| aresetn | Async reset | Active-low |

### 14.2.2	Reset Behavior

### 14.2.3	Figure 4.3: Reset Timing

Reset Timing

Reset is active-low: the bridge is in reset whenever aresetn = 0.

### 14.2.4	Reset Effects

During reset (aresetn = 0):

Table 4.6: Reset Effects

| Component | State |
| --- | --- |
| Arbiters | Cleared (no grants) |
| ID tables | Cleared (no outstanding) |
| FIFOs | Emptied |
| Outputs | Deasserted (VALID = 0) |

### 14.2.5	Outstanding Transactions

Warning: reset discards transactions in flight:

- Master may see timeout (no response)

- Slave may receive partial transaction

- System must ensure quiescence before reset

### 14.2.6	Reset Release

After reset release (aresetn = 1):

- Bridge accepts new transactions after 1 cycle

- All state machines start in IDLE

- No residual grants or locks

## 14.3	Timing Constraints

### 14.3.1	Setup and Hold

All inputs must meet setup/hold relative to aclk:

Table 4.7: Timing Constraints

| Constraint | Typical Value |
| --- | --- |
| Setup time | Process-dependent |
| Hold time | Process-dependent |

### 14.3.2	Clock-to-Output

All outputs appear after aclk rising edge:

Table 4.8: Clock-to-Output Delays

| Path | Typical Delay |
| --- | --- |
| aclk to VALID | 1 cycle (registered) |
| aclk to READY | Combinational |
| aclk to DATA | 1 cycle (registered) |

### 14.3.3	Recommended Constraints

# Example SDC constraints
create_clock -period 10 [get_ports aclk]  ;# 100 MHz

# Reset is async - synchronize externally
set_false_path -from [get_ports aresetn]

## 14.4	Multi-Clock Systems

### 14.4.1	External CDC Required

For systems with multiple clock domains:

### 14.4.2	Figure 4.2: Multi-Clock CDC

Multi-Clock CDC

### 14.4.3	CDC Recommendations

- Use AXI4 async FIFO bridges

- Consider AXI4 clock converter IP

- Ensure proper gray-coding for pointers

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	AXI5 and APB5 Interfaces (AMBA5 Support)

The bridge remains AMBA4-shaped internally — the crossbar fabric is always AXI4 — but any master or slave port can be declared AMBA5. This chapter covers the external surfaces; the mechanism lives in the MAS (AMBA5 Boundary and Native Sideband).

## 15.1	Declaring AMBA5 Ports

[[bridge.masters]]
name = "cpu_wr"
protocol = "axi5"
axi5_features = ["trace", "atomic"]   # optional; empty = base AXI5
# ... standard fields unchanged

[[bridge.slaves]]
name = "periph5"
protocol = "apb5"                      # APB5 peripheral via the apb5 shim
channels = "rw"                        # APB rules unchanged (rw-only, 32-bit)

## 15.2	AXI5 Port Surface

An axi5 port exposes the AXI4 signal set minus AW/ARREGION (AXI5 removed REGION) plus the enabled features’ sideband signals. Disabled features’ signals are not exposed at all — the boundary wrapper ties them off internally.

| Feature | Signals added | Class |
| --- | --- | --- |
| nsaid | aw/arnsaid[3:0] | Droppable sideband |
| trace | aw/artrace, btrace, rtrace | Droppable sideband |
| mpam | aw/armpam[10:0] | Droppable sideband |
| mecid | aw/armecid[15:0] | Droppable sideband |
| unique | aw/arunique | Droppable sideband |
| poison | wpoison, rpoison | Connectivity-gated |
| atomic | awatop[5:0] | Connectivity-gated (store-class only) |
| mte, chunking | — | Rejected at config time (deferred) |

Droppable sideband passes natively end-to-end when both ends of a path are AXI5, feature-enabled, and width-matched; on any other path it terminates at the fabric boundary with a generation-time warning.

Connectivity-gated features are a config error unless every connected path is native (AXI5 both ends, feature enabled both ends, data widths matched — dwidth converters cannot carry per-beat sideband). Dropping POISON silently would launder corrupted data; dropping ATOP would turn an atomic into a plain write.

Atomics are store-class only. AWATOP = 01xxxx (AtomicStore) and plain writes forward natively. Read-return classes (AtomicLoad 10xxxx, AtomicSwap/Compare 11000x) return their data on the R channel of a path the split-wr/rd fabric cannot route, so the master boundary’s axi5_atomic_filter answers them locally with DECERR — no slave-side AW handshake, no memory side effect.

## 15.3	APB5 Slave Surface

An apb5 slave exposes the APB4 requester surface plus the APB5 sideband, mirroring rtl/amba/apb5/apb5_slave.sv pin-for-pin:

| Direction | Signals |
| --- | --- |
| Requester → completer | APB4 set + PAUSER, PWUSER (driven '0 — nothing upstream sources them) |
| Completer → requester | APB4 set + PWAKEUP, PRUSER, PBUSER (accepted and terminated) |

The transfer protocol is unchanged from APB4, so the axi4_to_apb5_shim is a sideband wrapper over the APB4 conversion core; APB constraints (rw-only, 32-bit data) apply unchanged.

## 15.4	Interop Matrix

| Master Slave | axi4 | axi5 | apb / apb5 / axil |
| --- | --- | --- | --- |
| axi4 | native | base subset | via shim |
| axi5 | sideband drops (warning) | native sideband when width-matched | sideband drops (warning) |

Connectivity-gated features tighten the axi5→axi5 cell: they require it.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Throughput Characteristics

## 16.1	Peak Throughput

### 16.1.1	Single Master to Single Slave

One master driving one slave — no contention, no conversion:

Table 5.1: Peak Throughput by Data Width

| Data Width | Frequency | Peak Throughput |
| --- | --- | --- |
| 32-bit | 100 MHz | 400 MB/s |
| 64-bit | 100 MHz | 800 MB/s |
| 128-bit | 100 MHz | 1.6 GB/s |
| 256-bit | 100 MHz | 3.2 GB/s |
| 512-bit | 100 MHz | 6.4 GB/s |

### 16.1.2	Formula

Peak Throughput = DATA_WIDTH (bits) × Frequency (Hz) / 8 bytes/bit

## 16.2	Sustained Throughput

### 16.2.1	Factors Affecting Throughput

Table 5.2: Factors Affecting Throughput

| Factor | Impact | Mitigation |
| --- | --- | --- |
| Arbitration contention | Reduces per-master throughput | Increase slave ports |
| Width conversion | 1-cycle penalty per direction | Match widths where possible |
| Protocol conversion | 2+ cycles for APB | Use AXI4 for high-bandwidth |
| Response routing | Minimal (pipelined) | N/A |

### 16.2.2	Multi-Master Scaling

With fair round-robin arbitration:

Per-Master Throughput = Peak Throughput / Active Masters (to same slave)

### 16.2.3	Example: 4 Masters, 2 Slaves

All 4 masters accessing Slave 0:
  Per-master = Peak / 4

2 masters to Slave 0, 2 masters to Slave 1:
  Per-master = Peak / 2 (full parallelism)

## 16.3	Burst Efficiency

### 16.3.1	Burst vs Single-Beat

Table 5.3: Burst Efficiency Comparison

| Transaction Type | Overhead | Efficiency |
| --- | --- | --- |
| Single beat | 1 cycle address + 1 cycle data | 50% |
| 4-beat burst | 1 cycle address + 4 cycle data | 80% |
| 16-beat burst | 1 cycle address + 16 cycle data | 94% |
| 256-beat burst | 1 cycle address + 256 cycle data | 99.6% |

### 16.3.2	Recommendation

- Use bursts whenever possible

- AXI4 supports up to 256 beats per burst

- APB does not support bursts (split internally)

## 16.4	Width Conversion Impact

### 16.4.1	Upsize (Narrow to Wide)

64-bit to 512-bit (8:1 ratio):
  Input: 8 beats × 64 bits = 512 bits
  Output: 1 beat × 512 bits = 512 bits

Throughput: Preserved (same data, fewer beats)
Latency: +1 cycle (packing delay)

### 16.4.2	Downsize (Wide to Narrow)

512-bit to 64-bit (8:1 ratio):
  Input: 1 beat × 512 bits = 512 bits
  Output: 8 beats × 64 bits = 512 bits

Throughput: Preserved (same data, more beats)
Latency: +7 cycles (sequential output)

## 16.5	Protocol Conversion Impact

### 16.5.1	AXI4 to APB

Table 5.4: AXI4 vs APB Protocol Impact

| Metric | AXI4 Direct | AXI4 to APB |
| --- | --- | --- |
| Min cycles/transfer | 1 | 2 |
| Burst support | Yes | No (split) |
| Pipeline depth | 1+ | 1 |
| Peak throughput | DATA_WIDTH/cycle | DATA_WIDTH/2 cycles |

### 16.5.2	Recommendation

- Keep high-bandwidth traffic on AXI4 paths

- Use APB only for low-bandwidth peripherals

- Group APB slaves to avoid impacting AXI4 traffic

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Latency Analysis

## 17.1	Latency Components

### 17.1.1	Address Path Latency

Table 5.5: Address Path Latency

| Stage | Cycles | Notes |
| --- | --- | --- |
| Master Adapter (skid) | 1 | Pipeline registration |
| Address Decode | 0 | Combinational |
| Arbitration | 0-1 | 0 if no contention, 1 if arbitrate |
| Slave Router | 1 | Pipeline registration |
| Total Address | 2-3 | Best/worst case |

### 17.1.2	Data Path Latency

Table 5.6: Data Path Latency

| Stage | Cycles | Notes |
| --- | --- | --- |
| W follows AW | 0 | Same grant |
| Width conversion | 0-1 | 0 if match, 1 if convert |
| Protocol conversion | 0-2+ | 0 for AXI4, 2+ for APB |
| Total Data | 0-3+ | Best/worst case |

### 17.1.3	Response Path Latency

Table 5.7: Response Path Latency

| Stage | Cycles | Notes |
| --- | --- | --- |
| Slave response | Variable | Slave-dependent |
| ID extraction | 0 | Combinational |
| Response routing | 1 | Pipeline registration |
| Master delivery | 0 | Direct connection |
| Total Response | 1 + slave | Plus slave latency |

## 17.2	End-to-End Latency

### 17.2.1	Write Transaction (Best Case)

Cycle 0: AW arrives at Bridge
Cycle 1: AW exits to Slave (skid buffer)
Cycle 2: Slave receives AW
Cycle 1: W arrives (same cycle as AW skid)
Cycle 2: W exits to Slave
Cycle 3: Slave receives W, generates B
Cycle 4: B routed back to Master

Total: 4 cycles (minimum)

### 17.2.2	Read Transaction (Best Case)

Cycle 0: AR arrives at Bridge
Cycle 1: AR exits to Slave (skid buffer)
Cycle 2: Slave receives AR, generates R
Cycle 3: R routed back to Master

Total: 3 cycles (minimum)

### 17.2.3	Contention Latency

When multiple masters contend for same slave:

Arbitration adds 1 cycle per contending master

Example: 4 masters requesting same slave
  Worst case: Wait for 3 other masters = 3 extra cycles
  Average case: Wait for 1.5 masters = 1-2 extra cycles

## 17.3	Width Conversion Latency

### 17.3.1	Upsize Latency

Table 5.8: Upsize Latency by Ratio

| Ratio | Extra Cycles | Notes |
| --- | --- | --- |
| 1:1 | 0 | No conversion |
| 2:1 | 0 | Pack on fly |
| 4:1 | 0 | Pack on fly |
| 8:1 | 0-1 | May need accumulation |

### 17.3.2	Downsize Latency

Table 5.9: Downsize Latency by Ratio

| Ratio | Extra Cycles | Notes |
| --- | --- | --- |
| 1:1 | 0 | No conversion |
| 1:2 | +1 | 2 output beats |
| 1:4 | +3 | 4 output beats |
| 1:8 | +7 | 8 output beats |

## 17.4	Protocol Conversion Latency

### 17.4.1	AXI4 to APB

Table 5.10: AXI4 to APB Conversion Latency

| Phase | Cycles |
| --- | --- |
| AXI4 AW decode | 1 |
| APB setup | 1 |
| APB access | 1+ (PREADY) |
| B generation | 1 |
| Total | 4+ cycles |

### 17.4.2	APB Wait States

APB slaves may insert wait states:

PREADY deasserted: +1 cycle per wait
Typical UART/GPIO: 0-2 wait states
Slow peripherals: May have many wait states

## 17.5	Latency Optimization

### 17.5.1	Design Recommendations

- Match data widths - Avoid conversion latency

- Use AXI4 for fast paths - Avoid APB latency

- Minimize contention - Spread traffic across slaves

- Use bursts - Amortize address latency

- Pipeline responses - Don’t block on slow slaves

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Resource Estimates

## 18.1	FPGA Resource Summary

### 18.1.1	Baseline Configuration (2x2, 64-bit)

Table 5.11: Baseline Resource Requirements

| Resource | Count | Notes |
| --- | --- | --- |
| LUTs | ~2,000-3,000 | Logic and routing |
| Registers | ~1,500-2,000 | Pipeline stages |
| Block RAM | 0-1 | Optional ID CAM |
| DSP | 0 | No arithmetic |

### 18.1.2	Scaling Factors

Table 5.12: Resource Scaling Factors

| Component | Scaling |
| --- | --- |
| Crossbar core | O(M x N) |
| Master adapters | O(M) |
| Slave routers | O(N) |
| ID tracking | O(M x Outstanding) |
| Width converters | O(per-path ratio) |

## 18.2	Component Breakdown

### 18.2.1	Per-Master Resources

Table 5.13: Per-Master Resource Breakdown

| Component | LUTs | Registers |
| --- | --- | --- |
| Skid buffer | ~50-100 | ~DATA_WIDTH |
| ID extension | ~20 | ~ID_WIDTH |
| Channel mux | ~100 | ~50 |
| Per Master Total | ~200-300 | ~DATA_WIDTH + 100 |

### 18.2.2	Per-Slave Resources

Table 5.14: Per-Slave Resource Breakdown

| Component | LUTs | Registers |
| --- | --- | --- |
| Address decode | ~50 | 0 |
| Arbiter | ~100-200 | ~50 |
| Response mux | ~100 | ~50 |
| Per Slave Total | ~250-350 | ~100 |

### 18.2.3	Crossbar Core

Table 5.15: Crossbar Core Resources

| Component | LUTs | Registers |
| --- | --- | --- |
| AW mux (N-way) | ~100 x N | ~50 x N |
| W mux (N-way) | ~100 x N | ~50 x N |
| B demux (M-way) | ~50 x M | ~50 x M |
| AR mux (N-way) | ~100 x N | ~50 x N |
| R demux (M-way) | ~50 x M | ~50 x M |

## 18.3	Width Converter Resources

Table 5.16: Width Converter Resources

| Ratio | LUTs | Registers | Notes |
| --- | --- | --- | --- |
| 1:2 upsize | ~200 | ~DATA_IN | Pack logic |
| 1:4 upsize | ~300 | ~DATA_IN | Pack logic |
| 1:8 upsize | ~400 | ~DATA_IN | Pack logic |
| 2:1 downsize | ~200 | ~DATA_OUT | Split logic |
| 4:1 downsize | ~300 | ~DATA_OUT | Split logic |
| 8:1 downsize | ~400 | ~DATA_OUT | Split logic |

## 18.4	Protocol Converter Resources

### 18.4.1	AXI4 to APB

Table 5.17: AXI4 to APB Converter Resources

| Component | LUTs | Registers |
| --- | --- | --- |
| FSM | ~100 | ~20 |
| Burst counter | ~50 | ~8 |
| Data buffer | ~100 | ~DATA_WIDTH |
| Response logic | ~50 | ~10 |
| Total | ~300 | ~DATA_WIDTH + 50 |

## 18.5	Example Configurations

Table 5.18: Complete Bridge Resource Estimates

| Config | Masters | Slaves | Data | LUTs | Registers |
| --- | --- | --- | --- | --- | --- |
| 2x2 Basic | 2 | 2 | 64 | ~2,500 | ~1,500 |
| 4x4 Standard | 4 | 4 | 64 | ~5,000 | ~3,000 |
| 4x4 Wide | 4 | 4 | 256 | ~8,000 | ~5,000 |
| 8x8 Large | 8 | 8 | 128 | ~12,000 | ~8,000 |
| RAPIDS (4x3) | 4 | 3 | 512 | ~10,000 | ~6,000 |

### 18.5.1	Notes

- Estimates include all converters and adapters

- Actual results vary by synthesis tool and FPGA family

- Block RAM usage depends on ID table implementation

- DSP usage is zero (no multiplication/division)

## 18.6	Optimization Strategies

### 18.6.1	Resource Reduction

- Use channel-specific masters - 40-60% port reduction

- Match data widths - Avoid converter logic

- Use APB sparingly - Reduces AXI4 overhead

- Limit outstanding transactions - Smaller ID tables

### 18.6.2	Performance/Resource Trade-off

Table 5.19: Performance/Resource Trade-offs

| Feature | Resource Impact | Performance Impact |
| --- | --- | --- |
| Deeper skid buffers | +registers | +timing margin |
| Larger ID tables | +BRAM | +OOO depth |
| Pipeline stages | +registers | +frequency |
| Wider data paths | +routing | +throughput |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	System Requirements

## 19.1	Hardware Requirements

### 19.1.1	Clock Infrastructure

Table 6.1: Clock Requirements

| Requirement | Specification |
| --- | --- |
| Clock source | Single clock domain (aclk) |
| Clock quality | Low jitter, stable frequency |
| Clock distribution | Must reach all Bridge ports |

### 19.1.2	Reset Infrastructure

Table 6.2: Reset Requirements

| Requirement | Specification |
| --- | --- |
| Reset type | Asynchronous, active-low (aresetn) |
| Reset synchronization | External synchronizer recommended |
| Reset distribution | Must reach all Bridge ports |

### 19.1.3	Power

Table 6.3: Power Requirements

| Requirement | Specification |
| --- | --- |
| Power domains | Single domain (no power gating) |
| Voltage | FPGA/ASIC process-dependent |

## 19.2	Interface Requirements

### 19.2.1	Master Requirements

Masters connecting to Bridge must:

- Comply with AXI4 protocol - Valid/ready handshake

- Use correct ID width - Match configured ID_WIDTH

- Use correct data width - Match configured DATA_WIDTH

- Use correct address width - Match configured ADDR_WIDTH

- Stay within address range - Target valid slave addresses

### 19.2.2	Slave Requirements

Slaves connecting to Bridge must:

- Comply with AXI4/APB protocol - Based on configuration

- Accept extended IDs - ID_WIDTH + BID_WIDTH

- Match configured data width - Or use width conversion

- Respond to all transactions - No hanging

## 19.3	Configuration Requirements

### 19.3.1	Address Map

Table 6.4: Address Map Requirements

| Requirement | Specification |
| --- | --- |
| Slave ranges | Must not overlap |
| Range alignment | Power-of-2 recommended |
| Coverage | All master addresses must map to slaves or OOR |

### 19.3.2	Connectivity

Table 6.5: Connectivity Requirements

| Requirement | Specification |
| --- | --- |
| Matrix completeness | All masters must access at least one slave |
| Reachability | Consider traffic patterns for performance |

## 19.4	Timing Requirements

### 19.4.1	Setup/Hold

Table 6.6: Timing Requirements

| Constraint | Requirement |
| --- | --- |
| Input setup | Meet FPGA/ASIC requirements |
| Input hold | Meet FPGA/ASIC requirements |
| Clock-to-output | Per process characterization |

### 19.4.2	Recommended Margins

Table 6.7: Recommended Timing Margins

| Margin | Value | Purpose |
| --- | --- | --- |
| Setup margin | 10-20% | Variation tolerance |
| Hold margin | Positive | No hold violations |
| Clock uncertainty | Include in constraints | PLL/MMCM jitter |

## 19.5	Verification Requirements

### 19.5.1	Pre-Integration Checks

- RTL lint clean - No Verilator warnings

- Parameter validation - All parameters in valid range

- Address map review - No overlaps, complete coverage

- ID width calculation - Sufficient for NUM_MASTERS

### 19.5.2	Integration Checks

- Signal connectivity - All ports connected

- Width matching - DATA_WIDTH, ADDR_WIDTH, ID_WIDTH

- Protocol matching - AXI4 vs APB correctly configured

- Clock/reset connectivity - All ports share same domain

### 19.5.3	Post-Integration Checks

- Basic transaction test - Read/write to each slave

- Arbitration test - Multi-master contention

- Error handling test - OOR address response

- Performance validation - Meet throughput targets

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Parameter Configuration

## 20.1	Core Parameters

Table 6.8: Bridge Core Parameters

| Parameter | Type | Range | Default | Description |
| --- | --- | --- | --- | --- |
| NUM_MASTERS | int | 1-32 | 2 | Number of master ports |
| NUM_SLAVES | int | 1-256 | 2 | Number of slave ports |
| ADDR_WIDTH | int | 12-64 | 32 | Address bus width |
| DATA_WIDTH | int | 32-512 | 64 | Data bus width |
| ID_WIDTH | int | 1-16 | 4 | Master ID width |
| USER_WIDTH | int | 1-16 | 1 | User signal width |

## 20.2	Derived Parameters

### 20.2.1	Calculated by Generator

Table 6.9: Derived Parameters

| Parameter | Formula | Example |
| --- | --- | --- |
| BID_WIDTH | clog2(NUM_MASTERS) | 4 masters = 2 bits |
| TOTAL_ID_WIDTH | ID_WIDTH + BID_WIDTH | 4 + 2 = 6 bits |
| STRB_WIDTH | DATA_WIDTH / 8 | 64b = 8 strobes |

## 20.3	Per-Port Configuration

### 20.3.1	Master Port Configuration

Table 6.10: Master Port Configuration

| Field | Type | Description |
| --- | --- | --- |
| name | string | Unique identifier |
| prefix | string | Signal prefix |
| channels | enum | “rw”, “wr”, or “rd” |
| data_width | int | Port data width |
| id_width | int | Port ID width |
| use_monitor | bool | Mandatory. Enable per-port monitor wrappers (true/false) |

### 20.3.2	Slave Port Configuration

Table 6.11: Slave Port Configuration

| Field | Type | Description |
| --- | --- | --- |
| name | string | Unique identifier |
| prefix | string | Signal prefix |
| protocol | enum | “axi4”, “axi4lite”, “apb” |
| data_width | int | Port data width |
| base_addr | hex | Base address (must be 4K-aligned) |
| addr_range | hex | Address range size (must be multiple of 4K) |
| use_monitor | bool | Mandatory. Enable per-port monitor wrappers (true/false) |

#### 20.3.2.1	Slave Address Window Rules (MANDATORY)

Every slave must occupy a 4K-aligned address window that is a multiple of 4 KB:

Rule 1: Base Address Alignment
  base_addr & 0xFFF == 0x000
  
  Valid:   0x00000000, 0x00001000, 0x80000000, 0xFFFF0000
  Invalid: 0x00000001, 0x80000800, 0xC0000100

Rule 2: Range Alignment
  addr_range % 0x1000 == 0
  
  Valid:   0x1000 (4 KB), 0x2000 (8 KB), 0x10000 (64 KB)
  Invalid: 0x800 (2 KB), 0x1001, 0x7FFF

Rule 3: Non-Overlapping Windows
  All slaves must have non-overlapping address ranges
  
  Valid:   Slave0: 0x00000000-0x0FFFFFFF, Slave1: 0x10000000-0x1FFFFFFF
  Invalid: Slave0: 0x00000000-0x10000000, Slave1: 0x0FFFFFFF-0x1FFFFFFF

Why This Rule Exists: - Real memory systems use 4K page boundaries (virtual memory, MMU) - Linker scripts and memory maps assume 4K granularity - Address decoders simplify to bit masks with 4K alignment - Prevents ambiguity in address decode logic

## 20.4	Top-Level Monitor Configuration

When monitor collection is desired, the bridge TOML configuration includes:

| Field | Type | Description |
| --- | --- | --- |
| variants | list | List of bridge variants to generate. Supported: ["no"] (no monitor), ["mon"] (monitor only), ["no", "mon"] (both). Default: ["no"] |

### 20.4.1	Monitor Identifiers

Each monitored port gets a unique (UNIT_ID, AGENT_ID) pair for identification in monitor packets:

UNIT_ID Assignment:
  - UNIT_ID = 1: Master-side monitor wrappers (axi4_master_{rd,wr}_mon)
  - UNIT_ID = 2: Slave-side monitor wrappers (axi4_slave_{rd,wr}_mon)

AGENT_ID Assignment (per port):
  - AGENT_ID = (port_index << 4) | channel_bit
  - channel_bit: 0 = read channel (AR/R), 1 = write channel (AW/W/B)
  - Example: Master port 2, write direction → AGENT_ID = (2 << 4) | 1 = 0x21

### 20.4.2	AXIL→Wider-Slave Master-Side Alignment

When an AXI4-Lite master connects to a wider AXI4 slave through the bridge (e.g., 32-bit AXIL master to 64-bit AXI4 slave), the generator emits a master-side alignment converter (the axil_to_axi4_wide_align_{rd,wr} modules) between the master adapter and the crossbar core. The converter handles partial-word alignment on the narrow side while preserving AXIL’s single-beat semantics; the protocol stays AXIL — protocol shims are a slave-boundary concern, applied separately.

## 20.5	Configuration File Format

### 20.5.1	TOML Configuration

[bridge]
  name = "my_bridge"
  description = "Example bridge configuration"

  defaults.skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

  masters = [
    {name = "cpu", prefix = "cpu_m_axi", channels = "rw",
     id_width = 4, addr_width = 32, data_width = 64, user_width = 1},
    {name = "dma", prefix = "dma_m_axi", channels = "rw",
     id_width = 4, addr_width = 32, data_width = 256, user_width = 1}
  ]

  slaves = [
    {name = "ddr", prefix = "ddr_s_axi", protocol = "axi4",
     id_width = 6, addr_width = 32, data_width = 512, user_width = 1,
     base_addr = 0x80000000, addr_range = 0x40000000},
    {name = "uart", prefix = "uart_apb", protocol = "apb",
     addr_width = 12, data_width = 32,
     base_addr = 0x00000000, addr_range = 0x00001000}
  ]

### 20.5.2	CSV Connectivity

master\slave,ddr,uart
cpu,1,1
dma,1,0

## 20.6	Parameter Validation

### 20.6.1	Generator Checks

Table 6.12: Generator Validation Checks

| Check | Error Condition |
| --- | --- |
| NUM_MASTERS | < 1 or > 32 |
| NUM_SLAVES | < 1 or > 256 |
| DATA_WIDTH | Not power of 2 |
| Address overlap | Slave ranges intersect |
| Connectivity | Master with no slaves |
| ID width | Insufficient for masters |

### 20.6.2	Runtime Validation

Bridge includes optional assertions for:

- Address alignment

- Burst boundary crossing

- Protocol violations

- ID mismatch

## 20.7	Example Configurations

### 20.7.1	Simple 2x2

[bridge]
  name = "bridge_simple"
  masters = [
    {name = "m0", prefix = "m0_axi", data_width = 64},
    {name = "m1", prefix = "m1_axi", data_width = 64}
  ]
  slaves = [
    {name = "s0", prefix = "s0_axi", base_addr = 0x0, addr_range = 0x10000000},
    {name = "s1", prefix = "s1_axi", base_addr = 0x10000000, addr_range = 0x10000000}
  ]

### 20.7.2	Mixed Protocol

[bridge]
  name = "bridge_mixed"
  masters = [
    {name = "cpu", prefix = "cpu_axi", data_width = 64, channels = "rw"}
  ]
  slaves = [
    {name = "mem", prefix = "mem_axi", protocol = "axi4", data_width = 512},
    {name = "gpio", prefix = "gpio_apb", protocol = "apb", data_width = 32},
    {name = "uart", prefix = "uart_apb", protocol = "apb", data_width = 32}
  ]

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Verification Strategy

## 21.1	Verification Levels

### 21.1.1	Unit Level

Test individual Bridge components:

Table 6.13: Unit Level Test Focus

| Component | Test Focus |
| --- | --- |
| Master Adapter | Skid buffer, ID extension |
| Slave Router | Address decode, arbitration |
| Width Converter | Upsize/downsize correctness |
| Protocol Converter | AXI4 to APB timing |
| Response Router | ID extraction, routing |

### 21.1.2	Integration Level

Test complete Bridge functionality:

Table 6.14: Integration Test Coverage

| Test Category | Coverage |
| --- | --- |
| Address routing | All master-slave paths |
| Arbitration | Contention, fairness |
| ID tracking | Response routing |
| Error handling | OOR, timeouts |
| Performance | Throughput, latency |

### 21.1.3	System Level

Test Bridge in target system:

Table 6.15: System Level Tests

| Test Category | Focus |
| --- | --- |
| End-to-end | CPU to memory |
| Multi-master | DMA + CPU |
| Mixed protocol | AXI4 + APB |
| Stress | Maximum traffic |

## 21.2	Test Infrastructure

### 21.2.1	CocoTB Framework with Protocol-BFM-Only Driving

Bridge verification runs on CocoTB with protocol BFMs only: all test stimulus is driven through protocol BFMs (AXI4Master, AXI4Slave, etc.) with no direct DUT signal manipulation. Each slave port is backed by an in-memory model (MemoryModel), and checking is assertion-based (readback matches write) rather than routing-based.

from CocoTBFramework.components.axi4 import AXI4Master, AXI4Slave
from TBClasses.shared.tbbase import TBBase

class BridgeTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)
        self.masters = [AXI4Master(dut, dut.clk, f"m{i}_axi") for i in range(NUM_MASTERS)]
        self.slaves = [MemoryModel(dut, dut.clk, f"s{i}_axi") for i in range(NUM_SLAVES)]

@cocotb.test()
async def cocotb_test_basic_write(dut):
    tb = BridgeTB(dut)
    await tb.setup_clocks_and_reset()

    # Write transaction via BFM only
    await tb.masters[0].write(addr=0x1000, data=0xDEADBEEF)
    
    # Verify: readback matches write (memory-model assertion)
    readback = await tb.masters[0].read(addr=0x1000)
    assert readback == 0xDEADBEEF

### 21.2.2	Test Categories

Table 6.16: Test Categories

| Category | Description | Example |
| --- | --- | --- |
| Basic | Single transactions | Read/write |
| Burst | Multi-beat transactions | 16-beat burst |
| Concurrent | Multiple masters active | M0 + M1 |
| Boundary Probe | Address window edges | Top/mid/bottom of each slave page |
| Stress | Maximum utilization | Back-to-back |
| Error | Error conditions | SLVERR injection, timeout |
| Monitor | Monitor packet collection | Packet capture, IRQ assertion |

## 21.3	Test Execution Model

### 21.3.1	Serial Execution

Bridge tests run serially only (no parallel execution via pytest-xdist). Each test function is named per-module to avoid cross-test cocotb function name pollution:

# Naming convention: test_bridge_{N}x{M}_{variant}_{scenario}
def test_bridge_1x2_rd_basic(request):        # Test name indicates 1 master, 2 slaves, read-only
def test_bridge_1x2_rd_monitor_smoke(request): # Monitor test on 1x2
def test_bridge_4x4_rw_concurrent(request):    # Concurrent test on 4x4

Cocotb test functions inside each file follow the same naming pattern but with cocotb_test_* prefix to avoid pytest collection conflicts.

### 21.3.2	Boundary Probe Testing

Address-window boundary testing probes the top, middle, and bottom of each slave’s address window to catch address-decode logic errors at window edges:

for slave_idx, (base, window_size) in enumerate(slave_windows):
    # Probe bottom (base address)
    await master.write(base + 0x000, data)
    
    # Probe middle (arbitrary address within window)
    await master.write(base + window_size // 2, data)
    
    # Probe top (base + window_size - 1)
    await master.write(base + window_size - 1, data)

## 21.4	Coverage Goals

### 21.4.1	Functional Coverage

Table 6.17: Functional Coverage Goals

| Category | Target |
| --- | --- |
| Address paths | 100% master-slave combinations via boundary probes |
| Transaction types | Read, write, burst, error (SLVERR) |
| Monitor modes | Enabled and disabled |
| ID values | Full ID range (representative) |

### 21.4.2	Code Coverage

Table 6.18: Code Coverage Goals

| Metric | Target |
| --- | --- |
| Line coverage | >95% |
| Branch coverage | >90% |
| FSM coverage | 100% state transitions |
| Toggle coverage | >80% signals |

## 21.5	Debug Features

### 21.5.1	Waveform Capture

Bridge tests support VCD/FST waveform dumps:

pytest test_bridge.py --vcd=waves.vcd
gtkwave waves.vcd

### 21.5.2	Debug Signals

Bridge includes optional debug signals:

Table 6.19: Debug Signals

| Signal | Description |
| --- | --- |
| dbg_aw_grant | Current AW grant |
| dbg_ar_grant | Current AR grant |
| dbg_outstanding | Outstanding transaction count |
| dbg_state | FSM states |

## 21.6	Regression Testing

### 21.6.1	Continuous Integration

Table 6.20: CI Test Schedule

| Stage | Tests | Frequency |
| --- | --- | --- |
| Pre-commit | Basic sanity | Each commit |
| Nightly | Full regression | Daily |
| Weekly | Performance | Weekly |

### 21.6.2	Test Matrix

Table 6.21: Configuration Test Matrix

| Config | Data Width | Protocol | Tested |
| --- | --- | --- | --- |
| 2x2 | 64 | AXI4 | Yes |
| 4x4 | 64 | AXI4 | Yes |
| 4x4 | 256 | AXI4 | Yes |
| 2x2 | 64 | AXI4+APB | Yes |
| RAPIDS | 512 | AXI4+APB | Yes |

## 21.7	Known Limitations

### 21.7.1	Test Gaps

Table 6.22: Known Test Gaps

| Gap | Status | Plan |
| --- | --- | --- |
| APB burst stress | Partial | Phase 3 |
| Width downsize stress | Partial | Planned |
| 32-master config | Not tested | Low priority |

### 21.7.2	Simulator Support

Table 6.23: Simulator Support Status

| Simulator | Status |
| --- | --- |
| Verilator | Fully supported |
| Icarus Verilog | Supported |
| Commercial (VCS, Questa) | Not tested |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Bridge Micro-Architecture Specification Index

Component: Bridge (Multi-Protocol Crossbar Generator) Version: 1.1 Date: 2026-06-04 Purpose: Detailed micro-architecture specification for Bridge component

## 22.1	Document Organization

The Bridge component is a CSV-driven generator that produces AXI4 crossbars with automatic width conversion, protocol conversion (AXI4, AXI4-Lite, APB), and channel-specific masters (read-only, write-only, or full). Each chapter below is one source file.

### 22.1.1	Front Matter

- Document Information

### 22.1.2	Chapter 1: Introduction

- Overview

### 22.1.3	Chapter 2: Block Descriptions

- Master Adapter

- Slave Router

- Crossbar Core

- Arbitration

- ID Management

- Width Conversion

- Protocol Conversion

- Response Routing

- Error Handling

- AMBA5 Boundary and Native Sideband

### 22.1.4	Chapter 3: FSM Design

- Arbiter FSMs

- Transaction Tracking

### 22.1.5	Chapter 4: ID Management

- CAM Architecture

- ID Tracking Tables

### 22.1.6	Chapter 5: Converters

- Width Converters

- APB Converters

### 22.1.7	Chapter 6: Generated RTL

- Module Structure

- Signal Naming

### 22.1.8	Chapter 7: Verification

- Test Strategy

- Debug Guide

## 22.2	Quick Navigation

### 22.2.1	For New Users

- Start with Overview for a tour of what Bridge can do

- Read Block Diagram for the architecture

- Study Arbitration for operational details

- Reference Module Structure for generated RTL

### 22.2.2	For Integration

- Protocol support: See Protocol Conversion for AXI4/APB/AXI-Lite

- Width handling: See Width Conversion for data width mismatches

- ID management: See ID Management for transaction tracking

- Error handling: See Error Handling for OOR and timeout

## 22.3	Visual Assets

Diagram sources and their renders live in two directories:

- Source Files:

  - assets/graphviz/*.gv - Graphviz source diagrams

  - assets/puml/*.puml - PlantUML FSM diagrams

- Rendered Files:

  - assets/graphviz/*.png - Rendered block diagrams

  - assets/puml/*.png - Rendered FSM diagrams

### 22.3.1	Architecture Diagrams

- Master Adapter - assets/graphviz/master_adapter.png

- Slave Router - assets/graphviz/slave_router.png

- Crossbar Core - assets/graphviz/crossbar_core.png

- ID Management - assets/graphviz/id_management.png

- Width Conversion - assets/graphviz/width_conversion.png

- Protocol Conversion - assets/graphviz/protocol_conversion_apb.png

- Response Routing - assets/graphviz/response_routing.png

- Error Handling - assets/graphviz/error_handling.png

### 22.3.2	FSM Diagrams

- AW Arbiter FSM - assets/puml/aw_arbiter_fsm.png

- AR Arbiter FSM - assets/puml/ar_arbiter_fsm.png

## 22.4	Component Overview

### 22.4.1	Key Features

- Multi-Protocol Support: AXI4, AXI4-Lite, and APB slave conversion

- CSV-Driven Configuration: Human-readable port and connectivity definitions

- Channel-Specific Masters: Write-only (wr), read-only (rd), or full (rw) support

- Automatic Width Conversion: Upsize/downsize for data width mismatches

- Out-of-Order Support: ID-based response routing with CAM tracking

- Custom Signal Prefixes: Unique prefixes per port for clean integration

### 22.4.2	Protocol Conversion Matrix

What happens to a transaction depends on the master/slave protocol pairing:

| Master Protocol | Slave Protocol | Conversion |
| --- | --- | --- |
| AXI4 | AXI4 | Direct or width convert |
| AXI4 | AXI4-Lite | Protocol downgrade |
| AXI4 | APB | Full protocol conversion |

### 22.4.3	Design Philosophy

Efficient Multi-Width Routing: - Direct connections where widths match (zero conversion overhead) - Width converters only where needed - No fixed internal crossbar width

Resource Optimization: - Channel-specific masters reduce port count by 40-60% - Per-path width converters instead of global conversion - Minimal logic for matching-width connections

## 22.5	Related Documentation

### 22.5.1	Companion Specifications

- Bridge HAS - Hardware Architecture Specification (high-level)

### 22.5.2	Project-Level

- PRD.md: ../../PRD.md - Complete product requirements document

- CLAUDE.md: ../../CLAUDE.md - AI assistant integration guide

### 22.5.3	Generator

- Generator: ../../bin/bridge_generator.py - CSV/TOML-based generator script (with ../../bin/bridge_pkg/)

- Test Configs: ../../bin/test_configs/ - Example TOML/CSV configurations

## 22.6	Version History

Version 1.0 (2026-01-03): - Initial specification release - Restructured from single spec to HAS/MAS format - Complete block-level documentation - FSM and ID management details

Last Updated: 2026-01-03 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Bridge: AXI4 Full Crossbar Generator - Product Requirements Document

Project: Bridge Version: 2.1 Status: Phase 2 Complete - Simplified Architecture with Hard Limits Created: 2025-10-18 Last Updated: 2025-11-02

## 23.1	Executive Summary

Bridge is a Python-based AXI4 crossbar generator that produces simple, performant SystemVerilog RTL for connecting multiple AXI4 masters to multiple AXI4 slaves. The name follows the infrastructure theme - bridges connect different regions, enabling communication across divides, just like crossbars connect masters and slaves.

Design Philosophy: A simple AMBA fabric that is performant, but makes no attempt to support all features. We enforce hard limits (8-bit ID width, 64-bit address width) to eliminate unnecessary complexity, focusing only on what real hardware needs.

Key Differentiator from Delta: - Delta: AXI-Stream crossbar (streaming data, single channel, simple routing) - Bridge: AXI4 full crossbar (memory-mapped, 5 channels, burst support, ID-based routing)

Key Differentiator from Commercial IP: - Commercial AXI4 Crossbars: Feature-complete, support every AXI4 edge case, complex configurability - Bridge: Simple and performant, supports common use cases, hard limits for simplicity

Target Use Case: High-performance memory-mapped interconnects for multi-core processors, accelerators, and memory controllers where simplicity and performance matter more than spec compliance.

## 23.2	Design Philosophy

Vision: A simple AMBA fabric that is performant, but makes no attempt to support all features.

### 23.2.1	Core Principles

1. Simplicity Over Completeness - Focus on common use cases, not edge cases - Implement what’s needed for real hardware, not AXI4 spec compliance theater - Prefer straightforward implementations over complex feature sets

2. Performance First - Direct connections where possible (matching widths = zero overhead) - Minimal conversion logic in critical paths - Optimize for throughput and latency, not feature count

3. Hard Limits for Simplicity

We enforce uniform widths on certain parameters to eliminate complexity:

| Parameter | Hard Limit | Rationale |
| --- | --- | --- |
| ID Width | 8 bits (fixed) | Eliminates ID width conversion logic. 256 unique IDs is sufficient for all realistic use cases. |
| Address Width | 64 bits (fixed) | Eliminates address width conversion. 64-bit addressing covers all memory-mapped systems. |
| Data Width | Variable (32b-512b) | Width converters ONLY for data. Commonly needed for bandwidth optimization. |

Why This Works: - ID width conversion adds complexity with minimal benefit (nobody needs >256 outstanding transactions per master) - Address width conversion is rarely needed (peripheral addresses fit in 32-bit, but using 64-bit everywhere is simpler) - Data width conversion is the ONLY width conversion that provides real value (bandwidth matching)

4. What We DON’T Support (By Design)

Features intentionally excluded for simplicity: - No variable ID width per port - No variable address width per port - No AXI4-Lite protocol variant (use standard AXI4 with len=0) - No ACE protocol extensions (cache coherency) - No AXI5 features - No complete AXI4 sideband signal support (QoS, Region, User signals declared but not routed)

5. What We DO Support

Core AXI4 features that matter: - Full 5-channel AXI4 protocol (AW, W, B, AR, R) - Burst transactions (INCR, WRAP, FIXED) - Out-of-order completion via transaction IDs - Multiple outstanding transactions per master - Channel-specific masters (write-only, read-only, read-write) - Data width conversion (32b ↔ 64b ↔ 128b ↔ 256b ↔ 512b) - Configurable M×N topology (1-32 masters, 1-256 slaves) - Fair round-robin arbitration per slave

### 23.2.2	Architecture Philosophy

Target: Intelligent width-aware routing, not fixed-width crossbar

OLD Approach (What We’re Moving Away From):

Master (64b) → Upsize(64→256) → Fixed 256b Crossbar → Downsize(256→64) → Slave (64b)

- Two conversions for same-width connections (wasteful!)

- Artificial bandwidth bottleneck

- Unnecessary logic and latency

NEW Approach (Target Architecture):

Master (64b) → Direct Connection → Slave (64b)    [0 conversions!]
Master (512b) → Conv(512→64) → Slave (64b)         [1 conversion]

- Per-master paths to each unique slave width it connects to

- Direct paths where widths match (zero overhead)

- Converters only where actually needed

- Router selects appropriate path based on address decode

Result: Minimal conversion logic, maximum performance, pragmatic simplicity.

## 23.3	CRITICAL: RTL Regeneration Requirements

ALL generated RTL files MUST be deleted and regenerated together whenever ANY generator code changes.

Why This Matters: - Generated RTL files may have interdependencies (bridges, wrappers, integrators) - Generator code changes can create version mismatches between files - Partial regeneration creates subtle incompatibilities that cause test failures - Even “small innocuous” generator changes can have cascading effects

The Rule:

# WRONG - Partial regeneration
./bridge_generator.py --masters 5 --slaves 3 --output ../rtl/
# Only regenerates bridge_axi4_flat_5x3.sv
# Other files (wrappers, integrators) now mismatched!

# CORRECT - Full regeneration
cd bin
make clean                               # Delete ALL generated bridges (rtl/generated/)
make all                                 # Regenerate everything from bridge_batch.csv

Generator Files That Trigger Full Regeneration: - bin/bridge_generator.py - Main bridge generator (CSV/TOML driven) - Any Python file in bin/bridge_pkg/ (components/, generators/, config, csv_parser, …) - Any Jinja template in bin/bridge_pkg/jinja_templates/ - bridge_wrapper_generator.py - Wrapper generation - Any Python file in projects/components/bridge/bin/

Symptoms of Version Mismatch: - Tests that previously passed now fail - Simulation errors about missing signals - Mismatched port widths or counts - Address decode routing to wrong slaves

Think of Generated RTL Like Compiled Code: When you update a compiler, you don’t selectively recompile - you rebuild everything. When you update a generator, you don’t selectively regenerate - you regenerate everything.

## 23.4	1. Product Overview

### 23.4.1	1.1 Purpose

Bridge automates generation of AXI4 crossbar infrastructure: - Python code generation - Parameterized RTL generation (similar to APB/Delta) - Performance modeling - Analytical + simulation validation - Flat topology - Full M×N interconnect matrix - ID-based routing - Out-of-order transaction support - Burst optimization - Pipelined burst transfers

### 23.4.2	1.2 Target Audience

Primary Users: - RTL designers building SoC interconnect - System architects evaluating interconnect topologies - Verification engineers needing crossbar testbenches - Students learning AXI4 protocol and interconnects

Educational Focus: - Demonstrates AXI4 protocol complexity - Shows arbitration strategies for memory-mapped busses - Teaches ID-based transaction tracking - Illustrates burst optimization techniques

### 23.4.3	1.3 Success Criteria

Functional: - [x] Generates working AXI4 crossbar RTL (bridge_generator.py) - [x] Generates CSV/TOML-configured bridges (bridge_generator.py + bridge_pkg/; the earlier separate bridge_csv_generator.py was merged in) - [x] Passes Verilator lint - [x] Supports 1-32 masters, 1-256 slaves - [x] Handles out-of-order completion via IDs (bridge_cam.sv) - [x] Supports burst lengths 1-256 beats - [x] Channel-specific masters (wr/rd/rw) for resource optimization (Phase 2) - [ ] APB converter integration (Phase 3 pending)

Performance: - [x] Latency ≤ 3 cycles for single-beat transactions - [x] Throughput: All M×N paths can transfer concurrently - [x] Performance models implemented (bridge_model.py - V1 Flat) - [ ] Fmax ≥ 300 MHz on UltraScale+ FPGAs (pending synthesis validation)

Quality: - [x] Complete specifications (PRD) before code - [x] Performance models validate requirements (bridge_model.py) - [x] All generated RTL Verilator verified - [x] Integration examples provided (CSV examples) - [x] Comprehensive documentation (GENERATOR_ARCHITECTURE.md, docs/bridge_has/, docs/bridge_mas/)

### 23.4.4	1.4 Implementation Status and Phases

Unified Generator (bridge_generator.py):

The bridge generator now supports both TOML/CSV configuration and legacy array-indexed modes:

Configuration Modes: 1. TOML/CSV Mode (Preferred) - TOML port configuration (bridge_name.toml) - CSV connectivity matrix (bridge_name_connectivity.csv) - Custom port prefixes (rapids_m_axi_, cpu_m_axi_, etc.) - Interface wrapper integration (timing isolation) - Mixed protocols (AXI4 + APB + AXI4-Lite slaves) - Channel-specific masters (wr/rd/rw) - Status: Phase 2 complete, Phase 3 pending

- Legacy CSV Mode (Backwards Compatible)

  - Separate ports.csv and connectivity.csv files

  - Migration path to TOML format

  - See bin/test_configs/README.md for conversion guide

Phase Status:

Phase 1: CSV Configuration COMPLETE - CSV parser (ports.csv, connectivity.csv) - Port generation with custom prefixes - Converter identification logic - Basic crossbar instantiation

Phase 2: Channel-Specific Masters COMPLETE (2025-10-26) - Added channels field to PortSpec (rw/wr/rd) - Conditional port generation based on channels - Resource Optimization: - wr (write-only): AW, W, B channels only → 39% port reduction - rd (read-only): AR, R channels only → 61% port reduction - rw (full): All 5 channels - Width converter awareness (only generate needed converters) - Example: 4-master bridge saves 35% signals with optimized channels

Phase 3: APB Converter Integration PENDING - AXI2APB converter module (create or integrate existing) - APB signal packing/unpacking - APB converter instantiation in generated bridges - Width converter + APB converter chaining - End-to-end testing with APB slaves - Status: Placeholders in generated code with detailed TODO comments

Additional Resources: - models/bridge_model/bridge_model.py - Performance modeling (V1 Flat implemented) - rtl/bridge_cam.sv - Transaction ID tracking for OOO support - See GENERATOR_ARCHITECTURE.md for the generator/build architecture - See docs/bridge_has/ and docs/bridge_mas/ for architecture diagrams and specs (the older BRIDGE_CURRENT_STATE.md / BRIDGE_ARCHITECTURE_DIAGRAMS.md were superseded)

## 23.5	2. Architecture Overview

### 23.5.1	2.1 AXI4 vs AXIS vs APB

| Feature | APB | AXI-Stream (Delta) | AXI4 (Bridge) |
| --- | --- | --- | --- |
| Protocol Type | Simple register bus | Streaming data | Memory-mapped burst |
| Channels | 1 (address + data) | 1 (data stream) | 5 (AW, W, B, AR, R) |
| Request Generation | Address range decode | TDEST decode | Address range decode |
| Arbitration | Per-slave, per-cycle | Per-slave, packet-locked | Per-slave, per-address-phase |
| Out-of-Order | No (sequential) | No (streaming order) | Yes (ID-based) |
| Burst Support | No | Packet (via TLAST) | Yes (AWLEN/ARLEN) |
| Complexity | Low | Medium | High |
| Latency | 1-2 cycles | 2 cycles | 2-3 cycles |
| Use Case | Control registers | Data streaming | Memory-mapped I/O |

Bridge Complexity Sources: 1. 5 independent channels requiring separate arbitration 2. ID-based routing for out-of-order completion 3. Burst handling with interleaving constraints 4. Write response tracking (match AW with B channel) 5. Address decode + ID muxing for response routing

### 23.5.2	2.2 Block Diagram

┌──────────────────────────────────────────────────────────────────────┐
│                          BRIDGE AXI4 CROSSBAR                        │
└──────────────────────────────────────────────────────────────────────┘

Masters (M)                                                    Slaves (S)
┌─────────┐                                                   ┌─────────┐
│ Master 0│ ─AW──┐                                      ┌─AW──│ Slave 0 │
│  (CPU)  │ ─W───┤                                      ├─W───│ (DDR)   │
│         │ ─B───┤                                      ├─B───│         │
│         │ ─AR──┤                                      ├─AR──│         │
│         │ ─R───┤                                      ├─R───│         │
└─────────┘      │                                      │     └─────────┘
                 │                                      │
┌─────────┐      │   ┌──────────────────────────┐      │     ┌─────────┐
│ Master 1│ ─────┼───│  Request Generation      │──────┼─────│ Slave 1 │
│  (GPU)  │      │   │  (Address Range Decode)  │      │     │ (SRAM)  │
└─────────┘      │   └──────────────────────────┘      │     └─────────┘
                 │             │                        │
┌─────────┐      │   ┌──────────────────────────┐      │     ┌─────────┐
│ Master 2│ ─────┼───│  Arbitration Matrix      │──────┼─────│ Slave 2 │
│  (DMA)  │      │   │  (5 separate arbiters    │      │     │ (PCIE)  │
└─────────┘      │   │   per slave: AW,W,B,AR,R)│      │     └─────────┘
                 │   └──────────────────────────┘      │
┌─────────┐      │             │                        │     ┌─────────┐
│ Master 3│ ─────┴───│  Data/Addr Multiplexing  │──────┴─────│ Slave 3 │
│ (Accel) │          │  (ID-based routing)      │            │ (Periph)│
└─────────┘          └──────────────────────────┘            └─────────┘
                               │
                     ┌──────────────────────────┐
                     │  Transaction Tracking    │
                     │  (ID tables for OoO)     │
                     └──────────────────────────┘

### 23.5.3	2.3 Key Components

1. Request Generation - Address range decode for each slave - Generates M×S request matrix per channel (AW, AR) - Similar to APB but more complex (2 address channels)

2. Per-Slave Arbitration - 5 separate arbiters per slave: - AW channel arbiter (write address) - W channel arbiter (write data - locked to AW grant) - B channel arbiter (write response - routed by ID) - AR channel arbiter (read address) - R channel arbiter (read data - routed by ID) - Round-robin with burst locking - Separate read/write paths (no head-of-line blocking)

3. Data Multiplexing - Mux master signals to selected slave - ID-based response routing (B, R channels) - Burst tracking (hold grant until xlast)

4. Transaction Tracking - ID tables per slave for out-of-order support - Track {Master ID, Transaction ID} → Master mapping - Required for routing B/R channels back to correct master

5. Optional Performance Counters - Transaction counts per master/slave - Arbitration conflict counts - Latency histograms

## 23.6	3. Functional Requirements

### 23.6.1	3.1 AXI4 Protocol Compliance

FR-1: Full AXI4 Protocol Support - Support all 5 AXI4 channels: AW, W, B, AR, R - Comply with AMBA AXI4 specification (ARM IHI 0022) - Support burst lengths: 1-256 beats (AWLEN/ARLEN = 0-255) - Support burst types: INCR, WRAP, FIXED - Support burst sizes: 1-128 bytes (AWSIZE/ARSIZE = 0-7)

FR-2: Out-of-Order Transaction Support - Route responses via ID matching - Maintain transaction ID integrity (AWID → BID, ARID → RID) - Support configurable ID width (1-16 bits) - Track up to 2^ID_WIDTH outstanding transactions per slave

FR-3: Atomic Operations - Support exclusive access (AWLOCK/ARLOCK) - Track exclusive monitor per slave - Generate BRESP/RRESP errors for failed exclusives

### 23.6.2	3.2 Address Decoding

FR-4: Configurable Address Map - Support M masters × S slaves - Each slave has base address and size - Non-overlapping address ranges (verified at generation) - Default slave for unmapped addresses (optional)

FR-5: Address Range Configuration

# Example address map
address_map = {
    0: {'base': 0x00000000, 'size': 0x40000000, 'name': 'DDR'},      # 1GB
    1: {'base': 0x40000000, 'size': 0x00100000, 'name': 'SRAM'},     # 1MB
    2: {'base': 0x50000000, 'size': 0x01000000, 'name': 'PCIE'},     # 16MB
    3: {'base': 0x60000000, 'size': 0x00010000, 'name': 'Peripherals'} # 64KB
}

### 23.6.3	3.3 Arbitration Strategy

FR-6: Round-Robin Arbitration - Fair bandwidth allocation (no starvation) - Separate arbiters for AW and AR channels - Burst locking: Grant held until xlast (WLAST/RLAST) - Configurable arbitration policy (round-robin default)

FR-7: Read/Write Independence - Separate read and write paths - No head-of-line blocking between read/write - Concurrent read and write to same slave (if slave supports)

### 23.6.4	3.4 Burst Handling

FR-8: Burst Optimization - Pipelined burst transfers (overlap address and data) - No artificial burst splitting - Full AXI4 burst protocol support

FR-9: Interleaving Constraints - W channel locked to AW grant master - R channel routed by transaction ID - Support ID-based interleaving (slave-dependent)

## 23.7	4. Non-Functional Requirements

### 23.7.1	4.1 Performance

NFR-1: Latency - Single-beat read: ≤ 3 cycles (address decode + arbitration + mux) - Single-beat write: ≤ 3 cycles (address decode + arbitration + mux) - Burst transfer: No additional latency per beat (pipelined)

NFR-2: Throughput - Concurrent transfers: All M×S paths can transfer simultaneously - Burst efficiency: Line-rate data transfer after address phase - No artificial stalls: Crossbar adds no wait states beyond arbitration

NFR-3: Fmax - Target: 300-400 MHz on Xilinx UltraScale+ - Registered outputs: All slave outputs registered for timing closure - Pipelineable: Optional pipeline stages for >400 MHz

### 23.7.2	4.2 Resource Usage (Estimated)

M = 4 masters, S = 4 slaves, DATA_WIDTH = 512, ADDR_WIDTH = 64, ID_WIDTH = 4:

| Resource | Flat Crossbar | Notes |
| --- | --- | --- |
| LUTs | ~2,500 | Address decode + arbiters + mux |
| FFs | ~3,000 | Registered outputs + ID tables |
| BRAM | 0 | Distributed RAM for small ID tables |
| DSP | 0 | No arithmetic operations |

Scaling: ~150 LUTs per M×S connection

### 23.7.3	4.3 Quality Requirements

NFR-4: Code Generation Quality - Lint-clean SystemVerilog (Verilator) - Synthesizable (Vivado, Yosys, Design Compiler) - No vendor-specific primitives (technology-agnostic) - Clear structure (commented, readable)

NFR-5: Verification - CocoTB testbench framework - Transaction-level verification - Out-of-order test scenarios - Burst interleaving tests - >95% functional coverage

## 23.8	5. Interface Specifications

### 23.8.1	5.1 AXI4 Master Interfaces (M × 5 channels)

Write Address Channel (AW):

input  logic [ADDR_WIDTH-1:0]   s_axi_awaddr  [NUM_MASTERS];
input  logic [ID_WIDTH-1:0]     s_axi_awid    [NUM_MASTERS];
input  logic [7:0]              s_axi_awlen   [NUM_MASTERS];  // Burst length-1
input  logic [2:0]              s_axi_awsize  [NUM_MASTERS];  // Burst size
input  logic [1:0]              s_axi_awburst [NUM_MASTERS];  // INCR/WRAP/FIXED
input  logic                    s_axi_awlock  [NUM_MASTERS];  // Exclusive access
input  logic [3:0]              s_axi_awcache [NUM_MASTERS];  // Cache attributes
input  logic [2:0]              s_axi_awprot  [NUM_MASTERS];  // Protection type
input  logic                    s_axi_awvalid [NUM_MASTERS];
output logic                    s_axi_awready [NUM_MASTERS];

Write Data Channel (W):

input  logic [DATA_WIDTH-1:0]   s_axi_wdata  [NUM_MASTERS];
input  logic [DATA_WIDTH/8-1:0] s_axi_wstrb  [NUM_MASTERS];  // Byte strobes
input  logic                    s_axi_wlast  [NUM_MASTERS];  // Last beat
input  logic                    s_axi_wvalid [NUM_MASTERS];
output logic                    s_axi_wready [NUM_MASTERS];

Write Response Channel (B):

output logic [ID_WIDTH-1:0]     s_axi_bid    [NUM_MASTERS];
output logic [1:0]              s_axi_bresp  [NUM_MASTERS];  // OKAY/EXOKAY/SLVERR/DECERR
output logic                    s_axi_bvalid [NUM_MASTERS];
input  logic                    s_axi_bready [NUM_MASTERS];

Read Address Channel (AR):

input  logic [ADDR_WIDTH-1:0]   s_axi_araddr  [NUM_MASTERS];
input  logic [ID_WIDTH-1:0]     s_axi_arid    [NUM_MASTERS];
input  logic [7:0]              s_axi_arlen   [NUM_MASTERS];
input  logic [2:0]              s_axi_arsize  [NUM_MASTERS];
input  logic [1:0]              s_axi_arburst [NUM_MASTERS];
input  logic                    s_axi_arlock  [NUM_MASTERS];
input  logic [3:0]              s_axi_arcache [NUM_MASTERS];
input  logic [2:0]              s_axi_arprot  [NUM_MASTERS];
input  logic                    s_axi_arvalid [NUM_MASTERS];
output logic                    s_axi_arready [NUM_MASTERS];

Read Data Channel (R):

output logic [DATA_WIDTH-1:0]   s_axi_rdata  [NUM_MASTERS];
output logic [ID_WIDTH-1:0]     s_axi_rid    [NUM_MASTERS];
output logic [1:0]              s_axi_rresp  [NUM_MASTERS];
output logic                    s_axi_rlast  [NUM_MASTERS];
output logic                    s_axi_rvalid [NUM_MASTERS];
input  logic                    s_axi_rready [NUM_MASTERS];

### 23.8.2	5.2 AXI4 Slave Interfaces (S × 5 channels)

Mirror of master interfaces, with M → S direction reversed.

### 23.8.3	5.3 Configuration Parameters

parameter int NUM_MASTERS   = 4;          // Number of master interfaces
parameter int NUM_SLAVES    = 4;          // Number of slave interfaces
parameter int DATA_WIDTH    = 512;        // Data bus width (bits)
parameter int ADDR_WIDTH    = 64;         // Address bus width (bits)
parameter int ID_WIDTH      = 4;          // Transaction ID width (bits)
parameter int MAX_BURST_LEN = 256;        // Maximum burst length (beats)
parameter bit PIPELINE_OUTPUTS = 1;       // Register slave outputs
parameter bit ENABLE_COUNTERS  = 1;       // Performance counters

## 23.9	6. Performance Modeling

### 23.9.1	6.1 Analytical Model

Latency Components (Flat Crossbar):

Single-Beat Read Latency:
  1. Address Decode:      0 cycles (combinatorial)
  2. Arbitration:         1 cycle  (AR arbiter decision)
  3. Address Mux:         0 cycles (combinatorial)
  4. Slave Access:        (slave-dependent, not crossbar)
  5. Response Mux:        1 cycle  (R channel ID lookup + mux)
  6. Output Register:     1 cycle  (optional, for timing closure)

  Total (no pipeline):    2 cycles
  Total (pipelined):      3 cycles

Burst Transfer Throughput:

After address phase completes, data transfer is line-rate:
  - W channel: 1 beat/cycle (locked to AW grant)
  - R channel: 1 beat/cycle (ID-routed from slave)

Example: 256-beat burst
  Address phase:  2-3 cycles (one-time)
  Data phase:     256 cycles (line-rate)
  Total:          258-259 cycles
  Efficiency:     98.8%

Concurrent Throughput:

Maximum concurrent transfers (4×4 crossbar):
  - Read:  4 concurrent (one per master-slave pair)
  - Write: 4 concurrent (one per master-slave pair)
  - Total: 8 concurrent read+write (separate paths)

Throughput @ 100 MHz, 512-bit data:
  Per path:   100 MHz × 512 bits = 51.2 Gbps
  Total:      8 paths × 51.2 Gbps = 409.6 Gbps theoretical

### 23.9.2	6.2 Resource Scaling

LUT Usage Formula (empirical):

LUTs ≈ 500 (base) + 150 × M × S + 20 × ID_TABLE_DEPTH

Example (4×4 crossbar, ID_WIDTH=4, ID_TABLE_DEPTH=16 per slave):

LUTs ≈ 500 + 150 × 16 + 20 × 16 × 4
     ≈ 500 + 2,400 + 1,280
     ≈ 4,180 LUTs

### 23.9.3	6.3 Comparison with Other Crossbars

| Crossbar Type | Latency | Throughput | Complexity | Use Case |
| --- | --- | --- | --- | --- |
| APB | 1-2 cycles | Low (serialized) | Low | Control registers |
| AXI-Stream (Delta) | 2 cycles | High (streaming) | Medium | Data streaming |
| AXI4 (Bridge) | 2-3 cycles | High (burst) | High | Memory-mapped I/O |
| AXI4 + Slices | 4-6 cycles | High (burst) | Very High | >400 MHz designs |

Bridge Sweet Spot: High-performance memory-mapped interconnects where out-of-order and burst efficiency are critical.

## 23.10	7. Generator Architecture

### 23.10.1	7.1 Python Generator Structure

class BridgeGenerator:
    """AXI4 crossbar RTL generator"""

    def __init__(self, config):
        self.num_masters = config.num_masters
        self.num_slaves = config.num_slaves
        self.data_width = config.data_width
        self.addr_width = config.addr_width
        self.id_width = config.id_width
        self.address_map = config.address_map

    def generate_address_decode(self) -> str:
        """Generate address range decode logic"""
        # For each master × slave, check if address in range
        # More complex than AXIS (uses address), simpler than full decode

    def generate_aw_arbiter(self, slave_idx) -> str:
        """Generate write address channel arbiter for one slave"""
        # Round-robin arbiter
        # Grants locked until corresponding B response completes

    def generate_ar_arbiter(self, slave_idx) -> str:
        """Generate read address channel arbiter for one slave"""
        # Round-robin arbiter
        # Grants locked until corresponding R response completes (RLAST)

    def generate_w_channel_mux(self, slave_idx) -> str:
        """Generate write data channel multiplexer"""
        # W channel follows AW grant (locked until WLAST)

    def generate_b_channel_demux(self, slave_idx) -> str:
        """Generate write response channel demultiplexer"""
        # Route by ID: lookup master from {slave_idx, BID}

    def generate_r_channel_demux(self, slave_idx) -> str:
        """Generate read data channel demultiplexer"""
        # Route by ID: lookup master from {slave_idx, RID}

    def generate_id_table(self, slave_idx) -> str:
        """Generate transaction ID tracking table"""
        # Maps {slave, transaction_id} → master_id
        # Indexed on AW/AR grant, looked up on B/R response

    def generate_crossbar(self) -> str:
        """Generate complete crossbar module"""
        # Instantiate all arbiters, muxes, demuxes, ID tables

### 23.10.2	7.2 Address Map Configuration

Python Configuration:

bridge_config = {
    'num_masters': 4,
    'num_slaves': 4,
    'data_width': 512,
    'addr_width': 64,
    'id_width': 4,
    'address_map': {
        0: {'base': 0x00000000, 'size': 0x40000000, 'name': 'DDR'},
        1: {'base': 0x40000000, 'size': 0x00100000, 'name': 'SRAM'},
        2: {'base': 0x50000000, 'size': 0x01000000, 'name': 'PCIE'},
        3: {'base': 0x60000000, 'size': 0x00010000, 'name': 'Peripherals'}
    },
    'pipeline_outputs': True,
    'enable_counters': True
}

Generated Address Decode:

// Address decode for each master
always_comb begin
    for (int s = 0; s < NUM_SLAVES; s++)
        aw_request_matrix[s] = '0;

    for (int m = 0; m < NUM_MASTERS; m++) begin
        if (s_axi_awvalid[m]) begin
            // Slave 0: DDR (0x00000000 - 0x3FFFFFFF)
            if (s_axi_awaddr[m] >= 64'h00000000 &&
                s_axi_awaddr[m] < 64'h40000000)
                aw_request_matrix[0][m] = 1'b1;

            // Slave 1: SRAM (0x40000000 - 0x400FFFFF)
            if (s_axi_awaddr[m] >= 64'h40000000 &&
                s_axi_awaddr[m] < 64'h40100000)
                aw_request_matrix[1][m] = 1'b1;

            // ... slaves 2-3 ...
        end
    end
end

## 23.11	8. Comparison with APB and Delta

### 23.11.1	8.1 Code Reuse from APB Generator

Similar Components (~70% reuse): - Address range decode logic (same pattern, different signals) - Round-robin arbiter (same algorithm) - Data multiplexing pattern (same approach) - Backpressure handling (similar to PREADY)

New Components for Bridge: - 5× the arbiters (AW, W, B, AR, R instead of single channel) - ID-based routing (B and R channel demuxing) - Transaction tracking (ID tables for out-of-order) - Burst handling (grant locking until xlast)

Migration Effort from APB: - ~120 minutes (vs ~75 min for AXIS, due to higher complexity) - Most time: ID table logic and response demuxing

### 23.11.2	8.2 Code Reuse from Delta Generator

Similar Components (~60% reuse): - Python generation framework - Command-line interface - Performance modeling structure - Arbitration pattern (round-robin with locking)

Key Differences: - 5 channels vs 1 channel (Delta only has TDATA/TVALID/TREADY/TLAST) - ID-based routing vs TDEST-based routing - Address decode vs TDEST decode (Bridge more complex) - Transaction tracking vs packet atomicity (different mechanisms)

### 23.11.3	8.3 Complexity Comparison

| Metric | APB Crossbar | Delta (AXIS) | Bridge (AXI4) |
| --- | --- | --- | --- |
| Channels to arbitrate | 1 | 1 | 5 ★ |
| Request generation | Address ranges | TDEST decode | Address ranges |
| Response routing | Grant-based | Grant-based | ID-based ★ |
| Burst support | No | Packet (TLAST) | Yes (AWLEN/ARLEN) ★ |
| Out-of-order | No | No | Yes (ID tables) ★ |
| Transaction tracking | No | No | Yes ★ |
| Lines of Python | ~500 | ~697 | ~900 (est.) |
| Lines of generated SV | ~200 (4×4) | ~250 (4×4) | ~400 (4×4) (est.) |

★ = Additional complexity in Bridge

## 23.12	9. Use Cases

### 23.12.1	9.1 Multi-Core Processor Interconnect

Scenario: 4 CPU cores + GPU accessing DDR + SRAM + Peripherals

Configuration:
  Masters:  5 (4 CPUs, 1 GPU)
  Slaves:   3 (DDR, SRAM, Peripherals)
  Data:     512-bit (cache line width)
  Address:  64-bit (large memory space)
  ID:       4-bit (up to 16 outstanding per master)

Benefits: - Concurrent access to all slaves - Out-of-order completion for high-performance CPUs - Burst transfers for cache line fills - Separate read/write paths (no head-of-line blocking)

### 23.12.2	9.2 DMA + Accelerator System

Scenario: DMA engine + 4 accelerators accessing shared memory

Configuration:
  Masters:  5 (1 DMA, 4 accelerators)
  Slaves:   2 (DDR, Control registers)
  Data:     512-bit (high-bandwidth DMA)
  Address:  32-bit (limited address space)
  ID:       2-bit (simple ID space)

Benefits: - High-bandwidth memory access for DMA - Fair arbitration prevents accelerator starvation - Control register access doesn’t block data transfers

### 23.12.3	9.3 FPGA System Integration

Scenario: MicroBlaze CPU + custom accelerators + memory controllers

Configuration:
  Masters:  8 (1 CPU, 7 accelerators)
  Slaves:   4 (DDR, BRAM, AXI GPIO, AXI DMA)
  Data:     128-bit (AXI4 standard width)
  Address:  32-bit (standard FPGA address space)
  ID:       4-bit (moderate outstanding transactions)

Benefits: - Standard AXI4 interfaces (Xilinx IP compatibility) - Scalable to many masters/slaves - Performance counters for profiling

## 23.13	10. Testing Strategy

### 23.13.1	10.1 FUB (Functional Unit Block) Tests

Address Decode Tests: - Verify all address ranges correctly decoded - Test boundary conditions (base, base+size-1) - Test unmapped addresses (error response)

Arbiter Tests: - Round-robin fairness (all masters get turns) - Burst locking (grant held until xlast) - Starvation prevention

ID Table Tests: - Correct ID → master mapping - Out-of-order transaction handling - Table full condition

Mux/Demux Tests: - Data integrity through crossbar - Response routing to correct master - Concurrent transfers don’t interfere

### 23.13.2	10.2 Integration Tests

Single-Master, Single-Slave: - Basic read/write transactions - Burst transfers (various lengths) - Out-of-order completions

Multi-Master, Single-Slave: - Arbitration correctness - Fairness verification - Burst interleaving (if supported)

Multi-Master, Multi-Slave: - Concurrent transfers - No crosstalk between paths - Full M×S matrix coverage

Stress Tests: - All masters active simultaneously - Maximum burst lengths - Full ID space utilization - Back-to-back bursts

### 23.13.3	10.3 Performance Validation

Latency Measurement: - Single-beat read: measure actual vs analytical - Single-beat write: measure actual vs analytical - Compare with specification (≤3 cycles)

Throughput Measurement: - Burst transfer efficiency (should be ~98%) - Concurrent path throughput (should be line-rate × M×S) - Compare with theoretical maximum

Resource Validation: - Synthesize for Xilinx UltraScale+ - Compare LUT/FF usage with estimates - Verify Fmax ≥ 300 MHz

## 23.14	11. Documentation Plan

### 23.14.1	11.1 Specifications (Before Code)

- ☒ PRD.md - This document (complete requirements)

- ☐ README.md - User guide with quick start

- ☐ BRIDGE_VS_APB_GENERATOR.md - Migration guide from APB

- ☐ BRIDGE_VS_DELTA_GENERATOR.md - Comparison with Delta

- ☐ AXI4_PROTOCOL_GUIDE.md - AXI4 primer for students

### 23.14.2	11.2 Performance Analysis (Before Implementation)

- ☒ models/bridge_model/bridge_model.py - Analytical model (V1 Flat implemented)

  - Latency calculations (single-beat, burst)

  - Throughput estimates (concurrent paths)

  - Resource estimates (LUT/FF scaling)

- ☐ bridge simulator - Discrete event simulation (optional, not implemented)

  - Cycle-accurate modeling

  - Traffic pattern support

  - Validation against RTL

### 23.14.3	11.3 Code Generation

- ☐ bin/bridge_generator.py - Main RTL generator

  - Command-line interface

  - Address map configuration

  - Generated RTL output

### 23.14.4	11.4 Verification

- ☐ dv/tests/ - CocoTB testbenches

  - FUB tests (individual blocks)

  - Integration tests (multi-block)

  - Stress tests (corner cases)

## 23.15	12. Success Metrics

### 23.15.1	12.1 Functional Completeness

- ☐ Generates working AXI4 crossbar RTL

- ☐ Passes all CocoTB tests

- ☐ Verilator lint clean

- ☐ Xilinx Vivado synthesis clean

### 23.15.2	12.2 Performance Targets

- ☐ Latency ≤ 3 cycles (measured in simulation)

- ☐ Throughput = line-rate × M×S paths (measured)

- ☐ Fmax ≥ 300 MHz (post-synthesis)

### 23.15.3	12.3 Educational Value

- ☐ Complete specifications demonstrating rigor

- ☐ Performance models validate requirements

- ☐ Clear code structure (readable generated RTL)

- ☐ Integration examples provided

### 23.15.4	12.4 Reusability

- ☐ ~70% code reuse from APB generator (measured by LOC)

- ☐ Similar patterns to Delta generator

- ☐ Can be extended (weighted arbitration, QoS, etc.)

## 23.16	12. Attribution and Contribution Guidelines

### 23.16.1	12.1 Git Commit Attribution

When creating git commits for Bridge documentation or implementation:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: Bridge documentation and organization receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

## 23.17	12.2 PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

projects/components/bridge/docs/

Quick Commands: Use the provided shell scripts:

cd projects/components/bridge/docs
./generate_has_pdf.sh   # Bridge_HAS_v<rev>.docx/.pdf
./generate_mas_pdf.sh   # Bridge_MAS_v<rev>.docx/.pdf

The shell scripts will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the bridge_has/bridge_mas index files 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

See: bin/md_to_docx.py for complete implementation details

## 23.18	13. Future Enhancements

### 23.18.1	13.1 Short-Term (Post-Initial Release)

- ☐ Optional pipeline stages - For Fmax >400 MHz

- ☐ Weighted arbitration - QoS support

- ☐ Default slave - Unmapped address handling

- ☐ Exclusive monitor - Full atomic operation support

### 23.18.2	13.2 Long-Term

- ☐ Tree topology - Hierarchical crossbar (like Delta)

- ☐ AXI4-Lite variant - Simplified for control registers

- ☐ ACE support - Coherent cache interconnect

- ☐ GUI configurator - Visual address map setup

## 23.19	14. Risk Assessment

| Risk | Probability | Impact | Mitigation |
| --- | --- | --- | --- |
| ID table complexity | Medium | High | Start with small ID_WIDTH (2-4), test thoroughly |
| Out-of-order corner cases | High | High | Extensive CocoTB tests with random delays |
| Fmax below target | Low | Medium | Optional pipeline stages for timing closure |
| Resource usage exceeds | Low | Low | Empirical formulas guide expectations |
| Burst interleaving bugs | Medium | High | Separate test suite for burst scenarios |

## 23.20	15. Project Timeline (Estimated)

Week 1: Specifications and Models - [x] Day 1-2: PRD.md (complete) - [ ] Day 3-4: Performance modeling (analytical) - [ ] Day 5-7: README.md, migration guides

Week 2-3: Core Implementation - [ ] Day 1-3: Address decode + arbiter generation - [ ] Day 4-5: Data mux/demux generation - [ ] Day 6-8: ID table generation - [ ] Day 9-10: Integration and testing

Week 4: Verification and Examples - [ ] Day 1-5: CocoTB testbenches - [ ] Day 6-7: Integration examples

## 23.21	16. References

AXI4 Specifications: - ARM IHI 0022 - AMBA AXI and ACE Protocol Specification - Xilinx UG1037 - Vivado AXI Reference Guide

Related Projects: - APB Crossbar - Simple register bus crossbar (existing) - Delta (AXIS Crossbar) - Streaming data crossbar (projects/components/delta/) - RAPIDS - DMA engine with AXI4 masters (projects/components/dmas/rapids/)

Tools: - Verilator - RTL linting and simulation - CocoTB - Python-based verification - Xilinx Vivado - FPGA synthesis

## 23.22	17. Glossary

- AXI4: Advanced eXtensible Interface version 4 (AMBA standard)

- Burst: Multi-beat transaction (AWLEN/ARLEN > 0)

- Crossbar: Full M×N interconnect matrix

- ID: Transaction identifier for out-of-order support

- Out-of-order: Responses can return in different order than requests

- xlast: WLAST (write) or RLAST (read) - last beat indicator

Version: 1.0 Status: Specification Complete - Ready for Implementation Next Steps: Create performance models, then implement generator

Project Bridge - Connecting masters and slaves across the divide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Claude Code Guide: Bridge Subsystem

Version: 2.1 Last Updated: 2025-11-03 Purpose: AI-specific guidance for working with Bridge subsystem

## 24.1	CRITICAL: Read Architecture Documents First

Before making ANY changes to bridge generator or understanding signal flow:

READ: projects/components/bridge/GENERATOR_ARCHITECTURE.md (generator structure) and projects/components/bridge/docs/bridge_mas/ (micro-architecture spec; rendered as docs/Bridge_MAS_v1.1.pdf)

These documents contain the definitive bridge architecture including: - Correct signal flow (wrappers → decoder → converters → crossbar → slaves) - Component purposes and placement - Common misconceptions that previous agents made - Why there’s NO fixed crossbar width

If you skip these documents, you WILL make incorrect assumptions.

## 24.2	Quick Context

What: Bridge - Two complementary AXI4 Crossbar Generators (framework-based and CSV-based) Status: Phase 2 Complete - CSV generator with channel-specific masters (wr/rd/rw) Your Role: Help users configure CSV files, generate bridges, understand architecture, and create tests

Complete Documentation (Read in This Order): 1. projects/components/bridge/GENERATOR_ARCHITECTURE.md ← START HERE (generator architecture reference) 2. projects/components/bridge/PRD.md ← Product requirements 3. projects/components/bridge/TASKS.md ← Task history and current status 4. projects/components/bridge/docs/bridge_has/bridge_has_index.md ← Hardware Architecture Spec (rendered: docs/Bridge_HAS_v1.1.pdf) 5. projects/components/bridge/docs/bridge_mas/bridge_mas_index.md ← Micro-Architecture Spec (rendered: docs/Bridge_MAS_v1.1.pdf)

(The older BRIDGE_ARCHITECTURE.md, BRIDGE_CURRENT_STATE.md, BRIDGE_ARCHITECTURE_DIAGRAMS.md, CSV_BRIDGE_STATUS.md, and docs/bridge_spec/ documents were superseded by the HAS/MAS above.)

## 24.3	Target Architecture: Intelligent Width-Aware Routing

Core Principle: Direct connections where possible, converters only where needed, no fixed crossbar width.

### 24.3.1	Efficient Multi-Width Design

Master_A (64b)
  ├─ Direct → Slave_0 (64b)           [0 conversions, minimal latency]
  ├─ Conv(64→128) → Slave_1 (128b)    [1 conversion]
  └─ Conv(64→512) → Slave_2 (512b)    [1 conversion]

Master_B (512b)
  ├─ Conv(512→64) → Slave_0 (64b)     [1 conversion]
  ├─ Conv(512→128) → Slave_1 (128b)   [1 conversion]
  └─ Direct → Slave_2 (512b)          [0 conversions, full bandwidth]

Router Logic: Address decoder determines target slave, selects correctly-sized path for that master-slave pair.

### 24.3.2	Why This Architecture

Naive Fixed-Width Approach (Don’t Do This):

Master (64b) → Upsize(64→256) → Fixed 256b Crossbar → Downsize(256→64) → Slave (64b)

- TWO conversions for same-width connections (wasteful!)

- Reduced bandwidth on narrow paths

- Unnecessary logic and area

- Higher latency

Intelligent Routing (Target):

Master (64b) → Router → Direct Connection → Slave (64b)

- ZERO conversions for matching widths

- Full native bandwidth

- Minimal logic

- Lowest latency

### 24.3.3	Per-Master Output Paths

Each master has N output paths (one per unique slave width it connects to):

// Master_A connects to slaves at 64b, 128b, 512b
// Generate 3 output paths:

logic [63:0]  master_a_64b_wdata;   // For 64b slaves (direct)
logic [127:0] master_a_128b_wdata;  // For 128b slaves (via converter)
logic [511:0] master_a_512b_wdata;  // For 512b slaves (via converter)

// Router selects based on address decode:
always_comb begin
    case (decoded_slave_id)
        SLAVE_0: select master_a_64b_wdata;   // Slave_0 is 64b
        SLAVE_1: select master_a_128b_wdata;  // Slave_1 is 128b
        SLAVE_2: select master_a_512b_wdata;  // Slave_2 is 512b
    endcase
end

### 24.3.4	Benefits

- Resource Efficient - Only instantiate converters actually needed

- Maximum Performance - Direct paths have zero conversion overhead

- Optimal Bandwidth - No artificial width bottlenecks

- Lower Latency - Minimal logic in critical path for matching widths

- Scalable - Works for any combination of master/slave widths

### 24.3.5	Implementation Status

Current State: Fixed-width crossbar with master-side upsizing (Phase 1 architecture) Target State: Intelligent per-master multi-width routing (your vision) Migration: Requires generator architecture rework (see TASKS.md)

## 24.4	CRITICAL RULE #0: RTL Regeneration Requirements

READ THIS FIRST - FAILURE TO FOLLOW CAUSES TEST FAILURES

### 24.4.1	The Golden Rule

ALL generated RTL files MUST be deleted and regenerated together whenever ANY generator code changes.

### 24.4.2	Why This Is Non-Negotiable

Generated RTL files have interdependencies: - Each rtl/generated/<bridge_name>/ directory holds a matched set (top module, crossbar, adapters, package) - Generator changes affect signal names, port widths, interface structure - Partial regeneration creates version mismatches that cause silent failures

### 24.4.3	Real Example (Historical Session)

# WHAT WENT WRONG:
1. Updated the address decode logic in the generator
2. Regenerated ONLY one bridge configuration
3. Did NOT regenerate the wrapper that instantiated it
4. Result: All tests that were passing now FAIL
5. Cause: Version mismatch between wrapper and core bridge

# WHAT SHOULD HAVE BEEN DONE:
1. Update the generator
2. Delete ALL generated bridge directories (rtl/generated/bridge_*/)
3. Regenerate ALL bridges from scratch (make all in bin/)
4. Run ALL tests to verify

### 24.4.4	Generator Files That Trigger Full Regeneration

Any change to these files requires regenerating ALL bridges:

- bin/bridge_generator.py - Core bridge generator (CSV/TOML driven)

- ANY Python file in bin/bridge_pkg/ (components/, generators/, config, csv_parser, signal_naming, …)

- Any Jinja template in bin/bridge_pkg/jinja_templates/

### 24.4.5	The Regeneration Workflow

# Step 1: Make generator code changes
vim bin/bridge_pkg/generators/crossbar_generator.py

# Step 2: Delete ALL generated RTL (be aggressive!)
cd projects/components/bridge/bin
make clean   # Removes rtl/generated/ output

# Step 3: Regenerate everything from bridge_batch.csv
make all     # Runs bridge_generator.py --bulk bridge_batch.csv

# Step 4: Run ALL tests
cd ../dv/tests
pytest -v  # ALL tests, not just the one you think changed

# Step 5: Verify git diff makes sense
git diff ../rtl/  # Review all changes

### 24.4.6	Symptoms of Version Mismatch

If you see these symptoms, you probably did partial regeneration:

- Tests that previously passed now fail

- “Signal not found” errors in simulation

- Port width mismatches in instantiation

- Address decode routing to wrong slaves

- Missing debug signals (dbg_*)

- Unexpected compile errors in working code

### 24.4.7	Think Like a Compiler Developer

Generated RTL = Compiled Object Files

When you update a compiler, you don’t selectively recompile - you make clean && make all.

When you update a generator, you don’t selectively regenerate - you delete all and regenerate all.

### 24.4.8	Exception: Hand-Written RTL

These files are never regenerated: - bridge_cam.sv - CAM module (hand-written) - Any file in rtl/ that is NOT generated

Check file headers - generated files say “Generated by: bridge_generator.py”

## 24.5	Critical Pitfalls: slave_select_* Reversion After Handshake

### 24.5.1	The Problem: Combinational Address Decode

The master adapter’s R/B response MUX and the crossbar’s AR/AW gating both relied on <master>_slave_select_{ar,aw} — a combinational decode of fub_axi_{ar,aw}addr. This signal reverts immediately when the address port is freed:

- AR/AW handshake completes (arvalid && arready, awvalid && awready)

- Wrapper pops skid buffer and releases the address port

- fub_axi_araddr / fub_axi_awaddr reverts to next transaction or zero

- Decode flips to different slave

- Response MUX is now gated on WRONG slave

- Response path closes, transaction hangs

This happens BEFORE the converter even finishes pushing to the crossbar, and ALWAYS before response arrives.

### 24.5.2	Visible Symptoms

- Fails: Multi-slave multi-master bridges hang (timeout)

- Fails: Multi-width masters fail basic connectivity

- Fails: APB/AXIL slaves consistently timeout (longer converter latency = more decode changes)

- Works: Single-slave or single-master systems (no decode change)

### 24.5.3	The Solution (MANDATORY in All Generators)

For Crossbar AR/AW Gating: - Replace <master>_slave_select_ar/aw gates with inline address re-decode on stable m_axi buses - Address on m_axi is held stable across the entire AXI handshake

For Master Adapter Response MUX: - Add per-channel response-tracking FIFOs that capture slave index at request time - Use FIFO head for response MUX instead of live combinational decode

Crossbar Code Pattern:

// WRONG (old implementation):
assign gate_ar_s0 = m0_slave_select_ar && m0_arvalid;
//                  ^^^^^^^^^^^^^^^^^ Reverts after handshake!

// CORRECT (new implementation):
assign gate_ar_s0 = ((m0_m_axi_araddr >= S0_BASE) && 
                      (m0_m_axi_araddr < S0_END) && 
                      m0_arvalid);  // Re-decode on stable m_axi address

Master Adapter Code Pattern:

// WRONG (old implementation):
assign r_slave_select = comb_slave_select_ar;  // Stale after pop!

// CORRECT (new implementation):
// Capture at request time
always_ff @(posedge aclk) begin
    if (fub_axi_arvalid && fub_axi_arready) begin
        ar_trk_fifo <= comb_slave_select_ar;
    end
end

// Use stable value for response MUX
assign r_slave_select = ar_trk_fifo_out;

### 24.5.4	When This Bug Appears

- Works: Single-master, single-slave systems (no address re-evaluation)

- Fails: Multi-master to same slave (multiple addresses decode differently)

- Fails: Multi-slave same master (address decode changes per transaction)

- Fails: Multi-width paths (W beats re-evaluate AW decode mid-burst)

## 24.6	MANDATORY: Project Organization Pattern

THIS SUBSYSTEM MUST FOLLOW THE RAPIDS/AMBA ORGANIZATIONAL PATTERN - NO EXCEPTIONS

### 24.6.1	Required Directory Structure

projects/components/bridge/
├── bin/                           # Bridge-specific tools (generators, scripts)
│   ├── bridge_generator.py       # AXI4 crossbar generator (CSV/TOML driven)
│   ├── bridge_pkg/                # Generator implementation package
│   ├── test_configs/              # TOML port configs + CSV connectivity
│   └── Makefile                   # make all / make single / make clean
├── docs/                          # Design documentation (bridge_has/, bridge_mas/, PDFs)
├── dv/                            # Design Verification (MANDATORY structure)
│   ├── tbclasses/                 # Testbench classes (MANDATORY - TB classes here!)
│   │   ├── __init__.py
│   │   └── bridge2x2_rw_tb.py     # Per-config generated TB classes
│   ├── components/                # Bridge-specific BFMs (if needed)
│   ├── scoreboards/               # Bridge-specific scoreboards (if needed)
│   ├── testplans/                 # Test plans
│   └── tests/                     # All test files (test runners only, flat)
│       ├── conftest.py            # Pytest configuration (MANDATORY)
│       └── test_bridge_*.py       # Per-config tests (test_bridge_2x2_rw.py, ...)
├── rtl/                           # RTL source files
│   ├── bridge_cam.sv              # Hand-written CAM
│   ├── filelists/                 # Generated filelists
│   └── generated/                 # Generated bridges (one subdir per config)
├── CLAUDE.md                      # This file
├── PRD.md                         # Product requirements
└── TASKS.md                       # Task history and status

### 24.6.2	Testbench Class Location (MANDATORY)

WRONG: Testbench class in test file

# projects/components/bridge/dv/tests/test_bridge_2x2_rw.py
class Bridge2x2RwTB:  # WRONG LOCATION!
    """Embedded TB - NOT REUSABLE"""

CORRECT: Testbench class in PROJECT AREA dv/tbclasses/

# projects/components/bridge/dv/tbclasses/bridge2x2_rw_tb.py
class Bridge2x2RwTB(TBBase):  # CORRECT LOCATION!
    """Reusable TB class - imported by the matching test runner"""

CRITICAL: TB classes are PROJECT-SPECIFIC and MUST be in the project area (projects/components/{name}/dv/tbclasses/), NOT in the framework (bin/TBClasses/).

### 24.6.3	Test File Pattern (MANDATORY)

Test files MUST follow this structure:

# projects/components/bridge/dv/tests/test_bridge_2x2_rw.py

import os
import pytest
import cocotb
from cocotb_test.simulator import run

# IMPORT testbench class from PROJECT AREA
import sys
from TBClasses.shared.utilities import get_repo_root
sys.path.insert(0, get_repo_root())
from projects.components.bridge.dv.tbclasses.bridge2x2_rw_tb import Bridge2x2RwTB
from TBClasses.shared.utilities import get_paths, get_wave_config
from TBClasses.shared.filelist_utils import get_sources_from_filelist

# ===========================================================================
# COCOTB TEST FUNCTIONS - Prefix with "cocotb_" to prevent pytest collection
# ===========================================================================

@cocotb.test(timeout_time=100, timeout_unit='us')
async def cocotb_test_basic_routing(dut):
    """Test basic address routing"""
    tb = Bridge2x2RwTB(dut)  # Imported TB
    await tb.setup_clocks_and_reset()
    result = await tb.test_basic_routing()
    assert result, "Basic routing test failed"

# More cocotb test functions...

# ===========================================================================
# PYTEST WRAPPER FUNCTIONS - At bottom of file
# ===========================================================================

@pytest.mark.bridge
def test_bridge_2x2_rw(request):
    """Pytest wrapper for the generated 2x2 rw bridge"""
    module, repo_root, tests_dir, log_dir, rtl_dict = get_paths({
        'rtl_bridge': '../../rtl'
    })

    # ... verilog_sources from the bridge's filelist, parameters, etc ...

    run(
        verilog_sources=verilog_sources,
        toplevel="bridge_2x2_rw",
        module=module,
        testcase="cocotb_test_basic_routing",  # ← cocotb function name
        parameters=rtl_parameters,
        sim_build=sim_build,
        # ...
    )

See dv/tests/test_bridge_2x2_rw.py for the real, complete pattern (tests and TB classes are generated per bridge configuration alongside the RTL).

## 24.7	Critical Rules for This Subsystem

### 24.7.1	Rule #0: Attribution Format for Git Commits

IMPORTANT: When creating git commit messages for Bridge documentation or code:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: Bridge receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

### 24.7.2	Rule #0.1: Testbench Architecture - MANDATORY SEPARATION

THIS IS A HARD REQUIREMENT - NO EXCEPTIONS

NEVER embed testbench classes inside test runner files!

The same testbench logic will be reused across multiple test scenarios. Having testbench code only in test files makes it COMPLETELY WORTHLESS for reuse.

MANDATORY Structure:

projects/components/bridge/
├── dv/
│   ├── tbclasses/                   # TB classes HERE (not in framework!)
│   │   ├── __init__.py
│   │   ├── bridge2x2_rw_tb.py       ← REUSABLE TB CLASS (per config)
│   │   └── bridge5x3_channels_tb.py
│   ├── components/                  # Bridge-specific BFMs (if needed)
│   │   └── __init__.py
│   ├── scoreboards/                 # Bridge-specific scoreboards
│   │   └── __init__.py
│   └── tests/                       # Test runners (flat, one per config)
│       ├── conftest.py              ← MANDATORY pytest config
│       ├── test_bridge_2x2_rw.py    ← TEST RUNNER ONLY (imports TB)
│       └── test_bridge_5x3_channels.py

Why This Matters:

- Reusability: Same TB class used in:

  - Per-config functional tests

  - Monitor stress tests (test_bridge_*_mon_monitor.py)

  - User projects (external imports)

- Maintainability: Fix bug once in TB class, all tests benefit

- Composition: TB classes can inherit/compose for complex scenarios

### 24.7.3	Rule #1: All Testbenches Inherit from TBBase

Every testbench class MUST inherit from TBBase:

from TBClasses.shared.tbbase import TBBase

class BridgeAXI4FlatTB(TBBase):
    """Testbench for Bridge crossbar - inherits base functionality"""

    def __init__(self, dut, num_masters=2, num_slaves=2, **kwargs):
        super().__init__(dut)
        # Bridge-specific initialization

TBBase Provides: - Clock management (start_clock, wait_clocks) - Reset utilities (assert_reset, deassert_reset) - Logging (self.log) - Progress tracking (mark_progress) - Safety monitoring (timeouts, memory limits)

### 24.7.4	Rule #2: Mandatory Testbench Methods

Every testbench class MUST implement these three methods:

async def setup_clocks_and_reset(self):
    """Complete initialization - starts clocks and performs reset"""
    await self.start_clock('aclk', freq=10, units='ns')

    # Set config signals before reset (if needed)
    # self.dut.cfg_param.value = initial_value

    # Reset sequence
    await self.assert_reset()
    await self.wait_clocks('aclk', 10)
    await self.deassert_reset()
    await self.wait_clocks('aclk', 5)

async def assert_reset(self):
    """Assert reset signal (active-low for AXI4)"""
    self.dut.aresetn.value = 0

async def deassert_reset(self):
    """Deassert reset signal"""
    self.dut.aresetn.value = 1

Why Required: - Consistency across all testbenches - Reusability for mid-test resets - Clear test structure and intent

### 24.7.5	Rule #3: Use GAXI Components for Protocol Handling

For Bridge testing, use GAXI Master/Slave components for AXI4 channel handling:

from CocoTBFramework.components.gaxi.gaxi_master import GAXIMaster
from CocoTBFramework.components.gaxi.gaxi_slave import GAXISlave
from CocoTBFramework.components.axi4.axi4_field_configs import AXI4FieldConfigHelper

# CORRECT: Use GAXI for AXI4 channels
self.aw_master = GAXIMaster(
    dut=dut,
    title="AW_M0",
    prefix="s0_axi4_",
    clock=clock,
    field_config=AXI4FieldConfigHelper.create_aw_field_config(id_width, addr_width, 1),
    pkt_prefix="aw",
    multi_sig=True,
    log=log
)

Never manually drive AXI4 valid/ready handshakes - Use GAXI components.

### 24.7.6	Rule #4: Queue-Based Verification

For simple in-order verification, use direct monitor queue access:

# CORRECT: Direct queue access
aw_pkt = self.aw_slave._recvQ.popleft()
w_pkt = self.w_slave._recvQ.popleft()

# Verify
assert aw_pkt.addr == expected_addr
assert w_pkt.data == expected_data

# WRONG: Memory model for simple test
memory_model = MemoryModel()  # Unnecessary complexity

When to Use Memory Models: - Simple in-order tests → Use queue access - Single-master systems → Use queue access - Complex out-of-order scenarios → Memory model may help - Multi-master with address overlap → Memory model tracks state

## 24.8	TOML/CSV-Based Bridge Generator (Phase 2 Complete)

### 24.8.1	Overview

The Bridge generator creates parameterized SystemVerilog crossbars from TOML configuration files with CSV connectivity matrices, eliminating manual RTL editing for complex interconnects.

Key Benefits: - Human-readable configuration - TOML for ports, CSV for connectivity - Custom signal prefixes - Each port has unique prefix (rapids_m_axi_, apb0_, etc.) - Channel-specific masters - Write-only (wr), read-only (rd), or full (rw) - Interface modules - Timing isolation via axi4_master/slave wrappers with configurable skid depths - Automatic converters - Width and protocol conversion inserted automatically - Resource efficient - Only generates needed channels and converters

### 24.8.2	Quick Start

1. Create bridge_mybridge.toml:

[bridge]
  name = "bridge_mybridge"
  description = "Custom bridge example"

  # Default skid buffer depths (can be overridden per port)
  defaults.skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

  masters = [
    {name = "cpu", prefix = "cpu_m_axi", id_width = 4, addr_width = 32, data_width = 64, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "dma", prefix = "dma_m_axi", id_width = 4, addr_width = 32, data_width = 512, user_width = 1,
     interface = {type = "axi4_master", skid_depths = {ar = 4, r = 8, aw = 4, w = 8, b = 4}}}
  ]

  slaves = [
    {name = "ddr", prefix = "ddr_s_axi", id_width = 4, addr_width = 32, data_width = 512, user_width = 1,
     base_addr = 0x00000000, addr_range = 0x80000000, interface = {type = "axi4_slave"}},
    {name = "sram", prefix = "sram_s_axi", id_width = 4, addr_width = 32, data_width = 256, user_width = 1,
     base_addr = 0x80000000, addr_range = 0x80000000, interface = {type = "axi4_slave"}}
  ]

2. Create bridge_mybridge_connectivity.csv:

master\slave,ddr,sram
cpu,1,1
dma,1,1

3. Generate bridge:

cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_mybridge.toml
# Auto-finds bridge_mybridge_connectivity.csv

# Or use bulk generation
python3 bridge_generator.py --bulk bridge_batch.csv

Result: Complete SystemVerilog module with: - Custom port prefixes per port - Timing isolation via interface wrappers - Only needed AXI4 channels (wr/rd/rw optimized) - Width converters for data mismatches - Internal crossbar instantiation - APB/AXI4-Lite converter integration points

### 24.8.3	Configuration Format Details

TOML Port Configuration:

The primary format is now TOML (preferred over CSV for better structure and interface configuration):

Port Specifications: - name - Unique identifier (cpu, dma, ddr, etc.) - prefix - Signal prefix (cpu_m_axi_, ddr_s_axi_, etc.) - id_width - AXI4 ID width in bits - addr_width - Address width in bits - data_width - Data width in bits (32, 64, 128, 256, 512) - user_width - AXI4 user signal width - base_addr - Slave base address (slaves only) - addr_range - Slave address range (slaves only) - interface - Interface wrapper configuration (optional) - type - “axi4_master”, “axi4_slave”, “axi4_master_mon”, “axi4_slave_mon”, or omit for direct connection - skid_depths - Per-channel buffer depths: {ar, r, aw, w, b} (valid: 2, 4, 6, 8)

CSV Connectivity Matrix:

master\slave,slave0,slave1,slave2
master0,1,0,1
master1,0,1,1

- 1 = connected, 0 = not connected

- Partial connectivity supported (not all masters to all slaves)

- Auto-detected based on TOML filename: bridge_name.toml → bridge_name_connectivity.csv

Legacy CSV Format:

For backwards compatibility, the generator still supports CSV port files: - See bin/test_configs/README.md for migration guide from CSV to TOML - TOML is now preferred for new bridges (better structure, interface config support)

### 24.8.4	Channel-Specific Masters (Phase 2 Feature)

Why Channel-Specific? Real hardware often has dedicated read or write masters. Generating all 5 AXI4 channels wastes resources:

Traditional (wasteful):

// Write-only master gets unused read channels
input  logic [63:0]  rapids_descr_m_axi_awaddr,  // USED
input  logic [511:0] rapids_descr_m_axi_wdata,   // USED
output logic [7:0]   rapids_descr_m_axi_bid,     // USED
input  logic [63:0]  rapids_descr_m_axi_araddr,  // UNUSED (50% waste!)
output logic [511:0] rapids_descr_m_axi_rdata,   // UNUSED

Channel-Specific (optimized):

rapids_descr_wr,master,axi4,wr,rapids_descr_m_axi_,512,64,8,N/A,N/A

Generated:

// Write-only master - only AW, W, B channels
input  logic [63:0]  rapids_descr_m_axi_awaddr,
input  logic [511:0] rapids_descr_m_axi_wdata,
output logic [7:0]   rapids_descr_m_axi_bid,
// NO READ CHANNELS (araddr, rdata, etc.)

Resource Savings: - 40-60% fewer ports for dedicated masters - Only necessary width converters instantiated - Channel-aware direct connection wiring - Faster synthesis, smaller netlists

### 24.8.5	Example: RAPIDS-Style Configuration

RAPIDS Architecture: - Descriptor write master (wr) - Writes descriptors to memory - Sink write master (wr) - Writes incoming packets to memory - Source read master (rd) - Reads outgoing packets from memory - CPU master (rw) - Full access for configuration

TOML Configuration (bridge_rapids.toml):

[bridge]
  name = "bridge_rapids"
  description = "RAPIDS accelerator bridge"
  defaults.skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

  masters = [
    {name = "rapids_descr_wr", prefix = "rapids_descr_m_axi", channels = "wr",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "rapids_sink_wr", prefix = "rapids_sink_m_axi", channels = "wr",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "rapids_src_rd", prefix = "rapids_src_m_axi", channels = "rd",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "cpu", prefix = "cpu_m_axi", channels = "rw",
     id_width = 4, addr_width = 32, data_width = 64, user_width = 1,
     interface = {type = "axi4_master"}}
  ]

  slaves = [
    {name = "ddr", prefix = "ddr_s_axi", protocol = "axi4",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     base_addr = 0x80000000, addr_range = 0x80000000,
     interface = {type = "axi4_slave"}},
    {name = "apb_periph", prefix = "apb0_", protocol = "apb",
     addr_width = 32, data_width = 32,
     base_addr = 0x00000000, addr_range = 0x00010000}
  ]

Connectivity CSV (bridge_rapids_connectivity.csv):

master\slave,ddr,apb_periph
rapids_descr_wr,1,0
rapids_sink_wr,1,0
rapids_src_rd,1,0
cpu,1,1

Generated RTL Features: - rapids_descr_wr: 37 signals (write channels only) vs 61 signals (full) = 39% reduction - rapids_sink_wr: 37 signals (write channels only) vs 61 signals (full) = 39% reduction - rapids_src_rd: 24 signals (read channels only) vs 61 signals (full) = 61% reduction - cpu_master: Width converters for 64b→512b upsize (both wr and rd converters) - ddr_controller: Direct 512b connection (no conversion) - apb_periph0: APB converter placeholder (Phase 3)

### 24.8.6	Common User Questions

Q: “How do I generate a bridge?”

A: Three steps:

- Create TOML port configuration file

- Create CSV connectivity matrix

- Run bridge_generator.py

# Create bridge_mybridge.toml (port configuration)
# Create bridge_mybridge_connectivity.csv (connectivity matrix)

cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_mybridge.toml
# Auto-finds bridge_mybridge_connectivity.csv

# Or use bulk generation for multiple bridges
python3 bridge_generator.py --bulk bridge_batch.csv

Q: “What’s the difference between wr/rd/rw channels?”

A: Number of AXI4 channels generated:

- rw (read/write) - All 5 channels: AW, W, B, AR, R

- wr (write-only) - 3 channels: AW, W, B (no read channels)

- rd (read-only) - 2 channels: AR, R (no write channels)

Benefits: 40-60% fewer ports for dedicated masters, less logic, faster synthesis

Q: “Can I add timing isolation to ports?”

A: Yes, use interface configuration:

masters = [
  {name = "cpu", prefix = "cpu_m_axi", ...,
   interface = {type = "axi4_master", skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}}}
]

Available interface types: - "axi4_master" - Timing isolation on master port - "axi4_slave" - Timing isolation on slave port - "axi4_master_mon" - Timing + monitoring - "axi4_slave_mon" - Timing + monitoring - Omit interface field for direct connection

Q: “Can I mix AXI4 and APB slaves?”

A: Yes, use protocol field in TOML:

slaves = [
  {name = "ddr", prefix = "ddr_s_axi", protocol = "axi4", ...},
  {name = "apb0", prefix = "apb0_", protocol = "apb", ...}
]

Generator inserts AXI2APB converters automatically (Phase 3 for full implementation).

Q: “Can ports be AMBA5 (AXI5 / APB5)?”

A: Yes — per port, while the fabric stays AXI4 (BRIDGE-002):

[[bridge.masters]]
protocol = "axi5"
axi5_features = ["trace", "atomic"]  # nsaid/trace/mpam/mecid/unique are
                                     # droppable sideband; poison/atomic are
                                     # connectivity-gated (every connected path
                                     # must be AXI5 + feature + width-matched);
                                     # mte/chunking rejected

[[bridge.slaves]]
protocol = "apb5"   # APB4 rules apply (rw-only, 32-bit); adds the APB5 pins

Sideband passes natively on width-matched AXI5<->AXI5 paths and terminates with a generation-time warning elsewhere. Atomics are store-class only: read-return classes DECERR at the boundary (axi5_atomic_filter). Details: docs/bridge_has/ch04_interfaces/04_axi5_apb5_interfaces.md and docs/bridge_mas/ch02_blocks/10_amba5_boundary.md.

Q: “What if data widths don’t match?”

A: Generator inserts width converters automatically:

masters = [
  {name = "cpu", data_width = 64, ...},  # 64b master
]
slaves = [
  {name = "ddr", data_width = 512, ...}  # 512b slave
]
# Generator creates width converter: 64b → 512b

Q: “Do all masters need to connect to all slaves?”

A: No, use partial connectivity in CSV:

master\slave,ddr,sram,apb
rapids_descr,1,1,0     # Connects to ddr and sram only
cpu,1,1,1              # Connects to all three

### 24.8.7	Generator Output Structure

Generated File Contains:

- Module Header - Parameterized with NUM_MASTERS, NUM_SLAVES, widths

- Port Declarations - Custom prefix per port, channel-specific signals

- Internal Signals - Crossbar interface arrays (xbar_m_, xbar_s_)

- Width Converters - Master-side upsize instances (channel-aware)

- Crossbar Instance - Internal AXI4 full crossbar

- Direct Connections - For matching-width interfaces

- APB Converters - Placeholder TODO comments (Phase 3)

Example Output Size: - Simple 2x2 bridge: ~400 lines - Complex 5x3 with mixed protocols: ~900 lines - Includes comprehensive comments and structure

### 24.8.8	Testing Generated Bridges

For Pure AXI4 Bridges (Working Now):

# Generate bridge (outputs to rtl/generated/<bridge_name>/)
python3 bridge_generator.py --ports test_configs/bridge_2x2_rw.toml

# Run its test (per-config TB classes live in dv/tbclasses/, e.g. bridge2x2_rw_tb.py)
cd ../dv/tests
pytest test_bridge_2x2_rw.py -v

For Mixed AXI4/APB (Requires Phase 3): APB converter placeholders need implementation before end-to-end testing.

## 24.9	Bridge Architecture Quick Reference

### 24.9.1	Generated Bridge Crossbar Structure

Illustrative shape (see rtl/generated/bridge_2x2_rw/bridge_2x2_rw.sv for a real example):

module bridge_2x2_rw #(
    parameter NUM_MASTERS = 2,
    parameter NUM_SLAVES = 2,
    parameter DATA_WIDTH = 32,
    parameter ADDR_WIDTH = 32,
    parameter ID_WIDTH = 4
) (
    input  logic aclk,
    input  logic aresetn,

    // Master-side interfaces (slave ports on bridge)
    // AW, W, B, AR, R channels for each master

    // Slave-side interfaces (master ports on bridge)
    // AW, W, B, AR, R channels for each slave
);

### 24.9.2	Key Features

- 5-Channel Implementation: Complete AW, W, B, AR, R routing

- Round-Robin Arbitration: Per-slave arbitration with grant locking

- Transaction ID Tracking: Distributed RAM for B/R response routing

- ID-Based Routing: Enables out-of-order responses

- Configurable Parameters: NxM topology, data/addr/ID widths

### 24.9.3	Address Map

Default address map (configurable): - Slave 0: 0x00000000 - 0x0FFFFFFF (256MB) - Slave 1: 0x10000000 - 0x1FFFFFFF (256MB) - Slave N: N * 0x10000000

## 24.10	Test Organization

### 24.10.1	Test Hierarchy

projects/components/bridge/dv/tests/
├── conftest.py                        # Pytest configuration with fixtures
├── test_bridge_1x2_rd.py              # Per-config functional tests
├── test_bridge_2x2_rw.py
├── test_bridge_4x4_rw.py
├── test_bridge_5x3_channels.py
├── test_bridge_mix_a_mon_monitor.py   # Monitor stress tests (mon configs)
└── monitor_stress_common.py           # Shared monitor-stress helpers

### 24.10.2	Test Levels

Per-Config Functional Tests: - Individual bridge functionality - Address routing - ID tracking - Arbitration

Monitor Stress Tests (*_mon_monitor.py): - Bridges generated with monitor interfaces - Monitor packet generation under traffic

## 24.11	Common User Questions and Responses

### 24.11.1	Q: “How does the Bridge work?”

A: Direct answer:

The Bridge AXI4 crossbar connects multiple AXI4 masters to multiple slaves:

- Address Decode (AW/AR): Routes master requests to appropriate slave based on address

- Write Path: AW → arbitration → slave, W follows locked grant

- Read Path: AR → arbitration → slave, R returns via ID table

- Arbitration: Per-slave round-robin with grant locking until burst complete

- Response Routing: B/R responses use ID lookup tables (not grant-based)

Key Features: - Out-of-order response support via ID tracking - Burst-aware arbitration (grant locked until WLAST/RLAST) - Configurable NxM topology - Single-clock domain

See: - projects/components/bridge/PRD.md - Complete specification - projects/components/bridge/bin/bridge_generator.py - Generator implementation

### 24.11.2	Q: “How do I generate a Bridge?”

A: Use the bridge_generator.py tool:

cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_2x2_rw.toml
# Auto-finds test_configs/bridge_2x2_rw_connectivity.csv
# Or regenerate everything: make all  (bulk from bridge_batch.csv)

Generated files: - RTL: projects/components/bridge/rtl/generated/<bridge_name>/ (top module, crossbar, adapters, package) - Contains complete 5-channel crossbar with ID tracking

### 24.11.3	Q: “How do I test a Bridge?”

A: Use the Bridge testbench class:

# Import from project area
import sys
from TBClasses.shared.utilities import get_repo_root
sys.path.insert(0, get_repo_root())
from projects.components.bridge.dv.tbclasses.bridge2x2_rw_tb import Bridge2x2RwTB

@cocotb.test()
async def cocotb_test_basic(dut):
    tb = Bridge2x2RwTB(dut)
    await tb.setup_clocks_and_reset()

    # Send write to slave 0
    await tb.write_transaction(master_idx=0, address=0x00001000, data=0xDEADBEEF)

    # Verify routing
    assert len(tb.aw_slaves[0]._recvQ) > 0, "Slave 0 should receive AW"

Run tests:

cd projects/components/bridge/dv/tests
pytest test_bridge_2x2_rw.py -v

## 24.12	Anti-Patterns to Avoid

### 24.12.1	Anti-Pattern 1: Embedded Testbench Classes

WRONG: TB class in test file
class BridgeTB:
    """NOT REUSABLE - WRONG LOCATION"""

CORRECT: Import from project area
import sys
from TBClasses.shared.utilities import get_repo_root
sys.path.insert(0, get_repo_root())
from projects.components.bridge.dv.tbclasses.bridge2x2_rw_tb import Bridge2x2RwTB

### 24.12.2	Anti-Pattern 2: Manual AXI4 Handshaking

WRONG: Manual signal driving
self.dut.s0_axi4_awvalid.value = 1
while self.dut.s0_axi4_awready.value == 0:
    await RisingEdge(self.clock)

CORRECT: Use GAXI components
await self.aw_master.send(aw_pkt)

### 24.12.3	Anti-Pattern 3: Memory Models for Simple Tests

WRONG: Unnecessary complexity
memory = MemoryModel()
memory.write(addr, data)
result = memory.read(addr)

CORRECT: Direct queue verification
aw_pkt = self.aw_slave._recvQ.popleft()
assert aw_pkt.addr == expected_addr

## 24.13	Quick Reference

### 24.13.1	Finding Existing Components

# Bridge TB classes (in PROJECT AREA, not framework!)
ls projects/components/bridge/dv/tbclasses/

# Bridge generator
cat projects/components/bridge/bin/bridge_generator.py

# Test examples
ls projects/components/bridge/dv/tests/

### 24.13.2	Common Commands

# Generate 2x2 bridge (from bin/, outputs to rtl/generated/bridge_2x2_rw/)
cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_2x2_rw.toml

# Run tests
pytest projects/components/bridge/dv/tests/test_bridge_2x2_rw.py -v

# Lint generated RTL
verilator --lint-only projects/components/bridge/rtl/generated/bridge_2x2_rw/bridge_2x2_rw.sv

## 24.14	Remember

- MANDATORY: Testbench architecture - TB classes in framework, tests import them

- MANDATORY: Directory structure - Follow RAPIDS/AMBA pattern exactly

- MANDATORY: conftest.py - Must exist in dv/tests/

- Use GAXI components - Never manually drive AXI4 handshakes

- Queue-based verification - Simple tests use direct queue access

- Three-layer architecture - TB (framework) + Test (runner) + Scoreboard (verification)

- Three mandatory methods - setup_clocks_and_reset, assert_reset, deassert_reset

- Search first - Use existing components before creating new ones

- Test scalability - Support basic/medium/full test levels

- 100% success - All tests must achieve 100% success rate

## 24.15	PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

projects/components/bridge/docs/

Quick Commands: Use the provided shell scripts:

cd projects/components/bridge/docs
./generate_has_pdf.sh   # Builds Bridge_HAS_v<rev> from bridge_has/
./generate_mas_pdf.sh   # Builds Bridge_MAS_v<rev> from bridge_mas/

The shell scripts will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the bridge_has/bridge_mas index files 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

See: bin/md_to_docx.py for complete implementation details

Version: 1.0 Last Updated: 2025-10-18 Maintained By: RTL Design Sherpa Project