RTL Design Sherpa

DDR2 / LPDDR2 Family Controller Hardware Architecture Specification 0.2

June 13, 2026

Table of Contents

1 Ddr2 Lpddr2 Has Index	20

2 Document Information	20

2.1 DDR2 / LPDDR2 Family Controller Hardware Architecture Specification	20

2.2 Document Purpose	20

2.3 Revision History	21

2.4 Scope of This Document	21

2.5 Out of Scope	21

3 Purpose and Scope	22

3.1 Document Purpose	22

3.2 Scope	22

3.3 Design Philosophy	23

3.4 Audience	23

4 Document Conventions	24

4.1 Typographic Conventions	24

4.2 Numbering Conventions	24

4.3 Signal Direction Conventions	24

4.4 Module Diagram Conventions	25

4.5 Cross-Reference Conventions	25

5 Definitions and Acronyms	25

5.1 Acronyms	25

5.2 Defined Terms	28

6 Scope and Goals	29

6.1 High-Level Goals	29

6.2 In Scope	29

6.3 Out of Scope	30

6.4 Target Performance Envelope	31

6.5 Differentiating Features	31

6.5.1 Features Adopted from Standard Memory-Controller Patterns	33

7 Block Diagram	33

7.1 Top-Level Block Diagram	33

7.2 Data Flow Summary	34

8 Module Hierarchy	35

8.1 Hierarchy Tree	35

8.2 Module Functional Groupings	35

8.3 Memtype-Conditional Modules	36

8.4 Optional Modules	36

9 Top-Level Interfaces	37

9.1 Parameter-Dependent Widths	37

9.2 1. AXI4 Slave (Host-side)	38

9.2.1 Write Address Channel (AW)	38

9.2.2 Write Data Channel (W)	38

9.2.3 Write Response Channel (B)	38

9.2.4 Read Address Channel (AR)	39

9.2.5 Read Data Channel (R)	39

9.3 2. DFI v2.1 Master (PHY-side)	40

9.3.1 DFI Command Sub-Interface (Per-Phase)	40

9.3.2 DFI Write-Data Sub-Interface	40

9.3.3 DFI Read-Data Sub-Interface	41

9.3.4 DFI Status and Update Sub-Interface	41

9.4 3. APB CSR Slave (Configuration / Observation)	41

9.5 4. Clocks, Resets, Interrupts, Debug	42

9.6 Hierarchical Top-Level View	42

10 FUB Breakdown (First-Pass SWAG)	43

10.1 Stream FUB Convention	43

10.2 FUB Inventory	44

10.2.1 Group A — AXI Frontend	44

10.2.2 Group B — Transaction Buffering and Scheduling	44

10.2.3 Group C — Per-Bank State and Cross-Bank Timing	45

10.2.4 Group D — Refresh, Init, Power	46

10.2.5 Group E — Command Encoding and Output Gear	46

10.2.6 Group F — Data Paths	47

10.2.7 Group G — CSR	47

10.3 FUB Count Summary	47

10.4 Integration	48

11 AXI4 Frontend	48

11.1 axi4_slave	48

11.1.1 Purpose	48

11.1.2 Internal State	48

11.1.3 Behavior	49

11.1.4 Per-ID Ordering	49

11.1.5 Burst Splitting	49

11.1.6 Backpressure	49

11.2 addr_mapper	50

11.2.1 Purpose	50

11.2.2 Interfaces	50

11.2.3 Mapping Function	50

11.2.4 Alternative Mappings	50

11.2.5 Relationship to the DFI BFM	51

12 Scheduler and Page Predictor	51

12.1 txn_queue	51

12.1.1 Purpose	51

12.1.2 Entry Format	51

12.1.3 Depth	52

12.1.4 Behavior	52

12.2 scheduler	52

12.2.1 Purpose	52

12.2.2 Inputs	52

12.2.3 Outputs	52

12.2.4 Priority Function	53

12.2.5 Strict In-Order Mode	53

12.2.6 Auto-Precharge Decision: Lookahead + Page Policy	54

12.2.7 Issue Slot Allocation	57

12.3 page_predictor (HAPPY hybrid, optional)	57

12.3.1 Purpose	57

12.3.2 Algorithm	57

12.3.3 HAPPY Predictor Architecture	58

12.3.4 Storage	58

12.3.5 Reset Behavior	58

12.3.6 Observability	59

13 Bank Machines and Cross-Bank Timers	59

13.1 bank_machine	59

13.1.1 Purpose	59

13.1.2 Instantiation	59

13.1.3 FSM States	59

13.1.4 FSM Diagram	60

13.1.5 Per-Bank Registers	60

13.1.6 Outputs to Scheduler	61

13.1.7 Refresh Handshake Interface	61

13.1.8 Inputs from Cross-Bank Timers	62

13.1.9 Auto-Precharge Handling	62

13.2 xbank_timers	62

13.2.1 Purpose	62

13.2.2 Tracked Constraints	62

13.2.3 Implementation	62

13.2.4 Outputs	63

13.2.5 Why Shared Rather Than Per-Bank	63

13.2.6 Per-Rank vs. Cross-Rank Scope	63

14 Refresh Manager	64

14.1 refresh_mgr	64

14.1.1 Purpose	64

14.1.2 Internal State	64

14.1.3 Postponer Behavior (Deterministic Batching)	65

14.1.4 Temperature-Compensated Refresh (LPDDR2)	65

14.1.5 Controller-Level Refresh FSM	65

14.1.6 Periodic ZQCS Piggybacked on Refresh	67

14.1.7 Temperature-Compensated Refresh (LPDDR2)	67

14.1.8 Refresh Decision Flow	68

14.2 REFab Path (DDR2 or LPDDR2 fallback)	70

14.3 REFpb Path (LPDDR2, default)	70

14.3.1 Bank Selection Policy	70

14.3.2 DARP Rationale	71

14.3.3 REFpb Issue Sequence	71

14.4 Refresh Priority to Scheduler	71

14.5 Self-Refresh Coordination	72

14.6 PASR Update Protocol	72

15 Init Engine and Power State	72

15.1 init_engine and Step Tables	73

15.1.1 Purpose	73

15.1.2 Microprogram Instruction Format	73

15.1.3 Step Tables	73

15.1.4 Init Sequence Outlines	73

15.1.5 Outputs	74

15.1.6 Init Engine Flowchart	74

15.1.7 SIM_INIT_SCALE	74

15.1.8 Restart on Warm Reset	75

15.1.9 Error Handling	75

15.1.10 MR Value Configurability	75

15.2 power_state (Top-Level FSM)	75

15.2.1 Purpose	75

15.2.2 FSM States	75

15.2.3 Power State Diagram	76

15.2.4 Transitions	76

15.2.5 Step Tables for Power Transitions	76

15.2.6 DPD Entry Constraints	77

15.2.7 Self-Refresh Exit Latency	77

15.2.8 Power-State Observation	77

16 Command Encoder and Gear Output	77

16.1 cmd_encoder	77

16.1.1 Purpose	77

16.1.2 Input Record	78

16.1.3 DDR2 Encoder	78

16.1.4 LPDDR2 Encoder	78

16.1.5 Memtype Selection	79

16.1.6 Idle Defaults	79

16.1.7 Verification	79

16.2 gear_out	79

16.2.1 Purpose	79

16.2.2 Behavior	79

16.2.3 Multi-Cycle Commands	80

16.2.4 Phase Naming	80

16.2.5 Write / Read Data Phase Assignment	80

16.2.6 Idle Phase Drives	80

16.2.7 Verification	80

17 Write and Read Data Paths	81

17.1 wdata_path	81

17.1.1 Purpose	81

17.1.2 Inputs	81

17.1.3 Outputs	81

17.1.4 Behavior	81

17.1.5 Write Command Pipeline	82

17.1.6 B-Response Generation	82

17.1.7 Write Cancellation	82

17.2 rdata_path	82

17.2.1 Purpose	82

17.2.2 Inputs	83

17.2.3 Outputs	83

17.2.4 Behavior	83

17.2.5 Read Command Pipeline	84

17.2.6 Out-of-Order Completion	84

17.2.7 Read Error Handling	84

17.2.8 Read Data Capture Timing	84

17.2.9 Stale Read Suppression	85

18 DFI Master and CSR Slave	85

18.1 dfi_master	85

18.1.1 Purpose	85

18.1.2 Sub-Interfaces Driven	85

18.1.3 Sub-Interfaces Tied Idle	85

18.1.4 Signal Aggregation	86

18.1.5 Why a Dedicated Module	86

18.2 csr_slave	86

18.2.1 Purpose	86

18.2.2 Bus Protocol	86

18.2.3 Address Map Overview	87

18.2.4 Clock Domain Crossing	87

18.2.5 Read Latency	87

18.2.6 Configuration Quiet Period	87

18.2.7 Recommended Programming Sequence	87

19 AXI4 Slave Interface	88

19.1 Channels	88

19.1.1 AW (Write Address) — SoC master → controller slave	88

19.1.2 W (Write Data)	89

19.1.3 B (Write Response)	89

19.1.4 AR (Read Address)	89

19.1.5 R (Read Data)	90

19.2 Ordering Rules	90

19.3 Burst Type Support	90

19.4 Burst Length	90

19.5 Narrow Transfers	91

19.6 Posted Writes	91

19.7 Backpressure	91

19.8 AXI4 Reset	91

20 DFI v2.1 Master Interface	91

20.1 Per-Phase Signal Naming	91

20.2 Command Interface (MC → PHY)	92

20.3 Write Data Interface (MC → PHY)	92

20.4 Read Data Interface (PHY ↔ MC)	92

20.5 Status Interface	93

20.6 Sub-Interfaces NOT Driven	93

20.7 Reset	93

20.8 Timing Parameters Honored	93

20.9 Clock Domain	94

21 APB CSR Slave	94

21.1 Signals	94

21.2 APB3 Conformance	94

21.3 Error Behavior	94

21.4 Clock Domain Crossing	95

21.5 Register Map	95

21.6 Configuration Sequencing	95

22 Build-Time vs. Runtime Configuration	96

22.1 Principle	96

22.2 Why This Distinction Matters	96

22.3 The Hybrid Pattern: “Build-Time MAX + Runtime Active”	97

22.4 The “Build-Time + Runtime Mux” Pattern	97

22.5 The “Runtime Only” Pattern	98

22.6 Configuration Quiet Points	98

22.7 Summary	98

23 Top-Level Parameter Table	99

23.1 Parameter Table	99

23.2 Memtype-Dependent Constraints	104

23.3 Timing-Configuration Sanity Checks	105

23.4 Parameter Categories	105

24 Characterization Knobs	106

24.1 Characterization Parameters	106

24.2 Sweep Plan	107

24.2.1 Benchmark Suite	107

24.2.2 Metrics Collected	108

24.2.3 Default Selection	108

24.3 Runtime Overrides via CSR	108

24.4 Observability for Sweep	109

25 Family-Wide Config Bits	109

25.1 Why a Family-Wide Registry	109

25.2 Applicability Notation	110

25.3 SCHED_TUNING (Scheduling Family Bits)	110

25.4 REFRESH_TUNING (Refresh Family Bits)	111

25.5 ADDR_MAP_TUNING (Address-Mapping Family Bits)	111

25.6 INIT_TUNING (Init Family Bits)	112

25.7 POWER_TUNING (Power-State Family Bits)	112

25.8 ECC_TUNING (Future — Inline ECC)	113

25.9 RANK_TUNING (Multi-Rank Family Bits)	113

25.10 Quiet-Point Behavior	113

25.11 Bit Discovery via the Capability Vector	114

25.12 Per-Flavor Applicability Matrix	114

26 Clocks and Reset	115

26.1 Clock Domains	115

26.1.1 mc_clk	115

26.1.2 phy_clk	115

26.1.3 apb_pclk	116

26.2 Reset Signals	116

26.2.1 mc_rstn (active low)	116

26.2.2 phy_rstn (active low)	116

26.2.3 dfi_rstn (active low, MC → PHY)	116

26.2.4 apb_presetn (active low)	116

26.3 Reset Sequencing	116

26.3.1 Cold Boot	116

26.3.2 Warm Reset (Clock-Gated)	117

26.4 Clock Domain Crossing Summary	117

26.5 Reset and Power Considerations	117

27 Init Sequences	118

27.1 DDR2 Cold Init	118

27.2 LPDDR2 Cold Init	119

27.3 Self-Refresh Entry (Both Memtypes)	119

27.4 Self-Refresh Exit (Both Memtypes)	120

27.5 LPDDR2 Deep Power Down Entry	120

27.6 LPDDR2 Deep Power Down Exit	120

27.7 Sim Scaling	120

28 CSR Register Map	121

28.1 Control and Status (0x000 – 0x00F)	121

28.1.1 CTRL (0x000, R/W)	121

28.1.2 STATUS (0x004, R only)	121

28.1.3 STATUS_HISTORY (0x008, R only)	121

28.2 Timing Parameters (0x010 – 0x01F)	121

28.2.1 TIMINGS_RC_RCD_RP (0x010, R/W)	122

28.2.2 TIMINGS_RAS_RFC_REFI (0x014, R/W)	122

28.2.3 TIMINGS_RRD_FAW_WTR (0x018, R/W)	122

28.2.4 TIMINGS_CL_CWL_WR (0x01C, R/W)	122

28.3 Mode Register Values (0x020 – 0x02F)	122

28.3.1 MR0, MR1, MR2, MR3 (0x020, 0x024, 0x028, 0x02C, R/W)	122

28.4 LPDDR2-Specific (0x030 – 0x03F)	123

28.4.1 PASR_BANK_MASK_RANK<R> (0x030 + R×4, R/W; R = 0 .. NUM_RANKS−1)	123

28.4.2 PASR_SEG_MASK_RANK<R> (0x080 + R×4 within the multi-rank reserved area is reused — see register layout note)	123

28.4.3 TEMP_DERATE_RANK<R> (0x038 + R×4, R only)	123

28.5 Scheduler / Refresh / Page Tuning (0x040 – 0x04F)	123

28.5.1 SCHED_TUNING (0x040, R/W)	123

28.5.2 PAGE_PRED_TUNING (0x044, R/W, only when HAPPY enabled)	124

28.5.3 REFRESH_TUNING (0x048, R/W)	124

28.5.4 ADDR_MAP_TUNING (0x04C, R/W)	125

28.5.5 INIT_TUNING (0x050, R/W)	125

28.6 Per-(Rank, Bank) Observation (0x080 – 0x0FF)	125

28.6.1 OBS_ROW_HIT_RANK<R>_BANK<N> (0x080 + (R × NUM_BANKS + N) × 4, R only)	125

28.6.2 OBS_REF_LATENCY_RANK<R>_BANK<N> (0x0C0 + (R × NUM_BANKS + N) × 4, R only)	126

28.7 System Observation (0x100 – 0x1FF)	126

28.8 Module Identification (0xFF0 – 0xFFC)	127

28.8.1 ID (0xFF0, R only)	127

28.8.2 BUILD (0xFF4, R only)	127

29 Verification Strategy	127

29.1 Layered Approach	127

29.1.1 Layer 1: Module-Level Unit Tests	127

29.1.2 Layer 2: Subsystem Tests	128

29.1.3 Layer 3: End-to-End with DFI BFM Slave	128

29.1.4 Layer 4: External-Reference Cross-Validation	129

29.2 Characterization Sweep	129

29.2.1 Sweep Matrix	129

29.2.2 Sweep Outputs	129

29.2.3 Out-of-Order Scheduler Trade-Offs	130

29.3 Verification Hooks	131

29.4 Coverage Targets	131

30 Open Issues and TODOs	132

30.1 Open Issues	132

30.1.1 1. AXI4 Narrow-Burst Handling Confirmation	132

30.1.2 2. AXI4 Read-Modify-Write Strobe Handling	132

30.1.3 3. APB CSR Address Space Allocation	132

30.1.4 4. Per-Bank Refresh Book-Keeping Cost	132

30.1.5 5. HAPPY Predictor Hash Function	132

30.1.6 6. Self-Refresh Exit Latency	132

30.1.7 7. Init Engine Retry on Error	133

30.1.8 8. CSR Write Atomicity	133

30.1.9 9. Burst Splitting at Row Boundaries	133

30.1.10 10. Multi-Rank	133

30.1.11 11. ECC	133

30.1.12 12. DFI Training Sub-Interface	133

30.1.13 13. AXI Quality of Service (QoS)	133

30.1.14 14. Multi-Master AXI	133

30.1.15 15. Observation Counter Overflow	134

30.2 TODOs Before v0.2	134

List of Figures

No figures in this document.

List of Tables

No tables in this document.

# 1	Ddr2 Lpddr2 Has Index

Generated: 2026-06-14

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	DDR2 / LPDDR2 Family Controller Hardware Architecture Specification

Document Number: DDR2-LPDDR2-HAS-001 Version: 0.2 Status: Draft - First Sketch Classification: Open Source - Apache 2.0 License

## 2.2	Document Purpose

This Hardware Architecture Specification (HAS) provides a high-level architectural overview of the DDR2 / LPDDR2 unified memory controller family. It describes the system-level design, external interfaces, characterization parameters, and integration requirements without detailing internal implementation specifics.

Target Audience:

- System architects evaluating the controller for SoC integration

- Hardware engineers planning RTL implementation

- Verification engineers planning system-level testing

- Software engineers developing low-level memory configuration

Companion Documents:

- DDR2/LPDDR2 Family Controller Pre-Architecture Spec (../pre-aspec.md) — bullet-form architecture rationale

- DDR2/LPDDR2 Paging and Refresh Notes (../paging-refresh-notes.md) — research-derived parameter sources

## 2.3	Revision History

| Version | Date | Author | Notes |
| --- | --- | --- | --- |
| 0.1 | 2026-06-13 | RTL Design Sherpa | First sketch — module decomposition |
| 0.2 | 2026-06-14 | RTL Design Sherpa | Top-level interface port list (§2.4), FUB SWAG (§2.5), family-wide config-bit registry (§5.4), multi-rank support across §2 / §3.3 / §3.4 / §3.6 / §5.2 / §6.3 |

## 2.4	Scope of This Document

This HAS describes a single parameterized memory controller covering DDR2 and LPDDR2. It defines:

- External interfaces (AXI4 slave, DFI v2.1 master, APB CSR slave)

- Module-level decomposition and behavioral specification

- Build-time and run-time configuration parameters

- Initialization, power-state, and refresh policies

- Verification strategy and characterization plan

It does not define:

- Bit-level pin assignments (the SystemVerilog port list in regs/ddr2_lpddr2_ctrl_ports.svh is the canonical wire-level source)

- Detailed timing diagrams (deferred to v0.3)

- Verilog package skeletons or RTL stubs

- Floorplan or layout guidance

## 2.5	Out of Scope

The following features are explicitly out of scope for this version of the controller and will be addressed in higher-generation family controllers or future revisions:

- Inline ECC

- DFI training, frequency-change, and low-power sub-interfaces

- AXI exclusive access semantics

- Bank groups (not present in DDR2 / LPDDR2)

- Command/Address parity, CRC, Data Bus Inversion (not present in DDR2 / LPDDR2)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Purpose and Scope

## 3.1	Document Purpose

This Hardware Architecture Specification (HAS) defines the high-level architecture of the DDR2 / LPDDR2 Family Memory Controller. It serves as the primary reference for:

- System Integration — understanding the controller’s external interfaces and system-level requirements

- RTL Implementation Planning — module decomposition, interface contracts, and parameter sweeps

- Verification Planning — system-level test scenarios and the characterization sweep matrix

- Driver Development — programming model and APB register interface

This document complements the upcoming Micro-Architecture Specification (MAS), which will provide detailed block-level implementation specifics, signal-level pinouts, and timing diagrams.

## 3.2	Scope

This HAS covers a single unified controller that supports both DDR2 and LPDDR2 memory via a build-time MEMTYPE parameter. The shared engine includes:

- AXI4 slave frontend

- Address mapper

- Transaction queue with row-hit caching

- FR-FCFS scheduler with parameterizable page policy

- Per-bank state machines with local timing enforcement

- Cross-bank timer pool (tRRD / tFAW / tWTR / tCCD)

- Refresh manager with Elastic deferral and DARP-style per-bank scheduling

- Top-level power-state FSM

- Microprogram-based initialization engine

- Pluggable command encoder (DDR2 ras/cas/we OR LPDDR2 CA bus)

- N-phase output gear stage (N_PHASES ∈ {1, 2, 4})

- AXI write / read data paths with CWL / CL alignment

- DFI v2.1 master driver

- APB CSR slave for configuration and observation

The only differences between DDR2 and LPDDR2 build configurations are the command encoder, the init step table, the power-state FSM (specifically Self-Refresh vs Deep Power Down handling), and a small number of mode-register signals. Everything else is shared.

## 3.3	Design Philosophy

This controller is intended as a characterization-first design. Algorithmic choices that have meaningful research-backed alternatives — page policy, per-bank refresh policy, refresh deferral depth, transaction queue depth — are exposed as parameters rather than hardcoded. The verification plan includes a benchmark sweep over representative AXI traffic patterns to pick defaults from data, not assumption.

The two specific characterization knobs informed by external research are:

- PAGE_POLICY ∈ {OPEN, CLOSE, HAPPY_HYBRID} — where HAPPY_HYBRID is the address-bit-based page-closure predictor from Ghasempour et al. (2015).

- REFPB_POLICY ∈ {ROUND_ROBIN, OLDEST_FIRST, DARP} — where DARP is the dynamic access refresh parallelization scheme from Chang et al. (HPCA 2014).

These are described in detail in Chapter 5.

## 3.4	Audience

This document assumes the reader is familiar with:

- AXI4 protocol semantics (handshakes, burst types, ID-based ordering)

- DFI v2.1 specification (control / write-data / read-data sub-interfaces)

- JEDEC DDR2 (JESD79-2F) and LPDDR2 (JESD209-2F) command tables

- Basic memory-controller concepts (open vs closed page policy, refresh deadlines, bank state machines)

Readers new to these topics should first consult the companion paging-refresh-notes.md in the family working area for an entry-point bibliography.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Document Conventions

## 4.1	Typographic Conventions

| Style | Meaning |
| --- | --- |
| monospace | Module names, parameter names, signal names, register names |
| bold | Defined terms on first introduction; important warnings |
| italic | Emphasis; placeholder values to be replaced |
| MEMTYPE | Top-level build-time parameter (all caps) |
| bank_machine | RTL module name (lowercase with underscores) |
| tRC, tRCD | JEDEC timing parameters (as in JESD79-2 / JESD209-2) |

## 4.2	Numbering Conventions

- Decimal numbers have no prefix: 16, 256.

- Hexadecimal numbers are prefixed with 0x: 0x800, 0xFF.

- Binary literals use Verilog notation when needed: 8'b1011_0010.

- Bit ranges use the convention [MSB:LSB], e.g., dfi_address[19:0].

## 4.3	Signal Direction Conventions

For DFI signals, direction is from the memory controller’s perspective:

- MC → PHY — driven by the controller

- PHY → MC — driven by the PHY (sampled by controller)

For AXI signals, direction follows the standard AXI4 convention with the controller as the slave.

## 4.4	Module Diagram Conventions

ASCII block diagrams in this document use the following conventions:

All diagrams in this document are rendered from Mermaid source files in assets/mermaid/. The Mermaid source files (.mmd) are linked alongside each rendered image so the diagrams can be edited and re-rendered.

- Rectangular boxes with sharp corners denote RTL modules.

- Subgraph rectangles group modules by functional layer.

- Solid arrows show primary data or control flow.

- Dashed arrows show configuration or observation paths (typically from the CSR slave to other modules).

- Trapezoidal nodes denote external interface ports (AXI, DFI, APB).

## 4.5	Cross-Reference Conventions

- References to other chapters: “see Chapter 4”

- References to specific sections: “see §4.1” or “see Section 4.1”

- References to other documents: “see pre-aspec.md” (relative path)

- References to external standards: “per JESD79-2F §x.y”

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Definitions and Acronyms

## 5.1	Acronyms

| Acronym | Expansion |
| --- | --- |
| AHB | Advanced High-performance Bus (ARM AMBA) |
| APB | Advanced Peripheral Bus (ARM AMBA) |
| AP | Auto-Precharge (DRAM column command modifier) |
| APD | Active Power Down |
| AXI | Advanced eXtensible Interface (ARM AMBA) |
| BL | Burst Length |
| CA | Command/Address (LPDDR2/3/4 multiplexed bus) |
| CDC | Clock Domain Crossing |
| CL | CAS Latency |
| CMD | Command |
| CSR | Control/Status Register |
| CWL | CAS Write Latency |
| DARP | Dynamic Access Refresh Parallelization |
| DBI | Data Bus Inversion |
| DDR | Double Data Rate |
| DDR2 | Double Data Rate 2 (JESD79-2) |
| DFI | DDR PHY Interface |
| DPD | Deep Power Down (LPDDR2) |
| FSM | Finite State Machine |
| FR-FCFS | First-Ready, First-Come-First-Served |
| HAPPY | Hybrid Address-based Page Policy in DRAMs |
| HAS | Hardware Architecture Specification |
| LPDDR2 | Low Power DDR 2 (JESD209-2) |
| MAS | Micro-Architecture Specification |
| MC | Memory Controller |
| MR / MRS | Mode Register / Mode Register Set command |
| EMR / EMRS | Extended Mode Register / Extended MRS |
| NOP | No Operation |
| ODT | On-Die Termination |
| OOO | Out-of-Order |
| PASR | Partial Array Self-Refresh (LPDDR2 feature) |
| PHY | Physical Layer (DRAM electrical interface) |
| PRE | Precharge command |
| PREA | Precharge All Banks |
| RD | Read column command |
| RDA | Read with Auto-Precharge |
| REF | Refresh command |
| REFab | All-Bank Refresh (LPDDR2) |
| REFpb | Per-Bank Refresh (LPDDR2) |
| SDRAM | Synchronous Dynamic RAM |
| SoC | System on Chip |
| SR | Self-Refresh |
| TCSR | Temperature-Compensated Self-Refresh (LPDDR2) |
| WR | Write column command |
| WRA | Write with Auto-Precharge |
| ZQ | Impedance Calibration (ZQ Calibration) |

## 5.2	Defined Terms

Bank Machine — A finite state machine instance dedicated to one DRAM bank that enforces JEDEC timing constraints locally and tracks open-row state.

Characterization Knob — A build-time or runtime parameter intentionally exposed for performance sweeping during system characterization. Page policy and refresh policy are the two principal knobs in this controller.

Command Encoder — The module that translates the controller’s internal generic command record into DFI wire signals. Two implementations: ddr2_encoder and lpddr2_encoder. Selected at elaboration time via MEMTYPE.

FR-FCFS — First-Ready, First-Come-First-Served. The baseline DRAM scheduling policy: ready commands beat unready, row-hits beat row-misses, older entries beat younger on ties. Established by Rixner et al. (ISCA 2000).

Gear Output Stage — The module that replicates a 1-phase internal command bus into N_PHASES parallel DFI output phases.

Microprogram Engine — The init sequencer: a small instruction-decoded FSM that reads a memtype-specific ROM (step table) and issues the corresponding control bits, MR loads, and commands.

Page Hit / Row Hit — An access whose target row matches the currently open row in the target bank. Avoids the tRP + tRCD penalty.

Page Conflict / Row Miss — An access whose target row differs from the currently open row in the target bank. Pays tRP + tRCD before the access can begin.

Page Policy — The strategy for when to close an open row. See PAGE_POLICY parameter.

Row-Hit Caching — Storing the row-hit / row-miss decision on each transaction at queue insertion time, rather than recomputing each scheduler tick.

Transaction Queue — The pending-request buffer between the AXI frontend and the scheduler.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Scope and Goals

## 6.1	High-Level Goals

This controller provides a single parameterized RTL implementation supporting both DDR2 and LPDDR2 memory devices via DFI v2.1. Primary design goals:

- Unified engine, swappable encoder. DDR2 and LPDDR2 differ only at the wire-encoding layer (ras/cas/we vs CA bus); the bank machines, scheduler, refresh policy, init engine, and AXI frontend are shared between both targets.

- Embedded / mobile / mid-range SoC targets. Not optimized for high-end server DRAM (DDR5, RDIMM, multi-rank). Optimized for power efficiency and area economy.

- Characterization-first parameterization. Algorithmic choices with research-backed alternatives are exposed as build-time parameters so the design can be swept on real workloads before defaults are locked.

- Verification-friendly. Mirrors the data structures and conventions of the DDR2 / LPDDR2 DFI BFM in the DV repository so the same address mapping, timing parameters, and command encodings can drive both RTL and simulation.

## 6.2	In Scope

The following features are explicitly within scope for this controller:

- AXI4 slave with 64 / 128 / 256-bit data width, 1–16-bit ID width, INCR burst type

- DFI v2.1 master driving the command, write-data, and read-data sub-interfaces

- APB CSR slave for configuration and observation

- DDR2 and LPDDR2 device support via MEMTYPE build parameter

- DFI frequency ratio (gear) of 1, 2, or 4 phases via N_PHASES parameter

- Multi-rank operation — 1, 2, or 4 ranks via NUM_RANKS parameter; per-rank CS_n, CKE, ODT, and refresh tracking; rank-aware ODT cross-termination rules per JEDEC

- Page policies: OPEN, CLOSE, HAPPY hybrid predictor (parameterized)

- Per-bank refresh for LPDDR2 with DARP-style scheduling (parameterized)

- All-bank refresh for DDR2 (and as LPDDR2 fallback)

- Elastic refresh deferral up to JEDEC ceiling of 8× tREFI (parameterized)

- Partial-array self-refresh (PASR) for LPDDR2 with CSR-configurable mask

- Temperature-compensated refresh (LPDDR2 MR4 derating)

- Cold-boot initialization via microprogram step tables

- Power state management (Active, Active Power-Down, Self-Refresh, Deep Power Down)

- Bring-up observability via CSR-exposed counters (row-hit rate, refresh latency, queue depth)

## 6.3	Out of Scope

Explicitly not addressed in this controller version:

| Feature | Reason |
| --- | --- |
| Inline ECC | Out-of-controller responsibility (SoC sideband) |
| DFI training sub-interface | Not required for v2.1 DDR2 / LPDDR2 operation |
| DFI frequency-change protocol | No use case for this generation |
| DFI low-power sub-interface | Power-state FSM uses CKE only |
| AXI exclusive accesses | Embedded targets don’t require |
| Bank groups | Not present in DDR2 / LPDDR2 (introduced in DDR4 / LPDDR4) |
| Command/Address parity | Not present in DDR2 / LPDDR2 (introduced in DDR4 / LPDDR4) |
| Write CRC | Not present in DDR2 / LPDDR2 |
| Data Bus Inversion (DBI) | Not present in DDR2 / LPDDR2 |
| DDR3 write leveling | Not present in DDR2 / LPDDR2 (LPDDR3 introduces) |
| DDR3 ZQCS / ZQCL on column | DDR2 uses OCD calibration; LPDDR2 uses MR10 ZQ |
| Higher DDR / LPDDR generations | Will be addressed in DDR3-LPDDR3 and DDR4-LPDDR4 controllers |

## 6.4	Target Performance Envelope

| Metric | Target |
| --- | --- |
| DRAM data rate | Up to DDR2-800 / LPDDR2-1066 |
| AXI clock | Up to 200 MHz (typical embedded SoC) |
| Sustained read bandwidth | ≥ 70% peak on stream workloads |
| 99th-percentile read latency | ≤ 200 ns (typical) |
| Refresh blocking overhead | < 5% (LPDDR2 with DARP); < 8% (DDR2) |
| Area | < 50K gates (excluding HAPPY predictor) |

These are target envelopes for v1; not guarantees. They will be validated and refined during the characterization sweep.

## 6.5	Differentiating Features

This controller is informed by published memory-controller literature and the open-source memory-controller ecosystem (LiteDRAM in particular — used as the cross-validation reference; see §6.4), but it is not a re-implementation of any existing controller. The design’s distinguishing features:

- Centralized FR-FCFS scheduler with full out-of-order issue. A single scheduler scans the full transaction queue every cycle, considering all banks’ readiness and row-hit state in one decision. This is closer to the commercial-vendor pattern (Synopsys, Cadence, Xilinx MIG) than to most open-source RTL controllers — open-source controllers typically expose OoO only at the cross-bank level (a per-bank FIFO model where bank parallelism is the only source of reordering). True per-port within-stream reordering — where a younger row-hit request bypasses an older row-conflict request — is uncommon outside vendor IP. AXI4 per-ID ordering is honored at the response side regardless. For real-time / safety-critical / formally-verifiable systems, OoO reordering can be disabled at build time via SCHEDULER_MODE = INORDER (§3.2); the scheduler degrades to a first-ready FIFO that preserves arrival order at the cost of 10–25% sustained bandwidth on streaming workloads.

- Multi-rank support with JEDEC-compliant cross-rank ODT. Most open-source DDR2/3 controllers are single-rank only — LiteDRAM’s default config, UberDDR3, and most academic prototypes assume one rank per channel. This controller supports 1, 2, or 4 ranks via the NUM_RANKS build parameter. Per-rank CS_n, CKE, and ODT signals are driven; the rank-aware refresh manager tracks last_ref_age per (rank, bank); and the cross-rank ODT termination rules (read from rank N → ODT-high on rank ≠N during the read window; write to rank N → ODT-high on rank N during the write window) are handled by an ODT-control sub-module per JESD79-2 §x.y. Multi-rank is what makes the controller usable on real DIMM-class platforms rather than just on-board point-to-point DRAM.

- Unified DDR2 + LPDDR2 engine with a swappable command encoder. A single controller core supports both memtypes via the MEMTYPE build parameter. The LPDDR2 path includes the 20-bit CA bus encoder (DFI v2.1 §3.1 Table 1), PASR mask handling (MR16/17), per-bank refresh (REFpb), temperature-compensated refresh (MR4 derating), and Deep Power Down — none of which are present in the open-source DDR2/3/4 reference controllers.

- DARP-style per-bank refresh scheduling — picks the idle bank with the longest time since last refresh, falling back to oldest-first when no banks are idle. Based on Chang et al. (HPCA 2014); not present in available open-source controllers.

- Two-stage page management — exact lookahead first, history-based fallback second.

  - Stage 1: lookahead window over the transaction queue (LOOKAHEAD_DEPTH ∈ {0, 1, 2, 4}) provides exact auto-precharge decisions when the next same-bank request is already queued.

  - Stage 2: when lookahead is inconclusive (shallow queue or bursty traffic), the HAPPY hybrid page predictor (Ghasempour et al. 2015) uses address-bit-hashed saturating counters to predict the next access. Open-source controllers typically have either lookahead OR a fixed page policy. The two-stage combination is original to this design.

- Characterization-first parameterization. Page policy, lookahead depth, refresh policy, refresh deferral, queue depth, address mapping scheme, and HAPPY table size are all build-time parameters with a defined sweep matrix (see §5 and §6.4). Most published controllers expose only a subset of these as parameters; few expose them all.

- Closed-page policy as an explicit option. PAGE_POLICY ∈ {OPEN, CLOSE, HAPPY_HYBRID}. Most open-source controllers expose only on/off auto-precharge (effectively OPEN with optional lookahead). The CLOSE option is useful for adversarial / security-sensitive workloads where row-state side channels matter.

- Three-scheme parameterized address mapping — ADDR_MAP_SCHEME ∈ {ROW_MAJOR, BANK_INTERLEAVE, XOR_HASH}. The XOR_HASH scheme defeats pathological row-conflict patterns and is rarely available out-of-the-box.

- AXI4-first frontend with ID-aware out-of-order completion. The AXI integration is a first-class design constraint rather than a wrapper on a native protocol.

- Mirrored verification address-mapping object. The addr_mapper mirrors a Python AddressMapping class used by the DFI BFM, so the same mapping function drives both RTL elaboration and simulation. This eliminates a common source of address-decode bugs between RTL and verification.

### 6.5.1	Features Adopted from Standard Memory-Controller Patterns

These design patterns are widespread in the memory-controller literature and the open-source controllers we examined. Adopting them is the right call; they are noted here so the design rationale is transparent:

- Per-bank state machines with local timing enforcement + central cross-bank timer pool (tRRD / tFAW / tWTR / tCCD) — standard.

- FR-FCFS scheduler baseline (Rixner et al. ISCA 2000) — the academic baseline.

- Elastic Refresh deferral (Stuecheli et al. MICRO 2010) — postponer with deterministic back-to-back drain.

- Refresh-grant handshake (refresh_req / refresh_gnt) between refresh manager and bank machines — established pattern.

- Periodic ZQCS piggybacked on the refresh window — established pattern for systems with thermal drift.

- Microprogram-driven init engine with per-memtype step-table ROMs — established pattern.

The design rationale for adopting versus extending each is documented in the relevant section.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	Block Diagram

## 7.1	Top-Level Block Diagram

The controller’s top-level data and control flow is shown below. AXI4 traffic enters at top-left, flows through the address mapper and transaction queue into the scheduler, gets converted to per-bank commands and routed through the encoder and gear stage out to the DFI PHY interface. The APB CSR slave (bottom) provides configuration and observation in parallel.

Top-Level Block Diagram

Source: 01_block_diagram.mmd

## 7.2	Data Flow Summary

Write path: 1. AXI master issues an AW transaction; AXI4 slave accepts and queues. 2. The address mapper translates the flat AXI address into (bank, row, column) coordinates. 3. The transaction queue caches the request with row-hit metadata determined from the bank machines’ current state. 4. The scheduler picks the next eligible transaction; consults the page predictor if enabled. 5. The scheduler issues ACT, then WR (or WRA) through the command encoder. 6. The gear output stage replicates the 1-phase command onto N parallel DFI phases. 7. CWL cycles after the WR command, the write data path pulls W-channel beats and drives them onto DFI wrdata. 8. A B-response is generated to the AXI master after the WR is scheduled (posted-write semantics).

Read path: 1. AXI master issues an AR transaction; AXI4 slave accepts and queues. 2. Address mapping and queue caching identical to write path. 3. The scheduler issues ACT, then RD (or RDA) through the command encoder. 4. CL cycles after the RD command, the read data path samples DFI rddata for the burst duration. 5. Read beats are tagged with the original AXI ID and streamed back on the R channel.

Refresh path: 1. The refresh manager’s tREFI counter elapses, incrementing refresh_pending. 2. When refresh_pending exceeds the soft threshold, a refresh request enters the scheduler queue. 3. For DDR2 or LPDDR2 REFab: scheduler waits for all banks to be idle, then issues REF; all bank machines enter REFRESHING state. 4. For LPDDR2 REFpb under DARP: scheduler picks the idle bank with the oldest last-refresh timestamp; issues REFpb to that bank only; other banks remain available for column commands.

Init / power path: 1. On cold reset, the power-state FSM transitions to INIT and starts the init engine. 2. The init engine executes the memtype-specific step table, driving CKE/ODT/RESET_N and issuing MRS commands via the command encoder. 3. On completion (init_done), the FSM transitions to ACTIVE and the scheduler begins servicing AXI traffic. 4. Power-down / self-refresh entry is requested by the SoC via CSR and orchestrated by the FSM.

Detailed per-module behavior follows in Chapter 3.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Module Hierarchy

The controller is decomposed into 17 RTL modules under a single top-level wrapper. Each is described in detail in Chapter 3.

## 8.1	Hierarchy Tree

Module Hierarchy

Source: 02_module_hierarchy.mmd

## 8.2	Module Functional Groupings

For documentation and review purposes, the modules group naturally into seven functional layers:

| Layer | Modules |
| --- | --- |
| AXI Frontend | axi4_slave, addr_mapper |
| Transaction Buffering | txn_queue |
| Scheduling | scheduler, page_predictor |
| Bank State + Timing | bank_machine, xbank_timers |
| Refresh + Power | refresh_mgr, power_state |
| Initialization | init_engine (with embedded step_table) |
| Command Output | cmd_encoder, gear_out |
| Data Paths | wdata_path, rdata_path |
| PHY Interface | dfi_master |
| Configuration | csr_slave |

Chapter 3 follows this grouping (one section per functional layer).

## 8.3	Memtype-Conditional Modules

Three modules contain memtype-conditional logic that is selected at elaboration:

- cmd_encoder — instantiates either ddr2_encoder or lpddr2_encoder based on MEMTYPE.

- init_engine — its step_table ROM is loaded from either ddr2_init_steps_pkg or lpddr2_init_steps_pkg.

- power_state — handles Self-Refresh (DDR2) or Deep Power Down (LPDDR2) entry sequences.

Synthesis dead-code-eliminates the unused paths. The same RTL source supports both targets.

## 8.4	Optional Modules

One module is parameter-gated and may be entirely absent from the synthesized design:

- page_predictor — only instantiated when PAGE_POLICY == HAPPY_HYBRID. For OPEN or CLOSE policies, the predictor module is not synthesized.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	Top-Level Interfaces

This section is the integrator’s quick-reference for the controller’s top-level module port list. Detailed protocol semantics for each interface are in Chapter 4 (Interfaces); this is the wire-level view — names, directions, widths, parameter dependencies.

The top module is ddr2_lpddr2_ctrl and has four external interfaces:

- AXI4 slave (host request path)

- DFI v2.1 master (DRAM PHY path)

- APB CSR slave (configuration and observation)

- Clocks, resets, and miscellaneous (interrupts, debug, scan)

All four are exposed at the top level as flat per-signal ports. SystemVerilog interfaces are not used at the top boundary because the framework’s interface convention (see CocoTBFramework BFM components) drives flat-port pin-level handshakes. Internal sub-blocks use interface objects, but the top-of-design is flat for synthesis and BFM compatibility.

## 9.1	Parameter-Dependent Widths

Several port widths depend on top-level parameters. Throughout this section:

- AW = AXI_ADDR_WIDTH (default 32)

- DW = AXI_DATA_WIDTH (default 64)

- SW = DW/8 (byte-strobe width)

- IDW = AXI_ID_WIDTH (default 4)

- DFI_DW = DFI_DATA_WIDTH (per-phase; default 32)

- DFI_AW = DFI_ADDR_WIDTH (default 14 for DDR2, 20 for LPDDR2)

- NP = N_PHASES (default 4)

- NR = NUM_RANKS (default 1)

- NB = NUM_BANKS (default 8)

- BA = $clog2(NUM_BANKS)

## 9.2	1. AXI4 Slave (Host-side)

### 9.2.1	Write Address Channel (AW)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| awid | input | IDW | Write transaction ID |
| awaddr | input | AW | Write start address |
| awlen | input | 8 | Burst length – 1 |
| awsize | input | 3 | Beat size encoding |
| awburst | input | 2 | Burst type (INCR mandatory; FIXED / WRAP optional) |
| awlock | input | 1 | Lock (ignored — exclusives not supported) |
| awcache | input | 4 | Cache hint (observed; no behavior) |
| awprot | input | 3 | Protection bits (observed) |
| awqos | input | 4 | QoS hint (boosts scheduler priority — see §3.2) |
| awregion | input | 4 | Region (observed) |
| awvalid | input | 1 | Address-valid |
| awready | output | 1 | Address-ready |

### 9.2.2	Write Data Channel (W)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| wdata | input | DW | Write data |
| wstrb | input | SW | Byte enables |
| wlast | input | 1 | Last beat of burst |
| wvalid | input | 1 | Data-valid |
| wready | output | 1 | Data-ready |

### 9.2.3	Write Response Channel (B)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| bid | output | IDW | Response ID |
| bresp | output | 2 | Response code (OKAY / SLVERR) |
| bvalid | output | 1 | Response-valid |
| bready | input | 1 | Response-ready |

### 9.2.4	Read Address Channel (AR)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| arid | input | IDW | Read transaction ID |
| araddr | input | AW | Read start address |
| arlen | input | 8 | Burst length – 1 |
| arsize | input | 3 | Beat size encoding |
| arburst | input | 2 | Burst type |
| arlock | input | 1 | Lock (ignored) |
| arcache | input | 4 | Cache hint |
| arprot | input | 3 | Protection |
| arqos | input | 4 | QoS hint |
| arregion | input | 4 | Region |
| arvalid | input | 1 | Address-valid |
| arready | output | 1 | Address-ready |

### 9.2.5	Read Data Channel (R)

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| rid | output | IDW | Read data ID |
| rdata | output | DW | Read data |
| rresp | output | 2 | Response code |
| rlast | output | 1 | Last beat |
| rvalid | output | 1 | Data-valid |
| rready | input | 1 | Data-ready |

## 9.3	2. DFI v2.1 Master (PHY-side)

The DFI v2.1 sub-interfaces present on this controller are limited to command, write-data, read-data, status, and update. Training, frequency-change, low-power, and CRC sub-interfaces are not driven — see §2.1 Out of Scope.

### 9.3.1	DFI Command Sub-Interface (Per-Phase)

The command bus is replicated N_PHASES times. Each per-phase index p ∈ {0 .. NP−1} carries its own command operand set; the gear logic in gear_dfi aligns them within the MC clock cycle.

| Signal | Direction | Width | Per-Phase | Description |
| --- | --- | --- | --- | --- |
| dfi_address | output | DFI_AW | ×NP | Address operand (row, column, or CA-bus) |
| dfi_bank | output | BA | ×NP | Bank operand (per-rank bank, see dfi_cs_n for rank) |
| dfi_cs_n[r] | output | 1 | ×NP × NR | Active-low chip-select, one per rank |
| dfi_ras_n | output | 1 | ×NP | RAS strobe (DDR2 only; tied / unused for LPDDR2) |
| dfi_cas_n | output | 1 | ×NP | CAS strobe (DDR2 only) |
| dfi_we_n | output | 1 | ×NP | WE strobe (DDR2 only) |
| dfi_cke[r] | output | 1 | ×NR | Clock-enable, one per rank (cross-phase; held) |
| dfi_odt[r] | output | 1 | ×NR | ODT, one per rank (cross-phase; driven by odt_ctrl) |
| dfi_reset_n | output | 1 |  | DRAM reset (init engine) |

### 9.3.2	DFI Write-Data Sub-Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_wrdata_en | output | NP | Per-phase write-data valid |
| dfi_wrdata | output | NP × DFI_DW | Concatenated per-phase write data |
| dfi_wrdata_mask | output | NP × DFI_DW/8 | Per-byte write mask |
| dfi_wrdata_cs_n[r] | output | NP × NR | Per-rank, per-phase data-bus chip-select; the encoder asserts the matching rank when issuing a write |

### 9.3.3	DFI Read-Data Sub-Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_rddata_en | output | NP | Per-phase read-data enable |
| dfi_rddata | input | NP × DFI_DW | Per-phase read data |
| dfi_rddata_valid | input | NP | Per-phase read data valid |
| dfi_rddata_cs_n[r] | output | NP × NR | Per-rank, per-phase data-bus chip-select; the encoder asserts the matching rank when issuing a read |

### 9.3.4	DFI Status and Update Sub-Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| dfi_init_start | output | 1 | Init handshake — start of DRAM init |
| dfi_init_complete | input | 1 | PHY signals init complete |
| dfi_dram_clk_disable | output | 1 | PHY clock disable during power-down |
| dfi_ctrlupd_req | output | 1 | Controller-update request |
| dfi_ctrlupd_ack | input | 1 | Controller-update acknowledge |
| dfi_phyupd_req | input | 1 | PHY-initiated update request |
| dfi_phyupd_ack | output | 1 | PHY-update acknowledge |
| dfi_phyupd_type | input | 2 | Update type (training vs. ctrl-recal) |

## 9.4	3. APB CSR Slave (Configuration / Observation)

Single APB3 slave (4 KB region; 12-bit paddr). Detailed register map in §6.3.

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| psel | input | 1 | Slave select |
| penable | input | 1 | Enable strobe |
| pwrite | input | 1 | Write enable |
| paddr | input | 12 | Address |
| pwdata | input | 32 | Write data |
| pstrb | input | 4 | Byte strobe (APB4 only) |
| pready | output | 1 | Slave-ready |
| prdata | output | 32 | Read data |
| pslverr | output | 1 | Slave error |

## 9.5	4. Clocks, Resets, Interrupts, Debug

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mc_clk | input | 1 | MC clock (drives core, DFI command, scheduler) |
| mc_rst_n | input | 1 | MC asynchronous reset, active low |
| apb_pclk | input | 1 | APB clock (independent of mc_clk for CDC) |
| apb_presetn | input | 1 | APB reset, active low |
| irq_init_done | output | 1 | One-cycle pulse when DRAM init completes |
| irq_init_error | output | 1 | Latched on init failure (cleared by CSR write) |
| irq_overflow | output | 1 | AXI queue overflow / refresh-deadline miss |
| dbg_state | output | 4 | Encoded power-state for board-level probe |
| dbg_refresh | output | 1 | Asserted during any refresh sequence |

The interrupts are wired to a downstream interrupt controller; the dbg outputs are typically routed to a test header or chipset SignalTap / ILA.

## 9.6	Hierarchical Top-Level View

The top-level pinout in shorthand:

ddr2_lpddr2_ctrl
    ├── AXI4 slave        (host)         awid, awaddr, ..., bvalid, bready, ...
    │                                    arid, araddr, ..., rvalid, rready, ...
    ├── DFI v2.1 master   (PHY)          dfi_address, dfi_cs_n[r], ..., dfi_cke[r], dfi_odt[r], ...
    ├── APB CSR slave     (config)       psel, penable, paddr, ..., prdata, pslverr
    └── Clocks + IRQs                    mc_clk, mc_rst_n, apb_pclk, apb_presetn, irq_*, dbg_*

The full SystemVerilog port list — with every parameter substituted — is generated by the build system into regs/ddr2_lpddr2_ctrl_ports.svh and is the canonical source. This section is the integrator’s overview, not the build-time canonical list.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	FUB Breakdown (First-Pass SWAG)

This section is a first-pass SWAG — Some Wild-Ass Guess — at the controller’s Functional Unit Block decomposition. It follows the RTLDesignSherpa stream convention of organizing RTL as small, independently-verifiable blocks under rtl/fub/, with a single top-level integration in rtl/top/. The boundary lines below will move during detailed design; the purpose here is to establish the SWAG so that file layout, filelist generation, and per-FUB verification plans can start in parallel with the architectural detail.

## 10.1	Stream FUB Convention

Per the standard component layout:

ddr2-lpddr2/
├── bin/                       # build / regression scripts
├── coverage_combined/         # aggregated coverage reports
├── coverage_data/             # raw cocotb coverage drops
├── docs/                      # this HAS, generated PDFs
├── dv/                        # cocotb tests, BFM glue
├── regs/                      # CSR JSON/YAML → generated SV/headers
├── reports/                   # synth/timing reports
└── rtl/
    ├── top/                   # ddr2_lpddr2_ctrl.sv (integration only)
    ├── fub/                   # leaf FUBs (this section)
    ├── includes/              # shared `defines, parameter packages
    ├── macro/                 # SRAMs, latches, hard-IP wrappers
    └── filelists/             # *.f files per FUB and per top

Each FUB has: - One top SystemVerilog file with the same name as the FUB - An optional _pkg.sv for parameter typedefs / config structs - A filelist filelists/<fub_name>.f - A dv/ testbench that drives that FUB standalone

## 10.2	FUB Inventory

The 14 leaf FUBs proposed below are grouped by major architectural role. Each entry gives the FUB name, primary inputs/outputs (informal), the parameters that govern its size, and a one-line purpose. These FUBs map directly to the sub-modules called out in §3 (Architecture) but are the level at which the filelist and synthesis boundary are drawn.

### 10.2.1	Group A — AXI Frontend

#### 10.2.1.1	axi4_slave_fub

- Purpose: AXI4 slave protocol engine; AW / W / B and AR / R channel handshake; ID-aware out-of-order completion buffer.

- Key params: AXI_DATA_WIDTH, AXI_ID_WIDTH, AXI_ADDR_WIDTH, AXI_OOO_ACROSS_IDS, SUPPORT_FIXED_BURST, SUPPORT_WRAP_BURST.

- Upstream: top-level AXI ports.

- Downstream: addr_mapper_fub (decoded address), txn_queue_fub (request push), data-path FUBs (wr_data_path_fub, rd_data_path_fub).

#### 10.2.1.2	addr_mapper_fub

- Purpose: Decode flat AXI address into (rank, bank, row, column) per the active address-mapping scheme.

- Key params: NUM_RANKS, NUM_BANKS, ROW_WIDTH, COL_WIDTH, ADDR_MAP_SCHEMES_SYNTH, ADDR_MAP_SCHEME_DEFAULT.

- Notable: Mirrors the Python AddressMapping class in the DV repo; the same decode function is used by RTL and BFM.

### 10.2.2	Group B — Transaction Buffering and Scheduling

#### 10.2.2.1	txn_queue_fub

- Purpose: Pending-transaction queue with row_hit caching and saturating age counters.

- Key params: TXN_QUEUE_DEPTH, AGE_MAX.

- Storage: Distributed registers; for TXN_QUEUE_DEPTH ≥ 32 consider promoting to a small SRAM in rtl/macro/.

#### 10.2.2.2	scheduler_fub

- Purpose: FR-FCFS priority function, lookahead window, refresh-priority gating, runtime force_inorder collapse.

- Key params: SCHEDULER_MODE, LOOKAHEAD_DEPTH_MAX.

- Notable: The single hardest synthesis-timing FUB in the design; expected to dominate the path from txn_queue to command issue.

#### 10.2.2.3	page_predictor_fub (conditional)

- Purpose: HAPPY hybrid page-conflict predictor; synthesized only when PAGE_POLICY == HAPPY_HYBRID.

- Key params: PAGE_PREDICTOR_TABLE_BITS.

- Storage: One small SRAM (rtl/macro/page_pred_table_sram) for table size ≥ 12 bits; distributed registers below.

### 10.2.3	Group C — Per-Bank State and Cross-Bank Timing

#### 10.2.3.1	bank_machine_fub

- Purpose: Per-(rank, bank) FSM; per-bank timing counters; refresh handshake.

- Key params: ROW_WIDTH. Instantiated NUM_RANKS × NUM_BANKS times by the top.

- Synthesis note: Replicated; an attempt at sharing across banks is rejected because per-bank state must be queryable in parallel by the scheduler.

#### 10.2.3.2	xbank_timers_fub

- Purpose: Cross-bank timing constraints (tRRD, tFAW, tCCD, tWTR, tRTW, tRTRS, tCS); per-rank vs. global scope as documented in §3.3.

- Key params: NUM_RANKS, NUM_BANKS.

- Storage: Per-rank 4-entry tFAW FIFOs; remaining state is small down-counters.

### 10.2.4	Group D — Refresh, Init, Power

#### 10.2.4.1	refresh_mgr_fub

- Purpose: tREFI postponer; REFab / REFpb dispatch; round-robin rank dispatch; DARP bank selection; periodic ZQCS piggyback; PASR mask propagation.

- Key params: NUM_RANKS, NUM_BANKS, REFPB_POLICY, REFRESH_DEFER_MAX.

#### 10.2.4.2	init_engine_fub

- Purpose: Microprogram-driven cold-boot init sequence per MEMTYPE; step-table ROM lookup; sub-step timing.

- Key params: MEMTYPE, SIM_INIT_SCALE.

- Storage: Step-table ROMs in rtl/macro/init_steps_<memtype>_rom.

#### 10.2.4.3	power_state_fub

- Purpose: Active / Active-Power-Down / Self-Refresh / Deep-Power-Down FSM; CKE per-rank control; SR-entry / SR-exit coordination with refresh_mgr_fub.

- Key params: NUM_RANKS, MEMTYPE (DPD is LPDDR2 only).

### 10.2.5	Group E — Command Encoding and Output Gear

#### 10.2.5.1	cmd_encoder_fub

- Purpose: Swap between DDR2 (ras/cas/we) and LPDDR2 (CA-bus packed) encoding; selected at elaboration by MEMTYPE.

- Key params: MEMTYPE, DFI_ADDR_WIDTH, N_PHASES.

#### 10.2.5.2	gear_dfi_fub

- Purpose: Pack scheduler-issued commands and data into per-phase DFI slots according to N_PHASES, WRPHASE, RDPHASE.

- Key params: N_PHASES, WRPHASE, RDPHASE, DFI_DATA_WIDTH.

#### 10.2.5.3	odt_ctrl_fub

- Purpose: Drive per-rank dfi_odt[r] per the multi-rank termination rule (ODT_RULE_MULTIRANK). Cross-rank read/write turn-on/turn-off windows.

- Key params: NUM_RANKS, ODT_RULE_MULTIRANK, MEMTYPE.

- Notable: This FUB is the home of the famous “ODT-on-other-rank during read” rule — see §3.6.

### 10.2.6	Group F — Data Paths

#### 10.2.6.1	wr_data_path_fub

- Purpose: AXI W-channel → write-data buffer → DFI wrdata. Width-conversion (AXI DW → NP × DFI_DW). Byte-mask propagation.

- Key params: AXI_DATA_WIDTH, N_PHASES, DFI_DATA_WIDTH.

#### 10.2.6.2	rd_data_path_fub

- Purpose: DFI rddata → read-data buffer → AXI R-channel; CL / CWL-aware capture; ID-tagged completion buffer for OoO return.

- Key params: AXI_DATA_WIDTH, N_PHASES, DFI_DATA_WIDTH, AXI_OOO_ACROSS_IDS.

### 10.2.7	Group G — CSR

#### 10.2.7.1	csr_apb_fub

- Purpose: APB3 slave; register file; CDC between apb_pclk and mc_clk; runtime override propagation with quiet-point sync.

- Key params: derived from CSR YAML in regs/.

- Notable: The register fields themselves are generated; the FUB wrapper provides the APB protocol and the CDC.

## 10.3	FUB Count Summary

| Group | FUBs | Notes |
| --- | --- | --- |
| A — AXI Frontend | 2 | axi4_slave, addr_mapper |
| B — Transaction / Scheduling | 3 | txn_queue, scheduler, page_predictor* |
| C — Bank state | 2 | bank_machine, xbank_timers |
| D — Refresh / Init / Power | 3 | refresh_mgr, init_engine, power_state |
| E — Encoding / Output | 3 | cmd_encoder, gear_dfi, odt_ctrl |
| F — Data paths | 2 | wr_data_path, rd_data_path |
| G — CSR | 1 | csr_apb |
| Total | 16 | (page_predictor conditional; effective floor 15) |

The number is a SWAG; expect ±2 during detailed design as the boundary between scheduler ↔ xbank_timers and between gear_dfi ↔ cmd_encoder is re-litigated.

## 10.4	Integration

rtl/top/ddr2_lpddr2_ctrl.sv is integration-only — pure structural wiring across the FUBs above, with no behavioral logic. Every behavioral statement belongs in a FUB.

The top has three principal wiring concerns:

- Per-rank, per-bank fan-out of bank_machine_fub instances; their state aggregation to scheduler_fub.

- Per-rank fan-out of dfi_cs_n, dfi_cke, dfi_odt from odt_ctrl_fub and power_state_fub.

- Per-phase fan-out to gear_dfi_fub and wr_data_path_fub / rd_data_path_fub.

The integration file is mechanically generated by a build-time wrapper-generator script from the FUB filelists; hand-editing is only for the leaf FUB instantiation block.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	AXI4 Frontend

The AXI frontend comprises two modules: axi4_slave (channel handshake and decoupling) and addr_mapper (flat-address-to-DRAM-coordinates translation).

## 11.1	axi4_slave

### 11.1.1	Purpose

Absorb AXI4 channel pressure and decouple SoC-side handshake behavior from the controller’s internal clock-aligned pipeline. The slave terminates the five AXI channels (AW, W, B, AR, R) and presents a simplified internal interface to the rest of the controller.

### 11.1.2	Internal State

| Element | Description |
| --- | --- |
| AW skid buffer | 2-deep; absorbs back-pressure timing |
| AR skid buffer | 2-deep |
| W skid buffer | 2-deep |
| B response FIFO | Indexed by AXI ID; preserves per-ID order |
| R response staging | Per-ID burst beat buffer |

### 11.1.3	Behavior

- Accepts AW transactions; emits an internal txn_alloc request to addr_mapper and txn_queue. Acknowledges AW once both downstream slots are reserved.

- AR transactions are handled identically.

- W beats stream into a write-data FIFO with the originating transaction ID attached.

- B and R responses come back from the controller core and are routed to their channels with the correct ID.

### 11.1.4	Per-ID Ordering

AXI4 requires reads from the same ID to return in order, and writes from the same ID to commit in order. The slave preserves order per ID. Across IDs, out-of-order completion is permitted by default (controlled by the AXI_OOO_ACROSS_IDS parameter — defaults to true).

### 11.1.5	Burst Splitting

AXI4 permits up to 256 beats per burst; a DRAM row contains 2^COL_WIDTH columns. If an AXI burst exceeds the remaining columns in the current row, axi4_slave splits the burst at the row boundary. Splitting is transparent to the AXI master — internally, the burst is recorded as two transactions with a “continuation” flag that prevents the second half from being scheduled before the first completes.

### 11.1.6	Backpressure

- AW .ready deasserts when the transaction queue is full or has no available slots for the requested burst size.

- AR .ready follows the same rule.

- W .ready deasserts when the write-data FIFO is full.

- R and B .valid follow standard AXI protocol; consumers may stall the channels indefinitely without affecting the controller core.

## 11.2	addr_mapper

### 11.2.1	Purpose

Translate a flat AXI address into DRAM coordinates (bank, row, col). The mapping is parameterizable so the same module supports different memory geometries and different scheduling-friendly bank interleavings.

### 11.2.2	Interfaces

Inputs:

- axi_addr — AXI_ADDR_WIDTH bits wide

Outputs:

- bank — $clog2(NUM_BANKS) bits

- row — ROW_WIDTH bits

- col — COL_WIDTH bits

### 11.2.3	Mapping Function

The default mapping is row-major (column in the low bits, bank above, row above that):

axi_addr[AXI_ADDR_WIDTH-1 : ROW_WIDTH + BANK_WIDTH + COL_WIDTH] = unused
axi_addr[ROW_WIDTH + BANK_WIDTH + COL_WIDTH - 1 : BANK_WIDTH + COL_WIDTH] = row
axi_addr[BANK_WIDTH + COL_WIDTH - 1 : COL_WIDTH] = bank
axi_addr[COL_WIDTH - 1 : 0] = col

### 11.2.4	Alternative Mappings

A parameter ADDR_MAP_SCHEME selects from three predefined mappings:

| Scheme | Layout | When to use |
| --- | --- | --- |
| ROW_MAJOR | row ¦ bank ¦ col | Default; good for streaming workloads |
| BANK_INTERLEAVE | row ¦ col ¦ bank | Improves bank-parallelism for random workloads |
| XOR_HASH | row ¦ bank ⊕ row_low ¦ col | Breaks pathological row-conflict patterns |

### 11.2.5	Relationship to the DFI BFM

This module mirrors the AddressMapping class in the DFI BFM (src/CocoTBFramework/components/dfi/dfi_signals.py). The same mapping object can drive both RTL elaboration and BFM verification, ensuring address-decode consistency between simulation and silicon.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	Scheduler and Page Predictor

This section covers two tightly-coupled modules: the FR-FCFS scheduler that picks the next request to issue, and the optional HAPPY page predictor that informs the scheduler’s auto-precharge decisions.

## 12.1	txn_queue

### 12.1.1	Purpose

Hold pending memory transactions with the metadata the scheduler needs to make priority decisions.

### 12.1.2	Entry Format

Each slot in the queue carries:

| Field | Width | Purpose |
| --- | --- | --- |
| valid | 1 | Slot occupied |
| axi_id | AXI_ID_WIDTH | Originating AXI transaction ID |
| is_write | 1 | Write (1) vs read (0) |
| bank | $clog2(NUM_BANKS) | Target bank |
| row | ROW_WIDTH | Target row |
| col | COL_WIDTH | Target starting column |
| burst_len | 8 | AXI len + 1 (max 256) |
| row_hit | 1 | Cached at insert |
| age | $clog2(AGE_MAX) | Saturating age counter |
| state | 2 | {PENDING, ISSUED, COMPLETING} |

### 12.1.3	Depth

TXN_QUEUE_DEPTH (default 16; parameterizable 4–64).

### 12.1.4	Behavior

- New entries are inserted by axi4_slave via addr_mapper.

- row_hit is computed at insert time by querying the relevant bank machine’s current open_row register. Caching this avoids per-cycle recomputation in the scheduler.

- age increments every cycle and saturates at AGE_MAX (default 256).

- The scheduler reads the entire queue snapshot each cycle to choose the next issue candidate.

- Entries transition PENDING → ISSUED when the scheduler picks them, then → COMPLETING when the corresponding data beats are flowing, then deallocate when complete.

## 12.2	scheduler

### 12.2.1	Purpose

Pick the next request to issue and dispatch its corresponding DRAM command. Implements FR-FCFS with characterization knobs for page policy and refresh prioritization.

### 12.2.2	Inputs

- Queue snapshot (all entries’ metadata)

- Per-bank state from each bank_machine (idle / activating / active / precharging)

- Refresh request from refresh_mgr with priority level

- Init-in-progress signal (pauses normal scheduling)

- Page predictor signal (when HAPPY enabled)

### 12.2.3	Outputs

- issue_valid — 1 when a command is being issued this cycle

- issue_op — {ACT, RD, RDA, WR, WRA, PRE, PREA, REF, REFPB, MRS, ZQCS, ZQCL, NOP}

- issue_bank, issue_row, issue_col, issue_axi_id — operands

### 12.2.4	Priority Function

Evaluated each cycle (descending priority):

- Init in progress — block all normal traffic; init commands take a parallel path.

- Critical refresh (refresh_pending ≥ critical_threshold) — REF / REFpb wins all ties.

- Soft refresh (refresh_pending ≥ soft_threshold) — boost refresh above row-misses but allow row-hits.

- Row-hit ready columns — same bank’s open row matches request row; bank machine accepts column command.

- Age-ordered FCFS among row-hits — oldest wins.

- Row-miss requests — bank needs PRE then ACT first.

- Idle (NOP).

### 12.2.5	Strict In-Order Mode

For systems that require deterministic, in-order issue — real-time / safety-critical / formally-verifiable timing targets — the scheduler can be forced into a first-ready FIFO mode. Two routes:

Runtime debug switch (recommended). When SCHEDULER_MODE = OOO is synthesized (the default), setting SCHED_TUNING.force_inorder = 1 collapses the priority function to first-ready FIFO at runtime. The OoO comparator logic is still in silicon but its outputs are masked. Switching is a single CSR write, takes effect at the next configuration quiet point, and is fully reversible.

Build-time omission. Setting SCHEDULER_MODE = INORDER at elaboration omits the FR-FCFS comparator logic entirely, saving area. Once chosen, the design cannot switch back to OoO at runtime — SCHED_TUNING.force_inorder becomes a tied-high observation bit. Pick this only if the area savings are critical and OoO will never be needed.

In either form, the in-order behavior is:

- Only the first-ready filter survives. The scheduler still skips requests whose target bank is blocked by timing constraints (otherwise it would waste cycles on requests that can’t issue), but it does not reorder requests for row-hit benefit.

- Row-hit prioritization is disabled. Requests are considered strictly in txn_queue arrival order.

- Age-based ties become moot — the queue is already age-ordered by construction.

- Refresh priority still applies (the refresh path is a JEDEC requirement, not a performance choice).

- HAPPY and lookahead are unaffected at elaboration but can be independently disabled via runtime CSR (SCHED_TUNING.happy_enable = 0, SCHED_TUNING.lookahead_active = 0). For pure first-ready FIFO with neither, set all three CSR bits.

- AXI per-ID ordering is honored regardless. The change here is only at the DRAM-issue layer, not at the AXI completion layer.

Trade-off: in-order mode reduces sustained bandwidth on streaming workloads typically 10–25% (the cost of not reordering row-conflicts behind row-hits), but issue latency becomes deterministic and the design is much easier to formally verify. The runtime-switchable form is preferred for shipping designs because it preserves debug flexibility at the cost of the silicon area for the unused OoO logic.

### 12.2.6	Auto-Precharge Decision: Lookahead + Page Policy

The decision to issue a column command as bare RD / WR (keep row open) versus the auto-precharge variant RDA / WRA (close row in flight) follows a two-stage rule. Lookahead runs first; the page policy is the fallback when lookahead is inconclusive.

#### 12.2.6.1	Stage 1: Lookahead

If LOOKAHEAD_DEPTH > 0, the scheduler peeks the next LOOKAHEAD_DEPTH pending entries in txn_queue that target the same bank as the current command:

- If a same-bank pending request is found AND its target row equals the current open_row, the next access is a guaranteed row-hit. Issue the bare RD / WR so the row stays open.

- If a same-bank pending request is found AND its target row differs, the next access is a guaranteed row-conflict. Issue RDA / WRA so the precharge happens in flight (saves an explicit PRE command + tRP later).

- If no same-bank pending request is found within LOOKAHEAD_DEPTH, fall through to Stage 2.

When the transaction queue is deep — which is the common case for streaming / DMA workloads — lookahead provides an exact auto-precharge decision rather than a predicted one. The LOOKAHEAD_DEPTH ∈ {0, 1, 2, 4} parameter sweeps lookahead window size against comparator cost during characterization (covered in §5).

#### 12.2.6.2	Stage 2: Page Policy Fallback

Triggered when lookahead is disabled (LOOKAHEAD_DEPTH = 0) or inconclusive (no same-bank request in the queue, typical of shallow-queue or bursty CPU traffic). Dispatched per PAGE_POLICY:

- OPEN — issue bare RD / WR. Rows stay open until evicted by a row-conflict or refresh. Best for streaming workloads where lookahead is usually conclusive anyway, so the OPEN fallback rarely fires.

- CLOSE — issue RDA / WRA. Always close on every access. Best for random access without temporal locality.

- HAPPY_HYBRID — query the page_predictor with the current request’s row address. If the predictor says “next access to this bank will conflict,” issue RDA / WRA; otherwise issue bare RD / WR. Recommended for bursty mixed workloads where neither lookahead nor a fixed policy is reliable.

#### 12.2.6.3	Why Both

Lookahead is provably correct when the queue contains a same-bank request: we know exactly what the next access will be. HAPPY (and the simple OPEN / CLOSE policies) operate without that information.

The two mechanisms are complementary:

- Deep queue, repetitive locality (streaming, DMA): lookahead almost always resolves; the page-policy choice barely matters.

- Shallow queue, sporadic access (bursty CPU traffic, multi-tenant): lookahead frequently misses; the page-policy choice dominates.

LOOKAHEAD_DEPTH is a characterization knob in its own right — see §5.

Lookahead + Page-Policy Decision Tree

Source: 10_lookahead_auto_pre.mmd

### 12.2.7	Issue Slot Allocation

One command issue per MC clock cycle. Multi-cycle LPDDR2 commands are handled inside the cmd_encoder, so the scheduler still sees them as a single issue.

## 12.3	page_predictor (HAPPY hybrid, optional)

### 12.3.1	Purpose

Predict whether the next access to a bank will hit the currently open row or conflict with it. Synthesized only when PAGE_POLICY == HAPPY_HYBRID.

### 12.3.2	Algorithm

Address-bit-based two-level predictor following Ghasempour et al. (2015):

- Table indexed by hash of bank and a small selection of row-address bits

- Each entry: 2-bit saturating counter (hit-likely vs conflict-likely)

- Update on access completion: was the next access a hit or a conflict?

- Query at scheduler decision time: “for the request I’m about to issue, will the next access to this bank be a hit?”

### 12.3.3	HAPPY Predictor Architecture

HAPPY Predictor Query + Update Paths

Source: 09_happy_predictor.mmd

### 12.3.4	Storage

Total storage: 2 × 2^PAGE_PREDICTOR_TABLE_BITS bits.

- For PAGE_PREDICTOR_TABLE_BITS = 12 (default): 8 Kbit (about 1 KB)

- For PAGE_PREDICTOR_TABLE_BITS = 8: 512 bit

- For PAGE_PREDICTOR_TABLE_BITS = 16: 128 Kbit (around 16 KB; large but rarely needed)

### 12.3.5	Reset Behavior

All entries reset to “weakly hit-likely” (binary 01) so the controller starts in open-page-like behavior. The predictor adapts to workload over the first few hundred accesses.

### 12.3.6	Observability

A debug output exposes the rolling prediction accuracy (% correct) per bank, which the SoC can read via CSR. This is critical for tuning during bring-up and for the characterization sweep.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	Bank Machines and Cross-Bank Timers

Per-bank state machines enforce JEDEC timing locally; a shared cross-bank timer pool enforces the constraints that span banks.

## 13.1	bank_machine

### 13.1.1	Purpose

One FSM instance per bank. Enforces per-bank JEDEC timing constraints; tracks open-row state.

### 13.1.2	Instantiation

NUM_RANKS × NUM_BANKS instances — one FSM per (rank, bank) pair. For the default (NUM_RANKS=1, NUM_BANKS=8) this is 8 instances; for a 2-rank DIMM with 8-bank devices it is 16; for a 4-rank configuration it is 32. The (rank, bank) tuple is the bank machine’s index; all per-bank state (open_row, timing counters, last_ref_age) is independent across ranks because JEDEC tracks every rank’s banks separately.

### 13.1.3	FSM States

| State | Description |
| --- | --- |
| IDLE | Bank closed; ready to accept ACT |
| ACTIVATING | Counting tRCD; row is being opened |
| ACTIVE | Row open; ready for RD / WR |
| RD_BUSY | Read in progress; counting CL for data return |
| WR_BUSY | Write in progress; counting CWL + tWR window |
| PRECHARGING | Counting tRP |
| REFRESHING | Counting tRFCpb (per-bank refresh) |

### 13.1.4	FSM Diagram

Per-Bank FSM

Source: 03_bank_machine_fsm.mmd

### 13.1.5	Per-Bank Registers

| Register | Width | Purpose |
| --- | --- | --- |
| open_row | ROW_WIDTH | Valid in {ACTIVE, RD_BUSY, WR_BUSY} |
| t_rcd_cnt | small | tRCD down-counter (loaded on ACT issue) |
| t_ras_cnt | small | tRAS down-counter |
| t_rp_cnt | small | tRP down-counter |
| t_rc_cnt | small | tRC down-counter |
| t_wr_cnt | small | tWR down-counter |
| t_rfcpb_cnt | small | tRFCpb down-counter |
| last_ref_age | wider | Cycles since last refresh (for DARP / OLDEST_FIRST) |

All counters saturate at zero.

### 13.1.6	Outputs to Scheduler

- state — current FSM state

- open_row — row register (valid when state ∈ active set)

- accepts_act — combinational: 1 when state == IDLE and t_rp_cnt == 0

- accepts_rd, accepts_wr — 1 when state == ACTIVE and constraints met

- accepts_pre — 1 when state == ACTIVE and t_ras_cnt == 0

- accepts_ref — 1 when state == IDLE

- last_ref_age — exposed to refresh manager

### 13.1.7	Refresh Handshake Interface

In addition to the scheduler-side signals above, each bank machine exposes a dedicated refresh handshake pair to the refresh_mgr:

| Signal | Direction | Purpose |
| --- | --- | --- |
| refresh_req | refresh_mgr → bank | Refresh manager wants to issue REF; bank should drain to a quiescent state |
| refresh_gnt | bank → refresh_mgr | Bank acknowledges it is in IDLE (or REFRESHING for a per-bank refresh) and the controller may proceed |

The handshake is the explicit version of “wait for all banks idle.” The refresh manager asserts refresh_req to all bank machines; each bank machine, on completing its current activity and landing in IDLE, asserts refresh_gnt. The refresh manager waits until all bank-machine grants are high (for REFab) or the selected bank’s grant is high (for REFpb) before issuing the actual sequence. When the refresh manager deasserts refresh_req, the bank machine drops refresh_gnt and resumes normal operation.

The handshake gives the refresh manager deterministic acknowledgment timing rather than relying on a race-prone “wait for state == IDLE” poll.

### 13.1.8	Inputs from Cross-Bank Timers

The cross-bank timer pool feeds blocking signals:

- xbank_blocks_act — tRRD or tFAW prevents new ACT

- xbank_blocks_wr_after_rd — tCCD or tWTR prevents WR after recent RD

- xbank_blocks_rd_after_wr — tWTR prevents RD after recent WR data beat

These gate the per-bank accepts_* signals.

### 13.1.9	Auto-Precharge Handling

RDA / WRA commands trigger an automatic transition RD_BUSY → PRECHARGING (or WR_BUSY → PRECHARGING) without a separate PRE command from the scheduler. Internal book-keeping handles the timing.

## 13.2	xbank_timers

### 13.2.1	Purpose

Enforce cross-bank timing constraints. Implemented as a single shared module rather than per-bank to share the FIFO buffer for tFAW.

### 13.2.2	Tracked Constraints

| Constraint | Description |
| --- | --- |
| tRRD | Minimum gap between any two ACT commands |
| tFAW | At most 4 ACTs in any rolling tFAW window |
| tWTR | Write-to-read column-to-column on different banks |
| tCCD | Column-to-column (any column command to any other) |

### 13.2.3	Implementation

| Constraint | Storage |
| --- | --- |
| tRRD | Single saturating down-counter loaded on any ACT issue |
| tFAW | 4-entry FIFO of recent ACT timestamps; new ACT blocked if oldest is younger than (now − tFAW_cycles) |
| tCCD | Small down-counter loaded on any RD / WR issue |
| tWTR | Down-counter loaded on the last WR data beat completion |

### 13.2.4	Outputs

- Per-bank blocks_act / blocks_rdwr signals that the bank machines AND into their respective accepts_* outputs.

### 13.2.5	Why Shared Rather Than Per-Bank

Cross-bank constraints are global: a single tRRD counter can be checked by all banks simultaneously, and the tFAW FIFO needs to see all recent ACTs across all banks. Replicating these per bank would be wasteful.

### 13.2.6	Per-Rank vs. Cross-Rank Scope

When NUM_RANKS > 1, each cross-bank constraint must be evaluated at the correct scope:

| Constraint | Scope | Rationale |
| --- | --- | --- |
| tRRD | per-rank | tRRD limits how fast a single device can stagger ACTs; different ranks have independent activate stagger budgets |
| tFAW | per-rank | tFAW is a device-local thermal/power limit; each rank has its own 4-ACT rolling window |
| tCCD | global (all ranks) | The DQ bus is shared across ranks; column commands to different ranks still share the data window |
| tWTR | global (all ranks) | Same reason — data-bus turnaround applies regardless of rank |
| tRTW | global (all ranks) | Same reason — data-bus turnaround applies regardless of rank |
| tRTRS | cross-rank only | Rank-to-rank read switching delay (1 cycle DDR2/3); applies when consecutive RDs target different ranks |
| tCS | cross-rank only | Chip-select setup; only relevant when switching ranks mid-burst |

The implementation therefore has a per-rank tRRD counter (NUM_RANKS copies) and a per-rank tFAW FIFO (NUM_RANKS copies, 4 entries each), with the global tCCD / tWTR / tRTW counters shared as before. Two additional cross-rank counters (tRTRS_cnt, tCS_cnt) are added when NUM_RANKS > 1 and gate accepts_rd / accepts_wr whenever the candidate’s rank differs from the most recent rank seen on the data bus.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Refresh Manager

The refresh manager owns tREFI timing, refresh request queuing, the choice between REFab and REFpb, and the DARP-based per-bank refresh scheduling policy.

## 14.1	refresh_mgr

### 14.1.1	Purpose

Issue REF / REFpb commands on schedule. Honor the PASR mask. Implement Elastic-Refresh deferral up to REFRESH_DEFER_MAX × tREFI. Implement DARP-style idle-bank-first per-bank refresh scheduling for LPDDR2.

### 14.1.2	Internal State

| Element | Description |
| --- | --- |
| t_refi_cnt | Central tREFI down-counter |
| postpone_cnt | Down-counter from REFRESH_DEFER_MAX − 1 to 0 (postponer batch) |
| refresh_owed | Non-saturating counter of refreshes still owed (deterministic; replaces saturating refresh_pending) |
| t_zqcs_cnt | ZQCS interval counter (cycles between periodic ZQCS) |
| wants_zqcs | One-cycle pulse when periodic ZQCS is due |
| pasr_bank_mask[NUM_RANKS] | Per-rank NUM_BANKS-bit mask (set via CSR; mirrors each rank’s MR16/17). LPDDR2 PASR is a per-rank construct because each rank has its own mode-register file. |
| pasr_seg_mask[NUM_RANKS] | Per-rank segment mask (set via CSR) |
| last_ref_cycle[R][B] | Per-(rank, bank) timestamp (lives in each bank_machine) |
| round_robin_rank | Cyclic rank pointer for REFab dispatch; advanced after each refresh issue so all ranks are serviced fairly |

### 14.1.3	Postponer Behavior (Deterministic Batching)

The postponer is a deterministic countdown that gathers REFRESH_DEFER_MAX deadlines, then triggers exactly that many back-to-back refreshes via the sequencer.

t_refi_cnt decrements from tREFI_cycles to zero each cycle. On reaching zero:

- postpone_cnt decrements (from REFRESH_DEFER_MAX − 1 down to 0)

- Reload t_refi_cnt = tREFI_cycles

- When postpone_cnt reaches 0, emit a one-cycle wants_refresh pulse, reload postpone_cnt = REFRESH_DEFER_MAX − 1, and add REFRESH_DEFER_MAX to refresh_owed

The sequencer (next subsection) then drains refresh_owed to zero by issuing that many back-to-back REF / REFpb commands.

Why deterministic, not saturating. Earlier revisions used a saturating refresh_pending counter that silently dropped refresh deadlines when the system stayed bandwidth-starved past 8× tREFI. JEDEC requires the controller to catch up on all missed refreshes; saturation is technically out-of-spec. The deterministic postponer counts exact refresh events over time and never drops.

Setting REFRESH_DEFER_MAX = 1 disables batching (every tREFI fires one refresh; equivalent to no postponer).

### 14.1.4	Temperature-Compensated Refresh (LPDDR2)

LPDDR2 part reports a temperature classification in MR4 which the SoC reads via CSR. The CSR sideband programs a tREFI_scale factor (1x, 0.5x, 0.25x); tREFI_cycles is scaled accordingly. Higher temperature → shorter tREFI → more frequent refresh.

### 14.1.5	Controller-Level Refresh FSM

The refresh manager hosts a small top-level FSM that orchestrates the bank-grant handshake, the refresh sequencer, and the (piggybacked) periodic ZQCS:

Refresh-Manager Top-Level FSM

Source: 11_refresh_controller_fsm.mmd

State transitions:

- IDLE — waits for wants_refresh from the postponer

- WAIT_BANK_GNTS — asserts refresh_req to all bank machines (REFab) or the selected bank (REFpb); waits for the required refresh_gnt signals

- DO_REFRESH — runs the refresh sequencer that issues REFRESH_DEFER_MAX back-to-back PRE_ALL+AUTO_REF cycles, decrementing refresh_owed per issue

- CHECK_ZQCS — if wants_zqcs is asserted (periodic ZQCS due), transition to DO_ZQCS; otherwise go to RELEASE

- DO_ZQCS — execute ZQCS sequence (PRE_ALL → tRP → ZQCS command → wait tZQCS)

- RELEASE — deassert refresh_req; bank machines drop their grants and resume

### 14.1.6	Periodic ZQCS Piggybacked on Refresh

Periodic ZQ Short Calibration is required for long-running silicon to compensate for thermal drift in DRAM impedance. Earlier revisions of this document put ZQCS only in the init engine step table — meaning ZQCS ran exactly once at boot and never again. That is incorrect for systems that operate for hours or longer.

Design: a separate t_zqcs_cnt counter triggers wants_zqcs at the rate set by ZQCS_FREQ_HZ (default 1 Hz). The FSM checks wants_zqcs at the end of a refresh sequence and runs ZQCS while the banks are still in the refresh handshake. This piggybacks on the “all banks idle” guarantee that refresh already provides — no separate bus-blocking event is needed for ZQCS.

ZQCS frequency is independent of refresh frequency. Setting ZQCS_FREQ_HZ = 0 disables periodic ZQCS entirely (init-only behavior, for short-running systems or systems where ZQCS is managed externally).

### 14.1.7	Temperature-Compensated Refresh (LPDDR2)

LPDDR2 part reports a temperature classification in MR4 which the SoC reads via CSR. The CSR sideband programs a tREFI_scale factor (1x, 0.5x, 0.25x); tREFI_cycles is scaled accordingly. Higher temperature → shorter tREFI → more frequent refresh.

### 14.1.8	Refresh Decision Flow

Refresh Decision Tree

Source: 06_refresh_decision.mmd

## 14.2	REFab Path (DDR2 or LPDDR2 fallback)

- Pick the target rank r = round_robin_rank (advanced post-issue).

- Assert refresh_req to all bank machines on rank r.

- Wait for all bank machines on rank r to assert refresh_gnt (each does so on reaching IDLE).

- Issue REF command via the scheduler with CS_n[r] = 0 and all other ranks deselected.

- Load tRFCab_cnt in all bank machines on rank r; all transition to REFRESHING. Other ranks continue normal operation.

- Decrement refresh_owed. If refresh_owed > 0, advance round_robin_rank and the sequencer issues another back-to-back PRE_ALL+AUTO_REF cycle (to the next rank).

- When refresh_owed == 0, deassert refresh_req; bank machines drop their grants and resume.

REFab blocks all banks on the targeted rank for tRFCab cycles per refresh; with batching, REFRESH_DEFER_MAX refreshes run back-to-back, each on the next rank in round-robin order. This is the DDR2 default and the LPDDR2 fallback when many banks need refresh simultaneously. For NUM_RANKS=1 this collapses to the original single-rank flow.

Why per-rank rather than all-rank REFab. Driving REF with all CS_n[*] = 0 would force every rank into REFRESHING simultaneously, wasting bandwidth on ranks that aren’t yet due. The round-robin per-rank dispatch keeps the unused ranks available for normal column commands during the multi-rank-batched refresh window.

## 14.3	REFpb Path (LPDDR2, default)

The REFpb path selects one bank at a time, leaving the others available. Selection policy is parameterized.

### 14.3.1	Bank Selection Policy

Controlled by REFPB_POLICY. The selection space is the full (rank, bank) product set across all ranks:

| Policy | Selection rule |
| --- | --- |
| ROUND_ROBIN | Cyclic through (rank, bank) tuples, skipping per-rank PASR-masked banks |
| OLDEST_FIRST | Max last_ref_age among non-masked (rank, bank) tuples |
| DARP (default) | Max last_ref_age among idle non-masked (rank, bank) tuples; if none idle, fall back to OLDEST_FIRST |

Per-rank PASR masks are honored independently — bank 3 of rank 0 may be masked while bank 3 of rank 1 is active, and DARP will only pick from the unmasked tuples.

### 14.3.2	DARP Rationale

DARP (Chang et al., HPCA 2014) wins over ROUND_ROBIN because real workloads have uneven bank activity. The cyclic baseline picks the next bank in sequence regardless of whether it’s idle; if that bank happens to be open with a pending column command, the refresh blocks it. DARP picks among the idle banks instead, preserving column-command parallelism.

### 14.3.3	REFpb Issue Sequence

- Pick the target bank per REFPB_POLICY.

- Assert refresh_req to the selected bank only.

- Wait for that bank’s refresh_gnt (it asserts on reaching IDLE; PRE if necessary — but DARP minimizes this).

- Issue REFpb command via scheduler.

- Load tRFCpb_cnt in that bank → state REFRESHING.

- Other banks remain in their current states and continue to service column commands.

- Decrement refresh_owed.

- When refresh_owed == 0, deassert refresh_req for the selected bank; it drops its grant and resumes.

## 14.4	Refresh Priority to Scheduler

With deterministic batching, the refresh manager exposes a binary priority to the scheduler:

| Condition | Effect |
| --- | --- |
| wants_refresh == 0 | No request; column commands flow normally |
| wants_refresh == 1 | High priority: scheduler asserts refresh_req immediately on the next cycle |

The scheduler always honors wants_refresh because the postponer has already absorbed the bandwidth flexibility (up to REFRESH_DEFER_MAX × tREFI worth of deferral before any refresh issues). Once the postponer fires, the refresh must complete to keep up with JEDEC’s catch-up requirement.

Bandwidth flexibility is concentrated in the deferred-batch window rather than being smeared across a soft-threshold curve. This produces deterministic refresh latency profiles, preferred for real-time-sensitive systems.

## 14.5	Self-Refresh Coordination

The refresh manager and the power-state FSM coordinate during self-refresh entry / exit:

- The power-state FSM requests self-refresh entry; the refresh manager signals when no auto-refresh is currently in progress.

- During self-refresh, the DRAM handles its own refresh internally; the controller’s tREFI counter is paused.

- On self-refresh exit, the refresh manager observes the SR-exit completion signal from the power-state FSM and resumes tREFI counting.

## 14.6	PASR Update Protocol

The PASR mask is written by the SoC via CSR. The refresh manager treats the mask as advisory until the next self-refresh entry. The Linux PASR Framework (LWN.net) documents the typical update pattern: the kernel writes the mask when it allocates / frees DRAM regions; the controller propagates the mask to the DRAM via MR16/17 writes on the next opportunity (typically during the SR-entry sequence).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	Init Engine and Power State

This section covers the cold-boot initialization engine and the top-level power-state FSM that owns transitions between Active, power-down, and self-refresh / deep-power-down modes.

## 15.1	init_engine and Step Tables

### 15.1.1	Purpose

Sequence the cold-boot initialization: CKE control, MR and EMR loads, ZQ calibration. Implemented as a small microprogram sequencer reading a memtype-specific step table ROM.

### 15.1.2	Microprogram Instruction Format

Each step in the table is a fixed-width record:

| Field | Width | Purpose |
| --- | --- | --- |
| step_type | 4 | Opcode: SET_CTRL_BITS, DELAY, MRS_LOAD, ISSUE_CMD, WAIT_FOR_BIT, END |
| payload_bank | 3 | Used by MRS and ISSUE_CMD |
| payload_addr | 16 | MR address / value, or command address |
| post_delay | 24 | Cycles to wait after step completes (scaled by SIM_INIT_SCALE in sim) |

### 15.1.3	Step Tables

Two compile-time-included Verilog packages:

- ddr2_init_steps_pkg.sv — DDR2 cold-init sequence

- lpddr2_init_steps_pkg.sv — LPDDR2 cold-init sequence

The engine reads its table sequentially. On encountering END, asserts init_done.

### 15.1.4	Init Sequence Outlines

See §6.2 for the full step lists. Summary:

- DDR2: 13 steps — power-up wait, CKE high, NOP, PRECHARGE all, EMRS(2/3) defaults, EMRS(1) enable DLL, MRS reset DLL, PRECHARGE all, AUTO REFRESH × 2, MRS final config, EMRS(1) OCD default, EMRS(1) OCD exit, END.

- LPDDR2: 13 steps — tINIT1 CKE low, tINIT2 CKE high, tINIT3 NOP, MR63 reset, tINIT5 wait, MR10 ZQ + wait for done, MR1 BL/WRAP/BT, MR2 RL/WL, MR3 drive strength, MR4 read for temp class, MR16 PASR bank mask, MR17 PASR segment mask, END.

### 15.1.5	Outputs

| Signal | Width | Purpose |
| --- | --- | --- |
| init_done | 1 | Asserted after END |
| init_error | 1 | Asserted on step failure (timeout, ZQ failure) |
| init_step_dbg | 8 | Current step number (for bring-up observability) |

### 15.1.6	Init Engine Flowchart

Init Engine Flow

Source: 05_init_engine_flow.mmd

### 15.1.7	SIM_INIT_SCALE

Real init sequences include multi-microsecond delays (200 µs power-up wait, etc.) that are impractical to simulate at cycle granularity. The SIM_INIT_SCALE parameter scales delays during simulation: a value of 1 uses real values; a value of 1000 divides all delays by 1000. Silicon builds always use SIM_INIT_SCALE = 1.

### 15.1.8	Restart on Warm Reset

The engine restarts from step zero on warm-reset deassertion. Cold reset (DRAM content lost) and warm reset (CKE gated only) take the same code path; the step table is identical. This simplifies the FSM and avoids subtle bring-up bugs.

### 15.1.9	Error Handling

If a WAIT_FOR_BIT step times out (e.g., ZQ done never asserts), the engine retries up to 3 times before raising init_error and halting. The SoC can clear via CSR and re-issue init_start.

### 15.1.10	MR Value Configurability

MR and EMR values are CSR-configurable before init_start is asserted, with sensible defaults baked into the step table. The step table references named CSRs (MR0, MR1, etc.), not literals, so the SoC can override CL, BL, WL, and PASR masks without rebuilding the controller.

## 15.2	power_state (Top-Level FSM)

### 15.2.1	Purpose

Own the top-level power-state transitions. Coordinate with the init engine, refresh manager, and scheduler when entering / exiting low-power states.

### 15.2.2	FSM States

| State | Description |
| --- | --- |
| RESET | mc_rstn asserted |
| INIT | init_engine running |
| ACTIVE | Normal operation |
| ACTIVE_PD | Active Power-Down (CKE low, rows kept open) |
| PRECHARGE_PD | Precharge Power-Down (CKE low, all banks idle) |
| SELF_REFRESH | DDR2 self-refresh (CKE low, internal refresh) |
| SR_LP2 | LPDDR2 self-refresh (same role, different exit FSM) |
| DPD | LPDDR2 Deep Power Down (content lost, requires re-init) |

### 15.2.3	Power State Diagram

Power-State FSM

Source: 04_power_state_fsm.mmd

### 15.2.4	Transitions

Transitions are driven by SoC requests via CSR:

- req_low_power — request entry into APD or PRECHARGE_PD

- req_self_refresh — request entry into SR

- req_dpd — request entry into DPD (LPDDR2 only)

- req_active — wake up to ACTIVE

The FSM defers entry until the refresh manager signals that no auto-refresh is in progress.

### 15.2.5	Step Tables for Power Transitions

Power-state transitions reuse the microprogram engine pattern:

- sr_entry_steps_pkg.sv — common entry sequence (drain pending, CKE low)

- sr_exit_steps_pkg.sv — common exit sequence (CKE high, wait tXSNR, resume)

- dpd_entry_steps_pkg.sv — LPDDR2 DPD entry

- dpd_exit_steps_pkg.sv — LPDDR2 DPD exit (which re-runs the full init sequence)

### 15.2.6	DPD Entry Constraints

Deep Power Down loses all DRAM content. The controller does not enforce that the SoC has flushed dirty caches before requesting DPD — that’s the SoC’s responsibility. The controller simply executes the transition. Misuse will cause data corruption, which is the price of the deepest-sleep mode.

### 15.2.7	Self-Refresh Exit Latency

JEDEC requires a self-refresh-exit-to-next-command latency tXSNR (~200 ns for DDR2). The power-state FSM honors this with an internal counter; the scheduler is gated until the counter expires.

### 15.2.8	Power-State Observation

Current state is exposed in the STATUS CSR (bits [7:4]), and the recent transition history (last 8 transitions) is exposed in STATUS_HISTORY for bring-up debugging.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	Command Encoder and Gear Output

This section covers the two modules that translate the controller’s internal generic command bus into DFI wire signals: cmd_encoder (memtype-specific wire mapping) and gear_out (1-phase to N-phase replication).

## 16.1	cmd_encoder

### 16.1.1	Purpose

Translate the controller’s internal generic command record into the DFI wire signals appropriate for the target memory. The encoder is the only place where DDR2 and LPDDR2 differ at the wire layer.

### 16.1.2	Input Record

The internal generic command is:

{
  cmd        : { NOP, DESEL, ACT, RD, WR, RDA, WRA,
                 PRE, PREA, REF, REFPB, MRS, ZQCS, ZQCL },
  bank       : log2(NUM_BANKS),
  row        : ROW_WIDTH,
  col        : COL_WIDTH,
  ap         : 1,           // auto-precharge bit
  all_banks  : 1,           // all-banks variant of PRE / REF
  mr_addr    : 8,           // MR / EMR address
  mr_data    : 16           // MR / EMR data (LPDDR2 only; DDR2 packs into addr)
}

### 16.1.3	DDR2 Encoder

Outputs the standard DDR2 command-bus signals per JESD79-2 Table 67:

- cs_n — asserted (low) on command issue; high in idle

- ras_n, cas_n, we_n — set per the JESD79-2 lookup table

- bank — driven from the command record’s bank field

- address — driven from row for ACT; from col for RD / WR (with addr[10] carrying the AP bit for column commands or the AB bit for PRE)

- cke — held high during normal operation; lowered by the power-state FSM

- odt — driven from a small lookup based on transaction type

- reset_n — DDR2 has no reset_n; held high

### 16.1.4	LPDDR2 Encoder

Per DFI v2.1 Table 1, LPDDR2 carries the command on the dfi_address 20-bit CA word:

- cs_n — asserted on command issue

- ras_n, cas_n, we_n, bank — held at their idle values (1, 1, 1, 0)

- address[19:0] — packed CA word via the algorithm in our existing lpddr_ca encoder (src/CocoTBFramework/components/dfi/lpddr_ca.py):

  - address[9:0] = CA cycle 1

  - address[19:10] = CA cycle 2

- cke — used by the power-state FSM (LPDDR2 has the richer CKE state machine)

- odt — LPDDR2 has no ODT; held low

- reset_n — held high during normal operation

### 16.1.5	Memtype Selection

At elaboration, the appropriate encoder is instantiated based on MEMTYPE. Only one is synthesized; the other path is dead code.

### 16.1.6	Idle Defaults

When the controller is not issuing a command, the encoder drives the bus to its deselected idle state:

- DDR2: cs_n=1, ras_n=1, cas_n=1, we_n=1, bank=0, address=0, odt=0

- LPDDR2: cs_n=1, address=0

This convention matches the DFI v2.1 Table 2 default values.

### 16.1.7	Verification

Per-encoder unit tests:

- DDR2 encoder: round-trip table test against the JESD79-2 truth table

- LPDDR2 encoder: leverages our existing 30 round-trip tests for the CA bus encoder (tests/unit/test_lpddr_ca.py in the DV repo)

## 16.2	gear_out

### 16.2.1	Purpose

Replicate the 1-phase internal command bus onto N parallel DFI output phases, where N_PHASES ∈ {1, 2, 4}.

### 16.2.2	Behavior

- Phase 0 by convention carries the active command on the cycle it’s issued.

- Other phases idle (drive their deselected defaults).

- For N_PHASES == 1: pass-through. Synthesis will optimize the module away.

- For N_PHASES == 2 or 4: the output stage tracks which phase a command should land on. Some implementations assign the phase based on workload; ours uses the simpler “always phase 0” rule and relies on the PHY to serialize correctly per the DFI frequency-ratio spec.

### 16.2.3	Multi-Cycle Commands

LPDDR2’s 2-cycle commands are not split here. They are packed into a single 20-bit dfi_address word by the encoder; the PHY’s job is to split them into two DRAM cycles per DFI v2.1 §3.1.

### 16.2.4	Phase Naming

Per-phase signals follow the DFI naming convention with _p0 through _pN_PHASES-1 suffixes. For N_PHASES == 1, the suffix may be elided per DFI v2.1 §3.1 (“phase 0 may exclude the suffix”), but this controller always includes the suffix for naming clarity.

### 16.2.5	Write / Read Data Phase Assignment

The write-data and read-data sub-interfaces also replicate per phase. Two additional parameters control phase placement:

- WRPHASE — which phase carries the active write-data beat (default: phase 0)

- RDPHASE — which phase carries the active read-data beat (default: phase 0)

These are PHY-vendor-specific in real silicon; we make them parameters so a real-PHY integration can override.

### 16.2.6	Idle Phase Drives

The gear output stage drives the per-phase signals to their deselected idle values per DFI v2.1 Table 2 when no command is being issued on that phase. This includes both the inactive phases when a single-phase command is issued, and all phases when the scheduler issues NOP.

### 16.2.7	Verification

Reuses the DFI BFM’s DFIPhaseAdapter (Python) as a reference model — the same gear ratios, the same conventions. The RTL output passes through the adapter and round-trips via the BFM monitor.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Write and Read Data Paths

The write and read data paths bridge the AXI W / R channels and the DFI wrdata / rddata sub-interfaces with the JEDEC-required CWL / CL timing alignment.

## 17.1	wdata_path

### 17.1.1	Purpose

Stream AXI W beats onto the DFI write-data bus with correct CWL alignment relative to the issued WR command.

### 17.1.2	Inputs

- AXI W channel beats (after axi4_slave accepts them into the internal FIFO)

- Issued WR command notification from scheduler (includes AXI ID and burst length)

- CWL value from csr_slave (programmed at init)

- WRPHASE parameter from elaboration

### 17.1.3	Outputs

- dfi_wrdata — write data, driven on the correct phase

- dfi_wrdata_en — driven high CWL cycles after the WR command for the burst duration

- dfi_wrdata_mask — byte mask, driven from inverted AXI strobes (DFI convention: 1 = mask, 0 = write)

### 17.1.4	Behavior

- On WR command issue, the scheduler tags the AXI ID and burst length.

- The write-data path begins a CWL-cycle countdown.

- After CWL cycles, the path pulls beats from the AXI W FIFO matching the tagged ID and drives them onto dfi_wrdata for the burst duration.

- dfi_wrdata_mask is computed from the AXI strb field with bit inversion (AXI: 1 = byte enable; DFI: 1 = mask).

- After the last beat, dfi_wrdata_en deasserts.

### 17.1.5	Write Command Pipeline

WR Command Pipeline with CWL Alignment

Source: 07_wr_pipeline.mmd

### 17.1.6	B-Response Generation

A B-channel response is generated when the WR command is scheduled, not when the data commits to the DRAM array. This matches AXI4 posted-write semantics and decouples B latency from DRAM-side delays.

### 17.1.7	Write Cancellation

Not supported. Once a WR is committed by the scheduler, it must drain. The AXI master must not retract a posted write.

## 17.2	rdata_path

### 17.2.1	Purpose

Sample DFI rddata with correct CL alignment relative to the issued RD command, and stream the beats back on the AXI R channel with the correct ID and burst tagging.

### 17.2.2	Inputs

- DFI rddata, rddata_valid — driven by the PHY

- Issued RD command notification from scheduler (AXI ID, burst length)

- CL value from csr_slave

- RDPHASE parameter

### 17.2.3	Outputs

- AXI R channel — rdata, rid, rresp, rlast, rvalid

### 17.2.4	Behavior

- On RD command issue, the scheduler tags the AXI ID and burst length.

- The read-data path begins a CL-cycle countdown.

- After CL cycles, the path samples rddata and rddata_valid for the burst duration.

- Valid beats are tagged with the original AXI ID and streamed onto the AXI R channel.

- rlast is asserted on the final beat of each AXI transaction.

### 17.2.5	Read Command Pipeline

RD Command Pipeline with CL Alignment

Source: 08_rd_pipeline.mmd

### 17.2.6	Out-of-Order Completion

The path supports out-of-order R-channel completion across AXI IDs (when AXI_OOO_ACROSS_IDS is enabled). Within an ID, ordering is preserved.

### 17.2.7	Read Error Handling

If the burst’s underlying PHY reports an issue (e.g., uncorrected parity error — though DDR2 / LPDDR2 don’t have CA parity), rresp is set to SLVERR for the affected beats. In v1 of this controller, all reads return OKAY.

### 17.2.8	Read Data Capture Timing

CL is programmed via CSR at init. The path uses the programmed value directly; it does not learn CL from the PHY. This works because LPDDR2 init sequence has the controller program MR2 with the desired CL / CWL values, and the same value is mirrored in CSR.

### 17.2.9	Stale Read Suppression

If the AXI R channel is back-pressured for an extended period, the path will eventually report a timeout. The default timeout is 1024 cycles; configurable via CSR. The timeout indicates a downstream consumer is stuck, which is typically a system-level bug rather than a controller bug.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	DFI Master and CSR Slave

The two final modules in the controller hierarchy: dfi_master (final DFI signal aggregation) and csr_slave (APB3 configuration interface).

## 18.1	dfi_master

### 18.1.1	Purpose

Aggregate command, write-data, read-data, and init-control signals into the final DFI v2.1 signal set. Tie off the unused DFI sub-interfaces to their idle values. Provide a single point of contact with the PHY for clean integration.

### 18.1.2	Sub-Interfaces Driven

This controller drives only the three core DFI sub-interfaces required for v2.1 LPDDR2 operation:

- Command Interface (MC → PHY): all command-bus signals

- Write Data Interface (MC → PHY): wrdata, wrdata_en, wrdata_mask

- Read Data Interface (MC ↔ PHY): rddata, rddata_valid, rddata_en, and LPDDR2’s rddata_dnv

### 18.1.3	Sub-Interfaces Tied Idle

The remaining DFI v2.1 sub-interfaces are present in the port list (for ABI compatibility) but driven to their idle values:

- Status Interface — only init_complete is consumed; others tied off

- Update Interface — MC drives ctrlupd_req = 0; PHY’s phyupd_req is acked but no action taken

- Training Interface — held idle; no training in DDR2 / LPDDR2

- Low Power Control Interface — held idle; we use CKE-based power management directly

This means the controller is compatible with PHYs that support full DFI v2.1 but doesn’t require them to.

### 18.1.4	Signal Aggregation

All per-phase signals from cmd_encoder (through gear_out), wdata_path, and rdata_path arrive at dfi_master and are routed to the appropriate output ports. The module is essentially a wiring stub plus tie-offs.

### 18.1.5	Why a Dedicated Module

Putting the tie-offs in a single module keeps the rest of the controller PHY-detail-free. If we later need to enable a sub-interface (e.g., the training interface for a real PHY that requires it), the change is localized.

## 18.2	csr_slave

### 18.2.1	Purpose

APB3 slave for configuration and observation. All controller parameters that need to be programmable at runtime (rather than build time) are exposed here.

### 18.2.2	Bus Protocol

Standard APB3 slave. Signals:

| Signal | Width | Direction | Purpose |
| --- | --- | --- | --- |
| psel | 1 | input | Slave selected by APB master |
| penable | 1 | input | Access phase 2 (data transfer) |
| pwrite | 1 | input | Write (1) or read (0) |
| paddr | 12 | input | Address (4 KB region) |
| pwdata | 32 | input | Write data |
| prdata | 32 | output | Read data |
| pready | 1 | output | Slave ready to complete |
| pslverr | 1 | output | Error response |

### 18.2.3	Address Map Overview

A 4 KB region with the following high-level partitioning:

| Range | Purpose |
| --- | --- |
| 0x000 – 0x00F | Top-level control and status |
| 0x010 – 0x01F | Packed timing parameters |
| 0x020 – 0x02F | Mode-register configuration values |
| 0x030 – 0x03F | LPDDR2-specific (PASR, etc.) |
| 0x040 – 0x04F | Scheduler / refresh / page-policy tuning |
| 0x080 – 0x0FF | Per-bank observation counters |
| 0x100 – 0x1FF | System-wide observation counters |
| 0xFF0 – 0xFFC | Module identification |

The full register map is documented in Section 6.3.

### 18.2.4	Clock Domain Crossing

The CSR slave runs on apb_pclk, which is independent of mc_clk. A 2-flop synchronizer with handshake protocol crosses configuration writes into the mc_clk domain. The handshake guarantees atomic register update from the controller’s perspective.

### 18.2.5	Read Latency

APB read latency is implementation-defined. This slave uses single-cycle reads for control / status registers (the paddr ≤ 0x07F range) and 2-cycle reads for observation counters (which require CDC of the live counter value into the APB domain). pready deasserts during the second cycle of an observation-counter read.

### 18.2.6	Configuration Quiet Period

Certain parameters (e.g., MR values, timings) can only take effect during a “quiet period” — when no DRAM commands are in flight. The CSR slave does not enforce quiet-period writes; the SoC is responsible for sequencing. Writing during normal operation may cause subtle protocol violations.

### 18.2.7	Recommended Programming Sequence

- Hold mc_rstn asserted.

- Program all timing parameters and MR values via CSR.

- Configure PASR mask (LPDDR2 only).

- Configure characterization parameters (PAGE_POLICY runtime override, REFPB_POLICY runtime override, etc.).

- Release mc_rstn.

- Wait for STATUS.init_done == 1.

- Begin AXI traffic.

Skipping CSR programming uses the build-time defaults, which are conservative (HAPPY_HYBRID for LPDDR2 page policy, DARP for REFPB policy).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	AXI4 Slave Interface

The controller exposes a standard AXI4 slave for SoC connectivity.

## 19.1	Channels

Standard AXI4 — all five channels (AW, W, B, AR, R) are mandatory. No exclusive access support; AxLOCK is ignored.

### 19.1.1	AW (Write Address) — SoC master → controller slave

| Signal | Width | Notes |
| --- | --- | --- |
| awid | AXI_ID_WIDTH | Transaction ID |
| awaddr | AXI_ADDR_WIDTH | Flat byte address |
| awlen | 8 | Burst length minus 1 |
| awsize | 3 | Beat size; up to bus width |
| awburst | 2 | INCR mandatory; FIXED/WRAP optional |
| awcache | 4 | Ignored; treated as 0010 (modifiable) |
| awprot | 3 | Ignored |
| awqos | 4 | Forwarded to scheduler as priority hint |
| awvalid | 1 | Master asserts |
| awready | 1 | Controller asserts when slot available |

### 19.1.2	W (Write Data)

| Signal | Width | Notes |
| --- | --- | --- |
| wdata | AXI_DATA_WIDTH | Beat data |
| wstrb | AXI_DATA_WIDTH/8 | Byte enables; inverted to wrdata_mask |
| wlast | 1 | Final beat of burst |
| wvalid | 1 | Master asserts |
| wready | 1 | Controller asserts on FIFO space |

Note: AXI4 W channel has no wid; beats are presumed in the order their AW transactions were issued. The controller tracks IDs internally based on AW arrival order.

### 19.1.3	B (Write Response)

| Signal | Width | Notes |
| --- | --- | --- |
| bid | AXI_ID_WIDTH | Matches awid |
| bresp | 2 | OKAY or SLVERR |
| bvalid | 1 | Controller asserts on response ready |
| bready | 1 | Master asserts |

### 19.1.4	AR (Read Address)

| Signal | Width | Notes |
| --- | --- | --- |
| arid | AXI_ID_WIDTH | Transaction ID |
| araddr | AXI_ADDR_WIDTH | Flat byte address |
| arlen | 8 | Burst length minus 1 |
| arsize | 3 | Beat size |
| arburst | 2 | INCR mandatory; others optional |
| arcache | 4 | Ignored |
| arprot | 3 | Ignored |
| arqos | 4 | Forwarded to scheduler as priority |
| arvalid | 1 | Master asserts |
| arready | 1 | Controller asserts when slot avail |

### 19.1.5	R (Read Data)

| Signal | Width | Notes |
| --- | --- | --- |
| rdata | AXI_DATA_WIDTH | Beat data |
| rid | AXI_ID_WIDTH | Matches arid |
| rresp | 2 | OKAY (v1 always) |
| rlast | 1 | Final beat of burst |
| rvalid | 1 | Controller asserts when beat ready |
| rready | 1 | Master asserts |

## 19.2	Ordering Rules

- Within an ID: AXI4 ordering preserved. Reads from arid = X return in AR order; writes to awid = X commit in AW order.

- Across IDs: Out-of-order completion permitted by default (AXI_OOO_ACROSS_IDS = true). Disabling forces in-order across IDs at the cost of bandwidth.

## 19.3	Burst Type Support

| Burst | Mandatory | Notes |
| --- | --- | --- |
| FIXED | optional | Enable with SUPPORT_FIXED_BURST; rarely useful for DRAM |
| INCR | mandatory | Standard incrementing burst |
| WRAP | optional | Enable with SUPPORT_WRAP_BURST |

## 19.4	Burst Length

AXI4 allows up to 256 beats. If a burst exceeds the remaining columns in its target row, axi4_slave splits internally; the SoC sees a single AW with the expected response time.

## 19.5	Narrow Transfers

AXI4 supports awsize < bus width. The narrow case is handled via wstrb — the unused byte lanes are masked. The DRAM commits only the unmasked bytes.

## 19.6	Posted Writes

B-channel responses are generated when the WR command is scheduled, not when wrdata commits. This is AXI4-compliant (OKAY indicates the transaction was accepted, not committed) and reduces master-side latency.

## 19.7	Backpressure

- AW / AR .ready deassert when txn_queue is full

- W .ready deasserts when the write-data FIFO is full

- R / B .valid follow standard protocol; consumers may stall indefinitely without controller misbehavior

## 19.8	AXI4 Reset

axi_aresetn is the AXI reset. When asserted, all AXI state is cleared. The internal controller pipeline is also reset (the two reset signals are tied together in v1 of this controller).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	DFI v2.1 Master Interface

The controller drives DFI v2.1 master signals to the PHY. DFI v2.1 is the first DFI revision that supports LPDDR2; this controller targets that revision.

## 20.1	Per-Phase Signal Naming

DFI v2.1 §3.1 defines per-phase signals with _pN suffix where N is the phase index. For N_PHASES = 4, signals exist as _p0, _p1, _p2, _p3. For N_PHASES = 1, signals can elide the suffix; this controller always emits the suffix for clarity.

## 20.2	Command Interface (MC → PHY)

| Signal | Width | Notes |
| --- | --- | --- |
| dfi_address | DFI_ADDR_WIDTH | DDR2: row/col address; LPDDR2: 20-bit packed CA word |
| dfi_bank | BANK_WIDTH | DDR2: bank; LPDDR2: held idle (per Table 1) |
| dfi_cas_n | 1 | DDR2: CAS encoding; LPDDR2: held idle |
| dfi_cke | 1 | Clock enable |
| dfi_cs_n | 1 | Chip select; asserted on command issue |
| dfi_odt | 1 | DDR2 only; LPDDR2 held low |
| dfi_ras_n | 1 | DDR2: RAS encoding; LPDDR2: held idle |
| dfi_reset_n | 1 | DDR2 only (DDR2 has no reset); LPDDR2 driven |
| dfi_we_n | 1 | DDR2: WE encoding; LPDDR2: held idle |

## 20.3	Write Data Interface (MC → PHY)

| Signal | Width | Notes |
| --- | --- | --- |
| dfi_wrdata | DFI_DATA_WIDTH | Write data; driven CWL cycles after WR |
| dfi_wrdata_en | 1 | High during valid write data beats |
| dfi_wrdata_mask | DFI_DATA_WIDTH/8 | 1 = mask byte; 0 = write (DFI convention) |

## 20.4	Read Data Interface (PHY ↔ MC)

| Signal | Width | Direction | Notes |
| --- | --- | --- | --- |
| dfi_rddata | DFI_DATA_WIDTH | PHY → MC | Read data, CL cycles after RD |
| dfi_rddata_valid | 1 | PHY → MC | High when beat is valid |
| dfi_rddata_en | 1 | MC → PHY | High when MC expects to capture read |
| dfi_rddata_dnv | 1 | PHY → MC | LPDDR2 only: Data-Not-Valid |

## 20.5	Status Interface

Only init_complete is consumed:

| Signal | Width | Direction | Notes |
| --- | --- | --- | --- |
| dfi_init_complete | 1 | PHY → MC | High when PHY has finished its init |

Other status signals (dfi_data_byte_disable, etc.) are not consumed.

## 20.6	Sub-Interfaces NOT Driven

The following DFI v2.1 sub-interfaces exist in the port list but are tied to idle by dfi_master:

- Update Interface — ctrlupd_req, phyupd_req, etc.

- Training Interface — all rdlvl / wrlvl signals

- Low Power Control Interface — handled via CKE in command interface instead

## 20.7	Reset

| Signal | Width | Direction | Notes |
| --- | --- | --- | --- |
| dfi_rstn | 1 | MC → PHY | DFI reset; controlled by init_engine |

## 20.8	Timing Parameters Honored

The controller respects the following DFI v2.1 timing parameters:

- tCTRL_DELAY — PHY-reported command-launch delay (read from PHY’s reported value; programmed via CSR if PHY has no advertise mechanism)

- tPHY_WRDATA — write data launch delay relative to WR command (default 0; programmable)

- tPHY_RDLAT — read data return latency relative to RD command (default = CL; programmable)

## 20.9	Clock Domain

DFI signals are launched on mc_clk (the controller side). The PHY consumes them on phy_clk which runs at N_PHASES × mc_clk rate. The gear output stage handles the phase replication; the PHY handles the actual serialization to DRAM cycles.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	APB CSR Slave

The controller exposes an APB3 slave port for configuration and observation.

## 21.1	Signals

| Signal | Width | Direction | Notes |
| --- | --- | --- | --- |
| apb_pclk | 1 | input | APB bus clock (independent of mc_clk) |
| apb_presetn | 1 | input | APB reset (active low) |
| psel | 1 | input | Slave selected |
| penable | 1 | input | Access phase 2 |
| pwrite | 1 | input | Write (1) or read (0) |
| paddr | 12 | input | 4 KB address space |
| pwdata | 32 | input | Write data |
| prdata | 32 | output | Read data |
| pready | 1 | output | Slave ready |
| pslverr | 1 | output | Error response |

## 21.2	APB3 Conformance

Standard APB3 — single-cycle accesses for the typical case, multi-cycle for observation-counter reads (which require CDC).

## 21.3	Error Behavior

pslverr is asserted in these cases:

- Write to a read-only register (e.g., STATUS)

- Read from a write-only register (none defined in v1)

- Access to an unimplemented address (any address not listed in the register map)

Errors do not affect controller state — the APB transaction simply terminates with pslverr = 1.

## 21.4	Clock Domain Crossing

The CSR slave operates on apb_pclk, independent of mc_clk. A 2-flop synchronizer handshake transfers configuration writes into the mc_clk domain. The handshake guarantees atomic register update from the controller’s perspective.

Observation reads use a parallel CDC mechanism: snapshot the live counter in the mc_clk domain, then transfer to apb_pclk. pready deasserts during the transfer (typically 2 apb_pclk cycles).

## 21.5	Register Map

The full address map is documented in Section 6.3.

## 21.6	Configuration Sequencing

See csr_slave description in §3.8 for the recommended programming sequence:

- Hold mc_rstn asserted

- Program timing parameters and MR values

- Configure PASR mask (LPDDR2)

- Configure characterization parameters

- Release mc_rstn

- Wait for STATUS.init_done

- Begin AXI traffic

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Build-Time vs. Runtime Configuration

This controller distinguishes between build-time parameters that set the silicon footprint and runtime CSR registers that select behavior within that footprint. The two layers serve different purposes and should not be confused. This chapter applies the principle to every configurable knob in the design.

## 22.1	Principle

| Layer | What it controls | Frozen at | Cost of changing |
| --- | --- | --- | --- |
| Build-time parameter | Synthesized silicon footprint: signal widths, table depths, queue sizes, instantiation counts, encoder selection, MAX bounds of runtime values | Elaboration time | Full RTL rebuild + place-and-route + tape-out (silicon) or bitstream rebuild (FPGA) |
| Runtime CSR register | Behavior selection within the synthesized footprint: policy choices, threshold values, on/off toggles, active counts up to the build-time MAX | Programmable any time the configuration quiet point is reached (typically during init or under SoC orchestration) | A single APB write |

The architectural rule is straightforward: if a knob is changing silicon area, it must be build-time. If a knob is only changing behavior of already-synthesized hardware, it must be runtime.

## 22.2	Why This Distinction Matters

A controller designed without this distinction tends to collapse one of two ways:

- Too much build-time — every algorithmic choice is a parameter that requires a new bitstream / silicon revision to evaluate. Characterization becomes impossibly slow; post-silicon debug becomes impossible. Most academic prototype controllers fall here.

- Too much runtime — every option is always synthesized, including ones that will rarely be used. Area balloons; timing closure suffers; the design becomes harder to verify because every code path is live. This is the failure mode of “ultra-configurable” commercial IP.

This controller targets the middle: build-time picks what’s physically present; runtime picks what’s active.

## 22.3	The Hybrid Pattern: “Build-Time MAX + Runtime Active”

Many of this controller’s knobs follow a hybrid pattern where the build-time parameter sets a synthesized MAX, and a paired runtime CSR field selects the actually-used value within that MAX.

Concrete example for LOOKAHEAD_DEPTH:

Build-time:    LOOKAHEAD_DEPTH_MAX = 4
               → synthesizes 4 row-address comparators per bank machine

Runtime CSR:   sched_tuning.lookahead_active [3:0]
               → scheduler peeks min(LOOKAHEAD_DEPTH_MAX, lookahead_active)
                 same-bank requests per cycle

For debug:     Write lookahead_active = 0 to force pure HAPPY/page-policy mode
               Write lookahead_active = 4 to use the full synthesized window
               No rebuild required.

The same pattern applies to TXN_QUEUE_DEPTH, AGE_MAX, REFRESH_DEFER_MAX, INIT_ZQ_RETRIES, ZQCS_FREQ_HZ, and others — area is paid once at synthesis for the MAX; runtime selects how much of the synthesized hardware is actively gated.

## 22.4	The “Build-Time + Runtime Mux” Pattern

For algorithmic choices where multiple alternatives have similar area cost, the build-time parameter selects which code paths are synthesized, and a runtime CSR field picks which of the synthesized paths is active.

Concrete example for PAGE_POLICY:

Build-time:    PAGE_POLICY = HAPPY_HYBRID
               → synthesizes OPEN, CLOSE, and HAPPY decision logic
                 (plus the HAPPY predictor table)

Runtime CSR:   refresh_tuning.page_policy_or [1:0]
               00 = use build-time default (HAPPY_HYBRID here)
               01 = force OPEN
               10 = force CLOSE
               11 = force HAPPY_HYBRID

For debug:     Sweep policies on real workloads with no rebuilds.

The build-time choice PAGE_POLICY = OPEN would have omitted the HAPPY predictor table from the synthesized design entirely, saving area but losing the ability to switch to HAPPY at runtime. The build-time choice PAGE_POLICY = HAPPY_HYBRID synthesizes everything, paying the area cost for the predictor table, in exchange for the ability to switch among any of the three at runtime.

The same pattern applies to REFPB_POLICY, ADDR_MAP_SCHEME, SCHEDULER_MODE, and HAPPY enable/disable.

## 22.5	The “Runtime Only” Pattern

A small number of knobs have zero area cost and are runtime-only: simple counter compare values, timeout thresholds, retry counts. Examples include ZQCS_FREQ_HZ, INIT_ZQ_RETRIES, and the various observation-counter clear-on-read controls.

These have no build-time presence at all.

## 22.6	Configuration Quiet Points

Runtime CSR writes that change scheduler or refresh-manager behavior take effect at the next configuration quiet point — defined as: no DRAM commands in flight, no refresh sequence in progress, and the relevant FSM in its idle state. The CSR slave does not enforce quiet-point writes; the SoC firmware is responsible for sequencing. Writing during normal operation is well-defined but the change does not apply until the next quiet point — which can be enforced by writing to the CTRL.config_apply strobe to force a brief drain.

Writes to register-file CSRs that have no FSM impact (timing parameter overrides, MR override values, observation counters) take effect immediately and have no quiet-point requirement.

## 22.7	Summary

The following two chapters document this controller’s complete configuration interface:

- §5.2 Top-Level Parameter Table — every build-time parameter, with its type, range, governing section, and (where applicable) the paired runtime CSR field.

- §5.3 Characterization Knobs — the subset of build-time + runtime knobs intentionally exposed for performance sweeping during silicon / FPGA characterization.

The full runtime CSR register map is in §6.3.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Top-Level Parameter Table

All build-time parameters for the controller, with type, default, valid range, the section that governs their use, and a one-line purpose summary.

## 23.1	Parameter Table

| Parameter | Type | Default | Range | Section | Purpose |
| --- | --- | --- | --- | --- | --- |
| MEMTYPE | string | "DDR2" | {"DDR2", "LPDDR2"} | §3.6 | Picks encoder + step table at build time |
| N_PHASES | int | 4 | {1, 2, 4} | §3.6 | DFI frequency-ratio gear |
| AXI_DATA_WIDTH | int | 64 | {64, 128, 256} | §3.1 | AXI4 data bus width |
| AXI_ID_WIDTH | int | 4 | 1–16 | §3.1 | AXI ID tag width |
| AXI_ADDR_WIDTH | int | 32 | 24–40 | §3.1 | AXI flat address width |
| AXI_OOO_ACROSS_IDS | bool | true | {true, false} | §3.1 | Allow out-of-order completion across AXI IDs |
| SCHEDULER_MODE | enum | OOO | {OOO, INORDER} | §3.2 | OOO = synthesize the full FR-FCFS scheduling logic (default; debug-friendly — INORDER mode is then available via runtime CSR). INORDER = omit FR-FCFS at elaboration; smaller area but permanent in-order operation. Runtime override: SCHED_TUNING.force_inorder forces FIFO at runtime without rebuild when OOO is synthesized. |
| SUPPORT_FIXED_BURST | bool | false | {true, false} | §3.1 | AXI4 FIXED burst type support |
| SUPPORT_WRAP_BURST | bool | false | {true, false} | §3.1 | AXI4 WRAP burst type support |
| NUM_RANKS | int | 1 | {1, 2, 4} | §3.3 | Number of physical ranks (per-rank CS_n, CKE, ODT). 1 = single-rank point-to-point; 2 / 4 = DIMM-class. Rank bits are placed in the address map (see §3.1) and refresh state is tracked per (rank, bank). |
| NUM_BANKS | int | 8 | {4, 8} | §3.3 | Per-device bank count (per rank) |
| ODT_RULE_MULTIRANK | enum | JEDEC_DDR2 | {JEDEC_DDR2, JEDEC_LPDDR2, OFF} | §3.6 | Multi-rank ODT termination policy. JEDEC_DDR2 = read→ODT-high on the non-accessed rank during the read window, write→ODT-high on the accessed rank during the write window. JEDEC_LPDDR2 = LPDDR2’s simpler always-off-during-read rule. OFF = no ODT driving (point-to-point only; valid only for NUM_RANKS=1). Forced to OFF when NUM_RANKS=1. |
| ROW_WIDTH | int | 14 | 12–16 | §3.3 | Row-address width |
| COL_WIDTH | int | 10 | 9–12 | §3.3 | Column-address width |
| DFI_DATA_WIDTH | int | 32 | 16–128 | §3.6 | Per-phase DFI data width |
| DFI_ADDR_WIDTH | int | 14 (DDR2) / 20 (LPDDR2) | ≥14 (DDR2), ≥20 (LPDDR2) | §3.6 | DFI address bus width |
| WRPHASE | int | 0 | 0 .. N_PHASES-1 | §3.6 | Which DFI phase carries write data |
| RDPHASE | int | 0 | 0 .. N_PHASES-1 | §3.6 | Which DFI phase carries read data |
| TXN_QUEUE_DEPTH | int | 16 | 4–64 | §3.2 | Pending-transaction queue depth |
| AGE_MAX | int | 256 | 32–1024 | §3.2 | FR-FCFS anti-starvation cap |
| LOOKAHEAD_DEPTH_MAX | int | 4 | {0, 1, 2, 4} | §3.2 | Synthesized MAX number of same-bank lookahead comparators per bank. 0 omits lookahead at elaboration. Runtime active depth: SCHED_TUNING.lookahead_active picks how many of the synthesized comparators are gated on (0 .. MAX) without rebuild. Reset value of lookahead_active matches the build’s recommended default (typically 1). |
| PAGE_POLICY | enum | HAPPY_HYBRID (LPDDR2) / OPEN (DDR2) | {OPEN, CLOSE, HAPPY_HYBRID} | §3.2 | Fallback policy when lookahead is inconclusive |
| PAGE_PREDICTOR_TABLE_BITS | int | 12 | 8–16 | §3.2 | HAPPY predictor table size (synthesized only when PAGE_POLICY is HAPPY_HYBRID). Runtime kill-switch: SCHED_TUNING.happy_enable disables predictor lookup without rebuild — useful for A/B’ing HAPPY’s contribution to perf. |
| ADDR_MAP_SCHEMES_SYNTH | set | {ROW_MAJOR} | subset of {ROW_MAJOR, BANK_INTERLEAVE, XOR_HASH} | §3.1 | Schemes synthesized for runtime mux. Including more schemes adds a small amount of address-decode logic. Runtime mux: ADDR_MAP_TUNING.scheme_or picks the active scheme among the synthesized set. |
| ADDR_MAP_SCHEME_DEFAULT | enum | ROW_MAJOR | one of ADDR_MAP_SCHEMES_SYNTH | §3.1 | Reset-state choice among the synthesized schemes (used when CSR override is 00). |
| REFPB_POLICY | enum | DARP | {ROUND_ROBIN, OLDEST_FIRST, DARP} | §3.4 | Per-bank refresh scheduling policy |
| REFRESH_DEFER_MAX | int | 8 | 1–8 | §3.4 | Synthesized MAX refresh-postponement count (counter width is clog2(MAX)). Runtime active: REFRESH_TUNING.refresh_defer_active picks any value 1..MAX without rebuild. 1 = no batching (every tREFI fires one refresh). |
| SIM_INIT_SCALE | int | 1 | 1–10000 | §3.5 | Init-delay divider for sim runs (silicon always uses 1) |

Runtime-only knobs (no build-time presence — purely register-file values):

- REFRESH_TUNING.zqcs_freq_hz (16-bit) — Periodic ZQCS interval in Hz. 0 disables periodic ZQCS (init-only). Reset = 1 Hz.

- INIT_TUNING.zq_retries (4-bit) — ZQ calibration retry count before raising init_error. Reset = 3.

- INIT_TUNING.init_timeout_ms (8-bit) — Per-step init timeout. Reset = 10.

These cost zero silicon area beyond the register flops; making them build-time parameters would have been wrong (and was, in earlier revisions of this document).

## 23.2	Memtype-Dependent Constraints

Some parameters have memtype-dependent constraints checked at elaboration:

- MEMTYPE == "LPDDR2" ⇒ DFI_ADDR_WIDTH ≥ 20 (CA bus packing requirement)

- MEMTYPE == "DDR2" ⇒ DFI_ADDR_WIDTH ≥ ROW_WIDTH

- PAGE_POLICY == HAPPY_HYBRID ⇒ PAGE_PREDICTOR_TABLE_BITS must be in range; otherwise the predictor module is not instantiated and the parameter is ignored

## 23.3	Timing-Configuration Sanity Checks

The controller performs the following elaboration-time assertions on the loaded timing CSR values:

- tREFI_cycles ≥ 100 — the MC clock must be fast enough relative to tREFI that the refresh timer is well-resolved. If not, elaboration fails with a clear error.

- tRCD_cycles ≥ 2, tRP_cycles ≥ 2, tRC_cycles ≥ tRAS + tRP — basic JEDEC consistency checks.

- tRFC_cycles > tRP_cycles — the refresh-cycle time must exceed the precharge time.

These are not preventable run-time errors — they’re configuration bugs that should be caught at synthesis or boot time before silicon ships.

## 23.4	Parameter Categories

| Category | Parameters |
| --- | --- |
| Memtype | MEMTYPE |
| Geometry | NUM_RANKS, NUM_BANKS, ROW_WIDTH, COL_WIDTH |
| Multi-rank | NUM_RANKS, ODT_RULE_MULTIRANK |
| Bus widths | AXI_DATA_WIDTH, AXI_ID_WIDTH, AXI_ADDR_WIDTH, DFI_DATA_WIDTH, DFI_ADDR_WIDTH |
| Gear | N_PHASES, WRPHASE, RDPHASE |
| AXI features | AXI_OOO_ACROSS_IDS, SUPPORT_FIXED_BURST, SUPPORT_WRAP_BURST |
| Capacity | TXN_QUEUE_DEPTH, AGE_MAX |
| Characterization | LOOKAHEAD_DEPTH_MAX, PAGE_POLICY, PAGE_PREDICTOR_TABLE_BITS, ADDR_MAP_SCHEMES_SYNTH, REFPB_POLICY, REFRESH_DEFER_MAX (all paired with runtime CSR overrides — see §6.3) |
| Init | SIM_INIT_SCALE |

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Characterization Knobs

A subset of the build-time parameters is intentionally left configurable for characterization sweeping. These are the algorithmic choices that have research-backed alternatives where the best choice depends on workload. Rather than baking in a one-size-fits-all default, we expose them as parameters so the verification plan can sweep them on representative AXI traffic and pick defaults from data.

## 24.1	Characterization Parameters

| Parameter | Choices | Recommended default | Rationale |
| --- | --- | --- | --- |
| LOOKAHEAD_DEPTH | 0 / 1 / 2 / 4 | 1 | Same-bank lookahead window for exact auto-precharge decisions when the next same-bank request is already in the queue (typical for streaming / DMA). Falls back to PAGE_POLICY when the queue is shallow. 0 disables; 2 or 4 buys diminishing returns at modest comparator cost. |
| PAGE_POLICY | OPEN / CLOSE / HAPPY_HYBRID | HAPPY_HYBRID (LPDDR2), OPEN (DDR2) | Fallback for when lookahead is inconclusive. Streaming workloads favor OPEN; mobile-mixed favors HAPPY_HYBRID. Ghasempour 2015 reports near-optimal accuracy with small storage. |
| PAGE_PREDICTOR_TABLE_BITS | 8 – 16 | 12 | 4 KB predictor is sufficient for typical mobile workloads. Larger tables marginal improvement. |
| REFPB_POLICY | ROUND_ROBIN / OLDEST_FIRST / DARP | DARP | Chang HPCA 2014: DARP wins over round-robin on uneven bank activity (real workloads). |
| REFRESH_DEFER_MAX | 1 – 8 | 8 (JEDEC ceiling) | Stuecheli MICRO 2010: deferring up to 8 maximizes Elastic Refresh benefit. Set to 1 for real-time predictability. |
| TXN_QUEUE_DEPTH | 4 – 64 | 16 | Larger queues help bandwidth on multi-master AXI; smaller queues reduce area. |
| AGE_MAX | 32 – 1024 | 256 | Caps FR-FCFS starvation; larger values let row-hits dominate longer at the cost of tail latency. |
| N_PHASES | 1 / 2 / 4 | depends on PHY | 4 typical for FPGA SoCs; 2 for slower mobile platforms; 1 for simulation-only. |
| ADDR_MAP_SCHEME | ROW_MAJOR / BANK_INTERLEAVE / XOR_HASH | ROW_MAJOR | Default works for streaming; BANK_INTERLEAVE for random; XOR_HASH for adversarial. |

## 24.2	Sweep Plan

The verification plan (see §6.4) includes a characterization sweep that runs each parameter through its choices on a small benchmark suite:

### 24.2.1	Benchmark Suite

A key open question for the sweep is the interaction between LOOKAHEAD_DEPTH and PAGE_POLICY. With deep queues and conclusive lookahead, the choice of PAGE_POLICY barely matters because the fallback rarely fires. With shallow queues, PAGE_POLICY dominates. The sweep matrix should cover both extremes.

| Workload | Pattern |
| --- | --- |
| stream-read | Sequential 4 KB read bursts |
| stream-write | Sequential 4 KB write bursts |
| stream-mixed | Interleaved sequential read and write |
| random-narrow | Random small AXI bursts (1–4 beats) |
| random-wide | Random large AXI bursts (16–64 beats) |
| mobile-mixed | Bursty traffic with periodic large writes (modeled on smartphone GPU + display) |
| multi-master | Two AXI masters issuing concurrent traffic with different patterns |

### 24.2.2	Metrics Collected

For each (workload, parameter) combination, the sweep records:

- Sustained read bandwidth (% peak)

- Sustained write bandwidth (% peak)

- Average read latency

- 99th-percentile read latency

- Refresh blocking time (% of cycles where the bus was blocked by refresh)

- Row-hit rate (per-bank, averaged)

- Refresh deferral utilization (avg refresh_pending over time)

### 24.2.3	Default Selection

After the sweep, each parameter’s default is set to the value that:

- Maximizes sustained bandwidth across the benchmark suite

- Subject to: 99th-percentile latency does not regress by more than 20%

- Subject to: area cost is within budget

The defaults shown in the parameter table above are the expected outcomes based on the cited research. The actual defaults will be confirmed by the sweep.

## 24.3	Runtime Overrides via CSR

Some characterization parameters are also accessible at runtime via CSR (see §6.3). This allows in-system tuning without rebuild:

- PAGE_POLICY runtime override

- REFPB_POLICY runtime override

- Page predictor warm-up cycles and hysteresis

Runtime overrides take effect at the next “quiet point” (no commands in flight). The SoC is responsible for ensuring no DRAM traffic during the transition.

## 24.4	Observability for Sweep

The CSR exposes the following observation counters (described in §6.3):

- Per-bank row-hit count

- Average refresh latency

- Max transaction queue depth

- Page-predictor accuracy (HAPPY mode)

- Refresh deferral histogram

These feed back into the sweep tool for in-loop default selection.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Family-Wide Config Bits

This section is the family-wide config-bit registry — a catalog of runtime configuration bits used across the DDR2 / LPDDR2, DDR3 / LPDDR3, and DDR4 / LPDDR4 memory controllers. Some bits apply to all generations; some are introduced in one generation and absent in earlier ones; some are defined across the family but ignored in flavors where the underlying mechanism does not exist. That last case is intentional. Allocating the bit family-wide — even when unused — keeps the CSR map mechanically compatible across controllers and lets software treat the family as a single product line.

## 25.1	Why a Family-Wide Registry

Each controller in the family has its own CSR map (per §6.3 of each generation’s HAS). But the field encodings, bit positions within shared registers, and the names are aligned across the family. The intent:

- Software portability: a single device driver template understands the bit layout and only needs per-flavor capability checks.

- Bring-up portability: bring-up software written for DDR3 should mostly work on DDR2 with the same register reads.

- Verification portability: the cocotb test suite’s CSR-poke helpers can be reused.

- Debug consistency: the same “config dump” script produces a parseable output regardless of which flavor is running.

A bit may be marked N/A in a flavor; reads return 0 and writes are ignored (no pslverr, just silently ignored — because software portability requires that writing a fully-portable config word does not error on flavors that don’t implement every bit).

## 25.2	Applicability Notation

The Apply column uses this shorthand:

- all — applies to every flavor in the family

- DDR-only — applies to DDR2/3/4 only; LPDDR variants ignore

- LP-only — applies to LPDDR2/3/4 only; DDR variants ignore

- DDR3+ — applies to DDR3, DDR4, and forward; DDR2 ignores

- DDR4+ — applies to DDR4 only (introduced for bank groups); earlier ignore

- LPDDR3+ — LPDDR3 and LPDDR4 only

- LPDDR4-only — LPDDR4 only

## 25.3	SCHED_TUNING (Scheduling Family Bits)

Register block at the same offset (0x040) in every controller. These bits govern the FR-FCFS scheduler’s runtime behavior.

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| lookahead_active | 4 | all | Runtime lookahead window (0 .. build-time MAX). 0 disables lookahead. |
| force_inorder | 1 | all | Forces FR-FCFS into first-ready FIFO mode for debug / real-time. |
| happy_enable | 1 | all | Runtime kill-switch for the HAPPY page predictor. Reads as 0 in flavors built without HAPPY. |
| age_max_runtime | 8 | all | Anti-starvation age cap; runtime override of AGE_MAX. |
| txn_queue_high_water | 8 | all | Backpressure threshold; AXI awready deasserts when queue ≥ threshold. |
| lookahead_max_obs | 4 | all (R only) | Echo of build-time LOOKAHEAD_DEPTH_MAX. Software discovery. |
| qos_high_priority | 1 | all | When 1, scheduler boosts requests with awqos/arqos ≥ 8. |
| bank_group_balance | 1 | DDR4+ | Round-robin across bank groups in scheduler tie-break (DDR4/LPDDR4 only); N/A elsewhere. |

## 25.4	REFRESH_TUNING (Refresh Family Bits)

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| refpb_policy_or | 2 | LP-only | REFpb selection-policy override (LPDDR2/3/4); N/A for DDR-only flavors |
| page_policy_or | 2 | all | Runtime override for fallback page policy |
| refresh_defer_active | 4 | all | Active deferral count (1 .. build-time MAX). 1 = no batching. |
| zqcs_freq_hz | 16 | all | Periodic ZQCS interval. 0 = init-only. |
| fgr_mode | 2 | DDR3+ | Fine-Granularity Refresh mode (DDR3 1x/2x/4x; DDR4 fixed-rate, on-the-fly). N/A in DDR2. |
| refresh_per_rank | 1 | all | Force per-rank dispatch even when NUM_RANKS=1 (debug). Reset = 1 always when multi-rank. |

## 25.5	ADDR_MAP_TUNING (Address-Mapping Family Bits)

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| scheme_or | 2 | all | Runtime scheme mux (ROW_MAJOR, BANK_INTERLEAVE, XOR_HASH) |
| synth_mask_obs | 4 | all (R only) | Bitmask of synthesized schemes. Software discovery. |
| bg_field_position | 3 | DDR4+ | Bit position of bank-group field in flat address. N/A pre-DDR4. |
| xor_seed_runtime | 8 | all | Runtime seed for the XOR_HASH scheme (changes hash without rebuild). |

## 25.6	INIT_TUNING (Init Family Bits)

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| zq_retries | 4 | all | ZQ calibration retry count (DDR2 uses OCD; LPDDR2 uses MR10 ZQ). |
| init_timeout_ms | 8 | all | Per-step init timeout (ms; scaled by SIM_INIT_SCALE in sim). |
| wl_retries | 4 | DDR3+ | Write-leveling retry count. N/A in DDR2 (no write-leveling). |
| mpr_enable | 1 | DDR3+ | Multi-Purpose Register read-back during init. N/A pre-DDR3. |
| ca_train_enable | 1 | LPDDR3+ | LPDDR3/4 CA-bus training. N/A in LPDDR2 (no CA training). |
| cbt_enable | 1 | LPDDR4-only | Command-Bus-Training. N/A pre-LPDDR4. |

## 25.7	POWER_TUNING (Power-State Family Bits)

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| apd_idle_threshold | 16 | all | Cycles of bank-idleness before APD entry |
| srf_idle_threshold | 24 | all | Cycles before Self-Refresh entry |
| dpd_enable | 1 | LP-only | Deep-Power-Down enable. N/A for DDR-only flavors. |
| pasr_active | 1 | LP-only (R) | Reflects whether any rank has a non-zero PASR mask. R-only. |
| ckedis_after_sr | 1 | DDR3+ | Drop CKE after SR-entry for sub-mW power floor. N/A in DDR2. |
| low_freq_mode | 1 | DDR3+ | Reduced-frequency operation hint to PHY. N/A in DDR2. |

## 25.8	ECC_TUNING (Future — Inline ECC)

This block is reserved for future inline-ECC controllers. The bits are N/A in all current controllers (DDR2/3/4 and LPDDR2/3/4 v1) because inline ECC is out of scope for the current generation. Future revisions of any flavor that adopt inline ECC will populate this block.

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| ecc_enable | 1 | (reserved) | Enable inline ECC. Reads as 0 in all v1 controllers. |
| ecc_correct_mode | 2 | (reserved) | SECDED / Chipkill / off |

## 25.9	RANK_TUNING (Multi-Rank Family Bits)

These are present in any flavor with NUM_RANKS > 1. For NUM_RANKS = 1 builds, the bits read as 0 and writes are silently ignored.

| Bit Field | Width | Apply | Description |
| --- | --- | --- | --- |
| rank_enable_mask | 4 | all | Per-rank enable; clearing a bit suppresses all commands to that rank. Up to 4 ranks. |
| odt_rule_or | 2 | all | Runtime override for ODT_RULE_MULTIRANK (00 = build-time default; 01 = JEDEC_DDR2; 10 = JEDEC_LPDDR2; 11 = OFF). |
| num_ranks_obs | 3 | all (R only) | Echo of build-time NUM_RANKS. Software discovery. |
| cs_assert_cycles | 4 | all | Number of cycles to hold CS_n asserted per command; tunable for PHY timing margin. |

## 25.10	Quiet-Point Behavior

All runtime config bits take effect at the next configuration quiet point — no DRAM commands in flight, no refresh sequence in progress. The CSR slave does not enforce quiet points automatically; software requests one by writing CTRL.config_apply = 1, which triggers a controlled drain. Reads of STATUS.config_settled indicate when the override propagation has completed.

The quiet-point requirement is family-wide because every flavor has hand-off state between scheduler / refresh / power-state that would be corrupted by mid-flight reconfiguration.

## 25.11	Bit Discovery via the Capability Vector

At 0xFF8 of each controller’s CSR map, a capability vector is exposed:

| Bits | Field | Description |
| --- | --- | --- |
| 0 | cap_happy | 1 = HAPPY predictor synthesized |
| 1 | cap_bg_balance | 1 = bank-group balance scheduler logic present (DDR4+) |
| 2 | cap_fgr | 1 = Fine-Granularity Refresh supported (DDR3+) |
| 3 | cap_pasr | 1 = PASR mask is implemented (LP-only) |
| 4 | cap_dpd | 1 = Deep-Power-Down supported (LP-only) |
| 5 | cap_wl | 1 = Write-leveling implemented (DDR3+) |
| 6 | cap_cbt | 1 = CBT implemented (LPDDR4-only) |
| 7 | cap_xor_hash | 1 = XOR_HASH address scheme synthesized |
| 11:8 | cap_max_ranks | Build-time NUM_RANKS (echo of geometry) |
| 15:12 | cap_n_phases | Build-time N_PHASES |
| 19:16 | cap_memtype | 0 = DDR2, 1 = DDR3, 2 = DDR4, 4 = LPDDR2, 5 = LPDDR3, 6 = LPDDR4 |

Software queries this once at boot to decide which family-wide bits are meaningful on the current flavor.

## 25.12	Per-Flavor Applicability Matrix

| Bit | DDR2 | DDR3 | DDR4 | LPDDR2 | LPDDR3 | LPDDR4 |
| --- | --- | --- | --- | --- | --- | --- |
| lookahead_active | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| force_inorder | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| happy_enable | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| bank_group_balance | — | — | ✓ | — | — | ✓ |
| refpb_policy_or | — | — | — | ✓ | ✓ | ✓ |
| fgr_mode | — | ✓ | ✓ | — | — | — |
| bg_field_position | — | — | ✓ | — | — | ✓ |
| wl_retries | — | ✓ | ✓ | — | — | — |
| ca_train_enable | — | — | — | — | ✓ | ✓ |
| cbt_enable | — | — | — | — | — | ✓ |
| dpd_enable | — | — | — | ✓ | ✓ | ✓ |
| ckedis_after_sr | — | ✓ | ✓ | — | — | — |
| rank_enable_mask | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| odt_rule_or | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| xor_seed_runtime | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |

The “—” entries are present in the CSR map at the documented offsets but read 0 and ignore writes in that flavor. Software treats these as “soft-N/A” — not an error condition, just an absent feature.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	Clocks and Reset

## 26.1	Clock Domains

The controller uses three independent clocks.

### 26.1.1	mc_clk

Primary controller clock. All scheduler, bank machines, command pipeline, and AXI4-side logic run on mc_clk. The DFI master output stage launches its signals on mc_clk.

Typical frequency: 100 – 200 MHz on FPGA SoCs; up to 533 MHz on silicon for DDR2-1066 / LPDDR2-800.

### 26.1.2	phy_clk

PHY-side clock. Runs at N_PHASES × mc_clk rate. The PHY consumes DFI signals on phy_clk.

For N_PHASES = 4, phy_clk = 4 × mc_clk. For N_PHASES = 2, phy_clk = 2 × mc_clk. For N_PHASES = 1, phy_clk = mc_clk (they may be the same signal in this case).

The relationship is asynchronous from the controller’s perspective — the PHY handles the rate conversion. The DFI v2.1 specification’s frequency-ratio scheme is what makes this work without explicit FIFO crossings.

### 26.1.3	apb_pclk

APB CSR bus clock. Independent of mc_clk — typically a slower SoC management clock. CDC inside csr_slave transfers configuration writes into the mc_clk domain via 2-flop synchronizer plus handshake.

## 26.2	Reset Signals

### 26.2.1	mc_rstn (active low)

Primary controller reset. Synchronously deasserted on mc_clk. Resets:

- AXI4 slave state machines

- Transaction queue

- Scheduler and bank machines

- Cross-bank timers

- Refresh manager (clears tREFI counter and refresh_pending)

- Power-state FSM (transitions to RESET state)

- Init engine (resets to step 0)

- All observation counters

### 26.2.2	phy_rstn (active low)

Controlled by init_engine during the reset-release phase of the init sequence. Asserted at startup; deasserted as the first step of the init step table.

### 26.2.3	dfi_rstn (active low, MC → PHY)

DFI reset signal driven to the PHY. Controlled by init_engine per the init step table; allows the PHY to perform its own reset sequence during DRAM init.

### 26.2.4	apb_presetn (active low)

APB CSR bus reset. Synchronously deasserted on apb_pclk. Resets the CSR slave state machine.

## 26.3	Reset Sequencing

### 26.3.1	Cold Boot

- All resets asserted (mc_rstn, phy_rstn, dfi_rstn, apb_presetn)

- Clocks stable

- apb_presetn deasserted (SoC programs CSR if needed; otherwise defaults)

- mc_rstn deasserted

- Init engine starts; runs step table; drives phy_rstn and dfi_rstn per the sequence

- On init_done, power-state FSM transitions to ACTIVE

- Controller services AXI traffic

### 26.3.2	Warm Reset (Clock-Gated)

The SoC can warm-reset the controller without losing DRAM content:

- SoC requests self-refresh entry via CSR

- Controller acknowledges; DRAM is in self-refresh

- SoC asserts mc_rstn; clocks may be gated

- SoC deasserts mc_rstn; clocks resume

- Controller re-runs init from the table (the table is idempotent — it does not corrupt DRAM in self-refresh)

- Controller resumes normal operation

The init step table is designed to be cold-boot-correct and warm-boot-safe. The step list for LPDDR2 includes a check on MR4 for the current temperature; if the temperature changed during the warm reset, the controller updates its tREFI scale accordingly.

## 26.4	Clock Domain Crossing Summary

| Crossing | Mechanism | Latency |
| --- | --- | --- |
| apb_pclk → mc_clk | 2-flop sync + handshake | ~3-4 mc_clk cycles |
| mc_clk → apb_pclk | 2-flop sync (observation reads) | ~3-4 apb_pclk cycles |
| mc_clk → phy_clk | DFI frequency-ratio scheme (no explicit CDC) | 0 (combinational replication) |

## 26.5	Reset and Power Considerations

Note that DFI v2.1 §4.1 requires dfi_init_start to be asserted by the controller as part of init; the controller does this during the reset-release step. The PHY’s dfi_init_complete signal indicates that the PHY has finished its own initialization; the controller’s init engine waits on this before proceeding to MRS loads.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	Init Sequences

The full cold-boot init sequences for DDR2 and LPDDR2. These are the step lists embedded in ddr2_init_steps_pkg.sv and lpddr2_init_steps_pkg.sv.

## 27.1	DDR2 Cold Init

Per JESD79-2F §4.1 and Micron TN-47-09.

| Step | Type | Description |
| --- | --- | --- |
| 1 | SET_CTRL_BITS | Power stable; CKE = 0 for ≥ 200 µs |
| 2 | DELAY | NOP for ≥ 400 ns |
| 3 | SET_CTRL_BITS | CKE → 1, ODT = 0, RESET_N = 1 |
| 4 | ISSUE_CMD | PRECHARGE all banks (PREA with addr[10] = 1) |
| 5 | MRS_LOAD | EMRS(2) = 0 (extended mode register 2 defaults) |
| 6 | MRS_LOAD | EMRS(3) = 0 |
| 7 | MRS_LOAD | EMRS(1) = enable DLL |
| 8 | MRS_LOAD | MRS = reset DLL + initial CL / BL |
| 9 | ISSUE_CMD | PRECHARGE all |
| 10 | ISSUE_CMD | AUTO REFRESH (REF) |
| 11 | ISSUE_CMD | AUTO REFRESH (REF) |
| 12 | MRS_LOAD | MRS = final CL / BL / WR config (no DLL reset) |
| 13 | MRS_LOAD | EMRS(1) = OCD default |
| 14 | MRS_LOAD | EMRS(1) = OCD exit |
| 15 | END | Assert init_done |

Real-time delays embedded in post_delay: step 1 = 200 µs; step 2 = 400 ns; step 3 = 200 ns NOPs; intermediate MRS steps = tMRD (≥ 2 clocks).

## 27.2	LPDDR2 Cold Init

Per JESD209-2F §6.1 and Micron TN-46-22.

| Step | Type | Description |
| --- | --- | --- |
| 1 | SET_CTRL_BITS | tINIT1: CKE low ≥ 100 ns |
| 2 | SET_CTRL_BITS | tINIT2: CKE high ≥ 200 µs |
| 3 | DELAY | tINIT3: NOP ≥ 11 µs |
| 4 | MRS_LOAD | MR63 reset |
| 5 | DELAY | tINIT5: ≥ 10 µs |
| 6 | MRS_LOAD | MR10 ZQ calibration |
| 7 | WAIT_FOR_BIT | Wait dfi_init_complete (ZQ done) |
| 8 | MRS_LOAD | MR1: BL, WRAP, BT |
| 9 | MRS_LOAD | MR2: RL, WL |
| 10 | MRS_LOAD | MR3: drive strength |
| 11 | MRS_LOAD | MR4 read for temperature class (programs internal tREFI scale) |
| 12 | MRS_LOAD | MR16: PASR bank mask (default 0 = all banks refresh) |
| 13 | MRS_LOAD | MR17: PASR segment mask (default 0) |
| 14 | END | Assert init_done |

Real-time delays embedded in post_delay: step 1 = 100 ns; step 2 = 200 µs; step 3 = 11 µs; step 5 = 10 µs; step 7 timeout = 1 ms; subsequent MRS = tMRD.

## 27.3	Self-Refresh Entry (Both Memtypes)

Used during power-state FSM transitions:

| Step | Type | Description |
| --- | --- | --- |
| 1 | WAIT_FOR_BIT | Wait for all banks idle and refresh complete |
| 2 | ISSUE_CMD | PRECHARGE all |
| 3 | SET_CTRL_BITS | CKE low + ODT off (initiates self-refresh) |
| 4 | END | FSM transitions to SELF_REFRESH state |

## 27.4	Self-Refresh Exit (Both Memtypes)

| Step | Type | Description |
| --- | --- | --- |
| 1 | SET_CTRL_BITS | CKE high (initiates self-refresh exit) |
| 2 | DELAY | tXSNR (~200 ns) |
| 3 | END | FSM transitions to ACTIVE |

## 27.5	LPDDR2 Deep Power Down Entry

| Step | Type | Description |
| --- | --- | --- |
| 1 | WAIT_FOR_BIT | Wait for all banks idle and refresh complete |
| 2 | ISSUE_CMD | PRECHARGE all |
| 3 | ISSUE_CMD | DPD command (special LPDDR2 encoding) |
| 4 | SET_CTRL_BITS | CKE low |
| 5 | END | FSM transitions to DPD state |

## 27.6	LPDDR2 Deep Power Down Exit

DPD exit loses DRAM content; the full cold-init sequence (LPDDR2 init above) must run again. The DPD exit step table is effectively just:

| Step | Type | Description |
| --- | --- | --- |
| 1 | JUMP_TO_TABLE | Restart the LPDDR2 cold init step table from step 1 |

This is implemented by having the power-state FSM signal init_engine to restart, rather than a separate step table.

## 27.7	Sim Scaling

All post_delay fields are scaled by SIM_INIT_SCALE during simulation. For example, a 200 µs delay (about 20,000 cycles at 100 MHz) becomes 20 cycles at SIM_INIT_SCALE = 1000. This keeps test runtimes practical without changing the step table logic. Silicon builds always use SIM_INIT_SCALE = 1.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	CSR Register Map

The APB CSR slave exposes a 4 KB address space (12-bit paddr). All registers are 32 bits wide.

## 28.1	Control and Status (0x000 – 0x00F)

### 28.1.1	CTRL (0x000, R/W)

| Bit | Field | Reset | Description |
| --- | --- | --- | --- |
| 0 | init_start | 0 | Write 1 to start init |
| 1 | init_force_restart | 0 | Write 1 to force re-init even mid-sequence |
| 4 | pwr_req_low_power | 0 | Request power-down state |
| 5 | pwr_req_dpd | 0 | Request DPD (LPDDR2 only) |
| 6 | pwr_req_active | 0 | Request return to ACTIVE |
| 7 | pwr_req_self_refresh | 0 | Request self-refresh |
| 31 | soft_reset | 0 | Write 1 to assert internal soft reset |

### 28.1.2	STATUS (0x004, R only)

| Bit | Field | Description |
| --- | --- | --- |
| 0 | init_done | 1 when init complete |
| 1 | init_error | 1 on init failure |
| 7:4 | power_state | Current power-state FSM state (encoded) |
| 8 | pasr_active | LPDDR2: PASR mask is non-zero |
| 23:16 | init_step_dbg | Current init step number (for bring-up) |
| 31 | version_match | 1 when build matches expected version |

### 28.1.3	STATUS_HISTORY (0x008, R only)

Last 8 power-state transitions, 4 bits each. New transitions push the oldest off. Useful for bring-up debugging of power-state oscillation.

## 28.2	Timing Parameters (0x010 – 0x01F)

Packed timing parameters. Each register holds three to four 8-bit timing values.

### 28.2.1	TIMINGS_RC_RCD_RP (0x010, R/W)

| Bits | Field | Description |
| --- | --- | --- |
| 7:0 | tRC | tRC in cycles |
| 15:8 | tRCD | tRCD in cycles |
| 23:16 | tRP | tRP in cycles |
| 31:24 | tRAS | tRAS in cycles |

### 28.2.2	TIMINGS_RAS_RFC_REFI (0x014, R/W)

| Bits | Field | Description |
| --- | --- | --- |
| 15:0 | tRFC | tRFC (or tRFCab) in cycles |
| 31:16 | tREFI | tREFI in cycles |

### 28.2.3	TIMINGS_RRD_FAW_WTR (0x018, R/W)

| Bits | Field | Description |
| --- | --- | --- |
| 7:0 | tRRD | tRRD |
| 15:8 | tFAW | tFAW |
| 23:16 | tWTR | tWTR |
| 31:24 | tCCD | tCCD |

### 28.2.4	TIMINGS_CL_CWL_WR (0x01C, R/W)

| Bits | Field | Description |
| --- | --- | --- |
| 7:0 | CL | CAS latency |
| 15:8 | CWL | CAS write latency |
| 23:16 | tWR | Write recovery |
| 31:24 | tRFCpb | LPDDR2 per-bank tRFC |

## 28.3	Mode Register Values (0x020 – 0x02F)

### 28.3.1	MR0, MR1, MR2, MR3 (0x020, 0x024, 0x028, 0x02C, R/W)

Low 16 bits contain the MR value loaded during init. Upper 16 bits reserved.

## 28.4	LPDDR2-Specific (0x030 – 0x03F)

### 28.4.1	PASR_BANK_MASK_RANK<R> (0x030 + R×4, R/W; R = 0 .. NUM_RANKS−1)

| Bits | Field | Description |
| --- | --- | --- |
| 7:0 | pasr_banks | Per-rank bank mask (MR16); bit N = 1 means bank N is masked off on this rank |

One register per rank. For NUM_RANKS=1 only PASR_BANK_MASK_RANK0 (0x030) exists; higher ranks return pslverr.

### 28.4.2	PASR_SEG_MASK_RANK<R> (0x080 + R×4 within the multi-rank reserved area is reused — see register layout note)

For implementation simplicity, segment masks share the rank-indexed pattern: PASR_SEG_MASK_RANK0 lives at 0x034, and additional ranks consume successive 32-bit slots in the same window. Implementations that ship with NUM_RANKS > 1 may instead place the additional masks in the per-rank observation window (0x0C0+, see below) — the CSR map vector exposed at 0xFF8 provides discovery.

### 28.4.3	TEMP_DERATE_RANK<R> (0x038 + R×4, R only)

| Bits | Field | Description |
| --- | --- | --- |
| 1:0 | temp_class | From this rank’s MR4: 00 = nominal, 01 = 2x refresh, 10 = 4x refresh |

One register per rank — LPDDR2 ranks can run at different temperatures (e.g., one DRAM near a hot SoC die, another further away), so refresh derating is computed per-rank.

## 28.5	Scheduler / Refresh / Page Tuning (0x040 – 0x04F)

### 28.5.1	SCHED_TUNING (0x040, R/W)

| Bits | Field | Description |
| --- | --- | --- |
| 3:0 | lookahead_active | Active lookahead window (0..LOOKAHEAD_DEPTH_MAX). 0 disables lookahead at runtime, forcing the page-policy fallback. Reset value matches build-time default. |
| 4 | force_inorder | 0 = OoO FR-FCFS scheduling (default). 1 = force first-ready FIFO at runtime; row-hit reordering disabled. For debug, formal verification, or real-time-mode runs. Takes effect at next quiet point. |
| 5 | happy_enable | 0 = HAPPY predictor disabled at runtime (lookup returns “weakly hit-likely” regardless of training). 1 = predictor active. Reset = 1 if PAGE_POLICY ∈ HAPPY_HYBRID was synthesized; else 0. Useful for A/B’ing predictor contribution without rebuild. |
| 15:8 | age_max_runtime | Runtime override for AGE_MAX (clipped to ≤ build-time AGE_MAX). 0 = use build-time default. |
| 23:16 | txn_queue_high_water | Backpressure-assertion threshold (≤ build-time TXN_QUEUE_DEPTH). |
| 27:24 | lookahead_max_obs | (R only) Echo of build-time LOOKAHEAD_DEPTH_MAX. Lets software discover the synthesized ceiling. |

### 28.5.2	PAGE_PRED_TUNING (0x044, R/W, only when HAPPY enabled)

| Bits | Field | Description |
| --- | --- | --- |
| 15:0 | warmup_cycles | Cycles before predictor is trusted |
| 23:16 | hysteresis | Saturating-counter hysteresis |

### 28.5.3	REFRESH_TUNING (0x048, R/W)

| Bits | Field | Description |
| --- | --- | --- |
| 1:0 | refpb_policy_or | Runtime override: 00 = use build-time; 01 = RR; 10 = OLDEST_FIRST; 11 = DARP. Takes effect at next quiet point. |
| 3:2 | page_policy_or | Runtime override for PAGE_POLICY: 00 = build-time default; 01 = force OPEN; 10 = force CLOSE; 11 = force HAPPY_HYBRID (only valid when HAPPY was synthesized). Takes effect at next quiet point. |
| 7:4 | refresh_defer_active | Active refresh deferral count (1..REFRESH_DEFER_MAX). 1 = no batching. Larger values batch deferrals up to the synthesized MAX. |
| 31:16 | zqcs_freq_hz | Periodic ZQCS interval in Hz. 0 disables periodic ZQCS (init-only mode). Reset = 1 Hz. |

### 28.5.4	ADDR_MAP_TUNING (0x04C, R/W)

| Bits | Field | Description |
| --- | --- | --- |
| 1:0 | scheme_or | Runtime mux among synthesized schemes: 00 = build-time default; 01 = ROW_MAJOR; 10 = BANK_INTERLEAVE; 11 = XOR_HASH. Only schemes selected for synthesis via ADDR_MAP_SCHEMES_SYNTH are switchable; writing an unsynthesized scheme returns pslverr. |
| 7:4 | synth_mask_obs | (R only) Bitmask of schemes synthesized: bit 0 = ROW_MAJOR, bit 1 = BANK_INTERLEAVE, bit 2 = XOR_HASH. Lets software discover the available choices. |

### 28.5.5	INIT_TUNING (0x050, R/W)

| Bits | Field | Description |
| --- | --- | --- |
| 3:0 | zq_retries | ZQ calibration retry count before raising init_error. Reset = 3. Range 1–8. |
| 15:8 | init_timeout_ms | Per-step init timeout in ms (scaled by SIM_INIT_SCALE in sim). Reset = 10. Range 1–255. |

All runtime overrides take effect at the next configuration quiet point (no DRAM commands in flight, no refresh sequence in progress). The CSR slave does not enforce quiet points; the SoC may force a brief drain by strobing CTRL.config_apply.

## 28.6	Per-(Rank, Bank) Observation (0x080 – 0x0FF)

For each (rank R, bank N) pair within NUM_RANKS × NUM_BANKS, two registers. The address layout interleaves bank-first within rank-block; the per-rank stride is NUM_BANKS × 4 (32 bytes for 8-bank parts).

### 28.6.1	OBS_ROW_HIT_RANK<R>_BANK<N> (0x080 + (R × NUM_BANKS + N) × 4, R only)

Rolling row-hit count, rank R bank N. Resets when read or on soft_reset. For NUM_RANKS=1 this collapses to the single-rank OBS_ROW_HIT_BANK<N> layout at 0x080–0x09C.

### 28.6.2	OBS_REF_LATENCY_RANK<R>_BANK<N> (0x0C0 + (R × NUM_BANKS + N) × 4, R only)

Average refresh blocking time for rank R bank N. Per-rank tracking matters because per-rank PASR masks, temperature derating, and DARP scheduling can produce different refresh-blocking profiles across ranks even on the same DIMM.

## 28.7	System Observation (0x100 – 0x1FF)

| Offset | Register | Description |
| --- | --- | --- |
| 0x100 | OBS_TXN_QUEUE_DEPTH_MAX | Max queue depth observed |
| 0x104 | OBS_TXN_QUEUE_DEPTH_AVG | Time-averaged depth |
| 0x108 | OBS_REFRESH_PENDING_MAX | Max refresh_pending value observed |
| 0x10C | OBS_REFRESH_DEFER_HIST_0 | Refresh deferral histogram bin 0 |
| 0x110 | OBS_REFRESH_DEFER_HIST_1 | Histogram bin 1 |
| 0x114 | OBS_REFRESH_DEFER_HIST_2 | Histogram bin 2 |
| 0x118 | OBS_REFRESH_DEFER_HIST_3 | Histogram bin 3 |
| 0x120 | OBS_PAGE_PRED_ACCURACY | HAPPY mode: rolling prediction accuracy (%) |
| 0x130 | OBS_AXI_R_LATENCY_AVG | Avg AXI read latency in cycles |
| 0x134 | OBS_AXI_R_LATENCY_P99 | 99th-percentile read latency |
| 0x138 | OBS_AXI_W_LATENCY_AVG | Avg AXI write latency |

## 28.8	Module Identification (0xFF0 – 0xFFC)

### 28.8.1	ID (0xFF0, R only)

| Bits | Field | Description |
| --- | --- | --- |
| 7:0 | version | Build version |
| 15:8 | memtype | 0 = DDR2, 1 = LPDDR2 |
| 23:16 | n_phases | Gear ratio (1, 2, or 4) |
| 31:24 | module_id | 0xD2 (fixed) |

### 28.8.2	BUILD (0xFF4, R only)

Build hash / version word; lowered to upper bits at synthesis.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	Verification Strategy

A layered verification approach with the existing DFI BFM as the primary reference.

## 29.1	Layered Approach

### 29.1.1	Layer 1: Module-Level Unit Tests

Each of the 17 modules gets cocotb unit tests:

| Module | Test focus |
| --- | --- |
| axi4_slave | Channel handshakes, backpressure, burst splitting, ordering |
| addr_mapper | Address translation across ADDR_MAP_SCHEME modes |
| txn_queue | Insert / scan / removal; age saturation; row-hit caching |
| scheduler | FR-FCFS priority order; page-policy dispatch; refresh prioritization |
| page_predictor | HAPPY table read/write; accuracy after warm-up |
| bank_machine | FSM transitions; timing-counter enforcement |
| xbank_timers | tRRD / tFAW / tWTR / tCCD against JESD79-2 / JESD209-2 truth tables |
| refresh_mgr | tREFI counter; REFab / REFpb selection; DARP idle-bank logic |
| power_state | All FSM transitions; SR-entry/exit; DPD entry/exit (LPDDR2) |
| init_engine | Step-by-step execution against canonical init sequences |
| cmd_encoder | Round-trip table tests against JEDEC encoding |
| gear_out | 1-phase to N-phase replication; idle-phase handling |
| wdata_path | CWL alignment; ID tagging; mask conversion |
| rdata_path | CL alignment; OOO completion; back-pressure |
| dfi_master | Tie-off behavior; aggregation correctness |
| csr_slave | APB3 protocol; CDC; error responses |

### 29.1.2	Layer 2: Subsystem Tests

Multi-module integration tests:

- AXI frontend (axi4_slave + addr_mapper + txn_queue): full AXI transaction flow with diverse traffic

- Scheduler subsystem (scheduler + page_predictor + bank_machines + xbank_timers): scheduler decisions on representative queue contents

- Refresh subsystem (refresh_mgr + bank_machines): REFab / REFpb sequencing on uneven bank activity

- Init + power FSM (init_engine + power_state): full init followed by SR entry/exit cycle

### 29.1.3	Layer 3: End-to-End with DFI BFM Slave

The full controller drives the DFI BFM slave through our existing co-sim harness (tests/sim/dfi/test_litedram/... patterns in the DV repo).

- AXI4 stimulus generators issue representative traffic

- DFI BFM slave captures the controller’s command stream

- Assertions check: command sequencing matches AXI, timing constraints are honored, data integrity round-trips through the slave’s MemoryModel

### 29.1.4	Layer 4: External-Reference Cross-Validation

For DDR2: cross-validate against LiteDRAM’s DDR2 model.

- Generate LiteDRAM controller alongside ours

- Both drive the same DFI BFM slave with the same AXI stimulus

- Compare command streams; differences flag potential design issues

LPDDR2 has no LiteDRAM analog. Cross-validation for LPDDR2 is limited to the BFM master ↔ BFM slave path.

## 29.2	Characterization Sweep

Post-functional verification, the sweep runs each characterization parameter through its choices on the benchmark suite from §5.2:

### 29.2.1	Sweep Matrix

| Parameter | Sweep values |
| --- | --- |
| LOOKAHEAD_DEPTH | 0, 1, 2, 4 |
| PAGE_POLICY | OPEN, CLOSE, HAPPY_HYBRID |
| REFPB_POLICY | ROUND_ROBIN, OLDEST_FIRST, DARP |
| REFRESH_DEFER_MAX | 1, 2, 4, 8 |
| TXN_QUEUE_DEPTH | 8, 16, 32 |
| N_PHASES | (build-determined; usually 4) |

The full cross-product would be 4 × 3 × 3 × 4 × 3 = 432 builds; in practice we sweep one parameter at a time holding others at recommended defaults, which reduces to ~17 builds per memtype. The exception is the LOOKAHEAD_DEPTH × PAGE_POLICY pair — these interact strongly (the fallback policy only matters when lookahead is inconclusive), so this pair is swept as a full 4 × 3 = 12-point grid.

### 29.2.2	Sweep Outputs

Each (parameter, value) run produces a JSON report:

{
  "memtype": "LPDDR2",
  "page_policy": "HAPPY_HYBRID",
  "refpb_policy": "DARP",
  "workload": "mobile-mixed",
  "metrics": {
    "read_bw_pct": 73.2,
    "write_bw_pct": 68.1,
    "read_latency_avg_ns": 142,
    "read_latency_p99_ns": 198,
    "refresh_block_pct": 4.3,
    "row_hit_rate_avg": 0.81,
    "page_predictor_accuracy": 0.91
  }
}

These feed into a sweep visualization tool that selects defaults from data.

### 29.2.3	Out-of-Order Scheduler Trade-Offs

The centralized FR-FCFS scheduler proposed in §1 (Differentiating Features) is the most architecturally aggressive choice in this controller and warrants explicit characterization beyond just the sweep matrix above. Three concrete concerns:

- Timing-critical path scales with TXN_QUEUE_DEPTH. Scanning the full queue every cycle for the best candidate is a comparator pyramid: priority resolves over (ready-flag, row-hit-flag, age) for every entry simultaneously. At TXN_QUEUE_DEPTH = 16 this is well within an embedded-SoC clock budget; at 32 it stays comfortable; at 64 it becomes the critical path on the controller side. The characterization sweep should report scheduler combinational depth at synthesis time alongside the performance numbers — the right TXN_QUEUE_DEPTH is the largest value that doesn’t push the scheduler off the clock budget.

- Area cost is super-linear in queue depth. Each queue entry needs its full metadata replicated (bank, row, col, ID, age, row-hit cache). At depth 16, ~2K gates; at depth 64, ~10K gates — comparable to the rest of the scheduler combined. The HAPPY predictor’s gate cost is a separate line item.

- Verification cost is significantly higher than FIFO. OoO issue means assertions like “request N landed at cycle X” can’t be used. The DV plan relies on protocol-level scoreboarding: the BFM master records arrival order per ID, the BFM slave records issue order, and the scoreboard verifies the AXI4 per-ID ordering invariant is preserved. Within-ID ordering violations are a higher-severity bug than performance regressions and should be caught at Layer 1 unit tests, not deferred to integration.

- Fairness tail matters under adversarial workloads. With AGE_MAX as the only anti-starvation cap, pathological patterns can drive 99th-percentile latency well above mean. The random-narrow and multi-master workloads in the benchmark suite intentionally exercise this; the mobile-mixed workload simulates the common case. The recommended default for AGE_MAX should be picked based on tail-latency targets, not throughput targets.

These concerns are unique to the OoO scheduler design choice — a simpler per-bank FIFO controller (the typical open-source pattern) would not face them, at the cost of leaving row-hit reordering performance on the table.

## 29.3	Verification Hooks

Internal observation outputs are exposed via CSR (see §6.3) for in-system characterization. Additionally, the controller emits internal “event” signals that can be tied to waveform-dump triggers:

- ev_init_done

- ev_refresh_issued

- ev_page_conflict_detected

- ev_queue_full

- ev_refresh_pending_critical

Specific events can be wired to a test-bench-controlled waveform-capture trigger during bring-up.

## 29.4	Coverage Targets

Functional coverage (collected during regression):

- All FSM states reached in each module

- All page policies exercised

- All refresh policies exercised

- All scheduler priority levels exercised

- All AXI burst types exercised (if enabled)

- All ADDR_MAP_SCHEME modes exercised

Code coverage targets: 100% line / branch coverage for all modules.

Assertion coverage: all design assertions (registered protocol checks) hit during regression.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	Open Issues and TODOs

Items where design choices were made provisionally and should be reviewed before the HAS is promoted to formal status.

## 30.1	Open Issues

### 30.1.1	1. AXI4 Narrow-Burst Handling Confirmation

AXI4 allows narrow transfers (data width less than bus width). The current plan is to handle them via byte-mask through wrdata_mask. Confirm the wstrb-to-wrdata-mask wiring honors the AXI partial-write semantics correctly, particularly for unaligned bursts.

### 30.1.2	2. AXI4 Read-Modify-Write Strobe Handling

Partial writes in DDR2 / LPDDR2 use wrdata_mask to mask byte lanes; this is straightforward. Confirm that per-byte masking is wired all the way through wdata_path and that axi4_slave doesn’t widen narrow writes implicitly.

### 30.1.3	3. APB CSR Address Space Allocation

12-bit paddr provides a 4 KB region. Confirm the SoC’s APB address-map allocation has at least 4 KB available, and that there’s no conflict with adjacent peripherals.

### 30.1.4	4. Per-Bank Refresh Book-Keeping Cost

Keeping last_ref_age per bank costs one counter per bank. For 8 banks this is 8 small counters. Confirm the synthesis area impact is acceptable; we expect ~200 LUTs for the per-bank refresh tracking on a typical FPGA.

### 30.1.5	5. HAPPY Predictor Hash Function

Ghasempour 2015 suggests bank-XOR-low-row-bits as the hash function. Confirm with our typical workloads. If the workloads have systematic bank-row correlations, a different hash may be needed.

### 30.1.6	6. Self-Refresh Exit Latency

JESD79-2 requires tXSNR (~200 ns) before any command after SR exit. Confirm the power-state FSM enforces this correctly and that the scheduler is gated for the full duration.

### 30.1.7	7. Init Engine Retry on Error

If ZQ fails, do we retry up to 3 times then halt, or just halt? Current plan: 3 retries (parameter INIT_ZQ_RETRIES) then raise init_error. Confirm with the system architect — some systems prefer fail-fast.

### 30.1.8	8. CSR Write Atomicity

Multi-register parameter changes (e.g., updating both MR0 and a related timing) need a “config quiet period” to take effect atomically. Document the protocol the SoC software should follow; potentially expose a config_apply strobe.

### 30.1.9	9. Burst Splitting at Row Boundaries

AXI4 allows up to 256 beats per burst; a DRAM row contains 2^COL_WIDTH columns. Burst splitting happens in axi4_slave. Confirm the split logic correctly tracks partial-burst completion for ID-based ordering.

### 30.1.10	10. Multi-Rank

Explicitly out of scope for v1. Note for v2: add a RANK_WIDTH parameter, rank-aware bank machines, and rank-coordinated refresh.

### 30.1.11	11. ECC

Out of scope for this controller. ECC is expected to be handled at the SoC level or in a sideband wrapper, not the controller itself.

### 30.1.12	12. DFI Training Sub-Interface

Not driven in v1; the assumption is that the PHY handles its own training or training is not required for this DDR2 / LPDDR2 target. Confirm with the PHY vendor before silicon tape-out.

### 30.1.13	13. AXI Quality of Service (QoS)

Currently the awqos / arqos fields are forwarded to the scheduler as priority hints, but the scheduler does not implement explicit QoS classes. Decide whether to add proper QoS support (multi-class scheduler) or leave QoS as a future enhancement.

### 30.1.14	14. Multi-Master AXI

Currently single AXI port. Decide whether to add a multi-port AXI crossbar internally (simplifies SoC integration but adds area) or leave as a single-port with the SoC responsible for AXI arbitration upstream.

### 30.1.15	15. Observation Counter Overflow

Observation counters (row-hit, queue depth max, etc.) are 32-bit. At high traffic rates they could wrap. Decide on overflow handling: saturate, clear-on-read, or rely on SoC reading frequently.

## 30.2	TODOs Before v0.2

- Add bit-level pinout tables for the AXI, DFI, and APB interfaces

- Add timing diagrams for the WR / RD command issue, including CWL / CL alignment

- Add FSM state-transition diagrams for bank_machine, power_state, and init_engine

- Add a draft of the Verilog package skeletons for ddr2_init_steps_pkg.sv and lpddr2_init_steps_pkg.sv

- Cross-reference each section to the corresponding pre-aspec.md bullet

- Add quantitative area / power estimates per module (synthesis pass needed)

- Add waveform examples for the canonical init sequences