RTL Design Sherpa

DDR2/LPDDR2 Characterization Nexys A7 Project Guide 0.90

July 2026

Table of Contents

1 Document Information	7

1.1 References	7

1.2 Terminology	8

1.3 Revision History	8

2 Overview	8

2.1 What it characterizes	8

2.2 The three flows	9

2.3 Board and tools	9

2.4 Workload families	10

3 How It Works	10

3.1 The transport spine	10

3.1.1 Figure 2.1: DDR2 Characterization Harness Spine	11

3.2 The 1→4 AXIL bridge	11

3.3 Key RTL blocks	11

3.4 The characterization engines	12

3.5 The controller, DFI, and PHY (black boxes)	12

4 Build and Run	13

4.1 flows-ours-uart make targets	13

4.2 The host programs (flows-ours-uart/host/)	14

4.3 Typical bring-up sequence	15

5 Harness CSR Configuration	15

5.1 Bridge address map	15

5.2 By-name access (RDL → regmap)	15

5.2.1 Waveform 4.1: Engine Kick and Poll	16

5.3 Control / status (offsets relative to HARNESS_CSR_BASE)	16

5.4 Timer	18

5.5 Runtime controller cfg + DFI tuning	18

5.6 a7ddrphy calibration passthrough (hardware only)	19

5.7 Engine config (WR 0x100–0x128, RD 0x180–0x1A8)	19

5.8 Perf observability (0x1C0–0x1E8)	20

6 Simulation / Silicon Equivalence	21

6.1 Testbenches	21

6.2 How the same program runs in sim	21

6.3 Match the board’s exact DFI config	22

6.4 What the sim can and cannot reproduce	22

7 Troubleshooting	22

7.1 Current board state	22

7.2 Sticky errors look like a wedge	22

7.3 Known bugs / bounds	23

7.4 Cannot find the board	23

7.5 a7ddrphy is not simulatable	23

7.6 debug_sram trace ring is disabled on this build	24

7.7 LiteDRAM baseline flow	24

List of Figures

Figure 2.1: DDR2 Characterization Harness Spine	11

List of Tables

Table 1: Reference documents	7

Table 2: Revision history	8

Table 3: The characterization flows	9

Table 4: Board and tool requirements	9

Table 5: Access-pattern families	10

Table 6: Key RTL blocks	11

Table 7: flows-ours-uart make targets	13

Table 8: Bridge slave address map	15

Table 9: Control / status registers	16

Table 10: Characterization timer registers	18

Table 11: Runtime controller config and live DFI tuning	18

Table 12: a7ddrphy calibration passthrough	19

Table 13: Write-engine config (read engine identical at +0x80)	19

Table 14: Perf observability (bin b covers [2^b, 2^(b+1)) cycles)	20

Table 15: Simulation testbenches	21

Table 16: Known issues and bounds	23

List of Waveforms

Waveform 4.1: Engine Kick and Poll	16

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 1	Document Information

This is the operator / developer guide for the Nexys A7 DDR2/LPDDR2 Characterization project (projects/NexysA7/ddr2-characterization). It explains what the project characterizes, how the harness works, how to build / simulate / program / run it, and how the harness CSR is configured.

The memory controller under test — pumice — is separate IP (projects/components/memory-controllers/pumice-ddr2-lpddr2). This guide treats it as a black box driven by the characterization harness; it does not document the controller’s internals.

Note: the project’s top-level README.md still carries an early “skeleton — RTL not yet written” status line. That is stale: the harness, host, sim, and both flows are built and running. Trust this guide and the RTL/host sources over that status.

## 1.1	References

Reference documents

| Source | Title |
| --- | --- |
| RTL Design Sherpa | flows-ours-uart/host/ADDRESS_MAP.md (bridge + CSR map) |
| RTL Design Sherpa | flows-ours-uart/bin/README_a7ddrphy.md (PHY regen) |
| RTL Design Sherpa | flows-litedram-uart/HARNESS_PLAN.md (baseline flow) |
| RTL Design Sherpa | bin/TBClasses/harness/ (shared UART-char collateral) |
| Micron | MT47H64M16HR-25E DDR2 SDRAM datasheet |
| ARM | AMBA AXI4 / AXI4-Lite / APB Protocol Specifications |

## 1.2	Terminology

DFI

DDR PHY Interface. The command/data protocol between the memory controller and the PHY. Here pumice emits a phase-packed flat DFI v2.1 bus.

a7ddrphy

LiteDRAM’s Artix-7 DDR2 PHY (4:1 serdes, 800 Mbps). Uses Xilinx OSERDESE2/ISERDESE2/IDELAYE2 and therefore does not simulate in Verilator.

GEAR / DFI_RATE

The DFI phase count. GEAR_RATIO = log2(DFI_RATE). Must match between the RTL build and the sim model or reads corrupt.

Engine

A master-side AXI4 traffic generator: the write engine emits an LFSR data pattern; the read engine re-reads and CRC-checks it.

Leveling

Firmware-driven a7ddrphy read/write training over the PHY-CSR passthrough window; there is no hardware leveling FSM.

## 1.3	Revision History

Revision history

| Version | Date | Notes |
| --- | --- | --- |
| 0.90 | 2026-07-18 | Initial project guide |

# 2	Overview

## 2.1	What it characterizes

This project performs end-to-end, board-level performance, latency, and data-integrity characterization of a DDR2/LPDDR2 memory controller on real silicon. It drives the onboard Micron MT47H64M16HR-25E DDR2 (x16, single-rank, 128 MiB, up to 800 Mbps) through the controller against the real device, and reports throughput (MB/s), latency histograms, and per-workload CRC integrity across burst-length / stride / gap / page-policy / scheme sweeps.

There is no soft CPU in the FPGA — an off-board Python host drives the harness over the FTDI USB-UART (115200 8N1). The same host programs run against a cocotb simulation (Chapter 5).

## 2.2	The three flows

The project is organized as sibling “flows” plus a shared framework:

The characterization flows

| Flow | Controller (DUT) | What it is |
| --- | --- | --- |
| flows-ours-uart/ | pumice DDR2 (black box) + DFI + a7ddrphy | Primary flow: UART-driven host + Vivado build of the pumice harness on the Nexys A7. |
| flows-litedram-uart/ | LiteDRAM litedram_core | Apples-to-apples baseline: LiteDRAM’s own DDR2 controller (self-initializing, own PLL/PHY) driven by the same engines/taps/host. |
| ddr2_char_framework/ | — | Shared instrumentation: harness RTL, the engine wrapper (ddr2_char_macro), the 1→4 AXIL bridge, and the cocotb DV. |

The naming convention is flows-<controller>-uart/ for on-Nexys builds (host over UART, no CPU). pumice is separate IP under projects/components/memory-controllers/pumice-ddr2-lpddr2/.

## 2.3	Board and tools

Board and tool requirements

| Item | Detail |
| --- | --- |
| Board | Digilent Nexys A7-100T, Artix-7 xc7a100tcsg324-1 |
| DRAM | Onboard Micron MT47H64M16 DDR2 x16, ROW_WIDTH = 13 |
| Build | Vivado (batch mode) |
| Host | Python 3 + pyserial |
| Sim | Verilator + cocotb / cocotb_test / pytest |
| PHY regen (only) | Python 3.10 + migen / litex 2024.12 / litedram 2024.12 |

## 2.4	Workload families

The characterization sweeps four access-pattern families (host/pumice_char.py):

Access-pattern families

| Family | Access | Purpose |
| --- | --- | --- |
| incremental | contiguous linear | best-case sequential |
| row_major | bounded to one page | guaranteed page HIT |
| col_major | stride = one row, same bank | guaranteed page MISS (worst case) |
| col_major_interleaved | stride = one bank | activates the bank pipeline |

Controller-config presets (baseline, bank_interleave, open_page, inorder, reorder, happy_hybrid, fast_refresh, slow_refresh) each flip one lever from baseline; they are driven over the pumice APB slave (black box). Run profiles smoke / matrix / full are shared by sim and board.

# 3	How It Works

## 3.1	The transport spine

Host commands reach the DUT over one UART link that fans out to four AXI4-Lite slaves:

### 3.1.1	Figure 2.1: DDR2 Characterization Harness Spine

DDR2 characterization harness spine

Source: 01_harness_spine.mmd

The UART wire protocol is ASCII, line-based, 115200 8N1: a write is W <hex-addr32> <hex-data32>\n → the FPGA replies OK\n; a read is R <hex-addr32>\n → replies 0x<hex-data32>\n. The bridge RTL is projects/components/converters/rtl/uart_to_axil4/uart_axil_bridge.sv; the host framing is UARTAxiBridge (projects/components/converters/bin/uart_axi_bridge.py).

## 3.2	The 1→4 AXIL bridge

bridge_ddr2_char_axil is generated from ddr2_char_framework/rtl/bridges/configs/bridge_ddr2_char_axil.toml (one master host, four slaves). It decodes the four windows in Chapter 4 and routes each host transaction to the right slave via per-slave adapters.

## 3.3	Key RTL blocks

Key RTL blocks

| File | Role |
| --- | --- |
| flows-ours-uart/rtl/ddr2_char_top.sv | FPGA pin-level top: wraps the harness, the flat-DFI→per-phase adapter, and the a7ddrphy black box; MMCM clock synthesis (sys / sys2x / sys4x / sys4x_dqs), IDELAYCTRL, DDR2 pads. |
| flows-ours-uart/rtl/ddr2_char_harness.sv | Internal integration: uart_axil_bridge + bridge_ddr2_char_axil + harness_csr + debug_sram + dfi_mon_ram + ddr2_char_macro + char timer + LED / 7-seg. |
| ddr2_char_framework/rtl/ddr2_char_macro.sv | Binds the two AXI4 engines to pumice’s s_axi, and holds the perf taps (bus meters + latency histogram) on the internal AXI wires. |
| ddr2_char_framework/rtl/harness_csr.sv | The AXIL CSR slave (Chapter 4). Hand-written (self-clearing pulses, latches, PHY passthrough). |
| ddr2_char_framework/rtl/dfi_v21_flat_to_a7ddrphy.sv | Combinational adapter: pumice’s phase-packed flat DFI v2.1 → a7ddrphy per-phase ports (NPHASES = 4). |
| ddr2_char_framework/rtl/a7ddrphy_stub.sv | Port-shape black box of a7ddrphy for Verilator (real body swapped in by Vivado). |

## 3.4	The characterization engines

Inside ddr2_char_macro, two master-side AXI4 engines generate and check traffic against the controller’s s_axi port:

- Write engine (axi4_master_wr_pattern_gen) — emits an LFSR data pattern across the programmed address pattern, and latches an expected CRC.

- Read engine (axi4_master_rd_crc_check) — re-reads the same pattern and computes an actual CRC, plus a per-beat mismatch count.

Alongside them, perf taps — axi_bus_meter (write and read) and axi_perf_latency_hist — observe the internal AXI wires to produce the producer/backpressure/starve/idle buckets and the latency histogram read back in Chapter 4.

## 3.5	The controller, DFI, and PHY (black boxes)

The controller (pumice) sits inside ddr2_char_macro between the AXI engines and the flat DFI output. Its configuration registers are reached over the ddr2_apb bridge slave at base 0x0. This guide documents only that the harness drives it — not its internals.

pumice’s flat DFI v2.1 bus is adapted to the a7ddrphy per-phase interface, then to the DDR2 pads. a7ddrphy does not simulate in Verilator (it uses Xilinx SERDES/IDELAY primitives); the sim connects at the DFI level with a behavioral model instead (Chapter 5). On hardware, a7ddrphy calibration is driven by firmware (the host) over the PHY-CSR passthrough window — there is no hardware leveling FSM.

# 4	Build and Run

All commands assume the Python environment is sourced (sets REPO_ROOT, SIM=verilator, PATH, PYTHONPATH):

cd /path/to/RTLDesignSherpa && source env_python
cd projects/NexysA7/ddr2-characterization/flows-ours-uart

## 4.1	flows-ours-uart make targets

UART ?= /dev/ttyUSB1, BAUD ?= 115200, VIVADO ?= vivado.

flows-ours-uart make targets

| Target | Action |
| --- | --- |
| make lint | Verilator lint of ddr2_char_top via the harness filelist |
| make project / make synth | create Vivado project / synth-only + reports |
| make bitstream | full synth + impl + bitgen (~10–30 min) → bitstream/ddr2_char.bit |
| make bitstream-ila | same, plus an ILA on the DFI boundary → .bit + .ltx |
| make program | flash the board |
| make utilization / make timing | print the latest reports |
| make smoke | host/run_smoke.py — link + one linear WR/RD integrity pass |
| make status | host/ddr2_char.py — one-shot BUILD_ID + status dump |
| make level | host/pumice_master.py --level-only — a7ddrphy leveling |
| make simple | host/pumice_master.py --simple — init + one write→read pass |
| make characterize | host/pumice_master.py --full — full workload sweep |
| make sweep-rddly / make train-deskew | PHY timing sweeps |
| make host-test | pytest host/test_pumice_master.py (mock UART, no board) |
| make sim / sim-smoke / sim-simple / sim-level | run the same host programs in cocotb sim |
| make clean / clean-all | remove build artifacts |

The other Makefiles: flows-litedram-uart/Makefile (make regen / make bitstream / make program for the LiteDRAM baseline) and ddr2_char_framework/dv/tests/Makefile (the cocotb macro suite: make run, run-smoke, run-<shape>, and parallel run-{gate,func,full}-parallel).

## 4.2	The host programs (flows-ours-uart/host/)

The port defaults to --port auto, which probes every /dev/ttyUSB* and keeps the board that answers BUILD_ID == 0x44445232 (“DDR2”).

run_smoke.py — the first thing to run after flashing. Programs a linear WR+RD workload, kicks both engines, and checks CRC / cycles / perf.

python3 host/run_smoke.py --port /dev/ttyUSB1 --txn 1024 --blen 8 --seed 0xDEADBEEF

Pass = BUILD_ID matches, both engines done, CRC_ACTUAL == CRC_EXPECTED, BEATS_MISMATCHED == 0, and TIMER.pass.

pumice_master.py — the orchestration program; one mode is required:

python3 host/pumice_master.py --port /dev/ttyUSB1 --level-only   # PHY read/write leveling
python3 host/pumice_master.py --port /dev/ttyUSB1 --simple       # init + one write→read pass
python3 host/pumice_master.py --port /dev/ttyUSB1 --full         # full workload sweep
python3 host/pumice_master.py --port /dev/ttyUSB1 --char --char-profile matrix --char-scale 1000 --csv out.csv

Useful options: --no-level, --rd-phase, --rd-delay, --char-configs, --char-scale N (≈1000 on FPGA), --level-cache JSON, --clk-mhz.

ddr2_char.py — the driver library plus a one-shot status CLI (python3 host/ddr2_char.py --port auto). It exports DDR2CharDriver, the base-address constants, and the enum constants used throughout.

## 4.3	Typical bring-up sequence

make bitstream
make program
make smoke   UART=/dev/ttyUSB1      # link + integrity
make level                          # find the read/write eye
make simple                         # one clean write→read
make characterize                   # full sweep -> CSV
# no board? prove the same programs in sim:
make sim

# 5	Harness CSR Configuration

## 5.1	Bridge address map

The UART bridge fans out to four AXI4-Lite slaves. Bases come from bridge_ddr2_char_axil.toml (mirrored in host/ADDRESS_MAP.md and as constants in ddr2_char.py):

Bridge slave address map

| Constant | Slave | Base | Size | Protocol |
| --- | --- | --- | --- | --- |
| DDR2_APB_BASE | ddr2_apb | 0x0000_0000 | 4 KB | APB — pumice controller CSR (black box) |
| HARNESS_CSR_BASE | harness_csr | 0x0001_0000 | 4 KB | AXIL — this chapter |
| DEBUG_SRAM_BASE | debug_sram | 0x0004_0000 | 256 KB | AXIL 64b — MonBus/DFI trace ring |
| DFI_MON_RAM_BASE | dfi_mon_ram | 0x0008_0000 | 4 KB | AXIL — DFI cmd observability |

## 5.2	By-name access (RDL → regmap)

harness_csr.sv is hand-written; harness_csr.rdl is a descriptor of it, from which the by-name host regmap is generated. Registers are accessed by name via UartRegisterMap, never by hardcoded offset. Regenerate after editing either:

python3 bin/peakrdl_generate.py \
  projects/NexysA7/ddr2-characterization/ddr2_char_framework/rtl/harness_csr.rdl \
  --regmap --docs-only --no-html --no-markdown \
  --regmap-output projects/NexysA7/ddr2-characterization/ddr2_char_framework/dv/tbclasses/harness_csr_regmap.py

A consistency test (host/test_harness_regmap_consistency.py) asserts the generated regmap matches the SV header table.

### 5.2.1	Waveform 4.1: Engine Kick and Poll

Engine kick and poll

Source: 01_engine_kick.json

## 5.3	Control / status (offsets relative to HARNESS_CSR_BASE)

Control / status registers

| Register | Offset | Field | Bits | Access | Description |
| --- | --- | --- | --- | --- | --- |
| CTRL | 0x00 | start_wr | [0] | W | Pulse: kick the write engine |
| CTRL | 0x00 | start_rd | [1] | W | Pulse: kick the read engine |
| CTRL | 0x00 | clear_stats | [2] | W | Pulse: clear perf + sticky errors |
| CTRL | 0x00 | freeze_trace | [3] | RW | Latch: freeze perf counters |
| CTRL | 0x00 | soft_reset | [4] | W | Pulse: soft reset (does not clear sticky errors) |
| STATUS | 0x04 | wr_done / rd_done | [0] / [1] | R | Engine completion |
| STATUS | 0x04 | wr_error / rd_error | [2] / [3] | R | Per-engine error |
| STATUS | 0x04 | any_error | [4] | R | Sticky OR of errors — cleared by clear_stats |
| STATUS | 0x04 | init_done / init_fail | [6] / [7] | R | Controller init status |
| DBG_WR_PTR | 0x08 | value | [31:0] | R | Words written to debug_sram since clear |
| DBG_OVERFLOW | 0x0C | overflow | [0] | R | Sticky trace-overflow |
| CRC_EXPECTED | 0x10 | value | [31:0] | R | Write-engine expected CRC |
| CRC_ACTUAL | 0x14 | value | [31:0] | R | Read-engine actual CRC |
| CRC_MATCH | 0x18 | match | [0] | R | CRC_ACTUAL == CRC_EXPECTED |
| CRC_MATCH | 0x18 | exp_valid / act_valid | [1] / [2] | R | CRC captured |
| CRC_MATCH | 0x18 | beats_mism_nz | [3] | R | Any beat mismatched |
| SCRATCH | 0x1C | value | [31:0] | RW | Host link ping |
| BUILD_ID | 0x20 | value | [31:0] | R | 0x44445232 (“DDR2”) |
| BEATS_MISM | 0x24 | value | [31:0] | R | Beats mismatched |

## 5.4	Timer

Characterization timer registers

| Register | Offset | Bits | Access | Description |
| --- | --- | --- | --- | --- |
| TIMER_CTRL | 0x28 | [0] clear | W | Pulse: clear the cycle counter |
| TIMER_STATUS | 0x2C | [0] done, [1] running, [2] pass | R | Timer state |
| TIMER_CYCLES_LO/HI | 0x30 / 0x34 | [31:0] | R | 64-bit cycle count (10 ns/cycle) |
| TIMER_EXP_BEATS | 0x38 | [31:0] | RW | Beat-count stop trigger (0 = disable) |
| TIMER_{R,W}_{FIRST,LAST}_{LO,HI} | 0x40–0x5C | [31:0] | R | First/last R/W beat timestamps (64-bit each) |

## 5.5	Runtime controller cfg + DFI tuning

Runtime controller config and live DFI tuning

| Register | Offset | Field | Bits | Access | Description |
| --- | --- | --- | --- | --- | --- |
| CTRLR_CFG | 0x60 | memtype | [0] | RW | 0 = DDR2, 1 = LPDDR2 |
| CTRLR_CFG | 0x60 | t_phy_wrlat | [15:8] | RW | PHY write latency (see Chapter 6 — board wants 0) |
| CTRLR_CFG | 0x60 | t_rddata_en | [23:16] | RW | Read-data-enable latency |
| CTRLR_CFG | 0x60 | rd_in_order | [24] | RW | Force in-order reads |
| CTRLR_CAP | 0x64 | cap_lookahead_max | [3:0] | RW | Advertised OOO look-ahead depth |
| CTRLR_CAP | 0x64 | cap_synth_mask | [7:4] | RW | Advertised feature mask |
| DFI_TUNING | 0x68 | cmd_delay | [3:0] | RW | Live DFI cmd delay (reset 1) — no rebuild |
| DFI_TUNING | 0x68 | rddata_delay | [7:4] | RW | Live DFI read-data delay (reset 0) |

## 5.6	a7ddrphy calibration passthrough (hardware only)

Indirect access to the a7ddrphy’s read/write-leveling knobs; firmware-driven, no HW FSM. Meaningless in sim (the stub ignores it). Knob map: rtl-vivado/a7ddrphy/a7ddrphy_csr_map.txt.

a7ddrphy calibration passthrough

| Register | Offset | Bits | Access | Description |
| --- | --- | --- | --- | --- |
| PHY_CSR_ADDR | 0x80 | [9:0] | RW | a7ddrphy CSR word index |
| PHY_CSR_WDATA | 0x84 | [31:0] | RW | Value to write |
| PHY_CSR_CTRL | 0x88 | [0] pulse | W | Drive one CSR-bus write |
| PHY_CSR_RDATA | 0x8C | [31:0] | R | dat_r for the current PHY_CSR_ADDR |

## 5.7	Engine config (WR 0x100–0x128, RD 0x180–0x1A8)

The read block mirrors the write block at +0x80.

Write-engine config (read engine identical at +0x80)

| Register | Offset | Field | Bits | Description |
| --- | --- | --- | --- | --- |
| WR_START_ADDR | 0x100 | value | [31:0] | Start address |
| WR_STRIDE_0/1 | 0x104 / 0x108 | value | [23:0] | Signed strides |
| WR_WRAP_MASK_0/1 | 0x10C / 0x110 | value | [31:0] | Address wrap masks |
| WR_BLEN_TXN | 0x114 | burst_len | [7:0] | AXI burst length |
| WR_BLEN_TXN | 0x114 | txn_count | [23:8] | Transaction count |
| WR_BLEN_TXN | 0x114 | gap | [27:24] | Inter-burst gap |
| WR_AXI_ATTR | 0x118 | axi_id | [7:0] | AXI ID |
| WR_AXI_ATTR | 0x118 | id_mode | [9:8] | 0 fixed / 1 counter / 2 LFSR |
| WR_AXI_ATTR | 0x118 | axi_size | [12:10] | AXI size (AXI_SIZE_8 = 3) |
| WR_AXI_ATTR | 0x118 | axi_burst | [14:13] | 0 fixed / 1 incr / 2 wrap |
| WR_AXI_ATTR | 0x118 | data_mode | [15] | Data pattern mode |
| WR_LFSR_SEED | 0x11C | value | [31:0] | Pattern seed (must match RD for CRC) |
| WR_HASH_SEED0/1/2 | 0x120–0x128 | value | [31:0] | Hash seeds |

## 5.8	Perf observability (0x1C0–0x1E8)

All 32-bit; cleared by CTRL.clear_stats, frozen by CTRL.freeze_trace.

Perf observability (bin b covers [2^b, 2^(b+1)) cycles)

| Register | Offset | Field | Bits | Description |
| --- | --- | --- | --- | --- |
| OBS_RD_{PROD,BP,STARV,IDLE} | 0x1C0–0x1CC | value | [31:0] | Read data-channel meter buckets |
| OBS_WR_{PROD,BP,STARV,IDLE} | 0x1D0–0x1DC | value | [31:0] | Write meter buckets |
| OBS_HIST_SEL | 0x1E0 | bus | [0] | 0 = read, 1 = write |
| OBS_HIST_SEL | 0x1E0 | metric | [1] | 0 = AR→firstR / AW→B, 1 = AR→RLAST |
| OBS_HIST_SEL | 0x1E0 | bin | [5:2] | Histogram bin index (0–15) |
| OBS_HIST_COUNT | 0x1E4 | value | [31:0] | Count in the selected bin |
| OBS_HIST_TOTAL | 0x1E8 | value | [31:0] | Total txns on the selected metric |

# 6	Simulation / Silicon Equivalence

This project follows the shared UART-characterization methodology (bin/TBClasses/harness/): one authored-once host program runs byte-for-byte identically on the FPGA and in a cocotb simulation. The equivalence boundary is the ASCII W/R byte stream on the UART wire; only the bottom ByteChannel differs — SerialChannel (pyserial) on the board vs CocotbUartChannel on the DUT pins in sim.

## 6.1	Testbenches

Simulation testbenches

| tb_top | Wraps | Drives cfg via |
| --- | --- | --- |
| ddr2_char_uart_tb_top.sv | the full ddr2_char_harness (real UART bridge + harness_csr + 1→N bridge + engines + pumice) | the real UART pins (cocotb UARTMaster/Monitor) |
| ddr2_char_macro_tb_top.sv | the inner ddr2_char_macro | direct ports + an APB BFM |

The UART tb is the equivalence vehicle: it exposes real UART pins so the sim consumes the identical byte stream the host sends to silicon. The DFI side is wired to internal phy_dfi_* nets so the framework’s DFISlavePHY + MemoryModel loopback auto-binds by prefix. There is no a7ddrphy in sim — it reproduces the digital DFI handshake, not the analog eye.

## 6.2	How the same program runs in sim

test_ddr2_char_uart.py builds the DFISlavePHY + MemoryModel backend, then:

r = await cocotb.external(prog)()   # prog is an UNMODIFIED pumice_master routine

make_uart_channel(dut, clock, clks_per_bit) returns a CocotbUartChannel that drives the UART pins; the synchronous host program runs in a worker thread under cocotb.external, and the channel’s read/write are cocotb.function wrappers that step the sim. The baud is lowered for sim (UART_BAUD = 6_250_000 → CLKS_PER_BIT = 16 vs 868 on board) — bit-timing only, the byte stream is unchanged. make sim / sim-smoke / sim-simple / sim-level run the same programs the board runs.

## 6.3	Match the board’s exact DFI config

GEAR_RATIO = log2(DFI_RATE) and must match the compile-time DFI_RATE or reads corrupt. The test threads TEST_DFI_RATE / TEST_GEAR_RATIO / TEST_DRAM_BEAT_BYTES into both the SV parameters={...} and the host via drv.set_dfi_phase(...). The board is built at DFI_RATE = 4 (GEAR = 2, AXI 64 / DRAM beat 32); a rate-2 / GEAR-1 sim will pass while the board fails — it skips the very paths that break. Named variants (..._smoke_rate4, ..._smoke_rate4_x16, ..._pagehit_rate4_x16, …) exist to run the board’s real geometry.

## 6.4	What the sim can and cannot reproduce

- CAN: everything digital — the controller/engines, the bridge RTL, DFI-level handshakes, register logic. This is where it earns its keep.

- CANNOT: analog PHY behavior. Xilinx SERDES/IDELAY in a7ddrphy do not simulate, so read-eye / DQS-gate / pin-LOC problems are silicon-only.

# 7	Troubleshooting

## 7.1	Current board state

Sim timing is closed; on-board DDR2 runs but is gated by PHY calibration. A clean read-leveling eye is found only at t_phy_wrlat = 0 (bitslip 0, taps 0–11, centre tap 5). The default of t_phy_wrlat = 4 used by --simple / --char breaks the write path (“no passing tap”).

Known-good board config: t_phy_wrlat = 0, t_rddata_en = 6, rd_phase = 0, rddata_delay = 8, cmd_delay = 1.

After the runtime page-policy fix, OPEN vs CLOSE page policy moved read throughput from ~12.7 MB/s to ~112 MB/s (8.8×) and write to ~44 MB/s (3.5×). For reference, LiteDRAM on the same board reaches ~600 MB/s peak (300 MT/s memtest).

## 7.2	Sticky errors look like a wedge

STATUS.any_error is sticky and is cleared by CTRL.clear_stats, not soft_reset. A single failed read latches rd_error; every subsequent engine wait then returns False and looks like a hardware wedge that isn’t one. Clear stats between phases. Note pumice_master.wait_engine() waits on one engine (write-then-read is phased); a “wait for both” would hang.

## 7.3	Known bugs / bounds

Known issues and bounds

| Symptom | Cause / workaround |
| --- | --- |
| Read engine wedges at ~4790 txns | rd_cmd_cam ceiling — keep board runs txn ≤ 1024 for clean completion; the perf rate is still valid at scale |
| row_major wedges immediately | every burst is a row-miss — needs the wedge fix before it can be characterized |
| Intermittent 1–11 beat mismatches at scale | clean at txn ≤ 1024 |
| col_major stride overflow | stride 16 KB × txn > 8192 exceeds 128 MB → address alias; host bounds col_major to txn ≤ 8192 |
| synth_mask_obs / lookahead_max_obs read 0 | init never set CTRLR_CAP — advertise capability first |

## 7.4	Cannot find the board

The USB-UART re-enumerates across reboots/replugs. --port auto (default) probes each /dev/ttyUSB*, reads BUILD_ID, and picks the one answering 0x44445232. If none respond it raises “no pumice DDR2 char harness responded … is the board powered and programmed?” — confirm the bitstream is flashed and the FTDI cable is free.

## 7.5	a7ddrphy is not simulatable

Verilator cannot model OSERDESE2 / ISERDESE2 / IDELAYE2, so sim connects at the DFI level with DFISlavePHY + MemoryModel. Read-eye / DQS-gate / pin-LOC problems are silicon-only. Regenerating the real PHY requires Python 3.10 only (migen’s CSR-name tracer breaks on 3.11/3.12):

uv venv --python 3.10 /tmp/litex-venv310 && source /tmp/litex-venv310/bin/activate
uv pip install migen "litex==2024.12" "litedram==2024.12" pyyaml
python3 bin/elaborate_a7ddrphy.py --out rtl-vivado/a7ddrphy/a7ddrphy_generated.v
python3 bin/elaborate_a7ddrphy.py --dump-csr-map      # CSR offsets for firmware

## 7.6	debug_sram trace ring is disabled on this build

DEBUG_SRAM_WORDS was shrunk 32768 → 512 — the full ring needed ~44 K distributed-RAM cells (2.4× over the 100T budget) and blocked place_design. The 256-KB address window is unchanged (accesses > 2 KB alias into the ring); the ring is effectively unused here. Raise it on a larger device.

## 7.7	LiteDRAM baseline flow

flows-litedram-uart is wired and lint-clean but board bring-up is gated. The default ./regen.sh builds an empty BIOS ROM (--no-compile-software) so the core never asserts init_done — use ./regen.sh --bios (needs riscv-gcc). The host variant (harness_csr at base 0, no pumice APB cfg) is still TODO.