RTL Design Sherpa

amber — Blocking MESI Snoopy L1 Cache Micro-Architecture Specification 1.0

October 6, 2026

Table of Contents

1 Amber Mas Index	17

2 amber — Blocking MESI Snoopy L1 Cache — Hardware Architecture Specification	18

2.1 Document Organization	18

2.1.1 Front Matter	18

2.1.2 Chapter 1: Introduction	18

2.1.3 Chapter 2: System Overview	18

2.1.4 Chapter 3: Architecture	19

2.1.5 Chapter 4: Interfaces	19

2.1.6 Chapter 5: Parameters	19

2.1.7 Chapter 6: Integration	19

2.2 Figures	19

2.3 Related Documentation	20

3 Document Information	22

3.1 References	22

3.1.1 Related Documents	22

3.2 Terminology	22

3.3 Revision History	24

4 Architecture Overview	25

4.1 Introduction	25

4.1.1 What this book covers that the HAS does not	25

4.2 High-Level Block Diagram	26

4.3 Key Architectural Concepts	26

4.3.1 1. Blocking Pipeline	26

4.3.2 2. Dual-Port Arrays	27

4.3.3 3. Snoop Responder and Ordering	27

4.3.4 4. No Register Block	27

4.4 Design Decisions	27

4.5 Related Documentation	28

5 Top-Level Port List	29

5.1 amber — Pair-Rig Top	29

5.1.1 Clock and Reset	29

5.1.2 CPU GAXI Slave	29

5.1.3 AXI4 Memory Masters	29

5.1.4 ACE Snoop Responder	31

5.1.5 MonBus	32

5.2 amber_ace — Onyx-Rig Top	32

5.3 Parameters	33

6 Clocks and Reset	34

6.1 Clock Domains	34

6.2 Reset	34

7 amber_control: Blocking Pipeline FSM	35

7.1 Overview	35

7.2 State Encoding	35

7.2.1 FSM states inside amber_control	35

7.3 Pipeline Staging	37

7.3.1 Stage 0: Request latch	37

7.3.2 Stage 1: Tag lookup (CTRL_LOOKUP)	37

7.3.3 Stage 2: Hit or miss decision	37

7.3.4 Stage 3: Miss handling	37

7.3.5 Snoop interleaving	37

7.4 Key Control Signals	38

7.5 Miss-Path Decision K-map	39

8 Pending-Fill Bypass Register	40

8.1 Purpose	40

8.2 Register Fields	40

8.3 Match Logic	41

8.4 Timing	41

8.5 Snoop vs Fill Ordering	41

9 amber_tag_array and amber_data_array	43

9.1 Overview	43

9.2 amber_tag_array	43

9.2.1 Tag + state packing	43

9.2.2 Port assignment	44

9.2.3 Addressing	44

9.2.4 Banking	44

9.3 amber_data_array	44

9.3.1 Organization	44

9.3.2 Port assignment	44

9.3.3 Write masking	45

9.3.4 Snoop data readout	45

9.4 Reset Behavior	45

10 amber_repl: Replacement Policy Engine	46

10.1 Overview	46

10.2 Interface	46

10.3 LRU (True LRU)	47

10.3.1 Implementation	47

10.4 FIFO	47

10.5 RANDOM	48

10.6 Tree-PLRU	48

10.7 Policy Selection	48

11 amber_victim: Depth-1 Victim Buffer	50

11.1 Overview	50

11.2 Fields	50

11.3 Handshake with amber_control	50

11.4 Bypass for Snoops	51

11.5 Timing	51

12 amber_fill and amber_drain	52

12.1 amber_fill	52

12.1.1 Purpose	52

12.1.2 Interface	52

12.1.3 Burst parameters	52

12.1.4 Timing	52

12.2 amber_drain	53

12.2.1 Purpose	53

12.2.2 Interface	53

12.2.3 Burst parameters	53

12.2.4 Timing	53

12.2.5 On the onyx rig	54

13 amber_snoop_resp: Snoop Responder Adapter	55

13.1 Overview	55

13.2 External Interface	55

13.3 Internal Interface	56

13.4 Adapter State Machine	56

13.4.1 State transitions	56

13.5 CR-after-CDLAST Ordering	57

13.6 Snoop Type Decoding	57

14 amber_ace_issue: Coherent Transaction Mapping	59

14.1 Overview	59

14.2 Event-to-Transaction Mapping	59

14.3 Interface	60

14.3.1 From amber_control / fill / drain	60

14.3.2 To axi4ace_master_rd / axi4ace_master_wr	60

14.4 CleanUnique / MakeUnique Data Behavior	60

14.5 Timing	61

15 amber_cpu_frontend and amber_monlite	62

15.1 amber_cpu_frontend	62

15.1.1 Purpose	62

15.1.2 Request payload	62

15.1.3 Response payload	62

15.1.4 Handshake	62

15.1.5 Replay path	63

15.2 amber_monlite	63

15.2.1 Purpose	63

15.2.2 Observation points	63

15.2.3 Packet format	63

15.2.4 Drop-and-count	64

16 CPU GAXI Request/Response Timing	65

16.1 Channel Structure	65

16.2 Request Timing	65

16.3 Response Timing	66

16.4 Write-Allocate Fills	66

17 Fabric AXI4 Master Timing	67

17.1 Read (Fill) Timing	67

17.2 Write (Drain) Timing	67

17.3 Single Outstanding	68

18 Fabric ACE Master Timing	69

18.1 Read Transactions	69

18.2 Write Transactions	69

18.3 Timing Example: ReadShared Fill	69

18.4 Timing Example: WriteBack Drain	70

19 Snoop AC/CD/CR Timing and Ordering	71

19.1 External ACE Snoop Port	71

19.2 Transaction Sequence	71

19.3 CR-After-CDLAST Rule	72

19.4 Multiple Outstanding Snoops	72

19.5 AC Ready Backpressure	72

20 MonBus Event Emission Points	73

20.1 Packet Format	73

20.2 Emission Timing	73

20.3 Drop-and-Count	73

20.4 Backpressure Integrity	74

21 MonBus Event Encodings and Emission Points	75

21.1 Overview	75

21.2 Event Encoding	75

21.3 Payload Fields	76

21.4 Miss-Class Encoding	76

22 Drop-and-Count Behavior and monbus_tally_axil	78

22.1 Drop-and-Count Policy	78

22.2 Counter Width	78

22.3 monbus_tally_axil Integration	78

22.4 Software View	78

23 What the K-maps Prove	79

23.1 Methodology	79

23.2 The Three Verdicts	79

23.3 First-Verdict Status (2026-10-06)	80

24 RTL Landing Diff Procedure	81

24.1 When RTL Lands	81

24.2 Diff Checklist	81

24.3 Regression Rule	81

25 gem5 FSM Cross-Check	83

25.1 Source	83

25.2 Cross-Check Mechanism	83

25.3 Scope	83

26 Elaboration Configurations	84

26.1 Default Config	84

26.2 Tiny Formal Config	84

26.3 Bring-Up Config	84

27 DV Matrix Dimensions	86

27.1 Pattern-B Cocotb Grids	86

27.1.1 Dimensions	86

27.1.2 Grid Coverage	86

27.2 Formal Targets	86

27.3 Golden-Model Parity	87

27.4 MonBus Coverage	87

28 amber — Product Requirements (DRAFT)	88

28.1 1. Purpose	88

28.2 2. Background	88

28.3 3. Decisions that pick the design	89

28.4 4. Success criteria	98

28.5 5. Non-goals	98

28.6 6. Where the block sits	98

29 amber — Pre-Hardware Architecture Specification (Pre-HAS)	99

29.1 1. Intent of this document	99

29.2 2. Feature sketches	99

29.2.1 2.1 Core datapath — the blocking bet	99

29.2.2 2.2 Coherence	100

29.2.3 2.3 Fabric side — where the two deployment rigs meet	101

29.2.4 2.4 Observation (D8 DECIDED — sketched, not re-decided)	102

29.2.5 2.5 Verification & formal surface (D9 OPEN — direction proposed)	102

29.2.6 2.6 Cost & characterisation (PRD criterion 4)	102

29.3 3. Proposed block architecture	103

29.4 4. Interface sketches (shapes, not pinouts)	104

29.5 5. Parameter sketch (illustrative — feeds the DV matrix)	105

29.6 6. Open questions this sketch surfaces	106

29.7 7. What the full HAS will add	107

29.8 8. Cross-references	107

30 jet — Product Requirements (DRAFT)	109

30.1 1. Purpose	109

30.2 2. Decisions jet adds to amber’s	109

30.3 3. Success criteria (additions to amber’s)	111

30.4 4. Non-goals (for jet, beyond amber’s)	111

31 onyx — Product Requirements (DRAFT)	112

31.1 1. Purpose	112

31.2 2. Background	112

31.3 3. Decisions that pick the design	113

31.4 4. Success criteria	121

31.5 5. Non-goals	121

31.6 6. Where the block sits	121

32 AMBA ACE — Working Interface Definition for cache-ip	122

32.1 1. What ACE is	122

32.2 2. Channel inventory	122

32.2.1 2.1 Coherent master → manager (front side, per master)	122

32.2.2 2.2 Manager → cache, cache → manager (snoop side, per cache)	123

32.3 3. Transaction types (the onyx D2 research subset in bold)	124

32.4 4. The CRRESP decision bits	125

32.5 5. ACE-Lite	126

32.6 6. What this family implements, and what it deliberately does not	126

32.7 7. Sources	126

33 gem5 Ruby protocol specs (reference extract)	128

34 Cache Associativity Simulator	129

34.1 Files	129

34.2 Model API	129

34.3 Running tests	130

34.4 UI features	130

35 Global Hard Requirements for RTL Design Sherpa	131

35.1 Overview	131

35.2 Category 1: RTL Standards	131

35.2.1 1.1 Reset Handling (MANDATORY - NO EXCEPTIONS)	131

35.2.2 1.2 FPGA Synthesis Attributes (MANDATORY)	132

35.2.3 1.3 Array Syntax Standard (MANDATORY)	133

35.2.4 1.4 SRAM Module Standards (MANDATORY)	133

35.2.5 1.5 Reset Convention (STANDARD)	134

35.3 Category 2: Testbench Architecture	134

35.3.1 2.1 TB Location Requirements (MANDATORY)	134

35.3.2 2.2 Three Mandatory TB Methods (MANDATORY)	135

35.3.3 2.3 TBBase Inheritance (MANDATORY)	135

35.3.4 2.4 Three-Layer Architecture (MANDATORY)	136

35.3.5 2.5 Queue-Based Verification (PREFERRED)	137

35.4 Category 3: Test Standards	137

35.4.1 3.1 Test Naming Convention (MANDATORY)	137

35.4.2 3.2 Test Pattern - CocoTB + Pytest (MANDATORY for projects/components/)	138

35.4.3 3.3 100% Success Requirement (MANDATORY)	138

35.4.4 3.4 Simulation Timestamps in Logs (MANDATORY)	139

35.5 Category 4: Framework Usage	140

35.5.1 4.1 Search Before Creating (MANDATORY)	140

35.5.2 4.2 BFM Usage for Protocols (MANDATORY for RAPIDS)	140

35.6 Category 5: Documentation	141

35.6.1 5.1 No Emojis in Technical Docs (MANDATORY)	141

35.7 Category 6: Project-Specific Requirements	141

35.7.1 6.1 Commit Messages Carry No Attribution Trailer	141

35.7.2 6.2 STREAM Tutorial Focus	141

35.8 Summary by Priority	142

35.8.1 P0 - Critical (Enforcement Required)	142

35.8.2 P1 - High Priority (Strong Recommendation)	142

35.8.3 P2 - Standard Practice (Best Practices)	142

35.8.4 P3 - Project-Specific	142

35.9 Compliance Checklist	143

35.10 Enforcement	143

35.11 References	144

List of Figures

No figures in this document.

List of Tables

Table 1: Table 0.0: Figures and their sources	19

Table 2: Table 0.1: Related documentation	20

Table 3: Related Documents and Specifications	22

Table 4: amber MAS Document Revision History	24

Table 5: Table 1.3.1: Clock domains	34

Table 6: Table 1.3.2: Reset signals	34

Table 7: Table 2.8.1: Cache events to ACE transactions	59

Table 8: Table 3.1.1: CPU GAXI channels	65

Table 9: Table 4.1.1: MonBus event encodings	75

List of Waveforms

No waveforms in this document.

# 1	Amber Mas Index

Generated: 2026-10-07

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	amber — Blocking MESI Snoopy L1 Cache — Hardware Architecture Specification

Version: 1.0 Date: 2026-10-06 Status: full HAS ratifies the Pre-HAS sketch; PRD decisions D1, D2, D4, D5, D6, D7, D8, D9, D10 are DECIDED. D3 (memory-side shape) and D11 (array construction) remain OPEN and are recorded honestly in the chapters they shape.

Scope: The architecture of amber (pair-rig top) and amber_ace (onyx-rig top) from the CPU GAXI slave through the shared cache core to the fabric masters and ACE snoop responder. The MAS closes the micro-architecture decisions this book defers.

How this book relates to its sources. The amber PRD is the binding requirements document. This HAS carries the Pre-HAS feature sketches (F1–F18) forward, drops their “PROPOSED” hedging where the PRD has decided, and fixes the module set and interfaces as architecture. Where D3 and D11 are still open, the chapters state the current direction and the consequences; nothing is disguised as decided. The GLOBAL_REQUIREMENTS.md reset-macro, array-syntax, no-bespoke-SRAM, and filelist rules apply throughout.

## 2.1	Document Organization

Note: All chapters linked below for automated document generation.

### 2.1.1	Front Matter

- Document Information

### 2.1.2	Chapter 1: Introduction

- Purpose and Scope

- Document Conventions

- Definitions and Acronyms

### 2.1.3	Chapter 2: System Overview

- Use Cases

- Key Features

- System Context

- Block Diagram

### 2.1.4	Chapter 3: Architecture

- Top Structure and the Two Rigs

- Data Flow: Hits, Misses, Fills, and Drains

- Control FSM: Blocking Pipeline and Pending-Fill Bypass

- Coherence FSM and the Snoop Responder

- Arbitration and the Victim Bypass

### 2.1.5	Chapter 4: Interfaces

- CPU-Side GAXI Slave

- Fabric-Side AXI4 Masters

- Fabric-Side ACE Masters and Coherent Issue

- Snoop Port: AC/CD/CR Adapter

- Observation: MonBus through amber_monlite

### 2.1.6	Chapter 5: Parameters

- amber_pkg and Build-Time Geometry

### 2.1.7	Chapter 6: Integration

- Clocking and Reset

- Verification Strategy

- Characterization

- Future Hooks

## 2.2	Figures

Table 0.0: Figures and their sources

| Figure | Source | Subject |
| --- | --- | --- |
| 2.1 | assets/mermaid/01_block_diagram.mmd | amber shared core, the two rig tops, and the peer/onyx snoop boundary |
| 3.1 | assets/mermaid/02_miss_flow.mmd | miss-to-fill-to-replay data flow, with victim drain |
| 3.2 | assets/mermaid/03_snoop_flow.mmd | probe path through tag port B and the pending-fill bypass |

## 2.3	Related Documentation

Table 0.1: Related documentation

| Document | Where | Relationship |
| --- | --- | --- |
| amber PRD | ../../PRD.md | binding requirements — all D-rows and success criteria originate here |
| amber Pre-HAS | ./amber_prehas.md | architecture sketch; this HAS supersedes and ratifies it |
| amber MAS | ../amber_mas/amber_mas_index.md | micro-architecture and pre-RTL signal-contract / kmap workbook |
| jet PRD | ../../../jet-mesi-l1/PRD.md | lockup-free follow-on; amber must not silently pre-decide jet’s J-rows |
| onyx PRD | ../../../onyx-ace-ccu/PRD.md | onyx D7 defines the ACE-shaped snoop port; onyx D2 defines the coherent-transaction subset |
| AMBA ACE Interface Definition | ../../../References/AMBA_ACE_Interface_Definition.md | in-repo ACE contract, verified against IHI 0022H |
| gem5 Ruby protocols | ../../../References/gem5-ruby-protocols/README.md | golden-protocol source for the Python reference model and FSM oracles |
| cache simulator | ../../../../../../bin/apps/cache_sim/README.md | golden parity model for LRU/FIFO/RANDOM |
| Global Requirements | ../../../../../../GLOBAL_REQUIREMENTS.md | reset macros, array syntax, no bespoke SRAM, filelist registry |

Last Updated: 2026-10-06 Maintained By: amber architecture

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Document Information

This document is the Micro-Architecture Specification (MAS) for amber, a parameterised, blocking, snoopy MESI L1 data cache. It closes the implementation decisions that the amber Hardware Architecture Specification (HAS) deliberately deferred to RTL bring-up: pipeline staging, array banking, the exact control FSM state encoding, replacement-policy implementation details, and the pre-RTL combinational decision-logic contracts captured in the kmap workbook.

## 3.1	References

### 3.1.1	Related Documents

Related Documents and Specifications

| Source | Title | Version |
| --- | --- | --- |
| RTL Design Sherpa | amber Product Requirements Document | 0.5 |
| RTL Design Sherpa | amber Hardware Architecture Specification | 1.0 |
| RTL Design Sherpa | amber Pre-HAS sketch | 1.0 |
| ARM | AMBA AXI and ACE Protocol Specification | IHI0022H |
| gem5 | Ruby MESI_Two_Level protocol extract | commit f5c5a6e |

## 3.2	Terminology

ACE

AXI Coherency Extensions. The ARM protocol carrying snoop channels (AC/CD/CR) and coherent master transactions.

AC / CD / CR

ACE snoop channels: Address Coherent (AC), Coherent Data (CD), Coherent Response (CR).

CRRESP

5-bit snoop response carrying DataTransfer, PassDirty, IsShared, WasUnique, and an error bit.

DT / PD / IS / WU

CRRESP sub-fields: DataTransfer[0], PassDirty[1], IsShared[2], WasUnique[3].

FUB

Functional Unit Block. A self-contained RTL module with a defined interface and testbench.

GAXI

House generic-AXI streaming pattern (wr_valid/wr_ready/wr_data, rd_valid/rd_ready/rd_data) used for the CPU-side port.

LRU / FIFO / RANDOM / tree-PLRU

Replacement policies supported by amber_repl. LRU, FIFO, and RANDOM have cache_sim golden-model parity; tree-PLRU is the timing fallback without a sim golden model today.

MAC

Macro. An integration-level block that instantiates and connects multiple FUBs.

MESI

Cache coherence states: Modified, Exclusive, Shared, Invalid.

MonBus

128-bit packet + 64-bit side-band timestamp monitor bus. amber_monlite is a drop-and-count observer that never stalls the observed path.

Pending-fill bypass register

Single register held by amber_control while a fill is outstanding, allowing snoops to the same line to be answered with the post-fill state before the array is updated.

RACK / WACK

ACE read/write acknowledge pulses. In amber’s adapters they are auto-pulsed one cycle after the last read beat / B handshake.

SDPRAM

Simple dual-port RAM. The house sdpram_core primitive is the only allowed tag/data storage.

Snoop responder

amber_snoop_resp, the ACE-shaped adapter that presents bus-agnostic {address, type, response, data} transactions to amber_core.

## 3.3	Revision History

amber MAS Document Revision History

| Rev | Date | Author | Notes |
| --- | --- | --- | --- |
| 1.0 | 2026-10-06 | seang | Initial amber MAS: index, ch01–ch06, styles, title, and pre-RTL kmap workbook. No RTL yet; all micro-architecture proposals are contracts for the landing implementation. |

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	Architecture Overview

Component: amber — Blocking MESI Snoopy L1 Cache Version: 1.0 Status: Pre-RTL micro-architecture contract

## 4.1	Introduction

The amber Hardware Architecture Specification (HAS) fixes the module set, interface shapes, and parameter decisions. This Micro-Architecture Specification (MAS) closes what the HAS deferred: the cycle-level behavior of every module, the exact control FSM state encoding, how tag and state pack into the dual-port sdpram_core, the replacement-policy datapaths, and the handshake timing of each external interface.

Because amber has no register block, there is no CSR programming chapter. All runtime visibility is through the MonBus observation fabric. The MAS therefore replaces the traditional register-map chapter with a MonBus observation map.

### 4.1.1	What this book covers that the HAS does not

- Per-module micro-architecture — how amber_control advances its FSM, how the pending-fill bypass register is matched and timed, how amber_tag_array and amber_data_array use sdpram_core port A and port B, how amber_repl implements each policy, and how amber_snoop_resp stages CR and CD.

- Cycle-level interface behavior — GAXI request/response handshake timing, AXI4/ACE master channel timing, AC/CD/CR snoop ordering including the CR-after-CDLAST rule, and MonBus event emission points.

- The kmap workbook as decision-logic contract — every important combinational decision signal is captured as a K-map with axis equations, checked relations, and a mechanically-derived minimal SOP. The workbook is generated by gen_amber_contracts_kmaps.py. Because RTL does not yet exist, rtl_sop is intentionally absent; when RTL lands the derived cover will be diffed against the RTL expression and one of the three methodology verdicts applied: identical, RTL-redundant, or RTL-differs.

## 4.2	High-Level Block Diagram

amber_core contains all logic shared between the two rig tops:

amber_core
├── amber_cpu_frontend   (GAXI slave request/response latch)
├── amber_control        (blocking pipeline FSM)
├── amber_tag_array      (dual-port tag + MESI state)
├── amber_data_array     (dual-port data store)
├── amber_repl           (replacement policy engine)
├── amber_victim         (depth-1 dirty victim buffer)
├── amber_fill           (line-fill read interface)
├── amber_drain          (dirty victim write-back interface)
├── amber_snoop_resp     (ACE snoop adapter)
└── amber_monlite        (MonBus observer)

The pair-rig top amber wraps amber_core with plain AXI4 read/write masters (axi4_master_rd/axi4_master_wr) toward memory and the snoop responder toward the peer cache. The onyx-rig top amber_ace wraps the same core with axi4ace_master_rd/axi4ace_master_wr masters and adds amber_ace_issue to map cache-side events to ACE coherent transactions.

There is no RIG parameter. The rig is selected by instantiating the appropriate top module.

## 4.3	Key Architectural Concepts

### 4.3.1	1. Blocking Pipeline

amber_control is the only FSM in the cache. A request flows through:

- Lookup — tag/state read on port A.

- Hit service — data read or write on port A, response returned.

- Miss resolution — ask amber_repl for a victim way.

- Victim drain — if dirty, stage the victim and launch amber_drain.

- Fill request — launch amber_fill for a whole-line burst.

- Fill completion — write data and install tag + state on port A.

- Replay — re-present the original request from the front-end latch; it now hits.

Because the pipeline is blocking, no new CPU request is accepted while a miss is in flight. This single-outstanding property is what makes the pending-fill bypass tractable.

### 4.3.2	2. Dual-Port Arrays

amber_tag_array and amber_data_array are built from the house sdpram_core primitive. Port A serves CPU lookups, fill writes, and replay reads. Port B serves snoop lookups and snoop data readout. The arrays themselves need no arbitration because CPU and snoop use separate ports; contention is resolved at the control FSM level.

### 4.3.3	3. Snoop Responder and Ordering

amber_snoop_resp is the only ACE-aware block inside amber_core. It presents a bus-agnostic {address, type, response, data} interface to amber_control. The adapter completes all CD beats for a transaction before asserting CR, matching the family convention and the cocotb-framework 1.2.0 compliance checker.

### 4.3.4	4. No Register Block

amber is parameter-only. Configuration, event counting, and debug visibility all come through MonBus. This is deliberate and keeps the CPU-side GAXI port the only software-visible surface.

## 4.4	Design Decisions

| Decision | Source | MAS consequence |
| --- | --- | --- |
| GAXI CPU-side slave | PRD D2 | amber_cpu_frontend is a GAXI slave with a latched request and in-order response stream. |
| Write-back + write-allocate | PRD D5 | Dirty victims are drained; fills are whole-line read-modify-write inside amber_control. |
| Plain MESI on 3-bit state | PRD D6 | amber_pkg encodes M/E/S/I; one bit is reserved for MOESI O-state. |
| Replacement policies {LRU, tree-PLRU, FIFO, RANDOM} | PRD D7 | amber_repl implements all four; LRU/FIFO/RANDOM match cache_sim; tree-PLRU is the timing fallback. |
| ACE-shaped snoop port | PRD D4 | amber_snoop_resp drives AC-ready, CR-valid, CD-valid; CR follows CDLAST. |
| MonBus observation only | PRD D8 | amber_monlite emits event packets and drops-and-counts under backpressure. |

## 4.5	Related Documentation

- amber HAS — architecture decisions and interface contracts.

- amber PRD — binding requirements.

- K-map workbook — pre-RTL combinational contracts.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	Top-Level Port List

This chapter lists the top-level ports of amber and amber_ace. Internal amber_core ports are defined in the per-block chapters of Chapter 2.

## 5.1	amber — Pair-Rig Top

### 5.1.1	Clock and Reset

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| aclk | input | 1 | System clock |
| aresetn | input | 1 | Active-low asynchronous reset, synchronous deassert |

### 5.1.2	CPU GAXI Slave

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| cpu_req_wr_valid | input | 1 | CPU request valid |
| cpu_req_wr_ready | output | 1 | Cache ready to accept request |
| cpu_req_wr_data | input | CPU_REQ_W | Packed {addr, we, be, wdata} |
| cpu_rsp_rd_valid | output | 1 | Cache response valid |
| cpu_rsp_rd_ready | input | 1 | CPU ready to accept response |
| cpu_rsp_rd_data | output | CPU_RSP_W | Packed {rdata} |

### 5.1.3	AXI4 Memory Masters

Plain AXI4 masters driven by amber_fill and amber_drain. Signal names and widths follow the house axi4_master_rd / axi4_master_wr wrappers.

| Channel | Direction | Signals |
| --- | --- | --- |
| AR | out | m_axi_arid, m_axi_araddr, m_axi_arlen, m_axi_arsize, m_axi_arburst, m_axi_arlock, m_axi_arcache, m_axi_arprot, m_axi_arqos, m_axi_arregion, m_axi_aruser, m_axi_arvalid |
| R | in/out | m_axi_rid, m_axi_rdata, m_axi_rresp, m_axi_rlast, m_axi_ruser, m_axi_rvalid, m_axi_rready |
| AW | out | m_axi_awid, m_axi_awaddr, m_axi_awlen, m_axi_awsize, m_axi_awburst, m_axi_awlock, m_axi_awcache, m_axi_awprot, m_axi_awqos, m_axi_awregion, m_axi_awuser, m_axi_awvalid |
| W | out | m_axi_wdata, m_axi_wstrb, m_axi_wlast, m_axi_wuser, m_axi_wvalid |
| B | in/out | m_axi_bid, m_axi_bresp, m_axi_buser, m_axi_bvalid, m_axi_bready |

### 5.1.4	ACE Snoop Responder

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_acaddr | input | ADDR_WIDTH | Snoop address |
| m_axi_acsnoop | input | 4 | Snoop transaction type |
| m_axi_acprot | input | 3 | Snoop protection |
| m_axi_acvalid | input | 1 | Snoop address valid |
| m_axi_acready | output | 1 | Cache ready for snoop |
| m_axi_crresp | output | 5 | Snoop response |
| m_axi_crvalid | output | 1 | Snoop response valid |
| m_axi_crready | input | 1 | Snoop response ready |
| m_axi_cddata | output | DATA_WIDTH | Snoop data |
| m_axi_cdlast | output | 1 | Snoop data last beat |
| m_axi_cdvalid | output | 1 | Snoop data valid |
| m_axi_cdready | input | 1 | Snoop data ready |

### 5.1.5	MonBus

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| mon_valid | output | 1 | Monitor packet valid |
| mon_ready | input | 1 | Monitor consumer ready |
| mon_packet | output | 128 | Monitor packet |
| mon_timestamp | output | 64 | Side-band timestamp |

## 5.2	amber_ace — Onyx-Rig Top

amber_ace has the same CPU GAXI slave, snoop responder, and MonBus ports as amber. The memory-side masters are full ACE masters with ARSNOOP / AWSNOOP fields and auto-pulsed RACK / WACK outputs. amber_ace_issue maps cache-side events to ACE transactions; see Chapter 2.8.

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| m_axi_arsnoop | output | 4 | ACE read transaction type |
| m_axi_rack | output | 1 | ACE read acknowledge (auto-pulsed) |
| m_axi_awsnoop | output | 3 | ACE write transaction type |
| m_axi_wack | output | 1 | ACE write acknowledge (auto-pulsed) |

All other AXI/ACE master signals mirror amber but connect to axi4ace_master_rd / axi4ace_master_wr instead of the plain AXI4 wrappers.

## 5.3	Parameters

All geometry, policy, and bus-width choices are elaboration parameters. See Chapter 6 and the HAS parameters chapter for the complete list. There is no runtime register block.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	Clocks and Reset

## 6.1	Clock Domains

Table 1.3.1: Clock domains

| Clock | Frequency | Usage |
| --- | --- | --- |
| aclk | 50–200 MHz | Primary — all amber logic, AXI4/ACE/GAXI interfaces, MonBus |

amber is a single-clock IP. The GAXI slave, AXI4/ACE masters, snoop responder, and MonBus all run on aclk.

## 6.2	Reset

Table 1.3.2: Reset signals

| Signal | Polarity | Type | Usage |
| --- | --- | --- | --- |
| aresetn | Active-low | Async assert, sync deassert | Primary reset for all amber logic and memories |

aresetn resets all flops. The sdpram_core instances inside amber_tag_array and amber_data_array are not reset; their contents are invalidated by clearing valid bits or by the first access after reset, depending on the implementation choice recorded in Chapter 2.3.

All sequential logic uses the house ALWAYS_FF_RST macro with RST_ASSERTED(aresetn). This is the same pattern used in the AMBA wrappers and STREAM.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	amber_control: Blocking Pipeline FSM

Module: amber_control.sv Location: projects/components/cache-ip/amber-mesi-l1/rtl/fub/ Status: Pre-RTL micro-architecture contract

## 7.1	Overview

amber_control is the cache’s only state machine. It owns the blocking pipeline, the tag-array port-A schedule, the pending-fill bypass register, victim-way selection, and the handshakes to amber_fill, amber_drain, amber_victim, and amber_snoop_resp. Because the cache is blocking, there is exactly one CPU transaction in flight at any time; snoops are serviced concurrently on tag-array port B.

## 7.2	State Encoding

amber_pkg defines a 3-bit cache-line state field for M, E, S, I. One extra code is reserved for a future MOESI O state; the control FSM v1.0 implements only MESI.

typedef enum logic [2:0] {
    STATE_I = 3'b000,  // Invalid
    STATE_S = 3'b001,  // Shared
    STATE_E = 3'b010,  // Exclusive
    STATE_M = 3'b011,  // Modified
    STATE_O = 3'b100,  // Reserved for future MOESI Owned state
    // 3'b101, 3'b110, 3'b111 reserved
} cache_state_t;

The 3-bit width is intentional: it gives one reserved code for O and three spare codes for future transient/stable elaborations without widening the tag store.

### 7.2.1	FSM states inside amber_control

The FSM itself is one-hot encoded. Stable states are the ones that can persist between CPU requests; transient states exist only while a miss or snoop is in progress, mirroring the gem5 Ruby MESI_Two_Level-L1cache.sm philosophy that every in-flight condition is an explicit enumerated state.

| State | Type | Meaning |
| --- | --- | --- |
| CTRL_IDLE | stable | Waiting for a CPU request from amber_cpu_frontend. |
| CTRL_LOOKUP | transient | Tag/state lookup on port A; hit/miss decision. |
| CTRL_HIT_RD | transient | Read hit: fetch data from amber_data_array port A, return response. |
| CTRL_HIT_WR | transient | Write hit: update data, promote to M, return response. |
| CTRL_MISS_VICTIM | transient | Miss: ask amber_repl for victim way; read victim state. |
| CTRL_MISS_DRAIN | transient | Victim is dirty: stage it in amber_victim, launch amber_drain. |
| CTRL_MISS_FILL | transient | Launch amber_fill, wait for RLAST. |
| CTRL_FILL_WRITE | transient | Write fill data into amber_data_array port A; install tag + state. |
| CTRL_REPLAY | transient | Re-present original request from front-end latch; it now hits. |
| CTRL_SNOOP | transient | Service a snoop on port B (can preempt / interleave with miss states). |
| CTRL_ERROR | stable | Sticky fatal error; cleared only by reset. |

The FSM is not a minimal two-state controller; the explicit transient states are the formal proof surface. Each state asserts exactly the control signals that are legal for that state, and illegal states default to CTRL_ERROR.

## 7.3	Pipeline Staging

The HAS deferred pipeline staging to the MAS. This section fixes it.

### 7.3.1	Stage 0: Request latch

amber_cpu_frontend accepts a GAXI request and holds {addr, we, be, wdata} in a register. amber_control sees a single req_valid pulse and the latched request fields. The front end does not accept a new request until req_ready is returned.

### 7.3.2	Stage 1: Tag lookup (CTRL_LOOKUP)

On the cycle after request acceptance, amber_control drives tag_a_addr and tag_a_en. The tag array returns {tag, state} on the next cycle. The lookup also selects the victim way from amber_repl combinatorially so that the victim is known on the same cycle the hit/miss decision is made.

### 7.3.3	Stage 2: Hit or miss decision

- Hit: transition to CTRL_HIT_RD or CTRL_HIT_WR. The data array is accessed in the same cycle as the state check (the set/way was already decoded from the lookup).

- Miss: transition to CTRL_MISS_VICTIM.

### 7.3.4	Stage 3: Miss handling

- CTRL_MISS_VICTIM: read the victim tag/state from the tag array and, if dirty, copy the victim line data from amber_data_array port A into amber_victim.

- CTRL_MISS_DRAIN: if the victim is M, launch amber_drain with the address and data from amber_victim. Wait for the drain-done handshake.

- CTRL_MISS_FILL: launch amber_fill with the missing line address. The pending-fill bypass register is loaded now.

- CTRL_FILL_WRITE: on fill_done, write the fill data into amber_data_array port A and install the tag + new state in amber_tag_array port A.

- CTRL_REPLAY: re-assert req_valid to the hit path using the original latched request; the next cycle is CTRL_LOOKUP again and the line is present.

### 7.3.5	Snoop interleaving

amber_control resolves CPU-vs-snoop contention with snoop priority. A pending snoop can cause the CPU pipeline to stall for one cycle at safe boundaries (after a tag-array access completes, never mid-burst). The snoop path uses tag_b_* and data_b_* ports exclusively; it never needs port A except when a snoop triggers an invalidation or downgrade, which is applied on the next available port-A cycle.

## 7.4	Key Control Signals

| Signal | Direction | Meaning |
| --- | --- | --- |
| ctrl_tag_a_addr | to amber_tag_array | Port A address (set index). |
| ctrl_tag_a_en | to amber_tag_array | Port A read/write enable. |
| ctrl_tag_a_wdata | to amber_tag_array | Port A write data {tag, state}. |
| ctrl_tag_a_way | to amber_tag_array | Way select for write. |
| ctrl_data_a_addr | to amber_data_array | Port A address {set, way, beat}. |
| ctrl_data_a_en | to amber_data_array | Port A byte-enable / write enable. |
| ctrl_data_a_wdata | to amber_data_array | Port A write data. |
| ctrl_repl_req | to amber_repl | Request replacement decision. |
| ctrl_repl_way | from amber_repl | Selected victim way. |
| ctrl_victim_wr | to amber_victim | Strobe to load victim line. |
| ctrl_fill_start | to amber_fill | Launch a fill for this line address. |
| ctrl_fill_done | from amber_fill | Fill completed (RLAST accepted). |
| ctrl_drain_start | to amber_drain | Launch a drain from amber_victim. |
| ctrl_drain_done | from amber_drain | Drain completed (B handshake). |
| ctrl_snoop_req | from amber_snoop_resp | Snoop address + type valid. |
| ctrl_snoop_ready | to amber_snoop_resp | Control accepts snoop this cycle. |
| ctrl_snoop_crresp | to amber_snoop_resp | 5-bit CRRESP result. |
| ctrl_snoop_cddata | to amber_snoop_resp | CD beat data. |
| ctrl_snoop_cdlast | to amber_snoop_resp | CD last beat. |
| ctrl_req_ready | to amber_cpu_frontend | Ready for next CPU request. |
| ctrl_rsp_valid | to amber_cpu_frontend | Response valid. |
| ctrl_rsp_data | to amber_cpu_frontend | Response read data. |

## 7.5	Miss-Path Decision K-map

The combinational decision that launches drain/fill/replay is captured in the kmap workbook:

- Outputs: start_drain, start_fill, replay_now.

- Axes: {hit, victim_dirty, pending_bypass_match}.

- Sheet: K-maps amber control in amber_signal_contracts.xlsx, generated by gen_amber_contracts_kmaps.py.

The sheet is marked PROPOSED because it is a micro-architecture proposal owned by this chapter; the RTL must match the derived cover when it lands.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	Pending-Fill Bypass Register

Module: amber_control sub-block Status: Pre-RTL micro-architecture contract

## 8.1	Purpose

While a fill is outstanding, a snoop may arrive for the same line. The pending-fill bypass register lets amber_control answer that snoop with the post-fill state and any fill beats already received, before the tag and data arrays have been updated. This is the only place in amber where a line can be observed in a state that has not yet been committed to the arrays, so its correctness is a SymbiYosys target.

## 8.2	Register Fields

| Field | Width | Meaning |
| --- | --- | --- |
| pf_addr | ADDR_WIDTH - OFFSET_WIDTH | Line address (tag + set index) of the fill. |
| pf_state | 3 | State that will be installed when the fill completes. |
| pf_data_valid | FILL_BEATS | Per-beat mask: bit[i] is 1 when fill beat i has been received. |
| pf_active | 1 | Register is valid and the fill is still outstanding. |

The register is loaded in CTRL_MISS_FILL when amber_fill is launched. pf_state is determined by the request type:

- Read miss to shared → STATE_S

- Read miss to exclusive → STATE_E

- Write miss (whole-line allocate) → STATE_M

pf_data_valid is updated as each R beat is accepted by amber_fill. The snoop responder can forward fill beats whose pf_data_valid bit is set; beats not yet received stall the CD channel until they arrive.

## 8.3	Match Logic

A snoop matches the pending-fill bypass when:

pf_active &&
(snoop_line_addr == pf_addr)

The line address is {tag, set_index}; the line offset is ignored. The kmap workbook contains a small optional bypass-match sheet, but the match is a single equality compare; the sheet is included only if it earns its place as a teaching aid.

## 8.4	Timing

| Cycle | Event |
| --- | --- |
| CTRL_MISS_FILL entry | pf_active set, pf_addr and pf_state loaded, pf_data_valid cleared. |
| Each accepted R beat | Corresponding pf_data_valid bit set by amber_fill done-strobe. |
| Snoop arrives during fill | If match, CRRESP and CD beats sourced from bypass register; otherwise normal tag lookup. |
| CTRL_FILL_WRITE | Bypass register contents written to arrays; pf_active cleared. |
| CTRL_REPLAY | Original request now hits the installed line. |

The critical correctness property is state-accuracy: the bypass register must answer with the post-fill state, never the pre-fill (Invalid) state. A snoop that would invalidate the line after the fill must see the post-fill state and then be allowed to perform its invalidation in the normal snoop path.

## 8.5	Snoop vs Fill Ordering

If a snoop arrives for the pending line and requires data transfer, the CD beats are sourced from whichever copy is available:

- If the beat has already been received (pf_data_valid[i] == 1), it is forwarded directly from the fill-path register.

- If the beat has not yet arrived, the snoop responder stalls on cd_ready until the beat arrives, then forwards it.

The fill still completes normally into the arrays after RLAST. The snoop does not cancel the fill; any state change caused by the snoop is applied after the fill commits.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	amber_tag_array and amber_data_array

Modules: amber_tag_array.sv, amber_data_array.sv Location: projects/components/cache-ip/amber-mesi-l1/rtl/fub/ Status: RTL landed 2026-10-06; this chapter is the micro-architecture contract the RTL implements. DV: dv/tests/fub/test_amber_{tag,data}_array.py, green at gate/func/full across the default and tiny-formal geometries.

## 9.1	Overview

Both arrays are per-way inferred RAMs — the same storage idiom the house FIFOs and sdpram_core use internally, with the GLOBAL_REQUIREMENTS 1.2 synthesis attributes. They are not sdpram_core instances: sdpram_core (rtl/amba/shared) is a FUB/AXI burst slave, the wrong shape for a multi-way parallel tag lookup or single-beat random data readout. Each array has one synchronous write port (one-hot way select) and two combinational lookup ports: port A serves CPU/fill traffic and port B serves snoop traffic, matching the port assignment below. Combinational read keeps the hit/miss decision and the snoop CRRESP path inside one cycle; it maps to distributed RAM (tags) / block RAM (data) rather than a wrapped burst slave.

The one-write / two-read shape exceeds a single physical 2-port BRAM; per-way flat storage (way * SETS + set for tags, way * (SETS*FILL_BEATS) + {set, beat} for data) lets synthesis bank per way at the default geometry. That is the HAS F8 dual-port decision implemented at L1-array granularity.

## 9.2	amber_tag_array

### 9.2.1	Tag + state packing

Each way has its own sdpram_core instance, or the ways are banked into one instance per way depending on the synthesis attributes allowed by GLOBAL_REQUIREMENTS.md. The data width of each instance is:

TAG_STATE_WIDTH = TAG_WIDTH + 3

where TAG_WIDTH = ADDR_WIDTH - SET_INDEX_WIDTH - LINE_OFFSET_WIDTH and the 3 bits carry cache_state_t.

The memory word is packed as {tag[TAG_WIDTH-1:0], state[2:0]} — verified by the DV against per-location values that place nonzero patterns in both fields.

### 9.2.2	Port assignment

| Port | Traffic | Operations |
| --- | --- | --- |
| A | CPU / fill | Read tag+state for lookup; write tag+state on fill or hit-state update. |
| B | Snoop | Read tag+state for snoop lookup. |

Port A is driven by amber_control. Port B is driven by amber_control during snoop handling.

### 9.2.3	Addressing

The address into the tag array is the set index:

tag_addr = req_addr[SET_INDEX_WIDTH + OFFSET_WIDTH - 1 : OFFSET_WIDTH]

Way selection is done by instantiating one sdpram_core per way and decoding the way index on writes. On reads, all ways return their tag+state in parallel and a comparator determines the hit way.

### 9.2.4	Banking

For timing closure at larger geometries, each way is a separate physical memory instance. There is no cross-way banking; a fill or snoop reads all ways in parallel and writes one. This keeps the critical hit/miss decision a simple combinational comparison across WAYS tag outputs.

## 9.3	amber_data_array

### 9.3.1	Organization

The data array stores cache-line data. A line is LINE_BYTES*8 bits wide and is split into FILL_BEATS = LINE_BYTES / (BUS_WIDTH/8) beats. Each way has its own sdpram_core instance, or one instance per way, with:

DATA_MEM_DEPTH = SETS * FILL_BEATS
DATA_MEM_WIDTH = BUS_WIDTH

The address is {set_index, beat_index}.

### 9.3.2	Port assignment

| Port | Traffic | Operations |
| --- | --- | --- |
| A | CPU / fill | Read data on hit or replay; write fill beats. |
| B | Snoop | Read whole line for snoop data transfer. |

### 9.3.3	Write masking

sdpram_core is instantiated with byte-write enable support. On a CPU write hit, only the bytes selected by be are written; the rest of the line is preserved. On a fill, the entire beat is written with all byte enables asserted. On a write-allocate miss, the fill data is merged with the CPU write data before installation.

### 9.3.4	Snoop data readout

A snoop that requires data transfer reads the matching way from port B beat-by-beat. amber_control drives b_addr = {snoop_set, beat}, increments the beat counter each cycle until the line is complete, and supplies the hit way on b_way (from the tag compare on port B). The data is forwarded to amber_snoop_resp for CD transmission.

## 9.4	Reset Behavior

The arrays carry no reset port (GLOBAL_REQUIREMENTS 1.4 — SRAM contents are not reset, and real BRAM has no reset pin). After aresetn deassertion, amber_control treats all ways as Invalid until the init walk completes; option 1 from the pre-RTL review is the contract: a hardware sequence walks all sets and writes STATE_I to every way (the DV TBs perform the same walk so unwritten locations cannot leak X’s). The formal proofs assume all lines are Invalid after reset.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	amber_repl: Replacement Policy Engine

Module: amber_repl.sv Location: projects/components/cache-ip/amber-mesi-l1/rtl/fub/ Status: RTL landed 2026-10-06; this chapter is the micro-architecture contract the RTL implements. DV: dv/tests/fub/test_amber_repl.py, green at gate/func/full across all four policies x {128/4, 16/2, 64/8} geometries. The TB carries an independent Python golden model per policy (tree-PLRU included — it still has no cache_sim golden; the TB model is the check).

## 10.1	Overview

amber_repl selects the victim way on a cache miss. The policy is an elaboration parameter REPL_POLICY with supported values lru, tree_plru, fifo, and random. LRU is the default; tree-PLRU is the timing-tight fallback.

In the RTL the parameter is declared int holding the amber_repl_t encoding from amber_pkg (house precedent: fifo_sync MEM_STYLE is typed, but a typed enum parameter receives -G overrides as raw 32-bit constants and verilator WIDTHTRUNCs values 2/3 — the int declaration keeps test-time overrides width-clean; the generate branches compare against int'(AMBER_REPL_*) casts).

## 10.2	Interface

| Signal | Direction | Width | Meaning |
| --- | --- | --- | --- |
| repl_req | input | 1 | Request a victim way this cycle. |
| repl_set | input | SET_INDEX_WIDTH | Set index for the access. |
| repl_hit_way | input | WAY_WIDTH | Way that hit (for update). |
| repl_hit | input | 1 | A hit occurred; update policy state. |
| repl_victim_way | output | WAY_WIDTH | Selected victim way. |
| repl_update | input | 1 | A miss install occurred; update policy state for repl_set. |

repl_hit and repl_update are mutually exclusive in the blocking pipeline: a cycle is either a hit service or a miss install, never both.

## 10.3	LRU (True LRU)

True LRU maintains a recency stack per set. For WAYS ways, each set stores WAYS*(WAYS-1)/2 ordering bits. On every hit or install, the accessed way is moved to the most-recent position and the other ways are shifted down. The victim is the least-recent way.

This matches cache_sim exactly and is the golden reference for policy parity.

### 10.3.1	Implementation

Use a per-set array of WAYS rank values. The rank value is 0 for most-recent, WAYS-1 for least-recent. On access:

for (int w = 0; w < WAYS; w++) begin
    if (w == accessed_way)
        rank[set][w] = 0;
    else if (rank[set][w] < old_rank_of_accessed_way)
        rank[set][w] += 1;
end

The victim way is the one with rank[set][w] == WAYS-1.

## 10.4	FIFO

Each set maintains a FIFO queue of way indices, stored as a ring with a head pointer. On a miss, the head of the queue is the victim; on an install, the ring slot at the head is overwritten with the installed way and the head advances — the popped victim slot becomes the tail, so one write plus one head bump is the whole update. On a hit, the queue is unchanged (FIFO does not recency-stack); the TB golden model encodes the same rule (UPDATES_ON_HIT = False).

This matches cache_sim exactly, under the invariant that the installed way equals the victim way — which amber_control guarantees because fills install into the victim way. Installing into any other way would require a mid-queue removal this ring does not perform; that is a contract, not an accident.

This matches cache_sim exactly.

## 10.5	RANDOM

Each set contains a small LFSR. On a replacement request, the LFSR value is truncated to WAY_WIDTH bits to select the victim way. The LFSR advances every replacement request. The seed is parameterizable for repeatable tests.

This matches cache_sim exactly when the same LFSR polynomial and seed are used.

## 10.6	Tree-PLRU

Tree-PLRU is carried as the timing-tight fallback per PRD D7. It uses WAYS-1 bits per set arranged as a binary tree. On each access, the tree bits point away from the accessed way. On a replacement request, the tree bits are followed to find the victim.

Tree-PLRU does not have a cache_sim golden model today. It is cross-checked by RTL self-checks and is recorded as a future cache_sim extension. This is an honest caveat, not a hidden assumption.

## 10.7	Policy Selection

The policy is selected at elaboration time; the RTL declares the parameter int (values are the amber_repl_t encodings) and generates exactly one branch:

parameter int REPL_POLICY = int'(AMBER_REPL_LRU)   // values: amber_repl_t

generate
    if (REPL_POLICY == int'(AMBER_REPL_LRU)) begin : gen_lru
        // true LRU rank array
    end else if (REPL_POLICY == int'(AMBER_REPL_FIFO)) begin : gen_fifo
        // FIFO ring array
    end else if (REPL_POLICY == int'(AMBER_REPL_RANDOM)) begin : gen_random
        // LFSR per set
    end else if (REPL_POLICY == int'(AMBER_REPL_TREE_PLRU)) begin : gen_tree_plru
        // binary tree PLRU
    end
endgenerate

Only one policy is instantiated per build. This keeps area predictable and matches the cache_sim per-policy parity model.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	amber_victim: Depth-1 Victim Buffer

Module: amber_victim.sv Location: projects/components/cache-ip/amber-mesi-l1/rtl/fub/ Status: Pre-RTL micro-architecture contract

## 11.1	Overview

amber_victim is a depth-1 buffer that holds one dirty cache line while its write-back (drain) is outstanding. The depth-1 size matches the single-outstanding-miss model: there can be only one dirty victim at a time because the pipeline blocks on the miss until it resolves.

## 11.2	Fields

| Field | Width | Meaning |
| --- | --- | --- |
| victim_addr | ADDR_WIDTH | Full line address (for AW). |
| victim_data | LINE_BYTES*8 | Dirty line data. |
| victim_valid | 1 | Buffer is occupied. |

## 11.3	Handshake with amber_control

| Signal | Direction | Meaning |
| --- | --- | --- |
| victim_load | input from amber_control | Load the buffer from amber_data_array port A this cycle. |
| victim_addr_in | input from amber_control | Line address to load. |
| victim_data_in | input from amber_control | Line data to load. |
| victim_busy | output to amber_control | Buffer already occupied. |
| victim_empty | output to amber_control | Buffer is empty. |

victim_load asserts for one cycle in CTRL_MISS_VICTIM when the victim is dirty. The buffer is busy until amber_drain reports done.

## 11.4	Bypass for Snoops

If a snoop arrives for the victim line while the drain is still outstanding, amber_victim can supply the data without re-reading amber_data_array. amber_control checks:

victim_valid && (snoop_line_addr == victim_addr)

If matched, the snoop responder sources CD beats from victim_data instead of from data_array port B. This is the victim bypass.

## 11.5	Timing

| Cycle | Event |
| --- | --- |
| CTRL_MISS_VICTIM, victim is M | victim_load asserted; victim_valid sets next cycle. |
| CTRL_MISS_DRAIN | amber_drain consumes victim_addr and victim_data; begins AW/W burst. |
| Drain done (ctrl_drain_done) | victim_valid cleared. |

The buffer is never loaded while valid; the single-outstanding property guarantees this cannot happen.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	amber_fill and amber_drain

Modules: amber_fill.sv, amber_drain.sv Location: projects/components/cache-ip/amber-mesi-l1/rtl/fub/ Status: Pre-RTL micro-architecture contract

## 12.1	amber_fill

### 12.1.1	Purpose

amber_fill issues whole-line read bursts to the memory-side master and forwards received beats to amber_data_array port A and to the pending-fill bypass register.

### 12.1.2	Interface

| Signal | Direction | Meaning |
| --- | --- | --- |
| fill_start | input from amber_control | Launch a fill for fill_addr. |
| fill_addr | input from amber_control | Line-aligned address. |
| fill_done | output to amber_control | All beats received (RLAST accepted). |
| fill_beat_valid | output to amber_control / data array | A fill beat is valid this cycle. |
| fill_beat_data | output | Fill beat data. |
| fill_beat_idx | output | Beat index within line. |
| fill_last | output | Last beat of burst. |

### 12.1.3	Burst parameters

- ARLEN = FILL_BEATS - 1

- ARSIZE = log2(BUS_WIDTH/8)

- ARBURST = INCR

- ARADDR line-aligned

### 12.1.4	Timing

- fill_start asserted in CTRL_MISS_FILL.

- amber_fill asserts fub_axi_arvalid to the memory-side read master.

- As R beats arrive, fill_beat_valid pulses; fill_beat_idx increments.

- Each beat is written to amber_data_array port A and the pf_data_valid mask is updated.

- On RLAST acceptance, fill_done pulses for one cycle.

## 12.2	amber_drain

### 12.2.1	Purpose

amber_drain takes a dirty line from amber_victim and issues a whole-line write-back burst to the memory-side write master.

### 12.2.2	Interface

| Signal | Direction | Meaning |
| --- | --- | --- |
| drain_start | input from amber_control | Launch drain from amber_victim. |
| drain_done | output to amber_control | B response received. |
| drain_beat_ready | input from memory master | W-channel ready. |
| drain_beat_valid | output | W-channel valid. |
| drain_beat_data | output | W-channel data. |
| drain_beat_strb | output | W-channel strobe (all-1s for whole-line). |
| drain_beat_last | output | W-channel last. |

### 12.2.3	Burst parameters

- AWLEN = FILL_BEATS - 1

- AWSIZE = log2(BUS_WIDTH/8)

- AWBURST = INCR

- AWADDR = victim_addr

- WSTRB = all-1s for every beat

### 12.2.4	Timing

- drain_start asserted in CTRL_MISS_DRAIN when amber_victim is valid.

- amber_drain asserts fub_axi_awvalid and begins driving W beats as the master accepts them.

- After WLAST is accepted and the B response arrives, drain_done pulses.

### 12.2.5	On the onyx rig

For amber_ace, dirty evictions are issued as WriteBack transactions. amber_ace_issue supplies AWSNOOP and the AW address; amber_drain supplies the W beats. Clean evictions are Evict transactions with no data.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	amber_snoop_resp: Snoop Responder Adapter

Module: amber_snoop_resp.sv Location: projects/components/cache-ip/amber-mesi-l1/rtl/fub/ Status: RTL landed 2026-10-06; this chapter is the contract the RTL implements. Wraps the house axi4ace_snoop_slave transport (which exists in rtl/amba/ace — confirmed). DV: dv/tests/fub/test_amber_snoop_resp.py, green at gate/func/full on both geometries; the FULL level is a 3000-snoop randomized soak over an evolving line-state model with per-transaction ACE compliance checks, not a token rerun.

## 13.1	Overview

amber_snoop_resp is the ACE-shaped snoop responder. It wraps the house axi4ace_snoop_slave module and adds the small state machine that sequences CR and CD for each snoop transaction. Inside amber_core the snoop is bus-agnostic; only amber_snoop_resp speaks ACE.

## 13.2	External Interface

The external side is the standard axi4ace_snoop_slave port:

| Signal | Direction | Width | Purpose |
| --- | --- | --- | --- |
| m_axi_acaddr | input | ADDR_WIDTH | Snoop address |
| m_axi_acsnoop | input | 4 | Snoop type |
| m_axi_acprot | input | 3 | Snoop protection |
| m_axi_acvalid | input | 1 | AC valid |
| m_axi_acready | output | 1 | AC ready |
| m_axi_crresp | output | 5 | CR response |
| m_axi_crvalid | output | 1 | CR valid |
| m_axi_crready | input | 1 | CR ready |
| m_axi_cddata | output | DATA_WIDTH | CD data |
| m_axi_cdlast | output | 1 | CD last |
| m_axi_cdvalid | output | 1 | CD valid |
| m_axi_cdready | input | 1 | CD ready |

## 13.3	Internal Interface

The core-facing side is a bus-agnostic handshake:

| Signal | Direction | Meaning |
| --- | --- | --- |
| ctrl_snoop_req | output to amber_control | Valid snoop address + type. |
| ctrl_snoop_addr | output | Snoop line address. |
| ctrl_snoop_type | output | Encoded snoop type. |
| ctrl_snoop_ready | input from amber_control | Control accepts snoop. |
| ctrl_crresp | input from amber_control | 5-bit response. |
| ctrl_cddata | input from amber_control | CD beat data. |
| ctrl_cdlast | input from amber_control | CD last beat. |
| ctrl_cdvalid | input from amber_control | CD beat valid. |
| ctrl_cdready | output to amber_control | Adapter ready for CD beat. |

## 13.4	Adapter State Machine

| State | Meaning |
| --- | --- |
| SR_IDLE | Waiting for AC handshake. |
| SR_LOOKUP | Snoop accepted; waiting for amber_control CRRESP. |
| SR_DATA | Driving CD beats. |
| SR_RESP | Driving CR after CDLAST. |

### 13.4.1	State transitions

- SR_IDLE → SR_LOOKUP: AC handshake completes (m_axi_acvalid && m_axi_acready).

- SR_LOOKUP → SR_DATA: amber_control returns ctrl_crresp and CRRESP.DataTransfer is 1.

- SR_LOOKUP → SR_RESP: amber_control returns ctrl_crresp and CRRESP.DataTransfer is 0.

- SR_DATA → SR_RESP: Last CD beat accepted (m_axi_cdvalid && m_axi_cdready && m_axi_cdlast).

- SR_RESP → SR_IDLE: CR handshake completes (m_axi_crvalid && m_axi_crready).

## 13.5	CR-after-CDLAST Ordering

amber strengthens the protocol by completing all CD beats before asserting CR. This matches:

- IHI0022: the snoop response is required only after the final CD beat.

- The cocotb-framework 1.2.0 ACE compliance checker (check_cr_order / check_cd_order).

- The family convention recorded with onyx D4.

The CR channel is held low while the adapter is in SR_DATA. This makes the gather logic simple: one closed response at a time.

## 13.6	Snoop Type Decoding

m_axi_acsnoop carries the six IHI0022 snoop encodings used by amber:

| Encoding | Type |
| --- | --- |
| ReadShared | Shared read snoop |
| ReadOnce | Single read snoop |
| ReadUnique | Exclusive read snoop |
| CleanShared | Clean shared snoop |
| CleanInvalid | Clean invalidate snoop |
| MakeInvalid | Invalidate snoop |

CleanUnique and MakeUnique are transactions a master issues, not snoop encodings, so they do not appear on the AC channel. The CRRESP bits for each {state, snoop} pair are the authority of the HAS Table 3.0 and are captured as K-maps in the workbook.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	amber_ace_issue: Coherent Transaction Mapping

Module: amber_ace_issue.sv Location: projects/components/cache-ip/amber-mesi-l1/rtl/fub/ Status: Pre-RTL micro-architecture contract

## 14.1	Overview

amber_ace_issue maps cache-side events to the onyx D2 coherent-transaction subset. It lives only in the amber_ace rig; the pair-rig amber top does not instantiate it. The module sits between amber_control / amber_fill / amber_drain and the axi4ace_master_rd / axi4ace_master_wr wrappers.

## 14.2	Event-to-Transaction Mapping

Table 2.8.1: Cache events to ACE transactions

| Cache event | ACE transaction | Snoop field | Notes |
| --- | --- | --- | --- |
| Read miss, shared intent | ReadShared | fub_axi_arsnoop[3:0] | Line will be installed in S. |
| Read miss, exclusive intent | ReadUnique | fub_axi_arsnoop[3:0] | Line will be installed in E. |
| Write to Shared line | CleanUnique | fub_axi_awsnoop[2:0] | Upgrades S → M without data transfer. |
| Write to Invalid line (whole line) | MakeUnique | fub_axi_awsnoop[2:0] | Invalidates other copies; line installed in M. |
| Dirty eviction | WriteBack | fub_axi_awsnoop[2:0] | Whole-line write-back with data. |
| Clean eviction | Evict | fub_axi_awsnoop[2:0] | No data; notifies interconnect. |

## 14.3	Interface

### 14.3.1	From amber_control / fill / drain

| Signal | Direction | Meaning |
| --- | --- | --- |
| ace_rd_req | input | Read transaction request. |
| ace_rd_type | input | Encoded event type (shared read / unique read). |
| ace_rd_addr | input | Line-aligned address. |
| ace_rd_len | input | Burst length. |
| ace_wr_req | input | Write transaction request. |
| ace_wr_type | input | Encoded event type (CleanUnique / MakeUnique / WriteBack / Evict). |
| ace_wr_addr | input | Line-aligned address. |
| ace_wr_len | input | Burst length. |

### 14.3.2	To axi4ace_master_rd / axi4ace_master_wr

amber_ace_issue drives the fub_axi_* ports of the ACE master wrappers. It supplies arsnoop/awsnoop and the standard AXI address/data fields. The wrappers auto-pulse RACK/WACK.

## 14.4	CleanUnique / MakeUnique Data Behavior

- CleanUnique is an AW-only transaction (no W data). It tells the interconnect to invalidate other copies so the local write can proceed. amber_control performs the actual data write into amber_data_array and promotes the line to M.

- MakeUnique is also an AW-only transaction for the full-line case. It invalidates other copies and grants exclusive ownership; the local line is installed or promoted to M.

- WriteBack is an AW+W burst with the dirty victim data.

- Evict is an AW-only transaction with no data.

## 14.5	Timing

amber_ace_issue is combinational for address/snoop mapping. It does not add pipeline stages; the timing path is from amber_control through this module into the axi4ace_master_* skid buffers.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	amber_cpu_frontend and amber_monlite

Modules: amber_cpu_frontend.sv, amber_monlite.sv Location: projects/components/cache-ip/amber-mesi-l1/rtl/fub/ Status: Pre-RTL micro-architecture contract

## 15.1	amber_cpu_frontend

### 15.1.1	Purpose

amber_cpu_frontend is the GAXI slave that accepts CPU requests and returns responses. Because the cache is blocking, it latches one request at a time and does not accept a new request until amber_control returns req_ready.

### 15.1.2	Request payload

CPU_REQ_W = ADDR_WIDTH + 1 + BUS_WIDTH/8 + BUS_WIDTH

Packed as {addr[ADDR_WIDTH-1:0], we, be[BUS_WIDTH/8-1:0], wdata[BUS_WIDTH-1:0]}.

### 15.1.3	Response payload

CPU_RSP_W = BUS_WIDTH

Packed as {rdata[BUS_WIDTH-1:0]}.

### 15.1.4	Handshake

| Signal | Direction | Meaning |
| --- | --- | --- |
| cpu_req_wr_valid | input | CPU has a request. |
| cpu_req_wr_ready | output | Frontend accepts request this cycle. |
| cpu_req_wr_data | input | Request payload. |
| cpu_rsp_rd_valid | output | Response ready. |
| cpu_rsp_rd_ready | input | CPU accepts response. |
| cpu_rsp_rd_data | output | Response payload. |

cpu_req_wr_ready is asserted only when amber_control is in CTRL_IDLE and no snoop priority stall is active. Once a request is accepted, the front end latches it and holds it until amber_control returns the response.

### 15.1.5	Replay path

On a miss, amber_control re-drives the original latched request into its own hit logic in CTRL_REPLAY. The front end does not see a new GAXI transaction; the response is generated from the same latched request.

## 15.2	amber_monlite

### 15.2.1	Purpose

amber_monlite is the drop-and-count MonBus observer. It taps the internal signals of amber_core and emits 128-bit MonBus event packets. It never stalls the observed path; under sustained backpressure it drops packets and counts the loss.

### 15.2.2	Observation points

| Event | Emit point | Payload highlights |
| --- | --- | --- |
| hit | CTRL_HIT_RD / CTRL_HIT_WR | set, way, old/new state |
| miss | CTRL_MISS_VICTIM | set, way, miss class |
| snoop | snoop accepted | snoop type, hit/miss, response class |
| eviction | CTRL_MISS_VICTIM dirty victim | clean/dirty, set, way |
| MESI transition | state update | old state, new state, cause |
| fill start | CTRL_MISS_FILL | address, direction |
| fill end | CTRL_FILL_WRITE | address |
| drain start | CTRL_MISS_DRAIN | address |
| drain end | drain done | address |

### 15.2.3	Packet format

Standard MonBus 128-bit packet with house UNIT/AGENT ids. The exact field layout follows the shared monitor_common_pkg spec; the MAS does not invent a new layout.

### 15.2.4	Drop-and-count

If mon_ready is low when a packet is ready, the packet is dropped and a saturating drop counter increments. The drop count is reported as an EVENT_DROPPED packet when the bus next has room, exactly like the STREAM monitor-lite behavior.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	CPU GAXI Request/Response Timing

## 16.1	Channel Structure

The CPU-side port is two house-GAXI streams:

Table 3.1.1: CPU GAXI channels

| Signal | Direction | Width | Payload |
| --- | --- | --- | --- |
| cpu_req_wr_valid | in | 1 | request valid |
| cpu_req_wr_ready | out | 1 | request accepted |
| cpu_req_wr_data | in | CPU_REQ_W | packed {addr, we, be, wdata} |
| cpu_rsp_rd_valid | out | 1 | response valid |
| cpu_rsp_rd_ready | in | 1 | response accepted |
| cpu_rsp_rd_data | out | CPU_RSP_W | packed {rdata} |

## 16.2	Request Timing

Because the cache is blocking, cpu_req_wr_ready is asserted only when amber_control is in CTRL_IDLE and no higher-priority snoop stall is active.

Cycle :  0   1   2   3   4   5
req_v :  1   0   0   0   0   0
req_r :  1   0   0   0   0   0   <- handshake at cycle 0
state :  L0  L1  HR  HR  RSP RSP

- Cycle 0: request accepted; amber_cpu_frontend latches it.

- Cycle 1: CTRL_LOOKUP drives tag-array port A.

- Cycle 2–3: hit service (CTRL_HIT_RD or CTRL_HIT_WR).

- Cycle 4–5: response valid until accepted.

On a miss, cpu_req_wr_ready stays low from cycle 1 through the entire miss sequence. The front end does not accept a new request until the replay completes.

## 16.3	Response Timing

cpu_rsp_rd_valid is asserted when amber_control enters the response state. The data is held stable until cpu_rsp_rd_ready is high. Because the response is single-beat and the front end blocks, backpressure is expected to be rare, but the handshake is still honored.

## 16.4	Write-Allocate Fills

A write miss triggers a fill, a merge of the CPU write data with the fill data, and then a replay of the write as a hit. The GAXI slave sees only the original single-beat write request and the eventual single-beat write response; the read-modify-write sequence is internal to amber_control.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Fabric AXI4 Master Timing

The amber pair-rig top uses the house axi4_master_rd and axi4_master_wr wrappers for plain AXI4 memory access. This chapter documents the cycle-level handoff between amber_fill / amber_drain and those wrappers.

## 17.1	Read (Fill) Timing

amber_fill drives the fub_axi_* side of axi4_master_rd:

Cycle :  0   1   2   3   4   5   6
AR_v  :  1   0   0   0   0   0   0
AR_r  :  1   0   0   0   0   0   0   <- AR handshake
R_v   :  0   0   0   1   1   1   0
R_r   :  0   0   0   1   1   1   0   <- R beats accepted
last  :  0   0   0   0   0   1   0

- ARLEN = FILL_BEATS - 1, ARSIZE = log2(BUS_WIDTH/8), ARBURST = INCR.

- RREADY is asserted as long as amber_data_array port A can accept the beat.

- Each accepted R beat is written to amber_data_array and the pending-fill bypass register is updated.

- fill_done pulses one cycle after RLAST is accepted.

## 17.2	Write (Drain) Timing

amber_drain drives the fub_axi_* side of axi4_master_wr:

Cycle :  0   1   2   3   4   5   6
AW_v  :  1   0   0   0   0   0   0
AW_r  :  1   0   0   0   0   0   0   <- AW handshake
W_v   :  0   1   1   1   0   0   0
W_r   :  0   1   1   1   0   0   0   <- W beats accepted
last  :  0   0   0   1   0   0   0
B_v   :  0   0   0   0   0   1   0
B_r   :  0   0   0   0   0   1   0   <- B handshake

- AWLEN = FILL_BEATS - 1, AWSIZE = log2(BUS_WIDTH/8), AWBURST = INCR.

- WSTRB is all-1s for every beat.

- drain_done pulses one cycle after the B handshake.

## 17.3	Single Outstanding

There is at most one AR transaction and one AW transaction in flight at any time because the blocking pipeline does not launch a new miss until the previous one completes. No reorder buffer is needed inside amber_fill or amber_drain.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	Fabric ACE Master Timing

The amber_ace onyx-rig top uses the house axi4ace_master_rd and axi4ace_master_wr wrappers. The timing is identical to the AXI4 case except for the ARSNOOP/AWSNOOP fields and the auto-pulsed RACK/WACK outputs.

## 18.1	Read Transactions

| Cache event | ACE transaction | fub_axi_arsnoop |
| --- | --- | --- |
| read miss, shared intent | ReadShared | ReadShared encoding |
| read miss, exclusive intent | ReadUnique | ReadUnique encoding |

RACK is auto-pulsed one cycle after RLAST is accepted. amber_ace_issue supplies fub_axi_arsnoop; amber_fill supplies address, length, size, and burst.

## 18.2	Write Transactions

| Cache event | ACE transaction | fub_axi_awsnoop |
| --- | --- | --- |
| write to Shared line | CleanUnique | CleanUnique encoding |
| write to Invalid line (whole line) | MakeUnique | MakeUnique encoding |
| dirty eviction | WriteBack | WriteBack encoding |
| clean eviction | Evict | Evict encoding |

WACK is auto-pulsed one cycle after the B handshake. CleanUnique, MakeUnique, and Evict are AW-only transactions with no W data. WriteBack is an AW+W burst with the dirty victim data.

## 18.3	Timing Example: ReadShared Fill

Cycle :  0   1   2   3   4   5   6
AR_v  :  1   0   0   0   0   0   0
AR_r  :  1   0   0   0   0   0   0   <- AR handshake, ARSNOOP=ReadShared
R_v   :  0   0   0   1   1   1   0
R_r   :  0   0   0   1   1   1   0
last  :  0   0   0   0   0   1   0
rack  :  0   0   0   0   0   0   1   <- auto-pulsed

## 18.4	Timing Example: WriteBack Drain

Cycle :  0   1   2   3   4   5   6
AW_v  :  1   0   0   0   0   0   0
AW_r  :  1   0   0   0   0   0   0   <- AW handshake, AWSNOOP=WriteBack
W_v   :  0   1   1   1   0   0   0
W_r   :  0   1   1   1   0   0   0
last  :  0   0   0   1   0   0   0
B_v   :  0   0   0   0   0   1   0
B_r   :  0   0   0   0   0   1   0
wack  :  0   0   0   0   0   0   1   <- auto-pulsed

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Snoop AC/CD/CR Timing and Ordering

## 19.1	External ACE Snoop Port

| Signal | Direction | Purpose |
| --- | --- | --- |
| m_axi_acvalid / m_axi_acready | in / out | AC handshake |
| m_axi_acaddr | in | snoop line address |
| m_axi_acsnoop | in | snoop type |
| m_axi_cdvalid / m_axi_cdready | out / in | CD handshake |
| m_axi_cddata | out | snoop data beat |
| m_axi_cdlast | out | last CD beat |
| m_axi_crvalid / m_axi_crready | out / in | CR handshake |
| m_axi_crresp | out | 5-bit response |

## 19.2	Transaction Sequence

Cycle :  0   1   2   3   4   5   6   7
AC_v  :  1   0   0   0   0   0   0   0
AC_r  :  1   0   0   0   0   0   0   0   <- AC handshake
lookup:      L0  L1
CD_v  :  0   0   0   1   1   1   0   0
CD_r  :  0   0   0   1   1   1   0   0
last  :  0   0   0   0   0   1   0   0
CR_v  :  0   0   0   0   0   0   1   0
CR_r  :  0   0   0   0   0   0   1   0   <- CR after CDLAST

- Cycle 0: AC handshake. The snoop address and type are captured.

- Cycles 1–2: amber_control performs tag lookup on port B. If the line is M/E and the snoop requires data, it reads the data array port B.

- Cycles 3–5: CD beats are driven and accepted.

- Cycle 6: CR is asserted after CDLAST has been accepted.

- Cycle 7: CR handshake completes; the adapter returns to idle.

## 19.3	CR-After-CDLAST Rule

amber_snoop_resp holds m_axi_crvalid low until the last CD beat has been accepted. This is stronger than the minimum IHI0022 requirement but matches the family convention and the cocotb-framework 1.2.0 compliance checker. It also makes the gather logic simple: the manager sees one closed response at a time.

## 19.4	Multiple Outstanding Snoops

The axi4ace_snoop_slave wrapper buffers up to SKID_DEPTH_AC snoops. However, amber_control processes them one at a time because it is a single FSM. Therefore the effective outstanding snoop count is 1 from the core’s perspective; the adapter FIFO absorbs bursts of AC arrivals.

## 19.5	AC Ready Backpressure

m_axi_acready is the skid-buffer wr_ready from axi4ace_snoop_slave. It can be low when the AC skid is full. The manager must respect this; amber never drops a snoop address because the adapter buffers it.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	MonBus Event Emission Points

## 20.1	Packet Format

amber_monlite emits standard 128-bit MonBus packets plus a 64-bit side-band timestamp. The packet layout is the house monitor_common_pkg spec; the timestamp is sampled at the same cycle as the packet and held stable while mon_valid is high.

## 20.2	Emission Timing

Events are emitted as one-cycle pulses. The event class and payload are captured combinatorially from the control FSM state and registered at the emission point to avoid glitches.

| Event class | Emit point | Payload |
| --- | --- | --- |
| hit | CTRL_HIT_RD / CTRL_HIT_WR | set, way, state before/after |
| miss | CTRL_MISS_VICTIM | set, way, miss class |
| snoop | AC handshake accepted | snoop type, hit/miss flag |
| eviction | CTRL_MISS_DRAIN start | clean/dirty, set, way |
| MESI transition | tag-array state write | old state, new state, cause code |
| fill_start | CTRL_MISS_FILL | line address |
| fill_end | CTRL_FILL_WRITE | line address |
| drain_start | CTRL_MISS_DRAIN | line address |
| drain_end | drain_done | line address |

## 20.3	Drop-and-Count

If mon_ready is low when mon_valid would assert, the packet is dropped and a saturating drop counter increments. The counter is reported as an EVENT_DROPPED packet when MonBus next has room. This is identical to the STREAM monitor-lite behavior and never stalls the observed path.

## 20.4	Backpressure Integrity

amber_monlite is a pure observer. Its FIFO state never gates the CPU GAXI port, the AXI4/ACE masters, or the snoop responder. This is the measurement-integrity rationale that chose *_monlite over the heavyweight _mon wrappers.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	MonBus Event Encodings and Emission Points

## 21.1	Overview

Because amber has no register block, the MonBus observation map replaces the traditional register-map chapter. Every software-visible event is emitted as a MonBus packet by amber_monlite.

## 21.2	Event Encoding

Table 4.1.1: MonBus event encodings

| Event code | Name | Emitting module | Trigger |
| --- | --- | --- | --- |
| AMBER_EV_HIT | hit | amber_monlite | CTRL_HIT_RD or CTRL_HIT_WR |
| AMBER_EV_MISS | miss | amber_monlite | CTRL_MISS_VICTIM |
| AMBER_EV_SNOOP | snoop | amber_monlite | AC handshake accepted |
| AMBER_EV_EVICT | eviction | amber_monlite | dirty victim staged to amber_victim |
| AMBER_EV_TRANSITION | MESI transition | amber_monlite | tag-array state write |
| AMBER_EV_FILL_START | fill start | amber_monlite | CTRL_MISS_FILL |
| AMBER_EV_FILL_END | fill end | amber_monlite | CTRL_FILL_WRITE |
| AMBER_EV_DRAIN_START | drain start | amber_monlite | CTRL_MISS_DRAIN |
| AMBER_EV_DRAIN_END | drain end | amber_monlite | drain_done |
| AMBER_EV_DROPPED | dropped packet | amber_monlite | packet dropped due to backpressure |

The exact numeric codes are assigned in amber_pkg and match the house monitor_common_pkg allocation rules.

## 21.3	Payload Fields

All packets carry:

- agent_id — house MON_AGENT_ID for amber.

- unit_id — sub-unit within amber (control, snoop, etc.).

- event_code — one of the codes above.

- timestamp — 64-bit side-band sampled at emission.

Event-specific payload:

| Event | Payload fields |
| --- | --- |
| hit | set, way, state_before, state_after, we |
| miss | set, way, miss_class, we |
| snoop | snoop_type, hit, response_class |
| eviction | set, way, dirty, line_addr |
| transition | set, way, old_state, new_state, cause |
| fill_start / fill_end | line_addr |
| drain_start / drain_end | line_addr |
| dropped | dropped_count |

## 21.4	Miss-Class Encoding

Miss classes follow the cache_sim computation:

| Code | Class | Meaning |
| --- | --- | --- |
| AMBER_MISS_COMPULSORY | compulsory | First access to this line. |
| AMBER_MISS_CAPACITY | capacity | Line was evicted by capacity pressure. |
| AMBER_MISS_CONFLICT | conflict | Line was evicted by mapping conflict. |

The golden model supplies the class; the RTL records what it is told. When the model is not present, the class is recorded as AMBER_MISS_UNKNOWN.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Drop-and-Count Behavior and monbus_tally_axil

## 22.1	Drop-and-Count Policy

amber_monlite is drop-and-count, not stall-and-backpressure. This preserves measurement integrity: the observer cannot perturb the cache hit/miss timing it is trying to measure.

When a packet is emitted and mon_ready is low, the packet is discarded and a saturating counter increments. When mon_ready next goes high, if the counter is non-zero an AMBER_EV_DROPPED packet is emitted with the count, and the counter clears.

## 22.2	Counter Width

The drop counter is 8 bits. The probability of 256 consecutive drops in a correctly sized system is negligible; if it occurs, the counter saturates and the overflow is visible in the dropped packet.

## 22.3	monbus_tally_axil Integration

Amber’s MonBus connects to the shared monbus_tally_axil agent exactly like STREAM and every other component in the repo. The agent:

- Accepts mon_valid/mon_ready.

- Parses the 128-bit packet using monitor_common_pkg.

- Maintains per-event-class counters in AXI-Lite addressable registers.

- Reports dropped-packet counts.

## 22.4	Software View

Software reads event counters through the monbus_tally_axil AXI-Lite slave. There is no APB CSR inside amber. The counter addresses and field layouts are defined by the tally agent, not by amber.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	What the K-maps Prove

## 23.1	Methodology

The kmap workbook (gen_amber_contracts_kmaps.py) applies the STREAM-derived methodology:

- Identify every important combinational decision signal.

- List axes as (name, expression, cite) triples.

- State depends_only_on evidence that justifies the axis list.

- Add checked relations that mark unreachable input combos as don’t-cares.

- Derive the minimal SOP mechanically with Quine-McCluskey.

- Diff the derived cover against the RTL SOP when RTL exists.

Because amber has no RTL yet, the workbook is a pre-RTL contract. The derived SOP is the logic the future RTL must implement. rtl_sop is intentionally absent in the workbook; when RTL lands the diff step produces one of three verdicts.

## 23.2	The Three Verdicts

The verdict logic is implemented in bin/kmaps/minimize.py:

| Verdict | Meaning | Action |
| --- | --- | --- |
| IDENTICAL | The RTL SOP matches the derived minimal cover. | No change needed; the RTL is already minimal. |
| RTL-REDUNDANT | The RTL SOP includes extra literals or terms but produces the same truth table. | Document why the redundancy exists (timing, readability, defensive coding). |
| RTL-DIFFERS | The RTL SOP produces a different truth table than the derived cover. | Investigate: bug, unstated invariant, or missing relation. |

## 23.3	First-Verdict Status (2026-10-06)

The first RTL slice landed: amber_pkg (the Table 3.0 decode functions amber_snoop_crresp / amber_snoop_next_state) and its module wrapper amber_snoop_kmap. Verdicts for the six snoop sheets (the four CRRESP maps and the two next-state maps):

- Truth-table equivalence: PASS at gate/func/full. The TB (dv/tests/fub/test_amber_snoop_kmap.py) carries an independent copy of HAS Table 3.0 and exhausts the reachable space at gate, adds the reserved encodings at func, and the whole 3-bit x 3-bit space at full. One real discrepancy was caught and resolved in the TB’s favor of the documentation: Modified + CleanInvalid carries IsShared=1 per Table 3.0.

- SOP-literal diff against the QM-derived covers: NOT CHECKED, deferred. The RTL implements the table as a case decode, not minimal SOP literals; the meaningful diff is the truth-table one above. Supplying rtl_sop= and re-deriving is deferred until amber_snoop_resp integrates this decode — if the integration restructures the logic, the literal diff would be repeated work.

All other sheets (address decode, control FSM, MonBus events) remain pre-RTL contracts with VERDICT: NOT CHECKED.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	RTL Landing Diff Procedure

## 24.1	When RTL Lands

After the first RTL commit for a module, update the corresponding sheet in gen_amber_contracts_kmaps.py:

- Add citations to the RTL file and line numbers in the CITES registry.

- Replace rtl_sop=None with the actual RTL expression as a sum-of-products string.

- Re-run the generator.

- Review each sheet’s verdict.

First landing (2026-10-06): the snoop CRRESP / next-state decode exists as amber_pkg functions + amber_snoop_kmap. See 01_kmap_verdicts.md for why its sheets record truth-table PASS but a deferred SOP-literal diff; the procedure above applies to every sheet still at rtl_sop=None.

## 24.2	Diff Checklist

For every sheet that reports RTL-DIFFERS:

- Is the difference a deliberate redundancy? Re-label as RTL-REDUNDANT and add a note.

- Is an unstated invariant doing work? Add it as a checked relation and re-derive.

- Is the RTL expression wrong? File a bug and fix it.

For every sheet that reports RTL-REDUNDANT:

- Does the redundancy serve timing closure? Keep it and document.

- Does the redundancy serve readability? Keep it if the cost is small.

- Is it accidental? Simplify the RTL.

## 24.3	Regression Rule

The generator is part of the CI check for amber. Any RTL edit that moves or changes a cited expression must cause the generator to fail loudly, not silently publish a stale map. This is enforced by the CITES registry in gen_amber_contracts_kmaps.py.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	gem5 FSM Cross-Check

## 25.1	Source

The Python reference model and the formal FSM oracles derive from the gem5 Ruby MESI_Two_Level SLICC tables, specifically L1cache.sm. The enumerated transient states in that file are the source for the amber_pkg state encoding and for the control FSM states in amber_control.

## 25.2	Cross-Check Mechanism

- Extract every stable and transient state from L1cache.sm.

- Map gem5 events to amber signals (CPU request, snoop type, fill response, drain done).

- Replay a randomized trace through both the gem5-derived Python model and a future cycle-accurate RTL model.

- Compare state transitions, CRRESP outputs, and next-state writes.

## 25.3	Scope

The cross-check covers:

- All M/E/S/I stable states.

- Transient states between stable states and pending responses.

- The six IHI0022 snoop types.

- The onyx D2 coherent issue subset (ReadShared, ReadUnique, CleanUnique, MakeUnique, WriteBack, Evict).

MOESI O-state is out of scope for v1.0; the 3-bit encoding reserves a code for it.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	Elaboration Configurations

## 26.1	Default Config

The default elaboration geometry is the PRD D1 center point:

| Parameter | Value |
| --- | --- |
| SETS | 128 |
| WAYS | 4 |
| LINE_BYTES | 64 |
| BUS_WIDTH | 64 |
| REPL_POLICY | "LRU" |
| WRITE_POLICY | "wb_wa" |
| VICTIM_DEPTH | 1 |

This is the configuration used for FPGA characterization, cache_sim parity, and the FULL cocotb grid.

## 26.2	Tiny Formal Config

The SymbiYosys control-layer proofs use a tiny geometry for tractability:

| Parameter | Value |
| --- | --- |
| SETS | 16 |
| WAYS | 2 |
| LINE_BYTES | 64 |
| BUS_WIDTH | 64 |
| REPL_POLICY | "LRU" |
| WRITE_POLICY | "wb_wa" |
| VICTIM_DEPTH | 1 |

The tiny config reduces tag-array depth and way count so that the formal tool can fully explore the control FSM and pending-fill bypass interactions.

## 26.3	Bring-Up Config

A minimal bring-up config may use:

| Parameter | Value |
| --- | --- |
| SETS | 4 |
| WAYS | 2 |
| LINE_BYTES | 32 |
| BUS_WIDTH | 32 |
| REPL_POLICY | "FIFO" |
| WRITE_POLICY | "wt_na" |
| VICTIM_DEPTH | 1 |

This is for initial module-level smoke tests only, not for characterization or formal proofs.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	DV Matrix Dimensions

## 27.1	Pattern-B Cocotb Grids

The primary DV layer is Pattern-B GATE/FUNC/FULL cocotb grids over the geometry × policy × rig matrix.

### 27.1.1	Dimensions

| Dimension | Values |
| --- | --- |
| Geometry | default (128/4/64/64), tiny (16/2/64/64), bring-up (4/2/32/32) |
| Policy | LRU, FIFO, RANDOM, tree-PLRU |
| Rig | amber (pair-rig AXI4), amber_ace (onyx-rig ACE) |
| Write policy | wb_wa, wt_na (bring-up only) |

### 27.1.2	Grid Coverage

| Grid | Geometry | Policy | Rig | Purpose |
| --- | --- | --- | --- | --- |
| GATE | tiny | LRU | amber | fast gate-level sanity |
| FUNC | default | LRU, FIFO, RANDOM | amber | functional regression |
| FUNC | default | LRU | amber_ace | coherent-transaction sanity |
| FULL | default | all four | both rigs | nightly regression + cache_sim parity |
| FULL | tiny | LRU | both rigs | formal companion runs |

## 27.2	Formal Targets

SymbiYosys is scoped to the control layer at the tiny geometry:

- No stale data served after an external write (pending-fill bypass correctness).

- No deadlock or livelock under fair arbitration.

- Victim-buffer handoff correctness.

- amber_snoop_resp AC/CR/CD ordering.

## 27.3	Golden-Model Parity

For LRU, FIFO, and RANDOM, amber hit/miss/miss-class counts must match cache_sim at identical geometry and policy. tree-PLRU has no cache_sim golden model today; it is cross-checked by RTL self-checks and recorded as a future sim extension.

## 27.4	MonBus Coverage

Every event class in Chapter 4 must be visible on MonBus in simulation. The _monlite wrapper is present-vs-absent in the regression so the gate-cost delta is part of the evidence.

Last Updated: 2026-10-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 28	amber — Product Requirements (DRAFT)

Version: 0.5 (draft, 2026-10-06) Status: nine of the eleven decision rows record a name and a date; D3 (memory-side, now shaped by the two-top structure) and D11 (arrays, house primitives by rule) remain OPEN. A decision becomes DECIDED when it records a name and a date, the way the reed-solomon PRD does it. v0.2: D8 decided — *_monlite observation wrappers (never _mon on the measured paths). v0.3: D4 decided — snoop transport is ACE-shaped (AC/CD/CR) per the onyx D7 port contract, family decision 2026-10-05. The snoop-filter (directory-lite) half of D4 stays deferred and is reopened as onyx D3. v0.4: D2 decided (Sean, 2026-10-06) — CPU-side interface is a GAXI slave. House BFM/monitor coverage and skid/FIFO plumbing already exist, and STREAM can attach as first real consumer without an adapter. v0.5 (Sean, 2026-10-06): D1, D5, D6, D7, D9, D10 decided in one working session — geometry 32 KiB/64 B/4-way center + a tiny 16-set/2-way formal config; write-back + write-allocate; plain MESI on a 3-bit state field; replacement {LRU, tree-PLRU, FIFO, RANDOM} with LRU default (tree-PLRU the timing fallback, no sim golden model — future sim extension recorded); verification = Pattern-B grids + control-layer SymbiYosys at tiny geometry + cache_sim parity on LRU/FIFO/RANDOM + FSM oracles derived from the gem5 Ruby MESI_Two_Level SLICC tables; first consumer = TB masters bring-up, pair rig the gated deliverable, STREAM attach the documented future hook.

## 28.1	1. Purpose

A parameterised, blocking, snoopy MESI L1 data cache for the Sherpa research/teaching flow: correct against a software golden model, provable with SymbiYosys, observable through the MonBus monitor fabric, and characterised for FPGA cost like every other component in this repo. It is the first cache-ip IP and the deliberate baseline for jet, the lockup-free follow-on.

## 28.2	2. Background

A blocking cache freezes on a miss: the pending request (and everything behind it) waits for the fill to complete. That is poor for performance and excellent for correctness work — one outstanding transaction means no MSHR bookkeeping, no hit-under-miss hazards, no merge logic. MESI (Modified / Exclusive / Shared / Invalid) is the coherence layer on top: each line carries a state that says who may write it and who must be told when they hold a stale copy. Snoopy coherence broadcasts probes on a shared bus; every cache watches and answers.

Blocking and MESI are orthogonal, which is the design bet: amber nails the coherence protocol with trivial miss handling; jet keeps the protocol and upgrades the miss handling. The pair then answers the classic question with measured, on-board data: what does lockup-freedom actually buy?

Repo anchors the design must respect: resets on the ALWAYS_FF_RST macros, storage on the shared sdpram_core / gaxi FIFO primitives (no bespoke SRAM, no reset on memories), [DEPTH] array syntax, valid/ready streaming contracts, and the monitor/MonBus observation fabric from rtl/amba/.

## 28.3	3. Decisions that pick the design

| # | Decision | Candidates | What it drives |
| --- | --- | --- | --- |
| D1 | Cache geometry | DECIDED 2026-10-06 (Sean): 32 KiB / 64 B lines / 4 ways (128 sets) as the default center, fully elaboration-parameterised (4–32 KiB, 32–64 B, 2–8 ways), plus a reserved tiny config — 16 sets / 2 ways / 64 B — for SymbiYosys tractability. Board BRAM does not bind (~9 of 135 36Kb RAMs on Nexys A7 at the default), so geometry serves the DV matrix and the proofs, not the board | tag/data array dimensions, the whole DV matrix |
| D2 | CPU-side interface | DECIDED 2026-10-06 (Sean): GAXI slave. Chosen over the plain valid/ready native port and the AXI4 slave: house BFM/monitor coverage and skid/FIFO plumbing already exist, and STREAM can attach as first real consumer without an adapter. Must express single-beat reads, write-allocate fills, and (if write-through is chosen) store-without-allocate | front-end FSM, adapter surface, first consumer |
| D3 | Memory-side interface | OPEN: AXI4 read/write masters on the house axi4_master_rd/wr wrappers (stream/rapids pattern) vs GAXI vs a simple direct SRAM port for bring-up | fill/drain engines, burst behavior on misses |
| D4 | Snoop transport | DECIDED 2026-10-05 (Sean): ACE-shaped transport — amber implements the AC/CD/CR snoop channels exactly as onyx D7 defines them (family decision: cache-ip is ACE-shaped), staying bus-agnostic internally behind a thin adapter. Chosen over a custom dedicated snoop bus and over snooping on the AXI4 fabric directly; the ACE-lite-shaped candidate was set aside because a snoop responder must drive CR responses and CD data — ACE-Lite carries no snoop channels at all. The snoop-filter (directory-lite) sub-question stays deferred, reopened as onyx D3. | the coherence bus, formal surface, gate count |
| D5 | Write policy | DECIDED 2026-10-06 (Sean): write-back + write-allocate. Write-through/no-allocate deletes the coherence traffic the research exists to measure — dirty snoop transfers (PassDirty), WriteBack drains, onyx’s memory-read elision — and leaves MESI’s M state with no job; it stays available as a bring-up mode only | M-state logic, memory traffic, formal targets |
| D6 | MESI variant | DECIDED 2026-10-06 (Sean): plain MESI on a 3-bit state field. MOESI’s O-state is a later elaboration upgrade (encoding headroom reserved now, ~512 extra tag bits); its win — dirty→shared without a memory read — is a measurement for the pair rig, not a v1 requirement | state machine count, snoop response matrix |
| D7 | Replacement policy | DECIDED 2026-10-06 (Sean): policy set {LRU, tree-PLRU, FIFO, RANDOM}, default LRU. LRU/FIFO/RANDOM match the cache simulator exactly (POLICIES = ['LRU','FIFO','RANDOM']), so golden-model parity covers every policy both sides implement; tree-PLRU is carried as the timing-tight fallback — the one policy with no sim golden model, cross-checked by RTL self-checks today and a future cache_sim extension (recorded, not silently assumed). Policy stays an elaboration parameter so the sim↔RTL cross-check holds per policy | policy engine, cross-check fidelity |
| D8 | Observation | DECIDED 2026-10-04 (Sean): *_monlite wrappers, not the heavyweight _mon. MonBus taps for hits, misses (class split if the model supplies it), snoops, evictions, and state transitions, observed through the axi4_monlite / axil4_monlite family — same 128-bit MonBus packets, UNIT/AGENT ids, and host tooling as _mon, ~1/5 the monitor gates, and a drop-and-count policy that never stalls the port. Rationale for a research cache: the observer must not perturb what it measures — a _mon wrapper that gates the port when its tracking tables fill would corrupt the very miss-latency numbers amber and jet exist to produce. The events D8 lists are emitted as MonBus packets and tallied by the standard agents (monbus_tally_axil); heavyweight _mon stays available as a DV cross-check, never on the measured paths. | perf counters, board capture, DV scoreboards |
| D9 | Verification strategy | DECIDED 2026-10-06 (Sean): Pattern-B cocotb GATE/FUNC/FULL grids over the geometry × policy × rig matrix (ACE BFMs: cocotb-framework 1.2.0); SymbiYosys scoped to the control layer — amber_control, amber_snoop_resp, the pending-fill bypass, the victim-buffer handoff — proven at the tiny 16-set/2-way geometry; cache_sim trace-replay parity across LRU/FIFO/RANDOM. The Python reference model and the FSM oracles derive from the gem5 Ruby MESI_Two_Level SLICC tables in ../References/gem5-ruby-protocols/ — L1cache.sm’s enumerated transient states are the derivation source for the amber_pkg state encoding, and every stable and transient transition is cross-checked against an executable protocol spec, not prose | DV structure, formal areas, CI time |
| D10 | First consumer | DECIDED 2026-10-06 (Sean): standalone TB masters for bring-up; the gated deliverable is the pair rig — two ambers + shared memory, the coherence research rig. STREAM attach is documented as the future integration hook (D2’s GAXI choice is what makes it cheap later); the onyx rig is sequenced per onyx D10 — onyx waits behind the pair rig | integration scope, what “done” means |
| D11 | Data/tag arrays | OPEN (direction: shared primitives): sdpram_core-based tag/data stores + house FIFOs for pending/fill queues; FPGA attributes per ../../../../GLOBAL_REQUIREMENTS.md | area/timing headroom, the no-bespoke-SRAM rule |

## 28.4	4. Success criteria

- Golden-model parity. Given the same trace + policy + geometry, amber’s hit/miss/miss-class counts match cache_sim exactly across a regression trace suite.

- Proofs. SymbiYosys: no stale data is ever served after an external write; the coherence protocol cannot deadlock under fair arbitration.

- Observation. Every event class above visible on MonBus in simulation, and captured on at least one FPGA board.

- Cost. Synthesis/implementation numbers (LUT/FF/RAM, fmax) on Nexys A7 and/or Genesys 2, recorded in docs/ the way pumice characterised.

## 28.5	5. Non-goals

Non-blocking / lockup-free miss handling (jet), directory-based coherence, L2/L3 or multi-level inclusion policies, ECC on cache arrays, virtual memory and TLB interaction, multi-port CPU issue. Each is a future IP or a future decision, not an amber requirement.

## 28.6	6. Where the block sits

Research cache on the AMBA fabric: CPU-side slave (D2), memory-side AXI4 masters (D3), snoop port toward onyx / peer caches (D4 — ACE-shaped AC/CD/CR, decided 2026-10-05), MonBus observation (D8). Standalone-testable with TB masters before any consumer exists (D10).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 29	amber — Pre-Hardware Architecture Specification (Pre-HAS)

Version: 0.3 (draft, 2026-10-06) — superseded by the full HAS (v1.0, same day) once the PRD closed its rows (v0.5): the chapter book ratifies this sketch, and this file is kept as the decision-path record. Status: architecture sketch written while the PRD decisions are still OPEN. This document records no decisions. “PROPOSED” marks a working default put forward so the decision rows can be argued with concretely; a decision becomes DECIDED only when PRD.md records a name and a date. When the decisions close, each PROPOSED item here is either ratified into the full amber_has chapter book (the stream/andesite convention) or overridden and this file edited. v0.2 (2026-10-06): D2 landed in the PRD — CPU-side is a GAXI slave. F9 now answers probe-during-fill with a pending-fill bypass instead of an unbounded stall. Q1 answered — amber_fabric carries both rig attachments and the RIG elaboration parameter generate-selects. v0.3 (2026-10-06): Q1 re-answered by the owner — both rigs as two tops, amber (pair rig) and amber_ace (onyx rig), on a shared amber_core. No RIG parameter: the rig is whichever top you instantiate.

Governing documents: PRD.md (binding) · onyx PRD D7 (snoop-port contract) · jet PRD (what must stay constant) · GLOBAL_REQUIREMENTS.md · cache simulator (golden model)

## 29.1	1. Intent of this document

The PRD carries eleven decision rows and has closed three: D2 (CPU-side is a GAXI slave), D4 (snoop transport is ACE-shaped AC/CD/CR), and D8 (observation through *_monlite wrappers only). The Pre-HAS makes the open rows concrete: for each, it sketches the feature the way a working default would build it, so the architecture can be reviewed before any RTL exists. It is deliberately cheap to change — one file, no chapter skeleton yet.

## 29.2	2. Feature sketches

### 29.2.1	2.1 Core datapath — the blocking bet

- F1 — Blocking single-outstanding pipeline. A miss freezes the front end until the fill completes and the original request replays. No MSHR, no hit-under-miss, no merge logic — that deletion is the entire point; it is jet’s delta (jet J1–J5).

- F2 — Geometry as elaboration parameters (D1 OPEN). PROPOSED default profile 32 KiB / 64 B lines / 4 ways (128 sets) as the DV matrix centre of gravity; ranges 4–32 KiB, 32–64 B, 2–8 ways.

- F3 — Pluggable replacement (D7 OPEN). PROPOSED: true-LRU as the correctness reference (the cache simulator models it exactly), tree-PLRU as the FPGA-area variant; FIFO/random kept because the sim models them too. Policy is an elaboration parameter, never a code hack, so the sim↔RTL cross-check holds for every policy both sides implement.

- F4 — Write policy (D5 OPEN). PROPOSED: write-back + write-allocate — MESI’s M state earns its keep, and dirty-victim handling is where blocking simplicity pays. Write-through/no-allocate retained as a bring-up mode because it deletes the victim path entirely.

- F5 — Whole-line fills. One burst per fill; sub-line fill granularity is a non-goal. Burst length derives from LINE_BYTES / (BUS_WIDTH/8).

### 29.2.2	2.2 Coherence

- F6 — MESI state per line (D6 OPEN). PROPOSED plain MESI; MOESI’s O-state recorded as a later elaboration upgrade, not v1 — ownership’s win (dirty-transfer reduction between caches) is a measurement for the pair rig, not a v1 requirement. Insurance: allocate a 3-bit state field now (MESI needs 2; MOESI needs 3) so the upgrade is an encoding change, not an array rebuild.

- F7 — ACE snoop responder (D4 DECIDED — sketched, not re-decided). amber_snoop_resp: a thin adapter speaking raw ACE AC/CR/CD on the outside per the onyx D7 contract (IHI 0022 the definition) and driving a bus-agnostic probe interface inside. The cache core never contains ACE — only the adapter does; that is exactly why D4 was chosen (onyx, or a real ACE interconnect, can supersede the custom bus without touching cache RTL).

- F8 — Snoop/local arbitration — the culsans problem. Probes must not starve and CPU tag latency must stay bounded. PROPOSED: tag array on the dual-port sdpram_core — port A for CPU lookups, port B for snoop lookups; data array port A for CPU/fill, port B for snoop readout feeding

  - No arbitration stall in the common case; the control collision (miss FSM vs snoop FSM wanting the same array control port) resolved with snoop-priority round-robin inside amber_control.

- F9 — Probe-during-fill, answered by a bypass (direction set 2026-10-06: add the bypass). Blocking means a fill is usually in flight, and a probe against a pending line must not observe pre-fill state. The answer is not an unbounded stall: amber keeps one pending-fill register — single outstanding by construction — recording {line address, state being installed, data availability}. A probe matching it is answered from the register: CRRESP reflects the post-fill state, and required CD beats are forwarded from the fill stream as its beats arrive. A probe that misses it takes the normal tag path. The snoop response is bounded either way. The correctness target sharpens rather than relaxes: the bypass must be state-accurate — a probe against a pending fill observes exactly the post-fill state, never pre-fill — alongside the response-bound liveness target. jet reopens the generalisation (jet J4/J6): multiple outstanding fills turn the register into a pending-fill table, which is jet’s problem, not amber’s.

- F10 — CRRESP response matrix per MESI state, mirroring the RDS-DV ACE BFM default matrix as corrected in cocotb-framework 1.2.0 (MakeInvalid on M/O invalidates without data transfer — the bug the release fixed). The matrix ships as an explicit table in the full HAS.

### 29.2.3	2.3 Fabric side — where the two deployment rigs meet

- F11 — The two rigs. Pair rig (amber D10): two ambers + shared memory, snoops peer-to-peer on the D4 port, misses are plain AXI4 to memory. Onyx rig: misses become coherent transactions toward onyx; onyx owns the memory side and drives the snoops. One cache core, two fabric attachments.

- F12 — Fabric side: two tops on one core (Q1 answered 2026-10-06: do both, as two tops). The rig-agnostic cache core is shared; the fabric difference is confined to the top wrapper and its master stack:

  - amber — the pair-rig top. Misses are plain AXI4 to memory on the house axi4_master_rd/wr wrappers; snoop encodings never driven; the ACE snoop-responder port faces the peer cache.

  - amber_ace — the onyx-rig top. Fabric side is the existing rtl/amba/ace/axi4ace_master_rd / axi4ace_master_wr modules (ACE fields and RACK/WACK auto-pulse already in place), driven by a thin amber_ace_issue coherent-transaction issuer mapping cache events to the onyx D2 subset — read miss → ReadShared / ReadUnique (by desired exclusivity), write promotion → CleanUnique / MakeUnique, dirty eviction → WriteBack, clean eviction → Evict. Memory masters not exposed; the snoop-responder port faces onyx. No parameter selects between them: the rig is whichever top is instantiated. Both tops are compiled, and both are exercised in the DV matrix — amber against a peer + memory environment, amber_ace against an onyx environment (§5).

- F13 — Fill/drain engines (D3 OPEN, direction: house wrappers). Fill = one AR burst, whole line. Drain = a small victim buffer (PROPOSED depth 1)

  - one AW burst. Single outstanding by construction — no reorder buffers.

### 29.2.4	2.4 Observation (D8 DECIDED — sketched, not re-decided)

- F14 — amber_monlite wrapper. MonBus event set: hit (read/write), miss with class split (cold/capacity/conflict, when the golden model supplies the class), snoop (type, hit/miss, response class), eviction (clean/dirty), MESI state transitions, fill/drain start+end. Tallied by monbus_tally_axil exactly like every other component; the wrapper never stalls the observed port — drop-and-count. The observer’s own gate cost is measured present vs absent (F18), so “the observer does not perturb what it measures” is a quantified claim, not an assertion.

### 29.2.5	2.5 Verification & formal surface (D9 OPEN — direction proposed)

- F15 — Pattern-B cocotb grids. GATE/FUNC/FULL over the geometry × policy × rig matrix. Instruments already exist: the RDS-DV AXI4ACESnoopMaster BFM drives amber’s snoop port and AXI4ACEMasterRead/   Write drive the fabric side (cocotb-framework 1.2.0).

- F16 — Golden-model parity. Trace replay from bin/apps/cache_sim/ at identical geometry + policy; hit/miss/miss-class counts must match exactly (PRD success criterion 1). The replacement policy parameter (F3) is what keeps this check meaningful across policies.

- F17 — SymbiYosys targets (PRD criterion 2): no stale data served after an external write (F9 ordering); no deadlock/livelock under fair arbitration. Proof surfaces named now so the RTL is built proof-shaped: amber_control FSM, amber_snoop_resp, tag-array port sharing, victim buffer handoff.

### 29.2.6	2.6 Cost & characterisation (PRD criterion 4)

- F18 — Synthesis sweep. Geometry params × replacement policy on Nexys A7 and/or Genesys 2; LUT/FF/RAM/fmax recorded in docs/ the way pumice characterised. Includes the F14 observer-present vs absent delta.

## 29.3	3. Proposed block architecture

Proposed module set (amber_* prefix, per the cache-ip convention):

| Module | Role | PRD anchor |
| --- | --- | --- |
| amber | Pair-rig top: amber_core + plain AXI4 master stack + snoop responder toward the peer | D10 |
| amber_ace | Onyx-rig top: amber_core + ACE master stack + coherent-transaction issuer + snoop responder toward onyx | D10, F12 |
| amber_core | Shared rig-agnostic core, instantiated by both tops | — |
| amber_cpu_frontend | GAXI slave (D2 decided); request accept, response return, replay latch | D2 |
| amber_control | Blocking pipeline FSM: lookup → hit / miss → evict → fill → replay; owns tag-array control arbitration (F8) and the pending-fill bypass register (F9) | D1, D5 |
| amber_tag_array | sdpram_core tag + state store (3-bit state field, F6) | D11 |
| amber_data_array | sdpram_core data store | D11 |
| amber_repl | Pluggable replacement (policy elaboration parameter) | D7 |
| amber_victim | Dirty victim buffer + drain trigger | D5 |
| amber_fill / amber_drain | AXI4 read/write engines; plain axi4_master_rd/wr under amber, axi4ace_master_rd/wr under amber_ace | D3 |
| amber_snoop_resp | ACE AC/CR/CD adapter; bus-agnostic inside — faces the peer in amber, onyx in amber_ace | D4 |
| amber_ace_issue | Coherent-transaction issuer for the onyx D2 subset — amber_ace only | F12 |
| amber_monlite | Observation wrapper (D8) | D8 |

Miss data flow: frontend accept → tag lookup (port A) → miss → amber_repl picks victim → dirty? amber_victim + drain engine WB → fill engine AR burst → data array write + state install → original request replays from the frontend latch → response.

Snoop data flow: AC arrives at amber_snoop_resp → tag lookup (port B) → line state determines CRRESP and whether CD beats follow → response driven. Conflict with an in-flight fill resolves per F9.

Observation taps sit on control-FSM decision points, not the datapath — the F8 port split means the datapath need not be touched to observe it.

## 29.4	4. Interface sketches (shapes, not pinouts)

| Side | Sketch | Anchor |
| --- | --- | --- |
| CPU-side | DECIDED (PRD D2, 2026-10-06): GAXI slave — house BFM/monitor coverage and skid/FIFO plumbing already exist, and STREAM can attach as first real consumer without an adapter. Must express single-beat reads, write-allocate fills, byte enables. | D2 |
| Fabric-side | Two tops, shared core (F12): amber = plain AXI4 masters to memory + AC/CR/CD snoop responder toward the peer; amber_ace = axi4ace_master_rd/wr + amber_ace_issue toward the CCU (onyx D2 subset: ReadShared / ReadUnique / CleanUnique / MakeUnique / WriteBack / Evict) + the same snoop responder toward onyx. The rig is the top you instantiate — no mode parameter. | D3, D4 |
| Observation | MonBus through amber_monlite; no other debug surface. | D8 |
| Register block | None. Parameter-only IP; all visibility through MonBus tallies, unlike every other component’s APB CSR. Recorded explicitly so it is a decision and not an accident. | — |

## 29.5	5. Parameter sketch (illustrative — feeds the DV matrix)

| Parameter | PROPOSED default | Range / set |
| --- | --- | --- |
| SETS | 128 | 16–512 |
| WAYS | 4 | 2–8 |
| LINE_BYTES | 64 | 32–64 |
| REPL_POLICY | lru | lru, tree_plru, fifo, random |
| WRITE_POLICY | wb_wa | wb_wa, wt_na |
| VICTIM_DEPTH | 1 | 1–2 |
| BUS_WIDTH | 64 | 32–128 |

Rig is not a parameter: it is the top you instantiate — amber or amber_ace (F12). Derived: tag width = ADDR_WIDTH − $clog2(SETS) − $clog2(LINE_BYTES) + 3 (state field, F6); fill beats = LINE_BYTES / (BUS_WIDTH/8).

## 29.6	6. Open questions this sketch surfaces

| # | Question | Where it resolves |
| --- | --- | --- |
| Q1 | One mode-parameter amber_fabric vs separate adapters per rig — RESOLVED 2026-10-06: two tops on a shared amber_core — amber (pair rig) and amber_ace (onyx rig); no rig parameter, both compiled, both in the DV matrix (F12) | Full HAS implements it |
| Q2 | GAXI vs plain valid/ready CPU-side — RESOLVED by PRD D2 (2026-10-06, Sean): GAXI slave | amber_cpu_frontend RTL |
| Q3 | Geometry default vs Nexys A7 RAM budget (F2) | With the F18 cost sweep |
| Q4 | 3-bit state field headroom for MOESI (F6) | Cheap now, expensive later — recommend taking the insurance |
| Q5 | Probe-during-fill stall bound — RESOLVED 2026-10-06: add the pending-fill bypass (F9); formal targets become bypass state-accuracy + response bound | Full HAS implements it; PRD criterion 2 depends on it |
| Q6 | Replacement default: LRU reference vs tree-PLRU area (F3) | D7 closure, informed by F18 |

## 29.7	7. What the full HAS will add

The chapter book per the stream/andesite convention: front matter; purpose, conventions, definitions; use cases / key features / system context; graphviz block diagram; data flow; pin-level interfaces; performance targets; integration (clocking, reset, verification). The MAS follows when micro-architecture decisions land (array banking, pipeline staging). This Pre-HAS is the draft those books grow from — sections 2–6 above map onto chapters 2–5.

## 29.8	8. Cross-references

- amber PRD — binding requirements; decision record

- onyx PRD D7/D2 — snoop-port contract and coherent-transaction subset the ONYX rig speaks

- jet PRD — J-rows hold amber constant; nothing here may silently pre-decide jet (F9’s stall policy is amber’s; jet J4/J6 explicitly reopen it)

- References/AMBA_ACE_Interface_Definition.md and References/gem5-ruby-protocols/ — protocol definition and golden-protocol source

- RDS-DV cocotb-framework 1.2.0 ACE BFMs — the DV instruments F15 assumes

Last Updated: 2026-10-06 Maintained By: amber architecture (pre-decision working document)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 30	jet — Product Requirements (DRAFT)

Version: 0.1 (draft, 2026-10-04) Status: scaffold only — inherits every DECIDED row of amber’s PRD. Decided inherited rows so far: D8 observation (*_monlite, 2026-10-04) and D4 snoop transport (ACE-shaped AC/CD/CR per the onyx D7 contract, 2026-10-05). The rows below are the deltas jet must decide for itself; all are OPEN until they record a name and a date.

## 30.1	1. Purpose

The lockup-free (non-blocking) MESI snoopy L1 cache: amber with MSHRs. The research question is measurement — with coherence, arrays, and observation held constant, what do outstanding misses, hit-under-miss, and miss merging actually buy in latency and bandwidth, in simulation and on board?

## 30.2	2. Decisions jet adds to amber’s

| # | Decision | Candidates | What it drives |
| --- | --- | --- | --- |
| J1 | MSHR organization | OPEN: full associativity keyed by address vs direct-mapped by line index vs CAM of pending lines; capacity 2–16 entries as an elaboration parameter | the miss queue, merge logic, formal surface |
| J2 | Hit-under-miss scope | OPEN: hits to any resident line (default) vs hits only to lines not conflicting with a pending fill vs reads-only (writes still block) | hazard logic between fill port and hit port |
| J3 | Miss merging policy | OPEN: merge only exact-line aliases (default) vs merge line + offset range vs no merging (issue duplicate fetches, dedupe on return) | memory traffic, MSHR occupancy, fairness |
| J4 | Ordering guarantees | OPEN: CPU-side responses in-issue-order vs out-of-order with ID tags; snoop-during-miss handling (a probe against a pending line must see the post-fill state or stall cleanly) | response path complexity, protocol proofs, consumer expectations |
| J5 | Eviction under misses | OPEN: a dirty victim behind an outstanding fill must write back without deadlocking against the fill; victim buffer depth (0 = block like amber, 1–4 = decouple) | write-back path, the classic lockup-free deadlock corner |
| J6 | Congestion/fairness | OPEN: policy when MSHRs are full — stall new misses (strict) vs drop-and-retry vs throttle CPU-side accepts; plus snoop-vs-fill arbitration priority | starvation/liveness proofs |
| J7 | The measured claim | OPEN: define the experiment now — same trace suite + same board as amber, report hit latency distribution, miss concurrency, bandwidth, and MSHR occupancy from MonBus capture | what “jet is faster by X” must mean, reproducibly |

## 30.3	3. Success criteria (additions to amber’s)

- Correctness inheritance. Every amber proof and golden-model parity result still passes with MSHRs present (proofs are re-run, not assumed).

- The delta, measured. On the amber trace suite and at least one board: reported latency/bandwidth improvement with MonBus evidence, and the cache_sim cross-check extended so the model predicts jet’s concurrency numbers too (or the model gap is documented as a finding).

- The deadlock corner, proven. J5/J6 formal targets: no deadlock when MSHRs are full and dirty victims wait behind fills.

## 30.4	4. Non-goals (for jet, beyond amber’s)

Speculative prefetching, adaptive replacement, quality-of-service policies, multi-banked parallelism beyond what MSHRs imply. Each is a future IP or a later decision.

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 31	onyx — Product Requirements (DRAFT)

Version: 0.3 (draft, 2026-10-05) Status: decisions pending — this page records the questions and the candidates, not answers. A decision becomes DECIDED when it records a name and a date, the way the reed-solomon PRD does it. Until then every row below is OPEN. v0.2: D7 decided 2026-10-05 (Sean) — onyx owns the ACE-shaped AC/CD/CR snoop port; amber/jet adapt to it behind a thin adapter. Decided together with amber D4, so the gate between the two is resolved. The sequencing constraint that remains: onyx still waits behind amber’s pair rig (amber D10), its first consumer. v0.3: D10 direction tightened 2026-10-05 — the ACE BFM extension of the RDS-DV axi4 family is now specified concretely (layering, four classes, build order); the row stays OPEN. RDS-DV implementation filed as RTLDesignSherpa-DV issue #86.

## 31.1	1. Purpose

A snoop-based cache coherency unit (CCU) in the shape of Arm ACE: the manager that sits between the repo’s coherent caches and memory on the house AXI4 fabric. An attached cache (amber instance, jet instance) is an ACE master; it issues coherent transactions (ReadShared, ReadUnique, CleanUnique, MakeUnique, WriteBack, Evict) instead of managing peers directly. onyx receives those transactions, broadcasts a snoop (AC request) to the other caches, gathers their CR responses (DataTransfer / IsShared / PassDirty) and CD data, decides whether the transaction may be completed from cache-supplied data alone or memory must be read, and serialises same-address traffic. The research question is the manager side itself: what do fanout, gather, adjudication, and ordering cost — in gates, in latency, and in verification effort.

## 31.2	2. Background

ACE is AXI4 plus three snoop channels (AC, CR, CD) — and this repo already speaks AXI4 (skid buffers, GAXI FIFOs, axi4_master_rd/wr wrappers, Pattern-B BFMs). The natural-protocol argument: every piece of onyx’s data path reuses existing, proven plumbing; the new logic is the coherence decision layer and the serializer. The definition is Arm IHI 0022 (the ACE chapters); the worked example is pulp-platform/culsans (ETH/PlanV), whose two halves are exactly the reading list:

- culsans’ CVA6 coherent WB cache — how a cache becomes a snoop responder: the AC/CD/CR channels, how snoop tag lookups arbitrate with the core’s own tag port, and the MOESI-ish line states.

- ace_ccu_top — the manager: broadcast snoops on a coherent read/write, collect responses, decide whether memory is read, and serialise same-address transactions.

Treat culsans as a worked example, not a dependency (last push 2024, lightly maintained). Note also what onyx is not: it is not a directory, not an L2, and not a general NoC — amber’s deferred “snoop filter / directory-lite” question (amber D4) shows up here only as one candidate row (D3).

Repo anchors the design must respect: resets on the ALWAYS_FF_RST macros, no bespoke SRAM (shared sdpram_core / house FIFO primitives), [DEPTH] array syntax, valid/ready streaming contracts, MonBus observation through *_monlite wrappers only (amber D8’s decision — the observer must never stall the measured paths), filelists registry, and GLOBAL_REQUIREMENTS.

## 31.3	3. Decisions that pick the design

| # | Decision | Candidates | What it drives |
| --- | --- | --- | --- |
| D1 | Functional scope | OPEN: CCU only — N ACE slave-side ports toward caches, one AXI4 master toward memory, one ACE-Lite IO port (the culsans shape; recommended starting scope) vs CCU + address striping / multiple memory ports (interconnect territory; non-goal today) | block boundary, what “onyx done” means |
| D2 | ACE transaction subset | OPEN: research subset — ReadShared / ReadUnique / CleanUnique / MakeUnique / WriteBack / Evict (recommended) vs subset + barrier support vs full ACE incl. DVM | front-end decoder, BFM scope, conformance matrix |
| D3 | Snoop fanout | OPEN: broadcast to all attached caches (culsans shape, simplest, recommended for v1) vs directory-lite snoop filter as an elaboration parameter (amber D4’s deferred question, reopened here) vs snoop filter mandatory | fanout logic, gate count, the coherence-traffic measurements |
| D4 | Response gather | OPEN: fixed response window (all caches respond within N beats, presumed-miss after timeout — simple, needs a proven bound) vs tagged outstanding snoop IDs with per-transaction completion (exact, more bookkeeping) | gather buffer, formal surface, ACE conformance |
| D5 | Same-address serialization | OPEN: per-address scoreboard allowing interleaving of different addresses (recommended; the interesting manager) vs full in-order single outstanding (simplest, strangles concurrency; fine as a bring-up mode) | the serializer, the deadlock/liveness proof, jet J4 interactions |
| D6 | Memory-read elision | OPEN: full adjudication — read memory only when no cache supplied data and no PassDirty (correct ACE semantics; recommended) vs always-read simplification for bring-up | memory traffic, the measurable win onyx exists to quantify |
| D7 | Cache-side interface ownership | DECIDED 2026-10-05 (Sean): onyx defines and owns an AC/CD/CR-shaped snoop port; amber/jet stay bus-agnostic internally and add a thin adapter to it. Chosen over caches natively growing ACE ports — the cache cores stay unpinned from ACE except at the adapter, so onyx (or a real ACE interconnect, e.g. culsans’ ace_ccu_top) can supersede the custom bus without touching cache RTL. | the amber D4 ↔ onyx contract, who changes when ACE supersedes the custom bus |
| D8 | Domain / barrier rules | OPEN: single shareability domain, barriers ignored (recommended non-goal, recorded here so it is a decision and not an accident) vs minimal barrier ordering vs full DVM | scope honesty, later upgrade path |
| D9 | Observation | OPEN (direction: amber D8 pattern): *_monlite taps for coherent-txn start, snoop fanout per transaction, response gather completion, memory-read elision hit/miss, serialization stalls, and WB/Evict drains; tallied by monbus_tally_axil | perf counters, board capture, DV scoreboard |
| D10 | Verification strategy | OPEN (direction: Pattern-B cocotb gate/func/full. ACE BFM — an extension of the axi4 BFM family in RDS-DV, layered the way that family is built (analysis from inspecting CocoTBFramework/components/axi4/): (1) field configs — an AXI4ACEFieldConfigHelper adds ARSNOOP[3:0] / AWSNOOP[2:0] to the AR/AW field configs; (2) signal-map patterns — new axi4ace_* protocol patterns (decide there: framework-shipped vs. user-loadable, the signal_mapping_helper question); (3) transaction/packet layer — the six D2 transaction types as an enum, CRRESP as a bitfield (DataTransfer/Error/PassDirty/IsShared/WasUnique); (4) four BFM classes — AXI4ACEMasterRead / AXI4ACEMasterWrite as subclasses (snoop type on commands; the BFM drives RACK/WACK when playing master against an onyx-port DUT, since the RTL auto-pulse in axi4ace_master_rd/wr only covers DUT-as-master), AXI4ACESnoopSlave (new, no base: receives AC, drives CR/CD in issue order — snoops carry no ID — with programmable cache-state behavior producing the correct CRRESP; this is the golden-model hook that replays gem5 traces through amber), AXI4ACESnoopMaster (new: drives AC, gathers CR/CD in order — drives onyx’s fanout in DV); (5) compliance checker — in-order CR/CD, no snoop to the requesting master, CD-before-CDLAST, CRRESP validity per snoop type; (6) main-repo TB classes under bin/TBClasses/axi4ace/ per consumer. Build order: snoop-slave BFM first (amber’s pair rig needs it), snoop-master BFM lands with onyx; ships as an RDS-DV framework release the way the irq BFM did. Python golden model derived from the gem5 Ruby MESI_Two_Level spec in ../References/gem5-ruby-protocols/; the ACE subset contract is the in-repo definition doc; SymbiYosys proofs of same-address serialisation and no-deadlock under fair arbitration; culsans behaviours as informal cross-checks, not oracles) | DV structure, formal areas, CI time |
| D11 | Master count & profile | OPEN: N coherent cache masters 2–4 as an elaboration parameter (culsans uses 2–4 CVA6) vs fixed 2 for the research rig; line size / profile locked to amber’s geometry vs independently parameterised | array dimensions, DV matrix, the measured system’s realism |

## 31.4	4. Success criteria

- Three-block coherence. Two amber instances (or amber + jet) plus onyx form a coherent system that passes the golden-model parity suite amber D9 defines — the pair rig grown by one block, same traces, same policy space.

- IHI 0022 conformance on the D2 subset. A scoreboard checks the AC/CR/CD behaviour per transaction type: the CR bits (DataTransfer, IsShared, PassDirty) of every gather are correct for the transaction and the caches’ states.

- The serializer, proven. Formal: no two same-address coherent transactions proceed incompatibly; no deadlock under fair arbitration at N ∈ {2, 3, 4} masters; the D4 response-gather mechanism cannot livelock (a presumed-miss bound exists and is proven).

- Observation. Every D9 event class visible on MonBus in simulation and captured on at least one FPGA board.

- Cost. Synthesis/implementation numbers (LUT/FF/RAM, fmax) on Nexys A7 and/or Genesys 2, recorded in docs/ the way pumice characterised.

## 31.5	5. Non-goals

Cache storage in onyx (it is a manager, not an L2 — tempting because a CCU’s gather point is the natural place to cache; explicitly out), directory-based coherence (the D3 snoop-filter candidate is the closest this IP comes), ACE DVM and full barrier semantics (D8), cache stash/dealloc features, multi-CCU / multi-domain systems, and any CPU-side or upstream-master port beyond ACE-Lite IO. Each is a future IP or a future decision, not an onyx requirement.

## 31.6	6. Where the block sits

The fabric’s coherence point, on three kinds of port: cache-side, N ACE-facing ports accepting coherent transactions from amber/jet instances acting as ACE masters (D7, decided 2026-10-05: onyx owns this port’s ACE shape; caches adapt); memory-side, one AXI4 master toward the house memory path (D6 fixes when it is read); inter-cache, the snoop fanout — AC requests driven into each peer cache, CD snoop data and CR snoop responses returned. No CPU port, no register block beyond what observation needs (D9 tallies live in monbus_tally_axil like every other component). Standalone-testable with BFM caches long before amber exists: culsans’ ace_ccu_top demonstrates exactly that decoupling.

# 32	AMBA ACE — Working Interface Definition for cache-ip

Distilled 2026-10-05 for the cache-ip family, then verified clause-by-clause against IHI 0022H on 2026-10-05 (Arm’s direct PDF, linked in §7; fetched to /tmp for comparison, not stored in-tree). Verification confirmed every transaction type, channel payload, CRRESP bit, and the ACE-Lite channel exclusions; it caught and fixed two errors in the ordering rules (the original draft claimed ACE serializes snoops one-at-a-time and mandates CD-before-CR — ACE requires only per-channel ordering and permits multiple outstanding snoops; the stricter behavior is recorded above as our own adapter convention). This document is the in-repo definition of the ACE-shaped port contract referenced by amber D4 and onyx D7 (family decision 2026-10-05: cache-ip is ACE-shaped).

Normative reference: Arm, AMBA AXI and ACE Protocol Specification, IHI 0022 (ACE chapters). Available from Arm without charge (direct PDF in §7); Arm copyright, so deliberately not mirrored here. Where this document and IHI 0022 disagree, IHI 0022 wins — file a docs bug. Worked example: pulp-platform/culsans (ace_ccu_top, Solderpad license) — read its RTL after this page, not before.

## 32.1	1. What ACE is

ACE (AXI Coherency Extensions) is AXI4 extended so that multiple caching masters can share memory coherently. An ACE master is typically a CPU’s cache; an ACE-Lite master is a non-caching agent (DMA, IO) that wants coherent access without being snooped. Between the masters and memory sits a coherency manager — in this family, onyx — which receives coherent transactions, broadcasts snoops to the peer caches, gathers their responses, decides whether DRAM must be read, and serializes same-address traffic.

ACE adds three channels to AXI4. Everything else — AW/W/B/AR/R semantics, burst rules, handshake rules — is unchanged AXI4 and is not restated here.

## 32.2	2. Channel inventory

### 32.2.1	2.1 Coherent master → manager (front side, per master)

The five AXI4 channels, with a transaction-type field added to each address channel:

| Channel | New/changed payload | Notes |
| --- | --- | --- |
| AR + R | ARSNOOP[3:0] | read-side coherent transaction type |
| AW + W + B | AWSNOOP[2:0] | write-side coherent transaction type |

Data and response channels are plain AXI4. A coherent read (e.g. ReadShared) delivers its data on the ordinary R channel, sourced by the manager from a peer cache or from memory as the adjudication decides.

Full ACE master interfaces also carry two acknowledge handshakes outside the five channels: RACK (read acknowledge — master pulses after the last read data beat of each transaction) and WACK (write acknowledge — master pulses after the write response). ACE-Lite excludes both (IHI 0022H §D11.1). Our D2 subset needs no ordering semantics from them; the family convention is to auto-pulse them promptly at the adapter and record that choice in the module docs.

### 32.2.2	2.2 Manager → cache, cache → manager (snoop side, per cache)

Three new channels. Direction below is from the point of view of the cache (amber/jet); the manager (onyx) sees the mirror image.

| Channel | Dir | Payload | Meaning |
| --- | --- | --- | --- |
| AC — snoop address | in | ACADDR, ACSNOOP[3:0], ACPROT[2:0] | “What state does this line have in you, and what should you do about it?” |
| CR — snoop response | out | CRRESP[4:0] | the answer: will you transfer data, is the line shared, is dirty responsibility passing |
| CD — snoop data | out | CDDATA, CDLAST | the line’s data, when CRRESP said it would come |

Handshake rules mirror AXI4 (VALID/READY, no combinatorial dependency of READY on VALID). Ordering rules that matter (IHI 0022H §D3.8–§D3.9, §D5.2, §D6.2):

- CR and CD only after AC. The snoop response and any snoop data must only be asserted after the ACVALID/ACREADY handshake.

- In-order responses, in-order data. CR responses must be returned in the same order as the AC addresses; CD data likewise. The spec permits a cache to have multiple outstanding snoop transactions — serialization is not required by ACE, only per-channel ordering is.

- Family convention (decided with onyx D4): the amber/jet adapter completes a transaction’s CD beats before asserting its CR, so onyx’s gather sees one fully-closed response at a time. This is stricter than ACE requires and simpler to verify; it is our contract, not the spec’s.

- Snoop vs. local access arbitration. A snoop reads the tag array and may evict/change a line’s state, so it must arbitrate with the cache’s own CPU-side tag port. This arbitration point is the classic cache-side cost of becoming a snoop responder (culsans’ CVA6 cache shows one arrangement).

## 32.3	3. Transaction types (the onyx D2 research subset in bold)

Read side (ARSNOOP): ReadOnce, ReadShared, ReadUnique, ReadClean, ReadNotSharedDirty, CleanUnique, MakeUnique, plus barrier/DVM forms (out of scope, onyx D8).

Write side (AWSNOOP): WriteUnique, WriteLineUnique, WriteBack, WriteClean, Evict, plus barrier/DVM forms (out of scope, onyx D8).

The six bolded transactions are the subset onyx D2 recommends; the definition of the subset is: everything amber or jet needs to run MESI write-back coherence against a manager, and nothing that requires DVM or barrier semantics.

Intent of each subset member, one line each:

- ReadShared — fetch a line to read; other caches may keep Shared copies; the line arrives in S (or E if nobody shared it).

- ReadUnique — fetch a line to write; all other copies must be invalidated; if a peer held it dirty, its data (or DRAM’s) is supplied.

- CleanUnique — “I already hold the line; make my copy Unique.” No data transfer; peers with Shared copies drop them.

- MakeUnique — like CleanUnique but the requester does not need the data (it will overwrite the whole line); peers invalidate.

- WriteBack — retire a dirty line to memory; the manager must merge the data into DRAM (or hand it to a subsequent reader).

- Evict — retire a clean line; a hint: no data, no DRAM write needed.

## 32.4	4. The CRRESP decision bits

CRRESP is where a snooped cache answers. The four bits the manager’s adjudication logic consumes (bit 4, WasUnique, also exists per IHI 0022):

| Bit | Name | Meaning when set |
| --- | --- | --- |
| [0] | DataTransfer | “I will drive the line’s data on CD.” Required of a Modified responder (or any responder told to transfer). |
| [1] | Error | “My lookup had an error.” Propagated, not interpreted. |
| [2] | PassDirty | “The line is/was dirty and dirty responsibility moves to whoever receives the data” — set with DataTransfer when the responder passes dirty data, and on its own when the data is already moving elsewhere. The manager uses it to decide whether DRAM must be (re)written. |
| [3] | IsShared | “Another master (or I) may hold a copy.” Tells the manager the requester must not be given Exclusive/Unique state, and that DRAM-read elision is safe when data was supplied. |

The adjudication onyx D6 specifies falls straight out of these bits: a coherent read completes from cache-supplied data alone iff some CR carried DataTransfer; DRAM must be read iff none did and no PassDirty moved dirty responsibility; DRAM must be written iff PassDirty passed dirty data that the requester did not absorb.

## 32.5	5. ACE-Lite

ACE-Lite is the non-snooped half: a master may issue coherent transaction types but never receives AC, so it never drives CR/CD. onyx’s IO port (onyx D1) is ACE-Lite — this is why the earlier “ACE-lite-shaped snoop channel” candidate in amber D4 was set aside: a snoop responder must drive CR/CD, and ACE-Lite has no snoop channels at all.

## 32.6	6. What this family implements, and what it deliberately does not

In scope (amber D4 / onyx D2 subset, single shareability domain, onyx D8): the six transaction types of §3, the AC/CR/CD contract of §2, the CRRESP semantics of §4, broadcast snoop fanout (onyx D3 v1), same-address serialization (onyx D5).

Transport for this contract exists in rtl/amba/ace/ (added 2026-10-05): axi4ace_master_rd/wr and axi4ace_slave_rd/wr (front-side movers with ARSNOOP/AWSNOOP, auto-pulsed RACK/WACK on the masters), the axi4ace_snoop_slave / axi4ace_snoop_master AC/CR/CD movers, and rtl/amba/monitor/axi4ace_snoop_monitor_lite for snoop-side observation.

Out of scope, recorded as decisions and not accidents (onyx D8): DVM (distributed virtual memory) messages, barrier ordering, multiple shareability domains, stash/dealloc hints. The repo’s port contract is ACE-shaped — same channels, signals, payload fields, and ordering rules on the pins — without claiming whole-spec conformance; see onyx D7 for why “shaped” rather than “compliant.”

## 32.7	7. Sources

- Arm, AMBA AXI and ACE Protocol Specification, IHI 0022, ACE chapters — the definition. Current issue H (2020), direct PDF: https://developer.arm.com/-/media/Arm%20Developer%20Community/PDF/IHI0022H_amba_axi_protocol_spec.pdf (landing page: https://developer.arm.com/documentation/ihi0022/). Available from Arm without charge; Arm copyright — cited, not mirrored. Sections this document was verified against: D2.2 (channels), D3.8–D3.9 (signaling, dependencies), D3.20–D3.22 (CRRESP), D5 (snoop transactions), D6.2 (sequencing), D11.1 (ACE-Lite).

- pulp-platform/culsans — rtl/src/ace_ccu_top.sv and the CVA6 coherent cache: a working ACE CCU and a working snoop-responder cache. Worked example, not a dependency. https://github.com/pulp-platform/culsans

- A Primer on Memory Consistency and Cache Coherence (Sorin, Hill, Wood, 2nd ed., stored alongside this file) — the coherence-theory background the transaction types above instantiate.

- gem5 Ruby SLICC specs (gem5-ruby-protocols/, stored alongside) — executable transient-state tables for the same protocol family; the golden-model source material, phrased in messages rather than ACE signals.

# 33	gem5 Ruby protocol specs (reference extract)

Executable, state-table-style cache-coherence protocol specifications in gem5’s SLICC language, extracted verbatim from gem5 master at commit f5c5a6e390f55dd5984977815bf9d0bd05da6945 (2026-09-07) via sparse clone.

Two protocols, as requested for the cache-ip research library:

- MESI_Two_Level — classic snoopy MESI with a shared L2. The L1 cache controller (MESI_Two_Level-L1cache.sm) is the closest executable model to amber: every transient state (IS, IM, SM, S^I, M^I, …) between a stable MESI state and a pending response is enumerated as an explicit state with its own transition table. MESI_Two_Level-msg.sm is the message-type contract (the shape a D4 snoop transport must express); MESI_Two_Level-dir.sm + -L2cache.sm show the point where a passive memory-side agent has to make coherence-visible decisions.

- MOESI_CMP_directory — the directory-based contrast set: what changes when a home/directory agent (not a broadcast medium) owns coherence serialization. Useful for ruling D4 options in or out with a working executable spec instead of prose.

Why these exist here: the Primer’s MESI diagrams show only stable states; most coherence bugs live in the transient states. These files are the enumerated transient-state tables — the raw material for deriving amber’s amber_pkg state encoding and for building the Python reference model the DV scoreboard replays against (amber D9, jet J7 cross-checks).

License: each .sm/.slicc file carries its complete copyright and BSD-3-clause license header inline (ARM Limited; Mark D. Hill and David A. Wood) — redistribution terms are inside the files themselves. LICENSE is gem5’s root file, an unrendered BSD-3 template, included as-shipped.

Upstream: https://github.com/gem5/gem5 (src/mem/ruby/protocol/). This directory is a read-only extract; track upstream for changes.

# 34	Cache Associativity Simulator

A standalone, trace-driven cache simulator for exploring set count, way count, block size, and replacement policy. Written in vanilla HTML5/CSS/ES5-ish JavaScript with no build step; open index.html directly as file://.

## 34.1	Files

bin/apps/cache_sim/
  index.html          -- UI
  style.css           -- mobile-first styles
  js/model.js         -- pure simulation model (no DOM)
  js/app.js           -- DOM glue, generators, and guided example
  test/run_tests.js   -- zero-dependency TAP-ish node test runner
  README.md           -- this file

## 34.2	Model API

var CACHESIM = require('./js/model.js');

var result = CACHESIM.simulate(
  { sets: 4, ways: 1, blockSize: 1, policy: 'LRU' },
  [0x00, 0x10, 0x20, 0x00],
  1   // seed for RANDOM policy / reproducibility
);

simulate(config, addresses, seed) returns:

{
  config: { sets, ways, blockSize, policy },
  perAccess: [
    { addr, hit, setIndex, way, missClass }, ...
  ],
  totals: {
    hits, misses, hitRate,
    compulsory, capacity, conflict
  },
  occupancy: [ [tagOrNull, ...], ... ]   // sets x ways
}

missClass is one of 'hit', 'compulsory', 'capacity', or 'conflict'. Capacity classification is computed by running a shadow fully-associative cache with the same total size and policy.

Other model helpers:

- CACHESIM.validateConfig(config) -> {ok, errors}

- CACHESIM.parseAddressText(text) -> address array (NaN for bad lines)

- CACHESIM.generateSequential(length, start)

- CACHESIM.generateStrided(length, start, stride)

- CACHESIM.generateRandom(length, seed, maxAddr) (default maxAddr = 256)

## 34.3	Running tests

node bin/apps/cache_sim/test/run_tests.js

Exit code is 0 when all tests pass.

## 34.4	UI features

- Sliders for sets, ways, and block size; dropdown for policy.

- Address trace textarea plus one-click sequential, strided, and random generators.

- Big-number hit rate, totals table, per-access log, and per-set occupancy.

- Guided example button that loads a cyclic trace and compares direct-mapped vs 2-way set-associative behavior.

# 35	Global Hard Requirements for RTL Design Sherpa

Version: 1.0 Created: 2025-10-31 Purpose: Consolidated mandatory requirements extracted from all CLAUDE.md files

## 35.1	Overview

This document consolidates all MANDATORY requirements found across repository CLAUDE.md files. These are HARD REQUIREMENTS that must be followed - not suggestions or guidelines.

Sources Analyzed: - Root /CLAUDE.md - projects/components/CLAUDE.md - projects/components/dma-ip/rapids/CLAUDE.md (cited below as rapids/CLAUDE.md) - projects/components/dma-ip/stream/CLAUDE.md (cited below as stream/CLAUDE.md) - rtl/amba/CLAUDE.md (cited below as amba/CLAUDE.md) - rtl/common/CLAUDE.md - CocoTBFramework/CLAUDE.md - the DV framework guide, now maintained in the separate RTLDesignSherpa-DV repo (the framework is editable-installed into the venv)

## 35.2	Category 1: RTL Standards

### 35.2.1	1.1 Reset Handling (MANDATORY - NO EXCEPTIONS)

Requirement: ALL RTL in projects/components/ MUST use standardized reset macros

Pattern:

`include "reset_defs.svh"

`ALWAYS_FF_RST(clk, rst_n,
    if (`RST_ASSERTED(rst_n)) begin
        r_state <= IDLE;
    end else begin
        r_state <= w_next_state;
    end
)

NOT Allowed:

// WRONG - Manual reset handling
always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_state <= IDLE;
    end else begin
        r_state <= w_next_state;
    end
end

Applies to: ALL synthesisable RTL – rtl/ and projects/ alike. Source: projects/components/CLAUDE.md Rule #0, widened 2026-09-06 (Sean: “All flops should use the global macro”).

Reset is asynchronous on assertion. Always. ALWAYS_FF_RST used to be conditional on USE_ASYNC_RESET and defaulted to SYNCHRONOUS, so make lint (which set the define) and simulation/synthesis (which did not) disagreed about what the design was. The define is now a no-op; passing it is harmless and changes nothing. Deassertion still has to be synchronised externally.

The one exception, and it is structural: rtl/cdc/reset_sync.sv. It is active-HIGH, which the macro cannot express without RESET_ACTIVE_HIGH set globally, so it keeps a raw always_ff. Documented at the flop itself. If you add another, document it there too and say WHY – “it has always been like that” is not a reason.

“Macro-free fork” is not a reason. Two trees claimed it and neither survived contact with evidence. timing_characterization/rtl/asic_only/ was a 37-file duplicate of rtl/ differing ONLY in reset spelling; synthesised through Yosys + slang against ASAP7 it produced a byte-identical mapped netlist (396 cells, 97 flops, every cell type matching), and the flow reading it still passed -DUSE_ASYNC_RESET, a define meaningful only to the macro tree it was cloned from. formal/cdc/cdc_handshake/cdc_handshake_formal.sv claimed Yosys could not take the macro, while 92 other .sby units stage reset_defs.svh and read it with -Iincludes; its own .sby had simply never staged it, and it now proves PASS with the macro. The macros exist to serve ASIC and FPGA both – that is the whole point of them – so a second copy of a module to spell reset differently is duplication, not portability. Deleted 2026-09-07.

Status: rtl/ converted 2026-09-06 (45 blocks / 20 files; the claim that it was “already compliant” was wrong). projects/components/utility-ip/converters/ done as projects/components/utility-ip/converters BUG-007 (was CONV-009). Hand-written RTL still outstanding elsewhere under projects/, plus ~928 blocks in GENERATED files, which must be fixed at their generators and never in the emitted .sv.

### 35.2.2	1.2 FPGA Synthesis Attributes (MANDATORY)

Requirement: ALL memory arrays MUST have FPGA synthesis hints

Pattern:

`ifdef XILINX
    (* ram_style = "auto" *)
`elsif SYNTH_PRAGMA
    /* synthesis ramstyle = "AUTO" */
`endif
logic [DATA_WIDTH-1:0] mem [DEPTH];

Attributes: - Xilinx: ram_style = "auto" | "block" | "distributed" | "ultra" - Quartus: ramstyle = "AUTO" | "M20K" | "MLAB" | "logic"

Applies to: All memory array declarations Source: projects/components/CLAUDE.md Rule #1

### 35.2.3	1.3 Array Syntax Standard (MANDATORY)

Requirement: Use [DEPTH] NOT [0:DEPTH-1]

Pattern:

// CORRECT
logic [DATA_WIDTH-1:0] mem [DEPTH];

// WRONG
logic [DATA_WIDTH-1:0] mem [0:DEPTH-1];

Reason: Iverilog compatibility, cleaner syntax Applies to: All unpacked array declarations Source: projects/components/CLAUDE.md Rule #2

### 35.2.4	1.4 SRAM Module Standards (MANDATORY)

Requirement: SRAM modules DO NOT have reset ports

Pattern:

module simple_sram #(
    parameter int DATA_WIDTH = 32,
    parameter int DEPTH = 1024
) (
    input  logic clk,
    // NO rst_n port!
    // ... other ports
);

Reason: - Real SRAM chips don’t have reset pins - FPGA block RAM doesn’t reset contents - Initialize via writes during system init

Applies to: All SRAM/memory modules Source: projects/components/CLAUDE.md Rule #3

### 35.2.5	1.5 Reset Convention (STANDARD)

Requirement: ALWAYS use active-low asynchronous reset

Signal Names: - rst_n - Active-low reset (projects/components/) - aresetn - Active-low asynchronous reset (AMBA standard) - i_rst_n - Active-low reset (rtl/common/)

NOT Allowed: - rst (positive reset) - reset (ambiguous polarity)

Applies to: All RTL modules Source: Root /CLAUDE.md, rtl/common/CLAUDE.md Rule #3

## 35.3	Category 2: Testbench Architecture

### 35.3.1	2.1 TB Location Requirements (MANDATORY)

Requirement: Project-specific TB classes MUST be in project area, NOT framework

Directory Structure:

CORRECT:
projects/components/{name}/dv/tbclasses/{module}_tb.py

WRONG:
bin/TBClasses/{name}/{module}_tb.py

Import Pattern:

# Add repo root to path
import os, sys
repo_root = os.path.abspath(os.path.join(os.path.dirname(__file__), '../../../../../..'))
sys.path.insert(0, repo_root)

# Import from PROJECT AREA
from projects.components.dma_ip.rapids.dv.tbclasses.scheduler_tb import SchedulerTB

# Shared framework utilities
from TBClasses.shared.tbbase import TBBase
from CocoTBFramework.components.axi4.axi4_master import AXI4Master

Applies to: ALL project-specific testbenches Source: Root /CLAUDE.md, projects/components/CLAUDE.md Rule #4, rapids/CLAUDE.md Rule #0.1, stream/CLAUDE.md Rule #0.1, amba/CLAUDE.md Rule #0

### 35.3.2	2.2 Three Mandatory TB Methods (MANDATORY)

Requirement: EVERY testbench class MUST implement these three methods

Pattern:

class MyModuleTB(TBBase):
    async def setup_clocks_and_reset(self):
        """Complete initialization - starts clocks and performs reset"""
        await self.start_clock('clk', freq=10, units='ns')

        # Set config before reset (if needed)
        self.dut.cfg_param.value = initial_value

        # Reset sequence
        await self.assert_reset()
        await self.wait_clocks('clk', 10)
        await self.deassert_reset()
        await self.wait_clocks('clk', 5)

    async def assert_reset(self):
        """Assert reset signal"""
        self.dut.rst_n.value = 0  # Active-low

    async def deassert_reset(self):
        """Deassert reset signal"""
        self.dut.rst_n.value = 1

Why Required: - Consistency across all testbenches - Reusability for mid-test resets - Clear test structure and intent - Configuration control (some RTL needs signals set BEFORE reset)

Applies to: ALL testbench classes Source: Root /CLAUDE.md, rapids/CLAUDE.md Rule #0.5, stream/CLAUDE.md Rule #0.2, CocoTBFramework/CLAUDE.md Rule #3

### 35.3.3	2.3 TBBase Inheritance (MANDATORY)

Requirement: ALL testbench classes MUST inherit from TBBase

Pattern:

from TBClasses.shared.tbbase import TBBase

class MyModuleTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)
        # Testbench-specific initialization

What TBBase Provides: - Clock management (start_clock, wait_clocks) - Reset utilities (assert_reset, deassert_reset) - Logging (self.log) - Progress tracking (mark_progress) - Common utilities (convert_to_int, format_dec)

Applies to: ALL testbench classes Source: CocoTBFramework/CLAUDE.md Rule #2

### 35.3.4	2.4 Three-Layer Architecture (MANDATORY)

Requirement: Verification MUST be separated into three layers

Layers:

- Testbench (TB): projects/components/{name}/dv/tbclasses/{module}_tb.py

  - Infrastructure + BFMs

  - Clock/reset control

  - Base test methods

  - NO verification logic

- Scoreboard: bin/TBClasses/scoreboards/{protocol}/{module}_scoreboard.py

  - Verification logic

  - Expected vs actual comparison

  - Coverage collection

- Test: projects/components/{name}/dv/tests/test_{module}.py

  - Test intelligence

  - Imports TB class (does NOT define it)

  - Pytest wrappers

  - Scenario logic

NEVER Embed TB Classes in Test Files

Applies to: ALL verification code Source: amba/CLAUDE.md Rule #0, rapids/CLAUDE.md Rule #0.1, CocoTBFramework/CLAUDE.md Rule #4

### 35.3.5	2.5 Queue-Based Verification (PREFERRED)

Requirement: Use direct queue access for simple tests, NOT memory models

Pattern:

# CORRECT: Direct queue access
aw_pkt = self.aw_monitor._recvQ.popleft()
w_pkt = self.w_monitor._recvQ.popleft()

# Verify
assert aw_pkt.addr == expected_addr
assert w_pkt.data == expected_data

# WRONG: Memory model for simple test
memory_model = MemoryModel()
written_data = memory_model.read(addr, 4)  # Unnecessary complexity

When to Use: - Queue Access: Simple in-order tests, single-master, no overlap - Memory Models: Complex OOO scenarios, multi-master with address overlap

Applies to: Simple in-order verification scenarios Source: rapids/CLAUDE.md Scoreboard Pattern, CocoTBFramework/CLAUDE.md Rule #5

## 35.4	Category 3: Test Standards

### 35.4.1	3.1 Test Naming Convention (MANDATORY)

Requirement: Pytest function MUST match module name

Pattern:

# Module: axi4_dwidth_converter_wr.sv

CORRECT:
def test_axi4_dwidth_converter_wr(request, params):
    """Test for axi4_dwidth_converter_wr.sv"""
    ...

WRONG:
def test_converter(request, params):  # Too generic!
    ...

Why: Prevents pytest collection conflicts between related modules

Applies to: ALL pytest test functions Source: Root /CLAUDE.md

### 35.4.2	3.2 Test Pattern - CocoTB + Pytest (MANDATORY for projects/components/)

Requirement: Use two-tier pattern with cocotb_test_* prefix

Pattern:

# 1. CocoTB test functions - prefix with "cocotb_test_*"
@cocotb.test(timeout_time=100, timeout_unit="ms")
async def cocotb_test_basic(dut):  # ← cocotb_test_* prefix!
    """CocoTB test function - NOT collected by pytest"""
    tb = SimpleSRAMTB(dut)
    await tb.setup_clocks_and_reset()
    # ... test logic

# 2. Pytest wrapper - calls specific cocotb_test_* function
@pytest.mark.parametrize("addr_width, data_width", params)
def test_basic(request, addr_width, data_width):
    """Pytest wrapper - calls cocotb_test_basic"""
    run(
        # ... setup
        testcase="cocotb_test_basic",  # ← Explicitly specify
        # ... compilation args
    )

Why: - Pytest collects only wrappers, not CocoTB functions - Each wrapper calls ONE specific CocoTB function - Allows parameter sweeps at pytest level

Applies to: ALL tests in projects/components/ Source: Root /CLAUDE.md

### 35.4.3	3.3 100% Success Requirement (MANDATORY)

Requirement: ALL tests MUST achieve 100% success rate

Pattern:

# WRONG: Accepting partial success
assert success_rate >= 70  # "Good enough" - NO!

# CORRECT: Require 100% success
assert success_rate >= 100  # 100% required

Rationale: - RTL is deterministic - 100% success is achievable - Partial success indicates bugs or timing issues - Lower thresholds mask real problems - Tests must reliably catch regressions

Applies to: ALL tests Source: rapids/CLAUDE.md, CocoTBFramework/CLAUDE.md

### 35.4.4	3.4 Simulation Timestamps in Logs (MANDATORY)

Requirement: ALL test log messages MUST include simulation time

Pattern:

# CORRECT: Include simulation timestamp
self.log.info(f"@ {cocotb.utils.get_sim_time('ns')}ns: Beat {i}: data=0x{data:X}, s_ready={ready}, occupancy={occ}")

# WRONG: No simulation timestamp
self.log.info(f"Beat {i}: data=0x{data:X}, s_ready={ready}, occupancy={occ}")

Output Example:

2025-11-06 19:11:20,688 - INFO - @ 150ns: Beat 1: data=0xA000, s_ready=1, occupancy=0
2025-11-06 19:11:20,688 - INFO - @ 160ns: Beat 2: data=0xA001, s_ready=1, occupancy=1
2025-11-06 19:11:20,688 - INFO - @ 170ns: Beat 3: data=0xA002, s_ready=1, occupancy=2
2025-11-06 19:11:20,688 - INFO - @ 180ns: Beat 4 BLOCKED: s_ready=0, occupancy=3

Why Critical: - Correlate test events with waveform timing - Debug timing-sensitive failures - Understand cycle-accurate behavior - Match log messages to specific clock edges

What Counts as “Simulation Time”: - Use cocotb.utils.get_sim_time('ns') for nanoseconds - Use cocotb.utils.get_sim_time('ps') for picoseconds - Format: @ {time}{unit}: at START of message

Anti-Pattern:

# WRONG: Only Python logging timestamp
self.log.info(f"Beat {i} completed")
# Output: "2025-11-06 19:11:20,688 - INFO - Beat 1 completed"
# Problem: Can't correlate with waveform!

Applies to: ALL test log messages showing DUT behavior Priority: P0 - CRITICAL for debugging Source: User feedback 2025-11-06

## 35.5	Category 4: Framework Usage

### 35.5.1	4.1 Search Before Creating (MANDATORY)

Requirement: ALWAYS search existing modules/components BEFORE creating new

Search Commands:

# Search for existing RTL modules
find rtl/{subsystem}/ -name "*.sv" | xargs grep -l "keyword"

# Search for existing BFMs (CocoTBFramework, editable-installed from RTLDesignSherpa-DV)
find "$(python3 -c 'import CocoTBFramework,os;print(os.path.dirname(CocoTBFramework.__file__))')/components" -name "*.py" | head

# Search for existing TB classes
find bin/TBClasses/ -name "*_tb.py"

Decision Tree: - Exact match exists → Use it - Close match exists → Adapt with parameters - No match found → Document search, propose new - Didn’t search → STOP, go back and search

Applies to: RTL modules, BFMs, TB components Source: rtl/common/CLAUDE.md Rule #1, CocoTBFramework/CLAUDE.md Rule #1

### 35.5.2	4.2 BFM Usage for Protocols (MANDATORY for RAPIDS)

Requirement: MUST use framework BFMs for all protocol interfaces

BFM Selection: - Custom valid/ready: GAXI Master/Slave (CocoTBFramework/components/gaxi/) - AXI4: AXI4 Master/Slave (CocoTBFramework/components/axi4/) - APB: APB drivers (CocoTBFramework/components/apb/) - AXI-Stream: AXIS drivers (CocoTBFramework/components/axis4/)

(BFMs come from the RTLDesignSherpa-DV framework repo; shared TB classes in this repo are bin/TBClasses/)

NEVER: - Manually drive valid/ready handshakes - Create custom protocol drivers when framework has them

Applies to: All RAPIDS FUB-level tests Source: rapids/CLAUDE.md Rule #1

## 35.6	Category 5: Documentation

### 35.6.1	5.1 No Emojis in Technical Docs (MANDATORY)

Requirement: No emojis in technical specifications

Rationale: - Break PDF generation tools (LaTeX) - Appear unprofessional in formal documentation - Use plain text for all technical documentation

Exception: User explicitly requests emojis (rare)

Applies to: All technical documentation Source: Root /CLAUDE.md

## 35.7	Category 6: Project-Specific Requirements

### 35.7.1	6.1 Commit Messages Carry No Attribution Trailer

Requirement: A commit message is the change and nothing else. No Co-Authored-By:, no Claude-Session:, no “generated with” line, and no “Documentation and implementation support by Claude.”

Applies to: every commit in this repo, every area, no exceptions. Decided: 2026-08-31 (owner). Supersedes the former per-component attribution formats that lived here as 6.1/6.2 and in six CLAUDE.md files; those required a trailer and contradicted this. Practice already matched this rule – the twenty commits before it was written carried zero trailers – but the rule itself was recorded nowhere in the repo, so every session rediscovered the conflict. It is written down here to stop that.

Not enforced by tooling. No commit-msg hook exists; bin/hooks/pre-commit checks task IDs, declaration order, protocol families, scenario verdicts, doc examples and .sv parse. This one is on the author.

Rationale and the history: vault/handbook/authoring/commit-messages.md.

### 35.7.2	6.2 STREAM Tutorial Focus

Requirement: STREAM has intentional simplifications - DO NOT “fix” them

Intentional Simplifications: 1. Aligned addresses only - No alignment fixup logic 2. Length in beats - Not bytes or chunks 3. No circular buffers - Explicit chain termination only 4. No credit management - Simple transaction limits 5. Pure memory-to-memory - No network interfaces

When users ask “Can we add alignment fixup?” - Correct answer: “STREAM intentionally keeps addresses aligned for tutorial simplicity. For complex alignment, see RAPIDS.” - Wrong answer: “Sure, let me add alignment logic…” (defeats tutorial purpose!)

Applies to: STREAM component only Source: stream/CLAUDE.md Rule #0.1

## 35.8	Summary by Priority

### 35.8.1	P0 - Critical (Enforcement Required)

- Reset macro usage (projects/components/)

- TB location (project area, not framework)

- Three mandatory TB methods

- TBBase inheritance

- Three-layer architecture

- 100% test success requirement

### 35.8.2	P1 - High Priority (Strong Recommendation)

- FPGA synthesis attributes

- Array syntax standard

- SRAM module standards

- Test naming convention

- CocoTB + Pytest pattern

- Queue-based verification

### 35.8.3	P2 - Standard Practice (Best Practices)

- Search before creating

- BFM usage for protocols

- Active-low reset convention

- No emojis in technical docs

### 35.8.4	P3 - Project-Specific

- No attribution trailer on commit messages

- STREAM tutorial focus

## 35.9	Compliance Checklist

Use this checklist when creating new RTL or testbenches:

RTL Module: - [ ] Uses ALWAYS_FF_RST macro (projects/components/) - [ ] Memory arrays have FPGA attributes - [ ] Arrays use [DEPTH] syntax - [ ] SRAM modules have no reset port - [ ] Uses active-low reset (rst_n or aresetn)

Testbench Class: - [ ] Located in projects/components/{name}/dv/tbclasses/ - [ ] Inherits from TBBase - [ ] Implements setup_clocks_and_reset() - [ ] Implements assert_reset() - [ ] Implements deassert_reset() - [ ] Uses framework BFMs (not manual handshaking)

Test File: - [ ] Imports TB class from project area - [ ] CocoTB functions prefixed with cocotb_test_* - [ ] Pytest function matches module name - [ ] Uses testcase= to specify CocoTB function - [ ] Requires 100% success rate - [ ] Uses queue-based verification (for simple tests)

Documentation: - [ ] No emojis in technical specs - [ ] Commit message carries no attribution trailer

## 35.10	Enforcement

These requirements are enforced via:

- Code Review - All PRs reviewed for compliance

- Automated Tools:

  - bin/update_resets.py - Reset macro conversion

  - bin/audit_signal_naming_conflicts.py - Signal naming checks

- CI/CD Checks:

  - Verilator linting

  - Pytest test execution

  - Test success rate validation

- Documentation:

  - CLAUDE.md files per subsystem

  - PRD.md requirements per component

Non-Compliance: - PRs will be REJECTED if they violate P0 requirements - P1 violations require justification and approval - P2/P3 violations should be corrected but may not block PR

## 35.11	References

Main Documentation: - /CLAUDE.md - Repository-wide guidance - /PRD.md - Master requirements - projects/components/CLAUDE.md - Project area standards - CLAUDE.md in the RTLDesignSherpa-DV repo - DV framework patterns (shared TB classes: bin/TBClasses/)

Subsystem Documentation: - projects/components/dma-ip/rapids/CLAUDE.md - RAPIDS guidance - projects/components/dma-ip/stream/CLAUDE.md - STREAM guidance - rtl/amba/CLAUDE.md - AMBA subsystem guidance - rtl/common/CLAUDE.md - Common RTL library guidance

Tools: - bin/update_resets.py - Reset macro conversion - bin/audit_signal_naming_conflicts.py - Signal naming audit - bin/md_to_docx.py - Documentation generation

Version: 1.0 Last Updated: 2025-10-31 Maintained By: RTL Design Sherpa Project