RTL Design Sherpa

Bridge Micro-Architecture Specification 1.1

September 6, 2026

Table of Contents

1 Bridge Mas Index	37

2 Document Information	38

2.1 Bridge Micro-Architecture Specification	38

2.2 Revision History	39

2.3 Document Purpose	41

2.4 Intended Audience	42

2.5 Related Documents	43

3 Overview	44

3.1 Bridge Micro-Architecture	44

3.2 Key Capabilities	45

3.2.1 Multi-Protocol Support	45

3.2.2 Channel-Specific Masters	45

3.2.3 Automatic Converters	45

3.3 Architecture Summary	46

3.3.1 Block Organization	46

3.4 Document Organization	47

3.5 Related Documentation	48

4 2.1 Master Adapter	49

4.1 2.1.1 Purpose and Function	50

4.2 2.1.2 Block Diagram	51

4.2.1 Figure 2.1: Master Adapter Architecture	51

4.3 2.1.3 Channel Specialization	52

4.3.1 Read-Only Masters	52

4.3.2 Write-Only Masters	52

4.3.3 Read-Write Masters	52

4.4 2.1.3b Per-Port Monitor Wrappers (when use_monitor = true)	53

4.5 2.1.4 Skid Buffer Architecture	54

4.5.1 Registered Forward Path	54

4.5.2 Single-Cycle Backpressure Response	54

4.5.3 Pipeline Depth	54

4.6 2.1.5 Bridge ID Management	55

4.6.1 ID Width Calculation	55

4.6.2 ID Injection (Request Path)	55

4.6.3 ID Stripping (Response Path)	55

4.7 2.1.6 Protocol Normalization	57

4.7.1 Valid Address Ranges	57

4.7.2 Burst Constraints	57

4.7.3 Signal Defaults	57

4.8 2.1.6 Response Path Slave Selection Tracking	58

4.8.1 Problem: Stale Slave Select Decode	58

4.8.2 Solution: Per-Channel Response-Tracking FIFOs	58

4.9 2.1.7 Interface Specifications	59

4.9.1 External Master Interface (per master)	59

4.9.2 Internal Crossbar Interface (per master)	59

4.10 2.1.8 Resource Utilization	60

4.10.1 Per-Master Adapter Resources (Typical)	60

4.10.2 Scaling Considerations	60

4.11 2.1.9 Timing Characteristics	61

4.11.1 Latency	61

4.11.2 Throughput	61

4.11.3 Critical Paths	61

4.12 2.1.10 Configuration Parameters	62

4.12.1 Per-Master Parameters (from TOML/CSV)	62

4.12.2 Global Parameters (affect all adapters)	62

4.13 2.1.11 Debug and Observability	63

4.13.1 Recommended Debug Signals	63

4.13.2 Common Issues and Debug	63

4.14 2.1.12 Future Enhancements	64

4.14.1 Planned Features	64

4.14.2 Under Consideration	64

5 2.2 Slave Router	65

5.1 2.2.1 Purpose and Function	66

5.2 2.2.2 Block Diagram	67

5.2.1 Figure 2.2: Slave Router Architecture	67

5.3 2.2.3 Address Decoding Algorithm	68

5.3.1 Configuration-Based Address Maps	68

5.3.2 Decoding Priority	68

5.3.3 Range Checking Logic	68

5.3.4 Power-of-Two Optimization	69

5.4 2.2.4 Request Routing	70

5.4.1 AR Channel Routing	70

5.4.2 AW/W Channel Routing	70

5.4.3 Write Data Tracking FSM	70

5.5 2.2.5 Out-of-Range Handling	72

5.5.1 Detection	72

5.5.2 Error Response Generation	72

5.5.3 Debug Data Patterns	72

5.6 2.2.6 Default Slave Support	74

5.6.1 Configuration	74

5.6.2 Behavior	74

5.7 2.2.7 Address Aliasing	75

5.7.1 Multiple Slaves, Same Address	75

5.7.2 Intentional Aliasing Use-Cases	75

5.8 2.2.8 Configuration Parameters	76

5.8.1 Per-Router Parameters	76

5.8.2 Global Parameters	76

5.9 2.2.9 Resource Utilization	77

5.9.1 Per-Router Resources (Typical)	77

5.9.2 Scaling Considerations	77

5.10 2.2.10 Timing Characteristics	78

5.10.1 Decode Latency	78

5.10.2 Throughput	78

5.10.3 Critical Paths	78

5.11 2.2.11 Debug and Observability	79

5.11.1 Recommended Debug Signals	79

5.11.2 Common Issues and Debug	79

5.12 2.2.12 Verification Considerations	80

5.12.1 Address Decode Tests	80

5.12.2 Write Tracking Tests	80

5.12.3 OOR Error Tests	80

5.13 2.2.13 Performance Optimization	81

5.13.1 Techniques	81

5.14 2.2.14 Future Enhancements	82

5.14.1 Planned Features	82

5.14.2 Under Consideration	82

6 2.3 Crossbar Core	83

6.1 2.3.1 Purpose and Function	84

6.2 2.3.2 Block Diagram	85

6.2.1 Figure 2.3: Crossbar Core Architecture	85

6.3 2.3.3 Connectivity Matrix	86

6.3.1 Full N×M Crossbar	86

6.3.2 Request Path Multiplexing	86

6.3.3 Response Path Demultiplexing	86

6.4 2.3.4 Request Path Architecture	87

6.4.1 Per-Slave Request Arbitration	87

6.4.2 Request Multiplexers with Stable Address Gating	87

6.4.3 AW→W Burst Tracking FIFO	88

6.4.4 Backpressure Propagation	88

6.5 2.3.5 Response Path Architecture	90

6.5.1 Bridge ID Extraction	90

6.5.2 CAM-Based Routing (Optional)	90

6.5.3 Response Demultiplexers	90

6.5.4 Multi-Slave Response Merging	91

6.6 2.3.6 AXI Ordering Requirements	92

6.6.1 Read-After-Write (RAW) Ordering	92

6.6.2 Write-After-Write (WAW) Ordering	92

6.6.3 Out-of-Order (OOO) Completion	92

6.7 2.3.7 Monitor Aggregation at Bridge Top (when variants includes mon)	93

6.8 2.3.8 Resource Utilization	94

6.8.1 Crossbar Core Resources	94

6.8.2 Scaling with Masters and Slaves	94

6.8.3 Optimization Techniques	94

6.9 2.3.8 Timing Characteristics	95

6.9.1 Latency	95

6.9.2 Throughput	95

6.10 2.3.9 Critical Paths	96

6.10.1 Common Critical Paths	96

6.10.2 Mitigation Strategies	96

6.11 2.3.10 Configuration Parameters	97

6.11.1 Crossbar Parameters (from TOML)	97

6.12 2.3.11 Debug and Observability	98

6.12.1 Recommended Debug Signals	98

6.12.2 Performance Counters	98

6.13 2.3.12 Common Issues and Debug	99

6.14 2.3.13 Verification Considerations	100

6.14.1 Functional Tests	100

6.14.2 Corner Cases	100

6.14.3 Protocol Compliance	100

6.15 2.3.14 Future Enhancements	101

6.15.1 Planned Features	101

6.15.2 Under Consideration	101

7 2.4 Arbitration	102

7.1 2.4.1 Purpose and Function	103

7.2 2.4.2 Arbitration Architecture	104

7.2.1 Per-Slave, Per-Channel Arbiters	104

7.3 2.4.3 Round-Robin Arbitration	105

7.3.1 Algorithm	105

7.3.2 Implementation	105

7.3.3 Characteristics	106

7.4 2.4.4 Fixed-Priority Arbitration	107

7.4.1 Algorithm	107

7.4.2 Implementation	107

7.4.3 Characteristics	107

7.4.4 Starvation Prevention	107

7.5 2.4.5 Weighted Arbitration	109

7.5.1 Algorithm	109

7.5.2 Implementation	109

7.5.3 Characteristics	110

7.6 2.4.6 Burst Handling	111

7.6.1 Grant Locking	111

7.6.2 Implementation	111

7.6.3 Fairness Considerations	111

7.7 2.4.7 Resource Utilization	113

7.7.1 Per-Arbiter Resources	113

7.7.2 Scaling with Master Count	113

7.8 2.4.8 Timing Characteristics	114

7.8.1 Arbitration Latency	114

7.8.2 Critical Paths	114

7.9 2.4.9 Configuration Parameters	115

7.9.1 Arbiter Configuration (TOML)	115

7.10 2.4.10 Debug and Observability	116

7.10.1 Recommended Debug Signals	116

7.10.2 Common Issues and Debug	116

7.11 2.4.11 Verification Considerations	117

7.11.1 Test Scenarios	117

7.11.2 Corner Cases	117

7.12 2.4.12 Performance Impact	118

7.12.1 Arbiter Choice Impact	118

7.12.2 Arbiter Efficiency	118

7.13 2.4.13 Future Enhancements	119

7.13.1 Planned Features	119

7.13.2 Under Consideration	119

8 2.5 ID Management	120

8.1 2.5.1 Purpose and Function	121

8.2 2.5.2 Bridge ID Concept	122

8.2.1 Problem Statement	122

8.2.2 Solution: Bridge ID Injection	122

8.3 2.5.3 Block Diagram	123

8.3.1 Figure 2.5: ID Management Architecture	123

8.4 2.5.4 Bridge ID Width Calculation	124

8.4.1 Formula	124

8.4.2 Examples	124

8.4.3 ID Width Growth	124

8.5 2.5.4a Multi-Master OR-Merge Gating	125

8.6 2.5.5 ID Injection (Request Path)	126

8.6.1 Read Address Channel (AR)	126

8.6.2 Write Address Channel (AW)	126

8.6.3 Injection Timing	126

8.7 2.5.6 ID Extraction (Response Path)	127

8.7.1 Read Data Channel (R)	127

8.7.2 Write Response Channel (B)	127

8.8 2.5.7 Content Addressable Memory (CAM)	128

8.8.1 When to Use CAM	128

8.8.2 CAM Structure	128

8.8.3 CAM Operations	128

8.9 2.5.8 CAM Implementation	129

8.9.1 Parallel CAM (Fast, Resource-Intensive)	129

8.9.2 Sequential CAM (Slow, Resource-Efficient)	129

8.10 2.5.9 Outstanding Transaction Limits	131

8.10.1 Configuration	131

8.10.2 Enforcement	131

8.10.3 Sizing Guidelines	131

8.11 2.5.10 Resource Utilization	132

8.11.1 ID Injection/Extraction Only	132

8.11.2 With CAM	132

8.11.3 Scaling	132

8.12 2.5.11 Timing Characteristics	133

8.12.1 ID Injection Latency	133

8.12.2 ID Extraction/Routing Latency	133

8.12.3 End-to-End Impact	133

8.13 2.5.12 Configuration Parameters	134

8.13.1 ID Management Configuration (TOML)	134

8.14 2.5.13 Debug and Observability	135

8.14.1 Recommended Debug Signals	135

8.14.2 Performance Counters	135

8.15 2.5.14 Common Issues and Debug	136

8.16 2.5.15 Verification Considerations	137

8.16.1 Test Scenarios	137

8.17 2.5.16 Future Enhancements	138

8.17.1 Planned Features	138

8.17.2 Under Consideration	138

9 2.6 Width Conversion	139

9.1 2.6.1 Purpose and Function	140

9.2 2.6.2 Internal Data Width	141

9.2.1 64-Bit Standard	141

9.2.2 Width Conversion Locations	141

9.3 2.6.3 Block Diagram	142

9.3.1 Figure 2.6: Width Conversion Architecture	142

9.4 2.6.4 Upsizing (Narrow to Wide)	143

9.4.1 Overview	143

9.4.2 Write Upsizing	143

9.4.3 Read Upsizing	143

9.4.4 Strobe Mapping (Upsizing)	144

9.5 2.6.5 Downsizing (Wide to Narrow)	145

9.5.1 Overview	145

9.5.2 Write Downsizing	145

9.5.3 Read Downsizing	145

9.5.4 Strobe Mapping (Downsizing)	145

9.6 2.6.6 Address Alignment	147

9.6.1 Address Adjustment for Width	147

9.6.2 Unaligned Access Handling	147

9.7 2.6.7 Burst Length Adjustment	148

9.7.1 Length Calculation	148

9.7.2 Odd Burst Lengths	148

9.8 2.6.8 Resource Utilization	149

9.8.1 Per-Converter Resources	149

9.8.2 Scaling	149

9.9 2.6.9 Timing Characteristics	150

9.9.1 Latency	150

9.9.2 Throughput	150

9.10 2.6.10 Configuration Parameters	151

9.10.1 Width Conversion Configuration (TOML)	151

9.11 2.6.11 Debug and Observability	152

9.11.1 Recommended Debug Signals	152

9.11.2 Common Issues and Debug	152

9.12 2.6.12 Verification Considerations	153

9.12.1 Test Scenarios	153

9.13 2.6.13 Performance Considerations	154

9.13.1 Conversion Overhead	154

9.13.2 Optimization Strategies	154

9.14 2.6.14 Future Enhancements	155

9.14.1 Planned Features	155

9.14.2 Under Consideration	155

10 2.7 Protocol Conversion	156

10.1 2.7.1 Purpose and Function	157

10.2 2.7.2 Supported Protocols	158

10.2.1 Current Support (Phase 2)	158

10.2.2 Current Limitation: AXI4-Lite Conversion	158

10.2.3 Future Support (Phase 2+)	158

10.3 2.7.3 Conversion Architecture Overview	159

10.3.1 Figure 2.7.1: Protocol Conversion Architecture	159

10.4 2.7.4 Block Diagram	160

10.4.1 Figure 2.7: Protocol Conversion Architecture	160

10.5 2.7.5 AXI4-Lite to AXI4 Conversion (Master-Side)	161

10.5.1 AXI4-Lite Protocol Overview	161

10.5.2 Conversion Requirements	161

10.5.3 Signal Mapping	161

10.5.4 Implementation	163

10.5.5 Resource Utilization	165

10.5.6 Performance Impact	165

10.5.7 Configuration	166

10.5.8 Common Issues and Debug	166

10.6 2.7.6 AXI4 to APB Conversion (Slave-Side)	167

10.6.1 APB Protocol Overview	167

10.6.2 APB Signals	167

10.6.3 APB State Machine	167

10.6.4 Read Transaction Conversion	168

10.6.5 Write Transaction Conversion	168

10.6.6 Burst Handling	169

10.6.7 Response Mapping	169

10.7 2.7.7 Implementation	170

10.7.1 AXI4-to-APB Converter FSM	170

10.7.2 Address Generation	171

10.8 2.7.8 Resource Utilization	172

10.8.1 APB Converter Resources	172

10.8.2 Scaling	172

10.9 2.7.9 Timing Characteristics	173

10.9.1 Latency	173

10.9.2 Throughput	173

10.10 2.7.10 Configuration Parameters	174

10.10.1 Protocol Conversion Configuration (TOML)	174

10.11 2.7.11 Debug and Observability	175

10.11.1 Recommended Debug Signals	175

10.11.2 Common Issues and Debug	175

10.12 2.7.12 Verification Considerations	176

10.12.1 Test Scenarios	176

10.13 2.7.13 Performance Considerations	177

10.13.1 When to Use APB	177

10.13.2 APB vs. AXI4 Comparison	177

10.14 2.7.14 Mixed Protocol Bridges	178

10.14.1 Example Configuration	178

10.14.2 Routing Optimization	178

10.15 2.7.15 Master-Side vs Slave-Side Conversion	179

10.15.1 Comparison	179

10.15.2 When to Use Each	179

10.16 2.7.16 Generator-Emitted Conversion Shims	180

10.16.1 AXI4 to AXI4-Lite Conversion	180

10.16.2 AXI4 to APB Conversion	180

10.16.3 Master-Side AXIL→Wider-Slave Alignment	180

10.17 2.7.17 Future Protocol Support	182

10.17.1 Planned Features	182

10.17.2 Under Consideration	182

10.18 2.7.18 Best Practices	183

10.18.1 Design Recommendations	183

10.18.2 Performance Tips	183

11 2.8 Response Routing	184

11.1 2.8.1 Purpose and Function	185

11.2 2.8.2 Block Diagram	186

11.2.1 Figure 2.8: Response Routing Architecture	186

11.3 2.8.3 Stable Response Routing via FIFO-Tracked Slave Select	187

11.3.1 Architecture: FIFO-Tracked Master Routing	187

11.3.2 BID Extraction	187

11.3.3 CAM-Based Extraction	188

11.4 2.8.4 Response Demultiplexing	189

11.4.1 Per-Master Response Paths	189

11.4.2 Demux Implementation	189

11.5 2.8.5 Multi-Slave Response Merging	190

11.5.1 Problem Statement	190

11.5.2 Response Arbitration	190

11.5.3 Response Buffering	190

11.6 2.8.6 Independent Write-Tracker FIFO (W-Channel Deadlock Fix)	192

11.7 2.8.7 Backpressure Management	193

11.7.1 Ready Signal Routing	193

11.7.2 Stall Conditions	193

11.8 2.8.7 Response Path Latency	194

11.8.1 End-to-End R Channel	194

11.8.2 End-to-End B Channel	194

11.9 2.8.8 Error Response Routing	195

11.9.1 Slave Error Responses	195

11.9.2 Out-of-Range Error Responses	195

11.10 2.8.9 Resource Utilization	196

11.10.1 Response Router Resources	196

11.10.2 Scaling	196

11.11 2.8.10 Configuration Parameters	197

11.11.1 Response Routing Configuration (TOML)	197

11.12 2.8.11 Debug and Observability	198

11.12.1 Recommended Debug Signals	198

11.12.2 Performance Counters	198

11.13 2.8.12 Common Issues and Debug	199

11.14 2.8.13 Verification Considerations	200

11.14.1 Test Scenarios	200

11.15 2.8.14 Performance Optimization	201

11.15.1 Techniques	201

11.16 2.8.15 Advanced Features	202

11.16.1 Response Reordering (Future)	202

11.16.2 Response Coalescing (Future)	202

11.16.3 Response QoS (Future)	202

11.17 2.8.16 Timing Considerations	203

11.17.1 Critical Paths	203

11.17.2 Optimization Strategies	203

11.18 2.8.17 Future Enhancements	204

11.18.1 Planned Features	204

11.18.2 Under Consideration	204

12 2.9 Error Handling	205

12.1 2.9.1 Purpose and Function	206

12.2 2.9.2 Error Categories	207

12.2.1 Transaction Errors	207

12.2.2 System Errors	207

12.3 2.9.3 Block Diagram	208

12.3.1 Figure 2.9: Error Handling Architecture	208

12.4 2.9.4 Out-of-Range Address Handling	209

12.4.1 Detection	209

12.4.2 Error Response Generation	209

12.4.3 Configurable Data Patterns	210

12.5 2.9.5 Protocol Violation Handling	211

12.5.1 Common Violations	211

12.5.2 Protocol Checker Implementation	211

12.6 2.9.6 Timeout Detection	213

12.6.1 Per-Transaction Watchdog	213

12.6.2 Timeout Configuration	213

12.7 2.9.7 Error Logging and Reporting	215

12.7.1 Error Status Register	215

12.7.2 Error History Buffer	215

12.7.3 Error Interrupts	216

12.8 2.9.8 Resource Utilization	217

12.8.1 Error Handling Resources	217

12.9 2.9.9 Configuration Parameters	218

12.9.1 Error Handling Configuration (TOML)	218

12.10 2.9.10 Debug and Observability	219

12.10.1 Recommended Debug Signals	219

12.11 2.9.11 Common Issues and Debug	220

12.12 2.9.12 Verification Considerations	221

12.12.1 Test Scenarios	221

12.13 2.9.13 Recovery Mechanisms	222

12.13.1 Automatic Recovery	222

12.13.2 Manual Recovery	222

12.14 2.9.14 Safety and Reliability	223

12.14.1 Error Containment	223

12.14.2 Fail-Safe Behavior	223

12.14.3 Error Injection (Debug/Test)	223

12.15 2.9.15 Future Enhancements	224

12.15.1 Planned Features	224

12.15.2 Under Consideration	224

13 AMBA5 Boundary and Native Sideband	225

13.1 Boundary Wrappers	226

13.2 Native Sideband Through the Structs (A5-2)	227

13.3 Connectivity Gating (poison, atomic)	228

13.4 Atomic Filter (A5-3a)	229

13.5 APB5 Slaves (A5-3c)	230

13.6 Verification Anchors	231

13.6.1 Bridge Arbiter Finite State Machines	231

13.6.2 Figure 5.3: AW Arbiter FSM	232

13.6.3 Figure 5.4: AR Arbiter FSM	233

14 Transaction Tracking FSMs	240

14.1 Overview	240

14.2 Write Data Tracking FSM	241

14.2.1 Purpose	241

14.2.2 States	241

14.2.3 Figure 3.1: Write Data Tracking FSM	241

14.2.4 Implementation	241

14.3 Response Pending Counter	243

14.3.1 Purpose	243

14.3.2 Implementation	243

14.4 Burst Counter FSM	244

14.4.1 Purpose	244

14.4.2 States	244

14.4.3 Implementation	244

14.5 Related Documentation	245

15 CAM Architecture	246

15.1 Overview	246

15.2 CAM Purpose	247

15.2.1 Transaction Tracking	247

15.2.2 Response Routing	247

15.3 CAM Structure	248

15.3.1 Figure 4.1: CAM Entry Format	248

15.3.2 CAM Sizing	248

15.3.3 Example Configuration	248

15.4 CAM Operations	249

15.4.1 Insertion (AR/AW Acceptance)	249

15.4.2 Lookup (R/B Response)	249

15.4.3 Deletion (Response Complete)	249

15.5 Implementation Options	250

15.5.1 Distributed RAM CAM	250

15.5.2 Block RAM CAM	250

15.6 CAM Overflow Handling	251

15.6.1 Prevention	251

15.6.2 Error Response	251

15.7 Related Documentation	252

16 ID Tracking Tables	253

16.1 Overview	253

16.2 Table Structure	254

16.2.1 Per-Slave ID Table	254

16.2.2 Figure 4.2: ID Table Structure	255

16.2.3 Table Sizing	255

16.3 ID Extension	256

16.3.1 Master-Side Extension	256

16.3.2 Slave-Side Presentation	256

16.4 ID Extraction	257

16.4.1 Response Parsing	257

16.5 Table Operations	258

16.5.1 Allocation (AR/AW Phase)	258

16.5.2 Lookup (R/B Phase)	258

16.5.3 Deallocation (Response Complete)	258

16.6 Multi-ID Considerations	259

16.6.1 Same External ID, Different Masters	259

16.6.2 Same Extended ID (Error)	259

16.7 Resource Utilization	260

16.7.1 Table Resources	260

16.8 Related Documentation	261

17 Width Converters	262

17.1 Overview	262

17.2 Upsize Converter	263

17.2.1 Purpose	263

17.2.2 Figure 5.1: Upsize Converter (64-bit to 512-bit)	263

17.2.3 Implementation	263

17.2.4 Burst Length Conversion	264

17.3 Downsize Converter	265

17.3.1 Purpose	265

17.3.2 Figure 5.2: Downsize Converter (512-bit to 64-bit)	265

17.3.3 Implementation	265

17.4 Strobe Handling	267

17.4.1 Upsize Strobe Packing	267

17.4.2 Downsize Strobe Splitting	267

17.5 Resource Utilization	268

17.5.1 Upsize Converter (64 to 512)	268

17.5.2 Downsize Converter (512 to 64)	268

17.6 AXIL-to-Wider-Slave Master-Side Alignment Converters	269

17.6.1 Use Case Example	269

17.6.2 Architecture	269

17.6.3 Modules	269

17.7 Related Documentation	271

18 APB Converters	272

18.1 Overview	272

18.2 Conversion Requirements	273

18.2.1 Protocol Differences	273

18.2.2 Conversion Strategy	273

18.3 Write Conversion	274

18.3.1 Figure 5.3: AXI4 Write to APB Write	274

18.3.2 State Machine	274

18.4 Read Conversion	276

18.4.1 Figure 5.4: AXI4 Read to APB Read	276

18.4.2 Read State Machine	276

18.5 Burst Handling	277

18.5.1 Burst to Single Conversion	277

18.5.2 Address Calculation	277

18.6 Error Handling	278

18.6.1 APB Error to AXI4 Response	278

18.6.2 Error Propagation	278

18.7 Performance Considerations	279

18.7.1 Throughput Impact	279

18.7.2 Latency Impact	279

18.8 Resource Utilization	280

18.8.1 APB Converter Resources	280

18.9 Related Documentation	281

19 Module Structure	282

19.1 Generated RTL Overview	282

19.2 Top-Level Module	283

19.2.1 Module Declaration	283

19.2.2 Port Organization	283

19.3 Internal Structure	284

19.3.1 Component Instantiation	284

19.4 Generated Variants	286

19.5 Signal Naming Convention	287

19.5.1 Master-Side Signals (External)	287

19.5.2 Slave-Side Signals (External)	287

19.5.3 Internal Signals	287

19.6 Generated File Structure	288

19.6.1 Single-File Output	288

19.6.2 Multi-File Output (Optional)	288

19.7 Parameterization	289

19.7.1 Compile-Time Parameters	289

19.7.2 Per-Port Parameters	289

19.8 Related Documentation	290

20 Signal Naming	291

20.1 Naming Convention Overview	291

20.2 External Port Naming	292

20.2.1 Pattern	292

20.2.2 Components	292

20.2.3 Master Port Examples	292

20.2.4 Slave Port Examples	292

20.3 APB Port Naming	293

20.3.1 Pattern	293

20.3.2 APB Signal Examples	293

20.4 Internal Signal Naming	294

20.4.1 Crossbar Signals	294

20.4.2 Arbitration Signals	294

20.4.3 ID Signals	294

20.5 Debug Signal Naming	295

20.5.1 Pattern	295

20.5.2 Debug Signal Examples	295

20.6 Verification Pattern Matching	296

20.6.1 Factory Pattern Support	296

20.6.2 Pattern Rules	296

20.7 Related Documentation	297

21 Test Strategy	298

21.1 Overview	298

21.2 Test Categories	299

21.2.1 Unit Tests	299

21.2.2 Integration Tests	299

21.3 Test Parameterization	300

21.3.1 Configuration Matrix	300

21.3.2 Protocol Combinations	300

21.4 Protocol-BFM-Only Testing with Memory-Backed Slaves	301

21.4.1 BFM Instantiation and Slave Models	301

21.4.2 Transaction Generation and Assertion	301

21.4.3 Boundary Probe Testing	302

21.5 Coverage Goals	304

21.5.1 Functional Coverage	304

21.5.2 Code Coverage	304

21.6 Test Execution	305

21.6.1 Serial Execution (No Parallel xdist)	305

21.6.2 Test Naming Convention	305

21.6.3 Test Levels and Variants	306

21.7 Related Documentation	307

22 Debug Guide	308

22.1 Overview	308

22.2 Debug Signal Access	309

22.2.1 There are no dbg_* ports	309

22.3 Common Issues	311

22.3.1 Issue: Transaction Timeout	311

22.3.2 Issue: Wrong Response Routing	311

22.3.3 Issue: Data Corruption	312

22.4 Waveform Analysis	313

22.4.1 Generating Waveforms	313

22.4.2 Key Signals to Observe	313

22.5 Assertion Failures	314

22.5.1 There are no built-in assertions	314

22.5.2 Handling Assertion Failures	314

22.6 Performance Debugging	315

22.6.1 Throughput Issues	315

22.6.2 Latency Issues	315

22.7 Related Documentation	316

23 Bridge Hardware Architecture Specification Index	317

23.1 Document Organization	318

23.1.1 Front Matter	318

23.1.2 Chapter 1: Introduction	318

23.1.3 Chapter 2: System Overview	318

23.1.4 Chapter 3: Architecture	318

23.1.5 Chapter 4: Interfaces	318

23.1.6 Chapter 5: Performance	318

23.1.7 Chapter 6: Integration	318

23.2 Related Documentation	320

24 Bridge: AXI4 Full Crossbar Generator - Product Requirements Document	321

24.1 Executive Summary	322

24.2 Design Philosophy	323

24.2.1 Core Principles	323

24.2.2 Architecture Philosophy	324

24.3 CRITICAL: RTL Regeneration Requirements	325

24.4 1. Product Overview	326

24.4.1 1.1 Purpose	326

24.4.2 1.2 Target Audience	326

24.4.3 1.3 Success Criteria	326

24.4.4 1.4 Implementation Status and Phases	326

24.5 2. Architecture Overview	328

24.5.1 2.1 AXI4 vs AXIS vs APB	328

24.5.2 2.2 Block Diagram	329

24.5.3 2.3 Key Components	330

24.6 3. Functional Requirements	331

24.6.1 3.1 AXI4 Protocol Compliance	331

24.6.2 3.2 Address Decoding	331

24.6.3 3.3 Arbitration Strategy	332

24.6.4 3.4 Burst Handling	332

24.7 4. Non-Functional Requirements	333

24.7.1 4.1 Performance	333

24.7.2 4.2 Resource Usage (Estimated)	333

24.7.3 4.3 Quality Requirements	334

24.8 5. Interface Specifications	335

24.8.1 5.1 AXI4 Master Interfaces (M × 5 channels)	335

24.8.2 5.2 AXI4 Slave Interfaces (S × 5 channels)	336

24.8.3 5.3 Configuration Is Generation-Time, Not Parameters	336

24.9 6. Performance Modeling	337

24.9.1 6.1 Analytical Model	337

24.9.2 6.2 Resource Scaling	337

24.9.3 6.3 Comparison with Other Crossbars	338

24.10 7. Generator Architecture	339

24.10.1 7.1 Python Generator Structure	339

24.10.2 7.2 Address Map Configuration	340

24.11 8. Comparison with APB and Delta	342

24.11.1 8.1 Code Reuse from APB Generator	342

24.11.2 8.2 Code Reuse from Delta Generator	342

24.11.3 8.3 Complexity Comparison	342

24.12 9. Use Cases	344

24.12.1 9.1 Multi-Core Processor Interconnect	344

24.12.2 9.2 DMA + Accelerator System	344

24.12.3 9.3 FPGA System Integration	344

24.13 10. Testing Strategy	345

24.13.1 10.1 FUB (Functional Unit Block) Tests	345

24.13.2 10.2 Integration Tests	345

24.13.3 10.3 Performance Validation	345

24.14 11. Documentation Plan	346

24.14.1 11.1 Specifications (Before Code)	346

24.14.2 11.2 Performance Analysis (Before Implementation)	346

24.14.3 11.3 Code Generation	346

24.14.4 11.4 Verification	346

24.15 12. Success Metrics	347

24.15.1 12.1 Functional Completeness	347

24.15.2 12.2 Performance Targets	347

24.15.3 12.3 Educational Value	347

24.15.4 12.4 Reusability	347

24.16 12. Attribution and Contribution Guidelines	348

24.16.1 12.1 Git Commit Attribution	348

24.17 12.2 PDF Generation Location	349

24.18 13. Future Enhancements	350

24.18.1 13.1 Short-Term (Post-Initial Release)	350

24.18.2 13.2 Long-Term	350

24.19 14. Risk Assessment	351

24.20 15. Project Timeline (Estimated)	352

24.21 16. References	353

24.22 17. Glossary	354

25 Claude Code Guide: Bridge Subsystem	355

25.1 CRITICAL: Read Architecture Documents First	356

25.2 Quick Context	357

25.3 Target Architecture: Intelligent Width-Aware Routing	358

25.3.1 Efficient Multi-Width Design	358

25.3.2 Why This Architecture	358

25.3.3 Per-Master Output Paths	358

25.3.4 Benefits	359

25.3.5 Implementation Status	359

25.4 CRITICAL RULE #0: RTL Regeneration Requirements	360

25.4.1 The Golden Rule	360

25.4.2 Why This Is Non-Negotiable	360

25.4.3 Real Example (Historical Session)	360

25.4.4 Generator Files That Trigger Full Regeneration	360

25.4.5 The Regeneration Workflow	360

25.4.6 Symptoms of Version Mismatch	361

25.4.7 Think Like a Compiler Developer	361

25.4.8 Exception: Hand-Written RTL	361

25.5 Critical Pitfalls: slave_select_* Reversion After Handshake	362

25.5.1 The Problem: Combinational Address Decode	362

25.5.2 Visible Symptoms	362

25.5.3 The Solution (MANDATORY in All Generators)	362

25.5.4 When This Bug Appears	363

25.6 MANDATORY: Project Organization Pattern	364

25.6.1 Required Directory Structure	364

25.6.2 Testbench Class Location (MANDATORY)	364

25.6.3 Test File Pattern (MANDATORY)	365

25.7 Critical Rules for This Subsystem	367

25.7.1 Rule #0: Attribution Format for Git Commits	367

25.7.2 Rule #0.1: Testbench Architecture - MANDATORY SEPARATION	367

25.7.3 Rule #1: All Testbenches Inherit from TBBase	368

25.7.4 Rule #2: Mandatory Testbench Methods	368

25.7.5 Rule #3: Use GAXI Components for Protocol Handling	369

25.7.6 Rule #4: Queue-Based Verification	369

25.8 TOML/CSV-Based Bridge Generator (Phase 2 Complete)	371

25.8.1 Overview	371

25.8.2 Quick Start	371

25.8.3 Configuration Format Details	372

25.8.4 Channel-Specific Masters (Phase 2 Feature)	373

25.8.5 Example: RAPIDS-Style Configuration	373

25.8.6 Common User Questions	374

25.8.7 Generator Output Structure	376

25.8.8 Testing Generated Bridges	377

25.9 Bridge Architecture Quick Reference	378

25.9.1 Generated Bridge Crossbar Structure	378

25.9.2 Key Features	378

25.9.3 Address Map	378

25.10 Test Organization	380

25.10.1 Test Hierarchy	380

25.10.2 Test Levels	380

25.11 Common User Questions and Responses	381

25.11.1 Q: “How does the Bridge work?”	381

25.11.2 Q: “How do I generate a Bridge?”	381

25.11.3 Q: “How do I test a Bridge?”	381

25.12 Anti-Patterns to Avoid	383

25.12.1 Anti-Pattern 1: Embedded Testbench Classes	383

25.12.2 Anti-Pattern 2: Manual AXI4 Handshaking	383

25.12.3 Anti-Pattern 3: Memory Models for Simple Tests	383

25.13 Quick Reference	384

25.13.1 Finding Existing Components	384

25.13.2 Common Commands	384

25.14 Remember	385

25.15 PDF Generation Location	386

List of Figures

Figure 2.1: Master Adapter Architecture	51

Figure 2.2: Slave Router Architecture	67

Figure 2.3: Crossbar Core Architecture	85

Figure 2.5: ID Management Architecture	123

Figure 2.6: Width Conversion Architecture	142

Figure 2.7.1: Protocol Conversion Architecture	159

Figure 2.7: Protocol Conversion Architecture	160

Figure 2.8: Response Routing Architecture	186

Figure 2.9: Error Handling Architecture	208

Figure 5.3: AW Arbiter FSM	232

Figure 5.4: AR Arbiter FSM	233

Figure 3.1: Write Data Tracking FSM	241

Figure 4.1: CAM Entry Format	248

Figure 4.2: ID Table Structure	255

Figure 5.1: Upsize Converter (64-bit to 512-bit)	263

Figure 5.2: Downsize Converter (512-bit to 64-bit)	265

Figure 5.3: AXI4 Write to APB Write	274

Figure 5.4: AXI4 Read to APB Read	276

List of Tables

Table 1: Table 1.1: Supported Protocols	45

Table 2: Table 1.2: Channel-Specific Master Types	45

Table 3: Table 1.3: Document Organization	47

Table 4: Table 5.3: Burst Length Conversion Examples	264

Table 5: Table 5.1: AXI4 vs APB Protocol Comparison	273

Table 6: Table 5.2: APB to AXI4 Error Mapping	278

Table 7: Table 7.1: Test Parameter Matrix	300

Table 8: Table 7.2: Test Level and Variant Definitions	306

Table 9: Table 7.3: Debug Signals That Exist	309

List of Waveforms

No waveforms in this document.

# 1	Bridge Mas Index

Generated: 2026-09-06

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Document Information

## 2.1	Bridge Micro-Architecture Specification

| Property | Value |
| --- | --- |
| Document Title | Bridge Micro-Architecture Specification |
| Version | 1.1 |
| Date | June 4, 2026 |
| Status | Released |
| Classification | Open Source - MIT License |

## 2.2	Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-01-03 | RTL Design Sherpa | Initial release - restructured from single spec |
| 1.1 | 2026-06-04 | RTL Design Sherpa | Content sync to RTL state at 2026-06-04. Adds (1) monitor-aggregation discussion in the crossbar-core chapter — per-port axi4_{master,slave}_{rd,wr}_mon wrappers feed a monbus_arbiter tree into a single bridge-top monbus_axil_group, packet+timestamp carried atomically through a 192-bit skid; references the canonical 128-bit packet spec at docs/markdown/rtl-amba/includes/monitor_package_spec.md; (2) generator-emitted protocol-conversion shims at the slave boundary — axi4_to_axil4_{rd,wr} for AXIL slaves, chained with the AXIL→APB shim for APB slaves; (3) AXIL→wider-slave master-side alignment converters in the width-converters chapter (axil_to_axi4_wide_align_{rd,wr} — partial-word alignment, not protocol bridging); (4) generated-RTL chapter now lists monitor-variant module emission and per-port (UNIT_ID, AGENT_ID) assignment conventions; (5) verification-strategy chapter rewritten for protocol-BFM-only TBs with memory-backed slaves, the new boundary_probe test pattern, and the four test_bridge_*_monitor_*.py canonical patterns (smoke/capture/error_inject/irq); (6) response-routing chapter notes the independent W-tracker FIFO + split AW/W-ready MUX fix (commit d24bd617) that closed an interleaved-master deadlock. |

## 2.3	Document Purpose

This Micro-Architecture Specification (MAS) documents how the Bridge component is built:

- Block-level architecture and internal structure

- FSM state diagrams and transition tables

- ID management and CAM architecture

- Signal-level interface details

- Width and protocol converter implementation

- Timing diagrams and debug information

For high-level architecture, system integration, and performance overview, refer to the companion Bridge Hardware Architecture Specification (HAS).

## 2.4	Intended Audience

- RTL designers implementing or modifying Bridge

- Verification engineers developing testbenches

- System integrators debugging Bridge behavior

- Technical staff requiring implementation details

## 2.5	Related Documents

- Bridge HAS - Hardware Architecture Specification (high-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	Overview

## 3.1	Bridge Micro-Architecture

Bridge is a multi-protocol AXI4 crossbar generator: it turns CSV/TOML configuration files into parameterized SystemVerilog RTL. This document describes what that RTL looks like on the inside.

## 3.2	Key Capabilities

### 3.2.1	Multi-Protocol Support

Bridge supports three AMBA protocols:

Table 1.1: Supported Protocols

| Protocol | Use Case | Conversion |
| --- | --- | --- |
| AXI4 Full | High-bandwidth memory | Direct or width convert |
| AXI4-Lite | Register access | Protocol downgrade |
| APB | Low-speed peripherals | Full conversion |

### 3.2.2	Channel-Specific Masters

Masters that only read or only write don’t pay for the channels they never use:

Table 1.2: Channel-Specific Master Types

| Type | Channels | Signals | Use Case |
| --- | --- | --- | --- |
| Full (rw) | AW, W, B, AR, R | 100% | CPU, config master |
| Write-only (wr) | AW, W, B | ~60% | DMA write engine |
| Read-only (rd) | AR, R | ~40% | DMA read engine |

### 3.2.3	Automatic Converters

Bridge inserts converters automatically:

- Width converters - Handle data width mismatches

- Protocol converters - AXI4 to APB conversion

- Per-path optimization - Only where needed

## 3.3	Architecture Summary

### 3.3.1	Block Organization

Bridge NxM
├── Master Adapters (M instances)
│   ├── Skid buffers
│   └── ID extension
├── Crossbar Core
│   ├── Address decode
│   ├── Per-slave arbiters
│   └── Channel muxes
├── Slave Routers (N instances)
│   ├── Protocol converters
│   └── Width converters
└── Response Routing
    ├── ID extraction
    └── Master demux

Signal Flow

## 3.4	Document Organization

Table 1.3: Document Organization

| Chapter | Content |
| --- | --- |
| Ch 2 | Block Descriptions |
| Ch 3 | FSM Design |
| Ch 4 | ID Management |
| Ch 5 | Converters |
| Ch 6 | Generated RTL |
| Ch 7 | Verification |

## 3.5	Related Documentation

- Bridge HAS - High-level architecture and integration

- PRD.md - Product requirements

- Generator source - bin/bridge_generator.py (with bin/bridge_pkg/)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	2.1 Master Adapter

Every master port in the bridge gets its own Master Adapter. The adapter sits between the master and the crossbar core, and it exists to make the crossbar’s life simple: timing is isolated at the boundary, channels are specialized to what the master actually uses, and every transaction carries a Bridge ID before it enters the fabric.

## 4.1	2.1.1 Purpose and Function

Concretely, the adapter does five things:

- Timing Isolation: Inserts pipeline registers (skid buffers) to break combinatorial paths from master to crossbar

- Channel Specialization: Separates read-only, write-only, and read-write masters for optimal resource utilization

- Bridge ID Injection: Adds internal tracking IDs to transactions for response routing

- Protocol Normalization: Ensures all transactions meet crossbar assumptions and constraints

- Backpressure Management: Handles ready/valid handshaking with single-cycle latency

## 4.2	2.1.2 Block Diagram

### 4.2.1	Figure 2.1: Master Adapter Architecture

Master Adapter Architecture

The figure shows Master Adapter architecture with per-master skid buffers, bridge ID injection/stripping, and channel specialization for rd/wr/rw configurations.

## 4.3	2.1.3 Channel Specialization

The Bridge generator creates three types of master adapters based on the channel usage specified in the configuration:

### 4.3.1	Read-Only Masters

Channels: AR (Address Read), R (Read Data)

Configuration: "rd" or "read"
RTL Generated: adapter_master_rd_<id>.sv

Optimizations:
- No write address channel logic
- No write data channel logic
- No write response channel logic
- Reduced arbitration participation (read path only)

### 4.3.2	Write-Only Masters

Channels: AW (Address Write), W (Write Data), B (Write Response)

Configuration: "wr" or "write"
RTL Generated: adapter_master_wr_<id>.sv

Optimizations:
- No read address channel logic
- No read data channel logic
- Reduced arbitration participation (write path only)

### 4.3.3	Read-Write Masters

Channels: AR, R, AW, W, B (Full AXI4 interface)

Configuration: "rw" or "readwrite"
RTL Generated: adapter_master_rw_<id>.sv

Full Functionality:
- All five AXI4 channels implemented
- Participates in both read and write arbitration
- Maximum flexibility but larger resource footprint

## 4.4	2.1.3b Per-Port Monitor Wrappers (when use_monitor = true)

When the bridge TOML configuration sets use_monitor = true for a master port, the generator instantiates per-direction monitor wrappers immediately downstream of the master adapter:

- axi4_master_rd_mon: Wraps the AR/R channels, emits packets with UNIT_ID=1

- axi4_master_wr_mon: Wraps the AW/W/B channels, emits packets with UNIT_ID=1

Each wrapper: - Samples all channel signals and timestamp at handshake points - Emits 128-bit monitor packets on internal monbus - Passes all signals through transparently (no stall) - Is instantiated only when requested (generator-time configuration)

The generated per-port monbus streams are later aggregated by a tree of monbus_arbiter instances at the bridge top.

## 4.5	2.1.4 Skid Buffer Architecture

Each AXI4 channel passes through a skid buffer for timing isolation. The skid buffer implements:

### 4.5.1	Registered Forward Path

// Simplified skid buffer concept
always_ff @(posedge clk) begin
    if (!rst_n) begin
        valid_reg <= 1'b0;
    end else if (!valid_reg || ready_out) begin
        valid_reg <= valid_in;
        data_reg  <= data_in;
    end
end

### 4.5.2	Single-Cycle Backpressure Response

- When downstream is not ready, accepts one additional beat into holding register

- Signals ready_in = 0 in same cycle to upstream

- No combinatorial paths between upstream and downstream

### 4.5.3	Pipeline Depth

- Default: 1 stage (skid buffer)

- Configurable: Up to 8 stages for high-frequency designs

- Trade-off: Latency vs. timing closure

Performance Impact: - Adds 1 cycle latency per channel - Enables higher clock frequencies - Prevents critical paths through crossbar

## 4.6	2.1.5 Bridge ID Management

### 4.6.1	ID Width Calculation

BID_WIDTH = clog2(num_masters)

Examples:
- 2 masters  → BID_WIDTH = 1
- 4 masters  → BID_WIDTH = 2
- 8 masters  → BID_WIDTH = 3
- 16 masters → BID_WIDTH = 4

### 4.6.2	ID Injection (Request Path)

For each master adapter, a unique constant Bridge ID is appended to the AXI transaction ID:

AR Channel:
  Internal ARID = {MASTER_BID[BID_WIDTH-1:0], External ARID[ARID_WIDTH-1:0]}
  Internal ARID Width = BID_WIDTH + ARID_WIDTH

AW Channel:
  Internal AWID = {MASTER_BID[BID_WIDTH-1:0], External AWID[AWID_WIDTH-1:0]}
  Internal AWID Width = BID_WIDTH + AWID_WIDTH

Example: 4-master system, external ARID_WIDTH = 4

Master 0: BID = 2'b00, External ID = 4'h3 → Internal ID = 6'b00_0011
Master 1: BID = 2'b01, External ID = 4'h3 → Internal ID = 6'b01_0011
Master 2: BID = 2'b10, External ID = 4'h5 → Internal ID = 6'b10_0101
Master 3: BID = 2'b11, External ID = 4'hA → Internal ID = 6'b11_1010

### 4.6.3	ID Stripping (Response Path)

The Bridge ID is removed from responses before returning to the master:

R Channel:
  External RID = Internal RID[ARID_WIDTH-1:0]
  Bridge routes based on Internal RID[BID_WIDTH+ARID_WIDTH-1:ARID_WIDTH]

B Channel:
  External BID = Internal BID[AWID_WIDTH-1:0]
  Bridge routes based on Internal BID[BID_WIDTH+AWID_WIDTH-1:AWID_WIDTH]

This ensures: - Master sees original transaction IDs - Bridge internally tracks which master originated each transaction - Responses route back to correct master even with ID reuse across masters

## 4.7	2.1.6 Protocol Normalization

The Master Adapter enforces several protocol requirements:

### 4.7.1	Valid Address Ranges

- Checks addresses against slave address maps

- Flags out-of-range accesses for error handling

- Prevents deadlocks from illegal addresses

### 4.7.2	Burst Constraints

- Validates ARLEN/AWLEN vs. 4KB boundary rules

- Ensures burst types are supported (FIXED, INCR, WRAP)

- Checks SIZE vs. DATA_WIDTH compatibility

### 4.7.3	Signal Defaults

- Provides default values for unused signals

- Ensures ARCACHE/AWCACHE have legal values

- Sets ARPROT/AWPROT based on configuration

## 4.8	2.1.6 Response Path Slave Selection Tracking

### 4.8.1	Problem: Stale Slave Select Decode

Here’s the part that bites people. The master adapter decodes the incoming address combinationally to pick a target slave. When the wrapper’s skid buffer pops and fub_axi_awaddr/fub_axi_araddr reverts, the decode changes even though the converter is still in flight pushing the request to the crossbar. This causes the response MUX to route read/write responses from the wrong slave.

### 4.8.2	Solution: Per-Channel Response-Tracking FIFOs

Added separate FIFOs for AR and AW channels that capture the slave index (comb_slave_select_ar/aw) at the moment of the FUB-level handshake:

// At request time (when fub_axi_arvalid && fub_axi_arready)
ar_trk_push <= comb_slave_select_ar;  // Capture decoded slave index

// At response time (when m_axi_rvalid && m_axi_rready && m_axi_rlast)
r_slave_select <= ar_trk_pop;  // Stable slave index for response MUX

Benefits: - R/B responses route correctly even after address reverts - Supports multi-slave masters spanning different widths - Stable path for both data width and target slave

Implementation: - One FIFO per (master, AW/AR channel) - Width: clog2(NUM_SLAVES) bits (4 bits typical for 16-slave systems) - Depth: Matches maximum outstanding transactions (8-16 typical)

## 4.9	2.1.7 Interface Specifications

### 4.9.1	External Master Interface (per master)

Input Channels (from master):
- ARVALID, ARREADY, ARADDR, ARBURST, ARCACHE, ARID, ARLEN, 
  ARLOCK, ARPROT, ARQOS, ARSIZE

- AWVALID, AWREADY, AWADDR, AWBURST, AWCACHE, AWID, AWLEN,
  AWLOCK, AWPROT, AWQOS, AWSIZE

- WVALID, WREADY, WDATA, WSTRB, WLAST

Output Channels (to master):
- RVALID, RREADY, RDATA, RID, RLAST, RRESP

- BVALID, BREADY, BID, BRESP

### 4.9.2	Internal Crossbar Interface (per master)

Output Channels (to crossbar):
- AR* signals + Internal ARID (wider)
- AW* signals + Internal AWID (wider)  
- W* signals (unchanged)

Input Channels (from crossbar):
- R* signals + Internal RID (wider)
- B* signals + Internal BID (wider)

## 4.10	2.1.8 Resource Utilization

### 4.10.1	Per-Master Adapter Resources (Typical)

Read-Write Master (64-bit data, 32-bit addr, 4-bit ID):

Logic Elements:  ~200-400 (depending on ID width and features)
Registers:       ~150-250 (pipeline stages + control)
Block RAM:       0 (CAM is in crossbar core)

Breakdown:
- Skid buffers (5 channels × ~30 regs each): ~150 regs
- ID manipulation logic: ~50 LEs
- Control FSMs: ~100 LEs
- Routing logic: ~50 LEs

Read-Only Master: ~60% of read-write resources
Write-Only Master: ~65% of read-write resources

### 4.10.2	Scaling Considerations

Resource usage scales with: - Number of masters (linear scaling, one adapter per master) - ID width (logarithmic: +BID_WIDTH bits per transaction) - Pipeline depth (linear: deeper pipelines = more registers) - Data width (linear: wider data = wider skid buffers)

## 4.11	2.1.9 Timing Characteristics

### 4.11.1	Latency

Request Path (Master → Crossbar): - Minimum: 1 cycle (skid buffer) - With 4-stage pipeline: 4 cycles - With ID injection: +0 cycles (combinatorial within stage)

Response Path (Crossbar → Master): - Minimum: 1 cycle (skid buffer) - With 4-stage pipeline: 4 cycles - With ID stripping: +0 cycles (combinatorial within stage)

Total Round-Trip Overhead: 2-8 cycles (depending on pipeline depth)

### 4.11.2	Throughput

- Maximum: 1 transaction per cycle per channel (fully pipelined)

- Backpressure: Propagates with 1-cycle latency

- Burst Performance: No degradation; bursts flow continuously when ready

### 4.11.3	Critical Paths

Typical critical paths (if skid buffers not used): - Master ARVALID → Crossbar arbitration logic - Crossbar grant → Master ARREADY - Slave RDATA → Master RDATA

Solution: Skid buffers break all these paths at the cost of 1-cycle latency.

## 4.12	2.1.10 Configuration Parameters

### 4.12.1	Per-Master Parameters (from TOML/CSV)

[[masters]]
name = "cpu"
channels = "rw"              # "rd", "wr", or "rw"
arid_width = 4               # External ID width
awid_width = 4               # Can differ from ARID
addr_width = 32              # Address bus width
data_width = 64              # Data bus width
pipeline_depth = 1           # Skid buffer stages (1-8)

### 4.12.2	Global Parameters (affect all adapters)

[bridge]
internal_data_width = 64     # Crossbar data width
enable_width_conversion = true
bid_width = 2                # Calculated: clog2(num_masters)

## 4.13	2.1.11 Debug and Observability

### 4.13.1	Recommended Debug Signals

For ILA or waveform capture:

- Master adapter input valid/ready (all channels)
- Master adapter output valid/ready (all channels)
- Bridge ID assignments per transaction
- Pipeline stage occupancy
- Backpressure events (ready = 0 while valid = 1)

### 4.13.2	Common Issues and Debug

Symptom: Master hangs waiting for READY
Check: - Is adapter receiving grants from arbiters? - Is slave responding to requests? - Is response path clear?

Symptom: Incorrect data returned to master
Check: - Bridge ID extraction logic - ID width mismatches - Response routing in crossbar

Symptom: Timing violations
Check: - Increase pipeline_depth parameter - Verify clock frequency vs. design complexity - Check for combinatorial loops

## 4.14	2.1.12 Future Enhancements

### 4.14.1	Planned Features

- Dynamic Pipeline Depth: Adjust depth based on operating frequency

- FIFO Mode: Deeper buffering (16-256 entries) for burst-intensive masters

- QoS Support: Priority-based arbitration hints

- Performance Counters: Transaction counts, stall cycles, utilization metrics

### 4.14.2	Under Consideration

- Clock Domain Crossing: Per-master clock domains with async FIFOs

- Width Conversion at Adapter: Move width logic to adapters for distributed conversion

- Outstanding Transaction Tracking: Windowing for improved OOO performance

Related Sections: - Section 2.3: Crossbar Core (interconnect architecture) - Section 2.4: Arbitration (how adapters compete for slaves) - Section 2.5: ID Management (CAM structures for response routing) - Section 3.2: Master Port Interface (signal-level specifications)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	2.2 Slave Router

Every master gets its own Slave Router. The router examines each request address, steers the transaction to one of the configured slaves, and produces the error response itself when the address lands nowhere.

## 5.1	2.2.1 Purpose and Function

The router does five things:

- Address Decoding: Matches request addresses against configured slave address ranges

- Request Routing: Directs AR/AW/W channels to the selected slave

- Out-of-Range Detection: Identifies addresses that don’t map to any slave

- Error Response Generation: Creates DECERR responses for invalid addresses

- Default Slave Support: Routes unmapped addresses to optional default slave

## 5.2	2.2.2 Block Diagram

### 5.2.1	Figure 2.2: Slave Router Architecture

Slave Router Architecture

Slave router architecture showing address decoding, routing matrix, and out-of-range detection for AW and AR channels.

## 5.3	2.2.3 Address Decoding Algorithm

### 5.3.1	Configuration-Based Address Maps

Each slave is configured with:

[[slaves]]
name = "memory"
base_address = 0x4000_0000
size = 0x1000_0000        # 256 MB
default = false           # Not a default slave

The router generates address ranges:

Start Address = base_address
End Address   = base_address + size - 1

### 5.3.2	Decoding Priority

When multiple slaves have overlapping address ranges, the router uses first-match priority:

Priority Order = Order in configuration file (top to bottom)

Example:
  Slave 0: 0x0000_0000 - 0x0FFF_FFFF  (checked first)
  Slave 1: 0x1000_0000 - 0x1FFF_FFFF  (checked second)
  Slave 2: 0x2000_0000 - 0x2FFF_FFFF  (checked third)
  
Address 0x1000_5000 → Matches Slave 1

### 5.3.3	Range Checking Logic

For each address, the router performs:

// Simplified address decode logic
logic [NUM_SLAVES-1:0] slave_match;

for (int i = 0; i < NUM_SLAVES; i++) begin
    slave_match[i] = (addr >= SLAVE_BASE[i]) && 
                     (addr <= SLAVE_END[i]);
end

// Priority encode: Select first matching slave
logic [$clog2(NUM_SLAVES)-1:0] slave_select;
always_comb begin
    slave_select = 0;
    for (int i = 0; i < NUM_SLAVES; i++) begin
        if (slave_match[i]) begin
            slave_select = i;
            break;  // First match wins
        end
    end
end

// Out-of-range detection
logic oor = ~(|slave_match);  // No slaves matched

### 5.3.4	Power-of-Two Optimization

For slaves with power-of-two sizes starting at aligned addresses, simplified decode:

// Optimized decode for: base=0x8000_0000, size=0x1000_0000 (256MB)
// Mask off lower bits: Only check upper bits
logic match = (addr[31:28] == 4'h8);  // Much simpler than range check

The generator automatically detects and applies this optimization.

## 5.4	2.2.4 Request Routing

### 5.4.1	AR Channel Routing

Read address routing flow: 1. Decode: Determine target slave from ARADDR 2. Check OOR: If no slave matches, flag for error response 3. Route: Send AR transaction to selected slave’s arbiter 4. Track: Remember routing decision for R channel responses

Diagram 2

### 5.4.2	AW/W Channel Routing

Write transactions require coordinated routing:

- AW Phase:

  - Decode AWADDR to determine target slave

  - Route AW transaction to selected slave’s arbiter

  - Store routing decision for subsequent W beats

- W Phase:

  - Follow AW routing (W channel has no address)

  - Route all W beats to same slave as AW

  - Continue until WLAST = 1

Diagram 3

### 5.4.3	Write Data Tracking FSM

State Machine for W Channel Routing:

IDLE:
  - Wait for AWVALID && AWREADY
  - On handshake: Store target slave, goto WRITING
  
WRITING:
  - Route W beats to stored slave
  - On WVALID && WREADY && WLAST: goto IDLE
  
Error Handling:
  - If AW was OOR: Discard W beats, generate BRESP error

## 5.5	2.2.5 Out-of-Range Handling

### 5.5.1	Detection

An address is out-of-range (OOR) if:

∀ slaves[i]: (address < base[i]) OR (address > end[i])

Unless a default slave is configured.

### 5.5.2	Error Response Generation

When OOR is detected:

Read Transactions (AR → R):

1. Accept ARVALID (assert ARREADY) 
2. Do NOT forward to any slave
3. Generate R response internally:
   - RID = Original ARID (with BID preserved)
   - RDATA = 0xBADDCAFE_DEADBEEF (debug pattern) or all zeros
   - RRESP = 2'b11 (DECERR)
   - RLAST = 1 (for each beat if burst)
4. Latency: Typically 2-3 cycles after AR acceptance

Write Transactions (AW/W → B):

1. Accept AWVALID (assert AWREADY)
2. Accept all W beats until WLAST (sink the data)
3. Do NOT forward to any slave
4. Generate B response internally:
   - BID = Original AWID (with BID preserved)
   - BRESP = 2'b11 (DECERR)
5. Latency: Typically 2-3 cycles after WLAST

### 5.5.3	Debug Data Patterns

For OOR read responses, configurable data patterns aid debugging:

Option 1: All zeros (default)
  RDATA = 64'h0000_0000_0000_0000

Option 2: Debug signature
  RDATA = 64'hBADD_CAFE_DEAD_BEEF

Option 3: Address echo (LSBs)
  RDATA = {32'hBADD_ADDR, ARADDR[31:0]}

Option 4: Master/Slave ID indicator  
  RDATA = {8'hEE, Master_ID[7:0], Slave_ID[7:0], ARADDR[47:0]}

## 5.6	2.2.6 Default Slave Support

### 5.6.1	Configuration

[[slaves]]
name = "error_responder"
base_address = 0x0        # Ignored for default slave
size = 0x0                # Ignored for default slave
default = true            # Catch-all for unmapped addresses

### 5.6.2	Behavior

When a default slave is configured: - Addresses that don’t match any specific slave → Routed to default slave - Default slave typically returns DECERR but with configurable response - Useful for prototyping (accept all addresses initially) - Can implement memory-mapped debug register for address capture

Note: Only ONE default slave allowed per bridge.

## 5.7	2.2.7 Address Aliasing

### 5.7.1	Multiple Slaves, Same Address

If configuration has overlapping ranges:

[[slaves]]
name = "fast_cache"
base_address = 0x8000_0000
size = 0x1000_0000

[[slaves]]
name = "slow_memory"
base_address = 0x8000_0000  # Same base!
size = 0x4000_0000

Result: First-match priority applies. All accesses go to fast_cache. The slow_memory range 0x9000_0000-0xBFFF_FFFF is unreachable from this master.

Warning: Generator can optionally flag this as error in DRC mode.

### 5.7.2	Intentional Aliasing Use-Cases

Legitimate uses of overlapping ranges: 1. Cache hierarchy: Small fast cache shadows larger slow memory 2. Memory remapping: Different views of same physical memory 3. Peripheral mirroring: Register block appears at multiple addresses

## 5.8	2.2.8 Configuration Parameters

### 5.8.1	Per-Router Parameters

# Router behavior is defined by slave configurations

[[slaves]]
name = "ddr_memory"
base_address = 0x8000_0000
size = 0x4000_0000           # 1 GB
default = false
oor_data_pattern = 0xDEAD    # Pattern for OOR reads

### 5.8.2	Global Parameters

[bridge]
enable_default_slave = false      # Allow default slave
strict_address_decode = true      # Flag overlapping ranges as errors
oor_response_latency = 2          # Cycles for OOR error response (2-4)

## 5.9	2.2.9 Resource Utilization

### 5.9.1	Per-Router Resources (Typical)

4-slave configuration (32-bit address):

Logic Elements:  ~150-200 LEs
Registers:       ~50 regs
Block RAM:       0

Breakdown:
- Address comparators (4 slaves × ~30 LEs): ~120 LEs
- Priority encoder: ~20 LEs
- W channel FSM: ~30 regs, ~20 LEs
- OOR error generator: ~20 regs, ~10 LEs

8-slave configuration:

Logic Elements:  ~250-350 LEs (scales roughly with slave count)
Registers:       ~60 regs

### 5.9.2	Scaling Considerations

Resource usage scales with: - Number of slaves: Linear (each slave adds comparator logic) - Address width: Minimal impact (wider comparators, but same structure) - Optimizations: Power-of-two sizes reduce logic significantly

## 5.10	2.2.10 Timing Characteristics

### 5.10.1	Decode Latency

Combinatorial Decode (Default): - Address → Slave selection: 0 cycles (combinatorial) - Critical path: ARADDR → Slave arbiter request - May limit maximum frequency in large systems

Registered Decode (Optional): - Address → Slave selection: 1 cycle (registered) - Adds latency but breaks critical path - Recommended for >8 slaves or >300 MHz operation

### 5.10.2	Throughput

- Maximum: 1 transaction per cycle per master

- No blocking: Router does not stall; arbiters handle conflicts

- Pipelining: AR and AW decode in parallel (independent paths)

### 5.10.3	Critical Paths

Typical critical paths: - ARADDR → Address comparators → Priority encoder → Arbiter request - w/4 slaves: ~10-15 logic levels (FPGA-dependent) - w/8 slaves: ~12-18 logic levels

Mitigation: - Enable registered decode mode (+1 cycle latency) - Use power-of-two slave sizes (simplified compare) - Synthesizer optimization directives

## 5.11	2.2.11 Debug and Observability

### 5.11.1	Recommended Debug Signals

- Address decode outputs (which slave matched)
- OOR flags (per channel)
- Default slave hit counter
- W channel FSM state
- Routing decision storage (for W tracking)

### 5.11.2	Common Issues and Debug

Symptom: Reads return all zeros
Check: - Is address out-of-range? - Check slave base/size configuration - Verify address decode logic in waveform

Symptom: Write data goes to wrong slave
Check: - W channel tracking FSM state - Did AW routing complete before W started? - Check for AWVALID/AWREADY handshake

Symptom: Unexpected DECERR responses
Check: - Address decode configuration - Overlapping slave ranges (wrong priority) - Off-by-one in size calculations

## 5.12	2.2.12 Verification Considerations

### 5.12.1	Address Decode Tests

Critical test scenarios: 1. Boundary conditions: base_address, base_address + size - 1 2. Just out-of-range: base_address - 1, base_address + size 3. Each slave: Verify routing to correct slave 4. Overlapping ranges: Verify priority encoding 5. Default slave: Unmapped addresses route correctly

### 5.12.2	Write Tracking Tests

W channel FSM testing: 1. Simple write: Single AW, single W (WLAST=1) 2. Burst write: AW with AWLEN=7, eight W beats 3. Back-to-back writes: New AW before previous W completes 4. Interleaved masters: Multiple masters writing simultaneously (if supported)

### 5.12.3	OOR Error Tests

Test: Read from unmapped address
- Send AR to 0xFFFF_FFFF (assuming unmapped)
- Verify R response with RRESP = DECERR
- Verify RID matches ARID
- Check response latency

Test: Write to unmapped address
- Send AW to invalid address
- Send W data
- Verify B response with BRESP = DECERR
- Verify BID matches AWID

## 5.13	2.2.13 Performance Optimization

### 5.13.1	Techniques

1. Registered Decode (High Frequency)

Trade-off: +1 cycle latency for timing closure
Best for: >8 slaves, >300 MHz targets

2. Simplified Address Decode (Power-of-2)

Optimization: Mask-based compare instead of range check
Best for: Slaves with aligned, power-of-2 sizes
Savings: ~50% reduction in comparator logic

3. Parallel Decode (Low Logic)

Implementation: Separate decoders per address bit-field
Best for: Many non-overlapping slaves
Savings: Reduces logic depth for priority encoding

4. CAM-Based Decode (Many Slaves)

Trade-off: Uses block RAM for address lookup table
Best for: >16 slaves, non-contiguous ranges
Note: Requires RAM resources

## 5.14	2.2.14 Future Enhancements

### 5.14.1	Planned Features

- Dynamic Address Remapping: Runtime-configurable slave ranges

- Transaction Filtering: Block certain address ranges per-master

- Priority Hints: QoS-based prioritization (beyond first-match)

- Address Translation: Offset/mask transformations before slave routing

### 5.14.2	Under Consideration

- Multi-region Slaves: Slave spans multiple non-contiguous ranges

- Secure Address Spaces: Per-master access control lists

- Debug Address Capture: Log invalid addresses to register

Related Sections: - Section 2.1: Master Adapter (upstream from router) - Section 2.3: Crossbar Core (downstream arbitration) - Section 2.4: Arbitration (how routed requests compete) - Section 3.3: Slave Port Interface (target of routed requests)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	2.3 Crossbar Core

The Crossbar Core is the fabric itself: any master can reach any slave through it. Arbitration, request routing, and response management — the machinery that makes any-to-any work — all live here.

## 6.1	2.3.1 Purpose and Function

The core does five things:

- Full Connectivity: Provides complete N×M master-to-slave interconnect matrix

- Independent Arbitration: Per-slave arbiters allow parallel access to different slaves

- Response Routing: Directs slave responses back to originating masters using Bridge IDs

- Transaction Ordering: Maintains AXI ordering requirements within address dependencies

- Backpressure Management: Handles flow control across multiple concurrent transactions

## 6.2	2.3.2 Block Diagram

### 6.2.1	Figure 2.3: Crossbar Core Architecture

Crossbar Core Architecture

Crossbar core architecture showing complete M×S switching fabric with address decode, per-slave arbitration, data path multiplexing, and ID-based response routing.

## 6.3	2.3.3 Connectivity Matrix

### 6.3.1	Full N×M Crossbar

The bridge implements a non-blocking crossbar where: - N masters can each access different slaves simultaneously - Conflicts occur only when multiple masters target the same slave - Independent read and write paths increase parallelism

Example: 4 masters, 3 slaves

S0   S1   S2
    M0   ●    ●    ●
    M1   ●    ●    ●
    M2   ●    ●    ●
    M3   ●    ●    ●

● = Connection available

### 6.3.2	Request Path Multiplexing

For each slave, requests from all masters are multiplexed:

Slave 0 Request Inputs:
  - M0 → S0 (AR, AW, W channels)
  - M1 → S0 (AR, AW, W channels)
  - M2 → S0 (AR, AW, W channels)
  - M3 → S0 (AR, AW, W channels)
  
Arbiter selects one master per channel per cycle
  → Grants propagate through MUX
  → Selected master's transaction forwarded to slave

### 6.3.3	Response Path Demultiplexing

Responses from slaves are demultiplexed back to masters:

Slave 0 Response Outputs (R, B channels):
  - Extract Bridge ID from RID/BID
  - Route to corresponding master (M0, M1, M2, or M3)
  - Strip Bridge ID before delivering to master adapter
  
Uses CAM (Content Addressable Memory) for fast ID lookup

## 6.4	2.3.4 Request Path Architecture

### 6.4.1	Per-Slave Request Arbitration

Each slave has independent arbiters for: 1. AR Channel: Read address arbitration (all masters competing) 2. AW/W Channels: Write address/data arbitration (all masters competing)

This separation allows: - Simultaneous read and write to same slave (if slave supports it) - Independent grant decisions for AR vs. AW channels - Better throughput for mixed read/write workloads

### 6.4.2	Request Multiplexers with Stable Address Gating

After arbitration, multiplexers select granted master’s signals. AR/AW gating uses inline address re-decode on the stable m_axi address bus (held stable across the full handshake):

// Inline address re-decode for stable gating (replaces slave_select_* gating)
// The address on m_axi is held stable from fub_axi handshake through m_axi handshake
always_comb begin
    // Re-decode each master's address to determine target slave
    m0_targets_s0 = (m0_m_axi_araddr >= S0_BASE) && (m0_m_axi_araddr < S0_END);
    m1_targets_s0 = (m1_m_axi_araddr >= S0_BASE) && (m1_m_axi_araddr < S0_END);
    m2_targets_s0 = (m2_m_axi_araddr >= S0_BASE) && (m2_m_axi_araddr < S0_END);
    m3_targets_s0 = (m3_m_axi_araddr >= S0_BASE) && (m3_m_axi_araddr < S0_END);
end

// Gate with valid signals
assign m0_ar_s0_valid = m0_arvalid && m0_targets_s0;
assign m1_ar_s0_valid = m1_arvalid && m1_targets_s0;
assign m2_ar_s0_valid = m2_arvalid && m2_targets_s0;
assign m3_ar_s0_valid = m3_arvalid && m3_targets_s0;

// Simplified AR channel MUX for Slave 0
always_comb begin
    case (ar_grant_s0)
        2'b00: begin  // Master 0 granted
            s0_arvalid = m0_arvalid;
            s0_araddr  = m0_m_axi_araddr;  // Stable across m_axi handshake
            s0_arid    = m0_arid;    // Includes Bridge ID
            // ... other AR signals
        end
        2'b01: begin  // Master 1 granted
            s0_arvalid = m1_arvalid;
            s0_araddr  = m1_m_axi_araddr;  // Stable across m_axi handshake
            s0_arid    = m1_arid;
            // ... other AR signals
        end
        // ... cases for M2, M3
    endcase
end

Why this matters: The address on m_axi_* is held stable for the entire AXI handshake (arvalid && arready). This allows address re-decoding to determine the target slave even as the master’s internal address state changes.

### 6.4.3	AW→W Burst Tracking FIFO

For masters with multiple slave connections, W beats must follow the slave that the AW was driven to. A per-(master, slave) FIFO tracks which slave the AW was routed to:

// Track which slave the AW was driven to
always_ff @(posedge aclk or negedge aresetn) begin
    if (!aresetn) begin
        aw_trk_slave_id <= '0;
        aw_trk_valid <= 1'b0;
    end else if (m0_awvalid && m0_m_axi_awready && s_selected_aw == S0) begin
        aw_trk_slave_id <= S0;  // Track that AW went to slave 0
        aw_trk_valid <= 1'b1;
    end else if (m0_wvalid && m0_m_axi_wready && m0_m_axi_wlast) begin
        aw_trk_valid <= 1'b0;  // AW->W burst complete
    end
end

// Gate W path based on tracked slave
logic w_to_s0, w_to_s1, w_to_s2;
always_comb begin
    w_to_s0 = m0_wvalid && (aw_trk_slave_id == S0);
    w_to_s1 = m0_wvalid && (aw_trk_slave_id == S1);
    w_to_s2 = m0_wvalid && (aw_trk_slave_id == S2);
end

Why this matters: Without tracking, W beats would re-evaluate address decode which may have stale or different values, routing to the wrong width converter or slave.

### 6.4.4	Backpressure Propagation

Ready signals flow back from slave through arbiter to granted master:

Slave S0 ARREADY → Arbiter → Granted Master ARREADY
                           ↘ Other Masters ARREADY = 0

This ensures: - Only granted master sees READY from slave - Non-granted masters see READY = 0 (backpressure) - No combinatorial loops in ready path (registered arbitration)

## 6.5	2.3.5 Response Path Architecture

### 6.5.1	Bridge ID Extraction

Response routing uses the Bridge ID embedded in transaction IDs:

R Channel:
  Slave RID = {BID[BID_WIDTH-1:0], Original_ID[ID_WIDTH-1:0]}
  Extract: Master_Index = BID
  Route R response to Master[Master_Index]

B Channel:
  Slave BID = {BID[BID_WIDTH-1:0], Original_ID[ID_WIDTH-1:0]}
  Extract: Master_Index = BID
  Route B response to Master[Master_Index]

### 6.5.2	CAM-Based Routing (Optional)

For large master counts or OOO responses, a CAM tracks outstanding transactions:

CAM Entry Structure:
  - Internal ID (with BID)
  - Master index
  - Transaction type (read/write)
  - Timestamp (for timeout detection)

Lookup:
  Input: RID or BID from slave
  Output: Master index for routing
  Latency: 1 cycle (registered CAM)

Benefit: Handles complex scenarios like ID reordering, burst interleaving

### 6.5.3	Response Demultiplexers

Based on extracted Bridge ID, responses are routed:

// Simplified R channel DEMUX from Slave 0
logic [BID_WIDTH-1:0] master_id;
assign master_id = s0_rid[TOTAL_ID_WIDTH-1:ID_WIDTH];  // Extract BID

always_comb begin
    // Default: No masters receive response
    m0_rvalid = 1'b0;
    m1_rvalid = 1'b0;
    m2_rvalid = 1'b0;
    m3_rvalid = 1'b0;
    
    // Route to indicated master
    case (master_id)
        2'b00: begin
            m0_rvalid = s0_rvalid;
            m0_rdata  = s0_rdata;
            m0_rid    = s0_rid[ID_WIDTH-1:0];  // Strip BID
            // ... other R signals
        end
        2'b01: begin
            m1_rvalid = s0_rvalid;
            // ... route to M1
        end
        // ... M2, M3
    endcase
end

### 6.5.4	Multi-Slave Response Merging

When multiple slaves can respond simultaneously, arbitration ensures: - Only one slave’s response delivered per master per cycle - Fair arbitration if multiple slaves have responses for same master - No response loss (responses queued until master ready)

## 6.6	2.3.6 AXI Ordering Requirements

The crossbar maintains AXI ordering rules:

### 6.6.1	Read-After-Write (RAW) Ordering

Rule: Read from address must see data from earlier write to same address

Crossbar Behavior: - Ordering enforced at slave level (slave handles RAW within itself) - Crossbar does NOT reorder transactions to same slave from same master - Different masters to same slave: No ordering guaranteed (slave must handle)

### 6.6.2	Write-After-Write (WAW) Ordering

Rule: Writes to overlapping addresses must complete in issue order

Crossbar Behavior: - Same master to same slave: Order preserved by arbiter (FIFO grant queue) - Different masters to same slave: Slave responsible for write ordering

### 6.6.3	Out-of-Order (OOO) Completion

Allowed: - Read responses can return out-of-order (different RIDs) - Reads to different slaves can complete in any order - Writes to different slaves can complete in any order

Crossbar Support: - Bridge ID tracking enables OOO response routing - Master sees responses in slave-determined order - Multi-master OOO requires careful slave design

## 6.7	2.3.7 Monitor Aggregation at Bridge Top (when variants includes mon)

When monitor collection is enabled, per-port monitor streams from axi4_master_{rd,wr}_mon and axi4_slave_{rd,wr}_mon wrappers are aggregated through a tree of monbus_arbiter instances. The final aggregated stream feeds a single monbus_axil_group instance at the bridge top level:

Aggregation Hierarchy:

Master-side monitors → monbus_arbiter tree (if >2 masters)
Slave-side monitors  → monbus_arbiter tree (if >2 slaves)
                    ↓
            monbus_axil_group
                    ↓
      s_mon_axil (slave), m_axil_mon (master), stream_irq

The monbus_axil_group instance: - Provides a 64-bit slave AXIL interface for CPU read access (s_mon_axil_*) - Provides a master AXIL interface for DMA writes to system memory (m_axil_mon_*) - Contains an error FIFO tracking packets with error flags - Exposes stream_irq (asserted when error FIFO has records) - Includes an internal 64-bit free-running timestamp counter sampled at each packet arrival

Reference: See docs/markdown/rtl-amba/_book_monitor_index.md (monitor documentation book, with per-module pages under docs/markdown/rtl-amba/monitor/) for complete monitor design-surface documentation and docs/markdown/rtl-amba/includes/monitor_package_spec.md for the packet layout.

## 6.8	2.3.8 Resource Utilization

### 6.8.1	Crossbar Core Resources

4 masters × 3 slaves configuration (64-bit data, 32-bit addr):

Logic Elements:  ~2000-3500 LEs
Registers:       ~800-1200 regs
Block RAM:       0-4 KB (if CAM used)

Breakdown per slave:
- Arbiter (4 masters, RR):        ~200 LEs, ~50 regs
- Request MUX (AR/AW/W):          ~400 LEs, ~100 regs
- Response DEMUX (R/B):           ~300 LEs, ~80 regs
- Control FSMs:                   ~100 LEs, ~50 regs

Total for 3 slaves: 3 × 1000 LEs = ~3000 LEs
Plus routing overhead: +500 LEs

### 6.8.2	Scaling with Masters and Slaves

Linear scaling: - Adding 1 master: +~500 LEs per slave (new arbiter input) - Adding 1 slave: +~1000 LEs (complete new slave port)

Example: 8 masters × 6 slaves

Estimated:  ~12,000 LEs, ~3000 regs
Block RAM:  ~8 KB (for CAM if enabled)

### 6.8.3	Optimization Techniques

- Read-Only/Write-Only Masters: Reduces arbiter complexity by 40-50%

- Power-of-Two Master Count: Simplifies BID width and routing logic

- Pipeline Stages: Trading latency for frequency (deeper pipelines)

## 6.9	2.3.8 Timing Characteristics

### 6.9.1	Latency

Request Path (Master → Slave): - Arbiter decision: 1 cycle (registered) - MUX selection: 0 cycles (combinatorial) or +1 cycle (registered) - Total: 1-2 cycles through crossbar

Response Path (Slave → Master): - BID extraction: 0 cycles (combinatorial) - DEMUX routing: 0 cycles (combinatorial) or +1 cycle (registered) - Total: 0-1 cycles through crossbar

End-to-End (Master adapter → Slave adapter): - Adapters: 2 cycles (skid buffers) - Router: 0-1 cycles (decode) - Crossbar: 1-2 cycles (arbitration + MUX) - Total: 3-5 cycles minimum latency

### 6.9.2	Throughput

Best Case (no conflicts): - Each master to different slave: N transactions/cycle (N = master count) - Maximum aggregate bandwidth: N × data_width bits/cycle

Arbitration Limits: - Multiple masters to one slave: 1 transaction/cycle to that slave - Other slaves remain available for parallel access

Burst Performance: - Once granted, bursts flow at 1 beat/cycle - Grant held until burst completes (RLAST or WLAST)

## 6.10	2.3.9 Critical Paths

### 6.10.1	Common Critical Paths

- Arbiter Request → Grant:

  - All masters’ VALID signals → Arbiter logic → Grant decision

  - Depth: ~5-8 logic levels for 4 masters

- Grant → Ready Backpressure:

  - Slave READY → Arbiter → Selected master READY

  - Depth: ~4-6 logic levels

- Response Demux:

  - Slave RDATA/RID → BID extraction → Master select → Master RDATA

  - Depth: ~6-10 logic levels for 64-bit data

### 6.10.2	Mitigation Strategies

- Registered MUX/DEMUX: +1 cycle latency, breaks paths

- Pipelined Arbitration: Multi-cycle arbiter for >8 masters

- Hierarchical Crossbar: For >16 masters, use tree structure

## 6.11	2.3.10 Configuration Parameters

### 6.11.1	Crossbar Parameters (from TOML)

[bridge]
num_masters = 4
num_slaves = 3
internal_data_width = 64
arbiter_type = "round_robin"       # "round_robin", "fixed_priority", "weighted"
registered_mux = false             # true = +1 cycle, better timing
registered_demux = false           # true = +1 cycle, better timing
enable_cam = false                 # true = CAM-based routing
cam_depth = 16                     # Outstanding transactions tracked

## 6.12	2.3.11 Debug and Observability

### 6.12.1	Recommended Debug Signals

Per Slave:
- Arbiter grants (which master granted)
- Arbiter requests (which masters requesting)
- Request MUX outputs (VALID, READY, ADDR, ID)
- Response DEMUX inputs (VALID, READY, DATA, ID)

Global:
- Active transactions count
- Stall counters (arbiter conflicts)
- BID extraction errors
- CAM hit/miss (if enabled)

### 6.12.2	Performance Counters

Useful metrics for profiling:

- Transactions per slave (read, write separate)
- Arbiter conflict rate (requests denied due to grant  contention)
- Average grant latency
- Utilization per master (% cycles active)
- Utilization per slave (% cycles busy)

## 6.13	2.3.12 Common Issues and Debug

Symptom: Master hangs with VALID=1, READY=0
Check: - Is another master holding grant to this slave? - Is arbiter logic functioning (check grant signals)? - Is slave responding with READY?

Symptom: Response goes to wrong master
Check: - Bridge ID values (verify correct BID per master) - CAM contents (if used) - BID extraction logic (check bit positions)

Symptom: Throughput lower than expected
Check: - Arbiter conflicts (multiple masters to same slave?) - Pipeline depth (excessive latency reducing effective bandwidth?) - Burst efficiency (are bursts being granted properly?)

## 6.14	2.3.13 Verification Considerations

### 6.14.1	Functional Tests

- Single Master to Each Slave: Verify basic connectivity

- All Masters to One Slave: Stress arbiter fairness

- All Masters to All Slaves: Maximum parallelism test

- OOO Responses: Issue transactions with different latencies

- Burst Interleaving: Multiple masters with bursts to same slave

### 6.14.2	Corner Cases

- Back-to-back grants (no idle cycles)
- Grant held for maximum burst length (256 beats)
- Rapid master switching (each gets 1 transaction then switches)
- Response while request in progress (pipelining)
- All masters requesting same slave simultaneously

### 6.14.3	Protocol Compliance

- AXI4 protocol checker at each master/slave interface

- Verify no READY → VALID dependencies (AXI violation)

- Check ID preservation through crossbar (modulo Bridge ID)

- Verify LAST signal handling

## 6.15	2.3.14 Future Enhancements

### 6.15.1	Planned Features

- Weighted Round-Robin: QoS support with configurable priorities

- Slave-Side Arbitration Policies: Per-slave arbiter configuration

- Grant Prediction: Speculative grant for lower latency

- Congestion Control: Throttling to prevent hotspots

### 6.15.2	Under Consideration

- Partial Crossbar: Configurable master-to-slave connectivity (not full mesh)

- Multi-Tier Hierarchy: For 32+ masters/slaves

- Virtual Channels: Separate channels for different traffic classes

- Register Slicing: Automatic pipeline insertion for timing

Related Sections: - Section 2.1: Master Adapter (request sources) - Section 2.2: Slave Router (address decode before arbitration) - Section 2.4: Arbitration (detailed arbiter algorithms) - Section 2.5: ID Management (CAM structures, Bridge ID tracking) - Section 3.1: Top-Level Integration (crossbar instantiation)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	2.4 Arbitration

When several masters ask for the same slave in the same cycle, something has to pick a winner — that’s the arbiter. Each slave gets dedicated arbiters for its read and write channels, so the pick is fair and nothing queues behind the wrong channel.

## 7.1	2.4.1 Purpose and Function

The arbiter does five things:

- Conflict Resolution: Selects one master when multiple masters request same slave

- Fairness: Ensures all masters eventually get access (starvation-free)

- Throughput Optimization: Minimizes idle cycles and maximizes utilization

- Grant Management: Maintains grants for burst transactions

- Priority Enforcement: Supports priority-based access policies (optional)

## 7.2	2.4.2 Arbitration Architecture

### 7.2.1	Per-Slave, Per-Channel Arbiters

Each slave has two independent arbiters:

Slave 0:
  - AR Arbiter (Read Address Channel)
    * Inputs: M0_ARVALID, M1_ARVALID, M2_ARVALID, M3_ARVALID
    * Output: Grant[1:0] → Selects one master
    
  - AW Arbiter (Write Address Channel)
    * Inputs: M0_AWVALID, M1_AWVALID, M2_AWVALID, M3_AWVALID
    * Output: Grant[1:0] → Selects one master
    * Coordinates with W channel data flow

Independence: AR and AW arbiters operate independently, allowing simultaneous read and write to same slave (if slave supports full-duplex operation).

Block Diagram

## 7.3	2.4.3 Round-Robin Arbitration

### 7.3.1	Algorithm

The Round-Robin (RR) arbiter cycles through masters in order, giving each master one opportunity to access the slave:

Priority Order (rotates after each grant):
  Cycle 0: M0 > M1 > M2 > M3
  Cycle 1: M1 > M2 > M3 > M0  (M0 was granted, now lowest priority)
  Cycle 2: M2 > M3 > M0 > M1  (M1 was granted)
  Cycle 3: M3 > M0 > M1 > M2  (M2 was granted)
  Cycle 4: M0 > M1 > M2 > M3  (M3 was granted, cycle repeats)

### 7.3.2	Implementation

// Simplified Round-Robin arbiter (4 masters)
logic [1:0] last_grant;      // Which master was granted last
logic [3:0] request;         // Current requests (VALID signals)
logic [1:0] grant;           // Grant decision

always_ff @(posedge clk) begin
    if (!rst_n) begin
        last_grant <= 2'b00;
    end else if (|request && slave_ready) begin
        // Update last_grant when transaction completes
        if (grant_accepted) begin
            last_grant <= grant;
        end
    end
end

always_comb begin
    // Default: No grant
    grant = 2'b00;
    
    // Priority encode starting after last grant
    case (last_grant)
        2'b00: begin  // Last grant to M0, try M1, M2, M3, M0
            if (request[1])      grant = 2'b01;
            else if (request[2]) grant = 2'b10;
            else if (request[3]) grant = 2'b11;
            else if (request[0]) grant = 2'b00;
        end
        2'b01: begin  // Last grant to M1, try M2, M3, M0, M1
            if (request[2])      grant = 2'b10;
            else if (request[3]) grant = 2'b11;
            else if (request[0]) grant = 2'b00;
            else if (request[1]) grant = 2'b01;
        end
        // ... similar for M2, M3
    endcase
end

### 7.3.3	Characteristics

Fairness: Excellent - Each master gets equal access over time
Latency: Bounded - Maximum wait = (N-1) × transaction_time
Throughput: Optimal - No idle cycles when requests pending
Starvation: None - Every master eventually gets grant

Best Use Cases: - Peers with similar priority - General-purpose interconnects - Default choice for most bridges

## 7.4	2.4.4 Fixed-Priority Arbitration

### 7.4.1	Algorithm

The Fixed-Priority arbiter always grants to the highest-priority requesting master:

Priority Order (never changes):
  M0 > M1 > M2 > M3  (M0 always highest priority)

Grant Decision:
  if (M0_VALID) → Grant = M0
  else if (M1_VALID) → Grant = M1
  else if (M2_VALID) → Grant = M2
  else if (M3_VALID) → Grant = M3

### 7.4.2	Implementation

// Fixed-priority arbiter (4 masters)
logic [3:0] request;
logic [1:0] grant;

always_comb begin
    // Priority encoder: M0 highest, M3 lowest
    if (request[0])      grant = 2'b00;  // M0 highest priority
    else if (request[1]) grant = 2'b01;
    else if (request[2]) grant = 2'b10;
    else if (request[3]) grant = 2'b11;  // M3 lowest priority
    else                 grant = 2'b00;  // Default (no requests)
end

### 7.4.3	Characteristics

Fairness: Poor - Low-priority masters can starve
Latency: Variable - High-priority: minimal, Low-priority: unbounded
Throughput: Optimal - No idle cycles when requests pending
Starvation: Possible for low-priority masters

Best Use Cases: - Real-time systems with priority requirements - CPU (high) vs. DMA (low) scenarios - Time-critical masters need guaranteed access

### 7.4.4	Starvation Prevention

Optional enhancement: Priority aging

- Track cycles since last grant per master
- Temporarily boost priority of starved masters
- Reset age counter after grant

Example:
  M0 age = 100 cycles → Boost M0 above normal priority
  M1 age = 5 cycles   → Normal priority

## 7.5	2.4.5 Weighted Arbitration

### 7.5.1	Algorithm

The Weighted arbiter assigns different access rates to masters based on weights:

Configuration:
  M0 weight = 4  (50% of bandwidth)
  M1 weight = 2  (25% of bandwidth)
  M2 weight = 1  (12.5% of bandwidth)
  M3 weight = 1  (12.5% of bandwidth)

Grant Sequence (repeating pattern):
  M0, M0, M0, M0, M1, M1, M2, M3  (8 cycles total)
  M0 gets 4/8, M1 gets 2/8, M2 gets 1/8, M3 gets 1/8

### 7.5.2	Implementation

// Weighted arbiter using credit counter
logic [7:0] credits [0:3];  // Credit counters per master
logic [1:0] grant;

always_ff @(posedge clk) begin
    if (!rst_n) begin
        credits[0] <= 8'd4;  // M0 weight
        credits[1] <= 8'd2;  // M1 weight
        credits[2] <= 8'd1;  // M2 weight
        credits[3] <= 8'd1;  // M3 weight
    end else if (grant_accepted) begin
        // Decrement granted master's credits
        credits[grant] <= credits[grant] - 1;
        
        // Reload when all credits exhausted
        if (all_credits_zero) begin
            credits[0] <= 8'd4;
            credits[1] <= 8'd2;
            credits[2] <= 8'd1;
            credits[3] <= 8'd1;
        end
    end
end

always_comb begin
    // Grant to master with highest credits and active request
    grant = select_highest_credit_with_request(credits, request);
end

### 7.5.3	Characteristics

Fairness: Configurable - Proportional to weights
Latency: Bounded - Depends on weight ratio
Throughput: Near-optimal - Small overhead for credit tracking
Starvation: None - All masters get share per weight

Best Use Cases: - QoS requirements (guaranteed bandwidth) - Mixed workload (video + CPU + DMA) - Service-level agreements

## 7.6	2.4.6 Burst Handling

### 7.6.1	Grant Locking

Once granted, a master holds the grant until its burst completes:

AR Channel Burst:
  1. Master asserts ARVALID with ARLEN = 7 (8-beat burst)
  2. Arbiter grants to this master
  3. Grant held until slave returns RLAST
  4. Other masters blocked during this time

AW/W Channel Burst:
  1. Master asserts AWVALID with AWLEN = 15 (16-beat burst)
  2. Arbiter grants to this master
  3. Grant held until master asserts WLAST
  4. Other masters blocked during W data transfer

### 7.6.2	Implementation

// Burst grant locking
logic grant_locked;
logic [1:0] locked_master;

always_ff @(posedge clk) begin
    if (!rst_n) begin
        grant_locked <= 1'b0;
    end else begin
        if (grant_accepted && !last_beat) begin
            // Lock grant for burst
            grant_locked <= 1'b1;
            locked_master <= grant;
        end else if (last_beat) begin
            // Release grant when burst completes
            grant_locked <= 1'b0;
        end
    end
end

assign grant = grant_locked ? locked_master : arbiter_decision;

### 7.6.3	Fairness Considerations

Long bursts can block other masters: - Maximum AXI burst: 256 beats - At 1 cycle/beat: Up to 256 cycles blocked - Impacts latency for other masters

Mitigation: - Limit burst lengths in master configuration - Use burst interleaving (complex, not implemented in Phase 1) - Monitor arbiter stall counters

## 7.7	2.4.7 Resource Utilization

### 7.7.1	Per-Arbiter Resources

Round-Robin (4 masters):

Logic Elements:  ~150 LEs
Registers:       ~40 regs

Breakdown:
- Priority encoder:        ~80 LEs
- Last grant tracking:     ~10 regs
- Request capture:         ~20 regs
- Grant FSM:               ~40 LEs, ~10 regs

Fixed-Priority (4 masters):

Logic Elements:  ~80 LEs
Registers:       ~20 regs

Simpler than RR (no rotation logic)

Weighted (4 masters):

Logic Elements:  ~200 LEs
Registers:       ~60 regs

Additional credit counters and comparison logic

### 7.7.2	Scaling with Master Count

Resource usage scales approximately O(N²) where N = master count:

2 masters:  ~50 LEs
4 masters:  ~150 LEs
8 masters:  ~400 LEs
16 masters: ~1200 LEs

The N² scaling comes from: - Priority encoder: Compares all pairs of masters - Grant MUX: N-to-1 multiplexing increases with N

## 7.8	2.4.8 Timing Characteristics

### 7.8.1	Arbitration Latency

Single-Cycle Arbitration (default):

Request → Grant Decision: 1 cycle
Grant Decision → MUX Output: 0 cycles (combinatorial)
Total: 1 cycle

Multi-Cycle Arbitration (high frequency):

Request → Grant Decision: 2-3 cycles (pipelined)
Improves timing at cost of latency

### 7.8.2	Critical Paths

Typical critical paths in arbiter: 1. Request collection: All master VALID signals → Arbiter input 2. Priority encode: Compare all requests → Grant decision 3. Grant propagate: Grant → MUX select → Slave interface

Path Depth: - 4 masters: ~6-8 logic levels - 8 masters: ~8-12 logic levels - 16 masters: ~12-16 logic levels

## 7.9	2.4.9 Configuration Parameters

### 7.9.1	Arbiter Configuration (TOML)

[bridge]
arbiter_type = "round_robin"    # "round_robin", "fixed_priority", "weighted"

[bridge.arbitration]
pipeline_stages = 1              # 1-3 (more = better timing, higher latency)
enable_priority_aging = false    # Prevent starvation in fixed-priority
aging_threshold = 1000           # Cycles before priority boost

# Weighted arbitration weights (only if arbiter_type = "weighted")
[[bridge.arbitration.weights]]
master = "cpu"
weight = 4                       # Relative weight (1-255)

[[bridge.arbitration.weights]]
master = "dma"
weight = 2

[[bridge.arbitration.weights]]
master = "periph"
weight = 1

## 7.10	2.4.10 Debug and Observability

### 7.10.1	Recommended Debug Signals

Per Arbiter:
- Request vector (all masters requesting)
- Grant decision (which master granted)
- Grant locked status (burst in progress)
- Last grant (for RR rotation tracking)

Performance Counters:
- Grants per master (utilization)
- Denied requests per master (contention)
- Average grant latency per master
- Arbiter conflict cycles (multiple requests, one grant)

### 7.10.2	Common Issues and Debug

Symptom: Master never gets grant (starvation)
Check: - Arbiter type (fixed-priority can starve low-priority masters) - Other masters continuously requesting? - Enable priority aging if using fixed-priority

Symptom: Unfair access (one master dominates)
Check: - Arbiter type (should be round-robin for fairness) - Burst lengths (long bursts block others) - Request patterns (one master requesting more frequently)

Symptom: Low throughput despite requests
Check: - Arbiter conflicts (too many masters to one slave) - Pipeline depth (excessive arbitration latency) - Burst efficiency (short bursts waste cycles)

## 7.11	2.4.11 Verification Considerations

### 7.11.1	Test Scenarios

- Fair Access Test (Round-Robin):

- All masters continuously request same slave
- Verify each master gets equal grants over 1000 cycles
- Expected: Each of 4 masters gets ~250 grants

- Priority Test (Fixed-Priority):

- High-priority master requests intermittently
- Low-priority masters request continuously
- Verify high-priority always wins when requesting

- Burst Locking Test:

- Master issues 16-beat burst
- Other masters request during burst
- Verify grant held until LAST
- Verify other masters blocked during burst

- Weighted Bandwidth Test:

- Configure M0=4, M1=2, M2=1, M3=1
- All masters request continuously for 10000 cycles
- Verify grants proportional: M0≈50%, M1≈25%, M2≈12.5%, M3≈12.5%

### 7.11.2	Corner Cases

- Single master requesting (trivial grant)
- All masters requesting simultaneously (worst-case arbitration)
- Grant released and new grant same cycle (back-to-back)
- Master drops request after arbiter latency (stale grant)
- Maximum length burst (256 beats blocking)

## 7.12	2.4.12 Performance Impact

### 7.12.1	Arbiter Choice Impact

Round-Robin: - Pros: Fair, starvation-free, predictable latency - Cons: Cannot prioritize time-critical masters - Use: General-purpose, peer masters

Fixed-Priority: - Pros: Guaranteed low latency for high-priority masters - Cons: Low-priority can starve, unpredictable for low-priority - Use: Real-time, priority-sensitive systems

Weighted: - Pros: Configurable bandwidth allocation, QoS support - Cons: More complex, small overhead - Use: Mixed workload, SLA requirements

### 7.12.2	Arbiter Efficiency

Assuming all masters request same slave continuously:

Best Case: 100% efficiency (one master)
  - No arbitration conflicts
  - Zero idle cycles

Typical Case: 25-50% per-master efficiency (4 masters, round-robin)
  - Each master gets ~25% of time grants
  - Load balancing to other slaves improves

Worst Case: Heavy contention
  - 4 masters to 1 slave: 25% efficiency per master
  - Solution: Add slaves or use more slaves in parallel

## 7.13	2.4.13 Future Enhancements

### 7.13.1	Planned Features

- Dynamic Weight Adjustment: Runtime-configurable weights via registers

- QoS Classes: Multiple priority levels with configurable policies

- Deadline-Based Arbitration: Grant based on transaction deadlines

- Predictive Arbitration: Speculative grants to reduce latency

### 7.13.2	Under Consideration

- Multi-Level Arbitration: Hierarchical for >16 masters

- Token Bucket: Rate limiting per master

- History-Based: Learn access patterns and optimize grants

- ECC Protection: For arbitration state (safety-critical systems)

Related Sections: - Section 2.3: Crossbar Core (arbiter integration) - Section 2.1: Master Adapter (request sources) - Section 2.5: ID Management (transaction tracking during grants) - Chapter 6: Performance (arbiter impact on throughput)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	2.5 ID Management

ID Management is how the bridge remembers which master sent each outstanding transaction, so the response can find its way back. Three pieces do the work: Bridge ID injection, Content Addressable Memory (CAM) structures, and ID translation logic.

## 8.1	2.5.1 Purpose and Function

The ID management system does five things:

- Transaction Tracking: Maintains association between requests and originating masters

- Response Routing: Directs responses to correct master using embedded IDs

- Out-of-Order Support: Handles responses returning in different order than issued

- ID Space Isolation: Prevents ID conflicts between multiple masters

- Burst Management: Tracks multi-beat bursts through the bridge

## 8.2	2.5.2 Bridge ID Concept

### 8.2.1	Problem Statement

Without ID management, the bridge cannot determine which master originated a transaction:

Scenario: Two masters with overlapping IDs
  Master 0: Issues read with ARID = 4'h5
  Master 1: Issues read with ARID = 4'h5 (same ID!)
  
When slave responds with RID = 4'h5, which master gets the response?
  → Ambiguous! Need additional information.

### 8.2.2	Solution: Bridge ID Injection

The bridge adds a Bridge ID (BID) to each transaction ID:

Internal ID = {Bridge_ID, Original_ID}

Master 0, ARID = 4'h5:
  Internal ARID = {2'b00, 4'h5} = 6'b00_0101
  
Master 1, ARID = 4'h5:
  Internal ARID = {2'b01, 4'h5} = 6'b01_0101
  
Now responses with RID = 6'b00_0101 → Route to Master 0
                    RID = 6'b01_0101 → Route to Master 1

## 8.3	2.5.3 Block Diagram

### 8.3.1	Figure 2.5: ID Management Architecture

ID Management Architecture

ID management architecture showing CAM and FIFO modes for transaction tracking and response routing.

## 8.4	2.5.4 Bridge ID Width Calculation

### 8.4.1	Formula

BID_WIDTH = clog2(NUM_MASTERS)

Where clog2(x) = ceiling(log2(x))

### 8.4.2	Examples

2 masters:  clog2(2)  = 1 bit   (BID: 0, 1)
3 masters:  clog2(3)  = 2 bits  (BID: 00, 01, 10)
4 masters:  clog2(4)  = 2 bits  (BID: 00, 01, 10, 11)
5 masters:  clog2(5)  = 3 bits  (BID: 000-100)
8 masters:  clog2(8)  = 3 bits  (BID: 000-111)
16 masters: clog2(16) = 4 bits  (BID: 0000-1111)

### 8.4.3	ID Width Growth

Configuration: 4 masters, external ARID_WIDTH = 4

Internal ARID_WIDTH = BID_WIDTH + External ARID_WIDTH
                    = 2 + 4
                    = 6 bits

Slave sees 6-bit IDs
Master sees 4-bit IDs
Bridge translates between them

## 8.5	2.5.4a Multi-Master OR-Merge Gating

When multiple masters drive address and ID signals toward a slave, an OR-merge combines them. To prevent idle masters with stale addresses from corrupting the routing tag, every OR-merge term is gated on both the slave_select condition AND the valid signal:

// Wrong (deprecated):
assign s0_bridge_id_aw = m0_bridge_id_aw | m1_bridge_id_aw | m2_bridge_id_aw;
//                       ↓ Master 1 contributes even if awvalid=0 and address is stale!

// Correct:
assign s0_bridge_id_aw = (m0_slave_select_aw && m0_awvalid ? m0_bridge_id_aw : '0) |
                        (m1_slave_select_aw && m1_awvalid ? m1_bridge_id_aw : '0) |
                        (m2_slave_select_aw && m2_awvalid ? m2_bridge_id_aw : '0);

Why this matters: An idle master with stale address (fub_axi_awaddr = 0) still decodes to slave 0 if that’s the address map. Without valid gating, its zero bridge_id corrupts the slave’s routing tag via OR-merge. When the slave responds, the B-response gets routed to the idle master instead of the actual requester.

Implementation: - All AW/AR/W data signals gated by {slave_select && <channel_valid>} - bridge_id_aw/ar signals gated by corresponding <channel>valid - Prevents any idle master from affecting the OR-merge result

## 8.6	2.5.5 ID Injection (Request Path)

### 8.6.1	Read Address Channel (AR)

// ID injection at master adapter
logic [BID_WIDTH-1:0] master_bid;         // Constant per master
logic [ARID_WIDTH-1:0] external_arid;     // From master
logic [TOTAL_ARID_WIDTH-1:0] internal_arid;  // To crossbar

assign master_bid = MASTER_INDEX;  // M0=0, M1=1, M2=2, M3=3
assign internal_arid = {master_bid, external_arid};

// Example: Master 2, ARID = 4'h7
// internal_arid = {2'b10, 4'h7} = 6'b10_0111

### 8.6.2	Write Address Channel (AW)

// ID injection for write transactions
logic [BID_WIDTH-1:0] master_bid;
logic [AWID_WIDTH-1:0] external_awid;
logic [TOTAL_AWID_WIDTH-1:0] internal_awid;

assign internal_awid = {master_bid, external_awid};

// Note: AWID and ARID widths can differ per master

### 8.6.3	Injection Timing

Combinatorial (default): - ID concatenation done in same cycle as VALID assertion - Zero latency overhead - May contribute to critical path in high-frequency designs

Registered (optional): - Add pipeline register after ID injection - +1 cycle latency - Breaks critical path

## 8.7	2.5.6 ID Extraction (Response Path)

### 8.7.1	Read Data Channel (R)

// ID extraction at crossbar response router
logic [TOTAL_RID_WIDTH-1:0] internal_rid;   // From slave
logic [BID_WIDTH-1:0] extracted_bid;        // Upper bits
logic [RID_WIDTH-1:0] external_rid;         // Lower bits

// Extract Bridge ID
assign extracted_bid = internal_rid[TOTAL_RID_WIDTH-1:RID_WIDTH];

// Strip Bridge ID for master
assign external_rid = internal_rid[RID_WIDTH-1:0];

// Route based on BID
always_comb begin
    case (extracted_bid)
        2'b00: master0_rvalid = slave_rvalid;
        2'b01: master1_rvalid = slave_rvalid;
        2'b10: master2_rvalid = slave_rvalid;
        2'b11: master3_rvalid = slave_rvalid;
    endcase
end

### 8.7.2	Write Response Channel (B)

// Similar extraction for write responses
logic [TOTAL_BID_WIDTH-1:0] internal_bid;
logic [BID_WIDTH-1:0] extracted_bid;
logic [BID_WIDTH-1:0] external_bid;

assign extracted_bid = internal_bid[TOTAL_BID_WIDTH-1:BID_WIDTH];
assign external_bid = internal_bid[BID_WIDTH-1:0];

// Route to originating master

## 8.8	2.5.7 Content Addressable Memory (CAM)

### 8.8.1	When to Use CAM

Simple Configurations (No CAM needed): - Single master (BID unnecessary) - Direct ID mapping suffices - Lower resource usage

Complex Configurations (CAM beneficial): - Many masters (>8) - Out-of-order responses common - Multiple outstanding transactions per master - ID reordering within slave

### 8.8.2	CAM Structure

Entry Format:
┌────────────┬──────────────┬─────────────┬───────────┬───────┐
│ Internal ID│ Master Index │ Transaction │ Timestamp │ Valid │
│  (6-12 bits│  (2-4 bits)  │    Type     │ (counter) │ (1b)  │
│            │              │   (R/W)     │           │       │
└────────────┴──────────────┴─────────────┴───────────┴───────┘
     10b           3b             1b          16b         1b

Total per entry: ~30 bits
CAM depth: 16-64 entries typical

### 8.8.3	CAM Operations

Allocation (Request Path):

1. Master issues AR/AW transaction
2. Find free CAM entry (Valid = 0)
3. Write entry:
   - Internal ID = {BID, External_ID}
   - Master Index = BID
   - Transaction Type = Read or Write
   - Timestamp = Current cycle count
   - Valid = 1
4. Forward transaction to slave

Lookup (Response Path):

1. Slave returns R/B response with Internal ID
2. CAM lookup: Search for matching Internal ID
3. Extract Master Index from matching entry
4. Route response to Master[Master Index]
5. If LAST: Deallocate entry (Valid = 0)

Associative Search: - All entries checked in parallel - 1-cycle lookup (registered CAM) - Match found → Returns master index - No match → Error condition (protocol violation)

## 8.9	2.5.8 CAM Implementation

### 8.9.1	Parallel CAM (Fast, Resource-Intensive)

// Parallel CAM structure (16 entries)
typedef struct packed {
    logic valid;
    logic [TOTAL_ID_WIDTH-1:0] internal_id;
    logic [BID_WIDTH-1:0] master_idx;
    logic txn_type;  // 0=read, 1=write
    logic [15:0] timestamp;
} cam_entry_t;

cam_entry_t cam [0:15];

// Parallel search
logic [15:0] match;
logic [3:0] match_idx;

for (genvar i = 0; i < 16; i++) begin
    assign match[i] = cam[i].valid && 
                      (cam[i].internal_id == response_id);
end

// Priority encode first match
assign match_idx = find_first_set(match);
assign routed_master = cam[match_idx].master_idx;

### 8.9.2	Sequential CAM (Slow, Resource-Efficient)

// Sequential CAM search (multi-cycle)
logic [3:0] search_idx;
logic searching;

always_ff @(posedge clk) begin
    if (search_start) begin
        search_idx <= 0;
        searching <= 1'b1;
    end else if (searching) begin
        if (cam[search_idx].valid && 
            cam[search_idx].internal_id == response_id) begin
            // Match found
            routed_master <= cam[search_idx].master_idx;
            searching <= 1'b0;
        end else if (search_idx == 15) begin
            // No match found (error)
            searching <= 1'b0;
            error <= 1'b1;
        end else begin
            search_idx <= search_idx + 1;
        end
    end
end

Trade-off: - Parallel: 1-cycle, ~2000 LEs for 16 entries - Sequential: 16 cycles, ~200 LEs for 16 entries

## 8.10	2.5.9 Outstanding Transaction Limits

### 8.10.1	Configuration

[bridge]
enable_cam = true
cam_depth = 16              # Max outstanding transactions

[[masters]]
name = "cpu"
max_outstanding_reads = 8   # Per-master limit
max_outstanding_writes = 8

### 8.10.2	Enforcement

When CAM full:

1. Master issues new request
2. Check CAM for free entry
3. If full:
   - ARREADY/AWREADY = 0 (backpressure)
   - Wait for response to free entry
4. When entry freed (RLAST/BVALID):
   - Accept new request

### 8.10.3	Sizing Guidelines

CAM Depth = Σ(max_outstanding per master) + Safety margin

Example: 4 masters, 4 outstanding each
  Minimum CAM depth = 4 × 4 = 16 entries
  Recommended depth = 20 entries (25% margin)

## 8.11	2.5.10 Resource Utilization

### 8.11.1	ID Injection/Extraction Only

Per Master (no CAM):

Logic Elements:  ~20-50 LEs
Registers:       ~10 regs

Simple bit concatenation and extraction
Minimal overhead

### 8.11.2	With CAM

CAM Resources (16 entries, 6-bit IDs):

Parallel CAM:
  Logic Elements:  ~2000 LEs
  Registers:       ~500 regs
  Block RAM:       0 (distributed)

BRAM-Based CAM:
  Logic Elements:  ~500 LEs
  Registers:       ~100 regs
  Block RAM:       1-2 KB

Sequential CAM:
  Logic Elements:  ~200 LEs
  Registers:       ~100 regs
  Block RAM:       0

### 8.11.3	Scaling

CAM Depth      Parallel      BRAM         Sequential
16 entries     ~2000 LEs     ~500 LEs     ~200 LEs
32 entries     ~4000 LEs     ~800 LEs     ~250 LEs
64 entries     ~8000 LEs     ~1200 LEs    ~300 LEs

## 8.12	2.5.11 Timing Characteristics

### 8.12.1	ID Injection Latency

Combinatorial (default): - 0 cycles overhead - Part of adapter pipeline stage

Registered: - +1 cycle latency - Breaks timing path

### 8.12.2	ID Extraction/Routing Latency

Direct Decode (no CAM): - 0 cycles (combinatorial BID extraction)

Parallel CAM: - 1 cycle (registered search)

Sequential CAM: - 1-N cycles (where N = CAM depth) - Average: N/2 cycles

### 8.12.3	End-to-End Impact

Configuration: No CAM, combinatorial ID management
  Request:  Master → +0 cycles → Crossbar
  Response: Slave → +0 cycles → Master
  Total: 0 cycles overhead

Configuration: Parallel CAM, registered
  Request:  Master → +1 cycle (allocation) → Crossbar
  Response: Slave → +1 cycle (lookup) → Master
  Total: 2 cycles overhead

## 8.13	2.5.12 Configuration Parameters

### 8.13.1	ID Management Configuration (TOML)

[bridge]
num_masters = 4
enable_cam = false              # Use CAM for ID tracking
cam_type = "parallel"           # "parallel", "bram", "sequential"
cam_depth = 16                  # Outstanding transaction capacity

[bridge.id_management]
registered_injection = false    # Register ID injection (+1 cycle)
registered_extraction = false   # Register ID extraction (+1 cycle)
enable_timeout = true           # Detect hung transactions
timeout_cycles = 10000          # Cycles before timeout error

[[masters]]
name = "cpu"
arid_width = 4                  # External ID width
awid_width = 4
max_outstanding_reads = 8       # CAM allocation limit
max_outstanding_writes = 8

## 8.14	2.5.13 Debug and Observability

### 8.14.1	Recommended Debug Signals

ID Injection:
- Master BID assignments (constant per master)
- External IDs (from masters)
- Internal IDs (after injection)

ID Extraction:
- Internal IDs (from slaves)
- Extracted BIDs
- External IDs (to masters)

CAM (if enabled):
- CAM occupancy (number of valid entries)
- Allocation/deallocation events
- Search hits/misses
- Timeout events

### 8.14.2	Performance Counters

- Total transactions tracked
- CAM hit rate
- CAM miss rate (should be 0)
- Average CAM occupancy
- Peak CAM occupancy
- Timeout errors
- ID width overhead (bits added per transaction)

## 8.15	2.5.14 Common Issues and Debug

Symptom: Response goes to wrong master
Check: - BID assignment (verify each master has unique BID) - ID width calculation (BID_WIDTH correct?) - Bit slicing in extraction logic - CAM contents (if used)

Symptom: CAM full, transactions stalling
Check: - CAM depth vs. outstanding transaction count - Are responses being received? (slave responsive?) - Timeout threshold (too short?) - Leaked entries (not deallocated after LAST)

Symptom: CAM miss errors
Check: - Slave returning incorrect IDs - ID corruption in transit - Race condition (deallocation before response complete)

## 8.16	2.5.15 Verification Considerations

### 8.16.1	Test Scenarios

- Basic ID Injection/Extraction:

- Single master, single transaction
- Verify BID added correctly
- Verify BID stripped before response delivery
- Check external ID unchanged

- Multi-Master ID Isolation:

- Multiple masters with same external IDs
- Verify responses route to correct master
- Check BID uniqueness prevents collisions

- CAM Capacity:

- Issue max_outstanding transactions
- Verify backpressure when CAM full
- Issue responses to free entries
- Verify new transactions accepted

- Out-of-Order Responses:

- Issue transactions: ID=1, ID=2, ID=3
- Return responses: ID=3, ID=1, ID=2
- Verify CAM correctly routes each

- CAM Timeout:

- Issue transaction
- Slave doesn't respond within timeout_cycles
- Verify timeout error signaled
- Verify CAM entry deallocated (or flagged)

## 8.17	2.5.16 Future Enhancements

### 8.17.1	Planned Features

- Dynamic CAM Depth: Runtime adjustment based on utilization

- Per-Master CAM Partition: Guaranteed entries per master

- ID Compression: Reduce internal ID width for slaves with limited ID support

- Transaction Ordering: CAM tracks issue order for reordering enforcement

### 8.17.2	Under Consideration

- Multi-Level CAM: Hierarchical for large transaction counts

- TCAM Support: Partial ID matching (wildcards)

- Error Injection: Debug mode to test error handling

- CAM Mirrors: Redundant CAMs for fail-safe operation

Related Sections: - Section 2.1: Master Adapter (BID injection location) - Section 2.3: Crossbar Core (BID extraction, response routing) - Section 2.8: Response Routing (detailed routing logic) - Section 3.2: Master Port Interface (ID width specifications)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	2.6 Width Conversion

Masters and slaves rarely agree on data width, and width conversion is how the bridge copes. The internal data path is 64 bits; conversion logic at the master and slave interfaces adapts whatever narrower or wider widths the ports bring.

## 9.1	2.6.1 Purpose and Function

Width conversion does five things:

- Data Path Adaptation: Converts between different data widths (8, 16, 32, 64, 128, 256 bits)

- Burst Splitting: Divides wide transactions into multiple narrow transactions

- Burst Merging: Combines multiple narrow beats into fewer wide beats

- Strobe Mapping: Translates write strobes across width boundaries

- Address Alignment: Adjusts addresses for width differences

## 9.2	2.6.2 Internal Data Width

### 9.2.1	64-Bit Standard

The bridge uses 64-bit (8-byte) internal data width for the crossbar:

Rationale:
- Balance between resource usage and performance
- Common width for modern embedded processors
- Efficient for both 32-bit and 64-bit masters
- Reasonably sized multiplexers in crossbar

### 9.2.2	Width Conversion Locations

Master (32-bit) → [Upsizer] → Crossbar (64-bit) → [Downsizer] → Slave (32-bit)
Master (64-bit) → [No conversion] → Crossbar (64-bit) → [No conversion] → Slave (64-bit)
Master (128-bit) → [Downsizer] → Crossbar (64-bit) → [Upsizer] → Slave (128-bit)

## 9.3	2.6.3 Block Diagram

### 9.3.1	Figure 2.6: Width Conversion Architecture

Width Conversion Architecture

Width conversion architecture showing data upsizing and downsizing with beat count adjustment and strobe handling.

## 9.4	2.6.4 Upsizing (Narrow to Wide)

### 9.4.1	Overview

Upsizing converts narrow data to wide data by buffering multiple narrow beats into a single wide beat.

Example: 32-bit master → 64-bit crossbar
  Master beat 0: 32'hAAAA_BBBB (bytes 3:0)
  Master beat 1: 32'hCCCC_DDDD (bytes 7:4)
  → Crossbar beat: 64'hCCCC_DDDD_AAAA_BBBB

### 9.4.2	Write Upsizing

Burst of 4 (32-bit) → Burst of 2 (64-bit):

Master AW: AWADDR = 0x1000, AWLEN = 3, AWSIZE = 2 (4 bytes)
Master W beats:
  W[0]: WDATA = 0xAAAA_BBBB, WSTRB = 4'b1111, WLAST = 0
  W[1]: WDATA = 0xCCCC_DDDD, WSTRB = 4'b1111, WLAST = 0
  W[2]: WDATA = 0xEEEE_FFFF, WSTRB = 4'b1111, WLAST = 0
  W[3]: WDATA = 0x1111_2222, WSTRB = 4'b1111, WLAST = 1

Crossbar AW: AWADDR = 0x1000, AWLEN = 1, AWSIZE = 3 (8 bytes)
Crossbar W beats:
  W[0]: WDATA = 0xCCCC_DDDD_AAAA_BBBB, WSTRB = 8'b1111_1111, WLAST = 0
  W[1]: WDATA = 0x1111_2222_EEEE_FFFF, WSTRB = 8'b1111_1111, WLAST = 1

### 9.4.3	Read Upsizing

Burst of 8 (32-bit) → Burst of 4 (64-bit):

Master AR: ARADDR = 0x2000, ARLEN = 7, ARSIZE = 2 (4 bytes)

Crossbar AR: ARADDR = 0x2000, ARLEN = 3, ARSIZE = 3 (8 bytes)

Crossbar R beats:
  R[0]: RDATA = 0x1111_2222_3333_4444, RLAST = 0
  R[1]: RDATA = 0x5555_6666_7777_8888, RLAST = 0
  R[2]: RDATA = 0x9999_AAAA_BBBB_CCCC, RLAST = 0
  R[3]: RDATA = 0xDDDD_EEEE_FFFF_0000, RLAST = 1

Master R beats (split from crossbar):
  R[0]: RDATA = 0x3333_4444, RLAST = 0  (from R[0] low)
  R[1]: RDATA = 0x1111_2222, RLAST = 0  (from R[0] high)
  R[2]: RDATA = 0x7777_8888, RLAST = 0  (from R[1] low)
  R[3]: RDATA = 0x5555_6666, RLAST = 0  (from R[1] high)
  R[4]: RDATA = 0xBBBB_CCCC, RLAST = 0  (from R[2] low)
  R[5]: RDATA = 0x9999_AAAA, RLAST = 0  (from R[2] high)
  R[6]: RDATA = 0xFFFF_0000, RLAST = 0  (from R[3] low)
  R[7]: RDATA = 0xDDDD_EEEE, RLAST = 1  (from R[3] high)

### 9.4.4	Strobe Mapping (Upsizing)

32-bit WSTRB → 64-bit WSTRB:

Beat 0 (lower 32 bits):
  32-bit WSTRB = 4'b1010 → 64-bit WSTRB = 8'b0000_1010

Beat 1 (upper 32 bits):
  32-bit WSTRB = 4'b1111 → 64-bit WSTRB = 8'b1111_0000

Combined:
  64-bit WSTRB = 8'b1111_1010

## 9.5	2.6.5 Downsizing (Wide to Narrow)

### 9.5.1	Overview

Downsizing converts wide data to narrow data by splitting a single wide beat into multiple narrow beats.

Example: 128-bit master → 64-bit crossbar
  Master beat: 128'h1111_2222_3333_4444_5555_6666_7777_8888
  → Crossbar beat 0: 64'h5555_6666_7777_8888 (lower)
  → Crossbar beat 1: 64'h1111_2222_3333_4444 (upper)

### 9.5.2	Write Downsizing

Burst of 2 (128-bit) → Burst of 4 (64-bit):

Master AW: AWADDR = 0x3000, AWLEN = 1, AWSIZE = 4 (16 bytes)
Master W beats:
  W[0]: WDATA = 0xAAAA...(128 bits), WSTRB = 16'hFFFF, WLAST = 0
  W[1]: WDATA = 0xBBBB...(128 bits), WSTRB = 16'hFFFF, WLAST = 1

Crossbar AW: AWADDR = 0x3000, AWLEN = 3, AWSIZE = 3 (8 bytes)
Crossbar W beats:
  W[0]: WDATA = 0xAAAA_low(64), WSTRB = 8'hFF, WLAST = 0
  W[1]: WDATA = 0xAAAA_high(64), WSTRB = 8'hFF, WLAST = 0
  W[2]: WDATA = 0xBBBB_low(64), WSTRB = 8'hFF, WLAST = 0
  W[3]: WDATA = 0xBBBB_high(64), WSTRB = 8'hFF, WLAST = 1

### 9.5.3	Read Downsizing

Burst of 1 (128-bit) → Burst of 2 (64-bit):

Master AR: ARADDR = 0x4000, ARLEN = 0, ARSIZE = 4 (16 bytes)

Crossbar AR: ARADDR = 0x4000, ARLEN = 1, ARSIZE = 3 (8 bytes)

Crossbar R beats:
  R[0]: RDATA = 0x1111_2222_3333_4444, RLAST = 0
  R[1]: RDATA = 0x5555_6666_7777_8888, RLAST = 1

Master R beat (merged):
  R[0]: RDATA = 0x5555_6666_7777_8888_1111_2222_3333_4444, RLAST = 1

### 9.5.4	Strobe Mapping (Downsizing)

128-bit WSTRB → 64-bit WSTRB (split):

128-bit WSTRB = 16'hF0F0_0FF0

Beat 0 (bytes 7:0):
  64-bit WSTRB = 8'b0000_1111_1111_0000 = 8'h0F0 → 8'hF0

Beat 1 (bytes 15:8):
  64-bit WSTRB = 8'b1111_0000_1111_0000 = 8'hF0F0 >> 8 → 8'hF0

## 9.6	2.6.6 Address Alignment

### 9.6.1	Address Adjustment for Width

When changing widths, addresses must align to the new width:

32-bit (4-byte) aligned address: 0x1004
Upsized to 64-bit (8-byte): 0x1000 (round down to 8-byte boundary)

Narrow access at 0x1004 within 64-bit word:
  - Byte offset = 0x1004 & 0x7 = 4
  - Data at bytes [7:4] of 64-bit word
  - Lower bytes [3:0] unused (WSTRB = 8'b1111_0000)

### 9.6.2	Unaligned Access Handling

Master: 32-bit, unaligned access at 0x1002
Problem: Not aligned to 4-byte boundary

Options:
1. Error response (strict mode)
2. Round down address, use WSTRB for actual bytes
3. Split into multiple aligned accesses

Bridge default: Option 2 (use WSTRB)

## 9.7	2.6.7 Burst Length Adjustment

### 9.7.1	Length Calculation

Formula:
  New_Length = (Old_Length + 1) × (Old_Width / New_Width) - 1

Example: Upsizing 32→64
  Old: AWLEN = 7 (8 beats), WIDTH = 32
  New: AWLEN = (7+1) × (32/64) - 1 = 8 × 0.5 - 1 = 3 (4 beats)

Example: Downsizing 128→64
  Old: ARLEN = 3 (4 beats), WIDTH = 128
  New: ARLEN = (3+1) × (128/64) - 1 = 4 × 2 - 1 = 7 (8 beats)

### 9.7.2	Odd Burst Lengths

When burst doesn’t divide evenly:

Example: 3 beats of 32-bit → 64-bit
  3 beats × 4 bytes = 12 bytes total
  12 bytes / 8 bytes per beat = 1.5 beats
  
Solution:
  - Beat 0: Full 64-bit (8 bytes)
  - Beat 1: Partial 64-bit (4 bytes, WSTRB = 8'b0000_1111)
  - AWLEN = 1 (2 beats)

## 9.8	2.6.8 Resource Utilization

### 9.8.1	Per-Converter Resources

32→64 Upsizer:

Logic Elements:  ~300 LEs
Registers:       ~100 regs (buffering + FSM)
Block RAM:       0

Breakdown:
- Data buffer (32 bits): ~32 regs
- Strobe buffer: ~4 regs
- Beat counter: ~8 regs
- Control FSM: ~50 LEs, ~20 regs
- MUX logic: ~150 LEs

64→32 Downsizer:

Logic Elements:  ~250 LEs
Registers:       ~120 regs
Block RAM:       0

Similar structure but includes split logic

128→64 Downsizer:

Logic Elements:  ~400 LEs
Registers:       ~180 regs
Block RAM:       0

Larger data paths, more complex MUX

### 9.8.2	Scaling

Resource usage scales primarily with: - Width ratio: 2:1 vs. 4:1 vs. 8:1 conversion - Data width: 128-bit vs. 256-bit buffers - Buffering depth: Single vs. multi-beat buffering

## 9.9	2.6.9 Timing Characteristics

### 9.9.1	Latency

Upsizing (combining beats): - Buffering latency: 1-2 cycles (wait for N narrow beats) - First beat output: After receiving required narrow beats - Example: 32→64 requires 2 master beats before 1 crossbar beat

Downsizing (splitting beats): - Minimal latency: 1 cycle (register stage) - First beat output: Immediately (split from wide beat) - Example: 128→64 outputs first 64-bit beat immediately

### 9.9.2	Throughput

Upsizing Throughput:

32→64 upsizing:
  Master: 32 bits/cycle = 4 bytes/cycle
  Crossbar: 64 bits per 2 cycles = 4 bytes/cycle (same)
  No throughput loss

Downsizing Throughput:

128→64 downsizing:
  Master: 128 bits/cycle = 16 bytes/cycle
  Crossbar: 64 bits/cycle = 8 bytes/cycle
  Throughput halved (crossbar becomes bottleneck)

## 9.10	2.6.10 Configuration Parameters

### 9.10.1	Width Conversion Configuration (TOML)

[bridge]
internal_data_width = 64        # Crossbar width
enable_width_conversion = true  # Allow width differences

[[masters]]
name = "cpu_32bit"
data_width = 32                 # Narrower than crossbar (upsizing)
addr_width = 32

[[masters]]
name = "dma_64bit"
data_width = 64                 # Matches crossbar (no conversion)
addr_width = 32

[[masters]]
name = "gpu_128bit"
data_width = 128                # Wider than crossbar (downsizing)
addr_width = 36

[[slaves]]
name = "memory_64bit"
data_width = 64                 # Matches crossbar (no conversion)

[[slaves]]
name = "periph_32bit"
data_width = 32                 # Narrower than crossbar (downsizing)

## 9.11	2.6.11 Debug and Observability

### 9.11.1	Recommended Debug Signals

Upsizer:
- Beat accumulator (partial wide beat being assembled)
- Beat counter (position in burst)
- Buffer valid (accumulated beats ready)
- Strobe accumulation

Downsizer:
- Beat splitter state (which sub-beat currently outputting)
- Remaining beats to output
- Source data register

### 9.11.2	Common Issues and Debug

Symptom: Data corruption after width conversion
Check: - Byte ordering (endianness) - Strobe mapping (correct bytes enabled) - Address alignment - Burst length calculation

Symptom: Stalls after width conversion
Check: - Buffer depths (upsizer needs buffering) - Backpressure propagation - LAST signaling

Symptom: Incorrect burst lengths
Check: - Length calculation formula - Odd burst handling - SIZE field matching width

## 9.12	2.6.12 Verification Considerations

### 9.12.1	Test Scenarios

- Power-of-2 Width Ratios:

- 32→64 (2:1)
- 64→128 (1:2)
- 32→128 (4:1)

- Various Burst Lengths:

- Single beat (ALEN=0)
- Even bursts (ALEN=3, 7, 15)
- Odd bursts (ALEN=1, 5, 9)

- Sub-Word Accesses:

- Byte writes (WSTRB with individual bytes)
- Half-word, word accesses
- Unaligned accesses

- Mixed Widths:

- 32-bit master → 64-bit crossbar → 32-bit slave
- 128-bit master → 64-bit crossbar → 32-bit slave
- All width combinations

## 9.13	2.6.13 Performance Considerations

### 9.13.1	Conversion Overhead

Upsizing Overhead: - Buffering latency: +1-2 cycles - Throughput maintained - Best for burst-oriented masters

Downsizing Overhead: - Split latency: +1 cycle - Throughput reduced by width ratio - Can bottleneck wide masters

### 9.13.2	Optimization Strategies

- Match Common Widths: Design masters/slaves to match internal width (64-bit)

- Burst Sizing: Use appropriate burst lengths for width ratios

- Parallel Paths: Multiple 32-bit masters can aggregate to 64-bit crossbar bandwidth

- Selective Conversion: Only convert where necessary

## 9.14	2.6.14 Future Enhancements

### 9.14.1	Planned Features

- Configurable Internal Width: Support 32, 64, 128, 256-bit crossbar

- Multi-Beat Buffering: Deeper upsizing buffers for smoother flow

- Byte Rotation: Handle unaligned access more efficiently

- Pipelined Conversion: Multi-stage for high frequency

### 9.14.2	Under Consideration

- Asymmetric Widths: Different widths for read vs. write paths

- Sparse Data Optimization: Skip unused bytes in conversions

- Tagged Data: Metadata preservation through width conversion

- Zero-Copy Bypass: Direct routing for matching widths

Related Sections: - Section 2.1: Master Adapter (upsizing location) - Section 2.3: Crossbar Core (internal data width) - Section 3.2: Master Port Interface (width specifications) - Section 3.3: Slave Port Interface (width specifications)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	2.7 Protocol Conversion

The crossbar speaks AXI4 internally. Protocol conversion is what lets the bridge reach slaves that speak something simpler — APB (Advanced Peripheral Bus) for low-bandwidth peripherals being the common case.

## 10.1	2.7.1 Purpose and Function

Protocol conversion does five things:

- Protocol Translation: Converts AXI4 transactions to target protocol (e.g., APB)

- Handshake Mapping: Translates ready/valid to protocol-specific handshakes

- Burst Decomposition: Breaks AXI bursts into single-beat target transactions

- Response Mapping: Converts protocol-specific responses back to AXI responses

- Timing Adaptation: Handles different timing requirements between protocols

## 10.2	2.7.2 Supported Protocols

### 10.2.1	Current Support (Phase 2)

AXI4 (Native): - Full AXI4 protocol - No conversion required - Maximum performance

AXI4-Lite (Master-Side): - Simplified AXI4 subset - Single-beat transactions only (ARLEN=0, AWLEN=0) - Converts to full AXI4 for crossbar - Common for control/status registers

APB (Slave-Side): - APB3 and APB4 support - For low-bandwidth peripherals - Simplified handshaking

### 10.2.2	Current Limitation: AXI4-Lite Conversion

IMPORTANT: AXI4-Lite slaves are currently treated as full AXI4 slaves internally. The bridge does NOT perform real AXI4-Lite protocol conversion (single-beat restriction, no ARLEN/AWLEN, etc.). AXIL slaves: - Use the standard AXI4 timing wrapper (same as AXI4 slaves) - Expose full 5-channel AXI4 interface at the bridge boundary - Are documented as “axil” for user reference only

The bridge now emits real AXI4-to-AXIL4 conversion shims (axi4_to_axil4_{rd,wr}.sv) at the slave boundary when protocol = "axil" is specified in the TOML. These shims downgrade bursts to single beats and enforce AXIL protocol constraints transparently. See “Generator-Emitted Conversion Shims” below.

### 10.2.3	Future Support (Phase 2+)

- Real AXI4-Lite Conversion: Enforce single-beat constraint, remove unused signals

- AHB: Advanced High-performance Bus

- Wishbone: Open-source bus standard

- Custom: User-defined protocols

## 10.3	2.7.3 Conversion Architecture Overview

### 10.3.1	Figure 2.7.1: Protocol Conversion Architecture

Protocol Conversion Architecture

The diagram shows the complete protocol conversion flow: AXI4-Lite masters are adapted to full AXI4 before entering the crossbar, while APB peripherals receive AXI4 transactions through a dedicated converter.

## 10.4	2.7.4 Block Diagram

### 10.4.1	Figure 2.7: Protocol Conversion Architecture

Protocol Conversion Architecture

Protocol conversion showing AXI4 to APB conversion with state machine, burst decomposition, and response mapping.

## 10.5	2.7.5 AXI4-Lite to AXI4 Conversion (Master-Side)

### 10.5.1	AXI4-Lite Protocol Overview

AXI4-Lite is a simplified subset of AXI4 designed for simple control/status register access:

Key Differences from Full AXI4:
- FIXED burst length: ARLEN/AWLEN always = 0 (single beat)
- FIXED burst size: No SIZE field, always full data width
- FIXED burst type: No BURST field, always INCR
- NO exclusive access: No LOCK support
- NO unaligned transfers: Address must be aligned
- Simpler ID: Typically 1-4 bits (fewer outstanding transactions)

Similarities to AXI4:
- Same 5 channels: AR, R, AW, W, B
- Same valid/ready handshaking
- Same response codes: OKAY, SLVERR, DECERR, EXOKAY
- Same data widths: 32 or 64 bits typically

### 10.5.2	Conversion Requirements

To adapt AXI4-Lite masters to the full AXI4 crossbar, the adapter must:

- Add Missing Signals: Provide default values for burst-related signals

- Validate Constraints: Ensure single-beat assumption holds

- Pass-Through Simplicity: Most signals connect directly

### 10.5.3	Signal Mapping

// AXI4-Lite to AXI4 Signal Mapping

// AR Channel (Read Address)
// AXI4-Lite Input          AXI4 Crossbar Output
axi4lite_arvalid      →     axi4_arvalid
axi4lite_arready      ←     axi4_arready
axi4lite_araddr       →     axi4_araddr
axi4lite_arprot       →     axi4_arprot
axi4lite_arid (opt)   →     axi4_arid

// Added by adapter (constants):
                            axi4_arlen    = 8'h00      // Always 1 beat
                            axi4_arsize   = log2(DW/8) // Full width
                            axi4_arburst  = 2'b01      // INCR
                            axi4_arlock   = 1'b0       // No lock
                            axi4_arcache  = 4'b0000    // Device non-buf
                            axi4_arqos    = 4'h0       // No QoS
                            axi4_arregion = 4'h0       // Region 0

// R Channel (Read Data)
// AXI4 Crossbar Input      AXI4-Lite Output
axi4_rvalid           →     axi4lite_rvalid
axi4_rready           ←     axi4lite_rready
axi4_rdata            →     axi4lite_rdata
axi4_rresp            →     axi4lite_rresp
axi4_rid (opt)        →     axi4lite_rid (opt)

// Discarded by adapter:
axi4_rlast                  // Always 1 for single beat

// AW Channel (Write Address)
axi4lite_awvalid      →     axi4_awvalid
axi4lite_awready      ←     axi4_awready
axi4lite_awaddr       →     axi4_awaddr
axi4lite_awprot       →     axi4_awprot
axi4lite_awid (opt)   →     axi4_awid

// Added by adapter:
                            axi4_awlen    = 8'h00
                            axi4_awsize   = log2(DW/8)
                            axi4_awburst  = 2'b01
                            axi4_awlock   = 1'b0
                            axi4_awcache  = 4'b0000
                            axi4_awqos    = 4'h0
                            axi4_awregion = 4'h0

// W Channel (Write Data)
axi4lite_wvalid       →     axi4_wvalid
axi4lite_wready       ←     axi4_wready
axi4lite_wdata        →     axi4_wdata
axi4lite_wstrb        →     axi4_wstrb

// Added by adapter:
                            axi4_wlast    = 1'b1       // Always last

// B Channel (Write Response)
axi4_bvalid           →     axi4lite_bvalid
axi4_bready           ←     axi4lite_bready
axi4_bresp            →     axi4lite_bresp
axi4_bid (opt)        →     axi4lite_bid (opt)

### 10.5.4	Implementation

The AXI4-Lite adapter is extremely simple, primarily providing constant values:

// AXI4-Lite to AXI4 Adapter (simplified)
module axi4lite_to_axi4_adapter #(
    parameter ADDR_WIDTH = 32,
    parameter DATA_WIDTH = 64,
    parameter ID_WIDTH = 4         // Optional, often 0 for AXI4-Lite
) (
    input logic clk,
    input logic rst_n,
    
    // AXI4-Lite Master Interface
    input  logic                     lite_arvalid,
    output logic                     lite_arready,
    input  logic [ADDR_WIDTH-1:0]    lite_araddr,
    input  logic [2:0]               lite_arprot,
    input  logic [ID_WIDTH-1:0]      lite_arid,    // Optional
    
    output logic                     lite_rvalid,
    input  logic                     lite_rready,
    output logic [DATA_WIDTH-1:0]    lite_rdata,
    output logic [1:0]               lite_rresp,
    output logic [ID_WIDTH-1:0]      lite_rid,     // Optional
    
    input  logic                     lite_awvalid,
    output logic                     lite_awready,
    input  logic [ADDR_WIDTH-1:0]    lite_awaddr,
    input  logic [2:0]               lite_awprot,
    input  logic [ID_WIDTH-1:0]      lite_awid,    // Optional
    
    input  logic                     lite_wvalid,
    output logic                     lite_wready,
    input  logic [DATA_WIDTH-1:0]    lite_wdata,
    input  logic [DATA_WIDTH/8-1:0]  lite_wstrb,
    
    output logic                     lite_bvalid,
    input  logic                     lite_bready,
    output logic [1:0]               lite_bresp,
    output logic [ID_WIDTH-1:0]      lite_bid,     // Optional
    
    // Full AXI4 Crossbar Interface
    output logic                     axi4_arvalid,
    input  logic                     axi4_arready,
    output logic [ADDR_WIDTH-1:0]    axi4_araddr,
    output logic [7:0]               axi4_arlen,
    output logic [2:0]               axi4_arsize,
    output logic [1:0]               axi4_arburst,
    output logic                     axi4_arlock,
    output logic [3:0]               axi4_arcache,
    output logic [2:0]               axi4_arprot,
    output logic [3:0]               axi4_arqos,
    output logic [3:0]               axi4_arregion,
    output logic [ID_WIDTH-1:0]      axi4_arid,
    
    input  logic                     axi4_rvalid,
    output logic                     axi4_rready,
    input  logic [DATA_WIDTH-1:0]    axi4_rdata,
    input  logic [1:0]               axi4_rresp,
    input  logic                     axi4_rlast,
    input  logic [ID_WIDTH-1:0]      axi4_rid,
    
    // ... (AW, W, B channels similar)
);

    // AR Channel: Pass-through with constants
    assign axi4_arvalid  = lite_arvalid;
    assign lite_arready  = axi4_arready;
    assign axi4_araddr   = lite_araddr;
    assign axi4_arprot   = lite_arprot;
    assign axi4_arid     = lite_arid;
    
    // Constants for single-beat burst
    assign axi4_arlen    = 8'h00;                    // 1 beat
    assign axi4_arsize   = $clog2(DATA_WIDTH/8);     // Full width
    assign axi4_arburst  = 2'b01;                    // INCR
    assign axi4_arlock   = 1'b0;                     // No lock
    assign axi4_arcache  = 4'b0000;                  // Device non-buf
    assign axi4_arqos    = 4'h0;                     // No QoS
    assign axi4_arregion = 4'h0;                     // Region 0
    
    // R Channel: Pass-through, ignore rlast
    assign lite_rvalid = axi4_rvalid;
    assign axi4_rready = lite_rready;
    assign lite_rdata  = axi4_rdata;
    assign lite_rresp  = axi4_rresp;
    assign lite_rid    = axi4_rid;
    // axi4_rlast ignored (always 1 for single beat)
    
    // AW Channel: Similar to AR
    assign axi4_awvalid  = lite_awvalid;
    assign lite_awready  = axi4_awready;
    assign axi4_awaddr   = lite_awaddr;
    assign axi4_awprot   = lite_awprot;
    assign axi4_awid     = lite_awid;
    assign axi4_awlen    = 8'h00;
    assign axi4_awsize   = $clog2(DATA_WIDTH/8);
    assign axi4_awburst  = 2'b01;
    assign axi4_awlock   = 1'b0;
    assign axi4_awcache  = 4'b0000;
    assign axi4_awqos    = 4'h0;
    assign axi4_awregion = 4'h0;
    
    // W Channel: Pass-through, add wlast
    assign axi4_wvalid = lite_wvalid;
    assign lite_wready = axi4_wready;
    assign axi4_wdata  = lite_wdata;
    assign axi4_wstrb  = lite_wstrb;
    assign axi4_wlast  = 1'b1;                       // Always last
    
    // B Channel: Pass-through
    assign lite_bvalid = axi4_bvalid;
    assign axi4_bready = lite_bready;
    assign lite_bresp  = axi4_bresp;
    assign lite_bid    = axi4_bid;

endmodule

### 10.5.5	Resource Utilization

AXI4-Lite Adapter Resources:

Logic Elements:  ~50-100 LEs (minimal, mostly wiring)
Registers:       ~50 regs (if skid buffers added)
Block RAM:       0

Breakdown:
- Signal pass-through: ~20 LEs (buffering)
- Constant generation: ~10 LEs
- Optional skid buffers: ~50 regs (for timing)

Note: Most implementations are purely combinatorial wire
      assignments with optional pipeline registers.

### 10.5.6	Performance Impact

Latency: - Zero-latency (combinatorial) if no pipeline stages - 1-2 cycles if skid buffers added for timing - No protocol conversion overhead

Throughput: - 1 transaction per cycle (same as native AXI4) - No degradation for single-beat transactions - Limited by AXI4-Lite’s single-beat constraint

### 10.5.7	Configuration

[[bridge.masters]]
name       = "control_processor"
prefix     = "cpu_axil_"       # required -- prefixes every external signal
protocol   = "axil"            # NOT "axi4lite"; see the protocol table in the HAS
channels   = "rw"              # "rw" | "rd" | "wr"
id_width   = 0                 # AXI4-Lite carries no ID
addr_width = 32
data_width = 32
user_width = 1

Field names are id_width / addr_width / data_width – one id_width per port, not the arid_width / awid_width pair an earlier revision of this page showed, which the loader does not read. The table is [[bridge.masters]], not [[masters]]. Compare bin/test_configs/bridge_1x5_wr_axil.toml for a config that actually generates.

### 10.5.8	Common Issues and Debug

Issue 1: Burst Detected on AXI4-Lite

Symptom: ARLEN/AWLEN != 0 on AXI4-Lite interface
Cause: Master not properly configured as AXI4-Lite
Check: Verify master only issues single-beat transactions

Issue 2: Unaligned Addresses

Symptom: ARADDR/AWADDR not aligned to data width
Cause: AXI4-Lite requires full-width aligned access
Check: Address[log2(DW/8)-1:0] should be zero

Issue 3: rlast/wlast Handling

Symptom: Master expects rlast/wlast but doesn't have them
Cause: True AXI4-Lite interface omits these signals
Solution: Adapter provides wlast=1 to crossbar, strips rlast

## 10.6	2.7.6 AXI4 to APB Conversion (Slave-Side)

### 10.6.1	APB Protocol Overview

APB is a simple, low-power bus protocol:

Characteristics:
- Single address phase
- Single data phase
- No burst support (one transfer per operation)
- Minimal logic
- Low power consumption
- Suitable for peripherals: UARTs, timers, GPIOs

### 10.6.2	APB Signals

Address Phase:
  PADDR[N-1:0]  - Address bus
  PSEL          - Slave select
  PENABLE       - Enable (2nd cycle of transfer)
  PWRITE        - Write direction (1=write, 0=read)
  
Data Phase (Write):
  PWDATA[N-1:0] - Write data
  PSTRB[N/8-1:0]- Write strobes (APB4 only)
  
Data Phase (Read):
  PRDATA[N-1:0] - Read data
  
Response:
  PREADY        - Slave ready (can extend transfer)
  PSLVERR       - Slave error (APB3+)

### 10.6.3	APB State Machine

APB requires a 2-phase handshake:

IDLE:
  - Wait for AXI request (ARVALID or AWVALID)
  - PSEL = 0, PENABLE = 0
  
SETUP:
  - Assert PSEL = 1
  - Drive PADDR, PWRITE, PWDATA (if write)
  - PENABLE = 0
  - Duration: 1 cycle
  
ACCESS:
  - Assert PENABLE = 1
  - Wait for PREADY = 1
  - Capture PRDATA (if read) or PSLVERR
  - Can extend multiple cycles if PREADY = 0
  
Complete:
  - De-assert PSEL, PENABLE
  - Return to IDLE or SETUP (if more beats)

### 10.6.4	Read Transaction Conversion

AXI4 Read:

Cycle 0: ARVALID=1, ARADDR=0x100, ARLEN=3 (4 beats)
Cycle 1: ARREADY=1
Cycles 2-5: R beats returning

Converted to APB (4 separate APB reads):

Beat 0:
  Cycle 0: PSEL=1, PENABLE=0, PADDR=0x100, PWRITE=0 (SETUP)
  Cycle 1: PSEL=1, PENABLE=1, wait PREADY (ACCESS)
  Cycle 2: PREADY=1, capture PRDATA → First R beat

Beat 1:
  Cycle 3: PSEL=1, PENABLE=0, PADDR=0x104 (SETUP)
  Cycle 4: PSEL=1, PENABLE=1, wait PREADY (ACCESS)
  Cycle 5: PREADY=1, capture PRDATA → Second R beat
  
... (beats 2 and 3 similar)

Latency: 2-3 cycles per beat (SETUP + ACCESS + ready)

### 10.6.5	Write Transaction Conversion

AXI4 Write:

Cycle 0: AWVALID=1, AWADDR=0x200, AWLEN=1 (2 beats)
Cycle 1: AWREADY=1
Cycle 1: WVALID=1, WDATA=0xAAAA_BBBB, WLAST=0
Cycle 2: WREADY=1
Cycle 2: WVALID=1, WDATA=0xCCCC_DDDD, WLAST=1
Cycle 3: WREADY=1
Cycle 4: BVALID=1, BRESP=OKAY

Converted to APB (2 separate APB writes):

Beat 0:
  Cycle 0: PSEL=1, PENABLE=0, PADDR=0x200, PWRITE=1, PWDATA=0xAAAA_BBBB
  Cycle 1: PSEL=1, PENABLE=1, wait PREADY
  Cycle 2: PREADY=1 → First write complete

Beat 1:
  Cycle 3: PSEL=1, PENABLE=0, PADDR=0x204, PWRITE=1, PWDATA=0xCCCC_DDDD
  Cycle 4: PSEL=1, PENABLE=1, wait PREADY
  Cycle 5: PREADY=1 → Second write complete, return B

### 10.6.6	Burst Handling

APB does not support bursts, so:

AXI Burst: AWLEN = 15 (16 beats)
→ 16 separate APB transfers
→ Address increments per AWBURST type:
   - INCR: Addr += SIZE each beat
   - WRAP: Wrapping within boundary
   - FIXED: Same address each beat

### 10.6.7	Response Mapping

APB → AXI Response Translation:

PREADY=1, PSLVERR=0 → RRESP/BRESP = 2'b00 (OKAY)
PREADY=1, PSLVERR=1 → RRESP/BRESP = 2'b10 (SLVERR)

PREADY stuck at 0:
  The converter WAITS. There is no timeout and no DECERR.

There is no PREADY timeout anywhere in the path. apb4_master waits unconditionally in ACCESS (if (m_apb_PREADY) ... with no counter), and neither axi4_to_apb4_convert, axi4_to_apb4_shim nor axi4_to_apb5_shim contains a threshold register or cycle counter. A slave that never asserts PREADY stalls that path indefinitely, and the stall propagates back through the crossbar as backpressure. Budget for it in the system, or put a watchdog outside the bridge; the bridge will not manufacture an error response.

## 10.7	2.7.7 Implementation

### 10.7.1	AXI4-to-APB Converter FSM

// Simplified AXI4-to-APB converter
typedef enum logic [2:0] {
    IDLE,
    AR_SETUP,
    AR_ACCESS,
    AW_SETUP,
    W_ACCESS,
    B_RESPONSE
} state_t;

state_t state, next_state;

always_ff @(posedge clk) begin
    if (!rst_n) state <= IDLE;
    else state <= next_state;
end

always_comb begin
    next_state = state;
    
    case (state)
        IDLE: begin
            if (arvalid) next_state = AR_SETUP;
            else if (awvalid) next_state = AW_SETUP;
        end
        
        AR_SETUP: begin
            next_state = AR_ACCESS;  // 1 cycle SETUP
        end
        
        AR_ACCESS: begin
            if (pready) begin
                if (more_beats) next_state = AR_SETUP;  // Next beat
                else next_state = IDLE;
            end
        end
        
        AW_SETUP: begin
            if (wvalid) next_state = W_ACCESS;
        end
        
        W_ACCESS: begin
            if (pready) begin
                if (!wlast) next_state = AW_SETUP;  // Next beat
                else next_state = B_RESPONSE;
            end
        end
        
        B_RESPONSE: begin
            if (bready) next_state = IDLE;
        end
    endcase
end

// APB signal generation
assign psel = (state != IDLE);
assign penable = (state == AR_ACCESS || state == W_ACCESS);
assign pwrite = (state == AW_SETUP || state == W_ACCESS);

### 10.7.2	Address Generation

// Address increment for burst
logic [ADDR_WIDTH-1:0] current_addr;
logic [7:0] beat_count;

always_ff @(posedge clk) begin
    if (state == IDLE) begin
        current_addr <= arvalid ? araddr : awaddr;
        beat_count <= 0;
    end else if ((state == AR_ACCESS || state == W_ACCESS) && pready) begin
        beat_count <= beat_count + 1;
        
        case (burst_type)
            2'b01: current_addr <= current_addr + (1 << size);  // INCR
            2'b10: current_addr <= wrap_address(current_addr);  // WRAP
            2'b00: current_addr <= current_addr;                // FIXED
        endcase
    end
end

assign paddr = current_addr;

## 10.8	2.7.8 Resource Utilization

### 10.8.1	APB Converter Resources

Per APB Slave Interface:

Logic Elements:  ~400 LEs
Registers:       ~150 regs
Block RAM:       0

Breakdown:
- FSM control:           ~100 LEs, ~20 regs
- Address generation:    ~80 LEs, ~40 regs
- Burst counter:         ~50 LEs, ~20 regs
- Response accumulation: ~80 LEs, ~30 regs
- Data path MUX:         ~90 LEs, ~40 regs

### 10.8.2	Scaling

Adding APB slaves: - Linear scaling: +~400 LEs per APB slave - Shared address decoder logic - Independent per-slave FSMs

## 10.9	2.7.9 Timing Characteristics

### 10.9.1	Latency

APB Read Latency (per beat):

Best case: 2 cycles (SETUP + ACCESS with PREADY=1)
Typical: 3-5 cycles (if slave extends with PREADY=0)
Worst case: unbounded -- the converter waits for PREADY with no timeout

For 8-beat AXI burst:
  Total: 8 × 3 = 24 cycles typical

APB Write Latency (per beat):

Similar to read: 2-5 cycles per beat

### 10.9.2	Throughput

Severely Limited:

APB: ~0.3-0.5 transactions/cycle (due to 2-3 cycle protocol)
AXI4: 1 transaction/cycle (burst mode)

APB suitable only for low-bandwidth peripherals

## 10.10	2.7.10 Configuration Parameters

### 10.10.1	Protocol Conversion Configuration (TOML)

[[slaves]]
name = "uart_peripheral"
protocol = "apb"            # "axi4", "apb", "ahb" (future)
base_address = 0xF000_0000
size = 0x1000
data_width = 32

[[slaves]]
name = "ddr_memory"
protocol = "axi4"           # Native, no conversion
base_address = 0x8000_0000
size = 0x4000_0000
data_width = 64

## 10.11	2.7.11 Debug and Observability

### 10.11.1	Recommended Debug Signals

APB Converter:
- FSM state
- APB phase (SETUP, ACCESS)
- Beat counter (progress through burst)
- Response accumulation (for burst)

APB Bus:
- PSEL, PENABLE, PWRITE
- PADDR, PWDATA, PRDATA
- PREADY, PSLVERR

### 10.11.2	Common Issues and Debug

Symptom: APB slave not responding (timeout)
Check: - PREADY signal (stuck at 0?) - APB slave clock/reset - PSEL assertion (There is no timeout threshold to check – the converter waits forever.)

Symptom: Data corruption on APB
Check: - SETUP phase duration (should be 1 cycle) - PENABLE assertion timing - Data sampling on correct cycle

Symptom: Burst to APB takes too long
Check: - Burst length (consider limiting ARLEN/AWLEN) - APB slave response time (PREADY) - Alternative: Use AXI4 slave instead

## 10.12	2.7.12 Verification Considerations

### 10.12.1	Test Scenarios

- Single APB Transfer:

- AXI ARLEN=0 (1 beat) → 1 APB read
- Verify SETUP → ACCESS sequence
- Check PREADY handling

- APB Burst Decomposition:

- AXI AWLEN=7 (8 beats) → 8 APB writes
- Verify address increment
- Check each beat completes before next

- APB PREADY Extension:

- Slave holds PREADY=0 for N cycles
- Verify converter waits
- Check no data corruption

- APB Error Response:

- Slave asserts PSLVERR
- Verify mapped to AXI SLVERR
- Check error propagated to master

- APB stall (no timeout to test):

- Slave never asserts PREADY
- Verify the path stalls and backpressures cleanly, with no lost or
  duplicated beats once PREADY finally arrives
- There is NO timeout and NO DECERR to check for -- see 2.7.6

## 10.13	2.7.13 Performance Considerations

### 10.13.1	When to Use APB

Good Use Cases: - Low-speed peripherals (UART, GPIO, timers) - Infrequent accesses - Simple register interfaces - Power-sensitive designs

Poor Use Cases: - High-bandwidth devices - Burst-intensive masters - Performance-critical paths - Memory interfaces

### 10.13.2	APB vs. AXI4 Comparison

Feature          APB              AXI4
Complexity       Simple           Complex
Throughput       Low (~0.3/cyc)   High (1/cyc burst)
Latency/beat     2-5 cycles       1 cycle
Burst Support    No               Yes (up to 256)
Resources        ~400 LEs         Native (no converter)
Power            Very low         Moderate
Use Case         Peripherals      Memory, DMA

## 10.14	2.7.14 Mixed Protocol Bridges

### 10.14.1	Example Configuration

# Bridge with mixed protocols
[bridge]
num_masters = 2
num_slaves = 3

[[masters]]
name = "cpu"
protocol = "axi4"

[[masters]]
name = "dma"
protocol = "axi4"

[[slaves]]
name = "ddr_memory"
protocol = "axi4"          # High bandwidth

[[slaves]]
name = "sram"
protocol = "axi4"          # Medium bandwidth

[[slaves]]
name = "peripherals"
protocol = "apb"           # Low bandwidth, simple

### 10.14.2	Routing Optimization

CPU → DDR Memory: AXI4-to-AXI4 (native, fast)
CPU → Peripherals: AXI4-to-APB (converted, slower)
DMA → SRAM: AXI4-to-AXI4 (native, fast)
DMA → Peripherals: AXI4-to-APB (rare, acceptable slowdown)

## 10.15	2.7.15 Master-Side vs Slave-Side Conversion

### 10.15.1	Comparison

Feature              Master-Side (AXI4-Lite)    Slave-Side (APB)
──────────────────────────────────────────────────────────────────
Complexity           Very Simple                Complex
Resource Usage       ~50-100 LEs                ~400 LEs
Latency Added        0-1 cycles                 2-5 cycles/beat
Throughput Impact    None                       Severe (3x slower)
Burst Handling       Single beat only           Decompose to singles
State Machine        None (combinatorial)       Multi-state FSM
Buffering Required   Optional (timing)          Essential
Use Case             Control registers          Peripherals

### 10.15.2	When to Use Each

AXI4-Lite Master Adapter: - Simple control/status register interfaces - Low-complexity masters (MCUs, simple CPUs) - Minimal resource overhead acceptable - No burst performance needed

APB Slave Converter: - Legacy peripheral integration - Very simple slave devices (GPIO, timers) - Low-bandwidth acceptable - Power optimization critical

## 10.16	2.7.16 Generator-Emitted Conversion Shims

The bridge generator automatically emits protocol conversion shims at the slave boundary based on the TOML configuration. These shims are instantiated between the crossbar core (uniformly AXI4 internally) and the external slave port.

### 10.16.1	AXI4 to AXI4-Lite Conversion

When protocol = "axil" is specified in the slave TOML: - Generator emits axi4_to_axil4_rd.sv (read path) and axi4_to_axil4_wr.sv (write path) - Shims downgrade bursts to single beats (set ARLEN/AWLEN = 0 on output) - Enforce single-beat semantics transparently - Data width remains unchanged (upsizing/downsizing done separately) - Shims are inserted between crossbar and slave port in the generated top-level module

Modules: projects/components/converters/rtl/axi4_to_axil4_{rd,wr}.sv

### 10.16.2	AXI4 to APB Conversion

When protocol = "apb" is specified in the slave TOML: - Generator emits ONE shim, axi4_to_apb4_shim – a direct full-AXI4-to-APB4 converter. There is no AXI4-Lite stage: the shim does its own burst decomposition (r_burst_count + axi_gen_addr), so nothing upstream has to reduce the burst first. - protocol = "apb5" emits axi4_to_apb5_shim, a sideband wrapper over that same shim – see AMBA5 Boundary. - Slave port externally presents APB signals; internally the crossbar is AXI4

Modules: projects/components/converters/rtl/axi4_to_apb4_shim.sv (axi4_to_apb5_shim.sv for apb5)

An earlier revision of this page described the path as an axi4_to_axil4 shim followed by an internal AXIL-to-APB bridge chain. No such chain exists, and no generated APB slave adapter instantiates axi4_to_axil4.

### 10.16.3	Master-Side AXIL→Wider-Slave Alignment

When an AXI4-Lite master interfaces with a wider AXI4 slave (e.g., 32-bit AXIL master to 64-bit AXI4 slave): - Generator emits axil_to_axi4_wide_align_rd.sv (read) and axil_to_axi4_wide_align_wr.sv (write) at the master adapter output (before crossbar core) - These modules handle width alignment (not protocol conversion — that’s done at the slave boundary if needed) - Preserves AXIL’s single-beat constraint on the master side - Properly aligns partial-word reads/writes on the wide slave side

Modules: projects/components/converters/rtl/axil_to_axi4_wide_align_{rd,wr}.sv

Example: 32-bit AXIL master → 64-bit AXI4 slave - Master writes to addr 0x04 with data 0xAABBCCDD. The shim selects the lane from addr[2] (= 1), so the data lands on the UPPER half: the slave sees wdata[63:32] = 0xAABBCCDD, WSTRB = 0xF0, at row-aligned address 0x00 – the lane bits move out of the address and into the strobe. Worked through in Width Converters. - Shim is inserted after master adapter, before crossbar

## 10.17	2.7.17 Future Protocol Support

### 10.17.1	Planned Features

AXI4-Lite: - Subset of AXI4 - No burst support (ALEN=0 always) - Simpler than full AXI4 - Common for control registers

AHB (AMBA High-performance Bus): - More capable than APB - Pipeline support - Burst support - Suitable for moderate-bandwidth peripherals

Wishbone: - Open-source bus standard - Common in FPGA designs - Multiple addressing modes - Configurable data widths

### 10.17.2	Under Consideration

- Custom Protocol: User-defined through configuration

- Stream Interface: AXI4-Stream for data streaming

- PCIe TLP: For PCIe endpoint integration

- CHI: ARM’s Coherent Hub Interface

## 10.18	2.7.18 Best Practices

### 10.18.1	Design Recommendations

- Limit APB Burst Lengths: Configure masters to use short bursts to APB slaves

- Watchdog Outside the Bridge: the converter has no PREADY timeout, so a wedged APB slave stalls its path forever. If the system needs to survive that, the watchdog belongs upstream.

- Protocol Matching: Use native AXI4 where possible, APB only when necessary

- Address Map Planning: Group APB peripherals together for efficient decoding

- Width Matching: Match APB data width to peripheral requirements

### 10.18.2	Performance Tips

Inefficient: 256-beat AXI burst → APB
  256 beats × 3 cyc/beat = 768 cycles

Better: Limit to 4-beat bursts → APB
  64 bursts of 4 beats each
  Still long, but more manageable

Best: Use AXI4-Lite slave for registers
  No burst, but native protocol

Related Sections: - Section 2.3: Crossbar Core (protocol integration point) - Section 3.3: Slave Port Interface (APB signal specifications) - Chapter 4: Programming (configuring protocol conversion) - Appendix A: Generator Deep Dive (protocol converter generation)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	2.8 Response Routing

Requests are easy to route — the address says where they go. Responses are harder: read data and write acknowledgments have to find their way back to whichever master asked. Response routing does that with Bridge IDs, demultiplexing logic, and optional CAM structures.

## 11.1	2.8.1 Purpose and Function

Response routing does five things:

- Response Direction: Routes R and B channel responses to correct master

- ID-Based Routing: Uses extracted Bridge IDs to determine destination

- Multi-Slave Merging: Combines responses from multiple slaves to single master

- Flow Control: Manages backpressure from master on response channels

- Error Propagation: Ensures error responses reach originating master

## 11.2	2.8.2 Block Diagram

### 11.2.1	Figure 2.8: Response Routing Architecture

Response Routing Architecture

Response routing architecture showing B and R channel merging from slaves and ID-based demultiplexing to masters.

## 11.3	2.8.3 Stable Response Routing via FIFO-Tracked Slave Select

### 11.3.1	Architecture: FIFO-Tracked Master Routing

For multi-slave masters, responses are routed based on a stable slave-select value captured at request time, not the current address decode:

// At AW/AR time: Capture slave index
assign aw_slave_select = address_decode(fub_axi_awaddr);
always_ff @(posedge aclk) begin
    if (fub_axi_awvalid && fub_axi_awready) begin
        aw_trk_fifo <= aw_slave_select;  // Capture now
    end
end

// At B time: Use captured slave from FIFO (not current decode)
assign b_slave_select = aw_trk_fifo_out;  // Stable value
logic [NUM_MASTERS-1:0] master_bvalid;
always_comb begin
    master_bvalid = '0;
    master_bvalid[extracted_bid] = slave_bvalid && (b_slave_select == current_slave);
end

Benefits: - B-responses route to correct master even after address reverts - Handles multi-slave masters spanning different widths - Prevents response loss due to stale address decodes

### 11.3.2	BID Extraction

For ID-based routing, Bridge ID is extracted from response:

// Extract Bridge ID from response ID
logic [TOTAL_RID_WIDTH-1:0] slave_rid;     // From slave
logic [BID_WIDTH-1:0] extracted_bid;       // Master index
logic [RID_WIDTH-1:0] external_rid;        // To master

// Upper bits = Bridge ID, lower bits = original ID
assign extracted_bid = slave_rid[TOTAL_RID_WIDTH-1:RID_WIDTH];
assign external_rid = slave_rid[RID_WIDTH-1:0];

// Route response based on BID (combined with slave select tracking above)
logic [NUM_MASTERS-1:0] master_rvalid;
always_comb begin
    master_rvalid = '0;  // Default: no master selected
    master_rvalid[extracted_bid] = slave_rvalid && (r_slave_select == current_slave);
end

### 11.3.3	CAM-Based Extraction

For complex configurations with OOO or ID reordering:

// CAM lookup for response routing
logic [TOTAL_RID_WIDTH-1:0] response_id;
logic [BID_WIDTH-1:0] master_index;
logic cam_hit;

// Search CAM for matching transaction
assign {cam_hit, master_index} = cam_lookup(response_id);

// Route response to found master
if (cam_hit) begin
    master_rvalid[master_index] = slave_rvalid;
end else begin
    // Error: No matching transaction (protocol violation)
    error_response();
end

## 11.4	2.8.4 Response Demultiplexing

### 11.4.1	Per-Master Response Paths

Each master has dedicated response channels from the demultiplexer:

Master 0 Response:
  - M0_RVALID, M0_RREADY, M0_RDATA, M0_RID, M0_RRESP, M0_RLAST
  - M0_BVALID, M0_BREADY, M0_BID, M0_BRESP

Source: Responses from any slave with BID=0

### 11.4.2	Demux Implementation

// Response demultiplexer (4 masters, 3 slaves)
// Combines responses from all slaves, routes to correct master

// Slave responses (multiple sources)
logic s0_rvalid, s1_rvalid, s2_rvalid;
logic [63:0] s0_rdata, s1_rdata, s2_rdata;
logic [5:0] s0_rid, s1_rid, s2_rid;  // Includes BID

// Master responses (multiple destinations)
logic m0_rvalid, m1_rvalid, m2_rvalid, m3_rvalid;
logic [63:0] m0_rdata, m1_rdata, m2_rdata, m3_rdata;
logic [3:0] m0_rid, m1_rid, m2_rid, m3_rid;  // BID stripped

// Demux logic per slave
for (genvar s = 0; s < 3; s++) begin
    logic [1:0] bid;
    assign bid = slave_rid[s][5:4];  // Extract BID
    
    always_comb begin
        case (bid)
            2'b00: begin
                if (slave_rvalid[s] && !m0_busy) begin
                    m0_rvalid = 1'b1;
                    m0_rdata = slave_rdata[s];
                    m0_rid = slave_rid[s][3:0];  // Strip BID
                end
            end
            2'b01: /* Route to M1 */
            2'b10: /* Route to M2 */
            2'b11: /* Route to M3 */
        endcase
    end
end

## 11.5	2.8.5 Multi-Slave Response Merging

### 11.5.1	Problem Statement

When multiple slaves can respond to same master simultaneously:

Scenario:
  Master 0 has outstanding transactions to Slave 0 and Slave 1
  Both slaves return R responses in same cycle
  
Problem: Master 0 can only accept one R response per cycle
Solution: Arbitrate between slave responses

### 11.5.2	Response Arbitration

// Arbitrate between multiple slave responses for same master
logic [2:0] slave_has_response;  // Which slaves have responses for M0
logic [1:0] selected_slave;      // Which slave wins arbitration

// Detect responses destined for M0
for (genvar s = 0; s < 3; s++) begin
    assign slave_has_response[s] = slave_rvalid[s] && 
                                   (extract_bid(slave_rid[s]) == 2'b00);
end

// Round-robin arbiter for fairness
always_ff @(posedge clk) begin
    if (!rst_n) begin
        selected_slave <= 0;
    end else if (m0_rready && (|slave_has_response)) begin
        // Rotate selection for fairness
        if (slave_has_response[selected_slave])
            ; // Keep current
        else
            selected_slave <= find_next_requesting_slave();
    end
end

// MUX selected slave's response to master
assign m0_rvalid = slave_has_response[selected_slave];
assign m0_rdata = slave_rdata[selected_slave];
assign m0_rid = strip_bid(slave_rid[selected_slave]);

### 11.5.3	Response Buffering

Optional: Add FIFOs to buffer pending responses:

Purpose:
  - Prevent response loss during arbitration conflicts
  - Allow slaves to return responses even if master busy
  - Smooth out bursty response patterns

Configuration:
  per_master_response_fifo_depth = 4-16 entries

Trade-off:
  + No response loss
  + Better throughput
  - Additional resources (~200 LEs per FIFO)
  - +1-2 cycle latency

## 11.6	2.8.6 Independent Write-Tracker FIFO (W-Channel Deadlock Fix)

Prior to commit d24bd617, the AW and W ready signals shared a single multiplexer at the write-address arbiter output. Under interleaved master traffic patterns, this could cause deadlock: a master’s W channel could back up waiting for a non-granted write-address slot, preventing the arbiter from servicing other masters’ AW transactions.

The corrected design (current) uses an independent W-tracker FIFO alongside the AW tracker:

// AW tracker FIFO (per master)
always_ff @(posedge aclk) begin
    if (master_awvalid && master_awready) begin
        aw_trk_fifo <= address_decode(master_awaddr);  // Capture slave
    end
end

// Independent W tracker and ready MUX
logic aw_ready_mux, w_ready_mux;  // Separate paths (prior: shared)

assign master_awready = aw_ready_mux;   // AW gets its own ready
assign master_wready = w_ready_mux;      // W gets independent ready

// W-tracker: Gate W valid against tracked slave select
always_comb begin
    w_ready_mux = (aw_trk_fifo != 0) && slave_wready;  // W independent
end

Result: AW and W channels can assert independently without blocking each other, preventing deadlock during interleaved multi-master write bursts.

## 11.7	2.8.7 Backpressure Management

### 11.7.1	Ready Signal Routing

Master RREADY/BREADY must reach correct slave:

// Route master ready back to slaves
logic m0_rready;  // From master
logic [2:0] slave_rready;  // To slaves

// Only selected slave sees master's ready
for (genvar s = 0; s < 3; s++) begin
    assign slave_rready[s] = (selected_slave == s) ? m0_rready : 1'b0;
end

// Non-selected slaves see ready = 0 (backpressure)

### 11.7.2	Stall Conditions

Response path can stall when:

1. Master not ready (M_RREADY = 0)
   → Selected slave stalls (S_RREADY = 0)
   → Other slaves buffer or stall

2. Arbitration conflict (multiple slaves ready)
   → Non-selected slaves stalled
   → Selected slave proceeds

3. Response FIFO full
   → Slave stalled until space available

## 11.8	2.8.7 Response Path Latency

### 11.8.1	End-to-End R Channel

Slave R response → Bridge → Master R channel

Components:
1. Slave response valid
2. BID extraction (0 cycles, combinatorial)
3. Demux routing (0-1 cycles)
4. Response arbitration (1 cycle if conflict)
5. Optional FIFO (0-1 cycles)
6. Master adapter (1 cycle, skid buffer)

Total: 2-4 cycles typical

### 11.8.2	End-to-End B Channel

Similar to R channel:
  Slave B → Extraction → Demux → Arbitration → Master B
  Total: 2-4 cycles

## 11.9	2.8.8 Error Response Routing

### 11.9.1	Slave Error Responses

When slave returns error:

RRESP = 2'b10 (SLVERR) or 2'b11 (DECERR)
BRESP = 2'b10 (SLVERR) or 2'b11 (DECERR)

Routing:
  - Extract BID normally
  - Route to originating master
  - Preserve error code
  - Master sees error response

### 11.9.2	Out-of-Range Error Responses

When router generates error for OOR address:

Process:
1. Router captures OOR request
2. Generates internal error response
   - Uses cached request ID (with BID)
   - Sets RRESP/BRESP = DECERR
3. Injects into response path
4. Routes to master using BID

## 11.10	2.8.9 Resource Utilization

### 11.10.1	Response Router Resources

4 masters, 3 slaves (64-bit data):

Logic Elements:  ~1500-2000 LEs
Registers:       ~400-600 regs
Block RAM:       0 (unless FIFOs enabled)

Breakdown:
- BID extraction (3 slaves):     ~150 LEs
- Demux logic (4 masters):        ~600 LEs, ~150 regs
- Response arbitration:           ~300 LEs, ~100 regs
- Backpressure routing:           ~200 LEs, ~50 regs
- Control FSMs:                   ~250 LEs, ~100 regs

Optional FIFOs (4 × 8 entries):  +800 LEs, +2KB BRAM

### 11.10.2	Scaling

Resource usage scales with:
- Number of masters: Linear (one demux path per master)
- Number of slaves: Linear (one extraction per slave)
- Data width: Linear (wider data = wider demux)
- FIFO depth: Linear (deeper = more BRAM)

## 11.11	2.8.10 Configuration Parameters

### 11.11.1	Response Routing Configuration (TOML)

[bridge]
num_masters = 4
num_slaves = 3

[bridge.response_routing]
enable_response_fifos = false       # Buffer responses per master
fifo_depth = 8                      # Entries per FIFO
response_arbiter_type = "round_robin"  # "round_robin", "fixed_priority"
registered_demux = false            # Register demux output (+1 cycle)

# Per-master response credits (optional)
[[masters]]
name = "cpu"
max_response_credits = 16           # Outstanding responses allowed

## 11.12	2.8.11 Debug and Observability

### 11.12.1	Recommended Debug Signals

BID Extraction:
- Slave RID/BID (from each slave)
- Extracted BID (master index)
- External RID/BID (to master, BID stripped)

Response Demux:
- Demux select signals (which master per slave)
- Response valid per master
- Response valid per slave

Arbitration:
- Conflict detection (multiple responses for same master)
- Arbiter grant (which slave wins)
- Stall counters

FIFOs (if enabled):
- FIFO occupancy per master
- FIFO full/empty flags
- FIFO overflow errors

### 11.12.2	Performance Counters

- Responses routed per master
- Arbitration conflicts (cycles with multiple responses to same master)
- Response stall cycles (master not ready)
- Average response latency per master
- FIFO overflow counts
- CAM hits/misses (if CAM enabled)

## 11.13	2.8.12 Common Issues and Debug

Symptom: Response not reaching master
Check: - BID extraction (correct bit slicing?) - Master index (BID → master mapping) - Demux select logic - Backpressure (is master READY?)

Symptom: Response goes to wrong master
Check: - BID values (verify each master has unique BID) - BID width calculation (clog2 correct?) - CAM contents (if used) - ID corruption in transit

Symptom: Response ordering incorrect
Check: - CAM lookup (should preserve order) - Arbitration fairness (round-robin working?) - Multiple outstanding transactions per master

Symptom: Response loss
Check: - FIFO overflows (enable FIFOs if needed) - Dropped responses during arbitration conflicts - Protocol violations (slave not following AXI rules)

## 11.14	2.8.13 Verification Considerations

### 11.14.1	Test Scenarios

- Single Master, Single Slave:

- Basic routing test
- Verify BID extraction and stripping
- Check response reaches correct master

- Multiple Masters, Single Slave:

- Interleaved responses
- Verify each response routes to correct master
- Check BID uniqueness prevents collisions

- Single Master, Multiple Slaves:

- Simultaneous responses from multiple slaves
- Verify arbitration fairness
- Check no response loss

- All Masters, All Slaves:

- Maximum stress test
- All masters issuing to all slaves
- Responses returning in various orders
- Verify correct routing under heavy load

- OOO Responses:

- Issue transactions: ID=1, ID=2, ID=3
- Return responses: ID=3, ID=1, ID=2
- Verify CAM correctly routes each
- Check response order at master

- Backpressure Test:

- Master asserts RREADY=0
- Verify slave stalls (RREADY=0 propagated)
- Master asserts RREADY=1
- Verify responses flow

## 11.15	2.8.14 Performance Optimization

### 11.15.1	Techniques

1. Registered Demux:

Trade-off: +1 cycle latency for timing closure
Best for: High-frequency designs, many masters

2. Response FIFOs:

Trade-off: +2KB BRAM, +1-2 cycle latency
Benefit: Prevent response loss, smooth conflicts
Best for: High-throughput systems

3. Distributed Arbitration:

Implementation: Per-master arbiters instead of global
Benefit: Reduced logic depth, better timing
Best for: >8 masters

4. Priority Response Routing:

Implementation: High-priority masters get preference in conflicts
Benefit: Guaranteed low latency for critical masters
Best for: Real-time systems

## 11.16	2.8.15 Advanced Features

### 11.16.1	Response Reordering (Future)

Allow bridge to reorder responses for efficiency:

Scenario:
  Master issues: Req A (slow slave), Req B (fast slave)
  Fast slave responds first
  Bridge can deliver B before A (if IDs different)

Benefit: Reduced latency for fast responses
Requirement: CAM to track outstanding in-order constraints

### 11.16.2	Response Coalescing (Future)

Combine multiple responses into fewer beats:

Scenario:
  Multiple single-beat reads to narrow slave
  Bridge coaleses into fewer wide beats
  
Benefit: Reduced overhead
Complexity: High (requires buffering and alignment)

### 11.16.3	Response QoS (Future)

Prioritize responses based on master QoS:

Feature: High-QoS masters get response priority
Implementation: Weighted arbitration in response path
Use case: RT masters need bounded response latency

## 11.17	2.8.16 Timing Considerations

### 11.17.1	Critical Paths

Common critical paths in response routing:

1. BID extraction → Demux select → Master RDATA
   Depth: ~8-12 logic levels
   
2. Slave RVALID → Arbitration → Master RVALID
   Depth: ~6-10 logic levels
   
3. Master RREADY → Backpressure → Slave RREADY
   Depth: ~5-8 logic levels

### 11.17.2	Optimization Strategies

- Register demux outputs (+1 cycle, breaks path)
- Pipeline arbitration (+1-2 cycles, multi-stage)
- Use CAM for complex ID scenarios (parallel lookup)
- Limit response buffering depth (less MUX levels)

## 11.18	2.8.17 Future Enhancements

### 11.18.1	Planned Features

- Dynamic Response Buffering: Adjust FIFO depth based on utilization

- Response Ordering Hints: Masters specify ordering requirements

- Multi-Cycle Arbitration: Pipelined for >16 masters

- Response Compression: Pack narrow responses into wide beats

### 11.18.2	Under Consideration

- Response Speculation: Predictive routing before full ID available

- Virtual Response Channels: Separate paths for different QoS classes

- Response Mirroring: Duplicate responses for redundancy

- Cross-Clock Response: Async response paths with CDC

Related Sections: - Section 2.3: Crossbar Core (response path architecture) - Section 2.5: ID Management (BID extraction details) - Section 2.4: Arbitration (response arbitration algorithms) - Section 3.2: Master Port Interface (response channel signals)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	2.9 Error Handling

Error handling is everything the bridge does when something goes wrong: detecting the problem, answering with the right AXI error response, logging what happened, and keeping the rest of the system running. The covered failure modes are protocol violations, out-of-range addresses, timeout conditions, and configuration errors.

## 12.1	2.9.1 Purpose and Function

Error handling does five things:

- Error Detection: Identifies violations and abnormal conditions

- Error Response Generation: Creates appropriate AXI error responses

- Error Reporting: Logs and signals errors for debug

- Graceful Degradation: Maintains system stability during errors

- Recovery Support: Enables system recovery after errors

## 12.2	2.9.2 Error Categories

### 12.2.1	Transaction Errors

Out-of-Range (OOR) Addresses:

Master requests address not mapped to any slave
Response: DECERR (Decode Error)
Action: Bridge generates error response internally

Protocol Violations:

Master violates AXI4 protocol rules
Examples:
  - VALID deasserted while READY=0
  - Incorrect burst parameters
  - ID width mismatches
Response: Configurable (error log, SLVERR, or ignore)

Slave Errors:

Slave returns SLVERR or DECERR response
Action: Bridge forwards error to master unchanged

### 12.2.2	System Errors

Timeout Conditions:

Slave fails to respond within configured timeout
Response: DECERR after timeout expires
Action: Force transaction completion

CAM Overflow:

Too many outstanding transactions (CAM full)
Response: Backpressure master (ARREADY/AWREADY=0)
Action: Wait for responses to free entries

Configuration Errors:

Invalid bridge configuration detected at generation time
Examples:
  - Overlapping slave address ranges
  - Invalid data width combinations
  - Illegal parameter values
Response: Generation error, fail early

## 12.3	2.9.3 Block Diagram

### 12.3.1	Figure 2.9: Error Handling Architecture

Error Handling System

Error handling system showing detection (OOR, protocol, timeout, CAM), generation, and logging subsystems.

## 12.4	2.9.4 Out-of-Range Address Handling

### 12.4.1	Detection

Router identifies OOR when no slave address range matches:

// Address decode with OOR detection
logic [NUM_SLAVES-1:0] slave_match;
logic oor_detected;

for (genvar i = 0; i < NUM_SLAVES; i++) begin
    assign slave_match[i] = (addr >= SLAVE_BASE[i]) && 
                            (addr <= SLAVE_END[i]);
end

assign oor_detected = ~(|slave_match) && ~default_slave_en;

### 12.4.2	Error Response Generation

Read OOR Response:

// Generate R response for OOR read
always_ff @(posedge clk) begin
    if (ar_oor_detected && arvalid && arready) begin
        // Capture transaction details
        oor_arid <= arid;
        oor_arlen <= arlen;
        oor_burst_count <= 0;
        oor_generating <= 1'b1;
    end else if (oor_generating) begin
        // Generate R beats
        rvalid <= 1'b1;
        rdata <= OOR_DATA_PATTERN;  // Configurable pattern
        rid <= oor_arid;
        rresp <= 2'b11;  // DECERR
        rlast <= (oor_burst_count == oor_arlen);
        
        if (rready) begin
            oor_burst_count <= oor_burst_count + 1;
            if (rlast) oor_generating <= 1'b0;
        end
    end
end

Write OOR Response:

// Generate B response for OOR write
always_ff @(posedge clk) begin
    if (aw_oor_detected && awvalid && awready) begin
        // Capture write ID
        oor_awid <= awid;
        oor_w_count <= 0;
        oor_sinking_w <= 1'b1;
    end else if (oor_sinking_w) begin
        // Sink W data (accept and discard)
        wready <= 1'b1;
        
        if (wvalid && wlast) begin
            oor_sinking_w <= 1'b0;
            oor_gen_bresp <= 1'b1;
        end
    end else if (oor_gen_bresp) begin
        // Generate B response
        bvalid <= 1'b1;
        bid <= oor_awid;
        bresp <= 2'b11;  // DECERR
        
        if (bready) begin
            oor_gen_bresp <= 1'b0;
        end
    end
end

### 12.4.3	Configurable Data Patterns

[bridge.error_handling]
oor_read_data_pattern = 0xDEADCAFE  # Pattern for OOR reads
oor_response_latency = 2            # Cycles to generate response

## 12.5	2.9.5 Protocol Violation Handling

### 12.5.1	Common Violations

VALID Dropped While READY=0:

AXI4 Rule: Once VALID asserted, must remain until READY
Violation: Master deasserts VALID before handshake
Detection: Track VALID history per channel
Action: Log error, optionally assert error signal

Incorrect Burst Parameters:

Violations:
  - SIZE > DATA_WIDTH
  - LEN > 255
  - Address not aligned to SIZE
  - Burst crosses 4KB boundary
Detection: Parameter checking logic
Action: SLVERR response or reject transaction

ID Width Mismatch:

Violation: Master uses ID wider than configured
Detection: Check ID values against expected width
Action: Truncate or error (configurable)

### 12.5.2	Protocol Checker Implementation

// AXI4 protocol checker (simplified)
module axi_protocol_checker #(
    parameter ID_WIDTH = 4,
    parameter ADDR_WIDTH = 32,
    parameter DATA_WIDTH = 64
) (
    input logic clk, rst_n,
    // AXI signals
    input logic arvalid, arready,
    input logic [ADDR_WIDTH-1:0] araddr,
    input logic [7:0] arlen,
    input logic [2:0] arsize,
    // Error outputs
    output logic protocol_error,
    output logic [7:0] error_code
);

    // Check: VALID held until READY
    logic arvalid_prev;
    always_ff @(posedge clk) begin
        arvalid_prev <= arvalid;
        if (arvalid_prev && !arvalid && !arready) begin
            protocol_error <= 1'b1;
            error_code <= 8'h01;  // VALID_DROPPED
        end
    end
    
    // Check: SIZE <= DATA_WIDTH
    always_comb begin
        if (arvalid && (arsize > $clog2(DATA_WIDTH/8))) begin
            protocol_error = 1'b1;
            error_code = 8'h02;  // INVALID_SIZE
        end
    end
    
    // Check: Address alignment
    always_comb begin
        if (arvalid && (araddr[arsize-1:0] != 0)) begin
            protocol_error = 1'b1;
            error_code = 8'h03;  // UNALIGNED_ADDR
        end
    end
    
    // Additional checks...
endmodule

## 12.6	2.9.6 Timeout Detection

### 12.6.1	Per-Transaction Watchdog

// Timeout detector for outstanding transactions
typedef struct packed {
    logic valid;
    logic [TOTAL_ID_WIDTH-1:0] id;
    logic [15:0] timestamp;
    logic is_read;  // 1=read, 0=write
} timeout_entry_t;

timeout_entry_t watchdog [0:15];  // Track up to 16 transactions
logic [15:0] global_timer;

always_ff @(posedge clk) begin
    if (!rst_n) begin
        global_timer <= 0;
    end else begin
        global_timer <= global_timer + 1;
        
        // Check for timeouts
        for (int i = 0; i < 16; i++) begin
            if (watchdog[i].valid) begin
                logic [15:0] elapsed;
                elapsed = global_timer - watchdog[i].timestamp;
                
                if (elapsed > TIMEOUT_THRESHOLD) begin
                    // Timeout detected
                    timeout_error <= 1'b1;
                    timeout_id <= watchdog[i].id;
                    timeout_type <= watchdog[i].is_read ? "READ" : "WRITE";
                    
                    // Force completion with DECERR
                    force_error_response(watchdog[i]);
                    watchdog[i].valid <= 1'b0;
                end
            end
        end
    end
end

### 12.6.2	Timeout Configuration

[bridge.error_handling]
enable_timeout = true
timeout_cycles = 10000         # Max cycles before timeout
timeout_action = "decerr"      # "decerr", "slverr", or "hang"
per_slave_timeout = [          # Optional per-slave overrides
    {name = "slow_periph", timeout = 50000},
    {name = "fast_mem", timeout = 1000}
]

## 12.7	2.9.7 Error Logging and Reporting

### 12.7.1	Error Status Register

Error Status Register (Read/Clear):
┌──────┬──────────────────────────────────────┐
│ Bits │ Description                          │
├──────┼──────────────────────────────────────┤
│ 0    │ OOR Read Error                       │
│ 1    │ OOR Write Error                      │
│ 2    │ Protocol Violation                   │
│ 3    │ Timeout Error                        │
│ 4    │ CAM Overflow                         │
│ 5    │ Slave Error (SLVERR received)        │
│ 6    │ Decode Error (DECERR received)       │
│ 7    │ ID Match Error                       │
│15:8  │ Reserved                             │
│31:16 │ Error Count (saturating)             │
└──────┴──────────────────────────────────────┘

### 12.7.2	Error History Buffer

// Circular buffer for error history
typedef struct packed {
    logic [7:0] error_type;
    logic [31:0] address;
    logic [TOTAL_ID_WIDTH-1:0] id;
    logic [31:0] timestamp;
    logic [7:0] master_index;
    logic [7:0] slave_index;
} error_entry_t;

error_entry_t error_log [0:15];  // 16-entry history
logic [3:0] error_wr_ptr;

always_ff @(posedge clk) begin
    if (error_detected) begin
        error_log[error_wr_ptr] <= {
            error_type,
            error_addr,
            error_id,
            global_timer,
            error_master,
            error_slave
        };
        error_wr_ptr <= error_wr_ptr + 1;
    end
end

### 12.7.3	Error Interrupts

Optional interrupt generation:

Interrupt Enable Register:
  - OOR_INT_EN
  - TIMEOUT_INT_EN
  - PROTOCOL_INT_EN
  - etc.

Interrupt when enabled error occurs
Clear on status register read or explicit clear

## 12.8	2.9.8 Resource Utilization

### 12.8.1	Error Handling Resources

Logic Elements:  ~800-1200 LEs
Registers:       ~400-600 regs
Block RAM:       0-2 KB (if error logging enabled)

Breakdown:
- OOR detection/response:     ~300 LEs, ~100 regs
- Protocol checkers:           ~250 LEs, ~50 regs
- Timeout watchers:            ~200 LEs, ~150 regs
- Error logging:               ~150 LEs, ~100 regs
- Status registers:            ~100 LEs, ~50 regs

Optional error log (16 entries): +2KB BRAM

## 12.9	2.9.9 Configuration Parameters

### 12.9.1	Error Handling Configuration (TOML)

[bridge.error_handling]
# OOR handling
enable_oor_errors = true
oor_response_type = "decerr"      # "decerr", "slverr"
oor_read_pattern = 0xDEADCAFE     # Data pattern for OOR reads

# Timeout detection
enable_timeout = true
timeout_cycles = 10000
timeout_response = "decerr"

# Protocol checking
enable_protocol_checker = true
protocol_checker_mode = "log"     # "log", "error", "ignore"

# Error logging
enable_error_log = true
error_log_depth = 16              # Entries in circular buffer

# Error reporting
enable_error_interrupts = false
error_status_register_addr = 0xFFFF_FFF0  # Optional mapped register

## 12.10	2.9.10 Debug and Observability

### 12.10.1	Recommended Debug Signals

Error Detection:
- OOR flags (per master)
- Protocol violation flags
- Timeout counters
- CAM occupancy

Error Response:
- Error response generation FSM states
- Active error responses
- Error data patterns

Error Logging:
- Error count
- Last error type
- Last error address
- Error log write pointer

## 12.11	2.9.11 Common Issues and Debug

Symptom: Unexpected DECERR responses
Check: - Address map configuration (gaps?) - Slave address ranges (overlaps?) - Default slave configuration - Timeout thresholds (too short?)

Symptom: System hangs on certain addresses
Check: - Timeout detection enabled? - Slave responsiveness - Protocol violations upstream - CAM overflow

Symptom: Error log filling quickly
Check: - What errors are occurring? - Master behavior (bad addresses?) - Slave configuration - Timeout thresholds

## 12.12	2.9.12 Verification Considerations

### 12.12.1	Test Scenarios

- OOR Address Tests:

- Read from unmapped address
- Write to unmapped address
- Burst to partially mapped range
- Verify DECERR response

- Timeout Tests:

- Slave never responds
- Verify timeout after N cycles
- Check error logged
- Verify forced completion

- Protocol Violation Tests:

- Drop VALID before READY
- Invalid burst parameters
- Misaligned addresses
- Verify detection and response

- Error Recovery Tests:

- Error occurs
- System continues operating
- Subsequent transactions succeed
- Error status readable

- Error Logging Tests:

- Generate multiple errors
- Verify all logged
- Check circular buffer wrap
- Verify timestamps

## 12.13	2.9.13 Recovery Mechanisms

### 12.13.1	Automatic Recovery

OOR Errors:
  - Generate error response
  - Continue normal operation
  - No intervention needed

Timeout Errors:
  - Force transaction completion
  - Free CAM entry
  - Allow new transactions

Protocol Violations:
  - Log error
  - Complete/reject transaction  
  - Continue with next transaction

### 12.13.2	Manual Recovery

CAM Stuck:
  - Software reset via control register
  - Clear CAM entries
  - Restart affected masters

Persistent Errors:
  - Read error log
  - Identify root cause
  - Reconfigure or reset subsystem

## 12.14	2.9.14 Safety and Reliability

### 12.14.1	Error Containment

Goal: Prevent error propagation

Mechanisms:
- Isolate error to originating master
- Don't corrupt other masters' transactions
- Maintain crossbar integrity
- Log for post-mortem analysis

### 12.14.2	Fail-Safe Behavior

On Critical Error:
1. Generate safe error response (DECERR)
2. Log error details
3. Assert error signal (optional)
4. Continue operation if possible
5. Halt only if configured (safety-critical mode)

### 12.14.3	Error Injection (Debug/Test)

[bridge.debug]
enable_error_injection = true

[[bridge.debug.error_injection]]
type = "oor"
trigger_address = 0xBAD_ADDR
action = "decerr"

[[bridge.debug.error_injection]]
type = "timeout"
trigger_after_n_cycles = 100
target_slave = 2

## 12.15	2.9.15 Future Enhancements

### 12.15.1	Planned Features

- Error Recovery FSM: Automatic recovery from certain error conditions

- Error Rate Limiting: Prevent error storm from misbehaving master

- Detailed Error Telemetry: Capture full transaction context on error

- Error Prediction: Detect patterns indicating impending failures

### 12.15.2	Under Consideration

- ECC Protection: Error correction for internal state

- Redundant Checking: Duplicate checkers for safety-critical

- Watchdog Timers: System-level health monitoring

- Error Severity Levels: Classify errors by impact

Related Sections: - Section 2.2: Slave Router (OOR detection) - Section 2.5: ID Management (CAM overflow) - Section 2.8: Response Routing (error response paths) - Chapter 5: Verification (error testing strategies)

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	AMBA5 Boundary and Native Sideband

How AXI5/APB5 ports ride the AXI4 fabric (BRIDGE-002 phases A5-1 through A5-3). One spec table drives every generator: bin/bridge_pkg/sideband.py::SIDEBAND_FIELDS maps each feature to its per-channel struct fields, widths, and wrapper port bases — package, adapter, crossbar, and slave-adapter emission all iterate it in the same order, so struct layout and wiring cannot drift apart.

## 13.1	Boundary Wrappers

- AXI5 master port → axi5_slave_{wr,rd}[_mon] at the master adapter (the bridge is a slave to the external master).

- AXI5 slave port → axi5_master_{wr,rd}[_mon] at the slave adapter.

- Both families carry every feature signal through their skid path, gated by ENABLE_<FEATURE> parameters; the generator binds every pin (enabled features connect through, the rest tie/open).

## 13.2	Native Sideband Through the Structs (A5-2)

The per-bridge _pkg channel structs gain feature fields as the union of features on any AXI5 port. Pure-AXI4 bridges emit no fields, so their RTL stays byte-identical — the zero-drift invariant.

- Master adapter: packs its own enabled fields from the wrapper’s fub_axi_* sideband on the direct (width-matched) arm only; converter arms pack '0 — per-beat and per-transaction sideband cannot traverse the dwidth-converter IP. Because non-qualifying sources are guaranteed '0, the crossbar forwards request fields unconditionally.

- Crossbar: explodes struct fields into discrete <slave>_axi_<sig> nets for feature-enabled AXI5 slaves; response fields (b.trace, r.trace, r.poison) mux from qualifying slaves and default '0 (they must be driven for every master since union fields exist in every struct).

- Slave adapter: rides xbar_<slave>_axi_<sig> nets into the wrapper via the component’s native_sideband flag, which stops terminating enabled features’ fub ports (fall-through to the connector prefix).

The struct field for AWUNIQUE/ARUNIQUE is named uniq (unique is an SV keyword).

## 13.3	Connectivity Gating (poison, atomic)

AXI5_CONNECTIVITY_GATED_FEATURES in the validator: these features are legal only when every connected path is AXI5-both-ends, feature-enabled, and width-matched — otherwise a config error naming the offending pair. Droppable sideband that terminates mid-path is legal but prints a generation-time warning per (master, slave, feature).

## 13.4	Atomic Filter (A5-3a)

An atomic-enabled master’s wr path inserts axi5_atomic_filter between the boundary wrapper and the fabric:

Diagram 5

Handshakes (AW/W/B) and the B payload (bid/bresp) go through the filter; all other payload gets pref → fub pass-through assigns. Store- class ATOP and plain writes forward; read-return classes are swallowed (W burst consumed) and answered with a local DECERR B. See the module doc: docs/markdown/rtl-amba/axi5/axi5_atomic_filter.md. Read-return atomics proper (A5-3b) need a per-ID tracker shared across a port’s split wr/rd paths and are deferred until a consumer exists.

## 13.5	APB5 Slaves (A5-3c)

protocol = "apb5" reuses the entire APB4 path with two deltas: the slave adapter instantiates axi4_to_apb5_shim (a sideband wrapper over the APB4 shim — see the converters MAS), and the external surface adds PAUSER/PWUSER out (driven '0) and PWAKEUP/PRUSER/PBUSER in (terminated). The generated TB drives the port with the APB4 BFM: APB5 keeps the APB4 transfer protocol.

## 13.6	Verification Anchors

- Generator unit tests: bin/tests/test_generator_pkg.py (feature gating, poison/atomic connectivity rules, generation smoke).

- Sideband values end-to-end: dv/tests/test_bridge_1x2_{rd,wr}_axi5n_sideband.py.

- Atomics: dv/tests/test_bridge_1x2_wr_axi5a_atomics.py and val/amba/test_axi5_atomic_filter.py.

- AXI5 compliance at the boundary: dv/tests/test_bridge_1x2_rd_axi5_bfm5.py (AXI5 BFM + AXI5ComplianceChecker, zero violations).

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

### 13.6.1	Bridge Arbiter Finite State Machines

#### 13.6.1.1	Overview

The Bridge AXI4 crossbar arbitrates per slave, and each arbiter is a simple 2-state finite state machine (FSM). Each slave has two dedicated arbiters:

- AW Arbiter - Write Address channel arbitration

- AR Arbiter - Read Address channel arbitration

Total FSM Count: 2 × NUM_SLAVES

Round-robin access keeps any master from starving, and two states keep the behavior easy to reason about.

#### 13.6.1.2	AW Channel Arbiter FSM

Purpose: Arbitrate write address requests from multiple masters to a single slave

States: 2 (IDLE, GRANT_ACTIVE)

Algorithm: Round-robin with grant locking

### 13.6.2	Figure 5.3: AW Arbiter FSM

AW Arbiter FSM

State Descriptions:

IDLE State: - Entry Actions: - aw_grant_matrix[s] = 0 - No grant issued - aw_grant_active[s] = 0 - Arbiter inactive - Behavior: - Monitor aw_request_matrix[s] for master requests - Track round-robin priority via aw_last_grant[s] - Remain in IDLE until at least one master requests access

GRANT_ACTIVE State: - Entry Actions: - aw_grant_matrix[s][m] = 1 - Issue grant to winning master - aw_grant_active[s] = 1 - Mark arbiter as active - Behavior: - Hold grant until AW handshake completes (AWVALID && AWREADY) - W channel data multiplexing follows AW grant - B channel response uses ID table (not grant-based)

State Transitions:

| From | To | Condition | Description |
| --- | --- | --- | --- |
| IDLE | GRANT_ACTIVE | \¦aw_request_matrix[s]\¦ > 0 | At least one master requesting access |
| GRANT_ACTIVE | IDLE | m_axi_awvalid[s] && m_axi_awready[s] | AW handshake complete |

#### 13.6.2.1	AR Channel Arbiter FSM

Purpose: Arbitrate read address requests from multiple masters to a single slave

States: 2 (IDLE, GRANT_ACTIVE)

Algorithm: Round-robin with grant locking (same as AW)

### 13.6.3	Figure 5.4: AR Arbiter FSM

AR Arbiter FSM

State Descriptions:

IDLE State: - Entry Actions: - ar_grant_matrix[s] = 0 - No grant issued - ar_grant_active[s] = 0 - Arbiter inactive - Behavior: - Monitor ar_request_matrix[s] for master requests - Track round-robin priority via ar_last_grant[s] - Independent from AW arbiter state

GRANT_ACTIVE State: - Entry Actions: - ar_grant_matrix[s][m] = 1 - Issue grant to winning master - ar_grant_active[s] = 1 - Mark arbiter as active - Behavior: - Hold grant until AR handshake completes (ARVALID && ARREADY) - R channel data multiplexing follows AR grant - R channel response uses ID table for routing

State Transitions:

| From | To | Condition | Description |
| --- | --- | --- | --- |
| IDLE | GRANT_ACTIVE | \¦ar_request_matrix[s]\¦ > 0 | At least one master requesting read |
| GRANT_ACTIVE | IDLE | m_axi_arvalid[s] && m_axi_arready[s] | AR handshake complete |

#### 13.6.3.1	Round-Robin Arbitration Algorithm

Algorithm Description:

Both AW and AR arbiters use the same fair round-robin arbitration algorithm:

Pseudocode:
1. Start search from (last_grant[s] + 1) % NUM_MASTERS
2. Search cyclically through all masters
3. Select first master with pending request
4. Update last_grant[s] = winning_master
5. Issue grant

Example with 4 Masters:

Initial state: last_grant[0] = 0

Request Pattern:
  Master 0: request
  Master 1: request
  Master 2: no request
  Master 3: request

Arbitration Sequence:
  Grant 1: Master 1 (start from master 1, first requester)
          last_grant = 1

  Grant 2: Master 3 (start from master 2, find master 3)
          last_grant = 3

  Grant 3: Master 0 (start from master 0, first requester)
          last_grant = 0

  Grant 4: Master 1 (start from master 1, first requester)
          last_grant = 1

Fairness Properties:

- No Starvation - Every requesting master will eventually receive grant

- Priority Rotation - Priority shifts after each grant

- Predictable Latency - Maximum wait time = (NUM_MASTERS - 1) × grant_duration

- Equal Opportunity - All masters treated equally

#### 13.6.3.2	Grant Locking Mechanism

Purpose: Prevent grant changes mid-transaction

AW Grant Locking:

// AW grant locked until address handshake completes
always_ff @(posedge aclk or negedge aresetn) begin
    if (!aresetn) begin
        aw_grant_active[s] <= 1'b0;
        aw_grant_matrix[s] <= '0;
    end else begin
        if (aw_grant_active[s]) begin
            // GRANT_ACTIVE state: Hold grant until handshake
            if (m_axi_awvalid[s] && m_axi_awready[s]) begin
                aw_grant_active[s] <= 1'b0;  // Return to IDLE
                aw_grant_matrix[s] <= '0;
            end
        end else if (|aw_request_matrix[s]) begin
            // IDLE state: Arbitrate if requests pending
            aw_grant_matrix[s] <= round_robin_select(aw_request_matrix[s]);
            aw_grant_active[s] <= 1'b1;  // Enter GRANT_ACTIVE
        end
    end
end

AR Grant Locking:

// AR grant locked until address handshake completes
always_ff @(posedge aclk or negedge aresetn) begin
    if (!aresetn) begin
        ar_grant_active[s] <= 1'b0;
        ar_grant_matrix[s] <= '0;
    end else begin
        if (ar_grant_active[s]) begin
            // GRANT_ACTIVE state: Hold grant until handshake
            if (m_axi_arvalid[s] && m_axi_arready[s]) begin
                ar_grant_active[s] <= 1'b0;  // Return to IDLE
                ar_grant_matrix[s] <= '0;
            end
        end else if (|ar_request_matrix[s]) begin
            // IDLE state: Arbitrate if requests pending
            ar_grant_matrix[s] <= round_robin_select(ar_request_matrix[s]);
            ar_grant_active[s] <= 1'b1;  // Enter GRANT_ACTIVE
        end
    end
end

Why Grant Locking Matters:

- Protocol Compliance - AXI4 spec requires stable AWID/ARID during handshake

- Data Integrity - W channel must follow granted master’s AW request

- Response Routing - B/R channels need consistent transaction tracking

- Simplicity - Single grant active per slave prevents mux conflicts

#### 13.6.3.3	Independent Read/Write Arbitration

Key Design Feature: AW and AR arbiters operate completely independently

Benefits:

- No Head-of-Line Blocking:

  - Read requests don’t wait for write completion

  - Write requests don’t wait for read completion

- Parallel Operation:

  - Master 0 can write to Slave 0 (AW path)

  - Master 1 can read from Slave 0 (AR path)

  - Both transactions proceed simultaneously

- Resource Efficiency:

  - Full utilization of read/write bandwidth

  - No artificial serialization

Example Scenario:

Time T0: Master 0 issues write to Slave 0
         - AW arbiter grants to Master 0
         - AW_GRANT_ACTIVE = 1
         - W data follows

Time T1: Master 1 issues read to Slave 0 (during Master 0 write)
         - AR arbiter grants to Master 1 (independent!)
         - AR_GRANT_ACTIVE = 1
         - R data returns

Result: Read and write happen in parallel - no blocking

#### 13.6.3.4	FSM Instance Breakdown by Configuration

2×2 Configuration (2 masters, 2 slaves): - Slave 0 AW Arbiter: 1 FSM - Slave 0 AR Arbiter: 1 FSM - Slave 1 AW Arbiter: 1 FSM - Slave 1 AR Arbiter: 1 FSM - Total: 4 FSMs

4×4 Configuration (4 masters, 4 slaves): - 4 slaves × 2 arbiters per slave = 8 FSMs

8×8 Configuration (8 masters, 8 slaves): - 8 slaves × 2 arbiters per slave = 16 FSMs

Scalability: - FSM count scales linearly with NUM_SLAVES - Arbitration complexity scales with NUM_MASTERS (search time) - Synthesis impact minimal for up to 16×16 configurations

#### 13.6.3.5	Performance Characteristics

Latency:

- Best Case: 1 clock cycle (IDLE → GRANT_ACTIVE → IDLE with immediate handshake)

- Worst Case: (NUM_MASTERS) clock cycles (round-robin search + contention)

- Average Case: ~(NUM_MASTERS / 2) clock cycles

Throughput:

- Back-to-Back Grants: Possible if different masters (no wait)

- Same Master Repeated: 1 cycle minimum between grants (FSM state change)

Fairness:

- Guaranteed: Every master gets grant within (NUM_MASTERS - 1) arbitration cycles

- No Priority: All masters treated equally (can be extended for QoS)

#### 13.6.3.6	Comparison with Other Crossbar Arbiters

| Feature | Bridge Arbiter | APB Crossbar | Commercial Tools |
| --- | --- | --- | --- |
| States | 2 (IDLE, GRANT_ACTIVE) | 1 (passthrough) | 3-5 (complex) |
| Algorithm | Round-robin | N/A (APB is 1:1) | Priority, QoS, weighted |
| Grant Locking | Yes (until handshake) | N/A | Burst-aware locking |
| Read/Write Indep | Yes (2 FSMs/slave) | N/A | Yes |
| Complexity | Low | Minimal | High |
| Predictability | High (round-robin) | N/A | Medium (priority-based) |

Simplicity Trade-off:

- Advantages:

  - Easy to verify (2-state FSM)

  - Predictable latency

  - Fair resource allocation

- Limitations (future enhancements):

  - No Quality-of-Service (QoS) support

  - No priority levels

  - No weighted arbitration

#### 13.6.3.7	Future Enhancements (Phase 3+)

QoS Support:

// Master QoS priority field (AXI4 optional)
input logic [3:0] s_axi_awqos [NUM_MASTERS];

// Arbitration considers QoS priority
function automatic [NUM_MASTERS-1:0] qos_arbitrate(
    input logic [NUM_MASTERS-1:0] requests,
    input logic [3:0] qos [NUM_MASTERS]
);
    // Higher QoS value = higher priority
    // Round-robin among same QoS level
endfunction

Weighted Arbitration:

// Master weight configuration
parameter int MASTER_WEIGHTS [NUM_MASTERS] = '{1, 2, 1, 4};

// Grant frequency proportional to weight
// Master 3 gets 4× more grants than Master 0

Burst-Aware Locking:

// Lock grant until entire burst completes
// Currently: Lock until address handshake
// Enhanced: Lock until WLAST (write) or RLAST (read)

See Also: - 1.1 - Introduction - Bridge overview - 3.1 - Module Structure - Generated RTL organization - 3.3 - Crossbar Core - Internal crossbar instantiation

Reference: - ARM AMBA AXI4 Specification (IHI 0022E) - Section A7 (Arbitration)

Next: 3.3 - Crossbar Core

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	Transaction Tracking FSMs

## 14.1	Overview

Several small FSMs track outstanding transactions so responses route correctly. They work alongside the ID management CAM, each holding one piece of the transaction state.

## 14.2	Write Data Tracking FSM

### 14.2.1	Purpose

The W channel carries no address of its own, so this FSM remembers which slave the AW handshake selected and steers the data beats to match.

### 14.2.2	States

| State | Description |
| --- | --- |
| IDLE | Waiting for AW handshake |
| WRITING | Routing W beats to target slave |

### 14.2.3	Figure 3.1: Write Data Tracking FSM

Write Data Tracking FSM

### 14.2.4	Implementation

typedef enum logic [0:0] {
    W_IDLE    = 1'b0,
    W_WRITING = 1'b1
} w_state_t;

w_state_t r_w_state;
logic [$clog2(NUM_SLAVES)-1:0] r_w_target;

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_w_state <= W_IDLE;
        r_w_target <= '0;
    end else begin
        case (r_w_state)
            W_IDLE: begin
                if (awvalid && awready) begin
                    r_w_state <= W_WRITING;
                    r_w_target <= decoded_slave;
                end
            end
            W_WRITING: begin
                if (wvalid && wready && wlast) begin
                    r_w_state <= W_IDLE;
                end
            end
        endcase
    end
end

## 14.3	Response Pending Counter

### 14.3.1	Purpose

Track number of outstanding transactions per master for flow control.

### 14.3.2	Implementation

logic [7:0] r_outstanding [NUM_MASTERS];

// Increment on AR/AW acceptance
always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        for (int i = 0; i < NUM_MASTERS; i++)
            r_outstanding[i] <= '0;
    end else begin
        for (int m = 0; m < NUM_MASTERS; m++) begin
            logic inc = (ar_accepted[m] || aw_accepted[m]);
            logic dec = (r_delivered[m] || b_delivered[m]);

            case ({inc, dec})
                2'b10: r_outstanding[m] <= r_outstanding[m] + 1;
                2'b01: r_outstanding[m] <= r_outstanding[m] - 1;
                default: r_outstanding[m] <= r_outstanding[m];
            endcase
        end
    end
end

// Backpressure when outstanding limit reached
assign accept_ar[m] = (r_outstanding[m] < MAX_OUTSTANDING);
assign accept_aw[m] = (r_outstanding[m] < MAX_OUTSTANDING);

## 14.4	Burst Counter FSM

### 14.4.1	Purpose

Track burst beat count for responses spanning multiple cycles.

### 14.4.2	States

| State | Description |
| --- | --- |
| IDLE | No burst in progress |
| COUNTING | Tracking burst beats |

### 14.4.3	Implementation

logic [7:0] r_burst_count;
logic r_burst_active;

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_burst_count <= '0;
        r_burst_active <= 1'b0;
    end else begin
        if (!r_burst_active && rvalid && rready) begin
            // Start new burst
            r_burst_active <= !rlast;
            r_burst_count <= rlast ? '0 : 8'd1;
        end else if (r_burst_active && rvalid && rready) begin
            // Continue burst
            r_burst_count <= r_burst_count + 1;
            r_burst_active <= !rlast;
        end
    end
end

## 14.5	Related Documentation

- Arbiter FSMs - Address channel arbitration

- ID Management - CAM for response tracking

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	CAM Architecture

## 15.1	Overview

The CAM is what makes out-of-order response routing possible: it records which master owns each outstanding transaction ID, and a lookup answers the routing question in one shot.

## 15.2	CAM Purpose

### 15.2.1	Transaction Tracking

The CAM stores: - Transaction ID (key) - Originating master index (value) - Transaction metadata (optional)

### 15.2.2	Response Routing

When a response arrives: 1. Extract transaction ID from response 2. CAM lookup returns originating master 3. Route response to correct master

## 15.3	CAM Structure

### 15.3.1	Figure 4.1: CAM Entry Format

CAM Entry Format

### 15.3.2	CAM Sizing

CAM Depth = MAX_OUTSTANDING × NUM_MASTERS
CAM Width = TOTAL_ID_WIDTH + BID_WIDTH + METADATA_WIDTH

### 15.3.3	Example Configuration

4 masters, 4-bit external ID, max 16 outstanding per master:
  BID_WIDTH = clog2(4) = 2
  TOTAL_ID_WIDTH = 4 + 2 = 6
  CAM Depth = 16 × 4 = 64 entries
  CAM Width = 6 + 2 + 8 = 16 bits

## 15.4	CAM Operations

### 15.4.1	Insertion (AR/AW Acceptance)

// On AR handshake
if (arvalid && arready) begin
    cam_write_en <= 1'b1;
    cam_write_id <= {bid, arid};  // Key
    cam_write_master <= master_idx;  // Value
end

### 15.4.2	Lookup (R/B Response)

// On R response from slave
logic [BID_WIDTH-1:0] response_master;
logic cam_hit;

cam_lookup(slave_rid, response_master, cam_hit);

if (cam_hit) begin
    route_response_to(response_master);
end else begin
    // Error: Unknown transaction
end

### 15.4.3	Deletion (Response Complete)

// On RLAST or B response
if ((rvalid && rready && rlast) || (bvalid && bready)) begin
    cam_delete_en <= 1'b1;
    cam_delete_id <= response_id;
end

## 15.5	Implementation Options

### 15.5.1	Distributed RAM CAM

For small outstanding counts (< 16 entries):

// Parallel comparator-based CAM
logic [DEPTH-1:0] valid;
logic [ID_WIDTH-1:0] id_table [DEPTH];
logic [BID_WIDTH-1:0] master_table [DEPTH];

// Parallel compare for lookup
logic [DEPTH-1:0] match;
for (genvar i = 0; i < DEPTH; i++) begin
    assign match[i] = valid[i] && (id_table[i] == lookup_id);
end

### 15.5.2	Block RAM CAM

For larger outstanding counts (> 16 entries):

// Sequential search CAM (saves logic)
logic [$clog2(DEPTH)-1:0] search_idx;
logic searching;

always_ff @(posedge clk) begin
    if (start_lookup) begin
        searching <= 1'b1;
        search_idx <= 0;
    end else if (searching) begin
        if (match_found || search_idx == DEPTH-1)
            searching <= 1'b0;
        else
            search_idx <= search_idx + 1;
    end
end

## 15.6	CAM Overflow Handling

### 15.6.1	Prevention

// Backpressure when CAM full
assign ar_ready = !cam_full && downstream_ready;
assign aw_ready = !cam_full && downstream_ready;

### 15.6.2	Error Response

If CAM overflow would occur: 1. Assert backpressure (ARREADY/AWREADY = 0) 2. Wait for responses to free entries 3. Never drop transactions

## 15.7	Related Documentation

- ID Tracking - ID table implementation

- Response Routing - Using CAM for routing

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	ID Tracking Tables

## 16.1	Overview

ID tracking tables hold the mapping from extended IDs (external ID plus Bridge ID) back to the originating master — the information response routing needs in a multi-master system.

## 16.2	Table Structure

### 16.2.1	Per-Slave ID Table

Each slave has its own ID tracking table:

### 16.2.2	Figure 4.2: ID Table Structure

ID Table Structure

### 16.2.3	Table Sizing

Entries = MAX_OUTSTANDING_PER_SLAVE
Width = TOTAL_ID_WIDTH + clog2(NUM_MASTERS) + 1 (valid)

## 16.3	ID Extension

### 16.3.1	Master-Side Extension

When transaction enters Bridge:

// Extend ID with Bridge ID (master index)
logic [TOTAL_ID_WIDTH-1:0] extended_id;
assign extended_id = {master_bid, external_id};

// Example: Master 2, external ID = 4'hA
// master_bid = 2'b10, external_id = 4'b1010
// extended_id = 6'b10_1010

### 16.3.2	Slave-Side Presentation

Extended ID goes to slave:

assign s_axi_arid = extended_id;  // 6 bits to slave
assign s_axi_awid = extended_id;  // 6 bits to slave

## 16.4	ID Extraction

### 16.4.1	Response Parsing

When response returns from slave:

// Extract Bridge ID and external ID
logic [BID_WIDTH-1:0] bridge_id;
logic [ID_WIDTH-1:0] external_id;

assign bridge_id = slave_rid[TOTAL_ID_WIDTH-1:ID_WIDTH];
assign external_id = slave_rid[ID_WIDTH-1:0];

// Route to master based on bridge_id
assign m_rvalid[bridge_id] = s_rvalid;
assign m_rid[bridge_id] = external_id;  // Strip Bridge ID

## 16.5	Table Operations

### 16.5.1	Allocation (AR/AW Phase)

always_ff @(posedge clk) begin
    if (arvalid && arready) begin
        // Find free entry
        for (int i = 0; i < DEPTH; i++) begin
            if (!valid[i]) begin
                id_table[i] <= extended_arid;
                master_table[i] <= master_idx;
                valid[i] <= 1'b1;
                break;
            end
        end
    end
end

### 16.5.2	Lookup (R/B Phase)

// Combinational lookup
logic [NUM_MASTERS-1:0] master_onehot;
always_comb begin
    master_onehot = '0;
    for (int i = 0; i < DEPTH; i++) begin
        if (valid[i] && (id_table[i] == response_id)) begin
            master_onehot[master_table[i]] = 1'b1;
        end
    end
end

### 16.5.3	Deallocation (Response Complete)

always_ff @(posedge clk) begin
    if (rvalid && rready && rlast) begin
        // Find and invalidate matching entry
        for (int i = 0; i < DEPTH; i++) begin
            if (valid[i] && (id_table[i] == response_rid)) begin
                valid[i] <= 1'b0;
                break;
            end
        end
    end
end

## 16.6	Multi-ID Considerations

### 16.6.1	Same External ID, Different Masters

Master 0 issues ID=5 → Extended: 00_0101
Master 1 issues ID=5 → Extended: 01_0101
Master 2 issues ID=5 → Extended: 10_0101

All three can be outstanding simultaneously!
The extended ID ensures uniqueness.

### 16.6.2	Same Extended ID (Error)

This cannot happen: - Bridge ID is unique per master - Extended ID = {unique BID, external ID} - Same extended ID implies same master + same external ID - AXI4 requires unique IDs per master for outstanding transactions

## 16.7	Resource Utilization

### 16.7.1	Table Resources

4 masters, 4-bit external ID, 16 outstanding per slave:

Per-slave table:
  Entries: 16
  Width: 6 + 2 + 1 = 9 bits
  Total: 16 × 9 = 144 bits

4 slaves:
  Total: 4 × 144 = 576 bits (~72 bytes)

Implementation:
  Distributed RAM: ~150 LEs
  Block RAM: 1 BRAM (minimum)

## 16.8	Related Documentation

- CAM Architecture - CAM implementation details

- Response Routing - Using tables for routing

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	Width Converters

## 17.1	Overview

When a master and slave disagree on data width, a converter sits between them: an upsizer for narrow-to-wide, a downsizer for wide-to-narrow. The generator inserts them automatically wherever the configuration needs one.

## 17.2	Upsize Converter

### 17.2.1	Purpose

Convert narrow master data to wide slave interface.

### 17.2.2	Figure 5.1: Upsize Converter (64-bit to 512-bit)

Upsize Converter

### 17.2.3	Implementation

module width_upsize #(
    parameter IN_WIDTH = 64,
    parameter OUT_WIDTH = 512
) (
    input  logic clk, rst_n,

    // Narrow input
    input  logic                    s_valid,
    output logic                    s_ready,
    input  logic [IN_WIDTH-1:0]     s_data,
    input  logic                    s_last,

    // Wide output
    output logic                    m_valid,
    input  logic                    m_ready,
    output logic [OUT_WIDTH-1:0]    m_data,
    output logic                    m_last
);

    localparam RATIO = OUT_WIDTH / IN_WIDTH;

    logic [OUT_WIDTH-1:0] r_buffer;
    logic [$clog2(RATIO)-1:0] r_count;

    always_ff @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            r_count <= '0;
            r_buffer <= '0;
        end else if (s_valid && s_ready) begin
            // Pack input into buffer
            r_buffer[r_count * IN_WIDTH +: IN_WIDTH] <= s_data;

            if (s_last || r_count == RATIO - 1) begin
                r_count <= '0;
            end else begin
                r_count <= r_count + 1;
            end
        end
    end

    // Output when buffer full or last beat
    assign m_valid = (r_count == RATIO - 1) || s_last;
    assign m_data = r_buffer;
    assign s_ready = !m_valid || m_ready;

endmodule

### 17.2.4	Burst Length Conversion

Table 5.3: Burst Length Conversion Examples

| Master AWLEN | Master Beats | Slave AWLEN | Slave Beats |
| --- | --- | --- | --- |
| 7 (8 beats) | 8 × 64b | 0 (1 beat) | 1 × 512b |
| 15 (16 beats) | 16 × 64b | 1 (2 beats) | 2 × 512b |
| 255 (256 beats) | 256 × 64b | 31 (32 beats) | 32 × 512b |

## 17.3	Downsize Converter

### 17.3.1	Purpose

Convert wide master data to narrow slave interface.

### 17.3.2	Figure 5.2: Downsize Converter (512-bit to 64-bit)

Downsize Converter

### 17.3.3	Implementation

module width_downsize #(
    parameter IN_WIDTH = 512,
    parameter OUT_WIDTH = 64
) (
    input  logic clk, rst_n,

    // Wide input
    input  logic                    s_valid,
    output logic                    s_ready,
    input  logic [IN_WIDTH-1:0]     s_data,
    input  logic                    s_last,

    // Narrow output
    output logic                    m_valid,
    input  logic                    m_ready,
    output logic [OUT_WIDTH-1:0]    m_data,
    output logic                    m_last
);

    localparam RATIO = IN_WIDTH / OUT_WIDTH;

    logic [IN_WIDTH-1:0] r_buffer;
    logic [$clog2(RATIO)-1:0] r_count;
    logic r_active;

    always_ff @(posedge clk or negedge rst_n) begin
        if (!rst_n) begin
            r_count <= '0;
            r_buffer <= '0;
            r_active <= 1'b0;
        end else begin
            if (!r_active && s_valid) begin
                // Load new wide word
                r_buffer <= s_data;
                r_active <= 1'b1;
                r_count <= '0;
            end else if (r_active && m_ready) begin
                if (r_count == RATIO - 1) begin
                    r_active <= 1'b0;
                end else begin
                    r_count <= r_count + 1;
                end
            end
        end
    end

    // Output from buffer slice
    assign m_valid = r_active;
    assign m_data = r_buffer[r_count * OUT_WIDTH +: OUT_WIDTH];
    assign m_last = r_active && (r_count == RATIO - 1);
    assign s_ready = !r_active;

endmodule

## 17.4	Strobe Handling

### 17.4.1	Upsize Strobe Packing

// Pack 8-byte strobes into 64-byte strobes
logic [7:0] in_strb;   // 64-bit input
logic [63:0] out_strb; // 512-bit output

always_ff @(posedge clk) begin
    if (s_valid && s_ready) begin
        out_strb[r_count * 8 +: 8] <= in_strb;
    end
end

### 17.4.2	Downsize Strobe Splitting

// Split 64-byte strobes into 8-byte strobes
logic [63:0] in_strb;  // 512-bit input
logic [7:0] out_strb;  // 64-bit output

assign out_strb = in_strb[r_count * 8 +: 8];

## 17.5	Resource Utilization

### 17.5.1	Upsize Converter (64 to 512)

Registers: ~520 (buffer + control)
Logic: ~200 LEs (packing + control)

### 17.5.2	Downsize Converter (512 to 64)

Registers: ~520 (buffer + control)
Logic: ~150 LEs (selection + control)

## 17.6	AXIL-to-Wider-Slave Master-Side Alignment Converters

An AXI4-Lite master talking to a wider AXI4 slave through the bridge has an alignment problem: AXIL single-beat transactions don’t know where they land in a wide word. The bridge generator emits master-side alignment converters (axil_to_axi4_wide_align_{rd,wr}.sv) to handle partial-word alignment on the AXIL side while producing properly-aligned transactions on the wide slave side.

### 17.6.1	Use Case Example

32-bit AXIL Master → Bridge → 64-bit AXI4 Slave

Master writes 32-bit word to offset 0x04:
  Data = 0xAABBCCDD (on bits [31:0])

The converter picks the lane from the address, then ROW-ALIGNS the address:
  slot      = addr[2]                 = 1      (SLOT_LSB=$clog2(4)=2, SLOT_W=$clog2(2)=1)
  m_axi_wdata[63:32] = 0xAABBCCDD             (slot 1 -> the UPPER half)
  m_axi_wstrb        = 0xF0                   (byte lanes 4..7)
  m_axi_awaddr       = 0x04 & ~0x7 = 0x00     (row_addr_mask clears the lane bits)

  Slave sees: 0xAABBCCDD_00000000 at address 0x00, WSTRB 0xF0

The address the slave sees is 0x00, NOT 0x04: the lane bits move out of the
address and into WSTRB. Byte lanes outside the strobe are don't-care, shown
here as zero.

### 17.6.2	Architecture

Read Path: - Master issues AXIL read (no burst, single beat) - Converter passes address to slave (unchanged) - Slave returns 64-bit word - Converter extracts the relevant 32-bit slice for the master

Write Path: - Master issues AXIL write with address offset and 32-bit data - Converter computes byte-lane position within 64-bit word - Converter expands 32-bit strobe to 64-bit strobe (8 lanes) - Converter sends full 64-bit write to slave - Slave performs a 64-bit write with byte strobes

### 17.6.3	Modules

- axil_to_axi4_wide_align_rd.sv: Read path (address → data conversion)

- axil_to_axi4_wide_align_wr.sv: Write path (strobe expansion, data placement)

Note: These converters handle width alignment, not protocol bridging. Protocol conversion (AXIL→AXI4 burst restriction) is applied separately at the slave boundary (via axi4_to_axil4_* shims if needed).

## 17.7	Related Documentation

- Width Conversion Block - Block-level description

- APB Converters - Protocol conversion

- Generator-Emitted Conversion Shims - Master-side vs. slave-side shim placement

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	APB Converters

## 18.1	Overview

APB converters turn AXI4 transactions into APB transfers — the price a high-performance master pays to talk to a low-speed peripheral.

## 18.2	Conversion Requirements

### 18.2.1	Protocol Differences

The two protocols differ in ways that force real work on the converter:

Table 5.1: AXI4 vs APB Protocol Comparison

| Feature | AXI4 | APB |
| --- | --- | --- |
| Channels | 5 (AW, W, B, AR, R) | 1 (combined) |
| Bursts | Up to 256 beats | None (single) |
| Pipelining | Yes | No |
| Min cycles | 1 per beat | 2 per transfer |

### 18.2.2	Conversion Strategy

- Split AXI4 bursts into individual APB transfers

- Serialize AW+W into APB write sequence

- Convert AR into APB read sequence

- Generate AXI4 responses from APB completions

## 18.3	Write Conversion

### 18.3.1	Figure 5.3: AXI4 Write to APB Write

AXI4 to APB Write

### 18.3.2	State Machine

typedef enum logic [2:0] {
    IDLE,
    AW_ACCEPT,
    W_ACCEPT,
    APB_SETUP,
    APB_ACCESS,
    B_GENERATE
} apb_wr_state_t;

apb_wr_state_t r_state;

always_ff @(posedge clk or negedge rst_n) begin
    if (!rst_n) begin
        r_state <= IDLE;
    end else begin
        case (r_state)
            IDLE: if (awvalid) r_state <= AW_ACCEPT;

            AW_ACCEPT: begin
                if (awready) r_state <= W_ACCEPT;
            end

            W_ACCEPT: begin
                if (wvalid && wready) r_state <= APB_SETUP;
            end

            APB_SETUP: r_state <= APB_ACCESS;

            APB_ACCESS: begin
                if (pready) begin
                    if (r_beat_count == r_awlen)
                        r_state <= B_GENERATE;
                    else
                        r_state <= W_ACCEPT;  // Next beat
                end
            end

            B_GENERATE: begin
                if (bready) r_state <= IDLE;
            end
        endcase
    end
end

## 18.4	Read Conversion

### 18.4.1	Figure 5.4: AXI4 Read to APB Read

AXI4 to APB Read

### 18.4.2	Read State Machine

typedef enum logic [2:0] {
    IDLE,
    AR_ACCEPT,
    APB_SETUP,
    APB_ACCESS,
    R_GENERATE
} apb_rd_state_t;

## 18.5	Burst Handling

### 18.5.1	Burst to Single Conversion

AXI4 Burst (AWLEN=7, 8 beats):
  Beat 0: AWADDR + 0×SIZE
  Beat 1: AWADDR + 1×SIZE
  Beat 2: AWADDR + 2×SIZE
  ...
  Beat 7: AWADDR + 7×SIZE

APB Sequence:
  Transfer 0: PADDR = AWADDR + 0×SIZE
  Transfer 1: PADDR = AWADDR + 1×SIZE
  ...
  Transfer 7: PADDR = AWADDR + 7×SIZE

### 18.5.2	Address Calculation

// Calculate APB address for each beat
logic [ADDR_WIDTH-1:0] apb_addr;
logic [7:0] beat_count;

always_comb begin
    case (r_burst_type)
        FIXED: apb_addr = r_base_addr;
        INCR:  apb_addr = r_base_addr + (beat_count << r_size);
        WRAP:  apb_addr = wrap_address(r_base_addr, beat_count, r_size, r_len);
    endcase
end

## 18.6	Error Handling

### 18.6.1	APB Error to AXI4 Response

Table 5.2: APB to AXI4 Error Mapping

| APB PSLVERR | AXI4 Response |
| --- | --- |
| 0 (OK) | OKAY (2’b00) |
| 1 (Error) | SLVERR (2’b10) |

### 18.6.2	Error Propagation

// Latch first error in burst
logic r_error_seen;

always_ff @(posedge clk) begin
    if (r_state == APB_ACCESS && pready && pslverr)
        r_error_seen <= 1'b1;
    if (r_state == IDLE)
        r_error_seen <= 1'b0;
end

// Final response reflects any errors
assign bresp = r_error_seen ? 2'b10 : 2'b00;

## 18.7	Performance Considerations

### 18.7.1	Throughput Impact

AXI4 alone: 1 beat per cycle (peak)
AXI4 to APB: 1 beat per 2+ cycles (minimum)

Throughput reduction: ~50% or more

### 18.7.2	Latency Impact

AXI4 read: 1-2 cycles (typical)
AXI4-to-APB read: 4+ cycles minimum
  - 1 cycle: AR accept
  - 1 cycle: APB setup
  - 1+ cycles: APB access (PREADY)
  - 1 cycle: R generate

## 18.8	Resource Utilization

### 18.8.1	APB Converter Resources

Logic Elements: ~300-400 LEs
Registers: ~100-150 regs
Block RAM: 0

Breakdown:
- State machine: ~50 LEs, ~20 regs
- Address calc: ~100 LEs, ~50 regs
- Beat counter: ~30 LEs, ~8 regs
- Data buffering: ~100 LEs, ~DATA_WIDTH regs

## 18.9	Related Documentation

- Protocol Conversion Block - Block description

- Width Converters - Data width conversion

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	Module Structure

## 19.1	Generated RTL Overview

The generator turns configuration files into parameterized SystemVerilog modules, and every topology gets the same internal structure. Once you’ve read one generated bridge, you can find your way around any of them.

## 19.2	Top-Level Module

### 19.2.1	Module Declaration

module bridge_4x3 #(
    parameter int NUM_MASTERS = 4,
    parameter int NUM_SLAVES = 3,
    parameter int ADDR_WIDTH = 32,
    parameter int DATA_WIDTH = 64,
    parameter int ID_WIDTH = 4
) (
    // Clock and reset
    input  logic aclk,
    input  logic aresetn,

    // Master interfaces (M0-M3)
    // ... master port signals ...

    // Slave interfaces (S0-S2)
    // ... slave port signals ...
);

### 19.2.2	Port Organization

Module Ports:
├── Clock/Reset
│   ├── aclk
│   └── aresetn
├── Master Ports (per master)
│   ├── AW Channel (if rw or wr)
│   ├── W Channel (if rw or wr)
│   ├── B Channel (if rw or wr)
│   ├── AR Channel (if rw or rd)
│   └── R Channel (if rw or rd)
└── Slave Ports (per slave)
    ├── AW Channel
    ├── W Channel
    ├── B Channel
    ├── AR Channel
    └── R Channel

## 19.3	Internal Structure

### 19.3.1	Component Instantiation

// Generated module internal structure

// Master adapters (per master)
master_adapter_rw u_m0_adapter (...);
master_adapter_wr u_m1_adapter (...);
master_adapter_rd u_m2_adapter (...);
master_adapter_rw u_m3_adapter (...);

// Address decoders (per master)
address_decoder u_m0_decoder (...);
address_decoder u_m1_decoder (...);
address_decoder u_m2_decoder (...);
address_decoder u_m3_decoder (...);

// Per-slave arbiters
arbiter_aw u_s0_aw_arb (...);
arbiter_ar u_s0_ar_arb (...);
arbiter_aw u_s1_aw_arb (...);
arbiter_ar u_s1_ar_arb (...);
arbiter_aw u_s2_aw_arb (...);
arbiter_ar u_s2_ar_arb (...);

// Width converters (as needed)
width_upsize_64_512 u_m0_s0_upsizer (...);

// Protocol converters (as needed)
axi4_to_apb4 u_s2_apb_conv (...);

// Response routing
response_router u_resp_router (...);

// Monitor system (when variants include "mon")
axi4_master_rd_mon u_m0_rd_mon (...);    // Per-port monitor wrappers
axi4_master_wr_mon u_m0_wr_mon (...);
axi4_slave_rd_mon  u_s0_rd_mon (...);
axi4_slave_wr_mon  u_s0_wr_mon (...);
// ... (repeat for remaining ports)

// Monitor aggregation tree (if multiple monitors)
monbus_arbiter u_mon_arb0 (...);         // Aggregate master-side streams
monbus_arbiter u_mon_arb1 (...);         // Aggregate slave-side streams

// Monitor AXIL group at bridge top
monbus_axil_group u_mon_group (
    .s_mon_axil_*(s_mon_axil_*),  // Slave AXIL for CPU
    .m_axil_mon_*(m_axil_mon_*),  // Master AXIL for DMA
    .stream_irq(stream_irq)        // IRQ output
);

## 19.4	Generated Variants

When variants list in the bridge TOML includes multiple entries, the generator produces one complete .sv file per variant:

variants = ["no", "mon"]

Generated files:
  bridge_4x3.sv              (variants = "no", no monitor)
  bridge_4x3_mon.sv          (variants = "mon", with monitor)

Each variant is a complete, standalone bridge module. Both can coexist in the same design or be selected at compile time.

## 19.5	Signal Naming Convention

### 19.5.1	Master-Side Signals (External)

{prefix}_aw{signal}    - Write address channel
{prefix}_w{signal}     - Write data channel
{prefix}_b{signal}     - Write response channel
{prefix}_ar{signal}    - Read address channel
{prefix}_r{signal}     - Read data channel

Example (prefix = "cpu_m_axi"):
  cpu_m_axi_awvalid
  cpu_m_axi_awready
  cpu_m_axi_awaddr
  cpu_m_axi_wdata
  cpu_m_axi_bvalid

### 19.5.2	Slave-Side Signals (External)

{prefix}_aw{signal}    - Write address channel
{prefix}_w{signal}     - Write data channel
{prefix}_b{signal}     - Write response channel
{prefix}_ar{signal}    - Read address channel
{prefix}_r{signal}     - Read data channel

Example (prefix = "ddr_s_axi"):
  ddr_s_axi_awvalid
  ddr_s_axi_awready
  ddr_s_axi_awaddr
  ddr_s_axi_wdata
  ddr_s_axi_bvalid

### 19.5.3	Internal Signals

// Crossbar internal signals
xbar_m{N}_aw_{signal}  - Master N to crossbar AW
xbar_m{N}_ar_{signal}  - Master N to crossbar AR
xbar_s{N}_aw_{signal}  - Crossbar to slave N AW
xbar_s{N}_ar_{signal}  - Crossbar to slave N AR

// Arbitration signals
grant_aw_s{N}[M-1:0]   - AW grants for slave N
grant_ar_s{N}[M-1:0]   - AR grants for slave N

// ID tracking signals
id_table_s{N}[...]     - ID table for slave N

## 19.6	Generated File Structure

### 19.6.1	Single-File Output

bridge_{name}.sv
├── Module declaration
├── Parameter section
├── Port declarations
├── Internal signal declarations
├── Master adapter instances
├── Address decoder instances
├── Arbiter instances
├── Converter instances
├── Response router instance
└── Debug signals (optional)

### 19.6.2	Multi-File Output (Optional)

bridge_{name}/
├── bridge_{name}.sv        - Top-level wrapper
├── master_adapter_m0.sv    - Master 0 adapter
├── master_adapter_m1.sv    - Master 1 adapter
├── address_decoder.sv      - Shared decoder
├── arbiter_aw.sv           - AW arbiter
├── arbiter_ar.sv           - AR arbiter
├── width_upsize_64_512.sv  - Width converter
├── axi4_to_apb4.sv          - Protocol converter
└── response_router.sv      - Response routing

## 19.7	Parameterization

### 19.7.1	Compile-Time Parameters

// Core parameters
parameter int NUM_MASTERS = 4;
parameter int NUM_SLAVES = 3;
parameter int ADDR_WIDTH = 32;
parameter int DATA_WIDTH = 64;
parameter int ID_WIDTH = 4;

// Derived parameters
localparam int BID_WIDTH = $clog2(NUM_MASTERS);
localparam int TOTAL_ID_WIDTH = ID_WIDTH + BID_WIDTH;
localparam int STRB_WIDTH = DATA_WIDTH / 8;

### 19.7.2	Per-Port Parameters

// Generated per-port widths
localparam int M0_DATA_WIDTH = 64;
localparam int M1_DATA_WIDTH = 256;
localparam int S0_DATA_WIDTH = 512;
localparam int S1_DATA_WIDTH = 32;  // APB

## 19.8	Related Documentation

- Signal Naming - Detailed naming conventions

- Generator Usage - How to run the generator

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	Signal Naming

## 20.1	Naming Convention Overview

Signal names in generated RTL follow one convention, which is what makes pattern-based verification and painless integration possible.

## 20.2	External Port Naming

### 20.2.1	Pattern

{prefix}_{channel}{signal}

### 20.2.2	Components

| Component | Description | Example |
| --- | --- | --- |
| prefix | User-defined port prefix | cpu_m_axi, ddr_s_axi |
| channel | AXI4 channel code | aw, w, b, ar, r |
| signal | Signal within channel | valid, ready, addr, data |

### 20.2.3	Master Port Examples

// Master with prefix "cpu_m_axi"
input  logic        cpu_m_axi_awvalid,
output logic        cpu_m_axi_awready,
input  logic [31:0] cpu_m_axi_awaddr,
input  logic [3:0]  cpu_m_axi_awid,
input  logic [7:0]  cpu_m_axi_awlen,
input  logic [2:0]  cpu_m_axi_awsize,
input  logic [1:0]  cpu_m_axi_awburst,
input  logic        cpu_m_axi_awlock,
input  logic [3:0]  cpu_m_axi_awcache,
input  logic [2:0]  cpu_m_axi_awprot,
input  logic [3:0]  cpu_m_axi_awqos,
input  logic        cpu_m_axi_awuser,

### 20.2.4	Slave Port Examples

// Slave with prefix "ddr_s_axi"
output logic        ddr_s_axi_awvalid,
input  logic        ddr_s_axi_awready,
output logic [31:0] ddr_s_axi_awaddr,
output logic [5:0]  ddr_s_axi_awid,   // Extended ID
output logic [7:0]  ddr_s_axi_awlen,
output logic [2:0]  ddr_s_axi_awsize,
// ...

## 20.3	APB Port Naming

### 20.3.1	Pattern

{prefix}p{signal}

### 20.3.2	APB Signal Examples

// APB slave with prefix "uart_apb_"
output logic        uart_apb_psel,
output logic        uart_apb_penable,
output logic [11:0] uart_apb_paddr,
output logic        uart_apb_pwrite,
output logic [31:0] uart_apb_pwdata,
output logic [3:0]  uart_apb_pstrb,
output logic [2:0]  uart_apb_pprot,
input  logic [31:0] uart_apb_prdata,
input  logic        uart_apb_pslverr,
input  logic        uart_apb_pready,

## 20.4	Internal Signal Naming

### 20.4.1	Crossbar Signals

// Master to crossbar
logic        xbar_m0_awvalid;
logic        xbar_m0_awready;
logic [31:0] xbar_m0_awaddr;

// Crossbar to slave
logic        xbar_s0_awvalid;
logic        xbar_s0_awready;
logic [31:0] xbar_s0_awaddr;

### 20.4.2	Arbitration Signals

// Request/grant matrices
logic [NUM_MASTERS-1:0] aw_request_s0;  // AW requests to slave 0
logic [NUM_MASTERS-1:0] aw_grant_s0;    // AW grants from slave 0

// Arbiter state
logic aw_grant_active_s0;               // Grant locked
logic [$clog2(NUM_MASTERS)-1:0] aw_last_grant_s0;  // Round-robin state

### 20.4.3	ID Signals

// Extended IDs (internal)
logic [TOTAL_ID_WIDTH-1:0] xbar_m0_awid;  // BID + external ID
logic [TOTAL_ID_WIDTH-1:0] xbar_s0_bid;   // Response with BID

// External IDs (to master)
logic [ID_WIDTH-1:0] cpu_m_axi_bid;       // Stripped ID

## 20.5	Debug Signal Naming

### 20.5.1	Pattern

dbg_{category}_{description}

### 20.5.2	Debug Signal Examples

// Arbitration debug
output logic [NUM_MASTERS-1:0] dbg_aw_grant_s0,
output logic [NUM_MASTERS-1:0] dbg_ar_grant_s0,

// Transaction debug
output logic [7:0] dbg_outstanding_m0,
output logic [7:0] dbg_outstanding_m1,

// State machine debug
output logic [1:0] dbg_aw_state_s0,
output logic [1:0] dbg_ar_state_s0,

## 20.6	Verification Pattern Matching

### 20.6.1	Factory Pattern Support

Signal naming enables automatic BFM connection:

# CocoTB/GAXI pattern matching
master = AXI4Master(
    dut=dut,
    prefix="cpu_m_axi_",  # Matches all cpu_m_axi_* signals
    clock=dut.aclk
)

### 20.6.2	Pattern Rules

- Prefix must end with underscore or be directly followed by channel code

- Channel codes are lowercase (aw, w, b, ar, r)

- Signal names match AXI4 specification (valid, ready, addr, data, etc.)

## 20.7	Related Documentation

- Module Structure - Overall RTL organization

- Verification - Pattern-based testing

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	Test Strategy

## 21.1	Overview

Bridge verification is CocoTB-based and parameterized: the same infrastructure covers every bridge topology and protocol combination.

## 21.2	Test Categories

### 21.2.1	Unit Tests

Per-block verification:

Unit Test Coverage:
├── Master Adapter Tests
│   ├── bridge_id sideband tagging (IDs pass through unchanged)
│   ├── Channel routing
│   └── Backpressure handling
├── Address Decoder Tests
│   ├── Address mapping
│   ├── Multi-region support
│   └── Default slave routing
├── Arbiter Tests
│   ├── Round-robin fairness
│   ├── Grant/lock behavior
│   └── Multi-master contention
└── Converter Tests
    ├── Width up/down sizing
    ├── APB protocol conversion
    └── Burst handling

### 21.2.2	Integration Tests

Full bridge verification:

Integration Test Matrix:
├── 2x2 Configuration
│   ├── Basic read/write
│   ├── Concurrent transactions
│   └── Same-ID aliasing across masters (bridge_id keeps them apart)
├── 4x4 Configuration
│   ├── Full arbitration coverage
│   ├── Width conversion paths
│   └── Mixed protocol targets
└── NxM Configurations
    ├── Asymmetric topologies
    ├── Channel-specific masters
    └── Protocol mixing

## 21.3	Test Parameterization

### 21.3.1	Configuration Matrix

Which parameters get swept, and why:

Table 7.1: Test Parameter Matrix

| Parameter | Test Values | Purpose |
| --- | --- | --- |
| NUM_MASTERS | 2, 4, 8 | Scalability |
| NUM_SLAVES | 2, 3, 4 | Routing coverage |
| DATA_WIDTH | 32, 64, 128, 256 | Width paths |
| ID_WIDTH | 4, 6, 8 | ID space |
| OUTSTANDING | 4, 8, 16 | Depth stress |

### 21.3.2	Protocol Combinations

# Test protocol matrix
PROTOCOL_COMBOS = [
    {"masters": ["axi4"], "slaves": ["axi4"]},
    {"masters": ["axi4"], "slaves": ["axi4", "apb"]},
    {"masters": ["axi4", "axil"], "slaves": ["axi4", "apb"]},
]

## 21.4	Protocol-BFM-Only Testing with Memory-Backed Slaves

Modern bridge tests use protocol BFMs only (no direct DUT signal manipulation) with memory-backed slave models:

### 21.4.1	BFM Instantiation and Slave Models

from CocoTBFramework.components.axi4 import AXI4Master, AXI4Slave
from TBClasses.shared.tbbase import TBBase
from TBClasses.shared.memory_model import MemoryModel

class BridgeTB(TBBase):
    def __init__(self, dut):
        super().__init__(dut)

        # Create masters using protocol BFMs
        self.masters = []
        for i in range(NUM_MASTERS):
            prefix = f"m{i}_axi_"
            master = AXI4Master(
                dut=dut,
                prefix=prefix,
                clock=dut.aclk,
                reset=dut.aresetn
            )
            self.masters.append(master)

        # Create slave memory models (NOT direct signal manipulation)
        self.slaves = []
        for i in range(NUM_SLAVES):
            prefix = f"s{i}_axi_"
            slave = MemoryModel(
                dut=dut,
                prefix=prefix,
                clock=dut.aclk,
                reset=dut.aresetn,
                size=0x100000  # 1 MB memory per slave
            )
            self.slaves.append(slave)

### 21.4.2	Transaction Generation and Assertion

async def cocotb_test_concurrent_writes(dut):
    """Test multiple masters writing simultaneously."""
    tb = BridgeTB(dut)
    await tb.setup_clocks_and_reset()

    # Generate transactions via BFMs only
    tasks = []
    for i, master in enumerate(tb.masters):
        addr = 0x1000 + (i * 0x100)
        data = 0xDEAD0000 + i
        # Drive via protocol BFM (no DUT signal poke)
        task = cocotb.start_soon(
            master.write(addr=addr, data=data, size=2)
        )
        tasks.append(task)

    # Wait for all writes to complete
    for task in tasks:
        await task

    # Verify via memory readback (not via routing logic check)
    for i, master in enumerate(tb.masters):
        addr = 0x1000 + (i * 0x100)
        readback = await master.read(addr=addr)
        assert readback == 0xDEAD0000 + i, \
            f"Master {i} readback mismatch at {addr:#x}"

### 21.4.3	Boundary Probe Testing

Address-window boundary probes catch address-decode errors at slave page edges:

async def test_boundary_probes(self):
    """Probe top, middle, bottom of each slave's address window."""
    
    for slave_idx, (base_addr, window_size) in enumerate(self.slave_windows):
        # Payloads carry the slave index and the probe position, so a readback
        # mismatch says BOTH which probe failed and whether the write landed on
        # the wrong slave -- the failure a boundary probe is actually hunting.
        bottom, middle, top = (0xB0000000 | (slave_idx << 8) | pos
                               for pos in (0x0, 0x1, 0x2))

        # Probe bottom (base address)
        await self.masters[0].write(base_addr + 0x000, data=bottom)

        # Probe middle (arbitrary offset)
        await self.masters[0].write(base_addr + window_size // 2, data=middle)

        # Probe top (last valid address)
        await self.masters[0].write(base_addr + window_size - 1, data=top)

        # Verify via readback
        rb_bottom = await self.masters[0].read(base_addr + 0x000)
        rb_middle = await self.masters[0].read(base_addr + window_size // 2)
        rb_top = await self.masters[0].read(base_addr + window_size - 1)

        assert rb_bottom == bottom
        assert rb_middle == middle
        assert rb_top == top

## 21.5	Coverage Goals

### 21.5.1	Functional Coverage

Coverage Targets:
├── Protocol Coverage
│   ├── All burst types (FIXED, INCR, WRAP)
│   ├── All sizes (1, 2, 4, 8, ... bytes)
│   └── All response types
├── Routing Coverage
│   ├── Every master to every slave path
│   ├── Default slave routing
│   └── Error response paths
├── Arbitration Coverage
│   ├── All grant combinations
│   ├── Priority scenarios
│   └── Lock sequences
└── Corner Cases
    ├── Maximum outstanding
    ├── ID exhaustion
    └── Backpressure saturation

### 21.5.2	Code Coverage

Code Coverage Targets:
├── Line Coverage: >95%
├── Branch Coverage: >90%
├── FSM State Coverage: 100%
└── Toggle Coverage: >85%

## 21.6	Test Execution

### 21.6.1	Serial Execution (No Parallel xdist)

Bridge tests execute serially only (pytest-xdist parallelization is not supported). Each cocotb test function is named with a cocotb_test_* prefix to avoid cross-test name collisions when multiple test modules are loaded.

# Run all bridge tests (serial)
cd projects/components/bridge/dv/tests
pytest -v

# Run specific configuration test
pytest test_bridge_4x4_rw_basic.py -v

# Run with coverage
pytest --cov=bridge -v

# Run with waveforms
WAVES=1 pytest test_bridge_2x2_rd_basic.py -v

### 21.6.2	Test Naming Convention

Cocotb test functions follow the pattern: cocotb_test_{N}x{M}_{variant}_{scenario}

# Example: test_bridge_4x4_mon_capture.py
@cocotb.test()
async def cocotb_test_4x4_mon_capture(dut):
    """Monitor packet capture on 4x4 bridge with monitor enabled."""
    ...

# Pytest wrapper function
def test_bridge_4x4_mon_capture(request):
    """Wrapper to invoke cocotb_test_4x4_mon_capture."""
    run(
        python_search=[tests_dir],
        verilog_sources=verilog_sources,
        toplevel="bridge_4x4_mon",
        module=module,
        testcase="cocotb_test_4x4_mon_capture",
        ...
    )

### 21.6.3	Test Levels and Variants

Table 7.2: Test Level and Variant Definitions

| Level | Duration | Coverage | Use Case |
| --- | --- | --- | --- |
| basic | ~30s | Smoke test (single transaction) | Quick verification |
| medium | ~90s | Core paths (concurrent, multi-slave) | Development |
| full | ~180s | Comprehensive (stress, boundary probes) | Pre-commit |
| monitor | ~120s | Monitor system validation | Monitor-specific scenarios |

Monitor Tests (when variants includes "mon"): - test_bridge_1x2_rd_monitor_smoke: Basic packet emission - test_bridge_1x2_rd_monitor_capture: Packet parsing and verification - test_bridge_1x2_rd_monitor_error_inject: SLVERR packet collection - test_bridge_1x2_rd_monitor_irq: IRQ assertion on error FIFO

## 21.7	Related Documentation

- Debug Guide - Debugging failed tests

- Signal Naming - Pattern matching for BFMs

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	Debug Guide

## 22.1	Overview

When a Bridge test fails, start here. This guide lists the failure modes that actually occur and how to chase each one down.

## 22.2	Debug Signal Access

### 22.2.1	There are no dbg_* ports

The generator emits no debug ports. Earlier revisions of this chapter showed dbg_aw_grant_s0, dbg_outstanding_m0, dbg_resp_master and a dbg_id_table_* CAM view; none of them exist in any generated module, and every procedure written against them fails immediately with a cocotb AttributeError.

Debug instead by reaching into the real internal signals by hierarchical name. They are named after the SLAVE (<slave>_...) in the xbar and are plain logic, so cocotb can read them directly:

@cocotb.test()
async def test_with_debug(dut):
    xbar = dut.u_xbar                     # instance name in the generated top
    grant   = xbar.ddr_aw_arb_gnt.value   # which master holds the AW grant
    locked  = xbar.ddr_aw_arb_locked.value
    rr      = xbar.ddr_aw_arb_rr.value    # round-robin pointer
    routed  = xbar.ddr_axi_rid_bridge_id.value  # bridge_id the R beat returns to

Table 7.3: Debug Signals That Exist

| Category | Real signals | Purpose |
| --- | --- | --- |
| Arbitration | <slave>_aw_arb_gnt, _locked, _rr, _req, _pick | Grant, lock and round-robin state, per slave, AW and AR |
| Response routing | <slave>_axi_bid_bridge_id, <slave>_axi_rid_bridge_id | The bridge_id a B/R beat is muxed to. This is the routing decision – the AXI ID is not used |
| Outstanding tracking | aw_trk_wptr / aw_trk_rptr, ar_trk_* (in the master adapter) | Depth of the in-order tracking FIFO |
| Target gating | aw_gate_ok / ar_gate_ok, r_aw_active_target (adapter) | Why a new AW/AR to a different slave is being held off |

There is no outstanding-count output and no FSM state vector – the arbiters are two-state (locked / not) rather than an encoded FSM, so _locked plus _gnt is the whole state.

## 22.3	Common Issues

### 22.3.1	Issue: Transaction Timeout

Symptoms: - Test hangs waiting for response - TimeoutError after configured timeout

Debug Steps:

# 1. Check if request was accepted
await RisingEdge(dut.aclk)
print(f"AWVALID: {dut.m0_axi_awvalid.value}")
print(f"AWREADY: {dut.m0_axi_awready.value}")

# 2. Check arbitration state
print(f"AW grant (ddr): {dut.u_xbar.ddr_aw_arb_gnt.value}")
print(f"AW locked     : {dut.u_xbar.ddr_aw_arb_locked.value}")

# 3. Check the tracking FIFO and the single-outstanding-target gate
print(f"AW trk wptr/rptr: {dut.u_cpu_adapter.aw_trk_wptr.value}"
      f"/{dut.u_cpu_adapter.aw_trk_rptr.value}")
print(f"aw_gate_ok      : {dut.u_cpu_adapter.aw_gate_ok.value}")

# 4. Check slave response
print(f"BVALID: {dut.s0_axi_bvalid.value}")
print(f"BREADY: {dut.s0_axi_bready.value}")

Common Causes: - Arbiter locked by another master (_arb_locked high for someone else) - A different slave is still outstanding. aw_gate_ok low means the adapter is holding AWREADY down because every outstanding write must target one slave at a time. This is by design, not a fault – see the single-outstanding-target note in the PRD. - Slave not responding - Tracking FIFO full (aw_trk_wptr has lapped aw_trk_rptr)

### 22.3.2	Issue: Wrong Response Routing

Symptoms: - Response arrives at wrong master - BID mismatch errors

Debug Steps:

# The AXI ID passes through UNCHANGED -- there is no ID extension.
print(f"Master AWID : {dut.cpu_m_axi_awid.value}")
print(f"Slave  AWID : {dut.ddr_axi_awid.value}")     # same value

# Routing is by the bridge_id sideband, not the ID:
print(f"B routed to bridge_id: {dut.u_xbar.ddr_axi_bid_bridge_id.value}")
print(f"R routed to bridge_id: {dut.u_xbar.ddr_axi_rid_bridge_id.value}")

Common Causes: - The slave returned responses out of order. Routing pops an in-order FIFO, so a reordering slave sends one master’s beats to another. Nothing detects it. This is the single most likely cause and it is a design constraint, not a bug in the bridge – see FR-2 in the PRD. - ID width mismatch between master and slave ports - Two masters using the same AXI ID to one slave: their IDs alias at the slave, and only the bridge_id sideband keeps the responses apart

### 22.3.3	Issue: Data Corruption

Symptoms: - Read data doesn’t match written data - WSTRB/data alignment issues

Debug Steps:

# Check width conversion
print(f"Master data width: {dut.M0_DATA_WIDTH.value}")
print(f"Slave data width: {dut.S0_DATA_WIDTH.value}")

# Check strobe packing
print(f"Master WSTRB: {dut.m0_axi_wstrb.value}")
print(f"Slave WSTRB: {dut.s0_axi_wstrb.value}")

# Check address alignment
print(f"AWADDR: {dut.m0_axi_awaddr.value}")
print(f"AWSIZE: {dut.m0_axi_awsize.value}")

Common Causes: - Width converter byte lane error - Unaligned access handling - Burst boundary crossing

## 22.4	Waveform Analysis

### 22.4.1	Generating Waveforms

# Enable VCD dump
WAVES=1 pytest test_bridge_2x2.py::test_basic -v

# View with GTKWave
gtkwave sim_build/dump.vcd

### 22.4.2	Key Signals to Observe

Waveform Signal Groups:
├── Clock/Reset
│   └── aclk, aresetn
├── Master 0 AW Channel
│   └── m0_axi_awvalid, m0_axi_awready, m0_axi_awaddr, m0_axi_awid
├── Slave 0 AW Channel
│   └── s0_axi_awvalid, s0_axi_awready, s0_axi_awaddr, s0_axi_awid
├── Arbitration (inside u_xbar)
│   └── ddr_aw_arb_gnt, ddr_aw_arb_locked, ddr_ar_arb_gnt, ddr_ar_arb_locked
├── Target gating (inside the master adapter)
│   └── aw_gate_ok, ar_gate_ok, r_aw_active_target
└── Response
    └── s0_axi_bvalid, m0_axi_bvalid, ddr_axi_bid_bridge_id

Timing Analysis

## 22.5	Assertion Failures

### 22.5.1	There are no built-in assertions

The generated RTL contains no assert statements of any kind – not in the top, the xbar or any adapter. An earlier revision of this section showed two protocol assertions as though they shipped; they never existed, and the signals they referenced (s_awvalid, r_current_awid) match no identifier in the generated design.

Protocol checking is the testbench’s job here. Use the AXI4 BFMs and the monitor wrappers rather than expecting the DUT to flag a violation itself.

### 22.5.2	Handling Assertion Failures

- Locate assertion in RTL - Search for error message

- Check signal history - Review 10-20 cycles before failure

- Identify root cause - Usually protocol or timing issue

- Fix test or RTL - Depending on where bug exists

## 22.6	Performance Debugging

### 22.6.1	Throughput Issues

# Measure transaction throughput
start_time = cocotb.utils.get_sim_time('ns')

for i in range(100):
    await master.write(addr=0x1000 + i*4, data=[i])

end_time = cocotb.utils.get_sim_time('ns')
throughput = 100 / (end_time - start_time) * 1e9  # tx/sec
print(f"Throughput: {throughput:.2f} transactions/sec")

### 22.6.2	Latency Issues

# Measure single transaction latency
start = cocotb.utils.get_sim_time('ns')
await master.read(addr=0x1000)
latency = cocotb.utils.get_sim_time('ns') - start
print(f"Read latency: {latency} ns")

## 22.7	Related Documentation

- Test Strategy - Overall test approach

- Arbiter FSMs - FSM state details

- ID Tracking - ID table operation

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	Bridge Hardware Architecture Specification Index

Version: 1.1 Date: 2026-06-04 Purpose: High-level hardware architecture specification for Bridge component

## 23.1	Document Organization

Note: Every chapter below is one source file; the document build assembles the spec from these links.

### 23.1.1	Front Matter

- Document Information

### 23.1.2	Chapter 1: Introduction

- Purpose and Scope

- Document Conventions

- Definitions and Acronyms

### 23.1.3	Chapter 2: System Overview

- Use Cases

- Key Features

- System Context

### 23.1.4	Chapter 3: Architecture

- Block Diagram

- Data Flow

- Protocol Support

### 23.1.5	Chapter 4: Interfaces

- AXI4 Interface Overview

- APB Interface Overview

- Clock and Reset

- AXI5 and APB5 Interfaces (AMBA5)

### 23.1.6	Chapter 5: Performance

- Throughput Characteristics

- Latency Analysis

- Resource Estimates

### 23.1.7	Chapter 6: Integration

- System Requirements

- Parameter Configuration

- Verification Strategy

## 23.2	Related Documentation

- Bridge MAS - Micro-Architecture Specification (detailed block-level)

- PRD.md - Product requirements and overview

- CLAUDE.md - AI development guide

Last Updated: 2026-01-03 Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	Bridge: AXI4 Full Crossbar Generator - Product Requirements Document

Project: Bridge Version: 2.1 Status: Phase 2 Complete - Simplified Architecture with Hard Limits Created: 2025-10-18 Last Updated: 2025-11-02

## 24.1	Executive Summary

Bridge is a Python-based AXI4 crossbar generator that produces simple, performant SystemVerilog RTL for connecting multiple AXI4 masters to multiple AXI4 slaves. The name follows the infrastructure theme - bridges connect different regions, enabling communication across divides, just like crossbars connect masters and slaves.

Design Philosophy: A simple AMBA fabric that is performant, but makes no attempt to support all features. We enforce hard limits (8-bit ID width, 64-bit address width) to eliminate unnecessary complexity, focusing only on what real hardware needs.

Key Differentiator from Delta: - Delta: AXI-Stream crossbar (streaming data, single channel, simple routing) - Bridge: AXI4 full crossbar (memory-mapped, 5 channels, burst support, bridge_id sideband routing)

Key Differentiator from Commercial IP: - Commercial AXI4 Crossbars: Feature-complete, support every AXI4 edge case, complex configurability - Bridge: Simple and performant, supports common use cases, hard limits for simplicity

Target Use Case: High-performance memory-mapped interconnects for multi-core processors, accelerators, and memory controllers where simplicity and performance matter more than spec compliance.

## 24.2	Design Philosophy

Vision: A simple AMBA fabric that is performant, but makes no attempt to support all features.

### 24.2.1	Core Principles

1. Simplicity Over Completeness - Focus on common use cases, not edge cases - Implement what’s needed for real hardware, not AXI4 spec compliance theater - Prefer straightforward implementations over complex feature sets

2. Performance First - Direct connections where possible (matching widths = zero overhead) - Minimal conversion logic in critical paths - Optimize for throughput and latency, not feature count

3. Hard Limits for Simplicity

We enforce uniform widths on certain parameters to eliminate complexity:

| Parameter | Hard Limit | Rationale |
| --- | --- | --- |
| ID Width | Per port, set at generation | NOT fixed at 8. The TOML sets id_width per port and the generated bridge bakes it in – bridge_2x2_rw has 4-bit IDs. This row previously said “8 bits (fixed)”, which the generator has never enforced. |
| Address Width | Per port, set at generation | NOT fixed at 64. bridge_2x2_rw is a 32-bit-address bridge. Same correction as the row above. |
| Data Width | Variable (32b-512b) | Width converters ONLY for data. Commonly needed for bandwidth optimization. |

Why This Works: - ID width conversion adds complexity with minimal benefit (nobody needs >256 outstanding transactions per master) - Address width conversion is rarely needed (peripheral addresses fit in 32-bit, but using 64-bit everywhere is simpler) - Data width conversion is the ONLY width conversion that provides real value (bandwidth matching)

4. What We DON’T Support (By Design)

Features intentionally excluded for simplicity: - (Both of these WERE excluded once and no longer are: ID width and address width are per-port generation inputs. Left here as history because the “design differentiator” argument above still cites them.) - No AXI4-Lite protocol variant (use standard AXI4 with len=0) - No ACE protocol extensions (cache coherency) - No AXI5 features - No complete AXI4 sideband signal support (QoS, Region, User signals declared but not routed)

5. What We DO Support

Core AXI4 features that matter: - Full 5-channel AXI4 protocol (AW, W, B, AR, R) - Burst transactions (INCR, WRAP, FIXED) - In-order completion. Responses are routed by a per-master bridge_id sideband and an in-order FIFO, NOT by matching AXI IDs – see FR-2. - Multiple outstanding transactions per master, to one slave at a time. The generated master adapters carry a single-outstanding-target gate (aw_gate_ok / ar_gate_ok): a new AW/AR to a DIFFERENT slave is held off until the outstanding ones drain. - Channel-specific masters (write-only, read-only, read-write) - Data width conversion (32b ↔ 64b ↔ 128b ↔ 256b ↔ 512b) - Configurable M×N topology (1-32 masters, 1-256 slaves) - Fair round-robin arbitration per slave

### 24.2.2	Architecture Philosophy

Target: Intelligent width-aware routing, not fixed-width crossbar

OLD Approach (What We’re Moving Away From):

Master (64b) → Upsize(64→256) → Fixed 256b Crossbar → Downsize(256→64) → Slave (64b)

- Two conversions for same-width connections (wasteful!)

- Artificial bandwidth bottleneck

- Unnecessary logic and latency

NEW Approach (Target Architecture):

Master (64b) → Direct Connection → Slave (64b)    [0 conversions!]
Master (512b) → Conv(512→64) → Slave (64b)         [1 conversion]

- Per-master paths to each unique slave width it connects to

- Direct paths where widths match (zero overhead)

- Converters only where actually needed

- Router selects appropriate path based on address decode

Result: Minimal conversion logic, maximum performance, pragmatic simplicity.

## 24.3	CRITICAL: RTL Regeneration Requirements

ALL generated RTL files MUST be deleted and regenerated together whenever ANY generator code changes.

Why This Matters: - Generated RTL files may have interdependencies (bridges, wrappers, integrators) - Generator code changes can create version mismatches between files - Partial regeneration creates subtle incompatibilities that cause test failures - Even “small innocuous” generator changes can have cascading effects

The Rule:

# WRONG - Partial regeneration
./bridge_generator.py --masters 5 --slaves 3 --output ../rtl/
# Only regenerates bridge_axi4_flat_5x3.sv
# Other files (wrappers, integrators) now mismatched!

# CORRECT - Full regeneration
cd bin
make clean                               # Delete ALL generated bridges (rtl/generated/)
make all                                 # Regenerate everything from bridge_batch.csv

Generator Files That Trigger Full Regeneration: - bin/bridge_generator.py - Main bridge generator (CSV/TOML driven) - Any Python file in bin/bridge_pkg/ (components/, generators/, config, csv_parser, …) - Any Jinja template in bin/bridge_pkg/jinja_templates/ - bridge_wrapper_generator.py - Wrapper generation - Any Python file in projects/components/bridge/bin/

Symptoms of Version Mismatch: - Tests that previously passed now fail - Simulation errors about missing signals - Mismatched port widths or counts - Address decode routing to wrong slaves

Think of Generated RTL Like Compiled Code: When you update a compiler, you don’t selectively recompile - you rebuild everything. When you update a generator, you don’t selectively regenerate - you regenerate everything.

## 24.4	1. Product Overview

### 24.4.1	1.1 Purpose

Bridge automates generation of AXI4 crossbar infrastructure: - Python code generation - Parameterized RTL generation (similar to APB/Delta) - Performance modeling - Analytical + simulation validation - Flat topology - Full M×N interconnect matrix - bridge_id sideband routing - in-order response return (see FR-2) - Burst optimization - Pipelined burst transfers

### 24.4.2	1.2 Target Audience

Primary Users: - RTL designers building SoC interconnect - System architects evaluating interconnect topologies - Verification engineers needing crossbar testbenches - Students learning AXI4 protocol and interconnects

Educational Focus: - Demonstrates AXI4 protocol complexity - Shows arbitration strategies for memory-mapped busses - Teaches ID-based transaction tracking - Illustrates burst optimization techniques

### 24.4.3	1.3 Success Criteria

Functional: - [x] Generates working AXI4 crossbar RTL (bridge_generator.py) - [x] Generates CSV/TOML-configured bridges (bridge_generator.py + bridge_pkg/; the earlier separate bridge_csv_generator.py was merged in) - [x] Passes Verilator lint - [x] Supports 1-32 masters, 1-256 slaves - [ ] Handles out-of-order completion via IDs. NOT IMPLEMENTED: bridge_cam.sv exists but is instantiated by no generated module. The slave adapters run “Bridge ID Tracking - FIFO Mode (In-Order)” and 93 generated files still declare cam_wr_allocate/cam_rd_allocate without driving them. - [x] Supports burst lengths 1-256 beats - [x] Channel-specific masters (wr/rd/rw) for resource optimization (Phase 2) - [ ] APB converter integration (Phase 3 pending)

Performance: - [ ] Latency ≤ 3 cycles for single-beat transactions. Unverified for the generated bridge. The generated path crosses four skid buffers on the round trip (adapter timing wrappers on both sides), and gaxi_skid_buffer registers its output side, so an ideal zero-wait slave still costs >=4 cycles of bridge-added latency before arbitration. The 2-3 cycle figure in 6.1 describes the wrapperless flat crossbar, not what the generator emits. - [x] Throughput: All M×N paths can transfer concurrently - [x] Performance models implemented (bridge_model.py - V1 Flat) - [ ] Fmax ≥ 300 MHz on UltraScale+ FPGAs (pending synthesis validation)

Quality: - [x] Complete specifications (PRD) before code - [x] Performance models validate requirements (bridge_model.py) - [x] All generated RTL Verilator verified - [x] Integration examples provided (CSV examples) - [x] Comprehensive documentation (GENERATOR_ARCHITECTURE.md, docs/bridge_has/, docs/bridge_mas/)

### 24.4.4	1.4 Implementation Status and Phases

Unified Generator (bridge_generator.py):

The bridge generator now supports both TOML/CSV configuration and legacy array-indexed modes:

Configuration Modes: 1. TOML/CSV Mode (Preferred) - TOML port configuration (bridge_name.toml) - CSV connectivity matrix (bridge_name_connectivity.csv) - Custom port prefixes (rapids_m_axi_, cpu_m_axi_, etc.) - Interface wrapper integration (timing isolation) - Mixed protocols (AXI4 + APB + AXI4-Lite slaves) - Channel-specific masters (wr/rd/rw) - Status: Phase 2 complete, Phase 3 pending

- Legacy CSV Mode (Backwards Compatible)

  - Separate ports.csv and connectivity.csv files

  - Migration path to TOML format

  - See bin/test_configs/README.md for conversion guide

Phase Status:

Phase 1: CSV Configuration COMPLETE - CSV parser (ports.csv, connectivity.csv) - Port generation with custom prefixes - Converter identification logic - Basic crossbar instantiation

Phase 2: Channel-Specific Masters COMPLETE (2025-10-26) - Added channels field to PortSpec (rw/wr/rd) - Conditional port generation based on channels - Resource Optimization: - wr (write-only): AW, W, B channels only → 39% port reduction - rd (read-only): AR, R channels only → 61% port reduction - rw (full): All 5 channels - Width converter awareness (only generate needed converters) - Example: 4-master bridge saves 35% signals with optimized channels

Phase 3: APB Converter Integration PENDING - AXI2APB converter module (create or integrate existing) - APB signal packing/unpacking - APB converter instantiation in generated bridges - Width converter + APB converter chaining - End-to-end testing with APB slaves - Status: Placeholders in generated code with detailed TODO comments

Additional Resources: - models/bridge_model/bridge_model.py - Performance modeling (V1 Flat implemented) - rtl/bridge_cam.sv - CAM for ID tracking. Not instantiated by any generated module; kept for the OOO work FR-2 describes as not implemented. - See GENERATOR_ARCHITECTURE.md for the generator/build architecture - See docs/bridge_has/ and docs/bridge_mas/ for architecture diagrams and specs (the older BRIDGE_CURRENT_STATE.md / BRIDGE_ARCHITECTURE_DIAGRAMS.md were superseded)

## 24.5	2. Architecture Overview

### 24.5.1	2.1 AXI4 vs AXIS vs APB

| Feature | APB | AXI-Stream (Delta) | AXI4 (Bridge) |
| --- | --- | --- | --- |
| Protocol Type | Simple register bus | Streaming data | Memory-mapped burst |
| Channels | 1 (address + data) | 1 (data stream) | 5 (AW, W, B, AR, R) |
| Request Generation | Address range decode | TDEST decode | Address range decode |
| Arbitration | Per-slave, per-cycle | Per-slave, packet-locked | Per-slave, per-address-phase |
| Out-of-Order | No (sequential) | No (streaming order) | No (in-order, bridge_id routed) |
| Burst Support | No | Packet (via TLAST) | Yes (AWLEN/ARLEN) |
| Complexity | Low | Medium | High |
| Latency | 1-2 cycles | 2 cycles | 2-3 cycles |
| Use Case | Control registers | Data streaming | Memory-mapped I/O |

Bridge Complexity Sources: 1. 5 independent channels requiring separate arbitration 2. bridge_id sideband routing, in-order (the AXI ID is not used for routing) 3. Burst handling with interleaving constraints 4. Write response tracking (match AW with B channel) 5. Address decode + ID muxing for response routing

### 24.5.2	2.2 Block Diagram

┌──────────────────────────────────────────────────────────────────────┐
│                          BRIDGE AXI4 CROSSBAR                        │
└──────────────────────────────────────────────────────────────────────┘

Masters (M)                                                    Slaves (S)
┌─────────┐                                                   ┌─────────┐
│ Master 0│ ─AW──┐                                      ┌─AW──│ Slave 0 │
│  (CPU)  │ ─W───┤                                      ├─W───│ (DDR)   │
│         │ ─B───┤                                      ├─B───│         │
│         │ ─AR──┤                                      ├─AR──│         │
│         │ ─R───┤                                      ├─R───│         │
└─────────┘      │                                      │     └─────────┘
                 │                                      │
┌─────────┐      │   ┌──────────────────────────┐      │     ┌─────────┐
│ Master 1│ ─────┼───│  Request Generation      │──────┼─────│ Slave 1 │
│  (GPU)  │      │   │  (Address Range Decode)  │      │     │ (SRAM)  │
└─────────┘      │   └──────────────────────────┘      │     └─────────┘
                 │             │                        │
┌─────────┐      │   ┌──────────────────────────┐      │     ┌─────────┐
│ Master 2│ ─────┼───│  Arbitration Matrix      │──────┼─────│ Slave 2 │
│  (DMA)  │      │   │  (AW + AR arbiters       │      │     │ (PCIE)  │
└─────────┘      │   │   per slave: AW,W,B,AR,R)│      │     └─────────┘
                 │   └──────────────────────────┘      │
┌─────────┐      │             │                        │     ┌─────────┐
│ Master 3│ ─────┴───│  Data/Addr Multiplexing  │──────┴─────│ Slave 3 │
│ (Accel) │          │  (bridge_id routing)     │            │ (Periph)│
└─────────┘          └──────────────────────────┘            └─────────┘
                               │
                     ┌──────────────────────────┐
                     │  Transaction Tracking    │
                     │  (ID tables for OoO)     │
                     └──────────────────────────┘

### 24.5.3	2.3 Key Components

1. Request Generation - Address range decode for each slave - Generates M×S request matrix per channel (AW, AR) - Similar to APB but more complex (2 address channels)

2. Per-Slave Arbitration - 2 arbiters per slave, not five: - AW channel arbiter (write address) - AR channel arbiter (read address) - W has no arbiter – a W-owner FIFO gives the channel to whichever master’s AW the slave accepted first. - B and R have no arbiter – they are muxed combinationally on the bridge_id carried alongside the response. - Round-robin with burst locking - Separate read/write paths (no head-of-line blocking)

3. Data Multiplexing - Mux master signals to selected slave - ID-based response routing (B, R channels) - Burst tracking (hold grant until xlast)

4. Transaction Tracking - ID tables per slave for out-of-order support - Track {Master ID, Transaction ID} → Master mapping - Required for routing B/R channels back to correct master

5. Optional Performance Counters - Transaction counts per master/slave - Arbitration conflict counts - Latency histograms

## 24.6	3. Functional Requirements

### 24.6.1	3.1 AXI4 Protocol Compliance

FR-1: Full AXI4 Protocol Support - Support all 5 AXI4 channels: AW, W, B, AR, R - Comply with AMBA AXI4 specification (ARM IHI 0022) - Support burst lengths: 1-256 beats (AWLEN/ARLEN = 0-255) - Support burst types: INCR, WRAP, FIXED - Support burst sizes: 1-128 bytes (AWSIZE/ARSIZE = 0-7)

FR-2: Response Routing (in-order) - Responses are routed by the bridge_id sideband, a static per-master tag, popped from an in-order FIFO. The AXI ID plays no part in routing. - Master IDs pass through unchanged – there is no ID extension. cpu_adapter drives cpu_32b_aw.id = fub_axi_awid, 4 bits in, 4 bits out. - Constraint this imposes: the slave must return responses in the order its requests were accepted. A reordering slave misroutes – the FIFO head names the wrong master, so one master receives another’s beats and the second starves. Nothing detects this. - Two masters using the same AXI ID to the same slave alias to one ID at that slave; the bridge_id sideband, not the ID, keeps their responses apart.

FR-3: Atomic Operations (the bridge does NOT implement a monitor – see below) - Pass AWLOCK/ARLOCK through to the slave. The bridge implements no exclusive monitor: there is no per-slave exclusive tracking and no EXOKAY generation anywhere in the generated RTL – awlock is muxed to the granted slave and nothing more. Exclusives work only if the attached slave implements the monitor itself. 13.1 correctly lists the monitor as future work; this requirement previously read as though the bridge provided it. - Track exclusive monitor per slave - Generate BRESP/RRESP errors for failed exclusives

### 24.6.2	3.2 Address Decoding

FR-4: Configurable Address Map - Support M masters × S slaves - Each slave has base address and size - Non-overlapping address ranges (verified at generation) - Default slave for unmapped addresses (optional)

FR-5: Address Range Configuration

# Example address map
address_map = {
    0: {'base': 0x00000000, 'size': 0x40000000, 'name': 'DDR'},      # 1GB
    1: {'base': 0x40000000, 'size': 0x00100000, 'name': 'SRAM'},     # 1MB
    2: {'base': 0x50000000, 'size': 0x01000000, 'name': 'PCIE'},     # 16MB
    3: {'base': 0x60000000, 'size': 0x00010000, 'name': 'Peripherals'} # 64KB
}

### 24.6.3	3.3 Arbitration Strategy

FR-6: Round-Robin Arbitration - Fair bandwidth allocation (no starvation) - Separate arbiters for AW and AR channels - Grant is locked only until the ADDRESS handshake (awvalid && awready), not until xlast. The RTL comment reads “lock until handshake”. Back-to-back AWs from different masters can therefore be accepted and their W bursts sequenced by the W-owner FIFO. - Configurable arbitration policy (round-robin default)

FR-7: Read/Write Independence - Separate read and write paths - No head-of-line blocking between read/write - Concurrent read and write to same slave (if slave supports)

### 24.6.4	3.4 Burst Handling

FR-8: Burst Optimization - Pipelined burst transfers (overlap address and data) - No artificial burst splitting - Full AXI4 burst protocol support

FR-9: Interleaving Constraints - W channel locked to AW grant master - R channel routed by transaction ID - Support ID-based interleaving (slave-dependent)

## 24.7	4. Non-Functional Requirements

### 24.7.1	4.1 Performance

NFR-1: Latency - Single-beat read: ≤ 3 cycles (address decode + arbitration + mux) - Single-beat write: ≤ 3 cycles (address decode + arbitration + mux) - Burst transfer: No additional latency per beat (pipelined)

NFR-2: Throughput - Concurrent transfers: All M×S paths can transfer simultaneously - Burst efficiency: Line-rate data transfer after address phase - No artificial stalls: Crossbar adds no wait states beyond arbitration

NFR-3: Fmax - Target: 300-400 MHz on Xilinx UltraScale+ - Registered outputs: All slave outputs registered for timing closure - Pipelineable: Optional pipeline stages for >400 MHz

### 24.7.2	4.2 Resource Usage (Estimated)

M = 4 masters, S = 4 slaves, DATA_WIDTH = 512, ADDR_WIDTH = 64, ID_WIDTH = 4:

| Resource | Flat Crossbar | Notes |
| --- | --- | --- |
| LUTs | ~2,500 | Address decode + arbiters + mux |
| FFs | ~3,000 | Registered outputs + ID tables |
| BRAM | 0 | No ID tables exist. Tracking is a small in-order FIFO per direction in each slave adapter. |
| DSP | 0 | No arithmetic operations |

Scaling: ~150 LUTs per M×S connection

### 24.7.3	4.3 Quality Requirements

NFR-4: Code Generation Quality - Lint-clean SystemVerilog (Verilator) - Synthesizable (Vivado, Yosys, Design Compiler) - No vendor-specific primitives (technology-agnostic) - Clear structure (commented, readable)

NFR-5: Verification - CocoTB testbench framework - Transaction-level verification - Out-of-order test scenarios - Burst interleaving tests - >95% functional coverage

## 24.8	5. Interface Specifications

### 24.8.1	5.1 AXI4 Master Interfaces (M × 5 channels)

Write Address Channel (AW):

input  logic [ADDR_WIDTH-1:0]   s_axi_awaddr  [NUM_MASTERS];
input  logic [ID_WIDTH-1:0]     s_axi_awid    [NUM_MASTERS];
input  logic [7:0]              s_axi_awlen   [NUM_MASTERS];  // Burst length-1
input  logic [2:0]              s_axi_awsize  [NUM_MASTERS];  // Burst size
input  logic [1:0]              s_axi_awburst [NUM_MASTERS];  // INCR/WRAP/FIXED
input  logic                    s_axi_awlock  [NUM_MASTERS];  // Exclusive access
input  logic [3:0]              s_axi_awcache [NUM_MASTERS];  // Cache attributes
input  logic [2:0]              s_axi_awprot  [NUM_MASTERS];  // Protection type
input  logic                    s_axi_awvalid [NUM_MASTERS];
output logic                    s_axi_awready [NUM_MASTERS];

Write Data Channel (W):

input  logic [DATA_WIDTH-1:0]   s_axi_wdata  [NUM_MASTERS];
input  logic [DATA_WIDTH/8-1:0] s_axi_wstrb  [NUM_MASTERS];  // Byte strobes
input  logic                    s_axi_wlast  [NUM_MASTERS];  // Last beat
input  logic                    s_axi_wvalid [NUM_MASTERS];
output logic                    s_axi_wready [NUM_MASTERS];

Write Response Channel (B):

output logic [ID_WIDTH-1:0]     s_axi_bid    [NUM_MASTERS];
output logic [1:0]              s_axi_bresp  [NUM_MASTERS];  // OKAY/EXOKAY/SLVERR/DECERR
output logic                    s_axi_bvalid [NUM_MASTERS];
input  logic                    s_axi_bready [NUM_MASTERS];

Read Address Channel (AR):

input  logic [ADDR_WIDTH-1:0]   s_axi_araddr  [NUM_MASTERS];
input  logic [ID_WIDTH-1:0]     s_axi_arid    [NUM_MASTERS];
input  logic [7:0]              s_axi_arlen   [NUM_MASTERS];
input  logic [2:0]              s_axi_arsize  [NUM_MASTERS];
input  logic [1:0]              s_axi_arburst [NUM_MASTERS];
input  logic                    s_axi_arlock  [NUM_MASTERS];
input  logic [3:0]              s_axi_arcache [NUM_MASTERS];
input  logic [2:0]              s_axi_arprot  [NUM_MASTERS];
input  logic                    s_axi_arvalid [NUM_MASTERS];
output logic                    s_axi_arready [NUM_MASTERS];

Read Data Channel (R):

output logic [DATA_WIDTH-1:0]   s_axi_rdata  [NUM_MASTERS];
output logic [ID_WIDTH-1:0]     s_axi_rid    [NUM_MASTERS];
output logic [1:0]              s_axi_rresp  [NUM_MASTERS];
output logic                    s_axi_rlast  [NUM_MASTERS];
output logic                    s_axi_rvalid [NUM_MASTERS];
input  logic                    s_axi_rready [NUM_MASTERS];

### 24.8.2	5.2 AXI4 Slave Interfaces (S × 5 channels)

Mirror of master interfaces, with M → S direction reversed.

### 24.8.3	5.3 Configuration Is Generation-Time, Not Parameters

The generated bridge exposes no parameters. Everything below is fixed when the generator runs and reaches the module through its package:

module bridge_2x2_rw
    import bridge_2x2_rw_pkg::*;
(
    input logic aclk, ...

Geometry and widths come from the TOML, per port, and appear inside the generated modules as localparams (localparam ADDR_WIDTH = 32;, localparam ID_WIDTH = 4; in cpu_adapter). To change any of them, regenerate – there is nothing to override at elaboration.

An earlier revision of this section listed a parameter block including MAX_BURST_LEN, PIPELINE_OUTPUTS and ENABLE_COUNTERS. None of those three exist in any generated module, and neither do the performance counters that ENABLE_COUNTERS implied – there is no counter logic anywhere in the generated set.

## 24.9	6. Performance Modeling

### 24.9.1	6.1 Analytical Model

Latency Components (Flat Crossbar):

Single-Beat Read Latency:
  1. Address Decode:      0 cycles (combinatorial)
  2. Arbitration:         1 cycle  (AR arbiter decision)
  3. Address Mux:         0 cycles (combinatorial)
  4. Slave Access:        (slave-dependent, not crossbar)
  5. Response Mux:        1 cycle  (R channel ID lookup + mux)
  6. Output Register:     1 cycle  (optional, for timing closure)

  Total (no pipeline):    2 cycles
  Total (pipelined):      3 cycles

Burst Transfer Throughput:

After address phase completes, data transfer is line-rate:
  - W channel: 1 beat/cycle (locked to AW grant)
  - R channel: 1 beat/cycle (ID-routed from slave)

Example: 256-beat burst
  Address phase:  2-3 cycles (one-time)
  Data phase:     256 cycles (line-rate)
  Total:          258-259 cycles
  Efficiency:     98.8%

Concurrent Throughput:

Maximum concurrent transfers (4×4 crossbar):
  - Read:  4 concurrent (one per master-slave pair)
  - Write: 4 concurrent (one per master-slave pair)
  - Total: 8 concurrent read+write (separate paths)

Throughput @ 100 MHz, 512-bit data:
  Per path:   100 MHz × 512 bits = 51.2 Gbps
  Total:      8 paths × 51.2 Gbps = 409.6 Gbps theoretical

### 24.9.2	6.2 Resource Scaling

LUT Usage Formula (empirical):

LUTs ≈ 500 (base) + 150 × M × S + 20 × ID_TABLE_DEPTH

Example (4×4 crossbar, ID_WIDTH=4, ID_TABLE_DEPTH=16 per slave):

LUTs ≈ 500 + 150 × 16 + 20 × 16 × 4
     ≈ 500 + 2,400 + 1,280
     ≈ 4,180 LUTs

### 24.9.3	6.3 Comparison with Other Crossbars

| Crossbar Type | Latency | Throughput | Complexity | Use Case |
| --- | --- | --- | --- | --- |
| APB | 1-2 cycles | Low (serialized) | Low | Control registers |
| AXI-Stream (Delta) | 2 cycles | High (streaming) | Medium | Data streaming |
| AXI4 (Bridge) | 2-3 cycles | High (burst) | High | Memory-mapped I/O |
| AXI4 + Slices | 4-6 cycles | High (burst) | Very High | >400 MHz designs |

Bridge Sweet Spot: High-performance memory-mapped interconnects where out-of-order and burst efficiency are critical.

## 24.10	7. Generator Architecture

### 24.10.1	7.1 Python Generator Structure

class BridgeGenerator:
    """AXI4 crossbar RTL generator"""

    def __init__(self, config):
        self.num_masters = config.num_masters
        self.num_slaves = config.num_slaves
        self.data_width = config.data_width
        self.addr_width = config.addr_width
        self.id_width = config.id_width
        self.address_map = config.address_map

    def generate_address_decode(self) -> str:
        """Generate address range decode logic"""
        # For each master × slave, check if address in range
        # More complex than AXIS (uses address), simpler than full decode

    def generate_aw_arbiter(self, slave_idx) -> str:
        """Generate write address channel arbiter for one slave"""
        # Round-robin arbiter
        # Grants locked until corresponding B response completes

    def generate_ar_arbiter(self, slave_idx) -> str:
        """Generate read address channel arbiter for one slave"""
        # Round-robin arbiter
        # Grants locked until corresponding R response completes (RLAST)

    def generate_w_channel_mux(self, slave_idx) -> str:
        """Generate write data channel multiplexer"""
        # W channel follows AW grant (locked until WLAST)

    def generate_b_channel_demux(self, slave_idx) -> str:
        """Generate write response channel demultiplexer"""
        # Route by ID: lookup master from {slave_idx, BID}

    def generate_r_channel_demux(self, slave_idx) -> str:
        """Generate read data channel demultiplexer"""
        # Route by ID: lookup master from {slave_idx, RID}

    def generate_id_table(self, slave_idx) -> str:
        """Generate transaction ID tracking table"""
        # Maps {slave, transaction_id} → master_id
        # Indexed on AW/AR grant, looked up on B/R response

    def generate_crossbar(self) -> str:
        """Generate complete crossbar module"""
        # Instantiate all arbiters, muxes, demuxes, ID tables

### 24.10.2	7.2 Address Map Configuration

Python Configuration:

bridge_config = {
    'num_masters': 4,
    'num_slaves': 4,
    'data_width': 512,
    'addr_width': 64,
    'id_width': 4,
    'address_map': {
        0: {'base': 0x00000000, 'size': 0x40000000, 'name': 'DDR'},
        1: {'base': 0x40000000, 'size': 0x00100000, 'name': 'SRAM'},
        2: {'base': 0x50000000, 'size': 0x01000000, 'name': 'PCIE'},
        3: {'base': 0x60000000, 'size': 0x00010000, 'name': 'Peripherals'}
    },
    'pipeline_outputs': True,
    'enable_counters': True
}

Generated Address Decode:

// Address decode for each master
always_comb begin
    for (int s = 0; s < NUM_SLAVES; s++)
        aw_request_matrix[s] = '0;

    for (int m = 0; m < NUM_MASTERS; m++) begin
        if (s_axi_awvalid[m]) begin
            // Slave 0: DDR (0x00000000 - 0x3FFFFFFF)
            if (s_axi_awaddr[m] >= 64'h00000000 &&
                s_axi_awaddr[m] < 64'h40000000)
                aw_request_matrix[0][m] = 1'b1;

            // Slave 1: SRAM (0x40000000 - 0x400FFFFF)
            if (s_axi_awaddr[m] >= 64'h40000000 &&
                s_axi_awaddr[m] < 64'h40100000)
                aw_request_matrix[1][m] = 1'b1;

            // ... slaves 2-3 ...
        end
    end
end

## 24.11	8. Comparison with APB and Delta

### 24.11.1	8.1 Code Reuse from APB Generator

Similar Components (~70% reuse): - Address range decode logic (same pattern, different signals) - Round-robin arbiter (same algorithm) - Data multiplexing pattern (same approach) - Backpressure handling (similar to PREADY)

New Components for Bridge: - 2 arbiters per slave (AW and AR). W is owned via a FIFO, B and R are muxed combinationally – no arbiter on any of the three. - bridge_id sideband routing (B and R channel demuxing) - In-order transaction tracking (a FIFO per direction, not an ID table) - Burst handling (grant locked to the ADDRESS handshake, not xlast)

Migration Effort from APB: - ~120 minutes (vs ~75 min for AXIS, due to higher complexity) - Most time: ID table logic and response demuxing

### 24.11.2	8.2 Code Reuse from Delta Generator

Similar Components (~60% reuse): - Python generation framework - Command-line interface - Performance modeling structure - Arbitration pattern (round-robin with locking)

Key Differences: - 5 channels vs 1 channel (Delta only has TDATA/TVALID/TREADY/TLAST) - bridge_id sideband routing vs TDEST-based routing - Address decode vs TDEST decode (Bridge more complex) - Transaction tracking vs packet atomicity (different mechanisms)

### 24.11.3	8.3 Complexity Comparison

| Metric | APB Crossbar | Delta (AXIS) | Bridge (AXI4) |
| --- | --- | --- | --- |
| Channels to arbitrate | 1 | 1 | 5 ★ |
| Request generation | Address ranges | TDEST decode | Address ranges |
| Response routing | Grant-based | Grant-based | ID-based ★ |
| Burst support | No | Packet (TLAST) | Yes (AWLEN/ARLEN) ★ |
| Out-of-order | No | No | Yes (ID tables) ★ |
| Transaction tracking | No | No | Yes ★ |
| Lines of Python | ~500 | ~697 | ~900 (est.) |
| Lines of generated SV | ~200 (4×4) | ~250 (4×4) | ~400 (4×4) (est.) |

★ = Additional complexity in Bridge

## 24.12	9. Use Cases

### 24.12.1	9.1 Multi-Core Processor Interconnect

Scenario: 4 CPU cores + GPU accessing DDR + SRAM + Peripherals

Configuration:
  Masters:  5 (4 CPUs, 1 GPU)
  Slaves:   3 (DDR, SRAM, Peripherals)
  Data:     512-bit (cache line width)
  Address:  64-bit (large memory space)
  ID:       4-bit (up to 16 outstanding per master)

Benefits: - Concurrent access to all slaves - Out-of-order completion for high-performance CPUs - Burst transfers for cache line fills - Separate read/write paths (no head-of-line blocking)

### 24.12.2	9.2 DMA + Accelerator System

Scenario: DMA engine + 4 accelerators accessing shared memory

Configuration:
  Masters:  5 (1 DMA, 4 accelerators)
  Slaves:   2 (DDR, Control registers)
  Data:     512-bit (high-bandwidth DMA)
  Address:  32-bit (limited address space)
  ID:       2-bit (simple ID space)

Benefits: - High-bandwidth memory access for DMA - Fair arbitration prevents accelerator starvation - Control register access doesn’t block data transfers

### 24.12.3	9.3 FPGA System Integration

Scenario: MicroBlaze CPU + custom accelerators + memory controllers

Configuration:
  Masters:  8 (1 CPU, 7 accelerators)
  Slaves:   4 (DDR, BRAM, AXI GPIO, AXI DMA)
  Data:     128-bit (AXI4 standard width)
  Address:  32-bit (standard FPGA address space)
  ID:       4-bit (moderate outstanding transactions)

Benefits: - Standard AXI4 interfaces (Xilinx IP compatibility) - Scalable to many masters/slaves - Performance counters for profiling. NOT IMPLEMENTED – no counter logic exists in any generated module.

## 24.13	10. Testing Strategy

### 24.13.1	10.1 FUB (Functional Unit Block) Tests

Address Decode Tests: - Verify all address ranges correctly decoded - Test boundary conditions (base, base+size-1) - Test unmapped addresses (error response)

Arbiter Tests: - Round-robin fairness (all masters get turns) - Grant locking to the ADDRESS handshake (not xlast – see FR-6) - Starvation prevention

ID Table Tests: - Correct ID → master mapping - Out-of-order transaction handling - Table full condition

Mux/Demux Tests: - Data integrity through crossbar - Response routing to correct master - Concurrent transfers don’t interfere

### 24.13.2	10.2 Integration Tests

Single-Master, Single-Slave: - Basic read/write transactions - Burst transfers (various lengths) - Out-of-order completions

Multi-Master, Single-Slave: - Arbitration correctness - Fairness verification - Burst interleaving (if supported)

Multi-Master, Multi-Slave: - Concurrent transfers - No crosstalk between paths - Full M×S matrix coverage

Stress Tests: - All masters active simultaneously - Maximum burst lengths - Full ID space utilization - Back-to-back bursts

### 24.13.3	10.3 Performance Validation

Latency Measurement: - Single-beat read: measure actual vs analytical - Single-beat write: measure actual vs analytical - Compare with specification (≤3 cycles)

Throughput Measurement: - Burst transfer efficiency (should be ~98%) - Concurrent path throughput (should be line-rate × M×S) - Compare with theoretical maximum

Resource Validation: - Synthesize for Xilinx UltraScale+ - Compare LUT/FF usage with estimates - Verify Fmax ≥ 300 MHz

## 24.14	11. Documentation Plan

### 24.14.1	11.1 Specifications (Before Code)

- ☒ PRD.md - This document (complete requirements)

- ☐ README.md - User guide with quick start

- ☐ BRIDGE_VS_APB_GENERATOR.md - Migration guide from APB

- ☐ BRIDGE_VS_DELTA_GENERATOR.md - Comparison with Delta

- ☐ AXI4_PROTOCOL_GUIDE.md - AXI4 primer for students

### 24.14.2	11.2 Performance Analysis (Before Implementation)

- ☒ models/bridge_model/bridge_model.py - Analytical model (V1 Flat implemented)

  - Latency calculations (single-beat, burst)

  - Throughput estimates (concurrent paths)

  - Resource estimates (LUT/FF scaling)

- ☐ bridge simulator - Discrete event simulation (optional, not implemented)

  - Cycle-accurate modeling

  - Traffic pattern support

  - Validation against RTL

### 24.14.3	11.3 Code Generation

- ☐ bin/bridge_generator.py - Main RTL generator

  - Command-line interface

  - Address map configuration

  - Generated RTL output

### 24.14.4	11.4 Verification

- ☐ dv/tests/ - CocoTB testbenches

  - FUB tests (individual blocks)

  - Integration tests (multi-block)

  - Stress tests (corner cases)

## 24.15	12. Success Metrics

### 24.15.1	12.1 Functional Completeness

- ☐ Generates working AXI4 crossbar RTL

- ☐ Passes all CocoTB tests

- ☐ Verilator lint clean

- ☐ Xilinx Vivado synthesis clean

### 24.15.2	12.2 Performance Targets

- ☐ Latency ≤ 3 cycles (measured in simulation)

- ☐ Throughput = line-rate × M×S paths (measured)

- ☐ Fmax ≥ 300 MHz (post-synthesis)

### 24.15.3	12.3 Educational Value

- ☐ Complete specifications demonstrating rigor

- ☐ Performance models validate requirements

- ☐ Clear code structure (readable generated RTL)

- ☐ Integration examples provided

### 24.15.4	12.4 Reusability

- ☐ ~70% code reuse from APB generator (measured by LOC)

- ☐ Similar patterns to Delta generator

- ☐ Can be extended (weighted arbitration, QoS, etc.)

## 24.16	12. Attribution and Contribution Guidelines

### 24.16.1	12.1 Git Commit Attribution

When creating git commits for Bridge documentation or implementation:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: Bridge documentation and organization receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

## 24.17	12.2 PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

projects/components/bridge/docs/

Quick Commands: Use the provided shell scripts:

cd projects/components/bridge/docs
./generate_has_pdf.sh   # Bridge_HAS_v<rev>.docx/.pdf
./generate_mas_pdf.sh   # Bridge_MAS_v<rev>.docx/.pdf

The shell scripts will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the bridge_has/bridge_mas index files 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

See: bin/md_to_docx.py for complete implementation details

## 24.18	13. Future Enhancements

### 24.18.1	13.1 Short-Term (Post-Initial Release)

- ☐ Optional pipeline stages - For Fmax >400 MHz

- ☐ Weighted arbitration - QoS support

- ☐ Default slave - Unmapped address handling

- ☐ Exclusive monitor - Full atomic operation support

### 24.18.2	13.2 Long-Term

- ☐ Tree topology - Hierarchical crossbar (like Delta)

- ☐ AXI4-Lite variant - Simplified for control registers

- ☐ ACE support - Coherent cache interconnect

- ☐ GUI configurator - Visual address map setup

## 24.19	14. Risk Assessment

| Risk | Probability | Impact | Mitigation |
| --- | --- | --- | --- |
| ID table complexity | Medium | High | Start with small ID_WIDTH (2-4), test thoroughly |
| Out-of-order corner cases | High | High | Extensive CocoTB tests with random delays |
| Fmax below target | Low | Medium | Optional pipeline stages for timing closure |
| Resource usage exceeds | Low | Low | Empirical formulas guide expectations |
| Burst interleaving bugs | Medium | High | Separate test suite for burst scenarios |

## 24.20	15. Project Timeline (Estimated)

Week 1: Specifications and Models - [x] Day 1-2: PRD.md (complete) - [ ] Day 3-4: Performance modeling (analytical) - [ ] Day 5-7: README.md, migration guides

Week 2-3: Core Implementation - [ ] Day 1-3: Address decode + arbiter generation - [ ] Day 4-5: Data mux/demux generation - [ ] Day 6-8: ID table generation - [ ] Day 9-10: Integration and testing

Week 4: Verification and Examples - [ ] Day 1-5: CocoTB testbenches - [ ] Day 6-7: Integration examples

## 24.21	16. References

AXI4 Specifications: - ARM IHI 0022 - AMBA AXI and ACE Protocol Specification - Xilinx UG1037 - Vivado AXI Reference Guide

Related Projects: - APB Crossbar - Simple register bus crossbar (existing) - Delta (AXIS Crossbar) - Streaming data crossbar (projects/components/delta/) - RAPIDS - DMA engine with AXI4 masters (projects/components/dmas/rapids/)

Tools: - Verilator - RTL linting and simulation - CocoTB - Python-based verification - Xilinx Vivado - FPGA synthesis

## 24.22	17. Glossary

- AXI4: Advanced eXtensible Interface version 4 (AMBA standard)

- Burst: Multi-beat transaction (AWLEN/ARLEN > 0)

- Crossbar: Full M×N interconnect matrix

- ID: Transaction identifier for out-of-order support

- Out-of-order: Responses can return in different order than requests

- xlast: WLAST (write) or RLAST (read) - last beat indicator

Version: 1.0 Status: Specification Complete - Ready for Implementation Next Steps: Create performance models, then implement generator

Project Bridge - Connecting masters and slaves across the divide

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	Claude Code Guide: Bridge Subsystem

Version: 2.1 Last Updated: 2025-11-03 Purpose: AI-specific guidance for working with Bridge subsystem

## 25.1	CRITICAL: Read Architecture Documents First

Before making ANY changes to bridge generator or understanding signal flow:

READ: projects/components/bridge/GENERATOR_ARCHITECTURE.md (generator structure) and projects/components/bridge/docs/bridge_mas/ (micro-architecture spec; rendered as docs/Bridge_MAS_v1.1.pdf)

These documents contain the definitive bridge architecture including: - Correct signal flow (wrappers → decoder → converters → crossbar → slaves) - Component purposes and placement - Common misconceptions that previous agents made - Why there’s NO fixed crossbar width

If you skip these documents, you WILL make incorrect assumptions.

## 25.2	Quick Context

What: Bridge - one AXI4/AXI5 crossbar generator, bin/bridge_generator.py, taking TOML or CSV configuration. There is no second generator: the separate bridge_csv_generator.py was merged into it and no longer exists. Status: Phase 2 Complete - CSV generator with channel-specific masters (wr/rd/rw) Your Role: Help users configure CSV files, generate bridges, understand architecture, and create tests

Complete Documentation (Read in This Order): 1. projects/components/bridge/GENERATOR_ARCHITECTURE.md ← START HERE (generator architecture reference) 2. projects/components/bridge/PRD.md ← Product requirements 3. projects/components/bridge/TASKS.md ← Task history and current status 4. projects/components/bridge/docs/bridge_has/bridge_has_index.md ← Hardware Architecture Spec (rendered: docs/Bridge_HAS_v1.1.pdf) 5. projects/components/bridge/docs/bridge_mas/bridge_mas_index.md ← Micro-Architecture Spec (rendered: docs/Bridge_MAS_v1.1.pdf)

(The older BRIDGE_ARCHITECTURE.md, BRIDGE_CURRENT_STATE.md, BRIDGE_ARCHITECTURE_DIAGRAMS.md, CSV_BRIDGE_STATUS.md, and docs/bridge_spec/ documents were superseded by the HAS/MAS above.)

## 25.3	Target Architecture: Intelligent Width-Aware Routing

Core Principle: Direct connections where possible, converters only where needed, no fixed crossbar width.

### 25.3.1	Efficient Multi-Width Design

Master_A (64b)
  ├─ Direct → Slave_0 (64b)           [0 conversions, minimal latency]
  ├─ Conv(64→128) → Slave_1 (128b)    [1 conversion]
  └─ Conv(64→512) → Slave_2 (512b)    [1 conversion]

Master_B (512b)
  ├─ Conv(512→64) → Slave_0 (64b)     [1 conversion]
  ├─ Conv(512→128) → Slave_1 (128b)   [1 conversion]
  └─ Direct → Slave_2 (512b)          [0 conversions, full bandwidth]

Router Logic: Address decoder determines target slave, selects correctly-sized path for that master-slave pair.

### 25.3.2	Why This Architecture

Naive Fixed-Width Approach (Don’t Do This):

Master (64b) → Upsize(64→256) → Fixed 256b Crossbar → Downsize(256→64) → Slave (64b)

- TWO conversions for same-width connections (wasteful!)

- Reduced bandwidth on narrow paths

- Unnecessary logic and area

- Higher latency

Intelligent Routing (Target):

Master (64b) → Router → Direct Connection → Slave (64b)

- ZERO conversions for matching widths

- Full native bandwidth

- Minimal logic

- Lowest latency

### 25.3.3	Per-Master Output Paths

Each master has N output paths (one per unique slave width it connects to):

// Master_A connects to slaves at 64b, 128b, 512b
// Generate 3 output paths:

logic [63:0]  master_a_64b_wdata;   // For 64b slaves (direct)
logic [127:0] master_a_128b_wdata;  // For 128b slaves (via converter)
logic [511:0] master_a_512b_wdata;  // For 512b slaves (via converter)

// Router selects based on address decode:
always_comb begin
    case (decoded_slave_id)
        SLAVE_0: select master_a_64b_wdata;   // Slave_0 is 64b
        SLAVE_1: select master_a_128b_wdata;  // Slave_1 is 128b
        SLAVE_2: select master_a_512b_wdata;  // Slave_2 is 512b
    endcase
end

### 25.3.4	Benefits

- Resource Efficient - Only instantiate converters actually needed

- Maximum Performance - Direct paths have zero conversion overhead

- Optimal Bandwidth - No artificial width bottlenecks

- Lower Latency - Minimal logic in critical path for matching widths

- Scalable - Works for any combination of master/slave widths

### 25.3.5	Implementation Status

Current State: Fixed-width crossbar with master-side upsizing (Phase 1 architecture) Target State: Intelligent per-master multi-width routing (your vision) Migration: Requires generator architecture rework (see TASKS.md)

## 25.4	CRITICAL RULE #0: RTL Regeneration Requirements

READ THIS FIRST - FAILURE TO FOLLOW CAUSES TEST FAILURES

### 25.4.1	The Golden Rule

ALL generated RTL files MUST be deleted and regenerated together whenever ANY generator code changes.

### 25.4.2	Why This Is Non-Negotiable

Generated RTL files have interdependencies: - Each rtl/generated/<bridge_name>/ directory holds a matched set (top module, crossbar, adapters, package) - Generator changes affect signal names, port widths, interface structure - Partial regeneration creates version mismatches that cause silent failures

### 25.4.3	Real Example (Historical Session)

# WHAT WENT WRONG:
1. Updated the address decode logic in the generator
2. Regenerated ONLY one bridge configuration
3. Did NOT regenerate the wrapper that instantiated it
4. Result: All tests that were passing now FAIL
5. Cause: Version mismatch between wrapper and core bridge

# WHAT SHOULD HAVE BEEN DONE:
1. Update the generator
2. Delete ALL generated bridge directories (rtl/generated/bridge_*/)
3. Regenerate ALL bridges from scratch (make all in bin/)
4. Run ALL tests to verify

### 25.4.4	Generator Files That Trigger Full Regeneration

Any change to these files requires regenerating ALL bridges:

- bin/bridge_generator.py - Core bridge generator (CSV/TOML driven)

- ANY Python file in bin/bridge_pkg/ (components/, generators/, config, csv_parser, signal_naming, …)

- Any Jinja template in bin/bridge_pkg/jinja_templates/

### 25.4.5	The Regeneration Workflow

# Step 1: Make generator code changes
vim bin/bridge_pkg/generators/crossbar_generator.py

# Step 2: Delete ALL generated RTL (be aggressive!)
cd projects/components/bridge/bin
make clean   # Removes rtl/generated/ output

# Step 3: Regenerate everything from bridge_batch.csv
make all     # Runs bridge_generator.py --bulk bridge_batch.csv

# Step 4: Run ALL tests
cd ../dv/tests
pytest -v  # ALL tests, not just the one you think changed

# Step 5: Verify git diff makes sense
git diff ../rtl/  # Review all changes

### 25.4.6	Symptoms of Version Mismatch

If you see these symptoms, you probably did partial regeneration:

- Tests that previously passed now fail

- “Signal not found” errors in simulation

- Port width mismatches in instantiation

- Address decode routing to wrong slaves

- Missing debug signals (dbg_*)

- Unexpected compile errors in working code

### 25.4.7	Think Like a Compiler Developer

Generated RTL = Compiled Object Files

When you update a compiler, you don’t selectively recompile - you make clean && make all.

When you update a generator, you don’t selectively regenerate - you delete all and regenerate all.

### 25.4.8	Exception: Hand-Written RTL

These files are never regenerated: - bridge_cam.sv - CAM module (hand-written) - Any file in rtl/ that is NOT generated

Check file headers - generated files say “Generated by: bridge_generator.py”

## 25.5	Critical Pitfalls: slave_select_* Reversion After Handshake

### 25.5.1	The Problem: Combinational Address Decode

The master adapter’s R/B response MUX and the crossbar’s AR/AW gating both relied on <master>_slave_select_{ar,aw} — a combinational decode of fub_axi_{ar,aw}addr. This signal reverts immediately when the address port is freed:

- AR/AW handshake completes (arvalid && arready, awvalid && awready)

- Wrapper pops skid buffer and releases the address port

- fub_axi_araddr / fub_axi_awaddr reverts to next transaction or zero

- Decode flips to different slave

- Response MUX is now gated on WRONG slave

- Response path closes, transaction hangs

This happens BEFORE the converter even finishes pushing to the crossbar, and ALWAYS before response arrives.

### 25.5.2	Visible Symptoms

- Fails: Multi-slave multi-master bridges hang (timeout)

- Fails: Multi-width masters fail basic connectivity

- Fails: APB/AXIL slaves consistently timeout (longer converter latency = more decode changes)

- Works: Single-slave or single-master systems (no decode change)

### 25.5.3	The Solution (MANDATORY in All Generators)

For Crossbar AR/AW Gating: - Replace <master>_slave_select_ar/aw gates with inline address re-decode on stable m_axi buses - Address on m_axi is held stable across the entire AXI handshake

For Master Adapter Response MUX: - Add per-channel response-tracking FIFOs that capture slave index at request time - Use FIFO head for response MUX instead of live combinational decode

Crossbar Code Pattern:

// WRONG (old implementation):
assign gate_ar_s0 = m0_slave_select_ar && m0_arvalid;
//                  ^^^^^^^^^^^^^^^^^ Reverts after handshake!

// CORRECT (new implementation):
assign gate_ar_s0 = ((m0_m_axi_araddr >= S0_BASE) && 
                      (m0_m_axi_araddr < S0_END) && 
                      m0_arvalid);  // Re-decode on stable m_axi address

Master Adapter Code Pattern:

// WRONG (old implementation):
assign r_slave_select = comb_slave_select_ar;  // Stale after pop!

// CORRECT (new implementation):
// Capture at request time
always_ff @(posedge aclk) begin
    if (fub_axi_arvalid && fub_axi_arready) begin
        ar_trk_fifo <= comb_slave_select_ar;
    end
end

// Use stable value for response MUX
assign r_slave_select = ar_trk_fifo_out;

### 25.5.4	When This Bug Appears

- Works: Single-master, single-slave systems (no address re-evaluation)

- Fails: Multi-master to same slave (multiple addresses decode differently)

- Fails: Multi-slave same master (address decode changes per transaction)

- Fails: Multi-width paths (W beats re-evaluate AW decode mid-burst)

## 25.6	MANDATORY: Project Organization Pattern

THIS SUBSYSTEM MUST FOLLOW THE RAPIDS/AMBA ORGANIZATIONAL PATTERN - NO EXCEPTIONS

### 25.6.1	Required Directory Structure

projects/components/bridge/
├── bin/                           # Bridge-specific tools (generators, scripts)
│   ├── bridge_generator.py       # AXI4 crossbar generator (CSV/TOML driven)
│   ├── bridge_pkg/                # Generator implementation package
│   ├── test_configs/              # TOML port configs + CSV connectivity
│   └── Makefile                   # make all / make single / make clean
├── docs/                          # Design documentation (bridge_has/, bridge_mas/, PDFs)
├── dv/                            # Design Verification (MANDATORY structure)
│   ├── tbclasses/                 # Testbench classes (MANDATORY - TB classes here!)
│   │   ├── __init__.py
│   │   └── bridge2x2_rw_tb.py     # Per-config generated TB classes
│   ├── components/                # Bridge-specific BFMs (if needed)
│   ├── scoreboards/               # Bridge-specific scoreboards (if needed)
│   ├── testplans/                 # Test plans
│   └── tests/                     # All test files (test runners only, flat)
│       ├── conftest.py            # Pytest configuration (MANDATORY)
│       └── test_bridge_*.py       # Per-config tests (test_bridge_2x2_rw.py, ...)
├── rtl/                           # RTL source files
│   ├── bridge_cam.sv              # Hand-written CAM
│   ├── filelists/                 # Generated filelists
│   └── generated/                 # Generated bridges (one subdir per config)
├── CLAUDE.md                      # This file
├── PRD.md                         # Product requirements
└── TASKS.md                       # Task history and status

### 25.6.2	Testbench Class Location (MANDATORY)

WRONG: Testbench class in test file

# projects/components/bridge/dv/tests/test_bridge_2x2_rw.py
class Bridge2x2RwTB:  # WRONG LOCATION!
    """Embedded TB - NOT REUSABLE"""

CORRECT: Testbench class in PROJECT AREA dv/tbclasses/

# projects/components/bridge/dv/tbclasses/bridge2x2_rw_tb.py
class Bridge2x2RwTB(TBBase):  # CORRECT LOCATION!
    """Reusable TB class - imported by the matching test runner"""

CRITICAL: TB classes are PROJECT-SPECIFIC and MUST be in the project area (projects/components/{name}/dv/tbclasses/), NOT in the framework (bin/TBClasses/).

### 25.6.3	Test File Pattern (MANDATORY)

Test files MUST follow this structure:

# projects/components/bridge/dv/tests/test_bridge_2x2_rw.py

import os
import pytest
import cocotb
from cocotb_test.simulator import run

# IMPORT testbench class from PROJECT AREA
import sys
from TBClasses.shared.utilities import get_repo_root
sys.path.insert(0, get_repo_root())
from projects.components.bridge.dv.tbclasses.bridge2x2_rw_tb import Bridge2x2RwTB
from TBClasses.shared.utilities import get_paths, get_wave_config
from TBClasses.shared.filelist_utils import get_sources_from_filelist

# ===========================================================================
# COCOTB TEST FUNCTIONS - Prefix with "cocotb_" to prevent pytest collection
# ===========================================================================

@cocotb.test(timeout_time=100, timeout_unit='us')
async def cocotb_test_basic_routing(dut):
    """Test basic address routing"""
    tb = Bridge2x2RwTB(dut)  # Imported TB
    await tb.setup_clocks_and_reset()
    result = await tb.test_basic_routing()
    assert result, "Basic routing test failed"

# More cocotb test functions...

# ===========================================================================
# PYTEST WRAPPER FUNCTIONS - At bottom of file
# ===========================================================================

@pytest.mark.bridge
def test_bridge_2x2_rw(request):
    """Pytest wrapper for the generated 2x2 rw bridge"""
    module, repo_root, tests_dir, log_dir, rtl_dict = get_paths({
        'rtl_bridge': '../../rtl'
    })

    # ... verilog_sources from the bridge's filelist, parameters, etc ...

    run(
        verilog_sources=verilog_sources,
        toplevel="bridge_2x2_rw",
        module=module,
        testcase="cocotb_test_basic_routing",  # ← cocotb function name
        parameters=rtl_parameters,
        sim_build=sim_build,
        # ...
    )

See dv/tests/test_bridge_2x2_rw.py for the real, complete pattern (tests and TB classes are generated per bridge configuration alongside the RTL).

## 25.7	Critical Rules for This Subsystem

### 25.7.1	Rule #0: Attribution Format for Git Commits

IMPORTANT: When creating git commit messages for Bridge documentation or code:

Use:

Documentation and implementation support by Claude.

Do NOT use:

Co-Authored-By: Claude <noreply@anthropic.com>

Rationale: Bridge receives AI assistance for structure and clarity, while design concepts and architectural decisions remain human-authored.

### 25.7.2	Rule #0.1: Testbench Architecture - MANDATORY SEPARATION

THIS IS A HARD REQUIREMENT - NO EXCEPTIONS

NEVER embed testbench classes inside test runner files!

The same testbench logic will be reused across multiple test scenarios. Having testbench code only in test files makes it COMPLETELY WORTHLESS for reuse.

MANDATORY Structure:

projects/components/bridge/
├── dv/
│   ├── tbclasses/                   # TB classes HERE (not in framework!)
│   │   ├── __init__.py
│   │   ├── bridge2x2_rw_tb.py       ← REUSABLE TB CLASS (per config)
│   │   └── bridge5x3_channels_tb.py
│   ├── components/                  # Bridge-specific BFMs (if needed)
│   │   └── __init__.py
│   ├── scoreboards/                 # Bridge-specific scoreboards
│   │   └── __init__.py
│   └── tests/                       # Test runners (flat, one per config)
│       ├── conftest.py              ← MANDATORY pytest config
│       ├── test_bridge_2x2_rw.py    ← TEST RUNNER ONLY (imports TB)
│       └── test_bridge_5x3_channels.py

Why This Matters:

- Reusability: Same TB class used in:

  - Per-config functional tests

  - Monitor stress tests (test_bridge_*_mon_monitor.py)

  - User projects (external imports)

- Maintainability: Fix bug once in TB class, all tests benefit

- Composition: TB classes can inherit/compose for complex scenarios

### 25.7.3	Rule #1: All Testbenches Inherit from TBBase

Every testbench class MUST inherit from TBBase:

from TBClasses.shared.tbbase import TBBase

class BridgeAXI4FlatTB(TBBase):
    """Testbench for Bridge crossbar - inherits base functionality"""

    def __init__(self, dut, num_masters=2, num_slaves=2, **kwargs):
        super().__init__(dut)
        # Bridge-specific initialization

TBBase Provides: - Clock management (start_clock, wait_clocks) - Reset utilities (assert_reset, deassert_reset) - Logging (self.log) - Progress tracking (mark_progress) - Safety monitoring (timeouts, memory limits)

### 25.7.4	Rule #2: Mandatory Testbench Methods

Every testbench class MUST implement these three methods:

async def setup_clocks_and_reset(self):
    """Complete initialization - starts clocks and performs reset"""
    await self.start_clock('aclk', freq=10, units='ns')

    # Set config signals before reset (if needed)
    # self.dut.cfg_param.value = initial_value

    # Reset sequence
    await self.assert_reset()
    await self.wait_clocks('aclk', 10)
    await self.deassert_reset()
    await self.wait_clocks('aclk', 5)

async def assert_reset(self):
    """Assert reset signal (active-low for AXI4)"""
    self.dut.aresetn.value = 0

async def deassert_reset(self):
    """Deassert reset signal"""
    self.dut.aresetn.value = 1

Why Required: - Consistency across all testbenches - Reusability for mid-test resets - Clear test structure and intent

### 25.7.5	Rule #3: Use GAXI Components for Protocol Handling

For Bridge testing, use GAXI Master/Slave components for AXI4 channel handling:

from CocoTBFramework.components.gaxi.gaxi_master import GAXIMaster
from CocoTBFramework.components.gaxi.gaxi_slave import GAXISlave
from CocoTBFramework.components.axi4.axi4_field_configs import AXI4FieldConfigHelper

# CORRECT: Use GAXI for AXI4 channels
self.aw_master = GAXIMaster(
    dut=dut,
    title="AW_M0",
    prefix="s0_axi4_",
    clock=clock,
    field_config=AXI4FieldConfigHelper.create_aw_field_config(id_width, addr_width, 1),
    pkt_prefix="aw",
    multi_sig=True,
    log=log
)

Never manually drive AXI4 valid/ready handshakes - Use GAXI components.

### 25.7.6	Rule #4: Queue-Based Verification

For simple in-order verification, use direct monitor queue access:

# CORRECT: Direct queue access
aw_pkt = self.aw_slave._recvQ.popleft()
w_pkt = self.w_slave._recvQ.popleft()

# Verify
assert aw_pkt.addr == expected_addr
assert w_pkt.data == expected_data

# WRONG: Memory model for simple test
memory_model = MemoryModel()  # Unnecessary complexity

When to Use Memory Models: - Simple in-order tests → Use queue access - Single-master systems → Use queue access - Complex out-of-order scenarios → Memory model may help - Multi-master with address overlap → Memory model tracks state

## 25.8	TOML/CSV-Based Bridge Generator (Phase 2 Complete)

### 25.8.1	Overview

The Bridge generator creates parameterized SystemVerilog crossbars from TOML configuration files with CSV connectivity matrices, eliminating manual RTL editing for complex interconnects.

Key Benefits: - Human-readable configuration - TOML for ports, CSV for connectivity - Custom signal prefixes - Each port has unique prefix (rapids_m_axi_, apb0_, etc.) - Channel-specific masters - Write-only (wr), read-only (rd), or full (rw) - Interface modules - Timing isolation via axi4_master/slave wrappers with configurable skid depths - Automatic converters - Width and protocol conversion inserted automatically - Resource efficient - Only generates needed channels and converters

### 25.8.2	Quick Start

1. Create bridge_mybridge.toml:

[bridge]
  name = "bridge_mybridge"
  description = "Custom bridge example"

  # Default skid buffer depths (can be overridden per port)
  defaults.skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

  masters = [
    {name = "cpu", prefix = "cpu_m_axi", id_width = 4, addr_width = 32, data_width = 64, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "dma", prefix = "dma_m_axi", id_width = 4, addr_width = 32, data_width = 512, user_width = 1,
     interface = {type = "axi4_master", skid_depths = {ar = 4, r = 8, aw = 4, w = 8, b = 4}}}
  ]

  slaves = [
    {name = "ddr", prefix = "ddr_s_axi", id_width = 4, addr_width = 32, data_width = 512, user_width = 1,
     base_addr = 0x00000000, addr_range = 0x80000000, interface = {type = "axi4_slave"}},
    {name = "sram", prefix = "sram_s_axi", id_width = 4, addr_width = 32, data_width = 256, user_width = 1,
     base_addr = 0x80000000, addr_range = 0x80000000, interface = {type = "axi4_slave"}}
  ]

2. Create bridge_mybridge_connectivity.csv:

master\slave,ddr,sram
cpu,1,1
dma,1,1

3. Generate bridge:

cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_mybridge.toml
# Auto-finds bridge_mybridge_connectivity.csv

# Or use bulk generation
python3 bridge_generator.py --bulk bridge_batch.csv

Result: Complete SystemVerilog module with: - Custom port prefixes per port - Timing isolation via interface wrappers - Only needed AXI4 channels (wr/rd/rw optimized) - Width converters for data mismatches - Internal crossbar instantiation - APB/AXI4-Lite converter integration points

### 25.8.3	Configuration Format Details

TOML Port Configuration:

The primary format is now TOML (preferred over CSV for better structure and interface configuration):

Port Specifications: - name - Unique identifier (cpu, dma, ddr, etc.) - prefix - Signal prefix (cpu_m_axi_, ddr_s_axi_, etc.) - id_width - AXI4 ID width in bits - addr_width - Address width in bits - data_width - Data width in bits (32, 64, 128, 256, 512) - user_width - AXI4 user signal width - base_addr - Slave base address (slaves only) - addr_range - Slave address range (slaves only) - interface - Interface wrapper configuration (optional) - type - “axi4_master”, “axi4_slave”, “axi4_master_mon”, “axi4_slave_mon”, or omit for direct connection - skid_depths - Per-channel buffer depths: {ar, r, aw, w, b} (valid: 2, 4, 6, 8)

CSV Connectivity Matrix:

master\slave,slave0,slave1,slave2
master0,1,0,1
master1,0,1,1

- 1 = connected, 0 = not connected

- Partial connectivity supported (not all masters to all slaves)

- Auto-detected based on TOML filename: bridge_name.toml → bridge_name_connectivity.csv

Legacy CSV Format:

For backwards compatibility, the generator still supports CSV port files: - See bin/test_configs/README.md for migration guide from CSV to TOML - TOML is now preferred for new bridges (better structure, interface config support)

### 25.8.4	Channel-Specific Masters (Phase 2 Feature)

Why Channel-Specific? Real hardware often has dedicated read or write masters. Generating all 5 AXI4 channels wastes resources:

Traditional (wasteful):

// Write-only master gets unused read channels
input  logic [63:0]  rapids_descr_m_axi_awaddr,  // USED
input  logic [511:0] rapids_descr_m_axi_wdata,   // USED
output logic [7:0]   rapids_descr_m_axi_bid,     // USED
input  logic [63:0]  rapids_descr_m_axi_araddr,  // UNUSED (50% waste!)
output logic [511:0] rapids_descr_m_axi_rdata,   // UNUSED

Channel-Specific (optimized):

rapids_descr_wr,master,axi4,wr,rapids_descr_m_axi_,512,64,8,N/A,N/A

Generated:

// Write-only master - only AW, W, B channels
input  logic [63:0]  rapids_descr_m_axi_awaddr,
input  logic [511:0] rapids_descr_m_axi_wdata,
output logic [7:0]   rapids_descr_m_axi_bid,
// NO READ CHANNELS (araddr, rdata, etc.)

Resource Savings: - 40-60% fewer ports for dedicated masters - Only necessary width converters instantiated - Channel-aware direct connection wiring - Faster synthesis, smaller netlists

### 25.8.5	Example: RAPIDS-Style Configuration

RAPIDS Architecture: - Descriptor write master (wr) - Writes descriptors to memory - Sink write master (wr) - Writes incoming packets to memory - Source read master (rd) - Reads outgoing packets from memory - CPU master (rw) - Full access for configuration

TOML Configuration (bridge_rapids.toml):

[bridge]
  name = "bridge_rapids"
  description = "RAPIDS accelerator bridge"
  defaults.skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}

  masters = [
    {name = "rapids_descr_wr", prefix = "rapids_descr_m_axi", channels = "wr",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "rapids_sink_wr", prefix = "rapids_sink_m_axi", channels = "wr",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "rapids_src_rd", prefix = "rapids_src_m_axi", channels = "rd",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     interface = {type = "axi4_master"}},
    {name = "cpu", prefix = "cpu_m_axi", channels = "rw",
     id_width = 4, addr_width = 32, data_width = 64, user_width = 1,
     interface = {type = "axi4_master"}}
  ]

  slaves = [
    {name = "ddr", prefix = "ddr_s_axi", protocol = "axi4",
     id_width = 8, addr_width = 64, data_width = 512, user_width = 1,
     base_addr = 0x80000000, addr_range = 0x80000000,
     interface = {type = "axi4_slave"}},
    {name = "apb_periph", prefix = "apb0_", protocol = "apb",
     addr_width = 32, data_width = 32,
     base_addr = 0x00000000, addr_range = 0x00010000}
  ]

Connectivity CSV (bridge_rapids_connectivity.csv):

master\slave,ddr,apb_periph
rapids_descr_wr,1,0
rapids_sink_wr,1,0
rapids_src_rd,1,0
cpu,1,1

Generated RTL Features: - rapids_descr_wr: 37 signals (write channels only) vs 61 signals (full) = 39% reduction - rapids_sink_wr: 37 signals (write channels only) vs 61 signals (full) = 39% reduction - rapids_src_rd: 24 signals (read channels only) vs 61 signals (full) = 61% reduction - cpu_master: Width converters for 64b→512b upsize (both wr and rd converters) - ddr_controller: Direct 512b connection (no conversion) - apb_periph0: APB converter placeholder (Phase 3)

### 25.8.6	Common User Questions

Q: “How do I generate a bridge?”

A: Three steps:

- Create TOML port configuration file

- Create CSV connectivity matrix

- Run bridge_generator.py

# Create bridge_mybridge.toml (port configuration)
# Create bridge_mybridge_connectivity.csv (connectivity matrix)

cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_mybridge.toml
# Auto-finds bridge_mybridge_connectivity.csv

# Or use bulk generation for multiple bridges
python3 bridge_generator.py --bulk bridge_batch.csv

Q: “What’s the difference between wr/rd/rw channels?”

A: Number of AXI4 channels generated:

- rw (read/write) - All 5 channels: AW, W, B, AR, R

- wr (write-only) - 3 channels: AW, W, B (no read channels)

- rd (read-only) - 2 channels: AR, R (no write channels)

Benefits: 40-60% fewer ports for dedicated masters, less logic, faster synthesis

Q: “Can I add timing isolation to ports?”

A: Yes, use interface configuration:

masters = [
  {name = "cpu", prefix = "cpu_m_axi", ...,
   interface = {type = "axi4_master", skid_depths = {ar = 2, r = 4, aw = 2, w = 4, b = 2}}}
]

Available interface types: - "axi4_master" - Timing isolation on master port - "axi4_slave" - Timing isolation on slave port - "axi4_master_mon" - Timing + monitoring - "axi4_slave_mon" - Timing + monitoring - Omit interface field for direct connection

Q: “Can I mix AXI4 and APB slaves?”

A: Yes, use protocol field in TOML:

slaves = [
  {name = "ddr", prefix = "ddr_s_axi", protocol = "axi4", ...},
  {name = "apb0", prefix = "apb0_", protocol = "apb", ...}
]

Generator inserts AXI2APB converters automatically (Phase 3 for full implementation).

Q: “Can ports be AMBA5 (AXI5 / APB5)?”

A: Yes — per port, while the fabric stays AXI4 (BRIDGE-002):

[[bridge.masters]]
protocol = "axi5"
axi5_features = ["trace", "atomic"]  # nsaid/trace/mpam/mecid/unique are
                                     # droppable sideband; poison/atomic are
                                     # connectivity-gated (every connected path
                                     # must be AXI5 + feature + width-matched);
                                     # mte/chunking rejected

[[bridge.slaves]]
protocol = "apb5"   # APB4 rules apply (rw-only, 32-bit); adds the APB5 pins

Sideband passes natively on width-matched AXI5<->AXI5 paths and terminates with a generation-time warning elsewhere. Atomics are store-class only: read-return classes DECERR at the boundary (axi5_atomic_filter). Details: docs/bridge_has/ch04_interfaces/04_axi5_apb5_interfaces.md and docs/bridge_mas/ch02_blocks/10_amba5_boundary.md.

Q: “What if data widths don’t match?”

A: Generator inserts width converters automatically:

masters = [
  {name = "cpu", data_width = 64, ...},  # 64b master
]
slaves = [
  {name = "ddr", data_width = 512, ...}  # 512b slave
]
# Generator creates width converter: 64b → 512b

Q: “Do all masters need to connect to all slaves?”

A: No, use partial connectivity in CSV:

master\slave,ddr,sram,apb
rapids_descr,1,1,0     # Connects to ddr and sram only
cpu,1,1,1              # Connects to all three

### 25.8.7	Generator Output Structure

Generated File Contains:

- Module Header - Parameterized with NUM_MASTERS, NUM_SLAVES, widths

- Port Declarations - Custom prefix per port, channel-specific signals

- Internal Signals - Crossbar interface arrays (xbar_m_, xbar_s_)

- Width Converters - Master-side upsize instances (channel-aware)

- Crossbar Instance - Internal AXI4 full crossbar

- Direct Connections - For matching-width interfaces

- APB Converters - Placeholder TODO comments (Phase 3)

Example Output Size: - Simple 2x2 bridge: ~400 lines - Complex 5x3 with mixed protocols: ~900 lines - Includes comprehensive comments and structure

### 25.8.8	Testing Generated Bridges

For Pure AXI4 Bridges (Working Now):

# Generate bridge (outputs to rtl/generated/<bridge_name>/)
python3 bridge_generator.py --ports test_configs/bridge_2x2_rw.toml

# Run its test (per-config TB classes live in dv/tbclasses/, e.g. bridge2x2_rw_tb.py)
cd ../dv/tests
pytest test_bridge_2x2_rw.py -v

For Mixed AXI4/APB (Requires Phase 3): APB converter placeholders need implementation before end-to-end testing.

## 25.9	Bridge Architecture Quick Reference

### 25.9.1	Generated Bridge Crossbar Structure

Illustrative shape (see rtl/generated/bridge_2x2_rw/bridge_2x2_rw.sv for a real example):

The generated top takes no parameters at all – every width is fixed at generation time and reaches the module through its package:

module bridge_2x2_rw
    import bridge_2x2_rw_pkg::*;
(
    input  logic aclk,
    input  logic aresetn,

    // Master-side interfaces (slave ports on bridge)
    // AW, W, B, AR, R channels for each master

    // Slave-side interfaces (master ports on bridge)
    // AW, W, B, AR, R channels for each slave
);

### 25.9.2	Key Features

- 5-Channel Implementation: Complete AW, W, B, AR, R routing

- Round-Robin Arbitration: Per-slave arbitration with grant locking

- Response routing: a small in-order FIFO per direction in each slave adapter, holding the bridge_id of the master whose request was accepted. Not a RAM, and not indexed by AXI ID.

- bridge_id sideband routing: responses return IN ORDER. The slave must complete in the order its requests were accepted; a reordering slave misroutes silently (see FR-2 in the PRD).

- Configurable Parameters: NxM topology, data/addr/ID widths

### 25.9.3	Address Map

There is no default address map and no 0x10000000 stride. Windows come from each slave’s base_addr / addr_range in the TOML and are baked into the decode. bridge_2x2_rw splits the space in half:

- Slave 0 (ddr): 0x00000000 - 0x7FFFFFFF

- Slave 1 (sram): 0x80000000 - 0xFFFFFFFF

Probing 0x1000_0000 expecting “slave 1” hits slave 0.

## 25.10	Test Organization

### 25.10.1	Test Hierarchy

projects/components/bridge/dv/tests/
├── conftest.py                        # Pytest configuration with fixtures
├── test_bridge_1x2_rd.py              # Per-config functional tests
├── test_bridge_2x2_rw.py
├── test_bridge_4x4_rw.py
├── test_bridge_5x3_channels.py
├── test_bridge_mix_a_mon_monitor.py   # Monitor stress tests (mon configs)
└── monitor_stress_common.py           # Shared monitor-stress helpers

### 25.10.2	Test Levels

Per-Config Functional Tests: - Individual bridge functionality - Address routing - ID tracking - Arbitration

Monitor Stress Tests (*_mon_monitor.py): - Bridges generated with monitor interfaces - Monitor packet generation under traffic

## 25.11	Common User Questions and Responses

### 25.11.1	Q: “How does the Bridge work?”

A: Direct answer:

The Bridge AXI4 crossbar connects multiple AXI4 masters to multiple slaves:

- Address Decode (AW/AR): Routes master requests to appropriate slave based on address

- Write Path: AW → arbitration → slave, W follows locked grant

- Read Path: AR → arbitration → slave, R returns via ID table

- Arbitration: Per-slave round-robin, AW and AR only, with the grant locked until the ADDRESS handshake – not until burst complete. W is owned via a FIFO; B and R have no arbiter.

- Response Routing: B/R responses use ID lookup tables (not grant-based)

Key Features: - Out-of-order response support via ID tracking - Burst-aware arbitration (grant locked until WLAST/RLAST) - Configurable NxM topology - Single-clock domain

See: - projects/components/bridge/PRD.md - Complete specification - projects/components/bridge/bin/bridge_generator.py - Generator implementation

### 25.11.2	Q: “How do I generate a Bridge?”

A: Use the bridge_generator.py tool:

cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_2x2_rw.toml
# Auto-finds test_configs/bridge_2x2_rw_connectivity.csv
# Or regenerate everything: make all  (bulk from bridge_batch.csv)

Generated files: - RTL: projects/components/bridge/rtl/generated/<bridge_name>/ (top module, crossbar, adapters, package) - Contains complete 5-channel crossbar with ID tracking

### 25.11.3	Q: “How do I test a Bridge?”

A: Use the Bridge testbench class:

# Import from project area
import sys
from TBClasses.shared.utilities import get_repo_root
sys.path.insert(0, get_repo_root())
from projects.components.bridge.dv.tbclasses.bridge2x2_rw_tb import Bridge2x2RwTB

@cocotb.test()
async def cocotb_test_basic(dut):
    tb = Bridge2x2RwTB(dut)
    await tb.setup_clocks_and_reset()

    # Send write to slave 0
    await tb.write_transaction(master_idx=0, address=0x00001000, data=0xDEADBEEF)

    # Verify routing
    assert len(tb.aw_slaves[0]._recvQ) > 0, "Slave 0 should receive AW"

Run tests:

cd projects/components/bridge/dv/tests
pytest test_bridge_2x2_rw.py -v

## 25.12	Anti-Patterns to Avoid

### 25.12.1	Anti-Pattern 1: Embedded Testbench Classes

WRONG: TB class in test file
class BridgeTB:
    """NOT REUSABLE - WRONG LOCATION"""

CORRECT: Import from project area
import sys
from TBClasses.shared.utilities import get_repo_root
sys.path.insert(0, get_repo_root())
from projects.components.bridge.dv.tbclasses.bridge2x2_rw_tb import Bridge2x2RwTB

### 25.12.2	Anti-Pattern 2: Manual AXI4 Handshaking

WRONG: Manual signal driving
self.dut.s0_axi4_awvalid.value = 1
while self.dut.s0_axi4_awready.value == 0:
    await RisingEdge(self.clock)

CORRECT: Use GAXI components
await self.aw_master.send(aw_pkt)

### 25.12.3	Anti-Pattern 3: Memory Models for Simple Tests

WRONG: Unnecessary complexity
memory = MemoryModel()
memory.write(addr, data)
result = memory.read(addr)

CORRECT: Direct queue verification
aw_pkt = self.aw_slave._recvQ.popleft()
assert aw_pkt.addr == expected_addr

## 25.13	Quick Reference

### 25.13.1	Finding Existing Components

# Bridge TB classes (in PROJECT AREA, not framework!)
ls projects/components/bridge/dv/tbclasses/

# Bridge generator
cat projects/components/bridge/bin/bridge_generator.py

# Test examples
ls projects/components/bridge/dv/tests/

### 25.13.2	Common Commands

# Generate 2x2 bridge (from bin/, outputs to rtl/generated/bridge_2x2_rw/)
cd projects/components/bridge/bin
python3 bridge_generator.py --ports test_configs/bridge_2x2_rw.toml

# Run tests
pytest projects/components/bridge/dv/tests/test_bridge_2x2_rw.py -v

# Lint generated RTL
verilator --lint-only projects/components/bridge/rtl/generated/bridge_2x2_rw/bridge_2x2_rw.sv

## 25.14	Remember

- MANDATORY: Testbench architecture - TB classes in the PROJECT area (projects/components/bridge/dv/tbclasses/), NOT the framework; tests import them. (This line used to say “in framework”, contradicting the hard rule above.)

- MANDATORY: Directory structure - Follow RAPIDS/AMBA pattern exactly

- MANDATORY: conftest.py - Must exist in dv/tests/

- Use GAXI components - Never manually drive AXI4 handshakes

- Queue-based verification - Simple tests use direct queue access

- Three-layer architecture - TB (PROJECT area, not the framework) + Test (runner) + Scoreboard (verification)

- Three mandatory methods - setup_clocks_and_reset, assert_reset, deassert_reset

- Search first - Use existing components before creating new ones

- Test scalability - Support basic/medium/full test levels

- 100% success - All tests must achieve 100% success rate

## 25.15	PDF Generation Location

IMPORTANT: PDF files should be generated in the docs directory:

projects/components/bridge/docs/

Quick Commands: Use the provided shell scripts:

cd projects/components/bridge/docs
./generate_has_pdf.sh   # Builds Bridge_HAS_v<rev> from bridge_has/
./generate_mas_pdf.sh   # Builds Bridge_MAS_v<rev> from bridge_mas/

The shell scripts will automatically: 1. Use the md_to_docx.py tool from bin/ 2. Process the bridge_has/bridge_mas index files 3. Generate both DOCX and PDF files in the docs/ directory 4. Create table of contents and title page

See: bin/md_to_docx.py for complete implementation details

Version: 1.0 Last Updated: 2025-10-18 Maintained By: RTL Design Sherpa Project