RTL Design Sherpa

Timing Characterization -- Synthesis Guide Per-FUB recipes, the multi-flow SDC, and how to read the results

September 29, 2026

Table of Contents

Timing Characterization – Synthesis Guide	6

1. What This Design Is and Why It Exists	6

2. Architecture Overview	6

3. FUB Catalog – Characterization Purpose of Each Block	7

3.1 nand_chain (NAND Gate Binary Tree)	7

3.2 inverter_chain (Inverter Chain)	8

3.3 xor_tree (XOR Gate Binary Tree)	8

3.4 carry_chain (Ripple-Carry Adder)	9

3.5 multiplier_tree (Multiplier Wrapper)	10

3.6 mux_tree (Binary Mux Tree)	10

3.7 queue_depth (FIFO Queue)	11

3.8 clock_divider_chain (Clock Divider Cascade)	12

3.9 gray_counter_chain (Gray Counter)	12

4. The LFSR – Anti-Optimization Engine	13

5. SDC Constraints Strategy	13

5.1 Multi-Flow SDC	13

5.2 Overridable Parameters	14

5.3 What Each Flow Requires Externally	14

5.4 The 80/20 I/O Split	15

5.5 Sweeping Frequency	15

5.6 Tool-Specific Features	16

6. Synthesis Workflow	16

6.1 Single-FUB Characterization	16

6.2 Parameter Sweep	17

6.3 Cross-FUB Comparison	17

6.4 Cross-Technology Comparison	17

7. Interpreting Results	17

7.1 Key Metrics	17

7.2 Expected Trends	18

7.3 Normalizing Across Technologies	19

8. File Organization	19

9. Quick Reference – Recommended First Runs	20

List of Figures

No figures in this document.

List of Tables

No tables in this document.

# Timing Characterization – Synthesis Guide

Version: 1.0 Last Updated: 2026-03-18 Purpose: Detailed guide to synthesizing and interpreting timing characterization results

## 1. What This Design Is and Why It Exists

char_top is a synthesis benchmarking harness. It does not implement a useful datapath – its sole purpose is to force synthesis tools to build specific combinational structures between registered endpoints so that the resulting timing reports can be compared across:

- FPGA families (Xilinx 7-series vs. UltraScale vs. Cyclone/Stratix)

- ASIC standard-cell libraries (TSMC, GF, Samsung at various nodes)

- Synthesis tool settings (effort levels, mapping strategies)

- Parameter sweeps (wider adders, deeper trees, bigger multipliers)

Every flip-flop input is driven by a 32-bit Galois LFSR that produces a maximal-length pseudo-random sequence. Every flip-flop output is routed to a top-level port. Together these two measures guarantee that the synthesis tool cannot constant-propagate inputs or prune unused outputs. Additionally, every internal combinational net carries vendor-appropriate dont_touch / preserve attributes as a second line of defense.

The design is organized as a single top-level wrapper (char_top) that conditionally instantiates nine Functional Unit Blocks (FUBs). Each FUB isolates a different class of combinational logic so that its timing can be reported independently.

## 2. Architecture Overview

+------------------------------------------+
                    |              char_top                    |
                    |                                          |
  i_seed_valid ---->|  +--------+                              |
  i_seed_data  ---->|  | 32-bit |   r_lfsr[31:0]               |
                    |  | Galois |--------------------------+-->| FUB inputs
                    |  |  LFSR  |                          |   |
                    |  +--------+                          |   |
                    |                                      |   |
                    |  +-------------+  +--------------+   |   |
                    |  | nand_chain  |  | inverter_chain|  |   |
                    |  | (NAND tree) |  | (INV chain)  |<--+   |
                    |  +------+------+  +------+-------+   |   |
                    |         |                |           |   |
                    |  +------+------+  +------+-------+   |   |
                    |  | xor_tree    |  | carry_chain  |<--+   |
                    |  | (XOR tree)  |  | (adder)      |   |   |
                    |  +------+------+  +------+-------+   |   |
                    |         |                |           |   |
                    |  +------+------+  +------+-------+   |   |
                    |  | mux_tree    |  | multiplier   |<--+   |
                    |  | (MUX tree)  |  |   _tree      |   |   |
                    |  +------+------+  +------+-------+   |   |
                    |         |                |           |   |
                    |  +------+------+  +------+-------+   |   |
                    |  | queue_depth |  | clock_divider|   |   |
                    |  | (FIFO)      |  |   _chain     |<--+   |
                    |  +------+------+  +------+-------+   |   |
                    |         |                |           |   |
                    |  +------+-----------+    |           |   |
                    |  | gray_counter     |<---+           |   |
                    |  |   _chain         |                |   |
                    |  +------+-----------+                |   |
                    |         |                            |   |
                    +---------+----------------------------+---+
                              |
                              v
                    o_nand, o_inverter, o_xor, o_carry, o_mult,
                    o_mux, o_queue_data, o_queue_valid, o_queue_count,
                    o_clk_div, o_gray_bin, o_gray_code

Each FUB is controlled by an EN_* parameter. When disabled (EN_*=0), the corresponding generate block emits only a tie-to-zero assignment and consumes zero logic. This allows targeted synthesis runs that isolate a single FUB or any combination.

## 3. FUB Catalog – Characterization Purpose of Each Block

### 3.1 nand_chain (NAND Gate Binary Tree)

| Attribute | Value |
| --- | --- |
| RTL file | rtl/fub/nand_chain.sv |
| Enable | EN_NAND_TREE |
| Key params | NAND_LEVELS, NUM_FLOPS |
| Critical path | LEVELS NAND gate delays (root to leaf) |
| What it measures | Raw 2-input NAND propagation through LUT fabric |

Structure: A complete binary tree of 2-input NAND gates. The leaf level reads from registered input flops; the root feeds a registered output flop. Heap-style indexing maps tree nodes to a flat array. When 2^LEVELS > NUM_FLOPS, leaf inputs reuse flops via modulo wrapping – this preserves the full tree depth (and critical path) while bounding flop count.

Synthesis goal: Measure how NAND gate depth maps to LUT levels on FPGA or gate levels on ASIC. A LEVELS=8 tree creates 255 NAND gates with an 8-gate critical path. Compare slack at 500 MHz across devices to understand per-gate delay.

### 3.2 inverter_chain (Inverter Chain)

| Attribute | Value |
| --- | --- |
| RTL file | rtl/fub/inverter_chain.sv |
| Enable | EN_INVERTER_CHAIN |
| Key params | INVERTER_COUNT |
| Critical path | INVERTER_COUNT inverter delays (linear) |
| What it measures | Pure gate delay without fan-in effects |

Structure: A simple linear chain of inverters: r_input_flop -> ~() -> ~() -> ... -> ~() -> r_out_flop. Even count = same polarity as input; odd count = inverted.

Synthesis goal: Baseline single-gate delay measurement. Inverters have zero fan-in overhead and minimal routing, so they represent the best-case per-gate delay. Compare against NAND/XOR to isolate fan-in overhead. Sweep INVERTER_COUNT to find the linear region and the point where routing delay begins to dominate.

### 3.3 xor_tree (XOR Gate Binary Tree)

| Attribute | Value |
| --- | --- |
| RTL file | rtl/fub/xor_tree.sv |
| Enable | EN_XOR_TREE |
| Key params | XOR_LEVELS, NUM_FLOPS |
| Critical path | LEVELS XOR gate delays |
| What it measures | XOR-specific LUT packing efficiency |

Structure: Identical topology to nand_chain but with XOR gates instead of NAND. Same heap indexing and modulo flop reuse.

Synthesis goal: XOR gates stress FPGA LUTs differently than NAND. On Xilinx, a 6-input LUT can absorb multiple XOR levels, while NAND chains often pack fewer levels per LUT. Comparing XOR tree slack against NAND tree slack at the same depth reveals LUT packing efficiency for each gate type.

### 3.4 carry_chain (Ripple-Carry Adder)

| Attribute | Value |
| --- | --- |
| RTL file | rtl/fub/carry_chain.sv |
| Enable | EN_CARRY_CHAIN |
| Key params | CARRY_WIDTH |
| Critical path | WIDTH carry-propagation delays |
| What it measures | Dedicated carry-chain resource speed |

Structure: Two registered input vectors feed a simple + operator. The synthesis tool infers a ripple-carry adder and maps it to dedicated fast-carry resources (CARRY4/CARRY8 on Xilinx, ALM carry on Quartus targets). The output is WIDTH+1 bits (including carry-out).

Synthesis goal: Measure the speed of the dedicated carry chain versus generic LUT routing. On modern FPGAs, carry chains run at sub-100ps per bit – far faster than LUT-routed equivalents. Sweep CARRY_WIDTH from 8 to 256+ to find the width at which carry propagation becomes the limiting factor at the target frequency. This directly informs max-width adder decisions for production datapaths.

### 3.5 multiplier_tree (Multiplier Wrapper)

| Attribute | Value |
| --- | --- |
| RTL file | rtl/fub/multiplier_tree.sv |
| Enable | EN_MULTIPLIER |
| Key params | MULT_WIDTH, MULT_TYPE |
| Critical path | Multiplier reduction tree depth |
| What it measures | DSP vs. LUT-fabric multiplier timing |

Structure: Two registered input vectors feed a multiplier that produces a 2*WIDTH output. MULT_TYPE selects the implementation:

| MULT_TYPE | Implementation | Available widths |
| --- | --- | --- |
| 0 | Inferred (* operator) | Any |
| 1 | Dadda tree (structural) | 8, 16, 32 |
| 2 | Wallace tree (structural) | 8, 16, 32 |
| 3 | Wallace tree CSA (structural) | 8, 16, 32 |
| 4 | Dadda 4:2 compressor (structural) | 8, 11, 24 |

Unsupported width/type combinations fall back to inferred.

Synthesis goal: Compare inferred multiplier timing (where the tool decides DSP vs. LUT) against explicit structural implementations. On Xilinx, inferred * at 16-bit width will map to a single DSP48E2 slice; structural types force LUT-only implementation. The slack difference directly quantifies the DSP advantage. Sweep MULT_WIDTH to find the crossover point where the tool stops using DSPs and switches to LUTs.

### 3.6 mux_tree (Binary Mux Tree)

| Attribute | Value |
| --- | --- |
| RTL file | rtl/fub/mux_tree.sv |
| Enable | EN_MUX_TREE |
| Key params | MUX_LEVELS, NUM_FLOPS |
| Critical path | LEVELS 2:1 mux delays |
| What it measures | Mux-to-LUT mapping and routing fabric |

Structure: A binary tree of 2:1 multiplexers. Data inputs come from registered flops; select inputs come from a separate registered select vector. The tree reduces 2^LEVELS data inputs down to a single output bit through LEVELS mux stages.

Synthesis goal: Muxes map to LUTs differently than logic gates. A 6-input LUT can implement a 4:1 mux (2 levels) in a single cell. Compare mux tree slack against NAND/XOR tree slack at the same depth to understand mux-to-LUT packing. Also reveals routing congestion effects at large tree sizes since every mux level requires routing from two subtrees.

### 3.7 queue_depth (FIFO Queue)

| Attribute | Value |
| --- | --- |
| RTL file | rtl/fub/queue_depth.sv |
| Enable | EN_QUEUE_DEPTH |
| Key params | FIFO_DATA_WIDTH, FIFO_DEPTH |
| Critical path | FIFO read path + output register |
| What it measures | Memory inference thresholds and FIFO timing |

Structure: Wraps gaxi_fifo_sync (from rtl/common). LFSR data is continuously written when the FIFO has space; reads are always enabled. An output flop captures the read data.

Synthesis goal: Sweep DATA_WIDTH and DEPTH to find the boundaries where the synthesis tool transitions between memory implementations:

| FPGA transition | Typical boundary |
| --- | --- |
| Distributed RAM (LUTRAM) -> BRAM | ~64 entries or ~2Kb total |
| BRAM -> URAM (UltraScale+) | ~4K entries or ~36Kb total |

Each transition changes the read latency and routing characteristics. The timing report reveals the cost of each memory type at the target frequency.

### 3.8 clock_divider_chain (Clock Divider Cascade)

| Attribute | Value |
| --- | --- |
| RTL file | rtl/fub/clock_divider_chain.sv |
| Enable | EN_CLK_DIVIDER |
| Key params | CLK_DIV_STAGES, CLK_DIV_CW, CLK_DIV_PICKOFF |
| Critical path | Counter increment + pickoff mux per stage |
| What it measures | Counter-based clock generation timing |

Structure: NUM_STAGES cascaded clock_divider instances. Stage 0 divides the input clock; each subsequent stage divides the previous stage’s output. Each stage contains a COUNTER_WIDTH-bit counter and a pickoff mux that selects the divided output.

Synthesis goal: Characterize the timing of counter-based clock generation. The pickoff point determines which counter bit drives the divided clock – higher pickoff points mean longer counter chains before the toggle. Sweep CLK_DIV_STAGES to measure cascaded divider overhead and CLK_DIV_CW to test counter width scaling.

### 3.9 gray_counter_chain (Gray Counter)

| Attribute | Value |
| --- | --- |
| RTL file | rtl/fub/gray_counter_chain.sv |
| Enable | EN_GRAY_COUNTER |
| Key params | GRAY_WIDTH |
| Critical path | Binary counter carry + XOR tree for Gray conversion |
| What it measures | Binary-to-Gray conversion timing at width |

Structure: Wraps counter_bingray (from rtl/common). The internal binary counter uses dedicated carry logic; the Gray conversion is gray = bin ^ (bin >> 1), which creates a WIDTH-1 wide XOR tree. Output flops capture both binary and Gray values.

Synthesis goal: The interesting path is the XOR tree for Gray conversion. At narrow widths, the XOR tree fits in a few LUTs. At wide widths (64+), it becomes a significant combinational depth. Sweep GRAY_WIDTH to find the crossover where Gray conversion dominates the counter carry chain in the critical path.

## 4. The LFSR – Anti-Optimization Engine

The shared 32-bit Galois LFSR (polynomial x^32 + x^22 + x^2 + x + 1) serves three purposes:

- Prevents constant propagation – Every FUB input changes every cycle with a pseudo-random pattern, so the synthesis tool cannot simplify logic based on constant input values.

- Prevents output pruning – All LFSR-driven data eventually reaches a top-level output port, so no internal logic can be removed as unused.

- Provides deterministic seeding – The i_seed_valid / i_seed_data interface allows testbenches to load a known seed and predict the exact LFSR sequence, enabling cycle-accurate functional verification.

The RTL implementation:

r_lfsr <= {r_lfsr[30:0], 1'b0} ^ (r_lfsr[31] ? 32'h0040_0007 : 32'h0);

Each FUB’s inputs are wired as combinational taps from r_lfsr with bit-modulo wrapping to fill widths wider than 32 bits. For two-input FUBs (carry, multiplier), the second operand uses a 16-bit-shifted mapping (r_lfsr[(b+16) % 32]) to ensure the two inputs are decorrelated.

## 5. SDC Constraints Strategy

### 5.1 Multi-Flow SDC

char_top.sdc supports three synthesis flows via the FLOW variable:

| Flow | Tool | Set before sourcing |
| --- | --- | --- |
| "asic" | Synopsys DC / Cadence Genus | Target library, operating conditions |
| "vivado" | Xilinx Vivado | Part number (e.g., xc7a200t...) |
| "quartus" | Quartus Prime | FAMILY, DEVICE in .qsf (fpga/quartus/sweep_config.example.tcl) |

All overridable parameters use if {![info exists ...]} guards so you can set them in your synthesis script before sourcing the SDC:

# Example: Vivado flow at 300 MHz
set FLOW "vivado"
set TARGET_FREQ_MHZ 300.0
source char_top.sdc

### 5.2 Overridable Parameters

| Parameter | Default | Rationale |
| --- | --- | --- |
| FLOW | "asic" | Selects tool-specific constraint blocks |
| TARGET_FREQ_MHZ | 500 | Aggressive enough to expose path differences |
| INPUT_DELAY_FRACTION | 0.80 | Models heavy external source |
| OUTPUT_DELAY_FRACTION | 0.20 | Models light external sink |
| CLK_UNCERTAINTY_NS | 0.100 | Covers jitter + skew |

### 5.3 What Each Flow Requires Externally

ASIC (Synopsys DC):

set target_library "your_stdcell.db"
set link_library   "* your_stdcell.db"
set_operating_conditions -max WC_COM
set FLOW "asic"
source char_top.sdc

ASIC (Cadence Genus):

set_db init_lib_search_path /path/to/libs
read_libs your_stdcell.lib
set FLOW "asic"
read_sdc char_top.sdc

Vivado:

set_property part xc7a200tffg1156-2 [current_project]
set FLOW "vivado"
read_xdc char_top.sdc
synth_design -top char_top -generic EN_CARRY_CHAIN=1 ...

Quartus:

# In .qsf:
set_global_assignment -name FAMILY "Cyclone V"
set_global_assignment -name DEVICE 5CGXFC5C6F27C7
set_global_assignment -name SDC_FILE timing_target.sdc   ;# a 6-line wrapper, see below
set_global_assignment -name OPTIMIZATION_MODE "HIGH PERFORMANCE EFFORT"

The Timing Analyzer reads SDC as Tcl, so the wrapper it is pointed at sets the flow and sources the master file (there is no “pre-SDC hook”):

# timing_target.sdc
set FLOW            quartus
set TARGET_FREQ_MHZ 200
source "/abs/path/to/rtl/syn/char_top.sdc"

This is exactly what fpga/quartus/syn_sweep_quartus.tcl generates per sweep point; cd fpga && make quartus-sweep runs the whole loop and fpga/quartus/sta_reports.tcl adds per-FUB slack / data delay / logic levels. Two Timing Analyzer differences are folded into the master SDC: set_clock_uncertainty takes -to <clock> with the value LAST, and set_load is not implemented (read_sdc aborts on it).

### 5.4 The 80/20 I/O Split

The 80% input delay / 20% output delay split is intentional:

- Input delay (80%): The LFSR register is the first stage. By consuming 80% of the period on the input side, we force the LFSR update to close with only 20% margin. Since the LFSR is a simple shift-XOR, it easily meets this constraint, and the “real” critical paths are the register-to-register paths within each FUB.

- Output delay (20%): FUB outputs go directly from registered output flops to top-level ports. The 20% output budget is trivially met for registered outputs.

The net effect is that all interesting timing is in the register-to-register paths within each FUB, which is exactly what we want to characterize.

### 5.5 Sweeping Frequency

To sweep the target frequency, override the TARGET_FREQ_MHZ variable before sourcing the SDC:

set TARGET_FREQ_MHZ 250.0
set FLOW "vivado"
source char_top.sdc

Typical sweep points: 100, 200, 300, 400, 500, 600, 750, 1000 MHz.

### 5.6 Tool-Specific Features

| Feature | ASIC | Vivado | Quartus |
| --- | --- | --- | --- |
| Per-FUB path groups | group_path (active) | report_timing -from (manual) | report_timing -from (manual) |
| Dont-touch enforcement | set_dont_touch (active) | DONT_TOUCH property (commented) | PRESERVE_REGISTER (commented) |
| I/O driving model | set_driving_cell (commented, needs lib cell) | IOSTANDARD (commented) | Via .qsf IO_STANDARD |
| Output load | Via library | set_load 5.0 | not supported by the Timing Analyzer; via .qsf IO_STANDARD |
| Per-FUB slack/delay/levels | report_timing -group | manual | fpga/quartus/sta_reports.tcl -> fub_slack.csv (automated) |

The ASIC flow enables path groups and dont-touch by default since these are standard DC/Genus commands. Vivado and Quartus equivalents are commented out because they use property-based syntax that depends on the specific target; uncomment as needed.

## 6. Synthesis Workflow

### 6.1 Single-FUB Characterization

Enable only the FUB of interest to minimize compile time:

# Vivado example: synthesize only carry_chain
synth_design -top char_top \
    -generic EN_NAND_TREE=0 \
    -generic EN_INVERTER_CHAIN=0 \
    -generic EN_XOR_TREE=0 \
    -generic EN_CARRY_CHAIN=1 \
    -generic EN_MULTIPLIER=0 \
    -generic EN_MUX_TREE=0 \
    -generic EN_QUEUE_DEPTH=0 \
    -generic EN_CLK_DIVIDER=0 \
    -generic EN_GRAY_COUNTER=0 \
    -generic CARRY_WIDTH=64

### 6.2 Parameter Sweep

For a carry-chain width sweep, iterate over CARRY_WIDTH values:

for W in 8 16 32 64 128 256 512; do
    vivado -mode batch -source syn_sweep.tcl -tclargs CARRY_WIDTH=$W
done

### 6.3 Cross-FUB Comparison

Enable all FUBs and use path groups to compare in a single run:

synth_design -top char_top   ;# All EN_* default to 1
report_timing -group GRP_NAND -max_paths 1
report_timing -group GRP_CARRY -max_paths 1
report_timing -group GRP_MULT -max_paths 1
# ... etc.

### 6.4 Cross-Technology Comparison

Synthesize the same RTL + SDC against different targets:

| Target | Tool | Command |
| --- | --- | --- |
| Xilinx Artix-7 | Vivado | set_property part xc7a200t... |
| Xilinx UltraScale+ | Vivado | set_property part xcvu9p... |
| Cyclone 10 | Quartus | set_global_assignment FAMILY "Cyclone 10 LP" |
| Stratix 10 | Quartus | set_global_assignment FAMILY "Stratix 10" |
| ASIC (TSMC 28nm) | DC/Genus | Point to .db / .lib library |

The SDC is tool-agnostic (standard Tcl/SDC syntax) and works unchanged across all these flows.

## 7. Interpreting Results

### 7.1 Key Metrics

For each FUB at each parameter point, extract:

| Metric | Source | Meaning |
| --- | --- | --- |
| Setup slack | Timing report | How much margin remains (negative = fails) |
| Data path delay | Timing report | Actual combinational delay |
| Logic levels | Timing report | Number of LUT/gate levels |
| Resource usage | Utilization report | LUTs, FFs, DSPs, BRAMs consumed |
| Fmax | 1 / (period - slack) | Maximum achievable frequency |

### 7.2 Expected Trends

| FUB | Parameter sweep | Expected trend |
| --- | --- | --- |
| nand_chain | LEVELS 1-12 | Linear delay increase per level |
| inverter_chain | COUNT 1-256 | Linear delay, flattens when routing dominates |
| xor_tree | LEVELS 1-12 | Similar to NAND but potentially fewer logic levels (better LUT packing) |
| carry_chain | WIDTH 8-512 | Sub-linear delay (dedicated carry is fast) |
| multiplier_tree | WIDTH 8-32, TYPE 0-4 | TYPE=0 uses DSP (fast), structural types use LUTs (slower) |
| mux_tree | LEVELS 1-12 | ~2 levels per LUT on 6-input LUT architectures |
| queue_depth | DEPTH 4-4096 | Step changes at LUTRAM/BRAM/URAM boundaries |
| clock_divider_chain | STAGES 1-8 | Linear per stage |
| gray_counter_chain | WIDTH 4-128 | Dominated by XOR tree above ~32 bits |

### 7.3 Normalizing Across Technologies

To compare across technologies, normalize to delay per logic level:

delay_per_level = data_path_delay / logic_levels

This removes the depth variable and isolates the per-gate (or per-LUT) delay, which is the true technology-dependent metric.

## 8. File Organization

rtl/syn/
  char_top.sdc           <-- This SDC file (constraints)
  SYNTHESIS_GUIDE.md      <-- This document

rtl/fub/                  <-- FUB RTL (one per characterization block)
  nand_chain.sv
  inverter_chain.sv
  xor_tree.sv
  carry_chain.sv
  multiplier_tree.sv
  mux_tree.sv
  queue_depth.sv
  clock_divider_chain.sv
  gray_counter_chain.sv

rtl/top/                  <-- Top-level wrappers
  char_top.sv             <-- Main synthesis target
  nand_chain_top.sv       <-- Standalone NAND chain wrapper

rtl/common/               <-- Shared dependencies (copied from rtl/common/)
  counter_bin.sv
  counter_bingray.sv
  clock_divider.sv
  fifo_control.sv
  gaxi_fifo_sync.sv
  math_adder_half.sv
  math_adder_full.sv
  math_adder_carry_save.sv
  math_compressor_4to2.sv
  math_multiplier_dadda_tree_*.sv
  math_multiplier_wallace_tree_*.sv
  math_multiplier_wallace_tree_csa_*.sv
  math_multiplier_dadda_4to2_*.sv
  math_adder_han_carlson_*.sv
  math_prefix_cell.sv
  math_prefix_cell_gray.sv

rtl/filelists/            <-- Filelist for synthesis and simulation
  char_top.f

## 9. Quick Reference – Recommended First Runs

| Run | Enable flags | Key parameter | Goal |
| --- | --- | --- | --- |
| Carry sweep | EN_CARRY_CHAIN=1, all others 0 | CARRY_WIDTH=8,16,32,64,128,256 | Map carry-chain speed vs. width |
| NAND depth | EN_NAND_TREE=1 | NAND_LEVELS=4,6,8,10,12 | Per-gate delay baseline |
| DSP vs. LUT | EN_MULTIPLIER=1 | MULT_TYPE=0 then MULT_TYPE=1, WIDTH=16 | Quantify DSP advantage |
| Memory inference | EN_QUEUE_DEPTH=1 | FIFO_DEPTH=4,16,64,256,1024,4096 | Find LUTRAM/BRAM boundary |
| Full comparison | All enabled | Defaults | Cross-FUB comparison at one point |

Version: 1.0 Last Updated: 2026-03-18 Maintained By: RTL Design Sherpa Project