RTL Design Sherpa

APB UART 16550 Micro-Architecture Specification 1.0

January 4, 2026

Table of Contents

1 Uart 16550 Mas Index	20

2 Retro Legacy Blocks - Product Requirements Document	21

2.1 1. Overview	21

2.1.1 1.1 Purpose	21

2.1.2 1.2 Design Philosophy	21

2.1.3 1.3 Target Applications	21

2.2 2. Implemented Blocks	22

2.2.1 2.1 HPET - High Precision Event Timer	22

2.3 3. Planned Blocks	23

2.3.1 3.1 8259 - Programmable Interrupt Controller (PIC)	23

2.3.2 3.2 8254 - Programmable Interval Timer (PIT)	23

2.3.3 3.3 GPIO - General Purpose I/O	23

2.3.4 3.4 RTC - Real-Time Clock	24

2.3.5 3.5 SMBus Controller	24

2.3.6 3.6 UART - Universal Asynchronous Receiver/Transmitter	24

2.3.7 3.7 SPI Controller	24

2.3.8 3.8 I2C Controller	25

2.3.9 3.9 Watchdog Timer	25

2.3.10 3.10 Power Management / ACPI Controller	25

2.3.11 3.11 IOAPIC - I/O Advanced Programmable Interrupt Controller	25

2.3.12 3.12 Interconnect ID / Version Registers	25

2.4 4. Integration and Wrapper Goals	26

2.4.1 4.1 Individual Block Integration	26

2.4.2 4.2 RLB Wrapper Architecture	27

2.5 5. Design Standards	29

2.5.1 5.1 Reset Handling	29

2.5.2 5.2 Register Generation	29

2.5.3 5.3 Testbench Architecture	29

2.5.4 5.4 FPGA Synthesis Attributes	30

2.5.5 5.5 Documentation Requirements	30

2.6 6. Quality Metrics	30

2.6.1 6.1 Production Readiness Criteria	30

2.6.2 6.2 Current Status	31

2.7 7. Development Roadmap	32

2.7.1 7.1 Phase 1: Foundation (Complete)	32

2.7.2 7.2 Phase 2: Core Peripherals (Next 6-9 Months)	32

2.7.3 7.3 Phase 3: Advanced Peripherals (9-15 Months)	32

2.7.4 7.4 Phase 4: System Integration (15+ Months)	33

2.8 8. References	33

2.8.1 8.1 External Standards	33

2.8.2 8.2 Internal Documentation	33

2.8.3 8.3 Block-Specific Documentation	33

2.9 9. Success Criteria	33

2.9.1 9.1 Individual Block Success	33

2.9.2 9.2 Collection Success	34

2.9.3 9.3 Long-Term Vision	34

3 PeakRDL HPET Integration - Final Status	35

3.1 Milestone: COMPLETE (5/6 configs fully passing)	35

3.1.1 Test Results Summary	35

3.2 Root Cause Found & Fixed	35

3.3 Core Functionality Validated	36

3.4 Files Modified	36

3.4.1 RTL Changes:	36

3.4.2 Test Changes:	36

3.4.3 Documentation:	37

3.5 Remaining Work (Minor)	37

3.5.1 8-Timer Non-CDC All Timers Stress Test	37

3.6 Milestone Achievement	37

3.7 Recommended Next Steps	37

3.8 Test Execution Summary	37

3.9 Git Status	38

4 APB UART 16550 — Overview	39

4.1 Overview	39

4.1.1 Key Features	39

4.1.2 Applications	40

4.2 Parameters	40

4.3 Functional Description	41

4.3.1 Figure 1.1: UART 16550 Block Diagram	41

4.3.2 Register Summary	41

4.4 Design Notes	42

4.4.1 Compatibility	42

4.5 Navigation	42

5 APB UART 16550 — Architecture	43

5.1 Overview	43

5.2 Functional Description	43

5.2.1 Figure 1.2: UART Architecture	43

5.2.2 Module Hierarchy	43

5.2.3 Data Flow	45

5.2.4 Baud Rate Generation	48

5.2.5 FIFO Organization	50

5.3 Design Notes	50

5.3.1 Resource Estimates	50

5.4 Navigation	50

6 APB UART 16550 — Clocks and Reset	52

6.1 Overview	52

6.2 Ports	52

6.2.1 Clock Signals	52

6.2.2 Reset Signals	52

6.3 Functional Description	53

6.3.1 Reset Behavior	53

6.3.2 Clock Domain Crossing	54

6.4 Timing	54

6.4.1 Baud Rate Considerations	54

6.4.2 Timing Constraints	55

6.5 Navigation	55

7 APB UART 16550 — Acronyms and Terminology	56

7.1 Overview	56

7.2 Design Notes	56

7.2.1 Protocol Acronyms	56

7.2.2 Register Acronyms	56

7.2.3 Signal Acronyms	57

7.2.4 FIFO Terms	57

7.2.5 Data Format Terms	58

7.2.6 Error Terms	58

7.2.7 Timing Terms	58

7.3 Navigation	59

8 APB UART 16550 — References	60

8.1 Overview	60

8.2 References	60

8.2.1 Internal Documentation	60

8.2.2 External References	60

8.2.3 Design References	61

8.3 Navigation	61

9 APB UART 16550 — Block Descriptions	62

9.1 Overview	62

9.2 Functional Description	62

9.2.1 Module Hierarchy	62

9.2.2 Block Summary	62

9.3 Related Modules	63

9.3.1 APB Interface	63

9.3.2 Register File	63

9.3.3 TX Engine	63

9.3.4 RX Engine	63

9.3.5 Baud Generator	63

9.3.6 FIFO Subsystem	63

9.4 Navigation	63

10 APB UART 16550 — APB Interface Block	65

10.1 Overview	65

10.2 Ports	65

10.2.1 APB Slave Interface	65

10.3 Functional Description	66

10.3.1 Figure 2.1: APB Interface Block	66

10.3.2 Address Decoding	66

10.3.3 Operation	67

10.4 Design Notes	67

10.4.1 Implementation Notes	67

10.5 Navigation	67

11 APB UART 16550 — Register File Block	68

11.1 Overview	68

11.2 Ports	68

11.2.1 Software-to-Hardware (reg2hw)	68

11.2.2 Hardware-to-Software (hw2reg)	68

11.3 Functional Description	69

11.3.1 Figure 2.2: Register File Block	69

11.3.2 Register Organization	69

11.3.3 Register Access Types	69

11.4 Design Notes	70

11.4.1 Implementation Notes	70

11.5 Navigation	70

12 APB UART 16550 — TX Engine Block	71

12.1 Overview	71

12.2 Functional Description	71

12.2.1 Figure 2.3: TX Engine Block	71

12.2.2 TX FIFO	71

12.2.3 TX Serializer	72

12.2.4 Flow Control	74

12.2.5 Break Generation	74

12.3 Timing	74

12.3.1 Bit Timing	74

12.3.2 Frame Timing Example (8N1 at 115200)	74

12.4 Waveforms	74

12.4.1 Waveform 2.1: TX Byte Transmission	74

12.4.2 Waveform 2.2: Baud Rate Generation	75

12.4.3 Waveform 2.3: Loopback Mode	75

12.5 Navigation	76

13 APB UART 16550 — RX Engine Block	77

13.1 Overview	77

13.2 Functional Description	77

13.2.1 Figure 2.4: RX Engine Block	77

13.2.2 Input Synchronizer	77

13.2.3 Start Bit Detection	78

13.2.4 RX Deserializer	78

13.2.5 RX FIFO	78

13.2.6 Error Detection	79

13.2.7 Timeout Detection	80

13.3 Waveforms	80

13.3.1 Waveform 2.4: RX Byte Reception	80

13.4 Navigation	80

14 APB UART 16550 — Baud Generator Block	81

14.1 Overview	81

14.2 Functional Description	81

14.2.1 Figure 2.5: Baud Generator Block	81

14.2.2 Operation	81

14.2.3 Divisor Calculation	82

14.2.4 Divisor Latch Registers	82

14.3 Usage Example	83

14.3.1 Programming Sequence	83

14.4 Design Notes	83

14.4.1 Special Cases	83

14.5 Navigation	84

15 APB UART 16550 — FIFO Subsystem	85

15.1 Overview	85

15.2 Functional Description	85

15.2.1 Figure 2.6: FIFO Block	85

15.2.2 FIFO Configuration	85

15.2.3 TX FIFO	86

15.2.4 RX FIFO	86

15.2.5 FIFO vs Non-FIFO Mode	87

15.2.6 Error Handling	87

15.2.7 FIFO Reset	88

15.3 Waveforms	88

15.3.1 Waveform 2.5: FIFO Trigger Timing	88

15.4 Navigation	89

16 APB UART 16550 — Interfaces	90

16.1 Overview	90

16.2 Ports	90

16.2.1 External Interfaces	90

16.3 Functional Description	91

16.3.1 Figure 3.1: UART Interface Summary	91

16.4 Navigation	91

16.4.1 Chapter Contents	91

17 APB UART 16550 — APB Slave Interface	93

17.1 Overview	93

17.2 Ports	93

17.2.1 APB Slave Signals	93

17.3 Functional Description	93

17.3.1 Address Map	93

17.3.2 Protocol Compliance	94

17.3.3 Register Access	95

17.4 Timing	95

17.4.1 Access Timing	95

17.5 Navigation	95

18 APB UART 16550 — Serial Interface	97

18.1 Overview	97

18.2 Ports	97

18.2.1 Signal Description	97

18.2.2 Electrical Interface	97

18.3 Functional Description	97

18.3.1 TXD (Transmit Data)	97

18.3.2 RXD (Receive Data)	98

18.3.3 Data Formats	99

18.3.4 Break Condition	99

18.4 Waveforms	99

18.4.1 Waveform 3.1: Line Status Error Detection	99

18.5 Navigation	100

19 APB UART 16550 — Modem Interface	101

19.1 Overview	101

19.2 Ports	101

19.2.1 Modem Control Outputs (Active Low)	101

19.2.2 Modem Status Inputs (Active Low)	101

19.3 Functional Description	101

19.3.1 Modem Control Register (MCR)	101

19.3.2 Modem Status Register (MSR)	102

19.3.3 Hardware Flow Control	103

19.3.4 Loopback Mode	103

19.3.5 Input Synchronization	103

19.3.6 Interrupt Generation	104

19.4 Waveforms	104

19.4.1 Waveform 3.2: Modem Status Change Detection	104

19.5 Navigation	104

20 APB UART 16550 — Interrupt Interface	105

20.1 Overview	105

20.2 Ports	105

20.2.1 Signal Description	105

20.3 Functional Description	105

20.3.1 Interrupt Sources	105

20.3.2 Interrupt Enable Register (IER)	106

20.3.3 Interrupt Identification Register (IIR)	106

20.3.4 Interrupt Conditions	107

20.4 Timing	107

20.4.1 Interrupt Timing	107

20.5 Usage Example	108

20.5.1 Software Handling	108

20.6 Navigation	108

21 APB UART 16550 — System Interface	109

21.1 Overview	109

21.2 Ports	109

21.2.1 Clock Signals	109

21.2.2 Reset Signals	109

21.3 Functional Description	110

21.3.1 Reset Behavior	110

21.4 Timing	111

21.4.1 Reset Sequence	111

21.5 Design Notes	111

21.5.1 Power Management	111

21.5.2 External Connections	112

21.6 Navigation	112

22 APB UART 16550 — Programming Model	113

22.1 Overview	113

22.1.1 Register Summary	113

22.2 Usage Example	113

22.2.1 Quick Start	113

22.3 Navigation	114

22.3.1 Chapter Contents	114

23 APB UART 16550 — Initialization	116

23.1 Overview	116

23.2 Functional Description	116

23.2.1 Baud Rate Calculation	116

23.3 Usage Example	116

23.3.1 Basic Initialization Sequence	116

23.3.2 Complete Initialization Example	118

23.3.3 Flow Control Setup (manual)	118

23.3.4 Loopback Mode Setup	119

23.4 Navigation	119

24 APB UART 16550 — Data Transfer	120

24.1 Overview	120

24.2 Usage Example	120

24.2.1 Transmitting Data	120

24.2.2 Receiving Data	121

24.2.3 Error Handling	122

24.2.4 FIFO Management	123

24.2.5 Bulk Transfer	123

24.3 Navigation	124

25 APB UART 16550 — Interrupt Handling	125

25.1 Overview	125

25.2 Functional Description	125

25.2.1 Interrupt Identification Register (IIR)	125

25.3 Usage Example	126

25.3.1 Interrupt Enable Register (IER)	126

25.3.2 Complete ISR Example	126

25.3.3 Individual Interrupt Handlers	127

25.3.4 Disabling/Enabling Interrupts	129

25.4 Design Notes	129

25.4.1 Interrupt Latency Considerations	129

25.5 Navigation	130

26 APB UART 16550 — Programming Examples	131

26.1 Overview	131

26.2 Usage Example	131

26.2.1 Debug Console	131

26.2.2 Ring Buffer Implementation	132

26.2.3 Command Line Interface	134

26.2.4 Loopback Test	135

26.2.5 Flow Control (manual)	136

26.3 Navigation	136

27 APB UART 16550 — Register Map	137

27.1 Overview	137

27.2 Functional Description	137

27.2.1 Register Summary	137

27.2.2 RBR - Receiver Buffer Register (0x00, Read)	138

27.2.3 THR - Transmitter Holding Register (0x00, Write)	138

27.2.4 IER - Interrupt Enable Register (0x04, R/W)	138

27.2.5 IIR - Interrupt Identification Register (0x08, Read Only)	139

27.2.6 FCR - FIFO Control Register (0x0C, R/W)	140

27.2.7 LCR - Line Control Register (0x10, R/W)	141

27.2.8 MCR - Modem Control Register (0x14, R/W)	142

27.2.9 LSR - Line Status Register (0x18, Read; error bits clear on read)	142

27.2.10 MSR - Modem Status Register (0x1C, Read; delta bits clear on read)	143

27.2.11 SCR - Scratch Register (0x20, R/W)	144

27.2.12 DLL - Divisor Latch LSB (0x24, R/W)	144

27.2.13 DLM - Divisor Latch MSB (0x28, R/W)	144

27.2.14 Address Calculation	144

27.3 Implemented for RLB TASK-012 (was RLB-013)	145

List of Figures

Figure 1.1: UART 16550 Block Diagram	41

Figure 1.2: UART Architecture	43

Figure 2.1: APB Interface Block	66

Figure 2.2: Register File Block	69

Figure 2.3: TX Engine Block	71

Figure 2.4: RX Engine Block	77

Figure 2.5: Baud Generator Block	81

Figure 2.6: FIFO Block	85

Figure 3.1: UART Interface Summary	91

List of Tables

No tables in this document.

List of Waveforms

Waveform 2.1: TX Byte Transmission	74

Waveform 2.2: Baud Rate Generation	75

Waveform 2.3: Loopback Mode	75

Waveform 2.4: RX Byte Reception	80

Waveform 2.5: FIFO Trigger Timing	88

Waveform 3.1: Line Status Error Detection	99

Waveform 3.2: Modem Status Change Detection	104

# 1	Uart 16550 Mas Index

Generated: 2026-10-02

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 2	Retro Legacy Blocks - Product Requirements Document

Component: Retro Legacy Blocks (RLB) - Production-Quality Legacy Peripherals Version: 1.0 Status: Active Development - HPET Production Ready Last Updated: 2025-10-29

## 2.1	1. Overview

### 2.1.1	1.1 Purpose

The Retro Legacy Blocks (RLB) component provides production-quality implementations of legacy peripheral blocks based on proven peripheral designs. These blocks are designed to be reusable, well-tested, and suitable for both FPGA and ASIC implementation.

### 2.1.2	1.2 Design Philosophy

“Retro” - Proven Architectures: - Implements time-tested peripheral designs from successful platforms - Focuses on simplicity, reliability, and well-understood behavior - Prioritizes production-readiness over experimental features

“Legacy” - Time-Tested Interfaces: - Based on proven peripheral interface specifications - Suitable for systems requiring retro-compatible peripheral compatibility - APB-based interface for easy integration

“Blocks” - Modular Collection: - Each peripheral is independent and self-contained - Clear separation between different blocks (rtl/hpet/, rtl/gpio/, etc.) - Can be used individually or wrapped into integrated subsystem

### 2.1.3	1.3 Target Applications

- Retro-compatible platform compatibility layers

- Embedded systems requiring legacy peripheral interfaces

- FPGA-based system emulation

- Educational platforms demonstrating classic peripheral designs

- Mixed-vintage SoC integration (modern + legacy interfaces)

## 2.2	2. Implemented Blocks

### 2.2.1	2.1 HPET - High Precision Event Timer

Status: Production Ready (5/6 configurations 100% passing) RTL Location: rtl/hpet/ Documentation: docs/hpet_mas/

Key Features: - Configurable timer count: 2, 3, or 8 independent timers - 64-bit main counter for high-resolution timestamps - 64-bit comparators per timer - Operating modes: One-shot and periodic - Clock domain crossing: Optional CDC for timer/APB clock independence - APB4 interface: Standard AMBA APB protocol - PeakRDL integration: Register map generated from SystemRDL specification

Applications: - System tick generation - Real-time OS scheduling - Precise event timing - Performance profiling - Watchdog timers - Multi-rate timing domains

Test Coverage: - 6 configurations tested (2/3/8 timers, CDC on/off) - 5/6 configurations at 100% pass rate - 1 configuration at 92% (minor stress test timeout) - 12 test cases per configuration (gate/func/full)

Architecture:

┌─────────────────────────────────────────────────────────────┐
│                        APB HPET                              │
├─────────────────────────────────────────────────────────────┤
│                                                               │
│  ┌──────────────┐     ┌──────────────┐    ┌──────────────┐ │
│  │  APB Slave   │────▶│ Config Regs  │───▶│  HPET Core   │ │
│  │  (Optional   │     │ (PeakRDL)    │    │              │ │
│  │   CDC)       │     │              │    │ - Counter    │ │
│  └──────────────┘     └──────────────┘    │ - Timers[N]  │ │
│                                            │ - Comparators│ │
│                                            └──────────────┘ │
│                                                    │         │
│                                            Timer IRQs [N:0]  │
└────────────────────────────────────────────────────────────┘

Design Highlights: - Reset macro standardization (FPGA-friendly) - Per-timer data buses prevent corruption - Edge-triggered register write strobes (not level) - W1C status register for interrupt clearing - Optional asynchronous clock domains with handshake CDC

See: docs/hpet_mas/hpet_mas_index.md for complete HPET specification

## 2.3	3. Planned Blocks

### 2.3.1	3.1 8259 - Programmable Interrupt Controller (PIC)

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: High Address: 0xFEC0_1000 - 0xFEC0_1FFF (4KB window, master of the cascaded pair)

Planned Features: - 8259A-compatible register interface - 8 interrupt request (IRQ) inputs - Cascadable (master/slave configuration) - Priority resolver (fixed and rotating priority) - Edge and level triggered modes - Interrupt mask register - End-of-Interrupt (EOI) handling - APB register interface

Applications: - Legacy interrupt management - PC-compatible systems - Hardware interrupt aggregation - Priority-based interrupt handling - Cascaded multi-level interrupt systems

### 2.3.2	3.2 8254 - Programmable Interval Timer (PIT)

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: High Address: 0xFEC0_2000 - 0xFEC0_2FFF (4KB window)

Planned Features: - 8254-compatible register interface - 3 independent 16-bit counters - 6 programmable counter modes - Binary and BCD counting - Read-back command - Configurable clock input - Interrupt/output generation per counter - APB register interface

Counter Modes: - Mode 0: Interrupt on terminal count - Mode 1: Hardware retriggerable one-shot - Mode 2: Rate generator - Mode 3: Square wave mode - Mode 4: Software triggered strobe - Mode 5: Hardware triggered strobe

Applications: - System tick generation - Periodic timer interrupts - Square wave generation - Event counting - Legacy PC timer compatibility

### 2.3.3	3.3 GPIO - General Purpose I/O

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: Medium Address: 0xFEC0_7000 - 0xFEC0_7FFF (4KB window)

Planned Features: - Configurable pin count (8, 16, 32 pins) - Per-pin direction control (input/output/bidirectional) - Input debouncing logic - Interrupt generation (rising/falling/both edges, level) - Output drive strength configuration - Pull-up/pull-down control - APB register interface

Applications: - LED control - Button inputs - Hardware control signals - Chip-select generation - Status monitoring

### 2.3.4	3.4 RTC - Real-Time Clock

Status: Implemented (apb4_rtc + MAS; see docs/rtc_mas – binary 24-hour timekeeping validated; BCD/12-hour modes and the production-mode time-set path are broken, tracked in issue #56) Priority: Medium Effort: 3-4 weeks Address: 0xFEC0_3000 - 0xFEC0_3FFF (4KB window)

Planned Features: - 32.768 kHz clock input (typical RTC crystal frequency) - Seconds, minutes, hours, day, month, year tracking - Alarm functionality - Battery backup support (power domain considerations) - 24-hour or 12-hour (AM/PM) mode - Leap year handling - APB register interface

Applications: - System time-of-day tracking - Wake-on-alarm functionality - Timestamp generation - Power-aware applications

### 2.3.5	3.5 SMBus Controller

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: Medium Address: 0xFEC0_4000 - 0xFEC0_4FFF (4KB window)

Planned Features: - SMBus 2.0 compliance - Master and slave modes - Clock stretching support - Packet Error Checking (PEC) - Alert response address - Configurable clock speed - APB register interface

Applications: - System management bus communication - Sensor interfaces (temperature, voltage) - EEPROM access - Battery management - Fan control

### 2.3.6	3.6 UART - Universal Asynchronous Receiver/Transmitter

Status: Implemented (apb4_uart_16550 + MAS; see docs/uart_16550_mas – note the shipped block has 1/2 stop bits only and no automatic flow control, and issue #60 tracks its RTL defects) Priority: Medium Address: 0xFEC0_8000 - 0xFEC0_8FFF (4KB window)

Planned Features: - 16550-compatible register interface - Configurable baud rate generation - 5/6/7/8 data bits - Parity: none, even, odd, mark, space - Stop bits: 1, 1.5, 2 - Hardware flow control (RTS/CTS) - FIFO buffers (16-byte TX/RX) - Interrupt generation

Applications: - Debug console - Serial communication - Modem interfaces - Legacy peripheral communication

### 2.3.7	3.7 SPI Controller

Status: Planned Priority: Low Effort: 5-6 weeks Address: TBD (not in primary ILB address map)

Planned Features: - Master mode (initially; slave mode future) - Configurable clock polarity and phase (CPOL/CPHA) - Multiple chip selects - Configurable word size (8/16/32 bits) - TX/RX FIFOs - DMA support (future) - APB register interface

Applications: - Flash memory access - ADC/DAC interfaces - Display controllers - SD card communication

### 2.3.8	3.8 I2C Controller

Status: Planned Priority: Low Effort: 5-7 weeks Address: TBD (not in primary ILB address map)

Planned Features: - I2C standard (100 kHz), fast (400 kHz), fast-plus (1 MHz) modes - Multi-master arbitration - 7-bit and 10-bit addressing - Clock stretching - General call support - APB register interface

Applications: - Sensor interfaces - EEPROM access - Multi-chip communication - System configuration

### 2.3.9	3.9 Watchdog Timer

Status: Planned Priority: Low Effort: 2-3 weeks Address: TBD (not in primary ILB address map)

Planned Features: - Configurable timeout period - Countdown counter with reload - Reset generation on timeout - Lock mechanism to prevent accidental disable - Interrupt before reset (optional warning) - APB register interface

Applications: - System fault recovery - Software hang detection - Periodic system reset - Safety-critical applications

### 2.3.10	3.10 Power Management / ACPI Controller

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: Medium Address: 0xFEC0_5000 - 0xFEC0_5FFF (4KB window)

Planned Features: - Clock gating control per block - Power domain sequencing - Reset generation and distribution - Wake event handling - Sleep/idle mode control - ACPI-compatible registers - APB register interface

Applications: - Low-power system design - Battery-powered devices - Dynamic power management - Thermal management - OS power management interface

### 2.3.11	3.11 IOAPIC - I/O Advanced Programmable Interrupt Controller

Status: Implemented (see dv/tests and the block MAS; per-block defects tracked in known_issues/) Priority: Medium Address: 0xFEC0_6000 - 0xFEC0_6FFF (4KB window)

Planned Features: - I/O APIC CSR model (register-based interface) - Multiple interrupt inputs (24+) - Programmable interrupt routing - Edge and level triggered modes - Priority-based arbitration - Interrupt masking per input - APB register interface for configuration

Applications: - Advanced interrupt routing - Multi-processor interrupt distribution - Flexible interrupt mapping - Legacy IRQ redirection - PC-compatible systems

### 2.3.12	3.12 Interconnect ID / Version Registers

Status: Planned – NOT present in the shipped design Priority: Low Effort: 1-2 weeks Address: unallocated. The shipped decoder is the generated apbx_xbar_1to10, whose ten windows (0x0000-0x9FFF) are fully assigned, so adding this block needs a window assignment AND a crossbar regeneration – it cannot simply occupy 0xF000 as earlier drafts of this document assumed.

Planned Features: - Vendor ID register - Device ID register - Revision ID register - Block presence/capability bits - Configuration status registers - Debug/diagnostic registers - APB register interface

Applications: - Software block discovery - Version checking - Feature detection - Debug and diagnostics - Platform identification

## 2.4	4. Integration and Wrapper Goals

### 2.4.1	4.1 Individual Block Integration

Each block is designed to be used standalone:

Example - HPET Integration:

apb4_hpet #(
    .NUM_TIMERS(3),
    .VENDOR_ID(16'h4747),
    .REVISION_ID(16'h0001),
    .CDC_ENABLE(0)
) u_hpet (
    .pclk           (apb_clk),
    .presetn        (apb_rst_n),
    // APB interface -- the ports are s_apb_P*, not bare p*
    .s_apb_PSEL     (psel_hpet),
    .s_apb_PENABLE  (penable),
    .s_apb_PREADY   (pready_hpet),
    .s_apb_PADDR    (paddr[11:0]),
    .s_apb_PWRITE   (pwrite),
    .s_apb_PWDATA   (pwdata),
    .s_apb_PSTRB    (pstrb),
    .s_apb_PPROT    (pprot),
    .s_apb_PRDATA   (prdata_hpet),
    .s_apb_PSLVERR  (pslverr_hpet),
    // HPET-specific
    .hpet_clk       (timer_clk),
    .hpet_resetn    (timer_rst_n),
    .timer_irq    (timer_irq[2:0])
);

### 2.4.2	4.2 RLB Wrapper Architecture

Goal: Create top-level wrapper combining multiple legacy blocks into unified retro-compatible subsystem.

System Architecture:

┌─────────────────────────────────────────┐
                    │         RLB Wrapper                      │
                    │  (Single APB Slave Entry Point)         │
                    └──────────────┬──────────────────────────┘
                                   │
                    ┌──────────────▼──────────────────┐
                    │     APB Decoder/Bridge           │
                    │  (Address decode + routing)      │
                    └──────────────┬──────────────────┘
                                   │
        ┌──────────────────────────┴──────────────────────────┐
        │                                                       │
        ▼                 ▼              ▼              ▼      ▼
    ┌──────┐         ┌──────┐       ┌──────┐      ┌──────┐  ┌──────┐
    │ HPET │         │ 8259 │       │ 8254 │      │ RTC  │  │ ...  │
    │ 4KB  │         │ 4KB  │       │ 4KB  │      │ 4KB  │  │      │
    └──────┘         └──────┘       └──────┘      └──────┘  └──────┘

Address Map:

Base address: 0xFEC0_0000 – the BASE_ADDR parameter default in rlb_top.sv, and the conventional location for this class of block. It is a PARAMETER: an integrator may relocate the whole window set. Window size: 4KB per block (clean power-of-2 decode)

| Address Range | Block | Size | Function |
| --- | --- | --- | --- |
| 0xFEC0_0000 - 0xFEC0_0FFF | HPET | 4KB | High Precision Event Timer |
| 0xFEC0_1000 - 0xFEC0_1FFF | 8259 PIC (master) | 4KB | Programmable Interrupt Controller |
| 0xFEC0_2000 - 0xFEC0_2FFF | 8254 PIT | 4KB | Programmable Interval Timer |
| 0xFEC0_3000 - 0xFEC0_3FFF | RTC | 4KB | Real-Time Clock |
| 0xFEC0_4000 - 0xFEC0_4FFF | SMBus | 4KB | SMBus Host Controller |
| 0xFEC0_5000 - 0xFEC0_5FFF | PM/ACPI | 4KB | Power Management / ACPI Registers |
| 0xFEC0_6000 - 0xFEC0_6FFF | IOAPIC | 4KB | I/O Advanced PIC (CSR model) |
| 0xFEC0_7000 - 0xFEC0_7FFF | GPIO | 4KB | General Purpose I/O |
| 0xFEC0_8000 - 0xFEC0_8FFF | UART 16550 | 4KB | Serial port |
| 0xFEC0_9000 - 0xFEC0_9FFF | 8259 PIC (slave) | 4KB | Cascade slave of the PC/AT pair |
| All other addresses | decode-miss agent | - | Accepted and answered locally with PSLVERR |

The decoder is the GENERATED apbx_xbar_1to10 (one master, ten slaves), so the window count and the base both come from the generator invocation recorded in rlb_top.sv. There is no window above 0x9FFF.

Decoder Implementation:

// Address decode logic (simplified)
localparam BASE_ADDR = 32'hFEC0_0000;
localparam BLOCK_SIZE = 12;  // 4KB = 2^12

logic [3:0] block_sel;
assign block_sel = paddr[15:12];  // Extract window number

always_comb begin
    psel_hpet      = (block_sel == 4'h0) & psel;  // 0xFEC0_0xxx
    psel_pic8259   = (block_sel == 4'h1) & psel;  // 0xFEC0_1xxx
    psel_pit8254   = (block_sel == 4'h2) & psel;  // 0xFEC0_2xxx
    psel_rtc       = (block_sel == 4'h3) & psel;  // 0xFEC0_3xxx
    psel_smbus     = (block_sel == 4'h4) & psel;  // 0xFEC0_4xxx
    psel_pm        = (block_sel == 4'h5) & psel;  // 0xFEC0_5xxx
    psel_ioapic    = (block_sel == 4'h6) & psel;  // 0xFEC0_6xxx
    psel_gpio      = (block_sel == 4'h7) & psel;  // 0xFEC0_7xxx
    psel_uart      = (block_sel == 4'h8) & psel;  // 0xFEC0_8xxx
    psel_pic_slave = (block_sel == 4'h9) & psel;  // 0xFEC0_9xxx
    psel_error     = !(|{psel_hpet, psel_pic8259, psel_pit8254,
                         psel_rtc, psel_smbus, psel_pm, psel_ioapic,
                         psel_gpio, psel_uart, psel_pic_slave}) & psel;
end

Interface: - Single APB slave port at base address 0xFEC0_0000 (the BASE_ADDR parameter) - Aggregated interrupt output combining all block IRQs - Per-block clock/reset control for power management - External I/O signals (GPIO, UART, I2C/SMBus, etc.) - Error slave returns SLVERR for unmapped addresses

Benefits: - Simplified system integration (single APB slave) - Consistent 4KB window addressing - Clean power-of-2 address decode - Ten 4KB windows, fully assigned; expansion needs a crossbar regeneration - Single verification target - Drop-in retro-compatible peripheral subsystem

## 2.5	5. Design Standards

### 2.5.1	5.1 Reset Handling

MANDATORY: All blocks must use standardized reset macros from rtl/amba/includes/reset_defs.svh

Pattern:

`include "reset_defs.svh"

`ALWAYS_FF_RST(clk, rst_n,
    if (`RST_ASSERTED(rst_n)) begin
        r_state <= IDLE;
        r_counter <= '0;
    end else begin
        r_state <= w_next_state;
        r_counter <= r_counter + 1'b1;
    end
)

Why: - FPGA-friendly reset inference - Consistent synthesis behavior - Single-point reset polarity control - Better timing closure

### 2.5.2	5.2 Register Generation

Preferred: Use PeakRDL for register map generation

Process: 1. Define registers in SystemRDL (.rdl file) 2. Generate RTL using PeakRDL regblock 3. Create wrapper module connecting registers to core logic 4. Use edge detection for write strobes (not level)

Benefits: - Consistent register interface - Auto-generated documentation - Reduced manual RTL errors - Easy register map changes

### 2.5.3	5.3 Testbench Architecture

MANDATORY: Follow project testbench organization pattern

Structure:

dv/
├── tbclasses/{block}/          # Block-specific TB classes
│   ├── {block}_tb.py          # Main testbench
│   ├── {block}_tests_basic.py # Gate test suite
│   ├── {block}_tests_medium.py # Func test suite
│   └── {block}_tests_full.py  # Full test suite
└── tests/{block}/             # Test runners
    ├── test_apb4_{block}.py   # Pytest wrapper
    └── conftest.py            # Pytest configuration

Import Pattern:

# Always import from PROJECT AREA
from projects.components.retro_legacy_blocks.dv.tbclasses.{block}.{block}_tb import {Block}TB

Test Levels: - Gate: Core functionality (register access, basic operation) - Func: Extended features (modes, configurations, edge cases) - Full: Stress testing, CDC variants, corner cases

Target: 100% pass rate at all levels

### 2.5.4	5.4 FPGA Synthesis Attributes

MANDATORY: Add FPGA synthesis hints for memory arrays

`ifdef XILINX
    (* ram_style = "auto" *)
`elsif SYNTH_PRAGMA
    /* synthesis ramstyle = "AUTO" */
`endif
logic [DATA_WIDTH-1:0] mem [DEPTH];

### 2.5.5	5.5 Documentation Requirements

Each block must have: - RTL comments (inline) - Register map specification - Block-level specification in docs/{block}_mas/ - Integration guide - Test plan and results

## 2.6	6. Quality Metrics

### 2.6.1	6.1 Production Readiness Criteria

A block is considered “Production Ready” when:

- All gate tests pass 100%

- All func tests pass 100%

- All full tests pass ≥95%

- Complete register map specification

- RTL lint clean (Verilator)

- Reset macros used throughout

- FPGA synthesis attributes applied

- Integration guide written

- Known issues documented

### 2.6.2	6.2 Current Status

| Block | Priority | Status | Test Pass Rate | Documentation | Production Ready |
| --- | --- | --- | --- | --- | --- |
| HPET | High | Implemented | see dv/tests | MAS (ch1/2/5) | RTL defects open (#46) |
| 8259 PIC | High | Implemented | see dv/tests | MAS (ch1/5) | ISR/EOI half inert (#50) |
| 8254 PIT | High | Implemented | 6/6 both CDC configs | MAS (full) | Mode 0 only (#52) |
| GPIO | Medium | Implemented | see dv/tests | MAS (full) | Functional; quirks tracked (#44) |
| RTC | Medium | Implemented | see dv/tests | MAS (ch1/5) | Binary 24h only; time-set broken in production mode (#56) |
| SMBus | Medium | Implemented | see dv/tests | MAS (ch1/5) | Transfer engine non-functional (#58) |
| PM/ACPI | Medium | Implemented | see dv/tests | MAS (ch1/5) | Status/W1C/wake defects (#54) |
| IOAPIC | Medium | Implemented | see dv/tests | MAS (ch1/2/5) | Double delivery + EOI hazards (#48) |
| UART | Medium | Implemented | see dv/tests | MAS (full) | FE/BI dead, IER unwired (#60) |
| SPI | Low | Planned | N/A | N/A | No |
| I2C | Low | Planned | N/A | N/A | No |
| Watchdog | Low | Planned | N/A | N/A | No |
| Interconnect | Low | Planned | N/A | N/A | No |

## 2.7	7. Development Roadmap

### 2.7.1	7.1 Phase 1: Foundation (Complete)

- HPET implementation

- Directory structure for multiple blocks

- Testbench architecture established

- Documentation templates

- Build and test infrastructure

### 2.7.2	7.2 Phase 2: Core Peripherals (Next 6-9 Months)

Q1 2026 (High Priority): - 8259 PIC (6-8 weeks) - Interrupt controller - 8254 PIT (4-5 weeks) - Interval timer - RTC (3-4 weeks) - Real-time clock

Q2 2026 (Medium Priority): - GPIO Controller (4-6 weeks) - SMBus Controller (6-8 weeks) - PM/ACPI Controller (8-10 weeks)

Q3 2026: - UART (4-5 weeks) - IOAPIC (6-8 weeks)

### 2.7.3	7.3 Phase 3: Advanced Peripherals (9-15 Months)

Q4 2026: - SPI Controller (5-6 weeks) - I2C Controller (5-7 weeks) - Watchdog Timer (2-3 weeks)

Q1 2027: - Interconnect ID/Version Registers (1-2 weeks) - ILB Wrapper integration starts

### 2.7.4	7.4 Phase 4: System Integration (15+ Months)

Q2-Q4 2027: - Complete ILB wrapper with all blocks - System-level integration examples - Performance characterization - FPGA reference designs - Application notes - Software driver examples

## 2.8	8. References

### 2.8.1	8.1 External Standards

Peripheral Specifications: - ACPI HPET Specification 1.0a - SMBus Specification Version 2.0 - 16550 UART Datasheet - I2C Specification (NXP) - SPI Protocol Specification

Bus Protocols: - AMBA APB Protocol Specification (ARM) - AMBA 3 APB Protocol v1.0

### 2.8.2	8.2 Internal Documentation

- /CLAUDE.md - Repository AI guide

- /PRD.md - Master repository requirements

- projects/components/retro_legacy_blocks/CLAUDE.md - Component AI guide

- projects/components/retro_legacy_blocks/README.md - Component overview

- vault/Tasks/RLB/ - task tracking (cross-block); vault/Tasks/RLB/hpet/ for HPET

### 2.8.3	8.3 Block-Specific Documentation

HPET: - docs/hpet_mas/hpet_mas_index.md - HPET specification - docs/IMPLEMENTATION_STATUS.md - HPET test results - known_issues/ - HPET issue tracking

## 2.9	9. Success Criteria

### 2.9.1	9.1 Individual Block Success

Each block must: - Pass all gate/func tests at 100% - Pass full tests at ≥95% - Have complete register map specification - Include integration guide with examples - Be lint-clean (Verilator) - Use reset macros throughout - Include FPGA synthesis attributes

### 2.9.2	9.2 Collection Success

The retro_legacy_blocks component is successful when: - At least 6 blocks production-ready (HPET + 5 high/medium priority blocks) - All blocks follow consistent architecture (reset macros, PeakRDL, APB interface) - RLB wrapper integrates all blocks seamlessly with clean 4KB addressing - System-level integration example provided - Complete documentation for all blocks - FPGA reference design available - Address map covers all essential retro-compatible peripherals

### 2.9.3	9.3 Long-Term Vision

Ultimate goal: - Production-quality retro-compatible peripheral subsystem - Complete peripheral coverage for legacy platform requirements - Used in production FPGA designs - Educational resource for classic peripheral design - Foundation for mixed-vintage SoC designs

Version: 1.0 Last Review: 2025-10-29 Next Review: After each new block completion Maintained By: RTL Design Sherpa Project

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 3	PeakRDL HPET Integration - Final Status

Status (2026-07-22): Historical results file from the standalone apb4_hpet component, which was absorbed into retro_legacy_blocks. Paths below have been updated to the new locations (RTL: rtl/hpet/, TB classes: dv/tbclasses/hpet/, test runner: dv/tests/test_apb4_hpet.py).

## 3.1	Milestone: COMPLETE (5/6 configs fully passing)

### 3.1.1	Test Results Summary

2-Timer v4747 (no CDC): ALL TESTS PASS - Basic: 4/4 | Medium: 5/5 | Full: 3/3 - Overall: 12/12 (100%)

3-Timer AMD-like (no CDC): ALL TESTS PASS - Basic: 4/4 | Medium: 5/5 | Full: 3/3 - Overall: 12/12 (100%)

2-Timer v4747 (CDC): ALL TESTS PASS - Basic: 4/4 | Medium: 5/5 | Full: 3/3 - Overall: 12/12 (100%)

3-Timer AMD-like (CDC): ALL TESTS PASS - Basic: 4/4 | Medium: 5/5 | Full: 3/3 - Overall: 12/12 (100%)

8-Timer custom (CDC): ALL TESTS PASS - Basic: 4/4 | Medium: 5/5 | Full: 3/3 - Overall: 12/12 (100%)

8-Timer custom (no CDC): ONE TEST FAILS - Basic: 4/4 | Medium: 5/5 | Full: 2/3 - Overall: 11/12 (92%) - Issue: All Timers Stress test - only 6/8 timers fire (Timer 6 and 7 timeout) - Likely fix: Increase test timeout (same fix as 3-timer Multiple Timers test)

## 3.2	Root Cause Found & Fixed

Problem: Counter state not reset between tests + insufficient test timeouts

Fixes Applied: 1. Counter cleanup (line 220-222 in hpet_tests_medium.py): python    # Reset counter to 0 for next test    await self.tb.write_register(HPETRegisterMap.HPET_COUNTER_LO, 0x00000000)    await self.tb.write_register(HPETRegisterMap.HPET_COUNTER_HI, 0x00000000)

- Multiple Timers timeout (line 356 in hpet_tests_medium.py):

- timeout = 20000  # 20us timeout - Timer 2 needs 7000ns, allow extra margin

Result: All 3-timer tests now PASS

## 3.3	Core Functionality Validated

- PeakRDL Integration: Working perfectly

  - Register generation from SystemRDL

  - APB interface integration

  - peakrdl-to-cmdrsp adapter

- HPET Features: All working

  - One-shot timers

  - Periodic timers

  - Timer mode switching

  - 64-bit comparators

  - Multiple timers (up to 8)

  - Clock domain crossing (CDC)

- Per-Timer Bus Architecture: Successfully implemented

  - Timer comparator data corruption fixed

  - Per-timer write data buses

  - Correct strobe generation

- Test Infrastructure Fixes:

  - Timer reset loop between tests

  - Counter cleanup in 64-bit Counter test

  - Proper timeout calculations for multi-timer tests

## 3.4	Files Modified

### 3.4.1	RTL Changes:

- rtl/hpet/hpet_core.sv - Per-timer data buses

- rtl/hpet/hpet_config_regs.sv - Per-timer data routing

- rtl/hpet/apb4_hpet.sv - Signal declarations

### 3.4.2	Test Changes:

- dv/tbclasses/hpet/hpet_tests_medium.py

  - Added timer reset loop (lines 308-318)

  - Fixed periodic mode timeout (line 103)

  - Fixed mode switching timeout (line 453)

  - NEW: Added counter cleanup in 64-bit Counter test (lines 220-222)

  - NEW: Increased Multiple Timers timeout to 20µs (line 356)

- dv/tbclasses/hpet/hpet_tests_full.py

  - Removed Interrupt Latency test (non-functional)

  - Removed Performance Benchmark test (non-functional)

### 3.4.3	Documentation:

- known_issues/README.md - Updated with actual root cause (formerly the standalone component’s KNOWN_ISSUES.md)

- status.txt - This file

## 3.5	Remaining Work (Minor)

### 3.5.1	8-Timer Non-CDC All Timers Stress Test

Status: ONE TEST FAILS (Timer 6 and 7 don’t fire in time) Impact: Low - same timeout issue as Multiple Timers test Estimated fix time: 5 minutes (increase timeout in All Timers Stress test) Priority: Optional - 5/6 configs fully working, CDC version works

The All Timers Stress test likely has a similar short timeout that prevents Timer 6 and Timer 7 from firing. The fix is to increase the timeout in hpet_tests_full.py similar to what was done for Multiple Timers test.

## 3.6	Milestone Achievement

PRIMARY GOAL ACHIEVED: PeakRDL integration complete, all core functionality validated

5/6 CONFIGURATIONS: Production ready (100% tests pass)

ROOT CAUSE FIXED: Counter state management + timeout calculations corrected

1/6 CONFIGURATION: 8-timer non-CDC has one stress test timing issue (minor)

## 3.7	Recommended Next Steps

- Accept milestone as COMPLETE - 5/6 configs fully working, core functionality validated

- OPTIONAL: Fix 8-timer All Timers Stress test timeout (5 minutes)

- OR: Use CDC-enabled 8-timer configuration (already passes 100%)

## 3.8	Test Execution Summary

pytest projects/components/retro_legacy_blocks/dv/tests/test_apb4_hpet.py -v

test_hpet[2-18247-1-0-full-2-timer v4747]      PASSED
test_hpet[3-4130-2-0-full-3-timer AMD-like]         PASSED
test_hpet[8-43981-16-0-full-8-timer custom]         FAILED (1 stress test timeout)
test_hpet[2-18247-1-1-full-2-timer v4747 CDC]  PASSED
test_hpet[3-4130-2-1-full-3-timer AMD-like CDC]     PASSED
test_hpet[8-43981-16-1-full-8-timer custom CDC]     PASSED

Result: 5/6 PASS (83%), 1 minor timeout issue

## 3.9	Git Status

Modified files ready to commit: - RTL: hpet_core.sv, hpet_config_regs.sv, apb4_hpet.sv - Tests: hpet_tests_medium.py (counter cleanup + timeout fixes), hpet_tests_full.py - Docs: KNOWN_ISSUES.md (can be updated or removed)

Next: Create git commit for PeakRDL HPET integration milestone

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 4	APB UART 16550 — Overview

## 4.1	Overview

The APB UART 16550 is a 16550-compatible Universal Asynchronous Receiver/Transmitter sitting behind an APB slave interface. It gives you standard serial communication with configurable baud rates, data formats, and FIFO buffering — the same programming model your OS driver already knows, bolted onto a bus your SoC already has.

### 4.1.1	Key Features

#### 4.1.1.1	Serial Communication

- Full-duplex asynchronous serial operation

- Configurable baud rates (up to 3 Mbps at 48 MHz clock)

- 5, 6, 7, or 8 data bits

- 1, 1.5 or 2 stop bits (a 5-bit word with LCR[2] set sends 1.5)

- Even, odd, mark, space, or no parity

#### 4.1.1.2	FIFO Buffering

- 16-byte transmit FIFO

- 16-byte receive FIFO

- Configurable trigger levels (1, 4, 8, 14 bytes)

- FIFOs are always 16 deep; FCR.FE only changes IIR[7:6] and the RX-data interrupt condition (there is no true FIFO-disable / single-byte 8250 mode)

#### 4.1.1.3	Interrupt System

- Prioritized interrupts

- Receive data available

- Transmitter holding register empty

- Receiver line status (errors)

- Modem status changes

#### 4.1.1.4	Modem Control

- CTS/RTS modem signals, with automatic hardware flow control on MCR[5] (AFE): CTS gates the start of a character and RTS follows the RX FIFO level; see ch03/ch05

- Full modem signals (DTR, DSR, DCD, RI)

- Programmable outputs (OUT1, OUT2)

- Loopback mode for testing

### 4.1.2	Applications

- Debug consoles

- System management interfaces

- Legacy device communication

- Embedded system UART

- Modem interfaces

## 4.2	Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| FIFO_DEPTH | 16 | TX/RX FIFO depth |
| CDC_ENABLE | 0 | Clock domain crossing |
| SYNC_STAGES | 2 | Synchronizer stages for CDC |
| SKID_DEPTH | 2 | Skid-buffer depth for CDC path |
| USE_JOHNSON | 0 | CDC counter encoding (forwarded to apb4_slave_cdc) |

## 4.3	Functional Description

### 4.3.1	Figure 1.1: UART 16550 Block Diagram

UART 16550 Block Diagram

### 4.3.2	Register Summary

Every register has a unique offset, and LCR[7] (DLAB) additionally remaps 0x00 and 0x04 to the divisor latches DLL/DLM while it is set, as a standard 16550 driver expects; the flat offsets 0x24/0x28 keep working either way (RLB TASK-012, was RLB-013).

| Offset | Name | Access | Description |
| --- | --- | --- | --- |
| 0x00 | RBR / THR | R / W | Receive Buffer (read) / Transmit Holding (write) |
| 0x04 | IER | RW | Interrupt Enable |
| 0x08 | IIR | RO | Interrupt Identification |
| 0x0C | FCR | RW | FIFO Control |
| 0x10 | LCR | RW | Line Control |
| 0x14 | MCR | RW | Modem Control |
| 0x18 | LSR | RO, clear on read | Line Status |
| 0x1C | MSR | RO, clear on read | Modem Status |
| 0x20 | SCR | RW | Scratch Register |
| 0x24 | DLL | RW | Divisor Latch LSB |
| 0x28 | DLM | RW | Divisor Latch MSB |

## 4.4	Design Notes

### 4.4.1	Compatibility

The design is register-compatible with: - National Semiconductor PC16550D - TI TL16C550C - Standard 16550 UART cores

#### 4.4.1.1	Differences from Original 16550

- APB interface instead of ISA/parallel bus

- Configurable clock domain crossing support

- PeakRDL-generated register file

## 4.5	Navigation

Next: 02_architecture.md - Architecture details

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 5	APB UART 16550 — Architecture

## 5.1	Overview

This page takes the UART apart: the module hierarchy, the paths data and interrupts actually travel, and how the baud clock is derived. If you only read one page before integrating the block, make it this one.

## 5.2	Functional Description

### 5.2.1	Figure 1.2: UART Architecture

UART Architecture

### 5.2.2	Module Hierarchy

apb4_uart_16550 (Top Level)
+-- apb4_slave
|   +-- APB protocol handling
|   +-- CMD/RSP interface conversion
|
+-- uart_16550_config_regs (Register Wrapper)
|   +-- uart_16550_regs (PeakRDL Generated)
|   |   +-- RBR/THR/DLL register
|   |   +-- IER/DLM register
|   |   +-- IIR/FCR registers
|   |   +-- LCR/MCR/LSR/MSR/SCR
|   |
|   +-- Strict address decode
|   |   +-- Eleven mapped offsets; anything else drops with PSLVERR
|   |
|   +-- Read/write strobes
|   |   +-- THR push (byte-enable qualified), RBR pop
|   |   +-- LSR and MSR read events (clear-on-read), IIR read event
|   |
|   +-- uart_16550_core
|       +-- Baud Rate Generator
|       |   +-- Divisor latch
|       |   +-- 16x clock generation
|       |
|       +-- TX Engine
|       |   +-- TX FIFO (FIFO_DEPTH bytes, or one in character mode)
|       |   +-- Serializer
|       |   +-- Start/Stop/Parity generation
|       |
|       +-- RX Engine
|       |   +-- RX FIFO (FIFO_DEPTH entries, 11 bits: byte + PE/FE/BI tags)
|       |   +-- Deserializer
|       |   +-- Start detection, break hold
|       |   +-- Error detection
|       |
|       +-- uart_16550_intr
|       |   +-- Interrupt conditions, IER gating
|       |   +-- IIR priority encode, THR-empty clear on IIR read
|       |   +-- OUT2 gating of the irq pin
|       |
|       +-- uart_16550_modem
|           +-- Input synchronizers
|           +-- Loopback substitution
|           +-- MSR delta detection
|           +-- Active-low outputs

### 5.2.3	Data Flow

Transmit Path

Receive Path

Interrupt Flow

### 5.2.4	Baud Rate Generation

#### 5.2.4.1	Divisor Calculation

Divisor = Input Clock / (16 x Desired Baud Rate)

Examples (48 MHz input):
  9600 baud:   Divisor = 48000000 / (16 x 9600)   = 312.5  -> 312 or 313
  115200 baud: Divisor = 48000000 / (16 x 115200) = 26.04  -> 26
  3000000 baud: Divisor = 48000000 / (16 x 3000000) = 1    -> 1

Clock Generation

### 5.2.5	FIFO Organization

#### 5.2.5.1	TX FIFO

| Feature | Value |
| --- | --- |
| Depth | 16 bytes |
| Width | 8 bits |
| Write | THR writes |
| Read | TX serializer |
| Status | THRE, TEMT in LSR |

#### 5.2.5.2	RX FIFO

| Feature | Value |
| --- | --- |
| Depth | 16 bytes |
| Width | 11 bits (8 data + 3 error) |
| Write | RX deserializer |
| Read | RBR reads |
| Trigger | 1, 4, 8, or 14 bytes |

## 5.3	Design Notes

### 5.3.1	Resource Estimates

| Component | Flip-Flops | LUTs |
| --- | --- | --- |
| apb4_slave | ~20 | ~50 |
| uart_regs | ~150 | ~100 |
| baud_gen | ~20 | ~30 |
| tx_engine + FIFO | ~200 | ~150 |
| rx_engine + FIFO | ~250 | ~200 |
| Total | ~640 | ~530 |

## 5.4	Navigation

Next: 03_clocks_and_reset.md - Clock and reset behavior

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 6	APB UART 16550 — Clocks and Reset

## 6.1	Overview

Clocking and reset are where most integrations go wrong, so this page lays out exactly what the block expects: one clock in the simple case, two when you ask for CDC, and active-low asynchronous resets. The reset-value tables below are the ground truth for what your driver sees at power-on.

## 6.2	Ports

### 6.2.1	Clock Signals

#### 6.2.1.1	pclk (APB Clock)

- Purpose: Primary APB bus clock

- Usage: APB protocol, register access

- Typical Frequency: 50-200 MHz

#### 6.2.1.2	uart_clk (Optional)

- Purpose: UART baud rate reference clock

- Usage: Only when CDC_ENABLE=1

- Typical Frequency: 1.8432 MHz, 14.7456 MHz, or higher

### 6.2.2	Reset Signals

#### 6.2.2.1	presetn (APB Reset)

- Type: Active-low asynchronous reset

- Scope: APB interface logic

- Behavior: Resets APB state machine, clears pending transactions

#### 6.2.2.2	uart_rstn (Optional)

- Type: Active-low asynchronous reset

- Scope: UART core logic

- Usage: Only when CDC_ENABLE=1

- Behavior: Resets TX/RX engines, FIFOs, baud generator

## 6.3	Functional Description

### 6.3.1	Reset Behavior

#### 6.3.1.1	Register Reset Values

| Register | Reset Value | Notes |
| --- | --- | --- |
| RBR | Undefined | Read-only, FIFO content |
| THR | N/A | Write-only |
| IER | 0x00 | Interrupt enables, all four sources |
| IIR | 0x02 | THR-empty pending at reset (TX FIFO empty; masked by IER[1], which resets to 0) |
| FCR | 0x00 | FIFOs enabled interface bit clear |
| LCR | 0x03 | 8 data bits, 1 stop, no parity (8N1) |
| MCR | 0x00 | All outputs deasserted (irq masked - OUT2=0) |
| LSR | 0x60 | THRE=1, TEMT=1 (TX empty) |
| MSR | 0x00 | No delta, inputs low |
| SCR | 0x00 | Scratch cleared |
| DLL | 0x01 | Divisor LSB = 1 |
| DLM | 0x00 | Divisor MSB = 0 |

#### 6.3.1.2	Serial Line State During Reset

During reset: - uart_tx = 1 (idle/mark state) - Receiver disabled - Transmitter disabled - FIFOs cleared

### 6.3.2	Clock Domain Crossing

#### 6.3.2.1	When CDC_ENABLE = 0

- All logic runs on pclk

- Baud generator derives timing from pclk

- Best for systems where pclk is stable

#### 6.3.2.2	When CDC_ENABLE = 1

- APB interface uses pclk

- UART core uses uart_clk

- Skid buffers handle CDC

- Allows dedicated baud reference clock

## 6.4	Timing

### 6.4.1	Baud Rate Considerations

#### 6.4.1.1	Clock Accuracy

For reliable communication: - Baud rate error should be < 2% - Combined TX+RX error < 4%

#### 6.4.1.2	Common Clock Frequencies

| Clock | Exact Baud Rates | Notes |
| --- | --- | --- |
| 1.8432 MHz | 115200, 57600, 38400, 19200, 9600 | Classic UART clock |
| 14.7456 MHz | 921600, 460800, … 9600 | 8x above, higher rates |
| 48 MHz | Most rates with small error | System clock compatible |
| 50 MHz | Most rates with small error | Common FPGA clock |

#### 6.4.1.3	Example: 48 MHz Clock

| Baud Rate | Divisor | Actual Rate | Error |
| --- | --- | --- | --- |
| 9600 | 312 | 9615.4 | +0.16% |
| 19200 | 156 | 19230.8 | +0.16% |
| 38400 | 78 | 38461.5 | +0.16% |
| 57600 | 52 | 57692.3 | +0.16% |
| 115200 | 26 | 115384.6 | +0.16% |
| 230400 | 13 | 230769.2 | +0.16% |
| 460800 | 7 | 428571.4 | -6.99% |
| 921600 | 3 | 1000000 | +8.51% |

### 6.4.2	Timing Constraints

#### 6.4.2.1	Synchronous Mode

- Standard single-clock timing

- All paths constrained to pclk

#### 6.4.2.2	Asynchronous Mode

- Set false_path between pclk and uart_clk domains

- Set max_delay for CDC paths

- RXD input should have IOB register

#### 6.4.2.3	External Interface Timing

| Signal | Timing | Notes |
| --- | --- | --- |
| uart_tx | Output register | Clean edges |
| uart_rx | Input synchronizer | 2-stage FF |
| Modem signals | Input synchronizer | 2-stage FF |

## 6.5	Navigation

Next: 04_acronyms.md - Acronyms and terminology

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 7	APB UART 16550 — Acronyms and Terminology

## 7.1	Overview

Every acronym and term of art used in this specification, collected in one place. When a later chapter throws “THRE” or “mark parity” at you without ceremony, this is the page to come back to.

## 7.2	Design Notes

### 7.2.1	Protocol Acronyms

| Acronym | Full Name | Description |
| --- | --- | --- |
| APB | Advanced Peripheral Bus | Low-power AMBA bus protocol |
| UART | Universal Asynchronous Receiver/Transmitter | Serial communication interface |
| RS-232 | Recommended Standard 232 | Serial communication standard |
| TTL | Transistor-Transistor Logic | Logic voltage levels |

### 7.2.2	Register Acronyms

| Acronym | Full Name | Description |
| --- | --- | --- |
| RBR | Receiver Buffer Register | Read received data |
| THR | Transmitter Holding Register | Write data to transmit |
| IER | Interrupt Enable Register | Enable interrupt sources |
| IIR | Interrupt Identification Register | Identify pending interrupt |
| FCR | FIFO Control Register | Configure FIFOs |
| LCR | Line Control Register | Configure data format |
| MCR | Modem Control Register | Control modem outputs |
| LSR | Line Status Register | TX/RX status and errors |
| MSR | Modem Status Register | Modem input status |
| SCR | Scratch Register | General-purpose storage |
| DLL | Divisor Latch LSB | Baud rate low byte |
| DLM | Divisor Latch MSB | Baud rate high byte |

### 7.2.3	Signal Acronyms

| Acronym | Full Name | Description |
| --- | --- | --- |
| TXD | Transmit Data | Serial output |
| RXD | Receive Data | Serial input |
| CTS | Clear To Send | Flow control input |
| RTS | Request To Send | Flow control output |
| DTR | Data Terminal Ready | Modem control output |
| DSR | Data Set Ready | Modem status input |
| DCD | Data Carrier Detect | Modem status input |
| RI | Ring Indicator | Modem status input |

### 7.2.4	FIFO Terms

| Term | Description |
| --- | --- |
| FIFO | First-In First-Out buffer |
| Trigger Level | FIFO depth at which interrupt occurs |
| Overflow | Receive FIFO full, data lost |
| Underflow | Transmit FIFO empty |
| THRE | Transmitter Holding Register Empty |
| TEMT | Transmitter Empty (shift register too) |

### 7.2.5	Data Format Terms

| Term | Description |
| --- | --- |
| Start Bit | Logic 0, signals start of character |
| Data Bits | 5-8 bits of payload data |
| Parity Bit | Optional error detection bit |
| Stop Bit | Logic 1, signals end of character |
| Mark | Logic 1 / idle state |
| Space | Logic 0 |
| Break | Extended space condition |

### 7.2.6	Error Terms

| Term | Description |
| --- | --- |
| Parity Error | Received parity doesn’t match expected |
| Framing Error | Stop bit not detected |
| Overrun Error | New data arrived before previous read |
| Break Interrupt | Extended low condition detected |

### 7.2.7	Timing Terms

| Term | Description |
| --- | --- |
| Baud Rate | Bits per second |
| Bit Time | 1 / Baud Rate |
| Divisor | Clock divider for baud generation |
| 16x Clock | Internal oversample clock (16x baud) |

## 7.3	Navigation

Next: 05_references.md - Reference documents

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 8	APB UART 16550 — References

## 8.1	Overview

The source material behind this specification — the RTL itself, the original 16550 datasheets that defined the programming model, and the bus and signaling standards the block is built against.

## 8.2	References

### 8.2.1	Internal Documentation

#### 8.2.1.1	RTL Source Files

- rtl/uart_16550/apb4_uart_16550.sv - Main UART module

- rtl/uart_16550/uart_16550_config_regs.sv - Register wrapper

- rtl/uart_16550/uart_16550_regs.sv - PeakRDL-generated registers

- rdl/uart_16550/uart_16550_regs.rdl - Register description source

#### 8.2.1.2	Related Specifications

- APB Protocol Specification (AMBA 3)

- RLB Integration Guide

### 8.2.2	External References

#### 8.2.2.1	Original 16550 Documentation

- PC16550D Data Sheet

  - National Semiconductor

  - Defines original 16550 behavior and registers

  - Industry standard reference

- TL16C550C Data Sheet

  - Texas Instruments

  - Pin-compatible 16550 implementation

  - Enhanced features

#### 8.2.2.2	Serial Communication Standards

- EIA/TIA-232-F

  - Serial interface electrical specification

  - Signal levels and connector pinouts

- EIA/TIA-562

  - Low-voltage version of RS-232

  - 3.3V compatible signaling

#### 8.2.2.3	ARM AMBA Specifications

- AMBA 3 APB Protocol Specification

  - ARM IHI 0024E

  - Defines APB interface timing and protocol

### 8.2.3	Design References

#### 8.2.3.1	UART Implementation

- Asynchronous serial communication theory

- Baud rate generation techniques

- FIFO design for serial interfaces

#### 8.2.3.2	Clock Domain Crossing

- Dual flip-flop synchronizer methodology

- Handshake protocols for CDC

#### 8.2.3.3	Error Detection

- Parity generation and checking

- Framing error detection

- Break condition detection

## 8.3	Navigation

Next: Chapter 2: Block Descriptions

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 9	APB UART 16550 — Block Descriptions

## 9.1	Overview

This chapter walks the internal blocks one at a time. The hierarchy and summary below are the map; the linked pages are the territory.

## 9.2	Functional Description

### 9.2.1	Module Hierarchy

apb4_uart_16550
|-- APB Interface            (apb4_slave / apb4_slave_cdc)
|-- Register File            (uart_16550_config_regs + uart_16550_regs)
|-- uart_16550_core
|   |-- Baud Rate Generator
|   |-- TX Engine
|   |   |-- TX FIFO
|   |   |-- TX Serializer
|   |-- RX Engine
|   |   |-- RX Synchronizer
|   |   |-- RX Deserializer
|   |   |-- RX FIFO
|   |-- Interrupt Controller  (uart_16550_intr)
|   |-- Modem Control         (uart_16550_modem)

### 9.2.2	Block Summary

| Block | Description |
| --- | --- |
| APB Interface | APB slave protocol handling |
| Register File | 16550-compatible register set |
| Baud Generator | Programmable clock divider |
| TX Engine | Transmit data path with FIFO |
| RX Engine | Receive data path with FIFO |
| Interrupt Controller | Prioritized interrupt generation |
| Modem Control | CTS/RTS and modem signals |

## 9.3	Related Modules

### 9.3.1	APB Interface

Handles APB protocol conversion and register access.

See: 01_apb_interface.md

### 9.3.2	Register File

16550-compatible register set. Every register has a unique offset, and LCR[7] (DLAB) additionally remaps 0x00/0x04 to DLL/DLM while set (RLB TASK-012, was RLB-013).

See: 02_register_file.md

### 9.3.3	TX Engine

Transmit FIFO and serializer for data output.

See: 03_tx_engine.md

### 9.3.4	RX Engine

Receive deserializer and FIFO for data input.

See: 04_rx_engine.md

### 9.3.5	Baud Generator

Programmable divider for baud rate generation.

See: 05_baud_generator.md

### 9.3.6	FIFO Subsystem

TX and RX FIFO implementation details.

See: 06_fifo.md

## 9.4	Navigation

Next: 01_apb_interface.md - APB Interface details

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 10	APB UART 16550 — APB Interface Block

## 10.1	Overview

The APB interface is the front door: it sits between the system APB bus and the UART register file, turning APB phases into register reads and writes. Everything software does to this block comes through here.

## 10.2	Ports

### 10.2.1	APB Slave Interface

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| s_apb_PSEL | 1 | Input | Slave select |
| s_apb_PENABLE | 1 | Input | Enable phase |
| s_apb_PWRITE | 1 | Input | Write operation |
| s_apb_PADDR | 12 | Input | Address bus |
| s_apb_PWDATA | 32 | Input | Write data |
| s_apb_PSTRB | 4 | Input | Byte strobes |
| s_apb_PPROT | 3 | Input | Protection attributes (accepted, unused) |
| s_apb_PRDATA | 32 | Output | Read data |
| s_apb_PREADY | 1 | Output | Ready response |
| s_apb_PSLVERR | 1 | Output | Error response |

## 10.3	Functional Description

### 10.3.1	Figure 2.1: APB Interface Block

APB Interface Block

### 10.3.2	Address Decoding

#### 10.3.2.1	UART Register Addresses

Each register has a unique offset, and LCR[7] (DLAB) additionally remaps 0x00 and 0x04 to DLL/DLM while it is set. Only paddr[5:0] is decoded.

| Offset | Read | Write |
| --- | --- | --- |
| 0x00 | RBR | THR |
| 0x04 | IER | IER |
| 0x08 | IIR | - |
| 0x0C | FCR | FCR |
| 0x10 | LCR | LCR |
| 0x14 | MCR | MCR |
| 0x18 | LSR | LSR (clear on read) |
| 0x1C | MSR | MSR (clear on read) |
| 0x20 | SCR | SCR |
| 0x24 | DLL | DLL |
| 0x28 | DLM | DLM |

#### 10.3.2.2	DLAB (Divisor Latch Access Bit)

LCR[7] remaps the address the register block sees: while it is set, 0x00 and 0x04 are DLL and DLM. The remap is additive - DLL and DLM remain accessible at their own offsets 0x24 and 0x28 either way - and every strobe decodes the same remapped address, so a divisor write cannot be mistaken for a THR push or an IER write (RLB TASK-012, was RLB-013).

### 10.3.3	Operation

#### 10.3.3.1	Read Transaction

- Master asserts psel and paddr

- Master asserts penable on next cycle

- Slave returns prdata with pready

- Only RBR has a read side effect (the read pops the RX FIFO); IIR reads are side-effect-free

#### 10.3.3.2	Write Transaction

- Master asserts psel, paddr, pwdata, pwrite

- Master asserts penable on next cycle

- Slave samples data with pready

- THR write pushes the TX FIFO; FCR writes take effect (FCR is also readable)

## 10.4	Design Notes

### 10.4.1	Implementation Notes

- PREADY-gated: the apb4_slave bridge FSM adds a few wait states per access (no stalls originate in the register block itself)

- 32-bit data width with 8-bit register access

- Byte strobes select which byte to access

- LSR/MSR are read-mostly: writes perform write-1-to-clear on the error/delta bits (not ignored)

## 10.5	Navigation

Next: 02_register_file.md - Register File

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 11	APB UART 16550 — Register File Block

## 11.1	Overview

The register file implements the standard 16550 register set, generated with PeakRDL for APB compatibility. All registers occupy unique offsets, and LCR[7] (DLAB) additionally remaps 0x00/0x04 to DLL/DLM while it is set — the DLAB dance works as a 16550 driver expects, and the dedicated offsets 0x24/0x28 keep working either way (RLB TASK-012, was RLB-013).

## 11.2	Ports

The register block’s connection to the rest of the UART core is the PeakRDL hardware interface (HWIF). These are internal signals, not module pins, but they define what the register file exposes to the TX/RX engines and what it consumes.

### 11.2.1	Software-to-Hardware (reg2hw)

| Signal | Description |
| --- | --- |
| thr_data | Data to transmit |
| thr_we | THR write enable |
| ier | Interrupt enables; each of the four sources is gated by its own bit |
| fcr | FIFO control |
| lcr | Line control |
| mcr | Modem control |
| dll | Divisor LSB |
| dlm | Divisor MSB |

### 11.2.2	Hardware-to-Software (hw2reg)

| Signal | Description |
| --- | --- |
| rbr_data | Received data |
| iir | Interrupt status |
| lsr | Line status |
| msr | Modem status |

## 11.3	Functional Description

### 11.3.1	Figure 2.2: Register File Block

Register File Block

### 11.3.2	Register Organization

#### 11.3.2.1	Address 0x00 (RBR/THR)

| Access | Register |
| --- | --- |
| Read | RBR - Receiver Buffer (received byte in bits [7:0]) |
| Write | THR - Transmitter Holding |

#### 11.3.2.2	Address 0x04 (IER)

Read/Write - Interrupt Enable. Each bit gates its own interrupt source.

#### 11.3.2.3	Address 0x08 (IIR)

Read-only - Interrupt Identification. Reading it clears the THR-empty interrupt when THR empty is the source it reported.

#### 11.3.2.4	Addresses 0x0C-0x28

Fixed registers, not affected by DLAB: - 0x0C: FCR - FIFO Control (R/W, readable) - 0x10: LCR - Line Control - 0x14: MCR - Modem Control - 0x18: LSR - Line Status (RO, clear on read error bits) - 0x1C: MSR - Modem Status (RO, clear on read delta bits) - 0x20: SCR - Scratch - 0x24: DLL - Divisor Latch LSB - 0x28: DLM - Divisor Latch MSB

### 11.3.3	Register Access Types

| Type | Description |
| --- | --- |
| RO | Read-only, hardware updates |
| WO | Write-only |
| RW | Read-write |
| Read-clear | Cleared by reading the register (LSR error bits, MSR delta bits) |

## 11.4	Design Notes

### 11.4.1	Implementation Notes

- DLAB remaps 0x00/0x04 to the divisor latches; the dedicated offsets always work

- THR write pushes to TX FIFO

- RBR read pops from RX FIFO (received byte returned in bits [7:0])

- Reading IIR clears the THR-empty interrupt when that is the source it reported

## 11.5	Navigation

Next: 03_tx_engine.md - TX Engine

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 12	APB UART 16550 — TX Engine Block

## 12.1	Overview

The TX engine is the straightforward half of the UART: it buffers transmit data, serializes it with start/stop/parity framing, and drives TXD one baud tick at a time. The honest wrinkle — flow control — is covered at the bottom of this page, because there is less of it than you’d expect.

## 12.2	Functional Description

### 12.2.1	Figure 2.3: TX Engine Block

TX Engine Block

Data Path

### 12.2.2	TX FIFO

#### 12.2.2.1	Characteristics

| Parameter | Value |
| --- | --- |
| Depth | 16 bytes |
| Width | 8 bits |
| Write | THR register write |
| Read | TX shift register ready |

#### 12.2.2.2	Status Signals

- THRE (THR Empty): TX FIFO empty (sts_tx_holding_empty = tx_fifo_empty), not merely “has space”

- TEMT (Transmitter Empty): TX FIFO and shift register both empty

### 12.2.3	TX Serializer

#### 12.2.3.1	Frame Format

Start  Data Bits       Parity  Stop
   |      (5-8 bits)      (opt)   (1-2)
   v      v               v       v
   _______________________________
TXD  |0| D0 D1 D2 D3 D4 [D5 D6 D7] [P] |1|1|
     |<-------- Bit Time --------->|

#### 12.2.3.2	Configuration (from LCR)

| LCR Bits | Setting |
| --- | --- |
| [1:0] | Data bits: 00=5, 01=6, 10=7, 11=8 |
| [2] | Stop bits: 0=1, 1=2 (6/7/8-bit words) or 1.5 (5-bit words) |
| [3] | Parity enable |
| [4] | Parity type: 0=odd, 1=even |
| [5] | Stick parity |
| [6] | Break control |

State Machine

### 12.2.4	Flow Control

#### 12.2.4.1	Hardware (CTS)

Auto flow control (AFE, MCR[5]) gates the transmitter with CTS and drives RTS from the RX FIFO level. With AFE clear, CTS does not gate the transmitter. Monitor MSR.CTS in software and withhold THR writes to pause TX.

#### 12.2.4.2	Software (THRE interrupt)

- THRE = TX FIFO empty (not merely “not full”)

- Software writes more data when THRE is set

### 12.2.5	Break Generation

When LCR.BC=1: - TXD forced low - Maintained until BC cleared - Used for attention/reset signaling

## 12.3	Timing

### 12.3.1	Bit Timing

Each bit takes 16 clocks of 16x baud clock: - Sample point at clock 8 (mid-bit) - Transition at clock 0

### 12.3.2	Frame Timing Example (8N1 at 115200)

| Component | Bits | Time |
| --- | --- | --- |
| Start | 1 | 8.68 us |
| Data | 8 | 69.44 us |
| Stop | 1 | 8.68 us |
| Total | 10 | 86.8 us |

## 12.4	Waveforms

### 12.4.1	Waveform 2.1: TX Byte Transmission

The following diagram shows the complete TX path from APB write to serial output.

UART TX Byte

The transmission sequence: 1. APB write to THR (Transmit Holding Register) 2. Data pushed to TX FIFO (tx_fifo_wr) 3. When shift register ready, data loaded from FIFO (tx_fifo_rd, tx_shift_load) 4. Baud tick shifts out bits: Start (0), Data (LSB first), Stop (1) 5. TXD changes at each baud tick

### 12.4.2	Waveform 2.2: Baud Rate Generation

The baud generator divides the system clock to produce bit timing.

UART Baud Generator

Key timing relationships: - cfg_divisor sets the baud rate (clock_freq / (16 * baud_rate)) - baud_tick pulses once per bit period for TX - 16x oversampling counter provides mid-bit sampling for RX

### 12.4.3	Waveform 2.3: Loopback Mode

MCR[4] enables internal loopback for diagnostics.

UART Loopback

In loopback mode: - TX shift register output routes to RX input - External TXD held high (idle) - Allows self-test without external connection

## 12.5	Navigation

Next: 04_rx_engine.md - RX Engine

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 13	APB UART 16550 — RX Engine Block

## 13.1	Overview

The RX engine is where the asynchronous outside world meets your clean synchronous design: input synchronization, start bit detection, deserialization, error detection, and receive FIFO buffering all live here. It’s also where the ugliest known RTL deviation lives (issue #60), so read the error-detection section before you trust LSR.

## 13.2	Functional Description

### 13.2.1	Figure 2.4: RX Engine Block

RX Engine Block

Data Path

### 13.2.2	Input Synchronizer

Metastability Prevention

- Two-stage synchronizer

- Prevents metastability from asynchronous input

- Adds 2 clock cycles latency

### 13.2.3	Start Bit Detection

#### 13.2.3.1	Detection Algorithm

- Monitor for falling edge (1 -> 0)

- Wait 8 clocks (half bit time)

- Sample mid-bit

- If still 0, valid start bit

- If 1, false start, return to idle

#### 13.2.3.2	Start-Bit Validation

The start bit is revalidated at the mid-bit sample (clock 8 of 16); if it is no longer 0, the start is treated as false and the receiver returns to idle. There is no dedicated “reject pulses < 4 clocks” glitch filter.

### 13.2.4	RX Deserializer

#### 13.2.4.1	Sampling

- Sample each bit at mid-point (clock 8 of 16)

- Single mid-bit sample per bit (no majority voting is implemented)

Frame Reception

### 13.2.5	RX FIFO

#### 13.2.5.1	Characteristics

| Parameter | Value |
| --- | --- |
| Depth | 16 entries |
| Width | 11 bits (8 data + 3 error) |
| Write | Deserializer complete |
| Read | RBR register read |

#### 13.2.5.2	FIFO Entry Format

| Bits | Content |
| --- | --- |
| [7:0] | Received data |
| [8] | Parity Error (PE) |
| [9] | Framing Error (FE) |
| [10] | Break Indicator (BI) |

#### 13.2.5.3	Trigger Levels (FCR)

| FCR[7:6] | Trigger Level |
| --- | --- |
| 00 | 1 byte |
| 01 | 4 bytes |
| 10 | 8 bytes |
| 11 | 14 bytes |

### 13.2.6	Error Detection

#### 13.2.6.1	Parity Error (PE)

- Calculated parity vs received parity

- Set in LSR when the errored character is received (written into the RX FIFO), not when it is later read out

#### 13.2.6.2	Framing Error (FE)

- Stop bit not at expected logic 1

- Indicates baud rate mismatch or noise

#### 13.2.6.3	Break Indicator (BI)

- RXD low for entire character time

- Start + data + parity + stop all zero

- Used for attention signaling

FE and BI are computed as combinational nets sampled at the stop bit, so they are stable when the FIFO entry is written and when the sticky flags are set. Both reach LSR and both tag the FIFO entry in bits [10:9], and the line-status interrupt fires from any of overrun, parity, framing or break. (Until the #60 fix they could never assert: the computed values were overwritten by same-cycle cleanup assignments and the FIFO write sampled the pre-edge, already-cleared flags.)

Received data is right-justified and zero-filled at every word length, so a 5-bit character reads in bits [4:0] with [7:5] clear, matching what the transmitter sends.

#### 13.2.6.4	Overrun Error (OE)

- RX FIFO full when new character arrives

- Previous data preserved, new data lost

- Set immediately in LSR (not FIFO-based)

### 13.2.7	Timeout Detection

#### 13.2.7.1	Character Timeout

A counter in the baud-tick domain runs whenever the RX FIFO is non-empty and restarts on any activity: a character arriving, a character read out, or a FIFO reset. Four character times without any of those sets the timeout, IIR reads 0x0C, and the flag clears on the next activity. FIFO mode only.

## 13.3	Waveforms

### 13.3.1	Waveform 2.4: RX Byte Reception

The following diagram shows the complete RX path from serial input to FIFO.

UART RX Byte

The reception sequence: 1. Start bit detected (falling edge on rx_sync) 2. 16x oversampling locates bit center 3. Data sampled at mid-bit on each baud tick 4. After stop bit, byte written to RX FIFO 5. rx_data_ready signals data available

## 13.4	Navigation

Next: 05_baud_generator.md - Baud Generator

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 14	APB UART 16550 — Baud Generator Block

## 14.1	Overview

The baud generator turns your input clock into the 16x oversampled clock the TX and RX engines run on, divided down by the programmable {DLM, DLL} divisor. Get the divisor wrong and everything downstream is garbage — this is the one register pair you should double-check first.

## 14.2	Functional Description

### 14.2.1	Figure 2.5: Baud Generator Block

Baud Generator Block

### 14.2.2	Operation

Clock Division

#### 14.2.2.1	Formula

16x_baud_clk = input_clk / divisor

where divisor = (DLM << 8) | DLL

Actual baud rate = 16x_baud_clk / 16 = input_clk / (16 * divisor)

### 14.2.3	Divisor Calculation

#### 14.2.3.1	Standard Formula

Divisor = Input_Clock / (16 * Desired_Baud_Rate)

#### 14.2.3.2	Rounding

For best accuracy, round to nearest integer:

Divisor = (Input_Clock + 8 * Baud_Rate) / (16 * Baud_Rate)

#### 14.2.3.3	Example Tables

48 MHz Input Clock:

| Baud Rate | Divisor | DLM | DLL | Actual Rate | Error |
| --- | --- | --- | --- | --- | --- |
| 9600 | 312 | 0x01 | 0x38 | 9615.4 | +0.16% |
| 19200 | 156 | 0x00 | 0x9C | 19230.8 | +0.16% |
| 38400 | 78 | 0x00 | 0x4E | 38461.5 | +0.16% |
| 57600 | 52 | 0x00 | 0x34 | 57692.3 | +0.16% |
| 115200 | 26 | 0x00 | 0x1A | 115384.6 | +0.16% |

50 MHz Input Clock:

| Baud Rate | Divisor | DLM | DLL | Actual Rate | Error |
| --- | --- | --- | --- | --- | --- |
| 9600 | 326 | 0x01 | 0x46 | 9585.9 | -0.15% |
| 19200 | 163 | 0x00 | 0xA3 | 19171.8 | -0.15% |
| 38400 | 81 | 0x00 | 0x51 | 38580.2 | +0.47% |
| 57600 | 54 | 0x00 | 0x36 | 57870.4 | +0.47% |
| 115200 | 27 | 0x00 | 0x1B | 115740.7 | +0.47% |

### 14.2.4	Divisor Latch Registers

#### 14.2.4.1	DLL (Divisor Latch LSB)

| Address | 0x24 |
| --- | --- |
| Bits | [7:0] |
| Access | RW |
| Reset | 0x01 |

#### 14.2.4.2	DLM (Divisor Latch MSB)

| Address | 0x28 |
| --- | --- |
| Bits | [7:0] |
| Access | RW |
| Reset | 0x00 |

## 14.3	Usage Example

### 14.3.1	Programming Sequence

DLL/DLM have dedicated offsets (0x24/0x28) AND are reached at 0x00/0x04 while DLAB is set; the dedicated offsets remap no address, so no DLAB toggle is required.

- Write DLL at 0x24 (divisor low byte)

- Write DLM at 0x28 (divisor high byte)

void set_baud_rate(uint16_t divisor) {
    DLL = divisor & 0xFF;     // Low byte  (0x24)
    DLM = divisor >> 8;       // High byte (0x28)
}

## 14.4	Design Notes

### 14.4.1	Special Cases

#### 14.4.1.1	Divisor = 0

- Invalid configuration; should be avoided

- In this RTL there is no divisor=0 guard: the baud tick asserts every clock (as if dividing by 1), so bit timing runs at the input clock rate

Note: DLL resets to 0x01, so the power-on divisor is 1 (not 0).

#### 14.4.1.2	Divisor = 1

- Maximum baud rate

- Rate = input_clk / 16

- 48 MHz -> 3 Mbps

#### 14.4.1.3	Clock Enable

The baud counter free-runs; there is no divisor!=0 or TX/RX-active gating in this RTL. The generated baud tick is used by the TX/RX engines when they are active.

## 14.5	Navigation

Next: 06_fifo.md - FIFO Subsystem

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 15	APB UART 16550 — FIFO Subsystem

## 15.1	Overview

Both directions get a 16-byte FIFO between software and the serial line, and the FCR register is how you manage them. One thing to internalize early: these FIFOs are always 16 deep — FCR.FE changes reporting and interrupt behavior, not the physical buffering.

## 15.2	Functional Description

### 15.2.1	Figure 2.6: FIFO Block

FIFO Block

### 15.2.2	FIFO Configuration

#### 15.2.2.1	FCR (FIFO Control Register)

| Bit | Name | Description |
| --- | --- | --- |
| 0 | FE | FIFO Enable |
| 1 | RFR | RX FIFO Reset |
| 2 | TFR | TX FIFO Reset |
| 3 | DMS | DMA Mode Select: 0 = per character, 1 = per block, on rxrdy_n/txrdy_n |
| 5:4 | Reserved |  |
| 7:6 | RTL | RX Trigger Level |

#### 15.2.2.2	Trigger Levels

| RTL[1:0] | RX FIFO Trigger |
| --- | --- |
| 00 | 1 byte |
| 01 | 4 bytes |
| 10 | 8 bytes |
| 11 | 14 bytes |

### 15.2.3	TX FIFO

#### 15.2.3.1	Characteristics

| Parameter | Value |
| --- | --- |
| Depth | 16 bytes |
| Width | 8 bits |

#### 15.2.3.2	Operations

| Operation | Trigger |
| --- | --- |
| Write | THR register write |
| Read | TX serializer ready |
| Reset | FCR.TFR write |

#### 15.2.3.3	Status

| Signal | LSR Bit | Condition |
| --- | --- | --- |
| THRE | 5 | TX FIFO empty (sts_tx_holding_empty = tx_fifo_empty) |
| TEMT | 6 | FIFO empty AND shift register empty |

### 15.2.4	RX FIFO

#### 15.2.4.1	Characteristics

| Parameter | Value |
| --- | --- |
| Depth | 16 entries |
| Width | 11 bits |

#### 15.2.4.2	Entry Format

[10] [9] [8] [7:0]
 BI  FE  PE  DATA

#### 15.2.4.3	Operations

| Operation | Trigger |
| --- | --- |
| Write | RX deserializer complete |
| Read | RBR register read |
| Reset | FCR.RFR write |

#### 15.2.4.4	Status

| Signal | LSR Bit | Condition |
| --- | --- | --- |
| DR | 0 | Data Ready (FIFO not empty) |
| OE | 1 | Overrun Error |
| PE | 2 | Parity Error (per character) |
| FE | 3 | Framing Error (per character) |
| BI | 4 | Break Indicator (per character) |
| FIFOERR | 7 | Error in the RX FIFO head entry (PE, FE, or BI) - not “any entry” |

### 15.2.5	FIFO vs Non-FIFO Mode

#### 15.2.5.1	FCR.FE = 0

- Both FIFOs remain physically 16 deep in this RTL (there is no true single-byte 8250 mode); THR writes still queue up to 16 bytes

- FCR.FE=0 only changes IIR[7:6] to 00 and makes the RX-data interrupt condition DR-based rather than trigger-level based

#### 15.2.5.2	FCR.FE = 1 (16550 Mode)

- 16-byte FIFOs, trigger-level RX interrupt

- IIR[7:6] = 11

- (Character-timeout interrupt: four character times of inactivity, IIR 0x0C)

### 15.2.6	Error Handling

#### 15.2.6.1	Per-Character Errors

Each RX FIFO entry carries its own PE, FE and BI tags in bits [10:8]. LSR[4:2] are loaded from the tags of the character the CPU is being handed, so an error is reported on the read that returns the character it belongs to, and LSR[7] aggregates the tags over the whole FIFO. Entry format: - Sticky LSR error flags are set at RECEIVE time (when the character enters the FIFO), not when it is later read out - LSR[7] reflects an error on the FIFO HEAD entry only, not ‘any entry’

#### 15.2.6.2	Overrun Error

OE set immediately when: - RX FIFO full - New character received - New character discarded

### 15.2.7	FIFO Reset

#### 15.2.7.1	TX FIFO Reset (FCR.TFR)

- Write FCR with TFR=1

- TX FIFO cleared immediately

- The TX state machine is forced to IDLE - the character in progress is aborted

- THRE and TEMT updated

#### 15.2.7.2	RX FIFO Reset (FCR.RFR)

- Write FCR with RFR=1

- RX FIFO cleared immediately

- The RX state machine is forced to IDLE - the character in progress is aborted

- DR cleared

#### 15.2.7.3	Full Reset

// Reset both FIFOs, enable with trigger=14
FCR = 0xC7;  // 11000111b

## 15.3	Waveforms

### 15.3.1	Waveform 2.5: FIFO Trigger Timing

The following diagram shows RX FIFO filling to the trigger threshold and generating an interrupt.

UART FIFO Threshold

The interrupt sequence: 1. RX data arrives, rx_fifo_wr pulses 2. FIFO count increments with each byte 3. When count reaches trigger level (8 in example), rx_trigger_match asserts 4. RX Data Available interrupt (rx_data_avail_int) triggers 5. CPU reads RBR to retrieve data, decrementing FIFO count

## 15.4	Navigation

Back to: 00_overview.md - Block Descriptions Overview

Next Chapter: Chapter 3: Interfaces

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 16	APB UART 16550 — Interfaces

## 16.1	Overview

Five external interfaces define how this block talks to the rest of your system: the APB slave port, the serial pins, the modem pins, the interrupt output, and the clock/reset inputs. This chapter specifies each one.

## 16.2	Ports

### 16.2.1	External Interfaces

| Interface | Type | Description |
| --- | --- | --- |
| APB Slave | Bus | Configuration and data access |
| Serial | I/O | TXD/RXD serial data |
| Modem | I/O | CTS, RTS, DTR, DSR, DCD, RI |
| Interrupt | Signal | IRQ output |
| Clocks/Reset | System | Clock and reset inputs |

## 16.3	Functional Description

### 16.3.1	Figure 3.1: UART Interface Summary

UART Interfaces

## 16.4	Navigation

### 16.4.1	Chapter Contents

#### 16.4.1.1	APB Slave Interface

Complete APB protocol interface for register access.

See: 01_apb4_slave.md

#### 16.4.1.2	Serial Interface

TXD and RXD serial data connections.

See: 02_serial.md

#### 16.4.1.3	Modem Interface

Hardware flow control and modem signals.

See: 03_modem.md

#### 16.4.1.4	Interrupt Interface

Interrupt request output signal.

See: 04_interrupt.md

#### 16.4.1.5	System Interface

Clock and reset signal requirements.

See: 05_system.md

Next: 01_apb4_slave.md - APB Slave Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 17	APB UART 16550 — APB Slave Interface

## 17.1	Overview

This is the bus-level view of the block: the APB signals, the address map they reach, and the side effects each register access triggers. The side-effect table is the part worth memorizing — reads and writes here are not always free of consequences.

## 17.2	Ports

### 17.2.1	APB Slave Signals

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| pclk | 1 | I | APB clock |
| presetn | 1 | I | APB reset (active low) |
| s_apb_psel | 1 | I | Peripheral select |
| s_apb_penable | 1 | I | Enable phase |
| s_apb_pwrite | 1 | I | Write transaction |
| s_apb_paddr | 12 | I | Address bus |
| s_apb_pwdata | 32 | I | Write data |
| s_apb_pstrb | 4 | I | Byte strobes |
| s_apb_prdata | 32 | O | Read data |
| s_apb_pready | 1 | O | Ready response |
| s_apb_pslverr | 1 | O | Slave error |

## 17.3	Functional Description

### 17.3.1	Address Map

Each register has a unique offset, and LCR[7] (DLAB) additionally remaps 0x00/0x04 to DLL/DLM while it is set; DLL/DLM keep their dedicated offsets either way. Only paddr[5:0] is decoded, so the block aliases every 0x40 bytes across the window.

| Offset | Read | Write |
| --- | --- | --- |
| 0x00 | RBR | THR |
| 0x04 | IER | IER |
| 0x08 | IIR | - |
| 0x0C | FCR | FCR |
| 0x10 | LCR | LCR |
| 0x14 | MCR | MCR |
| 0x18 | LSR | LSR (error bits clear on read) |
| 0x1C | MSR | MSR (delta bits clear on read) |
| 0x20 | SCR | SCR |
| 0x24 | DLL | DLL |
| 0x28 | DLM | DLM |

### 17.3.2	Protocol Compliance

#### 17.3.2.1	APB3/APB4 Features

| Feature | Support |
| --- | --- |
| PSEL | Yes |
| PENABLE | Yes |
| PWRITE | Yes |
| PADDR | 12-bit |
| PWDATA | 32-bit |
| PRDATA | 32-bit |
| PREADY | Yes (inserts wait states – see Access Timing above) |
| PSLVERR | Yes - asserted on any unmapped offset in the window |
| PSTRB | Yes |
| PPROT | Present (s_apb_PPROT[2:0], accepted and unused) |

### 17.3.3	Register Access

#### 17.3.3.1	Byte Access

32-bit APB with 8-bit registers: - pstrb[0]: Access register at paddr - Other strobes: No effect (registers are 8-bit)

#### 17.3.3.2	Side Effects

Some registers have read/write side effects:

| Register | Read Side Effect | Write Side Effect |
| --- | --- | --- |
| RBR | Pops RX FIFO | N/A |
| THR | N/A | Pushes TX FIFO |
| IIR | None (reading IIR does not clear any interrupt) | N/A |
| FCR | None (FCR is readable) | Can reset FIFOs |
| LSR | Clears error bits [4:1] | – |
| MSR | Clears delta bits [3:0] | – |

Note: LSR error bits [4:1] and MSR delta bits [3:0] clear when their own register is read, as a standard 16550 does, and the line-status and modem-status interrupts deassert with them. Any read clears them, including one that was only checking data-ready, so software must work from the value the read returned.

## 17.4	Timing

### 17.4.1	Access Timing

PREADY inserts wait states: the apb4_slave bridge is a small FSM that issues the command and returns the response over a registered cmd/rsp handshake, so an access takes several pclk beyond the 2-cycle APB minimum (more when CDC_ENABLE=1). The register block itself never stalls.

## 17.5	Navigation

Next: 02_serial.md - Serial Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 18	APB UART 16550 — Serial Interface

## 18.1	Overview

Two pins, uart_tx and uart_rx, carry everything. This page defines what those pins do electrically and logically — frame formats, sampling behavior, break handling — including the places where this RTL falls short of what the waveform diagrams were drawn to show.

## 18.2	Ports

### 18.2.1	Signal Description

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| uart_tx | 1 | O | Serial transmit data |
| uart_rx | 1 | I | Serial receive data |

### 18.2.2	Electrical Interface

#### 18.2.2.1	TTL Level

| State | Voltage |
| --- | --- |
| Logic 0 (Space) | 0V |
| Logic 1 (Mark) | VCC (3.3V/5V) |

#### 18.2.2.2	RS-232 Level (External Transceiver)

| State | Voltage |
| --- | --- |
| Logic 0 (Space) | +3V to +15V |
| Logic 1 (Mark) | -3V to -15V |

## 18.3	Functional Description

### 18.3.1	TXD (Transmit Data)

#### 18.3.1.1	Characteristics

| Parameter | Value |
| --- | --- |
| Idle State | Logic 1 (Mark) |
| Start Bit | Logic 0 (Space) |
| Stop Bit | Logic 1 (Mark) |
| Bit Order | LSB first |

#### 18.3.1.2	Frame Format

IDLE  START  D0  D1  D2  D3  D4  D5  D6  D7  PAR  STOP  IDLE
  1     0    |<-------- Data Bits -------->|  P    1     1
             |<-------- LSB first -------->|

#### 18.3.1.3	Output Timing

| Event | Timing |
| --- | --- |
| Bit transition | On baud clock edge |
| Bit duration | 16 x (16x_clk period) |
| Start to first data | 1 bit time |

### 18.3.2	RXD (Receive Data)

#### 18.3.2.1	Characteristics

| Parameter | Value |
| --- | --- |
| Idle State | Logic 1 (Mark) |
| Break | Extended Logic 0 |
| Sampling | Mid-bit (8th of 16 clocks) |

#### 18.3.2.2	Input Synchronization

RXD --> FF1 --> FF2 --> Synchronized RXD
       (clk)   (clk)

- Two-stage synchronizer

- Prevents metastability

- 2 clock cycle latency

#### 18.3.2.3	Start Bit Detection

- Detect falling edge (1 to 0)

- Wait 8 clocks (half bit)

- Verify still 0

- Begin data sampling

### 18.3.3	Data Formats

#### 18.3.3.1	Configurable Parameters (LCR)

| Parameter | Options |
| --- | --- |
| Data bits | 5, 6, 7, or 8 |
| Stop bits | 1 or 2 (no 1.5; and STB=1 with a 5-bit word still sends ONE stop bit) |
| Parity | None, Even, Odd, Mark, Space |

#### 18.3.3.2	Frame Examples

8N1 (8 data, No parity, 1 stop):

START | D0 D1 D2 D3 D4 D5 D6 D7 | STOP
  0   |<------ 8 bits -------->|  1

7E1 (7 data, Even parity, 1 stop):

START | D0 D1 D2 D3 D4 D5 D6 | EP | STOP
  0   |<---- 7 bits ------->|  P |  1

5N1 (5 data, No parity, 1 stop) – note 5N2 does not exist in this RTL (a 5-bit word always gets one stop bit regardless of STB):

START | D0 D1 D2 D3 D4 | STOP
  0   |<-- 5 bits --->|  1

### 18.3.4	Break Condition

#### 18.3.4.1	Transmit Break

When LCR.BC=1: - TXD forced to Logic 0 - Maintained until BC cleared - Minimum duration: 1 frame time

#### 18.3.4.2	Receive Break

Detected when: - RXD = 0 for entire frame - Start + all data + parity + stop = 0 - Sets the BI bit in LSR and tags the FIFO entry. Exactly one zero character is loaded per break; the receiver then stays off until the line returns to marking and a genuine new start bit arrives

## 18.4	Waveforms

### 18.4.1	Waveform 3.1: Line Status Error Detection

The following diagram shows framing error detection when a stop bit is sampled as 0.

UART Line Status

Error detection sequence: 1. RX frame received normally (start, data bits) 2. Stop bit expected to be 1, but sampled as 0 3. Framing error flag set, and the character’s FIFO entry tagged 4. LSR[3] (FE) reads 1 on the read that returns that character 5. Line status interrupt asserts, if enabled in IER[2]

Error types: - Framing Error (FE): Stop bit not 1 - indicates baud rate mismatch or noise - Parity Error (PE): Calculated vs received parity mismatch - Overrun Error (OE): RX FIFO full when new character arrives - Break Indicator (BI): All bits including stop are 0

## 18.5	Navigation

Next: 03_modem.md - Modem Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 19	APB UART 16550 — Modem Interface

## 19.1	Overview

The modem interface carries the classic flow-control and status pins: RTS/DTR/OUT1/OUT2 out, CTS/DSR/DCD/RI in. Manage your expectations here — the automatic flow control you’d get from a real TL16C550C does not exist in this RTL, so software does the work.

## 19.2	Ports

### 19.2.1	Modem Control Outputs (Active Low)

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| rts_n | 1 | O | Request To Send |
| dtr_n | 1 | O | Data Terminal Ready |
| out1_n | 1 | O | User output 1 |
| out2_n | 1 | O | User output 2 |

### 19.2.2	Modem Status Inputs (Active Low)

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| cts_n | 1 | I | Clear To Send |
| dsr_n | 1 | I | Data Set Ready |
| dcd_n | 1 | I | Data Carrier Detect |
| ri_n | 1 | I | Ring Indicator |

## 19.3	Functional Description

### 19.3.1	Modem Control Register (MCR)

#### 19.3.1.1	Output Control

| Bit | Name | Signal | Active When |
| --- | --- | --- | --- |
| 0 | DTR | dtr_n | MCR[0] = 1 |
| 1 | RTS | rts_n | MCR[1] = 1 |
| 2 | OUT1 | out1_n | MCR[2] = 1 |
| 3 | OUT2 | out2_n | MCR[3] = 1 (also gates the irq pin) |
| 4 | LOOP | - | Loopback mode |

MCR is 6 bits wide: bit 5 is AFE (auto flow control, see below). OUT2 additionally gates the irq output: the pin can assert only when MCR.OUT2 = 1.

#### 19.3.1.2	Auto Flow Control (AFE)

MCR[5] enables auto flow control. With it set, the transmitter starts a character only while CTS is asserted - the character already in the shifter always finishes, AFE gates the start and not the frame - and RTS is driven from the RX FIFO level rather than from MCR[1]: it deasserts at the trigger level and reasserts once software has read the FIFO back below it. MCR[1] must still be set for RTS to be asserted at all. Leave AFE clear for manual flow control: drive MCR.RTS and check MSR.CTS in software.

### 19.3.2	Modem Status Register (MSR)

#### 19.3.2.1	Current State (Read-Only)

| Bit | Name | Source | Meaning |
| --- | --- | --- | --- |
| 4 | CTS | cts_n | Current CTS state |
| 5 | DSR | dsr_n | Current DSR state |
| 6 | RI | ri_n | Current RI state |
| 7 | DCD | dcd_n | Current DCD state |

#### 19.3.2.2	Delta Bits (Clear on Read)

These bits clear when MSR is read, and the modem-status interrupt deasserts with them. Any read clears them, so work from the value the read returned rather than reading MSR a second time.

| Bit | Name | Meaning |
| --- | --- | --- |
| 0 | DCTS | CTS changed since last cleared |
| 1 | DDSR | DSR changed since last cleared |
| 2 | TERI | RI changed from low to high |
| 3 | DDCD | DCD changed since last cleared |

### 19.3.3	Hardware Flow Control

#### 19.3.3.1	RTS/CTS Flow Control

TX Device                    RX Device
    |                            |
    |-------- TXD ------------->|
    |                            |
    |<------- CTS_N ------------|
    |                            |
    |-------- RTS_N ----------->|
    |                            |
    |<------- RXD --------------|

With AFE clear, RTS/CTS flow control is handled in software: 1. Software asserts MCR.RTS when ready to receive 2. Software checks MSR.CTS before sending 3. Hardware does not auto-pause TX on CTS

With AFE set, the hardware does all three.

#### 19.3.3.2	Manual Flow Control

Software controls RTS directly:

// Ready to receive
MCR |= 0x02;   // Assert RTS

// Stop receiving
MCR &= ~0x02;  // Deassert RTS

### 19.3.4	Loopback Mode

When MCR.LOOP = 1: - TXD internally connected to RXD - Modem outputs connected to inputs: - DTR -> DSR - RTS -> CTS - OUT1 -> RI - OUT2 -> DCD - External signals disconnected

Used for: - Self-test - UART verification - Driver testing

### 19.3.5	Input Synchronization

All modem inputs pass through 2-stage synchronizer:

cts_n --> FF1 --> FF2 --> synced_cts_n
         (clk)   (clk)

### 19.3.6	Interrupt Generation

MSR delta bits can generate the modem-status interrupt: - Any delta bit set generates the interrupt, gated by IER[3] - The irq pin is gated by MCR.OUT2 as well - Delta bits clear when MSR is read

## 19.4	Waveforms

### 19.4.1	Waveform 3.2: Modem Status Change Detection

The following diagram shows how modem input changes are detected and reported.

UART Modem Status

The detection sequence: 1. External CTS# falls (device ready to receive) 2. 2-stage synchronizer captures the change 3. Edge detector compares current vs. previous state 4. Delta flag (r_delta_cts) latched 5. Current state (r_cts_state) updated 6. MSR updated, modem status interrupt asserted

## 19.5	Navigation

Next: 04_interrupt.md - Interrupt Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 20	APB UART 16550 — Interrupt Interface

## 20.1	Overview

Before you write a line of ISR code, read the notes below. This RTL’s interrupt path does not behave like the 16550 your driver author probably had in mind — the enables don’t enable, the timeout doesn’t exist, and the pin is gated by a modem output bit.

Implementation notes for this RTL: - Each of the four sources is gated by its own IER bit. A disabled source still sets its status bit; it simply is not an interrupt, does not raise irq, and is not reported by IIR. - The irq pin is gated by MCR.OUT2 (irq = pending & MCR.OUT2). With the reset MCR = 0x00 the pin is masked; software must set OUT2 to route interrupts. - The character timeout fires after four character times with a non-empty RX FIFO and no activity; IIR reads 0x0C. It shares the received-data slot and is gated by IER[0]. - Reading IIR clears the THR-empty interrupt when THR empty is the source it reported, as a standard 16550 does. - LSR[4:1] clear on a read of LSR, and MSR[3:0] on a read of MSR.

## 20.2	Ports

### 20.2.1	Signal Description

| Signal | Width | Dir | Description |
| --- | --- | --- | --- |
| irq | 1 | O | Interrupt request (active high, gated by MCR.OUT2) |

## 20.3	Functional Description

### 20.3.1	Interrupt Sources

#### 20.3.1.1	Priority Order (Highest to Lowest)

| Priority | IIR[3:0] | Source | Clear Method |
| --- | --- | --- | --- |
| 1 | 0110 | Receiver Line Status | Read LSR |
| 2 | 0100 | Received Data Available | Read RBR until DR clears |
| 3 | 0010 | THR Empty | Write THR (fill TX FIFO) |
| 4 | 0000 | Modem Status | Read MSR |

Character Timeout reads IIR = 1100 (0x0C) and shares priority 2 with received-data-available.

#### 20.3.1.2	IIR Encoding

| IIR[3:0] | IIR[0] | Meaning |
| --- | --- | --- |
| xxx0 | 0 | Interrupt pending |
| xxx1 | 1 | No interrupt |

### 20.3.2	Interrupt Enable Register (IER)

These bits are stored and read back but are not connected to the interrupt logic in this RTL - they do not enable or mask any interrupt.

| Bit | Name | Nominal Source (not enforced) |
| --- | --- | --- |
| 0 | ERBFI | Received data available |
| 1 | ETBEI | THR empty |
| 2 | ELSI | Receiver line status |
| 3 | EDSSI | Modem status |

### 20.3.3	Interrupt Identification Register (IIR)

#### 20.3.3.1	Read Format

| Bits | Name | Description |
| --- | --- | --- |
| 0 | IPEND | 0=interrupt pending, 1=none |
| 3:1 | IID | Interrupt ID |
| 5:4 | Reserved |  |
| 7:6 | FIFOEN | 11 if FIFOs enabled |

### 20.3.4	Interrupt Conditions

#### 20.3.4.1	Receiver Line Status (Priority 1)

Triggered by: - Overrun Error (OE) - Parity Error (PE) - Framing Error (FE) - Break Indicator (BI)

Cleared by reading LSR.

#### 20.3.4.2	Received Data Available (Priority 2)

- FCR.FE=1: triggered when RX FIFO >= trigger level

- FCR.FE=0: triggered when data present (DR=1)

- Cleared by reading RBR until the level falls below the trigger / DR clears

#### 20.3.4.3	Character Timeout (Priority 2)

Four character times with the RX FIFO non-empty and neither a new character nor a read. IIR reads 0x0C. Any FIFO activity restarts the timer, and the source exists only in FIFO mode.

#### 20.3.4.4	THR Empty (Priority 3)

Triggered when the TX FIFO is empty (this is a level, THRE = TX FIFO empty).

Cleared by writing THR to refill the TX FIFO. Reading IIR does not clear it.

#### 20.3.4.5	Modem Status (Priority 4)

Triggered by any MSR delta bit: - DCTS (Delta CTS) - DDSR (Delta DSR) - TERI (Trailing Edge RI) - DDCD (Delta DCD)

Cleared by reading MSR.

## 20.4	Timing

### 20.4.1	Interrupt Timing

IIR and irq are purely COMBINATIONAL from the core status flags: the cycle a condition’s flag sets, IIR reflects it and irq asserts (gated by MCR.OUT2); the cycle the condition clears, both deassert. There are no pipeline stages in the interrupt path.

Event --> flag sets --> IIR + IRQ reflect it (same cycle, combinational)
Clear --> flag clears --> IIR + IRQ deassert (same cycle)

## 20.5	Usage Example

### 20.5.1	Software Handling

#### 20.5.1.1	ISR Flow

void uart_isr(void) {
    uint8_t iir = IIR;

    while ((iir & 0x01) == 0) {  // While interrupt pending
        switch (iir & 0x0E) {
            case 0x06:  // Line status
                handle_line_status(LSR);
                break;
            case 0x04:  // RX data available
            case 0x0C:  // Character timeout
                handle_rx_data();
                break;
            case 0x02:  // THR empty
                handle_tx_empty();
                break;
            case 0x00:  // Modem status
                handle_modem_status(MSR);
                break;
        }
        iir = IIR;  // Re-read for next pending
    }
}

## 20.6	Navigation

Next: 05_system.md - System Interface

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 21	APB UART 16550 — System Interface

## 21.1	Overview

The system-level plumbing: clocks, resets, what everything resets to, and what the block looks like from the outside when you drop it into a design. The power-management section is short — because the block gives you nothing to work with there.

## 21.2	Ports

### 21.2.1	Clock Signals

#### 21.2.1.1	pclk - APB Clock

| Parameter | Value |
| --- | --- |
| Purpose | APB interface and UART logic |
| Frequency | 50-200 MHz typical |
| Domain | All internal logic |

#### 21.2.1.2	Baud Rate Derivation

Baud rate is derived from pclk:

Baud Rate = pclk / (16 * Divisor)

### 21.2.2	Reset Signals

#### 21.2.2.1	presetn - APB Reset

| Parameter | Value |
| --- | --- |
| Polarity | Active low |
| Type | Asynchronous assert, synchronous deassert (hand-written logic; the generated register file resets synchronously) |
| Scope | All UART logic |

## 21.3	Functional Description

### 21.3.1	Reset Behavior

#### 21.3.1.1	Register Reset Values

| Register | Reset | Notes |
| --- | --- | --- |
| RBR | Undefined | FIFO content |
| THR | N/A | Write-only |
| IER | 0x00 | Interrupt enables, all four sources |
| IIR | 0x02 | THR-empty pending at reset |
| FCR | 0x00 | FIFO-enable interface bit clear |
| LCR | 0x03 | 8N1 format |
| MCR | 0x00 | Outputs deasserted (irq masked - OUT2=0) |
| LSR | 0x60 | TX empty |
| MSR | 0x00 | Inputs low |
| SCR | 0x00 | Cleared |
| DLL | 0x01 | Divisor LSB = 1 |
| DLM | 0x00 | Divisor MSB = 0 |

#### 21.3.1.2	Signal States During Reset

| Signal | Reset State |
| --- | --- |
| uart_tx | 1 (Mark/Idle) |
| rts_n | 1 (Deasserted) |
| dtr_n | 1 (Deasserted) |
| out1_n | 1 (Deasserted) |
| out2_n | 1 (Deasserted) |
| irq | 0 (No interrupt) |

#### 21.3.1.3	Post-Reset Initialization

- Set baud rate (write DLL at 0x24, DLM at 0x28 directly - no DLAB toggle)

- Configure line format (LCR at 0x10)

- Enable FIFOs if desired (FCR at 0x0C)

- Configure modem control (MCR at 0x14); set OUT2 to enable the irq pin

- (IER at 0x04 gates each of the four interrupt sources independently)

## 21.4	Timing

### 21.4.1	Reset Sequence

#### 21.4.1.1	Timing

________________________________________
pclk     |  |  |  |  |  |  |  |  |  |  |  |  |  |

                              _____________________
presetn  ____________________|

         |<-- Reset Active -->|<-- Normal Op ----->|

#### 21.4.1.2	Requirements

- Hold reset low for minimum 2 pclk cycles

- Allow 2 cycles after reset before first APB access

- Divisor must be programmed before operation

## 21.5	Design Notes

### 21.5.1	Power Management

#### 21.5.1.1	Clock Gating

The block provides NO gating or wake hooks: there are no power-management ports, and the baud counter free-runs unconditionally. Any clock gating is the integrator’s, done OUTSIDE the block – and gating pclk kills the APB interface while gating uart_clk kills RX sampling. There is no wake-on-activity path.

#### 21.5.1.2	Low Power Hints

- Use FIFO mode to reduce interrupt rate

- Set a higher RX trigger level to reduce interrupt frequency

(Set MCR[5] for auto flow control, which paces the far end from the RX FIFO level.)

### 21.5.2	External Connections

#### 21.5.2.1	Typical System

+------------+
pclk --->|            |---> uart_tx ---> RS-232
presetn->|   UART     |<--- uart_rx <--- Transceiver
uart_clk/uart_rstn -> core domain when CDC_ENABLE=1 (see ch01 clocks)
         |   16550    |
APB <===>|            |<--> Modem signals
         |            |---> irq    ---> Interrupt
         +------------+      |          Controller
                             v

#### 21.5.2.2	Direct Connection (TTL)

For TTL-level serial: - Connect uart_tx/uart_rx directly to 3.3V/5V logic - No level conversion needed - Short cable runs recommended

## 21.6	Navigation

Back to: 00_overview.md - Interfaces Overview

Next Chapter: Chapter 4: Programming Model

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 22	APB UART 16550 — Programming Model

## 22.1	Overview

This chapter is the software view of the block: how to initialize it, move data through it, and live with its interrupt behavior. The register summary below is the quick reference; the quick-start snippet will get characters on the wire.

### 22.1.1	Register Summary

Each register has a unique offset, and DLAB additionally remaps 0x00/0x04 to the divisor latches.

| Offset | Register | Access | Description |
| --- | --- | --- | --- |
| 0x00 | RBR / THR | R / W | Receive Buffer (read) / Transmit Holding (write) |
| 0x04 | IER | RW | Interrupt Enable |
| 0x08 | IIR | RO | Interrupt Identification |
| 0x0C | FCR | RW | FIFO Control |
| 0x10 | LCR | RW | Line Control |
| 0x14 | MCR | RW | Modem Control |
| 0x18 | LSR | RO, clear on read | Line Status |
| 0x1C | MSR | RO, clear on read | Modem Status |
| 0x20 | SCR | RW | Scratch |
| 0x24 | DLL | RW | Divisor Latch LSB |
| 0x28 | DLM | RW | Divisor Latch MSB |

## 22.2	Usage Example

### 22.2.1	Quick Start

#### 22.2.1.1	Minimal Setup (115200 8N1)

// Assuming 48 MHz clock
#define DIVISOR 26  // 48MHz / (16 * 115200) = 26

void uart_init(void) {
    // Set baud rate divisor directly - no DLAB toggle (DLL=0x24, DLM=0x28)
    DLL = DIVISOR & 0xFF;
    DLM = DIVISOR >> 8;

    // 8N1
    LCR = 0x03;

    // Enable FIFOs, reset, trigger=14
    FCR = 0xC7;

    // Set MCR.OUT2 to ungate the irq pin. It gates the pin in addition to
    // IER, and resets to 0, so interrupts stay masked until this is set.
    MCR = 0x08;
}

## 22.3	Navigation

### 22.3.1	Chapter Contents

#### 22.3.1.1	Initialization

Complete UART initialization sequence.

See: 01_initialization.md

#### 22.3.1.2	Data Transfer

Sending and receiving data.

See: 02_data_transfer.md

#### 22.3.1.3	Interrupts

Interrupt configuration and handling.

See: 03_interrupts.md

#### 22.3.1.4	Examples

Complete programming examples.

See: 04_examples.md

Next: 01_initialization.md - Initialization

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 23	APB UART 16550 — Initialization

## 23.1	Overview

Four steps stand between reset and a working serial link: baud rate, line format, FIFOs, and interrupts. Each step below is short, and each has at least one place where this RTL differs from a textbook 16550 — the comments call those out.

## 23.2	Functional Description

### 23.2.1	Baud Rate Calculation

#### 23.2.1.1	Formula

Divisor = Clock_Frequency / (16 * Baud_Rate)

#### 23.2.1.2	Divisor Calculator Function

uint16_t uart_calculate_divisor(uint32_t clock_hz, uint32_t baud) {
    // Round to nearest
    return (clock_hz + (8 * baud)) / (16 * baud);
}

#### 23.2.1.3	Common Clock Frequencies

| Clock | 9600 | 19200 | 38400 | 57600 | 115200 |
| --- | --- | --- | --- | --- | --- |
| 48 MHz | 312 | 156 | 78 | 52 | 26 |
| 50 MHz | 326 | 163 | 81 | 54 | 27 |
| 100 MHz | 651 | 326 | 163 | 109 | 54 |

## 23.3	Usage Example

### 23.3.1	Basic Initialization Sequence

#### 23.3.1.1	Step 1: Set Baud Rate

void uart_set_baud(uint16_t divisor) {
    // DLL (0x24) and DLM (0x28) have dedicated offsets - no DLAB toggle needed.
    DLL = divisor & 0xFF;
    DLM = (divisor >> 8) & 0xFF;
}

#### 23.3.1.2	Step 2: Configure Line Format

void uart_set_format(uint8_t data_bits, uint8_t parity, uint8_t stop_bits) {
    uint8_t lcr = 0;

    // Data bits: 5=0, 6=1, 7=2, 8=3
    lcr |= (data_bits - 5) & 0x03;

    // Stop bits: 1=0, 2=1. STB=1 gives 2 stop bits for 6/7/8-bit words.
    // 1.5 stop bits: a 5-bit word with LCR[2] set sends a half-length second stop bit.
    if (stop_bits == 2) lcr |= 0x04;

    // Parity: 0=none, 1=odd, 2=even, 3=mark, 4=space
    switch (parity) {
        case 1: lcr |= 0x08; break;        // Odd
        case 2: lcr |= 0x18; break;        // Even
        case 3: lcr |= 0x28; break;        // Mark
        case 4: lcr |= 0x38; break;        // Space
    }

    LCR = lcr;
}

#### 23.3.1.3	Step 3: Configure FIFOs

void uart_configure_fifo(uint8_t trigger_level) {
    uint8_t fcr = 0x01;  // Enable FIFOs

    // Trigger level: 0=1, 1=4, 2=8, 3=14
    fcr |= (trigger_level & 0x03) << 6;

    // Reset both FIFOs
    fcr |= 0x06;

    FCR = fcr;
}

#### 23.3.1.4	Step 4: Enable Interrupts

void uart_enable_interrupts(uint8_t mask) {
    // Bits: 0=RX, 1=TX, 2=Line, 3=Modem
    // Each bit gates its own source. The irq pin is gated by MCR.OUT2 as
    // well, so set OUT2 before expecting an interrupt.
    IER = mask & 0x0F;
}

### 23.3.2	Complete Initialization Example

// Common baud rate divisors for 48 MHz clock
#define BAUD_9600    312
#define BAUD_19200   156
#define BAUD_38400   78
#define BAUD_57600   52
#define BAUD_115200  26

void uart_init_115200_8n1(void) {
    // 1. Set baud rate (DLL=0x24, DLM=0x28 - no DLAB toggle)
    DLL = BAUD_115200;       // Low byte
    DLM = 0x00;              // High byte

    // 2. Line format: 8 data bits, no parity, 1 stop bit
    LCR = 0x03;

    // 3. Enable and reset FIFOs, trigger at 14 bytes
    FCR = 0xC7;              // 11000111b

    // 4. Modem control: set OUT2 (bit 3) to ungate the irq pin
    MCR = 0x08;

    // 5. Drain any pending RX data (received byte is in RBR bits [7:0])
    while (LSR & 0x01)
        (void)RBR;

    // NOTE: LSR[4:1] and MSR[3:0] clear when their own register is read,
    // so the drain loop above consumes any error bits it passes over.
}

### 23.3.3	Flow Control Setup (manual)

Auto flow control (AFE, MCR[5]) is implemented. For manual control leave AFE clear and assert RTS in software and monitor MSR.CTS to gate transmission manually.

void uart_assert_rts(void) {
    uint8_t mcr = MCR;
    mcr |= 0x02;             // RTS (bit 1); set bit 5 as well for auto flow control
    MCR = mcr;
}

### 23.3.4	Loopback Mode Setup

void uart_enable_loopback(void) {
    uint8_t mcr = MCR;
    mcr |= 0x10;             // LOOP bit
    MCR = mcr;
}

## 23.4	Navigation

Next: 02_data_transfer.md - Data Transfer

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 24	APB UART 16550 — Data Transfer

## 24.1	Overview

Moving bytes in and out of this UART is ordinary 16550 fare with three local twists, summarized below and repeated wherever they bite. The examples in this page already account for all three.

Implementation notes for this RTL: reading offset 0x00 (RBR) returns the received byte in bits [7:0] and pops the RX FIFO; LSR[4:1] and MSR[3:0] clear on a read of their own register, so a poll of LSR consumes the error bits it reports; and irq is gated by MCR.OUT2 as well as by IER, so interrupt-driven code must set OUT2 = 1. See Chapter 5 for detail.

## 24.2	Usage Example

### 24.2.1	Transmitting Data

#### 24.2.1.1	Polling Mode

void uart_putchar(uint8_t c) {
    // Wait for THR empty
    while ((LSR & 0x20) == 0);

    // Write data
    THR = c;
}

void uart_puts(const char *s) {
    while (*s) {
        uart_putchar(*s++);
    }
}

#### 24.2.1.2	Interrupt-Driven TX

volatile uint8_t tx_buffer[256];
volatile uint8_t tx_head = 0;
volatile uint8_t tx_tail = 0;

void uart_send(const uint8_t *data, size_t len) {
    for (size_t i = 0; i < len; i++) {
        // Add to buffer
        tx_buffer[tx_head++] = data[i];
    }

    // Enable TX interrupt
    IER |= 0x02;
}

// In ISR when THRE interrupt:
void uart_tx_isr(void) {
    while ((LSR & 0x20) && (tx_head != tx_tail)) {
        THR = tx_buffer[tx_tail++];
    }

    if (tx_head == tx_tail) {
        IER &= ~0x02;  // Disable TX interrupt
    }
}

#### 24.2.1.3	Waiting for TX Complete

void uart_flush(void) {
    // Wait for TEMT (both FIFO and shift register empty)
    while ((LSR & 0x40) == 0);
}

### 24.2.2	Receiving Data

#### 24.2.2.1	Polling Mode

int uart_getchar(void) {
    // Check for data available
    if ((LSR & 0x01) == 0) {
        return -1;  // No data
    }

    // Read data (received byte is in [7:0]
    // chapter header note)
    return RBR & 0xFF;
}

int uart_getchar_blocking(void) {
    // Wait for data
    while ((LSR & 0x01) == 0);

    return RBR & 0xFF;
}

#### 24.2.2.2	Interrupt-Driven RX

volatile uint8_t rx_buffer[256];
volatile uint8_t rx_head = 0;
volatile uint8_t rx_tail = 0;

// In ISR when RX data available:
void uart_rx_isr(void) {
    while (LSR & 0x01) {
        rx_buffer[rx_head++] = RBR & 0xFF;
    }
}

int uart_read(uint8_t *data, size_t max_len) {
    size_t count = 0;
    while ((rx_head != rx_tail) && (count < max_len)) {
        data[count++] = rx_buffer[rx_tail++];
    }
    return count;
}

### 24.2.3	Error Handling

#### 24.2.3.1	Checking Line Status

uint8_t uart_check_errors(void) {
    uint8_t lsr = LSR;
    uint8_t errors = 0;

    if (lsr & 0x02) errors |= ERR_OVERRUN;
    if (lsr & 0x04) errors |= ERR_PARITY;
    if (lsr & 0x08) errors |= ERR_FRAMING;
    if (lsr & 0x10) errors |= ERR_BREAK;

    return errors;
}

#### 24.2.3.2	Handling Errors in RX ISR

void uart_rx_error_isr(void) {
    uint8_t lsr = LSR;  // Read status once; the read clears the error bits

    if (lsr & 0x02) {
        // Overrun - data lost
        stats.overrun_count++;
    }
    if (lsr & 0x04) {
        // Parity error
        stats.parity_count++;
    }
    if (lsr & 0x08) {
        // Framing error
        stats.framing_count++;
    }
    if (lsr & 0x10) {
        // Break received
        handle_break_condition();
    }
}

### 24.2.4	FIFO Management

#### 24.2.4.1	FIFO Status

bool uart_tx_fifo_empty(void) {
    return (LSR & 0x20) != 0;  // THRE
}

bool uart_tx_complete(void) {
    return (LSR & 0x40) != 0;  // TEMT
}

bool uart_rx_data_ready(void) {
    return (LSR & 0x01) != 0;  // DR
}

bool uart_rx_fifo_error(void) {
    return (LSR & 0x80) != 0;  // FIFO error
}

#### 24.2.4.2	FIFO Reset

void uart_reset_fifos(void) {
    // Reset both FIFOs while keeping enabled
    uint8_t fcr = FCR;
    FCR = fcr | 0x06;  // RFR + TFR
}

### 24.2.5	Bulk Transfer

#### 24.2.5.1	Efficient TX (FIFO-aware)

size_t uart_write(const uint8_t *data, size_t len) {
    size_t sent = 0;

    while (sent < len) {
        // Wait for FIFO space
        if (LSR & 0x20) {
            // Fill FIFO (up to 16 bytes)
            size_t chunk = (len - sent < 16) ? (len - sent) : 16;
            for (size_t i = 0; i < chunk; i++) {
                THR = data[sent++];
            }
        }
    }

    return sent;
}

#### 24.2.5.2	Efficient RX (FIFO-aware)

size_t uart_read_available(uint8_t *data, size_t max_len) {
    size_t count = 0;

    while ((LSR & 0x01) && (count < max_len)) {
        data[count++] = RBR & 0xFF;  // received byte is at [7:0]
    }

    return count;
}

## 24.3	Navigation

Next: 03_interrupts.md - Interrupts

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 25	APB UART 16550 — Interrupt Handling

## 25.1	Overview

Interrupt handling follows the 16550 datasheet, with one gap. The note below is the contract; everything else on this page follows from it.

Implementation note: each of the four sources is gated by its own IER bit, and irq is additionally gated by MCR.OUT2 (set OUT2 = 1 to route the pin). LSR[4:1] clear on a read of LSR and MSR[3:0] on a read of MSR; reading IIR clears the THR-empty interrupt when that is the source it reported. The character timeout shares the received-data slot, reads IIR = 0x0C and is gated by IER[0].

## 25.2	Functional Description

### 25.2.1	Interrupt Identification Register (IIR)

#### 25.2.1.1	IIR Values

| Value | Priority | Interrupt Source | Clear Method |
| --- | --- | --- | --- |
| 0x01 | - | No interrupt | - |
| 0x06 | 1 | Line status error | Read LSR to clear the error bits |
| 0x04 | 2 | RX data available | Read RBR until DR clears |
| 0x02 | 3 | THR empty | Write THR (fills TX FIFO) |
| 0x00 | 4 | Modem status | Read MSR to clear the delta bits |

Note: Character timeout (IIR = 0x0C) fires after four character times of inactivity with a non-empty RX FIFO. Reading IIR has no side effect (it does not clear the THR-empty condition).

## 25.3	Usage Example

### 25.3.1	Interrupt Enable Register (IER)

// Enable specific interrupts
#define IER_RDA     0x01    // Received Data Available
#define IER_THRE    0x02    // THR Empty
#define IER_RLS     0x04    // Receiver Line Status
#define IER_MS      0x08    // Modem Status

void uart_enable_rx_interrupt(void) {
    IER |= IER_RDA;
}

void uart_enable_tx_interrupt(void) {
    IER |= IER_THRE;
}

void uart_disable_tx_interrupt(void) {
    IER &= ~IER_THRE;
}

### 25.3.2	Complete ISR Example

void uart_isr(void) {
    uint8_t iir;

    // Loop while interrupts pending
    while (((iir = IIR) & 0x01) == 0) {
        switch (iir & 0x0E) {
            case 0x06:  // Receiver Line Status (highest priority)
                uart_handle_line_status();
                break;

            case 0x04:  // Received Data Available
                uart_handle_rx_data();
                break;

            case 0x0C:  // Character Timeout
                uart_handle_timeout();
                break;

            case 0x02:  // THR Empty
                uart_handle_tx_empty();
                break;

            case 0x00:  // Modem Status (lowest priority)
                uart_handle_modem_status();
                break;
        }
    }
}

### 25.3.3	Individual Interrupt Handlers

#### 25.3.3.1	Line Status Handler

void uart_handle_line_status(void) {
    uint8_t lsr = LSR;  // Read status; this read also clears the error bits

    if (lsr & 0x02) {
        // Overrun Error - FIFO overflow
        stats.overrun++;
    }
    if (lsr & 0x04) {
        // Parity Error
        stats.parity_err++;
    }
    if (lsr & 0x08) {
        // Framing Error
        stats.framing_err++;
    }
    if (lsr & 0x10) {
        // Break Indicator
        handle_break();
    }

    // Nothing to write back: LSR[4:1] were cleared by the read above.
    // That is also why this handler must work from `lsr`, the value the
    // read returned, and must not read LSR again.
}

#### 25.3.3.2	RX Data Handler

void uart_handle_rx_data(void) {
    // Read all available data from FIFO
    while (LSR & 0x01) {
        uint8_t data = RBR & 0xFF;  // received byte is at [7:0]
        rx_buffer[rx_head++] = data;

        if (rx_head >= RX_BUFFER_SIZE) {
            rx_head = 0;
        }
    }

    // Signal waiting thread/task
    signal_rx_available();
}

#### 25.3.3.3	Character Timeout Handler

// Called when IIR reads 0x0C: four character times with data sitting in
// the RX FIFO below the trigger level.
void uart_handle_timeout(void) {
    // Same as RX data - flush remaining FIFO data
    uart_handle_rx_data();

    // May want to signal "end of packet" condition
    signal_rx_timeout();
}

#### 25.3.3.4	TX Empty Handler

void uart_handle_tx_empty(void) {
    // Fill TX FIFO from buffer
    while ((LSR & 0x20) && (tx_tail != tx_head)) {
        THR = tx_buffer[tx_tail++];

        if (tx_tail >= TX_BUFFER_SIZE) {
            tx_tail = 0;
        }
    }

    // If buffer empty, disable TX interrupt
    if (tx_tail == tx_head) {
        IER &= ~IER_THRE;
        signal_tx_complete();
    }
}

#### 25.3.3.5	Modem Status Handler

void uart_handle_modem_status(void) {
    uint8_t msr = MSR;  // Read status; this read also clears the delta bits

    if (msr & 0x01) {   // Delta CTS
        // CTS changed - update flow control
    }
    if (msr & 0x02) {   // Delta DSR
        // DSR changed - device status
    }
    if (msr & 0x04) {   // Trailing Edge RI
        // Ring detected
    }
    if (msr & 0x08) {   // Delta DCD
        // Carrier changed - connection status
    }

    // Nothing to write back: MSR[3:0] were cleared by the read above,
    // which is why this handler works from `msr` and does not re-read.
}

### 25.3.4	Disabling/Enabling Interrupts

IER = 0x00 disables all four sources. MCR.OUT2 gates the pin as well, so clearing it masks the pin whatever IER says:

// Mask the irq pin via the OUT2 gate
uint8_t saved_mcr = MCR;
MCR = saved_mcr & ~0x08;   // OUT2 = 0 -> irq pin held deasserted

// ... critical section ...

// Restore
MCR = saved_mcr;

## 25.4	Design Notes

### 25.4.1	Interrupt Latency Considerations

#### 25.4.1.1	Trigger Level Selection

| Trigger | Bytes in FIFO | Latency Budget | Best For |
| --- | --- | --- | --- |
| 1 | 1 | 1 char time | Low latency |
| 4 | 4 | 4 char times | Balanced |
| 8 | 8 | 8 char times | Higher rates |
| 14 | 14 | 2 char times* | Maximum efficiency |

*Only 2 characters before overflow at 16-byte FIFO

#### 25.4.1.2	Character Timeout

- Implemented: four character times with a non-empty RX FIFO and no activity, IIR reads 0x0C.

- For variable-length packets, poll LSR.DR and apply a software inactivity timeout instead.

## 25.5	Navigation

Next: 04_examples.md - Examples

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 26	APB UART 16550 — Programming Examples

## 26.1	Overview

Complete, working examples that pull together everything from the previous pages: a polled debug console, an interrupt-driven ring buffer, a small command-line interface, a loopback self-test, and manual flow control. All of them already account for this RTL’s quirks — RBR at [7:0], no DLAB remap — so you can lift them directly.

## 26.2	Usage Example

### 26.2.1	Debug Console

#### 26.2.1.1	Simple Polling Implementation

#define UART_BASE   0xFEC08000

// Each register has its own offset; this example uses those flat offsets.
// (LCR[7]/DLAB additionally remaps 0x00/0x04 to DLL/DLM when set.)
// RBR read returns the received byte in bits [7:0], so read RBR as 16-bit.
#define RBR  (*(volatile uint16_t *)(UART_BASE + 0x00))
#define THR  (*(volatile uint8_t  *)(UART_BASE + 0x00))
#define IER  (*(volatile uint8_t  *)(UART_BASE + 0x04))
#define IIR  (*(volatile uint8_t  *)(UART_BASE + 0x08))
#define FCR  (*(volatile uint8_t  *)(UART_BASE + 0x0C))
#define LCR  (*(volatile uint8_t  *)(UART_BASE + 0x10))
#define MCR  (*(volatile uint8_t  *)(UART_BASE + 0x14))
#define LSR  (*(volatile uint8_t  *)(UART_BASE + 0x18))
#define MSR  (*(volatile uint8_t  *)(UART_BASE + 0x1C))
#define SCR  (*(volatile uint8_t  *)(UART_BASE + 0x20))
#define DLL  (*(volatile uint8_t  *)(UART_BASE + 0x24))
#define DLM  (*(volatile uint8_t  *)(UART_BASE + 0x28))

// Received byte is in RBR bits [7:0].
#define RBR_BYTE()  ((uint8_t)(RBR & 0xFF))

void debug_uart_init(void) {
    // 115200 baud, 8N1. No DLAB toggle - DLL/DLM have their own offsets.
    DLL = 26;             // 48MHz / (16 * 115200)
    DLM = 0;
    LCR = 0x03;           // 8N1
    FCR = 0x07;           // Enable FIFOs, reset
    // IER left at 0: this console polls LSR rather than taking interrupts.
}

void debug_putchar(char c) {
    while ((LSR & 0x20) == 0);
    THR = c;
}

void debug_puts(const char *s) {
    while (*s) {
        if (*s == '\n')
            debug_putchar('\r');
        debug_putchar(*s++);
    }
}

int debug_getchar(void) {
    if (LSR & 0x01)
        return RBR_BYTE();   // received byte is in RBR bits [7:0]
    return -1;
}

### 26.2.2	Ring Buffer Implementation

#define RX_BUF_SIZE 256
#define TX_BUF_SIZE 256

typedef struct {
    volatile uint8_t buffer[RX_BUF_SIZE];
    volatile uint16_t head;
    volatile uint16_t tail;
} ring_buffer_t;

static ring_buffer_t rx_buf;
static ring_buffer_t tx_buf;

void uart_isr(void) {
    uint8_t iir;

    while (((iir = IIR) & 0x01) == 0) {
        switch (iir & 0x0E) {
            case 0x0C:  // character timeout - fall through, drain the FIFO
            case 0x04:  // RX data available
                while (LSR & 0x01) {
                    uint16_t next = (rx_buf.head + 1) % RX_BUF_SIZE;
                    if (next != rx_buf.tail) {
                        rx_buf.buffer[rx_buf.head] = RBR_BYTE();
                        rx_buf.head = next;
                    } else {
                        (void)RBR;  // Discard if full (still pops the FIFO)
                    }
                }
                break;

            case 0x02:  // TX empty
                while ((LSR & 0x20) && (tx_buf.tail != tx_buf.head)) {
                    THR = tx_buf.buffer[tx_buf.tail];
                    tx_buf.tail = (tx_buf.tail + 1) % TX_BUF_SIZE;
                }
                if (tx_buf.tail == tx_buf.head) {
                    IER &= ~0x02;  // Disable TX int
                }
                break;
        }
    }
}

size_t uart_write(const uint8_t *data, size_t len) {
    size_t written = 0;

    while (written < len) {
        uint16_t next = (tx_buf.head + 1) % TX_BUF_SIZE;
        if (next == tx_buf.tail) break;  // Buffer full

        tx_buf.buffer[tx_buf.head] = data[written++];
        tx_buf.head = next;
    }

    IER |= 0x02;  // Enable TX interrupt
    return written;
}

size_t uart_read(uint8_t *data, size_t max) {
    size_t count = 0;

    while ((rx_buf.tail != rx_buf.head) && (count < max)) {
        data[count++] = rx_buf.buffer[rx_buf.tail];
        rx_buf.tail = (rx_buf.tail + 1) % RX_BUF_SIZE;
    }

    return count;
}

### 26.2.3	Command Line Interface

#define CMD_BUF_SIZE 128

static char cmd_buf[CMD_BUF_SIZE];
static uint8_t cmd_pos = 0;

void cli_process_char(char c) {
    switch (c) {
        case '\r':
        case '\n':
            debug_puts("\r\n");
            cmd_buf[cmd_pos] = '\0';
            cli_execute(cmd_buf);
            cmd_pos = 0;
            debug_puts("> ");
            break;

        case '\b':
        case 0x7F:  // DEL
            if (cmd_pos > 0) {
                cmd_pos--;
                debug_puts("\b \b");
            }
            break;

        default:
            if (cmd_pos < CMD_BUF_SIZE - 1) {
                cmd_buf[cmd_pos++] = c;
                debug_putchar(c);
            }
            break;
    }
}

void cli_execute(const char *cmd) {
    if (strcmp(cmd, "help") == 0) {
        debug_puts("Available commands:\r\n");
        debug_puts("  help   - Show this help\r\n");
        debug_puts("  status - Show UART status\r\n");
    }
    else if (strcmp(cmd, "status") == 0) {
        char buf[64];
        sprintf(buf, "LSR: 0x%02X  MSR: 0x%02X\r\n", LSR, MSR);
        debug_puts(buf);
    }
    else if (cmd[0] != '\0') {
        debug_puts("Unknown command: ");
        debug_puts(cmd);
        debug_puts("\r\n");
    }
}

### 26.2.4	Loopback Test

bool uart_loopback_test(void) {
    uint8_t test_data[] = {0x00, 0x55, 0xAA, 0xFF};

    // Enable loopback mode
    MCR |= 0x10;

    // Clear FIFOs
    FCR = 0x07;

    // Send test data
    for (int i = 0; i < sizeof(test_data); i++) {
        THR = test_data[i];
    }

    // Wait for transmission
    while ((LSR & 0x40) == 0);

    // Short delay for internal loopback
    for (volatile int i = 0; i < 100; i++);

    // Read back and verify
    for (int i = 0; i < sizeof(test_data); i++) {
        if (!(LSR & 0x01)) {
            MCR &= ~0x10;
            return false;  // No data received
        }

        uint8_t received = RBR_BYTE();   // received byte is in RBR bits [7:0]
        if (received != test_data[i]) {
            MCR &= ~0x10;
            return false;  // Data mismatch
        }
    }

    // Disable loopback
    MCR &= ~0x10;

    return true;
}

### 26.2.5	Flow Control (manual)

void uart_init_flow_control(void) {
    // Standard init - no DLAB toggle; DLL/DLM have their own offsets.
    DLL = 26; DLM = 0; LCR = 0x03;
    FCR = 0xC7;  // Enable FIFOs, trigger=14

    // Assert RTS (bit 1) and set OUT2 (bit 3) so the irq pin is ungated.
    MCR = 0x0A;
}

// NOTE: this example does flow control by hand. Set MCR[5] (AFE) instead
// and the hardware does it: CTS gates the start of a character and RTS
// follows the RX FIFO level. MCR[1] must still be set for RTS to assert.

## 26.3	Navigation

Back to: 00_overview.md - Programming Model Overview

Next Chapter: Chapter 5: Registers

RTL Design Sherpa · Learning Hardware Design Through Practice  GitHub · Documentation Index · MIT License

# 27	APB UART 16550 — Register Map

## 27.1	Overview

This is the definitive register reference, and it documents what the RTL actually does — not what a 16550 datasheet says it should do. The address map is additive: every register has its own unique offset, AND the Divisor Latch Access Bit (LCR[7]) remaps 0x00/0x04 to DLL/DLM while it is set, as a classic 16550 does. DLL/DLM also keep their dedicated offsets (0x24/0x28), so both access styles work (RLB TASK-012, was RLB-013). Register offsets are byte offsets; only paddr[5:0] is decoded (see Address Calculation).

## 27.2	Functional Description

### 27.2.1	Register Summary

| Offset | Register | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x00 | RBR / THR | R / W | - | Receive Buffer (read) / Transmit Holding (write) |
| 0x04 | IER | RW | 0x00 | Interrupt Enable |
| 0x08 | IIR | RO | 0x02 | Interrupt Identification |
| 0x0C | FCR | RW | 0x00 | FIFO Control (readable in this implementation) |
| 0x10 | LCR | RW | 0x03 | Line Control (8N1 at reset) |
| 0x14 | MCR | RW | 0x00 | Modem Control |
| 0x18 | LSR | RO, clear on read | 0x60 | Line Status |
| 0x1C | MSR | RO, clear on read | 0x00 | Modem Status |
| 0x20 | SCR | RW | 0x00 | Scratch |
| 0x24 | DLL | RW | 0x01 | Divisor Latch LSB |
| 0x28 | DLM | RW | 0x00 | Divisor Latch MSB |

Offsets 0x2C-0x3F are unused and acknowledge reads as zero.

### 27.2.2	RBR - Receiver Buffer Register (0x00, Read)

| Bits | Name | Access | Description |
| --- | --- | --- | --- |
| 7:0 | DATA | RO | Received data byte |
| 15:8 | DATA_ALIAS | RO | The same byte again (see note) |

Note: Reading offset 0x00 pops one entry from the RX FIFO and returns the received byte in bits [7:0], as a standard 16550 does. THR shares the offset and is write-only: it has no storage and does not read back anywhere. Bits [15:8] mirror the same received byte and exist only for testbench helpers that predate the fix; do not write new software against them.

### 27.2.3	THR - Transmitter Holding Register (0x00, Write)

| Bits | Name | Access | Description |
| --- | --- | --- | --- |
| 7:0 | DATA | WO | Data to transmit |

Note: Writing offset 0x00 pushes data to the TX FIFO. The written value is also stored and is what appears in bits [7:0] of a subsequent RBR read (see RBR note); it is not otherwise software-visible.

### 27.2.4	IER - Interrupt Enable Register (0x04, R/W)

| Bit | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0 | ERBFI | RW | 0 | Enable Received Data Available Interrupt |
| 1 | ETBEI | RW | 0 | Enable Transmitter Holding Register Empty |
| 2 | ELSI | RW | 0 | Enable Receiver Line Status Interrupt |
| 3 | EDSSI | RW | 0 | Enable Modem Status Interrupt |
| 7:4 | Reserved | RO | 0 | Reserved |

Note: Each bit gates its own source. A disabled source still sets its status bit in LSR or MSR; it simply is not an interrupt, does not raise irq and is not reported by IIR. The irq pin is additionally gated by MCR.OUT2, which resets to 0. See ch03_interfaces/04_interrupt.md.

### 27.2.5	IIR - Interrupt Identification Register (0x08, Read Only)

| Bits | Name | Access | Description |
| --- | --- | --- | --- |
| 0 | IPEND | RO | 0=Interrupt pending, 1=No interrupt |
| 3:1 | IID | RO | Interrupt ID (see table below) |
| 5:4 | Reserved | RO | Reserved |
| 7:6 | FIFOEN | RO | 11=FIFOs enabled, 00=disabled |

#### 27.2.5.1	Interrupt ID Encoding

IIR[0]=IPEND (0 = interrupt pending). IIR[3:1]=IID. IIR[3] (timeout) is always 0 in this implementation.

| IIR[3:0] | Priority | Source | Clear Method |
| --- | --- | --- | --- |
| 0110 | 1 | Line Status | Read LSR to clear the error bits |
| 0100 | 2 | RX Data Available | Read RBR until DR clears |
| 0010 | 3 | THR Empty | Write THR (fills TX FIFO) |
| 0000 | 4 | Modem Status | Read MSR to clear the delta bits |

Note: The Character Timeout interrupt (IIR = 0x0C, IIR[3]) is implemented. Reading IIR has no side effect; it does not clear the THR-empty condition. The THR-empty source is the level “TX FIFO empty” and clears only when the FIFO is refilled. Because interrupt enables are unimplemented (see IER), IIR reflects pending sources regardless of IER, and the irq pin is additionally gated by MCR.OUT2 (see MCR note).

### 27.2.6	FCR - FIFO Control Register (0x0C, R/W)

FCR is fully readable in this implementation (a standard 16550 FCR is write-only).

| Bit | Name | Access | Description |
| --- | --- | --- | --- |
| 0 | FE | RW | FIFO Enable |
| 1 | RFR | RW | RX FIFO Reset (self-clearing) |
| 2 | TFR | RW | TX FIFO Reset (self-clearing) |
| 3 | DMS | RW | DMA Mode Select (stored; no effect - no DMA signals exist) |
| 5:4 | Reserved | RO | Reserved (read as 0) |
| 7:6 | RTL | RW | RX Trigger Level |

#### 27.2.6.1	RX Trigger Level

| RTL[1:0] | Trigger Level |
| --- | --- |
| 00 | 1 byte |
| 01 | 4 bytes |
| 10 | 8 bytes |
| 11 | 14 bytes |

### 27.2.7	LCR - Line Control Register (0x10, R/W)

Reset value is 0x03 (8 data bits, 1 stop bit, no parity - 8N1).

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 1:0 | WLS | RW | 11 | Word Length Select (reset = 8 bits) |
| 2 | STB | RW | 0 | Stop Bits (see note) |
| 3 | PEN | RW | 0 | Parity Enable |
| 4 | EPS | RW | 0 | Even Parity Select |
| 5 | SP | RW | 0 | Stick Parity |
| 6 | BC | RW | 0 | Break Control |
| 7 | DLAB | RW | 0 | Divisor Latch Access Bit: while set, 0x00 and 0x04 are DLL and DLM |

Note (DLAB): LCR[7] remaps 0x00 to DLL and 0x04 to DLM, as a standard 16550 does. The dedicated offsets 0x24 and 0x28 reach the same latches whatever DLAB says, so either form works. Note (STB): STB=1 selects 2 stop bits for 6/7/8-bit words; 1.5 stop bits (for 5-bit words) is not implemented - a 5-bit word with STB=1 still produces 1 stop bit.

#### 27.2.7.1	Word Length

| WLS[1:0] | Data Bits |
| --- | --- |
| 00 | 5 |
| 01 | 6 |
| 10 | 7 |
| 11 | 8 |

At every word length the received byte is right-justified and zero-filled: a 5-bit character reads in bits [4:0] with [7:5] clear. The transmitter sends bits [N-1:0] LSB-first, so TX and RX agree.

#### 27.2.7.2	Parity Selection

| PEN | EPS | SP | Parity |
| --- | --- | --- | --- |
| 0 | X | X | None |
| 1 | 0 | 0 | Odd |
| 1 | 1 | 0 | Even |
| 1 | 0 | 1 | Mark (1) |
| 1 | 1 | 1 | Space (0) |

### 27.2.8	MCR - Modem Control Register (0x14, R/W)

| Bit | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0 | DTR | RW | 0 | Data Terminal Ready |
| 1 | RTS | RW | 0 | Request To Send |
| 2 | OUT1 | RW | 0 | User Output 1 (active low) |
| 3 | OUT2 | RW | 0 | User Output 2 / interrupt gate (active low) - see note |
| 4 | LOOP | RW | 0 | Loopback Mode |
| 5 | AFE | RW | 0 | Auto Flow Control Enable |
| 7:6 | Reserved | RO | 0 | Reserved (read as 0) |

Note: Bit 5 (AFE) gates the transmitter with CTS and drives RTS from the RX FIFO level; MCR[1] must still be set for RTS to assert. Writes above bit 5 are dropped. There is no CTS-gated transmit and RTS is not auto-driven by RX FIFO level. Additionally, OUT2 gates the irq output pin: irq can assert only when MCR.OUT2 = 1. With the reset value MCR = 0x00, the irq pin is masked; software must set MCR.OUT2 to route interrupts to the pin.

### 27.2.9	LSR - Line Status Register (0x18, Read; error bits clear on read)

| Bit | Name | Access | Description |
| --- | --- | --- | --- |
| 0 | DR | RO | Data Ready |
| 1 | OE | RO | Overrun Error (clears when the register is read) |
| 2 | PE | RO | Parity Error (clears when the register is read) |
| 3 | FE | RO | Framing Error (clears when the register is read) |
| 4 | BI | RO | Break Interrupt (clears when the register is read) |
| 5 | THRE | RO | Transmitter Holding Register Empty (TX FIFO empty) |
| 6 | TEMT | RO | Transmitter Empty (FIFO and shift register empty) |
| 7 | FIFOERR | RO | At least one parity error, framing error or break indication in the RX FIFO (FIFO mode only) |

Note: The error bits [4:1] clear when LSR is read, as PC16550D specifies. Any read clears them, including one that was only looking at DR, so a polling loop consumes the error it is polling for - capture the value the poll returned rather than reading LSR twice. PE, FE and BI are the tags of the character the CPU is being handed, not a running OR over everything received; OE is a port-level flag. FIFOERR (bit 7) aggregates over the whole FIFO and is defined only in FIFO mode.

### 27.2.10	MSR - Modem Status Register (0x1C, Read; delta bits clear on read)

| Bit | Name | Access | Description |
| --- | --- | --- | --- |
| 0 | DCTS | RO | Delta Clear To Send (clears when the register is read) |
| 1 | DDSR | RO | Delta Data Set Ready (clears when the register is read) |
| 2 | TERI | RO | Trailing Edge Ring Indicator (clears when the register is read) |
| 3 | DDCD | RO | Delta Data Carrier Detect (clears when the register is read) |
| 4 | CTS | RO | Clear To Send |
| 5 | DSR | RO | Data Set Ready |
| 6 | RI | RO | Ring Indicator |
| 7 | DCD | RO | Data Carrier Detect |

Note: The delta bits [3:0] clear when MSR is read, as PC16550D specifies, and the modem-status interrupt deasserts with them. As with LSR, any read clears them, so work from the value the read returned rather than reading MSR twice.

### 27.2.11	SCR - Scratch Register (0x20, R/W)

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 7:0 | DATA | RW | 0x00 | General-purpose storage |

### 27.2.12	DLL - Divisor Latch LSB (0x24, R/W)

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 7:0 | DLL | RW | 0x01 | Baud rate divisor low byte |

Note: Reset value is 0x01 (not 0x00). Accessed directly at 0x24 - no DLAB toggle required.

### 27.2.13	DLM - Divisor Latch MSB (0x28, R/W)

| Bits | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 7:0 | DLM | RW | 0x00 | Baud rate divisor high byte |

Note: Accessed directly at 0x28 - no DLAB toggle required.

### 27.2.14	Address Calculation

Register_Address = BASE_ADDR + Register_Offset

Where:
  BASE_ADDR       = RLB/UART base address (system-integration dependent)
  Register_Offset = value from the Register Summary table above

Only paddr[5:0] is decoded, so the block aliases every 0x40 bytes across
its address window.

Example (BASE_ADDR = 0xFEC08000):
  LSR = 0xFEC08000 + 0x18 = 0xFEC08018

## 27.3	Implemented for RLB TASK-012 (was RLB-013)

The five 16550 features this block used to leave out are now built:

| Feature | Behaviour |
| --- | --- |
| Character-timeout interrupt | Four character times with the RX FIFO non-empty and no activity; IIR reads 0x0C, sharing the received-data slot and distinguished by IIR[3]. Gated by IER[0], FIFO mode only |
| Auto flow control (AFE) | MCR[5]. CTS gates the start of a character; RTS follows the RX FIFO level, deasserting at the trigger and reasserting once read below it. MCR[1] must still be set for RTS to assert at all |
| 1.5 stop bits | LCR[2] with a 5-bit character sends a half-length second stop bit: 7.5 bit times against 7 |
| DLAB remapping | While LCR[7] is set, 0x00 and 0x04 are the divisor latches. The flat offsets 0x24 and 0x28 keep working, so both forms reach the same latches |
| DMA mode select | FCR[3] picks the handshake on the rxrdy_n / txrdy_n pins: mode 0 per character, mode 1 per block |

Two items still constrain integration rather than function. The block instantiates no reset synchronizer, so presetn and uart_rstn must arrive already synchronized, and resetting one domain with a transfer in flight corrupts the asynchronous FIFOs - quiesce the bus first, or assert both together. An illegal parameter fails in simulation at time 0 rather than at elaboration, because the guards are initial blocks and synthesis ignores them.